CALIFORNIA REGIONAL WATER QUALITY CONTROL BOARD

SAN FRANCISCO BAY REGION

August 1993

STANDARD PROVISIONS AND REPORTING REQUIREMENTS

For

NPDES SURFACE WATER DISCHARGE PERMITS

A. GENERAL PROVISIONS

1. Neither the treatment nor the discharge of pollutants shall create a pollution, contamination, or nuisance as defined by Section 13050 of the California Water Code.

2. All discharges authorized by this Order shall be consistent with the terms and conditions of this Order.

3. Duty to Comply

a. If a toxic effluent standard or prohibition (including any schedule of compliance specified in such effluent standard or prohibition) is established under Section 307(a) of the Clean Water Act, or amendments thereto, for a toxic pollutant which is present in the discharge authorized herein and such standard or prohibition is more stringent than any limitation upon such pollutant in a Board adopted Order, discharger must comply with the new standard or prohibition.  The Board will revise or modify the Order in accordance with such toxic effluent standard or prohibition and so notify the discharger.

b. If more stringent applicable water quality standards are approved pursuant to Section 303 of the Clean Water Act, or amendments thereto, the discharger must comply with the new standard. The Board will revise and modify this Order in accordance with such more stringent standards.

c. The filing of a request by the discharger for a permit modification, revocation and reissuance, or termination, or a notification of planned changes or anticipated noncompliance does not stay any permit condition. [40 CFR 122.41(f)]

4. Duty to Mitigate

The discharger shall take all reasonable steps to minimize or prevent any discharge in violation of this order and permit which has a reasonable likelihood of adversely affecting public health or the environment, including such accelerated or additional monitoring as requested by the Board or Executive Officer to determine the nature and impact of the violation.  [40 CFR 122.41(d)]

5. Pursuant to U.S. Environmental Protection Agency regulations the discharger must notify the Regional Board as soon as it knows or has reason to believe (1) that they have begun or expect to begin, use or manufacture of a pollutant not reported in the permit application, or (2) a discharge of toxic pollutants not limited by this permit has occurred, or will occur, in concentrations that exceed the limits specified in 40 CFR 122.42(a). 

6. The discharge of any radiological, chemical, or biological warfare agent waste is prohibited.

7. All facilities used for transport, treatment, or disposal of wastes shall be adequately protected against overflow or washout as the result of a 100‑year frequency flood. 

8. Collection, treatment, storage and disposal systems shall be operated in a manner that precludes public contact with wastewater, except where excluding the public is inappropriate, warning signs shall be posted.

9. Property Rights

This Order and Permit does not convey any property rights of any sort or any exclusive privileges. The requirements prescribed herein do not authorize the commission of any act causing injury to the property of another, nor protect the discharger from liabilities under federal, state or local laws, nor create a vested right for the discharge to continue the waste discharge or guarantee the discharger a capacity right in the receiving water. [40 CFR 122.41(g)]

10. Inspection and Entry

The Board or its authorized representatives shall be allowed:

a. Entry upon premises where a regulated facility or activity is located or conducted, or where records are kept under the conditions of the order and permit;

b. Access to and copy at, reasonable times, any records that must be kept under the conditions of the order and permit;

c. To inspect at reasonable times any facility, equipment (including monitoring and control equipment), practices, or operations regulated or required under the order and 
permit; and

d. To photograph, sample, and monitor, at reasonable times for the purpose of assuring compliance with the order and permit or as otherwise authorized by the Clean Water Act, any substances or parameters at any locations. [40 CFR 122.41(i)]

11. Permit Actions

This Order and Permit may be modified, revoked and reissued, or terminated in accordance with applicable State and/or Federal regulations.  Cause for taking such action includes, but is not limited to any of the following:

a. Violation of any term or condition contained in the Order and Permit;

b. Obtaining the Order and Permit by misrepresentation, or by failure to disclose fully all relevant facts;

c. Endangerment to public health or environment that can only be regulated to acceptable levels by order and permit modification or termination; and

d. Any condition that requires either a temporary or permanent reduction or elimination of the authorized discharge. 

12. Duty to Provide Information

The discharger shall furnish, within a reasonable time, any information the Board may request to determine whether cause exists for modifying, revoking and reissuing, or terminating the permit.  The discharger shall also furnish to the Board, upon request, copies of records required to be kept by its permit. [40 CFR 122.41(h)]

13. Bypass (the intentional diversion of waste streams from any portion of a treatment facility) is prohibited. The Board may take enforcement action against the discharger for plant bypass unless:

a. Bypass was unavoidable to prevent loss of life, personal injury, or severe property damage.  (Severe property damage means substantial physical damage to property, damage to the treatment facilities that causes them to become inoperable, or substantial and permanent loss of natural resources that can reasonably be expected to occur in the absence of a bypass.  Severe property damage does not mean economic loss caused by delays in production.);

b. There were no feasible alternatives to the bypass, such as the use of auxiliary treatment facilities, retention of untreated wastes, or maintenance during normal periods of equipment down time.  This condition is not satisfied if adequate back‑up equipment should have been installed in the exercise of reasonable engineering judgment to prevent a bypass which occurred during normal periods of equipment downtime or preventive maintenance; and 

c. The discharger submitted advance notice of the need for a bypass to the Board.  If the discharger knows in advance of the need for a bypass, it shall submit prior notice, if possible at least 10 days before the date of the bypass. The discharger shall submit notice of an unanticipated bypass as required by 40 CFR 122.41(l)(6) (24 hour notice), as required in paragraph E.6.d.   

The discharger may allow a bypass to occur that does not cause effluent limitations to be exceeded, but only if it is for essential maintenance to assure efficient operation. 

14. Availability

A copy of this permit shall be maintained at the discharge facility and be available at all times to operating personnel.

15. Continuation of Expired Permit

This permit continues in force and effect until a new permit is issued or the Board rescinds the permit. Only those dischargers authorized to discharge under the expiring permit are covered by the continued permit.

B. STANDARD STORM WATER PROVISIONS

These provisions apply to facilities which do not direct all storm water flows to the wastewater treatment plant headworks.

1. The Storm Water Pollution Prevention Plan (SWPP Plan) shall be designed in accordance with good engineering practices and shall address the following objectives:

a. to identify pollutant sources that may affect the quality of storm water discharges; and

b. to identify, assign, and implement control measures and management practices to reduce pollutants in storm water discharges.

The SWPP Plan may be combined with the existing spill prevention plan as required in accordance with Provision E.5. The SWPP Plan shall be retained on-site and made available upon request of a representative of the Board.

2. Source Identification

The SWPP Plan shall provide a description of potential sources which may be expected to add significant quantities of pollutants to storm water discharges, or which may result in non-storm water discharges from the facility. The SWPP Plan shall include, at a minimum, the following items:

a. A topographical map (or other acceptable map if a topographical map is unavailable), extending one-quarter mile beyond the property boundaries of the facility, showing: the wastewater treatment facility process areas, surface water bodies (including springs and wells), and the discharge point(s) where the facility's storm water discharges to a municipal storm drain system or other points to waters of the State. The requirements of this paragraph may be included in the site map required under the following paragraph if appropriate.

b. A site map showing:

i. Storm water conveyance, drainage, and discharge structures;

ii. An outline of the storm water drainage areas for each storm water discharge point;

iii. Paved areas and buildings;

iv. Areas of pollutant contact with storm water or release to storm water, actual or potential, including but not limited to outdoor storage, and process areas, material loading, unloading, and access areas, and waste treatment, storage, and disposal areas;

v. Location of existing storm water structural control measures (i.e., berms, coverings, etc.);

vi. Surface water locations, including springs and wetlands;

vii. Vehicle service areas.

c. A narrative description of the following:

i. Wastewater treatment process activity areas;

ii. Materials, equipment, and vehicle management practices employed to minimize contact of significant materials of concern with storm water 
discharges;

iii. Material storage, loading, unloading, and access areas;

iv. Existing structural and non-structural control measures (if any) to reduce pollutants in storm water discharge;

v. Methods of on-site storage and disposal of significant materials.

d. A list of pollutants that have a reasonable potential to be present in storm water discharge in significant quantities.

3. Storm Water Management Controls

The SWPP Plan shall describe the storm water management controls appropriate for the facility and a time schedule for fully implementing such controls. The appropriateness and priorities of controls in the SWPP Plan shall reflect identified potential sources of pollutants. The description of storm water management controls to be implemented shall include, as appropriate:

a. Storm Water Pollution Prevention Personnel

Identify specific individuals (and job titles) who are responsible for developing, implementing, and reviewing the SWPP Plan.

b. Good Housekeeping

Good housekeeping requires the maintenance of clean, orderly facility areas that discharge storm water. Material handling areas shall be inspected and cleaned to reduce potential for pollutants to enter the storm drain conveyance system.

c. Spill Prevention and Response

Identify areas where significant materials can spill into or otherwise enter the storm water conveyance systems and their accompanying drainage points. Specific 
material handling procedures, storage requirements, cleanup equipment and procedures should be identified, as appropriate. The necessary equipment to implement a clean up shall be available and personnel trained in proper response, containment and cleanup of spills. Internal reporting procedures for spills of significant materials shall be established.

d. Source Control

Source controls, such as elimination or reduction of the use of toxic pollutants, covering of pollutant source areas, sweeping of paved areas, containment of potential pollutants, labeling all storm drain inlets with "No Dumping" signs, isolation/separation of industrial from non-industrial pollutant sources so that runoff from these areas does not mix, etc.

e. Storm Water Management Practices

Storm water management practices are practices other than those which control the sources of pollutants. They include treatment/conveyance structures such as drop inlets, channels, retention/detention basins, treatment vaults, infiltration galleries, filters, oil/water separators, etc. Based on assessment of the potential of various sources to contribute pollutants to storm water discharges in significant quantities, additional storm water management practices to remove pollutants from storm water discharges shall be implemented and design criteria shall be described.

f. Sediment and Erosion Control

Measures to minimize erosion around the storm water drainage and discharge points such as riprap, revegetation, slope stabilization, etc. shall be described and 
implemented.

g. Employee Training

Employee training programs shall inform all personnel responsible for implementing the SWPP Plan. Training should address spill response, good housekeeping, and material management practices. New employee and refresher training schedules should be identified.

h. Inspections

All inspections shall be done by trained personnel. Material handling areas shall be inspected for evidence of, or the potential for, pollutants entering storm water discharges. A tracking or follow up procedure shall be used to ensure appropriate response has been taken in response to an inspection. Inspections and maintenance activities shall be documented and recorder. Inspection records shall be retained for five years.

i. Records

A tracking and follow-up procedure shall be described to ensure that adequate response and corrective actions have been taken in response to inspections. 

4. An annual facility inspection shall be conducted to verify that all elements of the SWPP Plan are accurate and up to date. This results of this review shall be reported in the annual report to the Board on October 1 of each year.

C. SLUDGE MONITORING AND REPORTING

1. When sewage sludge is either sent to a landfill or applied to land as a soil amendment it should be monitored as follows:

a. Sewage sludge disposal shall be monitored at the following frequency:




Metric tons sludge/365 days
Frequency






0-290 
Once per year






290-1500
Quarterly






1500-15,000
Six times per year






Over 15,000
Once per month






(Metric tons are on a dry weight basis)

b. Sludge shall be monitored for the following constituents:




Land Application: As, Cd, Cr, Cu, Hg, Mo, Ni, Pb, Se, Zn




Municipal Landfill: Paint filter test (pursuant 40 CFR 258)




Sludge-only Landfill: As, Cd, Ni, (if no liner and leachate system)

2. The sludge must meet the following requirements prior to land application. The discharger must either demonstrate compliance or, if it sends the sludge to another party for further treatment and/or distribution, must give the recipient the information necessary to assure compliance.

a. Exceptional quality sludge: Sludge that meets the pollutant concentration limits in Table III of 40 CFR Part 503.13, Class A pathogen limits, and one of the vector 
attraction reduction requirements in 503.33(b)(1)-(b)(8) is exceptional quality sludge and does not have to be tracked further for compliance with general requirements (503.12) and management practices (503.14).

b. Sludge used for agricultural land, forest, or reclamation shall meet the pollutant limits in Table I (ceiling concentrations) and Table II or Table III (cumulative loadings or pollutant concentration limits) of 503.13. It shall also meet the general requirements (503.12) and management practices (503.14) (if not exceptional quality), Class A or Class B pathogen levels with associated access restrictions (503.32) and one of the 10 vector attraction reduction requirements in 503.33(b)(1)-(b)(10).

c. Sludge used for lawn or home gardens must meet exceptional quality sludge limits.

d. Sludge that is sold or given away in a bag or other container shall meet the pollutant limits in either Table III or Table IV (pollutant concentration limits or annual pollutant loading rate limits) of 503.13. If Table IV is used, a label or information sheet must be attached that explains Table IV (see 503.14). The sludge must also meet the Class A pathogen limits and one of the vector attraction reduction requirements in 503.33(b)(1)-(b)(8).

D. TREATMENT RELIABILITY

1. The discharger shall, at all times, properly operate and maintain all facilities and systems of treatment disposal and control (and related appurtenances) which are installed or used by the discharger to achieve compliance with this order and permit. Proper operation and maintenance also includes adequate laboratory controls and appropriate quality assurance procedures.  All of these procedures shall be described in an Operation and Maintenance Manual.  The discharger shall keep in a state of readiness all systems necessary to achieve compliance with the conditions of this order and permit. All systems, both those in service and reserve, shall be inspected and maintained on a regular basis.  Records shall be kept of the tests and made available to the Board.  [40 CFR 122.41(e)]

2. Safeguard to electric power failure:

a. The discharger shall, within ninety (90) days of the effective date of this permit, submit to the Board for approval a description of the existing safeguards provided to assure that, should there be reduction, loss, or failure of electric power, the discharger shall comply with the terms and conditions of its Order.  Such safeguards may include alternate power sources, standby generators, retention capacity, operating procedures or other means.  A description of the safeguards provided shall include an analysis of the frequency, duration, and impact of power failures experienced over the past five years on effluent quality and on the capability of the discharger to comply with the terms and conditions of the Order.  The adequacy of the safeguards is subject to the approval of the Regional Board.

b. Should the Board not approve the existing safeguards, the discharger shall, within ninety (90) days of having been advised by the Board that the existing safeguards are inadequate, provide to the Board and the U.S. Environmental Protection Agency a schedule of compliance for providing safeguards such that in the event of reduction, loss, or failure of electric power, the permittee shall comply with the terms and conditions of this permit. The schedule of compliance shall, upon approval of the Board Executive Officer, become a condition of the Order.

c. If the discharger already has approved plan(s), the plan shall be revised and updated as specified in the plan or whenever there has been a material change in design or operation.  A revised plan shall be submitted to the Board within ninety (90) days of the material change.

3. POTW facilities subject to this order and permit shall be supervised and operated by persons possessing certificates of appropriate grade pursuant to Division 4, Chapter 14, Title 23 of the California Code of Regulations.

E. GENERAL REPORTING REQUIREMENTS

1. Signatory Requirements

a. All reports required by the order and permit and other information requested by the Board or USEPA Region 9 shall be signed by a principal executive officer or ranking elected official of the discharger, or by a duly authorized representative of that 
person. [40 CFR 122.22(b)]

b. Certification

All reports signed by a duly authorized representative under Provision E.1.a. shall contain the following certification:

"I certify under penalty of law that this document and all attachments are prepared under my direction or supervision in accordance with a system designed to assure that qualified personnel properly gathered and evaluated the information submitted. Based on my inquiry of the person or persons who managed the system, or those persons directly responsible for gathering the information, the information submitted is, to the best of my knowledge and belief, true, accurate, and complete.  I am aware that there are significant penalties for submitting false information, including the possibility of fine and imprisonment for knowing violations. [40 CFR 122.22(d)]

2. Should the discharger discover that it failed to submit any relevant facts or that it submitted incorrect information in any report, it shall promptly submit the missing or correct information. [40 CFR 122.41(l)(8)]

3. False Reporting

Any person who knowingly makes any false statement, representation, or certification in any record or other document submitted or required to be maintained under this permit, including monitoring reports or reports of compliance or noncompliance shall be subject to enforcement procedures as identified in Section F of these Provisions.

4. Transfers

a. This permit is not transferable to any person except after notice to the Board. The Board may require modification or revocation and reissuance of the permit to change the name of the permittee and incorporate such other requirements as may be necessary under the Clean Water Act.

b. Transfer of control or ownership of a waste discharge facility under an National Pollutant Discharge Elimination System permit must be preceded by a notice to the Board at least 30 days in advance of the proposed transfer date.  The notice must include a written agreement between the existing discharger and proposed discharger containing specific dates for transfer of responsibility, coverage, and liability 
between them. Whether an order and permit may be transferred without modification or revocation and reissuance is at the discretion of the Board.  If order and permit modification or revocation and reissuance is necessary, transfer may be delayed 180 days after the Board's receipt of a complete application for waste discharge requirements and an NPDES permit.

5. Spill Prevention and Contingency Plans

The discharger shall file with the Board, for Executive Officer review and approval within ninety (90) days after the effective date of this Order, a technical report or a statement that the existing plan(s) was reviewed and updated, as appropriate, on preventive (failsafe) and contingency (cleanup) plans for controlling accidental discharges, and for minimizing the effect of such events.  The technical report or updated revisions should:

a. Identify the possible sources of accidental loss, untreated or partially treated waste bypass, and polluted drainage. Loading and storage areas, power outage, waste treatment unit outage, and failure of process equipment, tanks and pipes should be considered. 

b. Evaluate the effectiveness of present facilities and procedures and state when they became operational.

c. Predict the effectiveness of the proposed facilities and procedures and provide an implementation schedule containing interim and final dates when they will be 
constructed, implemented, or operational.  

This Board, after review of the technical report or updated revisions, may establish conditions which it deems necessary to control accidental discharges and to minimize the effects of such events.  Such conditions may be incorporated as part of this Order, upon notice to the discharger.  If the discharger already has an approved plan(s) he shall update them as specified in the plan(s).

6. Compliance Reporting 

a. Planned Changes

The discharger shall file with the Board a report of waste discharge at least 120 days before making any material change or proposed change in the character, location or volume of the discharge.

b. Compliance Schedules

Reports of compliance or noncompliance with, or any progress reports on, interim and final compliance dates contained in any compliance schedule shall be submitted within 10 working days following each scheduled date unless otherwise specified within this order and permit.  If reporting noncompliance, the report shall include a description of the reason for failure to comply, a description and schedule of tasks necessary to achieve compliance and an estimated date for achieving full compliance.  A final report shall be submitted within 10 working days of achieving full compliance, documenting full compliance

c. Anticipated Non-compliance

All POTWs must provide adequate notice to the Board of:

i. Any introduction of new pollutants into the POTW from an indirect discharger that would be subject to Sections 301 or 306 of the Clean Water Act if it were 
directly discharging those pollutants.

ii. Any substantial or material change in the volume or character of pollutants being introduced into that POTW by an input source at the time of issuance of the permit.

Adequate notice shall include information on the quality and quantity of influent introduced into the POTW as well as any anticipated impact of the change on the quantity or quality of effluent to be discharged from the POTW. 

d. Non-compliance Reporting (Twenty‑four hour reporting:)

i. The discharger shall report any noncompliance that may endanger health or the environment. All pertinent information shall be provided orally within 24 hours from the time the discharger becomes aware of the circumstances. A written submission shall also be provided within five working days of the time the discharger becomes aware of the circumstances. The written submission shall contain a description of the noncompliance and its cause; the period of noncompliance, including exact dates and times and, if the noncompliance has not been corrected, the anticipated time it is expected to continue; and steps taken or planned to reduce, eliminate, and prevent reoccurrence of the noncompliance.

ii. The following shall be included as information that must be reported within 24 hours under this paragraph:

(1) Any unanticipated bypass that exceeds any effluent limitation in the permit.

(2) Any upset that exceeds any effluent limitation in the permit.

(3) Violation of a maximum daily discharge limitation for any of the pollutants listed in this permit to be reported within 24 hours.

(4) The Board may waive the above‑required written report on a case‑by‑case basis.

F. ENFORCEMENT

1. The provision contained in this enforcement section shall not act as a limitation on the statutory or regulatory authority of the Board.

2. Any violation of the permit constitutes violation of the California Water Code and regulations adopted hereunder and the provisions of the Clean Water Act, and is the basis for enforcement action, permit termination, permit revocation and reissuance, denial of an application for permit reissuance; or a combination thereof.

3. The Board may impose administrative civil liability, may refer a discharger to the State Attorney General to seek civil monetary penalties, may seek injunctive relief or take other appropriate enforcement action as provided in the California Water Code or federal law for violation of Board orders.

4. It shall not be a defense for a discharger in an enforcement action that it would have been necessary to halt or reduce the permitted activity in order to maintain compliance with the conditions of this order and permit.

5. A discharger seeking to establish the occurrence of any upset (See Definitions, G. 24) has the burden of proof.  A discharger who wishes to establish the affirmative defense of any upset in an action brought for noncompliance shall demonstrate, through properly signed contemporaneous operating logs, or other relevant evidence that:

a. an upset occurred and that the permittee can identify the cause(s) or the upset;

b. the permitted facility was being properly operated at the time of the upset;

c. the discharger submitted notice of the upset as required in paragraph E.6.d.; and 

d. the discharger complied with any remedial measures required under A.4.

No determination made before an action for noncompliance, such as during administrative review of claims that noncompliance was caused by an upset, is final administrative action subject to judicial review.

In any enforcement proceeding, the discharger seeking to establish the occurrence of any upset has the burden of proof.  [40 CFR 122.41(n)]

G. DEFINITIONS

1. Bypass means the intentional diversion of waste streams from any portion of treatment facility.

2. Daily discharge means:

a. For flow rate measurements, the average flow rate measured during a calendar day or during any 24‑hour period reasonably representative of the calendar day for purposes of sampling.

b. For pollutant measurements, the concentration or mass emission rate measured during a calendar day or during any 24‑hour period reasonably representative of the calendar day for purposes of sampling.

3. Daily Maximum Limit means the maximum acceptable daily discharge.  For pollutant measurements, unless otherwise specified, the results to be compared to the daily 
maximum limit are based on composite samples.

4. DDT and Derivatives shall mean the sum of the p,p' and o,p' isomers of DDT, DDD (TDE), and DDE.

5. Duly authorized representative is one whose:

a. Authorization is made in writing by a principal executive officer or ranking elected official;

b. Authorization specifies either an individual or a position having responsibility for the overall operation of the regulated facility or activity, such as general manager in a partnership, manager, operator of a well or a well field, superintendent, position of equivalent responsibility, or an individual or position having overall responsibility for environmental matters for the company. (A duly authorized representative may thus be either a named individual or any individual occupying a 
named position.); and

c. Written authorization is submitted to the USEPA Region 9. If an authorization becomes no longer accurate because a different individual or position has responsibility for the overall operation of the facility, a new authorization satisfying the requirements above must be submitted to the Board and USEPA Region 9 prior to or together with any reports, information, or applications to be signed by an authorized representative.

6. Hazardous substance means any substance designated under 40 CFR 116 pursuant to Section 311 of the Clean Water Act.

7. HCH shall mean the sum of the alpha, beta, gama (Lindane), and delta isomers of hexachlorocyclohexane.

8. Inadequately Treated Waste is wastewater receiving partial treatment but failing to meet discharge requirements.

9. Incompatible pollutants are:

a. Pollutants which create a fire or explosion hazard in the POTW;

b. Pollutants which will cause corrosive structural damage to the POTW, or wastewaters with pH lower than 5.0 pH units, unless the facilities are specifically designed to accommodate such wastewater;

c. Solid or viscous pollutants in amounts which will cause obstruction to the flow in the POTW resulting in interference;

d. Any pollutant, including oxygen‑demanding pollutants (e.g., BOD) released into the wastewater system at a flow rate and/or pollutant concentration which will cause interference with the POTW.

e. Heat in amounts which will inhibit biological activity in the POTW and result in interference, or heat in such quantities that the temperature at the POTW treatment plant exceeds 40oC (104oF) unless the works is designed to accommodate such heat or the Board approves alternate temperature limits.  

10. Indirect discharger means a non‑domestic discharger introducing pollutants into a publicly owned treatment and disposal system.

11. Initial dilution is the process which results in the rapid and irreversible turbulent mixing of wastewater with receiving water around the point of discharge.

12. Mass emission rate is obtained from the following calculation for any calendar day:

N

Mass emission rate (lb/day) = 8.345 (( QiCi )

N
i=1

N

Mass emission rate (kg/day) = 3.785 (( QiCi)

N
i=1 

In which 'N' is the number of samples analyzed in any calendar day.  'Qi' and 'Ci' are the flow rate (MGD) and the constituent concentration (mg/L), respectively, which are associated with each of the 'N' grab samples which may be taken in any calendar day.  If a composite sample is taken, 'Ci' is the concentration measured in the composite sample and 'Qi' is the average flow rate occurring during the period over which samples are composited. The daily concentration measured over any calendar day of all constituents shall be determined from the flow‑ weighted average of the same constituents in the combined waste streams as follows:

N                   

Cd = Average daily concentration =  1 (( QiCi)

Qt
i=1

In which 'N' is the number of component waste streams.  'Q' and 'C' are the flow rate (MGD) and the constituent concentration (mg/L), respectively, which are associated with each of the 'N' waste streams.  'Qt' is the total flow rate of the combined waste streams.

13. Maximum allowable mass emission rate, whether for a 24‑hour, weekly 7‑day, monthly 30‑day, or 6‑month period, is a limitation expressed as a daily rate determined with the formulas in paragraph above, using the effluent concentration limit specified in the order and permit for the period and the specified allowable flow.  (Refer to Section C of Part A of Self‑ Monitoring Program for definitions of limitation period)

14. Overflow is defined as the intentional or unintentional spilling or forcing out of untreated 
or partially treated wastes from a transport system (e.g. through manholes, at pump stations, and at collection points) upstream from the plant headworks or from any treatment plant facilities.

15. POTW means Publicly Owned Treatment Works.

16. POTW Removal efficiency is expressed as the percentage of the ratio of pollutants removed by the treatment facilities to pollutants entering the treatment facilities.  Removal efficiencies of a treatment plant shall be determined using monthly averages of pollutant concentration of influent and effluent samples collected at about the same time and using the following equation (or its equivalent):

Removal Efficiency (%) = 100 X [1‑(Effluent Conc./Influent Conc.)]

When preferred, the discharger may substitute mass loadings and mass emissions for the concentrations.

17. Priority pollutants are those constituents referred to in 40 CFR S122, Appendix D and listed in the USEPA NPDES Application Form 2C, (dated 6/80) Items V‑3 through V‑9.

18. Sludge means the solids, semi‑liquid suspensions of solids, residues, screenings, grit, scum, and precipitates separated from, or created in wastewater by the unit processes of a treatment system.  It also includes but is not limited to, all supernatant, filtrate, centrate, decantate, and thickener overflow/underflow in the solids handling parts of the wastewater treatment system.

19. Storm Water means storm water runoff, snow melt runoff, and surface runoff and drainage. It excludes infiltration and runoff from agricultural land.

20. Toxic pollutant means any pollutant listed as toxic under Section 307(a)(1) of the Clean Water Act or under 40 CFR S401.15. 

21. Total Identifiable Chlorinated hydrocarbons (TICH) shall be measured by summing the individual concentrations of DDT, DDD, DDE, aldrin, BHC, chlordane, endrin, heptachlor, lindane, dieldrin, PCBs and other identifiable chlorinated hydrocarbons.

22. Severe property damage means substantial physical damage to property, damage to the treatment facilities which causes them to become inoperable or substantial and permanent loss of natural resources which can reasonably be expected to occur in the absence of a bypass or overflow.  It does not mean economic loss caused by delays in production.

23. Untreated waste is defined as raw wastewater.

24. Upset means an exceptional incident in which there is unintentional temporary noncompliance with effluent technology based permit limitations in the order and permit because of factors beyond the reasonable control of the discharger.  It does not include noncompliance to the extent caused by operational error, improperly designed treatment facilities, inadequate treatment facilities, lack of preventive maintenance, or careless or improper operation.

25. Waste, waste discharge, discharge of waste, and discharge are used interchangeably in this order and permit.  The requirements of this order and permit are applicable to the entire volume of water, and the material therein, which is disposed of to surface and 
ground waters of the State of California.

