The Electronic

Deliverable Format

(EDF)

Version 1.2b

DATA DICTIONARY

January 2001

Prepared by

ArsenaultLegg, Inc.

9600 Main Tree Drive

Anchorage, Alaska 99516

Phone: (907) 346-3827

Fax: (907) 346-1577

E-mail: information@arsenaultlegg.com

Web site: www.arsenaultlegg.com

Table of Contents

Introduction	1

ANADATE	10

ANMCODE	11

APPRVD	22

BASIS	23

CLCODE	24

CLREVDATE	26

CNTSHNUM	27

COC_MATRIX	28

COCNUM	29

DILFAC	30

DQO_ID	31

EXLABLOT	32

EXMCODE	33

EXPECTED	36

EXTDATE	37

LAB_METH_GRP	38

LAB_REPNO	39

LABCODE	40

LABDL	46

LABLOTCTL	47

LABQCID	48

LABREFID	49

LABSAMPID	50

LNOTE	51

LOCID	58

LOGCODE	59

LOGDATE	76

Table of Contents (continued)

LOGTIME	77

LOWERCL	78

MATRIX	79

METH_DESIGN_ID	82

MODPARLIST	83

NPDLWO	84

PARLABEL	85

PARUN	154

PARVAL	155

PARVQ	156

PRESCODE	157

PROJNAME	158

PVCCODE	159

QCCODE	160

RECDATE	166

REP_DATE	167

REPDL	168

REPDLVQ	169

REQ_METHOD_GRP	170

RLNOTE	171

RT	172

RUN_NUMBER	173

SAMPID	174

SRM	175

SUB	177

TLNOTE	183

UNITS	184

UPPERCL	190

List of Tables

Summary of Data Elements	3

Acceptable ANMCODE Values	11

Acceptable BASIS Values	23

CLCODE Groups	24

Acceptable CLCODE Values	25

Acceptable COC_MATRIX Values	28

Acceptable EXMCODE Values	33

Acceptable LABCODE Values Sorted by Code	40

Acceptable LABCODE Values Sorted by Description	43

Acceptable LNOTE Values	51

Acceptable LOGCODE Values Sorted by Code	59

Acceptable LOGCODE Values Sorted by Description	67

Acceptable MATRIX Values	79

Acceptable PARLABEL Values Sorted by Code	85

Acceptable PARLABEL Values Sorted by Description	119

Acceptable PARVQ Values	156

Acceptable PRESCODE Values	157

Acceptable PVCCODE Values	159

Acceptable QCCODE Values	160

Acceptable REPDLVQ Values	169

Acceptable SRM Values	175

Acceptable SUB Values Sorted by Code	177

Acceptable SUB Values Sorted by Description	180

Appropriate UNITS for Percent Moisture Parameters:	184

Acceptable UNITS Values	185

Introduction

This Data Dictionary provides the details and formats of each field in the Electronic Deliverable Format (EDF), Version 1.2b, January 2001, structure. The document is organized alphabetically by field name and includes the following information:

Definition:  A brief definition of the data field.

| Attributes: | Field Type and Size (defined in Field Attributes below) |  |
| --- | --- | --- |
| Included in Tables: |  | List of the tables in which the field exists in EDF, where “SAMPLE” represents the EDFSAMP table, “TEST” is EDFTEST, “RESULTS” is EDFRES, “QC” is EDFQC, and “CL” is EDFCL. |

Guidelines & Restrictions:

- Details and special instructions for appropriate entry into the field.

Acceptable [FIELDNAME] Values:

| Code | Description |
| --- | --- |
| A table listing available codes for a valid value field. |  |

## Field Attributes

Attributes are defined as follows:

- C7 is a 7-character field (alphanumeric).

- N5 is a numeric field with a total of 5 spaces available for numbers and decimals, with no restriction on the number of digits to the right of the decimal point other than the overall field size (e.g., 12345 or 123.4 or 1.234).

- D8 is a date field with the expected format of YYYYMMDD (e.g., 20010122).

- L1 is a logic field with the expected values of “T” (true) or “F” (false).

- The Time format is 4 digits using the military 24-hour clock without the colon, and ranging from 0000 to 2359 (e.g., 1630).

## Valid Value Fields

Various data fields in the EDF require entry of valid values (codes, also known as “VVLs”). Valid values are built-in codes that the format requires for certain fields, such as contractor names, matrices, and laboratories. The reason for using specific values for these fields is to standardize the data entry, to ensure data consistency and prevent errors. Freely entered data might contain extra spaces, commas, or dashes that would make meaningful data manipulation and thorough or accurate data searches impossible.

Most valid values are abbreviations of common or proper names; hence selecting the correct code is generally straightforward. However, some valid values are also used to link data properly (e.g., QCCODE is used to help link a laboratory replicate [“LR1”] to its original field sample [“CS”]). This Data Dictionary provides lists of the valid value codes and their definitions for each valid value field in the EDF.

New valid value codes can be requested Monday through Friday between 9:00 a.m. and 6:00 p.m. Pacific Standard Time through the office of ArsenaultLegg, Inc., by phone (907) 346-3827, fax (907) 346-1577, or e-mail information@arsenaultlegg.com. Please allow 72 hours for code generation.

## Summary of Data Elements

| Field Name | In Table(s) | Attrb | Null Allowed | VVL | Descr. Name | Definition | Guidelines & Restrictions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| anadate | TEST RESULTS | D8 |  |  | Analysis Date | The date the sample (aliquot, extract, digest and/or leachate) is analyzed. | Must be in the format YYYYMMDD. Must be later than or equal to EXTDATE, RECDATE, LOGDATE, and earlier than or equal to REP_DATE. |
| anmcode | TEST RESULTS QC CL | C7 |  | x | Analytical Method | The code identifying the method of analysis. | Must contain a valid value. |
| apprvd | TEST | C3 | x |  | Approved By | The initials of the individual approving the laboratory report. | No entry for laboratory-generated QC and non-client samples. |
| basis | TEST | C1 |  | x | Basis | The code used to distinguish whether a sample is reported as dry or wet weight, filtered or not filtered. | Must contain a valid value. Valid values for soil samples are "W" or "D" or leachate codes; for water samples "F," "L," or "N." |
| clcode | CL | C6 |  | x | Control Limit Type | The code identifying the type of quality control limit. | Must contain a valid value. |
| clrevdate | RESULTS CL | D8 | x |  | Control Limit Revision Date | The date a control limit is established. | Must be in the format YYYYMMDD. No entry when QCCODE is "CS," "NC," "LB," or "RS," and non-spiked parameters (except when PARVQ is "SU" or "IN"). |
| cntshnum | SAMPLE | C12 |  |  | Control Sheet Number | The administratively-assigned identification used to track contracts. | Entry of "NA" is acceptable. |
| coc_matrix | SAMPLE | C2 | x | x | COC Matrix | The code identifying the sample matrix as noted on the chain-of-custody (e.g., water, soil, etc.). | Optional. This field provides a link with the COC EDD from EDMS2000. It represents the sample matrix as identified by the field organization, and must contain a valid value. |
| cocnum | SAMPLE | C16 | x |  | Chain-of-Custody Number | The number assigned to the chain-of-custody. | No entry for laboratory-generated QC and non-client samples. |
| dilfac | RESULTS | N10 |  |  | Dilution Factor | The numeric factor indicating the level of sample dilution. | Must be greater than zero. (Formerly in the format N10,3 in EDF 1.2a.) |
| dqo_id | SAMPLE | C25 | x |  | Data Quality Objectives ID | The unique identifier representing the data quality objectives. | Optional. This field provides a link with the COC EDD from EDMS2000. |
| exlablot | TEST | C10 | x |  | OBSOLETE | OBSOLETE | OBSOLETE |
| exmcode | TEST RESULTS CL | C7 |  | x | Preparation Method | The code identifying the method of preparation. | Must contain a valid value. If no preparation performed enter "NONE;" if preparation method is included in analysis method enter "METHOD." |
| expected | QC | N14 | x |  | Expected Parameter Value | The target result for a quality control sample or surrogate spike. | No entry when QCCODE is "CS," "NC," "LB," or "RS." For matrix spikes, this value is the amount spiked plus the reference sample PARVAL. Enter "100" when UNITS are "PERCENT." (Formerly in the format N14,4 in EDF 1.2a.) |
| extdate | TEST RESULTS | D8 |  |  | Preparation Date | The date that a sample is prepared for analysis. | Must be in the format YYYYMMDD. If no preparation performed, enter ANADATE. |
| lab_meth_grp | TEST RESULTS QC CL | C25 | x |  | Lab Method Group | The unique identifier for a group of methods as defined by the laboratory. | Optional. This field provides a link with the EDMS2000. |
| lab_repno | TEST | C20 | x |  | Laboratory Report Number | The unique identifier for the laboratory report, assigned by the laboratory. | No entry for laboratory-generated QC and non-client samples. |
| labcode | SAMPLE TEST RESULTS QC CL | C4 |  | x | Laboratory | The code identifying the laboratory that analyzes the sample. | This field represents the laboratory that received the sample and is responsible for producing the electronic deliverable, and must contain a valid value. |
| labdl | RESULTS | N9 |  |  | Method Detection Limit | The laboratory-established method detection limit. | Enter zero when UNITS is "PERCENT" or PARVQ is "TI." Must be adjusted for dilution. Must be greater than or equal to zero. (Formerly in the format N9,4 in EDF 1.2a.) |
| lablotctl | TEST QC | C10 |  |  | Preparation Batch Number | The unique identifier for a preparation and handling batch. | Must uniquely define a group of samples prepared together. |
| labqcid | QC | C12 |  |  | Laboratory QC Sample ID | The unique identification number assigned to the sample by the laboratory. | This is equivalent to the LABSAMPID. |
| labrefid | QC | C12 | x |  | Laboratory Reference ID | The laboratory sample ID of the quality control reference sample. | This is the LABSAMPID of the reference sample. No entry unless QCCODE is "MS/SD" or "LR." |
| labsampid | TEST RESULTS | C12 |  |  | Laboratory Sample ID | The unique identification number assigned to the sample by the laboratory. | Must be unique. |
| locid | SAMPLE TEST | C10 | x |  | Location ID | The unique identifier for the sample's location, as identified by the laboratory. | No entry for laboratory-generated QC and non-client samples. |
| logcode | SAMPLE TEST | C4 | x | x | Field Organization | The code identifying the company collecting the samples or performing field tests. | Must contain a valid value. No entry for laboratory-generated QC and non-client samples. |
| logdate | SAMPLE TEST | D8 | x |  | Collection Date | The date a field sample is collected. | Must be in the format YYYYMMDD. No entry for laboratory-generated QC and non-client samples. Must be earlier than RECDATE, EXTDATE, ANADATE, and REP_DATE. |
| logtime | SAMPLE TEST | C4 | x |  | Collection Time | The time that a field sample is collected, recorded using 24-hour military time. | Must be a valid time between 0000 and 2359. No entry for laboratory-generated QC and non-client samples. |
| lowercl | CL | N4 |  |  | Lower Control Limit | The lower control limit of a quality control criterion. | Must be an integer greater than or equal to zero and less than UPPERCL. Enter zero for precision limit. |
| matrix | SAMPLE TEST RESULTS QC CL | C2 |  | x | Matrix | The code identifying the sample matrix as determined by the laboratory (e.g., water, soil, etc.). | This field represents the sample matrix as identified by the laboratory, and must contain a valid value. |
| meth_design_id | SAMPLE TEST RESULTS QC CL | C25 | x |  | Method Design ID | The unique identifier for the design of an analytical method. | Optional. This field provides a link with the COC EDD from EDMS2000. |
| modparlist | TEST | L1 |  |  | Modified Parameter List | A field indicating whether the parameter list of an analytical method has been modified. | Must enter "T" (true) or "F" (false) if a parameter from the method parameter list is not reported. The parameter list is not considered modified if extra parameters are reported. |
| npdlwo | SAMPLE | C7 |  |  | NPDL Work Order Number | A delivery order number associated with the contract. | Entry of "NA" is acceptable. |
| parlabel | RESULTS QC CL | C12 |  | x | Parameter | The code or CAS number identifying the analyte (parameter). | Must contain a valid value. |
| parun | RESULTS | N12 | x |  | Parameter Uncertainty | The uncertainty of a measured value due to a measuring technique (expressed as plus or minus some value). | No entry necessary for non-radiochemical results. If entered, must be greater than or equal to zero. (Formerly in the format N12,4 in EDF 1.2a.) |
| parval | RESULTS | N14 |  |  | Parameter Value | The analytical value for a compound, analyte, or physical parameter. | (Formerly in the format N14,4 in EDF 1.2a.) |
| parvq | RESULTS | C2 |  | x | Parameter Value Qualifier | The code identifying the qualifier of an analytical result (e.g., greater than, equal to, etc.). | Must contain a valid value. |
| prescode | TEST | C15 | x | x | Preservative | The code identifying the type of preservative added to the sample. | Must contain a valid value. Multiple codes may be entered, separated by commas (no spaces between values). |
| projname | SAMPLE | C25 | x |  | Project Name | The identification assigned to the project by the organization performing the work. | No entry for laboratory-generated QC and non-client samples. |
| pvccode | RESULTS | C2 |  | x | Primary Value Type | The code identifying whether a sample result is a primary or a confirmatory value. | Must contain a valid value. There may be only one "PR" result per LABSAMPID, ANMCODE, EXMCODE, and PARLABEL. |
| qccode | TEST RESULTS QC | C3 |  | x | QC Type | The code identifying the type of sample (e.g., laboratory-generated, environmental, etc.). | Must contain a valid value. |
| recdate | TEST | D8 |  |  | Received Date | The date the sample is received by the laboratory doing the analysis. | Must be in the format YYYYMMDD. For laboratory-generated QC samples enter date sample was created (e.g., EXTDATE). |
| rep_date | TEST | D8 | x |  | Report Date | The date of the laboratory report. | Must be in the format YYYYMMDD. No entry for laboratory-generated QC and non-client samples. |
| repdl | RESULTS | N9 |  |  | Reporting Detection Limit | The laboratory-established method detection limit, adjusted for the particular sample preparation (e.g., weight, volume, or dilution). | Enter zero when UNITS is "PERCENT" or PARVQ is "TI." Must be adjusted for dilution. Must be greater than or equal to zero. (Formerly in the format N9,4 in EDF 1.2a.) |
| repdlvq | RESULTS | C3 |  | x | Reporting Detection Limit Qualifier | The code identifying the type of reporting limit (e.g., practical quantitation limit, instrument detection limit, etc.). | Must contain a valid value. Enter "NA" when UNITS is "PERCENT" or PARVQ is "TI." |
| req_method_grp | SAMPLE | C25 | x |  | Requested Method Group | The unique identifier for the method or group of methods requested by the client for analysis of the sample. | Optional. This field provides a link with the COC EDD from EDMS2000. |
| rlnote | RESULTS | C20 | x | x | Laboratory Result Notes | The code identifying notes pertaining to analytical performance irregularities that apply to a single analyte. | Must contain a valid value. Multiple codes may be entered, separated by commas (no spaces between values). |
| rt | RESULTS | N7 | x |  | Retention Time | The retention time of a tentatively identified compound (TIC), reported in minutes (min). | No entry necessary except when PARVQ is "TI." If entered must be greater than or equal to zero. (Formerly in the format N7,2 in EDF 1.2a.) |
| run_number | TEST RESULTS | N2 |  |  | Run Number | The numeric code distinguishing multiple or repeat analysis of a sample by the same method on the same day. | Must be an integer greater than or equal to 1. |
| sampid | SAMPLE TEST | C25 | x |  | COC Sample ID | The unique identifier representing a sample, assigned by the consultant, as submitted to the laboratory on a chain-of-custody. | This field represents the sample ID as it appears on the COC. No entry for laboratory-generated QC and non-client samples. |
| srm | RESULTS | C12 |  | x | Standard Reference Material | The code identifying the standard reference material used in the analysis. | Must contain a valid value. Enter "NA" if no reference material. |
| sub | TEST | C4 |  | x | Subcontracted Laboratory | The code identifying the subcontracted laboratory. | Must contain a valid value. Enter "NA" if no analyses are subcontracted. |
| tlnote | TEST | C20 | x | x | Laboratory Test Notes | The code identifying notes pertaining to analytical performance irregularities that apply to the entire test. | Must contain a valid value. Multiple codes may be entered, separated by commas (no spaces between values). |
| units | RESULTS QC | C10 |  | x | Units of Measure | The units for the parameter value measurement. | Must contain a valid value. |
| uppercl | CL | N4 |  |  | Upper Control Limit | The upper control limit of a quality control criterion. | Must be an integer greater than or equal to one and greater than LOWERCL. |

# ANADATE

Definition: The Analysis Date is the date a sample or extract is analyzed.

| Attributes: | D8 |  |
| --- | --- | --- |
| Included in Tables: |  | TEST RESULTS |

Guidelines & Restrictions:

- All date fields must be in the YYYYMMDD format.

- ANADATE cannot be left blank.

- ANADATE must be later than or equal to EXTDATE.

- ANADATE must be later than or equal to RECDATE.

- ANADATE must be later than or equal to LOGDATE.

- ANADATE must be earlier than or equal to REP_DATE.

# ANMCODE

Definition:  The ANMCODE represents the Analytical Method performed on a sample by the analytical laboratory.

| Attributes: | C7 |  |
| --- | --- | --- |
| Included in Tables: |  | TEST RESULTS QC CL |

Guidelines & Restrictions:

- ANMCODE cannot be left blank.

- ANMCODE must contain a valid value.

- Although many of the analytical methods are similar, compound lists are often slightly different (i.e., SW8260B and E524.2). Each ANMCODE implies a specific list of analytes (refer to the actual method). All of these analytes are expected to be reported.  If they are not all reported, the list must be identified as modified by entering “T” (“true”) into the modified parameter list field (MODPARLIST) of the test record.

Acceptable ANMCODE Values:

| Code | Description |
| --- | --- |
| 3810HVO | Halogenated Volatile Organics by Headspace |
| 8260SIM | Volatile Organic Compounds by GC/MS SIM |
| 8270SIM | Semivolatile Organic Compounds by GC/MS SIM |
| A10200H | Standard Method (19th ed.) 10200 H: Chlorophyll |
| A1030F1 | Standard Method 1030F1: Cation-Anion Balance Calculation |
| A203 | Calcium Carbonate Saturation |
| A210B | Standard Method 210B: Salinity by Hydrometric Method |
| A2120B | Standard Method 2120 B: Color by Visual Comparison |
| A2150B | Standard Method 2150 B: Threshold Odor Test |
| A2320B | Standard Method 2320 B: Alkalinity by Titration Method |
| A2330B | Standard Method 2330 B: Langelier Index |
| A2340B | Standard Method 2340 B: Hardness by Calculation |
| A2340C | Standard Method 2340 C: Hardness by Calc.-EDTA Titrimetric Method |
| A2510B | Standard Method (19th ed.) 2510 B: Specific Conductance |
| A2520B | Standard Method 2520 B: Salinity |
| A2540C | Standard Method (19th ed.) 2540 C: Total Diss. Solids at 180 deg. |
| A2540G | Standard Method (19th ed.) 2540 G: Total, Fixed and Vol. Solids |
| A2580B | Standard Method (19th ed.) 2580 B: Oxidation-Reduction Potential |
| A2710F | Standard Method 2710 F: Specific Gravity |
| A3500FE | Standard Method (19th ed.) 3500-Fe D: Penanthroline Method |
| A4500B | Standard Method 4500-Cl: Chloride (Argentometric Method) |
| A4500C | Standard Method (19th ed.) 4500-Cl C: Mercuric Nitrate Method |
| A4500CL | Standard Method (19th ed.) 4500-Cl G: Chlorine: DPD Colorimetric |
| A4500CN | Standard Method (19th ed.) 4500-CN I: Weak Acid Dissociable CN |
| A4500DA | Standard Method 4500-CO2 D: CO2 and Forms of Alkalinity by Calc. |
| A4500F | Standard Method (18th ed.) 4500NO3F: Auto Cd Reduction |
| A4500NH | Standard Method (19th ed.) 4500-NH: Nitrogen (Ammonia) |
| A5520C | Standard Method 5520 C: Oil and Grease Partition-Infrared Method |
| A5520F | Standard Method 5520 F: Oil and Grease Hydrocarbons |
| A5540A | Standard Method 5540 A: Surfactants (MBAS) |
| A5540C | Standard Method 5540 C: Determ. of Methylene Blue Active Subst. |
| A5550B | Standard Method 5550 B: Tannin and Lignin |
| A9215B | Standard Method 9215 B: Heterotrophic Plate Count-Pour Plate |
| A9215D | Standard Method 9215 D: Heterotrophic Plate Count-Membrane Filter |
| A9221B | Standard Method 9221 B: Total Coliform Fermentation Technique |
| A9221E | Standard Method 9221 E: Fecal Coliform |
| A9222B | Standard Method 9222 B: Total Coliform Membrane Filter Procedure |
| A9222D | Standard Method 9222 D: Fecal Coliform |
| A9223B | Standard Method (19th ed) 9223 B: Chromogenic Substrate Coliform |
| A9230B | Standard Method (19th ed) 9230 B: Fecal Strep Multiple Tube Tech. |
| A9240D | Standard Method 9240 D: Isolation of Iron and Sulfur Bacteria |
| A9260D | Standard Method (18th ed.) 9260 D: Quantitative Salmonella Proc. |
| AG7-2.2 | Percent Moisture |
| AHERA | Bulk Asbestos-40CFR763, Subpart E, Appendix A |
| AK101 | Gasoline Range Organics, Alaska Dept. of Environment. Conserv. |
| AK101E | State of Alaska Method 101 Extended (AK101/BTEX) |
| AK102 | Diesel Range Organics, Alaska Dept. of Environment. Conserv. |
| AK102E | State of Alaska Method 102 Extended (AK102/AK103) |
| AK103 | State of Alaska Residual Range Hydrocarbons |
| AKD | State of Alaska Method for Diesel |
| AKG | State of Alaska Method for Gasoline |
| ASA1033 | ASA (1982) 10-3.3: Electrical Conductivity |
| ASA2451 | ASA (1982) 24-5.1: Phosphorus Soluble in Dilute Acid-Fluoride |
| ASA39 | ASA (1982) 39: Heterotrophic Plate Count |
| ASTMD93 | Flash Point, (Open Cup) |
| BDTL | Bligh and Dyer Method for Total Lipids |
| BTSNTOT | Total Butyltins |
| CAPBO | Determination of Organic Lead DHS Method |
| CATFH | Total Fuel Hydrocarbons: LUFT Method (California) |
| CATPH-D | State of California Diesel Range Organics |
| CATPH-G | State of California Gasoline Range Organics |
| CENPD | Fuel Identification and Quantification-COE |
| CLPPM | CLP Method for Percent Moisture (ILM03.0) |
| CSGAS | Analytical Determination of CS Gas |
| D1217 | Specific Gravity (Pycnometer) |
| D129 | ASTM Method for Sulfur in Petroleum Products |
| D1744 | ASTM Method for Water in Liquid Petroleum Prods. by K.F. Reagent |
| D1945 | ASTM Method for Natural Gas by GC |
| D2015 | High Heat Value Determination in Liquid |
| D2196 | Viscosity (Viscometer) |
| D2216 | Percent Solid |
| D240 | Heat of Combustion of Hydrocarbon Liquids by Bomb Calorimeter |
| D2500 | ASTM Method Cloud Point of Petroleum Oils |
| D287 | Specific Gravity (Petroleum Product Hydrometer Sp) |
| D2972 | Arsenic by Atomic Absorption; Furnace |
| D3341 | ASTM Method for Organic Lead |
| D3416 | Total Hydrocarbons, Methane and Carbon Monoxide in Atmosphere GC |
| D3828 | ASTM Method Flash Point by Setaflash Closed Tester |
| D3859 | ASTM Method for Selenium in Water |
| D4129 | Total Organic Carbon (TOC) |
| D412982 | ASTM Method for Total and Organic Carbon in Water Oxidation |
| D445 | Viscosity (petroleum product, kinematic viscosity) |
| D482 | ASTM Method for Ash from Petroleum Products |
| D776 | ASTM Method for Forms of Chlorine in Refuse-Derived Fuel |
| D808 | ASTM Method Chlorine in New and Used Petroleum Products |
| D91AVSM | Draft 1991-Determination of Acid Volatile in Sediment |
| D97 | ASTM Method Pour Point of Petroleum Oils |
| DE1109 | ASTM Method for Determining Bulk Density of Solid Waste Fractions |
| DOCH4 | Diss. Oxygen & Methane (Inter'l J. Env. Anal. Chem. 1988) |
| DU | Data Unavailable |
| E110.2 | Color (Colorimetric-Platinum-Cobalt) |
| E120.1 | Specific Conductance |
| E130.1 | Hardness, Total (Colorimetric, Automated EDTA) |
| E130.2 | Hardness, Total (Titrimetric) |
| E150.1 | pH, Electrometric |
| E160.1 | Residue, Filterable (TDS) |
| E160.2 | Residue, Non-Filterable |
| E160.3 | Residue, Total (Gravimetric, Dried at 103-105 Degrees) |
| E160.3M | Residue, Total (Gravimetric Dried) Modified |
| E160.4 | Residue, Volatile (Gravimetric, Ignition at 550 Degrees) |
| E160.4M | Residue, Volatile (Gravimetric, Ignition) Modified |
| E160.5 | Settleable Matter |
| E1613A | Tetra thru Octa-Chlorinated Dioxins and Furans by Isotope Dilutin |
| E1624 | Volatile Organic Compounds by Isotope Diltuion GC/MS |
| E1631 | Mercury in Water by Oxidation, P&T, and Cold Vapor |
| E1658 | Determination of Phenoxy-acid Herbicides |
| E1664 | HEM and SGT-HEM by Extraction and Gravimetry |
| E170.1 | Temperature |
| E180.1 | Turbidity (Nephelometric) |
| E200.7 | Inductively Coupled Plasma Emission |
| E200.8 | Inductively Coupled Plasma/Mass Spectroscopy |
| E200.9 | Atomic Absorption, Platform |
| E202.1 | Aluminum (AA, Direct Aspiration) |
| E202.2 | Aluminum (AA, Furnace Technique) |
| E204.1 | Antimony (AA, Direct Aspiration) |
| E204.2 | Antimony (AA, Furnace Technique) |
| E206.2 | Arsenic (AA, Furnace) |
| E206.3 | Arsenic (AA, Hydride) |
| E206.4 | Spectrophotometric, SDDC |
| E208.1 | Barium (AA, Direct Aspiration) |
| E208.2 | Barium (AA, Furnace) |
| E210.1 | Beryllium (AA, Direct Aspiration) |
| E210.2 | Beryllium (AA, Furnace Technique) |
| E212.3 | Boron (Colorimetric, Curcumin) |
| E213.1 | Cadmium (AA, Direct Aspiration) |
| E213.2 | Cadmium (AA, Furnace) |
| E215.1 | Calcium (AA, Direct Aspiration) |
| E215.2 | Calcium (Titrimetric, EDTA ) |
| E218.1 | Chromium (AA, Direct Aspiration) |
| E218.2 | Chromium (AA, Furnace) |
| E218.3 | Chromium by Chelation - Extraction |
| E218.4 | Chromium Hexavalent (AA, Chelation-Extraction) |
| E218.5 | Chromium Hexavalent, Dissolved (AA, Furnace) |
| E219.1 | Cobalt (AA, Direct Aspiration) |
| E219.2 | Cobalt (AA, Furnace Technique) |
| E220.1 | Copper (AA, Direct Aspiration) |
| E220.2 | Copper (AA, Furnace) |
| E231.1 | Gold (AA, Direct Aspiration) |
| E231.2 | Gold (AA, Furnace) |
| E235.1 | Iridium (AA, Direct Aspiration ) |
| E235.2 | Iridium (AA, Furnace) |
| E236.1 | Iron (AA, Direct Aspiration) |
| E236.2 | Iron (AA, Furnace Technique) |
| E239.1 | Lead (AA, Direct Aspiration) |
| E239.2 | Lead (AA, Furnace) |
| E242.1 | Magnesium (AA, Direct Aspiration) |
| E243.1 | Manganese (AA, Direct Aspiration) |
| E243.2 | Manganese (AA, Furnace Technique) |
| E245.1 | Mercury (Cold Vapor, Manual) |
| E245.2 | Mercury (Cold Vapor, Automated) |
| E245.5 | Mercury (Cold Vapor, Sediments) |
| E246.1 | Molybdenum (AA, Direct Aspiration) |
| E246.2 | Molybdenum (AA, Furnace Technique) |
| E249.1 | Nickel (AA, Direct Aspiration) |
| E249.2 | Nickel (AA, Furnace) |
| E252.1 | Osmium (AA, Direct Aspiration) |
| E252.2 | Osmium (AA, Furnace ) |
| E253.1 | Palladium, (AA, Direct Aspiration ) |
| E253.2 | Palladium (AA, Furnace) |
| E255.1 | Platinum, (AA, Direct Aspiration) |
| E255.2 | Platinum (AA, Furnace ) |
| E258.1 | Potassium (AA, Direct Aspiration) |
| E265.1 | Rhodium (AA, Direct Aspiration) |
| E265.2 | Rhodium (AA, Furnace) |
| E267.1 | Ruthenium (AA, Direct Aspiration) |
| E267.2 | Ruthenium (AA, Furnace) |
| E270.1 | Selenium (AA, Direct Aspiration) |
| E270.2 | Selenium (AA, Furnace) |
| E270.3 | Selenium (AA, Hydride) |
| E272.1 | Silver (AA, Direct Aspiration) |
| E272.2 | Silver (AA, Furnace) |
| E273.1 | Sodium (AA, Direct Aspiration) |
| E273.2 | Sodium (AA, Furnace Technique) |
| E279.2 | Thallium (AA, Furnace) |
| E282.1 | Tin (AA, Direct Aspiration) |
| E282.2 | Atomic Absorption, Furnace |
| E283.1 | Titanium (AA, Direct Aspiration) |
| E283.2 | Titanium (AA, Furnace Technique) |
| E286.1 | Vanadium (AA, Direct Aspiration) |
| E286.2 | Vanadium (AA, Furnace Technique) |
| E289.1 | Zinc (AA, Direct Aspiration) |
| E289.2 | Zinc (AA, Furnace) |
| E300 | Inorganic Anions by Ion Chromatography |
| E300.0 | Inorganic Anions by Ion Chromatography |
| E300A | Inorganic Anions by Ion Chromatography, Part A |
| E300B | Inorganic Anions by Ion Chromatography, Part B |
| E305.1 | Acidity, Total |
| E305.2 | Acidity, Total |
| E310.1 | Alkalinity, Total (as Carbonate) |
| E310.2 | Alkalinity, Total (as Carbonate) |
| E320.1 | Bromide |
| E325.1 | Chloride (as Cl) |
| E325.2 | Chloride (as Cl) |
| E325.2M | Chloride (as Cl) Modified |
| E325.3 | Chloride (as Cl) |
| E325.3M | Chloride (as CI) Modified |
| E335.1 | Cynanides, Amenable to Chlorination |
| E335.2 | Total Cyanide |
| E335.3 | Total Cyanide (Colorimetric, Automated UV) |
| E335.4 | Total Cyanide by Semi-automated Colorimetry |
| E340.1 | Fluoride |
| E340.2 | Fluoride |
| E340.2M | Fluoride Modified |
| E340.3 | Fluoride |
| E345.1 | Iodide (as I) |
| E350.1 | Nitrogen, Ammonia (as N) |
| E350.1M | Nitrogen, Ammonia (as N) Modified |
| E350.2 | Nitrogen, Ammonia (as N) |
| E350.3 | Nitrogen, Ammonia (as N) |
| E351.1 | Nitrogen, Kjeldahl, Total |
| E351.2 | Nitrogen, Kjeldahl, Total |
| E351.3 | Nitrogen, Kjeldahl, Total |
| E351.4 | Nitrogen, Kjeldahl, Total |
| E351.4M | Nitrogen, Kjeldahl, Total Modified |
| E352.1 | Nitrogen, Nitrate (as N) |
| E353.1 | Nitrogen, Nitrate-Nitrite |
| E353.2 | Nitrogen, Nitrate-Nitrite |
| E353.2M | Nitrogen, Nitrate-Nitrate Modified |
| E353.3 | Nitrogen, Nitrate-Nitrite |
| E354.1 | Nitrogen, Nitrite |
| E360.1 | Oxygen, Dissolved |
| E360.2 | Oxygen, Dissolved |
| E365.1 | Phosphorus, All Forms, (Colorimetric, Automated, Ascorbic Acid ) |
| E365.2 | Phosphorus, All Forms (as P) |
| E365.3 | Phosphorus, All Forms (Colorimetric, Ascorbic Acid ) |
| E365.3M | Phosphorus, Reactive Soluble |
| E365.4 | Phosphorus, Total (Colorimetric, Automated Block Digestor, AA II) |
| E370.1 | Silica |
| E375.1 | Sulfate |
| E375.2 | Sulfate |
| E375.3 | Sulfate |
| E375.4 | Sulfate |
| E376.1 | Sulfide |
| E376.2 | Sulfide |
| E377.1 | Sulfite |
| E405.1 | Biochemical Oxygen Demand |
| E410 | Chemical Oxygen Demand |
| E410.1 | Chemical Oxygen Demand |
| E410.2 | Chemical Oxygen Demand |
| E410.3M | COD (Titrimetric, High Level for Saline Water) Modified |
| E410.4 | Chemical Oxygen Demand - Colorimetric |
| E413.1 | Oil and Grease, Total Recoverable |
| E413.2 | Oil and Grease, Total Recoverable (Spectrophotometric IR) |
| E415.1 | Total Organic Carbon (Combustion or Oxidation) |
| E415.2 | Total Organic Carbon (UV Promoted, Persulfate Oxidation) |
| E418.1 | Petroleum Hydrocarbons, Total Recoverable |
| E420.1 | Phenolics, Total Recoverable (Spectrophotometric, Manual) |
| E420.1M | Phenolics, Total Recoverable Modified |
| E420.4 | Total Recoverable Phenolics by Semi-automated Colorimetry |
| E425.1 | Methylene Blue Active Substances (MBAS) |
| E502.2 | Volatile Organic Compounds in Water by Purge and Trap |
| E504 | EDB and DBCP In Water by Microextraction and Gas Chromatography |
| E508 | Chlorinated Pesticides in Groundwater |
| E515.1 | Determination of Chlorinated Acids in Water by GC/ECD |
| E524.2 | Volatile Organic Compounds by Purge and Trap |
| E525.1M | Semivolatile Organic Compounds Modified |
| E600M4 | Asbestos in Bulk Insulation, Int. Method (Pol Lt. Microscopy & Di |
| E601 | Purgeable Halocarbons |
| E601-2 | Combined Methods E601/E602 |
| E602 | Purgeable Aromatics |
| E6045 | PCB Oil |
| E608 | Organochlorine Pesticides and PCBs |
| E610 | Polynuclear Aromatic Hydrocarbons |
| E614 | Determination of Organophosphorus Pesticides |
| E615 | Chlorinated Herbicides |
| E624 | Volatile Organic Compounds EPA Method 624 |
| E625 | Extractable Priority Pollutants |
| E900 | Gross Alpha and Beta Radiation |
| E901.1 | Gamm-Emitting Radionuclides in Drinking Water |
| E903.0 | Alpha-Emitting Radium Isotopes in Drinking Water |
| E903.1 | Radium |
| E905.0 | Radioactive Strontium in Water |
| E906.0 | Tritium in Drinking Water |
| ETO12 | Determination of Non-Methane Organic Compounds in Ambient Air |
| ETO14A | Determination of Volatile Organic Compounds in Ambient Air by GC |
| ETO3 | Determination of Volatile Organic Compounds in Ambient Air |
| ISOPU | 1990 EML Procedures Manual, Pu-11, HASL-300 |
| ISOU | 34th ORNL-DOE Conf.: Measure of Isotopic Uranium |
| KAHNTOC | Determination of TOC in Soil by Modified Lloyd Kahn Method |
| LPFE3 | Lovely/Phillips (1987) Rapid Assay for Microbially Reducible Fe3+ |
| M8015 | Modified SW8015 for Gasoline or Diesel Determination |
| M8100 | Determination of Diesel Range Organics |
| MAEPH | State of Massachusetts Dept. of Env. Protection EPH Method |
| MAVPH | State of Massachusetts Dept. of Env. Protection VPH Method |
| ME418.1 | Modified E418.1 TRPH (Alaska) |
| MSACAT | Cation Exchange Capacity & Exchange Coefficients |
| MTTPH-D | Montana Total Petroleum Hydrocarbons-Diesel Range |
| MTTPH-G | Montana Total Petroleum Hydrocarbons-Gasoline Range |
| N0502 | Dustfall from the Atmosphere |
| N1501 | Aromatic Hydrocarbons in Air |
| NWTPHDX | State of Washington Dept. of Ecology TPH-Dx Method |
| NWTPHGX | State of Washington Dept. of Ecology TPH-Gx Method |
| NWTPHHC | State of Washington Dept. of Ecology TPH-HCID Method |
| OHCID | State of Oregon Hydrocarbon Identification Method |
| OPHC | State of Oregon Total Petroleum Hydrocarbons |
| OR418.1 | Modified for State of Oregon Total Petroleum Hydrocarbons |
| OSCACO3 | Oregon State University, CaCO3 Soils Method |
| OTPH-D | State of Oregon Diesel Range Organics |
| OTPH-G | State of Oregon Gasoline Range Organics |
| PAHSIM | PAH Specific Ion Monitoring |
| PS1986 | Particle Size by PSEP Protocols |
| PSEPSID | Puget Sound Estuary Program, Sulfide Analysis |
| RSK175 | Diss. Gasses in Water by GC (Inter'l J. Env. Anal. Chem. 1991) |
| SACIDSL | Acid Soluble Sulfide (EPA Draft 1991) |
| SCID | Hydrocarbon Screening Method |
| SHEEN | Brown & Braddock Method for Sheen Screening (1989) |
| SIM | GC/MS SIM Method |
| SW1010 | Flash Point (Closed-Cup Tester) |
| SW1020 | Ignitability |
| SW1020A | Setaflash Closed-Cup Method for Determining Ignitability |
| SW1110 | Corrosivity Toward Steel |
| SW6010 | Inductively Coupled Plasma-Emission |
| SW6010A | Inductively Coupled Plasma-Atomic Emission Spectroscopy |
| SW6010B | Inductively Coupled Plasma-Atomic Emission Spectroscopy |
| SW6020 | Inductively Coupled Plasma-Mass Spectrometry |
| SW7.1 | SW-846, chpt. 7.1: Ignitability |
| SW7.2 | SW-846, chpt. 7.2: Corrosivity |
| SW7.3 | SW-846, chpt. 7.3: Cyanide/Sulfide Reactivity |
| SW7020 | Aluminum (AA, Direct Aspiration) |
| SW7040 | Antimony (AA, Direct Aspiration) |
| SW7041 | Antimony (AA, Furnace Technique) |
| SW7060 | Arsenic (AA, Furnace Technique) |
| SW7060A | Arsenic (AA, Furnace Technique) |
| SW7061 | Arsenic by Hydride Generation |
| SW7061A | Arsenic (AA, Gaseous Hydride) |
| SW7080 | Barium (AA, Direct Aspiration) |
| SW7081 | Barium (AA, Furnace Technique) |
| SW7090 | Beryllium (AA, Direct Aspiration) |
| SW7091 | Beryllium (AA, Furnace Technique) |
| SW7130 | Cadmium (AA, Direct Aspiration) |
| SW7131 | Cadmium (AA, Furnace Technique) |
| SW7131A | Cadmium (AA, Furnace Technique) |
| SW7140 | Calcium (AA, Direct Aspiration) |
| SW7190 | Chromium (AA, Direct Aspiration) |
| SW7191 | Chromium (AA, Furnace Technique) |
| SW7195 | Chromium, Hexavalent (Coprecipitation) |
| SW7196 | Chromium, Hexavalent (Colorimetric) |
| SW7196A | Chromium, Hexavalent (Colorimetric) |
| SW7197 | Chromium, Hexavalent (Chelation/Extraction) |
| SW7198 | Chromium, Hexavalent (Differential Pulse Polarography) |
| SW7200 | Cobalt (AA, Direct Aspiration) |
| SW7201 | Cobalt (AA, Furnace Technique) |
| SW7210 | Copper (AA, Direct Aspiration) |
| SW7211 | Copper (AA, Furnace Technique) |
| SW7380 | Iron (AA, Direct Aspiration) |
| SW7381 | Iron (AA, Furnace Technique) |
| SW7420 | Lead (AA, Direct Aspiration) |
| SW7421 | Lead (AA, Furnace Technique) |
| SW7430 | Lithium (AA, Direct Aspiration) |
| SW7450 | Magnesium (AA, Direct Aspiration) |
| SW7460 | Manganese (AA, Direct Aspiration) |
| SW7461 | Manganese (AA, Furnace Technique) |
| SW7470 | Mercury in Liquid Waste (Manual Cold-Vapor Technique) |
| SW7470A | Mercury in Liquid Waste (Manual Cold-Vapor Technique) |
| SW7471 | Mercury in Solid or Semisolid Waste (Manual Cold-Vapor Technique) |
| SW7471A | Mercury in Solid or Semisolid Waste (Manual Cold-Vapor Technique) |
| SW7480 | Molybdenum (AA, Direct Aspiration) |
| SW7481 | Molybdenum (AA, Furnace Technique) |
| SW7520 | Nickel (AA, Direct Aspiration) |
| SW7550 | Osmium (AA, Direct Aspiration) |
| SW7610 | Potassium (AA, Direct Aspiration) |
| SW7740 | Selenium (AA, Furnace Technique) |
| SW7741 | Selenium (AA, Gaseous Hydride) |
| SW7760 | Silver (AA, Direct Aspiration) |
| SW7760A | Silver (AA, Direct Aspiration) |
| SW7761 | Silver (AA, Furnace Technique) |
| SW7770 | Sodium (AA, Direct Aspiration) |
| SW7780 | Strontium (AA, Direct Aspiration) |
| SW7840 | Thallium (AA, Direct Aspiration) |
| SW7841 | Thallium (AA, furnace Technique) |
| SW7870 | Tin (AA, Direct Aspiration) |
| SW7910 | Vanadium (AA, Direct Aspiration) |
| SW7911 | Vanadium (AA, Furnace Technique) |
| SW7950 | Zinc (AA, Direct Aspiration) |
| SW7951 | Zinc (AA, Furnace Technique) |
| SW8010 | Halogenated Volatile Organics by Gas Chromatography |
| SW8010A | Halogenated Volatile Organics by Gas Chromatography |
| SW8010B | Halogenated Volatile Organics by Gas Chromatography |
| SW8015 | Non-Halogenated Volatile Organics |
| SW8015B | Non-Halogenated Organics Using GC/FID |
| SW8020 | Aromatic Volatile Organics |
| SW8020A | Aromatic Volatile Organics by Gas Chromatography |
| SW8020F | BTEX/Gasoline Range Organics (SW8020/8015) |
| SW8021A | Halogenated Volatiles by Gas Chromatography using Photoionization |
| SW8021B | Halogenated and Aromatic Volatiles by GC using Photoionization |
| SW8021F | Volatiles by GC/Gasoline Range Organics (SW8021B/8015) |
| SW8040A | Phenols by Gas Chromatography |
| SW8060 | Phthalate Esters |
| SW8080 | Organochlorine Pesticides and PCBs |
| SW8080A | Organochlorine Pesticides and PCBs by GC |
| SW8081 | Organochlorine Pesticides & PCBs as Aroclors by GC: Capillary Col |
| SW8081A | Organochlorine Pesticides by Gas Chromatography |
| SW8082 | Polychlorinated Biphenyls (PCBs) by Gas Chromatography |
| SW8100 | Polynuclear Aromatic Hydrocarbons |
| SW8140 | Organophosphorus Pesticides |
| SW8141 | Organophosphorus Compounds by Gas Chromatography |
| SW8150 | Chlorinated Herbicides by GC |
| SW8150A | Chlorinated Herbicides by GC |
| SW8151 | Chlorinated Herbicides by GC |
| SW8151A | Chlorinated Herbicides by GC |
| SW8240 | GC/MS for Volatile Organics |
| SW8240A | Volatile Organic Compounds (SW-846 Method) |
| SW8240B | Volatile Organic Compounds by GC/MS |
| SW8260 | Volatile Organic Compounds by GC/MS |
| SW8260A | Volatile Organic Compounds by GC/MS |
| SW8260B | Volatile Organic Compounds by GC/MS |
| SW8270 | Semivolatile Organic Compounds by GC/MS |
| SW8270A | Semivolatile Organic Compounds by GC/MS |
| SW8270B | Semivolatile Organic Compounds by GC/MS |
| SW8270C | Semivolatile Organic Compounds by GC/MS |
| SW8280 | Polychlorinated Dibenzodioxins/Polychlorinated Dibenzofurans |
| SW8290 | Polychlorinated Dibenzodioxins/Polychlorinated Dibenzofurans |
| SW8290D | Draft Polychlorinated Dibenzodioxins/Polychlorinated Dibenzofuran |
| SW8310 | Polynuclear Aromatic Hydrocarbons |
| SW8321 | Solvent Extractable Non-Volatile Compounds by HPLC/TSP/MS or UV D |
| SW8330 | Nitroaromatics and Nitramines by HPLC |
| SW9010 | Total and Amenable Cyanide |
| SW9010A | Total and Amenable Cyanide |
| SW9010B | Total and Amenable Cyanide: Distillation |
| SW9012 | Total and Amenable Cyanide (Colorimetric, Automated UV) |
| SW9012A | Total and Amenable Cyanide (Auto. Colorimetric/Off-line Distill.) |
| SW9013 | Cyanide Extraction Procedure for Solids and Oils |
| SW9014 | Titrimetric and Manual Spectrophotometric Determ. for Cyanide |
| SW9020 | Total Organic Halides (TOX) |
| SW9020A | Total Organic Halides (TOX) |
| SW9030A | Acid Soluble and Acid Insoluble Sulfides |
| SW9031 | Extractable Sulfides |
| SW9034 | Titrimetric Proc. for Acid-Soluble/Insoluble Sulfides |
| SW9038 | Sulfate (Turbidimetric) |
| SW9040 | pH, Electrometric Measurement |
| SW9040B | pH, Electrometric Measurement |
| SW9045A | pH |
| SW9045B | Soil and Waste pH |
| SW9045C | Soil and Waste pH |
| SW9050A | Specific Conductance |
| SW9056 | Determination of Inorganic Anions by Ion Chromatography |
| SW9060 | Total Organic Carbon (TOC) |
| SW9065 | Phenolics (Spectrophotometric, Manual 4-AAP with Distillation) |
| SW9066 | Phenolics (Colorimetric, Automated 4-AAP with Distillation) |
| SW9071 | Oil and Grease Extraction Method for Sludge Samples |
| SW9076D | Draft Total Chlorine in Petroleum Products by Oxidative Combust. |
| SW9077 | Total Chlorine in New and Used Petroleum Products |
| SW9081 | EPA 9081 Cation-Exchange Capacity of Soils |
| SW9095 | Paint Filter Liquids Test |
| SW9131 | Total Coliform: Multiple Tube Fermentation Technique |
| SW9132 | Total Coliform: Membrane-Filter Technique |
| SW9252 | Chloride (Titrimetric, Mercuric Nitrate) |
| SW9253 | Chloride (Titrimetric, Silver Nitrate) |
| SW9310 | Gross Alpha and Gross Beta |
| SW9315 | Alpha-Emitting Radium Isotopes |
| SW9320 | Radium-228 |
| SWVOL | Combined Methods SW8010/SW8020, Same Column |
| UL09 | Method UL09, Rocky Mountain Arsenal |
| ULK09 | Determination of DIMP and DMMP in Soil by GC/FPD |
| ULL05 | Determination of Organosulfur Compounds in Soil by GC |
| ULL9 | Determination of Thiodiglycol and Chloroacetic Acid in Soil |
| ULT04 | Determination of Organic Acids in Soil by Ion Chromatography |
| USDA60 | USDA Handbook No. 60: Exchangeable Sodium Percentage |
| UW46 | Method UW46, Rocky Mountain Arsenal |
| WA418.1 | Modified for State of Washington Total Petroleum Hydrocarbons |
| WBLACK | Walkley-Black Method, Organic Carbon (TOC) |
| WDOEEPH | State of Washington Dept. of Ecology EPH Method |
| WDOEVPH | State of Washington Dept. of Ecology VPH Method |
| WHCID | State of Washington Hydrocarbon Identification |
| WPHC | State of Washington Total Petroleum Hydrocarbons |
| WTPH-D | State of Washington Diesel Range Organics |
| WTPH-G | State of Washington Gasoline Range Organics |

# APPRVD

Definition: The Approved By field contains the initials of the individual approving the laboratory report.

| Attributes: | C3 |  |
| --- | --- | --- |
| Included in Tables: |  | TEST |

Guidelines & Restrictions:

- APPRVD cannot be left blank for test records where QCCODE = “CS,” and must be blank in all other cases.

# BASIS

Definition: BASIS (Basis) identifies the basis on which an analytical result is reported. This field is also used to indicate leaching procedures performed.

| Attributes: | C1 |  |
| --- | --- | --- |
| Included in Tables: |  | TEST |

Guidelines & Restrictions:

- BASIS cannot be left blank, and only one BASIS code may be applied to a test record.

- BASIS must contain a valid value.

- For soil samples, BASIS may be “W” for wet-weight basis, or “D” for dry-weight basis.

- For water samples, BASIS may be “F” for field filtered, “L” for lab filtered, or “N” for not filtered.

- If sample preparation includes a leaching procedure, the EXMCODE assigned to the sample is not the leachate method, but the preparation procedure listed in the analytical method that has been performed on the leachate. The BASIS field indicates the leaching procedure that was performed (“B,” “E,” “H,” “S,” “T,” or “V”).

Acceptable BASIS Values:

| Code | Description |
| --- | --- |
| A | Air |
| B | SW-924 Leaching Procedure |
| C | California Waste Extraction Test |
| D | Dry |
| E | Method 1310A EP Toxicity Test |
| F | Field Filtered |
| G | Centrifuge supernatant |
| H | Modified SW1311 - Water Leachate |
| L | Lab Filtered |
| N | Not Filtered |
| S | Method SW1312 TCLP |
| T | Method SW1311 TCLP |
| U | Data Unavailable |
| V | D91AVSM Leachate |
| W | Wet |

# CLCODE

Definition: The CLCODE represents the Quality Control Limit Type that is associated with a given result.

| Attributes: | C6 |  |
| --- | --- | --- |
| Included in Tables: |  | CL |

Guidelines & Restrictions:

- CLCODE cannot be left blank.

- CLCODE must contain a valid value.

- CLCODEs are assigned based upon the type of quality assurance sample being analyzed, as well as the system of validation being used.

- A single PARLABEL may have multiple sets of control limits, distinguished by the CLCODE and (in some cases) the CLREVDATE.

- CLCODEs are separated into six groups, with codes for surrogates, initial calibration, continuing calibration, laboratory replicates, standard reference material, and spiked samples.

CLCODE Groups:

| GROUP | AcceptableCLCODEs |
| --- | --- |
| Surrogates | SLSA/SLSP |
|  | SMSA/SMSP |
|  | SBSA/SBSP |
|  | SMEA/SMEP |
|  | SCLA/SCLP |
| Initial Calibration | LIC |
|  | MEIC |
|  | CLPIC |
| Continuing Calibration | LCC |
|  | MECC |
|  | CLPCC |
| Standard Reference | SRAD/SRPD |
| Material | SRMA/SRMP |
| Laboratory Replicates | LLR |
|  | MLR |
|  | MELR |
|  | CLPLR |
| Spiked Samples | LSA/LSP |
| (Matrix or Blank Solution) | MSA/MSP |
|  | CLPA/CLPP |
|  | SRMA/SRMP |
|  | SRAD/SRPD |

Acceptable CLCODE Values:

| Code | Description |
| --- | --- |
| CLPA | Contract Laboratory Program Accuracy Limits for Spiked Samples |
| CLPCC | CLP Continuing Calibration Acceptance Criteria |
| CLPIC | CLP Initial Calibration Acceptance Criteria |
| CLPLR | Contract Laboratory Program Precision for Lab Replicates |
| CLPP | Contract Laboratory Program Precision Limits for Spiked Samples |
| DU | Data Unavailable |
| LCC | Laboratory Continuing Calibration Accuracy |
| LIC | Laboratory Initial Calibration Accuracy |
| LLR | Laboratory Established Precision for Lab Replicates |
| LSA | Laboratory Sample Accuracy for Spiked Samples |
| LSP | Laboratory Sample Precision for Spiked Samples |
| MEA | Method Established Accuracy for Spiked Samples |
| MECC | Method Established Continuing Calibration Acceptance Criteria |
| MEIC | Method Established Initial Calibration Acceptance Criteria |
| MELR | Method Established Precision for Laboratory Replicates |
| MEP | Method Established Precision for Spiked Samples |
| MLR | Matrix Laboratory Replicate Precision |
| MSA | Matrix Spike Accuracy for Spiked Samples |
| MSP | Matrix Spike Precision for Spiked Samples |
| SBSA | Both Reagent and Matrix Sample Accuracy for Surrogates |
| SBSP | Both Reagent and Matrix Sample Precision for Surrogates |
| SCLA | Contract Laboratory Program Limits for Surrogate Accuracy |
| SCLP | Contract Laboratory Program Limits for Surrogate Precision |
| SLSA | Laboratory Sample Limits for Accuracy for Surrogates |
| SLSP | Laboratory Sample Limits for Precision for Surrogates |
| SMEA | Method Established Limits for Accuracy for Surrogates |
| SMEP | Method Established Limits for Precision for Surrogates |
| SMSA | Sample Matrix Limits for Accuracy for Surrogates |
| SMSP | Sample Matrix Limits for Precision for Surrogates |
| SRAD | Standard Reference Accuracy Defined by Agency/Manufacturer |
| SRMA | Standard Reference Material Accuracy Limits Determined by Lab |
| SRMP | Standard Reference Material Precision Limits Determined by Lab |
| SRPD | Standard Reference Precision Defined by Agency/Manufacturer |

# CLREVDATE

Definition: The Control Limit Revision Date is the date that the control limit is established.

| Attributes: | D8 |  |
| --- | --- | --- |
| Included in Tables: |  | RESULTS CL |

Guidelines & Restrictions:

- All date fields must be in the YYYYMMDD format.

- CLREVDATE must be blank for all result records where QCCODE = “CS,” “NC,” “LB,” or “RS,” and non-spiked parameters, except for surrogates (PARVQ = “SU”).

- CLREVDATE cannot be blank when QCCODE = “MS/SD,” “BS/BD,” “RM/KD,” “LR,” “IC,” or “CC.”

- CLREVDATE cannot be blank when PARVQ = “SU” or “IN.”

# CNTSHNUM

Definition: The Control Sheet Number was historically a COE-assigned administration number. This field is no longer used as such, and may be used by the laboratory for internal tracking numbers.

| Attributes: | C12 |  |
| --- | --- | --- |
| Included in Tables: |  | SAMPLE |

Guidelines & Restrictions:

- CNTSHNUM cannot be left blank.

- Enter “NA,” or use this field for internal tracking purposes.

# COC_MATRIX

Definition: The Chain-of-Custody Matrix is the code identifying the sample matrix as noted on the chain-of-custody (e.g., water, soil, etc.).

| Attributes: | C2 |  |
| --- | --- | --- |
| Included in Tables: |  | SAMPLE |

Guidelines & Restrictions:

- COC_MATRIX is an optional field and may be left blank.

- COC_MATRIX must contain a valid value if populated.

- COC_MATRIX is a linking field with the EDMS2000 electronic COC tables.

Acceptable COC_MATRIX Values:

| Code | Description |
| --- | --- |
| A | Air |
| L | Liquid Organic |
| M | Multi-Phase |
| SO | Soil/Solid |
| T | Tissue |
| W | Water |

# COCNUM

Definition: The Chain-of-Custody Number is the number assigned to the chain-of-custody by the field organization.

| Attributes: | C16 |  |
| --- | --- | --- |
| Included in Tables: |  | SAMPLE |

Guidelines & Restrictions:

- COCNUM cannot be left blank when QCCODE = “CS,” and must be left blank for all other QCCODEs.

# DILFAC

Definition: The Dilution Factor is the numeric factor indicating the level of sample dilution.

| Attributes: | N10 |  |
| --- | --- | --- |
| Included in Tables: |  | RESULTS |

Guidelines & Restrictions:

- DILFAC cannot be left blank.

- DILFAC must be greater than zero.  Use “1” as the default.

- Detection limits should be adjusted for dilution.

# DQO_ID

Definition: The Data Quality Objectives ID is the unique identifier representing the data quality objectives.

| Attributes: | C25 |  |
| --- | --- | --- |
| Included in Tables: |  | SAMPLE |

Guidelines & Restrictions:

- DQO_ID is an optional field and may be left blank.

- DQO_ID is a linking field with the EDMS2000 electronic COC tables.

# EXLABLOT

Definition: The Extraction QC Lot Number is an obsolete field and should always be left blank.

| Attributes: | C10 |  |
| --- | --- | --- |
| Included in Tables: |  | TEST |

Guidelines & Restrictions:

- EXLABLOT must always be left blank.

# EXMCODE

Definition: The EXMCODE represents the Preparation Method performed on a sample.

| Attributes: | C7 |  |
| --- | --- | --- |
| Included in Tables: |  | TEST RESULTS CL |

Guidelines & Restrictions:

- EXMCODE cannot be left blank.

- EXMCODE must contain a valid value.

- There are five categories to differentiate the extraction or digestion procedure used in the analysis of a sample. They are:

- NONE - Selected when no preparation procedure is used or called for in the analytical method. Examples include determinations such as pH, temperature, percent moisture, etc.

- METHOD - Most commonly used with EPA drinking water procedures or laboratory modified methods where the preparation procedure is directly specified within the analytical method.

- DI - Sample is directly injected into the instrument.

- Specific EPA Methods - Documented, published methods for which a code exists in the EXMCODE valid value list.

- Field Preparation - For ANMCODE AK101 (Gasoline Range Organics), preparation can be performed in the field. The EXMCODE is “AK101PR” in this situation.

Acceptable EXMCODE Values:

| Code | Description |  |
| --- | --- | --- |
| 3510ALI |  | Separatory Funnel Liq.-Liq. Ext. (Aliphatic Fraction) |
| 3510ARO |  | Separatory Funnel Liq.-Liq. Ext. (Aromatic Fraction) |
| 3550ALI |  | Sonication Extraction (Aliphatic Fraction) |
| 3550ARO |  | Sonication Extraction (Aromatic Fraction) |
| A5520D |  | Soxhlet Extraction Method for Oil and Grease |
| A9221C |  | Standard Method 9221 C: Estimation of Bacterial Density |
| AK101PR |  | Field Preparation Method for AK101 |
| CAPBO |  | Determination of Organic Lead DHS Method |
| CLAA |  | Contract Lab Prog. Digestion for AA/ICP Analysis |
| CLFAA |  | Contract Lab Prog. Digestion for Furnace Analysis |
| D91AVSM |  | Draft 1991-Determination of Acid Volatile in Sediment |
| DI |  | Direct Injection |
| DU |  | Data Unavailable |
| E200.2 |  | Prep. for Spectral Chemical Determin. of Total Recov. Elements |
| E200.3 |  | Preparation for Total Recoverable Metals in Biological Tissue |
| E200.8 |  | Inductively Coupled Plasma/Mass Spectroscopy |
| E245.1 |  | Mercury (Cold Vapor, Manual) |
| E6045 |  | PCB Oil |
| LA29B |  | State of Louisiana (29-B) Extraction of Total Barium |
| METHOD |  | Extraction Method Specified in Analytical Method |
| MSADTPA |  | Extraction with Ammonium Bicarbonate-DTPA |
| MSANH4N |  | Extraction of Exchangeable Ammonium and Nitrate/Nitrite |
| MSASAT |  | Saturation Extraction |
| N5503 |  | PCBs in Air: Method 5503, NIOSH Man. of Anal. Methods, 4th Ed. |
| NH4OAC |  | USDA Handbook No. 60: NH4OAC Extract |
| NONE |  | No Extraction Required for this Method |
| REACT |  | Reactivity |
| SW1310A |  | Extraction Procedure (EP) Toxicity Test Method |
| SW1311 |  | Toxicity Characteristic Leaching Procedure |
| SW1312 |  | Synthetic Precipitation Leaching Procedure |
| SW3005 |  | Digestion for Total Recoverable Metals for Flame AA |
| SW3005A |  | Acid Digestion of Waters for Total Recov. or Dissolved Metals |
| SW3010 |  | Digestion for Total Metals for Flame AA and ICP |
| SW3010A |  | Acid Digest. of Aqueous Samples/Extracts for Total Metals FAA/ICP |
| SW3015 |  | Microwave-Assisted Acid Digestion of Aqueous Samples |
| SW3020 |  | Digestion for Total Metals for Furnace AA |
| SW3020A |  | Acid Digest. of Aqueous Samples/Extracts for Total Metals GFAA |
| SW3040 |  | Dissolution Procedure for Oils, Greases, or Waxes |
| SW3050 |  | Acid Digestion of Sediments, Sludges, and Soils |
| SW3050A |  | Acid Digestion of Sediments, Sludges, and Soils |
| SW3050B |  | Acid Digestion of Sediments, Sludges, and Soils |
| SW3051 |  | Microwave-Assisted Acid Digestion of Soils and Sediments |
| SW3051M |  | Modified SW-846, Method 3051 (Closed Vessel Oil Digestion) |
| SW3060 |  | Alkaline Digestion of Soil and Solid Waste |
| SW3060A |  | Alkaline Digestion for Hexavalent Chromium |
| SW3500 |  | Organic Extraction and Sample Preparation |
| SW3500A |  | Organic Extraction and Sample Preparation |
| SW3510 |  | Separatory Funnel Liquid-Liquid Extraction |
| SW3510A |  | Separatory Funnel Liquid-Liquid Extraction |
| SW3510B |  | Separatory Funnel Liquid-Liquid Extraction |
| SW3510C |  | Separatory Funnel Liquid-Liquid Extraction |
| SW3520 |  | Continuous Liquid-Liquid Extraction |
| SW3520A |  | Continuous Liquid-Liquid Extraction |
| SW3520B |  | Continuous Liquid-Liquid Extraction |
| SW3520C |  | Continuous Liquid-Liquid Extraction |
| SW3535 |  | Solid Phase Extraction (SPE) |
| SW3540 |  | Soxhlet Extraction |
| SW3540A |  | Soxhlet Extraction |
| SW3540B |  | Soxhlet Extraction |
| SW3540C |  | Soxhlet Extraction |
| SW3541 |  | Automated Soxhlet Extraction |
| SW3545 |  | Pressurized Fluid Extraction |
| SW3550 |  | Sonication Extraction |
| SW3550A |  | Ultrasonic Extraction |
| SW3550B |  | Ultrasonic Extraction |
| SW3580 |  | Waste Dilution |
| SW3580A |  | Waste Dilution |
| SW3810 |  | Headspace |
| SW5030 |  | Purge-and-Trap |
| SW5030A |  | Purge-and-Trap |
| SW5030B |  | Purge-and-Trap for Aqueous Samples |
| SW5035 |  | Closed-System Purge-and-Trap/Extraction for Volatile Organics |
| SW5040 |  | Protocol for Analysis of Sorbent Cartridges from Volatile Organic |
| SW5050 |  | Total Halogens as Chloride |
| SW7.3.3 |  | Reactive Cyanide |
| SW7.3.4 |  | Reactive Sulfide |
| SW7060 |  | Arsenic |
| SW7060A |  | Arsenic |
| SW7061 |  | Arsenic |
| SW7061A |  | Arsenic |
| SW7470 |  | Mercury |
| SW7470A |  | Mercury |
| SW7471 |  | Mercury Preparation in Solid or Semisolid Waste |
| SW7471A |  | Mercury Preparation in Solid or Semisolid Waste |
| SW7741 |  | Selenium |
| SW7741A |  | Selenium |
| SW7742 |  | Selenium |
| SW824D |  | SW8240(B) Direct Injection Technique |
| SW9071 |  | Oil and Grease Extraction Method for Sludge Samples |
| WOS |  | Water Extraction of Soils |

# EXPECTED

Definition: The Expected value is the target result for a quality control sample.

| Attributes: | N14 |  |
| --- | --- | --- |
| Included in Tables: |  | QC |

Guidelines & Restrictions:

- EXPECTED must be blank when QCCODE = “CS,” “NC,” “LB,” or “RS.”

- EXPECTED cannot be left blank if CLREVDATE is populated.

- If UNITS = “PERCENT,” enter “100” into EXPECTED.

- For spiked environmental samples (i.e., matrix spikes), enter the amount of the spike added plus the sample result value (PARVAL) into EXPECTED.

- EXPECTED must be greater than or equal to zero.

# EXTDATE

Definition: The Preparation Date is the date a sample is extracted or prepared for analysis.

| Attributes: | D8 |  |
| --- | --- | --- |
| Included in Tables: |  | TEST RESULTS |

Guidelines & Restrictions:

- All date fields must be in the YYYYMMDD format.

- EXTDATE cannot be left blank.

- EXTDATE must be earlier than or equal to ANADATE.

- EXTDATE must be later than or equal to RECDATE.

- EXTDATE must be later than or equal to LOGDATE.

- EXTDATE must be earlier than or equal to REP_DATE.

# LAB_METH_GRP

Definition: The Lab Method Group is the unique identifier for a group of methods as defined by the laboratory.

| Attributes: | C25 |  |
| --- | --- | --- |
| Included in Tables: |  | SAMPLE TEST RESULTS QC CL |

Guidelines & Restrictions:

- LAB_METH_GRP is an optional field and may be left blank.

- LAB_METH_GRP is a linking field with the EDMS2000 electronic COC tables.

# LAB_REPNO

Definition: The Laboratory Report Number is the laboratory-assigned number uniquely identifying the hard copy and electronic data deliverable (EDD) report.

| Attributes: | C20 |  |
| --- | --- | --- |
| Included in Tables: |  | TEST |

Guidelines & Restrictions:

- LAB_REPNO cannot be left blank when QCCODE = “CS,” and must be left blank in all other cases.

- LAB_REPNO must be unique.

# LABCODE

Definition: The LABCODE represents the analytical Laboratory that receives the samples (not necessarily the laboratory that performs the analyses).

| Attributes: | C4 |  |
| --- | --- | --- |
| Included in Tables: |  | SAMPLE TEST RESULTS QC CL |

Guidelines & Restrictions:

- LABCODE cannot be left blank.

- LABCODE must contain a valid value.

Acceptable LABCODE Values Sorted by Code:

| Code | Description |
| --- | --- |
| ACZ | ACZ Laboratories, Steamboat, CO |
| AEHA | Army Environmental Hygiene Agency (AEHA), APG, MD |
| AELF | American Environmental Laboratories, Pensacola, FL |
| AENP | American Environmental Network, Portland, OR |
| ALTC | Alta Analytical Lab Incorporated, El Dorado Hills, CA |
| APPL | Agriculture & Priority Pollutants Laboratories, Fresno, CA |
| ARDL | Applied Research and Development Lab, Inc., (ARDL) Mt. Vernon, IL |
| ARI | Analytical Resources, Inc., Seattle, WA |
| ATCA | Analytica, Anchorage, AK |
| ATCC | Analytica, CO |
| ATIA | Analytical Technologies, Inc., Anchorage, AK |
| ATIR | Analytical Technologies, Inc., Renton, WA |
| ATIS | Analytical Technologies, Inc., San Diego, CA |
| ATOX | Air Toxics LTD, Folsom, CA |
| AXYS | Axys Analytical Services, Ltd., Sidney, B.C., Canada |
| BCE | Brown & Caldwell Analytical Lab, Emeryville, CA |
| BCLB | BC Laboratories, Bakersfield, CA |
| BMLA | Boreochem Mobile Lab & Analytical Services |
| BRS | Brelje & Race, Santa Rosa, CA |
| CASB | Columbia Analytical Services, Inc., Bothell, WA |
| CASD | Columbia Analytical Services, Inc., Redding, CA |
| CASK | Columbia Analytical Services, Inc., Kelso, WA |
| CASL | Columbia Analytical Services, Inc., Canoga Park, CA |
| CAWL | California Water Labs, Inc., Modesto, CA |
| CCAC | Coast-to-Coast Analytical Services, Inc., Camarillo, CA |
| CCSJ | Coast-to-Coast Analytical Services, Inc., San Jose, CA |
| CDM | CDM Federal Programs Corporation |
| CHEM | Chemic Laboratory, San Diego, CA |
| CHMC | CH2M Hill Analytical Services, Corvallis, OR |
| CHMM | CH2M Hill Analytical Services, Montgomery, AL |
| CHRP | Chromalab, Inc., Pleasanton, CA |
| CKY | CKY Inc., Torrance, CA |
| CLSR | California Laboratory Services, Rancho Cordova, CA |
| CLTP | Clayton Environmental Consultants, Inc., Pleasanton, CA |
| CRLB | Century Refining (CENREF) Labs, Inc., Brighton, CO |
| CTB | Curtis & Tompkins, Berkeley, CA |
| CTE | CT&E Environmental Services, Inc., Anchorage, AK |
| CTEC | CT&E Environmental Services, Inc., Charleston, WV |
| DCHM | DataChem Laboratories, Inc., Salt Lake City, UT |
| DMP | D & M Laboratories, Petaluma, CA |
| DOWL | Dowl Engineering Alaska Test Labs, Anchorage, AK |
| DU | Data Unavailable |
| ECEN | Ecology & Environment, Inc. |
| ECI | EcoChem, Inc., Seattle, WA |
| ECLL | Environmental Chemistry Lab at LLNL, Livermore, CA |
| EEIS | Envirodyne Engineers, Inc., St. Louis, MO |
| EMXT | EMAX Laboratories, Inc., Torrance, CA |
| ETCS | ETC, Santa Rosa, CA |
| FGIS | Frontier Geosciences, Inc., Seattle, WA |
| FGL | Fruit Growers Lab, Stockton, CA |
| FORA | Forensic Analytical |
| GELC | General Engineering Laboratories, Inc., Charleston, SC |
| GENC | GTEL Environmental Labs, Inc., Concord, CA |
| KIC | KIC Lab, Prudhoe Bay, AK |
| LAL | Lockheed Analytical Laboratory, Las Vegas, NV |
| LASL | Los Alamos Scientific Laboratory, Los Alamos, NM |
| LDC | Laboratory Data Consultants |
| LTL | Laucks Testing Lab, Inc. |
| MEC | MEC Analytical Systems, Inc., Carlsbad, CA |
| MSSL | Mountain States Analytical, Salt Lake City, UT |
| MWLP | Montgomery Watson Laboratories, Pasadena, CA |
| NA | Not Applicable |
| NCAB | North Creek Analytical, Bothell, WA |
| NCAC | North Creek Analytical, Bend, OR |
| NCAP | North Creek Analytical, Beaverton, OR |
| NCAS | North Creek Analytical, Spokane, WA |
| NTL | Northern Testing Laboratories, Anchorage, AK |
| NTLF | Northern Testing Laboratories, Fairbanks, AK |
| NWCC | Northwest Colorado Consultants, Inc., Steamboat Springs, CO |
| OEIR | OnSite Environmental, Inc., Redmond, WA |
| PAC | Pacific Analytical, Carlsbad, CA |
| PAIS | Performance Analytical, Inc., Simi Valley, CA |
| PARA | Paragon Analytics, Inc., CO |
| PHLE | Philip Environmental |
| QALA | Quality Analytical Laboratores, Inc., Montgomery, AL |
| QALC | Quality Analytical Laboratories, Inc., Redding, CA |
| RFWC | Roy F. Weston, West Chester, PA |
| RFWS | Roy F. Weston, Stockton, CA |
| SAS | Sound Analytical Services, Inc., Tacoma, WA |
| SC3S | S-Cubed, A Division of Maxwell Laboratories, Inc., San Diego, CA |
| SEQR | Sequoia Analytical Laboratories, Inc., Redwood City, CA |
| SPEC | Spectra Laboratory, Inc., Tacoma, WA |
| STL1 | Severn Trent Laboratories, Arvada, CO |
| STL2 | Severn Trent Laboratories, Edison, NJ |
| STL3 | Severn Trent Laboratories, Santa Ana, CA |
| STL4 | Severn Trent Laboratories, Miramar, FL |
| STL5 | Severn Trent Laboratories, Newburgh, NY |
| STL6 | Severn Trent Laboratories, Colchester, VT |
| STL7 | Severn Trent Laboratories, Aurora, CO |
| STLB | Severn Trent Laboratories, Sparks, MD |
| STLC | Severn Trent Laboratories, North Canton, OH |
| STLD | Severn Trent Laboratories, Austin, TX |
| STLE | Severn Trent Laboratories, Tallahassee, FL |
| STLF | Severn Trent Laboratories, Tampa, FL (Quanterra) |
| STLG | Severn Trent Laboratories, Savannah, GA |
| STLH | Severn Trent Laboratories, Houston, TX |
| STLI | Severn Trent Laboratories, Pensacola, FL |
| STLJ | Severn Trent Laboratories, N. Billerica, MA |
| STLK | Severn Trent Laboratories, Knoxville, TN |
| STLL | Severn Trent Laboratories, Earth City, MO |
| STLM | Severn Trent Laboratories, Monroe, CT |
| STLN | Severn Trent Laboratories, Anaheim, CA |
| STLO | Severn Trent Laboratories, Mobile, AL |
| STLP | Severn Trent Laboratories, Pittsburgh, PA |
| STLQ | Severn Trent Laboratories, Amherst, NY |
| STLR | Severn Trent Laboratories, Richland, WA |
| STLS | Severn Trent Laboratories, West Sacramento, CA |
| STLT | Severn Trent Laboratories, Austin, TX (Quanterra) |
| STLU | Severn Trent Laboratories, University Park, IL |
| STLV | Severn Trent Laboratories, Valparaiso, IN |
| STLW | Severn Trent Laboratories, Westfield, MA |
| STLX | Severn Trent Laboratories, Tampa, FL (Savannah) |
| STLY | Severn Trent Laboratories, Whippany, NJ |
| STLZ | Severn Trent Laboratories, Corpus Christi, TX |
| SWAA | Shannon & Wilson, Inc., Anchorage, AK |
| SWLB | Southwest Laboratory |
| SWRI | Southwest Resarch Institute, San Antonio, TX |
| TRID | Triangle Laboratories, Inc., Durham, NC |

Acceptable LABCODE Values Sorted by Description:

| Code | Description |
| --- | --- |
| ACZ | ACZ Laboratories, Steamboat, CO |
| APPL | Agriculture & Priority Pollutants Laboratories, Fresno, CA |
| ATOX | Air Toxics LTD, Folsom, CA |
| ALTC | Alta Analytical Lab Incorporated, El Dorado Hills, CA |
| AELF | American Environmental Laboratories, Pensacola, FL |
| AENP | American Environmental Network, Portland, OR |
| ATCA | Analytica, Anchorage, AK |
| ATCC | Analytica, CO |
| ARI | Analytical Resources, Inc., Seattle, WA |
| ATIA | Analytical Technologies, Inc., Anchorage, AK |
| ATIR | Analytical Technologies, Inc., Renton, WA |
| ATIS | Analytical Technologies, Inc., San Diego, CA |
| ARDL | Applied Research and Development Lab, Inc., (ARDL) Mt. Vernon, IL |
| AEHA | Army Environmental Hygiene Agency (AEHA), APG, MD |
| AXYS | Axys Analytical Services, Ltd., Sidney, B.C., Canada |
| BCLB | BC Laboratories, Bakersfield, CA |
| BMLA | Boreochem Mobile Lab & Analytical Services |
| BRS | Brelje & Race, Santa Rosa, CA |
| BCE | Brown & Caldwell Analytical Lab, Emeryville, CA |
| CLSR | California Laboratory Services, Rancho Cordova, CA |
| CAWL | California Water Labs, Inc., Modesto, CA |
| CDM | CDM Federal Programs Corporation |
| CRLB | Century Refining (CENREF) Labs, Inc., Brighton, CO |
| CHMC | CH2M Hill Analytical Services, Corvallis, OR |
| CHMM | CH2M Hill Analytical Services, Montgomery, AL |
| CHEM | Chemic Laboratory, San Diego, CA |
| CHRP | Chromalab, Inc., Pleasanton, CA |
| CKY | CKY Inc., Torrance, CA |
| CLTP | Clayton Environmental Consultants, Inc., Pleasanton, CA |
| CCAC | Coast-to-Coast Analytical Services, Inc., Camarillo, CA |
| CCSJ | Coast-to-Coast Analytical Services, Inc., San Jose, CA |
| CASB | Columbia Analytical Services, Inc., Bothell, WA |
| CASL | Columbia Analytical Services, Inc., Canoga Park, CA |
| CASK | Columbia Analytical Services, Inc., Kelso, WA |
| CASD | Columbia Analytical Services, Inc., Redding, CA |
| CTE | CT&E Environmental Services, Inc., Anchorage, AK |
| CTEC | CT&E Environmental Services, Inc., Charleston, WV |
| CTB | Curtis & Tompkins, Berkeley, CA |
| DMP | D & M Laboratories, Petaluma, CA |
| DU | Data Unavailable |
| DCHM | DataChem Laboratories, Inc., Salt Lake City, UT |
| DOWL | Dowl Engineering Alaska Test Labs, Anchorage, AK |
| ECI | EcoChem, Inc., Seattle, WA |
| ECEN | Ecology & Environment, Inc. |
| EMXT | EMAX Laboratories, Inc., Torrance, CA |
| EEIS | Envirodyne Engineers, Inc., St. Louis, MO |
| ECLL | Environmental Chemistry Lab at LLNL, Livermore, CA |
| ETCS | ETC, Santa Rosa, CA |
| FORA | Forensic Analytical |
| FGIS | Frontier Geosciences, Inc., Seattle, WA |
| FGL | Fruit Growers Lab, Stockton, CA |
| GELC | General Engineering Laboratories, Inc., Charleston, SC |
| GENC | GTEL Environmental Labs, Inc., Concord, CA |
| KIC | KIC Lab, Prudhoe Bay, AK |
| LDC | Laboratory Data Consultants |
| LTL | Laucks Testing Lab, Inc. |
| LAL | Lockheed Analytical Laboratory, Las Vegas, NV |
| LASL | Los Alamos Scientific Laboratory, Los Alamos, NM |
| MEC | MEC Analytical Systems, Inc., Carlsbad, CA |
| MWLP | Montgomery Watson Laboratories, Pasadena, CA |
| MSSL | Mountain States Analytical, Salt Lake City, UT |
| NCAP | North Creek Analytical, Beaverton, OR |
| NCAC | North Creek Analytical, Bend, OR |
| NCAB | North Creek Analytical, Bothell, WA |
| NCAS | North Creek Analytical, Spokane, WA |
| NTL | Northern Testing Laboratories, Anchorage, AK |
| NTLF | Northern Testing Laboratories, Fairbanks, AK |
| NWCC | Northwest Colorado Consultants, Inc., Steamboat Springs, CO |
| NA | Not Applicable |
| OEIR | OnSite Environmental, Inc., Redmond, WA |
| PAC | Pacific Analytical, Carlsbad, CA |
| PARA | Paragon Analytics, Inc., CO |
| PAIS | Performance Analytical, Inc., Simi Valley, CA |
| PHLE | Philip Environmental |
| QALA | Quality Analytical Laboratores, Inc., Montgomery, AL |
| QALC | Quality Analytical Laboratories, Inc., Redding, CA |
| RFWS | Roy F. Weston, Stockton, CA |
| RFWC | Roy F. Weston, West Chester, PA |
| SC3S | S-Cubed, A Division of Maxwell Laboratories, Inc., San Diego, CA |
| SEQR | Sequoia Analytical Laboratories, Inc., Redwood City, CA |
| STLQ | Severn Trent Laboratories, Amherst, NY |
| STLN | Severn Trent Laboratories, Anaheim, CA |
| STL1 | Severn Trent Laboratories, Arvada, CO |
| STL7 | Severn Trent Laboratories, Aurora, CO |
| STLD | Severn Trent Laboratories, Austin, TX |
| STLT | Severn Trent Laboratories, Austin, TX (Quanterra) |
| STL6 | Severn Trent Laboratories, Colchester, VT |
| STLZ | Severn Trent Laboratories, Corpus Christi, TX |
| STLL | Severn Trent Laboratories, Earth City, MO |
| STL2 | Severn Trent Laboratories, Edison, NJ |
| STLH | Severn Trent Laboratories, Houston, TX |
| STLK | Severn Trent Laboratories, Knoxville, TN |
| STL4 | Severn Trent Laboratories, Miramar, FL |
| STLO | Severn Trent Laboratories, Mobile, AL |
| STLM | Severn Trent Laboratories, Monroe, CT |
| STLJ | Severn Trent Laboratories, N. Billerica, MA |
| STL5 | Severn Trent Laboratories, Newburgh, NY |
| STLC | Severn Trent Laboratories, North Canton, OH |
| STLI | Severn Trent Laboratories, Pensacola, FL |
| STLP | Severn Trent Laboratories, Pittsburgh, PA |
| STLR | Severn Trent Laboratories, Richland, WA |
| STL3 | Severn Trent Laboratories, Santa Ana, CA |
| STLG | Severn Trent Laboratories, Savannah, GA |
| STLB | Severn Trent Laboratories, Sparks, MD |
| STLE | Severn Trent Laboratories, Tallahassee, FL |
| STLF | Severn Trent Laboratories, Tampa, FL (Quanterra) |
| STLX | Severn Trent Laboratories, Tampa, FL (Savannah) |
| STLU | Severn Trent Laboratories, University Park, IL |
| STLV | Severn Trent Laboratories, Valparaiso, IN |
| STLS | Severn Trent Laboratories, West Sacramento, CA |
| STLW | Severn Trent Laboratories, Westfield, MA |
| STLY | Severn Trent Laboratories, Whippany, NJ |
| SWAA | Shannon & Wilson, Inc., Anchorage, AK |
| SAS | Sound Analytical Services, Inc., Tacoma, WA |
| SWLB | Southwest Laboratory |
| SWRI | Southwest Resarch Institute, San Antonio, TX |
| SPEC | Spectra Laboratory, Inc., Tacoma, WA |
| TRID | Triangle Laboratories, Inc., Durham, NC |

# LABDL

Definition: The LABDL represents the Laboratory-Established Method Detection Limit (i.e., the minimum detectable concentration of an analyte that can be measured and reported with 99% confidence that the analyte concentration is different from a blank for a given matrix).

| Attributes: | N9 |  |
| --- | --- | --- |
| Included in Tables: |  | RESULTS |

Guidelines & Restrictions:

- LABDL cannot be left blank, except when UNITS = “PERCENT” (e.g., surrogate parameters), or PARVQ = “TI” (i.e., for TIC parameters).

- LABDL must be adjusted for dilution.

- LABDL may contain the same value as the REPDL field, depending on the reporting format of the individual laboratory.  In this case, the REPDLVQ should indicate that the REPDL is actually the LABDL value (e.g., “MDL” would be an appropriate REPDLVQ when LABDL and REPDL are equal).

- LABDL must be greater than or equal to zero.

# LABLOTCTL

Definition: The Lab QC Lot Number is a unique identifier for an autonomous batch or group of environmental samples prepared together, and sharing the same quality control within the same time period. This group is equivalent to the EPA SW-846 concept of a “Quality Assurance Batch.”

| Attributes: | C10 |  |
| --- | --- | --- |
| Included in Tables: |  | TEST QC |

Guidelines & Restrictions:

- LABLOTCTL cannot be left blank.

- LABLOTCTL in the TEST file must have a matching record in the QC file.

# LABQCID

Definition: The Laboratory QC Sample ID is a laboratory-assigned QC sample ID. This field is equivalent to the LABSAMPID in the TEST file.

| Attributes: | C12 |  |
| --- | --- | --- |
| Included in Tables: |  | QC |

Guidelines & Restrictions:

- LABQCID cannot be left blank.

- LABQCID must be unique.

# LABREFID

Definition: The Laboratory Reference Sample ID is the laboratory-assigned sample ID of the sample upon which the QC sample is referenced in order to calculate the QC result. A reference sample is used in conjunction with a QC sample (LABQCID) to determine precision and accuracy.

| Attributes: | C12 |  |
| --- | --- | --- |
| Included in Tables: |  | QC |

Guidelines & Restrictions:

- LABREFID cannot be left blank when QCCODE = “MS/SD” or “LR,” and must be left blank in all other cases.

- Enter the LABSAMPID of the client sample that was spiked or replicated in the LABREFID field.

# LABSAMPID

Definition: The Laboratory Sample ID is the unique identifier assigned to a sample by the laboratory performing the analysis.

| Attributes: | C12 |  |
| --- | --- | --- |
| Included in Tables: |  | TEST RESULTS |

Guidelines & Restrictions:

- LABSAMPID cannot be left blank.

- LABSAMPID must be unique.

- In the QC file, LABSAMPID is equivalent to the LABQCID.

# LNOTE

Definition: The LNOTE field is used for Laboratory Notes (descriptive notes and/or data qualifiers that may be used to more completely describe the analytical data). Notes may be applied to both the test and result records, and are also known as “TLNOTE” and “RLNOTE” respectively.

| Attributes: | C20 |  |
| --- | --- | --- |
| Included in Tables: |  | TEST RESULTS |

Guidelines & Restrictions:

- LNOTE may be left blank.

- LNOTE must contain a valid value if populated.

- The same set of LNOTEs may be used to qualify entire analytical tests or individual results.

- If more than one LNOTE is used, commas without spaces separate the codes (e.g., “AZ,B,CI”).

- LNOTEs beginning with “V” are to be used by validators, and not by the analytical laboratory.

Acceptable LNOTE Values:

| Code | Description |  |
| --- | --- | --- |
| A |  | EPA Flag - TIC is a suspected aldol-condensation product |
| AA |  | Kerosene range not reported due to overlap of hydrocarbons |
| AB |  | Diesel range not reported due to overlap of hydrocarbon range |
| AC |  | Heavier hydrocarbons contributing to diesel range quantitation |
| AD |  | Typical pattern for diesel |
| AE |  | Unknown hydrocarbon with a single peak |
| AF |  | Hydrocarbon response is in the C7-C12 range |
| AG |  | Hydrocarbon response is in the C9-C12 range |
| AH |  | Hydrocarbon response is in excess of C22 |
| AJ |  | Heavier hydrocarbon than diesel |
| AK |  | Lighter hydrocarbon than diesel |
| AL |  | Hydrocarbon response is in the C8-C12 range |
| AM |  | Hydrocarbon response is in the C12-C22 range |
| AN |  | Unknown hydrocarbon with several peaks |
| AO |  | Typical pattern for gasoline |
| AP |  | Hydrocarbon response is in the C7-C14 range |
| AQ |  | Hydrocarbon response is in the C9-C14 range |
| AR |  | Hydrocarbon response is in excess of C10 |
| AS |  | Heavier hydrocarbon than gasoline |
| AT |  | Lighter hydrocarbon than gasoline |
| AU |  | Injection precision not met |
| AW |  | Detection limit increased due to dilution factor |
| AX |  | Sample too dilute to quantify surrogate |
| AY |  | Matrix interference suspected |
| AZ |  | Surr. recovery outside of acceptance limits due to matrix interf. |
| B |  | EPA Flag - Analyte present in the blank and the sample |
| BA |  | Relative percent difference out of control |
| BB |  | Sample > 4x spike concentration |
| BC |  | Matrix spike out of control, lab control sample within limits |
| BD |  | Concentration value greater than 25% difference between columns |
| BE |  | Low surrogate recovery; analyzed twice |
| BF |  | Reporting limits raised due to high hydrocarbon background |
| BG |  | Reporting limits raised due to interelement interference |
| BH |  | Reporting limits raised due to high level of non-target analytes |
| BI |  | Sample does not resemble standard |
| BJ |  | Analyte detected in blank and sample |
| BK |  | Hexavalent chromiun not available; total chromium analyzed |
| BL |  | Compound unidentified at a second dilution |
| BM |  | Sustains ignition |
| BN |  | Ignites but does not sustain ignition |
| BO |  | Foaming during purge cycle |
| BP |  | Sample type i, a millable solid |
| BQ |  | Sample type ii, a liquid solid mixture |
| BR |  | Sample type iii, a non-filterable, non-millable sludge |
| BS |  | Insufficient sample available to follow standard QC procedures |
| BT |  | Insufficient sample to perform the analysis |
| BU |  | Sample analyzed after holding time expired |
| BV |  | Sample received after holding time expired |
| BW |  | Sample extract analyzed after holding time expired |
| BX |  | Sample stored at improper temperature |
| BY |  | Sample received at improper temperature |
| BZ |  | Sample preserved improperly |
| C |  | EPA Flag - Pesticide result confirmed using GC/MS |
| CA |  | Sample contains white precipitate |
| CB |  | Sample contains floculant material |
| CC |  | Sample contains free product |
| CD |  | Sample contains multiple phases |
| CE |  | Sample not homogeneous |
| CF |  | Sample releases strong sulfur odor |
| CG |  | Sample releases strong solvent odor |
| CH |  | Sample releases strong petroleum odor |
| CI |  | See narrative |
| CJ |  | Analyte concentration is in excess of the instrument calibration |
| CK |  | Initial analysis within holding time but failed QA/QC criteria |
| CL |  | Initial analysis within holding time but required dilution |
| CM |  | Reporting limits elevated due to low percent solids |
| CN |  | Hydrocarbon response in diesel range but does not resemble diesel |
| CO |  | Hydrocarbon response in gasoline range but does not resemble gas |
| CP |  | Analyte conc. detectable, but less than 10 times blank conc. |
| CQ |  | Analyte conc. greater that 10 times the blank conc. |
| CR |  | QC criteria not met, sample re-analyzed with similar results |
| CS |  | QC criteria not met due to analyte concentration near RDL |
| CT |  | QC criteria not met due to high level of analyte concentration |
| CU |  | Surrogate concentration diluted to not detectable during analysis |
| CV |  | Analysis was performed on a 2M KCl extract |
| CW |  | Carbonate alkalinity is zero because the sample's pH is < 8.3 |
| CX |  | Analysis was performed on a 0.01N HCl extract |
| CY |  | Analysis was performed on a deionized water extract |
| CZ |  | Homogeneity could not be readily achieved using routine methods |
| D |  | EPA Flag - Analytes analyzed at a secondary dilution |
| DA |  | Non-fuel components excluded from the results calculation |
| DB |  | QA results outside of acceptance limits due to matrix effects |
| DC |  | Sample produced an emulsion during preparation |
| DD |  | A precipitate formed in the extract during sample preparation |
| DE |  | Toxic interferences suspected |
| DF |  | Reporting limits elevated due to matrix interferences |
| DG |  | Reporting limits elevated due to sample dilution |
| DH |  | Reporting limits elevated due to insufficient sample quantity |
| DI |  | The estimated TIC conc. is < the standard method reporting limit |
| DJ |  | TIC identification based on mass fragmentation pattern only |
| DK |  | TIC identification based on RT and mass fragmentation pattern |
| DL |  | Quantified using 30-weight motor oil standard |
| DM |  | Multi-component hydrocarbon mixture |
| DN |  | 2-Sigma Error |
| DO |  | Coelution |
| DP |  | Detected at or above the MRL; refer to reports |
| DQ |  | QC results not applicable because of sample matrix |
| DR |  | Spiked amount less than five times background level |
| DS |  | Batch Quality Assurance data from another project |
| DT |  | Method detection limit elevated |
| DU |  | Insufficient sample quantity for matrix spike/dup matrix spike |
| DV |  | Corr. coef. for Method of Standard Addition is less than 0.995 |
| DW |  | Sample result is less than reported value |
| DX |  | Value < lowest standard (MQL), but > than MDL |
| DY |  | Compound identified is an analysis at a secondary dilution factor |
| DZ |  | The positive result is an atypical pattern for diesel analysis |
| E |  | EPA Flag - Analyte exceeded the concentration range of the GC/MS |
| EA |  | The positive result is an atypical pattern for gasoline analysis |
| EB |  | Value is estimated |
| EC |  | Value determined by the Method of Standard Addition |
| ED |  | Concentration of total analyte ND; therefore this analyte is ND |
| EF |  | Compound quantitated at a different dilution |
| EG |  | Compound quantitated at a 2x dilution factor |
| EH |  | Compound quantitated at a 5x dilution factor |
| EI |  | Compound quantitated at a 10x dilution factor |
| EJ |  | Compound quantitated at a 20x dilution factor |
| EK |  | Compound quantitated at a 50x dilution factor |
| EL |  | Compound quantitated at a 100x dilution factor |
| EM |  | Compound quantitated at a 200x dilution factor |
| EN |  | Compound quantitated at a 500x dilution factor |
| EO |  | Compound quantitated at a 1000x dilution factor |
| EP |  | Compound quantitated at > 1000x dilution factor |
| EQ |  | Holding time is 28 days by anal. NO3-NO2 on pres. split & sub NO2 |
| ER |  | Thiocyanate not analyzed separately, free CN+SCN below RL free C |
| ES |  | Value undetected at the MDL |
| ET |  | Sample was extracted past end of recommended max. holding time |
| EU |  | LCS is outside of acceptance limits. MS/DMS are accept., no corr. |
| EV |  | Modified method; see narrative |
| EW |  | Surr. recovery outside of acceptance limits, spike recov. accept. |
| EX |  | Matrix spike diluted to not detectable during analysis |
| EY |  | Result exceeds normal dynamic range; reported as a min. est. |
| EZ |  | Peak coelution, sample concentration may be artificially high |
| FA |  | Peak coelution, sample concentration may be artificially low |
| FB |  | QC ion deviation > 20% relative to labeled std. used for quant. |
| FC |  | Quantitative interference |
| FD |  | Interferences affect 10% or more of the total PCDD/PCDF peak area |
| FE |  | Carbon-labeled int. std. with s/n ratio of < 10:1 |
| FF |  | S/N ratio 10:1 for the int. std. and assoc. analytes |
| FG |  | Results rejected due to int. std. rec. outside QC limits |
| FH |  | Results are valid even though int. std. rec. outside QC limits |
| FI |  | Analyte in field sample and associated blank |
| FJ |  | Total DPE contribution to the total PCDF value > 10% |
| FK |  | EMPC values could be overestimated due to coelution |
| FL |  | Int. std. ion-abundance ratio measured out of accept. range |
| FM |  | Poorly resolved GC peak(s) |
| FN |  | Large closely eluting peak cannot be separated from result |
| FO |  | Est. maximum possible concentration (EMPC) |
| FP |  | Extractable hydrocarbons |
| FQ |  | Gasoline range organics calculated as JP-4 jet fuel |
| FR |  | Diesel range organics, calculated as JP-4 jet fuel |
| FS |  | Diesel range organics, as diesel |
| FT |  | Methanol extraction |
| FU |  | Heated purge |
| FV |  | Purge volume 5 mL |
| FW |  | Purge volume 25 mL |
| FX |  | Analyte present in the instrument blank |
| FY |  | Chromatogram not typical of calibration standard |
| FZ |  | Identified by MS only; based on 1:1 response w/ internal stnd. |
| GA |  | Components not separable by this method; quantified together |
| GB |  | Matrix spike recovery not within control limits |
| GC |  | Second column confirmation performed |
| GD |  | Secondary column result. |
| GE |  | Post-digestion spike out of control limits |
| GF |  | Sample absorbance is less than 50% of spike absorbance |
| GG |  | Analytical results not reliable for this common lab contaminant |
| GH |  | Reporting limit must be considered an approximation |
| GI |  | Analyte confirmed by GC/MS |
| GJ |  | Result determined using SIM mode |
| GK |  | Quantitated as an isomer pair |
| GL |  | 2,3,7,8-TCDF results have been confirmed on a DB-225 column |
| GM |  | Result calc. by avg. response factor from init. calib. curve |
| GN |  | Surrogate recovery is outside of control limits |
| GO |  | Result est. below lower calib. limit but above target limit |
| GP |  | Duplicate analysis not within control limits |
| GQ |  | Post-digest. spike recov. 40% - 85% due to matrix interference |
| GR |  | Internal standard recovery is outside method recovery limit |
| GS |  | Reporting limit(s) raised: high level target analyte in sample |
| GT |  | Post-digestion spike recovery <40% due to matrix interference |
| GU |  | >10% breakthrough in second carbon col. due to matrix effects |
| GV |  | Result detected is below the lowest standard and above zero |
| GW |  | Post-digestion spike recovery between 115%-150% |
| GX |  | All reporting limits raised due to high conc. dissolved solids |
| GY |  | Analyte assoc. with sample processing & analysis in lab environ. |
| GZ |  | Secondary ion used for quantitation |
| H |  | A holding time violation has occurred. |
| HA |  | Quantit. of unknown hydrocarbon(s) in sample based on diesel |
| HB |  | Quantit. of unknown hydrocarbon(s) in sample based on gasoline |
| HC |  | Analyte is this compound or an isomer |
| HD |  | Chromat. profile inconsistent with pattern(s) of ref. fuel stnds. |
| HE |  | Extract was run through separation method; see case narrative |
| HF |  | SW3630B clean-up method performed |
| HG |  | Result exceeds calib. range, but not lab's; considered valid |
| HH |  | Result exceeds linear range; concentration may be understated |
| HI |  | SW3611B clean-up method performed |
| HJ |  | Blank spike recovery not within control limits |
| HK |  | Calcium carbonate is greater than 5% of the sample weight |
| HL |  | Analyte recovery above established limit |
| HM |  | Analyte recovery below established limit |
| HN |  | Low concentration matrix spike recovery out of limits |
| HO |  | High concentration matrix spike recovery out of limits |
| HP |  | Low concentration blank spike recovery out of limits |
| HQ |  | High concentration blank spike recovery out of limits |
| HR |  | Post-digestion spike |
| HS |  | Spike analyte recovery is outside stated control limits |
| HT |  | Analytical value calculated using results from associated tests |
| HU |  | Sample diluted beyond ability to quantitate % recovery and RPD |
| HV |  | Standard preparation factor adjusted, or aliquot diluted |
| J |  | EPA Flag - Estimated value |
| M |  | A matrix effect is present. |
| N |  | EPA Flag - Presumptive evidence of a compound |
| P |  | EPA Flag - > 25% D for detected concentrations between 2 columns |
| PR |  | Preliminary result |
| R |  | EPA Flag - Data rejected |
| SG |  | A silica gel cleanup procedure was performed. |
| U |  | EPA Flag - Compound was analyzed for, but was not detected |
| UN |  | Data Unavailable |
| VA |  | Val. Qual.: Refer to report assoc. w/ sampling event for details |
| VB |  | Val. Qual.: Analyte present in the blank and the sample |
| VBX |  | Val. Qual.: Sample stored at improper temperature |
| VBY |  | Val. Qual.: Sample received at improper temperature |
| VC |  | Val. Qual.: Calibration nonconformances |
| VCD |  | Val. Qual.: Lab Control Sample dup. RPD outside of estab. limits |
| VDT |  | Val. Qual.: Diss. metal result > total, beyond stnd. meth. var. |
| VDX |  | Val. Qual.: Value < lowest standard (MQL), but > than MDL |
| VDZ |  | Val. Qual.: Positive result is atypical pattern for diesel anal. |
| VF |  | Val. Qual.: Compound is common field contaminant |
| VFB |  | Val. Qual.: Analyte detected in associated field blank |
| VFD |  | Val. Qual.: Field duplicate RPD outside of established limits |
| VFLB |  | Val. Qual.: Analyte detected in associated filter blank |
| VH |  | Val. Qual.: Holding Time exceedence |
| VHB |  | Val. Qual.: Result positively biased |
| VJ |  | Val. Qual.: Estimated value |
| VL |  | Val. Qual.: Compound is common lab contaminant |
| VLB |  | Val. Qual.: Result negatively biased |
| VLH |  | Val. Qual.: Lab control sample recoveries above estab. limits |
| VLL |  | Val. Qual.: Lab control sample recoveries below estab. limits |
| VM |  | Val. Qual.: Nonconformance due to matrix effects |
| VMB |  | Val. Qual.: Analyte detected in associated method blank |
| VMD |  | Val. Qual.: Matrix Spike dup. RPD outside of established limits |
| VMH |  | Val. Qual.: Matrix spike recoveries above established limits |
| VML |  | Val. Qual.: Matrix spike recoveries below established limits |
| VNB |  | Val. Qual.: Result bias cannot be determined |
| VP |  | Val. Qual.: Sample Chromat. pattern does not match calib. pattern |
| VPH |  | Val. Qual.: Post prep. spike recoveries above established limits |
| VPL |  | Val. Qual.: Post prep. spike recoveries below established limits |
| VQ |  | Val. Qual.: QA/QC protocols were not met |
| VQB |  | Val. Qual.: QA/QC protocols not met for method blank |
| VQC |  | Val. Qual.: QA/QC protocols not met for calibration |
| VQH |  | Val. Qual.: QA/QC protocols not met for holding times |
| VQI |  | Val. Qual.: QA/QC protocols not met for internal standard |
| VQL |  | Val. Qual.: QA/QC protocols not met for lab control sample |
| VQM |  | Val. Qual.: QA/QC protocols not met for matrix spike/spike dup. |
| VQN |  | Val. Qual.: QC data does not exist (hist. data) or is unavailable |
| VQQ |  | Val. Qual.: PQL approx. due to QC or matrix effects |
| VQS |  | Val. Qual.: QA/QC protocols not met for surrogate recovery |
| VQT |  | Val. Qual.: QA/QC protocols not met for instr.12-hr tuning crit. |
| VQU |  | Val. Qual.: Non-detect above PQL; final result is now PQL |
| VR |  | Val. Qual.: Rejected value |
| VRB |  | Val. Qual.: Analyte detected in assoc. equipment rinsate blank |
| VRL |  | Val. Qual.: The MQL is above the regulatory limit |
| VS |  | Val. Qual.: Sample Receipt nonconformance |
| VSG |  | Val. Qual.: A silica gel cleanup procedure was performed |
| VSH |  | Val. Qual.: Surrogate recoveries above established limits |
| VSL |  | Val. Qual.: Surrogate recoveries below established limits |
| VSR |  | Val. Qual.: Semi-quantitative result |
| VTB |  | Val. Qual.: Analyte detected in associated trip blank sample |
| VU |  | Val. Qual.: Compound analyzed for but not detected |

# LOCID

Definition: The Location ID is a unique identifier assigned to a specific point (location) where measurements or samples are taken.

| Attributes: | C10 |  |
| --- | --- | --- |
| Included in Tables: |  | SAMPLE TEST |

Guidelines & Restrictions:

- LOCID may be left blank.

- If LOCID is unknown (i.e., not present on the chain-of-custody), enter “DU” for “Data Unavailable.”

# LOGCODE

Definition: The LOGCODE represents the Field Organization that is responsible for collecting samples and related field data (i.e., environmental sampling information).

| Attributes: | C4 |  |
| --- | --- | --- |
| Included in Tables: |  | SAMPLE TEST |

Guidelines & Restrictions:

- LOGCODE cannot be left blank when QCCODE = “CS,” and must be left blank in all other cases.

- LOGCODE must contain a valid value.

Acceptable LOGCODE Values Sorted by Code:

| Code | Description |
| --- | --- |
| ABB | ABB Environmental Services Inc., Portland, ME |
| ABRF | ABR, Inc., Fairbanks, AK |
| ACUX | Acurex Corporation |
| ADPM | Aquarius Drilling and Pumps, Merced, CA |
| ADW | Arizona Department of Water Resources |
| AEHA | Army Environmental Hygiene Agency (AEHA), APG, MD |
| AEI | Ambler Exploration, Inc., Anchorage, AK |
| AEMC | American Environmental Management Corp., Rancho Cordova, CA |
| AERO | Aerovironment, Inc. |
| AGRA | AGRA Earth & Environmental, Fairbanks, AK |
| ANDX | Anderson Excavating |
| ANI | Anacon, Inc. |
| ANL | Argonne National Laboratory, Argonne, IL |
| APC | Anderson Pump Co., Chowchilla, CA |
| APER | Applied Environmental, Inc., Reston, VA |
| APPL | Agriculture & Priority Pollutant |
| ARAS | Applied Research Associates, Inc., South Royalton, VT |
| ARI | Analytical Resources, Inc., Seattle, WA |
| ATEC | ATEC Associates |
| ATIS | Analytical Technologies, Inc., San Diego, CA |
| AWR | A & W Environmental Drilling, RI |
| AWSP | Andrews Well Service, Perry, GA |
| AWWS | Allen Water Well Service |
| BAKC | Baker Environmental, Inc., Coraopolis, PA |
| BAT | Battelle Columbus Division |
| BBR | Bill Belknap, Reedley, CA |
| BCG | Brown & Caldwell Analytical, Glendale, CA |
| BESC | Bristol Environmental Services Corporation |
| BESD | Bioenvironmental Eng. Services Division, USAF |
| BESH | Brewer Environmental Services, Honolulu, HI |
| BEYL | Beylik Drilling Co. |
| BLUR | Blue Ridge Associates, Inc., Spokane, WA |
| BMCD | Burns & McDonnell, Kansas City, MO |
| BOW | Bowser-Morner |
| BREA | Brown and Root Environmental, Albuquerque, NM |
| BREO | Brown and Root Environmental, Oak Ridge, TN |
| BSKF | BSK & Associates, Fresno, CA |
| BVA | B & V Waste Science & Technology Corp., Atlanta, GA |
| BVEA | Black & Veatch, Overland, KS |
| BVKC | B & V Waste Science & Technology Corp., Kansas City, MO |
| BWD | Bills Well and Drilling Co., Fayetteville, NC |
| CAL | California Analytical Lab |
| CALT | Calwater Drilling Co., Turlock, CA |
| CASS | Continental Analytical Services, Inc. |
| CAWL | California Water Labs, Inc. |
| CDMA | Camp, Dresser and McKee Federal Program Corp., Atlanta, GA |
| CECS | Converse Environmental Consultants, San Francisco, CA |
| CEEP | C-E Environmental, Inc., Portland, MN |
| CEFL | Cecon Corporation, Fort Lewis |
| CHEN | Chen Northern, Inc., San Antonio, TX |
| CHM | CH2M Hill |
| CHMA | CH2M Hill, Anchorage, AK |
| CHMR | CH2M Hill , Reno, NV |
| CHMS | CH2M Hill, Salt Lake City, UT |
| CLEA | Clearwater Environmental, AK |
| CLMK | USACE, Vicksburg District |
| CLMM | USACE, Memphis District |
| CLMN | USACE, New Orleans District |
| CLMS | USACE, St. Louis District |
| CLMV | USACE, Lower Mississippi Valley Division |
| CLWT | Calwater Drilling |
| CMRD | USACE, Mississippi River Division |
| CMRK | USACE, Kansas City District |
| CMRO | USACE, Omaha District |
| CNAB | USACE, Baltimore District |
| CNAD | USACE, North Atlantic Division |
| CNAN | USACE, New York District |
| CNAO | USACE, Norfolk District |
| CNAP | USACE, Philadelphia District |
| CNCB | USACE, Buffalo District |
| CNCC | USACE, Chicago District |
| CNCD | USACE, North Central Division |
| CNCE | USACE, Detroit District |
| CNCR | USACE, Rock Island District |
| CNCS | USACE, St. Paul District |
| CNED | USACE, New England Division |
| CNGT | Chem-Nuclear Geotech, Grand Junction, CO |
| CNPD | USACE, North Pacific Division |
| CNPP | USACE, Portland District |
| CNPS | USACE, Seattle District |
| CNPW | USACE, Walla Walla District |
| CORD | USACE, Ohio River Division |
| CORH | USACE, Huntington District |
| CORL | USACE, Louisville District |
| CORN | USACE, Nashville District |
| CORP | USACE, Pittsburgh District |
| CPOA | USACE, Alaska District |
| CPOD | USACE, Pacific Ocean Division |
| CPOF | USACE, Far East District |
| CPOH | USACE, Honolulu District |
| CPOJ | USACE, Japan Engineer District |
| CRCT | CRC & Associates, Inc., Tulsa, OK |
| CRU | Charles R. Underwood, Sanford, NC |
| CSAC | USACE, Charleston District |
| CSAD | USACE, South Atlantic Division |
| CSAJ | USACE, Jacksonville District |
| CSAM | USACE, Mobile District |
| CSAS | USACE, Savannah District |
| CSAW | USACE, Wilmington District |
| CSPD | USACE, South Pacific Division |
| CSPK | USACE, Sacramento District |
| CSPL | USACE, Los Angeles District |
| CSPN | USACE, San Francisco District |
| CSWA | USACE, Albuquerque District |
| CSWD | USACE, Southwestern Division |
| CSWF | USACE, Fort Worth District |
| CSWG | USACE, Galveston District |
| CSWL | USACE, Little Rock District |
| CSWT | USACE, Tulsa District |
| CTAC | USACE, Transatlantic Programs Center |
| CTAE | USACE, Europe Programs Center |
| CTEA | CT&E Environmental Services, Inc., Anchorage, AK |
| CWP | Carolina Well and Pump, Co., Sanford, NC |
| DCPT | DataChem Professionals, Tampa, FL |
| DDA | Discovery Drilling, Anchorage, AK |
| DDI | Denali Drilling, Inc., Anchorage, AK |
| DEE | Desert Earth Engineering |
| DHSC | California Department of Health Services |
| DLMV | Delmarva Drilling |
| DONC | SEC Donohue, Charleston, SC |
| DOWL | Dowl Engineers, Anchorage, AK |
| DPAS | Deer Park Analytical Services Division, Deer Park, TX |
| DU | Data Unavailable |
| DWR | Department of Water Resources |
| DYNC | Dynamac |
| EAH | E.A. Hoffman, CA |
| EAL | EAL Lab |
| EALM | Everett A. Loewenstein, Merced, CA |
| EBAA | EBASCO Services, Inc., Arlington, VA |
| EBAB | EBASCO Environmental, Bellevue, WA |
| EBAC | EBASCO Services, Inc., Santa Ana, CA |
| EBAN | EBASCO Environmental, Norcross, GA |
| EBAS | EBASCO Services, Inc., Oak Ridge, TN |
| EBRS | Evans Brothers |
| ECA | New York State Department of Environmental Conservation |
| ECCA | Environmental Chemical Corp., Aiea, HI |
| ECEN | Ecology & Environment, Inc. |
| ECI | EcoChem, Inc. |
| ECJP | E.C. Jordan Co., Portland, ME |
| EDP | Effinger Drill & Pump Service |
| EEIS | Envirodyne Engineers, Inc., St. Louis, MO |
| EISR | Environmental Industrial Research Associates, Inc. |
| EMIA | Environmental Management, Inc., Anchorage, AK |
| ENAH | Edward K. Noda and Associates, Honolulu, HI |
| ENSE | Enserch Environmental, Bellevue, WA |
| ENSR | Enseco-Rocky Mountain Analytical, Denver, CO |
| ENVD | Environmental Drilling Corp. |
| EPJ | E.P. Johnson Construction & Environmental, Inc., Pasco, WA |
| EPJO | E.P. Johnson Construction & Environmental, Inc. |
| EPT | El Paso Testing Laboratories, Inc., El Paso, TX |
| ERG | Environmental Research Group |
| ERM | Environmental Resources Management, Inc. |
| ESA | Engineering-Science (ES), Atlanta, GA |
| ESCE | Environmental Services (ENSR) Consulting Engineer, Anchorage, AK |
| ESCI | Engineering-Science |
| ESE | Environmental Science & Engineering, Inc. (ESE Inc) |
| ESEG | Environmental Science & Engineering, Inc., Gainsville, FL |
| ESTA | EA Engineering Science & Technology, Inc., Atlanta, GA |
| ESTB | EA Engineering Science & Technology, Inc., Baltimore, MD |
| ESTK | EA Engineering Science & Technology, Inc., Fairbanks, AK |
| ESTL | EA Engineering Science & Technology, Inc., Lincoln, NE |
| ESTM | EA Engineering Science & Technology, Inc., Boston, MA |
| ESTR | EA Engineering Science & Technology, Inc., Redmond, WA |
| ESTS | EA Engineering Science & Technology, Inc., Sparks, MD |
| ETC | Earth Technology Corporation |
| FA | Flood and Assoc. |
| FAA | Federal Aviation Administration |
| FEP | Foss Environmental, Portland, OR |
| FES | Foss Environmental, Seattle, WA |
| FHEA | Foothill Engineering Consultants, Inc., Anchorage, AK |
| FLD | Field Analysis |
| FOS | Foster-Wheeler, Bellevue, WA |
| FOX | Fox & Associates |
| GAM | Geraghty & Miller |
| GBC | Gobike and Bloomer, Chowchilla, CA |
| GEOE | Geoengineers, Inc., AK |
| GEOS | IHS Geotech/CMT, Inc., San Antonio, TX |
| GERC | Geochemical Environmental Research Group, College Station, TX |
| GERI | Geotechnical Resources, Inc., Beaverton, OR |
| GLDC | Golder & Associates, Anchorage, AK |
| GMA | Geraghty & Miller, Aiken, SC |
| GMP | Geraghty & Miller, Phoenix, AZ |
| GMSA | Geomatrix Consultants, Inc., Santa Ana Heights, CA |
| GOCB | G. Oscar Clayton Drilling Service, Buckeye, AZ |
| GRAM | Gram, Inc., Albuquerque, NM |
| GSI | Garry Struthers, Inc., Bellevue, WA |
| GSIM | Geosciences, Inc., Macon, GA |
| GTL | General Testing Labs of Kansas City, MO |
| GWDS | Graves Well Drilling, Inc., Sylacauga, AL |
| GWTC | Groundwater Technology, Inc., Chesapeake, VA |
| GWWG | Greenes Water Wells, Inc., Gray, GA |
| HARA | Hart Crowser, Anchorage, AK |
| HARC | Hart Crowser, Seattle, WA |
| HARG | Hargis & Associates |
| HART | Fred C. Hart Associates, Inc. |
| HBEH | USAF Hospital, Holloman Bioenvironmental Eng. Svcs., Holloman AFB |
| HDRO | Henningson, Duram & Richardson Engineering, Omaha, NE |
| HENM | Hennings Brothers Drilling Company, Inc., Modesto, CA |
| HESC | Harza Environmental Services, Inc., Chicago, IL |
| HET | H, E, and T, Inc. |
| HETA | Hydro-Environmental Technologies, Inc., Albany, NY |
| HGCL | HGCL Environmental Scientists and Engineers, Albuquerque, NM |
| HLA | Harding Lawson Associates, Anchorage, AK |
| HLAA | Harding Lawson Associates, Aiea, HI |
| HLAD | Harding Lawson Associates, Denver, CO |
| HLAN | Harding Lawson Associates, Novato, CA |
| HLTH | California State Health Lab |
| HRGT | Hargis & Associates, Tucson, AZ |
| HTI | Hydro-Terra, Inc., Columbia, MD |
| ICFK | ICF Kaiser Engineers, Boston, MA |
| ICFP | ICF Kaiser Engineers, Pittsburgh, PA |
| ICFR | ICF Technology, Inc., Richland, WA |
| IEA | Industrial & Environmental Analysis |
| INSO | INSITU Technologies, Orlando, FL |
| ITC | International Technology Corporation, Knoxville, TN |
| ITCC | IT Corporation, Anchorage, AK |
| ITCM | IT (International Technology) Corporation, Martinez, CA |
| ITCN | International Technology Corporation, Nome, AK |
| ITCP | International Technology Corporation, St. Paul, MN |
| JA | JAYCOR, Vienna, VA |
| JBRF | J. B. Rogers, San Francisco, CA |
| JCCM | J. C. Click Driller, Mesa, AZ |
| JEG | Jacobs Engineering Group, Inc., Denver, CO |
| JEGA | Jacobs Engineering Group, Inc., Anchorage, AK |
| JEGM | Jacobs Engineering Group, Inc., Martinez, CA |
| JEGP | Jacobs Engineering Group, Inc., Pasadena, CA |
| JFHM | J. F. Hoffman, Merced, CA |
| JM | J. Mason |
| JMCW | James M. Montgomery, Consulting Engineers, Walnut Creek, CA |
| JMH | J. M. Hensley |
| JMSL | James M. Montgomery, Consulting Engineers, Salt Lake City, UT |
| JOHN | Johnson Drilling, Reedley, CA |
| JRB | JRB Associates |
| JSM | J. S. Murk Engineers |
| KBES | Kelly Air Force Base & Environmental Services |
| KJMM | Kenneth J. McAvoy, Madera, CA |
| KLNF | J. H. Kleinfelder & Associates |
| KSAF | Ken Schmidt and Associates, Fresno, CA |
| LACO | Layne Atlantic Company |
| LAW | Law Environmental, Inc., Kennesaw, GA |
| LAYN | Layne West |
| LDC | Laboratory Data Consultants |
| LES | Layne Environmental Services |
| LETC | Law Engineering Test Co. (Letco) |
| LFE | Louis F. Evans |
| LIND | Linde Company |
| LIVR | J. W. Livermore, CA |
| LKHD | Lockheed Georgia Co. |
| LLNL | Lawrence Livermore National Laboratory, Livermore, CA |
| LOHI | Larsen, Ohlinger and Hill, Inc., Merced, CA |
| LTL | Laucks Testing Lab, Inc. |
| MASM | Masellis Drilling, Modesto, CA |
| MCA | Molzin-Corbin & Assoc. |
| MCLD | McClelland Engineers |
| MDM | Mitchell Drilling, Inc., Merced, CA |
| MDNR | Michigan Department of Natural Resources |
| MEI | Metcalf and Eddy, Inc. |
| MGIZ | Middle Georgia Water System, Inc., Zedulan, GA |
| MMES | Martin Marietta Energy Systems, Inc., Oakridge, TN |
| MOOD | Moodys of Dayton, OH |
| MPSO | Martin Pump and Supply, Oakdale, CA |
| MPWP | Malcolm Pirnie, White Plains, NY |
| MRDO | Missouri River Division, Corps. of Engineers Div. Lab., Omaha, NE |
| MTIE | Metatrace, Inc., Earth City, Mo |
| MWA | Montgomery Watson, Anchorage, AK |
| MWI | Montgomery Watson, Irvine, CA |
| MWS | Montgomery Watson, Seattle, WA |
| MWSL | Montgomery Watson, Salt Lake City, UT |
| MWSS | Montgomery Watson, Steamboat Springs, CO |
| MWWC | Montgomery Watson, Walnut Creek, CA |
| NA | Not Available |
| NOW | Nowicki & Associates, Inc. |
| NUS | NUS Corporation, Pittsburgh, PA |
| NUSH | Halliburton NUS Environmental Corporation, Oak Ridge, TN |
| OASI | OASIS Environmental, Inc., Anchorage, AK |
| OBG | O'Brien & Gere Engineers, Inc., Syracuse, NY |
| OCCA | Osterberg and Carroll, CA |
| OEHL | OEHL Brooks Air Force Base |
| OEIR | OnSite Environmental, Inc., Redmond, WA |
| OHM | O. H. Materials Corporation, Findlay, OH |
| OKDH | Oklahoma Department of Health |
| OLST | Olson Plumbing and Well Drilling, Turlock, CA |
| OSCA | Oil Spill Consultants, Inc., Anchorage, AK |
| OSM | Osterberg and Stewart, Modesto, CA |
| OSTB | Osterberg Brothers, CA |
| OSTR | Osterberg & Stewart, Inc. |
| OWRB | Oklahoma Water Resource Board |
| PAI | Phillips Alaska, Inc., Anchorage, AK |
| PEI | PEI Associates, Inc. |
| PHLE | Philip Environmental |
| PITL | Pittsburg Testing Laboratory |
| PPEC | Phelps Pump Equipment Co. |
| PRSN | Petrochem Recovery Services, Inc., Norfolk, VA |
| PSID | Professional Services Industries, Inc., Dallas, TX |
| PSIM | Professional Services Industries, Inc., Marietta, GA |
| PT | Pat Thompson |
| PVDN | Paug-Vik Development Corp., Naknek, AK |
| PWES | Parrat & Wolff, Inc., East Syracuse, NY |
| QALF | Quality Analytical Laboratories, Inc., Alachua, FL |
| QDA | Quality Drillers, Atwater, CA |
| QTA | Quest Environmental, Anchorage, AK |
| QUES | Quest Environmental |
| RABA | Raba-Kistner Consultants, Inc., San Antonio, TX |
| RAD | Radian Corporation |
| RAS | Radian Analytical Services Lab - Austin, TX |
| RASA | Radian/SAIC |
| RDL | Research & Development Lab |
| REI | Resource Engineering, Houston, TX |
| REIG | Rust Environment & Infrastructure, Greenville, SC |
| RFW | Roy F. Weston, Inc. |
| RFWC | Roy F. Weston, Inc., West Chester, PA |
| RFWL | Roy F. Weston, Lionville Lab |
| RFWS | Roy F. Weston, Stockton Lab |
| RGGJ | Rust Geotech, Inc., Grand Junction, CO |
| RMCA | Roscoe Moss Company, AZ |
| RMI | Residuals Management, Inc. (RMI) |
| RMTM | RMT, Inc., Madison, WI |
| ROWE | Rowe Drilling Company |
| RRC | Riverbend Research Center |
| RTI | Research Triangle Institute, Research Triangle Park, NC |
| RWQD | Regional Water Quality Control Board |
| SAIB | Science Applications International Corporation, Bellevue, WA |
| SAIC | Science Applications International Corporation |
| SANL | Sandia National Laboratories, California, Livermore, CA |
| SBFC | San Bernardino Flood Control |
| SCS | USDA Soil Conservation Service |
| SDH | Simpson Drilling Co., Hilmar, CA |
| SEC | Soil Exploration Company, St. Paul, MN |
| SERB | C.E. Schmidt, PhD., Environmental Consultant, Red Bluff, CA |
| SHD | Sacramento County Health Department |
| SHI | Stang Hydronics, Inc. |
| SLAC | Singer-Layne-Atlantic Company |
| SLS | Savannah Labs, Savannah, GA |
| SRM | S. R. McKinney & Sons |
| SSI | SSI, Inc., TN |
| SWAA | Shannon & Wilson, Inc., Anchorage, AK |
| SWAF | Shannon & Wilson, Inc., Fairbanks, AK |
| SWL | Southwestern Laboratories, Inc., Austin, TX |
| SWLS | Southwestern Laboratories, Inc., San Antonio, TX |
| SWS | Shannon & Wilson, Inc., Seattle, WA |
| SWSA | Shannon & Wilson, Inc. |
| TARC | Target Environmental Services, Co., Columbia, MD |
| TCT | Twin City Testing, Inc. |
| TDS | Tonto Drilling Services, Inc. |
| TECI | Testing Engineers and Consultants, Inc., Troy, MI |
| TFMC | Thomas F. Madison, Cressey, CA |
| TGGA | Trans Global Environmental Geochemistry, Austin, TX |
| TLLA | Tellus Ltd., Anchorage, AK |
| TRC | Tracer Research Corporation, Tucson, AZ |
| TRI | Triangle Laboratories of Houston, Inc., Sugarland, TX |
| TSI | Technical Services, Inc. |
| TTI | Tetra Tech, Inc., Pasadena, CA |
| TTIB | Tetra Tech, Inc., Bellevue, WA |
| TWC | Texas Water Commission, Austin, TX |
| UBTL | Utah Biomedical Testing Lab |
| UNCG | UNC Geotech, Grand Junction, CO |
| UOK | University of Oklahoma |
| URSA | URS Corporation, Anchorage, AK |
| URSC | URS Corporation, Seattle, WA |
| URSH | URS Corporation, Honolulu, HI |
| USAC | U.S. Army Corps of Engineers |
| USAF | U.S. Air Force |
| USBR | U.S. Bureau of Reclamation, Boulder City, NV |
| USDA | U.S. Department of Agriculture |
| USGS | U.S. Geological Survey |
| UURI | University of Utah Research Institute |
| WAFB | Wurtsmith Air Force Base |
| WAR | Water and Air Research, Inc. |
| WHAI | Walk, Haydel & Associates, Inc. |
| WLIC | Wellscon, Inc., Chandler, AZ |
| WS | 17th Weather Squadron (MAC) |
| WSC | Water & Soil Consultants, Inc., Norman, OK |
| WSWP | W. S. Williams, Phoenix, AZ |
| WTRD | Water Development |
| WZN | Warzyn, Inc., Madison, WI |

Acceptable LOGCODE Values Sorted by Description:

| Code | Description |
| --- | --- |
| WS | 17th Weather Squadron (MAC) |
| AWR | A & W Environmental Drilling, RI |
| ABB | ABB Environmental Services Inc., Portland, ME |
| ABRF | ABR, Inc., Fairbanks, AK |
| ACUX | Acurex Corporation |
| AERO | Aerovironment, Inc. |
| AGRA | AGRA Earth & Environmental, Fairbanks, AK |
| APPL | Agriculture & Priority Pollutant |
| AWWS | Allen Water Well Service |
| AEI | Ambler Exploration, Inc., Anchorage, AK |
| AEMC | American Environmental Management Corp., Rancho Cordova, CA |
| ANI | Anacon, Inc. |
| ARI | Analytical Resources, Inc., Seattle, WA |
| ATIS | Analytical Technologies, Inc., San Diego, CA |
| ANDX | Anderson Excavating |
| APC | Anderson Pump Co., Chowchilla, CA |
| AWSP | Andrews Well Service, Perry, GA |
| APER | Applied Environmental, Inc., Reston, VA |
| ARAS | Applied Research Associates, Inc., South Royalton, VT |
| ADPM | Aquarius Drilling and Pumps, Merced, CA |
| ANL | Argonne National Laboratory, Argonne, IL |
| ADW | Arizona Department of Water Resources |
| AEHA | Army Environmental Hygiene Agency (AEHA), APG, MD |
| ATEC | ATEC Associates |
| BVA | B & V Waste Science & Technology Corp., Atlanta, GA |
| BVKC | B & V Waste Science & Technology Corp., Kansas City, MO |
| BAKC | Baker Environmental, Inc., Coraopolis, PA |
| BAT | Battelle Columbus Division |
| BEYL | Beylik Drilling Co. |
| BBR | Bill Belknap, Reedley, CA |
| BWD | Bills Well and Drilling Co., Fayetteville, NC |
| BESD | Bioenvironmental Eng. Services Division, USAF |
| BVEA | Black & Veatch, Overland, KS |
| BLUR | Blue Ridge Associates, Inc., Spokane, WA |
| BOW | Bowser-Morner |
| BESH | Brewer Environmental Services, Honolulu, HI |
| BESC | Bristol Environmental Services Corporation |
| BCG | Brown & Caldwell Analytical, Glendale, CA |
| BREA | Brown and Root Environmental, Albuquerque, NM |
| BREO | Brown and Root Environmental, Oak Ridge, TN |
| BSKF | BSK & Associates, Fresno, CA |
| BMCD | Burns & McDonnell, Kansas City, MO |
| SERB | C.E. Schmidt, PhD., Environmental Consultant, Red Bluff, CA |
| CAL | California Analytical Lab |
| DHSC | California Department of Health Services |
| HLTH | California State Health Lab |
| CAWL | California Water Labs, Inc. |
| CLWT | Calwater Drilling |
| CALT | Calwater Drilling Co., Turlock, CA |
| CDMA | Camp, Dresser and McKee Federal Program Corp., Atlanta, GA |
| CWP | Carolina Well and Pump, Co., Sanford, NC |
| CEEP | C-E Environmental, Inc., Portland, MN |
| CEFL | Cecon Corporation, Fort Lewis |
| CHM | CH2M Hill |
| CHMR | CH2M Hill , Reno, NV |
| CHMA | CH2M Hill, Anchorage, AK |
| CHMS | CH2M Hill, Salt Lake City, UT |
| CRU | Charles R. Underwood, Sanford, NC |
| CNGT | Chem-Nuclear Geotech, Grand Junction, CO |
| CHEN | Chen Northern, Inc., San Antonio, TX |
| CLEA | Clearwater Environmental, AK |
| CASS | Continental Analytical Services, Inc. |
| CECS | Converse Environmental Consultants, San Francisco, CA |
| CRCT | CRC & Associates, Inc., Tulsa, OK |
| CTEA | CT&E Environmental Services, Inc., Anchorage, AK |
| DU | Data Unavailable |
| DCPT | DataChem Professionals, Tampa, FL |
| DPAS | Deer Park Analytical Services Division, Deer Park, TX |
| DLMV | Delmarva Drilling |
| DDI | Denali Drilling, Inc., Anchorage, AK |
| DWR | Department of Water Resources |
| DEE | Desert Earth Engineering |
| DDA | Discovery Drilling, Anchorage, AK |
| DOWL | Dowl Engineers, Anchorage, AK |
| DYNC | Dynamac |
| EAH | E.A. Hoffman, CA |
| ECJP | E.C. Jordan Co., Portland, ME |
| EPJO | E.P. Johnson Construction & Environmental, Inc. |
| EPJ | E.P. Johnson Construction & Environmental, Inc., Pasco, WA |
| ESTA | EA Engineering Science & Technology, Inc., Atlanta, GA |
| ESTB | EA Engineering Science & Technology, Inc., Baltimore, MD |
| ESTM | EA Engineering Science & Technology, Inc., Boston, MA |
| ESTK | EA Engineering Science & Technology, Inc., Fairbanks, AK |
| ESTL | EA Engineering Science & Technology, Inc., Lincoln, NE |
| ESTR | EA Engineering Science & Technology, Inc., Redmond, WA |
| ESTS | EA Engineering Science & Technology, Inc., Sparks, MD |
| EAL | EAL Lab |
| ETC | Earth Technology Corporation |
| EBAB | EBASCO Environmental, Bellevue, WA |
| EBAN | EBASCO Environmental, Norcross, GA |
| EBAA | EBASCO Services, Inc., Arlington, VA |
| EBAS | EBASCO Services, Inc., Oak Ridge, TN |
| EBAC | EBASCO Services, Inc., Santa Ana, CA |
| ECI | EcoChem, Inc. |
| ECEN | Ecology & Environment, Inc. |
| ENAH | Edward K. Noda and Associates, Honolulu, HI |
| EDP | Effinger Drill & Pump Service |
| EPT | El Paso Testing Laboratories, Inc., El Paso, TX |
| ESCI | Engineering-Science |
| ESA | Engineering-Science (ES), Atlanta, GA |
| ENSR | Enseco-Rocky Mountain Analytical, Denver, CO |
| ENSE | Enserch Environmental, Bellevue, WA |
| EEIS | Envirodyne Engineers, Inc., St. Louis, MO |
| ECCA | Environmental Chemical Corp., Aiea, HI |
| ENVD | Environmental Drilling Corp. |
| EISR | Environmental Industrial Research Associates, Inc. |
| EMIA | Environmental Management, Inc., Anchorage, AK |
| ERG | Environmental Research Group |
| ERM | Environmental Resources Management, Inc. |
| ESE | Environmental Science & Engineering, Inc. (ESE Inc) |
| ESEG | Environmental Science & Engineering, Inc., Gainsville, FL |
| ESCE | Environmental Services (ENSR) Consulting Engineer, Anchorage, AK |
| EBRS | Evans Brothers |
| EALM | Everett A. Loewenstein, Merced, CA |
| FAA | Federal Aviation Administration |
| FLD | Field Analysis |
| FA | Flood and Assoc. |
| FHEA | Foothill Engineering Consultants, Inc., Anchorage, AK |
| FEP | Foss Environmental, Portland, OR |
| FES | Foss Environmental, Seattle, WA |
| FOS | Foster-Wheeler, Bellevue, WA |
| FOX | Fox & Associates |
| HART | Fred C. Hart Associates, Inc. |
| GOCB | G. Oscar Clayton Drilling Service, Buckeye, AZ |
| GSI | Garry Struthers, Inc., Bellevue, WA |
| GTL | General Testing Labs of Kansas City, MO |
| GERC | Geochemical Environmental Research Group, College Station, TX |
| GEOE | Geoengineers, Inc., AK |
| GMSA | Geomatrix Consultants, Inc., Santa Ana Heights, CA |
| GSIM | Geosciences, Inc., Macon, GA |
| GERI | Geotechnical Resources, Inc., Beaverton, OR |
| GAM | Geraghty & Miller |
| GMA | Geraghty & Miller, Aiken, SC |
| GMP | Geraghty & Miller, Phoenix, AZ |
| GBC | Gobike and Bloomer, Chowchilla, CA |
| GLDC | Golder & Associates, Anchorage, AK |
| GRAM | Gram, Inc., Albuquerque, NM |
| GWDS | Graves Well Drilling, Inc., Sylacauga, AL |
| GWWG | Greenes Water Wells, Inc., Gray, GA |
| GWTC | Groundwater Technology, Inc., Chesapeake, VA |
| HET | H, E, and T, Inc. |
| NUSH | Halliburton NUS Environmental Corporation, Oak Ridge, TN |
| HLAA | Harding Lawson Associates, Aiea, HI |
| HLA | Harding Lawson Associates, Anchorage, AK |
| HLAD | Harding Lawson Associates, Denver, CO |
| HLAN | Harding Lawson Associates, Novato, CA |
| HARG | Hargis & Associates |
| HRGT | Hargis & Associates, Tucson, AZ |
| HARA | Hart Crowser, Anchorage, AK |
| HARC | Hart Crowser, Seattle, WA |
| HESC | Harza Environmental Services, Inc., Chicago, IL |
| HENM | Hennings Brothers Drilling Company, Inc., Modesto, CA |
| HDRO | Henningson, Duram & Richardson Engineering, Omaha, NE |
| HGCL | HGCL Environmental Scientists and Engineers, Albuquerque, NM |
| HETA | Hydro-Environmental Technologies, Inc., Albany, NY |
| HTI | Hydro-Terra, Inc., Columbia, MD |
| ICFK | ICF Kaiser Engineers, Boston, MA |
| ICFP | ICF Kaiser Engineers, Pittsburgh, PA |
| ICFR | ICF Technology, Inc., Richland, WA |
| GEOS | IHS Geotech/CMT, Inc., San Antonio, TX |
| IEA | Industrial & Environmental Analysis |
| INSO | INSITU Technologies, Orlando, FL |
| ITC | International Technology Corporation, Knoxville, TN |
| ITCN | International Technology Corporation, Nome, AK |
| ITCP | International Technology Corporation, St. Paul, MN |
| ITCM | IT (International Technology) Corporation, Martinez, CA |
| ITCC | IT Corporation, Anchorage, AK |
| JBRF | J. B. Rogers, San Francisco, CA |
| JCCM | J. C. Click Driller, Mesa, AZ |
| JFHM | J. F. Hoffman, Merced, CA |
| KLNF | J. H. Kleinfelder & Associates |
| JMH | J. M. Hensley |
| JM | J. Mason |
| JSM | J. S. Murk Engineers |
| LIVR | J. W. Livermore, CA |
| JEGA | Jacobs Engineering Group, Inc., Anchorage, AK |
| JEG | Jacobs Engineering Group, Inc., Denver, CO |
| JEGM | Jacobs Engineering Group, Inc., Martinez, CA |
| JEGP | Jacobs Engineering Group, Inc., Pasadena, CA |
| JMSL | James M. Montgomery, Consulting Engineers, Salt Lake City, UT |
| JMCW | James M. Montgomery, Consulting Engineers, Walnut Creek, CA |
| JA | JAYCOR, Vienna, VA |
| JOHN | Johnson Drilling, Reedley, CA |
| JRB | JRB Associates |
| KBES | Kelly Air Force Base & Environmental Services |
| KSAF | Ken Schmidt and Associates, Fresno, CA |
| KJMM | Kenneth J. McAvoy, Madera, CA |
| LDC | Laboratory Data Consultants |
| LOHI | Larsen, Ohlinger and Hill, Inc., Merced, CA |
| LTL | Laucks Testing Lab, Inc. |
| LETC | Law Engineering Test Co. (Letco) |
| LAW | Law Environmental, Inc., Kennesaw, GA |
| LLNL | Lawrence Livermore National Laboratory, Livermore, CA |
| LACO | Layne Atlantic Company |
| LES | Layne Environmental Services |
| LAYN | Layne West |
| LIND | Linde Company |
| LKHD | Lockheed Georgia Co. |
| LFE | Louis F. Evans |
| MPWP | Malcolm Pirnie, White Plains, NY |
| MMES | Martin Marietta Energy Systems, Inc., Oakridge, TN |
| MPSO | Martin Pump and Supply, Oakdale, CA |
| MASM | Masellis Drilling, Modesto, CA |
| MCLD | McClelland Engineers |
| MTIE | Metatrace, Inc., Earth City, Mo |
| MEI | Metcalf and Eddy, Inc. |
| MDNR | Michigan Department of Natural Resources |
| MGIZ | Middle Georgia Water System, Inc., Zedulan, GA |
| MRDO | Missouri River Division, Corps. of Engineers Div. Lab., Omaha, NE |
| MDM | Mitchell Drilling, Inc., Merced, CA |
| MCA | Molzin-Corbin & Assoc. |
| MWA | Montgomery Watson, Anchorage, AK |
| MWI | Montgomery Watson, Irvine, CA |
| MWSL | Montgomery Watson, Salt Lake City, UT |
| MWS | Montgomery Watson, Seattle, WA |
| MWSS | Montgomery Watson, Steamboat Springs, CO |
| MWWC | Montgomery Watson, Walnut Creek, CA |
| MOOD | Moodys of Dayton, OH |
| ECA | New York State Department of Environmental Conservation |
| NA | Not Available |
| NOW | Nowicki & Associates, Inc. |
| NUS | NUS Corporation, Pittsburgh, PA |
| OHM | O. H. Materials Corporation, Findlay, OH |
| OASI | OASIS Environmental, Inc., Anchorage, AK |
| OBG | O'Brien & Gere Engineers, Inc., Syracuse, NY |
| OEHL | OEHL Brooks Air Force Base |
| OSCA | Oil Spill Consultants, Inc., Anchorage, AK |
| OKDH | Oklahoma Department of Health |
| OWRB | Oklahoma Water Resource Board |
| OLST | Olson Plumbing and Well Drilling, Turlock, CA |
| OEIR | OnSite Environmental, Inc., Redmond, WA |
| OSTR | Osterberg & Stewart, Inc. |
| OCCA | Osterberg and Carroll, CA |
| OSM | Osterberg and Stewart, Modesto, CA |
| OSTB | Osterberg Brothers, CA |
| PWES | Parrat & Wolff, Inc., East Syracuse, NY |
| PT | Pat Thompson |
| PVDN | Paug-Vik Development Corp., Naknek, AK |
| PEI | PEI Associates, Inc. |
| PRSN | Petrochem Recovery Services, Inc., Norfolk, VA |
| PPEC | Phelps Pump Equipment Co. |
| PHLE | Philip Environmental |
| PAI | Phillips Alaska, Inc., Anchorage, AK |
| PITL | Pittsburg Testing Laboratory |
| PSID | Professional Services Industries, Inc., Dallas, TX |
| PSIM | Professional Services Industries, Inc., Marietta, GA |
| QALF | Quality Analytical Laboratories, Inc., Alachua, FL |
| QDA | Quality Drillers, Atwater, CA |
| QUES | Quest Environmental |
| QTA | Quest Environmental, Anchorage, AK |
| RABA | Raba-Kistner Consultants, Inc., San Antonio, TX |
| RAS | Radian Analytical Services Lab - Austin, TX |
| RAD | Radian Corporation |
| RASA | Radian/SAIC |
| RWQD | Regional Water Quality Control Board |
| RDL | Research & Development Lab |
| RTI | Research Triangle Institute, Research Triangle Park, NC |
| RMI | Residuals Management, Inc. (RMI) |
| REI | Resource Engineering, Houston, TX |
| RRC | Riverbend Research Center |
| RMTM | RMT, Inc., Madison, WI |
| RMCA | Roscoe Moss Company, AZ |
| ROWE | Rowe Drilling Company |
| RFW | Roy F. Weston, Inc. |
| RFWC | Roy F. Weston, Inc., West Chester, PA |
| RFWL | Roy F. Weston, Lionville Lab |
| RFWS | Roy F. Weston, Stockton Lab |
| REIG | Rust Environment & Infrastructure, Greenville, SC |
| RGGJ | Rust Geotech, Inc., Grand Junction, CO |
| SRM | S. R. McKinney & Sons |
| SHD | Sacramento County Health Department |
| SBFC | San Bernardino Flood Control |
| SANL | Sandia National Laboratories, California, Livermore, CA |
| SLS | Savannah Labs, Savannah, GA |
| SAIC | Science Applications International Corporation |
| SAIB | Science Applications International Corporation, Bellevue, WA |
| DONC | SEC Donohue, Charleston, SC |
| SWSA | Shannon & Wilson, Inc. |
| SWAA | Shannon & Wilson, Inc., Anchorage, AK |
| SWAF | Shannon & Wilson, Inc., Fairbanks, AK |
| SWS | Shannon & Wilson, Inc., Seattle, WA |
| SDH | Simpson Drilling Co., Hilmar, CA |
| SLAC | Singer-Layne-Atlantic Company |
| SEC | Soil Exploration Company, St. Paul, MN |
| SWL | Southwestern Laboratories, Inc., Austin, TX |
| SWLS | Southwestern Laboratories, Inc., San Antonio, TX |
| SSI | SSI, Inc., TN |
| SHI | Stang Hydronics, Inc. |
| TARC | Target Environmental Services, Co., Columbia, MD |
| TSI | Technical Services, Inc. |
| TLLA | Tellus Ltd., Anchorage, AK |
| TECI | Testing Engineers and Consultants, Inc., Troy, MI |
| TTIB | Tetra Tech, Inc., Bellevue, WA |
| TTI | Tetra Tech, Inc., Pasadena, CA |
| TWC | Texas Water Commission, Austin, TX |
| TFMC | Thomas F. Madison, Cressey, CA |
| TDS | Tonto Drilling Services, Inc. |
| TRC | Tracer Research Corporation, Tucson, AZ |
| TGGA | Trans Global Environmental Geochemistry, Austin, TX |
| TRI | Triangle Laboratories of Houston, Inc., Sugarland, TX |
| TCT | Twin City Testing, Inc. |
| USAF | U.S. Air Force |
| USAC | U.S. Army Corps of Engineers |
| USBR | U.S. Bureau of Reclamation, Boulder City, NV |
| USDA | U.S. Department of Agriculture |
| USGS | U.S. Geological Survey |
| UNCG | UNC Geotech, Grand Junction, CO |
| UOK | University of Oklahoma |
| UURI | University of Utah Research Institute |
| URSA | URS Corporation, Anchorage, AK |
| URSH | URS Corporation, Honolulu, HI |
| URSC | URS Corporation, Seattle, WA |
| CPOA | USACE, Alaska District |
| CSWA | USACE, Albuquerque District |
| CNAB | USACE, Baltimore District |
| CNCB | USACE, Buffalo District |
| CSAC | USACE, Charleston District |
| CNCC | USACE, Chicago District |
| CNCE | USACE, Detroit District |
| CTAE | USACE, Europe Programs Center |
| CPOF | USACE, Far East District |
| CSWF | USACE, Fort Worth District |
| CSWG | USACE, Galveston District |
| CPOH | USACE, Honolulu District |
| CORH | USACE, Huntington District |
| CSAJ | USACE, Jacksonville District |
| CPOJ | USACE, Japan Engineer District |
| CMRK | USACE, Kansas City District |
| CSWL | USACE, Little Rock District |
| CSPL | USACE, Los Angeles District |
| CORL | USACE, Louisville District |
| CLMV | USACE, Lower Mississippi Valley Division |
| CLMM | USACE, Memphis District |
| CMRD | USACE, Mississippi River Division |
| CSAM | USACE, Mobile District |
| CORN | USACE, Nashville District |
| CNED | USACE, New England Division |
| CLMN | USACE, New Orleans District |
| CNAN | USACE, New York District |
| CNAO | USACE, Norfolk District |
| CNAD | USACE, North Atlantic Division |
| CNCD | USACE, North Central Division |
| CNPD | USACE, North Pacific Division |
| CORD | USACE, Ohio River Division |
| CMRO | USACE, Omaha District |
| CPOD | USACE, Pacific Ocean Division |
| CNAP | USACE, Philadelphia District |
| CORP | USACE, Pittsburgh District |
| CNPP | USACE, Portland District |
| CNCR | USACE, Rock Island District |
| CSPK | USACE, Sacramento District |
| CSPN | USACE, San Francisco District |
| CSAS | USACE, Savannah District |
| CNPS | USACE, Seattle District |
| CSAD | USACE, South Atlantic Division |
| CSPD | USACE, South Pacific Division |
| CSWD | USACE, Southwestern Division |
| CLMS | USACE, St. Louis District |
| CNCS | USACE, St. Paul District |
| CTAC | USACE, Transatlantic Programs Center |
| CSWT | USACE, Tulsa District |
| CLMK | USACE, Vicksburg District |
| CNPW | USACE, Walla Walla District |
| CSAW | USACE, Wilmington District |
| HBEH | USAF Hospital, Holloman Bioenvironmental Eng. Svcs., Holloman AFB |
| SCS | USDA Soil Conservation Service |
| UBTL | Utah Biomedical Testing Lab |
| WSWP | W. S. Williams, Phoenix, AZ |
| WHAI | Walk, Haydel & Associates, Inc. |
| WZN | Warzyn, Inc., Madison, WI |
| WSC | Water & Soil Consultants, Inc., Norman, OK |
| WAR | Water and Air Research, Inc. |
| WTRD | Water Development |
| WLIC | Wellscon, Inc., Chandler, AZ |
| WAFB | Wurtsmith Air Force Base |

# LOGDATE

Definition: The Collection Date (or “Log Date”) is the date that a sample is collected in the field.

| Attributes: | D8 |  |
| --- | --- | --- |
| Included in Tables: |  | SAMPLE TEST |

Guidelines & Restrictions:

- All date fields must be in the YYYYMMDD format.

- LOGDATE cannot be left blank when QCCODE = “CS,” and must be blank in all other cases.

- LOGDATE must be earlier than or equal to RECDATE.

- LOGDATE must be earlier than or equal to EXTDATE.

- LOGDATE must be earlier than or equal to ANADATE.

- LOGDATE must be earlier than or equal to REP_DATE.

# LOGTIME

Definition: The Collection Time (or “Log Time”) is the time that a sample is collected in the field.

| Attributes: | C4 |  |
| --- | --- | --- |
| Included in Tables: |  | SAMPLE TEST |

Guidelines & Restrictions:

- All time fields must be entered using the military 24-hour clock (0000-2359), HHMM.

- LOGTIME cannot be left blank when QCCODE = “CS,” and must be blank in all other cases.

# LOWERCL

Definition: The Lower Control Limit is the lower limit of a quality control acceptance criterion.

| Attributes: | N4 |  |
| --- | --- | --- |
| Included in Tables: |  | CL |

Guidelines & Restrictions:

- LOWERCL must be an integer greater than or equal to zero and less than or equal to 9999.

- LOWERCL must be less than UPPERCL.

- Enter zero for precision limit.

# MATRIX

Definition: The MATRIX field identifies the sample’s medium or make-up (e.g., soil, air, water, etc.), as categorized by the analytical laboratory.

| Attributes: | C2 |  |
| --- | --- | --- |
| Included in Tables: |  | SAMPLE TEST RESULTS QC CL |

Guidelines & Restrictions:

- MATRIX cannot be left blank.

- MATRIX must contain a valid value.

- Laboratory-generated QC samples using only laboratory reagents may be assigned QC MATRIX codes such as “WQ” (“Water QC Matrix”) for a blank spike. (The use of “*Q” MATRIX codes is recommended for data that will be converted into the Air Force Center for Environmental Excellence [AFCEE] Environmental Resources Program Information Management System [ERPIMS] formats, but is not required.)

- Laboratory-generated samples which use the original environmental sample matrix are assigned the MATRIX code that describes the original sample matrix, rather than the QC sample matrix, (e.g., a matrix spiked waste water sample would be assigned “WW” [“Waste Water”] rather than “WQ” [“Water QC Matrix”]).

- When the laboratory is not completely informed about the exact sample matrix, it should enter the more general MATRIX codes (such as “W” for “Water” and “SO” for “Soil”).

Acceptable MATRIX Values:

| Code | Description |
| --- | --- |
| AA | Ambient Air |
| AD | Drilling Air |
| AQ | Air QC |
| AX | Air - Unk. Origin |
| CF | Fly Ash Cinder |
| DC | Drill Cuttings |
| DU | Data Unavailable |
| GE | Gaseous Effluent (Stack Gas) |
| GL | Gaseous Phase of Liquid |
| GQ | Gaseous Phase QC |
| GS | Soil Gas |
| GV | Vapor Extraction System Vapor |
| LC | Liquid Condensate |
| LD | Drilling Fluid |
| LE | Liquid Emulsion |
| LF | Floating/Free Product - Groundwater |
| LH | Liquid Waste |
| LO | Organic Liquid |
| LT | Floating/Free Product - Tank |
| LV | Vadose Zone Liquid |
| MH | Hazardous Multi-Phase Waste |
| MX | Multi-Phase - Unk. Origin |
| RG | Radioactive Groundwater |
| RK | Rock |
| SB | Bentonite |
| SC | Cement |
| SE | Sediment |
| SF | Filter Sandpack |
| SH | Solid Waste |
| SL | Sludge |
| SM | Water Filter |
| SN | Misc. Solid |
| SO | Soil |
| SP | Well Casing |
| SQ | Soil/Solid QC |
| SR | Water Filter Residue |
| SS | Surface Scrapings |
| ST | Sorbents |
| SW | Swab or Wipe |
| SX | Soil/Solid - Unk. Origin |
| TA | Animal Tissue |
| TP | Plant Tissue |
| TQ | Tissue QC |
| TX | Tissue - Unk. Origin |
| W | Water |
| WC | Drilling Water |
| WD | Well Development Water |
| WE | Estuary |
| WG | Groundwater |
| WH | Equipment Wash Water |
| WL | Leachate |
| WO | Ocean Water |
| WP | Drinking Water |
| WQ | Water QC |
| WS | Surface Water |
| WT | Treatment System Water |
| WV | Vadose Zone Water |
| WW | Waste Water |
| WX | Water - Unk. Origin |
| WZ | Special Water QC |

# METH_DESIGN_ID

Definition: The Method Design ID is the unique identifier for the design of an analytical method.

| Attributes: | C25 |  |
| --- | --- | --- |
| Included in Tables: |  | SAMPLE TEST RESULTS QC CL |

Guidelines & Restrictions:

- METH_DESIGN_ID is an optional field and may be left blank.

- METH_DESIGN_ID is a linking field with the EDMS2000 electronic COC tables.

# MODPARLIST

Definition: The Modified Parameter List field indicates whether the compound list of a method being reported has been amended by omitting compounds (the addition of compounds is not considered a modification).

| Attributes: | L1 |  |
| --- | --- | --- |
| Included in Tables: |  | TEST |

Guidelines & Restrictions:

- MODPARLIST cannot be left blank.

- MODPARLIST must be entered as “T” (“true”) or “F” (“false”).

- Enter “T” if an analyte has been omitted from the reported method list.

# NPDLWO

Definition: The Work Order Number was historically a COE-assigned administration number. This field is no longer used as such, and may be used by the laboratory for internal tracking numbers.

| Attributes: | C7 |  |
| --- | --- | --- |
| Included in Tables: |  | SAMPLE |

Guidelines & Restrictions:

- NPDLWO cannot be left blank.

- Enter “NA,” or use this field for internal tracking purposes.

# PARLABEL

Definition: The PARLABEL represents the Analyte name or Chemical Abstract Services (CAS) number associated with a given parameter being measured.

| Attributes: | C12 |  |
| --- | --- | --- |
| Included in Tables: |  | RESULTS QC CL |

Guidelines & Restrictions:

- PARLABEL cannot be left blank.

- PARLABEL must contain a valid value.

Acceptable PARLABEL Values Sorted by Code:

| Code | Description |
| --- | --- |
| 1002-84-2 | Pentadecanoic acid |
| 107-83-5 | Pentane, 2-methyl- |
| 107-86-8 | 3-Methyl-2-butenal |
| 1074-17-5 | 1-Methyl-2-n-propylbenzene |
| 108-87-2 | Cyclohexane, methyl- |
| 111-01-3 | Squalane |
| 111-06-8 | Hexadecanoic acid, butyl ester |
| 111-76-2 | 2-Butoxy-ethanol |
| 111-77-3 | 2-(2-Methoxyethoxy) ethanol |
| 112-37-8 | Undecanoic acid |
| 112-40-3 | Dodecane |
| 112-85-6 | Docosanoic acid |
| 112-95-8 | Eicosane |
| 1120-21-4 | Undecane |
| 1127-76-0 | 1-Ethylnaphthalene |
| 115-11-7 | 1-Propene, 2-methyl- |
| 11DCPROPN | 1,1-Dichloropropanone |
| 121-43-7 | Boric acid, trimethyl ester |
| 123-95-5 | Octadecanoic acid, butyl ester |
| 124-18-5 | Decane |
| 13427-43-5 | 1-Hexene, 3,3,5-trimethyl- |
| 13BZDIOL | Resorcinol |
| 13CPCB209 | 13C-PCB 209 |
| 13DCPR20H | 1,3-Dichloro-2-propanol |
| 141-79-7 | 4-Methyl-3-penten-2-one |
| 14167-59-0 | Tetratriacontane |
| 142-82-5 | Heptane |
| 143-07-7 | Dodecanoic acid |
| 1430-97-3 | 9H-Fluorene, 2-methyl- |
| 14BZDIOL | 1,4-Benzenediol |
| 14DITH | 1,4-Dithiane |
| 14HYDROQ | 1,4-Hydroquinone |
| 16538-93-5 | Cyclooctane, butyl- |
| 16747-32-3 | 3-Ethyl-2,2-dimethylpentane |
| 1678-92-8 | Cyclohexane, propyl- |
| 1719-03-5 | Pentadecane, 2,6,10,14-tetramethyl- |
| 17301-27-8 | Undecane, 1,10-dimethyl- |
| 17312-81-1 | Undecane, 3,5-dimethyl- |
| 1BR2FLET | 1-Bromo-2-fluoroethane |
| 1CL2MEPE | 1-Chloro-2-methylpropene |
| 1CLOCT | 1-Chlorooctane |
| 1E2MBZ | 1-Ethyl-2-methylbenzene |
| 2027-17-0 | 2-Isopropylnaphthalene |
| 2049-95-8 | Benzene, (1,1-Dimethylpropyl)- |
| 2050-24-0 | Benzene, 1,3-diethyl-5-methyl- |
| 2088-07-5 | 1-Penten-3-ol, 2-methyl- |
| 2131-42-2 | Naphthalene, 1,4,6-trimethyl- |
| 2207-04-7 | Cyclohexane, 1,4-dimethyl-, trans- |
| 2216-33-3 | Octane, 3-methyl- |
| 2234-75-5 | Cyclohexane, 1,2,4-trimethyl- |
| 2245-38-7 | Naphthalene, 1,6,7-trimethyl- |
| 234TFBZME | 2,3,4-Trifluorotoluene |
| 2363-71-5 | Heneicosanoic acid |
| 245T | 2,4,5-T |
| 245TBE | 2,4,5-T, butyl ester |
| 245TBEE | 2,4,5-T, butoxyethanol ester |
| 24642-72-6 | Cydohexane carboxylic acid |
| 24D | 2,4-D |
| 24DB | 2,4-DB |
| 24DBE | 2,4-D, butoxyethanol ester |
| 24DCPHYAA | 2,4-Dichlorophenylacetic acid |
| 24DEE | 2,4-D, 2-ethylhexyl ester |
| 24NO2FBZ | 2,4-Dinitrofluorobenzene |
| 25044-01-3 | 1-Penten-3-one, 2-methyl- |
| 2532-58-3 | Cylcopentane, 1,3-dimethyl-, cis- |
| 26DIM6NITRO | 2,6-Dimethyl-6-nitro-2-hepten-4-one |
| 26DIMENONANE | Nonane, 2,6-dimethyl- |
| 27133-93-3 | 2,3-Dihydro-1-methylindene |
| 2870-04-4 | 2-Ethyl-1,3-dimethylbenzene |
| 2958-76-1 | Napthalene, decahydro-2-methyl- |
| 29949-27-7 | n-Amylcyclohexane |
| 2BR46DCP | 2-Bromo-4,6-dichlorophenol |
| 2BRPROPENE | 2-Bromopropene |
| 2BUTENAL | Crotonaldehyde |
| 2CLANILINE | 2-Chloroaniline |
| 2CLANTH | 2-Chloroanthracene |
| 2HPROPN | 2-Hydroxypropionitrile |
| 2MPA1E | 2-Methylpropanoic acid |
| 2NO2MXYL | 2-Nitro-m-Xylene |
| 2PROPENOL | 2-Propenol |
| 2PRYN1OL | 2-Propyn-1-ol |
| 2PYRR1M | 2-Pyrrolidinone, 1-methyl- |
| 334-48-5 | Decanoic acid |
| 3674-66-6 | Phenanthrene, 2,5-dimethyl- |
| 4057-42-5 | 2,6-Dimethyl-2-octene |
| 4175-53-5 | 2,3-Dihydro-1,3-dimethyl-1H-indene |
| 4292-92-6 | Cyclohexane, pentyl- |
| 4860-03-1 | 1-Chlorohexadecane |
| 4926-78-7 | Cyclohexane, 1-ethyl-4-methyl-, cis- |
| 4N2PHEN | 4-Nitrobiphenyl |
| 4NQO | 4-Nitroquinoline-n-oxide |
| 506-12-7 | Heptadecanoic acid |
| 506-30-9 | Eicosanoic acid |
| 50876-31-8 | Cyclohexane, 1,1,3,5-tetramethyl-, trans- |
| 527-53-7 | 1,2,3,5-Tetramethylbenzene |
| 535-77-3 | 1-Methyl-3-isopropylbenzene |
| 53771-88-3 | Cyclopentane, 1-methyl-3-(1-methylethyl)- |
| 54120-62-6 | Ethyl-1,2,4-trimethylbenzene |
| 544-63-8 | Tetradecanoic acid |
| 544-76-3 | Hexadecane |
| 544-85-4 | Dotriacontane |
| 557-59-5 | Tetracosanoic acid |
| 565-75-3 | 2,3,4-Trimethylpentane |
| 571-61-9 | 1,5-Dimethylnaphthalene |
| 581-40-8 | Naphthalene, 2,3-dimethyl- |
| 582-16-1 | Naphthalene, 2,7-dimethyl- |
| 591-04-6 | Nonane, 4-methyl- |
| 592-41-6 | 1-Hexene |
| 593-45-3 | Octadecane |
| 593-49-7 | Heptacosane |
| 620-14-4 | 1-Methyl-3-ethylbenzene |
| 6236-88-0 | Cyclohexane, 1-ethyl-4-methyl-, trans- |
| 624-29-3 | Cyclohexane, 1,4-dimethyl-, cis- |
| 625-33-2 | 3-Penten-2-one |
| 629-07-0 | Docosane |
| 629-50-5 | Tridecane |
| 629-59-4 | Tetradecane |
| 629-62-9 | Pentadecane |
| 629-78-7 | Heptadecane |
| 629-92-5 | Nonadecane |
| 629-94-7 | Heneicosane |
| 629-99-2 | Pentacosane |
| 630-01-3 | Hexacosane |
| 630-02-4 | Octacosane |
| 630-04-6 | Hentriacontane |
| 630-05-7 | Tritriacontane |
| 630-06-8 | Hexatriacontane |
| 630-07-9 | Pentatriacontane |
| 630-32-8 | Nonacosane |
| 638-53-9 | Tridecanoic acid |
| 638-67-5 | Tricosane |
| 638-68-6 | Tricontane |
| 646-30-0 | Nonadecanoic acid |
| 646-31-1 | Tetracosane |
| 6682-71-9 | 2,3-Dihydro-4,7-dimethyl-1H-indene |
| 767-58-8 | 2,3-Dihydro-1-methyl-1H-indene |
| 7683-64-9 | Squalene |
| 78-78-4 | 2-Methylbutane |
| 79-20-9 | Acetic acid, methyl ester |
| 7METDECANE | Tridecane, 7-methyl- |
| 824-22-6 | 2,3-Dihydro-4-methyl-1H-indene |
| 832-71-3 | Phenanthrene, 3-methyl- |
| 874-35-1 | 2,3-Dihydro-5-methyl-1H-indene |
| 874-41-9 | 1,3-Dimethyl-4-ethylbenzene |
| 89-82-7 | Pulegone |
| 933-98-2 | 1-Ethyl-2,3-dimethylbenzene |
| 9PHENAN | 9-Phenylanthracene |
| A2DNT46 | 2-Amino-4,6-dinitrotoluene |
| A3ECBZ9 | 3-Amino-9-ethylcarbazole |
| A4DNT26 | 4-Amino-2,6-dinitrotoluene |
| AAATFBZME | a,a,a-Trifluorotoluene |
| AC-228 | Actinium-228 |
| AC2T | 1-Acetyl-2-thiourea |
| ACAMFL2 | 2-Acetylaminofluorene |
| ACCN | Acetonitrile |
| ACE | Acetone |
| ACID | Acidity, Total |
| ACNP | Acenaphthene |
| ACNPD10 | Acenaphthene-d10 |
| ACNPY | Acenaphthylene |
| ACPHN | Acetophenone |
| ACRAMD | Acrylonitrile |
| ACRL | Acrolein |
| ACRN | Acrylamide |
| AFN | Acifluorfen |
| AG | Silver |
| AGP | Acid Generating Potential |
| AISULFIDE | Acid-Insoluble Sulfide |
| AL | Aluminum |
| ALACL | Alachlor |
| ALDICARB | Aldicarb |
| ALDRIN | Aldrin |
| ALDSULFOX | Aldicarb sulfoxide |
| ALK | Alkalinity, Total |
| ALKB | Bicarbonate Alkalinity as CACO3 |
| ALKC | Carbonate Alkalinity as CACO3 |
| ALKH | Hydroxide Alkalinity as CACO3 |
| ALLYLALCOHOL | Allyl alcohol |
| ALPHA | Alpha, Gross |
| AMAQ2 | 2-Aminoanthraquinone |
| AMAZOBENZ | Aminoazobenzene |
| AMINOBPH4 | 4-Aminobiphenyl |
| AMINONAPH1 | 1-Naphthylamine |
| AMINONAPH2 | 2-Naphthylamine |
| AMOSITE | Amosite |
| ANC | Acid Neutralizing Capacity |
| ANDROSTANE5A | 5a-Androstane |
| ANILINE | Aniline |
| ANILINED5 | Aniline-d5 |
| ANLNAM4 | 1,4-Phenylenediamine |
| ANSD2 | o-Anisidine |
| ANTH | Anthracene |
| ANTHD10 | Anthracene-d10 |
| ANZIN | Anilazine |
| APIGRAVITY | API Gravity |
| APINENE | alpha-Pinene |
| ARAMITE | Aramite |
| AS | Arsenic |
| ASBESTOS | Asbestos |
| ASH | Ash Content |
| ASULAM | Asulam |
| ASULFIDE | Acid-Soluble Sulfide |
| ATRAZINE | Atrazine |
| AU | Gold |
| AVS | Acid-Volatile Sulfide |
| AZIPM | Azinphos methyl |
| AZOBENZENE | Azobenzene |
| B | Boron |
| B2CES | bis-(2-Chloroethyl) sulfide |
| BA | Barium |
| BACE | Bromoacetone |
| BARBAN | Barban |
| BATOT | Total Barium |
| BBP | Benzyl butyl phthalate |
| BCHPD | Bicyclo[2,2,1]hepta-2,5-diene |
| BDCME | Bromodichloromethane |
| BDENSITY | Bulk Density |
| BE | Beryllium |
| BECEM | bis-(2-chloroethoxy)methane |
| BETA | Beta, Gross |
| BHCALPHA | alpha-BHC |
| BHCBETA | beta-BHC |
| BHCDELTA | delta-BHC |
| BHCGAMMA | gamma-BHC (Lindane) |
| BI | Bismuth |
| BI-212 | Bismuth-212 |
| BI-214 | Bismuth-214 |
| BICACO3 | Bicarbonate as CaCO3 |
| BIDRIN | Dicrotophos |
| BIPHENYL | Biphenyl |
| BIS2BEE | Bis(2-butoxyethyl) ether |
| BIS2CEE | bis-(2-Chloroethyl)ether |
| BIS2CIE | bis(2-Chloroisopropyl)ether |
| BIS2EHP | bis-(2-ethylhexyl)phthalate |
| BOD5 | Biologic Oxygen Demand, Five-Day |
| BPLACTONE | b-Propiolactone |
| BPPE4 | 4-Bromophenyl phenyl ether |
| BPROPACT | beta-Propiolactone |
| BR | Bromide |
| BR4DCP26 | 4-Bromo-2,6-dichlorophenol |
| BR4FBZ | 4-Bromofluorobenzene |
| BRBZ | Bromobenzene |
| BRCLBZ4 | 4-Bromochlorobenzene |
| BRCLME | Bromochloromethane |
| BREA | Bromoethane |
| BRME | Bromomethane |
| BROMCIL | Bromacil |
| BROXL | Bromoxynil |
| BTA | Butanoic acid |
| BTALT | t-Butyl alcohol |
| BTBZN | n-Butylbenzene |
| BTBZS | sec-Butylbenzene |
| BTBZT | tert-Butylbenzene |
| BTCL | 1-Chlorobutane |
| BTEX | Benzene, Toluene, Ethylbenzene, and Xylenes |
| BTOH | n-Butanol |
| BTOXETETL | 2-(2-Butoxyethoxy) ethanol |
| BTSN | Butyltin |
| BTZ | Bentazon |
| BUNKERC | Fuel Oil No. 6 (BUNKER C) |
| BUTACHLOR | Butachlor |
| BZ | Benzene |
| BZAA | Benzo(a)anthracene |
| BZACID | Benzoic acid |
| BZALD | Benzaldehyde |
| BZAP | Benzo(a)pyrene |
| BZBF | Benzo(b)fluoranthene |
| BZBFBZKF | Benzo(b)fluoranthene and Benzo(k)fluoranthene |
| BZD | Benzidine |
| BZD6 | Benzene-d6 |
| BZEP | Benzo(e)pyrene |
| BZGHIP | Benzo(g,h,i)perylene |
| BZJF | Benzo(j)fluoranthene |
| BZKF | Benzo(k)fluoranthene |
| BZLAL | Benzyl alcohol |
| BZLCL | Chlorotoluene |
| BZME | Toluene |
| BZMED8 | Toluene-d8 |
| BZS | Thiophenol (Benzenethiol) |
| BZTZ | Benzothiazole |
| C10C10N | C10 as n-Decane |
| C10C12ALIPH | C10-C12 Aliphatics |
| C10C12AROM | C10-C12 Aromatics |
| C10C24ALIPH | C10-C24 Aliphatics |
| C10C24AROM | C10-C24 Aromatics |
| C11C11N | C11 as n-Undecane |
| C12C12N | C12 as n-Dodecane |
| C12C16ALIPH | C12-C16 Aliphatics |
| C12C16AROM | C12-C16 Aromatics |
| C13C13N | C13 as n-Tridecane |
| C14C14N | C14 as n-Tetradecane |
| C15C15N | C15 as n-Pentadecane |
| C16C16N | C16 as n-Hexadecane |
| C16C21ALIPH | C16-C21 Aliphatics |
| C16C21AROM | C16-C21 Aromatics |
| C17C17N | C17 as n-Heptadecane |
| C18C18N | C18 as n-Octadecane |
| C21C34ALIPH | C21-C34 Aliphatics |
| C21C34AROM | C21-C34 Aromatics |
| C25C36ALIPH | C25-C36 Aliphatics |
| C25C36AROM | C25-C36 Aromatics |
| C25N | Pentacosane |
| C2H4 | Ethene |
| C2H6 | Ethane |
| C30N | n-Triacontane |
| C3H8 | Propane |
| C4BZ1245 | 1,2,4,5-Tetrachlorobenzene |
| C4M2PH | 4-Chloro-2-methylphenol |
| C4M3PH | 4-Chloro-3-methylphenol |
| C5C16TPH | C5-C16 Total Petroleum Hydrocarbons |
| C5C6ALIPH | C5-C6 Aliphatics |
| C6C10ALIPH | C6-C10 Aliphatics |
| C6C10AROM | C6-C10 Aromatics |
| C6C8ALIPH | C6-C8 Aliphatics |
| C6HEXANE | C6 as n-Hexane |
| C7HEPTANE | C7 as n-Heptane |
| C8C10ALIPH | C8-C10 Aliphatics |
| C8C10AROM | C8-C10 Aromatics |
| C8C8N | C8 as n-Octane |
| C9C9N | C9 as n-Nonane |
| CA | Calcium |
| CAA | Chloroacetic acid |
| CACO3 | Carbonate as CaCO3 |
| CACO3EQ | Calcium Carbonate Equivalent |
| CAFFEINE | Caffeine |
| CAMPHENE | Camphene |
| CAPT | Captafol |
| CAPTAN | Captan |
| CARBAZOLE | Carbazole |
| CARBOPHENOTH | Carbophenothion |
| CATION-EX | Cation Exchange Capacity |
| CBPH2 | 2-Chlorobiphenyl |
| CD | Cadmium |
| CDS | Carbon disulfide |
| CE | Cerium |
| CELLFIBER | Cellulose fiber |
| CEVETH | 2-Chloroethyl vinyl ether |
| CH4 | Methane |
| CHLORALHY | Chloral hydrate |
| CHLORDANE | Chlordane |
| CHLORDANEA | alpha-Chlordane |
| CHLORDANEG | gamma-Chlordane |
| CHLORINE | Chlorine, Total residual |
| CHLORMEPHOS | Chlormephos |
| CHLOROPHYLLA | Chlorophyll a |
| CHLOROPRENE | 2-Chloro-1,3-butadiene (Chloroprene) |
| CHRYSENE | Chrysene |
| CHRYSENEC1 | C1-Chrysenes |
| CHRYSENEC2 | C2-Chrysenes |
| CHRYSENEC3 | C3-Chrysenes |
| CHRYSENEC4 | C4-Chrysenes |
| CHRYSENED12 | Chrysene-d12 |
| CHRYSO | Chrysotile |
| CL | Chloride |
| CL10BZ2 | Decachlorobiphenyl |
| CL2ETOH | 2-Chloroethanol |
| CL3NATE | Trichloronate |
| CL4PED12 | 4-Chloro-1,2-phenylenediamine |
| CL4PED13 | 4-Chloro-1,3-phenylenediamine |
| CL5MANIL2 | 5-Chloro-2-methylaniline |
| CLACRN2 | 2-Chloroacrylonitrile |
| CLACTH | Chloroacetaldehyde |
| CLAN | Chloroacetonitrile |
| CLANIL4 | 4-Chloroaniline |
| CLBEN | Chloramben |
| CLBZ | Chlorobenzene |
| CLBZALDO | o-Chlorobenzaldehyde |
| CLBZD5 | Chlorobenzene-d5 |
| CLBZLATE | Chlorobenzilate |
| CLBZME2 | 2-Chlorotoluene |
| CLBZME4 | 4-Chlorotoluene |
| CLCYHXAL2 | 2-Chlorocyclohexanol |
| CLEA | Chloroethane |
| CLFBZ2 | 1-Chloro-2-fluorobenzene |
| CLFBZ4 | 1-Chloro-4-fluorobenzene |
| CLHX1 | 1-Chlorohexane |
| CLHXDC1 | 1-Chlorohexadecane |
| CLHYD | Chlorinated Hydrocarbon |
| CLM3CPYRDN | 3-(Chloromethyl)pyridine hydrochloride |
| CLME | Chloromethane |
| CLMME | Chloromethyl methyl ether |
| CLNO2BZ3 | 1-Chloro-3-nitrobenzene |
| CLNPH1 | 1-Chloronaphthalene |
| CLODC1 | 1-Chlorooctadecane |
| CLOUDPT | Cloud Point |
| CLPE3 | Allyl chloride |
| CLPH2 | 2-Chlorophenol |
| CLPH2D4 | 2-Chlorophenol-d4 |
| CLPROP | Chloropropylate |
| CLPYRIFOS | Chlorpyrifos |
| CN | Cyanide |
| CNA | Amenable Cyanide |
| CNPH2 | 2-Chloronaphthalene |
| CO | Cobalt |
| CO-60 | Cobalt-60 |
| CO2 | Carbon dioxide |
| CO3 | Carbonate as CO3 |
| COD | Chemical Oxygen Demand |
| COLIFORM | Coliform, Total |
| COLOR | Color |
| COLORAPPRNT | Color, Apparent |
| COLORTRUE | Color, True |
| COND | Conductivity |
| CORROS | Corrosivity Toward Steel |
| CORRPH | Corrosivity as pH |
| COUMAPHOS | Coumaphos |
| COUMARIN | Coumarin Dyes |
| CPENTANEME | Methyl cyclopentane |
| CPMS | p-Chloromethyl sulfide |
| CPMSO | p-Chloromethyl sulfoxide |
| CPMSO2 | p-Chloromethyl sulfone |
| CPN3 | 3-Chloropropionitrile |
| CPPE4 | 4-Chlorophenyl phenyl ether |
| CR | Chromium |
| CR3 | Chromium III |
| CR6 | Chromium, Hexavalent |
| CRBFN | Carbofuran |
| CRESP | p-Cresidine |
| CROCID | Crocidolite |
| CROTOX | Crotoxyphos |
| CS | Cesium |
| CS-134 | Cesium-134 |
| CS-137 | Cesium-137 |
| CTCL | Carbon tetrachloride |
| CU | Copper |
| CVP | Chlorfenvinphos |
| CYC5N | Cyclopentane |
| CYHEKET | Cyclohexanone |
| CYHEX2DNP46 | 2-Cyclohexyl-4,6-dinitrophenol |
| CYHEXANE | Cyclohexane |
| CYHEXPROP | Cyclohexanepropanol |
| CYMP | 4-Isopropyltoluene |
| DACTH | DCPA (Dacthal) |
| DALAPON | Dalapon |
| DB7HCGCBZ | 7H-Dibenzo(c,g)carbazole |
| DBA | Dibromoethane |
| DBAHA | Dibenzo(a,h)anthracene |
| DBAHACR | Dibenz(a,h)acridine |
| DBAHAD14 | Dibenzo(a,h)anthracene-d14 |
| DBAJACR | Dibenz(a,j)acridine |
| DBCME | Dibromochloromethane |
| DBCP | 1,2-Dibromo-3-chloropropane |
| DBF | Dibenzofuran |
| DBFM | Dibromofluoromethane |
| DBMA | Dibromomethane |
| DBT | Dibenzothiophene |
| DBTC1 | C1-Dibenzothiophenes |
| DBTC2 | C2-Dibenzothiophenes |
| DBTC3 | C3-Dibenzothiophenes |
| DBTSN | Dibutyltin |
| DBUTYLC | Dibutylchlorendate |
| DBZAEP | Dibenzo(a,e)pyrene |
| DBZAHP | Dibenzo(a,h)pyrene |
| DBZAIP | Dibenzo(a,i)pyrene |
| DBZD33 | 3,3'-Dichlorobenzidine |
| DCA11 | 1,1-Dichloroethane |
| DCA12 | 1,2-Dichloroethane |
| DCA12D4 | 1,2-Dichloroethane-d4 |
| DCBE14C | cis-1,4-Dichloro-2-butene |
| DCBE14T | trans-1,4-Dichloro-2-butene |
| DCBETOT | 1,4-Dichloro-2-butene, Total |
| DCBPH23 | 2,3-Dichlorobiphenyl |
| DCBTA14 | 1,4-Dichlorobutane |
| DCBZ12 | 1,2-Dichlorobenzene |
| DCBZ12D4 | 1,2-Dichlorobenzene-d4 |
| DCBZ13 | 1,3-Dichlorobenzene |
| DCBZ14 | 1,4-Dichlorobenzene |
| DCBZ14D4 | 1,4 Dichlorobenzene-d4 |
| DCBZA35 | 3,5-Dichlorobenzoic acid |
| DCBZALD24 | 2,4-Dichlorobenzaldehyde |
| DCE11 | 1,1-Dichloroethene |
| DCE12C | cis-1,2-Dichloroethene |
| DCE12T | trans-1,2-Dichloroethene |
| DCE12TOT | 1,2-Dichloroethene, Total |
| DCLN | Dichlone |
| DCMA | Dichloromethane |
| DCP11 | 1,1-Dichloropropene |
| DCP13 | 1,3-Dichloropropene (Total) |
| DCP13C | cis-1,3-Dichloropropene |
| DCP13T | trans-1,3-Dichloropropene |
| DCP24 | 2,4-Dichlorophenol |
| DCP26 | 2,6-Dichlorophenol |
| DCPA12 | 1,2-Dichloropropane |
| DCPA13 | 1,3-Dichloropropane |
| DCPA22 | 2,2-Dichloropropane |
| DCPANE | 1,1-Dichloropropane |
| DCPD | Dicyclopentadiene |
| DCPROP | Dichlorprop |
| DD1234678C13 | 1,2,3,4,6,7,8-Heptachlorodibenzo-p-dioxin-C13 |
| DD123478C13 | 1,2,3,4,7,8-Hexachlorodibenzo-p-dioxin-C13 |
| DD123678C13 | 1,2,3,6,7,8-Hexachlorodibenzo-p-dioxin-C13 |
| DD12378C13 | 1,2,3,7,8-Pentachlorodibenzo-p-dioxin-C13 |
| DDD24 | 2,4'-DDD |
| DDD44 | 4,4'-DDD |
| DDE24 | 2,4'-DDE |
| DDE44 | 4,4'-DDE |
| DDE44DIEL | 4,4'-DDE/Dieldrin |
| DDT24 | 2,4'-DDT |
| DDT44 | 4,4'-DDT |
| DEMETON | Demeton, -O and -S |
| DEMETONO | Demeton-O |
| DEMETONS | Demeton-S |
| DENSITY | Density |
| DEPH | Diethyl phthalate |
| DEPHD4 | Diethyl phthalate-D4 |
| DES | Diethylstilbestrol |
| DESO4 | Diethyl sulfate |
| DF1234678C13 | 1,2,3,4,6,7,8-Heptachlorodibenzofuran-C13 |
| DF1234789C13 | 1,2,3,4,7,8,9-Heptachlorodibenzofuran-C13 |
| DF123478C13 | 1,2,3,4,7,8-Hexachlorodibenzofuran-C13 |
| DF123678C13 | 1,2,3,6,7,8-Hexachlorodibenzofuran-C13 |
| DF123789C13 | 1,2,3,7,8,9-Hexachlorodibenzofuran-C13 |
| DF12378C13 | 1,2,3,7,8-Pentachlorodibenzofuran-C13 |
| DF234678C13 | 2,3,4,6,7,8-Hexachlorodibenzofuran-C13 |
| DF23478C13 | 2,3,4,7,8-Pentachlorodibenzofuran-C13 |
| DFBZ14 | 1,4-Difluorobenzene |
| DHNAPH | Naphthalene, decahydro- |
| DHNAPHT | Naphthalene, decahydro-, trans- |
| DIACOH | 2-Pentanone, 4-hydroxy-4-methyl- |
| DIALLATE | Diallate (cis- or trans-) |
| DIAZ | Diazinon |
| DICAMBA | Dicamba |
| DICHLORVOS | Dichlorovos |
| DICOFOL | Dicofol |
| DIELDRIN | Dieldrin |
| DIESEL2 | Diesel Fuel #2 |
| DIHYDROSAF | Dihydrosafrole |
| DIMETHAT | Dimethoate |
| DIMP | Diisopropylmethylphosphonate |
| DINOSEB | Dinoseb |
| DIOP | Diisooctyl phthalate |
| DIOXANE14 | 1,4-Dioxane |
| DIOXATHION | Dioxathion |
| DISBLUE14 | Disperse Blue 14 |
| DISBLUE3 | Disperse Blue 3 |
| DISBRN1 | Disperse Brown 1 |
| DISORNG3 | Disperse Orange 3 |
| DISORNG30 | Disperse Orange 30 |
| DISRED1 | Disperse Red 1 |
| DISRED13 | Disperse Red 13 |
| DISRED5 | Disperse Red 5 |
| DISRED60 | Disperse Red 60 |
| DISUL | Disulfoton |
| DISYEL5 | Disperse Yellow 5 |
| DITH | Dithiane |
| DM12NPH | 1,2-Dimethylnaphthalene |
| DM13NBZ2 | 1,3-Dimethyl-2-nitrobenzene |
| DM16NPH | 1,6-Dimethylnaphthalene |
| DM26NPH | 2,6-Dimethylnaphthalene |
| DMBZA712 | 7,12-Dimethylbenz(a)anthracene |
| DMBZD33 | 3,3'-Dimethylbenzidine |
| DMC10N | Dimethyl decane |
| DMCPS | Decamethylcyclopentasiloxane |
| DMDS | Dimethyl disulfide |
| DMMP | Dimethylmethylphosphonate |
| DMOBZD33 | 3,3'-Dimethoxybenzidine |
| DMP24 | 2,4-Dimethylphenol |
| DMPH | Dimethyl phthalate |
| DN46M | 2-Methyl-4,6-dinitrophenol |
| DNA24 | 2,4-Dinitroaniline |
| DNB13 | 1,3-Dinitrobenzene |
| DNBP | Di-n-butyl phthalate |
| DNBZ12 | 1,2-Dinitrobenzene |
| DNBZ14 | 1,4-Dinitrobenzene |
| DNOCP | Dinocap |
| DNOP | Di-n-octyl phthalate |
| DNOPD4 | Di-n-octyl phthalate-d4 |
| DNP24 | 2,4-Dinitrophenol |
| DNT24 | 2,4-Dinitrotoluene |
| DNT26 | 2,6-Dinitrotoluene |
| DNT34 | 3,4-Dinitrotoluene |
| DO | Oxygen, Dissolved |
| DOA | bis(2-ethylhexyl)adipate |
| DOC | Dissolved Organic Carbon |
| DPA | Diphenylamine |
| DPAMIDE | Diphenamide |
| DPD10 | Diphenyl-d10 |
| DPHANTH910 | 9,10-Diphenylanthracene |
| DPHY12 | 1,2-Diphenylhydrazine |
| DRO | Diesel Range Organics |
| DROALIPHATIC | Diesel Range Organics, Aliphatic |
| DROAROMATIC | Diesel Range Organics, Aromatic |
| DROC10C25 | Diesel Range Organics (C10-C25) |
| DROC10C28 | Diesel Range Organics (C10-C28) |
| DS | Dissolved Sulfide |
| DU | Data Unavailable |
| DUST | Dust |
| DY | Dysprosium |
| EAH | Erythritol anhydride |
| EBZ | Ethylbenzene |
| EBZD10 | Ethylbenzene-D10 |
| EBZME4 | 4-Ethyltoluene |
| ECARB | Ethyl carbamate |
| ECOLI | Escherichia coli |
| EDB | 1,2-Dibromoethane |
| EE | Diethyl ether |
| EMETHACRY | Ethyl methacrylate |
| EMSULFN | Ethyl methanesulfonate |
| ENDOSULFANA | Endosulfan I |
| ENDOSULFANB | Endosulfan II |
| ENDOSULFANS | Endosulfan sulfate |
| ENDRIN | Endrin |
| ENDRINALD | Endrin aldehyde |
| ENDRINKET | Endrin ketone |
| ENTCOCCUS | Enterococcus |
| EPICLHDRN | Epichlorhydrin |
| EPN | EPN |
| ERYTHRENE | 1,3-Butadiene |
| ESP | Exchangeable Sodium Percentage |
| ET2GLY | Diethylene glycol |
| ET2HEOH | 1-Hexanol, 2-ethyl- |
| ET3GLY | Triethylene glycol |
| ETACET | Ethyl acetate |
| ETBE | tert-Butyl ethyl ether |
| ETEGLY | Ethylene glycol |
| ETETH | 2-(2-Ethoxyethoxy) ethanol |
| ETHANOL | Ethanol |
| ETHION | Ethion |
| ETHOPROP | Ethoprop |
| ETOX | Ethylene oxide |
| ETRID | Etridiazole |
| EU | Europium |
| EU-152 | Europium-152 |
| EU-154 | Europium-154 |
| EU-155 | Europium-155 |
| F | Fluoride |
| FAA | Fluoroacetic acid |
| FAMPHUR | Famphur |
| FARN | 3,7,11-Trimethyl-2,6,10-dodecatrien-1-ol |
| FBZ | Fluorobenzene |
| FC11 | Trichlorofluoromethane |
| FC113 | 1,1,2-Trichloro-1,2,2-trifluoroethane |
| FC114 | 1,2-Dichloro-1,1,2,2-tetrafluoroethane |
| FC12 | Dichlorodifluoromethane |
| FC21 | Dichlorofluoromethane |
| FE | Iron |
| FE2 | Ferrous Iron |
| FE3 | Ferric Iron |
| FECCOLIFORM | Fecal Coliform, 0.7 Micron Filter |
| FECSTREP | Fecal Streptococcus |
| FENSTHION | Fensulfothion |
| FENTHION | Fenthion |
| FIBGLASS | Fibrous Glass |
| FIOBPH | Decafluorobiphenyl |
| FIOTPHPINE | Decafluorotriphenylphosphine |
| FL | Fluorene |
| FLA | Fluoranthene |
| FLAC1PYR | C1-Fluoranthenes/Pyrenes |
| FLAD10 | Fluoranthene-d10 |
| FLASHPT | Flash Point |
| FLBRIGHT236 | Fluorescent Brightener 236 |
| FLBRIGHT61 | Fluorescent Brightener 61 |
| FLC1 | C1-Fluorenes |
| FLC2 | C2-Fluorenes |
| FLC3 | C3-Fluorenes |
| FLD10 | Fluorene-d10 |
| FLUCHLOR | Fluchloralin |
| FREELIQUIDS | Free Liquids |
| GAMMA-GELI | Gamma Spectral Analysis, Ge(Li) |
| GASOLINE | Gasoline |
| GRO | Gasoline Range Organics |
| GROALIPHATIC | Gasoline Range Organics, Aliphatic |
| GROAROMATIC | Gasoline Range Organics, Aromatic |
| GROC6C10 | Gasoline Range Organics (C6-C10) |
| H-3 | Tritium (Hydrogen 3) |
| H2M2P | Hexafluoro-2-methyl-2-propanol |
| H2S | Sulfide (as H2S) |
| HALIDES | Halides |
| HALOWAX1000 | Halowax 1000 |
| HALOWAX1001 | Halowax 1001 |
| HALOWAX1013 | Halowax 1013 |
| HALOWAX1014 | Halowax 1014 |
| HALOWAX1051 | Halowax 1051 |
| HALOWAX1099 | Halowax 1099 |
| HARD | Hardness (as CaCO3) |
| HARDC | Hardness (as CO3), Carbonate |
| HARDNC | Hardness (as CaCO3), Noncarbonate |
| HCBPH2233446 | 2,2,3,3,4,4,6-Heptachlorobiphenyl |
| HCBU | Hexachlorobutadiene |
| HCCP | Hexachlorocyclopentadiene |
| HCLBZ | Hexachlorobenzene |
| HCLEA | Hexachloroethane |
| HCN | Cyanide (as HCN) |
| HCO3 | Bicarbonate as HCO3 |
| HCPR | Hexachloropropene |
| HEM | n-Hexane Extractable Material |
| HEPT-EPOX | Heptachlor epoxide |
| HEPT-EPOXA | Heptachlor epoxide A |
| HEPTACHLOR | Heptachlor |
| HEXANE | Hexane |
| HF | Hafnium |
| HFACID | Hydrofluoric acid |
| HFP2 | Hexafluoro-2-propanol |
| HG | Mercury |
| HHV | High Heat Value |
| HMPA | Hexamethylphosphoramide |
| HMX | Octahydro-1,3,5,7-tetranitro-1,3,5,7-tetrazocine |
| HOIL | Heavy Oil |
| HPC | Heterotrophic Plate Count |
| HPCDD | Total Heptachlorodibenzo-p-dioxins (HpCDD) |
| HPCDD1234678 | 1,2,3,4,6,7,8-Heptachlorodibenzo-p-dioxin |
| HPCDF | Total Heptachlorodibenzofurans (HpCDF) |
| HPCDF1234678 | 1,2,3,4,6,7,8-Heptachlorodibenzofuran |
| HPCDF1234789 | 1,2,3,4,7,8,9-Heptachlorodibenzofuran |
| HXALD | Hexanal |
| HXBRBZ | Hexabromobenzene |
| HXCBPH224456 | 2,2,4,4,5,6-Hexachlorobiphenyl |
| HXCDD | Total Hexachlorodibenzo-p-dioxins (HxCDD) |
| HXCDD123478 | 1,2,3,4,7,8-Hexachlorodibenzo-p-dioxin |
| HXCDD123678 | 1,2,3,6,7,8-Hexachlorodibenzo-p-dioxin |
| HXCDD123789 | 1,2,3,7,8,9-Hexachlorodibenzo-p-dioxin |
| HXCDF | Total Hexachlorodibenzofurans (HxCDF) |
| HXCDF123478 | 1,2,3,4,7,8-Hexachlorodibenzofuran |
| HXCDF123678 | 1,2,3,6,7,8-Hexachlorodibenzofuran |
| HXCDF123789 | 1,2,3,7,8,9-Hexachlorodibenzofuran |
| HXCDF234678 | 2,3,4,6,7,8-Hexachlorodibenzofuran |
| HXCLCYHX | Hexachlorocyclohexane |
| HXCP | Hexachlorophene |
| HXO2 | 2-Hexanone |
| HYDDICAM | 5-Hydroxydicamba |
| HYDROQUIN | Hydroquinone |
| I | Iodide (as I) |
| I-129 | Iodine-129 |
| IGNITB | Ignitability |
| IME | Methyl iodide |
| IMPA | Isopropylmethyl phosphonic acid |
| INDENE | Indene |
| INP123 | Indeno(1,2,3-cd)pyrene |
| IONBAL | Ion Balance |
| IPBZ | Isopropylbenzene |
| IR | Iridium |
| ISC10H12 | C10H12 Isomer |
| ISC11H120 | C11H120 Isomer |
| ISC8H803 | C8H803 Isomer |
| ISOBTOH | Isobutanol |
| ISOBUTANE | Isobutane |
| ISODRIN | Isodrin |
| ISOP | Isophorone |
| ISOPRE | Isopropyl ether |
| ISOPROH | Isopropanol |
| ISOSAFR | Isosafrole |
| JETA | Jet Fuel as Jet A |
| JETFUEL | Jet Fuel |
| JP4 | Jet Fuel #4 (JP4) |
| K | Potassium |
| K-40 | Potassium-40 |
| KEP | Kepone |
| KEROSENE | Kerosene |
| KEXT | Extractable Potassium |
| KN | Nitrogen, Kjeldahl, Total |
| LA | Lanthanum |
| LAI | Langelier Index |
| LAI140 | Langlier Index at 140 degrees F |
| LAI40 | Langlier Index at 40 degrees F |
| LEPTO | Leptophos |
| LI | Lithium |
| LU | Lutetium |
| MACRYLATE | Methyl acrylate |
| MALA | Malathion |
| MALANH | Maleic anhydride |
| MALNTRL | Malononitrile |
| MARACHIDATE | Methyl arachidate |
| MB2CAN44 | 4,4'-Methylenebis(2-chloraniline) |
| MBAS | Methylene Blue Active Substances |
| MCPA | MCPA |
| MCPP | MCPP |
| ME5CHRYSENE | 6-Methylchrysene |
| MECHLAN3 | 3-Methylcholanthrene |
| MEDIUM | Medium |
| MEDS | Methyl disulfide |
| MEK | 2-Butanone |
| MEOH | Methanol |
| MEPH1314 | m,p-Cresol |
| MEPH2 | 2-Methylphenol (o-Cresol) |
| MEPH3 | 3-Methylphenol |
| MEPH34CO | 3-Methylphenol/4-Methylphenol Coelution |
| MEPH4 | 4-Methylphenol (p-Cresol) |
| MEPHS | Cresols (Methyl Phenols) |
| MERPHOS | Merphos |
| METABOLITES | DCPA acid metabolites (a) |
| METHACRN | Methacrylonitrile |
| METHIOCARB | Methiocarb |
| METHOMYL | Methomyl |
| METOCHLOR | Metolachlor |
| METRIBUZ | Metribuzin |
| MEVINPHOS | Mevinphos |
| MEXACARBATE | Mexacarbate |
| MG | Magnesium |
| MGCAC03 | Magnesium as CaCO3 |
| MIBK | 4-Methyl-2-pentanone |
| MICHLER | 4,4'-Methylenebis(N,N-dimethylaniline) |
| MINSPRT | Mineral Spirits |
| MIREX | Mirex |
| MMETHACRY | Methylmethacrylate |
| MMSULFN | Methyl methanesulfonate |
| MN | Manganese |
| MO | Molybdenum |
| MOIL | Motor Oils |
| MOIST | Percent Moisture |
| MONOCROPHOS | Monocrotophos |
| MP22BZ | 1-Methyl-2-(2-propenyl)-benzene |
| MPA | Methyl phosphonic dichloride |
| MPEA11 | a,a-Dimethylphenethylamine |
| MPHAN1 | 1-Methylphenanthrene |
| MPK | 2-Pentanone |
| MSNL | Mestranol |
| MTD | 2,4-Diaminotoluene |
| MTLNCL | Methylene chloride |
| MTNPH1 | 1-Methylnaphthalene |
| MTNPH2 | 2-Methylnaphthalene |
| MTNPH2D10 | 2-Methylnaphthalene-d10 |
| MTPYRLN | Methapyrilene |
| MTXYCL | Methoxychlor |
| N | Nitrogen |
| N2ANSD5 | 5-Nitro-o-anisidine |
| NA | Sodium |
| NACLO3 | Sodium chlorate |
| NACN5 | 5-Nitroacenaphthene |
| NAEXT | Extractable Sodium |
| NALED | Naled |
| NAPH | Naphthalene |
| NAPHD8 | Naphthalene-d8 |
| NAPHF | 1-Fluoronaphthalene |
| NAPHQ14 | 1,4-Naphthoquinone |
| NB | Niobium |
| NBZME2 | 2-Nitrotoluene |
| NBZME24 | 2-Nitrotoluene and 4-Nitrotoluene (Total) |
| NBZME3 | 3-Nitrotoluene |
| NBZME4 | 4-Nitrotoluene |
| NCT | trans-Nonachlor |
| NH3 | Ammonia |
| NH3N | Nitrogen, Ammonia (as N) |
| NH4N | Ammonium as Nitrogen |
| NI | Nickel |
| NICOTINE | Nicotine |
| NITROFEN | Nitrofen |
| NMOC | Non-Methane Organic Compounds |
| NNSBU | n-Nitroso-di-n-butylamine |
| NNSE | n-Nitrosodiethylamine |
| NNSM | n-Nitrosodimethylamine |
| NNSME | n-Nitrosomethylethylamine |
| NNSMRPH | n-Nitrosomorpholine |
| NNSPH | n-Nitrosodiphenylamine |
| NNSPPRD | n-Nitrosopiperidine |
| NNSPR | n-Nitrosodi-n-propylamine |
| NNSPYRL | n-Nitrosopyrrolidine |
| NO2ANIL2 | 2-Nitroaniline |
| NO2ANIL3 | 3-Nitroaniline |
| NO2ANIL4 | 4-Nitroaniline |
| NO2BZ | Nitrobenzene |
| NO2BZD5 | Nitrobenzene-d5 |
| NO2N | Nitrogen, Nitrite |
| NO3 | Nitrate |
| NO3N | Nitrogen, Nitrate (as N) |
| NO3NO2N | Nitrogen, Nitrate-Nitrite |
| NONANE | Nonane |
| NPHC2 | C2-Naphthalenes |
| NPHC3 | C3-Naphthalenes |
| NPHC4 | C4-Naphthalenes |
| NPHD | Naphtha distillate |
| NPR2 | 2-Nitropropane |
| NTG | Nitroglycerin |
| NTPH2 | 2-Nitrophenol |
| NTPH4 | 4-Nitrophenol |
| OC | Organic Carbon |
| OCBPH2233456 | 2,2,3,3,4,5,6,6-Octachlorobiphenyl |
| OCDD | Octachlorodibenzo-p-dioxin |
| OCDDC13 | Octachlorodibenzo-p-dioxin-C13 |
| OCDF | Octachlorodibenzofuran |
| OCDFC13 | Octachlorodibenzofuran-C13 |
| OCDNA | Octadecanoic acid |
| ODA | 4,4'-Oxydianiline |
| ODB | Hydrocarbon Degrading Bacteria |
| ODOR | Odor |
| OFBZME | Octafluorotoluene |
| OH | Hydroxide |
| OILGREASE | Oil and Grease |
| OILM | Oil, Misc. |
| OMCYTSX | Octamethylcyclotetrasiloxane |
| OMPA | Octamethyl pyprophosphoramide |
| OS | Osmium |
| OTHERS | Unidentified Light- and/or Medium-Weight Fuels |
| OTPH-D | State of Oregon Diesel Range Organics |
| OTPH-G | State of Oregon Gasoline Range Organics |
| OXAMYL | Oxamyl |
| OXAT | Oxathiane |
| OXATH14 | 1,4-Oxathiane |
| OXYGEN | Oxygen |
| P | Phosphorus, Total (as P) |
| PA-231 | Protactinium-231 |
| PA-234 | Protactinium-234 |
| PACN | Propionitrile |
| PALMA | Hexadecanoic acid |
| PARAE | Parathion ethyl |
| PARALD | Paraldehyde |
| PARAM | Parathion methyl |
| PB | Lead |
| PB-210 | Lead-210 |
| PB-212 | Lead-212 |
| PB-214 | Lead-214 |
| PBO | Organic Lead |
| PBTE | Tetraethyl lead |
| PBTED20 | Tetraethyl lead-d20 |
| PBZN | n-Propylbenzene |
| PBZQUINONE | p-Benzoquinone |
| PCA | 1,1,2,2-Tetrachloroethane |
| PCATE | Perchlorate |
| PCB0005 | 1,1'-Biphenyl, pentachloro- |
| PCB0006 | Hexachlorobiphenyls |
| PCB101 | 2,2',4,5,5'-Pentachlorobiphenyl |
| PCB1016 | PCB-1016 (Aroclor 1016) |
| PCB110 | 2,3,3',4',6-Pentachlorobiphenyl |
| PCB1221 | PCB-1221 (Aroclor 1221) |
| PCB1232 | PCB-1232 (Aroclor 1232) |
| PCB1242 | PCB-1242 (Aroclor 1242) |
| PCB1242/1016 | PCB-1242/1016 (Aroclor 1242/1016) |
| PCB1248 | PCB-1248 (Aroclor 1248) |
| PCB1254 | PCB-1254 (Aroclor 1254) |
| PCB1260 | PCB-1260 (Aroclor 1260) |
| PCB1262 | PCB-1262 (Aroclor 1262) |
| PCB1268 | PCB-1268 (Aroclor 1268) |
| PCB138 | 2,2',3,4,4',5'-Hexachlorobiphenyl |
| PCB139 | 1,1'-Biphenyl, 2,2',3,4,4',6-hexachloro- |
| PCB141 | 2,2',3,4,5,5'-Hexachlorobiphenyl |
| PCB151 | 2,2',3,5,5',6-Hexachlorobiphenyl |
| PCB153 | 2,2',4,4',5,5'-Hexachlorobiphenyl |
| PCB170 | 2,2',3,3',4,4',5-Heptachlorobiphenyl |
| PCB18 | 2,2',5-Trichlorobiphenyl |
| PCB180 | 2,2',3,4,4',5,5'-Heptachlorobiphenyl |
| PCB182 | 1,1'-Biphenyl, 2,2',3,4,4',5,6'-heptachloro- |
| PCB187 | 2,2',3,4',5,5',6-Heptachlorobiphenyl |
| PCB199 | 1,1'-Biphenyl, 2,2',3,3',4,5,5',6'-octachloro- |
| PCB206 | 2,2',3,3',4,4',5,5',6-Nonachlorobiphenyl |
| PCB44 | 2,2',3,5'-Tetrachlorobiphenyl |
| PCB52 | 2,2',5,5'-Tetrachlorobiphenyl |
| PCB66 | 2,3',4,4'-Tetrachlorobiphenyl |
| PCB87 | 2,2',3,4,5'-Pentachlorobiphenyl |
| PCE | Tetrachloroethene |
| PCLEA | Pentachloroethane |
| PCNB | PCNB (Quintozene) |
| PCP | Pentachlorophenol |
| PDMAABZ | p-Dimethylaminoazobenzene |
| PECBPH | Total Pentachlorinatedbiphenyls |
| PECDD | Total Pentachlorodibenzo-p-dioxin (PeCDD) |
| PECDD12378 | 1,2,3,7,8-Pentachlorodibenzo-p-dioxin |
| PECDF | Total Pentachlorodibenzofurans (PeCDF) |
| PECDF12378 | 1,2,3,7,8-Pentachlorodibenzofuran |
| PECDF23478 | 2,3,4,7,8-Pentachlorodibenzofuran |
| PECLBZ | Pentachlorobenzene |
| PECLNO2BZ | Pentachloronitrobenzene |
| PERMETHRIN | Permethrin |
| PERTHANE | Perthane |
| PERY | Perylene |
| PERYD12 | Perylene-d12 |
| PETN | Pentaerythritol tetranitrate |
| PEXT | Extractable Phosphorus |
| PFBZ | Pentafluorobenzene |
| PFP | Pentafluorophenol |
| PFTEST | Paint Filter Test |
| PH | pH |
| PH246BR | 2,4,6-Tribromophenol |
| PH2F | 2-Fluorophenol |
| PHAN | Phenanthrene |
| PHANC1A | C1-Phenanthrenes/Anthrace |
| PHANC2A | C2-Phenanthrenes/Anthrace |
| PHANC3A | C3-Phenanthrenes/Anthrace |
| PHANC4A | C4-Phenanthrenes/Anthrace |
| PHAND10 | Phenanthrene-d10 |
| PHANHY | Phthalic anhydride |
| PHC | Petroleum Hydrocarbons (TPH) |
| PHCD | PHC as Diesel Fuel (TPHD) |
| PHCDUNK | Non-PHC as Diesel |
| PHCFO | PHC as Fuel Oils |
| PHCFOUNK | Non-PHC as Fuel Oils |
| PHCG | PHC as Gasoline (TPHG) |
| PHCGC7C12 | PHC as Gasoline (C7-C12) |
| PHCGUNK | Non-PHC as Gasoline |
| PHCHFO | PHC as Heavy Fuel Oils |
| PHCHFOUNK | Non-PHC as Heavy Fuel Oils |
| PHCIO | PHC as Insulating Oil |
| PHCJP4 | PHC as JP-4 |
| PHCK | PHC as Kerosene |
| PHCLUB | PHC as Lube Oil |
| PHCLUBUNK | Non-PHC as Lube Oil |
| PHD5 | Phenol-d5 |
| PHEN2F | 2-Fluorobiphenyl |
| PHEND14 | Terphenyl-d14 |
| PHENM | m-Terphenyl |
| PHENO | o-Terphenyl |
| PHENOBAL | Phenobarbital |
| PHENOL | Phenol |
| PHENOLD6 | Phenol-d6 |
| PHENP | p-Terphenyl |
| PHENYTOIN | 5,5-Diphenylhydantoin |
| PHNACTN | Phenacetin |
| PHORATE | Phorate |
| PHOSAL | Phosalone |
| PHOSMET | Phosmet |
| PHOSPHAM | Phosphamidon |
| PHTHL | Phthalates |
| PICLORAM | Picloram |
| PICOLINE2 | 2-Picoline |
| PL | Palladium |
| PO4 | Phosphorus, Total Orthophosphate (as PO4) |
| PO4RS | Phosphorus, Reactive soluble |
| PORG | Phosphorus, Total Organic (as P) |
| PORTHO | Phosphorus, Total Orthophosphate (as P) |
| POURPT | Pour Point |
| PR2BRCL | 2-Bromo-1-chloropropane |
| PRIMICID | Pirimphos-ethyl |
| PROH | n-Propanol |
| PRONAMD | Pronamide |
| PROPACHLOR | Propachlor |
| PROPENE | Propylene |
| PROPGLY | Propylene glycol |
| PROPYCIL | Propylthiouracil |
| PROPYL | n-Propylamine |
| PT | Platinum |
| PTCBPH22346 | 2,2,3,4,6-Pentachlorobiphenyl |
| PU-238 | Plutonium-238 |
| PU239240 | Plutonium-239/240 |
| PYR | Pyrene |
| PYRD10 | Pyrene-d10 |
| PYRDN | Pyridine |
| QUINO | Quinoline |
| RA | Radium |
| RA-223 | Radium-223 |
| RA-224 | Radium-224 |
| RA-226 | Radium-226 |
| RA-228 | Radium-228 |
| RA226228 | Radium-226 and Radium-228 |
| RB | Rubidium |
| RDX | Hexahydro-1,3,5-trinitro-1,3,5-triazine |
| RECN | Reactive Cyanide |
| REDOX | Oxidation-Reduction Potential |
| RH | Rhodium |
| RONNEL | Ronnel |
| RRO | Residual Range Organics |
| RROALIPHATIC | Residual Range Organics, Aliphatic |
| RROAROMATIC | Residual Range Organics, Aromatic |
| RROC25C36 | Residual Range Organics (C25-C36) |
| RROC28C40 | Residual Range Organics (C28-C40) |
| RU | Ruthenium |
| S | Sulfide |
| SAFROLE | Safrole |
| SALINITY | Salinity |
| SALMONELLA | Salmonella |
| SAR | Sodium Absorption Ratio |
| SB | Antimony |
| SC | Specific Conductance |
| SCANDIUM | Scandium |
| SE | Selenium |
| SEM | Simultaneously Extracted Metals (Cd, Cu, Pb, Hg, Ni, and Zn) |
| SETMAT | Settleable Matter |
| SEVIN | Carbaryl |
| SG | Specific Gravity |
| SGTHEM | Silica Gel Treated Hexane Extractable Material |
| SI | Silicon |
| SIEVE10 | Sieve No. 10, Percent Passing |
| SIEVE10F | Sieve No. 10, Fractional Percent Retained |
| SIEVE10PHI | Sieve, 10 Phi, Percent Passing |
| SIEVE10PHIF | Sieve, 10 Phi, Fractional Percent Retained |
| SIEVE140 | Sieve No. 140, Percent Passing |
| SIEVE140F | Sieve No. 140, Fractional Percent Retained |
| SIEVE20 | Sieve No. 20, Percent Passing |
| SIEVE200 | Sieve No. 200, Percent Passing |
| SIEVE200F | Sieve No. 200, Fractional Percent Retained |
| SIEVE20F | Sieve No. 20, Fractional Percent Retained |
| SIEVE230 | Sieve No. 230, Percent Passing |
| SIEVE230F | Sieve No. 230, Fractional Percent Retained |
| SIEVE4 | Sieve No. 4, Percent Passing |
| SIEVE40 | Sieve No. 40, Percent Passing |
| SIEVE40F | Sieve No. 40, Fractional Percent Retained |
| SIEVE4F | Sieve No. 4, Fractional Percent Retained |
| SIEVE4PHI | Sieve, 4 Phi, Percent Passing |
| SIEVE4PHIF | Sieve, 4 Phi, Fractional Percent Retained |
| SIEVE5PHI | Sieve, 5 Phi, Percent Passing |
| SIEVE5PHIF | Sieve, 5 Phi, Fractional Percent Retained |
| SIEVE60 | Sieve No. 60, Percent Passing |
| SIEVE60F | Sieve No. 60, Fractional Percent Retained |
| SIEVE6PHI | Sieve, 6 Phi, Percent Passing |
| SIEVE6PHIF | Sieve, 6 Phi, Fractional Percent Retained |
| SIEVE7PHI | Sieve, 7 Phi, Percent Passing |
| SIEVE7PHIF | Sieve, 7 Phi, Fractional Percent Retained |
| SIEVE8PHI | Sieve, 8 Phi, Percent Passing |
| SIEVE8PHIF | Sieve, 8 Phi, Fractional Percent Retained |
| SIEVE9PHI | Sieve, 9 Phi, Percent Passing |
| SIEVE9PHIF | Sieve, 9 Phi, Fractional Percent Retained |
| SIL | Silica |
| SILICATE | Silicate |
| SILSI | Silicon, as Silica |
| SILVEX | 2,4,5-TP (Silvex) |
| SIMAZINE | Simazine |
| SM | Samarium |
| SN | Tin |
| SO3 | Sulfite |
| SO4 | Sulfate |
| SOLID | Solids, Percent |
| SOLIDVOA | Solids, Percent Volatile Components |
| SOLVRED23 | Solvent Red 23 |
| SOLVRED3 | Solvent Red 3 |
| SR | Strontium |
| SR-90 | Strontium-90 |
| SRB | Sulfate Reducing Bacteria |
| SRTOT | Total Strontium |
| SS | Suspended Solids |
| STIROFOS | Tetrachlorvinphos (Stirophos) |
| STROBANE | Strobane |
| STRYCHNINE | Strychnine |
| STY | Styrene |
| SU | Sulfur |
| SUB2MEPA3 | Substituted Propanoic acid |
| SUB2MOTENE | Substituted Dimethyl octene |
| SUBACEAC | Substituted Acetic acid |
| SUBALKANE | Substituted Alkane |
| SUBALKENE | Substituted Alkene |
| SUBBEN1 | Substituted Benzene #1 |
| SUBBEN2 | Substituted Benzene #2 |
| SUBBEN3 | Substituted Benzene #3 |
| SUBBEN4 | Substituted Benzene #4 |
| SUBBEN5 | Substituted Benzene #5 |
| SUBBZ | Substituted Benzene |
| SUBBZACID | Substituted Benzoic acid |
| SUBBZALD | Substituted Benzaldehyde |
| SUBBZAMIDE | Substituted Benzamide |
| SUBBZPA | Substituted Benzenepropanoic acid |
| SUBBZSAMIDE | Substituted Benzenesulfonamide |
| SUBCBT | Substituted Cyclobutane |
| SUBCHXN | Substituted Cyclohexane |
| SUBCHYD | Substituted Cyclic hydrocarbon |
| SUBCPT | Substituted Cyclopentane |
| SUBCPTO | Substituted Cyclopentanone |
| SUBDIOXIN | Substituted Dioxin |
| SUBDIOXLANE | Substituted Dioxolane Compound |
| SUBDS | Substituted Disulfide |
| SUBETHANOL | Substituted Ethanol |
| SUBETHONE | Substituted Ethanone |
| SUBH3PO4 | Substituted Phosphonic acid |
| SUBHDIOIC | Substituted Hexanedioic acid |
| SUBHEPTANONE | Substituted Heptanone |
| SUBIND1 | Substituted Indene #1 |
| SUBIND2 | Substituted Indene #2 |
| SUBIND3 | Substituted Indene #3 |
| SUBIND4 | Substituted Indene #4 |
| SUBIND5 | Substituted Indene #5 |
| SUBINDENE | Substituted Indene |
| SUBINDENONE | Substituted Indenone |
| SUBMALKANE | Methyl substituted alkane |
| SUBMCHX | Methyl substituted cyclohexane |
| SUBNAPH | Substituted Naphthalene |
| SUBOCTENE | Substituted Octene |
| SUBOXIRANE | Substituted Oxirane |
| SUBPAH | Substituted PAH |
| SUBPENTENE | Substituted Pentene |
| SUBPHAN | Substituted Phenanthrene |
| SUBPHENOL | Substituted Phenol |
| SUBPLENE | Substituted Pentalene |
| SUBPROPANOL | Substituted Propanol |
| SUBPYR | Substituted Pyrene |
| SULFAL | Sulfallate |
| SULFID-R | Reactive Sulfide |
| SULFOTEP | Sulfotep |
| SULFX | Piperonyl sulfoxide |
| SULPROFOS | Bolstar (Sulprofos) |
| SURFACT | Surfactants |
| SYNTHETIC | Synthetic |
| T23P | Tris(2,3-dibromopropyl)phosphate |
| TA | Tantalum |
| TAL | Tannin and Lignin |
| TAME | tert-Amyl methyl ether |
| TB | Terbium |
| TBME | Bromoform |
| TBP | Tributyl phosphate |
| TBTSN | Tributyltin |
| TBUTMEE | Methyl-t-butyl ether |
| TC1112 | 1,1,1,2-Tetrachloroethane |
| TCA111 | 1,1,1-Trichloroethane |
| TCA112 | 1,1,2-Trichloroethane |
| TCB123 | 1,2,3-Trichlorobenzene |
| TCB124 | 1,2,4-Trichlorobenzene |
| TCBZME | a,a,a-Trichlorotoluene |
| TCDD | Total Tetrachlorodibenzo-p-dioxins (TCDD) |
| TCDD2378 | 2,3,7,8-Tetrachlorodibenzo-p-dioxin |
| TCDD2378C13 | 2,3,7,8-Tetrachlorodiibenzo-p-dioxin-C13 |
| TCDD2378CL37 | 2,3,7,8-Tetrachlorodibenzo-p-dioxin-CL37 |
| TCDF | Total Tetrachlorodibenzofurans (TCDF) |
| TCDF2378 | 2,3,7,8-Tetrachlorodibenzofuran |
| TCDF2378C13 | 2,3,7,8-Tetrachlorodibenzofuran-C13 |
| TCE | Trichloroethene |
| TCLME | Chloroform |
| TCLMED | Chloroform-D |
| TCP2346 | 2,3,4,6-Tetrachlorophenol |
| TCP235 | 2,3,5-Trichlorophenol |
| TCP245 | 2,4,5-Trichlorophenol |
| TCP246 | 2,4,6-Trichlorophenol |
| TCPR | Trichloropropane |
| TCPR123 | 1,2,3-Trichloropropane |
| TDGCL | Thiodiglycol |
| TDI | Toluene diisocyanate |
| TDS | Total Dissolved Solids |
| TEBTSN | Tetrabutyltin |
| TECLPHS | Tetrachlorophenols |
| TECMXYL | Tetrachlorometaxylene |
| TEGLY | Tetraethylene glycol |
| TEMP | Temperature |
| TEP | Triethyl phosphate |
| TEPP | Tetraethyl pyrophosphate |
| TEPTH | o,o,o-Triethyl phosphorothioate |
| TERBUFOS | Terbufos |
| TERMIL | Chlorothalonil |
| TETCBPH2244 | 2,2,4,4-Tetrachlorobiphenyl |
| TETRALIN | Naphthalene, 1,2,3,4-tetrahydro- |
| TETRYL | Methyl-2,4,6-trinitrophenylnitramine |
| TFBZME | Trifluorotoluene |
| TFS | Total Fixed Solids |
| TH | Thorium |
| TH1234N1 | 1,2,3,4-Tetrahydro-1-naphthol |
| THF | Tetrahydrofuran |
| THIOFANOX | Thiofanox |
| THM | Total Trihalomethanes |
| THNAPH | Tetrahydronaphthol |
| TI | Titanium |
| TIME | Time |
| TL | Thallium |
| TLDNO | o-Toluidine |
| TLDNONT5 | 5-Nitro-o-toluidine |
| TLIPID | Total Lipids |
| TM224C5N | 2,2,4-Trimethylpentane (Isoctane) |
| TM235NPH | 2,3,5-Trimethylnaphthalene |
| TM236NPH | 2,3,6-Trimethylnaphthalene |
| TMANIL245 | 2,4,5-Trimethylaniline |
| TMB123 | 1,2,3-Trimethylbenzene |
| TMB124 | 1,2,4-Trimethylbenzene |
| TMB135 | 1,3,5-Trimethylbenzene |
| TMCYHX | Trimethyl cyclohexane |
| TMEBZ1234 | 1,2,3,4-Tetramethylbenzene |
| TMEBZ1245 | 1,2,4,5-Tetramethylbenzene |
| TMEHX | Trimethyl hexane |
| TMEP | Trimethyl phosphate |
| TMGLY | Tetramethylene glycol |
| TNB135 | 1,3,5-Trinitrobenzene |
| TNT | 2,4,6-Trinitrotoluene |
| TOC | Total Organic Carbon (TOC) |
| TOKUTHION | Tokuthion (Prothiofos) |
| TOTBACTERIA | Total Bacteria |
| TOTCHLRN | Total Chlorine |
| TOTPCB | Total Polychlorinatedbiphenyls |
| TOTPHEN | Phenolics, Total recoverable |
| TOTX | Total Halogens |
| TOX | Total Organic Halides (TOX) |
| TOXAP | Toxaphene |
| TPH | Triphenylene |
| TPHP | Triphenyl phosphate |
| TPROPSN | Tripropyltin |
| TPTSN | Tripentyltin |
| TRICBPH245 | 2,4,5,-Trichlorobiphenyl |
| TRICHLORFON | Trichlorfon |
| TRICLPHS | Trichlorophenols |
| TRIFLURALIN | Trifluralin |
| TSO | Total Solids |
| TT4P | Tri-p-tolyl phosphate |
| TTCT | Tetratetracontane |
| TURB | Turbidity |
| TVO | Total Volatile Organics |
| TVS | Total Volatile Solids |
| TX | Total Halides |
| U | Uranium |
| U-234 | Uranium-234 |
| U-235 | Uranium-235 |
| U-238 | Uranium-238 |
| UNALD10 | Unknown Aldehyde #10 |
| UNALD8 | Unknown Aldehyde #8 |
| UNALD9 | Unknown Aldehyde #9 |
| UNK | Unknown |
| UNK1 | Unknown #1 |
| UNK10 | Unknown #10 |
| UNK11 | Unknown #11 |
| UNK12 | Unknown #12 |
| UNK13 | Unknown #13 |
| UNK14 | Unknown #14 |
| UNK15 | Unknown #15 |
| UNK16 | Unknown #16 |
| UNK17 | Unknown #17 |
| UNK18 | Unknown #18 |
| UNK19 | Unknown #19 |
| UNK2 | Unknown #2 |
| UNK20 | Unknown #20 |
| UNK3 | Unknown #3 |
| UNK4 | Unknown #4 |
| UNK5 | Unknown #5 |
| UNK6 | Unknown #6 |
| UNK7 | Unknown #7 |
| UNK8 | Unknown #8 |
| UNK9 | Unknown #9 |
| UNKAH | Unknown Aromatic hydrocarbon |
| UNKALC1 | Unknown Alcohol #1 |
| UNKALC2 | Unknown Alcohol #2 |
| UNKALC3 | Unknown Alcohol #3 |
| UNKALC4 | Unknown Alcohol #4 |
| UNKALC5 | Unknown Alcohol #5 |
| UNKALCOHOL | Unknown Alcohol |
| UNKALD1 | Unknown Aldehyde #1 |
| UNKALD2 | Unknown Aldehyde #2 |
| UNKALD3 | Unknown Aldehyde #3 |
| UNKALD4 | Unknown Aldehyde #4 |
| UNKALD5 | Unknown Aldehyde #5 |
| UNKALD6 | Unknown Aldehyde #6 |
| UNKALD7 | Unknown Aldehyde #7 |
| UNKALDEHYDE | Unknown Aldehyde |
| UNKALH1 | Unknown Aliphatic hydrocarbon #1 |
| UNKALH2 | Unknown Aliphatic hydrocarbon #2 |
| UNKALH3 | Unknown Aliphatic hydrocarbon #3 |
| UNKALH4 | Unknown Aliphatic hydrocarbon #4 |
| UNKALH5 | Unknown Aliphatic hydrocarbon #5 |
| UNKALIPHY | Unknown Aliphatic hydrocarbon |
| UNKALK1 | Unknown Alkene #1 |
| UNKALK10 | Unknown Alkene #10 |
| UNKALK2 | Unknown Alkene #2 |
| UNKALK3 | Unknown Alkene #3 |
| UNKALK4 | Unknown Alkene #4 |
| UNKALK5 | Unknown Alkene #5 |
| UNKALK6 | Unknown Alkene #6 |
| UNKALK7 | Unknown Alkene #7 |
| UNKALK8 | Unknown Alkene #8 |
| UNKALK9 | Unknown Alkene #9 |
| UNKALKANE | Unknown Alkane |
| UNKALKANE1 | Unknown Alkane #1 |
| UNKALKANE10 | Unknown Alkane #10 |
| UNKALKANE11 | Unknown Alkane #11 |
| UNKALKANE12 | Unknown Alkane #12 |
| UNKALKANE13 | Unknown Alkane #13 |
| UNKALKANE14 | Unknown Alkane #14 |
| UNKALKANE15 | Unknown Alkane #15 |
| UNKALKANE16 | Unknown Alkane #16 |
| UNKALKANE17 | Unknown Alkane #17 |
| UNKALKANE18 | Unknown Alkane #18 |
| UNKALKANE19 | Unknown Alkane #19 |
| UNKALKANE2 | Unknown Alkane #2 |
| UNKALKANE20 | Unknown Alkane #20 |
| UNKALKANE3 | Unknown Alkane #3 |
| UNKALKANE4 | Unknown Alkane #4 |
| UNKALKANE5 | Unknown Alkane #5 |
| UNKALKANE6 | Unknown Alkane #6 |
| UNKALKANE7 | Unknown Alkane #7 |
| UNKALKANE8 | Unknown Alkane #8 |
| UNKALKANE9 | Unknown Alkane #9 |
| UNKALKBZ | Unknown Alkyl benzene |
| UNKALKBZ1 | Unknown Alkyl benzene #1 |
| UNKALKBZ10 | Unknown Alkyl benzene #10 |
| UNKALKBZ11 | Unknown Alkyl benzene #11 |
| UNKALKBZ12 | Unknown Alkyl benzene #12 |
| UNKALKBZ13 | Unknown Alkyl benzene #13 |
| UNKALKBZ14 | Unknown Alkyl benzene #14 |
| UNKALKBZ15 | Unknown Alkyl benzene #15 |
| UNKALKBZ16 | Unknown Alkyl benzene #16 |
| UNKALKBZ2 | Unknown Alkyl benzene #2 |
| UNKALKBZ3 | Unknown Alkyl benzene #3 |
| UNKALKBZ4 | Unknown Alkyl benzene #4 |
| UNKALKBZ5 | Unknown Alkyl benzene #5 |
| UNKALKBZ6 | Unknown Alkyl benzene #6 |
| UNKALKBZ7 | Unknown Alkyl benzene #7 |
| UNKALKBZ8 | Unknown Alkyl benzene #8 |
| UNKALKBZ9 | Unknown Alkyl benzene #9 |
| UNKALKENE | Unknown Alkene |
| UNKALKENOL | Unknown Branched alkenol |
| UNKALKON1 | Unknown Alkanone #1 |
| UNKALKSUBBZ | Unknown Alkyl substituted benzene |
| UNKALKYNE | Unknown Alkyne |
| UNKAMIDE | Unknown Amide |
| UNKAMINE | Unknown Amine |
| UNKARH1 | Unknown Aromatic hydrocarbon #1 |
| UNKARH2 | Unknown Aromatic hydrocarbon #2 |
| UNKARH3 | Unknown Aromatic hydrocarbon #3 |
| UNKARH4 | Unknown Aromatic hydrocarbon #4 |
| UNKARH5 | Unknown Aromatic hydrocarbon #5 |
| UNKARO | Unknown Aromatic |
| UNKAROKET | Unknown Aromatic ketone |
| UNKBALK | Unknown Branched alkane |
| UNKBALK1 | Unknown Branched alkane #1 |
| UNKBALK2 | Unknown Branched alkane #2 |
| UNKBALK3 | Unknown Branched alkane #3 |
| UNKBALK4 | Unknown Branched alkane #4 |
| UNKBALK5 | Unknown Branched alkane #5 |
| UNKBALKANOL | Unknown Branched alkanol |
| UNKBALKENE | Unknown Branched alkene |
| UNKBCARBA | Unknown Branched carboxylic acid |
| UNKBCYALK | Unknown Branched cycloalkane |
| UNKBICYCLIC | Unknown Bicyclic |
| UNKBKETONE | Unknown Branched ketone |
| UNKBNAPH | Unknown Branched naphthalene |
| UNKBPAH | Unknown Branched PAH |
| UNKBRTRIENOL | Unknown Branched trienol |
| UNKBZALD | Unknown Benzaldehyde |
| UNKCALKANE | Unknown Cyclic alkane |
| UNKCARA1 | Unknown Carboxcylic acid #1 |
| UNKCARA2 | Unknown Carboxcylic acid #2 |
| UNKCARA3 | Unknown Carboxcylic acid #3 |
| UNKCARA4 | Unknown Carboxcylic acid #4 |
| UNKCARA5 | Unknown Carboxcylic acid #5 |
| UNKCARBAEST | Unknown Carboxylic acid ester |
| UNKCARBOXA | Unknown Carboxylic acid |
| UNKCHYD | Unknown Cyclic hydrocarbon |
| UNKCKETONE | Unknown Cyclic ketone |
| UNKCPYRDN | Unknown Chlorinated pyridine |
| UNKCYAL1 | Unknown Cyclic alkane #1 |
| UNKCYAL10 | Unknown Cyclic alkane #10 |
| UNKCYAL2 | Unknown Cyclic alkane #2 |
| UNKCYAL3 | Unknown Cyclic alkane #3 |
| UNKCYAL4 | Unknown Cyclic alkane #4 |
| UNKCYAL5 | Unknown Cyclic alkane #5 |
| UNKCYAL6 | Unknown Cyclic alkane #6 |
| UNKCYAL7 | Unknown Cyclic alkane #7 |
| UNKCYAL8 | Unknown Cyclic alkane #8 |
| UNKCYAL9 | Unknown Cyclic alkane #9 |
| UNKCYH1 | Unknown Cyclic hydrocarbon #1 |
| UNKCYH2 | Unknown Cyclic hydrocarbon #2 |
| UNKCYH3 | Unknown Cyclic hydrocarbon #3 |
| UNKCYH4 | Unknown Cyclic hydrocarbon #4 |
| UNKCYH5 | Unknown Cyclic hydrocarbon #5 |
| UNKCYHEX | Unknown Cyclohexane |
| UNKCYHEX1 | Unknown Cyclohexane #1 |
| UNKCYHEX2 | Unknown Cyclohexane #2 |
| UNKCYKET1 | Unknown Cycloketone #1 |
| UNKCYKET2 | Unknown Cycloketone #2 |
| UNKCYKET3 | Unknown Cycloketone #3 |
| UNKCYKET4 | Unknown Cycloketone #4 |
| UNKCYKET5 | Unknown Cycloketone #5 |
| UNKESTER | Unknown Ester |
| UNKESTER1 | Unknown Ester #1 |
| UNKESTER2 | Unknown Ester #2 |
| UNKESTER3 | Unknown Ester #3 |
| UNKESTER4 | Unknown Ester #4 |
| UNKESTER5 | Unknown Ester #5 |
| UNKFATACID | Unknown Fatty acid |
| UNKHOPANE | Unknown Hopane |
| UNKHOPANE1 | Unknown Hopane #1 |
| UNKHOPANE2 | Unknown Hopane #2 |
| UNKHOPANE3 | Unknown Hopane #3 |
| UNKHYD | Unknown Hydrocarbon |
| UNKHYD1 | Unknown Hydrocarbon #1 |
| UNKHYD10 | Unknown Hydrocarbon #10 |
| UNKHYD2 | Unknown Hydrocarbon #2 |
| UNKHYD3 | Unknown Hydrocarbon #3 |
| UNKHYD4 | Unknown Hydrocarbon #4 |
| UNKHYD5 | Unknown Hydrocarbon #5 |
| UNKHYD6 | Unknown Hydrocarbon #6 |
| UNKHYD7 | Unknown Hydrocarbon #7 |
| UNKHYD8 | Unknown Hydrocarbon #8 |
| UNKHYD9 | Unknown Hydrocarbon #9 |
| UNKINDOLE | Unknown Indole |
| UNKKET1 | Unknown Ketone #1 |
| UNKKET2 | Unknown Ketone #2 |
| UNKKET3 | Unknown Ketone #3 |
| UNKKET4 | Unknown Ketone #4 |
| UNKKET5 | Unknown Ketone #5 |
| UNKKETONE | Unknown Ketone |
| UNKMS1 | Unknown Methylated siloxane #1 |
| UNKMS2 | Unknown Methylated siloxane #2 |
| UNKMS3 | Unknown Methylated siloxane #3 |
| UNKOXYCOM | Unknown Oxygenated compound |
| UNKOXYCOM1 | Unknown Oxygenated compound #1 |
| UNKOXYCOM2 | Unknown Oxygenated compound #2 |
| UNKOXYCOM3 | Unknown Oxygenated compound #3 |
| UNKOXYCOM4 | Unknown Oxygenated compound #4 |
| UNKPAH | Unknown Polynuclear aromatic hydrocarbon |
| UNKPAH1 | Unknown Polynuclear aromatic hydrocarbon #1 |
| UNKPAH2 | Unknown Polynuclear aromatic hydrocarbon #2 |
| UNKPAH3 | Unknown Polynuclear aromatic hydrocarbon #3 |
| UNKPAH4 | Unknown Polynuclear aromatic hydrocarbon #4 |
| UNKPAH5 | Unknown Polynuclear aromatic hydrocarbon #5 |
| UNKPCB | Unknown Polychloronated biphenyl |
| UNKPCB1 | Unknown Polychloronated biphenyl #1 |
| UNKPCB10 | Unknown Polychloronated biphenyl #10 |
| UNKPCB11 | Unknown Polychloronated biphenyl #11 |
| UNKPCB12 | Unknown Polychloronated biphenyl #12 |
| UNKPCB13 | Unknown Polychloronated biphenyl #13 |
| UNKPCB2 | Unknown Polychloronated biphenyl #2 |
| UNKPCB3 | Unknown Polychloronated biphenyl #3 |
| UNKPCB4 | Unknown Polychloronated biphenyl #4 |
| UNKPCB5 | Unknown Polychloronated biphenyl #5 |
| UNKPCB6 | Unknown Polychloronated biphenyl #6 |
| UNKPCB7 | Unknown Polychloronated biphenyl #7 |
| UNKPCB8 | Unknown Polychloronated biphenyl #8 |
| UNKPCB9 | Unknown Polychloronated biphenyl #9 |
| UNKPHLATE | Unknown Phthalate |
| UNKPHT1 | Unknown Phthalate #1 |
| UNKPHT2 | Unknown Phthalate #2 |
| UNKPHT3 | Unknown Phthalate #3 |
| UNKPHT4 | Unknown Phthalate #4 |
| UNKPHT5 | Unknown Phthalate #5 |
| UNKSILOXANE | Unknown Siloxane |
| UNKSTE1 | Unknown Sterol #1 |
| UNKSTE2 | Unknown Sterol #2 |
| UNKSTE3 | Unknown Sterol #3 |
| UNKSTE4 | Unknown Sterol #4 |
| UNKSTE5 | Unknown Sterol #5 |
| UNKSTERANE | Unknown Sterane |
| UNKSTERANE1 | Unknown Sterane #1 |
| UNKSTERANE2 | Unknown Sterane #2 |
| UNKSTERANE3 | Unknown Sterane #3 |
| UNKSTERANE4 | Unknown Sterane #4 |
| UNKSTEROL | Unknown Sterol |
| UNKSUBALCOH | Unknown Substituted alcohol |
| UNKSUBALK1 | Unknown Substituted alkane #1 |
| UNKSUBALK2 | Unknown Substituted alkane #2 |
| UNKSUBALK3 | Unknown Substituted alkane #3 |
| UNKSUBALK4 | Unknown Substituted alkane #4 |
| UNKSUBALK5 | Unknown Substituted alkane #5 |
| UNKSUBARO1 | Unknown Substituted aromatic #1 |
| UNKSUBARO2 | Unknown Substituted aromatic #2 |
| UNKSUBARO3 | Unknown Substituted aromatic #3 |
| UNKSUBARO4 | Unknown Substituted aromatic #4 |
| UNKSUBARO5 | Unknown Substituted aromatic #5 |
| UNKSUBBZ1 | Unknown Substituted benzene #1 |
| UNKSUBBZ2 | Unknown Substituted benzene #2 |
| UNKSUBBZ3 | Unknown Substituted benzene #3 |
| UNKSUBBZ4 | Unknown Substituted benzene #4 |
| UNKSUBBZ5 | Unknown Substituted benzene #5 |
| UNKSUBCYALK | Unknown Substituted cycloalkane |
| UNKSUBIND1 | Unknown Substituted indene #1 |
| UNKSUBIND2 | Unknown Substituted indene #2 |
| UNKSUBIND3 | Unknown Substituted indene #3 |
| UNKSUBINDENE | Unknown Substituted indene |
| UNKSUBPAH | Unknown Substituted PAH |
| UNKSUBPAH1 | Unknown Substituted PAH #1 |
| UNKSUBPAH2 | Unknown Substituted PAH #2 |
| UNKSUBPHENOL | Unknown Substituted phenol |
| UNKSULPHUR | Unknown Sulphur |
| UNKTHIAZOLE | Unknown Thiazole |
| UTOT | Uranium, Total |
| V | Vanadium |
| VA | Vinyl acetate |
| VC | Vinyl chloride |
| VISCOS | Viscosity |
| W | Tungsten |
| WATER | Water Content |
| WICHLRN | Water Insoluble Chlorine |
| WOLLASTONITE | Wollastonite |
| WSCL | Water Soluble Chlorides |
| WTPH-D | State of Washington, Diesel Range Organics |
| WTPH-G | State of Washington, Gasoline Range Organics |
| XYL2456CLM | 2,4,5,6-Tetrachloro-meta-xylene |
| XYL246CLM | 2,4,5,6-Tetrachloro-meta-xylene (obsolete in Jan. 2000) |
| XYLENES | Xylenes |
| XYLENES1214 | Xylene, Isomers o & p |
| XYLENES1314 | Xylene, Isomers m & p |
| XYLM | m-Xylene |
| XYLMP | m,p-Xylene (Sum of Isomers) |
| XYLO | o-Xylene |
| XYLP | p-Xylene |
| Y | Yttrium |
| YB | Ytterbium |
| ZINOPHOS | Thionazine |
| ZN | Zinc |
| ZR | Zirconium |

Acceptable PARLABEL Values Sorted by Description:

| Code | Description |
| --- | --- |
| TC1112 | 1,1,1,2-Tetrachloroethane |
| TCA111 | 1,1,1-Trichloroethane |
| PCA | 1,1,2,2-Tetrachloroethane |
| FC113 | 1,1,2-Trichloro-1,2,2-trifluoroethane |
| TCA112 | 1,1,2-Trichloroethane |
| PCB199 | 1,1'-Biphenyl, 2,2',3,3',4,5,5',6'-octachloro- |
| PCB182 | 1,1'-Biphenyl, 2,2',3,4,4',5,6'-heptachloro- |
| PCB139 | 1,1'-Biphenyl, 2,2',3,4,4',6-hexachloro- |
| PCB0005 | 1,1'-Biphenyl, pentachloro- |
| DCA11 | 1,1-Dichloroethane |
| DCE11 | 1,1-Dichloroethene |
| DCPANE | 1,1-Dichloropropane |
| 11DCPROPN | 1,1-Dichloropropanone |
| DCP11 | 1,1-Dichloropropene |
| HPCDF1234678 | 1,2,3,4,6,7,8-Heptachlorodibenzofuran |
| DF1234678C13 | 1,2,3,4,6,7,8-Heptachlorodibenzofuran-C13 |
| HPCDD1234678 | 1,2,3,4,6,7,8-Heptachlorodibenzo-p-dioxin |
| DD1234678C13 | 1,2,3,4,6,7,8-Heptachlorodibenzo-p-dioxin-C13 |
| HPCDF1234789 | 1,2,3,4,7,8,9-Heptachlorodibenzofuran |
| DF1234789C13 | 1,2,3,4,7,8,9-Heptachlorodibenzofuran-C13 |
| HXCDF123478 | 1,2,3,4,7,8-Hexachlorodibenzofuran |
| DF123478C13 | 1,2,3,4,7,8-Hexachlorodibenzofuran-C13 |
| HXCDD123478 | 1,2,3,4,7,8-Hexachlorodibenzo-p-dioxin |
| DD123478C13 | 1,2,3,4,7,8-Hexachlorodibenzo-p-dioxin-C13 |
| TH1234N1 | 1,2,3,4-Tetrahydro-1-naphthol |
| TMEBZ1234 | 1,2,3,4-Tetramethylbenzene |
| 527-53-7 | 1,2,3,5-Tetramethylbenzene |
| HXCDF123678 | 1,2,3,6,7,8-Hexachlorodibenzofuran |
| DF123678C13 | 1,2,3,6,7,8-Hexachlorodibenzofuran-C13 |
| HXCDD123678 | 1,2,3,6,7,8-Hexachlorodibenzo-p-dioxin |
| DD123678C13 | 1,2,3,6,7,8-Hexachlorodibenzo-p-dioxin-C13 |
| HXCDF123789 | 1,2,3,7,8,9-Hexachlorodibenzofuran |
| DF123789C13 | 1,2,3,7,8,9-Hexachlorodibenzofuran-C13 |
| HXCDD123789 | 1,2,3,7,8,9-Hexachlorodibenzo-p-dioxin |
| PECDF12378 | 1,2,3,7,8-Pentachlorodibenzofuran |
| DF12378C13 | 1,2,3,7,8-Pentachlorodibenzofuran-C13 |
| PECDD12378 | 1,2,3,7,8-Pentachlorodibenzo-p-dioxin |
| DD12378C13 | 1,2,3,7,8-Pentachlorodibenzo-p-dioxin-C13 |
| TCB123 | 1,2,3-Trichlorobenzene |
| TCPR123 | 1,2,3-Trichloropropane |
| TMB123 | 1,2,3-Trimethylbenzene |
| C4BZ1245 | 1,2,4,5-Tetrachlorobenzene |
| TMEBZ1245 | 1,2,4,5-Tetramethylbenzene |
| TCB124 | 1,2,4-Trichlorobenzene |
| TMB124 | 1,2,4-Trimethylbenzene |
| DBCP | 1,2-Dibromo-3-chloropropane |
| EDB | 1,2-Dibromoethane |
| FC114 | 1,2-Dichloro-1,1,2,2-tetrafluoroethane |
| DCBZ12 | 1,2-Dichlorobenzene |
| DCBZ12D4 | 1,2-Dichlorobenzene-d4 |
| DCA12 | 1,2-Dichloroethane |
| DCA12D4 | 1,2-Dichloroethane-d4 |
| DCE12TOT | 1,2-Dichloroethene, Total |
| DCPA12 | 1,2-Dichloropropane |
| DM12NPH | 1,2-Dimethylnaphthalene |
| DNBZ12 | 1,2-Dinitrobenzene |
| DPHY12 | 1,2-Diphenylhydrazine |
| TMB135 | 1,3,5-Trimethylbenzene |
| TNB135 | 1,3,5-Trinitrobenzene |
| ERYTHRENE | 1,3-Butadiene |
| 13DCPR20H | 1,3-Dichloro-2-propanol |
| DCBZ13 | 1,3-Dichlorobenzene |
| DCPA13 | 1,3-Dichloropropane |
| DCP13 | 1,3-Dichloropropene (Total) |
| DM13NBZ2 | 1,3-Dimethyl-2-nitrobenzene |
| 874-41-9 | 1,3-Dimethyl-4-ethylbenzene |
| DNB13 | 1,3-Dinitrobenzene |
| DCBZ14D4 | 1,4 Dichlorobenzene-d4 |
| 14BZDIOL | 1,4-Benzenediol |
| DCBETOT | 1,4-Dichloro-2-butene, Total |
| DCBZ14 | 1,4-Dichlorobenzene |
| DCBTA14 | 1,4-Dichlorobutane |
| DFBZ14 | 1,4-Difluorobenzene |
| DNBZ14 | 1,4-Dinitrobenzene |
| DIOXANE14 | 1,4-Dioxane |
| 14DITH | 1,4-Dithiane |
| 14HYDROQ | 1,4-Hydroquinone |
| NAPHQ14 | 1,4-Naphthoquinone |
| OXATH14 | 1,4-Oxathiane |
| ANLNAM4 | 1,4-Phenylenediamine |
| 571-61-9 | 1,5-Dimethylnaphthalene |
| DM16NPH | 1,6-Dimethylnaphthalene |
| 13CPCB209 | 13C-PCB 209 |
| AC2T | 1-Acetyl-2-thiourea |
| 1BR2FLET | 1-Bromo-2-fluoroethane |
| CLFBZ2 | 1-Chloro-2-fluorobenzene |
| 1CL2MEPE | 1-Chloro-2-methylpropene |
| CLNO2BZ3 | 1-Chloro-3-nitrobenzene |
| CLFBZ4 | 1-Chloro-4-fluorobenzene |
| BTCL | 1-Chlorobutane |
| 4860-03-1 | 1-Chlorohexadecane |
| CLHXDC1 | 1-Chlorohexadecane |
| CLHX1 | 1-Chlorohexane |
| CLNPH1 | 1-Chloronaphthalene |
| CLODC1 | 1-Chlorooctadecane |
| 1CLOCT | 1-Chlorooctane |
| 933-98-2 | 1-Ethyl-2,3-dimethylbenzene |
| 1E2MBZ | 1-Ethyl-2-methylbenzene |
| 1127-76-0 | 1-Ethylnaphthalene |
| NAPHF | 1-Fluoronaphthalene |
| ET2HEOH | 1-Hexanol, 2-ethyl- |
| 592-41-6 | 1-Hexene |
| 13427-43-5 | 1-Hexene, 3,3,5-trimethyl- |
| MP22BZ | 1-Methyl-2-(2-propenyl)-benzene |
| 1074-17-5 | 1-Methyl-2-n-propylbenzene |
| 620-14-4 | 1-Methyl-3-ethylbenzene |
| 535-77-3 | 1-Methyl-3-isopropylbenzene |
| MTNPH1 | 1-Methylnaphthalene |
| MPHAN1 | 1-Methylphenanthrene |
| AMINONAPH1 | 1-Naphthylamine |
| 2088-07-5 | 1-Penten-3-ol, 2-methyl- |
| 25044-01-3 | 1-Penten-3-one, 2-methyl- |
| 115-11-7 | 1-Propene, 2-methyl- |
| BTOXETETL | 2-(2-Butoxyethoxy) ethanol |
| ETETH | 2-(2-Ethoxyethoxy) ethanol |
| 111-77-3 | 2-(2-Methoxyethoxy) ethanol |
| PCB206 | 2,2',3,3',4,4',5,5',6-Nonachlorobiphenyl |
| PCB170 | 2,2',3,3',4,4',5-Heptachlorobiphenyl |
| HCBPH2233446 | 2,2,3,3,4,4,6-Heptachlorobiphenyl |
| OCBPH2233456 | 2,2,3,3,4,5,6,6-Octachlorobiphenyl |
| PCB180 | 2,2',3,4,4',5,5'-Heptachlorobiphenyl |
| PCB138 | 2,2',3,4,4',5'-Hexachlorobiphenyl |
| PCB187 | 2,2',3,4',5,5',6-Heptachlorobiphenyl |
| PCB141 | 2,2',3,4,5,5'-Hexachlorobiphenyl |
| PCB87 | 2,2',3,4,5'-Pentachlorobiphenyl |
| PTCBPH22346 | 2,2,3,4,6-Pentachlorobiphenyl |
| PCB151 | 2,2',3,5,5',6-Hexachlorobiphenyl |
| PCB44 | 2,2',3,5'-Tetrachlorobiphenyl |
| PCB153 | 2,2',4,4',5,5'-Hexachlorobiphenyl |
| HXCBPH224456 | 2,2,4,4,5,6-Hexachlorobiphenyl |
| TETCBPH2244 | 2,2,4,4-Tetrachlorobiphenyl |
| PCB101 | 2,2',4,5,5'-Pentachlorobiphenyl |
| TM224C5N | 2,2,4-Trimethylpentane (Isoctane) |
| PCB52 | 2,2',5,5'-Tetrachlorobiphenyl |
| PCB18 | 2,2',5-Trichlorobiphenyl |
| DCPA22 | 2,2-Dichloropropane |
| PCB110 | 2,3,3',4',6-Pentachlorobiphenyl |
| PCB66 | 2,3',4,4'-Tetrachlorobiphenyl |
| HXCDF234678 | 2,3,4,6,7,8-Hexachlorodibenzofuran |
| DF234678C13 | 2,3,4,6,7,8-Hexachlorodibenzofuran-C13 |
| TCP2346 | 2,3,4,6-Tetrachlorophenol |
| PECDF23478 | 2,3,4,7,8-Pentachlorodibenzofuran |
| DF23478C13 | 2,3,4,7,8-Pentachlorodibenzofuran-C13 |
| 234TFBZME | 2,3,4-Trifluorotoluene |
| 565-75-3 | 2,3,4-Trimethylpentane |
| TCP235 | 2,3,5-Trichlorophenol |
| TM235NPH | 2,3,5-Trimethylnaphthalene |
| TM236NPH | 2,3,6-Trimethylnaphthalene |
| TCDF2378 | 2,3,7,8-Tetrachlorodibenzofuran |
| TCDF2378C13 | 2,3,7,8-Tetrachlorodibenzofuran-C13 |
| TCDD2378 | 2,3,7,8-Tetrachlorodibenzo-p-dioxin |
| TCDD2378CL37 | 2,3,7,8-Tetrachlorodibenzo-p-dioxin-CL37 |
| TCDD2378C13 | 2,3,7,8-Tetrachlorodiibenzo-p-dioxin-C13 |
| DCBPH23 | 2,3-Dichlorobiphenyl |
| 4175-53-5 | 2,3-Dihydro-1,3-dimethyl-1H-indene |
| 767-58-8 | 2,3-Dihydro-1-methyl-1H-indene |
| 27133-93-3 | 2,3-Dihydro-1-methylindene |
| 6682-71-9 | 2,3-Dihydro-4,7-dimethyl-1H-indene |
| 824-22-6 | 2,3-Dihydro-4-methyl-1H-indene |
| 874-35-1 | 2,3-Dihydro-5-methyl-1H-indene |
| XYL2456CLM | 2,4,5,6-Tetrachloro-meta-xylene |
| XYL246CLM | 2,4,5,6-Tetrachloro-meta-xylene (obsolete in Jan. 2000) |
| TRICBPH245 | 2,4,5,-Trichlorobiphenyl |
| 245T | 2,4,5-T |
| 245TBEE | 2,4,5-T, butoxyethanol ester |
| 245TBE | 2,4,5-T, butyl ester |
| SILVEX | 2,4,5-TP (Silvex) |
| TCP245 | 2,4,5-Trichlorophenol |
| TMANIL245 | 2,4,5-Trimethylaniline |
| PH246BR | 2,4,6-Tribromophenol |
| TCP246 | 2,4,6-Trichlorophenol |
| TNT | 2,4,6-Trinitrotoluene |
| 24D | 2,4-D |
| 24DEE | 2,4-D, 2-ethylhexyl ester |
| 24DBE | 2,4-D, butoxyethanol ester |
| 24DB | 2,4-DB |
| DDD24 | 2,4'-DDD |
| DDE24 | 2,4'-DDE |
| DDT24 | 2,4'-DDT |
| MTD | 2,4-Diaminotoluene |
| DCBZALD24 | 2,4-Dichlorobenzaldehyde |
| DCP24 | 2,4-Dichlorophenol |
| 24DCPHYAA | 2,4-Dichlorophenylacetic acid |
| DMP24 | 2,4-Dimethylphenol |
| DNA24 | 2,4-Dinitroaniline |
| 24NO2FBZ | 2,4-Dinitrofluorobenzene |
| DNP24 | 2,4-Dinitrophenol |
| DNT24 | 2,4-Dinitrotoluene |
| DCP26 | 2,6-Dichlorophenol |
| 4057-42-5 | 2,6-Dimethyl-2-octene |
| 26DIM6NITRO | 2,6-Dimethyl-6-nitro-2-hepten-4-one |
| DM26NPH | 2,6-Dimethylnaphthalene |
| DNT26 | 2,6-Dinitrotoluene |
| ACAMFL2 | 2-Acetylaminofluorene |
| A2DNT46 | 2-Amino-4,6-dinitrotoluene |
| AMAQ2 | 2-Aminoanthraquinone |
| PR2BRCL | 2-Bromo-1-chloropropane |
| 2BR46DCP | 2-Bromo-4,6-dichlorophenol |
| 2BRPROPENE | 2-Bromopropene |
| MEK | 2-Butanone |
| 111-76-2 | 2-Butoxy-ethanol |
| CHLOROPRENE | 2-Chloro-1,3-butadiene (Chloroprene) |
| CLACRN2 | 2-Chloroacrylonitrile |
| 2CLANILINE | 2-Chloroaniline |
| 2CLANTH | 2-Chloroanthracene |
| CBPH2 | 2-Chlorobiphenyl |
| CLCYHXAL2 | 2-Chlorocyclohexanol |
| CL2ETOH | 2-Chloroethanol |
| CEVETH | 2-Chloroethyl vinyl ether |
| CNPH2 | 2-Chloronaphthalene |
| CLPH2 | 2-Chlorophenol |
| CLPH2D4 | 2-Chlorophenol-d4 |
| CLBZME2 | 2-Chlorotoluene |
| CYHEX2DNP46 | 2-Cyclohexyl-4,6-dinitrophenol |
| 2870-04-4 | 2-Ethyl-1,3-dimethylbenzene |
| PHEN2F | 2-Fluorobiphenyl |
| PH2F | 2-Fluorophenol |
| HXO2 | 2-Hexanone |
| 2HPROPN | 2-Hydroxypropionitrile |
| 2027-17-0 | 2-Isopropylnaphthalene |
| DN46M | 2-Methyl-4,6-dinitrophenol |
| 78-78-4 | 2-Methylbutane |
| MTNPH2 | 2-Methylnaphthalene |
| MTNPH2D10 | 2-Methylnaphthalene-d10 |
| MEPH2 | 2-Methylphenol (o-Cresol) |
| 2MPA1E | 2-Methylpropanoic acid |
| AMINONAPH2 | 2-Naphthylamine |
| NO2ANIL2 | 2-Nitroaniline |
| 2NO2MXYL | 2-Nitro-m-Xylene |
| NTPH2 | 2-Nitrophenol |
| NPR2 | 2-Nitropropane |
| NBZME2 | 2-Nitrotoluene |
| NBZME24 | 2-Nitrotoluene and 4-Nitrotoluene (Total) |
| MPK | 2-Pentanone |
| DIACOH | 2-Pentanone, 4-hydroxy-4-methyl- |
| PICOLINE2 | 2-Picoline |
| 2PROPENOL | 2-Propenol |
| 2PRYN1OL | 2-Propyn-1-ol |
| 2PYRR1M | 2-Pyrrolidinone, 1-methyl- |
| CLM3CPYRDN | 3-(Chloromethyl)pyridine hydrochloride |
| DBZD33 | 3,3'-Dichlorobenzidine |
| DMOBZD33 | 3,3'-Dimethoxybenzidine |
| DMBZD33 | 3,3'-Dimethylbenzidine |
| DNT34 | 3,4-Dinitrotoluene |
| DCBZA35 | 3,5-Dichlorobenzoic acid |
| FARN | 3,7,11-Trimethyl-2,6,10-dodecatrien-1-ol |
| A3ECBZ9 | 3-Amino-9-ethylcarbazole |
| CPN3 | 3-Chloropropionitrile |
| 16747-32-3 | 3-Ethyl-2,2-dimethylpentane |
| 107-86-8 | 3-Methyl-2-butenal |
| MECHLAN3 | 3-Methylcholanthrene |
| MEPH3 | 3-Methylphenol |
| MEPH34CO | 3-Methylphenol/4-Methylphenol Coelution |
| NO2ANIL3 | 3-Nitroaniline |
| NBZME3 | 3-Nitrotoluene |
| 625-33-2 | 3-Penten-2-one |
| DDD44 | 4,4'-DDD |
| DDE44 | 4,4'-DDE |
| DDE44DIEL | 4,4'-DDE/Dieldrin |
| DDT44 | 4,4'-DDT |
| MB2CAN44 | 4,4'-Methylenebis(2-chloraniline) |
| MICHLER | 4,4'-Methylenebis(N,N-dimethylaniline) |
| ODA | 4,4'-Oxydianiline |
| A4DNT26 | 4-Amino-2,6-dinitrotoluene |
| AMINOBPH4 | 4-Aminobiphenyl |
| BR4DCP26 | 4-Bromo-2,6-dichlorophenol |
| BRCLBZ4 | 4-Bromochlorobenzene |
| BR4FBZ | 4-Bromofluorobenzene |
| BPPE4 | 4-Bromophenyl phenyl ether |
| CL4PED12 | 4-Chloro-1,2-phenylenediamine |
| CL4PED13 | 4-Chloro-1,3-phenylenediamine |
| C4M2PH | 4-Chloro-2-methylphenol |
| C4M3PH | 4-Chloro-3-methylphenol |
| CLANIL4 | 4-Chloroaniline |
| CPPE4 | 4-Chlorophenyl phenyl ether |
| CLBZME4 | 4-Chlorotoluene |
| EBZME4 | 4-Ethyltoluene |
| CYMP | 4-Isopropyltoluene |
| MIBK | 4-Methyl-2-pentanone |
| 141-79-7 | 4-Methyl-3-penten-2-one |
| MEPH4 | 4-Methylphenol (p-Cresol) |
| NO2ANIL4 | 4-Nitroaniline |
| 4N2PHEN | 4-Nitrobiphenyl |
| NTPH4 | 4-Nitrophenol |
| 4NQO | 4-Nitroquinoline-n-oxide |
| NBZME4 | 4-Nitrotoluene |
| PHENYTOIN | 5,5-Diphenylhydantoin |
| ANDROSTANE5A | 5a-Androstane |
| CL5MANIL2 | 5-Chloro-2-methylaniline |
| HYDDICAM | 5-Hydroxydicamba |
| NACN5 | 5-Nitroacenaphthene |
| N2ANSD5 | 5-Nitro-o-anisidine |
| TLDNONT5 | 5-Nitro-o-toluidine |
| ME5CHRYSENE | 6-Methylchrysene |
| DMBZA712 | 7,12-Dimethylbenz(a)anthracene |
| DB7HCGCBZ | 7H-Dibenzo(c,g)carbazole |
| DPHANTH910 | 9,10-Diphenylanthracene |
| 1430-97-3 | 9H-Fluorene, 2-methyl- |
| 9PHENAN | 9-Phenylanthracene |
| TCBZME | a,a,a-Trichlorotoluene |
| AAATFBZME | a,a,a-Trifluorotoluene |
| MPEA11 | a,a-Dimethylphenethylamine |
| ACNP | Acenaphthene |
| ACNPD10 | Acenaphthene-d10 |
| ACNPY | Acenaphthylene |
| 79-20-9 | Acetic acid, methyl ester |
| ACE | Acetone |
| ACCN | Acetonitrile |
| ACPHN | Acetophenone |
| AGP | Acid Generating Potential |
| ANC | Acid Neutralizing Capacity |
| AISULFIDE | Acid-Insoluble Sulfide |
| ACID | Acidity, Total |
| ASULFIDE | Acid-Soluble Sulfide |
| AVS | Acid-Volatile Sulfide |
| AFN | Acifluorfen |
| ACRL | Acrolein |
| ACRN | Acrylamide |
| ACRAMD | Acrylonitrile |
| AC-228 | Actinium-228 |
| ALACL | Alachlor |
| ALDICARB | Aldicarb |
| ALDSULFOX | Aldicarb sulfoxide |
| ALDRIN | Aldrin |
| ALK | Alkalinity, Total |
| ALLYLALCOHOL | Allyl alcohol |
| CLPE3 | Allyl chloride |
| ALPHA | Alpha, Gross |
| BHCALPHA | alpha-BHC |
| CHLORDANEA | alpha-Chlordane |
| APINENE | alpha-Pinene |
| AL | Aluminum |
| CNA | Amenable Cyanide |
| AMAZOBENZ | Aminoazobenzene |
| NH3 | Ammonia |
| NH4N | Ammonium as Nitrogen |
| AMOSITE | Amosite |
| ANZIN | Anilazine |
| ANILINE | Aniline |
| ANILINED5 | Aniline-d5 |
| ANTH | Anthracene |
| ANTHD10 | Anthracene-d10 |
| SB | Antimony |
| APIGRAVITY | API Gravity |
| ARAMITE | Aramite |
| AS | Arsenic |
| ASBESTOS | Asbestos |
| ASH | Ash Content |
| ASULAM | Asulam |
| ATRAZINE | Atrazine |
| AZIPM | Azinphos methyl |
| AZOBENZENE | Azobenzene |
| BARBAN | Barban |
| BA | Barium |
| BTZ | Bentazon |
| BZALD | Benzaldehyde |
| BZ | Benzene |
| 2049-95-8 | Benzene, (1,1-Dimethylpropyl)- |
| 2050-24-0 | Benzene, 1,3-diethyl-5-methyl- |
| BTEX | Benzene, Toluene, Ethylbenzene, and Xylenes |
| BZD6 | Benzene-d6 |
| BZD | Benzidine |
| BZAA | Benzo(a)anthracene |
| BZAP | Benzo(a)pyrene |
| BZBF | Benzo(b)fluoranthene |
| BZBFBZKF | Benzo(b)fluoranthene and Benzo(k)fluoranthene |
| BZEP | Benzo(e)pyrene |
| BZGHIP | Benzo(g,h,i)perylene |
| BZJF | Benzo(j)fluoranthene |
| BZKF | Benzo(k)fluoranthene |
| BZACID | Benzoic acid |
| BZTZ | Benzothiazole |
| BZLAL | Benzyl alcohol |
| BBP | Benzyl butyl phthalate |
| BE | Beryllium |
| BETA | Beta, Gross |
| BHCBETA | beta-BHC |
| BPROPACT | beta-Propiolactone |
| ALKB | Bicarbonate Alkalinity as CACO3 |
| BICACO3 | Bicarbonate as CaCO3 |
| HCO3 | Bicarbonate as HCO3 |
| BCHPD | Bicyclo[2,2,1]hepta-2,5-diene |
| BOD5 | Biologic Oxygen Demand, Five-Day |
| BIPHENYL | Biphenyl |
| BIS2BEE | Bis(2-butoxyethyl) ether |
| BECEM | bis-(2-chloroethoxy)methane |
| B2CES | bis-(2-Chloroethyl) sulfide |
| BIS2CEE | bis-(2-Chloroethyl)ether |
| BIS2CIE | bis(2-Chloroisopropyl)ether |
| DOA | bis(2-ethylhexyl)adipate |
| BIS2EHP | bis-(2-ethylhexyl)phthalate |
| BI | Bismuth |
| BI-212 | Bismuth-212 |
| BI-214 | Bismuth-214 |
| SULPROFOS | Bolstar (Sulprofos) |
| 121-43-7 | Boric acid, trimethyl ester |
| B | Boron |
| BPLACTONE | b-Propiolactone |
| BROMCIL | Bromacil |
| BR | Bromide |
| BACE | Bromoacetone |
| BRBZ | Bromobenzene |
| BRCLME | Bromochloromethane |
| BDCME | Bromodichloromethane |
| BREA | Bromoethane |
| TBME | Bromoform |
| BRME | Bromomethane |
| BROXL | Bromoxynil |
| BDENSITY | Bulk Density |
| BUTACHLOR | Butachlor |
| BTA | Butanoic acid |
| BTSN | Butyltin |
| C10C10N | C10 as n-Decane |
| C10C12ALIPH | C10-C12 Aliphatics |
| C10C12AROM | C10-C12 Aromatics |
| C10C24ALIPH | C10-C24 Aliphatics |
| C10C24AROM | C10-C24 Aromatics |
| ISC10H12 | C10H12 Isomer |
| C11C11N | C11 as n-Undecane |
| ISC11H120 | C11H120 Isomer |
| C12C12N | C12 as n-Dodecane |
| C12C16ALIPH | C12-C16 Aliphatics |
| C12C16AROM | C12-C16 Aromatics |
| C13C13N | C13 as n-Tridecane |
| C14C14N | C14 as n-Tetradecane |
| C15C15N | C15 as n-Pentadecane |
| C16C16N | C16 as n-Hexadecane |
| C16C21ALIPH | C16-C21 Aliphatics |
| C16C21AROM | C16-C21 Aromatics |
| C17C17N | C17 as n-Heptadecane |
| C18C18N | C18 as n-Octadecane |
| CHRYSENEC1 | C1-Chrysenes |
| DBTC1 | C1-Dibenzothiophenes |
| FLAC1PYR | C1-Fluoranthenes/Pyrenes |
| FLC1 | C1-Fluorenes |
| PHANC1A | C1-Phenanthrenes/Anthrace |
| C21C34ALIPH | C21-C34 Aliphatics |
| C21C34AROM | C21-C34 Aromatics |
| C25C36ALIPH | C25-C36 Aliphatics |
| C25C36AROM | C25-C36 Aromatics |
| CHRYSENEC2 | C2-Chrysenes |
| DBTC2 | C2-Dibenzothiophenes |
| FLC2 | C2-Fluorenes |
| NPHC2 | C2-Naphthalenes |
| PHANC2A | C2-Phenanthrenes/Anthrace |
| CHRYSENEC3 | C3-Chrysenes |
| DBTC3 | C3-Dibenzothiophenes |
| FLC3 | C3-Fluorenes |
| NPHC3 | C3-Naphthalenes |
| PHANC3A | C3-Phenanthrenes/Anthrace |
| CHRYSENEC4 | C4-Chrysenes |
| NPHC4 | C4-Naphthalenes |
| PHANC4A | C4-Phenanthrenes/Anthrace |
| C5C16TPH | C5-C16 Total Petroleum Hydrocarbons |
| C5C6ALIPH | C5-C6 Aliphatics |
| C6HEXANE | C6 as n-Hexane |
| C6C10ALIPH | C6-C10 Aliphatics |
| C6C10AROM | C6-C10 Aromatics |
| C6C8ALIPH | C6-C8 Aliphatics |
| C7HEPTANE | C7 as n-Heptane |
| C8C8N | C8 as n-Octane |
| C8C10ALIPH | C8-C10 Aliphatics |
| C8C10AROM | C8-C10 Aromatics |
| ISC8H803 | C8H803 Isomer |
| C9C9N | C9 as n-Nonane |
| CD | Cadmium |
| CAFFEINE | Caffeine |
| CA | Calcium |
| CACO3EQ | Calcium Carbonate Equivalent |
| CAMPHENE | Camphene |
| CAPT | Captafol |
| CAPTAN | Captan |
| SEVIN | Carbaryl |
| CARBAZOLE | Carbazole |
| CRBFN | Carbofuran |
| CO2 | Carbon dioxide |
| CDS | Carbon disulfide |
| CTCL | Carbon tetrachloride |
| ALKC | Carbonate Alkalinity as CACO3 |
| CACO3 | Carbonate as CaCO3 |
| CO3 | Carbonate as CO3 |
| CARBOPHENOTH | Carbophenothion |
| CATION-EX | Cation Exchange Capacity |
| CELLFIBER | Cellulose fiber |
| CE | Cerium |
| CS | Cesium |
| CS-134 | Cesium-134 |
| CS-137 | Cesium-137 |
| COD | Chemical Oxygen Demand |
| CHLORALHY | Chloral hydrate |
| CLBEN | Chloramben |
| CHLORDANE | Chlordane |
| CVP | Chlorfenvinphos |
| CL | Chloride |
| CLHYD | Chlorinated Hydrocarbon |
| CHLORINE | Chlorine, Total residual |
| CHLORMEPHOS | Chlormephos |
| CLACTH | Chloroacetaldehyde |
| CAA | Chloroacetic acid |
| CLAN | Chloroacetonitrile |
| CLBZ | Chlorobenzene |
| CLBZD5 | Chlorobenzene-d5 |
| CLBZLATE | Chlorobenzilate |
| CLEA | Chloroethane |
| TCLME | Chloroform |
| TCLMED | Chloroform-D |
| CLME | Chloromethane |
| CLMME | Chloromethyl methyl ether |
| CHLOROPHYLLA | Chlorophyll a |
| CLPROP | Chloropropylate |
| TERMIL | Chlorothalonil |
| BZLCL | Chlorotoluene |
| CLPYRIFOS | Chlorpyrifos |
| CR | Chromium |
| CR3 | Chromium III |
| CR6 | Chromium, Hexavalent |
| CHRYSENE | Chrysene |
| CHRYSENED12 | Chrysene-d12 |
| CHRYSO | Chrysotile |
| DCE12C | cis-1,2-Dichloroethene |
| DCP13C | cis-1,3-Dichloropropene |
| DCBE14C | cis-1,4-Dichloro-2-butene |
| CLOUDPT | Cloud Point |
| CO | Cobalt |
| CO-60 | Cobalt-60 |
| COLIFORM | Coliform, Total |
| COLOR | Color |
| COLORAPPRNT | Color, Apparent |
| COLORTRUE | Color, True |
| COND | Conductivity |
| CU | Copper |
| CORRPH | Corrosivity as pH |
| CORROS | Corrosivity Toward Steel |
| COUMAPHOS | Coumaphos |
| COUMARIN | Coumarin Dyes |
| MEPHS | Cresols (Methyl Phenols) |
| CROCID | Crocidolite |
| 2BUTENAL | Crotonaldehyde |
| CROTOX | Crotoxyphos |
| CN | Cyanide |
| HCN | Cyanide (as HCN) |
| CYHEXANE | Cyclohexane |
| 50876-31-8 | Cyclohexane, 1,1,3,5-tetramethyl-, trans- |
| 2234-75-5 | Cyclohexane, 1,2,4-trimethyl- |
| 624-29-3 | Cyclohexane, 1,4-dimethyl-, cis- |
| 2207-04-7 | Cyclohexane, 1,4-dimethyl-, trans- |
| 4926-78-7 | Cyclohexane, 1-ethyl-4-methyl-, cis- |
| 6236-88-0 | Cyclohexane, 1-ethyl-4-methyl-, trans- |
| 108-87-2 | Cyclohexane, methyl- |
| 4292-92-6 | Cyclohexane, pentyl- |
| 1678-92-8 | Cyclohexane, propyl- |
| CYHEXPROP | Cyclohexanepropanol |
| CYHEKET | Cyclohexanone |
| 16538-93-5 | Cyclooctane, butyl- |
| CYC5N | Cyclopentane |
| 53771-88-3 | Cyclopentane, 1-methyl-3-(1-methylethyl)- |
| 24642-72-6 | Cydohexane carboxylic acid |
| 2532-58-3 | Cylcopentane, 1,3-dimethyl-, cis- |
| DALAPON | Dalapon |
| DU | Data Unavailable |
| DACTH | DCPA (Dacthal) |
| METABOLITES | DCPA acid metabolites (a) |
| CL10BZ2 | Decachlorobiphenyl |
| FIOBPH | Decafluorobiphenyl |
| FIOTPHPINE | Decafluorotriphenylphosphine |
| DMCPS | Decamethylcyclopentasiloxane |
| 124-18-5 | Decane |
| 334-48-5 | Decanoic acid |
| BHCDELTA | delta-BHC |
| DEMETON | Demeton, -O and -S |
| DEMETONO | Demeton-O |
| DEMETONS | Demeton-S |
| DENSITY | Density |
| DIALLATE | Diallate (cis- or trans-) |
| DIAZ | Diazinon |
| DBAHACR | Dibenz(a,h)acridine |
| DBAJACR | Dibenz(a,j)acridine |
| DBZAEP | Dibenzo(a,e)pyrene |
| DBAHA | Dibenzo(a,h)anthracene |
| DBAHAD14 | Dibenzo(a,h)anthracene-d14 |
| DBZAHP | Dibenzo(a,h)pyrene |
| DBZAIP | Dibenzo(a,i)pyrene |
| DBF | Dibenzofuran |
| DBT | Dibenzothiophene |
| DBCME | Dibromochloromethane |
| DBA | Dibromoethane |
| DBFM | Dibromofluoromethane |
| DBMA | Dibromomethane |
| DBUTYLC | Dibutylchlorendate |
| DBTSN | Dibutyltin |
| DICAMBA | Dicamba |
| DCLN | Dichlone |
| FC12 | Dichlorodifluoromethane |
| FC21 | Dichlorofluoromethane |
| DCMA | Dichloromethane |
| DICHLORVOS | Dichlorovos |
| DCPROP | Dichlorprop |
| DICOFOL | Dicofol |
| BIDRIN | Dicrotophos |
| DCPD | Dicyclopentadiene |
| DIELDRIN | Dieldrin |
| DIESEL2 | Diesel Fuel #2 |
| DRO | Diesel Range Organics |
| DROC10C25 | Diesel Range Organics (C10-C25) |
| DROC10C28 | Diesel Range Organics (C10-C28) |
| DROALIPHATIC | Diesel Range Organics, Aliphatic |
| DROAROMATIC | Diesel Range Organics, Aromatic |
| EE | Diethyl ether |
| DEPH | Diethyl phthalate |
| DEPHD4 | Diethyl phthalate-D4 |
| DESO4 | Diethyl sulfate |
| ET2GLY | Diethylene glycol |
| DES | Diethylstilbestrol |
| DIHYDROSAF | Dihydrosafrole |
| DIOP | Diisooctyl phthalate |
| DIMP | Diisopropylmethylphosphonate |
| DIMETHAT | Dimethoate |
| DMC10N | Dimethyl decane |
| DMDS | Dimethyl disulfide |
| DMPH | Dimethyl phthalate |
| DMMP | Dimethylmethylphosphonate |
| DNBP | Di-n-butyl phthalate |
| DNOCP | Dinocap |
| DNOP | Di-n-octyl phthalate |
| DNOPD4 | Di-n-octyl phthalate-d4 |
| DINOSEB | Dinoseb |
| DIOXATHION | Dioxathion |
| DPAMIDE | Diphenamide |
| DPA | Diphenylamine |
| DPD10 | Diphenyl-d10 |
| DISBLUE14 | Disperse Blue 14 |
| DISBLUE3 | Disperse Blue 3 |
| DISBRN1 | Disperse Brown 1 |
| DISORNG3 | Disperse Orange 3 |
| DISORNG30 | Disperse Orange 30 |
| DISRED1 | Disperse Red 1 |
| DISRED13 | Disperse Red 13 |
| DISRED5 | Disperse Red 5 |
| DISRED60 | Disperse Red 60 |
| DISYEL5 | Disperse Yellow 5 |
| DOC | Dissolved Organic Carbon |
| DS | Dissolved Sulfide |
| DISUL | Disulfoton |
| DITH | Dithiane |
| 629-07-0 | Docosane |
| 112-85-6 | Docosanoic acid |
| 112-40-3 | Dodecane |
| 143-07-7 | Dodecanoic acid |
| 544-85-4 | Dotriacontane |
| DUST | Dust |
| DY | Dysprosium |
| 112-95-8 | Eicosane |
| 506-30-9 | Eicosanoic acid |
| ENDOSULFANA | Endosulfan I |
| ENDOSULFANB | Endosulfan II |
| ENDOSULFANS | Endosulfan sulfate |
| ENDRIN | Endrin |
| ENDRINALD | Endrin aldehyde |
| ENDRINKET | Endrin ketone |
| ENTCOCCUS | Enterococcus |
| EPICLHDRN | Epichlorhydrin |
| EPN | EPN |
| EAH | Erythritol anhydride |
| ECOLI | Escherichia coli |
| C2H6 | Ethane |
| ETHANOL | Ethanol |
| C2H4 | Ethene |
| ETHION | Ethion |
| ETHOPROP | Ethoprop |
| ETACET | Ethyl acetate |
| ECARB | Ethyl carbamate |
| EMETHACRY | Ethyl methacrylate |
| EMSULFN | Ethyl methanesulfonate |
| 54120-62-6 | Ethyl-1,2,4-trimethylbenzene |
| EBZ | Ethylbenzene |
| EBZD10 | Ethylbenzene-D10 |
| ETEGLY | Ethylene glycol |
| ETOX | Ethylene oxide |
| ETRID | Etridiazole |
| EU | Europium |
| EU-152 | Europium-152 |
| EU-154 | Europium-154 |
| EU-155 | Europium-155 |
| ESP | Exchangeable Sodium Percentage |
| PEXT | Extractable Phosphorus |
| KEXT | Extractable Potassium |
| NAEXT | Extractable Sodium |
| FAMPHUR | Famphur |
| FECCOLIFORM | Fecal Coliform, 0.7 Micron Filter |
| FECSTREP | Fecal Streptococcus |
| FENSTHION | Fensulfothion |
| FENTHION | Fenthion |
| FE3 | Ferric Iron |
| FE2 | Ferrous Iron |
| FIBGLASS | Fibrous Glass |
| FLASHPT | Flash Point |
| FLUCHLOR | Fluchloralin |
| FLA | Fluoranthene |
| FLAD10 | Fluoranthene-d10 |
| FL | Fluorene |
| FLD10 | Fluorene-d10 |
| FLBRIGHT236 | Fluorescent Brightener 236 |
| FLBRIGHT61 | Fluorescent Brightener 61 |
| F | Fluoride |
| FAA | Fluoroacetic acid |
| FBZ | Fluorobenzene |
| FREELIQUIDS | Free Liquids |
| BUNKERC | Fuel Oil No. 6 (BUNKER C) |
| GAMMA-GELI | Gamma Spectral Analysis, Ge(Li) |
| BHCGAMMA | gamma-BHC (Lindane) |
| CHLORDANEG | gamma-Chlordane |
| GASOLINE | Gasoline |
| GRO | Gasoline Range Organics |
| GROC6C10 | Gasoline Range Organics (C6-C10) |
| GROALIPHATIC | Gasoline Range Organics, Aliphatic |
| GROAROMATIC | Gasoline Range Organics, Aromatic |
| AU | Gold |
| HF | Hafnium |
| HALIDES | Halides |
| HALOWAX1000 | Halowax 1000 |
| HALOWAX1001 | Halowax 1001 |
| HALOWAX1013 | Halowax 1013 |
| HALOWAX1014 | Halowax 1014 |
| HALOWAX1051 | Halowax 1051 |
| HALOWAX1099 | Halowax 1099 |
| HARD | Hardness (as CaCO3) |
| HARDNC | Hardness (as CaCO3), Noncarbonate |
| HARDC | Hardness (as CO3), Carbonate |
| HOIL | Heavy Oil |
| 629-94-7 | Heneicosane |
| 2363-71-5 | Heneicosanoic acid |
| 630-04-6 | Hentriacontane |
| HEPTACHLOR | Heptachlor |
| HEPT-EPOX | Heptachlor epoxide |
| HEPT-EPOXA | Heptachlor epoxide A |
| 593-49-7 | Heptacosane |
| 629-78-7 | Heptadecane |
| 506-12-7 | Heptadecanoic acid |
| 142-82-5 | Heptane |
| HPC | Heterotrophic Plate Count |
| HXBRBZ | Hexabromobenzene |
| HCLBZ | Hexachlorobenzene |
| PCB0006 | Hexachlorobiphenyls |
| HCBU | Hexachlorobutadiene |
| HXCLCYHX | Hexachlorocyclohexane |
| HCCP | Hexachlorocyclopentadiene |
| HCLEA | Hexachloroethane |
| HXCP | Hexachlorophene |
| HCPR | Hexachloropropene |
| 630-01-3 | Hexacosane |
| 544-76-3 | Hexadecane |
| PALMA | Hexadecanoic acid |
| 111-06-8 | Hexadecanoic acid, butyl ester |
| H2M2P | Hexafluoro-2-methyl-2-propanol |
| HFP2 | Hexafluoro-2-propanol |
| RDX | Hexahydro-1,3,5-trinitro-1,3,5-triazine |
| HMPA | Hexamethylphosphoramide |
| HXALD | Hexanal |
| HEXANE | Hexane |
| 630-06-8 | Hexatriacontane |
| HHV | High Heat Value |
| ODB | Hydrocarbon Degrading Bacteria |
| HFACID | Hydrofluoric acid |
| HYDROQUIN | Hydroquinone |
| OH | Hydroxide |
| ALKH | Hydroxide Alkalinity as CACO3 |
| IGNITB | Ignitability |
| INDENE | Indene |
| INP123 | Indeno(1,2,3-cd)pyrene |
| I | Iodide (as I) |
| I-129 | Iodine-129 |
| IONBAL | Ion Balance |
| IR | Iridium |
| FE | Iron |
| ISOBUTANE | Isobutane |
| ISOBTOH | Isobutanol |
| ISODRIN | Isodrin |
| ISOP | Isophorone |
| ISOPROH | Isopropanol |
| ISOPRE | Isopropyl ether |
| IPBZ | Isopropylbenzene |
| IMPA | Isopropylmethyl phosphonic acid |
| ISOSAFR | Isosafrole |
| JETFUEL | Jet Fuel |
| JP4 | Jet Fuel #4 (JP4) |
| JETA | Jet Fuel as Jet A |
| KEP | Kepone |
| KEROSENE | Kerosene |
| LAI | Langelier Index |
| LAI140 | Langlier Index at 140 degrees F |
| LAI40 | Langlier Index at 40 degrees F |
| LA | Lanthanum |
| PB | Lead |
| PB-210 | Lead-210 |
| PB-212 | Lead-212 |
| PB-214 | Lead-214 |
| LEPTO | Leptophos |
| LI | Lithium |
| LU | Lutetium |
| MEPH1314 | m,p-Cresol |
| XYLMP | m,p-Xylene (Sum of Isomers) |
| MG | Magnesium |
| MGCAC03 | Magnesium as CaCO3 |
| MALA | Malathion |
| MALANH | Maleic anhydride |
| MALNTRL | Malononitrile |
| MN | Manganese |
| MCPA | MCPA |
| MCPP | MCPP |
| MEDIUM | Medium |
| HG | Mercury |
| MERPHOS | Merphos |
| MSNL | Mestranol |
| METHACRN | Methacrylonitrile |
| CH4 | Methane |
| MEOH | Methanol |
| MTPYRLN | Methapyrilene |
| METHIOCARB | Methiocarb |
| METHOMYL | Methomyl |
| MTXYCL | Methoxychlor |
| MACRYLATE | Methyl acrylate |
| MARACHIDATE | Methyl arachidate |
| CPENTANEME | Methyl cyclopentane |
| MEDS | Methyl disulfide |
| IME | Methyl iodide |
| MMSULFN | Methyl methanesulfonate |
| MPA | Methyl phosphonic dichloride |
| SUBMALKANE | Methyl substituted alkane |
| SUBMCHX | Methyl substituted cyclohexane |
| TETRYL | Methyl-2,4,6-trinitrophenylnitramine |
| MBAS | Methylene Blue Active Substances |
| MTLNCL | Methylene chloride |
| MMETHACRY | Methylmethacrylate |
| TBUTMEE | Methyl-t-butyl ether |
| METOCHLOR | Metolachlor |
| METRIBUZ | Metribuzin |
| MEVINPHOS | Mevinphos |
| MEXACARBATE | Mexacarbate |
| MINSPRT | Mineral Spirits |
| MIREX | Mirex |
| MO | Molybdenum |
| MONOCROPHOS | Monocrotophos |
| MOIL | Motor Oils |
| PHENM | m-Terphenyl |
| XYLM | m-Xylene |
| NALED | Naled |
| 29949-27-7 | n-Amylcyclohexane |
| NPHD | Naphtha distillate |
| NAPH | Naphthalene |
| TETRALIN | Naphthalene, 1,2,3,4-tetrahydro- |
| 2131-42-2 | Naphthalene, 1,4,6-trimethyl- |
| 2245-38-7 | Naphthalene, 1,6,7-trimethyl- |
| 581-40-8 | Naphthalene, 2,3-dimethyl- |
| 582-16-1 | Naphthalene, 2,7-dimethyl- |
| DHNAPH | Naphthalene, decahydro- |
| DHNAPHT | Naphthalene, decahydro-, trans- |
| NAPHD8 | Naphthalene-d8 |
| 2958-76-1 | Napthalene, decahydro-2-methyl- |
| BTOH | n-Butanol |
| BTBZN | n-Butylbenzene |
| HEM | n-Hexane Extractable Material |
| NI | Nickel |
| NICOTINE | Nicotine |
| NB | Niobium |
| NO3 | Nitrate |
| NO2BZ | Nitrobenzene |
| NO2BZD5 | Nitrobenzene-d5 |
| NITROFEN | Nitrofen |
| N | Nitrogen |
| NH3N | Nitrogen, Ammonia (as N) |
| KN | Nitrogen, Kjeldahl, Total |
| NO3N | Nitrogen, Nitrate (as N) |
| NO3NO2N | Nitrogen, Nitrate-Nitrite |
| NO2N | Nitrogen, Nitrite |
| NTG | Nitroglycerin |
| NNSE | n-Nitrosodiethylamine |
| NNSM | n-Nitrosodimethylamine |
| NNSBU | n-Nitroso-di-n-butylamine |
| NNSPR | n-Nitrosodi-n-propylamine |
| NNSPH | n-Nitrosodiphenylamine |
| NNSME | n-Nitrosomethylethylamine |
| NNSMRPH | n-Nitrosomorpholine |
| NNSPPRD | n-Nitrosopiperidine |
| NNSPYRL | n-Nitrosopyrrolidine |
| 630-32-8 | Nonacosane |
| 629-92-5 | Nonadecane |
| 646-30-0 | Nonadecanoic acid |
| NONANE | Nonane |
| 26DIMENONANE | Nonane, 2,6-dimethyl- |
| 591-04-6 | Nonane, 4-methyl- |
| NMOC | Non-Methane Organic Compounds |
| PHCDUNK | Non-PHC as Diesel |
| PHCFOUNK | Non-PHC as Fuel Oils |
| PHCGUNK | Non-PHC as Gasoline |
| PHCHFOUNK | Non-PHC as Heavy Fuel Oils |
| PHCLUBUNK | Non-PHC as Lube Oil |
| PROH | n-Propanol |
| PROPYL | n-Propylamine |
| PBZN | n-Propylbenzene |
| C30N | n-Triacontane |
| TEPTH | o,o,o-Triethyl phosphorothioate |
| ANSD2 | o-Anisidine |
| CLBZALDO | o-Chlorobenzaldehyde |
| OCDF | Octachlorodibenzofuran |
| OCDFC13 | Octachlorodibenzofuran-C13 |
| OCDD | Octachlorodibenzo-p-dioxin |
| OCDDC13 | Octachlorodibenzo-p-dioxin-C13 |
| 630-02-4 | Octacosane |
| 593-45-3 | Octadecane |
| OCDNA | Octadecanoic acid |
| 123-95-5 | Octadecanoic acid, butyl ester |
| OFBZME | Octafluorotoluene |
| HMX | Octahydro-1,3,5,7-tetranitro-1,3,5,7-tetrazocine |
| OMPA | Octamethyl pyprophosphoramide |
| OMCYTSX | Octamethylcyclotetrasiloxane |
| 2216-33-3 | Octane, 3-methyl- |
| ODOR | Odor |
| OILGREASE | Oil and Grease |
| OILM | Oil, Misc. |
| OC | Organic Carbon |
| PBO | Organic Lead |
| OS | Osmium |
| PHENO | o-Terphenyl |
| TLDNO | o-Toluidine |
| OXAMYL | Oxamyl |
| OXAT | Oxathiane |
| REDOX | Oxidation-Reduction Potential |
| OXYGEN | Oxygen |
| DO | Oxygen, Dissolved |
| XYLO | o-Xylene |
| PFTEST | Paint Filter Test |
| PL | Palladium |
| PARALD | Paraldehyde |
| PARAE | Parathion ethyl |
| PARAM | Parathion methyl |
| PBZQUINONE | p-Benzoquinone |
| PCB1016 | PCB-1016 (Aroclor 1016) |
| PCB1221 | PCB-1221 (Aroclor 1221) |
| PCB1232 | PCB-1232 (Aroclor 1232) |
| PCB1242 | PCB-1242 (Aroclor 1242) |
| PCB1242/1016 | PCB-1242/1016 (Aroclor 1242/1016) |
| PCB1248 | PCB-1248 (Aroclor 1248) |
| PCB1254 | PCB-1254 (Aroclor 1254) |
| PCB1260 | PCB-1260 (Aroclor 1260) |
| PCB1262 | PCB-1262 (Aroclor 1262) |
| PCB1268 | PCB-1268 (Aroclor 1268) |
| CPMS | p-Chloromethyl sulfide |
| CPMSO2 | p-Chloromethyl sulfone |
| CPMSO | p-Chloromethyl sulfoxide |
| PCNB | PCNB (Quintozene) |
| CRESP | p-Cresidine |
| PDMAABZ | p-Dimethylaminoazobenzene |
| PECLBZ | Pentachlorobenzene |
| PCLEA | Pentachloroethane |
| PECLNO2BZ | Pentachloronitrobenzene |
| PCP | Pentachlorophenol |
| 629-99-2 | Pentacosane |
| C25N | Pentacosane |
| 629-62-9 | Pentadecane |
| 1719-03-5 | Pentadecane, 2,6,10,14-tetramethyl- |
| 1002-84-2 | Pentadecanoic acid |
| PETN | Pentaerythritol tetranitrate |
| PFBZ | Pentafluorobenzene |
| PFP | Pentafluorophenol |
| 107-83-5 | Pentane, 2-methyl- |
| 630-07-9 | Pentatriacontane |
| MOIST | Percent Moisture |
| PCATE | Perchlorate |
| PERMETHRIN | Permethrin |
| PERTHANE | Perthane |
| PERY | Perylene |
| PERYD12 | Perylene-d12 |
| PHC | Petroleum Hydrocarbons (TPH) |
| PH | pH |
| PHCD | PHC as Diesel Fuel (TPHD) |
| PHCFO | PHC as Fuel Oils |
| PHCGC7C12 | PHC as Gasoline (C7-C12) |
| PHCG | PHC as Gasoline (TPHG) |
| PHCHFO | PHC as Heavy Fuel Oils |
| PHCIO | PHC as Insulating Oil |
| PHCJP4 | PHC as JP-4 |
| PHCK | PHC as Kerosene |
| PHCLUB | PHC as Lube Oil |
| PHNACTN | Phenacetin |
| PHAN | Phenanthrene |
| 3674-66-6 | Phenanthrene, 2,5-dimethyl- |
| 832-71-3 | Phenanthrene, 3-methyl- |
| PHAND10 | Phenanthrene-d10 |
| PHENOBAL | Phenobarbital |
| PHENOL | Phenol |
| PHD5 | Phenol-d5 |
| PHENOLD6 | Phenol-d6 |
| TOTPHEN | Phenolics, Total recoverable |
| PHORATE | Phorate |
| PHOSAL | Phosalone |
| PHOSMET | Phosmet |
| PHOSPHAM | Phosphamidon |
| PO4RS | Phosphorus, Reactive soluble |
| P | Phosphorus, Total (as P) |
| PORG | Phosphorus, Total Organic (as P) |
| PORTHO | Phosphorus, Total Orthophosphate (as P) |
| PO4 | Phosphorus, Total Orthophosphate (as PO4) |
| PHTHL | Phthalates |
| PHANHY | Phthalic anhydride |
| PICLORAM | Picloram |
| SULFX | Piperonyl sulfoxide |
| PRIMICID | Pirimphos-ethyl |
| PT | Platinum |
| PU-238 | Plutonium-238 |
| PU239240 | Plutonium-239/240 |
| K | Potassium |
| K-40 | Potassium-40 |
| POURPT | Pour Point |
| PRONAMD | Pronamide |
| PROPACHLOR | Propachlor |
| C3H8 | Propane |
| PACN | Propionitrile |
| PROPENE | Propylene |
| PROPGLY | Propylene glycol |
| PROPYCIL | Propylthiouracil |
| PA-231 | Protactinium-231 |
| PA-234 | Protactinium-234 |
| PHENP | p-Terphenyl |
| 89-82-7 | Pulegone |
| XYLP | p-Xylene |
| PYR | Pyrene |
| PYRD10 | Pyrene-d10 |
| PYRDN | Pyridine |
| QUINO | Quinoline |
| RA | Radium |
| RA-223 | Radium-223 |
| RA-224 | Radium-224 |
| RA-226 | Radium-226 |
| RA226228 | Radium-226 and Radium-228 |
| RA-228 | Radium-228 |
| RECN | Reactive Cyanide |
| SULFID-R | Reactive Sulfide |
| RRO | Residual Range Organics |
| RROC25C36 | Residual Range Organics (C25-C36) |
| RROC28C40 | Residual Range Organics (C28-C40) |
| RROALIPHATIC | Residual Range Organics, Aliphatic |
| RROAROMATIC | Residual Range Organics, Aromatic |
| 13BZDIOL | Resorcinol |
| RH | Rhodium |
| RONNEL | Ronnel |
| RB | Rubidium |
| RU | Ruthenium |
| SAFROLE | Safrole |
| SALINITY | Salinity |
| SALMONELLA | Salmonella |
| SM | Samarium |
| SCANDIUM | Scandium |
| BTBZS | sec-Butylbenzene |
| SE | Selenium |
| SETMAT | Settleable Matter |
| SIEVE10F | Sieve No. 10, Fractional Percent Retained |
| SIEVE10 | Sieve No. 10, Percent Passing |
| SIEVE140F | Sieve No. 140, Fractional Percent Retained |
| SIEVE140 | Sieve No. 140, Percent Passing |
| SIEVE20F | Sieve No. 20, Fractional Percent Retained |
| SIEVE20 | Sieve No. 20, Percent Passing |
| SIEVE200F | Sieve No. 200, Fractional Percent Retained |
| SIEVE200 | Sieve No. 200, Percent Passing |
| SIEVE230F | Sieve No. 230, Fractional Percent Retained |
| SIEVE230 | Sieve No. 230, Percent Passing |
| SIEVE4F | Sieve No. 4, Fractional Percent Retained |
| SIEVE4 | Sieve No. 4, Percent Passing |
| SIEVE40F | Sieve No. 40, Fractional Percent Retained |
| SIEVE40 | Sieve No. 40, Percent Passing |
| SIEVE60F | Sieve No. 60, Fractional Percent Retained |
| SIEVE60 | Sieve No. 60, Percent Passing |
| SIEVE10PHIF | Sieve, 10 Phi, Fractional Percent Retained |
| SIEVE10PHI | Sieve, 10 Phi, Percent Passing |
| SIEVE4PHIF | Sieve, 4 Phi, Fractional Percent Retained |
| SIEVE4PHI | Sieve, 4 Phi, Percent Passing |
| SIEVE5PHIF | Sieve, 5 Phi, Fractional Percent Retained |
| SIEVE5PHI | Sieve, 5 Phi, Percent Passing |
| SIEVE6PHIF | Sieve, 6 Phi, Fractional Percent Retained |
| SIEVE6PHI | Sieve, 6 Phi, Percent Passing |
| SIEVE7PHIF | Sieve, 7 Phi, Fractional Percent Retained |
| SIEVE7PHI | Sieve, 7 Phi, Percent Passing |
| SIEVE8PHIF | Sieve, 8 Phi, Fractional Percent Retained |
| SIEVE8PHI | Sieve, 8 Phi, Percent Passing |
| SIEVE9PHIF | Sieve, 9 Phi, Fractional Percent Retained |
| SIEVE9PHI | Sieve, 9 Phi, Percent Passing |
| SIL | Silica |
| SGTHEM | Silica Gel Treated Hexane Extractable Material |
| SILICATE | Silicate |
| SI | Silicon |
| SILSI | Silicon, as Silica |
| AG | Silver |
| SIMAZINE | Simazine |
| SEM | Simultaneously Extracted Metals (Cd, Cu, Pb, Hg, Ni, and Zn) |
| NA | Sodium |
| SAR | Sodium Absorption Ratio |
| NACLO3 | Sodium chlorate |
| SOLID | Solids, Percent |
| SOLIDVOA | Solids, Percent Volatile Components |
| SOLVRED23 | Solvent Red 23 |
| SOLVRED3 | Solvent Red 3 |
| SC | Specific Conductance |
| SG | Specific Gravity |
| 111-01-3 | Squalane |
| 7683-64-9 | Squalene |
| OTPH-D | State of Oregon Diesel Range Organics |
| OTPH-G | State of Oregon Gasoline Range Organics |
| WTPH-D | State of Washington, Diesel Range Organics |
| WTPH-G | State of Washington, Gasoline Range Organics |
| STROBANE | Strobane |
| SR | Strontium |
| SR-90 | Strontium-90 |
| STRYCHNINE | Strychnine |
| STY | Styrene |
| SUBACEAC | Substituted Acetic acid |
| SUBALKANE | Substituted Alkane |
| SUBALKENE | Substituted Alkene |
| SUBBZALD | Substituted Benzaldehyde |
| SUBBZAMIDE | Substituted Benzamide |
| SUBBZ | Substituted Benzene |
| SUBBEN1 | Substituted Benzene #1 |
| SUBBEN2 | Substituted Benzene #2 |
| SUBBEN3 | Substituted Benzene #3 |
| SUBBEN4 | Substituted Benzene #4 |
| SUBBEN5 | Substituted Benzene #5 |
| SUBBZPA | Substituted Benzenepropanoic acid |
| SUBBZSAMIDE | Substituted Benzenesulfonamide |
| SUBBZACID | Substituted Benzoic acid |
| SUBCHYD | Substituted Cyclic hydrocarbon |
| SUBCBT | Substituted Cyclobutane |
| SUBCHXN | Substituted Cyclohexane |
| SUBCPT | Substituted Cyclopentane |
| SUBCPTO | Substituted Cyclopentanone |
| SUB2MOTENE | Substituted Dimethyl octene |
| SUBDIOXIN | Substituted Dioxin |
| SUBDIOXLANE | Substituted Dioxolane Compound |
| SUBDS | Substituted Disulfide |
| SUBETHANOL | Substituted Ethanol |
| SUBETHONE | Substituted Ethanone |
| SUBHEPTANONE | Substituted Heptanone |
| SUBHDIOIC | Substituted Hexanedioic acid |
| SUBINDENE | Substituted Indene |
| SUBIND1 | Substituted Indene #1 |
| SUBIND2 | Substituted Indene #2 |
| SUBIND3 | Substituted Indene #3 |
| SUBIND4 | Substituted Indene #4 |
| SUBIND5 | Substituted Indene #5 |
| SUBINDENONE | Substituted Indenone |
| SUBNAPH | Substituted Naphthalene |
| SUBOCTENE | Substituted Octene |
| SUBOXIRANE | Substituted Oxirane |
| SUBPAH | Substituted PAH |
| SUBPLENE | Substituted Pentalene |
| SUBPENTENE | Substituted Pentene |
| SUBPHAN | Substituted Phenanthrene |
| SUBPHENOL | Substituted Phenol |
| SUBH3PO4 | Substituted Phosphonic acid |
| SUB2MEPA3 | Substituted Propanoic acid |
| SUBPROPANOL | Substituted Propanol |
| SUBPYR | Substituted Pyrene |
| SULFAL | Sulfallate |
| SO4 | Sulfate |
| SRB | Sulfate Reducing Bacteria |
| S | Sulfide |
| H2S | Sulfide (as H2S) |
| SO3 | Sulfite |
| SULFOTEP | Sulfotep |
| SU | Sulfur |
| SURFACT | Surfactants |
| SS | Suspended Solids |
| SYNTHETIC | Synthetic |
| TAL | Tannin and Lignin |
| TA | Tantalum |
| BTALT | t-Butyl alcohol |
| TEMP | Temperature |
| TB | Terbium |
| TERBUFOS | Terbufos |
| PHEND14 | Terphenyl-d14 |
| TAME | tert-Amyl methyl ether |
| ETBE | tert-Butyl ethyl ether |
| BTBZT | tert-Butylbenzene |
| TEBTSN | Tetrabutyltin |
| PCE | Tetrachloroethene |
| TECMXYL | Tetrachlorometaxylene |
| TECLPHS | Tetrachlorophenols |
| STIROFOS | Tetrachlorvinphos (Stirophos) |
| 646-31-1 | Tetracosane |
| 557-59-5 | Tetracosanoic acid |
| 629-59-4 | Tetradecane |
| 544-63-8 | Tetradecanoic acid |
| PBTE | Tetraethyl lead |
| PBTED20 | Tetraethyl lead-d20 |
| TEPP | Tetraethyl pyrophosphate |
| TEGLY | Tetraethylene glycol |
| THF | Tetrahydrofuran |
| THNAPH | Tetrahydronaphthol |
| TMGLY | Tetramethylene glycol |
| TTCT | Tetratetracontane |
| 14167-59-0 | Tetratriacontane |
| TL | Thallium |
| TDGCL | Thiodiglycol |
| THIOFANOX | Thiofanox |
| ZINOPHOS | Thionazine |
| BZS | Thiophenol (Benzenethiol) |
| TH | Thorium |
| TIME | Time |
| SN | Tin |
| TI | Titanium |
| TOKUTHION | Tokuthion (Prothiofos) |
| BZME | Toluene |
| TDI | Toluene diisocyanate |
| BZMED8 | Toluene-d8 |
| TOTBACTERIA | Total Bacteria |
| BATOT | Total Barium |
| TOTCHLRN | Total Chlorine |
| TDS | Total Dissolved Solids |
| TFS | Total Fixed Solids |
| TX | Total Halides |
| TOTX | Total Halogens |
| HPCDF | Total Heptachlorodibenzofurans (HpCDF) |
| HPCDD | Total Heptachlorodibenzo-p-dioxins (HpCDD) |
| HXCDF | Total Hexachlorodibenzofurans (HxCDF) |
| HXCDD | Total Hexachlorodibenzo-p-dioxins (HxCDD) |
| TLIPID | Total Lipids |
| TOC | Total Organic Carbon (TOC) |
| TOX | Total Organic Halides (TOX) |
| PECBPH | Total Pentachlorinatedbiphenyls |
| PECDF | Total Pentachlorodibenzofurans (PeCDF) |
| PECDD | Total Pentachlorodibenzo-p-dioxin (PeCDD) |
| TOTPCB | Total Polychlorinatedbiphenyls |
| TSO | Total Solids |
| SRTOT | Total Strontium |
| TCDF | Total Tetrachlorodibenzofurans (TCDF) |
| TCDD | Total Tetrachlorodibenzo-p-dioxins (TCDD) |
| THM | Total Trihalomethanes |
| TVO | Total Volatile Organics |
| TVS | Total Volatile Solids |
| TOXAP | Toxaphene |
| DCE12T | trans-1,2-Dichloroethene |
| DCP13T | trans-1,3-Dichloropropene |
| DCBE14T | trans-1,4-Dichloro-2-butene |
| NCT | trans-Nonachlor |
| TBP | Tributyl phosphate |
| TBTSN | Tributyltin |
| TRICHLORFON | Trichlorfon |
| TCE | Trichloroethene |
| FC11 | Trichlorofluoromethane |
| CL3NATE | Trichloronate |
| TRICLPHS | Trichlorophenols |
| TCPR | Trichloropropane |
| 638-68-6 | Tricontane |
| 638-67-5 | Tricosane |
| 629-50-5 | Tridecane |
| 7METDECANE | Tridecane, 7-methyl- |
| 638-53-9 | Tridecanoic acid |
| TEP | Triethyl phosphate |
| ET3GLY | Triethylene glycol |
| TFBZME | Trifluorotoluene |
| TRIFLURALIN | Trifluralin |
| TMCYHX | Trimethyl cyclohexane |
| TMEHX | Trimethyl hexane |
| TMEP | Trimethyl phosphate |
| TPTSN | Tripentyltin |
| TPHP | Triphenyl phosphate |
| TPH | Triphenylene |
| TPROPSN | Tripropyltin |
| TT4P | Tri-p-tolyl phosphate |
| T23P | Tris(2,3-dibromopropyl)phosphate |
| H-3 | Tritium (Hydrogen 3) |
| 630-05-7 | Tritriacontane |
| W | Tungsten |
| TURB | Turbidity |
| 1120-21-4 | Undecane |
| 17301-27-8 | Undecane, 1,10-dimethyl- |
| 17312-81-1 | Undecane, 3,5-dimethyl- |
| 112-37-8 | Undecanoic acid |
| OTHERS | Unidentified Light- and/or Medium-Weight Fuels |
| UNK | Unknown |
| UNK1 | Unknown #1 |
| UNK10 | Unknown #10 |
| UNK11 | Unknown #11 |
| UNK12 | Unknown #12 |
| UNK13 | Unknown #13 |
| UNK14 | Unknown #14 |
| UNK15 | Unknown #15 |
| UNK16 | Unknown #16 |
| UNK17 | Unknown #17 |
| UNK18 | Unknown #18 |
| UNK19 | Unknown #19 |
| UNK2 | Unknown #2 |
| UNK20 | Unknown #20 |
| UNK3 | Unknown #3 |
| UNK4 | Unknown #4 |
| UNK5 | Unknown #5 |
| UNK6 | Unknown #6 |
| UNK7 | Unknown #7 |
| UNK8 | Unknown #8 |
| UNK9 | Unknown #9 |
| UNKALCOHOL | Unknown Alcohol |
| UNKALC1 | Unknown Alcohol #1 |
| UNKALC2 | Unknown Alcohol #2 |
| UNKALC3 | Unknown Alcohol #3 |
| UNKALC4 | Unknown Alcohol #4 |
| UNKALC5 | Unknown Alcohol #5 |
| UNKALDEHYDE | Unknown Aldehyde |
| UNKALD1 | Unknown Aldehyde #1 |
| UNALD10 | Unknown Aldehyde #10 |
| UNKALD2 | Unknown Aldehyde #2 |
| UNKALD3 | Unknown Aldehyde #3 |
| UNKALD4 | Unknown Aldehyde #4 |
| UNKALD5 | Unknown Aldehyde #5 |
| UNKALD6 | Unknown Aldehyde #6 |
| UNKALD7 | Unknown Aldehyde #7 |
| UNALD8 | Unknown Aldehyde #8 |
| UNALD9 | Unknown Aldehyde #9 |
| UNKALIPHY | Unknown Aliphatic hydrocarbon |
| UNKALH1 | Unknown Aliphatic hydrocarbon #1 |
| UNKALH2 | Unknown Aliphatic hydrocarbon #2 |
| UNKALH3 | Unknown Aliphatic hydrocarbon #3 |
| UNKALH4 | Unknown Aliphatic hydrocarbon #4 |
| UNKALH5 | Unknown Aliphatic hydrocarbon #5 |
| UNKALKANE | Unknown Alkane |
| UNKALKANE1 | Unknown Alkane #1 |
| UNKALKANE10 | Unknown Alkane #10 |
| UNKALKANE11 | Unknown Alkane #11 |
| UNKALKANE12 | Unknown Alkane #12 |
| UNKALKANE13 | Unknown Alkane #13 |
| UNKALKANE14 | Unknown Alkane #14 |
| UNKALKANE15 | Unknown Alkane #15 |
| UNKALKANE16 | Unknown Alkane #16 |
| UNKALKANE17 | Unknown Alkane #17 |
| UNKALKANE18 | Unknown Alkane #18 |
| UNKALKANE19 | Unknown Alkane #19 |
| UNKALKANE2 | Unknown Alkane #2 |
| UNKALKANE20 | Unknown Alkane #20 |
| UNKALKANE3 | Unknown Alkane #3 |
| UNKALKANE4 | Unknown Alkane #4 |
| UNKALKANE5 | Unknown Alkane #5 |
| UNKALKANE6 | Unknown Alkane #6 |
| UNKALKANE7 | Unknown Alkane #7 |
| UNKALKANE8 | Unknown Alkane #8 |
| UNKALKANE9 | Unknown Alkane #9 |
| UNKALKON1 | Unknown Alkanone #1 |
| UNKALKENE | Unknown Alkene |
| UNKALK1 | Unknown Alkene #1 |
| UNKALK10 | Unknown Alkene #10 |
| UNKALK2 | Unknown Alkene #2 |
| UNKALK3 | Unknown Alkene #3 |
| UNKALK4 | Unknown Alkene #4 |
| UNKALK5 | Unknown Alkene #5 |
| UNKALK6 | Unknown Alkene #6 |
| UNKALK7 | Unknown Alkene #7 |
| UNKALK8 | Unknown Alkene #8 |
| UNKALK9 | Unknown Alkene #9 |
| UNKALKBZ | Unknown Alkyl benzene |
| UNKALKBZ1 | Unknown Alkyl benzene #1 |
| UNKALKBZ10 | Unknown Alkyl benzene #10 |
| UNKALKBZ11 | Unknown Alkyl benzene #11 |
| UNKALKBZ12 | Unknown Alkyl benzene #12 |
| UNKALKBZ13 | Unknown Alkyl benzene #13 |
| UNKALKBZ14 | Unknown Alkyl benzene #14 |
| UNKALKBZ15 | Unknown Alkyl benzene #15 |
| UNKALKBZ16 | Unknown Alkyl benzene #16 |
| UNKALKBZ2 | Unknown Alkyl benzene #2 |
| UNKALKBZ3 | Unknown Alkyl benzene #3 |
| UNKALKBZ4 | Unknown Alkyl benzene #4 |
| UNKALKBZ5 | Unknown Alkyl benzene #5 |
| UNKALKBZ6 | Unknown Alkyl benzene #6 |
| UNKALKBZ7 | Unknown Alkyl benzene #7 |
| UNKALKBZ8 | Unknown Alkyl benzene #8 |
| UNKALKBZ9 | Unknown Alkyl benzene #9 |
| UNKALKSUBBZ | Unknown Alkyl substituted benzene |
| UNKALKYNE | Unknown Alkyne |
| UNKAMIDE | Unknown Amide |
| UNKAMINE | Unknown Amine |
| UNKARO | Unknown Aromatic |
| UNKAH | Unknown Aromatic hydrocarbon |
| UNKARH1 | Unknown Aromatic hydrocarbon #1 |
| UNKARH2 | Unknown Aromatic hydrocarbon #2 |
| UNKARH3 | Unknown Aromatic hydrocarbon #3 |
| UNKARH4 | Unknown Aromatic hydrocarbon #4 |
| UNKARH5 | Unknown Aromatic hydrocarbon #5 |
| UNKAROKET | Unknown Aromatic ketone |
| UNKBZALD | Unknown Benzaldehyde |
| UNKBICYCLIC | Unknown Bicyclic |
| UNKBALK | Unknown Branched alkane |
| UNKBALK1 | Unknown Branched alkane #1 |
| UNKBALK2 | Unknown Branched alkane #2 |
| UNKBALK3 | Unknown Branched alkane #3 |
| UNKBALK4 | Unknown Branched alkane #4 |
| UNKBALK5 | Unknown Branched alkane #5 |
| UNKBALKANOL | Unknown Branched alkanol |
| UNKBALKENE | Unknown Branched alkene |
| UNKALKENOL | Unknown Branched alkenol |
| UNKBCARBA | Unknown Branched carboxylic acid |
| UNKBCYALK | Unknown Branched cycloalkane |
| UNKBKETONE | Unknown Branched ketone |
| UNKBNAPH | Unknown Branched naphthalene |
| UNKBPAH | Unknown Branched PAH |
| UNKBRTRIENOL | Unknown Branched trienol |
| UNKCARA1 | Unknown Carboxcylic acid #1 |
| UNKCARA2 | Unknown Carboxcylic acid #2 |
| UNKCARA3 | Unknown Carboxcylic acid #3 |
| UNKCARA4 | Unknown Carboxcylic acid #4 |
| UNKCARA5 | Unknown Carboxcylic acid #5 |
| UNKCARBOXA | Unknown Carboxylic acid |
| UNKCARBAEST | Unknown Carboxylic acid ester |
| UNKCPYRDN | Unknown Chlorinated pyridine |
| UNKCALKANE | Unknown Cyclic alkane |
| UNKCYAL1 | Unknown Cyclic alkane #1 |
| UNKCYAL10 | Unknown Cyclic alkane #10 |
| UNKCYAL2 | Unknown Cyclic alkane #2 |
| UNKCYAL3 | Unknown Cyclic alkane #3 |
| UNKCYAL4 | Unknown Cyclic alkane #4 |
| UNKCYAL5 | Unknown Cyclic alkane #5 |
| UNKCYAL6 | Unknown Cyclic alkane #6 |
| UNKCYAL7 | Unknown Cyclic alkane #7 |
| UNKCYAL8 | Unknown Cyclic alkane #8 |
| UNKCYAL9 | Unknown Cyclic alkane #9 |
| UNKCHYD | Unknown Cyclic hydrocarbon |
| UNKCYH1 | Unknown Cyclic hydrocarbon #1 |
| UNKCYH2 | Unknown Cyclic hydrocarbon #2 |
| UNKCYH3 | Unknown Cyclic hydrocarbon #3 |
| UNKCYH4 | Unknown Cyclic hydrocarbon #4 |
| UNKCYH5 | Unknown Cyclic hydrocarbon #5 |
| UNKCKETONE | Unknown Cyclic ketone |
| UNKCYHEX | Unknown Cyclohexane |
| UNKCYHEX1 | Unknown Cyclohexane #1 |
| UNKCYHEX2 | Unknown Cyclohexane #2 |
| UNKCYKET1 | Unknown Cycloketone #1 |
| UNKCYKET2 | Unknown Cycloketone #2 |
| UNKCYKET3 | Unknown Cycloketone #3 |
| UNKCYKET4 | Unknown Cycloketone #4 |
| UNKCYKET5 | Unknown Cycloketone #5 |
| UNKESTER | Unknown Ester |
| UNKESTER1 | Unknown Ester #1 |
| UNKESTER2 | Unknown Ester #2 |
| UNKESTER3 | Unknown Ester #3 |
| UNKESTER4 | Unknown Ester #4 |
| UNKESTER5 | Unknown Ester #5 |
| UNKFATACID | Unknown Fatty acid |
| UNKHOPANE | Unknown Hopane |
| UNKHOPANE1 | Unknown Hopane #1 |
| UNKHOPANE2 | Unknown Hopane #2 |
| UNKHOPANE3 | Unknown Hopane #3 |
| UNKHYD | Unknown Hydrocarbon |
| UNKHYD1 | Unknown Hydrocarbon #1 |
| UNKHYD10 | Unknown Hydrocarbon #10 |
| UNKHYD2 | Unknown Hydrocarbon #2 |
| UNKHYD3 | Unknown Hydrocarbon #3 |
| UNKHYD4 | Unknown Hydrocarbon #4 |
| UNKHYD5 | Unknown Hydrocarbon #5 |
| UNKHYD6 | Unknown Hydrocarbon #6 |
| UNKHYD7 | Unknown Hydrocarbon #7 |
| UNKHYD8 | Unknown Hydrocarbon #8 |
| UNKHYD9 | Unknown Hydrocarbon #9 |
| UNKINDOLE | Unknown Indole |
| UNKKETONE | Unknown Ketone |
| UNKKET1 | Unknown Ketone #1 |
| UNKKET2 | Unknown Ketone #2 |
| UNKKET3 | Unknown Ketone #3 |
| UNKKET4 | Unknown Ketone #4 |
| UNKKET5 | Unknown Ketone #5 |
| UNKMS1 | Unknown Methylated siloxane #1 |
| UNKMS2 | Unknown Methylated siloxane #2 |
| UNKMS3 | Unknown Methylated siloxane #3 |
| UNKOXYCOM | Unknown Oxygenated compound |
| UNKOXYCOM1 | Unknown Oxygenated compound #1 |
| UNKOXYCOM2 | Unknown Oxygenated compound #2 |
| UNKOXYCOM3 | Unknown Oxygenated compound #3 |
| UNKOXYCOM4 | Unknown Oxygenated compound #4 |
| UNKPHLATE | Unknown Phthalate |
| UNKPHT1 | Unknown Phthalate #1 |
| UNKPHT2 | Unknown Phthalate #2 |
| UNKPHT3 | Unknown Phthalate #3 |
| UNKPHT4 | Unknown Phthalate #4 |
| UNKPHT5 | Unknown Phthalate #5 |
| UNKPCB | Unknown Polychloronated biphenyl |
| UNKPCB1 | Unknown Polychloronated biphenyl #1 |
| UNKPCB10 | Unknown Polychloronated biphenyl #10 |
| UNKPCB11 | Unknown Polychloronated biphenyl #11 |
| UNKPCB12 | Unknown Polychloronated biphenyl #12 |
| UNKPCB13 | Unknown Polychloronated biphenyl #13 |
| UNKPCB2 | Unknown Polychloronated biphenyl #2 |
| UNKPCB3 | Unknown Polychloronated biphenyl #3 |
| UNKPCB4 | Unknown Polychloronated biphenyl #4 |
| UNKPCB5 | Unknown Polychloronated biphenyl #5 |
| UNKPCB6 | Unknown Polychloronated biphenyl #6 |
| UNKPCB7 | Unknown Polychloronated biphenyl #7 |
| UNKPCB8 | Unknown Polychloronated biphenyl #8 |
| UNKPCB9 | Unknown Polychloronated biphenyl #9 |
| UNKPAH | Unknown Polynuclear aromatic hydrocarbon |
| UNKPAH1 | Unknown Polynuclear aromatic hydrocarbon #1 |
| UNKPAH2 | Unknown Polynuclear aromatic hydrocarbon #2 |
| UNKPAH3 | Unknown Polynuclear aromatic hydrocarbon #3 |
| UNKPAH4 | Unknown Polynuclear aromatic hydrocarbon #4 |
| UNKPAH5 | Unknown Polynuclear aromatic hydrocarbon #5 |
| UNKSILOXANE | Unknown Siloxane |
| UNKSTERANE | Unknown Sterane |
| UNKSTERANE1 | Unknown Sterane #1 |
| UNKSTERANE2 | Unknown Sterane #2 |
| UNKSTERANE3 | Unknown Sterane #3 |
| UNKSTERANE4 | Unknown Sterane #4 |
| UNKSTEROL | Unknown Sterol |
| UNKSTE1 | Unknown Sterol #1 |
| UNKSTE2 | Unknown Sterol #2 |
| UNKSTE3 | Unknown Sterol #3 |
| UNKSTE4 | Unknown Sterol #4 |
| UNKSTE5 | Unknown Sterol #5 |
| UNKSUBALCOH | Unknown Substituted alcohol |
| UNKSUBALK1 | Unknown Substituted alkane #1 |
| UNKSUBALK2 | Unknown Substituted alkane #2 |
| UNKSUBALK3 | Unknown Substituted alkane #3 |
| UNKSUBALK4 | Unknown Substituted alkane #4 |
| UNKSUBALK5 | Unknown Substituted alkane #5 |
| UNKSUBARO1 | Unknown Substituted aromatic #1 |
| UNKSUBARO2 | Unknown Substituted aromatic #2 |
| UNKSUBARO3 | Unknown Substituted aromatic #3 |
| UNKSUBARO4 | Unknown Substituted aromatic #4 |
| UNKSUBARO5 | Unknown Substituted aromatic #5 |
| UNKSUBBZ1 | Unknown Substituted benzene #1 |
| UNKSUBBZ2 | Unknown Substituted benzene #2 |
| UNKSUBBZ3 | Unknown Substituted benzene #3 |
| UNKSUBBZ4 | Unknown Substituted benzene #4 |
| UNKSUBBZ5 | Unknown Substituted benzene #5 |
| UNKSUBCYALK | Unknown Substituted cycloalkane |
| UNKSUBINDENE | Unknown Substituted indene |
| UNKSUBIND1 | Unknown Substituted indene #1 |
| UNKSUBIND2 | Unknown Substituted indene #2 |
| UNKSUBIND3 | Unknown Substituted indene #3 |
| UNKSUBPAH | Unknown Substituted PAH |
| UNKSUBPAH1 | Unknown Substituted PAH #1 |
| UNKSUBPAH2 | Unknown Substituted PAH #2 |
| UNKSUBPHENOL | Unknown Substituted phenol |
| UNKSULPHUR | Unknown Sulphur |
| UNKTHIAZOLE | Unknown Thiazole |
| U | Uranium |
| UTOT | Uranium, Total |
| U-234 | Uranium-234 |
| U-235 | Uranium-235 |
| U-238 | Uranium-238 |
| V | Vanadium |
| VA | Vinyl acetate |
| VC | Vinyl chloride |
| VISCOS | Viscosity |
| WATER | Water Content |
| WICHLRN | Water Insoluble Chlorine |
| WSCL | Water Soluble Chlorides |
| WOLLASTONITE | Wollastonite |
| XYLENES1314 | Xylene, Isomers m & p |
| XYLENES1214 | Xylene, Isomers o & p |
| XYLENES | Xylenes |
| YB | Ytterbium |
| Y | Yttrium |
| ZN | Zinc |
| ZR | Zirconium |

# PARUN

Definition: The Parameter Uncertainty is the analytical uncertainty associated with a laboratory result.

| Attributes: | N12 |  |
| --- | --- | --- |
| Included in Tables: |  | RESULTS |

Guidelines & Restrictions:

- PARUN should be left blank for non-radiochemical results.

- PARUN is to be used only for radiochemical results.

- PARUN must be greater than or equal to zero.

# PARVAL

Definition: The Parameter Value is the actual analytical value for a compound or analyte; the result generated after a sample has been analyzed or a test has been performed.

| Attributes: | N14 |  |
| --- | --- | --- |
| Included in Tables: |  | RESULTS |

Guidelines & Restrictions:

- PARVAL cannot be left blank.

- PARVAL may contain a negative number.

- For results detected below LABDL, enter zero into PARVAL and “ND” into PARVQ.

- For results not calculated due to multiple runs, enter zero into PARVAL and “ND” into PARVQ.

# PARVQ

Definition: The PARVQ field indicates the Qualifier of an analytical result, or identifies a special type of parameter, such as a surrogate.

| Attributes: | C2 |  |
| --- | --- | --- |
| Included in Tables: |  | RESULTS |

Guidelines & Restrictions:

- PARVQ cannot be left blank.

- PARVQ must contain a valid value.

- The PARVQ field may be used in several ways. The field is most commonly used to qualify results. Standard analytical results will be qualified with “=” or “ND” (“Not Detected”). The PARVQ field may also be used to identify a special type of parameter such as a tentatively identified compound (“TI”), surrogates (“SU”), or internal standards (“IN”). And lastly, the PARVQ field may be used to indicate that data is not usable for a given parameter, such as “NR” (“Not Reported”).

Acceptable PARVQ Values:

| Code | Description |  |
| --- | --- | --- |
| < |  | Less Than |
| = |  | Equal To |
| > |  | Greater Than |
| DU |  | Data Unavailable |
| IN |  | Internal Standard |
| NA |  | Not Available - Result Not Available |
| ND |  | Not Detected |
| NR |  | Not Reported - Data Not Reported |
| SU |  | Surrogate |
| TI |  | Tentatively Identified Compound |

# PRESCODE

Definition: The PRESCODE represents the type of Preservative added to a sample upon collection in the field (e.g., pH adjusted to < 2 with hydrochloric acid).

| Attributes: | C15 |  |
| --- | --- | --- |
| Included in Tables: |  | TEST |

Guidelines & Restrictions:

- PRESCODE may be left blank.

- PRESCODE must contain a valid value if populated.

- Multiple PRESCODEs may be used; commas without spaces separate the codes (e.g., “P08,P12”).

Acceptable PRESCODE Values:

| Code | Description |
| --- | --- |
| DU | Data Unavailable |
| NONE | No preservation |
| P01 | 4 drops of 10% sodium thiosulfate to 4 oz. |
| P02 | Adjust to pH 4-5 |
| P03 | Add 3 mL 10% sodium thiosulfate per 1 gal. |
| P04 | Adjust to pH < 2 with sulfuric acid |
| P05 | Adjust to pH < 2 with hydrochloric acid |
| P06 | Adjust to pH < 2 with sodium hydrogen sulfate |
| P07 | Adjust to pH > 12 with sodium hydroxide |
| P08 | Adjust to pH < 2 with nitric acid |
| P09 | 0.6 g of ascorbic acid to 500 mLs |
| P10 | Add 2 mL of zinc acetate to 500 mLs |
| P11 | Adjust to pH > 9 with sodium hydroxide |
| P12 | 4 degrees Celsius |
| P13 | Methanol preservation |
| P14 | Not noted on Chain-of-Custody |
| P15 | Cooler temperature > 6 degrees C |
| P16 | Cooler temperature < 2 degrees C or freezing |
| P17 | Add 5mL 20% sodium bisulfate |

# PROJNAME

Definition: The Project Name is the name assigned to the project by the client, and can usually be found on the chain-of-custody.

| Attributes: | C25 |  |
| --- | --- | --- |
| Included in Tables: |  | SAMPLE |

Guidelines & Restrictions:

- PROJNAME cannot be left blank when QCCODE = “CS,” and must be blank in all other cases.

# PVCCODE

Definition: The PVCCODE distinguishes between primary and confirmatory results.

| Attributes: | C2 |  |
| --- | --- | --- |
| Included in Tables: |  | RESULTS |

Guidelines & Restrictions:

- PVCCODE cannot be left blank.

- PVCCODE must contain a valid value.

- There may be only one result with PVCCODE = “PR” per LABSAMPID, ANMCODE, EXMCODE, and PARLABEL, and there must be a “PR” result reported.

- PVCCODEs are used to report supporting gas chromatography (GC) confirmation information (used to verify compound identification). The confirmation results are entered using the first column (“1C”), second column (“2C”), and Gas Chromatography/Mass Spectroscopy (“MS”) PVCCODEs. For example, if the sample is confirmed using the first column, “1C” is entered into the PVCCODE field of the confirmation result. The primary result (PVCCODE = “PR”) will be assigned to the column result in which the laboratory places the most confidence. (The primary result will generally be assigned to the first column results.)

- If a dilution is required for a sample, both analytical determinations must be provided with the appropriate dilution factors and adjusted reporting limits. However, the laboratory must select which value they wish to report as the primary result (“PR”). The value that is not chosen to report should have the PVCCODE, “SR” (“Semi-Qualitative Result”).

Acceptable PVCCODE Values:

| Code | Description |
| --- | --- |
| 1C | First Column Result - The Value Obtained from the First Column |
| 2C | Second Column Result - The Value Obtained from the Second Column |
| DU | Data Unavailable |
| MS | GC/MS Result - Value Confirmed Using GC/MS |
| NR | Not Reported - Data Not Reported |
| NU | Not Usable - Data Not Usable |
| PR | Primary Result - The Primary Result for a Parameter |
| SR | Semi-Quantitative Result |

# QCCODE

Definition: The QCCODE represents the Quality Control Sample Type.

| Attributes: | C3 |  |
| --- | --- | --- |
| Included in Tables: |  | TEST RESULTS QC |

Guidelines & Restrictions:

- QCCODE cannot be left blank.

- QCCODE must contain a valid value.

- Standard field samples are assigned a QCCODE of “CS.”

- Tests performed on spiked field samples are assigned QCCODEs of “MS” or “SD.”

- Tests performed on replicates of a field sample are assigned codes of “LR.”

- All other available QCCODEs are assigned to laboratory-generated QC samples, with the exception of the “NC” code that identifies “Non-Client Samples” that have been included in the database to provide QC information.

Acceptable QCCODE Values:

| Code | Description |
| --- | --- |
| BD1 | Blank Spike Duplicate #1 |
| BD2 | Blank Spike Duplicate #2 |
| BD3 | Blank Spike Duplicate #3 |
| BD4 | Blank Spike Duplicate #4 |
| BD5 | Blank Spike Duplicate #5 |
| BD6 | Blank Spike Duplicate #6 |
| BD7 | Blank Spike Duplicate #7 |
| BD8 | Blank Spike Duplicate #8 |
| BD9 | Blank Spike Duplicate #9 |
| BDA | Blank Spike Duplicate #10 |
| BDB | Blank Spike Duplicate #11 |
| BDC | Blank Spike Duplicate #12 |
| BDD | Blank Spike Duplicate #13 |
| BDE | Blank Spike Duplicate #14 |
| BDF | Blank Spike Duplicate #15 |
| BDG | Blank Spike Duplicate #16 |
| BDH | Blank Spike Duplicate #17 |
| BDI | Blank Spike Duplicate #18 |
| BDJ | Blank Spike Duplicate #19 |
| BDK | Blank Spike Duplicate #20 |
| BDL | Blank Spike Duplicate #21 |
| BDM | Blank Spike Duplicate #22 |
| BDN | Blank Spike Duplicate #23 |
| BDO | Blank Spike Duplicate #24 |
| BDP | Blank Spike Duplicate #25 |
| BDQ | Blank Spike Duplicate #26 |
| BDR | Blank Spike Duplicate #27 |
| BDS | Blank Spike Duplicate #28 |
| BDT | Blank Spike Duplicate #29 |
| BDU | Blank Spike Duplicate #30 |
| BDV | Blank Spike Duplicate #31 |
| BDW | Blank Spike Duplicate #32 |
| BDX | Blank Spike Duplicate #33 |
| BDY | Blank Spike Duplicate #34 |
| BDZ | Blank Spike Duplicate #35 |
| BS1 | Blank Spike #1 |
| BS2 | Blank Spike #2 |
| BS3 | Blank Spike #3 |
| BS4 | Blank Spike #4 |
| BS5 | Blank Spike #5 |
| BS6 | Blank Spike #6 |
| BS7 | Blank Spike #7 |
| BS8 | Blank Spike #8 |
| BS9 | Blank Spike #9 |
| BSA | Blank Spike #10 |
| BSB | Blank Spike #11 |
| BSC | Blank Spike #12 |
| BSD | Blank Spike #13 |
| BSE | Blank Spike #14 |
| BSG | Blank Spike #16 |
| BSH | Blank Spike #17 |
| BSI | Blank Spike #18 |
| BSJ | Blank Spike #19 |
| BSK | Blank Spike #20 |
| BSL | Blank Spike #21 |
| BSM | Blank Spike #22 |
| BSN | Blank Spike #23 |
| BSO | Blank Spike #24 |
| BSP | Blank Spike #25 |
| BSQ | Blank Spike #26 |
| BSR | Blank Spike #27 |
| BSS | Blank Spike #28 |
| BST | Blank Spike #29 |
| BSU | Blank Spike #30 |
| BSV | Blank Spike #31 |
| BSW | Blank Spike #32 |
| BSX | Blank Spike #33 |
| BSY | Blank Spike #34 |
| BSZ | Blank Spike #35 |
| CB1 | Calibration Blank #1 |
| CB2 | Calibration Blank #2 |
| CB3 | Calibration Blank #3 |
| CB4 | Calibration Blank #4 |
| CB5 | Calibration Blank #5 |
| CB6 | Calibration Blank #6 |
| CB7 | Calibration Blank #7 |
| CB8 | Calibration Blank #8 |
| CB9 | Calibration Blank #9 |
| CBA | Calibration Blank #10 |
| CBB | Calibration Blank #11 |
| CBC | Calibration Blank #12 |
| CBD | Calibration Blank #13 |
| CBE | Calibration Blank #14 |
| CBF | Calibration Blank #15 |
| CC | Continuing Calibration Verification |
| CC1 | Continuing Calibration Verification #1 |
| CC2 | Continuing Calibration Verification #2 |
| CC3 | Continuing Calibration Verification #3 |
| CC4 | Continuing Calibration Verification #4 |
| CC5 | Continuing Calibration Verification #5 |
| CC6 | Continuing Calibration Verification #6 |
| CC7 | Continuing Calibration Verification #7 |
| CC8 | Continuing Calibration Verification #8 |
| CC9 | Continuing Calibration Verification #9 |
| CCA | Continuing Calibration Verification #10 |
| CCB | Continuing Calibration Verification #11 |
| CCC | Continuing Calibration Verification #12 |
| CCD | Continuing Calibration Verification #13 |
| CCE | Continuing Calibration Verification #14 |
| CCF | Continuing Calibration Verification #15 |
| CCG | Continuing Calibration Verification #16 |
| CCH | Continuing Calibration Verification #17 |
| CCI | Continuing Calibration Verification #18 |
| CCJ | Continuing Calibration Verification #19 |
| CCK | Continuing Calibration Verification #20 |
| CS | Client Sample |
| DU | Data Unavailable |
| IC | Initial Calibration Verification |
| IC1 | Initial Calibration Verification #1 |
| IC2 | Initial Calibration Verification #2 |
| IC3 | Initial Calibration Verification #3 |
| IC4 | Initial Calibration Verification #4 |
| IC5 | Initial Calibration Verification #5 |
| IC6 | Initial Calibration Verification #6 |
| IC7 | Initial Calibration Verification #7 |
| IC8 | Initial Calibration Verification #8 |
| IC9 | Initial Calibration Verification #9 |
| KD1 | Known (External Reference Material) Duplicate #1 |
| KD2 | Known (External Reference Material) Duplicate #2 |
| KD3 | Known (External Reference Material) Duplicate #3 |
| KD4 | Known (External Reference Material) Duplicate #4 |
| KD5 | Known (External Reference Material) Duplicate #5 |
| KD6 | Known (External Reference Material) Duplicate #6 |
| KD7 | Known (External Reference Material) Duplicate #7 |
| KD8 | Known (External Reference Material) Duplicate #8 |
| KD9 | Known (External Reference Material) Duplicate #9 |
| LB1 | Lab Blank #1 |
| LB2 | Lab Blank #2 |
| LB3 | Lab Blank #3 |
| LB4 | Lab Blank #4 |
| LB5 | Lab Blank #5 |
| LB6 | Lab Blank #6 |
| LB7 | Lab Blank #7 |
| LB8 | Lab Blank #8 |
| LB9 | Lab Blank #9 |
| LBA | Lab Blank #10 |
| LBB | Lab Blank #11 |
| LBC | Lab Blank #12 |
| LBD | Lab Blank #13 |
| LBE | Lab Blank #14 |
| LBF | Lab Blank #15 |
| LBG | Lab Blank #16 |
| LBH | Lab Blank #17 |
| LBI | Lab Blank #18 |
| LBJ | Lab Blank #19 |
| LBK | Lab Blank #20 |
| LBL | Lab Blank #21 |
| LBM | Lab Blank #22 |
| LBN | Lab Blank #23 |
| LBO | Lab Blank #24 |
| LBP | Lab Blank #25 |
| LBQ | Lab Blank #26 |
| LBR | Lab Blank #27 |
| LBS | Lab Blank #28 |
| LBT | Lab Blank #29 |
| LBU | Lab Blank #30 |
| LBV | Lab Blank #31 |
| LBW | Lab Blank #32 |
| LBX | Lab Blank #33 |
| LBY | Lab Blank #34 |
| LBZ | Lab Blank #35 |
| LR1 | Lab Replicate #1 |
| LR2 | Lab Replicate #2 |
| LR3 | Lab Replicate #3 |
| LR4 | Lab Replicate #4 |
| LR5 | Lab Replicate #5 |
| LR6 | Lab Replicate #6 |
| LR7 | Lab Replicate #7 |
| LR8 | Lab Replicate #8 |
| LR9 | Lab Replicate #9 |
| LRA | Lab Replicate #10 |
| LRB | Lab Replicate #11 |
| LRC | Lab Replicate #12 |
| LRD | Lab Replicate #13 |
| LRE | Lab Replicate #14 |
| MS1 | Lab Matrix Spike #1 |
| MS2 | Lab Matrix Spike #2 |
| MS3 | Lab Matrix Spike #3 |
| MS4 | Lab Matrix Spike #4 |
| MS5 | Lab Matrix Spike #5 |
| MS6 | Lab Matrix Spike #6 |
| MS7 | Lab Matrix Spike #7 |
| MS8 | Lab Matrix Spike #8 |
| MS9 | Lab Matrix Spike #9 |
| MSA | Lab Matrix Spike #10 |
| MSB | Lab Matrix Spike #11 |
| MSC | Lab Matrix Spike #12 |
| MSD | Lab Matrix Spike #13 |
| MSE | Lab Matrix Spike #14 |
| NC | Non-Client Sample |
| RM1 | Known (External Reference Material) #1 |
| RM2 | Known (External Reference Material) #2 |
| RM3 | Known (External Reference Material) #3 |
| RM4 | Known (External Reference Material) #4 |
| RM5 | Known (External Reference Material) #5 |
| RM6 | Known (External Reference Material) #6 |
| RM7 | Known (External Reference Material) #7 |
| RM8 | Known (External Reference Material) #8 |
| RM9 | Known (External Reference Material) #9 |
| RS1 | Reagent Solvent #1 |
| RS2 | Reagent Solvent #2 |
| RS3 | Reagent Solvent #3 |
| SD1 | Lab Matrix Spike Duplicate #1 |
| SD2 | Lab Matrix Spike Duplicate #2 |
| SD3 | Lab Matrix Spike Duplicate #3 |
| SD4 | Lab Matrix Spike Duplicate #4 |
| SD5 | Lab Matrix Spike Duplicate #5 |
| SD6 | Lab Matrix Spike Duplicate #6 |
| SD7 | Lab Matrix Spike Duplicate #7 |
| SD8 | Lab Matrix Spike Duplicate #8 |
| SD9 | Lab Matrix Spike Duplicate #9 |
| SDA | Lab Matrix Spike Duplicate #10 |
| SDB | Lab Matrix Spike Duplicate #11 |
| SDC | Lab Matrix Spike Duplicate #12 |
| SDD | Lab Matrix Spike Duplicate #13 |
| SDE | Lab Matrix Spike Duplicate #14 |

# RECDATE

Definition: The Received Date is the date the laboratory physically takes custody of the sample.

| Attributes: | D8 |  |
| --- | --- | --- |
| Included in Tables: |  | TEST |

Guidelines & Restrictions:

- All date fields must be in the YYYYMMDD format.

- RECDATE cannot be left blank.

- For laboratory-generated QC samples enter the EXTDATE into RECDATE.

- RECDATE must be later than or equal to LOGDATE.

- RECDATE must be earlier than or equal to EXTDATE.

- RECDATE must be earlier than or equal to ANADATE.

- RECDATE must be earlier than or equal to REP_DATE.

# REP_DATE

Definition: The Report Date is the date that the laboratory generates the hard copy report and EDD.

| Attributes: | D8 |  |
| --- | --- | --- |
| Included in Tables: |  | TEST |

Guidelines & Restrictions:

- All date fields must be in the YYYYMMDD format.

- REP_DATE cannot be left blank when QCCODE = “CS,” and must be blank in all other cases.

- REP_DATE must be later than or equal to LOGDATE.

- REP_DATE must be later than or equal to EXTDATE.

- REP_DATE must be later than or equal to ANADATE.

- REP_DATE must be later than or equal to RECDATE.

# REPDL

Definition: The REPDL represents the laboratory’s Reported Detection Limit that determines whether a parameter is detectable.

| Attributes: | N9 |  |
| --- | --- | --- |
| Included in Tables: |  | RESULTS |

Guidelines & Restrictions:

- REPDL cannot be left blank, except when UNITS = “PERCENT” (e.g., surrogate parameters), or PARVQ = “TI” (i.e., for TIC parameters).

- REPDL must be adjusted for dilution.

- REPDL may contain the same value as the LABDL field, depending on the reporting format of the individual laboratory.  In this case, the REPDLVQ should indicate that the LABDL is actually the REPDL value (e.g., “MDL” would be an appropriate REPDLVQ when LABDL and REPDL are equal).

- REPDL must be greater than or equal to zero.

# REPDLVQ

Definition: The REPDLVQ represents the Reported Detection Limit Qualifier used by the analytical laboratory (e.g., practical quantitation limit [PQL], instrument detection limit [IDL], method detection limit [MDL], etc.).

| Attributes: | C3 |  |
| --- | --- | --- |
| Included in Tables: |  | RESULTS |

Guidelines & Restrictions:

- REPDLVQ cannot be left blank.

- REPDLVQ must contain a valid value.

- When UNITS = “PERCENT” or PARVQ = “TI,” enter “NA” for REPDLVQ.

Acceptable REPDLVQ Values:

| Code | Description |
| --- | --- |
| DDL | Method Defined Detection Limit |
| DU | Data Unavailable |
| EQL | Estimated Quantitation Limit |
| IDL | Instrument Detection Limit |
| LLD | Lowest Level of Detection |
| MDL | Method Detection Limit |
| MRL | Method Reporting Limit (lowest standard adjusted for prep.) |
| NA | Not Applicable |
| PQL | Practical Quantitation Limit |
| PRL | Parameter Range Limit |

# REQ_METHOD_GRP

Definition: The Requested Method Group is the unique identifier for the method or group of methods requested by the client for analysis of the sample.

| Attributes: | C25 |  |
| --- | --- | --- |
| Included in Tables: |  | SAMPLE |

Guidelines & Restrictions:

- REQ_METHOD_GRP is an optional field and may be left blank.

- REQ_METHOD_GRP is a linking field with the EDMS2000 electronic COC tables.

# RLNOTE

Laboratory Result Note. Refer to LNOTE entry on page 51.

# RT

Definition: The RT represents the Retention Time of a tentatively identified compound (TIC).

| Attributes: | N7 |  |
| --- | --- | --- |
| Included in Tables: |  | RESULTS |

Guidelines & Restrictions:

- RT cannot be left blank when PARVQ = “TI,” and should be blank in all other cases.

- RT must be greater than or equal to zero.

- RT is reported in minutes.

# RUN_NUMBER

Definition: The Run Number is a numerical “coding” of multiple or repeat analyses of a sample (one LABSAMPID) by the same analytical method.

| Attributes: | N2 |  |
| --- | --- | --- |
| Included in Tables: |  | TEST RESULTS |

Guidelines & Restrictions:

- RUN_NUMBER cannot be left blank.

- RUN_NUMBER must be an integer greater than or equal to one and less than or equal to 99.

# SAMPID

Definition: The Sample ID is the identification assigned to a sample during collection in the field, and as it appears on the chain-of-custody.

| Attributes: | C25 |  |
| --- | --- | --- |
| Included in Tables: |  | SAMPLE TEST |

Guidelines & Restrictions:

- SAMPID cannot be left blank when QCCODE = “CS,” and must be blank in all other cases.

- SAMPID must be unique.

# SRM

Definition: The SRM identifies the source of the Standard Reference Material for the calibration method used by the analytical laboratory.

| Attributes: | C12 |  |
| --- | --- | --- |
| Included in Tables: |  | RESULTS |

Guidelines & Restrictions:

- SRM cannot be left blank.

- SRM must contain a valid value.

- When no reference material is used, enter “NA.”

Acceptable SRM Values:

| Code | Description |
| --- | --- |
| ABSSTD | Absolute Standards |
| ACCUSTD | AccuStandard |
| ALDRICH | Aldrich Chemical Co. |
| APG | Analytical Products Group |
| BURJAC | Burdick & Jackson |
| CAMBRIDGE | Cambridge Isotope Labs |
| CHEMSERV | Chem Services, Inc. |
| CPI | CPI, Santa Rosa, CA |
| DU | Data Unavailable |
| EMSCIENCE | EM Science |
| EMSL | Environmental Monitoring Systems Laboratory (EMSL), Las Vegas, NV |
| ERAS | Environmental Research Associated Standards |
| ETHYLCORP | Ethyl Corp. |
| FISHER | Fisher Scientific |
| HACH | HACH Chemical |
| INVENT | Inorganic Ventures |
| JTBAKER | J. T. Baker |
| KODAK | Eastman Kodak Co. |
| LEEMAN | Leeman Laboratories |
| MALLINBKRO | Mallinbkrodt |
| NA | Not Applicable |
| NIST | U.S.D.C., National Institute of Standards & Technology |
| PLASMA | Plasma Chem, Inc. |
| PROTOCOL | Protocol |
| RADIAN | Radian Corporation |
| RESTEK | Restek |
| SGAS | Scotty Specialty Gases |
| SIGMA | Sigma Chemical Co. |
| SOLPUS | Solutions Plus |
| SOURCE | The Source |
| SPEX | SPEX Industries |
| SUPELCO | Supelco |
| ULTRA | Ultra Scientific |
| USATHAMA | U.S. Army |

# SUB

Definition: The SUB field contains the LABCODE of the Subcontracted Laboratory for an analysis.

| Attributes: | C4 |  |
| --- | --- | --- |
| Included in Tables: |  | TEST |

Guidelines & Restrictions:

- SUB cannot be left blank.

- SUB must contain a valid value.

- Enter “NA” if no subcontracting occurred.

Acceptable SUB Values Sorted by Code:

| Code | Description |
| --- | --- |
| ACZ | ACZ Laboratories, Steamboat, CO |
| AEHA | Army Environmental Hygiene Agency (AEHA), APG, MD |
| AELF | American Environmental Laboratories, Pensacola, FL |
| AENP | American Environmental Network, Portland, OR |
| ALTC | Alta Analytical Lab Incorporated, El Dorado Hills, CA |
| APPL | Agriculture & Priority Pollutants Laboratories, Fresno, CA |
| ARDL | Applied Research and Development Lab, Inc., (ARDL) Mt. Vernon, IL |
| ARI | Analytical Resources, Inc., Seattle, WA |
| ATCA | Analytica, Anchorage, AK |
| ATCC | Analytica, CO |
| ATIA | Analytical Technologies, Inc., Anchorage, AK |
| ATIR | Analytical Technologies, Inc., Renton, WA |
| ATIS | Analytical Technologies, Inc., San Diego, CA |
| ATOX | Air Toxics LTD, Folsom, CA |
| AXYS | Axys Analytical Services, Ltd., Sidney, B.C., Canada |
| BCE | Brown & Caldwell Analytical Lab, Emeryville, CA |
| BCLB | BC Laboratories, Bakersfield, CA |
| BMLA | Boreochem Mobile Lab & Analytical Services |
| BRS | Brelje & Race, Santa Rosa, CA |
| CASB | Columbia Analytical Services, Inc., Bothell, WA |
| CASD | Columbia Analytical Services, Inc., Redding, CA |
| CASK | Columbia Analytical Services, Inc., Kelso, WA |
| CASL | Columbia Analytical Services, Inc., Canoga Park, CA |
| CAWL | California Water Labs, Inc., Modesto, CA |
| CCAC | Coast-to-Coast Analytical Services, Inc., Camarillo, CA |
| CCSJ | Coast-to-Coast Analytical Services, Inc., San Jose, CA |
| CDM | CDM Federal Programs Corporation |
| CHEM | Chemic Laboratory, San Diego, CA |
| CHMC | CH2M Hill Analytical Services, Corvallis, OR |
| CHMM | CH2M Hill Analytical Services, Montgomery, AL |
| CHRP | Chromalab, Inc., Pleasanton, CA |
| CKY | CKY Inc., Torrance, CA |
| CLSR | California Laboratory Services, Rancho Cordova, CA |
| CLTP | Clayton Environmental Consultants, Inc., Pleasanton, CA |
| CRLB | Century Refining (CENREF) Labs, Inc., Brighton, CO |
| CTB | Curtis & Tompkins, Berkeley, CA |
| CTE | CT&E Environmental Services, Inc., Anchorage, AK |
| CTEC | CT&E Environmental Services, Inc., Charleston, WV |
| DCHM | DataChem Laboratories, Inc., Salt Lake City, UT |
| DMP | D & M Laboratories, Petaluma, CA |
| DOWL | Dowl Engineering Alaska Test Labs, Anchorage, AK |
| DU | Data Unavailable |
| ECEN | Ecology & Environment, Inc. |
| ECI | EcoChem, Inc., Seattle, WA |
| ECLL | Environmental Chemistry Lab at LLNL, Livermore, CA |
| EEIS | Envirodyne Engineers, Inc., St. Louis, MO |
| EMXT | EMAX Laboratories, Inc., Torrance, CA |
| ETCS | ETC, Santa Rosa, CA |
| FGIS | Frontier Geosciences, Inc., Seattle, WA |
| FGL | Fruit Growers Lab, Stockton, CA |
| FORA | Forensic Analytical |
| GELC | General Engineering Laboratories, Inc., Charleston, SC |
| GENC | GTEL Environmental Labs, Inc., Concord, CA |
| KIC | KIC Lab, Prudhoe Bay, AK |
| LAL | Lockheed Analytical Laboratory, Las Vegas, NV |
| LASL | Los Alamos Scientific Laboratory, Los Alamos, NM |
| LDC | Laboratory Data Consultants |
| LTL | Laucks Testing Lab, Inc. |
| MEC | MEC Analytical Systems, Inc., Carlsbad, CA |
| MSSL | Mountain States Analytical, Salt Lake City, UT |
| MWLP | Montgomery Watson Laboratories, Pasadena, CA |
| NA | Not Applicable |
| NCAB | North Creek Analytical, Bothell, WA |
| NCAC | North Creek Analytical, Bend, OR |
| NCAP | North Creek Analytical, Beaverton, OR |
| NCAS | North Creek Analytical, Spokane, WA |
| NTL | Northern Testing Laboratories, Anchorage, AK |
| NTLF | Northern Testing Laboratories, Fairbanks, AK |
| NWCC | Northwest Colorado Consultants, Inc., Steamboat Springs, CO |
| OEIR | OnSite Environmental, Inc., Redmond, WA |
| PAC | Pacific Analytical, Carlsbad, CA |
| PAIS | Performance Analytical, Inc., Simi Valley, CA |
| PARA | Paragon Analytics, Inc., CO |
| PHLE | Philip Environmental |
| QALA | Quality Analytical Laboratores, Inc., Montgomery, AL |
| QALC | Quality Analytical Laboratories, Inc., Redding, CA |
| RFWC | Roy F. Weston, West Chester, PA |
| RFWS | Roy F. Weston, Stockton, CA |
| SAS | Sound Analytical Services, Inc., Tacoma, WA |
| SC3S | S-Cubed, A Division of Maxwell Laboratories, Inc., San Diego, CA |
| SEQR | Sequoia Analytical Laboratories, Inc., Redwood City, CA |
| SPEC | Spectra Laboratory, Inc., Tacoma, WA |
| STL1 | Severn Trent Laboratories, Arvada, CO |
| STL2 | Severn Trent Laboratories, Edison, NJ |
| STL3 | Severn Trent Laboratories, Santa Ana, CA |
| STL4 | Severn Trent Laboratories, Miramar, FL |
| STL5 | Severn Trent Laboratories, Newburgh, NY |
| STL6 | Severn Trent Laboratories, Colchester, VT |
| STL7 | Severn Trent Laboratories, Aurora, CO |
| STLB | Severn Trent Laboratories, Sparks, MD |
| STLC | Severn Trent Laboratories, North Canton, OH |
| STLD | Severn Trent Laboratories, Austin, TX |
| STLE | Severn Trent Laboratories, Tallahassee, FL |
| STLF | Severn Trent Laboratories, Tampa, FL (Quanterra) |
| STLG | Severn Trent Laboratories, Savannah, GA |
| STLH | Severn Trent Laboratories, Houston, TX |
| STLI | Severn Trent Laboratories, Pensacola, FL |
| STLJ | Severn Trent Laboratories, N. Billerica, MA |
| STLK | Severn Trent Laboratories, Knoxville, TN |
| STLL | Severn Trent Laboratories, Earth City, MO |
| STLM | Severn Trent Laboratories, Monroe, CT |
| STLN | Severn Trent Laboratories, Anaheim, CA |
| STLO | Severn Trent Laboratories, Mobile, AL |
| STLP | Severn Trent Laboratories, Pittsburgh, PA |
| STLQ | Severn Trent Laboratories, Amherst, NY |
| STLR | Severn Trent Laboratories, Richland, WA |
| STLS | Severn Trent Laboratories, West Sacramento, CA |
| STLT | Severn Trent Laboratories, Austin, TX (Quanterra) |
| STLU | Severn Trent Laboratories, University Park, IL |
| STLV | Severn Trent Laboratories, Valparaiso, IN |
| STLW | Severn Trent Laboratories, Westfield, MA |
| STLX | Severn Trent Laboratories, Tampa, FL (Savannah) |
| STLY | Severn Trent Laboratories, Whippany, NJ |
| STLZ | Severn Trent Laboratories, Corpus Christi, TX |
| SWAA | Shannon & Wilson, Inc., Anchorage, AK |
| SWLB | Southwest Laboratory |
| SWRI | Southwest Resarch Institute, San Antonio, TX |
| TRID | Triangle Laboratories, Inc., Durham, NC |

Acceptable SUB Values Sorted by Description:

| Code | Description |
| --- | --- |
| ACZ | ACZ Laboratories, Steamboat, CO |
| APPL | Agriculture & Priority Pollutants Laboratories, Fresno, CA |
| ATOX | Air Toxics LTD, Folsom, CA |
| ALTC | Alta Analytical Lab Incorporated, El Dorado Hills, CA |
| AELF | American Environmental Laboratories, Pensacola, FL |
| AENP | American Environmental Network, Portland, OR |
| ATCA | Analytica, Anchorage, AK |
| ATCC | Analytica, CO |
| ARI | Analytical Resources, Inc., Seattle, WA |
| ATIA | Analytical Technologies, Inc., Anchorage, AK |
| ATIR | Analytical Technologies, Inc., Renton, WA |
| ATIS | Analytical Technologies, Inc., San Diego, CA |
| ARDL | Applied Research and Development Lab, Inc., (ARDL) Mt. Vernon, IL |
| AEHA | Army Environmental Hygiene Agency (AEHA), APG, MD |
| AXYS | Axys Analytical Services, Ltd., Sidney, B.C., Canada |
| BCLB | BC Laboratories, Bakersfield, CA |
| BMLA | Boreochem Mobile Lab & Analytical Services |
| BRS | Brelje & Race, Santa Rosa, CA |
| BCE | Brown & Caldwell Analytical Lab, Emeryville, CA |
| CLSR | California Laboratory Services, Rancho Cordova, CA |
| CAWL | California Water Labs, Inc., Modesto, CA |
| CDM | CDM Federal Programs Corporation |
| CRLB | Century Refining (CENREF) Labs, Inc., Brighton, CO |
| CHMC | CH2M Hill Analytical Services, Corvallis, OR |
| CHMM | CH2M Hill Analytical Services, Montgomery, AL |
| CHEM | Chemic Laboratory, San Diego, CA |
| CHRP | Chromalab, Inc., Pleasanton, CA |
| CKY | CKY Inc., Torrance, CA |
| CLTP | Clayton Environmental Consultants, Inc., Pleasanton, CA |
| CCAC | Coast-to-Coast Analytical Services, Inc., Camarillo, CA |
| CCSJ | Coast-to-Coast Analytical Services, Inc., San Jose, CA |
| CASB | Columbia Analytical Services, Inc., Bothell, WA |
| CASL | Columbia Analytical Services, Inc., Canoga Park, CA |
| CASK | Columbia Analytical Services, Inc., Kelso, WA |
| CASD | Columbia Analytical Services, Inc., Redding, CA |
| CTE | CT&E Environmental Services, Inc., Anchorage, AK |
| CTEC | CT&E Environmental Services, Inc., Charleston, WV |
| CTB | Curtis & Tompkins, Berkeley, CA |
| DMP | D & M Laboratories, Petaluma, CA |
| DU | Data Unavailable |
| DCHM | DataChem Laboratories, Inc., Salt Lake City, UT |
| DOWL | Dowl Engineering Alaska Test Labs, Anchorage, AK |
| ECI | EcoChem, Inc., Seattle, WA |
| ECEN | Ecology & Environment, Inc. |
| EMXT | EMAX Laboratories, Inc., Torrance, CA |
| EEIS | Envirodyne Engineers, Inc., St. Louis, MO |
| ECLL | Environmental Chemistry Lab at LLNL, Livermore, CA |
| ETCS | ETC, Santa Rosa, CA |
| FORA | Forensic Analytical |
| FGIS | Frontier Geosciences, Inc., Seattle, WA |
| FGL | Fruit Growers Lab, Stockton, CA |
| GELC | General Engineering Laboratories, Inc., Charleston, SC |
| GENC | GTEL Environmental Labs, Inc., Concord, CA |
| KIC | KIC Lab, Prudhoe Bay, AK |
| LDC | Laboratory Data Consultants |
| LTL | Laucks Testing Lab, Inc. |
| LAL | Lockheed Analytical Laboratory, Las Vegas, NV |
| LASL | Los Alamos Scientific Laboratory, Los Alamos, NM |
| MEC | MEC Analytical Systems, Inc., Carlsbad, CA |
| MWLP | Montgomery Watson Laboratories, Pasadena, CA |
| MSSL | Mountain States Analytical, Salt Lake City, UT |
| NCAP | North Creek Analytical, Beaverton, OR |
| NCAC | North Creek Analytical, Bend, OR |
| NCAB | North Creek Analytical, Bothell, WA |
| NCAS | North Creek Analytical, Spokane, WA |
| NTL | Northern Testing Laboratories, Anchorage, AK |
| NTLF | Northern Testing Laboratories, Fairbanks, AK |
| NWCC | Northwest Colorado Consultants, Inc., Steamboat Springs, CO |
| NA | Not Applicable |
| OEIR | OnSite Environmental, Inc., Redmond, WA |
| PAC | Pacific Analytical, Carlsbad, CA |
| PARA | Paragon Analytics, Inc., CO |
| PAIS | Performance Analytical, Inc., Simi Valley, CA |
| PHLE | Philip Environmental |
| QALA | Quality Analytical Laboratores, Inc., Montgomery, AL |
| QALC | Quality Analytical Laboratories, Inc., Redding, CA |
| RFWS | Roy F. Weston, Stockton, CA |
| RFWC | Roy F. Weston, West Chester, PA |
| SC3S | S-Cubed, A Division of Maxwell Laboratories, Inc., San Diego, CA |
| SEQR | Sequoia Analytical Laboratories, Inc., Redwood City, CA |
| STLQ | Severn Trent Laboratories, Amherst, NY |
| STLN | Severn Trent Laboratories, Anaheim, CA |
| STL1 | Severn Trent Laboratories, Arvada, CO |
| STL7 | Severn Trent Laboratories, Aurora, CO |
| STLD | Severn Trent Laboratories, Austin, TX |
| STLT | Severn Trent Laboratories, Austin, TX (Quanterra) |
| STL6 | Severn Trent Laboratories, Colchester, VT |
| STLZ | Severn Trent Laboratories, Corpus Christi, TX |
| STLL | Severn Trent Laboratories, Earth City, MO |
| STL2 | Severn Trent Laboratories, Edison, NJ |
| STLH | Severn Trent Laboratories, Houston, TX |
| STLK | Severn Trent Laboratories, Knoxville, TN |
| STL4 | Severn Trent Laboratories, Miramar, FL |
| STLO | Severn Trent Laboratories, Mobile, AL |
| STLM | Severn Trent Laboratories, Monroe, CT |
| STLJ | Severn Trent Laboratories, N. Billerica, MA |
| STL5 | Severn Trent Laboratories, Newburgh, NY |
| STLC | Severn Trent Laboratories, North Canton, OH |
| STLI | Severn Trent Laboratories, Pensacola, FL |
| STLP | Severn Trent Laboratories, Pittsburgh, PA |
| STLR | Severn Trent Laboratories, Richland, WA |
| STL3 | Severn Trent Laboratories, Santa Ana, CA |
| STLG | Severn Trent Laboratories, Savannah, GA |
| STLB | Severn Trent Laboratories, Sparks, MD |
| STLE | Severn Trent Laboratories, Tallahassee, FL |
| STLF | Severn Trent Laboratories, Tampa, FL (Quanterra) |
| STLX | Severn Trent Laboratories, Tampa, FL (Savannah) |
| STLU | Severn Trent Laboratories, University Park, IL |
| STLV | Severn Trent Laboratories, Valparaiso, IN |
| STLS | Severn Trent Laboratories, West Sacramento, CA |
| STLW | Severn Trent Laboratories, Westfield, MA |
| STLY | Severn Trent Laboratories, Whippany, NJ |
| SWAA | Shannon & Wilson, Inc., Anchorage, AK |
| SAS | Sound Analytical Services, Inc., Tacoma, WA |
| SWLB | Southwest Laboratory |
| SWRI | Southwest Resarch Institute, San Antonio, TX |
| SPEC | Spectra Laboratory, Inc., Tacoma, WA |
| TRID | Triangle Laboratories, Inc., Durham, NC |

# TLNOTE

Laboratory Test Note. Refer to LNOTE entry on page 51.

# UNITS

Definition: The UNITS field identifies the Units of Measure for an analytical result.

| Attributes: | C10 |  |
| --- | --- | --- |
| Included in Tables: |  | RESULTS QC |

Guidelines & Restrictions:

- UNITS cannot be left blank.

- UNITS must contain a valid value.

- Blank spikes, blank spike duplicates, matrix spike and matrix spike duplicates must be expressed in absolute units.

- Report surrogates (PARVQ = “SU”) and internal standards (PARVQ = “IN”) with UNITS = “PERCENT.”

- For all analytes reporting as “PERCENT,” enter zero into the LABDL field and REPDL fields, and “NA” into the REPDLVQ field.

- If soil samples are expressed on a dry-weight basis, then percent moisture must be reported and detection limits should be provided on a dry-weight basis. Whenever multiple percent moisture determinations have been performed on a sample, (i.e., one determination for each analytical method), report the percent moisture results (PARLABEL and PARVAL) within the analytical method for that particular ANMCODE. (Note: Not all analytical methods require percent moisture determinations.) When entering percent moisture and solids data, use the PARLABEL and the following UNITS:

Appropriate UNITS for Percent Moisture Parameters:

| PARLABEL | UNITS |
| --- | --- |
| SOLIDVOA | PERCENT |
| MOIST | PERCENT |
| SOLID | PERCENT |
| SS | per unit volume |
| TDS | per unit volume |
| TSO | per unit volume |
| TVS | per unit volume |

Acceptable UNITS Values:

| Code | Description |
| --- | --- |
| %V/V | Percent by volume |
| 1/S | Per Second |
| ACRE FT | Acre feet |
| ACRES | Acres |
| ADMI COLOR | ADMI (American Dye Manufacturers Institute) color |
| BARS | Bars |
| BTU/GAL | British Thermal Units per gallon |
| BTU/LB | British Thermal Units per pound |
| CC | Cubic centimeters |
| CFS | Cubic feet per second |
| CFU/100ML | Colony Forming Units per 100 milliliters |
| CFU/G | Colony Forming Units per gram |
| CFU/ML | Colony Forming Units per milliliter |
| CM | Centimeters |
| CM/HR | Centimeters per hour |
| CM/SEC | Centimeters per second |
| CM/YR | Centimeters per year |
| CM2/SEC | Square centimeters per second |
| COLF/100ML | Coliform bacteria per 100 milliliters |
| COLF/G | Coliform bacteria per gram |
| COLOR UNIT | Color unit |
| COUNT/L | Count per liter |
| CP | Centipoise |
| CST | Centistokes |
| DAY | Days |
| DEG | Degrees |
| DEG C | Degrees Celsius |
| DEG C/HR | Degrees Celsius per hour |
| DEG F | Degrees Fahrenheit |
| DIGITS | Number of digits to the right of the decimal point |
| DOLLARS | Dollars |
| DPY | Drums per year |
| DU | Data Unavailable |
| DYNES/CM | Dynes per centimeter |
| E | Natural logarithm |
| FIBERS/L | Fibers per liter |
| FT | Feet |
| FT CANDLES | Foot candles |
| FT MSL | Feet above mean sea level |
| FT/DAY | Feet per day |
| FT/IN | Feet per inch |
| FT/MIN | Feet per minute |
| FT/SEC | Feet per second |
| FT2 | Square feet |
| FT2/DAY | Square feet per day (cubic feet/day-foot) |
| FT2/MIN | Feet squared per minute (for units of transmissivity) |
| FT3 | Cubic feet |
| FT3/YR | Cubic feet per year |
| G | Grams |
| G/CC | Grams per cubic centimeter |
| G/G | Grams per gram |
| G/KG | Grams per kilogram |
| G/L | Grams per liter |
| G/M2/YR | Grams per square meter per year |
| G/M3/DAY | Grams per cubic meter per day |
| G/ML | Grams per milliliter |
| GAL | Gallons |
| GAL/MIN | Gallons per minute |
| GPD | Gallons per day |
| GPD/FT | Gallons per day per foot |
| GPD/FT2 | Gallons per day per foot squared |
| GPM/FT | Gallons per minute per foot |
| GPY | Gallons per year |
| HRS | Hours |
| HRS/DAY | Hours per day |
| IN | Inches |
| IN(HG) | Inches of mercury |
| IN/DAY | Inches per day |
| IN/FT | Inches per foot |
| IN/HR | Inches per hour |
| IN/IN | Inches per inch |
| IN/WK | Inches per week |
| IN2/FT | Square inches per foot |
| IN3 | Cubic inches |
| JCU | Jackson Candle Units |
| JTU | Jackson Turbidity Units |
| KG/1000GAL | Kilograms per 1000 gallons |
| KG/BATCH | Kilograms per batch |
| KG/DAY | Kilograms per day |
| KG/M3 | Kilogram per meter cubed |
| KG/M3/S | Kilogram per meter cubed per second |
| KG/S | Kilogram per second |
| KM2 | Square kilometers |
| KNOTS | Knots |
| L | Liter |
| LAIU | Langelier Index Units |
| LB/1000LB | Pounds per thousand pounds |
| LB/BARREL | Pound per barrel |
| LB/IN2 | Pounds per square inch |
| LB/TON | Pounds per ton |
| LBS | Pounds |
| LBS/DAY | Pounds per day |
| LBS/MON | Pounds per month |
| LBS/YR | Pounds per year |
| M | Meter |
| M/S | Meter per second |
| M2 | Meter squared |
| M2/S | Meter squared per second |
| M3 X 10(6) | Meter cubed (in millions) |
| M3/KG | Meter cubed per kilogram |
| M3/S | Meter cubed per second |
| MEQ/100G | Milliequivalents per 100 grams |
| MG | Milligrams |
| MG/CM2 | Milligrams per centimeter squared |
| MG/FLT | Milligrams per filter |
| MG/G | Milligrams per gram |
| MG/KG | Milligrams per kilogram |
| MG/L | Milligrams per liter |
| MG/M2 | Milligrams per square meter |
| MG/M2/DAY | Milligrams per meter squared per day |
| MG/M3 | Milligrams per cubic meter (PPBV) |
| MG/ML | Milligrams per milliliter |
| MGAL | Million gallons |
| MGCACO3/L | Milligrams of calcium carbonate per liter |
| MGD | Millions of gallons per day |
| MGDO/L | Milligrams dissolved oxygen per liter |
| MGM | Millions of gallons per month |
| MGY | Millions of gallons per year |
| MILE2 | Square miles |
| MILES | Miles |
| MILL FT3 | Million feet cubed |
| MILLIVOLTS | Millivolts |
| MIN | Minutes |
| ML | Milliliter |
| ML/L | Milliliter per liter |
| MM | Millimeter |
| MM/M2/HR | Millimeter per meter squared per hour |
| MM/YR | Millimeter per year |
| MMHOS/CM | Millimhos (MMHOS) per centimeter |
| MOL % | Mole percent |
| MON | Month |
| MPH | Miles per hour |
| MPN/100ML | Most Probable Number per 100 milliliters |
| MPN/G | Most Probable Number per gram |
| MS/CM | Microsiemens per centimeter |
| NAUT.MILE | Nautical mile |
| NG | Nanogram |
| NG/CC | Nanogram per cubic centimeter |
| NG/G | Nanograms per gram |
| NG/KG | Nanogram per kilogram |
| NG/L | Nanogram per liter |
| NG/M3 | Nanogram per cubic meter |
| NG/SAMPLE | Nanograms per sample |
| NONE | No unit of measure |
| NTU | Nephelometric Turbidity Units |
| OZ | Ounces |
| PCF | Pounds per cubic foot |
| PCI/G | PicoCuries per gram |
| PCI/L | PicoCuries per liter |
| PER LOSS | Percent loss |
| PER WGT | Percent by weight |
| PERCENT | Percent |
| PG | Picogram |
| PG/G | Picogram per gram |
| PG/KG | Picograms per kilogram |
| PG/L | Picogram per liter |
| PH UNITS | pH units |
| PPB | Parts per billion |
| PPBV | Parts per billion by volume |
| PPM | Parts per million |
| PPMB | Parts per million, benzene equivalent (for soil/gas) |
| PPMC | Parts per million carbon |
| PPMM | Parts per million, methane equivalent (for soil/gas) |
| PPMV | Parts per million by volume |
| PPQ | Parts per quadrillion |
| PPT | Parts per trillion |
| PPTV | Parts per trillion by volume |
| PSF | Pounds per square foot |
| PSI | Pounds per square inch |
| S | Second |
| T.O.N. | Threshold Odor Number |
| TONS/DAY | Tons per day |
| UG | Micrograms |
| UG/100CM2 | Micrograms per 100 square centimeters |
| UG/CM2 | Micrograms per centimeter squared |
| UG/G | Micrograms per gram |
| UG/KG | Micrograms per kilogram |
| UG/L | Micrograms per liter |
| UG/M3 | Micrograms per cubic meter |
| UG/ML | Micrograms per milliliter |
| UG/SAMPLE | Micrograms per total air sample taken |
| UG/WIPE | Micrograms per wipe |
| UG/YR | Micrograms per year |
| UMHOS/CM | Micromhos (UMHOS) per centimeter |
| UMOLES/G | Micromoles per gram |
| UPY | Units per year |

# UPPERCL

Definition: The Upper Control Limit is the upper limit of a quality control acceptance criterion.

| Attributes: | N4 |  |
| --- | --- | --- |
| Included in Tables: |  | CL |

Guidelines & Restrictions:

- UPPERCL must be an integer greater than or equal to one and less than or equal to 9999.

- UPPERCL must be greater than LOWERCL.