# Proposed Amendments to the Water Quality Control Plan for the Central Coastal Basin to Establish Water Quality Objectives for Disinfection Byproducts and Disinfectant Residuals

Draft Project Report

April 15, 2026

California Regional Water Quality Control Board, Central Coast Region
California Environmental Protection Agency

State of California

Gavin Newsom, Governor
Yana Garcia, Secretary for Environmental Protection

California State Water Resources Control Board

E. Joaquin Esquivel, Chair
Dorene D’Adamo, Vice Chair 
Sean Maguire, Member
Laurel Firestone, Member
Nichole Morgan, Member

Eric Oppenheimer, Executive Director

California Regional Water Quality Control Board, Central Coast Region

Jane Gray, Chair
Dominic Roques, Vice Chair
Alex Rodriguez, Member
Anne Hoskins, Member
Dr. Jean-Pierre Wolff, Member
Stephanie Harlan, Member

Ryan E. Lodge, Executive Officer
Angela Schroeter, Assistance Executive Officer

895 Aerovista Place, Suite 101, San Luis Obispo, California 93401
Phone: (805) 549-3147
http://www.waterboards.ca.gov/centralcoast

Table of Contents

rI.	Introduction	4

I.I.	The Basin Plan	4

I.II.	Amending the Basin Plan	4

I.III.	Triennial review of the Basin Plan	4

II.	Overview	5

II.I.	Scope and content	5

II.II.	Necessity	5

II.III.	Purpose	6

II.IV.	Antidegradation	6

II.V.	Need for peer review	7

III.	Proposed Changes	7

III.I.	Disinfection Byproducts and Disinfectant Residuals	7

III.II.	Editorial corrections and clarifications	8

IV.	Environmental Analysis	91

V.	References	92

VI.	Appendix – CEQA Environmental Checklist	93

## Introduction

### I.I.	The Basin Plan

The Water Quality Control Plan for the Central Coastal Basin (Basin Plan) was first adopted in 1975 and is periodically amended by the California Regional Water Quality Control Board, Central Coast Region (Central Coast Water Board). The most recent Basin Plan edition (2024) is available on the Central Coast Water Board’s Basin Plan Program website.

The Basin Plan establishes designated uses (beneficial uses) for surface waters and groundwaters and the water quality that must be maintained to support those uses (water quality objectives). The Basin Plan describes the programs, projects, prohibitions, and other implementation actions that are necessary to achieve water quality objectives, summarizes California State Water Resources Control Board (State Water Board) and Central Coast Water Board plans and policies to protect water quality, and describes statewide and regional surveillance and monitoring assessment programs.

The Basin Plan forms the basis for regulatory actions taken by Central Coast Water Board to protect waters of the state and to ensure compliance with applicable federal and state laws, including the federal Clean Water Act (CWA) and the California Water Code (CWC). Section 303 of the CWA requires states to adopt water quality standards, which consist of three parts (1) the designated uses of waters, (2) water quality criteria (referred to as “water quality objectives” in California) necessary to protect those designated uses, and (3) an antidegradation policy. Under CWC section 13240, each regional water quality control board is required to formulate and adopt a water quality control plan (i.e., basin plan) for all areas within their region.

### I.II.	Amending the Basin Plan

In addition to requiring the adoption of a water quality control plan, CWC section 13240 requires them to be periodically reviewed and revised. Amendments to the Basin Plan are adopted by the Central Coast Water Board and are subsequently approved by the State Water Board and the California Office of Administrative Law. Additionally, the United States Environmental Protection Agency must approve any water quality control plan amendments that involve changes to water quality standards for surface waters.

### I.III.	Triennial review of the Basin Plan

Section 303(c) of the CWA requires water quality standards be reviewed at least once every three years in a “triennial review.” The triennial review results in a prioritized list of proposed Basin Plan amendments (referred to as proposals) that if undertaken, could improve the Basin Plan’s clarity and usefulness. The Central Coast Water Board’s most recent triennial review of the Basin Plan was in October 2024. The Central Coast Water Board initiated this amendment to implement Proposal 28: Update Reference to Maximum Contaminant Levels that was prioritized during the 2024 Triennial Review.

## Overview

This section provides the scope and content, objectives, and necessity of the Basin Plan amendments, as well as an antidegradation discussion and explains why external scientific peer review is not needed.

### II.I.	Scope and content

The proposed Basin Plan amendments include regulatory and non-regulatory amendments. The regulatory amendments incorporate maximum contaminant levels (MCLs) for disinfection byproducts and maximum residual disinfectant levels (MRDLs) for disinfectant residuals from the California Code of Regulations (CCR), title 22, or, where more protective, MCL goals (MCLGs) and MRDL goals (MRDLGs) from title 40 of the Code of Federal Regulations (CFR) (40 CFR), as numeric water quality objectives for surface waters and groundwaters designated with the Municipal and Domestic Supply (MUN) beneficial use. MCLGs and MRDLGs are health-based and recommended by the United States Environmental Protection Agency (USEPA) for use as water quality standards for untreated water when they are more protective than MCLs and MRDLs. The non-regulatory amendments make editorial corrections and clarifications (e.g., correcting typos, updating information, and following the Central Coast Water Board writing style) for improved accuracy and clarity.

The regulatory amendments are subject to the California Environmental Quality Act (CEQA). The non-regulatory amendments are editorial in nature and, therefore, not subject to CEQA. The non-regulatory amendments do not materially alter any requirement, right, responsibility, condition, prescription, or other regulatory element of the Basin Plan, and are therefore without regulatory effect under CCR, title 1, section 100, subdivision (a).

### II.II.	Necessity

The CWC division 7, section 13241 specifies that each regional water quality control board shall establish water quality objectives that are necessary for the reasonable protection of beneficial uses. The Central Coast Water Board has established water quality objectives for the protection of the MUN beneficial use in the Basin Plan. The Central Coast Water Board has incorporated MCLs for inorganic chemicals, radionuclides and organic chemicals (CCR, title 22, sections 64431, 64433.2, 64442, 64443, and 64444) as numeric water quality objectives for the MUN beneficial use but has not yet incorporated MCLs for disinfection byproducts or MRDLs for disinfectant residuals (CCR, title 22, sections 64533 and 64533.5). The Central Coast Water Board has determined that the Basin Plan needs to incorporate MCLs for disinfection byproducts and MRDLs for disinfectant residuals or, where more protective, MCLGs and MRDLGs, for the protection of domestic water supplies. The amendments provide additional protection for public health and make the Basin Plan a more effective regulatory instrument.

Editorial corrections and clarifications improve the accuracy and clarity of the Basin Plan. These changes ensure that staff and members of the public have a common understanding of foundational information in the Basin Plan.

A detailed discussion of proposed amendments is provided in section III of this project report. This discussion contains substantial evidence for the need for these Basin Plan amendments, based on facts, studies, or expert opinion, as required by the California Government Code section 11349(a).

### II.III.	Purpose

The purpose of the regulatory amendments is to establish numeric water quality objectives for disinfection byproducts and disinfectant residuals for surface waters and groundwaters designated with the MUN beneficial use to protect domestic water supplies and public health. The purpose of the non-regulatory amendments is to improve the accuracy and clarity of the Basin Plan. The purpose of the proposed Basin Plan amendments is consistent with the mission of the State Water Board and the requirements of the CWA and the CWC. These laws require the Central Coast Water Board to preserve, enhance, and restore the quality of water in the Central Coast region.

### II.IV.	Antidegradation

The Basin Plan amendments must comply with the requirements of the State Water Board’s Antidegradation Policy (State Water Board Resolution 68-16) and the federal antidegradation regulations included in 40 CFR, part 131.12. Under the State Water Board’s Antidegradation Policy, the quality of some of the waters of the state are higher than the quality that would be established by adopted policies. The proposed amendments will not result in degradation of water quality in waters currently achieving high water quality. Moreover, the regulatory Basin Plan amendments may maintain and protect the existing Municipal and Domestic Water Supply (MUN) beneficial use and associated water quality.

### II.V.	Need for peer review

California Health and Safety Code section 57004(d) requires an external scientific peer review for the scientific portion of a proposed rule. The proposed Basin Plan amendments, however, do not include a “scientific portion,” and thus do not require a scientific peer review. The proposed regulatory amendments incorporate MCLs for disinfection byproducts and MRDLs for disinfectant residuals from the CCR, title 22, or, where more protective, MCLGs and MRDLGs from 40 CFR that underwent scientific peer review prior to their adoption., The proposed non-regulatory amendments are editorial in nature, without regulatory effect.

According to the 1998 Unified California Environmental Protection Agency Policy and Guiding Principles for External Scientific Peer Review document (Cal/EPA 1998), no peer review is necessary when the “work product is a new application of an adequately peer reviewed work product [which] does not depart significantly from its scientific approach.” No scientific methodologies or evaluations were conducted in the development of this Basin Plan amendment. For these reasons, no peer review is necessary for these Basin Plan amendments.

## Proposed Changes

The Basin Plan amendments are organized into two categories based on the purpose: (I) to incorporate water quality objectives for disinfection byproducts and disinfectant residuals (MCLs and MRDLs or MCLGs and MRDLGs), or (II) to make editorial corrections and clarifications. Within these two categories, proposed amendments are organized by Basin Plan section. The removal of text is shown in dark red strikethrough (i.e.,) and the addition of text is shown in dark red underline (i.e., dark red underline).

### III.I.	Disinfection Byproducts and Disinfectant Residuals

Proposed Basin Plan amendments incorporate MCLs for disinfection byproducts and MRDLs for disinfectant residuals from the CCR, title 22, or, where more protective, MCLGs and MRDLGs from 40 CFR, as numeric water quality objectives for the MUN beneficial use to protect domestic water supplies and human health. Water quality objectives for surface waters designated with the MUN beneficial use are proposed for Basin Plan section 3.3.2.2., and water quality objectives for groundwater designated with the MUN beneficial use are proposed for Basin Plan section 3.3.4.2.

#### Proposed changes to Basin Plan section 3.3.2.2, below “Municipal and Domestic Supply (MUN)”

Disinfection Byproducts and Disinfectant Residuals

Waters shall not contain concentrations of disinfection byproducts or disinfectant residuals in excess of the primary MCLs and maximum residual disinfectant levels (MRDLs) for drinking water, as specified in CCR, title 22, division 4, chapter 15.5, article 2, sections 64533 and 64533.5, or, where lower and therefore more protective, the MCL goal (MCLG) or MRDL goal (MRDLG), as specified in 40 CFR sections 141.53 and 141.54, unless lower water quality objectives have been adopted by the Central Coast Water Board or the State Water Board. Compliance with objectives that are lower than their detection limit for purposes of reporting (DLR) will be based on the DLR. This incorporation by reference is prospective, including future changes to the incorporated provisions as the changes take effect.

#### Proposed changes to Basin Plan section 3.3.4.2, below “Municipal and Domestic Supply (MUN)”

Disinfection Byproducts and Disinfectant Residuals

Groundwaters shall not contain concentrations of disinfection byproducts or disinfectant residuals in excess of the primary MCLs and MRDLs for drinking water, as specified in CCR, title 22, division 4, chapter 15.5, article 2, sections 64533 and 64533.5, or, where lower and therefore more protective, the MCLG or MRDLG, as specified in 40 CFR sections 141.53 and 141.54, unless lower water quality objectives have been adopted by the Central Coast Water Board or the State Water Board. Compliance with objectives that are lower than their DLR will be based on the DLR. This incorporation by reference is prospective, including future changes to the incorporated provisions as the changes take effect.

### III.II.	Editorial corrections and clarifications

Proposed changes correct typos, update information, or follow the Central Coast Water Board writing style. These changes improve the accuracy and clarity of the Basin Plan for Water Board staff and interested persons. These changes create consistency within the Basin Plan so that there can be a common understanding of terminology and how data and information is referenced by Water Board staff. Proposed changes include:

More accurately describing the Central Coast region boundaries

Defining, correcting, and consistently using acronyms and abbreviations (e.g., California Water Code (CWC), Clean Water Act (CWA))

Updating Table 2-1 waterbody names for consistency with other State Water Board databases (e.g., Lockhart Gulch instead of Lockhart Gulch creek, Kelly Lake instead of Kelley Lake)

Correcting Table 2-1 waterbody names and hydrologic unit (HU) location (e.g., correcting the spelling of Werner Lake and listing the waterbody under Bolsa Nueva HU instead of Pajaro River HU)

Revising Table 2-2 for consistency with the State Water Board’s Water Quality Control Plan for Ocean Waters of California (Ocean Plan)

#### Proposed changes to Basin Plan List of Appendices

A-2	Statement of Policy with Respect to Maintaining High Quality of Waters in California (Antidegradation Policy). State Water Board Resolution 68-16.

#### Proposed changes to Basin Plan after List of Appendices

List of Acronyms and Abbreviations

ASBS			Area of Special Biological Significance

BLM			United States Bureau of Land Management

BMPs			Best Management Practices

CalEPA		California Environmental Protection Agency

Caltrans		California Department of Transportation

CAO			Cleanup and Abatement Order

CCAMP 		Central Coast Ambient Monitoring Program

CCAMP-GAP	Central Coast Ambient Monitoring Program – Groundwater Assessment and Protection

CCC			Criterion Continuous Concentration

CCR 			California Code of Regulations

CDFW 		California Department of Fish and Wildlife

CDO			Cease and Desist Order

CFR			Code of Federal Regulations

CMC			Criterion Maximum Concentration

CSA			county service area

CWA			Clean Water Act

CWC			California Water Code

DHCS			California Department of Health Care Services

DLR			detection limit for purposes of reporting

DoD			United States Department of Defense

DPR			California Department of Pesticide Regulation

DWR			California Department of Water Resources

EC			electrical conductivity

HA			hydrologic area

HSA			hydrologic subarea

HU			hydrologic unit

HUC			hydrologic unit code

LA			load allocation

LCPs			Local Coastal Programs

MCL			maximum contaminant level

MCLG			maximum contaminant level goal

MPN			most probable number

MRDL			maximum residual disinfectant level

MRDLG		maximum residual disinfectant level goal

meq/L			milliequivalents per liter

mgd			million gallons per day

mg/L			milligram per liter

mL			milliliter

MS4			municipal separate storm sewer system

µS/cm			microSiemens per centimeter

µg/L			micrograms per liter

NPDES		National Pollutant Discharge Elimination System

NPS			nonpoint source

NRCS			Natural Resources Conservation Service

NTU			nephelometric turbidity units

pH			potential of hydrogen

ppm			parts per million

RCD			Resource Conservation District

SAR			sodium absorption ratio

SWAMP		Surface Water Ambient Monitoring Program

SWMP		Stormwater Management Plan

SWPPP		Stormwater Pollution Prevention Plan

SWQPA		State Water Quality Protection Area

TDS			total dissolved solids

TIE			toxicity identification evaluation

TMDL			Total Maximum Daily Load

TSO			Time Schedule Order

TST			Test of Significant Toxicity

UCCE			University of California Cooperative Extension

US			United States

USEPA		United States Environmental Protection Agency

USFS			United State Forest Service

USFWS		United States Fish and Wildlife Service

USGS			United States Geological Survey

WDR			waste discharge requirement

WLA			waste load allocation

WMU			Waste Management Unit

WOTUS		waters of the United States

#### Proposed changes to Basin Plan section 1.1, paragraph 1

The objective of this Water Quality Control Plan for the Central Coastal Basin (Basin Plan) is to show how the quality of surface waters and groundwaters in the Central Coast region should be managed to provide the highest water quality reasonably possible.

#### Proposed changes to Basin Plan section 1.1, paragraph 2

Federal terminology is somewhat different, in that beneficial uses and water quality objectives are combined and the combination is called water quality standards. Chapter Four, Implementation Plan, then describes the programs, projects, and other actions that are necessary to achieve the water quality standards established in this plan. Chapter Five, Plans and Policies, summarizes State Water Resources Control Board (State Water Board) and Regional Water Quality Control Board (Regional Water Board) plans and policies to protect water quality.

#### Proposed changes to Basin Plan section 1.1, paragraph 3

These requirements can be either state waste discharge requirements (WDRs) for discharges to land or federally delegated National Pollutant Discharge Elimination System (NPDES) permits for point source discharges to surface waters.

#### Proposed changes to Basin Plan section 1.1, paragraph 4

Public works or other projects that can affect water quality are reviewed and their impacts identified. Proposals that implement or help achieve the goals of the Basin Plan are supported; the Regional Water Board makes water quality control recommendations for other projects.

#### Proposed changes to Basin Plan section 1.2, paragraphs 1 and 2

California’s Porter-Cologne Water Quality Control Act (1969), which became division 7 (Water Quality) of the California Water Code (CWC), establishes the responsibilities and authorities of the nine Regional Water Boards (previously called Water Pollution Control Boards) and the State Water Board. The Porter-Cologne Act names the State Water Board and the Regional Water Boards “…the principal State agencies with primary responsibility for the coordination and control of water quality” (section 13001). Each Regional Water Board is directed to “…formulate and adopt water quality control plans for all areas within the region.” A water quality control plan for the waters of an area is defined as having three components: beneficial uses which are to be protected, water quality objectives which protect those uses, and an implementation plan which accomplishes those objectives (section 13050). Further, “such plans shall be periodically reviewed and may be revised” (section 13240). The federal Clean Water Act (CWA) (Public Law 92-500, as amended) provides for the delegation of certain responsibilities in water quality control and water quality planning to the states. Where the United States Environmental Protection Agency (USEPA) and the State Water Board have agreed to such delegation, the Regional Water Boards implement portions of the CWA, such as the NPDES program and toxic substance control programs.

The Porter-Cologne Act and CWA also describe how enforcement of waste discharge regulations is to be carried out. Enforcement tools available to the Regional Water Board range from simple letters to the discharger, through formal Regional Water Board order, and direct penalty assessments, to judicial abatement for civil and/or criminal penalties. Legally noticed public hearings are required for most actions, but some enforcement actions (e.g., Cleanup and Abatement Orders) have been delegated to staff to allow for a quicker response than regularly scheduled Regional Water Board meetings can provide.

#### Proposed changes to Basin Plan section 1.3, paragraph 1

The California Regional Water Quality Control Board, Central Coast Region (Central Coast Water Board) is one of nine Regional Water Boards in California and has jurisdiction over a 300-mile-long by 40-mile-wide section of the state’s Central Coast. Its geographic area encompasses all of Monterey County and major portions of Santa Cruz, San Benito, San Luis Obispo, and Santa Barbara counties (including San Miguel, Santa Rosa, and Santa Cruz Channel Islands), the southern one-third of Santa Clara County, and small portions of San Mateo, Kern, and Ventura counties. Figure 1-1 shows the Central Coast Water Board’s boundary. Included in the region are urban areas such as the Monterey Peninsula and the Santa Barbara coastal plain; prime agricultural lands such as the Salinas, Santa Maria, and Lompoc Valleys; National Park and National Forest lands; National Marine Sanctuaries; and wet areas like the Santa Cruz mountains as well as arid areas like the Carrizo Plain. Some physical characteristics of the Central Coast region are listed below:

Central Coast Region1

| Characteristics | Number | Measure |
| --- | --- | --- |
| Area of Region | — | 11,274 square miles |
| Streams | Unknown | 2,360 miles |
| Lakes | 99 | 25,040 acres |
| Groundwater Basins | 53 | 3,559 square miles |
| Mainland Coast | — | 378 miles |
| Wetlands and Estuaries | 59 | 8,387 acres |
| Areas of Special Biological Significance2 | 7 | 293,971 acres |

1 Water Quality Assessment for Water Years 1986 and 1987, Water Quality Monitoring Report Number 88-1 Water Quality, Division of Water Quality, State Water Resources Control Board, July 1988.

2 State Water Board’s State Water Quality Protection Areas map: https://www.waterboards.ca.gov/water_issues/programs/ocean/asbs.html

#### Proposed changes to Basin Plan section 1.3, paragraphs 8 to 12

The Central Coast region receives three times the volume of average annual precipitation (12,090,000 acre-feet) as the Los Angeles region but has one-seventh the population (1.5 million versus 11 million). The North Coast region receives 52 million acre-feet of precipitation on average with a population of 460,000. These three regions demonstrate the range of California’s water and population distribution imbalance:

| Region | Annual Average Precipitation (Ac. Ft. per Person) |
| --- | --- |
| North Coast | 113.0 |
| Central Coast | 9.9 |
| Los Angeles | 0.56 |

Although this table shows the Central Coast is somewhat in the middle of the state’s water-versus-population distribution, the region is considered arid for the most part. An exception is the Santa Cruz mountain area with its relatively high average precipitation.

Total population of the region is estimated to be more than 1.5 million people as of 2020:

Central Coast Region Population

| County | 1970 | 1988 | 20202 |
| --- | --- | --- | --- |
| Santa Cruz | 124,000 | 225,400 | 270,861 |
| San Benito | 18,000 | 34,100 | 64,209 |
| Monterey | 249,000 | 346,100 | 439,035 |
| San Luis Obispo | 107,000 | 204,300 | 282,424 |
| Santa Barbara | 265,000 | 345,000 | 448,229 |
| Total1 | 763,000 | 1,154,900 | 1,504,758 |

1 Table does not include relatively small populations of portions of Ventura, Kern, Santa Clara, and San Mateo Counties that are within the Central Coast Region.

2 Data retrieved from the United States Census Bureau (January 2025): https://data.census.gov/

Adequate quality water for many beneficial uses in the Central Coast region is in short supply. Water rationing for domestic purposes is seriously considered and sometimes implemented during water shortages. The use of water by the human population and its activities is increasing in the region. Water mining and seawater intrusion have resulted in some locations. Consequently, the competition for waters of adequate quality will become more intense in the future.

Water quality problems most frequently encountered in the Central Coast region pertain to excessive salinity or hardness of local groundwaters. Groundwater basins containing 1,000 mg/L Total Dissolved Solids (TDS) or higher are found near Hollister, the Lower Forebay of the Salinas Valley Basin, the Carrizo Plain, the Santa Maria and Cuyama Valleys, San Antonio Creek Valley, Lompoc and Santa Rita Basins of the Santa Ynez River Valley, and Goleta and Santa Barbara. The Carrizo Plain groundwaters are most highly mineralized averaging over 5,000 mg/L TDS. Increasing nitrate concentrations is a problem in the Salinas Groundwater Basin, the Los Osos Valley Basin, the Santa Maria Valley Basin, and near Arroyo Grande. Surface water problems are less frequently evident, although bacteriological contamination of coastal waters has been a problem in Morro Bay and South Santa Barbara County. Eutrophication occurs in Pajaro River and Llagas Creek, Salinas River below Spreckels, and in the lower reaches of San Luis Obispo Creek. Some streams in the basin are naturally highly mineralized and contribute to the excessive salinity of local groundwaters; examples include Pancho Rico Creek in the Salinas River Hydrologic Unit, and the Cuyama River in the Santa Maria Hydrologic Unit. Both surface waters contain  more than 1,000 mg/L TDS.

#### Proposed changes to Basin Plan section 1.4, paragraphs 1 and 2

The Central Coast Water Board consists of seven members appointed by the Governor to serve staggered four-year terms. Members must reside or maintain a place of business within the Central Coast region and must be associated with or have special knowledge of specific activities related to the control of water quality. Members of the Central Coast Water Board conduct their business at regular meetings and public hearings at which public participation is encouraged.

All duties and responsibilities of the Central Coast Water Board are directed at providing reasonable protection and enhancement of the quality of all waters in the Central Coast region, both surface waters and groundwaters. The programs by which these duties and responsibilities are carried out include:

- Preparing new or revised policies addressing regionwide water quality concerns;

- Adopting, monitoring compliance with, and enforcing Orders;

- Implementing and enforcing water quality laws, regulations, policies, plans, and prohibitions to protect human health, water quality, and the environment;

- Developing water quality standards, assessing water quality standards attainment, and developing watershed and waterbody plans to address problem pollutants and improve water quality;

- Providing recommendations to the State Water Board on financial assistance programs, proposals for water diversion, budget development, and other statewide programs and policies;

- Coordinating with the State Water Board and other public agencies that are concerned with water quality control; and

- Informing and involving the public on water quality issues.

More information about Central Coast Water Board programs is available on the Central Coast Water Board’s website at https://www.waterboards.ca.gov/centralcoast/.

#### Proposed changes to Basin Plan section 1.5, paragraphs 1 to 6

Prior to 1970, the Regional Water Board did not have an active water quality planning function. Water quality problems in surface waters and groundwaters were responded to by setting controls on discharges. Those discharge controls generally consisted of limiting the allowable increases in TDS concentrations and certain other parameters. Normally, the only additional requirement specified by the Regional Water Board was that the discharge could not create a nuisance or pollution.

At the request of the federal Water Quality Administration, predecessor to the USEPA (and successor to the federal Water Pollution Control Administration), the so-called 1967 Standards were developed and published. These standards applied to coastal and estuarine waters.

By 1970, the Central Coast Water Board was actively involved in the formulation of plans to meet established water quality objectives. The CWA and Porter-Cologne Act, requiring regionwide planning on order to qualify for state and federal funding, plus the National Pollution Discharge Elimination System (NPDES), which empowers the states to set discharge standards, placed new tools in the hands of the Regional Water Boards and encouraged the development of new approaches to water quality management.

The first single plan for the Central Coast region was the 1971 Interim Water Quality Control Plan. It represented significant progress in that the 1967 Standards were incorporated and standards were designated for fresh water streams as well.

Following adoption of the 1971 Interim Plan, the State Water Board developed and adopted the Water Quality Control Plan for Ocean Waters of California (Ocean Plan) and the Water Quality Control Plan for Control of Temperature in the Coastal and Interstate Waters and Enclosed Bays and Estuaries of California (Thermal Plan). The Central Coast Water Board expanded objectives for municipal and domestic water supplies. Chemical objectives for the San Lorenzo River Subbasin (i.e., the Big Basin Hydrological Unit) were made more stringent. Incorporation of these State Water Board plans and Central Coast Water Board revisions produced the Revised Interim Water Quality Control Plan of 1973.

Work then began in earnest on a complete Water Quality Control Plan, the 1975 Basin Plan, which has been the foundation of the Central Coast Water Board’s planning operations since its adoption in 1975. Basin Plans were being developed statewide at that time under the direction of the State Water Board. In this region, the prime contractors for basin planning were Brown and Caldwell Consulting Engineers; Water Resources Engineers, Inc.; and Yoder, Trottner, Orlob and Associates. Water quality objectives were based largely on existing water quality.

#### Proposed changes to Basin Plan section 1.6, paragraph 1

The CWA (section 303(c)) requires states to hold public hearings for review of water quality standards at least once every three years. Water quality standards consist of beneficial use designations, water quality criteria (objectives) necessary to protect those uses, and consideration of antidegradation requirements to protect existing uses and high-quality waterbodies.

Proposed changes to Basin Plan section 1.6, paragraphs 2 to 4

At the conclusion of the triennial review public hearing, Regional Water Board staff prepares a priority list of issues that may enhance water quality protection. Placing an issue on the priority list only requires Regional Water Board staff to investigate the need for an amendment. It does not necessarily mean a revision of the Basin Plan will be made.

Other items completed after the triennial review public hearing include:

Detailed workplans of each issue;

Regional Water Board identification of issues that can be completed with existing resource allocations over a three-year period; and

List of issues requiring additional resources to complete.

Once the triennial review process is complete, Regional Water Board staff begins investigating the issues in order of rank. After each investigation, staff determines the need for a Basin Plan amendment.

Proposed changes to Basin Plan section 1.6, paragraphs 6 and 7

Public notices for Basin Plan amendment meetings are posted in newspapers circulated in affected areas. Persons interested in a particular issue can also notify Regional Water Board staff of their interest.

Basin Plan amendments do not become effective until approved by the State Water Board and the Office of Administrative Law (OAL). Changes to surface water standards also require USEPA approval to become effective.

#### Proposed changes to Basin Plan section 1.7

The Vision for the Central Coast Water Board is Healthy Watersheds. The Vision represents a framework for how the Central Coast Water Board implements the CWC and the Basin Plan and is intended to achieve measurable results in water quality and watershed improvements over time. The Vision creates a structure to focus the Central Coast Water Board on the highest priorities for beneficial use protection and more strategically aligns the Central Coast Water Board with current and future challenges and opportunities in watershed protection.

Consistent with the Vision, the Central Coast Water Board will continue to focus on program priorities and consider metrics to evaluate progress towards the following measurable goals:

Healthy Aquatic Habitat

Sustainable Land Management

Clean Groundwater

Consistent with the Vision, the Central Coast Water Board is also committed to implementing the following regional priorities:

Central Coast Water Board Resolution R3-2017-0004, Adopting the Human Right to Water as a Core Value and Directing Its Implementation in Central Coast Water Board Programs and Activities

Central Coast Water Board Resolution R3-2023-0002, Condemning Racism, Xenophobia, Bigotry, and Racial Injustice and Strengthening Commitment to Racial Equity, Diversity, Inclusion, Access, and Anti-Racism in the Central Coast Region

State Water Board Resolution 2017-0012, Comprehensive Response to Climate Change

State Water Board Tribal Consultation Policy (June 2019)

#### Proposed changes to Basin Plan Chapter 2, paragraphs 1 and 2

State policy for water quality control in California is directed toward achieving the highest water quality consistent with maximum benefit to the people of the state (Appendix A-1). Therefore, all water resources must be protected from pollution and nuisance that may occur as a result of waste discharges.

Establishing beneficial uses to be protected in the Central Coast region is a cornerstone of this comprehensive plan. Once beneficial uses are established, compatible water quality objectives can be established as well as the level of treatment necessary to ensure protection of beneficial uses. This chapter will examine and identify historical, present, and potential beneficial uses in the region.

#### Proposed changes to Basin Plan section 2.1, paragraphs 1 to 4

Beneficial uses are presented for inland surface waters, enclosed bays, and estuaries in Table 2-1. Beneficial uses for inland surface waters are arranged by hydrologic unit. A map of the hydrologic units is shown in Figure 2-1, and a table of hydrologic units is shown in Table 2-3. Beneficial uses are regarded as existing whether the waterbody is perennial or ephemeral, or the flow is intermittent or continuous. The Ocean Plan establishes beneficial uses for Pacific Ocean waters adjacent to the California Coast outside of enclosed bays, estuaries, and coastal lagoons, and extending three nautical miles into the Pacific Ocean from the line of mean lower low water marking the seaward limits of inland waters and three nautical miles from the line of mean lower low water on the mainland and each offshore island (CWC section 13200). Beneficial uses of ocean waters are shown in Table 2-2.

Surface waterbodies within the Central Coast region that do not have beneficial uses designated for them in Table 2-1 are assigned the following designations:

Municipal and Domestic Water Supply

Protection of both recreation and aquatic life.

Municipal and Domestic Water Supply is designated in accordance with the provisions of State Water Board Resolution 88-63, Sources of Drinking Water Policy. These MUN designations in no way affect the presence or absence of other beneficial use designations in these waterbodies.

Groundwater throughout the Central Coastal region, except for that found in the Carrizo Plain groundwater basin, is suitable for agricultural water supply, municipal and domestic water supply, and industrial use. Groundwater basins, adapted from the California Department of Water Resources (DWR) 2003 Bulletin 118, are listed in Table 2-4. A map showing these groundwater basins is displayed in Figure 2-2.

Proposed changes to Basin Plan section 2.2.1, paragraph 1

Uses of water for community, military, or individual water supply systems including, but not limited to, drinking water supply. According to State Water Board Resolution 88-63, Sources of Drinking Water Policy all surface waters and groundwaters of the state are considered suitable, or potentially suitable, for municipal or domestic water supply except where:

TDS exceeds 3,000 mg/L (5,000 μS/cm, electrical conductivity);

Proposed changes to Basin Plan section 2.2.6, paragraph 1

This includes only immediate upstream waterbodies and not their tributaries.

Proposed changes to Basin Plan section 2.2.16, paragraph 1

Included are waterbodies which would naturally fit the definition if not controlled by tide gates or other such devices.

#### Proposed changes to Basin Plan section 2.2.19, paragraphs 1, 2 and 3

Uses of water that support designated areas or habitats, such as established refuges, parks, sanctuaries, ecological reserves, or Areas of Special Biological Significance (ASBS), where the preservation or enhancement of natural resources requires special protection.

ASBS are ocean areas designated by the State Water Board as requiring protection of species or biological communities to the extent that maintenance of natural water quality is assured. Waste discharges to ASBS are prohibited and must be at sufficient distance to ensure the maintenance of natural water quality conditions unless an exception to the Ocean Plan is granted by the State Water Board. Nomination, designation, and implementation provisions for ASBS are in the Ocean Plan.

The Ocean Plan includes the following list of ASBS designated in ocean waters of the Central Coast region, as of 2019:

Año Nuevo, San Mateo County

Pacific Grove, Monterey County

Point Lobos, Monterey County

Carmel Bay, Monterey County

Julia Pfeiffer Burns, Monterey County

Salmon Creek Coast, Monterey County

San Miguel, Santa Rosa, and Santa Cruz Islands, Santa Barbara County

An interactive map of ASBS is available on the State Water Board’s ASBS website.

#### Proposed changes to Basin Plan Table 2-1

Table 2-1. Beneficial Uses of Inland Surface Waters, Enclosed Bays, and Estuaries

| Waterbody Names | M U N | A G R | P R O C | I N D | G W R | R E C 1 | R E C 2 | W I L D | C O L D | W A R M | M I G R | S P W N | B I O L | R A R E | E S T | F R S H | N A V | P O W | C O M M | A Q U A | S A L | S H E L L | C U L | T - S U B | S U B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Big Basin Hydrologic Unit 304 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lucerne Lake Estuary |  |  |  |  |  | X | X | X | X |  |  | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Lucerne Lake | X | X |  |  |  | X | X | X | X |  |  |  |  |  |  | X |  |  | X |  |  |  |  |  |  |
| Arroyo de los Frejoles Creek | X | X |  |  | X | X | X | X | X | X | X | X | X | X |  | X |  |  | X |  |  |  |  |  |  |
| Arroyo de los Frejoles Reservoir | X | X |  |  | X | X | X | X | X | X |  |  |  |  |  | X | X |  | X |  |  |  |  |  |  |
| Gazos Creek Lagoon/Estuary |  |  |  |  |  | X | X | X | X | X | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Gazos Creek | X | X |  |  | X | X | X | X | X |  | X | X |  |  |  | X |  |  | X |  |  |  |  |  |  |
| Old Womans Creek | X |  |  |  |  | X | X | X | X |  | X | X | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Whitehouse Creek | X |  |  |  |  | X | X | X | X |  | X | X | X |  | X | X |  |  | X |  |  |  |  |  |  |
| Cascade Creek Lagoon/Estuary |  |  |  |  |  | X | X | X | X |  | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Cascade Creek | X | X |  |  | X | X | X | X | X |  | X | X | X | X |  | X |  |  | X |  |  |  |  |  |  |
| Green Oaks Creek Lagoon/Estuary |  |  |  |  |  | X | X | X | X |  |  | X |  | X | X |  |  |  | X |  |  | X |  |  |  |
| Green Oaks Creek | X | X |  |  | X | X | X | X | X | X | X | X | X |  | X | X |  |  | X |  |  |  |  |  |  |
| Año Nuevo Creek | X | X |  |  | X | X | X | X | X |  | X | X | X | X | X | X |  |  | X |  |  |  |  |  |  |
| Finney Creek | X | X |  |  |  | X | X | X | X |  |  |  | X |  | X | X |  |  | X |  |  |  |  |  |  |
| Elliot Creek | X | X |  |  |  | X | X | X | X |  |  |  | X |  | X | X |  |  | X |  |  |  |  |  |  |
| Waddell Creek Lagoon |  |  |  |  | X | X | X | X | X |  | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Waddell Creek (Main Stem) | X | X |  | X | X | X | X | X | X |  | X | X | X | X |  | X |  |  | X |  |  |  |  |  |  |
| Waddell Creek, east branch | X |  |  |  | X | X | X | X | X |  | X | X | X | X |  | X |  |  | X |  |  |  |  |  |  |
| Last Chance Creek | X | X |  |  | X | X | X | X | X |  | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Blooms Creek | X |  |  |  | X | X | X | X | X |  |  | X | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Sempervirens Creek | X |  |  |  | X | X | X | X | X |  | X | X | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Union Creek | X |  |  |  |  | X | X | X | X |  |  |  | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Sempervirens Reservoir | X |  |  |  |  | X | X | X | X |  |  |  | X |  |  | X |  |  | X |  |  | X |  |  |  |
| Opal Creek | X |  |  |  | X | X | X | X | X |  |  |  | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Rogers Creek | X |  |  |  |  | X | X | X | X |  |  |  | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Maddock’s Creek | X |  |  |  |  | X | X | X | X |  |  |  | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Waddell Creek, west branch | X |  |  |  | X | X | X | X | X |  | X | X | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Kelly Creek | X |  |  |  | X | X | X | X | X |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Berry Creek (tributary of Waddell Creek West) | X |  |  |  | X | X | X | X | X |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Henry Creek | X |  |  |  | X | X | X | X | X |  |  |  | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Scott Creek Lagoon |  |  |  |  |  | X | X | X | X |  | X | X |  | X | X |  |  |  | X |  |  | X |  |  |  |
| Scott Creek | X | X |  | X | X | X | X | X | X |  | X | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Little Creek | X | X |  | X | X | X | X | X | X |  | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Big Creek | X | X |  | X | X | X | X | X | X |  | X | X |  | X |  |  |  | X | X |  |  |  |  |  |  |
| Berry Creek (tributary of Big Creek) | X |  |  |  | X | X | X | X | X |  |  |  | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Deadman Gulch Creek | X |  |  |  | X | X | X | X | X |  |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Boyer Creek | X |  |  |  | X | X | X | X | X |  |  | X |  |  |  |  |  | X | X |  |  |  |  |  |  |
| Mill Creek (tributary of Scott Creek) | X | X |  | X | X | X | X | X | X |  | X | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Mill Creek Reservoir | X |  |  |  |  | X | X | X | X | X | X | X |  |  |  | X | X |  | X |  |  |  |  |  |  |
| Molino Creek | X | X |  |  | X | X | X | X | X |  |  |  |  |  | X | X |  |  | X |  |  |  |  |  |  |
| San Vicente Creek | X | X | X | X | X | X | X | X | X |  | X | X |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Mill Creek (tributary of San Vicente Creek) | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Liddell Creek | X | X |  |  | X | X | X | X | X |  | X | X |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Liddell Creek, east branch | X | X |  | X | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Liddell Creek, west branch | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Laguna Creek Estuary |  |  |  |  | X | X | X | X | X |  | X | X |  | X | X |  |  |  | X |  |  | X |  |  |  |
| Laguna Creek | X | X |  | X | X | X | X | X | X |  | X | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Reggiardo Creek | X |  |  |  | X | X | X | X | X |  |  |  |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Majors Creek | X | X |  | X | X | X | X | X | X |  | X | X |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Baldwin Creek Estuary |  |  |  |  |  | X | X | X | X | X | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Baldwin Creek | X | X |  |  | X | X | X | X | X |  | X | X | X | X |  | X |  |  | X |  |  |  |  |  |  |
| Wilder Creek Estuary |  |  |  |  |  | X | X | X | X | X | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Wilder Creek | X | X |  |  | X | X | X | X | X | X | X | X | X |  |  | X |  |  | X |  |  |  |  |  |  |
| Cave Gulch | X |  |  |  | X | X | X | X | X | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Younger Lagoon |  |  |  |  | X | X | X | X | X | X |  | X | X |  |  |  |  |  | X |  |  | X |  |  |  |
| Antonellis Pond |  |  |  |  | X | X | X | X |  | X | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Moore Creek | X | X |  |  | X | X | X | X | X | X |  | X | X |  |  | X |  |  | X |  |  |  |  |  |  |
| Nearys Lagoon |  |  |  |  | X | X | X | X |  | X |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| San Lorenzo River Lagoon |  |  |  |  |  | X | X | X | X |  | X | X | X | X | X |  |  |  | X |  |  |  |  |  |  |
| San Lorenzo River | X | X |  | X | X | X | X | X | X |  | X | X | X | X |  | X |  |  | X |  |  |  |  |  |  |
| Branciforte Creek | X | X |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Blackburn Gulch | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Tie Gulch | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Granite Creek | X |  |  | X | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Carbonera Creek | X | X |  | X | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Zayante Creek | X | X |  | X | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Bean Creek | X | X |  | X | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Mackenzie Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Ruins Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Lockhart Gulch | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Mountain Charlie Gulch | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Lompico Creek | X | X |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Mill Creek (tributary of Lompico Creek) | X |  |  |  | X | X | X | X | X |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Newell Creek | X | X |  | X | X | X | X | X | X |  | X | X |  |  |  | X |  | X | X |  |  |  |  |  |  |
| Loch Lomond | X | X |  | X | X | X | X | X | X | X | X | X |  | X |  | X | X |  | X |  |  | X |  |  |  |
| Love Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Fritch Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Smith Creek | X |  |  |  | X | X | X | X | X |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Spring Creek Gulch | X |  |  |  | X | X | X | X | X |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Bear Creek | X | X |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Connely Gulch | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Shear Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Deer Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Hopkins Gulch | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Two Bar Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Kings Creek | X |  |  |  | X | X | X | X | X |  | X | X | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Logan Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Sleeper Gulch | X |  |  |  | X | X | X | X | X |  |  |  | X |  |  |  |  |  | X |  |  |  |  |  |  |
| McDonald Gulch | X |  |  |  | X | X | X | X | X |  | X | X | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Spring Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Boulder Creek | X | X |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Bracken Brae Creek | X |  |  |  | X | X | X | X | X |  |  |  |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Hare Creek | X |  |  |  | X | X | X | X | X |  | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Jamison Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Peavine Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Silver Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Foreman Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Malosky Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Clear Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Alba Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Marshall Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Manson Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Fall Creek | X | X |  | X | X | X | X | X | X |  | X | X | X |  |  |  |  |  | X |  |  |  |  |  |  |
| South Fall Creek | X | X |  |  | X | X | X | X | X |  | X | X | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Bennett Creek | X | X |  | X | X | X | X | X | X |  | X | X | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Bull Creek | X |  |  |  | X | X | X | X | X |  |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Shingle Mill Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Gold Gulch Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Woods Lagoon |  |  |  |  |  | X | X | X |  |  | X | X |  |  | X |  |  |  | X |  |  | X |  |  |  |
| Arana Gulch | X |  |  |  | X | X | X | X | X |  | X | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Schwan Lake |  |  |  |  |  | X | X | X |  | X |  | X | X | X |  |  |  |  | X |  |  | X |  |  |  |
| Corcoran Lagoon |  |  |  |  | X | X | X | X |  | X |  | X |  | X | X |  |  |  | X |  |  | X |  |  |  |
| Rodeo Creek Gulch (Doyle Gulch) | X | X |  | X | X | X | X | X | X |  |  | X |  |  |  | X |  |  | X |  |  |  |  |  |  |
| Moran Lake |  |  |  |  | X | X | X | X |  | X |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Soquel Lagoon |  |  |  |  |  | X | X | X | X |  | X | X |  | X | X |  |  |  | X |  |  |  |  |  |  |
| Soquel Creek | X | X |  | X | X | X | X | X | X |  | X | X | X |  |  | X |  |  | X |  |  |  |  |  |  |
| Bates Creek | X |  |  |  |  | X | X | X | X |  | X | X | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Grover Gulch | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Soquel Creek, east branch | X |  |  | X | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Hinckley Creek | X | X |  | X | X | X | X | X | X |  | X | X | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Amaya Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Soquel Creek, west branch | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Hester Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Laurel Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Burns Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Moores Gulch | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Miners Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Aptos Creek | X | X |  | X | X | X | X | X | X |  | X | X | X |  | X | X |  |  | X |  |  |  |  |  |  |
| Valencia Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Trout Gulch | X |  |  |  | X | X | X | X | X |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Bridge Creek | X | X |  |  |  | X | X | X | X |  | X | X | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Valencia Lagoon |  |  |  |  |  | X | X | X |  | X |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Pajaro River Hydrologic Unit 305 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Corralitos Lagoon |  |  |  |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Palm Beach Pond | X |  |  |  |  | X | X | X |  | X |  |  |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Pinto Lake | X | X |  |  | X | X | X | X |  | X |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Kelly Lake | X | X |  |  | X | X | X | X |  | X |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Drew Lake | X | X |  |  | X | X | X | X |  | X |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Tynan Lake | X | X |  |  | X | X | X | X |  | X |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Pajaro River Estuary |  |  |  |  |  | X | X | X | X | X | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Pajaro River | X | X |  | X | X | X | X | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |  |  |  |
| San Benito River | X | X |  | X | X | X | X | X |  | X |  | X |  |  |  | X |  |  | X |  |  |  |  |  |  |
| Bird Creek | X | X |  |  | X | X | X | X |  | X |  |  | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Pescadero Creek (San Benito County) | X | X |  |  | X | X | X | X | X | X | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Tres Pinos Creek | X | X |  | X | X | X | X | X |  | X |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Hernandez Reservoir | X | X |  |  | X | X | X | X |  | X |  | X |  |  |  | X | X |  | X |  |  |  |  |  |  |
| Tequisquita Slough |  |  |  |  | X | X | X | X |  | X |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| San Felipe Lake | X | X |  |  | X | X | X | X | X | X | X |  |  |  |  | X | X |  | X |  |  |  |  |  |  |
| Pacheco Creek | X | X |  |  | X | X | X | X | X | X | X | X | X | X |  | X |  |  | X |  |  |  |  |  |  |
| Pacheco Lake | X | X |  |  | X | X | X | X | X | X |  | X |  | X |  | X | X |  | X |  |  |  |  |  |  |
| Llagas Creek, upstream of Chesbro Reservoir | X | X |  |  | X | X | X | X | X | X |  |  |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Chesbro Reservoir | X | X |  |  | X | X | X | X |  | X | X | X |  | X |  | X | X |  | X |  |  |  |  |  |  |
| Llagas Creek, downstream of Chesbro Reservoir | X | X |  | X | X | X | X | X | X | X | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Alamias Creek | X | X |  |  | X | X | X | X | X | X | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Live Oak Creek | X | X |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Little Llagas Creek | X | X |  |  | X | X | X | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Carnadero Creek | X |  |  |  | X | X | X | X | X | X | X |  |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Uvas Creek, downstream of Uvas Reservoir | X | X |  | X | X | X | X | X | X | X | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Uvas Reservoir | X | X |  |  | X | X | X | X |  | X |  | X |  | X |  | X | X |  | X |  |  |  |  |  |  |
| Little Arthur Creek | X | X |  |  | X | X | X | X | X | X | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Bodfish Creek | X | X |  |  | X | X | X | X | X | X | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Black Hawk Canyon Creek | X |  |  |  |  | X | X | X |  | X | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Uvas Creek, upstream of Uvas Reservoir | X |  |  |  | X | X | X | X | X |  | X | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Little Uvas Creek | X | X |  |  | X | X | X | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Swanson Canyon Creek | X |  |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Alec Canyon Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Croy Creek | X |  |  |  | X | X | X | X |  | X |  |  |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Eastman Canyon Creek | X | X |  |  | X | X | X | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Pescadero Creek (Santa Clara County) | X | X |  |  | X | X | X | X | X |  | X | X | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Soda Lake (Santa Cruz County) |  |  |  |  |  |  | X | X |  | X |  |  |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Salsipuedes Creek (Santa Cruz County) | X | X |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Corralitos Creek | X | X |  | X | X | X | X | X | X | X | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Browns Creek | X | X |  | X | X | X | X | X | X | X | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Gamecock Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Ramsey Gulch | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Redwood Creek | X |  |  |  |  | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Mormon Gulch | X |  |  |  | X | X | X | X | X |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Clipper Gulch | X |  |  |  | X | X | X | X | X |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Cookhouse Gulch | X |  |  |  | X | X | X | X | X |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Shingle Mill Gulch | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Rattlesnake Gulch | X |  |  |  | X | X | X | X | X |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Diablo Gulch | X |  |  |  | X | X | X | X | X |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Eureka Gulch | X |  |  |  | X | X | X | X | X |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Rider Creek | X |  |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Watsonville Slough |  |  |  |  |  | X | X | X |  | X |  | X | X | X | X |  |  |  | X |  |  |  |  |  |  |
| Struve Slough |  |  |  |  |  | X | X | X |  | X |  | X | X | X | X |  |  |  | X |  |  |  |  |  |  |
| Hanson Slough |  |  |  |  |  | X | X | X |  | X |  | X | X | X | X |  |  |  | X |  |  |  |  |  |  |
| Harkins Slough |  |  |  |  |  | X | X | X |  | X |  | X | X | X | X |  |  |  | X |  |  |  |  |  |  |
| Gallighan Slough |  |  |  |  |  | X | X | X |  | X |  | X |  | X | X |  |  |  | X |  |  |  |  |  |  |
| Bolsa Nueva Hydrologic Unit 306 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| McClusky Slough |  |  |  |  | X | X | X | X |  | X |  | X |  | X |  |  |  |  | X |  |  | X |  |  |  |
| Elkhorn Slough |  |  |  |  |  | X | X | X | X | X | X | X | X | X | X |  | X |  | X | X |  | X |  |  |  |
| Werner Lake | X | X |  |  | X |  | X | X |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Carneros Creek (Monterey County) | X |  |  |  |  | X | X | X | X | X | X | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Bennett Slough |  |  |  |  |  | X | X | X | X | X |  | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Parsons Slough |  |  |  |  |  | X | X | X | X |  |  | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Carmel River Hydrologic Unit 307 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Carmel River Estuary |  |  |  |  | X | X | X | X | X |  | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Carmel River | X | X |  | X | X | X | X | X | X | X | X | X | X | X |  | X |  |  | X |  |  |  |  |  |  |
| San Clemente Reservoir | X | X |  |  | X | X | X | X | X |  | X | X |  |  |  | X | X |  | X |  |  |  |  |  |  |
| San Clemente Creek | X | X |  |  | X | X | X | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |  |  |  |
| Pine Creek | X |  |  |  | X | X | X | X | X | X | X | X | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Los Padres Reservoir | X |  |  |  | X | X | X | X | X | X | X | X |  |  |  | X | X |  | X |  |  |  |  |  |  |
| Cachagua Creek | X | X | X | X | X | X | X | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |  |  |  |
| Finch Creek | X |  |  |  | X | X | X | X | X | X | X | X | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Tularcitos Creek | X | X |  |  | X | X | X | X | X | X | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Rana Creek | X |  |  |  | X | X | X | X | X | X | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Chupines Creek | X |  |  |  | X | X | X | X | X | X | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Black Rock Creek | X |  |  |  |  | X | X | X | X |  | X | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| White Rock Lake | X |  |  |  |  | X | X | X | X | X | X | X |  |  |  |  | X |  | X |  |  |  |  |  |  |
| Santa Lucia Hydrologic Unit 308 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| San Jose Creek Estuary |  |  |  |  |  | X | X | X | X |  | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| San Jose Creek (Monterey County) | X | X |  |  | X | X | X | X | X | X | X | X | X |  |  | X |  |  | X |  |  |  |  |  |  |
| Garrapata Creek | X |  |  |  |  | X | X | X | X |  | X | X |  | X | X | X |  |  | X | X |  |  |  |  |  |
| Palo Colorado Canyon | X | X |  |  | X | X | X | X | X | X |  | X |  |  | X | X |  |  | X |  |  |  |  |  |  |
| Rocky Creek | X |  |  |  |  | X | X | X | X | X | X | X |  |  | X | X |  |  | X |  |  |  |  |  |  |
| Bixby Creek | X |  |  |  |  | X | X | X | X |  | X | X |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Mill Creek (tributary of Bixby Creek) | X |  |  |  |  | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Little Sur River Estuary |  |  |  |  |  | X | X | X | X |  | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Little Sur River | X | X |  |  | X | X | X | X | X |  | X | X | X | X |  | X |  |  | X |  |  |  |  |  |  |
| Big Sur River Estuary |  |  |  |  |  | X | X | X | X | X | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Big Sur River | X | X |  |  | X | X | X | X | X | X | X | X | X | X |  | X |  |  | X |  |  |  |  |  |  |
| Big Creek (Big Sur Coast) | X |  |  |  |  | X | X | X | X | X | X | X | X | X | X | X |  |  | X |  |  |  |  |  |  |
| Devils Canyon Creek, south fork | X |  |  |  |  | X | X | X | X |  | X | X | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Devils Canyon Creek, middle fork | X |  |  |  |  | X | X | X | X |  | X | X | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Devils Canyon Creek, north fork | X |  |  |  |  | X | X | X | X |  | X | X | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Big Creek, north fork | X |  |  |  |  | X | X | X | X |  |  |  | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Limekiln Creek | X | X |  |  | X | X | X | X | X |  | X | X | X | X | X | X |  |  | X |  |  |  |  |  |  |
| Mill Creek (North of Cape San Martin) | X |  |  |  |  | X | X | X | X | X | X | X |  |  | X | X |  |  | X |  |  |  |  |  |  |
| Willow Creek | X |  |  |  | X | X | X | X | X |  | X | X |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Salmon Creek (Big Sur Coast) | X |  |  |  |  | X | X | X | X |  | X | X |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Salinas Hydrologic Unit 309 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Old Salinas River Estuary, downstream of Potrero Road |  |  |  |  |  | X | X | X | X | X | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Moro Cojo Slough |  |  |  |  | X | X | X | X | X | X |  | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Tembladero Slough |  |  |  |  |  | X | X | X |  | X | X | X |  | X | X |  |  |  | X |  |  | X |  |  |  |
| Espinosa Lake |  |  |  |  |  | X | X | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Espinosa Slough |  |  |  |  |  | X | X | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Salinas Reclamation Canal |  |  |  |  |  | X | X | X |  | X | X |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Gabilan Creek | X | X |  |  | X | X | X | X | X | X | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Alisal Creek | X | X |  |  | X | X | X | X | X | X |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Blanco Drain |  |  |  |  |  | X | X | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Old Salinas River |  |  |  |  |  | X | X | X | X | X | X | X | X | X | X |  |  |  | X |  |  |  |  |  |  |
| Salinas River Refuge Lagoon (South) |  |  |  |  |  | X | X | X | X | X | X |  | X | X |  |  |  |  | X |  |  | X |  |  |  |
| Marina Pond #1 |  |  |  |  | X | X | X | X | X |  |  | X | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Marina Pond #2 |  |  |  |  | X | X | X | X | X |  |  |  | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Marina Pond #3 |  |  |  |  | X | X | X | X | X |  |  |  | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Marina Pond #4/5 |  |  |  |  | X | X | X | X | X |  |  |  | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Marina Pond #6 |  |  |  |  | X | X | X | X | X |  |  |  | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Marina Pond #7 |  |  |  |  | X | X | X | X | X |  |  | X | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Laguna Grande/Roberts Lake | X |  |  |  |  | X | X | X | X | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Del Monte Lake | X |  |  |  |  | X | X | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| El Estero Lake | X |  |  |  | X | X | X | X | X | X |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Salinas River Lagoon (North) |  |  |  |  |  | X | X | X | X | X | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Salinas River, downstream of Spreckels Gage | X | X |  |  |  | X | X | X | X | X | X |  |  |  |  | X |  |  | X |  |  |  |  |  |  |
| Salinas River, Spreckels Gage to Chualar | X | X | X | X | X | X | X | X | X | X | X |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Salinas River, Chualar to Nacimiento River | X | X | X | X | X | X | X | X | X | X | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Arroyo Seco River | X | X |  | X | X | X | X | X | X | X | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Abbott Lakes (The Lakes) | X |  |  |  | X | X | X | X | X | X |  | X |  |  |  |  | X |  | X |  |  |  |  |  |  |
| Piney Creek | X |  |  |  |  | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Paloma Creek | X | X |  |  | X | X | X | X | X | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Tassajara Creek | X | X |  |  | X | X | X | X | X | X | X | X | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Santa Lucia Creek | X | X |  |  | X | X | X | X | X | X | X | X | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Vaqueros Creek | X | X |  |  |  | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Reliz Creek | X | X |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Hames Creek | X | X |  |  | X | X | X | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| San Antonio River, downstream of San Antonio Reservoir | X | X |  | X | X | X | X | X |  | X | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| San Antonio Reservoir | X | X |  |  | X | X | X | X | X | X |  | X |  | X |  | X | X | X | X |  |  |  |  |  |  |
| San Antonio River, upstream of San Antonio Reservoir | X | X |  | X | X | X | X | X | X | X | X | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Pancho Rico Creek | X | X |  |  | X | X | X | X |  | X |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| San Lorenzo Creek | X | X |  |  | X | X | X | X |  | X |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Chalone Creek | X | X |  |  | X | X | X | X |  | X |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Salinas River, between Nacimiento River and Santa Margarita Lake | X | X | X |  | X | X | X | X | X | X | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Nacimiento River, upstream of Nacimiento Reservoir | X | X |  |  | X | X | X | X | X | X |  | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Salmon Creek (tributary of Nacimiento River) | X |  |  |  |  | X | X | X | X |  | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Nacimiento Reservoir | X | X |  |  | X | X | X | X | X | X |  | X |  | X |  | X | X |  | X |  |  |  |  |  |  |
| Nacimiento River, downstream of Nacimiento Reservoir | X | X |  | X | X | X | X | X | X | X | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Las Tablas Creek | X | X |  |  | X | X | X | X | X | X |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Las Tablas Creek, north fork | X | X |  |  | X | X | X | X | X |  |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Las Tablas Creek, south fork | X | X |  |  | X | X | X | X | X |  |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Franklin Creek (San Luis Obispo County) | X | X |  |  | X | X | X | X |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| San Marcos Creek | X | X |  |  | X | X | X | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Paso Robles Creek | X | X |  |  | X | X | X | X | X |  | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Jack Creek | X | X |  |  | X | X | X | X | X |  | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Santa Rita Creek (San Luis Obispo County) | X | X |  | X | X | X | X | X | X | X | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Atascadero Creek (San Luis Obispo County) | X | X |  |  | X | X | X | X | X |  |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Santa Margarita Lake | X | X |  | X | X | X | X | X | X | X |  | X |  | X |  | X | X | X | X |  |  |  |  |  |  |
| Salinas River, upstream of Santa Margarita Lake | X | X |  |  | X | X | X | X | X |  | X | X |  |  |  | X |  |  | X |  |  |  |  |  |  |
| Huerhuero Creek | X | X |  |  | X | X | X | X |  | X |  |  |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Vineyard Canyon Creek | X | X |  |  | X | X | X | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Big Sandy Creek | X | X |  |  | X | X | X | X |  | X |  |  | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Atascadero Lake | X |  |  |  | X | X | X | X | X | X |  | X |  |  |  |  | X |  | X |  |  |  |  |  |  |
| Estero Bay Hydrologic Unit 310 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| San Carpoforo Creek Estuary |  |  |  |  |  | X | X | X | X |  | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| San Carpoforo Creek | X | X |  | X | X | X | X | X | X | X | X | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Estrada Creek | X | X |  |  | X | X | X | X | X | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Chris Flood Creek | X | X |  |  | X | X | X | X | X | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Wagner Creek | X | X |  |  | X | X | X | X | X | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Dutra Creek | X | X |  |  | X | X | X | X | X | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Arroyo de los Chinos | X | X |  |  | X | X | X | X | X | X |  |  |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Arroyo de la Cruz Estuary |  |  |  |  |  | X | X | X | X |  | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Arroyo de la Cruz Creek | X | X |  | X | X | X | X | X | X | X | X | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Burnett Creek | X | X |  |  | X | X | X | X | X | X | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Arroyo del Oso | X | X |  |  | X | X | X | X | X |  |  |  |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Arroyo del Corral | X | X |  |  | X | X | X | X | X | X | X | X |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Oak Knoll Creek | X | X |  |  | X | X | X | X | X | X |  |  |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Arroyo Laguna |  |  |  |  |  | X | X | X | X |  |  | X |  | X | X |  |  |  | X |  |  | X |  |  |  |
| Little Pico Creek Estuary |  |  |  |  |  | X | X | X | X |  | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Little Pico Creek | X | X |  |  | X | X | X | X | X |  | X | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Pico Creek Estuary |  |  |  |  | X | X | X | X | X | X | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Pico Creek | X | X |  |  | X | X | X | X | X | X | X | X | X | X |  | X |  |  | X |  |  |  |  |  |  |
| Pico Creek, south fork | X | X |  |  | X | X | X | X | X |  | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Pico Creek, north fork | X | X |  |  | X | X | X | X | X |  | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| San Simeon Creek Estuary |  |  |  |  | X | X | X | X | X |  | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| San Simeon Creek | X | X |  | X | X | X | X | X | X | X | X | X | X | X |  | X |  |  | X |  |  |  |  |  |  |
| Steiner Creek | X | X |  |  | X | X | X | X | X | X | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Santa Rosa Creek Estuary |  |  |  |  | X | X | X | X | X | X | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Santa Rosa Creek (San Luis Obispo County) | X | X |  | X | X | X | X | X | X | X | X | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Perry Creek | X | X |  |  | X | X | X | X | X |  |  |  |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Green Valley Creek | X | X |  |  | X | X | X | X | X | X |  |  |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Villa Creek | X | X |  |  | X | X | X | X | X |  | X | X |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Cayucos Creek | X | X |  |  | X | X | X | X | X | X | X | X | X | X | X | X |  |  | X |  |  |  |  |  |  |
| Old Creek, downstream of Whale Rock Reservoir | X | X |  |  | X | X | X | X |  | X |  |  |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Whale Rock Reservoir | X | X | X | X | X | X | X | X | X | X |  | X |  | X |  | X | X |  | X |  |  |  |  |  |  |
| Old Creek, upstream of Whale Rock Reservoir | X | X | X | X | X | X | X | X | X | X |  | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Toro Creek | X | X |  |  | X | X | X | X | X | X | X | X |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Morro Creek | X | X |  |  | X | X | X | X | X | X | X | X |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Little Morro Creek | X | X |  |  | X | X | X | X | X |  | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Morro Bay Estuary |  |  |  | X |  | X | X | X | X |  | X | X | X | X | X |  |  |  | X | X |  | X |  |  |  |
| Chorro Creek | X | X |  |  | X | X | X | X | X | X | X | X | X | X |  | X |  |  | X |  |  |  |  |  |  |
| Dairy Creek | X | X |  |  | X | X | X | X | X |  | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| San Luisito Creek | X | X |  |  | X | X | X | X | X |  | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| San Bernardo Creek | X | X |  |  | X | X | X | X | X |  | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Los Osos Creek | X | X |  |  | X | X | X | X | X | X | X | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Warden Lake Wetland |  | X |  |  | X | X | X | X |  | X |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Islay Creek | X | X |  |  | X | X | X | X | X |  | X | X | X | X | X | X |  |  | X |  |  |  |  |  |  |
| Coon Creek | X | X |  |  | X | X | X | X | X |  | X | X | X | X | X | X |  |  | X |  |  |  |  |  |  |
| Diablo Canyon Creek | X | X |  | X | X | X | X | X | X |  |  | X |  | X | X | X |  |  | X |  |  |  |  |  |  |
| San Luis Obispo Creek Estuary |  |  |  |  | X | X | X | X | X | X | X | X | X | X | X |  |  |  | X | X |  | X |  |  |  |
| San Luis Obispo Creek, upstream of West Marsh Street | X | X |  |  | X | X | X | X | X | X | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| San Luis Obispo Creek, downstream of West Marsh Street | X | X |  |  | X | X | X | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |  |  |  |
| Froom Creek | X |  |  |  |  | X | X | X |  |  |  |  |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Davenport Creek | X | X |  |  | X | X | X | X | X |  |  |  |  | X |  |  |  |  | X |  |  |  |  |  |  |
| San Luis Obispo Creek, east fork | X | X |  |  | X | X | X | X | X |  | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Stenner Creek | X | X |  |  | X | X | X | X | X |  | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Brizziolari Creek | X | X |  |  | X | X | X | X | X |  | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Prefumo Creek | X | X |  |  | X | X | X | X | X |  | X | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Laguna Lake | X | X |  |  | X | X | X | X |  | X | X | X |  | X |  |  | X |  | X |  |  |  |  |  |  |
| Pismo Creek Estuary |  |  |  |  | X | X | X | X | X |  | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Pismo Creek | X | X |  | X | X | X | X | X | X | X | X | X | X | X |  | X |  |  | X |  |  |  |  |  |  |
| Arroyo Grande Creek Estuary |  |  |  |  | X | X | X | X | X |  | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Arroyo Grande Creek, downstream of Lopez Lake | X | X |  | X | X | X | X | X | X | X | X |  |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Oceano Lagoon |  |  |  |  |  | X | X | X |  | X |  | X | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Meadow Creek | X | X |  |  | X | X | X | X | X |  |  |  | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Pismo Marsh (Lake) |  |  |  |  | X | X | X | X |  | X |  |  | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Los Berros Creek | X | X |  |  | X | X | X | X | X |  | X |  |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Lopez Lake | X | X | X | X | X | X | X | X | X | X |  | X |  | X |  | X | X |  | X |  |  |  |  |  |  |
| Arroyo Grande Creek, upstream of Lopez Lake | X | X | X | X | X | X | X | X | X | X | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Big Pocket Lake (Dunes Lakes) |  |  |  |  | X |  | X | X |  |  |  |  |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Willow Lake (Dunes Lakes) |  |  |  |  | X | X | X | X |  | X |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Pipeline Lake (Dunes Lakes) |  |  |  |  | X | X | X | X |  | X |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Celery Lake (Dunes Lakes) |  |  |  |  | X | X | X | X |  | X |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Hospital Lake (Dunes Lakes) |  |  |  |  | X | X | X | X |  | X |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Big Twin Lake (Dunes Lakes) |  |  |  |  | X | X | X | X |  | X |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Small Twin Lake (Dunes Lakes) |  |  |  |  |  | X | X | X |  | X |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Bolsa Chico Lake (Dunes Lakes) |  |  |  |  | X | X | X | X |  | X |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| White Lake (Dunes Lakes) |  |  |  |  | X | X | X | X |  | X |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Mud Lake (Dunes Lakes) |  |  |  |  | X | X | X | X |  | X |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Black Lake (Dunes Lakes) |  |  |  |  | X | X | X | X |  | X |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Dune Lakes Marsh Area (Dunes Lakes) |  |  |  |  | X | X | X | X |  | X |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Carrizo Plain Hydrologic Unit 311 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| San Diego Creek | X | X |  |  | X | X | X | X |  | X |  |  | X | X |  | X |  |  | X |  |  |  |  |  |  |
| Soda Lake (San Luis Obispo County) |  |  |  | X |  |  | X | X |  | X |  |  | X | X |  |  |  |  | X |  | X |  |  |  |  |
| Santa Maria Hydrologic Unit 312 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Oso Flaco Lake |  |  |  |  | X | X | X | X |  | X |  | X | X | X |  |  | X |  | X |  |  |  |  |  |  |
| Oso Flaco Creek | X | X |  |  | X | X | X | X |  | X |  |  | X | X |  | X |  |  | X |  |  |  |  |  |  |
| Santa Maria River Estuary |  |  |  |  | X | X | X | X |  | X | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Santa Maria River | X | X |  | X | X | X | X | X | X | X | X |  |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Corralitos Canyon Creek | X | X |  |  |  | X | X | X |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Sisquoc River, downstream of San Rafael wilderness boundary | X | X |  | X | X | X | X | X | X | X | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Sisquoc River, upstream of San Rafael wilderness boundary | X |  |  |  | X | X | X | X | X |  | X | X | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Cuyama River, downstream of Twitchell Reservoir | X | X |  |  | X | X | X | X |  | X |  |  |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Twitchell Reservoir | X | X |  |  | X |  | X | X |  | X |  |  |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Cuyama River, upstream of Twitchell Reservoir | X | X | X | X | X | X | X | X | X | X |  | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Alamo Creek | X | X |  |  | X | X | X | X | X | X |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Huasna River | X | X |  |  | X | X | X | X |  | X |  |  |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Orcutt Creek | X | X |  |  | X | X | X | X | X | X |  |  |  | X | X | X |  |  | X |  |  |  |  |  |  |
| San Antonio Hydrologic Unit 313 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Shuman Canyon Creek | X | X |  |  |  | X | X | X |  | X |  | X |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Casmalia Canyon Creek | X | X |  |  |  | X | X | X |  | X |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| San Antonio Creek Estuary |  |  |  |  | X | X | X | X | X | X | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| San Antonio Creek | X | X |  |  | X | X | X | X | X | X | X | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Barka Slough |  |  |  |  | X | X | X | X |  | X |  | X |  | X | X |  |  |  | X |  |  | X |  |  |  |
| Santa Ynez Hydrologic Unit 314 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Santa Ynez River Estuary |  |  |  |  |  | X | X | X |  | X | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Santa Ynez River, downstream of Cachuma Lake | X | X | X | X | X | X | X | X | X | X | X | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Graves Wetland |  |  |  |  |  | X | X | X |  | X |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Lompoc Canyon | X | X |  | X | X | X | X | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| La Salle Canyon Creek | X | X |  |  | X | X | X | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Sloans Canyon Creek | X |  |  |  | X | X | X | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| San Miguelito Creek | X | X |  |  | X | X | X | X | X | X |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Salsipuedes Creek (Santa Barbara County) | X | X |  | X | X | X | X | X | X | X | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| El Jaro Creek | X | X |  | X | X | X | X | X | X | X | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| El Callejon Creek | X |  |  |  | X | X | X | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Llanito Creek | X |  |  |  | X | X | X | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Yridisis Creek | X | X |  |  | X | X | X | X |  | X |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Cañada de la Vina | X | X |  |  | X | X | X | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Nojoqui Creek | X | X |  |  | X | X | X | X | X | X |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Alamo Pintado Creek | X | X |  | X | X | X | X | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Zaca Creek | X | X |  |  | X | X | X | X | X | X |  |  |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Zaca Lake | X |  |  |  |  | X | X | X | X | X |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Santa Rosa Creek (Santa Barbara County) | X | X |  |  | X | X | X | X | X | X | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Santa Rita Creek (Santa Barbara County) | X | X |  | X | X | X | X | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Davis Creek | X |  |  |  | X | X | X | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Santa Lucia Canyon Creek | X | X |  |  | X | X | X | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Oak Canyon Creek | X | X |  | X | X | X | X | X |  | X |  |  | X |  |  |  |  |  | X |  |  |  |  |  |  |
| Hilton Creek | X | X |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Cachuma Lake | X | X | X |  | X | X | X | X | X | X |  | X |  | X |  | X | X |  | X |  |  |  |  |  |  |
| Santa Ynez River, upstream of Cachuma Lake | X | X | X | X | X | X | X | X | X | X | X | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Gibralter Reservoir | X | X | X | X | X | X | X | X | X | X |  | X |  | X |  | X | X |  | X |  |  |  |  |  |  |
| Jameson Lake | X | X | X |  | X | X | X | X | X | X |  | X |  | X |  | X | X |  | X |  |  |  |  |  |  |
| Agua Caliente Canyon | X | X |  | X | X | X | X | X | X | X |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Mono Creek | X | X |  | X | X | X | X | X | X | X | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Indian Creek | X | X |  | X | X | X | X | X | X | X | X | X | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Santa Cruz Creek | X | X |  | X | X | X | X | X | X | X | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Cachuma Creek | X |  |  |  | X | X | X | X | X | X | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| South Coast Hydrologic Unit 315 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cañada Honda Creek Estuary |  |  |  |  |  | X | X | X | X | X | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Cañada Honda Creek | X | X |  |  | X | X | X | X | X | X | X | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Cañada Agua Viva | X |  |  |  | X | X | X | X |  | X |  |  |  |  | X | X |  |  | X |  |  |  |  |  |  |
| Water Canyon Creek (Santa Barbara County) | X |  |  |  | X | X | X | X |  | X |  |  | X |  | X | X |  |  | X |  |  |  |  |  |  |
| Cañada del Jolloru | X |  |  |  |  | X | X | X |  | X |  |  |  |  | X | X |  |  | X |  |  |  |  |  |  |
| Jalama Creek Estuary |  |  |  |  |  | X | X | X |  | X | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Jalama Creek | X | X |  |  | X | X | X | X |  | X |  | X |  |  |  | X |  |  | X |  |  |  |  |  |  |
| Escondido Creek | X |  |  |  | X | X | X | X | X | X | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Gasper Creek | X |  |  |  | X | X | X | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Espada Creek | X |  |  |  | X | X | X | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Wood Canyon Creek | X |  |  |  | X | X | X | X |  | X |  |  |  |  | X | X |  |  | X |  |  |  |  |  |  |
| Cañada del Cojo | X |  |  |  | X | X | X | X |  | X |  |  |  |  | X | X |  |  | X |  |  |  |  |  |  |
| Barranca Honda | X | X |  |  | X | X | X | X |  | X |  |  |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Arroyo Bulito | X | X |  |  | X | X | X | X |  | X |  |  |  |  | X | X |  |  | X |  |  |  |  |  |  |
| Cañada de Santa Anita | X | X |  |  | X | X | X | X |  | X |  |  |  |  | X | X |  |  | X |  |  |  |  |  |  |
| Cañada del Sacate | X | X |  |  | X | X | X | X |  | X |  |  |  |  | X | X |  |  | X |  |  |  |  |  |  |
| Cañada Alegria | X |  |  |  | X | X | X | X |  | X |  |  |  |  | X | X |  |  | X |  |  |  |  |  |  |
| Cañada del Agua Caliente | X | X |  |  | X | X | X | X | X | X |  |  |  |  | X | X |  |  | X |  |  |  |  |  |  |
| Cañada de la Gaviota | X | X |  |  | X | X | X | X | X | X | X | X | X | X | X | X |  |  | X |  |  |  |  |  |  |
| Cañada San Onofre | X |  |  |  |  | X | X | X | X | X | X | X |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Cañada del Molino | X |  |  |  |  | X | X | X |  | X |  |  |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Arroyo Hondo | X |  |  |  |  | X | X | X | X | X | X | X |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Arroyo Quemado | X | X |  |  |  | X | X | X | X |  | X | X |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Tajiguas Creek | X | X |  |  | X | X | X | X | X | X | X | X |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Cañada del Refugio | X | X |  |  | X | X | X | X | X | X | X | X | X | X | X | X |  |  | X |  |  |  |  |  |  |
| Cañada del Capitan | X | X |  |  | X | X | X | X | X | X | X | X | X | X | X | X |  |  | X |  |  |  |  |  |  |
| Dos Pueblos Canyon Creek | X | X | X | X | X | X | X | X | X | X | X | X |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Tecolote Creek | X | X | X | X | X | X | X | X | X | X | X | X |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Devereux Ranch Lagoon |  |  |  |  |  | X | X | X |  | X | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Devereux Creek | X |  |  |  | X | X | X | X |  | X |  |  |  |  |  | X |  |  | X |  |  |  |  |  |  |
| Goleta Point Marsh |  |  |  |  |  | X | X | X |  | X |  | X | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Goleta Slough |  |  |  |  |  | X | X | X |  | X | X | X | X | X | X |  |  |  | X |  |  | X |  |  |  |
| Carneros Creek (Santa Barbara County) | X | X |  |  | X | X | X | X | X | X |  |  |  |  |  | X |  |  | X |  |  |  |  |  |  |
| Tecolotito Creek | X |  |  |  | X | X | X | X | X | X | X |  |  |  |  | X |  |  | X |  |  |  |  |  |  |
| Glen Annie Canyon | X | X | X | X | X | X | X | X | X | X | X | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Los Carneros Wetland |  |  |  |  | X | X | X | X |  | X |  | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Lake Los Carneros | X | X |  |  | X | X | X | X |  | X |  | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Atascadero Creek (Santa Barbara County) | X | X |  |  | X | X | X | X | X | X | X | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Maria Ygnacio Creek | X | X |  |  | X | X | X | X | X |  | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| San Antonio Creek (South Coast Watershed) | X | X |  |  | X | X | X | X | X | X | X | X |  | X |  |  |  |  | X |  |  |  |  |  |  |
| San Jose Creek (Santa Barbara County) | X | X |  |  | X | X | X | X | X | X | X | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Las Vegas Creek | X |  |  |  | X | X | X | X | X | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| San Pedro Creek | X | X |  |  | X | X | X | X | X | X | X |  |  |  |  | X |  |  | X |  |  |  |  |  |  |
| Las Palmas Creek | X |  |  |  | X | X | X | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Arroyo Burro Estuary |  |  |  |  |  | X | X | X |  | X |  | X |  |  | X |  |  |  | X |  |  |  |  |  |  |
| Arroyo Burro Creek | X |  |  |  | X | X | X | X |  | X |  | X | X | X |  | X |  |  | X |  |  |  |  |  |  |
| Mission Creek | X |  |  |  | X | X | X | X | X | X | X | X |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Rattlesnake Canyon | X |  |  |  | X | X | X | X | X | X | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Waste Slough |  |  |  |  | X | X | X | X |  | X |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Sycamore Creek | X | X |  |  | X | X | X | X | X | X | X | X |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Andree Clark Bird Refuge |  |  |  |  |  | X | X | X |  | X |  |  | X | X |  |  |  |  | X |  |  | X |  |  |  |
| San Ysidro Creek | X |  |  |  | X | X | X | X |  | X |  |  |  |  | X | X |  |  | X |  |  |  |  |  |  |
| Romero Creek | X |  |  |  | X | X | X | X |  | X |  |  |  |  | X | X |  |  | X |  |  |  |  |  |  |
| Toro Canyon Creek | X |  |  |  | X | X | X | X |  | X |  |  |  |  | X | X |  |  | X |  |  |  |  |  |  |
| Arroyo Paredon | X | X |  |  | X | X | X | X | X | X | X | X |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Carpinteria Marsh (El Estero) |  |  |  |  |  | X | X | X |  | X | X | X | X | X | X |  |  |  | X |  |  |  |  |  |  |
| Santa Monica Creek | X | X |  |  | X | X | X | X | X | X |  | X | X |  |  | X |  |  | X |  |  |  |  |  |  |
| Franklin Creek (Santa Barbara County) | X | X |  |  | X | X | X | X | X | X | X | X |  | X |  | X |  |  | X |  |  |  |  |  |  |
| Carpinteria Creek | X | X |  |  | X | X | X | X | X | X | X | X | X | X | X | X |  |  | X |  |  |  |  |  |  |
| Gobernador Creek | X |  |  |  | X | X | X | X | X | X |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Steer Creek | X |  |  |  |  | X | X | X | X | X | X | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Rincon Creek | X | X |  |  | X | X | X | X | X | X | X | X |  | X | X | X |  |  | X |  |  |  |  |  |  |
| Santa Barbara Channel Hydrologic Unit 316 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Santa Rosa Island |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lobo Canyon Creek | X | X |  |  |  | X | X | X |  | X |  |  | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Old Ranch Canyon Creek | X | X |  |  |  | X | X | X |  | X |  |  | X | X |  | X |  |  | X |  |  |  |  |  |  |
| Arlington Canyon Creek | X | X |  |  |  | X | X | X |  | X |  |  | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Water Canyon Creek (Santa Rosa Island) | X | X |  |  |  | X | X | X |  | X |  |  | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Cow Canyon Creek | X | X |  |  |  | X | X | X |  | X |  |  | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Clapp Springs | X | X |  |  |  | X | X | X |  | X |  |  | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Old Ranch Canyon Creek Estuaries |  | X |  |  |  | X | X | X |  | X |  |  | X | X | X |  |  |  | X |  |  |  |  |  |  |
| Old Ranch House Canyon Creek | X | X |  |  |  | X | X | X |  | X |  |  | X | X |  | X |  |  | X |  |  |  |  |  |  |
| Cherry Canyon Creek | X | X |  |  |  | X | X | X |  | X |  |  | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Santa Cruz Island |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Willow Canyon Creek | X |  |  |  |  | X | X | X |  | X |  |  | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Coches Prieto Canyon Creek | X |  |  |  |  | X | X | X |  | X |  |  | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Almos Anchorage Canyon Creek | X |  |  |  |  | X | X | X |  | X |  |  | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Cañada del Puerto, upstream of Prisoners Harbor | X |  |  |  |  | X | X | X |  | X |  |  | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Cañada Larga Creek | X |  |  |  |  | X | X | X |  | X |  |  | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Upper Pozo Canyon Creek | X |  |  |  |  | X | X | X |  | X |  |  | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Sauces Canyon Creek | X |  |  |  |  | X | X | X |  | X |  |  | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Twin Harbors Canyon Creek, east fork | X |  |  |  |  | X | X | X |  | X |  |  | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Lady’s Harbor Canyon Creek | X |  |  |  |  | X | X | X |  | X |  |  | X | X |  |  |  |  | X |  |  |  |  |  |  |
| Estrella River Hydrologic Unit 317 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Estrella River | X | X |  |  | X | X | X | X |  | X |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |
| San Juan Creek | X | X |  |  | X | X | X | X |  | X |  |  |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Cholame Creek | X | X |  |  | X | X | X | X |  | X |  |  |  | X |  |  |  |  | X |  |  |  |  |  |  |
| Little Cholame Creek | X | X |  |  | X | X | X | X |  | X |  |  |  | X |  |  |  |  | X |  |  |  |  |  |  |

#### Proposed changes to Basin Plan Table 2-2

Table 2-2. Existing and Anticipated Beneficial Uses of Ocean Watersa

| Ocean Water | R E C 1 | R E C 2 | I N D | N A V | M A R | S H E L L | C O M M | R A R E | B I O L | W I L D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pescadero Point to Point Año Nuevo | E | E | E | E | E | E | E | E |  | E |
| Point Año Nuevo to Soquel Point | E | E | E | E | E | E | E |  |  | E |
| Año Nuevo ASBS | E | E |  |  | E |  |  | E | E | E |
| Santa Cruz Harbor | E | E | E | E | E |  | E |  |  |  |
| San Lorenzo Estuary | E | E |  | E | E | E | E |  |  | E |
| Soquel Point to Salinas River | E | E | E | E | E | E | E | E |  | E |
| Elkhorn Sloughb | E | E |  |  | E | E | E | E |  | E |
| Moss Landing Harbor | E | E | E | E | E | Ec | E | E |  | E |
| Salinas River to Point Piños | E | E | E | E | E | E | E |  |  | E |
| Monterey Harbor | A | E | E | E | E | E | A | E |  |  |
| Pacific Grove ASBS | E | E |  |  | E |  | E | E | E | E |
| Hopkins Marine Life Refuge | E | E |  |  | E |  | E | E | E | E |
| Point Piños to Point Piedras Blancas | E | E |  | E | E |  | E | E |  | E |
| Carmel Bay ASBS | E | E |  |  | E |  | E | E | E | E |
| Point Lobos ASBS | E | E |  |  | E |  |  | E | E | E |
| Point Sur | E | E |  |  | E | E | E |  |  | E |
| Julia Pfeiffer Burns ASBS | E | E |  |  | E |  |  | E | E | E |
| Salmon Creek Coast ASBS | E | E |  |  | E |  |  |  | E | E |
| Point Piedras Blancas to Point Estero | E | E |  | E | E | E | E | E |  | E |
| Estero Bay | E | E | E | E | E | E | E | E |  | E |
| Morro Bay | E | E | E | E | E | E | E | E |  | E |
| Point Buchon to Point San Luis | E | E | E | E | E | E | E |  |  | E |
| Point San Luis to Point Sal | E | E | E | E | E | E | E | E |  | E |
| Point Sal to Point Arguello | E | E |  | E | E | E | E |  |  | E |
| Point Arguello to Coal Oil Point | E | E | E | E | E | E | E |  |  |  |
| Coal Oil Point to Rincon Point | E | E | E | E | E | E | E | E |  | E |
| Goleta Slough | E | E |  |  | E | E |  | E |  | E |
| Santa Barbara Harbor | E | E | E | E | E |  | E |  |  |  |
| Beach Parks | E | E |  | E | E |  |  |  |  |  |
| San Miguel Island ASBS | E | E |  | E | E | E | E | E | E | E |
| Santa Rosa Island ASBS | E | E |  | E | E | E | E |  | E | E |
| Santa Cruz Island ASBS | E | E |  | E | E | E | E | E | E | E |
| El Estero | E | E |  |  | E | E |  | E |  | E |

a This table lists selected coastal segments. It is not a complete inventory for the Central Coast region. All ocean waters, including ocean waters not listed above, have designated beneficial uses established by the Ocean Plan.  The Ocean Plan defines ocean waters: territorial marine waters of the state as defined by California law to the extent these waters are outside of enclosed bays, estuaries, and coastal lagoons. If a discharge outside the territorial waters of the state could affect the quality of the waters of the state, the discharge may be regulated to ensure no violation of the Ocean Plan will occur in ocean waters.

b Elkhorn Slough has been designated an ecological reserve by the California Department of Fish and Wildlife and recognized as a National Estuary Sanctuary by the Federal Government.

c Clamming is an existing beneficial use (SHELL) in the North Harbor and on the south side of the entrance channel to Elkhorn Slough (north of the Pacific Gas and Electric Cooling Water Intake). Presently, no shellfishing use occurs south of the Pacific Gas and Electric Intake.

Notes:

E = Existing beneficial water use

A = Anticipated beneficial water use

#### Proposed changes to Basin Plan Table 2-3

Table 2-3. Central Coastal Surface Water Hydrologic Planning Areas

| Number | Surface Waterbody Name |
| --- | --- |
| 304.00 | Big Basin Hydrologic Unit |
| 304.10 | Santa Cruz HA |
| 304.11 | Davenport HSA |
| 304.12 | San Lorenzo HSA |
| 304.13 | Aptos-Soquel HSA |
| 304.20 | Año Nuevo HA |
| 305.00 | Pajaro River Hydrologic Unit |
| 305.10 | Watsonville HA |
| 305.20 | Santa Cruz Mountains HA |
| 305.30 | South Santa Clara Valley HA |
| 305.40 | Pacheco-Santa Ana Creek HA |
| 305.50 | San Benito River HA |
| 306.00 | Bolsa Nueva Hydrologic Unit |
| 307.00 | Carmel River Hydrologic Unit |
| 308.00 | Santa Lucia Hydrologic Unit |
| 309.00 | Salinas Hydrologic Unit |
| 309.10 | Lower Salinas Valley HA |
| 309.20 | Chualar HA |
| 309.30 | Soledad HA |
| 309.40 | Upper Salinas Valley HA |
| 309.50 | Monterey Peninsula HA |
| 309.60 | Arroyo Seco HA |

#### Proposed changes to Basin Plan Table 2-4

Table 2-4. Central Coastal Groundwater Basins

| Basin/Subbasin Number | Basin Name | County |
| --- | --- | --- |
| 3-20 | Año Nuevo Area | San Mateo |

#### Proposed changes to Basin Plan Chapter 3, paragraphs 1, 2 and 3

Section 13241, division 7 of the CWC specifies that each Regional Water Board shall establish water quality objectives that, in the Regional Water Board’s judgment, are necessary for the reasonable protection of beneficial uses and for the prevention of nuisance.

Section 303 of the 1972 amendments to the CWA requires the state to submit all new or revised water quality standards established for surface waters to USEPA for approval. Under federal terminology, water quality standards consist of beneficial uses enumerated in Chapter Two and water quality objectives contained in this chapter.

Water quality objectives contained herein are designed to satisfy all state and federal requirements.

#### Proposed changes to Basin Plan section 3.1, paragraph 1

The aforementioned 1972 amendments to the CWA declare that a national goal is elimination of discharge of pollutants into navigable waters.

#### Proposed changes to Basin Plan section 3.1, paragraph 3

Water quality objectives have been adopted by the state and, when applicable, extended as federal water quality standards. Water quality standards, previously mentioned in this chapter’s introduction, pertain to navigable waters and become legally enforceable criteria when approved by the USEPA.

#### Proposed changes to Basin Plan section 3.1, paragraphs 4 and 5

Point and nonpoint water pollution sources described herein have the same meaning as defined in the CWA. Point sources are waste loads from identifiable sources such as municipal discharges, industrial discharges, vessels, controllable stormwaters, fish hatchery discharges, and confined animal operations. Nonpoint sources are loads resulting from land use practices where wastes are not collected and disposed of in any readily identifiable manner. Examples include agricultural runoff, rural road drainage, mining, grassland management, logging and other harvest activities, and natural sources such as effects of fire, flood, and landslide. The distinction between point sources and nonpoint sources is not always clear but generally applies to the practicality of waste load control.

Water quality objectives for the Central Coast region satisfy state and federal requirements to protect waters for the beneficial uses in Chapter Two and are consistent with applicable provisions of statewide plans and policies.

#### Proposed changes to Basin Plan section 3.2, heading and paragraph 1

3.2. Antidegradation Policy

Wherever the existing quality of water is better than the quality of water established herein as objectives, such existing quality shall be maintained unless otherwise provided by State Water Board Resolution 68-16, Statement of Policy with Respect to Maintain High Quality of Waters in California (Antidegradation Policy), including any revisions thereto.

#### Proposed changes to Basin Plan section 3.3, paragraphs 3 and 4

Controllable water quality conditions are those actions or circumstances resulting from human activities that may influence the quality of waters of the state and that may be reasonably controlled.

Water quality objectives are considered to be necessary to protect those present and probable future beneficial uses enumerated in Chapter Two of this plan and to protect existing high quality waters of the state. These objectives will be achieved primarily through the establishment of waste discharge requirements and through implementation of this water quality control plan.

#### Proposed changes to Basin Plan section 3.3, paragraphs 5 and 6

In setting waste discharge requirements, the Regional Water Board will consider the potential impact on beneficial uses within the area of influence of the discharge, the existing quality of receiving waters, and the appropriate water quality objectives. The Regional Water Board will make a finding of beneficial uses to be protected and establish waste discharge requirements to protect those uses and to meet water quality objectives.

Several water quality objectives listed herein originate from the California Code of Regulations (CCR), title 22. If title 22 concentrations are amended, Basin Plan objectives are automatically amended to correspond with the new regulations.

Several water quality objectives listed herein originate from the California Toxics Rule, adopted by USEPA, that apply to California’s inland surface waters, enclosed bays, and estuaries (Code of Federal Regulations, title 40 (40 CFR), part 131.38). Future amendments to concentrations in 40 CFR, part 131.38 may supersede the Basin Plan objectives.

#### Proposed changes to Basin Plan section 3.3.1, paragraph 1

The provisions of the Ocean Plan and Thermal Plan and any revisions thereto shall apply in their entireties to affected waters of the Central Coast region and should be referred to for a complete accounting of their components. The Ocean Plan and Thermal Plan shall also apply in their entireties to Monterey Bay and Carmel Bay.

#### Proposed changes to Basin Plan section 3.3.2.1, list item 1 below “Turbidity”

Where natural turbidity is between 0 and 50 nephelometric turbidity units (NTU), increases shall not exceed 20 percent.

#### Proposed changes to Basin Plan section 3.3.2.1, paragraph 1 below “Temperature”

Temperature objectives for enclosed bays and estuaries are as specified in the Thermal Plan including any revisions thereto.

#### Proposed changes to Basin Plan section 3.3.2.1, paragraphs 1 and 2 below “Toxicity”

All waters shall be maintained free of toxic substances in concentrations that are toxic to, or that produce detrimental physiological responses in human, plant, animal, or aquatic life. Compliance with this objective will be determined by use of indicator organisms, analyses of species diversity, population density, growth anomalies, toxicity bioassays of appropriate duration, and other appropriate methods as specified by the Regional Board.

Survival of aquatic life in surface waters subjected to a waste discharge or other controllable water quality conditions shall not be less than that for the same waterbody in areas unaffected by the waste discharge or, when necessary, for other control water that is consistent with the requirements for "experimental water" as described in Standard Methods for the Examination of Water and Wastewater, latest edition. As a minimum, compliance with this objective shall be evaluated with a 96-hour bioassay.

Proposed changes to Basin Plan section 3.3.2.1, paragraph 2 below “Pesticides”

For waters where existing concentrations are presently not detectable or where beneficial uses would be impaired by concentrations in excess of detectable levels, total identifiable chlorinated hydrocarbon pesticides shall not be present at concentrations detectable within the accuracy of analytical methods prescribed in Standard Methods for the Examination of Water and Wastewater, latest edition, or other equivalent methods approved by the Executive Officer.

#### Proposed changes to Basin Plan section 3.3.2.1, below “Chemical Constituents”

Where domestic wastewater effluents are returned to land for irrigation uses, regulatory controls shall comply with or be as effective and protective as those found in the water recycling criteria specified in CCR, title 22, division 4, chapter 3 and other relevant local controls.

#### Proposed changes to Basin Plan section 3.3.2.2, below “Municipal and Domestic Supply (MUN)”

Organic Chemicals

No inland surface waters, enclosed bays, and estuaries shall contain concentrations of organic chemicals in excess of the maximum contaminant levels (MCLs) for primary drinking water standards specified in CCR, title 22, division 4, chapter 15, article 5.5, section 64444, table 64444-A. This incorporation by reference is prospective, including future changes to the incorporated provisions as the changes take effect.

Inorganic Chemicals

Waters shall not contain concentrations of inorganic chemicals in excess of the MCLs for primary drinking water standards specified in CCR, title 22, division 4, chapter 15, article 4, section 64431 and article 4.1, section 64433.2. This incorporation by reference is prospective, including future changes to the incorporated provisions as the changes take effect.

Radioactivity

Waters shall not contain concentrations of radionuclides in excess of the limits specified in CCR, title 22, division 4, chapter 15, article 5, sections 64442 and 64443. This incorporation by reference is prospective, including future changes to the incorporated provisions as the changes take effect.

#### Proposed changes to Basin Plan section 3.3.2.2, below “Agricultural Supply (AGR)” and “Chemical Constituents”

Waters shall not contain concentrations of chemicals in amounts that adversely affect the agricultural beneficial use. Interpretation of adverse effect shall be as derived from the University of California Agricultural Extension Service guidelines provided in Table 3-1.

In addition, waters used for irrigation and livestock watering shall not exceed concentrations of those chemicals listed in Table 3-2. Salts concentrations for irrigation waters shall be controlled through implementation of the Antidegradation Policy (Appendix A-2) to the effect that mineral concentrations in currently or potentially usable waters shall not be increased. It is emphasized that no controllable water quality factor shall degrade the quality of any groundwater resource or adversely affect long-term soil productivity.

Where wastewater effluents are returned to land for irrigation uses, regulatory controls shall comply with or be as effective and protective as those found in  CCR, title 22, division 4, chapter 3 and with relevant controls for local irrigation sources.

#### Proposed changes to Basin Plan section 3.3.2.2, below “Non-Contact Water Recreation (REC-2)” and “Bacteria”

Fecal coliform concentration, based on a minimum of five samples for any 30-day period, shall not exceed a log mean of 2,000 most probable number (MPN) per 100 mL, nor shall more than ten percent of samples collected during any 30-day period exceed 4,000 MPN per 100 mL.

#### Proposed changes to Basin Plan section 3.3.2.2, below “Shellfish Harvesting (SHELL)” and “Chromium”

The maximum permissible value for waters designated SHELL shall be 0.01 mg/L as total chromium (Cr).

#### Proposed changes to Basin Plan section 3.3.2.2, below “Shellfish Harvesting (SHELL)” and “Bacteria”

At all areas where shellfish may be harvested for human consumption, the median total coliform concentration throughout the water column for any 30-day period shall not exceed 70 MPN per 100 mL, nor shall more than ten percent of the samples collected during any 30-day period exceed 230 MPN per 100 mL for a five-tube decimal dilution test or 330 MPN per 100 mL when a three-tube decimal dilution test is used.

#### Proposed changes to Basin Plan section 3.3.3, paragraph 1

Certain water quality objectives have been established for selected surface waters; these objectives are intended to serve as a water quality baseline for evaluating water quality management in the waterbody. Mean values, shown in Table 3-5 for surface waters, are based on available data.

#### Proposed changes to Basin Plan section 3.3.3, paragraphs 2 and 3

Therefore, application of these objectives must be based upon consideration of the surface water quality naturally present; i.e., waste discharge requirements must adhere to the previously stated objectives and issuance of requirements must be tempered by consideration of beneficial uses within the immediate influence of the discharge, the existing quality of receiving waters, and water quality objectives. Consideration of beneficial uses includes: (1) a specific enumeration of all beneficial uses potentially to be affected by the waste discharge, (2) a determination of the relative importance of competing beneficial uses, and (3) impact of the discharge on existing beneficial uses. The Regional Water Board will make a judgment as to the priority of dominant use and minimize the impact on competing uses while not allowing the discharge to violate receiving water quality objectives.

As part of the State Water Board’s continuing planning process, data will be collected and numerical water quality objectives will be developed for those mineral and nutrient constituents where sufficient information is presently not available for the establishment of such objectives.

#### Proposed changes to Basin Plan section 3.3.4.1, paragraphs 1 and 3

The following objectives apply to groundwaters of the Central Coast region.

Radioactivity

Radionuclides shall not be present in concentrations that are deleterious to human, plant, animal, or aquatic life; or result in the accumulation of radionuclides in the food web to an extent that presents a hazard to human, plant, animal, or aquatic life.

#### Proposed changes to Basin Plan section 3.3.4.2, below “Municipal and Domestic Supply (MUN)”

Organic Chemicals

Groundwaters shall not contain concentrations of organic chemicals in excess of the MCLs for primary drinking water standards specified in CCR, title 22, division 4, chapter 15, article 5.5, section 64444, table 64444-A.  This incorporation by reference is prospective, including future changes to the incorporated provisions as the changes take effect.

Inorganic Chemicals

Groundwaters shall not contain concentrations of inorganic chemicals in excess of the MCLs for primary drinking water standards specified in CCR, title 22, division 4, chapter 15, article 4, section 64431 and article 4.1, section 64433.2.  This incorporation by reference is prospective, including future changes to the incorporated provisions as the changes take effect.

Radioactivity

Groundwaters shall not contain concentrations of radionuclides in excess of the limits specified in CCR, title 22, division 4, chapter 15, article 5, section 64443.  This incorporation by reference is prospective, including future changes to the incorporated provisions as the changes take effect.

#### Proposed changes to Basin Plan section 3.3.5, paragraphs 2 and 3

The restrictions specified for Table 3-5 in the second paragraph of section 3.3.3. are applicable to the values indicated in Table 3-6; i.e., the values are at best representative of gross areas only. Groundwaters in the Upper Valley Aquifer groundwater basin have average total dissolved solids (TDS) concentrations that range from 300 mg/L to over 3,000 mg/L. Therefore, application of these objectives must be consistent with the objectives previously stated in this chapter and synchronously reflect the actual groundwater quality naturally present. The Regional Water Board must afford full consideration to: (1) present and probable future beneficial uses affected by the waste discharge; (2) competing beneficial uses; (3) degree of impact on existing beneficial uses; (4) receiving water quality; and (5) water quality objectives, before adjudging priority of dominant use and promulgating waste discharge requirements.

As part of the State Water Board’s continuing planning process, data will be collected and numerical water quality objectives will be developed for those mineral and nutrient constituents where sufficient information is presently not available for the establishment of such objectives.

#### Proposed changes to Basin Plan Table 3-3, header

Table 3-3. Toxic Metal Concentrations not to be Exceeded in Aquatic Life Habitats, mg/La

|  | Fresh Water (COLD, WARM) |  |
| --- | --- | --- |
| Metal | Hard (> 100 mg/L CaCO3) | Soft (≤ 100 mg/L CaCO3) |

#### Proposed changes to Basin Plan Table 3-5, header and text

Table 3-5. Mean Surface Water Quality Objectives, mg/La

| Hydrologic Unit/Sub-Area | TDS | Chlorine | Sulfate | Boron | Sodium |
| --- | --- | --- | --- | --- | --- |
| Santa Ynez (314) |  |  |  |  |  |
| Cachuma Lake | 600 | 20 | 220 | 0.4 | 50 |
| Solvang | 700 | 50 | 250 | 0.4 | 60 |
| Lompoc | 1000 | 100 | 350 | 0.4 | 100 |

#### Proposed changes to Basin Plan Table 3-6, header and footnotes

Table 3-6. Median Groundwater Objectives, mg/La

| Basin/ Sub-Area | TDS | Chlorine | Sulfate | Boron | Sodium | Nitrogenb |
| --- | --- | --- | --- | --- | --- | --- |
| Big Basin |  |  |  |  |  |  |
| Near Felton | 100 | 20 | 10 | 0.2 | 10 | 1 |
| Near Boulder Creek | 250 | 30 | 50 | 0.2 | 20 | 5 |
| Pajaro Valley |  |  |  |  |  |  |
| Hollister | 1200 | 150 | 250 | 1.0 | 200 | 5 |
| Tres Pinos | 1000 | 150 | 250 | 1.0 | 150 | 5 |
| Llagas | 300 | 20 | 50 | 0.2 | 20 | 5 |
| Salinas Valley |  |  |  |  |  |  |
| Upper Valleyf | 600 | 150 | 150 | 0.5 | 70 | 5 |
| Upper Forebayf | 800 | 100 | 250 | 0.5 | 100 | 5 |
| Lower Forebayf | 1500 | 250 | 850 | 0.5 | 150 | 8 |
| 180 foot Aquiferf | 1500 | 250 | 600 | 0.5 | 250 | 1 |
| 400 foot Aquiferf | 400 | 50 | 100 | 0.2 | 50 | 1 |
| Paso Robles Areag |  |  |  |  |  |  |
| Central Basinf | 400 | 60 | 45 | 0.3 | 80 | 3.4 |
| San Miguelf | 750 | 100 | 175 | 0.5 | 105 | 4.5 |
| Paso Roblesf | 1050 | 270 | 200 | 2.0 | 225 | 2.3 |
| Templetonf | 730 | 100 | 120 | 0.3 | 75 | 2.7 |
| Atascaderof | 550 | 70 | 85 | 0.3 | 65 | 2.3 |
| Estrellaf | 925 | 130 | 240 | 0.75 | 170 | 3.2 |
| Shandon | 1390 | 430 | 1025h | 2.8 | 730 | 2.3 |
| Estero Bay |  |  |  |  |  |  |
| Santa Rosa | 700 | 100 | 80 | 0.2 | 50 | 5 |
| Chorro | 1000 | 250 | 100 | 0.2 | 50 | 5 |
| San Luis Obispo | 900 | 200 | 100 | 0.2 | 50 | 5 |
| Arroyo Grande | 800 | 100 | 200 | 0.2 | 50 | 10 |
| Carrizo Plain | e | e | e | e | e | e |
| Santa Maria River Valleyc |  |  |  |  |  |  |
| Upper Guadalupef | 1000d | 165 | 500d | 0.5 | 230 | 1.4e |
| Lower Guadalupef | 1000d | 85 | 500d | 0.2 | 90 | 2.0e |
| Lower Nipomo Mesaf | 710 | 95 | 250 | 0.15 | 90 | 5.7e |
| Orcuttf | 740 | 65 | 300 | 0.1 | 65 | 2.3e |
| Santa Mariaf | 1000d | 90 | 510 | 0.2 | 105 | 8.0e |

a Objectives shown are median values based on data averages; objectives are based on preservation of existing quality or water quality enhancement believed attainable following control of point sources.

b Measured as total nitrogen

c Basis for objectives is in “Water Quality Objectives for the Santa Maria Ground Water Basin Revised Staff Report, May 1985” and February 1986 staff report.

d These are maximum objectives in accordance with Title 22 of the Code of Regulations.

e Groundwater basin currently exceeds usable mineral quality.

f Groundwater basin boundary maps available in the appendix: Salinas (Appendix A-32), Paso Robles (Appendix A-33), Santa Maria (Appendix A-34), and Lompoc (Appendix A-35).

g Basis for objectives is in the report “A Study of the Paso Robles Ground Water Basin to Establish Best Management Practices and Establish Salt Objectives,” Coastal Resources Institute, June 1993.

h Standard exceeds California Secondary Drinking Water Standards contained in CCR, title 22. Water quality standard is based upon existing water quality. If water quality degradation occurs, the Regional Water Board may consider salt limits on appropriate discharges.

#### Proposed changes to Basin Plan section 4.9.23, paragraph 2 below “Problem Statement”

Waterbodies identified as impaired in this TMDL Project include:

Lower Santa Ynez River from Floradale Road downstream to the confluence with the estuary: nitrate and dissolved oxygen.

Santa Ynez River (Cachuma Lake to below city of Lompoc): dissolved oxygen.

Santa Ynez River (above Cachuma Lake): dissolved oxygen.

San Miguelito Creek from monitoring site 314MCM (treatment plant effluent discharge point) downstream to the Santa Ynez River: nitrate, dissolved oxygen, and un-ionized ammonia.

Sloans Canyon Creek, all reaches upstream of West Central Avenue downstream to the confluence with the Santa Ynez River: un-ionized ammonia.

#### Proposed changes to Basin Plan Chapter 5, paragraph 1

In addition to the Implementation Plan, many other plans and policies direct State and Regional Water Board actions or clarify the Regional Water Board’s intent. The following pages contain brief descriptions of State Water Board plans and policies and numerous Regional Board plans and policies.

#### Proposed changes to Basin Plan section 5.1, paragraph 1

The State Water Board has adopted a number of plans and policies for statewide water quality management, available on the State Water Board’s Plans and Policies website at https://www.waterboards.ca.gov/plans_policies, including those referred to below:

State Policy for Water Quality Control, 1972 (Appendix A-1)

Antidegradation Policy (Appendix A-2)

#### Proposed changes to Basin Plan section 5.1, paragraph 2

Should any of these plans and policies be amended by the State water Board, the Regional Water Board will implement the amended version.

#### Proposed changes to Basin Plan section 5.1.1, paragraph 1

The State Water Board has developed a set of twelve general principles to implement the provisions and intent of the Porter-Cologne Act.

#### Proposed changes to Basin Plan section 5.1.2, header and paragraph 1

5.1.2. Antidegradation Policy

On October 28, 1968, the State Water Board adopted Resolution 68-16, Statement of Policy with Respect to Maintaining High Quality of Waters in California, (Appendix A-2). While requiring continued maintenance of existing high quality waters, the policy provides conditions under which a change in water quality is allowable. A change must:

- be consistent with maximum benefit to the people of the state;

#### Proposed changes to Basin Plan section 5.1.4, paragraph 2

Decisions by the Regional Water Boards are required to be consistent with the provisions designed to prevent water quality degradation and to protect beneficial uses.

#### Proposed changes to Basin Plan section 5.1.6, paragraph 1

This policy requires the Regional Water Boards to conduct reclamation surveys and specifies reclamation actions to be implemented by the State and Regional Water Boards as well as other agencies.

#### Proposed changes to Basin Plan section 5.1.7, paragraph 1

The Policy on the Disposal of Shredder Waste designates specific conditions to be enforced by the Regional Water Board by which mechanically destructed car bodies, old appliances, or other similar castoffs can be disposed at certain landfills.

#### Proposed changes to Basin Plan section 5.1.8, paragraph 1

It allows the Regional Water Board some discretion to evaluate whether bodies of water are presently or potentially suitable for MUN designation.

#### Proposed changes to Basin Plan section 5.1.9, paragraph 1

The NPS Management Plan identifies NPS control programs and milestones for their accomplishment. It emphasizes cooperation with local governments and other agencies to promote the implementation of BMPs and remedial projects. The policy describes the required “key elements” of a NPS pollution control implementation program including how significant non-compliance will be addressed.

#### Proposed changes to Basin Plan section 5.1.10, paragraph 2

The Ocean Plan also provides that the State Water Board shall designate ASBS and requires wastes to be discharged a sufficient distance from these areas to assure maintenance of natural water quality conditions.

#### Proposed changes to Basin Plan section 5.1.11, paragraph 1

This policy implements state regulations of waste discharge to land (CCR, title 27, division 2, subdivision 1) and federal regulations related to municipal solid waste disposal (40 CFR, sections 257 and 258). The policy directs Regional Water Boards to revise or adopt, prior to the federal deadline (currently October 9, 1993), WDRs for all municipal solid waste landfills subject to state and federal regulations.

#### Proposed changes to Basin Plan section 5.1.12, paragraph 1

This policy implements CWC, chapter 4.5, division 7, section 13290-13291.7 by establishing statewide regulations and standards for permitting onsite waste water systems. The OWTS Policy specifies criteria for existing and new onsite systems and establishes a conditional waiver of WDRs for onsite systems that comply with the policy.

#### Proposed changes to Basin Plan section 5.1.17, paragraph 1

This policy provides guidance for the application of the CWC enforcement provisions and defines a consistent and transparent enforcement process and penalty methodology application.

#### Proposed changes to Basin Plan section 5.1.18, paragraph 1

The policy provides guidance on principles that can be applied to resolve impairments in surfaces waters of the state and on processes that can be used to adopt TMDLs in California.

#### Proposed changes to Basin Plan section 5.1.19, paragraph 1

This policy establishes requirements for the implementation of CWA section 316(b), using best professional judgment in determining best technology available (BTA) for cooling water intake structures at existing coastal and estuarine power plants that must be implemented in NPDES permits.

#### Proposed changes to Basin Plan section 5.2, list item 3

Erosion and sedimentation control policies should be established based on (a) pilot studies conducted by the Natural Resources Conservation Service (NRCS) that recommended BMPs for erosion problems, (b) a statewide study by the California Association of RCDs on institutional solutions to sedimentation problems, and (c) findings of erosion studies conducted in the Central Coast region as part of nondesignated area 208 planning.

#### Proposed changes to Basin Plan section 5.2, list item 6

- The State Water Board should consider water quality effects when reviewing water rights permits.

#### Proposed changes to Basin Plan section 5.3.1, list items 1 and 2

Land use practices should ensure protection of beneficial uses and aquatic environmental values

There shall be no waste discharged into areas that possess unique or uncommon cultural, scenic, aesthetic, historical or scientific values. Such areas will be defined by the Regional Water Board.

#### Proposed changes to Basin Plan section 5.3.6, list items 1 to 6

- General recommendations for erosion control, numbered one through six under “Land Disturbance Activities” in the Implementation Plan, Chapter Four, are considered by the Regional Water Board to be BMPs, as are those BMPs identified in approved areawide Water Quality Management Plans.

Local units of government should have the lead role in controlling land use activities that cause erosion and may, as necessary, impose further conditions, restrictions, or limitations on waste disposal and other activities that might degrade the quality of waters of the state.

In implementing BMPs through local units of government, or through state and federal agencies for lands under their control, working relationships, priorities, and time schedules will be defined in management agency agreements between the areawide waste treatment planning agency and the local management agency. Agreements will be reviewed and updated annually to reflect recent achievements, new information and new concerns.

Regional Water Board participation in sediment control programs shall include assistance in the establishment of local control programs, participation in the determination of water quality problems, and a cooperative program evaluation with local units of government. Regional Water Board enforcement authority will be exercised where local volunteer programs fail to correct sediment problems within a reasonable period.

Emergency projects undertaken or approved by a public agency and necessary to prevent or mitigate loss of, or damage to, life, health, property, or essential public services from an unexpected occurrence involving a clear and imminent danger are exempt from this chapter providing such exemption is in the public interest.

Regulation of discharges from state and federal lands managed by agencies operating in accordance with approved management agency agreements is waived except where such activity is causing, or threatening to cause, a pollution or nuisance.

#### Proposed changes to Basin Plan section 5.4, paragraphs 1 and 2

Due to unique cultural, scenic, aesthetic, historical, scientific, and ecological values of the Central Coastal region, and the necessity to protect the public health and the desire to achieve water quality objectives, the Regional Water Board has established certain discharge prohibitions.

CWC section 13243 provides that a Regional Water Board, in a water quality control plan, may specify certain conditions or areas where the discharge of waste, or certain types of waste is prohibited. The following discharge prohibitions are applicable to any person, as defined by section 13050(c) of the CWC, including but not limited to, any city, county, district, the state, and the United States (US), to the extent authorized by federal law, whose activities in California could affect the quality of waters of the state within the boundaries of the Central Coastal region.

#### Proposed changes to Basin Plan section 5.4.1, list items 1 to 6

- Waste discharges shall not contain materials in concentrations that are hazardous to human, plant, animal, or aquatic life.

- The discharge of oil or any residual products of petroleum to the waters of the state, except in accordance with waste discharge requirements or other provisions of CWC division 7, is prohibited.

- Discharge of elevated temperature wastes into COLD intrastate waters is prohibited where it may cause the natural temperature of the receiving water to exceed limits specified in Chapter Three, Water Quality Objectives.

- Unless authorized, or exempt by the CWC or the Basin Plan, the discharge of sediment, well development water, or construction dewatering water, to waters of the state in a manner causing or threatening to cause a condition of pollution, contamination, or nuisance is prohibited.

- Any discharge to a stormwater conveyance system that is not composed entirely of stormwater is prohibited unless authorized by the Regional Water Board. Federal regulations, 40 CFR section 122.26(b)(13), define stormwater as stormwater runoff, snow melt runoff, and surface runoff and drainage. 40 CFR section 122.26(b)(2) defines an illicit discharge as any discharge to a stormwater conveyance system that is not composed entirely of stormwater except discharges pursuant to a NPDES permit and discharges resulting from firefighting activities.

- Unless authorized, the discharge of treated or untreated sewage to waters of the state or to a stormwater conveyance system is prohibited.

#### Proposed changes to Basin Plan section 5.4.2, paragraphs 1 and 2

Wastes discharged to inland surface waters of the state shall be essentially free of toxic substances, grease, oil, and phenolic compounds.

Unless authorized, waste discharges to the following inland surface waters of the state are prohibited:

#### Proposed changes to Basin Plan section 5.4.2.1, paragraph 1

Discharges containing fecal material from domestic animals to the waters of the state that cause or contribute to exceedance of water quality objectives in the areas listed below are prohibited.

#### Proposed changes to Basin Plan section 5.4.2.1, paragraph 1

Discharges containing fecal material from humans to the waters of the state in the areas listed below are prohibited. Exceptions to this prohibition include discharges in accordance with WDRs or other provisions of the CWC, division 7, as amended:

#### Proposed changes to Basin Plan section 5.4.2.1, list item 8

Submits a NPS Pollution Control Implementation Program, consistent with the Policy for Implementation and Enforcement of the Nonpoint Source Pollution Control Program that is approved by the Executive Officer, or

Demonstrates to the satisfaction of the Executive Officer that its activities do not cause livestock waste to pass into waters of the state within the Watsonville Slough Watershed, or

Is regulated under WDRs or an NPDES permit, or a conditional waiver of WDRs that explicitly addresses compliance with the Watsonville Slough TMDL for Pathogens.

#### Proposed changes to Basin Plan section 5.4.3, paragraph 3

Discharges to the Monterey Bay Prohibition Zone from desalinization units and circulating seawater system discharges may be permitted after each proposal satisfies California Environmental Quality Act (CEQA) requirements and completes the NPDES process.

#### Proposed changes to Basin Plan section 5.4.3.1, paragraph 1

Discharge of waste is prohibited where it will alter natural water quality conditions in ASBS. ASBS are:

#### Proposed changes to Basin Plan section 5.4.6, paragraph 1

The Central Coast Water Board may, subsequent to a public hearing, grant exceptions to any provision of this Basin Plan where the Central Coast Water Board determines:

#### Proposed changes to Basin Plan section 5.4.6, paragraph 2

Central Coast Water Board exceptions will be effective upon State Water Board approval, unless exceptions involve surface water beneficial use designations or surface water quality objectives (i.e., federally accepted water quality standards). Such water quality standard related exceptions will also require USEPA approval to become effective.

#### Proposed changes to Basin Plan section 5.5.1, list items

- The Regional Water Board will implement water quality control plan provisions through establishment or requirements and timetables for compliance with plan actions.

- WDRs will be established for all (operating) solid waste sites and where inactivated sites may contribute to water quality impairment.

- WDRs will be established for all existing oil well fields, mines, or other well fields which threaten water quality.

- WDRs will be established for all irrigation, feedlot, dairy, and poultry operations which are so located as to pose a clear and direct threat to water quality; such operations need not be so large as to require a permit under NPDES.

#### Proposed changes to Basin Plan section 5.5.2, header and list item 1

5.5.2. State Water Board Grants and Loans

- Priorities for State Water Board grants and State Revolving Fund loans, and financial assistance opportunities will be established by the Regional Water Board and provide ever increasing emphasis toward environmental justice and correction of water quality problems.

#### Proposed changes to Basin Plan section 5.5.4, paragraph 1

The Regional Water Board will initiate coordination with the appropriate Coastal Commission, as well as other state, federal, and local agencies which possess related or overlapping planning responsibilities.

#### Proposed changes to Basin Plan section 5.5.5, paragraph 1

The CCR, title 27, division 2, subdivision 1, chapter 7, subchapter 2 defines a confined animal facility as “any place where cattle, calves, sheep, swine, horses, mules, goats, fowl, or other domestic animals are corralled, penned, tethered, or otherwise enclosed or held and where feeding is by means other than grazing.”

#### Proposed changes to Basin Plan section 5.5.5, list item 5

- Washwater and surface drainage from manure storage areas shall be contained, applied to croplands, or discharged to treatment systems subject to approval by the Regional Water Board.

#### Proposed changes to Basin Plan section 5.5.5, list item 12

- A monitoring program may be required by the Regional Water  Board as a condition to issuance or waiver of WDRs.

#### Proposed changes to Basin Plan section 5.5.7.5, paragraph 1

Local agricultural representatives and the University of California Cooperative Extension (UCCE) should maintain liaison with the Regional Water Board and the State Water Board relative to agricultural wastewater management.

#### Proposed changes to Basin Plan section 5.5.7.6, paragraph 1

Water quality in offshore oil lease areas should be monitored by state and federal agencies preferably by arrangements with independent oceanographic institutions.

#### Proposed changes to Basin Plan section 5.5.7.8, paragraph 2

Local and state agencies having jurisdiction to help control seawater intrusion should assist in implementing seawater intrusion remedies.

#### Proposed changes to Basin Plan section 5.5.7.9, list items 1 to 8

- The federal government should increase its support of erosion and sediment control programs by increasing its technical staff, increasing cost-share funds, increasing the availability of low-interest loans, and changing its income tax laws to encourage the use of BMPs for erosion and sediment control.

- The state should establish an erosion and sediment control program that includes incentives for the individual—such as cost-sharing, changes in state law that would reduce property taxes for enduring erosion and sediment control practices, and incentives through state income taxes.

- RCDs within the Central Coast region should develop management agency agreements with the Regional Water Board agreeing to work jointly with the Regional Water Board to integrate soil and water resource programs in the application of BMPs to correct existing erosion and sediment problems and to prevent new problems from occurring.

- Local units of government should improve land use plans to establish a clear policy, and shall adopt or improve ordinances to include definitive performance standards, for the control of erosion and sedimentation, including consistency with this Basin Plan and BMPs identified under Regional Water Board “Management Principles.”

- Local units of government developing Local Coastal Programs (LCPs) shall establish a clear policy on erosion and sedimentation and adopt an ordinance consistent with BMPs for their land areas within the Coastal Zone.

- RCDs, the NRCS, Caltrans, and the UCCE, in conjunction with the cities and counties, should develop and carry out an erosion and sediment control training program for employees who check erosion and sediment control plans and who enforce local ordinances and regulations relating to erosion and sediment control practices.

- Counties and cities should work with the Regional Water Board to identify priorities, time schedules, and limitations and to negotiate management agency agreements concerning implementation of BMPs for control of erosion and sedimentation.

- Review and assessment of erosion and sediment control plans for new land developments in those counties and cities that have signed management agency agreements with the Regional Water Board will be processed entirely by that county or city.

#### Proposed changes to Basin Plan section 5.6, paragraph 1

Formal specific policies adopted by the Regional Water Board are presented below according to various categories.

#### Proposed changes to Basin Plan section 5.6.1, header

5.6.1. Area of Special Biological Significance

Proposed changes to Basin Plan section 5.6.1, paragraphs 2 and 3

This policy recommended the State Water Board not designate Terrace Point as an ASBS. The State Water Board concurred with the Regional Water Board in Resolution 77-21.

Further information concerning ASBS can be found in Chapter Two.

#### Proposed changes to Basin Plan section 5.6.2, paragraph 3

Regional Water Board adopted prohibition zones for tidal waters can be found under “Waters Subject to Tidal Action” under “Discharge Prohibitions” in this chapter.

#### Proposed changes to Basin Plan section 5.6.5, paragraphs 2 and 3

State law allows Regional Water Boards to waive WDRs for a specific discharge or types of discharges where it is not against the public interest (CWC section 13269). These waivers are conditional and may be terminated at any time.

On April 15, 1983, the Regional Water Board held a public hearing regarding the types and nature of waste discharges considered for waiver. Following this hearing, the Regional Water Board established certain discharges which waived WDRs. The types of dischargers which may be waived are shown in Appendix A-23.

#### Proposed changes to Basin Plan section 5.6.6, paragraph 1

This policy addresses the manner in which the Regional Water Board will protect water quality protection and improvement at the most cost effective manner to society.

#### Proposed changes to Basin Plan Appendix A-2, cover page

Statement of Policy with Respect to Maintaining High Quality of Waters in California (Antidegradation Policy). State Water Board Resolution 68-16.

#### Proposed changes to Basin Plan Appendix A-17, cover page

Adopting Amendments to the Water Quality Control Plan And Requesting Approval from the State Water Resources Control Board, Resolution R3-89-04 amendment by Resolution R3-2005-0013.

#### Proposed changes to Basin Plan Appendix A-18, cover page

Recommendation to the State Water Resources Control Board Concerning the Designation of Terrace Point in Santa Cruz County as an Area of Special Biological Significance, Resolution R3-76-10.

#### Proposed changes to Basin Plan Appendix A-20, cover page

Regarding Marina County Water District’s Petition to Delete the Southern Monterey Bay Discharge Prohibition Zones from the Basin Plan, Resolution R3-79-06.

#### Proposed changes to Basin Plan Appendix A-21, cover page

Certification of Santa Cruz County’s Wastewater Management Program for the San Lorenzo River Watershed, Resolution R3-87-04.

#### Proposed changes to Basin Plan Appendix A-25, cover page

Appreciation for Discharger Compliance, Resolution R3-93-04.

#### Proposed changes to Basin Plan Appendix A-30, cover page

Loso Osos Baywood Park Individual and Community Sewage Disposal System Prohibition Area, Resolution R3-83-13.

## Environmental Analysis

This section presents the regulatory analyses required under the California Environmental Quality Act (CEQA) when the Central Coast Water Board adopts a Basin Plan amendment under the Water Board’s certified regulatory program (California Public Resources Code (PRC) section 15251 (g)).

A CEQA scoping meeting for this project was held on December 4, 2025. Section 3775.5 of the CCR, title 23, describes the purpose of a CEQA scoping meeting for exempt regulatory programs, including Basin Plan amendments. The purpose of the public scoping process is to seek early input from public agencies and members of the public on the environmental analysis for the proposed project.

The PRC, section 21159 requires a state agency to perform an environmental analysis of the reasonably foreseeable methods of compliance, at the time of the adoption of a rule or regulation requiring the installation of pollution control equipment or a performance standard or treatment requirement. In this case, the proposed Basin Plan amendment does not require the installation of pollution control equipment, or compliance with a performance standard or treatment requirement. No implementation plan is proposed, because no actions are required to comply with the amendment. Thus, the amendment would have no environmental or economic impacts.

The Central Coast Water Board is the Lead Agency for evaluating the environmental impacts of Basin Plan amendments pursuant to CEQA. In compliance with the State Water Board’s CEQA implementation guidelines, the Central Coast Water Board prepared the required environmental documents, which include a written report (this project report) and a CEQA Environmental Checklist (Appendix to this project report).  The project report discloses any potentially significant environmental impacts of the reasonably foreseeable methods of compliance with the Basin Plan amendment. This project report, including the CEQA Environmental Checklist and these analyses, constitute a part of the substitute environmental document under CEQA.

As shown in the CEQA Environmental Checklist (Appendix to this project report), there are no potentially significant environmental impacts from the implementation of these Basin Plan amendments. Therefore, an analysis of alternatives is not needed to lessen or mitigate impacts. The finding of no environmental impacts is based on the fact that these amendments will not result in any physical change, nor will it affect any other plan, regulation, or policy. The amendments merely incorporate MCLs for disinfection byproducts and MRDLs for disinfectant residuals from the CCR, title 22, or, where more protective, MCLGs and MRDLGs from 40 CFR, as numeric water quality objectives for the MUN beneficial use to protect domestic water supplies and human health. The proposed amendments do not have any direct effect on the environment, because they make changes to numeric objectives associated with the MUN beneficial use listed in the Basin Plan.

The proposed amendments also make non-regulatory changes to the Basin Plan text to update information, make corrections, and improve overall clarity. Because these changes are solely clarifications of the Basin Plan, there are no potentially significant environmental or economic impacts associated with compliance with these revisions.

Lastly, the Central Coast Water Board must, when feasible, avoid or mitigate damaging effects on tribal cultural resources. Assembly Bill 52 (2013-2014 Reg. Sess.) established a new category of resources in CEQA called Tribal Cultural Resources (PRC, section 21074). The proposed Basin Plan amendments will not cause a substantial adverse change to a tribal cultural resource.

## References

California Regional Water Quality Control Board, Central Coast Region. 2024. Water Quality Control Plan for the Central Coastal Basin, June 2024 Edition. California Environmental Protection Agency. https://www.waterboards.ca.gov/centralcoast/water_issues/programs/basin_plan/docs/2024_basin_plan_r3.pdf

United States Environmental Protection Agency. 1998. National Primary Drinking Water Regulations: Disinfectants and Disinfection Byproducts. https://www.govinfo.gov/content/pkg/FR-1998-12-16/pdf/98-32887.pdf

United States Environmental Protection Agency. 2005. Occurrence Assessment for the Final Stage 2 Disinfectants and Disinfection Byproducts Rule. https://nepis.epa.gov/Exe/ZyPDF.cgi?Dockey=P1005ED2.txt

United States Environmental Protection Agency. 2023. Water Quality Standards Handbook Chapter 3: Water Quality Criteria. https://www.epa.gov/sites/default/files/2014-10/documents/handbook-chapter3.pdf

## Appendix – CEQA Environmental Checklist

### CEQA Environmental Checklist

This CEQA Environmental Checklist is based on the State Water Board CEQA Environmental Checklist for Exempt Regulatory Programs (CCR, title 23, division 3, chapter 27, appendix A) and is also consistent with the Guidelines for Implementation of CEQA Environmental Checklist Form for agencies (CCR, title 14, division 6, chapter 3, appendix G).

Project Title:

Proposed Amendments to the Water Quality Control Plan for the Central Coastal Basin to Establish Water Quality Objectives for Disinfection Byproducts and Disinfectant Residuals

Lead Agency Name and Address:

Central Coast Water Board
895 Aerovista Place, Suite 101
San Luis Obispo, CA 93401

Contact Person and Phone Number:

Jamie Pratt, Environmental Scientist
Central Coast Water Board
(805) 549-3761
jamie.pratt@waterboards.ca.gov

Project Location:

Central Coast Hydrologic Region (Region 3)

Project Description:

This project amends the Basin Plan to incorporate MCLs for disinfection byproducts and MRDLs for disinfectant residuals from the CCR, title 22 or, where more protective, MCLGs and MRDLGs from 40 CFR, as numeric water quality objectives for surface waters and groundwaters designated with the MUN beneficial use to protect domestic water supplies and human health.

The objectives of the Basin Plan amendments are consistent with the mission of the State Water Board and the requirements of the CWA and the CWC. These laws require the Central Coast Water Board to preserve, enhance, and restore the quality of water in the Central Coast region.

Consistent with the State Water Board’s CEQA implementation guidelines, the Central Coast Water Board has prepared a written Project Report and this CEQA Environmental Checklist.

CEQA Environmental Checklist:

This proposed action to amend the Basin Plan would have no direct or indirect impact on the environment, including aquatic and terrestrial wildlife and flora and humans. The “No Impact” box is checked in all checklist categories. The basis for these responses is contained in the Project Report.

This CEQA Environmental Checklist is a series of questions grouped by subject that identify different types of potential environmental impacts that a project may cause. CEQA considers the existing conditions of the physical project site as a baseline. It then compares how much change will occur to the site if the project is implemented. Based on the CEQA Guidelines, the impact severity is rated on a scale of four impact levels. The four levels are: potentially significant impact, less than significant with mitigation incorporated, less than significant impact, or no impact.

Aesthetics

Except as provided in Public Resources Code (PRC) section 21099, would the project:

| Item | Impact Description | Potentially Significant Impact | Less Than Significant With Mitigation Incorporated | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- | --- | --- |
| A | Have a substantial adverse effect on a scenic vista? |  |  |  | No Impact |
| B | Substantially damage scenic resources, including, but not limited to, trees, rock outcroppings, and historic buildings within a state scenic highway? |  |  |  | No Impact |
| C | In non-urbanized areas, substantially degrade the existing visual character or quality of public views of the site and its surroundings? (Public views are those that are experienced from publicly accessible vantage point). If the project is in an urbanized area, would the project conflict with applicable zoning and other regulations governing scenic quality? |  |  |  | No Impact |
| D | Create a new source of substantial light or glare which would adversely affect day or nighttime views in the area? |  |  |  | No Impact |

Agriculture / Forestry Resources

In determining whether impacts to agricultural resources are significant environmental effects, lead agencies may refer to the California Agricultural Land Evaluation and Site Assessment Model (1997) prepared by the California Department of Conservation as an optional model to use in assessing impacts on agriculture and farmland. In determining whether impacts to forest resources, including timberland, are significant environmental effects, lead agencies may refer to information compiled by the California Department of Forestry and Fire Protection regarding the state’s inventory of forest land, including the Forest and Range Assessment Project and the Forest Legacy Assessment project; and forest carbon measurement methodology provided in Forest Protocols adopted by the California Air Resources Boards. Would the project:

| Item | Impact Description | Potentially Significant Impact | Less Than Significant With Mitigation Incorporated | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- | --- | --- |
| A | Convert Prime Farmland, Unique Farmland or Farmland of Statewide Importance (Farmland), as shown on the maps prepared pursuant to the Farmland Mapping and Monitoring Program of the California Resources Agency, to non-agricultural use? |  |  |  | No Impact |
| B | Conflict with existing zoning for agricultural use, or a Williamson Act contract? |  |  |  | No Impact |
| C | Conflict with existing zoning for, or cause rezoning of, forest land (as defined in Public Resources Code section 12220(g)), timberland (as defined by Public Resources Code section 4526), or timberland zoned Timberland Production (as defined by Government Code section 51104(g))? |  |  |  | No Impact |
| D | Result in the loss of forest land or conversion of forest land to non-forest use? |  |  |  | No Impact |
| E | Involve other changes in the existing environment which, due to their location or nature, could result in conversion of Farmland, to non-agricultural use or conversion of forest land to non-forest use? |  |  |  | No Impact |

Air Quality

Where available, the significance criteria established by the applicable air quality management district or air pollution control district may be relied upon to make the following determinations. Would the project:

| Item | Impact Description | Potentially Significant Impact | Less Than Significant With Mitigation Incorporated | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- | --- | --- |
| A | Conflict with or obstruct implementation of the applicable air quality plan? |  |  |  | No Impact |
| B | Result in a cumulatively considerable net increase of any criteria pollutant for which the project region is non-attainment under an applicable federal or state ambient air quality standard? |  |  |  | No Impact |
| C | Expose sensitive receptors to substantial pollutant concentrations? |  |  |  | No Impact |
| D | Result in other emissions (such as those leading to odors) adversely affecting a substantial number of people? |  |  |  | No Impact |

Biological Resources

Would the project:

| Item | Impact Description | Potentially Significant Impact | Less Than Significant With Mitigation Incorporated | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- | --- | --- |
| A | Have a substantial adverse effect, either directly or through habitat modifications, on any species identified as a candidate, sensitive, or special status species in local or regional plans, policies, or regulations, or by the California Department of Fish and Wildlife or U.S. Fish and Wildlife Service? |  |  |  | No Impact |
| B | Have a substantial adverse effect on any riparian habitat or other sensitive natural community identified in local or regional plans, policies, regulations or by the California Department of Fish and Wildlife or U.S. Fish and Wildlife Service? |  |  |  | No Impact |
| C | Have a substantial adverse effect on federally protected wetlands (including, but not limited to, marsh, vernal, pool, coastal, etc.) through direct removal, filling, hydrological interruption, or other means? |  |  |  | No Impact |
| D | Interfere substantially with the movement of any native resident or migratory fish or wildlife species or with established native resident or migratory wildlife corridors, or impede the use of native wildlife nursery sites? |  |  |  | No Impact |
| E | Conflict with any local policies or ordinances protecting biological resources, such as a tree preservation policy or ordinance? |  |  |  | No Impact |
| F | Conflict with the provisions of an adopted Habitat Conservation Plan, Natural Community Conservation Plan, or other approved local, regional, or state habitat conservation plan? |  |  |  | No Impact |

Cultural Resources

Would the project:

| Item | Impact Description | Potentially Significant Impact | Less Than Significant With Mitigation Incorporated | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- | --- | --- |
| A | Cause a substantial adverse change in the significance of a historical resource pursuant to section 15064.5? |  |  |  | No Impact |
| B | Cause a substantial adverse change in the significance of an archaeological resource pursuant to section 15064.5? |  |  |  | No Impact |
| C | Disturb any human remains, including those interred outside of dedicated cemeteries? |  |  |  | No Impact |

Energy

Would the project:

| Item | Impact Description | Potentially Significant Impact | Less Than Significant With Mitigation Incorporated | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- | --- | --- |
| A | Result in potentially significant environmental impact due to wasteful, inefficient, or unnecessary consumption of energy resources, during project construction or operation? |  |  |  | No Impact |
| B | Conflict with or obstruct a state or local plan for renewable energy or energy efficiency? |  |  |  | No Impact |

Geology / Soils

Would the project:

| Item | Impact Description | Potentially Significant Impact | Less Than Significant With Mitigation Incorporated | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- | --- | --- |
| A | Expose people or structures to potential substantial adverse effects, including the risk of loss, injury, or death involving: Rupture of a known earthquake fault, as delineated on the most recent Alquist-Priolo Earthquake Fault Zoning Map issued by the State Geologist for the area or based on other substantial evidence of a known fault? Refer to Division of Mines and Geology Special Publication 42. |  |  |  | No Impact |
|  | Strong seismic ground shaking? |  |  |  | No Impact |
|  | Seismic-related ground failure, including liquefaction? |  |  |  | No Impact |
|  | Landslides? |  |  |  | No Impact |
| B | Result in substantial soil erosion or the loss of topsoil? |  |  |  | No Impact |
| C | Be located on a geologic unit or soil that is unstable, or that would become unstable as a result of the project, and potentially result in on- or off-site landslide, lateral spreading, subsidence, liquefaction, or collapse? |  |  |  | No Impact |
| D | Be located on expansive soil, as defined in Table 18-1-B of the Uniform Building Code (1994), creating substantial direct or indirect risks to life or property? |  |  |  | No Impact |
| E | Have soils incapable of adequately supporting the use of septic tanks or alternative wastewater disposal systems where sewers are not available for the disposal of wastewater? |  |  |  | No Impact |
| F | Directly or indirectly destroy a unique paleontological resource or site or unique geologic feature? |  |  |  | No Impact |

Greenhouse Gas Emissions

Would the project:

| Item | Impact Description | Potentially Significant Impact | Less Than Significant With Mitigation Incorporated | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- | --- | --- |
| A | Generate greenhouse gas emissions, either directly or indirectly, that may have a significant impact on the environment? |  |  |  | No Impact |
| B | Conflict with an applicable plan, policy or regulation adopted for the purpose of reducing the emissions of greenhouse gases? |  |  |  | No Impact |

Hazards and Hazardous Materials

Would the project:

| Item | Impact Description | Potentially Significant Impact | Less Than Significant With Mitigation Incorporated | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- | --- | --- |
| A | Create a significant hazard to the public or the environment through the routine transport, use, or disposal of hazardous materials? |  |  |  | No Impact |
| B | Create a significant hazard to the public or the environment through reasonably foreseeable upset and accident conditions involving the release of hazardous materials into the environment? |  |  |  | No Impact |
| C | Emit hazardous emissions or handle hazardous or acutely hazardous materials, substances, or waste within one-quarter mile of an existing or proposed school? |  |  |  | No Impact |
| D | Be located on a site which is included on a list of hazardous materials sites compiled pursuant to Government Code section 65962.5 and, as a result, would it create a significant hazard to the public or the environment? |  |  |  | No Impact |
| E | For a project located within an airport land use plan or, where such a plan has not been adopted, within two miles of a public airport or public use airport, would the project result in a safety hazard or excessive noise for people residing or working in the project area? |  |  |  | No Impact |
| F | Impair implementation of or physically interfere with an adopted emergency response plan or emergency evacuation plan? |  |  |  | No Impact |
| G | Expose people or structures, either directly or indirectly, to a significant risk of loss, injury or death involving wildland fires? |  |  |  | No Impact |

Hydrology and Water Quality

Would the project:

| Item | Impact Description | Potentially Significant Impact | Less Than Significant With Mitigation Incorporated | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- | --- | --- |
| A | Violate any water quality standards or waste discharge requirements or otherwise substantially degrade surface or ground water quality? |  |  |  | No Impact |
| B | Substantially decrease groundwater supplies or interfere substantially with groundwater recharge such that the project may impede sustainable groundwater management of the basin? |  |  |  | No Impact |
| C | Substantially alter the existing drainage pattern of the site or area, including through the alteration of the course of a stream or river or through the addition of impervious surfaces, in a manner which would: result in substantial erosion or siltation on- or off-site? |  |  |  | No Impact |
|  | substantially increase the rate or amount of surface runoff in a manner which would result in flooding on- or offsite; |  |  |  | No Impact |
|  | create or contribute runoff water which would exceed the capacity of existing or planned stormwater drainage systems or provide substantial additional sources of polluted runoff; or |  |  |  | No Impact |
|  | impede or redirect flood flows? |  |  |  | No Impact |
| D | In flood hazard, tsunami, or seiche zones, risk release of pollutants due to project inundation? |  |  |  | No Impact |
| E | Conflict with or obstruct implementation of a water quality control plan or sustainable groundwater management plan? |  |  |  | No Impact |

Land Use / Planning

Would the project:

| Item | Impact Description | Potentially Significant Impact | Less Than Significant With Mitigation Incorporated | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- | --- | --- |
| A | Physically divide an established community? |  |  |  | No Impact |
| B | Cause a significant environmental impact due to a conflict with any land use plan, policy, or regulation adopted for the purpose of avoiding or mitigating an environmental effect? |  |  |  | No Impact |

Mineral Resources

Would the project:

| Item | Impact Description | Potentially Significant Impact | Less Than Significant With Mitigation Incorporated | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- | --- | --- |
| A | Result in the loss of availability of a known mineral resource that would be a value to the region and the residents of the state? |  |  |  | No Impact |
| B | Result in the loss of availability of a locally-important mineral resource recovery site delineated on a local general plan, specific plan or other land use plan? |  |  |  | No Impact |

Noise

Would the project result in:

| Item | Impact Description | Potentially Significant Impact | Less Than Significant With Mitigation Incorporated | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- | --- | --- |
| A | Generation of a substantial temporary or permanent increase in ambient noise levels in the vicinity of the project in excess of standards established in the local general plan or noise ordinance, or applicable standards of other agencies? |  |  |  | No Impact |
| B | Generation of excessive groundborne vibration or groundborne noise levels? |  |  |  | No Impact |
| C | For a project located within the vicinity of a private airstrip or an airport land use plan or, where such a plan has not been adopted, within two miles of a public airport or public use airport, would the project expose people residing or working in the project area to excessive noise levels? |  |  |  | No Impact |

Population / Housing

Would the project:

| Item | Impact Description | Potentially Significant Impact | Less Than Significant With Mitigation Incorporated | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- | --- | --- |
| A | Induce substantial unplanned population growth in an area, either directly (for example, by proposing new homes and businesses) or indirectly (for example, through extension of roads or other infrastructure)? |  |  |  | No Impact |
| B | Displace substantial numbers of existing people or housing, necessitating the construction of replacement housing elsewhere? |  |  |  | No Impact |

Public Services

Would the project result in substantial adverse physical impacts associated with the provision of new or physically altered governmental facilities, need for new or physically altered governmental facilities, the construction of which could cause significant environmental impacts, in order to maintain acceptable service ratios, response times or other performance objectives for any of the public services:

| Item | Impact Description | Potentially Significant Impact | Less Than Significant With Mitigation Incorporated | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- | --- | --- |
| A | Fire protection? |  |  |  | No Impact |
| B | Police protection? |  |  |  | No Impact |
| C | Schools? |  |  |  | No Impact |
| D | Parks? |  |  |  | No Impact |
| E | Other public facilities? |  |  |  | No Impact |

Recreation

| Item | Impact Description | Potentially Significant Impact | Less Than Significant With Mitigation Incorporated | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- | --- | --- |
| A | Would the project increase the use of existing neighborhood and regional parks or other recreational facilities such that substantial physical deterioration of the facility would occur or be accelerated? |  |  |  | No Impact |
| B | Does the project include recreational facilities or require the construction or expansion of recreational facilities which might have an adverse physical effect on the environment? |  |  |  | No Impact |

Transportation

Would the project:

| Item | Impact Description | Potentially Significant Impact | Less Than Significant With Mitigation Incorporated | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- | --- | --- |
| A | Conflict with a program, plan, ordinance or policy addressing the circulation system, including transit, roadway, bicycle and pedestrian facilities? |  |  |  | No Impact |
| B | Would the project conflict or be inconsistent with CEQA Guidelines section 15064.3, subdivision (b)? |  |  |  | No Impact |
| C | Substantially increase hazards due to a geometric design feature (e.g., sharp curves or dangerous intersections) or incompatible uses (e.g., farm equipment)? |  |  |  | No Impact |
| D | Result in inadequate emergency access? |  |  |  | No Impact |

Tribal Cultural Resources

Would the project cause a substantial adverse change in the significance of a tribal cultural resource, defined in PRC section 21074 as either a site, feature, place, cultural landscape that is geographically defined in terms of the size and scope of the landscape, sacred place, or object with cultural value to a California Native American tribe, and that is:

| Item | Impact Description | Potentially Significant Impact | Less Than Significant With Mitigation Incorporated | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- | --- | --- |
| A | Listed or eligible for listing in the California Register of Historical Resources, or in a local register of historical resources as defined in Public Resources Code section 5020.1(k)? |  |  |  | No Impact |
| B | A resource determined by the lead agency, in its discretion and supported by substantial evidence, to be significant pursuant to criteria set forth in subdivision (c) of Public Resources Code section 5024.1? In applying the criteria set forth in subdivision (c) of Public Resource Code section 5024.1, the lead agency shall consider the significance of the resource to a California Native American tribe. |  |  |  | No Impact |

Utilities / Service Systems

Would the project:

| Item | Impact Description | Potentially Significant Impact | Less Than Significant With Mitigation Incorporated | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- | --- | --- |
| A | Require or result in the relocation or construction of new or expanded water, wastewater treatment or storm water drainage, electric power, natural gas, or telecommunications facilities, the construction or relocation of which could cause significant environmental effects? |  |  |  | No Impact |
| B | Have sufficient water supplies available to serve the project and reasonably foreseeable future development during normal, dry and multiple dry years? |  |  |  | No Impact |
| C | Result in a determination by the wastewater treatment provider, which serves or may serve the project that it has adequate capacity to serve the project’s projected demand in addition to the provider’s existing commitments? |  |  |  | No Impact |
| D | Generate solid waste in excess of state or local standards, or in excess of the capacity of local infrastructure, or otherwise impair the attainment of solid waste reduction goals? |  |  |  | No Impact |
| E | Comply with federal, state, and local management and reduction statutes and regulations related to solid waste? |  |  |  | No Impact |

Wildfire

If located in or near state responsibility areas or lands classified as very high fire hazard severity zones, would the project:

| Item | Impact Description | Potentially Significant Impact | Less Than Significant With Mitigation Incorporated | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- | --- | --- |
| A | Substantially impair an adopted emergency response plan or emergency evacuation plan? |  |  |  | No Impact |
| B | Due to slope, prevailing winds, and other factors, exacerbate wildfire risks, and thereby expose project occupants to pollutant concentrations from a wildfire or the uncontrolled spread of a wildfire? |  |  |  | No Impact |
| C | Require the installation or maintenance of associated infrastructure (such as roads, fuel breaks, emergency water sources, power lines or other utilities) that may exacerbate fire risk or that may result in temporary or ongoing impacts to the environment? |  |  |  | No Impact |
| D | Expose people or structures to significant risks, including downslope or downstream flooding or landslides, as a result of runoff, post-fire slope instability, or drainage changes? |  |  |  | No Impact |

Mandatory Findings of Significance

| Item | Impact Description | Potentially Significant Impact | Less Than Significant With Mitigation Incorporated | Less Than Significant Impact | No Impact |
| --- | --- | --- | --- | --- | --- |
| A | Does the project have the potential to substantially degrade the quality of the environment, substantially reduce the habitat of a fish or wildlife species, cause a fish or wildlife population to drop below self-sustaining levels, threaten to eliminate a plant or animal community, substantially reduce the number or restrict the range of a rare or endangered plant or animal or eliminate important examples of the major periods of California history or prehistory? |  |  |  | No Impact |
| B | Does the project have impacts that are individually limited, but cumulatively considerable? (“Cumulatively considerable” means that the incremental effects of a project are considerable when viewed in connection with the effects of past projects, the effects of other current projects, and the effects of probable future projects.) |  |  |  | No Impact |
| C | Does the project have environmental effects which will cause substantial adverse effects on human beings, either directly or indirectly? |  |  |  | No Impact |

The Central Coast Water Board’s Basin Plan website: https://www.waterboards.ca.gov/centralcoast/water_issues/programs/basin_plan/

Information about the 2024 Triennial Review is available on the Central Coast Water Board’s Triennial Review webpage: https://www.waterboards.ca.gov/centralcoast/water_issues/programs/basin_plan/tri-review.html

The USEPA Water Quality Standards Handbook, Chapter 3, discusses development of water quality standards consistent with relevant regulatory and statutory requirements: https://www.epa.gov/sites/default/files/2014-10/documents/handbook-chapter3.pdf

Antidegradation Policy (State Water Board Resolution 68-16): https://www.waterboards.ca.gov/board_decisions/adopted_orders/resolutions/1968/rs68_016.pdf

USEPA Occurrence Assessment for the Final Stage 2 Disinfectants and Disinfection Byproducts Rule:  https://nepis.epa.gov/Exe/ZyPDF.cgi?Dockey=P1005ED2.txt

USEPA National Primary Drinking Water Regulations: Disinfectants and Disinfection Byproducts (Federal Register, Volume 63, Number 241, 1998): https://www.govinfo.gov/content/pkg/FR-1998-12-16/pdf/98-32887.pdf#page=1