fTITLE 22, CALIFORNIA CODE OF REGULATIONS

DIVISION 4, CHAPTER 15, ARTICLE 5.5

(1)  Amend Section 64444 to read as follows:

§ 64444. Maximum Contaminant Levels – Organic Chemicals.

The MCLs for the primary drinking water chemicals shown in Ttable 64444-A shall not be exceeded in the water supplied to the public.

Table 64444-A

Maximum Contaminant Levels

Organic Chemicals

|  | Maximum |
| --- | --- |
|  | Contaminant |
| Chemical | Level, mg/L |
| (a) Volatile Organic Chemicals (VOCs) |  |
| Benzene. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.001 |
| Carbon Tetrachloride. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.0005 |
| 1,2-Dichlorobenzene. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.6 |
| 1,4-Dichlorobenzene. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.005 |
| 1,1-Dichloroethane. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.005 |
| 1,2-Dichloroethane. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.0005 |
| 1,1-Dichloroethylene. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.006 |
| cis-1,2-Dichloroethylene. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.006 |
| trans-1,2-Dichloroethylene. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.01 |
| Dichloromethane. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.005 |
| 1,2-Dichloropropane. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.005 |
| 1,3-Dichloropropene. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.0005 |
| Ethylbenzene. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.3 |
| Methyl-tert-butyl ether. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.013 |
| Monochlorobenzene. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.07 |
| Styrene. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.1 |
| 1,1,2,2-Tetrachloroethane. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.001 |
| Tetrachloroethylene. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.005 |
| Toluene. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.15 |
| 1,2,4-Trichlorobenzene. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.005 |
| 1,1,1-Trichloroethane. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.200 |
| 1,1,2-Trichloroethane. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.005 |
| Trichloroethylene. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.005 |
| Trichlorofluoromethane. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.15 |
| 1,1,2-Trichloro-1,2,2-Trifluoroethane. . . . . . . . . . . . . . . . . . . . . . . . . . . | 1.2 |
| Vinyl Chloride. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.0005 |
| Xylenes. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 1.750* |
| (b) Non-Volatile Synthetic Organic Chemicals (SOCs) |  |
| Alachlor. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.002 |
| Atrazine. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.001 |
| Bentazon. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.018 |
| Benzo(a)pyrene. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.0002 |
| Carbofuran. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.018 |
| Chlordane. . . . . . . . . . . . . . .. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.0001 |
| 2,4-D. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.07 |
| Dalapon. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.2 |
| Dibromochloropropane. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.0002 |
| Di(2-ethylhexyl)adipate. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.4 |
| Di(2-ethylhexyl)phthalate. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.004 |
| Dinoseb. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.007 |
| Diquat. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.02 |
| Endothall. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.1 |
| Endrin. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.002 |
| Ethylene Dibromide. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.00005 |
| Glyphosate. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.7 |
| Heptachlor. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.00001 |
| Heptachlor Epoxide. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.00001 |
| Hexachlorobenzene. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.001 |
| Hexachlorocyclopentadiene. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.05 |
| Lindane. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.0002 |
| Methoxychlor. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.03 |
| Molinate. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.02 |
| Oxamyl. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.05 |
| Pentachlorophenol. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.001 |
| Picloram. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.5 |
| Polychlorinated Biphenyls. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.0005 |
| Simazine. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.004 |
| Thiobencarb. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.07 |
| Toxaphene. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.003 |
| 1,2,3-Trichloropropane. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.000005 |
| 2,3,7,8-TCDD (Dioxin). . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 3 x 10-8 |
| 2,4,5-TP (Silvex) | 0.05 |

*MCL is for either a single isomer or the sum of the isomers.

NOTE:  Authority cited:  Sections 116271, 116350 and 116365, Health and Safety Code.  Reference:  Sections 116365, 116385, and 116555, Health and Safety Code.

(2)  Amend Section 64445 to read as follows:

§ 64445. Initial Sampling – Organic Chemicals

- No change to text

- No change to text

- No change to text

- No change to text

- No change to text

- No change to text

- MTBE Ddata (i.e., a single sample) collected in a manner consistent with this section after January 1, 1998 in which no MTBE is detected, along with a designation of nonvulnerability pursuant to subsection (d), may be used to satisfy the initial monitoring requirements in subsection (a).  If the requirements are satisfied in this way by a water system, the system shall begin annual monitoring pursuant to section 64445.1(b)(1).

- No change to text

- Results obtained from groundwater monitoring performed for an organic chemical in accordance with this section and not more than two calendar years prior to the effective date of a regulation establishing the MCL for that organic chemical may be substituted to partially satisfy the initial monitoring requirements required by this section for that organic chemical.  Requests to substitute groundwater monitoring results shall be made in accordance with the following:

  - Requests shall be made in writing by the water system to the State Board; and

  - If the State Board approves the request then results from a given calendar quarter will only be eligible to substitute for a single required initial monitoring result during that same quarter of initial monitoring.     (e.g. the second quarter of 2016 may be substituted for the second quarter of 2018).

  - No more than three of the four quarterly samples as required by 64445(a) or (b) may be substituted.

NOTE:  Authority cited:  Sections 116271, 116350 and 116375, Health and Safety Code.  Reference:  Sections 116385 and 116555, Health and Safety Code.

(3)  Amend Section 64445.1 to read as follows:

§ 64445.1. Repeat Monitoring and Compliance – Organic Chemicals.

(a)  For the purposes of this article, detection shall be defined by the detection limits for purposes of reporting (DLRs) in Ttable 64445.1-A:

Table 64445.1-A

Detection Limits for Purposes of Reporting (DLRs)

for Regulated Organic Chemicals

|  | Detection Limit for |
| --- | --- |
|  | Purposes of Reporting |
| Chemical | (DLR) (mg/L) |
| (a) All VOCs, except as listed. . . . . . . . . . . . . . . . . . . . . . . . . . | 0.0005 |
| Methyl-tert-butyl ether. . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.003 |
| Trichlorofluoromethane. . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.005 |
| 1,1,2-Trichloro-1,2,2-Trifluoroethane.. . . . . . .. . . . . . . . . . | 0.01 |
| (b) SOCs |  |
| Alachlor. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.001 |
| Atrazine. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.0005 |
| Bentazon. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.002 |
| Benzo(a)pyrene. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.0001 |
| Carbofuran. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.005 |
| Chlordane. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.0001 |
| 2,4-D. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.01 |
| Dalapon. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.01 |
| Dibromochloropropane (DBCP). . . . . . . . . . . . . . . . . . . . . . | 0.00001 |
| Di(2-ethylhexyl)adipate. . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.005 |
| Di(2-ethylhexyl)phthalate. . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.003 |
| Dinoseb. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.002 |
| Diquat. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.004 |
| Endothall. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.045 |
| Endrin. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.0001 |
| Ethylene dibromide (EDB). . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.00002 |
| Glyphosate. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.025 |
| Heptachlor. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.00001 |
| Heptachlor epoxide. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.00001 |
| Hexachlorobenzene. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.0005 |
| Hexachlorocyclopentadiene. . . . . . . . . . . . . . . . . . . . . . . . . | 0.001 |
| Lindane. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.0002 |
| Methoxychlor. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.01 |
| Molinate. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.002 |
| Oxamyl. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.02 |
| Pentachlorophenol. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.0002 |
| Picloram. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.001 |
| Polychlorinated biphenyls (PCBs) |  |
| (as decachlorobiphenyl). . . . . . . . .. . . . . . . . . . . . . . . . . . . . | 0.0005 |
| Simazine. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.001 |
| Thiobencarb. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.001 |
| Toxaphene. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.001 |
| 1,2,3-Trichloropropane. . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.000005 |
| 2,3,7,8-TCDD (Dioxin). . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 5 x 10-9 |
| 2,4,5-TP (Silvex). . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . | 0.001 |

(b)  When organic chemicals are not detected pursuant to Ttable 64445.1-A.

(1)  A water system which has not detected any of the VOCs on Ttable 64444-A during the initial four quarters of monitoring, shall collect and analyze one sample annually.  After a minimum of three years of annual sampling with no detection of a VOC in Ttable 64444-A, a system using groundwater may reduce the monitoring frequency to one sample during each compliance period.  A system using surface water shall continue monitoring annually.

(2)  A system serving more than 3,300 persons which has not detected an SOC on Ttable 64444-A during the initial four quarters of monitoring shall collect a minimum of two quarterly samples for that SOC in one year during the year designated by the State Board of each subsequent compliance period.  The year will be designated on the basis of historical monitoring frequency and laboratory capacity.

(3)  A system serving 3,300 persons or less which has not detected an SOC on Ttable 64444-A during the initial four quarters of monitoring shall collect a minimum of one sample for that SOC during the year designated by the State Board of each subsequent compliance period.  The year will be designated on the basis of historical monitoring frequency and laboratory capacity.

(c)  When organic chemicals are detected pursuant to Ttable 64445.1-A.

(1)  Prior to proceeding with the requirements of paragraphs (c)(2) through (7), the water supplier may first confirm the analytical result, as follows:  Within seven days from the notification of an initial finding from a laboratory reporting the presence of one or more organic chemicals in a water sample, the water supplier shall collect one or two additional sample(s) to confirm the initial finding.  Confirmation of the initial finding shall be shown by the presence of the organic chemical in either the first or second additional sample, and the detected level of the contaminant for compliance purposes shall be the average of the initial and confirmation sample(s).  The initial finding shall be disregarded if two additional samples do not show the presence of the organic chemical.

(2)  If one or both of the related organic chemicals heptachlor and heptachlor epoxide are detected, subsequent monitoring shall analyze for both chemicals until there has been no detection of either chemical for one compliance period.

(3)  A groundwater sampling site at which one or more of the following chemicals has been detected shall be monitored quarterly for vinyl chloride:  trichloroethylene, tetrachloroethylene, 1,2-dichloroethane, 1,1,1-trichloroethane, cis-1,2-dichloroethylene, trans-1,2-dichloroethylene, or 1,1-dichloroethylene.  If vinyl chloride is not detected in the first quarterly sample, the sampling site shall be monitored once for vinyl chloride during each compliance period.

(4)  If the detected level of organic chemicals for any sampling site does not exceed any shown in Ttable 64444-A, the water source shall be resampled every three months and the samples analyzed for the detected chemicals.  After one year of sampling an approved surface water system or two quarters of sampling a groundwater system, the State Board will consider allowing the water supplier to reduce the sampling to once per year upon request, based on a review of previous sampling data.  Systems shall monitor during the quarter(s) which previously yielded the highest analytical results.

(5)  If the detected level of an organic chemical for any sampling site exceeds that listed in Ttable 64444-A, the water supplier shall report this information to the State Board within 48 hours of receipt of the result.  Unless use of the contaminated source is discontinued, the water supplier shall resample the contaminated source and compliance shall be determined as follows:

(A)  Water systems serving more than 3,300 persons shall sample monthly for six months and shall submit the results to the State Board as specified in Ssection 64469.  If the average concentration of the initial finding, confirmation sample(s), and six subsequent monthly samples does not exceed the MCL shown in Ttable 64444-A the water supplier may reduce the sampling frequency to once every three months.  If the running annual average or the average concentration of the initial finding, confirmation sample(s), and six subsequent monthly samples exceeds the MCL shown in Ttable 64444-A, the water system shall be deemed to be in violation of Ssection 64444.

(B)  Water systems serving 3,300 persons or less shall sample quarterly for a minimum of one year and shall submit the results to the State Board as specified in Ssection 64469.  If the running annual average concentration does not exceed the MCL in Ttable 64444-A, the water supplier may reduce the sampling frequency to once every year during the quarter that previously yielded the highest analytical result.  Quarterly monitoring shall resume if any reduced frequency sample result exceeds the MCL.  If the running annual average concentration exceeds the MCL in Ttable 64444-A, the water system shall be deemed to be in violation of Ssection 64444.

(C)  If any sample would cause the running annual average to exceed the MCL, the water system is immediately in violation.  If a system takes more than one sample in a quarter, the average of all the results for that quarter shall be used when calculating the running annual average.  If a system fails to complete four consecutive quarters of monitoring, the running annual average shall be based on an average of the available data.

(6)  If any resample, other than those taken in accordance with paragraph (c)(5) of this section, of a water sampling site shows that the concentration of any organic chemical exceeds a MCL shown in Ttable 64444-A, the water supplier shall proceed in accordance with paragraphs (c)(1) and (c)(4), or paragraph (c)(5).

(7)  If an organic chemical is detected and the concentration exceeds ten times the MCL, the water supplier shall notify the State Board within 48 hours of the receipt of the results and the contaminated site shall be resampled within 48 hours to confirm the result.  The water supplier shall notify the State Board of the result of the confirmation sample(s) within 24 hours of the receipt of the confirmation result(s).

(A)  If the average concentration of the original and confirmation sample(s) is less than or equal to ten times the MCL, the water supplier shall proceed in accordance with subsectionparagraph (c)(5).

(B)  If the average concentration of the original and confirmation samples exceeds ten times the MCL, use of the contaminated water source shall immediately be discontinued, if directed by the State Board.  Such a water source shall not be returned to service without written approval from the State Board.

NOTE:  Authority cited:  Sections 116271, 116350 and 116375, Health and Safety Code.  Reference:  Sections 116385, 116450, 116460, and 116555, Health and Safety Code.

TITLE 22, CALIFORNIA CODE OF REGULATIONS

DIVISION 4, CHAPTER 15, ARTICLE 12

(4)  Amend Section 64447.4 to read as follows:

§ 64447.4. Best Available Technologies (BAT) – Organic Chemicals.

The technologies listed in Ttable 64447.4-A are the best available technology, treatment technologies, or other means available for achieving compliance with the MCLs in Ttable 64444-A for organic chemicals.

Table 64447.4-A

Best Available Technologies (BATs)

Organic Chemicals

| Chemical |  |  | Best Available Technologies |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Granular | Packed |  |
|  |  |  | Activated | Tower |  |
|  |  |  | Carbon | Aeration | Oxidation |
| (a) Volatile Organic Chemicals (VOCs) |  |  |  |  |  |
| Benzene |  |  | X | X |  |
| Carbon Tetrachloride |  |  | X | X |  |
| 1,2-Dichlorobenzene |  |  | X | X |  |
| 1,4-Dichlorobenzene |  |  | X | X |  |
| 1,1-Dichloroethane |  |  | X | X |  |
| 1,2-Dichloroethane |  |  | X | X |  |
| 1,1-Dichloroethylene |  |  | X | X |  |
| cis-1,2-Dichloroethylene |  |  | X | X |  |
| trans-1,2-Dichloroethylene |  |  | X | X |  |
| Dichloromethane |  |  |  | X |  |
| 1,2-Dichloropropane |  |  | X | X |  |
| 1,3-Dichloropropene |  |  | X | X |  |
| Ethylbenzene |  |  | X | X |  |
| Methyl-tert-butyl ether |  |  |  | X |  |
| Monochlorobenzene |  |  | X | X |  |
| Styrene |  |  | X | X |  |
| 1,1,2,2-Tetrachloroethane |  |  | X | X |  |
| Tetrachloroethylene |  |  | X | X |  |
| Toluene |  |  | X | X |  |
| 1,2,4-Trichlorobenzene |  |  | X | X |  |
| 1,1,1-Trichloroethane |  |  | X | X |  |
| 1,1,2-Trichloroethane |  |  | X | X |  |
| Trichlorofluoromethane |  |  | X | X |  |
| Trichlorotrifluoroethane |  |  | X | X |  |
| Trichloroethylene |  |  | X | X |  |
| Vinyl Chloride |  |  |  | X |  |
| Xylenes |  |  | X | X |  |
| (b) Synthetic Organic Chemicals (SOCs) |  |  |  |  |  |
| Alachlor |  |  | X | X |  |
| Atrazine |  |  | X |  |  |
| Bentazon |  |  |  | X |  |
| Benzo(a)pyrene |  |  | X |  |  |
| Carbofuran |  |  | X |  |  |
| Chlordane |  |  | X |  |  |
| 2,4-D |  |  | X |  |  |
| Dalapon |  |  | X |  |  |
| Di(2-ethylhexyl)adipate |  |  | X | X |  |
| Dinoseb |  |  | X |  |  |
| Diquat |  |  | X |  |  |
| 1,2-Dibromo-3-chloropropane |  |  | X | X |  |
| Di(2-ethylhexyl)phthalate |  |  | X |  |  |
| Endothall |  |  | X |  |  |
| Endrin |  |  | X |  |  |
| Ethylene Dibromide |  |  | X | X |  |
| Glyphosate |  |  |  |  | X |
| Heptachlor |  |  | X |  |  |
| Heptachlor epoxide |  |  | X |  |  |
| Hexachlorocyclopentadiene |  |  | X | X |  |
| Lindane |  |  | X |  |  |
| Methoxychlor |  |  | X |  |  |
| Molinate |  |  | X |  |  |
| Oxamyl |  |  | X |  |  |
| Picloram |  |  | X |  |  |
| Pentachlorophenol |  |  | X |  |  |
| Polychlorinated Biphenyls |  |  | X |  |  |
| Simazine |  |  | X |  |  |
| Thiobencarb |  |  | X |  |  |
| [delete this empty line] |  |  |  |  |  |
| Toxaphene |  |  | X | X |  |
| 1,2,3-Trichloropropane |  |  | X |  |  |
| 2,3,7,8-TCDD (Dioxin) |  |  | X |  |  |
| 2,4,5-TP (Silvex) |  |  | X |  |  |

NOTE:  Authority cited:  Sections 116271, 116350, 116370, and 116375, Health and Safety Code.  Reference:  Section 116350116370, Health and Safety Code.

TITLE 22, CALIFORNIA CODE OF REGULATIONS

DIVISION 4, CHAPTER 15, ARTICLE 18

(5)  Amend Section 64465 to read as follows:

§ 64465. Public Notice Content and Format.

- through (d)  No Change to Text.

Appendix 64465-A. Health Effects Language

Microbiological Contaminants.

| Contaminant | Health Effects language |
| --- | --- |
| Total Coliform | No Change to Text. |
| Fecal coliform/E.coli | No Change to Text. |
| Turbidity | No Change to Text. |

Appendix 64465-B.  Health Effects Language

Surface Water Treatment

| Contaminant | Health Effects language |
| --- | --- |
| Giardia lamblia Viruses Heterotrophic plate count bacteria Legionella Cryptosporidium | No Change to Text. |

Appendix 64465-C.  Health Effects Language

Radioactive Contaminants.

| Contaminant | Health Effects Language |
| --- | --- |
| Gross Beta particle activity | No Change to Text. |
| Strontium-90 | No Change to Text. |
| Tritium | No Change to Text. |
| Gross Alpha particle activity | No Change to Text. |
| Combined Radium 226/228 | No Change to Text. |
| Total Radium (for nontransient noncommunity water systems) | No Change to Text. |
| Uranium | No Change to Text. |

Appendix 64465-D.  Health Effects Language

Inorganic Contaminants.

| Contaminant | Health Effects Language |
| --- | --- |
| Aluminum | No Change to Text. |
| Antimony | No Change to Text. |
| Arsenic | No Change to Text. |
| Asbestos | No Change to Text. |
| Barium | No Change to Text. |
| Beryllium | No Change to Text. |
| Cadmium | No Change to Text. |
| Chromium | No Change to Text. |
| Copper | No Change to Text. |
| Cyanide | No Change to Text. |
| Fluoride | No Change to Text. |
| Lead | No Change to Text. |
| Mercury | No Change to Text. |
| Nickel | No Change to Text. |
| Nitrate | No Change to Text. |
| Nitrite | No Change to Text. |
| Perchlorate | No Change to Text. |
| Selenium | No Change to Text. |
| Thallium | No Change to Text. |

Appendix 64465-E.  Health Effects Language

Volatile Organic Contaminants.

| Contaminant | Health Effects Language |
| --- | --- |
| Benzene | No Change to Text. |
| Carbon Tetrachloride | No Change to Text. |
| 1,2-Dichlorobenzene | No Change to Text. |
| 1,4-Dichlorobenzene | No Change to Text. |
| 1,1-Dichloroethane | No Change to Text. |
| 1,2-Dichloroethane | No Change to Text. |
| 1,1-Dichloroethylene | No Change to Text. |
| cis-1,2-Dichloroethylene | No Change to Text. |
| trans-1,2-Dichloroethylene | No Change to Text. |
| Dichloromethane | No Change to Text. |
| 1,2-Dichloropropane | No Change to Text. |
| 1,3-Dichloropropene | No Change to Text. |
| Ethylbenzene | No Change to Text. |
| Methyl-tert-butyl ether | No Change to Text. |
| Monochlorobenzene | No Change to Text. |
| Styrene | No Change to Text. |
| 1,1,2,2-Tetrachloroethane | No Change to Text. |
| Tetrachloroethylene | No Change to Text. |
| 1,2,4-Trichlorobenzene | No Change to Text. |
| 1,1,1,-Trichloroethane | No Change to Text. |
| 1,1,2-Trichloroethane | No Change to Text. |
| Trichloroethylene (TCE) | No Change to Text. |
| Toluene | No Change to Text. |
| Trichlorofluoromethane | No Change to Text. |
| 1,1,2-Trichloro-1,2,2-trifluoroethane | No Change to Text. |
| Vinyl Chloride | No Change to Text. |
| Xylenes | No Change to Text. |

Appendix 64465-F.  Health Effects Language

Synthetic Organic Contaminants.

| Contaminant | Health Effects Language |
| --- | --- |
| 2,4-D | No Change to Text. |
| 2,4,5-TP (Silvex) | No Change to Text. |
| Alachlor | No Change to Text. |
| Atrazine | No Change to Text. |
| Bentazon | No Change to Text. |
| Benzo(a)pyrene [PAH] | No Change to Text. |
| Carbofuran | No Change to Text. |
| Chlordane | No Change to Text. |
| Dalapon | No Change to Text. |
| Dibromochloropropane (DBCP) | No Change to Text. |
| Di (2-ethylhexyl) adipate | No Change to Text. |
| Di (2-ethylhexyl) phthalate | No Change to Text. |
| Dinoseb | No Change to Text. |
| Dioxin (2,3,7,8-TCDD): | No Change to Text. |
| Diquat | No Change to Text. |
| Endothall | No Change to Text. |
| Endrin | No Change to Text. |
| Ethylene dibromide (EDB) | No Change to Text. |
| Glyphosate | No Change to Text. |
| Heptachlor | No Change to Text. |
| Heptachlor epoxide | No Change to Text. |
| Hexachlorobenzene | No Change to Text. |
| Hexachlorocyclopentadiene | No Change to Text. |
| Lindane | No Change to Text. |
| Methoxychlor | No Change to Text. |
| Molinate (Ordram) | No Change to Text. |
| Oxamyl [Vydate]: | No Change to Text. |
| PCBs [Polychlorinated biphenyls]: | No Change to Text. |
| Pentachlorophenol | No Change to Text. |
| Picloram | No Change to Text. |
| Simazine | No Change to Text. |
| Thiobencarb | No Change to Text. |
| Toxaphene | No Change to Text. |
| 1,2,3-Trichloropropane | Some people who drink water containing 1,2,3-trichloropropane in excess of the MCL over many years may have an increased risk of getting cancer. |

Appendix 64465-G. Health Effects Language

Disinfection Byproducts, Byproduct Precursors, and Disinfectant Residuals

| Contaminant | Health Effects language |
| --- | --- |
| TTHMs [Total Trihalomethanes]: | No Change to Text. |
| Haloacetic Acids | No Change to Text. |
| Bromate | No Change to Text. |
| Chloramines | No Change to Text. |
| Chlorine | No Change to Text. |
| Chlorite | No Change to Text. |
| Chlorine dioxide (2 consecutive daily samples at the entry point to the distribution system that are greater than the MRDL) | No Change to Text. |
| Chlorine dioxide (one or more distribution system samples are above the MRDL.) | No Change to Text. |
| Control of DBP precursors (TOC) | No Change to Text. |

Appendix 64465-H. Health Effects Language

Other Treatment Techniques

| Contaminant | Health Effects language |
| --- | --- |
| Acrylamide | No Change to Text. |
| Epichlorohydrin | No Change to Text. |

NOTE:  Authority cited:  Sections 116271, 116325, 116350 and 116375, Health and Safety Code.  Reference:  Section 116450, Health and Safety Code.

TITLE 22, CALIFORNIA CODE OF REGULATIONS

DIVISION 4, CHAPTER 15, ARTICLE 20

(6)  Amend Section 64481 to read as follows:

§ 64481. Content of the Consumer Confidence Report.

(a)  through (m)  No Change to Text.

Appendix 64481-A.

Typical Origins of Contaminants with Primary MCLs, MRDLs, Regulatory Action Levels, and Treatment Techniques

| Contaminant | Major origins in drinking water |
| --- | --- |
| Microbiological |  |
| Total coliform bacteria | No Change to Text. |
| Fecal coliform and E. coli | No Change to Text. |
| Turbidity | No Change to Text. |
| Surface water treatment |  |
| Giardia lamblia | No Change to Text. |
| Viruses |  |
| Heterotrophic plate count bacteria |  |
| Legionella |  |
| Cryptosporidium |  |
| Radioactive |  |
| Gross Beta particle activity | No Change to Text. |
| Strontium-90 | No Change to Text. |
| Tritium | No Change to Text. |
| Gross Alpha particle activity | No Change to Text. |
| Combined rRadium 226/228 | No Change to Text. |
| Total Radium (for nontransient noncommunity water systems) | No Change to Text. |
| Uranium | No Change to Text. |
| Inorganic |  |
| Aluminum | No Change to Text. |
| Antimony | No Change to Text. |
| Arsenic | No Change to Text. |
| Asbestos | No Change to Text. |
| Barium | No Change to Text. |
| Beryllium | No Change to Text. |
| Cadmium | No Change to Text. |
| Chromium | No Change to Text. |
| Copper | No Change to Text. |
| Cyanide | No Change to Text. |
| Fluoride | No Change to Text. |
| Hexavalent chromium | No Change to Text. |
| Lead | No Change to Text. |
| Mercury | No Change to Text. |
| Nickel | No Change to Text. |
| Nitrate | No Change to Text. |
| Nitrite | No Change to Text. |
| Perchlorate | No Change to Text. |
| Selenium | No Change to Text. |
| Thallium | No Change to Text. |
| Synthetic organic |  |
| 2,4-D | No Change to Text. |
| 2,4,5-TP (Silvex) | No Change to Text. |
| Acrylamide | No Change to Text. |
| Alachlor | No Change to Text. |
| Atrazine | No Change to Text. |
| Bentazon | No Change to Text. |
| Benzo(a)pyrene [PAH] | No Change to Text. |
| Carbofuran | No Change to Text. |
| Chlordane | No Change to Text. |
| Dalapon | No Change to Text. |
| Dibromochloropropane (DBCP) | No Change to Text. |
| Di(2-ethylhexyl) adipate | No Change to Text. |
| Di(2-ethylhexyl) phthalate | No Change to Text. |
| Dinoseb | No Change to Text. |
| Dioxin [2,3,7,8-TCDD] | No Change to Text. |
| Diquat | No Change to Text. |
| Endothall | No Change to Text. |
| Endrin | No Change to Text. |
| Epichlorohydrin | No Change to Text. |
| Ethylene dibromide (EDB) | No Change to Text. |
| Glyphosate | No Change to Text. |
| Heptachlor | No Change to Text. |
| Heptachlor epoxide | No Change to Text. |
| Hexachlorobenzene | No Change to Text. |
| Hexachlorocyclo-[delete this hyphen]pentadiene | No Change to Text. |
| Lindane | No Change to Text. |
| Methoxychlor | No Change to Text. |
| Molinate [Ordram] | No Change to Text. |
| Oxamyl [Vydate] | No Change to Text. |
| Pentachlorophenol | No Change to Text. |
| Picloram | No Change to Text. |
| Polychlorinated biphenyls [PCBs] | No Change to Text. |
| Simazine | No Change to Text. |
| Thiobencarb | No Change to Text. |
| Toxaphene | No Change to Text. |
| 1,2,3-Trichloropropane | Discharge from industrial and agricultural chemical factories; leaching from hazardous waste sites; used as cleaning and maintenance solvent, paint and varnish remover, and cleaning and degreasing agent; byproduct during the production of other compounds and pesticides. |
| Volatile organic |  |
| Benzene | No Change to Text. |
| Carbon tetrachloride | No Change to Text. |
| 1,2-Dichlorobenzene | No Change to Text. |
| 1,4-Dichlorobenzene | No Change to Text. |
| 1,1-Dichloroethane | No Change to Text. |
| 1,2-Dichloroethane | No Change to Text. |
| 1,1-Dichloroethylene | No Change to Text. |
| cis-1,2-Dichloroethylene | No Change to Text. |
| trans-1,2-Dichloroethylene | No Change to Text. |
| Dichloromethane | No Change to Text. |
| 1,2-Dichloropropane | No Change to Text. |
| 1,3-Dichloropropene | No Change to Text. |
| Ethylbenzene | No Change to Text. |
| Methyl-tert-butyl ether (MTBE) | No Change to Text. |
| Monochlorobenzene | No Change to Text. |
| Styrene | No Change to Text. |
| 1,1,2,2-Tetrachloroethane | No Change to Text. |
| Tetrachloroethylene (PCE) | No Change to Text. |
| 1,2,4-Trichlorobenzene | No Change to Text. |
| 1,1,1-Trichloroethane | No Change to Text. |
| 1,1,2-Trichloroethane | No Change to Text. |
| Trichloroethylene (TCE) | No Change to Text. |
| Toluene | No Change to Text. |
| Trichlorofluoromethane | No Change to Text. |
| 1,1,2-Trichloro-1,2,2- Trifluoroethane | No Change to Text. |
| Vinyl chloride | No Change to Text. |
| Xylenes | No Change to Text. |

Appendix 64481-B.

Typical Origins of Contaminants with Secondary MCLs

| Contaminant | Major origins in drinking water |
| --- | --- |
| Aluminum | No Change to Text. |
| Color | No Change to Text. |
| Copper | No Change to Text. |
| Foaming Agents (MBAS) | No Change to Text. |
| Iron | No Change to Text. |
| Manganese | No Change to Text. |
| Methyl-tert-butyl ether (MTBE) | No Change to Text. |
| Odor---Threshold | No Change to Text. |
| Silver | No Change to Text. |
| Thiobencarb | No Change to Text. |
| Turbidity | No Change to Text. |
| Zinc | No Change to Text. |
| Total dissolved solids | No Change to Text. |
| Specific Conductance | No Change to Text. |
| Chloride | No Change to Text. |
| Sulfate | No Change to Text. |

NOTE:  Authority cited:  Sections 116271, 116350 and 116375, Health and Safety Code.  Reference:  Sections 116275 and 116470, Health and Safety Code.