| Contract No. GS-35F-4461G SDWIS Project Product Control No. SAIC-SDWIS-1.2d7 March 31, 2013 | SDWIS/XML Sampling 3.1 Schema Mapping Document |
| --- | --- |

SDWIS/XML

SAMPLING 3.1

SCHEMA MAPPING

DOCUMENT

CONTRACT NO. GS-35F-4461G

SDWIS PROJECT

Prepared for:

United States Environmental Protection Agency

Office of Ground Water and Drinking Water

Drinking Water Protection Division

1200 Pennsylvania Ave., NW

Washington, DC 20460

Contracting Officer Representative

Clint Lemmons

Prepared by:

SAIC Solutions Delivery Center

Science Applications International Corporation

8301 Greensboro Drive

McLean, VA 22102

Contents

1.0	INTRODUCTION	1

1.1	Overview of SDWIS/XML Sampling Schemas	1

1.1.1	SDWIS_eDWR_v3.0.xsd Schema	1

1.1.2	SDWIS_Summary_v3.0.xsd Schema	2

1.1.3	SDWIS_SummaryResult_v3.0.xsd Schema	2

1.1.4	SDWIS_MDBPSummary_v3.0.xsd Schema	3

1.2	Scope and Structure of Document and Accompanying Schema	3

1.2.1	Staging Table Structure and Business Rules	3

1.2.2	Enumerated Values	3

1.2.3	Mapping from XML Schema Tag to SDWIS/STATE Entities and   Attributes	4

1.2.4	Definitions	4

2.0	LABORATORY SAMPLE BUSINESS OBJECT	4

2.1	Staging Table Structure and Business Rules	5

2.1.1	Schema Changes	5

2.1.2	Coliform Sample Staging Table Structure and Business Rules	6

2.1.3	Lead and Copper Sample Staging Table Structure and Business Rules	21

2.1.4	General Sample Staging Table Structure and Business Rules	35

2.2	Enumerated Values	50

2.2.1	Sample Enumerated Values	50

2.2.2	Other Sample Measures Enumerated Values	53

2.3	Mapping from XML Schema Tag to SDWIS/STATE Entities and Attributes	54

2.3.1	Sample Mapping from XML Schema Tag to SDWIS/STATE Entities and Attributes	54

2.4	Definitions	71

2.4.1	Sample Definitions	71

2.4.2	Other Sample Measures Definitions	73

3.0	LABORATORY RESULT BUSINESS OBJECT	74

3.1	Staging Table Structure and Business Rules	75

3.1.1	Schema Changes	75

3.1.2	Microbiological Results Staging Table Structure and Business Rules	75

3.1.3	General Results Staging Table Structure and Business Rules	96

3.1.4	Radiological Results Staging Table Structure and Business Rules	117

3.1.5	Other Analysis Result Measures Staging Table Structure and Business   Rules	138

3.2	Enumerated Values	140

3.2.1	Sample Result Enumerated Values	140

3.2.2	Other Analysis Result Measures Enumerated Values	146

3.3	Sample Results Mapping from XML Schema Tag to SDWIS/STATE Entities and Attributes	147

4.0	SCHEMA MAPPING FOR SAMPLE SUMMARY REPORTING	174

4.1	Sample Summary Staging Table Structure and Business Rules	175

4.1.1	Schema Changes	175

None	175

4.1.2	Structure Set	175

4.2	Sample Summary Enumerated Values	178

4.3	Sample Summary Mapping from XML Schema Tag to SDWIS/STATE Entities and Attributes	179

4.4	Sample Summary Definitions	181

5.0	SCHEMA MAPPING FOR SAMPLE SUMMARY RESULT BUSINESS OBJECT	182

5.1	Sample Summary Result Staging Table Structure and Business Rules	183

5.1.1	Schema Changes	183

5.1.2	Structure Set	183

5.2	Sample Summary Result Enumerated Values	186

5.3	Summary Result Mapping from XML Schema Tag to SDWIS/STATE Entities and Attributes	188

5.4	Sample Summary Result Definitions	191

6.0	SCHEMA MAPPING FOR MDBP_SUMMARY BUSINESS OBJECT	192

6.1	Schema Changes	193

6.2	MDBP: Generic or User-Defined Summaries (OTHR or State-Owned) Staging Table Structure and Business Rules	194

6.3	MDBP: Combined Filter Turbidity Summaries (AVGT, MAXT, 95PT) Staging Table Structure and Business Rules	200

6.4	MDBP: Individual Filter Turbidity (IFT) Staging Table Structure and Business	208

6.5	MDBP: Minimal Residual Disinfection Summaries (EPRD, SERD, DSRD, SDRD) Staging Table Structure and  Business	213

6.6	MDBP:  Chlorine Dioxide/Chlorite Entry Point Summaries (CLO2, CLO3) Staging Table Structure and Business	219

6.7	MDBP: Chlorine/Chloramine MRDL Summaries (MRDL) Staging Table Structure and Business Rules	225

6.8	MDBP_Summary Enumerated Values	231

6.9	MDBP Summary Mapping from XML Schema Tag to SDWIS/STATE Entities and Attributes	236

6.10	MDBP Summary Definitions	240

# INTRODUCTION

The SDWIS/XML Sampling Schema Mapping document defines the primary four schemas used by SDWIS/XML Sampling to migrate sample and sample result reporting into SDWIS/STATE.  It defines the schema elements, the business rules imposed on the element values, and maps the schema tags to the SDWIS/STATE staging tables and SDWIS/STATE database tables.

## Overview of SDWIS/XML Sampling Schemas

Depending on the type of sampling data, SDWIS/XML Sampling will work with one of the following schemas to transfer data to SDWIS/STATE:

- SDWIS_eDWR_v3.0.xsd Schema

- SDWIS_Summary_v3.0.xsd Schema

- SDWIS_SummaryResult_v3.0.xsd Schema

- SDWIS_MDBPSummary_v3.0.xsd Schema

- Each schema may include several business objects, which are essentially sub-groupings of related data.

### SDWIS_eDWR_v3.0.xsd Schema

- The SDWIS_eDWR schema is the one that laboratories will use to report sample results for batch input into the SDWIS/STATE database.  SDWIS_eDWR_v3.0.xsd encompasses the Sample, Other Sample Measure, Sample Result and the Other Analysis Measure business objects for SDWIS/XML Sampling (Laboratory Sample Result Data). While the SDWIS_eDWR_v3.0 schema and the EDWR_eDWR_v2.0.xsd schema share the same hierarchy and element names, the SDWIS_eDWR schema does not contain all the elements that exist in the e-DWR schema.  The SDWIS_eDWR schema only contains those elements needed to move laboratory sample results into a SDWIS/STATE database.

- To give immediate feedback to the laboratory and to optimize data quality, the following constraints have been added to the SDWIS_eDWR schema elements:

- Data Type

- Length

- Permitted Values

- Optionality

- Data Range (Valid date range is between 1901-01-01 to 2100-01-01)

- Time Range (Valid time range is between 00:00:00 to 23:59:59)

### SDWIS_Summary_v3.0.xsd Schema

- The SDWIS_Summary_v3.0.xsd schema defines the data elements needed to migrate a Coliform or Lead and Copper Sample Summary into a SDWIS/STATE database.  This schema is not a part of the e-DWR schemas but follows the data standards and recommendations of the Data Exchange Registry. The schema elements, type, optionality, and enumerated values are determined by the SDWIS/STATE Summary structure.  Data quality constraints, like those listed in section 1.1.1, have been added to this schema.

### SDWIS_SummaryResult_v3.0.xsd Schema

- The SDWIS_SummaryResult_v3.0.xsd schema defines the data elements needed to migrate a Coliform or Lead and Copper Sample Summary Result into a SDWIS/STATE database.  This schema is not a part of the e-DWR schemas but follows the data standards and recommendations of the Data Exchange Registry. The schema elements, type, optionality, and enumerated values are determined by the SDWIS/STATE Summary Result structure.  Data quality constraints, like those listed in section 1.1.1, have been added to this schema.

### SDWIS_MDBPSummary_v3.0.xsd Schema

- The SDWIS_MDBPSummary_v3.0.xsd schema defines the data elements needed to migrate an MDBP Summary Result into a SDWIS/STATE database.  (This is, for example, the schema used to migrate Water Treatment Rule Summaries into SDWIS/STATE.)  This schema is not a part of the e-DWR schemas but follows the data standards and recommendations of the Data Exchange Registry. The schema elements, type, optionality, and enumerated values are determined by the SDWIS/STATE MDBP Summary structure.  Data quality constraints, like those listed in section 1.1.1, have been added to this schema.

## Scope and Structure of Document and Accompanying Schema

- This document describes the schema and target SDWIS/STATE tables that are used to allow a laboratory to report sample results are migrated into a SDWIS/STATE database.  It includes information about the three SDWIS summary schemas and a section that groups together a schema’s elements into business objects, presented from the perspective of the XML schema.

### Staging Table Structure and Business Rules

- The first section describes the structure of the SDWIS/STATE table that stages the sample data.  Since the business rules of the several different types of samples and sample results are different, there are several tables for samples and sample results.

### Enumerated Values

- This section lists the enumerated or “permitted” values for each element.  The enumerated values for an element are defined as either “Fixed” or “User Defined”.  Fixed values cannot be modified by a user and are defined within the XML schema definition.  User Defined values are maintained by the SDWIS System Administrator using the SDWIS/STATE System Administration component, and are stored in SDWIS/STATE’s  TINPVALS table.

### Mapping from XML Schema Tag to SDWIS/STATE Entities and   Attributes

- This section presents the mapping from the SDWIS/ XML Sampling Schemas to the SDWIS/STATE staging table and on through to the SDWIS/STATE database table.  When viewing the SDWIS/ XML Sampling Schemas, use this section to map between the schema and the appropriate SDWIS/STATE tables.

### Definitions

- This section provides definitions for each element.

# LABORATORY SAMPLE BUSINESS OBJECT

- Schema mapping for Sample is divided into four main sections. The first section describes the schema tag hierarchy, staging table/columns, their length, optionality, and the business rules associated with them. The second describes the permitted values for some of the schema elements. The third section describes the mapping between the staging table/columns, SDWIS/STATE table/columns, and the schema elements in the SDWIS_eDWR schema. The fourth section defines the staging table/columns.

- The Other Sample Measures is a supplement to the Sample data. They are grouped together as a single unit in the SDWIS_eDWR schema.  They always require a sample report and cannot be submitted without one.

- Note that edit rules listed in the Optionality column and in the Permitted Value List (where certain values are only accepted as stated) apply samples where B_Compliance_Indicator is value as for compliance (“Y”.)  Setting this indicator to “N” bypasses most of the edit checks imposed on “for compliance” samples except those noted in B_Compliance_Indicator as well as the permitted value edit checks for fields that have permitted values (that simply ensure that the permitted value supplied is one of the valid values for that field).

- Laboratory Sample Result Schema SDWIS_eDWR_v3.0.xsd maps to the Sample and the Other Sample business unit.

- Note: The MeasurementValue element in the Measurement group element is a number field.  If MeasurementSignificantDigits is not valued, the software will disregard trailing zeros and not preserve the trailing zeroes.  If there is a value in the MeasurementSignificantDigits, the trailing zeroes will be preserved.

- Example:

- MeasurementValue equals 1.2 and MeasurementSignificantDigits equals 2, number will be 1.2 and text will be 1.20.

- MeasurementValue equals 1.023 and MeasurementSignificantDigits equals 2, number will be 1.023 and text will be 1.02.

## Staging Table Structure and Business Rules

### Schema Changes

None this release.

### Coliform Sample Staging Table Structure and Business Rules

- This section lists the schema tag hierarchy, staging table/column name and its domain, size, optionality, and the business rules associated with Coliform Samples.

| COLIFORM SAMPLE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| LINE NO (field) | SCHEMA TAG NAME | STAGING TABLE (B_SAMPLE ) COLUMN NAME | DOMAIN | SIZE | OPTIONALITY |
| 1. | N/A | TMGJOB_IS_NUMBER | N | 10 | MANDATORY. Populated by the XML Sampling Application. |
| 2. | N/A | B_TMGSAMPL_IS_NUMBER | N | 7 | MANDATORY. Populated by the XML Sampling Application. |
| 3. | N/A | B_STATUS | AN | 1 | OPTIONAL. Populated by the XML Sampling Application. |
| 4. | eDWR Submission LabReport Sample SampleIdentification LabSampleIdentifier | B_LAB_SAMPLE_NUMBER | AN | 20 | MANDATORY |
| 5. | eDWR Submission LabReport Sample SampleIdentification StateSampleIdentifier | B_STATE_SAMPLE_NUMBER | AN | 20 | OPTIONAL |
| 6. | eDWR Submission LabReport Sample SampleIdentification PWSIdentifier | B_PWS_NUMBER | AN | 9 | MANDATORY |
| 7. | eDWR Submission LabReport Sample SampleIdentification AdditionalSampleIndicator | B_REPLACEMENT_INDICATOR | AN | 1 | OPTIONAL |
| 8. | eDWR Submission LabReport LabIdentification LabAccreditation LabAccreditationIdentifier | B_LAB_ID_NUMBER | AN | 10 | MANDATORY |
| 9. | eDWR Submission LabReport LabIdentification LabAccreditationAuthorityName | B_CERT_AGENCY | AN | 10 | MANDATORY |
| 10. | eDWR Submission LabReport Sample SampleIdentification PWSFacilityIdentifier | B_WSF_STATE_ASGN_ID | AN | 12 | MANDATORY - Must belong to the Water System supplied in field 6 (B_PWS_NUMBER). |
| 11. | eDWR Submission LabReport Sample SampleLocationIdentification SampleLocationIdentifier | B_SAMPLING_POINT | AN | 12 | CONDITIONALLY MANDATORY - Must be valued if field 12 (B_SAMPLING_LOCATION) is not valued. |
| 12. | eDWR Submission LabReport Sample SampleLocationIdentification SampleLocationName | B_SAMPLING_LOCATION | AN | 40 | CONDITIONALLY MANDATORY - Must be valued if field 11 (B_SAMPLING_POINT) is not valued. |
| 12a | eDWR Submission LabReport Sample SampleLocationIdentification SampleLocationCollectionAddress | B_COLLECTION_ADDRS | AN | 200 | OPTIONAL |
| 13.* | eDWR Submission LabReport Sample SampleIdentification SampleRuleCode | B_SAMPLE_CATEGORY | AN | 2 | MANDATORY - Must equal “TC” |
| 14.* | eDWR Submission LabReport Sample SampleIdentification ComplianceSampleIndicator | B_COMPLIANCE_INDICATOR | AN | 1 | MANDATORY |
| 15. | eDWR Submission LabReport Sample SampleIdentification SampleCollectionEndDate | B_COLLECTION_DATE | DT | 8 | MANDATORY - Value must be less than or equal to the current date. |
| 16. | eDWR Submission LabReport Sample SampleIdentification SampleCollectionEndTime | B_COLLECTION_TIME | TM | 6 | OPTIONAL |
| 17.* | eDWR Submission LabReport Sample SampleIdentification SampleMonitoringTypeCode | B_SAMPLE_TYPE | AN | 2 | MANDATORY |
| 18.* | eDWR Submission LabReport Sample SampleLocationIdentification SampleRepeatLocationCode | B_REPEAT_LOCATION | AN | 2 | OPTIONAL |
| 19. | eDWR Submission LabReport Sample SampleIdentification SampleLaboratoryReceiptDate | B_LAB_RECEIPT_DATE | DT | 8 | OPTIONAL |
| 20. | eDWR Submission LabReport Sample SampleIdentification SampleCollector IndividualFullName | B_COLLECTOR_NAME | AN | 40 | OPTIONAL - If supplied, format must be: LASTNAME, FIRSTNAME (There is a single space after the comma) if collector is an Individual (Legal Entity). Otherwise, value is stored with Sample as Informal Collector. |
| 21.* | eDWR Submission LabReport Sample SampleIdentification SampleVolume MeasurementValue eDWR Submission LabReport Sample SampleIdentification SampleVolume MeasurementUnit eDWR Submission LabReport Sample SampleIdentification SampleVolume MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) | B_SAMPLE_VOLUME | AN | 9 | N/A |
| 22.* | eDWR Submission LabReport Sample SampleIdentification SampleCollectionTypeCode | B_LEAD_COPPER_SAMPLE_TYPE | AN | 3 | NOT USED (Blank Fill) |
| 23.* | eDWR Submission LabReport Sample SampleIdentification SampleRejectionReasonCode | B_SAMPLE_REJECTION_REASON | AN | 2 | OPTIONAL - If this field is valued (e.g., for a rejected sample), the sample may be inserted without the sample result. This is the only condition where a sample may be inserted without a result. |
| 24. | eDWR Submission LabReport Sample SampleIdentification SampleCollectionMethod MethodIdentifier | B_COLLECTION_METHOD_CODE | AN | 12 | OPTIONAL |
| 25. | eDWR Submission LabReport Sample SampleIdentification OriginalSampleIdentification OriginalSampleIdentifier | B_ORIGINAL_LAB_SAMPLE_NUMBER | AN | 20 | CONDITIONALLY MANDATORY - This field must be valued if field 17 (B_SAMPLE_TYPE) = “RP.” |
| 26. | eDWR Submission LabReport Sample SampleIdentification OriginalSampleIdentification OriginalSampleCollectionDate | B_ORIGINAL_COLLECTION_DATE | DT | 8 | CONDITIONALLY MANDATORY - This field must be valued if field 17 (B_SAMPLE_TYPE) = “RP.” |
| 27. | eDWR Submission LabReport Sample SampleIdentification LabSampleCompositeNumber | B_LAB_COMPOSITE_NUMBER | AN | 20 | N/A |
| 28. | eDWR Submission LabReport Sample SampleIdentification LabSampleCompositeDate | B_COMPOSITE_DATE | DT | 8 | N/A |
| 29. | eDWR Submission LabReport SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “FreeChlorineResidual”) | B_FREE_CHLORINE_RESIDUAL | N | 5(3) | OPTIONAL - Note that if valued with spaces, the software will populate field TSAMCSMP.FF_CHLOR_RES_MSR with null. |
| 30. | eDWR Submission LabReport SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “TotalChlorineResidual”) | B_TOTAL_CHLORINE_RESIDUAL | N | 5(3) | OPTIONAL - Note that if valued with spaces, the software will populate field TSAMCSMP.FLDTOT_CHL_RES_MSR with null. |
| 31.. | eDWR Submission LabReport SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “WaterTemperature”) | B_SAMPLE_WATER_TEMPERATURE | N | 3(1) | OPTIONAL - Note that if valued with spaces, the software will populate field TSAMCSMP.FIELD_TEMP_MSR with null. |
| 32.* | eDWR Submission LabReport SampleIdentification SpecializedMeasurement MeasurementUnit eDWR Submission LabReport Sample SampleIdentification SampleVolume MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “WaterTemperature”) | B_TEMPERATURE_UNIT_MEASURE | AN | 1 | CONDITIONALLY MANDATORY - This field must be valued if field 31 (B_SAMPLE_WATER_TEMPERATURE) is valued. |
| 33. | eDWR Submission LabReport SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “Turbidity”) | B_TURBIDITY_MEASURE | N | 5(2) | OPTIONAL - Note that if valued with spaces, the software will populate field TSAMCSMP.FIELD_TURBID_MSR with null. |
| 34. | eDWR Submission LabReport SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “PH”) | B_PH_MEASURE | N | 4(1) | OPTIONAL - Note that if valued with spaces, the software will populate field TSAMCSMP.FIELD_PH_MEASURE with null. |
| 35 | eDWR Submission LabReport SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “FlowRate”) | B_FLOW_RATE | N | 7(4) | OPTIONAL - Note that if valued with spaces, the software will populate field TSAMCSMP.FIELD_FLOW_RATE with null. |
| 36. | eDWR Submission LabReport SampleIdentification SamplePurpose | B_SAMPLE_PURPOSE | AN | 2 | OPTIONAL |
| 37. | eDWR Submission LabReport SampleIdentification StateClassificationCode | B_STATE_CLASSIFICATION_CODE | AN | 6 | OPTIONAL |
| 38. | eDWR Submission LabReport SampleIdentification SampleComments Comments | B_SAMPLE_COMMENTS | ANmc | 2000 | OPTIONAL |
| 39. | eDWR Submission LabReport Sample SampleIdentification OriginalSampleIdentification OriginalSampleLabAccreditation LabAccreditationIdentifier | B_ORIG_ LAB_ID_NUM | AN | 10 | CONDITIONALLY MANDATORY - This field must be valued if field 17 (B_SAMPLE_TYPE) = “RP” Or “TG”. |
| 40. | eDWR Submission LabReport Sample SampleIdentification OriginalSampleIdentification OriginalSampleLabAccreditation LabAccreditationAuthorityName | B_ORIG_LAB_CERT_AGE | AN | 10 | CONDITIONALLY MANDATORY - This field must be valued if field 17 (B_SAMPLE_TYPE) = “RP” Or “TG” and field 39 is valued |
| 41. | eDWR Submission LabReport Sample SampleIdentification RecordID | B_XMLDOC_RECORD_ID | N | 7 | OPTIONAL |

### Lead and Copper Sample Staging Table Structure and Business Rules

- This section lists the schema tag hierarchy, staging table/column name and its domain, size, optionality, and the business rules associated with Lead and Copper Samples.

| LEAD AND COPPER SAMPLE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| LINE NO. (field) | SCHEMA TAG NAME | STAGING TABLE ( B_SAMPLE ) COLUMN NAME | DOMAIN | SIZE | OPTIONALITY |
| 1. | N/A | TMGJOB_IS_NUMBER | N | 10 | MANDATORY. Populated by the XML Sampling Application. |
| 2. | N/A | B_TMGSAMPL_IS_NUMBER | N | 7 | MANDATORY. Populated by the XML Sampling Application. |
| 3. | N/A | B_STATUS | AN | 1 | OPTIONAL. Populated by the XML Sampling Application. |
| 4. | eDWR Submission LabReport Sample SampleIdentification LabSampleIdentifier | B_LAB_SAMPLE_NUMBER | AN | 20 | MANDATORY |
| 5. | eDWR Submission LabReport Sample SampleIdentification StateSampleIdentifier | B_STATE_SAMPLE_NUMBER | AN | 20 | OPTIONAL |
| 6. | eDWR Submission LabReport Sample SampleIdentification PWSIdentifier | B_PWS_NUMBER | AN | 9 | MANDATORY |
| 7. * | eDWR Submission LabReport Sample SampleIdentification AdditionalSampleIndicator | B_REPLACEMENT_INDICATOR | AN | 1 | OPTIONAL |
| 8. | eDWR Submission LabReport LabIdentification LabAccreditation LabAccreditationIdentifier | B_LAB_ID_NUMBER | AN | 10 | MANDATORY |
| 9. | eDWR Submission LabReport LabIdentification LabAccreditation LabAccreditationAuthortity | B_CERT_AGENCY | AN | 10 | MANDATORY |
| 10. | eDWR Submission LabReport Sample SampleIdentification PWSFacilityIdentifier | B_WSF_STATE_ASGN_ID | AN | 12 | MANDATORY - Must belong to the Water System supplied in field 6 (B_PWS_NUMBER). |
| 11. | eDWR Submission LabReport Sample SampleLocationIdentification SampleLocationIdentifier | B_SAMPLING_POINT | AN | 12 | CONDITIONALLY MANDATORY - Must be valued if field 12 (B_SAMPLING_LOCATION) is not valued. |
| 12. | eDWR Submission LabReport Sample SampleLocationIdentification SampleLocationName | B_SAMPLING_LOCATION | AN | 40 | CONDITIONALLY MANDATORY - Must be valued if field 11 (B_SAMPLING_POINT) is not valued. |
| 12a | eDWR Submission LabReport Sample SampleLocationIdentification SampleLocationCollectionAddress | B_COLLECTION_ADDRS | AN | 200 | OPTIONAL |
| 13.* | eDWR Submission LabReport Sample SampleIdentification SampleRuleCode | B_SAMPLE_CATEGORY | AN | 2 | MANDATORY - Field must = “PB.” |
| 14.* | eDWR Submission LabReport Sample SampleIdentification ComplianceSampleIndicator | B_COMPLIANCE_INDICATOR | AN | 1 | MANDATORY |
| 15. | eDWR Submission LabReport Sample SampleIdentification SampleCollectionEndDate | B_COLLECTION_DATE | DT | 8 | MANDATORY |
| 16. | eDWR Submission LabReport Sample SampleIdentification SampleCollectionEndTime | B_COLLECTION_TIME | TM | 6 | OPTIONAL |
| 17.* | eDWR Submission LabReport Sample SampleIdentification SampleMonitoringTypeCode | B_SAMPLE_TYPE | AN | 2 | MANDATORY |
| 18.* | eDWR Submission LabReport Sample SampleLocationIdentification SampleRepeatLocationCode | B_REPEAT_LOCATION | AN | 2 | N/A |
| 19. | eDWR Submission LabReport Sample SampleIdentification SampleLaboratoryReceiptDate | B_LAB_RECEIPT_DATE | DT | 8 | OPTIONAL |
| 20. | eDWR Submission LabReport Sample SampleIdentification SampleCollector IndividualFullName | B_COLLECTOR_NAME | AN | 40 | OPTIONAL - If supplied, format must be: LASTNAME, FIRSTNAME (there is a single space after the comma) if collector is an Individual (Legal Entity). Otherwise, value is stored with Sample as Informal Collector. |
| 21.* | eDWR Submission LabReport Sample SampleIdentification SampleVolume MeasurementValue eDWR Submission LabReport Sample SampleIdentification SampleVolume MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) | B_SAMPLE_VOLUME | AN | 9 | OPTIONAL |
| 22.* | eDWR Submission LabReport Sample SampleIdentification SampleCollectionTypeCode | B_LEAD_COPPER_SAMPLE_TYPE | AN | 3 | OPTIONAL |
| 23.* | eDWR Submission LabReport Sample SampleIdentification SampleRejectionReasonCode | B_SAMPLE_REJECTION_REASON | AN | 2 | OPTIONAL - If this field is valued (e.g., for a rejected sample), the sample may be inserted without the sample result. This is the only condition where a sample may be inserted without a result. |
| 24. | eDWR Submission LabReport Sample SampleIdentification SampleCollectionMethod MethodIdentifier | B_COLLECTION_METHOD_CODE | AN | 12 | OPTIONAL |
| 25. | eDWR Submission LabReport Sample SampleIdentification OriginalSampleIdentification OriginalSampleIdentifier | B_ORIGINAL_LAB_SAMPLE_NUMBER | AN | 20 | CONDITIONALLY MANDATORY - This field must be valued if field 17 (B_SAMPLE_TYPE) = “RP.” |
| 26. | eDWR Submission LabReport Sample SampleIdentification OriginalSampleIdentification OriginalSampleCollectionDate | B_ORIGINAL_COLLECTION_DATE | DT | 8 | CONDITIONALLY MANDATORY - This field must be valued if field 17 (B_SAMPLE_TYPE) = “RP.” |
| 27. | eDWR Submission LabReport Sample SampleIdentification LabSampleCompositeNumber | B_LAB_COMPOSITE_NUMBER | AN | 20 | N/A |
| 28. | eDWR Submission LabReport Sample SampleIdentification LabSampleCompositeDate | B_COMPOSITE_DATE | DT | 8 | N/A |
| 29. | eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “FreeChlorineResidual”) | B_FREE_CHLORINE_RESIDUAL | N | 5(3) | OPTIONAL - Note that if valued with spaces, the software will populate field TSAMCSMP.FF_CHLOR_RES_MSR with null. |
| 30. | eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “TotalChlorineResidual”) | B_TOTAL_CHLORINE_RESIDUAL | N | 5 (3) | OPTIONAL - Note that if valued with spaces, the software will populate field TSAMCSMP.FLDTOT_CHL_RES_MSR with null. NOT USED (Blank Fill) |
| 31. | eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “WaterTemperature”) | B_SAMPLE_WATER_TEMPERATURE | N | 3(1) | OPTIONAL - Note that if valued with spaces, the software will populate field TSAMCSMP.FIELD_TEMP_MSR with null. |
| 32.* | eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementUnit eDWR Submission LabReport Sample SampleIdentification SampleVolume MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “WaterTemperature”) | B_TEMPERATURE_UNIT_MEASURE | AN | 1 | CONDITIONALLY MANDATORY - This field must be valued if field 31 (B_SAMPLE_WATER_TEMPERATURE) is valued. |
| 33. | eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “Turbidity”) | B_TURBIDITY_MEASURE | N | 5(2) | OPTIONAL - Note that if valued with spaces, the software will populate field TSAMCSMP.FIELD_TURBID_MSR with null. |
| 34. | eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “PH”) | B_PH_MEASURE | N | 4(1) | OPTIONAL - Note that if valued with spaces, the software will populate field TSAMCSMP.FIELD_PH_MEASURE with null. |
| 35. | eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “FlowRate”) | B_FLOW_RATE | N | 7(4) | OPTIONAL - Note that if valued with spaces, the software will populate field TSAMCSMP.FIELD_FLOW_RATE with null. |
| 36. | eDWR Submission LabReport Sample SampleIdentification SamplePurpose | B_SAMPLE_PURPOSE | AN | 2 | OPTIONAL |
| 37. | eDWR Submission LabReport Sample SampleIdentification StateClassificationCode | B_STATE_CLASSIFICATION_CODE | AN | 6 | OPTIONAL |
| 38. | eDWR Submission LabReport Sample SampleIdentification SampleComments Comments | B_SAMPLE_COMMENTS | ANmc | 2000 | OPTIONAL |
| 39. | eDWR Submission LabReport Sample SampleIdentification OriginalSampleIdentification OriginalSampleLabAccreditation LabAccreditationIdentifier | B_ORIG_LAB_ID_NUM | AN | 10 | CONDITIONALLY MANDATORY - This field must be valued if field 17 (B_SAMPLE_TYPE) = “RP.” |
| 40. | eDWR Submission LabReport Sample SampleIdentification OriginalSampleIdentification OriginalSampleLabAccreditation LabAccreditationAuthorityName | B_ORIG_LAB_CERT_AG | AN | 10 | CONDITIONALLY MANDATORY - This field must be valued if field 17 (B_SAMPLE_TYPE) = “RP.” and field 39 is valued |
| 41. | eDWR Submission LabReport Sample SampleIdentification RecordID | B_XMLDOC_RECORD_ID | N | 7 | Optional – A recommended record identifier unique to each xml document. |

### General Sample Staging Table Structure and Business Rules

- This section lists the schema tag hierarchy, staging table/column name and its domain, size, optionality, and the business rules associated with General Samples.

| GENERAL SAMPLE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| LINE NO (field) | SCHEMA TAG NAME | STAGING TABLE ( B_SAMPLE ) COLUMN NAME | DOMAIN | SIZE | OPTIONALITY |
| 1. | N/A | TMGJOB_IS_NUMBER | N | 10 | MANDATORY. Populated by the XML Sampling Application. |
| 2. | N/A | B_TMGSAMPL_IS_NUMBER | N | 7 | MANDATORY. Populated by the XML Sampling Application. |
| 3. | N/A | B_STATUS | AN | 1 | OPTIONAL. Populated by the XML Sampling Application. |
| 4. | eDWR Submission LabReport Sample SampleIdentification LabSampleIdentifier | B_LAB_SAMPLE_NUMBER | AN | 20 | MANDATORY |
| 5. | eDWR Submission LabReport Sample SampleIdentification StateSampleIdentifier | B_STATE_SAMPLE_NUMBER | AN | 20 | OPTIONAL |
| 6. | eDWR Submission LabReport Sample SampleIdentification PWSIdentifier | B_PWS_NUMBER | AN | 9 | MANDATORY |
| 7. * | eDWR Submission LabReport Sample SampleIdentification AdditionalSampleIndicator | B_REPLACEMENT_INDICATOR | AN | 1 | OPTIONAL |
| 8. | eDWR Submission LabReport LabIdentification LabAccreditation LabAccreditationIdentifier | B_LAB_IS_NUMBER | AN | 10 | MANDATORY |
| 9. | eDWR Submission LabReport LabIdentification LabAccreditation LabAccreditationAuthortity | B_CERT_AG | AN | 10 | MANDATORY |
| 10. | eDWR Submission LabReport Sample SampleIdentification PWSFacilityIdentifier | B_WSF_STATE_ASGN_ID | AN | 12 | MANDATORY - Must belong to the Water System supplied in field 6 (B_PWS_NUMBER). |
| 11. | eDWR Submission LabReport Sample SampleLocationIdentification SampleLocationIdentifier | B_SAMPLING_POINT | AN | 12 | CONDITIONALLY MANDATORY - Must be valued if field 12 (B_SAMPLING_LOCATION) is not valued. |
| 12. | eDWR Submission LabReport Sample SampleLocationIdentification SampleLocationName | B_SAMPLING_LOCATION | AN | 40 | CONDITIONALLY MANDATORY - Must be valued if field 11 (B_SAMPLING_POINT) is not valued. |
| 12a | eDWR Submission LabReport Sample SampleLocationIdentification SampleLocationCollectionAddress | B_COLLECTION_ADDRS | AN | 200 | OPTIONAL |
| 13.* | eDWR Submission LabReport Sample SampleIdentification SampleRuleCode | B_SAMPLE_CATEGORY | AN | 2 | MANDATORY - Must equal “GE.” |
| 14.* | eDWR Submission LabReport Sample SampleIdentification ComplianceSampleIndicator | B_COMPLIANCE_INDICATOR | AN | 1 | MANDATORY |
| 15. | eDWR Submission LabReport Sample SampleIdentification SampleCollectionEndDate | B_COLLECTION_DATE | DT | 8 | MANDATORY |
| 16. | eDWR Submission LabReport Sample SampleIdentification SampleCollectionEndTime | B_COLLECTION_TIME | TM | 6 | OPTIONAL |
| 17.* | eDWR Submission LabReport Sample SampleIdentification SampleMonitoringTypeCode | B_SAMPLE_TYPE | AN | 2 | MANDATORY |
| 18.* | eDWR Submission LabReport Sample SampleLocationIdentification SampleRepeatLocationCode | B_REPEAT_LOCATION | AN | 2 | N/A |
| 19. | eDWR Submission LabReport Sample SampleIdentification SampleLaboratoryReceiptDate | B_LAB_RECEIPT_DATE | DT | 8 | OPTIONAL |
| 20. | eDWR Submission LabReport Sample SampleIdentification SampleCollector IndividualFullName | B_COLLECTOR_NAME | AN | 40 | OPTIONAL - If supplied, format must be: LASTNAME, FIRSTNAME (there is a single space after the comma) if collector is an Individual (Legal Entity). Otherwise, value is stored with Sample as Informal Collector. |
| 21.* | eDWR Submission LabReport Sample SampleIdentification SampleVolume MeasurementValue eDWR Submission LabReport Sample SampleIdentification SampleVolume MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) | B_SAMPLE_VOLUME | AN | 9 | OPTIONAL |
| 22.* | eDWR Submission LabReport Sample SampleIdentification SampleCollectionTypeCode | B_LEAD_COPPER_SAMPLE_TYPE | AN | 3 | N/A |
| 23.* | eDWR Submission LabReport Sample SampleIdentification SampleRejectionReasonCode | B_SAMPLE_REJECTION_REASON | AN | 2 | OPTIONAL - If this field is valued (e.g., for a rejected sample), the sample may be inserted without the sample result. This is the only condition where a sample may be inserted without a result. |
| 24. | eDWR Submission LabReport Sample SampleIdentification SampleCollectionMethod MethodIdentifier | B_COLLECTION_METHOD_CODE | AN | 12 | OPTIONAL |
| 25. | eDWR Submission LabReport Sample SampleIdentification OriginalSampleIdentification OriginalSampleIdentifier | B_ORIGINAL_LAB_SAMPLE_NUMBER | AN | 20 | CONDITIONALLY MANDATORY - This field must be valued if field 17 (B_SAMPLE_TYPE) = “RP.” |
| 26. | eDWR Submission LabReport Sample SampleIdentification OriginalSampleIdentification OriginalSampleCollectionDate | B_ORIGINAL_COLLECTION_DATE | DT | 8 | CONDITIONALLY MANDATORY - This field must be valued if field 17 (B_SAMPLE_TYPE) = “RP.” |
| 27. | eDWR Submission LabReport Sample SampleIdentification LabSampleCompositeNumber | B_LAB_COMPOSITE_NUMBER | AN | 20 | N/A |
| 28. | eDWR Submission LabReport Sample SampleIdentification LabSampleCompositeDate | B_COMPOSITE_DATE | DT | 8 | N/A |
| 29. | eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “FreeChlorineResidual”) | B_FREE_CHLORINE_RESIDUAL | N | 5 (3) | OPTIONAL - Note that if valued with spaces, the software will populate field TSAMCSMP.FF_CHLOR_RES_MSR with null. |
| 30. | eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “TotalChlorineResidual”) | B_TOTAL_CHLORINE_RESIDUAL | N | 5 (3) | OPTIONAL - Note that if valued with spaces, the software will populate field TSAMCSMP.FLDTOT_CHL_RES_MSR with null. |
| 31. | eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “WaterTemperature”) | B_SAMPLE_WATER_TEMPERATURE | N | 3(1) | OPTIONAL - Note that if valued with spaces, the software will populate field TSAMCSMP.FIELD_TEMP_MSR with null. |
| 32.* | eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementUnit eDWR Submission LabReport Sample SampleIdentification SampleVolume MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “WaterTemperature”) | B_TEMPERATURE_UNIT_MEASURE | AN | 1 | CONDITIONALLY MANDATORY - This field must be valued if field 31 (B_SAMPLE_WATER_TEMPERATURE) is valued. |
| 33. | eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “Turbidity”) | B_TURBIDITY_MEASURE | N | 5(2) | OPTIONAL - Note that if valued with spaces, the software will populate field TSAMCSMP.FIELD_TURBID_MSR with null. |
| 34. | eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “PH”) | B_PH_MEASURE | N | 4(1) | OPTIONAL - Note that if valued with spaces, the software will populate field TSAMCSMP.FIELD_PH_MEASURE with null. |
| 35. | eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “FlowRate”) | B_FLOW_RATE | N | 7(4) | OPTIONAL - Note that if valued with spaces, the software will populate field TSAMCSMP.FIELD_FLOW_RATE with null. |
| 36. | eDWR Submission LabReport Sample SampleIdentification SamplePurpose | B_SAMPLE_PURPOSE | AN | 2 | OPTIONAL |
| 37. | eDWR Submission LabReport Sample SampleIdentification StateClassificationCode | B_STATE_CLASSIFICATION_CODE | AN | 6 | OPTIONAL |
| 38. | eDWR Submission LabReport Sample SampleIdentification SampleComments Comments | B_SAMPLE_COMMENTS | ANmc | 2000 | OPTIONAL |
| 39. | eDWR Submission LabReport Sample SampleIdentification OriginalSampleIdentification OriginalSampleLabAccreditation LabAccreditationIdentifier | B_ORIG_LAB_ID_NUM | AN | 10 | CONDITIONALLY MANDATORY - This field must be valued if field 17 (B_SAMPLE_TYPE) = “RP.” |
| 40. | eDWR Submission LabReport Sample SampleIdentification OriginalSampleIdentification OriginalSampleLabAccreditation LabAccreditationAuthorityName | B_ORIG_LAB_CERT_AG | AN | 10 | CONDITIONALLY MANDATORY - This field must be valued if field 17 (B_SAMPLE_TYPE) = “RP.” and field 39 is valued |
| 41. | eDWR Submission LabReport Sample SampleIdentification RecordID | B_XMLDOC_RECORD_ID | N | 7 | Optional – A recommended record identifier unique to each xml document. |

## Enumerated Values

### Sample Enumerated Values

- This section lists the staging table/column name and the permitted values associated with the columns.

| SAMPLE ENUMERATED VALUES |  |  |  |
| --- | --- | --- | --- |
| LINE NO (field) | STAGING TABLE ( B_SAMPLE ) COLUMN NAME | PERMITTED VALUES | PERMITTED VALUE TYPE |
| 7. | B_REPLACEMENT_INDICATOR | SPACE Y Yes N No | Fixed |
| 9. | B_CERT_AGENCY | User Defined | User Defined in TINPVALS where CODE_NAME = CERT_AG |
| 13. | B_SAMPLE_CATEGORY | SPACE GE General PB Lead and Copper TC Coliform | Fixed |
| 14. | B_COMPLIANCE_INDICATOR | Y Yes N No | Fixed |
| 17. | B_SAMPLE_TYPE for Coliform Sample (B_SAMPLE_CATEGORY = TC) | RT Routine RP Repeat SP Special BB Batch Blank FB Field Blank PE Performance Evaluation SB Shipping Blank ST Split Blank MR Maximum Residence Time MS Matrix Spike TG Triggered | Fixed |
| 17. | B_SAMPLE_TYPE for General and Lead & Copper Samples (B_SAMPLE_CATEGORY = GE or PB) | RT Routine RP Repeat CO Confirmation SP Special BB Batch Blank FB Field Blank PE Performance Evaluation SB Shipping Blank ST Split Blank MR Max Residence Time MS Matrix Spike | Fixed |
| 18. | B_REPEAT_LOCATION_CODE | SPACE DN Downstream within 5 connections of original sample location NF Near First Service Connection OR Original Site OT Other UP Upstream within 5 connections of original sample location | Fixed |
| 21. | B_SAMPLE_VOLUME | SPACE 1 100 300 400 500 (All in l/ml) | Fixed |
| 22. | B_LEAD_COPPER_SAMPLE_TYPE | SPACE ATS At Source FLS Flushed FSD First Draw LSL Lead Service Line | Fixed |
| 23. | B_SAMPLE_REJECTION_REASON | User Defined | 2.2 Change User Defined in TINPVALS where CODE_NAME = SAMPREJECT |
| 32. | B_TEMPERATURE_UNIT_MEASURE | C Celsius (Centigrade) F Fahrenheit | Fixed |
| 36. | B_SAMPLE_PURPOSE | User Defined | User Defined in TINPVALS where CODE_NAME = SMPLPURPOS |
| 37. | B_STATE_CLASSIFICATION_CODE | User Defined | User Defined in TINPVALS where CODE_NAME = SMPSTCLASS |
| 40 | B_ORIG_LAB_CERT_AG | User Defined | User Defined in TINPVALS where CODE_NAME = CERT_AG |

### Other Sample Measures Enumerated Values

- This section lists the staging table/column name and the permitted values associated with the columns.

| OTHER SAMPLE MEASURE ENUMERATED VALUES |  |  |  |
| --- | --- | --- | --- |
| LINE NO (field) | STAGING TABLE ( TMGOSPM ) COLUMN NAME | PERMITTED VALUE | PERMITTED VALUE TYPE |
| 3.* | B_NAME | User Defined | The value should exist in the table TSASMMT and column NAME |
| 6.* | B_UOM_CODE | User Defined | The value should exist in the table TSASMMT and column |

## Mapping from XML Schema Tag to SDWIS/STATE Entities and Attributes

### Sample Mapping from XML Schema Tag to SDWIS/STATE Entities and Attributes

- This section has the mapping between the staging table/columns, SDWIS/STATE Table/Column in both the data model and the physical database and the Schema tags in the XML Sampling schemas for Sample business object.

| SAMPLE MAPPING FROM SCHEMA TAG TO SDWIS/STATE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| LINE NO. (field) | SCHEMA TAG NAME | Staging Table (TMGSAMPL) Column Name | SDWIS/STATE Table and Column Name | SDWIS/ STATE Entity Name | SDWIS/STATE ATTRIBUTE |
| 1. # | N/A | TMGJOB_IS_NUMBER | N/A | N/A | N/A |
| 2. # | N/A | B_TMGSAMPL_IS_NUMBER | N/A | N/A | N/A |
| 3. # | N/A | B_STATUS | N/A | N/A | N/A |
| 4. | eDWR Submission LabReport Sample SampleIdentification LabSampleIdentifier | B_LAB_SAMPLE_NUMBER | TSASAMPL.LAB_ASGND_ID_NUM | Sample | LAB_ASSIGNED_ID_NUMBER |
| 5. | eDWR Submission LabReport Sample SampleIdentification StateSampleIdentifier | B_STATE_SAMPLE_NUMBER | TSASAMPL.ST_ASGN_IDENT_NUM | Sample | STATE_ASGN_IDENTIFICATION_NUMBER |
| 6. | eDWR Submission LabReport Sample SampleIdentification PWSIdentifier | B_PWS_NUMBER | TINWSYS.NUMBER | Water System | NUMBER (Foreign Key to either SBS Sample or Sample Summary) |
| 7. | eDWR Submission LabReport Sample SampleIdentification AdditionalSampleIndicator | B_REPLACEMENT_INDICATOR | TSASAMPL.REPLACEMENT_IND_CD | Sample | REPLACEMENT_INDICATOR_CODE |
| 8. | eDWR Submission LabReport LabIdentification LabAccreditation LabAccreditationIdentifier | B_LAB_IS_NUMBER | TSALAB.LAB_ID_NUMBER | Laboratory | LAB_ID_NUMBER (Foreign Key to either SBS Sample or Sample Summary) |
| 9. | eDWR Submission LabReport LabIdentification LabAccreditation LabAccreditationAuthortity | B_CERT_AGENCY | TSALAB.CERT_AGENCY | Laboratory | CERT_AGENCY (Foreign Key to either SBS Sample or Sample Summary) |
| 10. | eDWR Submission LabReport Sample SampleIdentification PWSFacilityIdentifier | B_WSF_STATE_ASGN_ID | TINWSF.ST_ASGN_IDENT_CD | Water System Facility | STATE_ASGN_IDENTIFICATION_CODE (Foreign Key to either SBS Sample or Sample Summary) |
| 11. | eDWR Submission LabReport Sample SampleLocationIdentification SampleLocationIdentifier | B_SAMPLING_POINT | TSASMPPT.IDENTIFICATION_CD | Sampling Point | IDENTIFICATION_CODE (Foreign Key to either SBS Sample or Sample Summary) |
| 12. | eDWR Submission LabReport Sample SampleLocationIdentification SampleLocationName | B_SAMPLING_LOCATION | TSASMPPT.DESCRIPTION_TEXT | Sampling Point | DESCRIPTION_TEXT (Foreign Key to either SBS Sample or Sample Summary) |
| 12a | eDWR Submission LabReport Sample SampleLocationIdentification SampleLocationCollectionAddress | B_COLLECTION_ADDRS | TSASAMPL.COLLECTION_ADDRESS | Sample | Collection_Address |
| 13. | eDWR Submission LabReport Sample SampleIdentification SampleRuleCode | B_SAMPLE_CATEGORY | TSASAMPL.D_CATEGORY_FLOW_CD | Sample | D_CATEGORY_FLOW_CODE |
| 14. | eDWR Submission LabReport Sample SampleIdentification ComplianceSampleIndicator | B_COMPLIANCE_INDICATOR | TSASAMPL.COMPL_PURP_IND_CD | Sample | COMPLIANCE_PURPOSE_INDICATOR_CODE |
| 15. | eDWR Submission LabReport Sample SampleIdentification SampleCollectionEndDate | B_COLLECTION_DATE | TSASAMPL.COLLLECTION_END_DT | Sample | COLLECTION_END_DATE |
| 16. | eDWR Submission LabReport Sample SampleIdentification SampleCollectionEndTime | B_COLLECTION_TIME | TSASAMPL.COLLCTN_END_TIME | Sample | COLLECTION_END_TIME |
| 17. | eDWR Submission LabReport Sample SampleIdentification SampleMonitoringTypeCode | B_SAMPLE_TYPE | TSASAMPL.TYPE_CODE | Sample | TYPE_CODE |
| 18. | eDWR Submission LabReport Sample SampleLocationIdentification SampleRepeatLocationCode | B_REPEAT_LOCATION | TSASAMPL.REPEAT_LOC_TYP_CD | Sample | REPEAT_LOCATION_TYPE_CODE |
| 19. | eDWR Submission LabReport Sample SampleIdentification SampleLaboratoryReceiptDate | B_LAB_RECEIPT_DATE | TSASAMPL.LAB_RECEIVED_DATE | Sample | LABORATORY_RECEIVED_DATE |
| 20. | eDWR Submission LabReport Sample SampleIdentification SampleCollector IndividualFullName | B_COLLECTOR_NAME | TININDIV.NAME TINLGENT.NAME or TSASAMPL.INFORMAL_COLLECTOR | Individual Legal Entity or Sample | NAME (Foreign Key to SBS Sample) (Where INDIVIDUAL_ Collector_Indicator_Code = ‘Y’) or INFORMAL_COLLECTOR |
| 21. | eDWR Submission LabReport Sample SampleIdentification SampleVolume MeasurementValue eDWR Submission LabReport Sample SampleIdentification SampleVolume MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) | B_SAMPLE_VOLUME | TSASAMPL.MICROBE_UOM_CODE | Sample | MICROBE_UOM_CODE |
| 22. | eDWR Submission LabReport Sample SampleIdentification SampleCollectionTypeCode | B_LEAD_COPPER_SAMPLE_TYPE | TSASAMPL.LD_COP_SAMP_TYP_CD | Sample | LEAD_AND_COPPER_SAMPLE_TYPE_CODE |
| 23. | eDWR Submission LabReport Sample SampleIdentification SampleRejectionReasonCode | B_SAMPLE_REJECTION_REASON | TSASAMPL.REJECT_REASON_CD | Sample | REJECTION_REASON_CODE |
| 24. | eDWR Submission LabReport Sample SampleIdentification SampleCollectionMethod MethodIdentifier | B_COLLECTION_METHOD_CODE | TSASMN.CODE | Standard Method Number | CODE (Foreign Key to SBS Sample) |
| 25. | eDWR Submission LabReport Sample SampleIdentification OriginalSampleIdentification OriginalSampleIdentifier | B_ORIGINAL_LAB_SAMPLE_NUMBER | TSASAMPL.LAB_ASGND_ID_NUM | Sample | LAB_ASSIGNED_ID_NUMBER (Foreign Key to SBS Sample) |
| 26. | eDWR Submission LabReport Sample SampleIdentification OriginalSampleIdentification OriginalSampleCollectionDate | B_ORIGINAL_COLLECTION_DATE | TSASAMLP.COLLCTN_END_DT | Sample | COLLECTION_END_DATE(Foreign Key to SBS Sample) |
| 27. | eDWR Submission LabReport Sample SampleIdentification LabSampleCompositeNumber | B_LAB_COMPOSITE_NUMBER | TSASAMPL.LAB_ASGND_ID_NUM | Sample | LAB_ASSIGNED_ID_NUMBER (Foreign Key to SBS Sample) |
| 28. | eDWR Submission LabReport Sample SampleIdentification LabSampleCompositeDate | B_COMPOSITE_DATE | TSASAMPL.COMPOSITE_DATE | Sample | COMPOSITE_DATE |
| 29. | eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “FreeChlorineResidual”) | B_FREE_CHLORINE_RESIDUAL | TSAMCSMP.FF_CHLOR_RES_MSR | Field Results | FIELD_FREE_CHLORINE_RESIDUAL_MSR |
| 30. | eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “TotalChlorineResidual”) | B_TOTL_CHLORINE_RESIDUAL | TSAMCSMP.FLDTOT_CHL_RES_MSR | Field Results | FIELD_TOTAL_CHLORINE_RESIDUAL_MSR |
| 31. | eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “WaterTemperature”) | B_SAMPLE_WATER_TEMPERATURE | TSAMCSMP.FIELD_TEMP_MSR | Field Results | FIELD_TEMPERATURE_MEASURE |
| 32. | eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementUnit eDWR Submission LabReport Sample SampleIdentification SampleVolume MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “WaterTemperature”) | B_TEMPERATURE_UNIT_MEASURE | TSAMCSMP.TEMP_MEAS_TYPE_CD | Field Results | TEMP_MEASUREMENT_TYPE_CODE |
| 33. | eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “Turbidity”) | B_TURBIDITY_MEASURE | TSAMCSMP.FIELD_TURBID_MSR | Field Results | FIELD_TURBIDITY_MEASURE |
| 34. | eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “PH”) | B_PH_MEASURE | TSAMCSMP.FIELD_PH_MEASURE | Field Results | FIELD_PH_MEASURE |
| 35. | eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementValue eDWR Submission LabReport Sample SampleIdentification SpecializedMeasurement MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) (Note: SpecializedMeasurementTypeCode = “FlowRate”) | B_FLOW_RATE | TSAMCSMP.FIELD_FLOW_RATE | Field Results | FIELD_FLOW_RATE |
| 36. | eDWR Submission LabReport Sample SampleIdentification SamplePurpose | B_SAMPLE_PURPOSE | TSASAMPL.SAMPLE_PURPOSE | Sample | SAMPLE_PURPOSE |
| 37. | eDWR Submission LabReport Sample SampleIdentification StateClassificationCode | B_STATE_CLASSIFICATION_CODE | TSASAMPL.STATE_CLASS_CODE | Sample | STATE_CLASSIFICATION_CODE |
| 38. | eDWR Submission LabReport Sample SampleIdentification SampleComments Comments | B_SAMPLE_COMMENTS | TSASAMPL.COMMENT_TEXT | Sample | COMMENT_TEXT |
| 39. | eDWR Submission LabReport Sample SampleIdentification OriginalSampleIdentification OriginalSampleLabAccreditation LabAccreditationIdentifier | B_ORIG_LAB_IS_NUM | TSALAB.LAB_ID_NUMBER | Laboratory | LAB_ID_NUMBER |
| 40. | eDWR Submission LabReport Sample SampleIdentification OriginalSampleIdentification OriginalSampleLabAccreditation LabAccreditationAuthorityName | B_ORIG_CERT_AG | TSALAB.CERT_AGENCY | Laboratory | CERT_AGENCY |
| 41 | eDWR Submission LabReport Sample SampleIdentification RecordID | B_XMLDOC_RECORD_ID | N/A | N/A | N/A |

## Definitions

### Sample Definitions

- This section defines the staging table/ column for the Sample business object and also mentions any business rules associated with it

| SAMPLE DEFINITIONS |  |  |
| --- | --- | --- |
| LINE NO (field) | STAGING TABLE COLUMN NAME | INDIVIDUAL SAMPLE |
| 1. | TMGJOB_IS_NUMBER | The identification number of the job. |
| 2. | B_TMGSAMPL_IS_NUMBER | A system_generated value which uniquely identifies the entity occurrence. |
| 3. | B_STATUS | The current status of the record. <SPACES> = unprocessed, C = processed w/ changes, P = processed, R = rejected |
| 4. | B_LAB_SAMPLE_NUMBER | Number/alphanumeric that identifies each sample. The LAB_SAMPLE_NUMBER and the WATER_SYSTEM_NUMBER together serve as the reference for each sample result, keeping sample results with their parent samples. This field is mandatory. |
| 5. | B_STATE_SAMPLE_NUMBER | Additional number/alphanumeric to identify the sample. Both LAB_SAMPLE_NUMBER and STATE_SAMPLE_NUMBER may be used. |
| 6. | B_PWS_NUMBER | The Public Water System (PWS) identification number. This field is mandatory. |
| 7. | B_REPLACEMENT_INDICATOR | Indicates whether or not the routine, repeat, confirmation, etc. type of sample is a replacement. |
| 8. | B_LAB_ID_NUMBER | Laboratory Number normally assigned to a laboratory by the certifying agency.. |
| 9. | B_CERT_AGENCY | The certifying agency that assigned the lab id number. . |
| 10. | B_WSF_STATE_ASGN_ID | Number/alphanumeric that uniquely identifies a Water System Facility (e.g., Treatment Plant/Distribution System/Well) within a Water System. |
| 11. | B_SAMPLING_POINT | Number/alphanumeric that uniquely identifies a point within a Water System Facility from which the sample is drawn. (Associated with SAMPLING LOCATION.) |
| 12. | B_SAMPLING_LOCATION | Alphanumeric that typically identifies a Sampling Point as an address or equivalent text description. (Associated with SAMPLING POINT). |
| 12a | B_COLLECTION_ADDRS | A free-form text area where the physical address of the sample collection site can be recorded. |
| 13. | B_SAMPLE_CATEGORY | Identifies the sample category as either Coliform/Lead & Copper/General. |
| 14. | B_COMPLIANCE_INDICATOR | When set to “Y,” indicates that the sample has been taken for compliance. |
| 15. | B_COLLECTION_DATE | The date in which the sample was collected. |
| 16. | B_COLLECTION_TIME | The time at which the sample was collected. |
| 17. | B_SAMPLE_TYPE | Indicates whether the sample is taken for “Routine” purposes or is a “Repeat,” “Confirmation,” etc. Several types are available although not all may be used with samples taken for compliance. |
| 18. | B_REPEAT_LOCATION | Indicates the location relative to the original Sampling Point at which the repeat/invalid replacement/confirmation sample was taken (upstream/downstream/original location/etc.). |
| 19. | B_LAB_RECEIPT_DATE | Date at which the Laboratory received the sample; cannot be prior to Collection Date. |
| 20. | B_COLLECTOR_NAME | Name of the Individual who is the collector of the sample. |
| 21. | B_SAMPLE_VOLUME | Value to indicate the size of the volume of water collected for the sample. |
| 22. | B_LEAD_COPPER_SAMPLE_TYPE | Type of Lead & Copper sample (for purposes of Lead & Copper rule compliance). |
| 23. | B_SAMPLE_REJECTION_REASON | Set of possible reasons to reject a sample prior to its analysis at the lab. |
| 24. | B_COLLECTION_METHOD_CODE | Code that indicates the Method used to collect the sample. |
| 25. | B_ORIGINAL_LAB_SAMPLE_NUMBER | LAB_SAMPLE_NUMBER of the sample that was originally taken and whose result required the current Repeat/Invalid Replacement/Confirmation sample to be taken. |
| 26. | B_ORIGINAL_COLLECTION_DATE | The date the Original sample was collected. |
| 27. | B_LAB_COMPOSITE_NUMBER | Number/alphanumeric that ties a composite sample to one or more other composite samples. Each sample that is part of the Composite must carry the same LAB_COMPOSITE_NUMBER. |
| 28. | B_COMPOSITE_DATE | Date in which the Lab composited one or more individual samples. |
| 29. | B_FREE_CHLORINE_RESIDUAL | Free Chlorine “Field Result” value measured at the time/location of sample collection. Reported in MG/L. |
| 30. | B_TOTAL_CHLORINE_RESIDUAL | Total Chlorine “Field Result” value measured at the time/location of sample collection. Reported in MG/L. |
| 31. | B_SAMPLE_WATER_TEMPERATURE | Water Temperature “Field Result” value measured at the time/location of sample collection. |
| 32. | B_TEMPERATURE_UNIT_MEASURE | Temperature Unit of Measure - either C (Celsius) or F (Fahrenheit). |
| 33. | B_TURBIDITY_MEASURE | Turbidity “Field Result” value measured at the time/location of sample collection. Reported in NTU. |
| 34. | B_PH_MEASURE | pH “Field Result” value measured at the time/location of sample collection. Reported in pH Units. |
| 35. | B_FLOW_RATE | Flow Rate “Field Result” value measured at the time/location of sample collection. Reported in gal/min. |
| 36. | B_SAMPLE_PURPOSE | The reason why the sample was taken. |
| 37. | B_STATE_CLASSIFICATION_CODE | The classification of the sample. If left blank, it will default to the classification field of the analyte of the first sample analytical result added. |
| 38 | B_SAMPLE_COMMENTS | Provides a place to record free text information |
| 39 | B_ORIG_LAB_ID_NUM | The Lab ID that reported the original sample. |
| 40 | B_ORIG_LAB_CERT_AG | The certifying agency of the Lab ID that reported the original sample. |
| 41. | B_XMLDOC_RECORD_ID | A unique number within the xml document that identifies an individual record. |

### Other Sample Measures Definitions

This section defines the Staging Table/ column for the Other Sample Measure Business object and also mentions any business rules associated with it.

| OTHER SAMPLE MEASURE Definitions |  |  |
| --- | --- | --- |
| LINE NO (field) | STAGING TABLE ( TMGOSPM ) COLUMN NAME | INDIVIDUAL SAMPLE (T)/ SAMPLE SUMMARY (S) |
| 1. | B_TMGOSPM_IS_NUMBER | A system-generated value which uniquely identifies the entity occurrence. |
| 2. | B_STATUS | The current status of the record. <SPACES> = unprocessed, C = processed w/ changes, P = processed, R = rejected. |
| 3. | B_NAME | The name of the measure type. |
| 4. | B_VALUE_NUMBER | The numeric value of the other sample measure |
| 5. | B_VALUE_TEXT | The text value of the other sample measure. |
| 6. | B_UOM_CODE | The unit of measure abbreviation for the other sample measure. |
| 7 | B_XMLDOC_RECORD_ID | A unique number within the xml document that identifies an individual record. |

# LABORATORY RESULT BUSINESS OBJECT

Laboratory Sample Result Schema SDWIS_eDWR_v3.0.xsd maps to the Sample Result business unit.

The schema mapping for Sample Result has four sections.  The first sections describe the staging table/column, their length optionality, and the business rules for the three different types of results, namely Microbiological Result, General Results and the Radiological Results, and also the Other Analysis Result.  The second section describes the Enumerated values for the Results elements and the business rules. The third section describes the mapping between the staging table/columns, SDWIS/STATE table/columns for both the Sample Result and Other Analysis Result and the schema elements in the SDWIS_eDWR.xsd  schema.

The Other Analysis Measures is a supplement to the Analysis Result data. They are grouped together as single unit in the SDWIS_eDWR schema. They must always accompany the Analysis Result details and cannot be submitted without them.

Note: The MeasurementValue element in the Measurement group element is a number field.  If MeasurementSignificantDigits is not valued, the software will disregard trailing zeros and not preserve the trailing zeroes. But if there is a value in the MeasurementSignificantDigits, the trailing zeroes will be preserved.

Example: MeasurementValue equals 1.2 and MeasurementSignificantDigits equals 2, number will be 1.2 and text will be 1.20.

Data in line number 19, 20, 21, 22, 23, 24 is only for analyte of type code “MOR”.

Data in line number 25,26,27,28,29,30,31 is only allowed for analytes whose type code type is not equal to "MOR".

Data in line number 32 is only allowed for analytes whose type code type is not equal "RA".

## Staging Table Structure and Business Rules

### Schema Changes

None this release.

### Microbiological Results Staging Table Structure and Business Rules

This section lists the schema tag hierarchy, staging table/column name and its domain, size, optionality, and the business rules associated with Microbiological Results.

| MICROBIOLOGICAL SAMPLE RESULT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| LINE NO (field) | SCHEMA TAG NAME | STAGING TABLE ( B_SAMPLE_RESULT ) COLUMN NAME | DOMAIN | SIZE | OPTIONALITY |
| 1. | N/A | TMGJOB_IS_NUMBER | N | 10 | MANDATORY. Populated by the XML Sampling Application. |
| 2. | N/A | B_TMGSAR_IS_NUMBER | N | 7 | MANDATORY. Populated by the XML Sampling Application. |
| 3. | N/A | B_STATUS | AN | 1 | OPTIONAL. Populated by the XML Sampling Application. |
| 4. | eDWR Submission LabReport SampleAnalysisResults LabSampleIdentifier OR if used along with the Sample object eDWR Submission LabReport Sample SampleIdentification LabSampleIdentifier | B_LAB_SAMPLE_NUMBER | AN | 20 | MANDATORY - Reference field required to associate the result to the parent sample. Must be same value as field B_LAB_SAMPLE_NUMBER in B_Sample structure set or result (and sample) will be rejected. |
| 5. | eDWR Submission LabReport SampleAnalysisResults SampleCollectionEndDate OR if used along with the Sample object eDWR Submission LabReport Sample SampleIdentification SampleCollectionEndDate | B_COLLECTION_DATE | DT | 8 | OPTIONAL |
| 6. | eDWR Submission LabReport SampleAnalysisResults PWSIdentifier OR if used along with the Sample object eDWR Submission LabReport Sample SampleIdentification PWSIdentifier | B_PWS_NUMBER | AN | 9 | MANDATORY - Reference field required to associate the result to the parent sample. Must be same value as field B_PWS_NUMBER in B_Sample structure set or result (and sample) will be rejected. |
| 7. | eDWR Submission LabReport LabIdentification LabAccreditation | B_LAB_ID_NUMBER | AN | 10 | MANDATORY |
| 8. | eDWR Submission LabReport LabIdentification LabAccreditation LabAccreditationAuthortity | B_CERT_AGENCY | AN | 10 | MANDATORY |
| 9. | eDWR Submission LabReport SampleAnalysisResults AnalyteIdentification AnalyteCode OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalyteIdentification AnalyteCode | B_ANALYTE_CODE | AN | 4 | MANDATORY |
| 10. | eDWR Submission LabReport SampleAnalysisResults LabAnalysisIdentification AnalysisStartDate OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification AnalysisStartDate | B_ANALYSIS_START_DATE | DT | 8 | OPTIONAL - Sample will be rejected if Analysis Start Date is supplied but is prior to Sample Collection Date. |
| 11 | eDWR Submission LabReport SampleAnalysisResults LabAnalysisIdentification AnalysisStartTime OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification AnalysisStartTime | B_ANALYSIS_START_TIME | TM | 6 | OPTIONAL |
| 12. | eDWR Submission LabReport SampleAnalysisResults LabAnalysisIdentification AnalysisEndDate OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification AnalysisEndDate | B_ANALYSIS_COMPLETE_DATE | DT | 8 | OPTIONAL - Sample will be rejected if Analysis Completion Date is supplied but is prior to Sample Collection Date. Sample will also be rejected if both Analysis Start and Completion Dates are supplied, but Completion Date is prior to Start Date. |
| 13. | eDWR Submission LabReport SampleAnalysisResults LabAnalysisIdentification AnalysisEndTime OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification AnalysisEndTime | B_ANALYSIS_COMPLETE_TIME | TM | 6 | OPTIONAL |
| 14. | eDWR Submission LabReport SampleAnalysisResults AnalysisResult ResultStateNotificationDate OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult ResultStateNotificationDate | B_STATE_NOTIFY_DATE | DT | 8 | OPTIONAL |
| 15. | eDWR Submission LabReport SampleAnalysisResults AnalysisResult PWSNotificationDate OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult PWSNotificationDate | B_WATER_SYSTEM_NOTIFY_DATE | DT | 8 | OPTIONAL |
| 16. * | eDWR Submisssion LabReport SampleAnalysisResults QAQCSummary DataQualityCode OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation QAQCSummary DataQualityCode | B_DATA_QUALITY | AN | 1 | MANDATORY |
| 17. * | eDWR Submission LabReport SampleAnalysisResults QAQCSummary DataQualityRejectCode OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation QAQCSummary DataQualityRejectCode | B_DATA_QUALITY_REASON | AN | 2 | CONDITIONALLY MANDATORY - Field must be valued if field 16 (B_DATA_QUALITY) = “R.” |
| 18.* | eDWR Submission LabReport SampleAnalysisResults LabAnalysisIdentification SampleAnalyticalMethod: MethodIdentifier OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification SampleAnalyticalMethod: MethodIdentifier | B_ANALYSIS_METHOD_CODE | AN | 30 | OPTIONAL - May be either a federally owned or state-owned analyte method pairing that exists in state/region’s SDWIS/STATE database. |
| 19. | eDWR Submission LabReport SampleAnalysisResult LabAnalysisIdentification SampleAnalyzedMeasure MeasurementValue eDWR Submission LabReport SampleAnalysisResult LabAnalysisIdentification SampleAnalyzedMeasure MeasurementUnit eDWR Submission LabReport SampleAnalysisResults LabAnalysisIdentification SampleAnalyzedMeasure MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification SampleAnalyzedMeasure MeasurementValue eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification SampleAnalyzedMeasure MeasurementUnit eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification SampleAnalyzedMeasure MeasurementSignificantDigits | B_VOLUME_ASSAYED | AN | 9 | OPTIONAL |
| 20.* | eDWR Submission LabReport SampleAnalysisResults AnalysisResult SampleInterferenceReasonCode OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult SampleInterferenceReasonCode | B_LAB_REJECTION_REASON | AN | 4 | CONDITIONALLY MANDATORY - See discussion in field 21. |
| 21.* | eDWR Submission LabReport SampleAnalysisResult AnalysisResult Result MeasurementQualifier OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementQualifier | B_MICROBE_PRESENCE_INDICATOR | AN | 1 | CONDITIONALLY MANDATORY - - If not valued, field 20 (B_LAB_REJECTION_REASON) must be valued and field 17 (B_COUNT) must not be valued. -  If valued with P, field 22 (B_COUNT) must be either not valued, or if valued, be greater than 0. -  If valued with A, field 20 (B_LAB_REJECTION_REASON) should not be valued and field 22 (B_COUNT) should not be valued with 0 or any other integer. |
| - 22. | - eDWR - Submission - LabReport - SampleAnalysisResult AnalysisResult Result MeasurementValue - eDWR - Submission - LabReport - SampleAnalysisResult AnalysisResult - Result MeasurementSignificantDigits - (This schema element has to be valued for precision / significant digits) - OR - if used along with the Sample object - eDWR - Submission - LabReport - Sample - AnalysisResultInformation AnalysisResult Result MeasurementValue - eDWR - Submission - LabReport - Sample - AnalysisResultInformation - AnalysisResult - Result MeasurementSignificantDigits | - B_COUNT | - N | - 15,5 | - CONDITIONALLY MANDATORY - See discussion in field 21. |
| - 23.* | - eDWR - Submission - LabReport - SampleAnalysisResults AnalysisResult Result - MicrobialResultCountTypeCode - OR - if used along with the Sample object - eDWR - Submission - LabReport - Sample - AnalysisResultInformation AnalysisResult Result - MicrobialResultCountTypeCode | - B_COUNT_TYPE | - AN | - 10 | - CONDITIONALLY MANDATORY - Field must be valued if field 22 (B_COUNT) > 0. |
| - 24.* | - eDWR - Submission - LabReport - SampleAnalysisResult AnalysisResult Result - MeasurementUnit - OR - if used along with the Sample object - eDWR - Submission - LabReport - Sample - AnalysisResultInformation AnalysisResult Result - MeasurementUnit | - B_COUNT_UNITS | - AN | - 9 | - CONDITIONALLY MANDATORY - Field must be valued if field 23 (B_COUNT_TYPE) is valued. |
| - 25. * | - eDWR - Submission - LabReport - SampleAnalysisResult AnalysisResult Result MeasurementQualifier - OR - if used along with the Sample object - eDWR - Submission - LabReport - Sample - AnalysisResultInformation AnalysisResult Result MeasurementQualifier | - B_LESS_THAN_INDICATOR | - AN | - 1 | - N/A |
| - 26. * | - eDWR - Submission - LabReport - SampleAnalysisResult AnalysisResult - DetectionLimitTypeCode - OR - if used along with the Sample object - eDWR - Submission - LabReport - Sample - AnalysisResultInformation AnalysisResult DetectionLimitTypeCode | - B_LESS_THAN_CODE | - AN | - 3 | - N/A |
| - 27. | - eDWR - Submission - LabReport - SampleAnalysisResults AnalysisResult DetectionLimit MeasurementValue - eDWR - Submission - LabReport - SampleAnalysisResults AnalysisResult DetectionLimit MeasurementSignificantDigits - (This schema element has to be valued for precision / significant digits) - OR - if used along with the Sample object - eDWR - Submission - LabReport - Sample - AnalysisResult AnalysisResultInformation DetectionLimit MeasurementValue - eDWR - Submission - LabReport - Sample - AnalysisResult AnalysisResultInformation DetectionLimit MeasurementSignificantDigits - (This schema element has to be valued for precision / significant digits) | - B_DETECTION_LEVEL | - N | - 15(9) | - N/A |
| - 28. | - eDWR - Submission - LabReport - SampleAnalysisResults AnalysisResult DetectionLimit MeasurementUnit - OR - if used along with the Sample object - eDWR - Submission - LabReport - Sample - AnalysisResultInformation AnalysisResult DetectionLimit MeasurementUnit | - B_DETECTION_LEVEL_UNIT_CODE | - ANmc | - 9 | - N/A |
| - 29. | - eDWR - Submission - LabReport - SampleAnalysisResults AnalysisResult Result MeasurementValue - eDWR - Submission - LabReport - SampleAnalysisResults AnalysisResult Result MeasurementSignificantDigits - (This schema element has to be valued for precision / significant digits) - OR - if used along with the Sample object - eDWR - Submission - LabReport - Sample - AnalysisResultInformation AnalysisResult Result MeasurementValue - eDWR - Submission - LabReport - Sample - AnalysisResultInformation AnalysisResult Result MeasurementSignificantDigits - (This schema element has to be valued for precision / significant digits) | - B_CONCENTRATION | - N | - 15(9) | - N/A |
| - 30. | - eDWR - Submission - LabReport - SampleAnalysisResults AnalysisResult Result MeasurementUnit - OR - if used along with the Sample object - eDWR - Submission - LabReport - Sample - AnalysisResultInformation AnalysisResult Result MeasurementUnit | - B_CONCENTRATION_UNIT_CODE | - ANmc | - 9 | - N/A |
| - 31. | - eDWR - Submission - LabReport - SampleAnalysisResults AnalysisResult Result MeasurementValue - eDWR - Submission - LabReport - SampleAnalysisResults AnalysisResult Result MeasurementSignificantDigits - (This schema element has to be valued for precision / significant digits) - OR - if used along with the Sample object - eDWR - Submission - LabReport - Sample - AnalysisResultInformation AnalysisResult Result MeasurementValue - eDWR - Submission - LabReport - Sample - AnalysisResultInformation AnalysisResult Result MeasurementSignificantDigits - (This schema element has to be valued for precision / significant digits) | - B_REPORTED_MEASURE | - AN | - 10 | - N/A |
| - 32. | - eDWR - Submission - LabReport - SampleAnalysisResults AnalysisResult RadiologicalResultCountError - OR - if used along with the Sample object - eDWR - Submission - LabReport - Sample - AnalysisResultInformation AnalysisResult RadiologicalResultCountError | - B_REPORTED_MEASURE_COUNT_ERROR | - N | - 8(3) | - N/A |
| - 33. | - eDWR - Submission - LabReport - SampleAnalysisResult AnalysisResult AnalysisComment Comments - OR - if used along with the Sample object - eDWR - Submission - LabReport - Sample - AnalysisResultInformation AnalysisResult AnalysisComment Comments | - B_COMMENT | - ANmc | - 1000 | - Optional |
| - 34. | eDWR Submission LabReport SampleAnalysisResult AnalysisResult - RecordID | - B_XMLDOC_RECORD_ID | - N | - 7 | - Optional |

### General Results Staging Table Structure and Business Rules

This section lists the schema tag hierarchy, staging table/column name and its domain, size, optionality, and the business rules associated with General Results.

| GENERAL SAMPLE RESULT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| LINE NO (field) | SCHEMA TAG NAME | STAGING TABLE ( B_SAMPLE_RESULT ) COLUMN NAME | DOMAIN | SIZE | OPTIONALITY |
| 1. | N/A | TMGJOB_IS_NUMBER | N | 10 | MANDATORY. Populated by the XML Sampling Application. |
| 2. | N/A | B_TMGSAR_IS_NUMBER | N | 7 | MANDATORY. Populated by the XML Sampling Application. |
| 3. | N/A | B_STATUS | AN | 1 | OPTIONAL. Populated by the XML Sampling Application. |
| 4. | eDWR Submission LabReport SampleAnalysisResults LabSampleIdentifier OR if used along with the Sample object eDWR Submission LabReport Sample SampleIdentification LabSampleIdentifier | B_LAB_SAMPLE_NUMBER | AN | 20 | MANDATORY - Reference field required to associate the result to the parent sample. Must be same value as field B_LAB_SAMPLE_NUMBER in B_Sample structure set or result (and sample) will be rejected. |
| 5. | eDWR Submission LabReport SampleAnalysisResults SampleCollectionEndDate OR if used along with the Sample object eDWR Submission LabReport Sample SampleIdentification SampleCollectionEndDate | B_COLLECTION_DATE | DT | 8 | OPTIONAL |
| 6. | eDWR Submission LabReport SampleAnalysisResults PWSIdentifier OR if used along with the Sample object eDWR Submission LabReport Sample SampleIdentification PWSIdentifier | B_PWS_NUMBER | AN | 9 | MANDATORY - Reference field required to associate the result to the parent sample. Must be same value as field B_PWS_NUMBER in B_Sample structure set or result (and sample) will be rejected. |
| 7. | eDWR Submission LabReport LabIdentification LabAccreditation LabAccreditationIdentifier | B_LAB_ID_NUMBER | AN | 10 | MANDATORY |
| 8. | eDWR Submission LabReport LabIdentification LabAccreditation LabAccreditationAuthortity | B_CERT_AGENCY | AN | 10 | MANDATORY |
| 9. | eDWR Submission LabReport SampleAnalysisResults AnalyteIdentification AnalyteCode OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalyteIdentification AnalyteCode | B_ANALYTE_CODE | AN | 4 | MANDATORY - Reference field required. |
| 10. | eDWR Submission LabReport SampleAnalysisResults LabAnalysisIdentification AnalysisStartDate OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification AnalysisStartDate | B_ANALYSIS_START_DATE | DT | 8 | OPTIONAL - Sample will be rejected if Analysis Start Date is supplied but is prior to Sample Collection Date. |
| 11. | eDWR Submission LabReport SampleAnalysisResults LabAnalysisIdentification AnalysisStartTime OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification AnalysisStartTime | B_ANALYSIS_START_TIME | TM | 6 | OPTIONAL |
| 12. | eDWR Submission LabReport SampleAnalysisResults LabAnalysisIdentification AnalysisEndDate OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification AnalysisEndDate | B_ANALYSIS_COMPLETE_DATE | DT | 8 | OPTIONAL - Sample will be rejected if Analysis Completion Date is supplied but is prior to Sample Collection Date. Sample will also be rejected if both Analysis Start and Completion Dates are supplied, but Completion Date is prior to Start Date. OPTIONAL |
| 13. | eDWR Submission LabReport SampleAnalysisResults LabAnalysisIdentification AnalysisEndTime OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification AnalysisEndTime | B_ANALYSIS_COMPLETE_TIME | TM | 6 | OPTIONAL |
| 14. | eDWR Submission LabReport SampleAnalysisResults AnalysisResult ResultStateNotificationDate OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult ResultStateNotificationDate | B_STATE_NOTIFY_DATE | DT | 8 | OPTIONAL |
| 15. | eDWR Submission LabReport SampleAnalysisResults AnalysisResult PWSNotificationDate OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult PWSNotificationDate | B_WATER_SYSTEM_NOTIFY_DATE | DT | 8 | OPTIONAL |
| 16.* | eDWR Submisssion LabReport SampleAnalysisResults QAQCSummary DataQualityCode OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation QAQCSummary DataQualityCode | B_DATA_QUALITY | AN | 1 | MANDATORY |
| 17.* | eDWR Submission LabReport SampleAnalysisResults QAQCSummary DataQualityRejectCode OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation QAQCSummary DataQualityRejectCode | B_DATA_QUALITY_REASON | AN | 2 | CONDITIONALLY MANDATORY - Field must be valued if field 16 (B_DATA_QUALITY) = “R.” |
| 18.* | eDWR Submission LabReport SampleAnalysisResults LabAnalysisIdentification SampleAnalyticalMethod: MethodIdentifier OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification SampleAnalyticalMethod: MethodIdentifier | B_ANALYSIS_METHOD_CODE | AN | 30 | OPTIONAL - May be either a federally owned or state-owned analyte method pairing that exists in state/region’s SDWIS/STATE database. |
| 19* | eDWR Submission LabReport SampleAnalysisResult LabAnalysisIdentification SampleAnalyzedMeasure MeasurementValue eDWR Submission LabReport SampleAnalysisResults LabAnalysisIdentification SampleAnalyzedMeasure MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification SampleAnalyzedMeasure MeasurementValue eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification SampleAnalyzedMeasure MeasurementSignificantDigits | B_VOLUME_ASSAYED | AN | 9 | N/A |
| 20.* | eDWR Submission LabReport SampleAnalysisResults AnalysisResult SampleInterferenceReasonCode OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult SampleInterferenceReasonCode | B_LAB_REJECTION_REASON | AN | 4 | N/A |
| 21.* | eDWR Submission LabReport SampleAnalysisResult AnalysisResult Result MeasurementQualifier OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementQualifier | B_MICROBE_PRESENCE_INDICATOR | AN | 1 | N/A |
| 22. | eDWR Submission LabReport SampleAnalysisResult AnalysisResult Result MeasurementValue eDWR Submission LabReport SampleAnalysisResult AnalysisResult Result MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementValue eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementSignificantDigits | B_COUNT | N | 15,5 | N/A |
| 23.* | eDWR Submission LabReport SampleAnalysisResults AnalysisResult Result MeasurementUnit OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementUnit | B_COUNT_TYPE | AN | 10 | N/A |
| 24.* | eDWR Submission LabReport SampleAnalysisResult AnalysisResult Result MicrobialResultCountTypeCode OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MicrobialResultCountTypeCode | B_COUNT_UNITS | AN | 9 | N/A |
| 25.* | eDWR Submission LabReport SampleAnalysisResult AnalysisResult Result MeasurementQualifier OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementQualifier | B_LESS_THAN_INDICATOR | AN | 1 | CONDITIONALLY MANDATORY - Must be valued if field 16 (B_DATA_QUALITY) = “A” or “P” and field 29 (B_CONCENTRATION) is either not valued or is valued with 0. |
| 26.* | eDWR Submission LabReport SampleAnalysisResult AnalysisResult DetectionLimitTypeCode OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult DetectionLimitTypeCode | B_LESS_THAN_CODE | AN | 3 | CONDITIONALLY MANDATORY - Must be valued if field 25 (B_LESS_THAN_INDICATOR) = “Y.” |
| 27. | eDWR Submission LabReport SampleAnalysisResults AnalysisResult DetectionLimit MeasurementValue eDWR Submission LabReport SampleAnalysisResults AnalysisResult DetectionLimit MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResult AnalysisResultInformation DetectionLimit MeasurementValue eDWR Submission LabReport Sample AnalysisResult AnalysisResultInformation DetectionLimit MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) | B_DETECTION_LEVEL | N | 15(9) | CONDITIONALLY MANDATORY - Must be valued if field 26 (B_LESS_THAN_CODE) = “MRL.” |
| 28. | eDWR Submission LabReport SampleAnalysisResults AnalysisResult DetectionLimit MeasurementUnit OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult DetectionLimit MeasurementUnit | B_DETECTION_LEVEL_UNIT_CODE | ANmc | 9 | CONDITIONALLY MANDATORY - Must be valued if field 27 (B_DETECTION_LEVEL) is valued. |
| 29. | eDWR Submission LabReport SampleAnalysisResults AnalysisResult Result MeasurementValue eDWR Submission LabReport SampleAnalysisResults AnalysisResult Result MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementValue eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) | B_CONCENTRATION | N | 15(9) | CONDITIONALLY MANDATORY - Must be valued if field 16 (B_DATA_QUALITY) = (“A” or “P”) and field 25 (B_LESS_THAN_INDICATOR) is not valued. |
| 30. | eDWR Submission LabReport SampleAnalysisResults AnalysisResult Result MeasurementUnit OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementUnit | B_CONCENTRATION_UNIT_CODE | ANmc | 9 | CONDITIONALLY MANDATORY - Must be valued if field 29 (B_CONCENTRATION) is valued. |
| 31. | eDWR Submission LabReport SampleAnalysisResults AnalysisResult Result MeasurementValue eDWR Submission LabReport SampleAnalysisResults AnalysisResult Result MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementValue eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) | B_REPORTED_MEASURE | AN | 10 | N/A |
| 32. | eDWR Submission LabReport SampleAnalysisResults AnalysisResult RadiologicalResultCountError OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult RadiologicalResultCountError | B_REPORTED_MEASURE_COUNT_ERROR | N | 8(3) | N/A |
| 33. | eDWR Submission LabReport SampleAnalysisResult AnalysisResult AnalysisComment Comments OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult AnalysisComment Comments | B_COMMENT | ANmc |  | Optional |
| - 34. | eDWR Submission LabReport SampleAnalysisResult AnalysisResult - RecordID | - B_XMLDOC_RECORD_ID | - N | - 7 | - Optional |

### Radiological Results Staging Table Structure and Business Rules

This section lists the schema tag hierarchy, staging table/column name and its domain, size, optionality, and the business rules associated with Radiological Results.

| RADIOLOGICAL SAMPLE RESULT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| LINE NO (field) | SCHEMA TAG NAME | STAGING TABLE ( B_SAMPLE_RESULT ) COLUMN NAME | DOMAIN | SIZE | OPTIONALITY |
| 1. | N/A | TMGJOB_IS_NUMBER | N | 10 | MANDATORY. Populated by the XML Sampling Application. |
| 2. | N/A | B_TMGSAR_IS_NUMBER | N | 7 | MANDATORY. Populated by the XML Sampling Application. |
| 3. | N/A | B_STATUS | AN | 1 | OPTIONAL. Populated by the XML Sampling Application. |
| 4. | eDWR Submission LabReport SampleAnalysisResults LabSampleIdentifier OR if used along with the Sample object eDWR Submission LabReport Sample SampleIdentification LabSampleIdentifier | B_LAB_SAMPLE_NUMBER | AN | 20 | MANDATORY - Reference field required to associate the result to the parent sample. Must be same value as field B_LAB_SAMPLE_NUMBER in B_SAMPLE structure set or result (and sample) will be rejected. |
| 5. | eDWR Submission LabReport SampleAnalysisResults SampleCollectionEndDate OR if used along with the Sample object eDWR Submission LabReport Sample SampleIdentification SampleCollectionEndDate | B_COLLECTION_DATE | DT | 8 | OPTIONAL |
| 6. | eDWR Submission LabReport SampleAnalysisResults PWSIdentifier OR if used along with the Sample object eDWR Submission LabReport Sample SampleIdentification PWSIdentifier | B_PWS_NUMBER | AN | 9 | MANDATORY - Reference field required to associate the result to the parent sample. Must be same value as field B_PWS_NUMBER in B_SAMPLE structure set or result (and sample) will be rejected. |
| 7. | eDWR Submission LabReport LabIdentification LabAccreditation LabAccreditationIdentifier | B_LAB_ID_NUMBER | AN | 10 | MANDATORY |
| 8. | eDWR Submission LabReport LabIdentification LabAccreditation LabAccreditationAuthortity | B_CERT_AGENCY | AN | 10 | MANDATORY |
| 9. | eDWR Submission LabReport SampleAnalysisResults AnalyteIdentification AnalyteCode OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalyteIdentification AnalyteCode | B_ANALYTE_CODE | AN | 4 | MANDATORY - Required field must be valued. |
| 10. | eDWR Submission LabReport SampleAnalysisResults LabAnalysisIdentification AnalysisStartDate OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification AnalysisStartDate | B_ANALYSIS_START_DATE | DT | 8 | OPTIONAL - Sample will be rejected if Analysis Start Date is supplied but is prior to Sample Collection Date. |
| 11. | eDWR Submission LabReport SampleAnalysisResults LabAnalysisIdentification AnalysisStartTime OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification AnalysisStartTime | B_ANALYSIS_START_TIME | TM | 6 | OPTIONAL |
| 12. | eDWR Submission LabReport SampleAnalysisResults LabAnalysisIdentification AnalysisEndDate OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification AnalysisEndDate | B_ANALYSIS_COMPLETE_DATE | DT | 8 | OPTIONAL - Sample will be rejected if Analysis Completion Date is supplied but is prior to Sample Collection Date. Sample will also be rejected if both Analysis Start and Completion Dates are supplied, but Completion Date is prior to Start Date. |
| 13. | eDWR Submission LabReport SampleAnalysisResults LabAnalysisIdentification AnalysisEndTime OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification AnalysisEndTime | B_ANALYSIS_COMPLETE_TIME | TM | 6 | OPTIONAL |
| 14. | eDWR Submission LabReport SampleAnalysisResults AnalysisResult ResultStateNotificationDate OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult ResultStateNotificationDate | B_STATE_NOTIFY_DATE | DT | 8 | OPTIONAL |
| 15. | eDWR Submission LabReport SampleAnalysisResults AnalysisResult PWSNotificationDate OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult PWSNotificationDate | B_WATER_SYSTEM_NOTIFY_DATE | DT | 8 | OPTIONAL |
| 16.* | eDWR Submisssion LabReport SampleAnalysisResults QAQCSummary DataQualityCode OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation QAQCSummary DataQualityCode | B_DATA_QUALITY | AN | 1 | MANDATORY |
| 17.* | eDWR Submission LabReport SampleAnalysisResults QAQCSummary DataQualityRejectCode OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation QAQCSummary DataQualityRejectCode | B_DATA_QUALITY_REASON | AN | 2 | CONDITIONALLY MANDATORY - Field must be valued if field 16 (B_DATA_QUALITY) = “R.” |
| 18.* | eDWR Submission LabReport SampleAnalysisResults LabAnalysisIdentification SampleAnalyticalMethod: MethodIdentifier OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification SampleAnalyticalMethod: MethodIdentifier | B_ANALYSIS_METHOD_CODE | AN | 30 | OPTIONAL - May be either a federally owned or state-owned analyte method pairing that exists in state/region’s SDWIS/STATE database. |
| 19.* | eDWR Submission LabReport SampleAnalysisResult LabAnalysisIdentification SampleAnalyzedMeasure MeasurementValue eDWR Submission LabReport SampleAnalysisResult LabAnalysisIdentification SampleAnalyzedMeasure MeasurementUnit eDWR Submission LabReport SampleAnalysisResults LabAnalysisIdentification SampleAnalyzedMeasure MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification SampleAnalyzedMeasure MeasurementValue eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification SampleAnalyzedMeasure MeasurementUnit eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification SampleAnalyzedMeasure MeasurementSignificantDigits | B_VOLUME_ASSAYED | AN | 9 | N/A |
| 20.* | eDWR Submission LabReport SampleAnalysisResults AnalysisResult SampleInterferenceReasonCode OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult SampleInterferenceReasonCode | B_LAB_REJECTION_REASON | AN | 4 | N/A |
| 21.* | eDWR Submission LabReport SampleAnalysisResult AnalysisResult Result MeasurementQualifier OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementQualifier | B_MICROBE_PRESENCE_INDICATOR | AN | 1 | N/A |
| 22. | eDWR Submission LabReport SampleAnalysisResult AnalysisResult Result MeasurementValue eDWR Submission LabReport SampleAnalysisResult AnalysisResult Result MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementValue eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementSignificantDigits | B_COUNT | N | 15,5 | N/A |
| 23.* | eDWR Submission LabReport SampleAnalysisResults AnalysisResult Result MeasurementUnit OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementUnit | B_COUNT_TYPE | AN | 10 | N/A |
| 24.* | eDWR Submission LabReport SampleAnalysisResult AnalysisResult Result MicrobialResultCountTypeCode OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MicrobialResultCountTypeCode | B_COUNT_UNITS | AN | 9 | N/A |
| 25.* | eDWR Submission LabReport SampleAnalysisResult AnalysisResult Result MeasurementQualifier OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementQualifier | B_LESS_THAN_INDICATOR | AN | 1 | CONDITIONALLY MANDATORY - Must be valued if field 16 (B_DATA_QUALITY) = (“A” or “P”) and field 29 (B_CONCENTRATION) is not valued. |
| 26.* | eDWR Submission LabReport SampleAnalysisResult AnalysisResult DetectionLimitTypeCode OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult DetectionLimitTypeCode | B_LESS_THAN_CODE | AN | 3 | CONDITIONALLY MANDATORY - Must be valued if field 25 (B_LESS_THAN_INDICATOR) = “Y.” |
| 27. | eDWR Submission LabReport SampleAnalysisResults AnalysisResult DetectionLimit MeasurementValue eDWR Submission LabReport SampleAnalysisResults AnalysisResult DetectionLimit MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResult AnalysisResultInformation DetectionLimit MeasurementValue eDWR Submission LabReport Sample AnalysisResult AnalysisResultInformation DetectionLimit MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) | B_DETECTION_LEVEL | N | 15(9) | CONDITIONALLY MANDATORY - Must be valued if field 26 (B_LESS_THAN_CODE) = “MRL.” |
| 28. | eDWR Submission LabReport SampleAnalysisResults AnalysisResult DetectionLimit MeasurementUnit OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult DetectionLimit MeasurementUnit | B_DETECTION_LEVEL_UNIT_CODE | ANmc | 9 | CONDITIONALLY MANDATORY - Must be valued if field 27 (B_DETECTION_LEVEL) is valued. |
| 29. | eDWR Submission LabReport SampleAnalysisResults AnalysisResult Result MeasurementValue eDWR Submission LabReport SampleAnalysisResults AnalysisResult Result MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementValue eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) | B_CONCENTRATION | N | 15(9) | CONDITIONALLY MANDATORY - Must be valued if field 16 (B_DATA_QUALITY) = (“A” or “P”) and field 25 (B_LESS_THAN_INDICATOR) is not valued. |
| 30. | eDWR Submission LabReport SampleAnalysisResults AnalysisResult Result MeasurementUnit OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementUnit | B_CONCENTRATION_UNIT_CODE | ANmc | 9 | CONDITIONALLY MANDATORY - Must be valued if field 29 (B_CONCENTRATION). |
| 31. | eDWR Submission LabReport SampleAnalysisResults AnalysisResult Result MeasurementValue eDWR Submission LabReport SampleAnalysisResults AnalysisResult Result MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementValue eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) | B_REPORTED_MEASURE | AN | 10 | N/A |
| 32. | eDWR Submission LabReport SampleAnalysisResult AnalysisResult AnalysisComment Comments OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult AnalysisComment Comments | B_REPORTED_MEASURE_COUNT_ERROR | N | 8(3) | OPTIONAL - If value supplied does not convert to valid number, result will be rejected. |
| 33. | eDWR Submission LabReport SampleAnalysisResult AnalysisResult AnalysisComment Comments OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult AnalysisComment Comments | B_COMMENT | ANmc |  | OPTIONAL |
| - 34. | eDWR Submission LabReport SampleAnalysisResult AnalysisResult - RecordID | - B_XMLDOC_RECORD_ID | - N | - 7 | - OPTIONAL |

### Other Analysis Result Measures Staging Table Structure and Business   Rules

This section lists the schema tag hierarchy, staging table/column name and its domain, size, optionality, and the business rules associated with Other Sample Analysis Measure.

| OTHER SAMPLE ANALYSIS MEASURE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| LINE NO (field) | SCHEMA TAG NAME | STAGING TABLE ( TMGOSAM ) COLUMN NAME | DOMAIN | SIZE | OPTIONALITY |
| 1. | N/A | B_TMGOSAM_IS_NUMBER | N | 7 | MANDATORY. Populated by the XML Sampling Application. |
| 2. | N/A | B_STATUS | AN | 1 | OPTIONAL. Populated by the XML Sampling Application. |
| 3. | eDWR Submission LabReport SampleAnalysisResults AnalysisResult SpecializedMeasurement SpecializedMeasurementTypeCode | B_NAME | AN | 40 | OPTIONAL |
| 4. | eDWR Submission LabReport SampleAnalysisResults AnalysisResult SpecializedMeasurement MeasurementValue eDWR Submisssion LabReport Sample SampleIdentification SampleVolume MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) | B_VALUE_NUMBER | N | 16(8) | OPTIONAL |
| 5. | N/A | B_VALUE_TEXT | AN | 20 | OPTIONAL |
| 6. | eDWR Submisssion LabReport SampleAnalysisResults AnalysisResult SpecializedMeasurement MeasurementUnit | B_UOM_CODE | ANmc | 9 | MANDATORY |
| - 7. | eDWR Submission LabReport SampleAnalysisResult AnalysisResult - RecordID | - B_XMLDOC_RECORD_ID | - N | - 7 | - Optional |

## Enumerated Values

### Sample Result Enumerated Values

This section lists the staging table/column name and the permitted values associated with the columns.

| SAMPLE RESULT ENUMERATED VALUES |  |  |  |
| --- | --- | --- | --- |
| LINE NO (field) | STAGING TABLE ( B_SAMPLE_RESULT ) COLUMN NAME | PERMITTED VALUES | PERMITTED VALUE TYPE |
| 8. | B_CERT_AGENCY | User Defined | User Defined in TINPVALS where CODE_NAME = CERT_AG |
| 16. | B_DATA_QUALITY | SPACE A Accepted R Rejected P Preliminary V Validated | Fixed |
| 17. | B_DATA_QUALITY_REASON | User Defined | User Defined in TINPVALS where CODE_NAME = SAR_DQ |
| 18. | B_ANALYSIS_METHOD_CODE | Federally owned Analyte Method Pairings are available in the SDWIS/STATE Online Data Dictionary. Federally owned and state-owned Analyte Method Pairings may be viewed by accessing the Analyte Method Pairing menu item available in the SDWIS Administration component of SDWIS/STATE. | User Defined. The value is maintained in the column CODE in table TSASMN |
| 19. | B_VOLUME_ASSAYED | 1ML 1 Milliliter 5ML 5 Milliliters 10ML 10 Milliliters 100ML 100 Milliliters 300ML 300 Milliliters 400ML 400 Milliliters 500ML 500 Milliliters SAMP VOL SLIDE Note: The enumerated values are defined for the TMGSAR table. The reporting method is to put the numeric portion in the MeasurementValue schema tag and the Alpha Numeric portion in the MeasurementUnit schema tag. | Fixed |
| 20. | B_LAB_REJECTION_REASON | CNFG Confluent Growth TCNG Turbid Culture No Gas TNTC Too Numerous to Count SPACE | Fixed |
| 21. | B_MICROBE_PRESENCE_INDICATOR | A Absence P Presence SPACE | Fixed |
| 23. | B_COUNT_TYPE | SPACE TUBES COLONIES CFU Colony Forming Units CYSTSC Cysts, Calculated CTSTSO Cysts, Observed MPN Most Probable Number OBSVNS Observations OCYSTSC Oocysts, Calculated OCYSTSO Oocysts, Observed PFU Plaque Forming Units | Fixed |
| 24. | B_COUNT_UNITS | 1ML 1 Milliliter 5ML 5 Milliliters 10ML 10 Milliliters 100ML 100 Milliliters 400ML 400 Milliliters 500ML 500 Milliliters ML Milliliters LITER Liters FLD100 Field at 100 Power (used in measuring numbers of observations) 100L 100 Liters GAL Gallons 100GAL 100 Gallons 400GAL 400 Gallons SAMP VOL SLIDE SPACE | Fixed |
| 25. | B_LESS_THAN_INDICATOR | Y Yes N No | Fixed |
| 26. | B_LESS_THAN_CODE | MDL Federal Minimum Detection Limit MRL Lab Reporting Level SPACES | Fixed |
| 28. | B_DETECTION_LEVEL_UNIT_CODE for RAD Results | User maintained in the TSAUOM table pCi/L Picocuries/Liter mRemY Millirems/Liter/Year mRem Millirems/Liter mg/L Milligrams/Liter (AKA Parts per Million - ppm) ug/L Micrograms/Liter (AKA Parts per Billion - ppb) ng/L Nanograms/Liter (AKA Parts per Trillion - ppt) | User Defined in TSAUOM in column UOM_CODE |
| 28. | B_DETECTION_LEVEL_UNIT_CODE for General Results | User maintained in the TSAUOM table mg/L Milligrams/Liter (AKS Parts per Million - ppm) ug/L Micrograms/Liter (AKA Parts per Billion - ppb) %LUM Percent of Luminance (for measuring color) %PUR Percent of Purity (for measuring color) ADMIU American Dye Manufacturers Institute Units (for measuring color) AGGR Aggressive Index (for corrosivity) C Degrees Celsius CM-1 Total Absorbance for UV CT Contact Time CU Color Units F Degrees Fahrenheit FTU Flavor Threshold Units (for measuring taste and odor) L/mg-m Liter per Milligram-Meter LANG Langlier Index (for measuring corrosivity) Lbs/CFt Pounds/Cubic Foot (for measuring density) Lbs/gal Pounds/Gallon (for measuring density) MFL Million of Fibers/Liter (for measuring asbestos) ng/L Nanograms/Liter (AKA Parts per Trillion - ppt) nm Nanometer NTU Nephelometric Units OBSVNS Observations/field at 100 Power pH pH Ratio Ratio of Actual/Required TOC removal SU Standard Units TON Threshold Odor Centimeter umho/cm Micromhos per Centimeter | User Defined in TSAUOM in column UOM_CODE |
| 30. | B_CONCENTRATION_UNIT_CODE for RAD Results | User maintained in the TSAUOM table pCi/L Picocuries/Liter mRemY Millirems/Liter/Year mRem Millirems/Liter mg/L Milligrams/Liter (AKA Parts per Million - ppm) ug/L Micrograms/Liter (AKA Parts per Billion - ppb) ng/L Nanograms/Liter (AKA Parts per Trillion - ppt) | User Defined in TSAUOM in column UOM_CODE |
| 30. | B_CONCENTRATION_UNIT_CODE for General Results | User maintained in the TSAUOM table mg/L Milligrams/Liter (AKS Parts per Million - ppm) ug/L Micrograms/Liter (AKA Parts per Billion - ppb) %LUM Percent of Luminance (for measuring color) %PUR Percent of Purity (for measuring color) ADMIU American Dye Manufacturers Institute Units (for measuring color) AGGR Aggressive Index (for corrosivity) C Degrees Celsius CM-1 Total Absorbance for UV CT Contact Time CU Color Units F Degrees Fahrenheit FTU Flavor Threshold Units (for measuring taste and odor) L/mg-m Liter per Milligram-Meter LANG Langlier Index (for measuring corrosivity) Lbs/CFt Pounds/Cubic Foot (for measuring density) Lbs/gal Pounds/Gallon (for measuring density) MFL Million of Fibers/Liter (for measuring asbestos) ng/L Nanograms/Liter (AKA Parts per Trillion - ppt) nm Nanometer NTU Nephelometric Units OBSVNS Observations/field at 100 Power pH pH Ratio Ratio of Actual/Required TOC removal SU Standard Units | User Defined in TSAUOM in column UOM_CODE |
|  | B_CONCENTRATION_UNIT_CODE for General Results | TON Threshold Odor Centimeter umho/cm Micromhos per Centimeter |  |

### Other Analysis Result Measures Enumerated Values

This section lists the staging table/column name and the permitted values associated with the columns.

| OTHER ANLYSIS RESULT MEASURE |  |  |  |
| --- | --- | --- | --- |
| LINE NO (field) | STAGING TABLE ( TMGOSAM ) COLUMN NAME | PERMITTED VALUE | PERMITTED VALUE TYPE |
| *3 | B_NAME | User Defined | The value should exist in the table TSASMMT and column NAME |
| *6. | B_UOM_CODE | User Defined | The value should exist in the table TSASMMT and column |

## Sample Results Mapping from XML Schema Tag to SDWIS/STATE Entities and Attributes

This section has the mapping between the staging table/columns, SDWIS/STATE Table/Column in both the data model and the physical database and the Schema tags in the XML Sampling schemas for Sample Result business object.

| SAMPLE RESULT SCHEMA TAG MAPPING TO SDWIS/STATE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| LINE NO. (field) | SCHEMA TAG NAME | Staging Table TMGSAR Column Name | SDWIS/STATE Table and Column Name | SDWIS/STATE Entity Name | SDWIS/STATE ATTRIBUTE |
| 1. # | N/A | N/A | N/A | N/A | N/A |
| 2. # | N/A | N/A | N/A | N/A | N/A |
| 3. # | N/A | N/A | N/A | N/A | N/A |
| 4. | eDWR Submission LabReport SampleAnalysisResults LabSampleIdentifier OR if used along with the Sample object eDWR Submission LabReport Sample SampleIdentification LabSampleIdentifier | B_LAB_SAMPLE_NUMBER | TSASAMPL.LAB_ASGND_ID_NUM | Sample | LAB_ASSIGNED_ID_NUMBER (Reference Key to SBS Sample) |
| 5. | eDWR Submission LabReport SampleAnalysisResults SampleCollectionEndDate OR if used along with the Sample object eDWR Submission LabReport Sample SampleIdentification SampleCollectionEndDate | B_COLLECTION_DATE | TSASAMPL.COLLCTN_END_DT | Sample | COLLCTN_END_DT |
| 6. | eDWR Submission LabReport SampleAnalysisResults PWSIdentifier OR if used along with the Sample object eDWR Submission LabReport Sample SampleIdentification PWSIdentifier | B_PWS_NUMBER | TINWSYS.NUMBER | Water System | NUMBER (Reference Key to SBS Sample) |
| 7. | eDWR Submission LabReport LabIdentification LabAccreditation | B_LAB_ID_NUMBER | TSALAB.LAB_ID_NUMBER | Laboratory | LAB_ID_NUMBER (Foreign Key to SBS Sample) |
| 8. | eDWR Submission LabReport LabIdentification LabAccreditation LabAccreditationAuthortity | B_CERT_AGENCY | TSALAB.CERT_AB | Laboratory | CERT_AGENCY (Foreign Key to SBS Sample) |
| 9. | eDWR Submission LabReport SampleAnalysisResults AnalyteIdentification AnalyteCode OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalyteIdentification AnalyteCode | B_ANALYTE_CODE | TSAANLYT.CODE | Analyte | CODE (Foreign Key to SBS Sample Analytical Result) |
| 10. | eDWR Submission LabReport SampleAnalysisResults LabAnalysisIdentification AnalysisStartDate OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification AnalysisStartDate | B_ANALYSIS_START_DATE | TSASAR.ANALYSIS_START_DT | Sample Analytical Result | ANALYSIS_START_DATE |
| 11. | eDWR Submission LabReport SampleAnalysisResults LabAnalysisIdentification AnalysisStartTime OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification AnalysisStartTime | B_ANALYSIS_START_TIME | TSASAR.ANALYSIS_ST_TIME | Sample Analytical Result | ANALYSIS_START_TIME |
| 12. | eDWR Submission LabReport SampleAnalysisResults LabAnalysisIdentification AnalysisEndDate OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification AnalysisEndDate | B_ANALYSIS_COMPLETE_DATE | TSASAR.ANALYSIS_COMPL_DT | Sample Analytical Result | ANALYSIS_COMPLETE_DATE |
| 13. | eDWR Submission LabReport SampleAnalysisResults LabAnalysisIdentification AnalysisEndTime OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification AnalysisEndTime | B_ANALYSIS_COMPLETE_TIME | TSASAR.ANALYSIS_COMP_TIME | Sample Analytical Result | ANALYSIS_COMPLETE_TIME |
| 14. | eDWR Submission LabReport SampleAnalysisResults AnalysisResult ResultStateNotificationDate OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult ResultStateNotificationDate | B_STATE_NOTIFY_DATE | TSASAR.ST_NOTIFICATION_DT | Sample Analytical Result | STATE_NOTIFICATION_DATE |
| 15. | eDWR Submission LabReport SampleAnalysisResults AnalysisResult PWSNotificationDate OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult PWSNotificationDate | B_WATER_SYSTEM_NOTIFY_DATE | TSASAR.WS_NOTIFICATION_DT | Sample Analytical Result | WATER_SYSTEM_NOTIFICATION_DATE |
| 16. | eDWR Submisssion LabReport SampleAnalysisResults QAQCSummary DataQualityCode OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation QAQCSummary DataQualityCode | B_DATA_QUALITY | TSASAR.DATA_QUALITY_CODE | Sample Analytical Result | DATA_QUALITY_CODE |
| 17. | eDWR Submission LabReport SampleAnalysisResults QAQCSummary DataQualityRejectCode OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation QAQCSummary DataQualityRejectCode | B_DATA_QUALITY_REASON | TSASAR.DATA_QTY_RSN_TXT | Sample Analytical Result | DATA_QUALITY_REASON_CODE |
| 18. | eDWR Submission LabReport SampleAnalysisResults LabAnalysisIdentification SampleAnalyticalMethod: MethodIdentifier OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation LabAnalysisIdentification SampleAnalyticalMethod: MethodIdentifier | B_ANALYSIS_METHOD_CODE | TSASMN.CODE | Standard Method Number | CODE (Foreign Key to SBS Sample Analytical Result) |
| 19. | eDWR Submission LabReport SampleAnalysisResult LabAnalysisIdentification SampleAnalyzedMeasure MeasurementValue eDWR Submission LabReport Sample SampleIdentification SampleVolume MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) OR if used along with the Sample object | B_VOLUME_ASSAYED | TSAMAR.ASSAY_UOM_CODE | Microbiological Analytical Result | ASSAY_UOM_CODE |
| 20. | eDWR Submission LabReport SampleAnalysisResults AnalysisResult SampleInterferenceReasonCode OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult SampleInterferenceReasonCode | B_LAB_REJECTION_REASON | TSAMAR.REJECT_REASON_CODE | Microbiological Analytical Result | REJECTION_REASON_CODE |
| 21. | eDWR Submission LabReport SampleAnalysisResult AnalysisResult Result MeasurementQualifier OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementQualifier | B_MICROBE_PRESENCE_INDICATOR | TSAMAR. PRESENCE_IND_CODE | Microbiological Analytical Result | PRESENCE_INDICATOR_CODE |
| 22. | eDWR Submission LabReport SampleAnalysisResult AnalysisResult Result MeasurementValue eDWR Submission LabReport SampleAnalysisResult AnalysisResult Result MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) | B_COUNT | TSAMAR.COUNT_QTY | Microbiological Analytical Result | COUNT_QTY |
| 23. | eDWR Submission LabReport SampleAnalysisResults AnalysisResult Result MeasurementUnit OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementUnit | B_COUNT_TYPE | TSAMAR.COUNT_TYPE | Microbiological Analytical Result | COUNT_TYPE |
| 24. | eDWR Submission LabReport SampleAnalysisResult AnalysisResult Result MicrobialResultCountTypeCode OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MicrobialResultCountTypeCode | B_COUNT_UNITS | TSAMAR.COUNT_UOM_CODE | Microbiological Analytical Result | COUNT_UOM_CODE |
| 25. | eDWR Submission LabReport SampleAnalysisResult AnalysisResult Result MeasurementQualifier OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementQualifier | B_LESS_THAN_INDICATOR | TSASAR.LESS_THAN_IND | Sample Analytical Result | LESS_THAN_INDICATOR |
| 26. | eDWR Submission LabReport SampleAnalysisResult AnalysisResult DetectionLimitTypeCode OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult DetectionLimitTypeCode | B_LESS_THAN_CODE | TSASAR.LESS_THAN_CODE | Sample Analytical Result | LESS_THAN_CODE |
| 27. | eDWR Submission LabReport SampleAnalysisResult AnalysisResult DetectionLimit MeasurementValue eDWR Submission LabReport Sample SampleIdentification SampleVolume MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) | B_DETECTION_LEVEL | TSASAR.DETECTN_LIMIT_NUM | Sample Analytical Result | DETECTION_LIMIT_NUMBER |
| 28. | eDWR Submission LabReport SampleAnalysisResult AnalysisResult DetectionLimit MeasurementUnit OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult DetectionLimit MeasurementUnit | B_DETECTION_LEVEL_UNIT_CODE | TSASAR.DETECTN_LIM_UOM_CD | Sample Analytical Result | DETECTION_LIMIT_UOM_CODE |
| 29. | eDWR Submission LabReport SampleAnalysisResult AnalysisResult Result MeasurementValue eDWR Submission LabReport Sample SampleIdentification SampleVolume MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) | B_CONCENTRATION | TSASAR.CONCENTRATION_MSR | Sample Analytical Result | CONCENTRATION_MSR |
| 30. | eDWR Submission LabReport SampleAnalysisResult AnalysisResult Result MeasurementUnit OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult Result MeasurementUnit | B_CONCENTRATION_UNIT_CODE | TSASAR.UOM_CODE | Sample Analytical Result | UOM_CODE |
| 31. | eDWR Submission LabReport SampleAnalysisResult AnalysisResult Result MeasurementValue eDWR Submission LabReport Sample SampleIdentification SampleVolume MeasurementSignificantDigits (This schema element has to be valued for precision / significant digits) | B_REPORTED_MEASURE | TSASAR.REPORTED_MSR | Sample Analytical Result | REPORTED_MEASURE |
| 32. | eDWR Submission LabReport SampleAnalysisResult AnalysisResult RadiologicalResultCountError OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult RadiologicalResultCountError | B_REPORTED_MEASURE_COUNT_ERROR | TSASAR.RAD_COUNTING_ERROR | Sample Analytical Result | RAD_COUNTING_ERROR |
| 33 | eDWR Submission LabReport SampleAnalysisResult AnalysisResult AnalysisComment Comments OR if used along with the Sample object eDWR Submission LabReport Sample AnalysisResultInformation AnalysisResult AnalysisComment Comments | B_COMMENTS | TSASAR.COMMENTS | Sample Analytical Result | COMMENTS |
| 34. | eDWR Submission LabReport SampleAnalysisResult AnalysisResult RecordID | B_XMLDOC_RECORD_ID | N/A | N/A | N/A |

# SCHEMA MAPPING FOR SAMPLE SUMMARY REPORTING

Schema SDWIS_Summary_v3.0.xsd maps to the Sample Summary business unit.

The Sample Summary has four sections. The first section describes the sample summary schema tag hierarchy, staging table/columns, their length, optionality and the business rules associated with them. The second section describes the Enumerated values for the Sample Summary elements and the business rules.

The third section describes the mappping between the staging table/columns, SDWIS State table/columns and the schema elements in the SDWIS_Summary.xsd schema. The fourth section defines the staging table/columns.

## Sample Summary Staging Table Structure and Business Rules

### Schema Changes

None this release.

### Structure Set

This section lists the schema tag hierarchy, staging table/column name and its domain, size, optionality, and the business rules associated with Sample Summary.

| SAMPLE SUMMARY |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| LINE NO (field) | SCHEMA TAG NAME | STAGING TABLE ( TMGSMPSM ) COLUMN NAME | DOMAIN | SIZE | OPTIONALITY |
| 1. | N/A | TMGJOB_IS_NUMBER | N | 10 | MANDATORY. Populated by the XML Sampling Application. |
| 2. | N/A | B_TMGSMPSM_IS_NUMBER | N | 7 | MANDATORY. Populated by the XML Sampling Application. |
| 3. | N/A | B_STATUS | AN | 1 | OPTIONAL. Populated by the XML Sampling Application. |
| 4. | SampleSummaries SampleSummaryDetails PWSIdentifier | B_PWS_NUMBER | AN | 9 | MANDATORY - Uniqueness/Duplicate check is on the combination of field 4 (B_PWS_NUMBER), field 7 (B_WSF_STATE_ASGN_ID), and field 11 (B_ANALYTE_CODE), and field 12 (B_MONITORING_PERIOD_START_DATE), and field 13 (B_MONITORING_PERIOD_END_DATE). |
| 5. | SampleSummary SampleSummaryDetails LabAccreditation LabAccreditationdentifier | B_LAB_ID_NUMBER | AN | 10 | MANDATORY |
| 6. | SampleSummary SampleSummaryDetails LabAccreditation LabAccreditationAuthorityName | B_CERT_AGENCY | AN | 10 | MANDATORY |
| 7. | SampleSummary SampleSummaryDetails PWSFacilityIndentifier | B_WSF_STATE_ASGN_ID | AN | 12 | MANDATORY |
| 8.* | SampleSummary SampleSummaryDetails SampleComplianceIndicator | B_COMPLIANCE_INDICATOR | AN | 1 | MANDATORY |
| 9. | SampleSummary SampleSummaryDetails SummaryCollectionStartDate | B_SUMMARY_COLLECTION_START_DATE | DT | 8 | OPTIONAL - If valued, must be less than or equal to the current date. |
| 10. | SampleSummary SampleSummaryDetails SummaryCollectionEndDate | B_SUMMARY_COLLECTION_END_DATE | DT | 8 | OPTIONAL - If valued, must be after the Summary Collection Start Date in field 9 (B_SUMMARY_COLLECTION_START_DATE). |
| 11. | SampleSummary SampleSummaryDetails AnalyteCode | B_ANALYTE_CODE | AN | 4 | MANDATORY - Uniqueness/Duplicate check is on the combination of field 4 (B_PWS_NUMBER), field 7 (B_WSF_STATE_ASGN_ID), and field 11 (B_ANALYTE_CODE), and field 12 (B_MONITORING_PERIOD_START_DATE), and field 13 (B_MONITORING_PERIOD_END_DATE). See note at end of SAMPLE SUMMARY RESULT table. |
| 12. | SampleSummary SampleSummaryDetails MonitoringPeriodBeginDate | B_MONITORING_PERIOD_START_DATE | DT | 8 | MANDATORY - Uniqueness/Duplicate check is on the combination of field 4 (B_PWS_NUMBER), field 7 (B_WSF_STATE_ASGN_ID), and field 11 (B_ANALYTE_CODE), and field 12 (B_MONITORING_PERIOD_START_DATE), and field 13 (B_MONITORING_PERIOD_END_DATE). Value supplied must be the first day of the first month of the monitoring period. |
| 13. | SampleSummary SampleSummaryDetails MonitoringPeriodEndDate | B_MONITORING_PERIOD_END_DATE | DT | 8 | MANDATORY - Uniqueness/Duplicate check is on the combination of field 4 (B_PWS_NUMBER), field 7 (B_WSF_STATE_ASGN_ID), and field 11 (B_ANALYTE_CODE), and field 12 (B_MONITORING_PERIOD_START_DATE), and field 13 (B_MONITORING_PERIOD_END_DATE). Value supplied must be the last day of the last month of the monitoring period. |
| 14. | SampleSummary SampleSummaryDetails SummaryReceivedDate | B_SUMMARY_RECEIVED_DATE | DT | 8 | OPTIONAL |
| 15. | SampleSummary SampleSummaryDetails SummaryCommentText | B_COMMENT_TEXT | ANmc | 2000 | OPTIONAL |
| 16. | SampleSummary SampleSummaryDetails RecordID | B_XMLDOC_RECORD_ID | N | 7 | OPTIONAL |

## Sample Summary Enumerated Values

This section lists the staging table/column name and the permitted values associated with the columns.

| SAMPLE SUMMARY ENUMERATED VALUES |  |  |  |
| --- | --- | --- | --- |
| LINE NO (field) | STAGING TABLE ( TMGSMPSM) COLUMN NAME | PERMITTED VALUES | PERMITTED VALUE TYPE |
| 6. | B_CERT_AGENCY |  | User Defined in table TINPVALS with code-name = ‘CERT_AG’ |
| 8. | B_COMPLIANCE_INDICATOR | Y Yes N No | Fixed |
| 11. | B_ANALYTE_CODE | The users can only add and maintain summaries with the following analyte codes using the SDWIS/STATE on-line pages. CU90-Copper PB90-Lead 3100-Total Coliform | Fixed |
|  |  | The users may add summaries with any analyte code; however, they will not able to maintain them using the SDWIS/STATE on-line pages. User Defined | User Defined in table TSAANLYT column CODE |

## Sample Summary Mapping from XML Schema Tag to SDWIS/STATE Entities and Attributes

This section has the mapping between the staging table/columns, SDWIS/STATE Table/Column in both the data model and the physical database and the Schema tags in the XML Sampling schemas for Sample Summary business object.

| SAMPLE SUMMARY SCHEMA TAG MAPPING TO SDWIS/STATE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| LINE NO. (field) | SCHEMA TAG NAME | Staging Table ( TMGSMPSM ) Column Name | SDWIS/STATE Table and Column Name | SDWIS/STATE Entity Name | SDWIS/STATE ATTRIBUTE |
| 1. # | N/A | TMGJOB_IS_NUMBER | N/A | N/A | N/A |
| 2. # | N/A | B_TMGSMPSM_IS_NUMBER | N/A | N/A | N/A |
| 3. # | N/A | B_STATUS | N/A | N/A | N/A |
| 4. | SampleSummary SampleSummaryDetails PWSIdentifier | B_PWS_NUMBER | TINWSYS.TINWSYS.NUMBER | Water System | NUMBER (Foreign Key to either SBS Sample or Sample Summary) |
| 5. | SampleSummary SampleSummaryDetails LabAccreditation LabAccreditationIdentifier | B_LAB_ID_NUMBER | TSALAB.LAB_IS_NUMBER | Laboratory | LAB_IS_NUMBER |
| 6. | SampleSummary SampleSummaryDetails LabAccreditation LabAccreditationAuthorityName | B_CERT_AGENCY | TSALAB.CERT_AGENCY | Laboratory | CERT_AGENCY |
| 7. | SampleSummary SampleSummaryDetails PWSFacilityIndentifier | B_WSF_STATE_ASGN_ID | TINWSF.ST_ASGN_IDENT_CD | Water System Facility | STATE_ASGN_IDENTIFICATION_CODE (Foreign Key to either SBS Sample or Sample Summary) |
| 8. | SampleSummary SampleSummaryDetails SampleComplianceIndicator | B_COMPLIANCE_INDICATOR | TSASMPSM.COMPL_PURP_IND_CD | Sample Summary | COMPLIANCE_PURPOSE_INDICATOR_CODE |
| 9. | SampleSummary SampleSummaryDetails SummaryCollectionStartDate | B_SUMMARY_COLLECTION_START_DATE | TSASMPSM.COLLECTION_STRT_DT | Sample Summary | SAMPLE_COLLECTION_START_DATE |
| 10. | SampleSummary SampleSummaryDetails SummaryCollectionEndDate | B_SUMMARY_COLLECTION_END_DATE | TSASMPSM.COLLECTION_END_DT | Sample Summary | SAMPLE_COLLECTION_END_DATE |
| 11. | SampleSummary SampleSummaryDetails AnalyteCode | B_ANALYTE_CODE | TSAANLYT.CODE | Analyte | CODE (Foreign Key to Sample Summary) |
| 12. | SampleSummary SampleSummaryDetails MonitoringPeriodBeginDate | B_MONITORING_PERIOD_START_DATE | TMNMPRD.BEGIN_DATE | Monitoring Period | BEGIN_DATE (Foreign Key to Sample Summary) |
| 13. | SampleSummary SampleSummaryDetails MonitoringPeriodEndDate | B_MONITORING_PERIOD_END_DATE | TMNMPRD.END_DATE | Monitoring Period | END_DATE (Foreign Key to Sample Summary) |
| 14. | SampleSummary SampleSummaryDetails SummaryReceivedDate | B_SUMMARY_RECEIVED_DATE | TSASMPSM.SUMMARY_RECPT_DT | Sample Summary | SUMMARY_RECEIVED_DATE |
| 15 | SampleSummary SampleSummaryDetails SummaryCommentText | B_COMMENT_TEXT | TSASMPSM.COMMENT_TEXT | Sample Summary | COMMENT_TEXT |
| 16 | SampleSummary SampleSummaryDetails RecordID | B_XMLDOC_RECORD_ID | N/A | N/A | N/A |

## Sample Summary Definitions

This section defines the staging table/ column for the Sample Summary business object and also mentions any business rules associated with it.

| SAMPLE SUMMARY DEFINITIONS |  |  |
| --- | --- | --- |
| LINE NO (field) | STAGING TABLE ( TMGSMPSM ) COLUMN NAME | SAMPLE SUMMARY (S) |
| 1. | TMGJOB_IS_NUMBER | The identification number of the job. |
| 2. | B_TMGSMPSM_IS_NUMBER | A system_generated value which uniquely identifies the entity occurrence. |
| 3. | B_STATUS | The current status of the record. <SPACES> = unprocessed, C = processed w/ changes, P = processed , R = rejected |
| 4. | B_PWS_NUMBER | The Public Water System (PWS) identification number. This field is mandatory. |
| 5. | B_LAB_ID_NUMBER | Laboratory Number assigned to the laboratory by the certifying agency. |
| 6. | B_CERT_AGENCY | The certifying agency that assigned the laboratory the lab id number under which the lab is reporting. |
| 7. | B_WATER_FACILITY_STATE_CODE | Number/alphanumeric that uniquely identifies a Water System Facility (e.g., Treatment Plant/Distribution System/Well) within a Water System. |
| 8. | B_COMPLIANCE_INDICATOR | When set to “Y,” indicates that the sample summary has been taken for compliance. |
| 9. | B_SUMMARY_COLLECTION_START_DATE | The start date of the collection time period during which the sample summary should be taken. |
| 10. | B_SUMMARY_COLLECTION_END_DATE | The end date of the collection time period during which the sample summary should be taken. |
| 11. | B_ANALYTE_CODE | Standard code used to represent a given analyte. |
| 12. | B_MONITORING_PERIOD_START_DATE | Start date of the Monitoring Period. |
| 13. | B_MONITORING_PERIOD_END_DATE | End date of the Monitoring Period. |
| 14. | B_SUMMARY_RECEIVED_DATE | The date the sample summary was received by the state. |
| 15. | B_COMMENT_TEXT | The comment for the record. |
| 16 | B_XMLDOC_RECORD_ID | A number used to uniquely identify this record within an xml document. |

# SCHEMA MAPPING FOR SAMPLE SUMMARY RESULT BUSINESS OBJECT

SDWIS/ XMLSampling Schema SDWIS_SummaryResult_v3.0.xsd maps to the Summary Result business unit.

The schema mapping for Summary Result has four sections.  The first sections describe the schema tag hierarchy, staging table/column, their length optionality, and the business rules.  The second section describes the Enumerated values for the Sample Summary Result elements and the business rules. The third section describes the mapping between the staging table/columns, SDWIS/STATE table/columns for the Sample Summary Result and the schema elements in the SDWIS_Summary_v3.0.xsd  schema.

## Sample Summary Result Staging Table Structure and Business Rules

### Schema Changes

None this release.

### Structure Set

This section lists the schema tag hierarchy, staging table/column name and its domain, size, optionality, and the business rules associated with Summary Result.

| SAMPLE SUMMARY RESULT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| LINE NO (field) | SCHEMA TAG NAME | STAGING TABLE ( TMGSSR ) COLUMN NAME | DOMAIN | SIZE | OPTIONALITY |
| 1. | N/A | TMGJOB_IS_NUMBER | N | 10 | MANDATORY. Populated by the XML Sampling Applicatio. |
| 2. | N/A | B_TMGSSR_IS_NUMBER | N | 7 | MANDATORY. Populated by the XML Sampling Applicatio. |
| 3. | N/A | B_STATUS | AN | 1 | OPTIONAL. Populated by the XML Sampling Applicatio. |
| 4. | SummaryResult SummaryResultDetails PWSIdentifier | B_PWS_NUMBER | AN | 9 | MANDATORY - Because Sample Summary has no natural identifier, the combination of field 4 (B_PWS_NUMBER), field 5 (B_WSF_STATE_ASGN_ID) and field 6 (B_ANALYTE_CODE) and field 8 (B_MONITORING_PERIOD_START_DATE) and field 9 (B_MONITORING_PERIOD_END_DATE) serves as the reference to the parent Sample Summary. Required to associate the summary result to the parent Sample Summary. |
| 5. | SummaryResult SummaryResultDetails PWSFacilityIdentifier | B_WSF_STATE_ASGN_ID | AN | 12 | MANDATORY-Because Sample Summary has no natural identifier, the combination of field 4 (B_PWS_NUMBER), field 5 (B_WSF_STATE_ASGN_ID) and field 6 (B_ANALYTE_CODE) and field 8 (B_MONITORING_PERIOD_START_DATE) and field 9 (B_MONITORING_PERIOD_END_DATE) serves as the reference to the parent Sample Summary. Required to associate the summary result to the parent Sample Summary. |
| 6. | SummaryResult SummaryResultDetails AnalyteCode | B_ANALYTE_CODE | AN | 4 | MANDATORY - Because Sample Summary has no natural identifier, the combination of field 4 (B_PWS_NUMBER), field 5 (B_WSF_STATE_ASGN_ID) and field 6 (B_ANALYTE_CODE) and field 8 (B_MONITORING_PERIOD_START_DATE) and field 9 (B_MONITORING_PERIOD_END_DATE) serves as the reference to the parent Sample Summary. Required to associate the summary result to the parent Sample Summary. See note at end of SAMPLE SUMMARY RESULT table. |
| 7. * | SummaryResult SummaryResultDetails SummaryDataQualityCode | B_DATA_QUALITY | AN | 1 | MANDATORY - If this field is not valued, will set it to “A” which is defined as “Accepted.” |
| 8. | SummaryResult SummaryResultDetails MonitoringPeriodBeginDate | B_MONITORING_PERIOD_START_DATE | DT | 8 | MANDATORY - Because Sample Summary has no natural identifier, the combination of field 4 (B_PWS_NUMBER), field 5 (B_WSF_STATE_ASGN_ID) and field 6 (B_ANALYTE_CODE) and field 8 (B_MONITORING_PERIOD_START_DATE) and field 9 (B_MONITORING_PERIOD_END_DATE) serves as the reference to the parent Sample Summary. Required to associate the summary result to the parent Sample Summary. |
| 9. | SummaryResult SummaryResultDetails MonitoringPeriodEndDate | B_MONITORING_PERIOD_END_DATE | DT | 8 | MANDATORY - Because Sample Summary has no natural identifier, the combination of field 4 (B_PWS_NUMBER), field 5 (B_WSF_STATE_ASGN_ID) and field 6 (B_ANALYTE_CODE) and field 8 (B_MONITORING_PERIOD_START_DATE) and field 9 (B_MONITORING_PERIOD_END_DATE) serves as the reference to the parent Sample Summary. Required to associate the summary result to the parent Sample Summary. |
| 10. * | SummaryResult SummaryResultDetails SummaryResultsType | B_RESULTS_TYPE | AN | 2 | MANDATORY - Uniqueness/Duplicate check is on the combination of field 4 (B_PWS_NUMBER), field 5 (B_WSF_STATE_ASGN_ID) and field 6 (B_ANALYTE_CODE) and field 8 (B_MONITORING_PERIOD_START_DATE) and field 9 (B_MONITORING_PERIOD_END_DATE) and field 10 (B_RESULTS_TYPE). See note at end of SAMPLE SUMMARY RESULT table. |
| 11. | SummaryResult SummaryResultDetails CountQuantity | B_COUNT_QUANTITY | N | 10 | CONDITIONALLY MANDATORY - Must be valued if field 12 (B_MEASURE) is not valued. See note at end of SAMPLE SUMMARY RESULT table. |
| 12. | SummaryResult SummaryResultDetails SummaryMeasurement MeasurementValue | B_MEASURE | N | 15(9) | CONDITIONALLY MANDATORY - Must be valued if field 11 (B_COUNT_QUANTITY) is not valued. Value must be numeric; non-numeric symbols (such as “<“) will cause sample to be rejected. A result of 0.0 or a value that converts to 0 may be recorded as a result. See note at end of SAMPLE SUMMARY RESULT table. |
| 13. | SummaryResult SummaryResultDetails SummaryMeasurement MeasurementUnit | B_MEASURE_UNIT_CODE | AN | 9 | CONDITIONALLY MANDATORY - Must be valued if field 12 (B_MEASURE) is valued. Only mg/L is valid. If the value in this field is other than mg/L, the summary will be rejected. |
| 14. | SummaryResult SummaryResultDetails RecordID | B_XMLDOC_RECORD_ID | N | 7 | OPTIONAL |

The SDWIS/STATE on-line software allows for addition and maintenance of Sample Summaries only for the analyte/result type combinations listed in the following table. To facilitate historical data the users can migrate sample summaries and results for other analtye/result type combinations but they may not be maintained on-line, users must create their own queries to view the data. This table provides guidance for valuing these five attributes for the sample summaries and results that may be maintained on-line.

| B_RESULT_TYPE | B_ANALYTE_CODE | B_COUNT_QUANTITY | B_MEASURE | B_MEASURE_UNIT_CODE |
| --- | --- | --- | --- | --- |
| AL | CU90 or PB90 | Value with number of samples exceeding action level | Do not value | Do not value |
| RT | 3100 | Value with number of negative coliform samples | Do not value | Do not value |
| 90 | CU90 or PB90 | Value with number of tap samples collected | Value with 90 | MG/L |
| 95 | CU90 or PB90 | Value with number of tap samples collected | Value with 95 | MG/L |

## Sample Summary Result Enumerated Values

This section lists the staging table/column name and the permitted values associated with the columns.

| SAMPLE SUMMARY RESULT ENUMERATED VALUES |  |  |  |
| --- | --- | --- | --- |
| LINE NO (field) | STAGING TABLE COLUMN NAME | PERMITTED VALUES | PERMITTED VALUE TYPE |
| 7. | B_DATA_QUALITY | A Accepted R Rejected P Preliminary V Validated | Fixed |
| 10. | B_RESULTS_TYPE | The users can only add and maintain the following analyte code/summary type combinations using the SDWIS/STATE on-line pages. With CU90 and PB90 90 90th Percentile (Lead & Copper) 95 95th Percentile (Lead & Copper) AL Samples above Action Level (Lead & Copper) With 3100 RT Routine Samples with Negative Results from Distribution System The following summary types are not maintained in SDWIS/STATE, but to facilitate historical data the users can migrate these types in the SDWIS/STATE tables. Users must create their own queries to view the data. CB Total Number of Disinfectant Residual Measurements below 0.2 mg/L (count). CK Count of Check samples taken CR Total Number of Disinfectant Residual Measurements (count). FV Count of days in Federal Violation period HR High result for period (measure) LR Low result for period (measure) MR Mean (average) result for period (measure) ND Count of negative sample analytical results from distribution system samples NE Count of negative sample analytical results NF Count of negative finished water samples NR Count of negative raw water samples PO Count of number of positive sample analytical results RF Count of required Repeats per Federal regulation RI Count of Replacements Samples for Invalid RJ Count of Rejected samples RL Count of Replacement Samles RP Count of Repeat samples RS Count of required Repeats per State regulation SP Count of Special samples SR Count of samples required SV Count of days in State Violation period TO Count of total samples collected VI Count of days of Violation Period | Fixed |
| 13. | B_MEASURE_UNIT_CODE | User Defined- For lead and copper summaries this must be mg/l. | User Defined in table TSAUOM column UOM_CODE |

## Summary Result Mapping from XML Schema Tag to SDWIS/STATE Entities and Attributes

This section has the mapping between the staging table/columns, SDWIS/STATE Table/Column in both the data model and the physical database and the Schema tags in the SDWIS/ XMLSampling schemas for Summary Result business object.

| SAMPLE SUMMARY RESULT SCHEMA TAG MAPPING TO SDWIS/STATE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| LINE NO. (field) | SCHEMA TAG NAME | Staging Table TMGSSR Column Name | SDWIS/STATE Table and Column Name | SDWIS/STATE Entity Name | SDWIS/STATE ATTRIBUTE |
| 1. # | N/A | TMGJOB_IS_NUMBER | N/A | N/A | N/A |
| 2. # | N/A | B_TMGSSR_IS_NUMBER | N/A | N/A | N/A |
| 3. # | N/A | B_STATUS | N/A | N/A | N/A |
| 4. | SummaryResult SummaryResultDetails PWSIdentifier | B_PWS_NUMBER | TINWSYS.NUMBER | Water System | NUMBER (Reference Key to SBS Sample or Reference Key to Sample Summary) |
| 5. | SummaryResult SummaryResultDetails PWSFacilityIdentifier | B_WSF_STATE_ASGN_ID | TINWSF.ST_ASGN_IDENT_CD | Water System Facility | STATE_ASGN_IDENTIFICATION_CODE (Reference Key to SBS Sample or Reference Key to Sample Summary) |
| 6. | SummaryResult SummaryResultDetails AnalyteCode | B_ANALYTE_CODE | TSAANLYT.CODE | Analyte | CODE (Foreign Key to SBS Sample Analytical Result or Reference Key to Sample Summary) |
| 7. | SummaryResult SummaryResultDetails SummaryDataQualityCode | B_DATA_QUALITY | TSASSR.DATA_QUALITY_CODE | Sample Summary Result | DATA_QUALITY_CODE |
| 8. | SummaryResult SummaryResultDetails MonitoringPeriodBeginDate | B_MONITORING_PERIOD_START_DATE | TMNMPRD.BEGIN_DATE | Monitoring Period | BEGIN_DATE (Foreign Key to SBS Sample Analytical Result or Reference Key to Sample Summary) |
| 9. | SummaryResult SummaryResultDetails MonitoringPeriodEndDate | B_MONITORING_PERIOD_END_DATE | TMNMPRD.END_DATE | Monitoring Period | END_DATE (Foreign Key to SBS Sample Analytical Result or Reference Key to Sample Summary) |
| 10. | SummaryResult SummaryResultDetails SummaryResultsType | B_RESULTS_TYPE | TSASSR.TYPE_CODE | Sample Summary Result | TYPE_CODE |
| 11. | SummaryResult SummaryResultDetails CountQuantity | B_COUNT_QUANTITY | TSASSR.COUNT_QTY | Sample Summary Result | COUNT_QTY |
| 12. | SummaryResult SummaryResultDetails SummaryMeasurement MeasurementValue | B_MEASURE | TSASSR.MEASURE | Sample Summary Result | MEASURE MEASURE_TEXT |
| 13. | SummaryResult SummaryResultDetails SummaryMeasurement MeasurementUnit | B_MEASURE_UNIT_CODE | TSASSR.UOM_CODE | Sample Summary Result | UOM_CODE |
| 14 | SummaryResult SummaryResultDetails RecordID | B_XMLDOC_RECORD_ID | n/a | n/a | n/a |

## Sample Summary Result Definitions

This section defines the staging table/ column for the Sample Summary business object and also mentions any business rules associated with it.

| SAMPLE SUMMARY RESULT DEFINITIONS |  |  |
| --- | --- | --- |
| LINE NO (field) | FIELD NAME | SAMPLE SUMMARY RESULT |
| 1. | TMGJOB_IS_NUMBER | The identification number of the job. |
| 2. | B_TMGSSR_IS_NUMBER | A system_generated value which uniquely identifies the entity occurrence. |
| 3. | B_STATUS | The current status of the record. <SPACES> = unprocessed, C = processed w/ changes, P = processed , R = rejected |
| 4. | B_PWS_NUMBER | Reference field - The Public Water System (PWS) Identification number. This field is mandatory and must be identical to the B_WATER_SYSTEM_ NUMBER in the Sample Summary. |
| 5. | B_WSF_STATE_ASGN_ID | Reference field - The Water System Facility State Identification number. This field is mandatory and must be identical to the B_WSF_STATE_ASGN_ID in the Sample Summary. |
| 6. | B_ANALYTE_CODE | Analyte Code for the result to be assessed or the Sample Summary to be referenced. |
| 7. | B_DATA_QUALITY | Code indicating whether or not the analytical result meets established data quality criteria. |
| 8. | B_MONITORING_PERIOD_START_DATE | Start date of the monitoring period to which the analytical result or the referenced sample summary is assigned. The monitoring period must be valid for the Water System that collected the sample and the rule against which the result is assessed. |
| 9. | B_MONITORING_PERIOD_END_DATE | End date of the monitoring period to which the analytical result or the referenced sample summary is assigned. The monitoring period must be valid for the Water System that collected the sample and the rule against which the result is assessed. |
| 10. | B_RESULTS_TYPE | List of possible types of Sample Summary results. |
| 11. | B_COUNT_QUANTITY | Count of each type of result within the Sample Summary. |
| 12. | B_MEASURE | Measure value that represents the result obtained from a sample analysis. |
| 13. | B_MEASURE_UNIT_CODE | Unit of measure associated with the Measure. |
| 14. | B_XMLDOC_RECORD_ID | A number used to uniquely identify a record within an xml document. |

# SCHEMA MAPPING FOR MDBP_SUMMARY BUSINESS OBJECT

Schema SDWIS_MDBPSummary_v3.0.xsd maps to the MDBP Summary business unit.

The MDBP summary has nine sections. The first six sections describe the staging table/column, their length optionality

and the business rules for the six different types of MDBP Summaries:

Generic or User Defined

Combined Filter Turbidity

Individual Filter Turbidity

Minimal Residual Disinfection

Chlorine Dioxide/ Chorite Entry point

Chlorine/Chloramine MRDL Summaries (MRDL)

The following chart provides an overview of the different types of MDBP Summaries.

|  | Summary Type | Analtye Code | WSF Type | Sampling Point | % Of Sample Exceeding | RAA, MPA and UOMs | Level Non-Compliance Basis |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Generic or User Defined |  | Any |  |  |  |  | Single Sample Exceeding |
| Combined Filter Turbidity | AVGT MAXT | 0100 | TP | May Value | Do Not Value | Do Not Value | Single Sample Exceeding |
|  | 95PT | 0100 | TP | May Value | Value | Do Not Value | More than 5.49% of Sample Exceeding |
| Individual Filter Turbidity | IFT | 0100 | TP | Do Not Value | Do Not Value | Do Not Value | Inappropriate Answer to Questions |
| Minimal Residual Disinfection | EPRD SERD | 0999 or 1006 | TP | May Value | Do Not Value | Do Not Value | Single Sample Exceeding |
|  | DSRD | 0999 or 1006 | DS | Do not Value | Value | Do Not Value | More than 5.49% of Sample Exceeding Two Consecutive Months |
|  | SDRD | 0999 or 1006 | DS | Do not Value | Value | Do Not Value | Single Sample Exceeding |
| Chlorine Dioxide/ Chorite Entry point | CLO2 | 1008 | TP | May Value | Do Not Value | Do Not Value | Single Sample Exceeding |
|  | CLO3 | 1009 | TP | May Value | Do Not Value | Do Not Value | Single Sample Exceeding |
| Chlorine/Chloramine MRDL Summaries (MRDL) | MRDL | 0999 or 1006 | DS | Do Not Value | Do Not Value | Value | Running Annual Average (RAA) exceeds level in Facility Analyte Level (FANL) |

## Schema Changes

None this release.

## MDBP: Generic or User-Defined Summaries (OTHR or State-Owned) Staging Table Structure and Business Rules

This section lists the staging table/column name and its domain, size, optionality, and the business rules associated with MDBP Generic or User Defined Summaries.

The seventh section describes the Enumerated values for the MDBP summary elements and the business rules. The eighth section describes the mapping between the staging table/columns, SDWIS State table/columns and the schema elements in the SDWIS_edwr.xsd schema.

| MDBP SUMMARY-Generic |  |  |  |  |
| --- | --- | --- | --- | --- |
| LINE NO (field) | STAGING TABLE ( TMGMDBPS ) COLUMN NAME | DOMAIN | SIZE | OPTIONALITY |
| 1. | TMGJOB_IS_NUMBER | N | 10 | MANDATORY. Populated by the XML Sampling Application. |
| 2. | B_TMGMDBPS_IS_NUMBER | N | 7 | MANDATORY. Populated by the XML Sampling Application. |
| 3. | B_STATUS | AN | 1 | OPTIONAL. Populated by the XML Sampling Application. |
| 4. * | B_MDBP_SUMMARY_TYPE | AN | 4 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8(B_MON_PERIOD_BEG_DATE), field 9(B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. For a MDBP Summary Type, see federally defined list in the permitted values section of this structure set. For a state-defined MDBP Summary type, the type must already exist in the database. System Administrators may add state-defined MDBP Summary types to Code Name TSAMDBPS1 using the online Permitted Values function in the System Administration component. |
| 5. | B_PWS_NUMBER | AN | 9 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8(B_MON_PERIOD_BEG_DATE), field 9(B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. |
| 6. | B_WSF_STATE_ASGN_ID | AN | 12 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. |
| 7. * | B_ANALYTE_CODE | AN | 4 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. |
| 8. | B_MON_PERIOD_BEG_DATE | DT | 8 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. |
| 9. | B_MON_PERIOD_END_DATE | DT | 8 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. |
| 10. | B_SAMPLING_POINT | AN | 12 | OPTIONAL - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. Must value if need to enter separate summaries for each sampling point. For example, if there are two entry points from one treatment plant. Recommend value when all samples NOT collected at same point. |
| 11. | B_LAB_ID_NUMBER | AN | 10 | OPTIONAL |
| 12. | B_CERT_AGENCY | AN | 10 | CONDITIONALLY MANDATORY – Required if the B_LAB_ID_NUMBER is valued. |
| 18. | B_REPORTED_DATE | DT | 8 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. If date exists and it is a future date, the software will set to the current date. |
| 19. | B_SAMPLES_REQUIRED | N | 5 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. |
| 20. | B_SAMPLES_COLLECT | N | 5 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. |
| 21. * | B_MR_COMPLIANCE_IND | AN | 3 | For any user-defined type (where permitted value for MDBP sample summary type is not federally-owned), If not valued and field 19 (B_SAMPLES_REQUIRED) and field 20 (B_SAMPLES_COLLECT) are valued calculate and value z = (x/y) * 100; where x= B_SAMPLES_REQUIRED and y=B_SAMPLES_COLLECT. If z>=100, default to “Yes”, If z<=,89.9 default to No Major, otherwise default to No Minor. For any user-defined type (where permitted value for MDBP sample summary type is not federally-owned), If field 19 (B_SAMPLES_REQUIRED), field 20 (B_SAMPLES_COLLECT), and field 21 (B_MR_COMPLIANCE_IND) are valued for any summary type, software will evaluate M&R Compliance and update and flag if inconsistent. Calculate and value z = (x/y) * 100; where x= B_SAMPLES_REQUIRED and y=B_SAMPLES_COLLECT. If z>=100, calculated indicator is Yes”, if z<=89.9, set calculated indicator to No Major, otherwise default to No Minor. If field 19 (B_SAMPLES_REQUIRED) > field 20 (B_SAMPLES_COLLECT), - if field 21 (B_MR_COMPLIANCE_IND) equals “Yes”, set value to calculated indicator and issue message 332 - if field 21 (B_MR_COMPLIANCE_IND) equals “No Minor”, and - Calculated indicator equals “Yes”, set value to “Yes” and issue message 333. - Calculated indicator equals “No Major”, set value to “No Major” and issue message 334. - if field 21 (B_MR_COMPLIANCE_IND) equals “No Major”, and - Calculated indicator equals “Yes”, set value to “Yes” and issue message 333. - Calculated indicator equals “No Minor”, set value to “No Minor” and issue message 334. If field 19 (B_SAMPLES_REQUIRED) <= field 20 (B_SAMPLES_COLLECT), - if calculated indicator equals “Yes”, - if field 21 (B_MR_COMPLIANCE_IND) equals “No Minor”, set value to “Yes” and issue message 334 - if field 21 (B_MR_COMPLIANCE_IND) equals “No Major”, set value to “Yes” and issue message 334. |
| 22. | B_TOTAL_SAMPLES_BEYOND_MSR_LVL | N | 5 | OPTIONAL |
| 23. | B_PERCENT_SAMPLES_BEYOND_MSR_LVL | N | 6(3) | OPTIONAL - If valued, value must be convertible to a numeric within the range of (6(3)). Developer note: The software uses the number supplied in this field to value both MDBP Summary PERCENT_SAMPLES_BEYOND_MSR_LVL and MDBP Summary PERCENT_SAMPS_BYND_MSR_LVL_TXT. |
| 24. | B_HIGHEST_MEASURE | N | 5(3) | OPTIONAL - If valued, value must be convertible to a numeric within the range of (5(3)). Developer note: The software uses the number supplied in this field to value both MDBP Summary HIGHEST_MEASURE and MDBP Summary HIGHEST_MSR_TXT. |
| 25. * | B_LEVEL_COMPLIANCE_INDICATOR | AN | 1 | OPTIONAL - Must value for level compliance determination to be made. If not valued, calculate and value, if field 22 (B_TOTAL_SAMPLES_BEYOND_MSR_LVL) is greater than= zero, “No,” else “Yes.” If field 25 (B_LEVEL_COMPLIANCE_INDICATOR) equals “Yes” and field 22 (B_TOTAL_SAMPLES_BEYOND_MSR_LVL) is greater than zero, value will be changed to “No” and message 338 will be issued. If field 25 (B_LEVEL_COMPLIANCE_INDICATOR) equals “No” and field 22 (B_TOTAL_SAMPLES_BEYOND_MSR_LVL) equals zero, value will be changed to “Yes” and message 338 will be issued. |
| 26. | B_MONITORING_PERIOD_AVG_MEASURE | N | 7(4) | OPTIONAL - If valued, value must be convertible to a numeric within the range of (7(4)). Developer note: The software uses the number supplied in this field to value both MDBP Summary MONITORING_PERIOD_AVG_MEASURE and MDBP Summary MONITORING_PERIOD_AVG_MSR_TXT. |
| 27. * | B_MPA_UOM_CODE | AN | 9 | CONDITIONALLY MANDATORY - If field 26 (B_MONITORING_PERIOD_AVG_MEASURE) is valued, field 27 (B_MPA_UOM_CODE) must be valued. |
| 28. | B_RUNNING_ANNUAL_AVERAGE_MEASURE | N | 7(4) | OPTIONAL - If valued, value must be convertible to a numeric within the range of (7(4)). Developer note: The software uses the number supplied in this field to value both MDBP Summary RUNNING_ANNUAL_AVERAGE_MEASURE and MDBP SummaryRUNNING_ANNUAL_AVERAGE_MSR_TXT. |
| 29. * | B_RAA_UOM_CODE | AN | 9 | CONDITIONALLY MANDATORY - If field 28 (B_RUNNING_ANNUAL_AVERAGE_MEASURE) is valued, field 29 (B_RAA_UOM_CODE) must be valued. |
| 30. * | B_Q1_IFT_MONITORED_INDICATOR | AN | 1 | OPTIONAL |
| 31. * | B_Q2_IFT_RECORDED_INDICATOR | AN | 1 | OPTIONAL |
| 32. * | B_Q3_IFT_EQUIPMENT_INDICATOR | AN | 1 | OPTIONAL |
| 33. * | B_Q4_IFT_GREATER_1_0_INDICATOR | AN | 1 | OPTIONAL |
| 34. * | B_Q5_IFT_GREATER_0_5_INDICATOR | AN | 1 | OPTIONAL |
| 35. * | B_Q6_IFT_GREATER_1_DUR_3_MON_IND | AN | 1 | OPTIONAL |
| 36. * | B_Q7_IFT_GREAT_2_0_DUR_2_MON_IND | AN | 1 | OPTIONAL |
| 37. | B_COMMENT_TEXT | ANmc | 2000 | OPTIONAL |
| 38. | B_XMLDOC_RECORD_ID | N | 7 | OPTIONAL |

Due to the complex nature of MDBP Summaries and their relationships to Facility Analyte Levels (FANL) and CDS Compliance determination, it is recommended that those three areas of the User’s Guide be reviewed throughly prior to creation of MDBP Summary XML documents for migration.

Every MDBP Summary must be associated with a Facility Analyte Level (FANL).  If a FANL that matches the information supplied in the structure set cannot be found, the MDBP Summary will be rejected.

Overlap check for a Summary is on the combination of:

- The values in the FANL Reference Group fields.

- AND the Monitoring Period specified in fields 8 and 9 (which is also used to retrieve the correct FANL).

- AND the Sampling Point.  In this definition, a null value for field 10 (B_Sampling_Point) is effectively a value in and of itself.

- Additional notes:

- In addition to retrieving the correct Monitoring Period, field 8, B_MON_PERIOD_BEG_DATE, and field 9, B_MON_PERIOD_END_DATE, are used to retrieve the correct FANL.  The software identifies the FANL where the Effective Period Begin and End dates cover the specified Monitoring Period Begin Date.  It is recommended that FANL Effective Begin and End dates correspond to Monitoring Period Begin and End dates.

- When the summary type is IFT, the software will associate the summary to a FANL whose MDBP Summary Type is 95PT and whose INDIV_FILTER_MNTRG_REQD_FLAG is “Y.”

- Even though the values in the fields 10, 26, and 27 will not show up in the Water Treatment Summary pages, these fields can be viewed in the General MDBP Summary Maintenance page.

## MDBP: Combined Filter Turbidity Summaries (AVGT, MAXT, 95PT) Staging Table Structure and Business Rules

This section lists the staging table/column name and its domain, size, optionality, and the business rules associated with MDBP Combined Filter Turbidity Summaries.

- The values in the FANL Reference Group fields.

- AND the Monitoring Period specified in fields 8 and 9 (which is also used to retrieve the correct FANL).

- AND the Sampling Point.  In this definition, a null value for field 10 (B_Sampling_Point) is effectively a value in and of itself.

- Additional notes:

- In addition to retrieving the correct Monitoring Period, field 8, B_MON_PERIOD_BEG_DATE, and field 9, B_MON_PERIOD_END_DATE, are used to retrieve the correct FANL.  The software identifies the FANL where Effective Period Begin and End dates cover the specified Monitoring Period Begin Date.  It is recommended that FANL Effective Begin and End dates correspond to Monitoring Period Begin and End dates.

- When the summary type is IFT, the software will associate the summary to a FANL whose MDBP Summary Type is 95PT and whose INDIV_FILTER_MNTRG_REQD_FLAG is “Y.”

- Even though the Values in the fields 10, 26, and 27 will not show up in the Water Treatment Summary pages these fields can be viewed in the General MDBP Summary Maintenance pages.

| MDBP SUMMARY-AVGT, MAXT, 95PT |  |  |  |  |
| --- | --- | --- | --- | --- |
| LINE NO (field) | STAGING TABLE ( TMGMDBPS ) COLUMN NAME | DOMAIN | SIZE | OPTIONALITY |
| 1. | TMGJOB_IS_NUMBER | N | 10 | MANDATORY. Populated by the XML Sampling Application. |
| 2. | B_TMGMDBPS_IS_NUMBER | N | 7 | MANDATORY. Populated by the XML Sampling Application. |
| 3. | B_STATUS | AN | 1 | OPTIONAL. Populated by the XML Sampling Application. |
| 4. * | B_MDBP_SUMMARY_TYPE | AN | 4 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. AVGT, MAXT, 95PT |
| 5. | B_PWS_NUMBER | AN | 9 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. |
| 6. | B_WSF_STATE_ASGN_ID | AN | 12 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. Recommend WSF of type TP - Treatment Plant. |
| 7. * | B_ANALYTE_CODE | AN | 4 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 1 - 4), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. Must be Turbidity 0100. |
| 8. | B_MON_PERIOD_BEG_DATE | DT | 8 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. |
| 9. | B_MON_PERIOD_END_DATE | DT | 8 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. |
| 10. | B_SAMPLING_POINT | AN | 12 | OPTIONAL - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. Must value if need to enter separate summaries for each sampling point. For example, if there are two entry points from one treatment plant. Recommend value when all samples NOT collected at same point. |
| 11. | B_LAB_ID_NUMBER | AN | 10 | OPTIONAL |
| 12. | B_CERT_AGENCY | AN | 10 | CONDITIONALLY MANDATORY – Required if the B_LAB_ID_NUMBER is valued. |
| 18. | B_REPORTED_DATE | DT | 8 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. If date exists and it is a future date, the software will set to the current date. |
| 19. | B_SAMPLES_REQUIRED | N | 5 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. |
| 20. | B_SAMPLES_COLLECT | N | 5 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. |
| 21. * | B_MR_COMPLIANCE_IND | AN | 3 | OPTIONAL - Must value for compliance determinations to be made for AVGT and 95PT. For type AVGT, 95PT, or MAXT If not valued and field 19 (B_SAMPLES_REQUIRED) and field 20 (B_SAMPLES_COLLECT) are valued calculate and value z = (x/y) * 100; where x= B_SAMPLES_REQUIRED and y=B_SAMPLES_COLLECT. If z>=100, default to “Yes”, If z<=89.9, default to No Major, otherwise default to No Minor. For type AVGT, 95PT, or MAXT, If field 19 (B_SAMPLES_REQUIRED), field 20 (B_SAMPLES_COLLECT), and field 21 (B_MR_COMPLIANCE_IND) are valued for any summary type, software will evaluate M&R Compliance and update and flag if inconsistent. Calculate and value z = (x/y) * 100; where x= B_SAMPLES_REQUIRED and y=B_SAMPLES_COLLECT. If z>=100, calculated indicator is Yes”, if z<=89.9, set calculated indicator to No Major, otherwise default to No Minor. If field 19 (B_SAMPLES_REQUIRED) > field 20 (B_SAMPLES_COLLECT), - if field 21 (B_MR_COMPLIANCE_IND) equals “Yes”, set value to calculated indicator and issue message 332 - if field 21 (B_MR_COMPLIANCE_IND) equals “No Minor”, and - Calculated indicator equals “Yes”, set value to “Yes” and issue message 333. - Calculated indicator equals “No Major”, set value to “No Major” and issue message 334. - if field 21 (B_MR_COMPLIANCE_IND) equals “No Major”, and - Calculated indicator equals “Yes”, set value to “Yes” and issue message 333. - Calculated indicator equals “No Minor”, set value to “No Minor” and issue message 334. If field 19 (B_SAMPLES_REQUIRED) <= field 20 (B_SAMPLES_COLLECT), - if calculated indicator equals “Yes”, - if field 21 (B_MR_COMPLIANCE_IND) equals “No Minor”, set value to “Yes” and issue message 334 - if field 21 (B_MR_COMPLIANCE_IND) equals “No Major”, set value to “Yes” and issue message 334. |
| 22. | B_TOTAL_SAMPLES_BEYOND_MSR_LVL | N | 5 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. |
| 23. | B_PERCENT_SAMPLES_BEYOND_MSR_LVL | N | 6(3) | OPTIONAL - If valued, value must be convertible to a numeric within the range of (6(3)). Developer note: The software uses the number supplied in this field to value both MDBP Summary PERCENT_SAMPLES_BEYOND_MSR_LVL and MDBP Summary PERCENT_SAMPS_BYND_MSR_LVL_TXT. Recommend value with 95PT type summary only. If not valued for summary type 95PT and field 20 (B_SAMPLES_COLLECT) and field 22 (B_TOTAL_SAMPLES_BEYOND_MSR_LVL) valued, calculate and value z = (x/y) * 100; where x=TOTAL_SAMPLES_BEYOND_MSR_LVL and y=B_SAMPLES_COLLECT. |
| 24. | B_HIGHEST_MEASURE | N | 5(3) | OPTIONAL - If valued, value must be convertible to a numeric within the range of (5(3)). Developer note: The software uses the number supplied in this field to value both MDBP Summary HIGHEST_MEASURE and MDBP Summary HIGHEST_MSR_TXT. Recommend value with MAXT type summary only (MAXT summary must be valued to satisfy Consumer Confidence Reporting - CCR requirements). If not valued for type MAXT, software will flag the summary. Use this field with summary type MAXT to enter the maximum two-day average for turbidity under the old Turbidity rule. |
| 25. * | B_LEVEL_COMPLIANCE_INDICATOR | AN | 1 | OPTIONAL - Must value for compliance determination to be made. If not valued, calculate and value. For type AVGT or MAXT if field 22 (B_TOTAL_SAMPLES_BEYOND_MSR_LVL is greater than 0 value “No,” else “Yes.” For type 95PT, if field 23 (B_PERCENT_SAMPLES_BEYOND_MSR_LVL) greater than 5.49%, value “No,” else “Yes.” For type AVGT or MAXT, if field 22 (B_TOTAL_SAMPLES_BEYOND_MSR_LVL ) and field 25 (B_LEVEL_COMPLIANCE_INDICATOR) are valued, software will evaluate level compliance and update and flag if inconsistent. If field 25 (B_LEVEL_COMPLIANCE_INDICATOR) equals “Yes” and field 22 (B_TOTAL_SAMPLES_BEYOND_MSR_LVL) is greater than zero, value will be changed to “No” and message 338 will be issued. If field 25 (B_LEVEL_COMPLIANCE_INDICATOR) equals “No” and field 22 (B_TOTAL_SAMPLES_BEYOND_MSR_LVL) equals zero, value will be changed to “Yes” and message #338 will be issued. For type 95PT, if field 23 (B_PERCENT_SAMPLES_BEYOND_MSR_LVL) and field 25 (B_LEVEL_COMPLIANCE_INDICATOR) are valued, software will evaluate level compliance and update and flag if inconsistent. If field 23 (B_PERCENT_SAMPLES_BEYOND_MSR_LVL) is greater than 5.49% and field 25 (B_LEVEL_COMPLIANCE_INDICATOR) equals “Yes,” value will be changed to “No” and summary will be flagged and message #338 will be issued. If field 23 (B_PERCENT_SAMPLES_BEYOND_MSR_LVL) is less than or equal to 5.49% and field 25 (B_LEVEL_COMPLIANCE_INDICATOR) equals “No,” value will be changed to “Yes” and summary will be flagged and message #338 will be issued. |
| 26. | B_MONITORING_PERIOD_AVG_MEASURE | N | 7(4) | OPTIONAL - If entering the average turbidity for a month reported under the old Turbidity rule, use this field and the next to record the average. These values will be stored but not displayed. |
| 27. | B_MPA_UOM_CODE | AN | 9 | OPTIONAL - If entering the average turbidity for a month reported under the old Turbidity rule, use this field and the previous to record the average. These values will be stored but not displayed. |
| 28. | B_RUNNING_ANNUAL_AVERAGE_MEASURE | N | 7(4) | NOT USED (Blank Fill) |
| 29. | B_RAA_UOM_CODE | AN | 9 | NOT USED (Blank Fill) |
| 30. | B_Q1_IFT_MONITORED_INDICATOR | AN | 1 | NOT USED (Blank Fill) |
| 31. | B_Q2_IFT_RECORDED_INDICATOR | AN | 1 | NOT USED (Blank Fill) |
| 32. | B_Q3_IFT_EQUIPMENT_INDICATOR | AN | 1 | NOT USED (Blank Fill) |
| 33. | B_Q4_IFT_GREATER_1_0_INDICATOR | AN | 1 | NOT USED (Blank Fill) |
| 34. | B_Q5_IFT_GREATER_0_5_INDICATOR | AN | 1 | NOT USED (Blank Fill) |
| 35. | B_Q6_IFT_GREATER_1_DUR_3_MON_IND | AN | 1 | NOT USED (Blank Fill) |
| 36. | B_Q7_IFT_GREATER_2_0_DUR_2_MON_IND | AN | 1 | NOT USED (Blank Fill) |
| 37. | B_COMMENTS_TEXT | ANmc | 2000 | OPTIONAL |
| 38. | B_XMLDOC_RECORD_ID | N | 7 | OPTIONAL |

Due to the complex nature of MDBP Summaries and their relationships to Facility Analyte Levels (FANL) and CDS Compliance determination, it is recommended that those three areas of the User’s Guide be reviewed throughly prior to creation of MDBP Summary XML documents for migration.

Every MDBP Summary must be associated with a Facility Analyte Level (FANL).  If a FANL that matches the information supplied in the structure set cannot be found, the MDBP Summary will be rejected.

Overlap check for a Summary is on the combination of:

- The values in the FANL Reference Group fields.

- AND the Monitoring Period specified in fields 8 and 9 (which is also used to retrieve the correct FANL).

- AND the Sampling Point.  In this definition, a null value for field 10 (B_Sampling_Point) is effectively a value in and of itself.

- Additional notes:

- In addition to retrieving the correct Monitoring Period, field 8, B_MON_PERIOD_BEG_DATE, and field 9, B_MON_PERIOD_END_DATE, are used to retrieve the correct FANL.  The software identifies the FANL where the Effective Period Begin and End dates cover the specified Monitoring Period Begin Date.  It is recommended that FANL Effective Begin and End dates correspond to Monitoring Period Begin and End dates.

- When the summary type is IFT, the software will associate the summary to a FANL whose MDBP Summary Type is 95PT and whose INDIV_FILTER_MNTRG_REQD_FLAG is “Y.

- Even though the Values in the fields 10, 26, and 27 will not show up in the Water Treatment Summary pages these fields can be viewed in the General MDBP Summary Maintenance pages.

## MDBP: Individual Filter Turbidity (IFT) Staging Table Structure and Business

This section lists the staging table/column name and its domain, size, optionality, and the business rules associated with MDBP Individual Filter Turbidity Summaries.

| MDBP SUMMARY-IFT |  |  |  |  |
| --- | --- | --- | --- | --- |
| LINE NO (field) | STAGING TABLE ( TMGMDBPS ) COLUMN NAME | DOMAIN | SIZE | OPTIONALITY |
| 1. | TMGJOB_IS_NUMBER | N | 10 | MANDATORY. Populated by the XML Sampling Application. |
| 2. | B_TMGMDBPS_IS_NUMBER | N | 7 | MANDATORY. Populated by the XML Sampling Application. |
| 3. | B_STATUS | AN | 1 | OPTIONAL. Populated by the XML Sampling Application. |
| 4. * | B_MDBP_SUMMARY_TYPE | AN | 4 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. IFT |
| 5. | B_PWS_NUMBER | AN | 9 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. |
| 6. | B_WSF_STATE_ASGN_ID | AN | 12 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. Recommend WSF of type “TP” Treatment Plant. |
| 7. * | B_ANALYTE_CODE | AN | 4 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. Must be Turbidity 0100. |
| 8. | B_MON_PERIOD_BEGIN_DATE | DT | 8 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. |
| 9. | B_MON_PERIOD_END_DATE | DT | 8 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. |
| 10. | B_SAMPLING_POINT | AN | 12 | NOT USED (Blank Fill) |
| 11. | B_LAB_ID_NUMBER | AN | 10 | OPTIONAL |
| 12. | B_CERT_AGENCY | AN | 10 | CONDITIONALLY MANDATORY – Required if the B_LAB_ID_NUMBER is valued. |
| 18. | B_REPORTED_DATE | DT | 8 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. If date exists and it is a future date, the software will set to the current date. |
| 19. | B_SAMPLES_REQUIRED | N | 5 | NOT USED (Blank Fill) |
| 20. | B_SAMPLES_COLLECT | N | 5 | NOT USED (Blank Fill) |
| 21. * | B_MR_COMPLIANCE_IND | AN | 3 | OPTIONAL - Must value for M&R compliance determination to be made. If not valued, calculate and value. For type IFT, If field 30 (B_Q1_IFT_MONITORED_INDICATOR LVL) is “No”, value “No,” ; else if field 31 (B_Q2_IFT_RECORDED_INDICATOR) is “No”, value “No” ; else if field 32 (B_Q3_IFT_EQUIPMENT_INDICATOR) is “Yes”, value “No”; else value “Yes” If field 21 (B_MR_COMPLIANCE_IND) is valued and field 30 (B_Q1_IFT_MONITORED_INDICATOR LVL) or field 31 (B_Q2_IFT_RECORDED_INDICATOR) or field 32 (B_Q3_IFT_EQUIPMENT_INDICATOR) is valued, software will evaluate MR compliance and update and flag if inconsistent. For type IFT, If field 21 (B_MR_COMPLIANCE_IND) equals “Yes”, if field 30 (B_Q1_IFT_MONITORED_INDICATOR LVL) is “No” or if field 31 (B_Q2_IFT_RECORDED_INDICATOR) is “No” or if field 32 (B_Q3_IFT_EQUIPMENT_INDICATOR) is “Yes” value will be changed to “No” and summary will be flagged issue message 257. If field 21 (B_MR_COMPLIANCE_IND) equals “No”, if field 30 (B_Q1_IFT_MONITORED_INDICATOR LVL) is “Yes” and field 31 (B_Q2_IFT_RECORDED_INDICATOR) is “Yes” and field 32 (B_Q3_IFT_EQUIPMENT_INDICATOR) is “No” value will be changed to “Yes” and summary will be flagged issue message 257. |
| 22. | B_TOTAL_SAMPLES_BEYOND_MSR_LVL | N | 5 | NOT USED (Blank Fill) |
| 23. | B_PERCENT_SAMPLES_BEYOND_MSR_LVL | N | 6(3) | NOT USED (Blank Fill) |
| 24. | B_HIGHEST_MEASURE | N | 5(3) | NOT USED (Blank Fill) |
| 25. * | B_LEVEL_COMPLIANCE_INDICATOR | AN | 1 | OPTIONAL - Must value for level compliance determination to be made. If not valued, calculate and value. If field 33 (B_Q4_IFT_GREATER_1_0_INDICATOR is “Yes”, value “No,” ; else if field 34 (B_Q5_IFT_GREATER_0_5_INDICATOR) is “Yes”, value “No” ; else if field 35 (B_Q6_IFT_GREATER_1_DUR_3_MON_IND) is “Yes”, value “No”; else if field 36 (B_Q7_IFT_GREATER_2_0_DUR_2_MON_IND) is “Yes”, value “No”; else value “Yes” If field 25 (B_LEVEL_COMPLIANCE_INDICATOR) is valued and field 33 (B_Q4_IFT_GREATER_1_0_INDICATOR) or field 34 (B_Q5_IFT_GREATER_0_5_INDICATOR) or field 35 (B_Q6_IFT_GREATER_1_DUR_3_MON_IND) or field 36 (B_Q7_IFT_GREATER_2_0_DUR_2_MON_IND) is valued, software will evaluate level compliance for inconsistency. If field 25 (B_LEVEL_COMPLIANCE_INDICATOR) equals “Yes,” and field 33 (B_Q4_IFT_GREATER_1_0_INDICATOR)=”Yes” or field 34 (B_Q5_IFT_GREATER_0_5_INDICATOR)=”Yes” or field 35 (B_Q6_IFT_GREATER_1_DUR_3_MON_IND)=”Yes” or field 36 (B_Q7_IFT_GREATER_2_0_DUR_2_MON_IND)=”Yes”, value will be changed to “No” and message 258 will be issued. If field 25 (B_LEVEL_COMPLIANCE_INDICATOR) equals “No” and field 33 (B_Q4_IFT_GREATER_1_0_INDICATOR)=”No”and field 34 (B_Q5_IFT_GREATER_0_5_INDICATOR)=”No”and field 35 (B_Q6_IFT_GREATER_1_DUR_3_MON_IND)=”No”and field 36 (B_Q7_IFT_GREATER_2_0_DUR_2_MON_IND)=”No”, value will be changed to “Yes” and message 258 will be issued. |
| 26. | B_MONITORING_PERIOD_AVG_MEASURE | N | 7(4) | NOT USED (Blank Fill) |
| 27. | B_MPA_UOM_CODE | AN | 9 | NOT USED (Blank Fill) |
| 28. | B_RUNNING_ANNUAL_AVERAGE_MEASURE | N | 7(4) | NOT USED (Blank Fill) |
| 29. | B_RAA_UOM_CODE | AN | 9 | NOT USED (Blank Fill) |
| 30. * | B_Q1_IFT_MONITORED_INDICATOR | AN | 1 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. |
| 31. * | B_Q2_IFT_RECORDED_INDICATOR | AN | 1 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. |
| 32. * | B_Q3_IFT_EQUIPMENT_INDICATOR | AN | 1 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. |
| 33. * | B_Q4_IFT_GREATER_1_0_INDICATOR | AN | 1 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. |
| 34. * | B_Q5_IFT_GREATER_0_5_INDICATOR | AN | 1 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. |
| 35. * | B_Q6_IFT_GREATER_1_DUR_3_MON_IND | AN | 1 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. |
| 36. * | B_Q7_IFT_GREATER_2_0_DUR_2_MON_IND | AN | 1 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. |
| 37. | B_COMMENTS_TEXT | ANmc | 2000 | OPTIONAL |
| 38. | B_XMLDOC_RECORD_ID | N | 7 | OPTIONAL |

Due to the complex nature of MDBP Summaries and their relationships to Facility Analyte Levels (FANL) and CDS Compliance determination, it is recommended that those three areas of the User’s Guide be reviewed throughly prior to creation of MDBP Summary XML	 documents for migration.

Every MDBP Summary must be associated with a Facility Analyte Level (FANL).  If a FANL that matches the information supplied in the structure set cannot be found, the MDBP Summary will be rejected.

Overlap check for a Summary is on the combination of:

- The values in the FANL Reference Group fields.

- AND the Monitoring Period specified in fields 8 and 9 (which is also used to retrieve the correct FANL).

- AND the Sampling Point.  In this definition, a null value for field 10 (B_Sampling_Point) is effectively a value in and of itself.

- Additional notes:

- In addition to retrieving the correct Monitoring Period, field 8, B_MON_PERIOD_BEG_DATE, and field 9, B_MON_PERIOD_END_DATE, are used to retrieve the correct FANL.  The software identifies the FANL where the Effective Period Begin and End dates cover the specified Monitoring Period Begin Date.  It is recommended that FANL Effective Begin and End dates correspond to Monitoring Period Begin and End dates.

- When the summary type is IFT, the software will associate the summary to a FANL whose MDBP Summary Type is 95PT and whose INDIV_FILTER_MNTRG_REQD_FLAG is “Y.”

- Even though the Values in the fields 10, 26, and 27 will not show up in the Water Treatment Summary pages these fields can be viewed in the General MDBP Summary Maintenance pages.

## MDBP: Minimal Residual Disinfection Summaries (EPRD, SERD, DSRD, SDRD) Staging Table Structure and 	Business

This section lists the staging table/column name and its domain, size, optionality, and the business rules associated with MDBP Minimal Residual Disinfection Summaries.

| MDBP SUMMARY- EPRD, SERD, DSRD, SDRD |  |  |  |  |
| --- | --- | --- | --- | --- |
| LINE NO (field) | STAGING TABLE ( TMGMDBPS ) COLUMN NAME | DOMAIN | SIZE | OPTIONALITY |
| 1. | TMGJOB_IS_NUMBER | N | 10 | MANDATORY. Populated by the XML Sampling Application. |
| 2. | B_TMGMDBPS_IS_NUMBER | N | 7 | MANDATORY. Populated by the XML Sampling Application. |
| 3. | B_STATUS | AN | 1 | OPTIONAL. Populated by the XML Sampling Application. |
| 4. * | B_MDBP_SUMMARY_TYPE | AN | 4 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. EPRD, SERD, DSRD, SDRD |
| 5. | B_PWS_NUMBER | AN | 9 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. |
| 6. | B_WSF_STATE_ASGN_ID | AN | 12 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. For summary type EPRD and SERD, should be type “TP” Treatment Plant. For summary type DSRD and SDRD, should be type “DS” Distribution System. |
| 7. * | B_ANALYTE_CODE | AN | 4 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. Must be chlorine-0999 or chloramine-1006. |
| 8. | B_MON_PERIOD_BEG_DATE | DT | 8 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. |
| 9. | B_MON_PERIOD_END_DATE | DT | 8 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. |
| 10. | B_SAMPLING_POINT | AN | 12 | CONDITIONALLY MANDATORY - Must value if need to enter separate summaries for each sampling point. For example, if there are two entry points for one treatment plant. Recommend value when all samples NOT collected at same point Overlap check for MDBP Summary is on the combination of, the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. For summary type EPRD and SERD, must be valued with sampling point of type “EP” Entry Point. For summary type DSRD and SDRD, should not be valued. |
| 11. | B_LAB_ID_NUMBER | AN | 10 | OPTIONAL |
| 12. | B_CERT_AGENCY | AN | 10 | CONDITIONALLY MANDATORY – Required when B_LAB_ID_NUMBER is valued. |
| 18. | B_REPORTED_DATE | DT | 8 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. If date exists and it is a future date, the software will set to the current date. |
| 19. | B_SAMPLES_REQUIRED | N | 5 | OPTIONAL - Recommend value for summary types EPRD or DSRD. |
| 20. | B_SAMPLES_COLLECT | N | 5 | OPTIONAL - Recommend value for summary types EPRD or DSRD. |
| 21. * | B_MR_COMPLIANCE_IND | AN | 3 | OPTIONAL - Must value for M&R compliance determination to be made. Recommend do not value for summary types SERD or SDRD, if do not want to issue both federal and state violations. For type EPRD, DSRD, SDRD, or SERD, If not valued and field 19 (B_SAMPLES_REQUIRED) and field 20 (B_SAMPLES_COLLECT) are valued calculate and value z = (x/y) * 100; where x= B_SAMPLES_REQUIRED and y=B_SAMPLES_COLLECT. If z>=100, default to “Yes”, If z<=89.9, default to No Major, otherwise default to No Minor. For type EPRD, DSRD, SDRD, or SERD, If field 19 (B_SAMPLES_REQUIRED), field 20 (B_SAMPLES_COLLECT), and field 21 (B_MR_COMPLIANCE_IND) are valued for any summary type, software will evaluate M&R Compliance and update and flag if inconsistent. Calculate and value z = (x/y) * 100; where x= B_SAMPLES_REQUIRED and y=B_SAMPLES_COLLECT. If z>=100, calculated indicator is Yes”, if z<=89.9, set calculated indicator to No Major, otherwise default to No Minor. If field 19 (B_SAMPLES_REQUIRED) > field 20 (B_SAMPLES_COLLECT), - if field 21 (B_MR_COMPLIANCE_IND) equals “Yes”, set value to calculated indicator and issue message 332 - if field 21 (B_MR_COMPLIANCE_IND) equals “No Minor”, and - Calculated indicator equals “Yes”, set value to “Yes” and issue message 333. - Calculated indicator equals “No Major”, set value to “No Major” and issue message 334. - if field 21 (B_MR_COMPLIANCE_IND) equals “No Major”, and - Calculated indicator equals “Yes”, set value to “Yes” and issue message 333. - Calculated indicator equals “No Minor”, set value to “No Minor” and issue message 334. If field 19 (B_SAMPLES_REQUIRED) <= field 20 (B_SAMPLES_COLLECT), - if calculated indicator equals “Yes”, - if field 21 (B_MR_COMPLIANCE_IND) equals “No Minor”, set value to “Yes” and issue message 334 - if field 21 (B_MR_COMPLIANCE_IND) equals “No Major”, set value to “Yes” and issue message 334. |
| 22. | B_TOTAL_SAMPLES_BEYOND_MSR_LVL | N | 5 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. |
| 23. | B_PERCENT_SAMPLES_BEYOND_MSR_LVL | N | 6(3) | OPTIONAL - If valued, value must be convertible to a numeric within the range of (6(3)). Developer note: The software uses the number supplied in this field to value both MDBP Summary PERCENT_SAMPLES_BEYOND_MSR_LVL and MDBP Summary PERCENT_SAMPS_BYND_MSR_LVL_TXT. For summary type EPRD and SERD should not be valued. For summary type DSRD and SDRD, should be valued. If not valued for summary type DSRD or SDRD and field 20 (B_SAMPLES_COLLECT) and field 22 (B_TOTAL_SAMPLES_BEYOND_MSR_LVL) valued, calculate and value z = (x/y) * 100; where x=B_TOTAL_SAMPLES_BEYOND_MSR_LVL and y=B_SAMPLES_COLLECT. |
| 24. | B_HIGHEST_MEASURE | N | 5(3) | OPTIONAL - If valued, value must be convertible to a numeric within the range of (5(3)). Developer note: The software uses the number supplied in this field to value both MDBP Summary HIGHEST_MEASURE and MDBP Summary HIGHEST_MSR_TXT. |
| 25. * | B_LEVEL_COMPLIANCE_INDICATOR | AN | 1 | OPTIONAL - Must value for compliance determination to be made. If not valued, calculate and value. For type EPRD, SERD and SDRD, if field 22 (B_TOTAL_SAMPLES_BEYOND_MSR_LVL) is greater than zero, value “No,” else “Yes.” For type DSRD, if field 23 (B_PERCENT_SAMPLES_BEYOND_MSR_LVL) is greater than 5.49%, value “No” and issue message 347 else “Yes.” For type EPRD, SERD and SDRD, if field 22 (B_TOTAL_SAMPLES_BEYOND_MSR_LVL) and field 25 (B_LEVEL_COMPLIANCE_INDICATOR) are valued, software will evaluate level compliance for inconsistency. If field 25 (B_LEVEL_COMPLIANCE_INDICATOR) equals “Yes” and field 22 (B_TOTAL_SAMPLES_BEYOND_MSR_LVL) is greater than zero, value will be changed to “No” and message 338 will be issued. If field 25 (B_LEVEL_COMPLIANCE_INDICATOR) equals “No” and field 22 (B_TOTAL_SAMPLES_BEYOND_MSR_LVL) equals zero, value will be changed to “Yes” and message 338 will be issued. For type DSRD, If field 22 (B_TOTAL_SAMPLES_BEYOND_MSR_LVL) and field 25 (B_LEVEL_COMPLIANCE_INDICATOR) are valued, software will evaluate level compliance for inconsistency. If field 22 field 23 (B_PERCENT_SAMPLES_BEYOND_MSR_LVL) is greater than 5.49%, and field 25 (B_LEVEL_COMPLIANCE_INDICATOR) equals “Yes,” value will be changed to “No” and summary will be flagged. Message # will note Compliance Officer must review previous monitoring period. |
| 26. | B_MONITORING_PERIOD_AVG_MEASURE | N | 7(4) | NOT USED (Blank Fill) |
| 27. | B_MPA_UOM_CODE | AN | 9 | NOT USED (Blank Fill) |
| 28. | B_RUNNING_ANNUAL_AVERAGE_MEASURE | N | 7(4) | NOT USED (Blank Fill) |
| 29. | B_RAA_UOM_CODE | AN | 9 | NOT USED (Blank Fill) |
| 30. | B_Q1_IFT_MONITORED_INDICATOR | AN | 1 | NOT USED (Blank Fill) |
| 31. | B_Q2_IFT_RECORDED_INDICATOR | AN | 1 | NOT USED (Blank Fill) |
| 32. | B_Q3_IFT_EQUIPMENT_INDICATOR | AN | 1 | NOT USED (Blank Fill) |
| 33. | B_Q4_IFT_GREATER_1_0_INDICATOR | AN | 1 | NOT USED (Blank Fill) |
| 34. | B_Q5_IFT_GREATER_0_5_INDICATOR | AN | 1 | NOT USED (Blank Fill) |
| 35. | B_Q6_IFT_GREATER_1_DUR_3_MON_IND | AN | 1 | NOT USED (Blank Fill) |
| 36. | B_Q7_IFT_GREATER_2_0_DUR_2_MON_IND | AN | 1 | NOT USED (Blank Fill) |
| 37. | B_COMMENTS_TEXT | ANmc | 2000 | OPTIONAL |
| 38. | B_XMLDOC_RECORD_ID | N | 7 | OPTIONAL |

Due to the complex nature of MDBP Summaries and their relationships to Facility Analyte Levels (FANL) and CDS Compliance determination, it is recommended that those three areas of the User’s Guide be reviewed throughly prior to creation of MDBP Summary XML documents for migration.

Every MDBP Summary must be associated with a Facility Analyte Level (FANL).  If a FANL that matches the information supplied in the structure set cannot be found, the MDBP Summary will be rejected.

Overlap check for a Summary is on the combination of:

- The values in the FANL Reference Group fields.

- AND the Monitoring Period specified in fields 8 and 9 (which is also used to retrieve the correct FANL).

- AND the Sampling Point.  In this definition, a null value for field 10 (B_Sampling_Point) is effectively a value in and of itself.

- Additional notes:

- In addition to retrieving the correct Monitoring Period, field 8, B_MON_PERIOD_BEG_DATE, and field 9, B_MON_PERIOD_END_DATE, are used to retrieve the correct FANL.  The software identifies the FANL where the Effective Period Begin and End dates cover the specified Monitoring Period Begin Date.  It is recommended that FANL Effective Begin and End dates correspond to Monitoring Period Begin and End dates.

- When the summary type is IFT, the software will associate the summary to a FANL whose MDBP Summary Type is 95PT and whose INDIV_FILTER_MNTRG_REQD_FLAG is “Y.”

- Even though the Values in the fields 10, 26, and 27 will not show up in the Water Treatment Summary pages these fields can be viewed in the General MDBP Summary Maintenance pages.

## MDBP:  Chlorine Dioxide/Chlorite Entry Point Summaries (CLO2, CLO3) Staging Table Structure and Business

This section lists the staging table/column name and its domain, size, optionality, and the business rules associated with MDBP Chlorine Dioxide/ Chlorite Entry Point Summaries.

| MDBP SUMMARY-CLO2, CLO3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| LINE NO (field) | STAGING TABLE ( TMGMDBPS ) COLUMN NAME | DOMAIN | SIZE | OPTIONALITY |
| 1. | TMGJOB_IS_NUMBER | N | 10 | MANDATORY. Populated by the XML Sampling Application. |
| 2. | B_TMGMDBPS_IS_NUMBER | N | 7 | MANDATORY. Populated by the XML Sampling Application. |
| 3. | B_STATUS | AN | 1 | OPTIONAL. Populated by the XML Sampling Application. |
| 4. * | B_MDBP_SUMMARY_TYPE | AN | 4 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 5 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. For a MDBP Summary Type, see federally defined list in the permitted values section of this structure set. For a state-defined MDBP Summary type, the type must already exist in the database. System Administrators may add state-defined MDBP Summary types to Code Name TSAMDBPS1 using the online Permitted Values function in the System Administration component. |
| 5. | B_PWS_NUMBER | AN | 9 | MANDATORY - Overlap check for MDBP Summary is on the combination of, the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. |
| 6. | B_WSF_STATE_ASGN_ID | AN | 12 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. Should be of type “TP” Treatment Plant. |
| 7. * | B_ANALYTE_CODE | AN | 4 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. Must be chlorine dioxide-1008 or chlorite-1009. |
| 8. | B_MON_PERIOD_BEG_DATE | DT | 8 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. |
| 9. | B_MON_PERIOD_END_DATE | DT | 8 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. |
| 10. | B_SAMPLING_POINT | AN | 12 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. Should be of type “EP” Entry Point. |
| 11. | B_LAB_ID_NUMBER | AN | 10 | OPTIONAL |
| 12. | B_CERT_AGENCY | AN | 10 | CONDITIONALLY MANDATORY – Required when the B_LAB_ID_NUMBER is valued. |
| 18. | B_REPORTED_DATE | DT | 8 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. If date exists and it is a future date, the software will set to the current date. |
| 19. | B_SAMPLES_REQUIRED | N | 5 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. |
| 20. | B_SAMPLES_COLLECT | N | 5 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. |
| 21. * | B_MR_COMPLIANCE_IND | AN | 3 | OPTIONAL - Must value for M&R compliance determination to be made. For type CLO2 or CLO3, If not valued and field 19 (B_SAMPLES_REQUIRED) and field 20 (B_SAMPLES_COLLECT) are valued calculate and value z = (x/y) * 100; where x= B_SAMPLES_REQUIRED and y=B_SAMPLES_COLLECT. If z>=100, default to “Yes”, If z<=89.9, default to No Major, otherwise default to No Minor. For type CLO2 or CLO3, If field 19 (B_SAMPLES_REQUIRED), field 20 (B_SAMPLES_COLLECT), and field 21 (B_MR_COMPLIANCE_IND) are valued for any summary type, software will evaluate M&R Compliance and update and flag if inconsistent. Calculate and value z = (x/y) * 100; where x= B_SAMPLES_REQUIRED and y=B_SAMPLES_COLLECT. If z>=100, calculated indicator is Yes”, if z<=89.9, set calculated indicator to No Major, otherwise default to No Minor. If field 19 (B_SAMPLES_REQUIRED) > field 20 (B_SAMPLES_COLLECT), - if field 21 (B_MR_COMPLIANCE_IND) equals “Yes”, set value to calculated indicator and issue message 332 - if field 21 (B_MR_COMPLIANCE_IND) equals “No Minor”, and - Calculated indicator equals “Yes”, set value to “Yes” and issue message 333. - Calculated indicator equals “No Major”, set value to “No Major” and issue message 334. - if field 21 (B_MR_COMPLIANCE_IND) equals “No Major”, and - Calculated indicator equals “Yes”, set value to “Yes” and issue message 333. - Calculated indicator equals “No Minor”, set value to “No Minor” and issue message 334. If field 19 (B_SAMPLES_REQUIRED) <= field 20 (B_SAMPLES_COLLECT), - if calculated indicator equals “Yes”, - if field 21 (B_MR_COMPLIANCE_IND) equals “No Minor”, set value to “Yes” and issue message 334 - if field 21 (B_MR_COMPLIANCE_IND) equals “No Major”, set value to “Yes” and issue message 334. |
| 22. | B_TOTAL_SAMPLES_BEYOND_MSR_LVL | N | 5 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. |
| 23. | B_PERCENT_SAMPLES_BEYOND_MSR_LVL | N | 6(3) | NOT USED (Blank Fill) |
| 24. | B_HIGHEST_MEASURE | N | 5(3) | OPTIONAL - If valued, value must be convertible to a numeric within the range of (5(3)). Developer note: The software uses the number supplied in this field to value both MDBP Summary HIGHEST_MEASURE and MDBP Summary HIGHEST_MSR_TXT. |
| 25. * | B_LEVEL_COMPLIANCE_INDICATOR | AN | 1 | OPTIONAL - Must value for level compliance determination to be made. If not valued, calculate and value. If field 22 (B_TOTAL_SAMPLES_BEYOND_MSR_LVL) is greater than zero, “No,” else “Yes.” If field 22 (B_TOTAL_SAMPLES_BEYOND_MSR_LVL ) and field 25 (B_LEVEL_COMPLIANCE_INDICATOR) are valued, software will evaluate level compliance and update and flag if inconsistent. If field 25 (B_LEVEL_COMPLIANCE_INDICATOR) equals “Yes” and field 22 (B_TOTAL_SAMPLES_BEYOND_MSR_LVL) is greater than zero, value will be changed to “No” and message 338 will be issued. If field 25 (B_LEVEL_COMPLIANCE_INDICATOR) equals “No” and field 22 (B_TOTAL_SAMPLES_BEYOND_MSR_LVL) equals zero, value will be changed to “Yes” and message 338 will be issued. |
| 26. | B_MONITORING_PERIOD_AVG_MEASURE | N | 7(4) | NOT USED (Blank Fill) |
| 27. | B_MPA_UOM_CODE | AN | 9 | NOT USED (Blank Fill) |
| 28. | B_RUNNING_ANNUAL_AVERAGE_MEASURE | N | 7(4) | NOT USED (Blank Fill) |
| 29. | B_RAA_UOM_CODE | AN | 9 | NOT USED (Blank Fill) |
| 30. | B_Q1_IFT_MONITORED_INDICATOR | AN | 1 | NOT USED (Blank Fill) |
| 31. | B_Q2_IFT_RECORDED_INDICATOR | AN | 1 | NOT USED (Blank Fill) |
| 32. | B_Q3_IFT_EQUIPMENT_INDICATOR | AN | 1 | NOT USED (Blank Fill) |
| 33. | B_Q4_IFT_GREATER_1_0_INDICATOR | AN | 1 | NOT USED (Blank Fill) |
| 34. | B_Q5_IFT_GREATER_0_5_INDICATOR | AN | 1 | NOT USED (Blank Fill) |
| 35. | B_Q6_IFT_GREATER_1_DUR_3_MON_IND | AN | 1 | NOT USED (Blank Fill) |
| 36. | B_Q7_IFT_GREATER_2_0_DUR_2_MON_IND | AN | 1 | NOT USED (Blank Fill) |
| 37. | B_COMMENTS_TEXT | ANmc | 2000 | OPTIONAL |
| 38. | B_XMLDOC_RECORD_ID | N | 7 | OPTIONAL |

Due to the complex nature of MDBP Summaries and their relationships to Facility Analyte Levels (FANL) and CDS Compliance determination, it is recommended that those three areas of the User’s Guide be reviewed throughly prior to creation of MDBP Summary XML documents for migration.

Every MDBP Summary must be associated with a Facility Analyte Level (FANL).  If a FANL that matches the information supplied in the structure set cannot be found, the MDBP Summary will be rejected.

Overlap check for a Summary is on the combination of:

- The values in the FANL Reference Group fields

- AND the Monitoring Period specified in fields 8 and 9 (which is also used to retrieve the correct FANL).

- AND the Sampling Point.  In this definition, a null value for field 10 (B_Sampling_Point) is effectively a value in and of itself.

- Additional notes:

- In addition to retrieving the correct Monitoring Period, field 8, B_MON_PERIOD_BEG_DATE, and field 9, B_MON_PERIOD_END_DATE, are used to retrieve the correct FANL.  The software identifies the FANL where the Effective Period Begin and End dates cover the specified Monitoring Period Begin Date.  It is recommended that FANL Effective Begin and End dates correspond to Monitoring Period Begin and End dates.

- When the summary type is IFT, the software will associate the summary to a FANL whose MDBP Summary Type is 95PT and whose INDIV_FILTER_MNTRG_REQD_FLAG is “Y.”

- Even though the Values in the fields 10, 26, and 27 will not show up in the Water Treatment Summary pages these fields can be viewed in the General MDBP Summary Maintenance pages.

## MDBP: Chlorine/Chloramine MRDL Summaries (MRDL) Staging Table Structure and Business Rules

This section lists the staging table/column name and its domain, size, optionality, and the business rules associated with MDBP Chlorine/Chloramine MRDL Summaries.

| MDBP SUMMARY-MRDL |  |  |  |  |
| --- | --- | --- | --- | --- |
| LINE NO (field) | STAGING TABLE ( TMGMDBPS ) COLUMN NAME | DOMAIN | SIZE | OPTIONALITY |
| 1. | TMGJOB_IS_NUMBER | N | 10 | MANDATORY. Populated by the XML Sampling Application. |
| 2. | B_TMGMDBPS_IS_NUMBER | N | 7 | MANDATORY. Populated by the XML Sampling Application. |
| 3. | B_STATUS | AN | 1 | OPTIONAL. Populated by the XML Sampling Application. |
| 4. * | B_MDBP_SUMMARY_TYPE | AN | 4 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. MRDL |
| 5. | B_PWS_NUMBER | AN | 9 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. |
| 6. | B_WSF_STATE_ASGN_ID | AN | 12 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. Should be of type “DS” Distribution System. |
| 7. * | B_ANALYTE_CODE | AN | 4 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. Must be chlorine-0999 or chloraqmine-1006. |
| 8. | B_MON_PERIOD_BEG_DATE | DT | 8 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. |
| 9. | B_MON_PERIOD_END_DATE | DT | 8 | MANDATORY - Overlap check for MDBP Summary is on the combination of the FANL (as defined by the FANL Reference Group, fields 4 - 7), field 8 (B_MON_PERIOD_BEG_DATE), field 9 (B_MON_PERIOD_END_DATE), and field 10 (B_SAMPLING_POINT). See Note at end of this layout. |
| 10. | B_SAMPLING_POINT | AN | 12 | NOT USED (Blank Fill) |
| 11. | B_LAB_ID_NUMBER | AN | 10 | OPTIONAL |
| 12. | B_CERT_AGENCY | AN | 10 | CONDITIONALLY MANDATORY – Required when B_LAB_ID_NUMBER is valued. |
| 18. | B_REPORTED_DATE | DT | 8 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. If date exists and it is a future date, the software will set to the current date. |
| 19. | B_SAMPLES_REQUIRED | N | 5 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. |
| 20. | B_SAMPLES_COLLECT | N | 5 | OPTIONAL - The SDWIS/STATE team recommends that you value this field. |
| 21. * | B_MR_COMPLIANCE_IND | AN | 3 | OPTIONAL - Must value for M&R compliance determination to be made. For type MRDL, If not valued and field 19 (B_SAMPLES_REQUIRED) and field 20 (B_SAMPLES_COLLECT) are valued calculate and value z = (x/y) * 100; where x= B_SAMPLES_REQUIRED and y=B_SAMPLES_COLLECT. If z>=100, default to “Yes”, If z<, default to No Major, otherwise default to No Minor. If field 19 (B_SAMPLES_REQUIRED), field 20 (B_SAMPLES_COLLECT), and field 21 (B_MR_COMPLIANCE_IND) are valued for any summary type, software will evaluate M&R Compliance and update and flag if inconsistent. Calculate and value z = (x/y) * 100; where x= B_SAMPLES_REQUIRED and y=B_SAMPLES_COLLECT. If z>=100, calculated indicator is Yes”, if z<=89.9, set calculated indicator to No Major, otherwise default to No Minor. For type MRDL, If field 19 (B_SAMPLES_REQUIRED) > field 20 (B_SAMPLES_COLLECT), - if field 21 (B_MR_COMPLIANCE_IND) equals “Yes”, set value to calculated indicator and issue message 332 - if field 21 (B_MR_COMPLIANCE_IND) equals “No Minor”, and - Calculated indicator equals “Yes”, set value to “Yes” and issue message 333. - Calculated indicator equals “No Major”, set value to “No Major” and issue message 334. - if field 21 (B_MR_COMPLIANCE_IND) equals “No Major”, and - Calculated indicator equals “Yes”, set value to “Yes” and issue message 333. - Calculated indicator equals “No Minor”, set value to “No Minor” and issue message 334. If field 19 (B_SAMPLES_REQUIRED) <= field 20 (B_SAMPLES_COLLECT), - if calculated indicator equals “Yes”, - if field 21 (B_MR_COMPLIANCE_IND) equals “No Minor”, set value to “Yes” and issue message 334 - if field 21 (B_MR_COMPLIANCE_IND) equals “No Major”, set value to “Yes” and issue message 334. |
| 22. | B_TOTAL_SAMPLES_BEYOND_MSR_LVL | N | 5 | OPTIONAL |
| 23. | B_PERCENT_SAMPLES_BEYOND_MSR_LVL | N | 6(3) | NOT USED (Blank Fill) |
| 24. | B_HIGHEST_MEASURE | N | 5(3) | OPTIONAL - If valued, value must be convertible to a numeric within the range of (5(3)). Developer note: The software uses the number supplied in this field to value both MDBP Summary HIGHEST_MEASURE and MDBP Summary HIGHEST_MSR_TXT. |
| 25. * | B_LEVEL_COMPLIANCE_INDICATOR | AN | 1 | OPTIONAL - Must value for level compliance determination to be made. If not valued, calculate and value. If field 28 (B_ RUNNING_ANNUAL_AVERAGE_MEASURE) valued and associated FANL (Same water system, water system facility, analyte code and unit of measure (UOM) as submitted Summary and Summary type=”MRDL”and monitoring period for submitted summary fits within FANL effective dates ) can be found , is greater than CONTROL_LVL_MSR in the associated FANL, “No,” else “Yes.” If no FANL located, set to spaces and issue message 348. If field 25 (B_LEVEL_COMPLIANCE_INDICATOR) and field 28 (RUNNING_ANNUAL_AVERAGE_MEASURE valued and associated FANL (Same water system, water system facility, analyte code and unit of measure (UOM) as submitted Summary and Summary type=”MRDL”and monitoring period for submitted summary fits within FANL effective dates ) can be found, software will evaluate level compliance for inconsistency. If field 25 (B_LEVEL_COMPLIANCE_INDICATOR) equals “Yes” and field 28 (RUNNING_ANNUAL_AVERAGE_MEASURE) is greater than CONTROL_LVL_MSR in the associated FANL, value will be changed to “No” and message 348 will be issued. If field 25 (B_LEVEL_COMPLIANCE_INDICATOR) equals “No” and field 28 (RUNNING_ANNUAL_AVERAGE_MEASURE) is less than or equal to CONTROL_LVL_MSR in the associated FANL, value will be changed to “Yes” and message 348 will be issued. |
| 26. | B_MONITORING_PERIOD_AVG_MEASURE | N | 7(4) | OPTIONAL - If valued, value must be convertible to a numeric within the range of (7(4)). Developer note: The software uses the number supplied in this field to value both MDBP Summary MONITORING_PERIOD_AVG_MEASURE and MDBP Summary MONITORING_PERIOD_AVG_MSR_TXT. The SDWIS/STATE team recommends that you value this field. |
| 27. * | B_MPA_UOM_CODE | AN | 9 | CONDITIONALLY MANDATORY - Must be valued if field 26 (MONITORING_PERIOD_AVG_MEASURE) is valued. The only acceptable value is MG/L. |
| 28. | B_RUNNING_ANNUAL_AVERAGE_MEASURE | N | 7(4) | OPTIONAL - If valued, value must be convertible to a numeric within the range of (7(4)). Developer note: The software uses the number supplied in this field to value both MDBP Summary RUNNING_ANNUAL_AVERAGE_MEASURE and MDBP SummaryRUNNING_ANNUAL_AVERAGE_MSR_TXT. The SDWIS/STATE team recommends that you value this field. |
| 29. * | B_RAA_UOM_CODE | AN | 9 | CONDITIONALLY MANDATORY - Must be valued if field 28 (RUNNING_ANNUAL_AVERAGE_MEASURE) is valued. The only acceptable value is MG/L. |
| 30. | B_Q1_IFT_MONITORED_INDICATOR | AN | 1 | NOT USED (Blank Fill) |
| 31. | B_Q2_IFT_RECORDED_INDICATOR | AN | 1 | NOT USED (Blank Fill) |
| 32. | B_Q3_IFT_EQUIPMENT_INDICATOR | AN | 1 | NOT USED (Blank Fill) |
| 33. | B_Q4_IFT_GREATER_1_0_INDICATOR | AN | 1 | NOT USED (Blank Fill) |
| 34. | B_Q5_IFT_GREATER_0_5_INDICATOR | AN | 1 | NOT USED (Blank Fill) |
| 35. | B_Q6_IFT_GREATER_1_DUR_3_MON_IND | AN | 1 | NOT USED (Blank Fill) |
| 36. | B_Q7_IFT_GREATER_2_0_DUR_2_MON_IND | AN | 1 | NOT USED (Blank Fill) |
| 37. | B_COMMENTS_TEXT | ANmc | 2000 | OPTIONAL |
| 38. | B_XMLDOC_RECORD_ID | N | 7 | OPTIONAL |

Due to the complex nature of MDBP Summaries and their relationships to Facility Analyte Levels (FANL) and CDS Compliance determination, it is recommended that those three areas of the User’s Guide be reviewed throughly prior to creation of MDBP Summary XML documents for migration.

Every MDBP Summary must be associated with a Facility Analyte Level (FANL).  If a FANL that matches the information supplied in the structure set cannot be found, the MDBP Summary will be rejected.

Overlap check for a Summary is on the combination of:

- The values in the FANL Reference Group fields.

- AND the Monitoring Period specified in fields 8 and 9 (which is also used to retrieve the correct FANL).

- AND the Sampling Point.  In this definition, a null value for field 10 (B_Sampling_Point) is effectively a value in and of itself.

- Additional notes:

- In addition to retrieving the correct Monitoring Period, field 8, B_MON_PERIOD_BEG_DATE, and field 9, B_MON_PERIOD_END_DATE, are used to retrieve the correct FANL.  The software identifies the FANL where the Effective Period Begin and End dates cover the specified Monitoring Period Begin Date.  It is recommended that FANL Effective Begin and End dates correspond to Monitoring Period Begin and End dates.

- When the summary type is IFT, the software will associate the summary to a FANL whose MDBP Summary Type is 95PT and whose INDIV_FILTER_MNTRG_REQD_FLAG is “Y.”

- Even though the Values in the fields 10, 26, and 27 will not show up in the Water Treatment Summary pages these fields can be viewed in the General MDBP Summary Maintenance pages.

## MDBP_Summary Enumerated Values

This section lists the staging table/column name and the permitted values associated with the columns. These permitted values are those that are maintained in the data model.

| MDBP SUMMARY ENUMERATED VALUES |  |  |  |
| --- | --- | --- | --- |
| LINE NO. (field) | STAGING TABLE ( TMGMDBPS ) COLUMN NAME | PERMITTED VALUES | PERMITTED VALUE TYPE |
| 4. | B_MDBP_SUMMARY_TYPE for Generic or User-Defined Summaries | All summary types in the SDWIS/STATE permitted values table (TINPVALS:TSAMDBPAS1) are permitted value. The current list of federally owned summary types is available in the SDWIS/STATE Online Data Dictionary, the online SDWIS Administration (Permitted Value Maintenance) component, and via RELATION.MDB. OTHR Other. | User Defined in TINPVALS where CODE_NAME = TSAMDBPS1 |
| 4. | B_MDBP_SUMMARY_TYPE for Combined Filter Turbidity Summaries | AVGT Combined Filter Effluent (CFT) Average Turbidity MAXT Combined Filter Effluent (CFT) Maximum Turbidity 95PT Combined Filter Effluent (CFT) 95% Turbidity | User Defined in TINPVALS where CODE_NAME = TSAMDBPS1 |
| 4. | B_MDBP_SUMMARY_TYPE for Individual Filter Turbidity Summaries | IFT Individual Filter Effluent Turbidity | User Defined in TINPVALS where CODE_NAME = TSAMDBPS1 |
| 4. | B_MDBP_SUMMARY_TYPE for Minimal Residual Disinfection Summaries | EPRD Entry Point (EP) Residual Disinfectant Concentration (RDC) SERD State Entry Point (EP) Residual Disinfectant Concentration (RDC) DSRD Distribution Residual Disinfectant Concentration (RDC) SDRD State Distribution Residual Disinfectant Concentration (RDC) | User Defined in TINPVALS where CODE_NAME = TSAMDBPS1 |
| 4. | B_MDBP_SUMMARY_TYPE for Chlorine Dioxide/Chlorite Entry Point Summaries | CLO2 Entry Point (EP) Chlorine Dioxide CLO3 Entry Point (EP) Chlorite | User Defined in TINPVALS where CODE_NAME = TSAMDBPS1 |
| 4. | B_MDBP_SUMMARY_TYPE for Chlorine/Chloramine MRDL Summaries | MRDL Chlorine/Chloramine Maximum Residual Disinfectant Level (MRDL) | User Defined in TINPVALS where CODE_NAME = TSAMDBPS1 |
| 7. | B_ANALYTE_CODE for Generic or User-Defined (OTHR) Summaries | All analytes in the SDWIS/STATE Analyte table (TSAANLYT) are permitted value. The current list of federally owned analytes is available in the SDWIS/STATE Online Data Dictionary, the online SDWIS Administration (Analyte Maintenance) component, and via RELATION.MDB. | N/A |
| 7. | B_ANALYTE_CODE for Combined and Individual Filter Turbidity (AVGT, MAXT, 95PT, IFT) Summaries | 0100 Turbidity | Fixed |
| 7. | B_ANALYTE_CODE for Minimal Residual Disinfection and Chlorine/Chloramine MRDL (EPRD, SERD, DSRD, SDRD, MRDL) Summaries | 0999 Chlorine 1006 Chloramine | Fixed |
| 7. | B_ANALYTE_CODE for Chlorine Dioxide/(CLO2) Summaries | 1008 Chlorine Dioxide | Fixed |
| 7. | B_ANALYTE_CODE for Chlorite (CLO3) Summaries | 1009 Chlorite | Fixed |
| 12. | B_CERT_AGENCY | Values are defined by the System Administrator in the Permitted Values function of SDWIS/STATE. | User Defined in TINPVALS with code_name ‘CERT_AG’ |
| 21. | B_MR_COMPLIANCE_IND | YES Yes-In Compliance NMJ No-Major Monitoring Non-Compliance NMN No-Minor Monitoring Non-Compliance | Fixed |
| 25. | B_LEVEL_COMPLIANCE_INDICATOR | Y Yes-In Compliance N No-Not Compliant | Fixed |
| 27. | B_MPA_UOM_CODE for Generic or User-Defined (OTHR) Summaries | %LUM Percent of Luminance (for measuring color) %PUR Percent of Purity (for measuring color) ADMIU American Dye Manufacturers Institute Units (for measuring color) AGGR Aggressive Index (for corrosivity) C Degrees Celsius CM-1 Total Absorbance for UV CT Contact Time CU Color Units F Degrees Fahrenheit FTU Flavor Threshold Units (for measuring taste and odor) LANG Langlier Index (for measuring corrosivity) LBS/CFT Pounds/Cubic Foot (for measuring density) LBS/GAL Pounds/Gallon (for measuring density) MFL Million of Fibers/Liter (for measuring asbestos) MG/L Milligrams/Liter (AKS Parts per Million - ppm) MREM Millirems/Liter MREMY Millirems/Liter/Year NG/L Nanograms/Liter (AKA Parts per Trillion - ppt) NMT Nanometers (to measure color wave length) NTU Nephlometric Turbidity Units OBSVNS Observations/Field at 100 power PH PH Measure (parts hydrogen (pH 0 - 14) PIC/L Picocuries/Liter RATIO Total Organic Carbon SU Standard Units (for measuring color) TON Threshold Odor Number (for odor) UG/L Micrograms/Liter (AKA Parts per Billion - ppb) UMHOS/CM Microhos/Centimeter (for measuring conductivity) | User Defined in TSAUOM in column UOM_CODE |
| 27. | B_MPA_UOM_CODE for Chlorine/Chloramine MRDL (MRDL) Summaries | MG/L Milligrams/Liter (AKS Parts per Million - ppm) | User Defined in TSAUOM in column UOM_CODE |
| 29. | B_RAA_UOM_CODE for Generic or User-Defined (OTHR) Summaries | %LUM Percent of Luminance (for measuring color) %PUR Percent of Purity (for measuring color) ADMIU American Dye Manufacturers Institute Units (for measuring color) AGGR Aggressive Index (for corrosivity) C Degrees Celsius CM-1 Total Absorbance for UV CT Contact Time CU Color Units F Degrees Fahrenheit FTU Flavor Threshold Units (for measuring taste and odor) LANG Langlier Index (for measuring corrosivity) LBS/CFT Pounds/Cubic Foot (for measuring density) LBS/GAL Pounds/Gallon (for measuring density) MFL Million of Fibers/Liter (for measuring asbestos) MG/L Milligrams/Liter (AKS Parts per Million - ppm) MREM Millirems/Liter MREMY Millirems/Liter/Year NG/L Nanograms/Liter (AKA Parts per Trillion - ppt) NMT Nanometers (to measure color wave length) NTU Nephlometric Turbidity Units OBSVNS Observations/Field at 100 power PH PH Measure (parts hydrogen (pH 0 - 14) PIC/L Picocuries/Liter RATIO Total Organic Carbon SU Standard Units (for measuring color) TON Threshold Odor Number (for odor) UG/L Micrograms/Liter (AKA Parts per Billion - ppb) UMHOS/CM Microhos/Centimeter (for measuring conductivity) | User Defined in TSAUOM in column UOM_CODE |
| 29. | B_RAA_UOM_CODE for Chlorine/Chloramine MRDL (MRDL) Summaries | MG/L Milligrams/Liter (AKS Parts per Million - ppm) | User Defined in TSAUOM in column UOM_CODE |
| 30. | B_Q1_IFT_MONITORED_INDICATOR for Generic or User-Defined and Individual Filter Turbidities (OTHR, IFT) Summaries | Y Yes N No | Fixed |
| 31. | B_Q2_IFT_RECORDED_INDICATOR for Generic or User-Defined and Individual Filter Turbidities (OTHR, IFT) Summaries | Y Yes N No | Fixed |
| 32. | B_Q3_IFT_EQUIPMENT_INDICATOR for Generic or User-Defined and Individual Filter Turbidities (OTHR, IFT) Summaries | Y Yes N No | Fixed |
| 33. | B_Q4_IFT_GREATER_1_0_INDICATOR for Generic or User-Defined and Individual Filter Turbidities (OTHR, IFT) Summaries | Y Yes N No | Fixed |
| 34. | B_Q5_IFT_GREATER_0_5_INDICATOR for Generic or User-Defined and Individual Filter Turbidities (OTHR, IFT) Summaries | Y Yes N No | Fixed |
| 35. | B_Q6_IFT_GREATER_1_DUR_3_MON_IND for Generic or User-Defined and Individual Filter Turbidities (OTHR, IFT) Summaries | Y Yes N No | Fixed |
| 36. | B_Q7_IFT_GREATER_2_0_DUR_2_MON_IND for Generic or User-Defined and Individual Filter Turbidities (OTHR, IFT) Summaries | Y Yes N No | Fixed |

## MDBP Summary Mapping from XML Schema Tag to SDWIS/STATE Entities and Attributes

This section has the mapping between the staging table/columns, SDWIS/STATE Table/Column in both the data model and the physical database and the Schema tags in the XML Sampling schemas for MDBP Summary business object.

| MDBP SUMMARY SCHEMA TAG MAPPING TO SDWIS/STATE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| LINE NO. (field) | SCHEMA TAG NAME | Staging Table Column Name | SDWIS/STATE Table and Column Name | SDWIS/STATE Entity Name | SDWIS/STATE ATTRIBUTE |
| 1. # | N/A | JOB_ID | N/A | N/A | N/A |
| 2. # | N/A | B_MDBP_SUMMARY_IS_NUMBER | N/A | N/A | N/A |
| 3. # | N/A | B_STATUS | N/A | N/A | N/A |
| 4. | MDBPSummary MDBPSummaryDetails MDBPSummaryTypeCode | B_MDBP_SUMMARY_TYPE | TSAMDBPS.TYPE_CODE_CV TMNFANL.SUM_TYPE_CODE_CV | MDBP Summary Facility Analyte Level | TYPE_CODE_CV SUMMARY_TYPE_CODE_CV (Foreign Key) |
| 5. | MDBPSummary MDBPSummaryDetails PWSIdentifier | B_PWS_NUMBER | TINWSYS.NUMBER | Water System | NUMBER (Foreign Key) |
| 6. | MDBPSummary MDBPSummaryDetails PWSFacilityIdentifier | B_WSF_STATE_ASGN_ID | TINWSF.ST_ASGN_IDENT_CD | Water System Facility | STATE_ASGN_IDENTIFICATION_CODE (Foreign Key) |
| 7. | MDBPSummary MDBPSummaryDetails AnalyteCode | B_ANALYTE_CODE | TSAANLYT.CODE | Analyte | CODE (Foreign Key) |
| 8. | MDBPSummary MDBPSummaryDetails MonitoringPeriodBeginDate | B_MON_PERIOD_BEG_DATE | TMNMPRD.BEGIN_DATE TMNFANL.EFFECTIVE_BEG_DAT | Monitoring Period Facility Analyte Level | BEGIN_DATE (Foreign Key) EFFECTIVE_BEGIN_DATE (Foreign Key) |
| 9. | MDBPSummary MDBPSummaryDetails MonitoringPeriodEndDate | B_MON_PERIOD_END_DATE | TMNMPRD.END_DATE | Monitoring Period | END_DATE (Foreign Key) |
| 10. | MDBPSummary MDBPSummaryDetails SampleLocationIdentifier | B_SAMPLING_POINT | TSASMPPT.IDENTIFICATION_CD | Sampling Point | IDENTIFICATION_CODE (Foreign Key) |
| 11. | MDBPSummary MDBPSummaryDetails LabAccreditation LabAccreditationIdentifier | B_LAB_ID_NUMBER | TSALAB.LAB_ID_NUMBER | Laboratory | LAB_ID_NUMBER |
| 12. | MDBPSummary MDBPSummaryDetails LabAccreditation LabAccreditationAuthorityName | B_CERT_AGENCY | TSALAB.CERT_AGENCY | Laboratory | CERT_AGENCY (Foreign Key) |
| 18. | MDBPSummary MDBPSummaryDetails ReportedDate | B_REPORTED_DATE | TSAMDBPS.REPORTED_DATE | MDBP Summary | REPORTED_DATE |
| 19. | MDBPSummary MDBPSummaryDetails SamplesRequiredQuantity | B_SAMPLES_REQUIRED | TSAMDBPS.SAMPLES_REQUIRED | MDBP Summary | SAMPLES_REQUIRED |
| 20. | MDBPSummary MDBPSummaryDetails SamplesCollectedQuantity | B_SAMPLES_COLLECT | TSAMDBPS.SAMPLES_COLLECTED | MDBP Summary | SAMPLES_COLLECTED |
| 21. | MDBPSummary MDBPSummaryDetails MonitoringRequirementComplianceIndicator | B_MR_COMPLIANCE_IND | TSAMDBPS.MR_COMPLIANCE_IND | MDBP Summary | MR_COMPLIANCE_INDICATOR |
| 22. | MDBPSummary MDBPSummaryDetails SamplesBeyondMeasureLevelQuantity | B_TOTAL_SAMPLES_BEYOND_MSR_LVL | TSAMDBPS.SMPLS_BYND_MEA_LVL | MDBP Summary | TOTAL_SAMPLES_BEYOND_MSR_LVL |
| 23. | MDBPSummary MDBPSummaryDetails SamplesBeyondMeasureLevelPercentage | B_PERCENT_SAMPLES_BEYOND_MSR_LVL | TSAMDBPS.PRCNT_BYND_MEA_LVL | MDBP Summary | PERCENT_SAMPLES_BEYOND_MSR_LVL |
|  |  |  |  |  | PERCENT_SAMPS_BYND_MSR_LVL_TXT |
| 24. | MDBPSummary MDBPSummaryDetails HighestMeasureValue | B_HIGHEST_MEASURE | TSAMDBPS.HIGHEST_MSR | MDBP Summary | HIGHEST_MEASURE |
|  |  |  |  |  | HIGHEST_MSR_TXT |
| 25. | MDBPSummary MDBPSummaryDetails LevelComplianceIndicator | B_LEVEL_COMPLIANCE_INDICATOR | TSAMDBPS.LVL_COMPLIANCE_IND | MDBP Summary | LEVEL_COMPLIANCE_INDICATOR |
| 26. | MDBPSummary MDBPSummaryDetails MonitoringPeriodAverage MeasurementValue | B_MONITORING_PERIOD_AVG_MEASURE | TSAMDBPS.MON_PER_AVG_MSR | MDBP Summary | MONITORING_PERIOD_AVG_MEASURE |
|  |  |  |  |  | MONITORING_PERIOD_AVG_MSR_TXT |
| 27. | MDBPSummary MDBPSummaryDetails MonitoringPeriodAverage MeasurementUnit | B_MPA_UOM_CODE | TSAMDBPS.MPA_UOM_CODE | MDBP Summary | MPA_UOM_CODE |
| 28. | MDBPSummary MDBPSummaryDetails RunningAnnualAverage MeasurementValue | B_RUNNING_ANNUAL_AVERAGE_MEASURE | TSAMDBPS.RUN_ANL_AVG_MSR | MDBP Summary | RUNNING_ANNUAL_AVERAGE_MEASURE |
|  |  |  |  |  | RUNNING_ANNUAL_AVERAGE_MSR_TXT |
| 29. | MDBPSummary MDBPSummaryDetails RunningAnnualAverage MeasurementUnit | B_RAA_UOM_CODE | TSAMDBPS.RAA_UOM_CODE | MDBP Summary | RAA_UOM_CODE |
| 30. | MDBPSummary MDBPSummaryDetails IFTMonitoredIndicator | B_Q1_IFT_MONITORED_INDICATOR | TSAMDBPS.Q1_IFT_MONITRD_IND | MDBP Summary | Q1_IFT_MONITORED_INDICATOR |
| 31. | MDBPSummary MDBPSummaryDetails IFTRecordedEveryFifteenMinuteIndicator | B_Q2_IFT_RECORDED_INDICATOR | TSAMDBPS.Q2_IFT_RECORED_IND | MDBP Summary | Q2_IFT_RECORDED_INDICATOR |
| 32. | MDBPSummary MDBPSummaryDetails IFTEquipmentFailureIndicator | B_Q3_IFT_EQUIPMENT_INDICATOR | TSAMDBPS.Q3_IFT_EQUIP_IND | MDBP Summary | Q3_IFT_EQUIPMENT_INDICATOR |
| 33. | MDBPSummary MDBPSummaryDetails IFTGreaterThan1In2ConsecutiveMeasurementIndicator | B_Q4_IFT_GREATER_1_0_INDICATOR | TSAMDBPS.Q4_IFT_OVER_1_IND | MDBP Summary | Q4_IFT_GREATER_1_0_INDICATOR |
| 34. | MDBPSummary MDBPSummaryDetails IFTGreaterThan05In2ConsecutiveMeasurementIndicator | B_Q5_IFT_GREATER_0_5_INDICATOR | TSAMDBPS.Q5_IFT_OVER_05_IND | MDBP Summary | Q5_IFT_GREATER_0_5_INDICATOR |
| 35. | MDBPSummary MDBPSummaryDetails IFTGreaterThan1In2ConsecutiveMeasurementin3MonthIndicator | B_Q6_IFT_GREATER_1_DUR_3_MON_IND | TSAMDBPS.Q6_IFT_1_IN_3M_IND | MDBP Summary | Q6_IFT_GREATER_1_DUR_3_MON_IND |
| 36. | MDBPSummary MDBPSummaryDetails IFTGreaterThan2In2ConsecutiveMeasurementin2MonthIndicator | B_Q7_IFT_GREATER_2_0_DUR_2_MON_IND | TSAMDBPS.Q7_IFT_2_in_2M_IND | MDBP Summary | Q7_IFT_GREATER_2_0_DUR_2_MON_IND |
| 37. | MDBPSummary MDBPSummaryDetails Comments | B_COMMENT_TEXT | TSAMDBPS.COMMENT_TEXT | MDBP Summary | COMMENT_TEXT |
| 38. | MDBPSummary MDBPSummaryDetails RecordID | B_XMLDOC_RECORDS_ID | N/A | N/A | N/A |

## MDBP Summary Definitions

This section has the mapping between the staging table/columns, SDWIS State Table/Column in both the data model and the physical database and the Schema tags in the XML Sampling schemas for MDBP Summary business object.

| MDBP SUMMARY DEFINITIONS |  |  |
| --- | --- | --- |
| LINE NO (field) | STAGING TABLE ( TMGMDBPS ) COLUMN NAME | MDBP SUMMARY COLUMN DESCRIPTION |
| 1. # | TMGJOB_IS_NUMBER | The identification number of the job. |
| 2. | B_TMGMDBPS_IS_NUMBER | A system_generated value which uniquely identifies the entity occurrence. |
| 3. | B_STATUS | The current status of the record. <SPACES> = unprocessed, C = processed w/ changes, P = processed , R = rejected |
| 4. * | B_MDBP_SUMMARY_TYPE | The type of MDBP summary. |
| 5. | B_PWS_NUMBER | The Public Water System identification number. |
| 6. | B_WSF_STATE_ASGN_ID | The water system facility state assigned id number. |
| 7. * | B_ANALYTE_CODE | The code of the substance being measure in the analytical result. |
| 8. | B_MON_PERIOD_BEG_DATE | The start date of the monitoring period to which the referenced MDBP summary is assigned. |
| 9. | B_MON_PERIOD_END_DATE | The end date of the monitoring period to which the referenced MDBP summary is assigned. |
| 10. | B_SAMPLING_POINT | The identification number of the sampling point at which the MDBP summary was taken. |
| 11. | B_LAB_ID_NUMBER | The identification code assigned by the the certifying agency to this laboratory. |
| 12. | B_CERT_AGENCY | The certifying agency that issued the Laboratory the ID Number under which is it reporting. |
| 13. | B_ANALYST_NO | The id number for the analyst for the MDBP summary. |
| 14. | B_ANALYST_LAST_NAME | The last name of the analyst for the MDBP summary. |
| 15. | B_ANALYST_FIRST_NAME | The first name of the analyst for the MDBP summary. |
| 16. | B_ANALYST_ORGANIZATION_NAME | The organization name of the analyst for the MDBP summary. |
| 17. | B_ANALYST_MAIL_STOP_TEXT | The mail stop of the analyst for the MDBP summary. |
| 18. | B_REPORTED_DATE | The date the MDBP summary was reported. |
| 19. | B_SAMPLES_REQUIRED | The number of samples required. |
| 20. | B_SAMPLES_COLLECT | The number of samples collected. |
| 21. * | B_MR_COMPLIANCE_IND | Indicates status of M&R compliance for specified analyte and water system facility. |
| 22. | B_TOTAL_SAMPLES_BEYOND_MSR_LVL | The number of samples beyond measure level. |
| 23. | B_PERCENT_SAMPLES_BEYOND_MSR_LVL | The percent of samples beyond measure level. |
| 24. | B_HIGHEST_MEASURE | The highest measure during the specified monitoring period. |
| 25. * | B_LEVEL_COMPLIANCE_INDICATOR | Indicates status of Level compliance for specified analyte and water system facility. |
| 26. | B_MONITORING_PERIOD_AVG_MEASURE | The monitoring period average measure. |
| 27. * | B_MPA_UOM_CODE | The unit of measure for the monitoring period average measure. |
| 28. | B_RUNNING_ANNUAL_AVERAGE_MEASURE | The running annual average measure. |
| 29. * | B_RAA_UOM_CODE | The unit of measure for the running annual average measure. |
| 30. * | B_Q1_IFT_MONITORED_INDICATOR | Individual Filtered Turbidity Monitored Indicator |
| 31. * | B_Q2_IFT_RECORDED_INDICATOR | Individual Filtered Turbidity Recorded Indicator |
| 32. * | B_Q3_IFT_EQUIPMENT_INDICATOR | Individual Filtered Turbidity Equipment Indicator |
| 33. * | B_Q4_IFT_GREATER_1_0_INDICATOR | Individual Filtered Turbidity Greater Than 1.0 Indicator |
| 34. * | B_Q5_IFT_GREATER_0_5_INDICATOR | Individual Filtered Turbidity Greater Than 0.5 Indicator |
| 35. * | B_Q6_IFT_GREATER_1_DUR_3_MON_IND | Individual Filtered Turbidity Greater Than 1.0 during 3 consecutive months Indicator |
| 36. * | B_Q7_IFT_GREAT_2_0_DUR_2_MON_IND | Individual Filtered Turbidity Greater Than 2.0 during 2 consecutive months Indicator |
| 37. | B_COMMENT_TEXT | Additional information about this MDBP summary. |
| 38. | B_XMLDOC_RECORD_ID | A number that uniquely identifiers a record within an XML document. |