Detailed Procedures

Storage tank cleaning procedure:

The sediment shall be drained every four months by opening the gate valve located at the base of the storage tank.  The water shall be discharged for at least two full minutes or until the water is clear and free of visible particles which ever is longer.  If the flushing takes more than fifteen minutes, then the storage tank shall be drained and cleaned in accordance with AWWA Standard C652-92.

Replacement of any system components shall be done following all applicable Standards and Guidance documents