50a

State Water Resources Control Board

Division of Drinking Water

50 D Street, Suite 200

Santa Rosa, CA  95404-3752

(707) 576-2145   Fax: (707) 576-2722

# LEAD AND COPPER SAMPLING PLAN

System Name: Type: Community NTNC Population: Min # of Sampling Sites: >100,000 100 10,001 to 100,000 60 3,301 to 10,000 40 501 to 3,300 20 101 to 500 10 < 100 5

System ID Number:

Plan Date:

SAMPLE SITE IDENTIFICATION

Sampling must first be performed at Tier 1 sites. If not enough Tier 1 sites are available, the public water system (System) must use Tier 2 sites. If not enough Tier 2 sites are available, then Tier 3 sites may be used to complete sampling for the required number of locations.

Sample Site Categories:							           # of Sites in Category:

- Tier 1: Single-family structures with copper pipes with lead solder

installed after 1982; or with lead pipes; or with lead service lines.			  ___________

- Tier 2: Multi-family structures or other buildings (i.e. schools) with copper pipes 
with lead solder installed after 1982; or lead pipes; or with lead service lines.	 ____________

- Tier 3: Single family structures or any other buildings (i.e. school, multi-family, 
mobile home) with copper pipes with lead solder installed prior to 1983.		  ____________

- Other:  Sample sites do not fit any of the above categories.			  ____________

Total:	  ____________

- Number of lead service lines present in the service area:				  ___________

- Number of samples collected from sites served by lead service lines:		  ___________

System does not contain any copper pipes, lead plasticizers, plastic service lines that contain lead plasticizers, lead service lines, lead pipes, or lead soldered pipe joints. (System must provide documentation that ALL residences, buildings, and services lines contain NONE of the above in order to receive a waiver.)

The Lead and Copper Rule requires the system do due diligence in research to determine the presence of lead and copper materials in the service area. The following sources have been explored to determine the number of structures which have interior copper pipe with lead solder or lead service lines:

| Distribution system maps and record drawings. Capital improvement plans and/or master plans for distribution system development. Standard operating procedures and/or operation & maintenance manuals for the types of materials used for service connections. Utility records including meter installations, customer complaint investigations. Conduct service line sampling where lead service lines are suspected to exist. | Water quality data Survey of service area plumbers about when and where lead solder was used from 1982 to present Survey of residents Interviews with senior personnel. Interviews with local pipe suppliers Plumbing and/or building codes and permits Contacts with the building department, when and where lead solder was used from municipal clerk's office, or state regulatory agencies Interviews with local contractors, developers, building inspectors |
| --- | --- |

# SAMPLING LOCATION AND TIER IDENTIFICATION

The System shall sample at any of the approved sites listed below. Attach additional sites if needed and a map showing the locations of the sampling sites.

Consider including additional sampling sites (for example, for a system that is required to sample at ten locations, include five additional alternative locations).

If a System must change to a site not listed below, the System must notify the Division and submit an updated Lead and Copper Plan.

|  | Sample Address | Tier |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |
| 8 |  |  |
| 9 |  |  |
| 10 |  |  |
| 11 |  |  |
| 12 |  |  |
| 13 |  |  |
| 14 |  |  |
| 15 |  |  |
| 16 |  |  |
| 17 |  |  |
| 18 |  |  |
| 19 |  |  |
| 20 |  |  |
| 21 |  |  |
| 22 |  |  |
| 23 |  |  |
| 24 |  |  |
| 25 |  |  |
| 26 |  |  |
| 27 |  |  |
| 28 |  |  |
| 29 |  |  |
| 30 |  |  |

# SAMPLING FREQUENCY

The System shall sample according to California Code of Regulations Title 22, Section 64675.5 and as directed by the Division. Lead and copper monitoring shall begin with two consecutive rounds of samples, six months apart. Based on the results of these consecutive samples, the system will either move to annual or triennial sampling frequency.

If a lead or copper action level is exceeded or there is a change in source or treatment, the System will restart the sampling process with two consecutive rounds of samples six months apart. Please notify the Division if there is a change in source or treatment.

# CORROSION CONTROL TREATMENT MONITORING

The System does not have corrosion control treatment.

The System has corrosion control treatment and is required to perform corrosion control treatment (CCT) monitoring. Results are submitted _______________________________ (monthly, quarterly) to the Division.

Type of corrosion control treatment: ____________________________________________________________

Corrosion Control Parameters Monitored

| Constituent Monitored (pH, alkalinity, orthophosphate, etc.) | Monitoring Frequency (Daily, Weekly, Monthly, etc.) | Monitoring Location |
| --- | --- | --- |

# SCHOOLS

The System serves one or more schools (K-12).

# LEAD ACTION LEVEL EXCEEDANCE – PUBLIC NOTIFICATION

In the case of a lead action level exceedance, the System will deliver the attached public notification content to consumers. The System will repeat public lead education every 12 months for as long as the System has a lead action level exceedance.

# NOTIFICATION OF RESULTS TO PARTICIPANTS

Within 30 days of learning of the lead and copper sampling results, the System shall notify the participants, by mailing or posting of the notice, of the sample results taken from their individual taps.

If lead is detected, the System shall deliver notification providing an explanation of the health effects of lead and listing the steps the consumer can take to reduce exposure to lead in drinking water.

SIGNATURE:							  	DATE:

/

Print Name						Title