Operations Plan for a Small Water System

(Groundwater Source)

Submit to: State Water Resources Control Board

Division of Drinking Water - Visalia District Office

265 W. Bullard Ave, Ste 101, Fresno, CA 93704

Phone: (559) 447-3300

Date of Plan:

System Name:      	             System No.

Physical Location Address:      		________________________________________________

Mailing Address:      		______________________________________________________

Contact Name:      		___    Phone:      	________    Email:

The Water System agrees to implement all outlined procedures in this Operations Plan.

- BRIEF DESCRIPTION OF SYSTEM:

Type of System:	 Community (CWS)     Nontransient-Noncommunity (NTNC)

Transient-Noncommunity (TNC)

Number of service connections:      			Population served:

Population: 	  Year-round population:      				______

Seasonal population & time frame:      		______

Total Population:      		______

Source(s) (date of drilling, depth, perforations, pump setting):

Storage and Pressure Tank(s) (capacity and material):

Chlorination (type of chlorinator pump, capacity of pump, manufacturer and model, size of the chlorine solution storage tank, type of chemical and NSF certification):

Other:

- OPERATOR CERTIFICATION

Distribution System Class:   D1      D2	  Treatment System Class:  T1     T2 	 N/A

| Position | Name | Contact Info (Phone & Email) | Certification Level & No. | Responsibilities |
| --- | --- | --- | --- | --- |
| Certified Distribution Operator (chief) |  |  |  |  |
| Certified Distribution Operator (shift) |  |  |  |  |
| Certified Treatment Operator |  |  |  |  |
| Trained Personnel |  |  |  |  |
| Cross Connection Specialist |  |  |  |  |
| Water Sampler |  |  |  |  |

- ROUTINE OPERATIONAL PROCEDURES FOR SYSTEM:

Table 1: Routine Operations Checklist

| System Component | Activity | Frequency | Record- keeping?* | Responsible Party (fill in) |
| --- | --- | --- | --- | --- |
| WELL(S) | Check potential sanitary hazards: water leaks that could contaminate well, unscreened or openings where sealants can be applied, electrical hazards, chemical hazards (proper use of chemicals around well head). | Monthly | N/A |  |
|  | Well has the ability to be pumped to waste and sampling tap is non-threaded down-turned hose bib). | Annually | N/A |  |
|  | Check the pump and controls for proper operation of well. | Monthly | N/A |  |
|  | General housekeeping: remove animal feces, dirt, vegetation, any standing water, control gophers/squirrel burrowing around well head to eliminate potential contamination hazards, etc. | Monthly | N/A |  |
| STORAGE and PRESSURE TANKS | Inspect vents and overflow outlets for proper protection (screens, flapper valve, etc.) to keep out rodents and insects. | Monthly | N/A |  |
|  | Inspect for any leaks or damage and repair as needed. | Monthly | Yes |  |
|  | Record system pressure. Record the pressure the pump turns on, the pressure the pump turns off and the duration of the run time so storage tank does not overflow. | Monthly | Yes |  |
|  | Scheduled inspection and cleaning of storage tank. | Every 5 years | Yes |  |
| GAUGES and FLOWMETERS | Inspect all gauges and flowmeters for leaks and proper function, and repair or replace as necessary. Schedule routine calibration checks to ensure accurate readings are being provided. | Monthly | Yes – when repaired or replaced. |  |
|  | Record monthly water production year-round and weekly water production during the summer months. | Monthly | Yes – upload to Electronic Annual Report |  |
| VALVES | Inspect valves for leaks, and repair or replace as necessary. | Monthly | Yes – when repaired or replaced. |  |
|  | Exercise valves on a schedule, as needed. | Annually | Yes |  |
| DISTRIBUTION FACITILIES | Visually inspect the distribution system for leaks. | Monthly | Yes |  |
|  | Flush dead end mains or lines periodically | Annually | yes |  |
|  | Test backflow prevention devices annually. | Annually | Yes – upload to Electronic Annual Report |  |
|  | Develop and maintain a cross-connection program to prevent contamination of potable water supply from unapproved source. Conduct cross-connection survey every 5 years. | Every 5 years | Yes – send copy of survey to Division | Certified Operator or Cross Connection Specialist |
|  | Review and update Bacteriological Sample Siting Plan (BSSP), at minimum of 10 years, to ensure it reflects current customer base and service area | Every 10 years | Yes – send to Division for approval |  |
| CHLORINATION (if applicable) | Inspect the pump and chlorine reservoir and fill up as needed. Ensure chlorine chemical is NSF approved. | Monthly | N/A | Certified Operator |
|  | Measure disinfectant residual. | Weekly | Yes – send results monthly to Division | Certified Operator or Trained Personnel |

*Tip:  Maintain a log book for each well site that records maintenance and monthly water production and flow rates, water table depths and any maintenance performed.

OTHER: Describe any other operational procedures for any other aspects of the water system (e.g. hauled water system).

- WATER QUALITY MONITORING:

Table 2: Routine Water Quality Sampling Checklist

| Location | Type of Monitoring | Which systems | Frequency | Reporting |
| --- | --- | --- | --- | --- |
| SOURCE WATER | Chemical & Radiological | All | See Water Quality Monitoring Schedule | Lab EDT’s to Division |
|  | Bacteriological | Only for systems that chlorinate | Monthly | System sends to Division monthly |
| DISTRIBUTION SYSTEM | Bacteriological | All | Monthly | System sends to Division monthly |
|  | Disinfection Byproducts (DBP) | Only CWS & NTNC with chlorination | See approved Stage 2 DBP Plan | Lab EDT’s to Division |
|  | Lead & Copper | Only CWS & NTNC | Consult with Division | System sends results to Division |

- REQUIRED PLANS

Table 3: Water System Planning Tools Checklist

| Type | Name | Frequency |
| --- | --- | --- |
| OPERATIONS PLANS | Emergency Notification Plan | Update whenever contact information changes |
|  | Small Water System Operations Plan | Update whenever there are changes |
|  | Chlorination Operations Plan (for systems that chlorinate) | Update whenever there are changes to personnel |
|  | Emergency Disinfection Plan (for systems that do not chlorinate) | Update whenever there are changes to personnel |
|  | Cross Connection Survey | Every 5 years |
|  | Emergency Response Plan | Update whenever there are changes to personnel |
|  | Service Area and Facility Map | Update whenever there are changes |
| WATER QUALITY PLANS | Bacteriological Sample Siting Plan | Every 10 years |
|  | Stage 2 Disinfection By Product Plan | Consult with Division |
|  | Lead and Copper Plan | Consult with Division |

- REQUIRED REPORTING

Table 4: Monitoring and Reporting Required Reports

| Name | Frequency | Required Activities |
| --- | --- | --- |
| Electronic Annual Report (EAR) | Annual, due March 31st | Fill out at EAR portal: http://drinc.waterboards.ca.gov/EAR/Login.aspx?ReturnUrl=DEHome.aspx |
| Consumer Confidence Report (CCR) | Annual, Due July 1st | Fill out a CCR document. Template at http://www.swrcb.ca.gov/drinking_water/certlic/drinkingwater/CCR.shtml |
|  |  | Upload CCR document and Certification of Delivery at EAR portal http://drinc.waterboards.ca.gov/EAR/Login.aspx?ReturnUrl=DEHome.aspx |
|  |  | Deliver to all customers |
| Water Production | Monthly | Record every month. Upload data annually in EAR. |
| Treatment Report (except chlorination) | Monthly | Send Treatment Reports summarizing effluent results by hardcopy or fax to the Division. |
| Chlorine Residual Report | Monthly | Measure weekly chlorine residuals and send monthly log to the Division. |
| Bacteriological Report | Monthly | Due to the Division of by the 10th day following the month of the sample collected. |
| Compliance Order | Quarterly* | Sampling, Public Notification, Progress Report |
| *Or as indicated otherwise in issued Compliance Order. |  |  |

- WATER QUALITY EXCEEDANCES (If applicable)

- If an MCL is exceeded, the water system must notify the Visalia District Office immediately upon receipt of the exceedance.

- If the water system is in violation of an MCL exceedance, the Division will issue a compliance order outlining directives to comply with, including the items listed below. Check all that apply if you have a compliance order.

Corrective Action Plan

Present Correction Action Plan in Division office hearing

Quarterly Progress Report to the Division

Quarterly monitoring, public notification, and send Division proof of notification

Other:      												_____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

__________________________________________________________________________

____________________________________________________________________________________________________________________________________________________

__________________________________________________________________________

____________________________________________________________________________________________________________________________________________________

__________________________________________________________________________

- EMERGENCY OPERATIONAL PRACTICES:

Table 5: List of Sources of Needed Equipment, Not on Hand.

| Name | Address | Phone # | Equipment | Rental/ Contract |
| --- | --- | --- | --- | --- |
|  |  |  | Steel Tank Welder |  |
|  |  |  | Electrical repair |  |
|  |  |  | Digging equipment |  |
|  |  |  | Generator |  |
|  |  |  | Chemicals |  |

Table 6: List of Distributors or Suppliers of Replacement Parts for the System

| Name | Address | Phone # | Equipment |
| --- | --- | --- | --- |
|  |  |  | PVC pipe, valves, and fittings |
|  |  |  | Pumps, pressure tank and gauges |
|  |  |  | Chlorinator |

Table 7: List of Emergency Contact Numbers

| # Name | Phone # |
| --- | --- |
| Tricia Wathen, District Engineer, SWRCB Visalia District Office | Office: (559) 447-3300 Cell: (559) 696-8506 |
| Water System Owner – |  |
| Certified Operators (include certification level) – |  |
| Laboratory – |  |
| Pump repair service – |  |
| Chemical disinfectant supplier – |  |
| Equipment supplier – |  |
| Electrician – |  |
| Law Enforcement – |  |

- FILE ATTACHMENTS

With this Operations Plan, we strongly recommend that you create a multi-tabbed binder that includes the following file attachments (check all that apply):

Permits:

Domestic Water Supply Permit

Domestic Water Supply Permit Amendment(s) (describing modification or addition of new sources, treatment, or transfer of ownership)

Water System Information:

System schematics, map of distribution system

Plans and specifications related to wells, water lines and valve locations, pump, storage tank, pressure tank, etc.

Department of Water Resources (DWR) Well Completion Report (a.ka. Well Log)

County well construction permit (describing adequate horizontal protection of well from sanitary hazards)

Operations:

Operational and service records

Chlorination Plan

Water production logs

Hardcopies of annual Electronic Annual Report (EAR)

Cross Connection Survey – every 5 years

Operator Certification

Customer Complaint Program

Sanitary survey inspection reports and list of deficiencies from Division

Emergency:

Emergency Notification Plan (ENP)

Emergency Response Plan

Emergency Chlorination Plan

Water Quality:

Bacteriological Sample Siting Plan (BSSP) – update every 10 years

Water Quality Monitoring Schedule for chemicals – ask Division for most recent version

Stage 2 Disinfection Byproduct Sampling Plan

Lead and Copper Monitoring Plan

Ground Water Rule Plan

Treatment or Blending Operations Plan – for systems that have treatment only (not chlorination)

Required monthly treatment or blending reports to the Division – for systems that have treatment only (not chlorination)

Annual Consumer Confidence Reports (CCRs) and Delivery Certification form to Division

Public notifications (water quality exceedances, boil water orders, etc.) and proof of public notification to Division

Hardcopies of all water quality laboratory results

Other:

Enforcement History from the Division – Enforcement Letters, Citations, or Compliance Orders

Funding projects

General correspondence with Division, operator, etc.

Other:

The Water System agrees to implement all outlined procedures in this Operations Plan.  The Water System shall update this Operations Plan as needed and also send an update version to the Division.

Water System Name:___________________________________	System No.__________________

Name of the Person Preparing the Operations Plan:_________________________________________

Title & Organization:__________________________________________________________________

Signature of the Person:____________________________	Date:_____________