# LOS ANGELES COUNTY DEPARTMENT OF PUBLIC WORKS

GEOTECHNICAL AND MATERIALS ENGINEERING DIVISION

## 2012 WATER QUALITY MONITORING FOR

## SOFT BOTTOM CHANNEL CLEANOUTS

Channel/Reach: Project 74							PCA: F7057920

Date: 09/19/12

| SAMPLE LOCATION | Upstream of Project* | Within Project* | Downstream of Project* |
| --- | --- | --- | --- |
| TIME# | 1022 | 1031 | 1045 |
| SAMPLE NO. | PROJ74-1A | PROJ74-2A | PROJ74-3A |
| TEMPERATURE (°F) | 73.5 | 71.6 | 72.6 |
| pH | 6.95 | 6.97 | 6.94 |
| TURBIDITY (NTUs) | 11.4 | 4.05 | 8.27 |
| DISSOLVED O2 (mg/L) | 11.61 | 2.641 | 3.941 |
| TOTAL SUSPENDED SOLIDS (mg/L) | 19.01 | 5.001 | 3.001 |

- * – Upstream of Project = 1, Within Project = 2, and Downstream of Project = 3 on sample labels and lab reports

- # – A = morning and P = afternoon on sample labels and lab reports

- Samples for Dissolved Oxygen and Total Suspended Solids were analyzed by GMED’s As-Needed Contract Environmental Laboratories, Sierra Analytical Labs (Sierra); 1 – Reference: Sierra Lab Report No. 1209225

- Field Equipment Used: Hanna Model HI 991300 pH/EC/TDS/T Probe; LaMotte 2020e Turbidimeter

FIELD NOTES:

- Met with Ben Limosnero, FMD 83rd St. Yard, on Wednesday, 09/19, to discuss cleanout operations, scope of project, estimated number of days to complete the project, and water quality monitoring; surface water not diverted during field operations; estimated completion on Saturday, 09/22; next sampling event scheduled for Thursday, 09/20.

- Upstream sampling point, #1: at the intersection of the open concrete box channel and the soft-bottom channel about 450 feet n/o the northeast corner of the intersection of the Artesia (91) Freeway and Vermont Avenue; see Figure 1 and Photo 1.

- Internal sampling point, #2: directly below the north edge of the Artesia (91) Freeway on the east bank of the soft-bottom channel; sampling point is approximately 100 feet e/o Vermont Avenue; see Figure 1 and Photo 2.

- Downstream sampling point, #3: in bottom of open concrete-lined trapezoidal channel immediately s/o the south wall of the overpass entrance to the Artesia Transit Center; about 95 feet e/o Vermont Avenue as measured along the sidewalk on the south side of the bridge/overpass; see Figure 2 and Photo 3.

C:/My Files/WQ-Soft Bottom Channels/Project 74/Fall 2012/wqmonitsheet.091912