# LOS ANGELES COUNTY DEPARTMENT OF PUBLIC WORKS

GEOTECHNICAL AND MATERIALS ENGINEERING DIVISION

## 2013 WATER QUALITY MONITORING FOR

## SOFT BOTTOM CHANNEL CLEANOUTS

Channel/Reach: San Jose Creek-Reach 42				PCA: F4004037

Date: 01/22/2013

| SAMPLE LOCATION | Upstream of Project* | Within Project* | Downstream of Project* |
| --- | --- | --- | --- |
| TIME# | 1344 | 1352 | 1403 |
| SAMPLE NO. | SJCRK1 | SJCRK2 | SJCRK3 |
| TEMPERATURE (°F) | 69.0 | 67.7 | 53.9 |
| pH | 6.99 | 7.00 | 7.01 |
| TURBIDITY (NTUs) | 0.88 | 0.76 | 0.69 |
| DISSOLVED O2 (mg/L) | 11.91 | 10.91 | 7.441 |
| TOTAL SUSPENDED SOLIDS (mg/L) | 10.01 | ND1 | 6.001 |

- * – Upstream of Project = 1, Within Project = 2, and Downstream of Project = 3 on sample labels and lab reports

- Samples for Dissolved Oxygen and Total Suspended Solids were analyzed by GMED’s As-Needed Contract Environmental Laboratories, American Environmental Testing Labs (AETL); 1 – Reference: AETL Lab Report No. 68223; ND – not detected at or above the Method Detection Limit

- Field Equipment Used: Hanna Model HI 991300 pH/EC/TDS/T Probe; LaMotte 2020e Turbidimeter

FIELD NOTES:

- First and last day of cleanout operations; spoke with Bob Holderness, FMD Longden Yard and met with personnel from FMD Longden Yard, on Tuesday, 01/22, to discuss cleanout operations, scope of project, estimated number of days to complete the project, water quality monitoring and locations of upstream, internal and downstream sampling points; surface water was not diverted during cleanout operations; only minor vegetation removal of castor bean plants along the north bank of San Jose Creek which took less than 1 hour to complete; no BMPs were installed due to limited time and effort to finish cleanout operations; no further monitoring or sampling.

- Upstream sampling point, #1: On south bank of San Jose Creek, at transition from concrete-lined channel to soft-bottom channel about 3720’ (.70 mile) east of (e/o) Workman Mill Road along top of south levee; see Figure 1 and Photo 1.

- Internal sampling point, #2: Located at “RP23” (relief point?) at base of grouted rip-rap slope on south bank of San Jose Creek; sampling point located about 410’ west of (w/o) boundary between end of open-box concrete channel and beginning of trapezoidal channel (sampling point #1); see Figure 1 and Photo 2.

- Downstream sampling point, #3: On south bank of San Jose Creek below south-bound Workman Mill Road about 2970’ (.56 mile) west of (w/o) sampling point #2; see Figure 2 and Photo 3.

C:/My Files/WQ-Soft Bottom Channels/San Jose Creek/Spring 2013/wqmonitsheet.012213