# LOS ANGELES COUNTY DEPARTMENT OF PUBLIC WORKS

GEOTECHNICAL AND MATERIALS ENGINEERING DIVISION

## 2012 WATER QUALITY MONITORING FOR

## SOFT BOTTOM CHANNEL CLEANOUTS

Channel/Reach: Walnut Creek/Reach 98				PCA: F4004048

Date: 10/19/2012

| SAMPLE LOCATION | Upstream of Project* | Within Project* | Downstream of Project* |
| --- | --- | --- | --- |
| TIME# | 1157 | 1204 | 1224 |
| SAMPLE NO. | WCRK1 | WCRK2 | WCRK3 |
| TEMPERATURE (°F) | 67.5 | 68.5 | 74.6 |
| pH | 6.98 | 6.98 | 6.97 |
| TURBIDITY (NTUs) | 8.42 | 4.60 | 1.87 |
| DISSOLVED O2 (mg/L) | 4.991 | 7.091 | 8.611 |
| TOTAL SUSPENDED SOLIDS (mg/L) | 16.01 | ND1 | 8.001 |

- * – Upstream of Project = 1, Within Project = 2, and Downstream of Project = 3 on sample labels and lab reports

- Samples for Dissolved Oxygen and Total Suspended Solids were analyzed by GMED’s As-Needed Contract Environmental Laboratories, American Environmental Testing Labs (AETL); 1 – Reference: AETL Lab Report No. 67205; ND – not detected at or above the Method Detection Limit

- Field Equipment Used: Hanna Model HI 991300 pH/EC/TDS/T Probe; LaMotte 2020e Turbidimeter

FIELD NOTES:

Pre-Work Monitoring

- Upstream sampling point, #1: located at the north end of the cul-de-sac of Chaparro Drive; access to sampling point through locked vehicle gate on west side of cul-de-sac between 2 single-family residences; sampling point about 100’ upstream of the intersection of the SBC and the concrete channel on the east bank of Walnut Creek where drainage swale from street drain enters creek; see Figure 1 and Photo 1.

- Internal sampling point, #2: previous upstream sampling point; located at the north end of the cul-de-sac of Chaparro Drive; access to sampling point through locked vehicle gate on west side of cul-de-sac between 2 single-family residences; sampling point located at the end of the access road down to Walnut Creek, adjacent to “C.S.D. Sewer” manhole on east side of Walnut Creek, above start of the open-box concrete channel, see Figure 1 and Photo 2..

- Downstream sampling point, #3: located slightly over 1½ miles downstream and southwest of upstream sampling point (#1); sampling point accessed through locked gate on the west side of open concrete box channel off of Fairway Lane, w/o Grand Avenue; rubber dam at pump house diverting surface flow from channel to Walnut Spreading Basin; sampling point about 50’ w/o Walnut Creek in small open-box concrete spillway between creek and spreading basin; see Figure 2 and Photo 3.

C:/My Files/WQ-Soft Bottom Channels/Walnut Creek/Fall 2012/wqmonitsheet.101912