# LOS ANGELES COUNTY DEPARTMENT OF PUBLIC WORKS

GEOTECHNICAL AND MATERIALS ENGINEERING DIVISION

## 2012 WATER QUALITY MONITORING FOR

## SOFT BOTTOM CHANNEL CLEANOUTS

Channel/Reach: Compton Creek/Reach 24				PCA: F6057916

Date: 09/18/2012

| SAMPLE LOCATION | Upstream of Project* | Within Project* | Downstream of Project* |
| --- | --- | --- | --- |
| TIME# | 1338P | 1355P | 1411P |
| SAMPLE NO. | CCRK1P | CCRK2P | CCRK3P |
| TEMPERATURE (°F) | 89.8 | 78.8 | 86.2 |
| pH | 6.95 | 6.94 | 7.00 |
| TURBIDITY (NTUs) | 10.50 | 4.08 | 1.45 |
| DISSOLVED O2 (mg/L) | 17.01 | 5.791 | 9.241 |
| TOTAL SUSPENDED SOLIDS (mg/L) | 45.01 | 6.001 | 4.001 |

- * – Upstream of Project = 1, Within Project = 2, and Downstream of Project = 3 on sample labels and lab reports

- # – A = morning and P = afternoon on sample labels and lab reports

- Samples for Dissolved Oxygen and Total Suspended Solids were analyzed by GMED’s As-Needed Contract Environmental Laboratories, Sierra Analytical Labs (Sierra); 1 – Reference: Sierra Lab Report No. 1209207; ND – not detected at or above the Method Detection Limit

- Field Equipment Used: Hanna Model HI 991300 pH/EC/TDS/T Probe; LaMotte 2020e Turbidimeter

FIELD NOTES:

- Met with FMD personnel from Imperial Yard on Tuesday, 9/18 to discuss cleanout operations, scope of project, estimated number of days to complete the project, and water quality monitoring results; surface water not diverted during field operations; hand crews and heavy equipment working on channel margins clearing vegetation at time of sampling; BMPs consist of 2 sets of straw waddles anchored by sand bags with 1 set placed across the channel at transition from soft-bottom channel to concrete-lined channel and 1 set placed at the confluence of Compton Creek and the L.A. River; increased turbidity from unknown upstream sources resulted in higher turbidity reading at Point #1; work estimated to be completed on Friday, 10/12.

- Upstream sampling point, #1: located on east bank of Compton Creek approximately 480 feet west of (w/o) S. Santa Fe Avenue and about .77 mile north of (n/o) Santa Fe Avenue bridge over Compton Creek measured along top of east levee; access road to sampling point on west side of Santa Fe Avenue parallel to south side of UPRR railroad tracks; see Figure 1 and Photo 1.

- Internal sampling point, #2: located on west side of Compton Creek at small concrete crib structure with weir across creek; sampling point located about 1800’ (.34 mile) south of (s/o) Santa Fe Avenue bridge over Compton Creek as measured along top of west levee; see Figure 2 and Photo 2.

- Downstream sampling point, #3: located in center of Compton Creek at confluence with L.A. River and about 440’ southeast of north-bound 710 (Long Beach) Freeway overcrossing; sampling point about 50’ downstream of transition from soft-bottom channel to concrete-lined channel and immediately below the BMP consisting of a single straw waddle across the channel anchored with sand bags; access to sampling point on the south side of Del Amo Blvd. at west bank of the L.A. River, then down access ramp to Compton Creek outlet; see Figure 3 and Photo 3.

C:/My Files/WQ-Soft Bottom Channels/Compton Creek/Fall 2012/wqmonitsheet.091812