# LOS ANGELES COUNTY DEPARTMENT OF PUBLIC WORKS

GEOTECHNICAL AND MATERIALS ENGINEERING DIVISION

## 2012 WATER QUALITY MONITORING FOR

## SOFT BOTTOM CHANNEL CLEANOUTS

Channel/Reach: Los Angeles River – West Side/Reach 25		PCA: F6057919

Date: 09/17/2012

| SAMPLE LOCATION | Upstream of Project* | Within Project* | Downstream of Project* |
| --- | --- | --- | --- |
| TIME# | 1308 | 1340 | 1358 |
| SAMPLE NO. | LARW-1P | LARW-2P | LARW-3P |
| TEMPERATURE (°F) | 88.7 | 87.3 | 79.0 |
| pH | 6.96 | 6.93 | 6.91 |
| TURBIDITY (NTUs) | 21.3 | 11.1 | 4.03 |
| DISSOLVED O2 (mg/L) | 16.81 | 13.91 | 7.471 |
| TOTAL SUSPENDED SOLIDS (mg/L) | 4291 | 1401 | 25.01 |

- * – Upstream of Project = 1, Within Project = 2, and Downstream of Project = 3 on sample labels and lab reports

- # – A = morning and P = afternoon on sample labels and lab reports

- Samples for Dissolved Oxygen and Total Suspended Solids were analyzed by GMED’s As-Needed Contract Environmental Laboratories, Sierra Analytical Labs, Inc. (Sierra); 1 – Reference: Sierra Lab Report No. 1209193; ND – not detected at or above the Method Detection Limit

- Field Equipment Used: Hanna Model HI 991300 pH/EC/TDS/T Probe; LaMotte 2020e Turbidimeter

FIELD NOTES:

- First day of cleanout operations; using bulldozers and loader to remove vegetation and create temporary haul road for dump trucks; no hand crews on site; met with FMD personnel from Imperial Yard on Monday, 09/17 to discuss cleanout operations, scope of project, estimated number of days to complete the project, water quality monitoring and locations of upstream, internal and downstream sampling points; surface water not diverted during field operations; no BMPs utilized due to limited access and excessive width of L.A. River at downstream limit of project; increased turbidity from unknown upstream sources resulted in higher turbidity reading at Point #1; work estimated to be completed on Wednesday, 11/21.

- Upstream sampling point, #1: located on the west bank of the L.A. River about 85’ s/o the south side of the Willow Street bridge over the L.A. River; sampling point located at the end of the trapezoidal concrete channel and beginning of the SBC; access to sampling point from the end of the n/b 710 off-ramp to East Willow Street at the locked vehicle gate to the west levee, then over concrete wall and down concrete slope to west bank of river; see Figure 1 and Photo 1.

- Internal sampling point, #2: located on the west bank of the L.A. River about 1175’ s/o the south side of the Willow Street bridge over the L.A. River at the petroleum pipeline support structure; sampling point on west bank between the 2nd and 3rd supports with access over concrete wall and down rip-rap slope; see Figure 2 and Photo 2.

- Downstream sampling point, #3: located on the west bank of the L.A. River directly below the north side of the Pacific Coast Highway (PCH) bridge over the river; access to sampling point at service road to levee from w/b PCH to n/b 710 on-ramp, then over concrete wall and down rip-rap slope; see Figure 3 and Photo 3.

C:/My Files/WQ-Soft Bottom Channels/LA River-West/Fall 2012/wqmonitsheet.091712