# LOS ANGELES COUNTY DEPARTMENT OF PUBLIC WORKS

GEOTECHNICAL AND MATERIALS ENGINEERING DIVISION

## 2012 WATER QUALITY MONITORING FOR

## SOFT BOTTOM CHANNEL CLEANOUTS

Channel/Reach: Los Angeles River – West Side/Reach 25		PCA: F6057919

Date: 09/24/2012

| SAMPLE LOCATION | Upstream of Project* | Within Project* | Downstream of Project* |
| --- | --- | --- | --- |
| TIME# | 1222 | 1206 | 1148 |
| SAMPLE NO. | LARW-1P | LARW-2P | LARW-3A |
| TEMPERATURE (°F) | 87.9 | 84.9 | 76.8 |
| pH | 6.91 | 6.93 | 6.96 |
| TURBIDITY (NTUs) | 5.25 | 5.42 | 2.66 |
| DISSOLVED O2 (mg/L) | 16.91 | 13.61 | 6.731 |
| TOTAL SUSPENDED SOLIDS (mg/L) | 76.01 | 36.01 | 24.01 |

- * – Upstream of Project = 1, Within Project = 2, and Downstream of Project = 3 on sample labels and lab reports

- # – A = morning and P = afternoon on sample labels and lab reports

- Samples for Dissolved Oxygen and Total Suspended Solids were analyzed by GMED’s As-Needed Contract Environmental Laboratories, Sierra Analytical Labs, Inc. (Sierra); 1 – Reference: Sierra Lab Report No. 1209270; ND – not detected at or above the Method Detection Limit

- Field Equipment Used: Hanna Model HI 991300 pH/EC/TDS/T Probe; LaMotte 2020e Turbidimeter

FIELD NOTES:

- Last day of daily monitoring, start of weekly monitoring; met with FMD personnel from Imperial Yard on Monday, 09/24 to discuss cleanout operations, scope of project, estimated number of days to complete the project, and water quality monitoring results; using bulldozers and loader to remove vegetation and create temporary haul road for dump trucks; hand crews present on site; surface water not diverted during field operations; no BMPs utilized due to limited access and excessive width of L.A. River at downstream limit of project; work estimated to be completed on Wednesday, 11/21.

- Upstream sampling point, #1: located on the west bank of the L.A. River about 85’ s/o the south side of the Willow Street bridge over the L.A. River; sampling point located at the end of the trapezoidal concrete channel and beginning of the SBC; access to sampling point from the west end of 34th Street, s/o Wardlow Road, on the east side of L.A. River about .95 mile n/o Willow Street, then down access ramp into concrete channel and travel south, crossing river on the south side of the Willow St. Bridge to sampling point on the west bank; see Figure 1 and Photo 1.

- Internal sampling point, #2: located on the west bank of the L.A. River about 1175’ s/o the south side of the Willow Street bridge over the L.A. River at the petroleum pipeline support structure; sampling point on west bank between the 2nd and 3rd supports; access to sampling point from Point #1 along temporary haul road on west bank of river; see Figure 2 and Photo 2.

- Downstream sampling point, #3: located on the west bank of the L.A. River directly below the north side of the Pacific Coast Highway (PCH) bridge over the river; access to sampling point at service road to levee from w/b PCH to n/b 710 on-ramp, then over concrete wall and down rip-rap slope; see Figure 3 and Photo 3.

C:/My Files/WQ-Soft Bottom Channels/LA River-West/Fall 2012/wqmonitsheet.092412