LOS ANGELES COUNTY DEPARTMENT OF PUBLIC WORKS

GEOTECHNICAL AND MATERIALS ENGINEERING DIVISION

2012 WATER QUALITY MONITORING FOR

SOFT BOTTOM CHANNEL CLEANOUTS

Channel/Reach: Los Angeles River – West Side/Reach 25

PCA: F6057919
Date: 10/01/2012
	SAMPLE LOCATION
	Upstream of Project*
	Within Project*
	Downstream of Project*

	TIME#
	1043
	1018
	1001

	SAMPLE NO.
	LARW-1A
	LARW-2A
	LARW-3A

	TEMPERATURE (°F)
	84.2
	77.9
	74.6

	pH
	6.94
	6.95
	6.98

	TURBIDITY (NTUs)
	4.69
	6.28
	4.01

	DISSOLVED O2 (mg/L)
	13.61
	7.801
	4.561

	TOTAL SUSPENDED SOLIDS (mg/L)
	1311
	54.01
	36.01


· * – Upstream of Project = 1, Within Project = 2, and Downstream of Project = 3 on sample labels and lab reports

· # – A = morning and P = afternoon on sample labels and lab reports

· Samples for Dissolved Oxygen and Total Suspended Solids were analyzed by GMED’s As-Needed Contract Environmental Laboratories, Sierra Analytical Labs, Inc. (Sierra); 1 – Reference: Sierra Lab Report No. 1210005; ND – not detected at or above the Method Detection Limit
· Field Equipment Used: Hanna Model HI 991300 pH/EC/TDS/T Probe; LaMotte 2020e Turbidimeter

FIELD NOTES:
· Weekly monitoring; met with FMD personnel from Imperial Yard on Monday, 10/01 to discuss cleanout operations, scope of project, estimated number of days to complete the project, and water quality monitoring results; using bulldozers and loader to remove vegetation and create temporary haul road for dump trucks; hand crews present on site; surface water not diverted during field operations; no BMPs utilized due to limited access and excessive width of L.A. River at downstream limit of project; work estimated to be completed on Wednesday, 11/21.
· Upstream sampling point, #1: located on the west bank of the L.A. River about 85’ s/o the south side of the Willow Street bridge over the L.A. River; sampling point located at the end of the trapezoidal concrete channel and beginning of the SBC; access to sampling point from the west end of 34th Street, s/o Wardlow Road, on the east side of L.A. River about .95 mile n/o Willow Street, then down access ramp into concrete channel and travel south, crossing river on the south side of the Willow St. Bridge to sampling point on the west bank; see Figure 1 and Photo 1.

· Internal sampling point, #2: located on the west bank of the L.A. River about 1175’ s/o the south side of the Willow Street bridge over the L.A. River at the petroleum pipeline support structure; sampling point on west bank between the 2nd and 3rd supports; access to sampling point from Point #1 along temporary haul road on west bank of river; see Figure 2 and Photo 2.
· Downstream sampling point, #3: located on the west bank of the L.A. River directly below the north side of the Pacific Coast Highway (PCH) bridge over the river; access to sampling point at service road to levee from w/b PCH to n/b 710 on-ramp, then over concrete wall and down rip-rap slope; see Figure 3 and Photo 3.

C:/My Files/WQ-Soft Bottom Channels/LA River-West/Fall 2012/wqmonitsheet.100112
