ATTACHMENT A

ACTIVITIES THAT CONSTITUTE ROUTINE MAINTENANCE IN WATERWAYS, PONDS AND LAKES IN THE EAST BAY REGIONAL PARK DISTRICT WATERSHEDS

The following activities, with their conditional requirements, are accepted as the routine maintenance activities that may be conducted by East Bay Regional Park District (EBRPD) within all streams, channels, catchment basins, ponds, and lakes within their Alameda and Contra Costa Counties wastersheds.

- Routine removal of the minimum vegetation to insure the proper functioning and operation of stream flow measuring stations and existing water control facilities or structures necessary for public health, safety and benefit, provided that heavy equipment shall not be used in the streambed and trees over 4 inches diameter at breast height (dbh) will not be removed.

- Planting of riparian vegetation by hand or with rubber-tired backhoe along gravel bars and banks of EBRPD lakes and banks of their tributaries, is subject to the following conditions:  (a) equipment shall not be operated in standing or flowing water; (b) trenches shall be excavated, planted and backfilled on the same day; (c) fueling shall not take place in the stream channels, on lake beds nor within 100 feet of open water; and (d) access shall be by existing access ramps only.

- The following sites may be dredged as needed to remove accumulated sediment and debris.  Dredging shall be done with a crawler excavator and limited to 200 cubic yards annually per site with less than 0.05 acre of wetland or waterbody impact for each single dredging activity.  Erosion, sediment and turbidity control measures and procedures shall be implemented to minimize siltation and turbidity downstream of the siltation basins during dredging operations.  EBRPD shall not cause suspended solids of the water column downstream of the siltation basins to increase more than 10% of the background levels.  Background level is equal to the turbidity of the stream immediately upstream of the siltation basin.  Sediment that is removed shall be hauled away to a landfill or other appropriate upland site for disposal.  Removal of riparian vegetation shall be minimized during dredging operations.  Routinely dredged sites include:

  - Lake Temescal Regional Park Siltation Basins located near the confluence of Caldecott Creek and Temescal Creek;

  - Tilden Nature Area man-made ecological interpretive ponds (4) which are fed with piped-in water (EBMUD);

  - Tilden Nature Area siltation basins (3), located in Wildcat Creek, specifically installed to protect Jewel Lake;

  - Lake Chabot Regional Park marina located at the confluence of a small ephemeral stream; and

  - Miller Knox Regional Park man-made pond, which is fed with pumped-in bay water.

4.	Removal of woody and herbaceous vegetation with hand tools or hand power tools in the stream bottom in that portion of the channel from the toe of one bank to the toe of the opposite bank.  Only that vegetation representing a bank erosion and/or flood threat shall be removed.  All such removal shall be in the dry stream channel when there is not flowing nor standing water at the removal sites.  No trees over 4 inches diameter at breast height (dbh) will be removed.

5.		Removal of fallen trees, branches, rubbish, garbage and associated debris from the stream channel, banks and culverts.  This is allowed only when material represents a bank erosion and/or flood threat.  Wherever reasonably possible, this activity shall be restricted to and/or staged from the dry streambanks and upland areas to keep in-stream disturbance and turbidity to a minimum.

- Removal of non-native, invasive vegetation (Arundo, tree tobacco, castor bean, pampas grass, eucalyptus, acacia, broom, etc.)

- Repair or replacement of damaged or failed sections of rock riprap, gabion, geocell, sacked concrete, concrete wall and/or cribwall bank revetments to maintain bank stabilization.  These activities shall be confined to the damaged or   failed sections and immediate adjacent bank area (not to exceed an additional 30 feet total) affected by the damage failure.  Routine revetment repair or replacement shall be conducted only when the channel is dry and only during the period of April 15 to October 31.  Riparian trees shall be protected from damage to the greatest extent possible during revetment repair and replacement.  Repair or replacement will utilize less bank hardening materials, and/or more bio-technical materials, as much as possible.

- Routine maintenance or replacement of culverts in stream channels associated with park trails and access roads and installation of energy dissipaters, headwalls, and tailwalls on new and existing culverts.  These activities shall be conducted only when the channel is dry and only during the period of April 15 to October 31.

- Annual swim beach sand recapture and maintenance.  These activities shall occur only in beach areas above lake level.  Sand shall be re-spread across the beach area above water line using a bulldozer.

- Maintenance of existing bridges and installation of clear span bridges.

- Maintenance of existing stream fords and installation of articulated concrete blocks for small stream crossings.