TABLE 2B –CONTINUOUS CIRCULATION MONITORING FOR BAUMBERG PONDS

| Sampling Station: | D.O. | pH | Temp | Salinity | Metals | Sample Function |
| --- | --- | --- | --- | --- | --- | --- |
| B-2-1 | A | A | A | A | C | Discharge |
| B-2-2a | B | B | B | B |  | Receiving Water |
| B-2-2b | B | B | B | B |  | Receiving Water |
| B-2-2c | B | B | B | B |  | Receiving Water |
| B-2-3a | B | B | B | B |  | Receiving Water |
| B-2-3b | B | B | B | B |  | Receiving Water |
| B-2-3c | B | B | B | B |  | Receiving Water |
| B-2C-1 | A | A | A | A | C | Discharge |
| B-2C-2 | A | A | A | A |  | Receiving Water |
| ##### B-2C-3 | A | A | A | A |  | Receiving Water |
| B-2C-4 | A | A | A | A |  | Receiving Water |
| B-2C-5 | A | A | A | A |  | Receiving Water |
| B-2C-7 | A | A | A | A |  | Receiving Water |
| B-6A-1 | A | A | A | A | C | Discharge |
| B-6A-2 | A | A | A | A |  | Receiving Water |
| B-6A-3 | A | A | A | A |  | Receiving Water |
| ##### B-6A-6 | A | A | A | A |  | Receiving Water |
| B-6A-7 | A | A | A | A |  | Receiving Water |
| B-8A-1 | A | A | A | A | C | Discharge |
| B-8A-2 | A | A | A | A |  | Receiving Water |
| B-8A-3 | A | A | A | A |  | Receiving Water |
| B-8A-4 | A | A | A | A |  | Receiving Water |
| B-10-1 | A | A | A | A | C | Discharge |
| B-11-1 | A | A | A | A | C | Discharge |
| B-10-11-2 | A | A | A | A |  | Receiving Water |
| B-10-11-3 | A | A | A | A |  | Receiving Water |
| B-10-11-4 | A | A | A | A |  | Receiving Water |
| B-10-11-5 | B | B | B | B |  | Receiving Water |

LEGEND FOR TABLE 2B

A = 	Receiving water samples shall be collected at discrete locations near the surface and bottom from downstream to upstream of the discharge point.  Receiving water slough samples shall be collected monthly from May through October as close to low tide as practicable.  Discharge pond samples shall be collected before pond water mixes with receiving water using a continuous monitoring device from May through October.  If dissolved oxygen levels at the discharge point fall below a 10th percentile of 3.3 mg/L (calculated on a calendar weekly basis), the Discharger shall accelerate receiving water monitoring to weekly, notify Water Board staff, and implement best management practices to increase dissolved oxygen levels in discharged water (including within pond monitoring).  For days it collects receiving water samples, the Discharger shall also report standard observations, as described in Section D of the SMP, and document if it collect samples at flood tide, ebb tide, or slack tide.  Additionally, the Discharger shall record a daily estimate of the quantity and time-period of discharge based on pond water levels and the strength of tides.

B =  Receiving water samples shall be collected at discrete locations near the surface and bottom from downstream to upstream of the discharge point.  Receiving water bay samples shall be collected monthly from May through October as close to low tide as practicable.  For days it collects receiving water samples,

LEGEND FOR TABLE 2B - Continued

the Discharger shall also report standard observations, as described in Section D of the SMP, and document if it collect samples at flood tide, ebb tide, or slack tide.

C =  Water column samples for total and dissolved arsenic, chromium, nickel, copper, zinc, selenium, silver, cadmium, lead, and mercury shall be collected annually in August or September.  When collecting metals samples, the Discharger shall also monitor for salinity, and total suspended solids.