Fact Sheet Attachment 5

General Basis for Final Compliance Dates [1]

for Discharges North of the Dumbarton Bridge

# Revised March 21, 2005

| Constituent | Reference for applicable standard | Maximum compliance schedule allowed | Compliance date and Basis |
| --- | --- | --- | --- |
| Cyanide Selenium | NTR | 10 years | April 28, 2010 (10 years from effective date of SIP). Basis is the SIP. |
| Copper (salt) | CTR | 5 years | May 18, 2010 (this is 10 years from effective date of CTR/SIP). Bases are CTR and SIP. |
| Cadmium (fresh) Mercury PAH EPA 610 | Numeric Basin Plan (BP) | 10 years | April 28, 2010, which is 10 years from effective date of SIP (April 28, 2000). Basis is the Basin Plan, See note [2a]. |
| Arsenic Cadmium (salt) Chromium (VI) Copper (fresh) Lead Nickel Silver (CMC) Zinc | Numeric BP | 10 years | January 1, 2015. This is 10 years (using full months) from effective date of 2004 BP amendment (January 5, 2005). Basis is the Basin Plan section 4.3.5.6. See note [2b]. Also, see note [3] for permits issued prior to effective date of 2004 BP amendment. |
| Dioxins/Furans Tributyltin Other toxic pollutants not in CTR | Narrative BP using SIP methodology | 10 years | 10-yr from effective date of permit (which is when new standard is adopted; no sunset date). Basis is the Basin Plan, see note [2c]. |
| Other priority pollutants on CTR and not listed above | CTR | 5 years | May 18, 2010 (this is 10 years from effective date of CTR/SIP). Basis is the CTR and SIP. |

[1] These dates are maximum allowable compliance dates applicable.  As required by the Basin Plan, CTR, SIP, and 40CFR122.47, compliance should be as short as possible.  These are only applicable for discharges north of the Dumbarton Bridge because applicable criteria for the south bay are different than those cited above.

- For pollutants where there are planned TMDLs or SSOs, and final WQBELs may be affected by those TMDLs and SSOs, maximum timeframes may be appropriate due the uncertain length of time it takes to develop the TMDL/SSO.

- However, for pollutants without planned TMDLs or SSOs, the State Water Board in the EBMUD remand order (WQO 2002-0012), directs the Regional Water Board to establish schedules that are as short as feasible in accordance with requirements.

[2] The Basin Plan provides for a 10-year compliance schedule for implementation of measures to comply with new standards as of the effective date of those standards.  This provision has been construed to authorize compliance schedules for new interpretations of existing standards, such as the numeric and narrative water quality objectives specified in the Basin Plan, if the new interpretations result in more stringent limits than in the previous permit.

a.  For the numeric objectives in place since the 1995 Basin Plan, due to the adoption of the SIP, the Regional Water Board has newly interpreted these objectives.  The effective date of this new interpretation is the effective date of the SIP (April 28, 2000) for implementation of these numeric Basin Plan objectives.

b.  For numeric objectives for the seven pollutants adopted in the 2004 Basin Plan (amendments), the Regional Water Board has newly adopted these objectives.  The effective date of these new objectives is the approval date of the 2004 Basin Plan by U.S. EPA (January 5, 2005) for implementation of these numeric Basin Plan objectives. December is the last full month directly preceding the sunset date. Compliance should be set on the first day of the month to ease determination of monthly average limits.  Therefore, compliance must begin on January 1, 2015.

- For narrative objectives, the Regional Water Board must newly interpreted these objectives using best professional judgment as defined in the Basin Plan for each permit.   Therefore, the effective date of this new interpretation will be the effective date of the permit.

[3] The schedules established in permits effective prior to the 2004 Basin Plan (amendments) should be continued into subsequent permits reissued after the 2004 Basin Plan. For example, Permit XX, adopted Nov 2004 became effective Feb 1, 2005. Permit XX establishes a compliance schedule for copper to end April 1, 2010. When next reissued in 2010, the compliance deadline for the same copper limit should remain April 1, 2010. However, if in applying the 2004 BP objective results in a more stringent limit for copper, then a new compliance schedule may extend to the new date in2015, provided discharger XX justifies the need for the longer compliance schedule.