# ATTACHMENT 1

CALCULATIONS FOR PRODUCTION-BASED

BPT, BCT, AND BAT EFFLUENT LIMITATIONS

FOR

TESORO GOLDEN EAGLE REFINERY

References:

- 40 CFR § 419 Subpart B Effluent Limitations Guidelines and New Source Performance Standards for the Petroleum Refining Point Source Category (Cracking Subcategory)

- Development Document for Effluent Limitations Guidelines and New Source Performance Standards for the Petroleum Refining Point Source Category

- Guide for the Application of Effluent Limitations Guidelines for the Petroleum Refining Industry

- NPDES Application for Permit Reissuance (August 2004)

- Refinery Production Data 1999 – 2003, provided by the facility (Data from 2003 was selected as the high year based on average production rates and was used in calculations)

Production-Based Effluent Limitations

STEP 1:	Determine the size factor based on the refinery feedstock rate.  Based on 40 CFR § 419 Subpart B, a total refinery throughput of 157 kbbl/d results in a

SIZE FACTOR = 1.41

STEP 2:	Determine the process configuration based on the process rates:

| Process | Process Feedstock Rate (kbbl/d) | Fraction of Total Throughput | Weight Factor | Process Configuration |
| --- | --- | --- | --- | --- |
| Total Refinery Throughput = 157 kbbl/d |  |  |  |  |
| CRUDE: |  |  |  |  |
| Atmospheric Distillation | 149.6 | 0.953 |  |  |
| Vacuum Crude Distillation | 56.9 | 0.362 |  |  |
| Desalting | 149.6 | 0.953 |  |  |
| TOTAL | 356 | 2.27 | 1 | 2.27 |
| CRACKING & COKING: |  |  |  |  |
| Fluid Catalytic Cracking | 66 | 0.42 |  |  |
| Hydrocracking | 27.3 | 0.174 |  |  |
| Coking | 42.7 | 0.272 |  |  |
| Hydrotreating | 128 | 0.815 |  |  |
| TOTAL | 264 | 1.68 | 6 | 10.08 |
| TOTAL PROCESS CONFIGURATION = |  |  |  | 12.35 |

(kbbl/d = Thousand Barrels per day)

STEP 3:	Determine the process factor.  Based on 40 CFR § 419 Subpart B, a total process configuration of 12.35 results in a

PROCESS FACTOR = 1.89

STEP 4:	Based on 40 CFR § 419.22(a), 419.23(a), and 419.24(a), the BPT/BAT/BCT effluent limit is equal to

(THROUGHPUT) X (SIZE FACTOR) X (PROCESS FACTOR) X (EFFLUENT LIMIT FACTOR)

EFFLUENT LIMIT = (157)(1.41)(1.89)(Effluent Limit Factor)

= (418.4)(Effluent Limit Factor)

| Pollutant | Effluent Limit in 40 CFR 419B |  |  |  |  |  | Multiplier | Final Limit Calculated |  |  |  |  |  | Final Limit |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BPT |  | BAT |  | BCT |  |  | BPT |  | BAT |  | BCT |  |  |  |
|  | Daily Max | 30-d Avg | Daily Max | 30-d Avg | Daily Max | 30-d Avg |  | Daily Max | 30-d Avg | Daily Max | 30-d Avg | Daily Max | 30-d Avg | Daily Max | 30-d Avg |
|  | lb/kbbl | lb/kbbl | lb/kbbl | lb/kbbl | lb/kbbl | lb/kbbl |  | lb/d | lb/d | lb/d | lb/d | lb/d | lb/d | lb/d | lb/d |
| BOD5 | 9.9 | 5.5 |  |  | 9.9 | 5.5 | 418.4 | 4142 | 2301 |  |  | 4142 | 2301 | 4142 | 2301 |
| TSS | 6.9 | 4.4 |  |  | 6.9 | 4.4 | 418.4 | 2887 | 1841 |  |  | 2887 | 1841 | 2887 | 1841 |
| COD | 74 | 38.4 | 74 | 38.4 |  |  | 418.4 | 30962 | 16067 | 30962 | 16067 |  |  | 30962 | 16067 |
| O&G | 3 | 1.6 |  |  | 3 | 1.6 | 418.4 | 1255 | 669 |  |  | 1255 | 669 | 1255 | 669 |
| Phenols (4AAP)* | 0.074 | 0.036 |  |  |  |  | 418.4 | 31 | 15 |  |  |  |  | 31 | 15 |
| NH3-N | 6.6 | 3 | 6.6 | 3 |  |  | 418.4 | 2761 | 1255 | 2761 | 1255 |  |  | 2761 | 1255 |
| Sulfide | 0.065 | 0.029 | 0.065 | 0.029 |  |  | 418.4 | 27 | 12 | 27 | 12 |  |  | 27 | 12 |
| Total Cr | 0.15 | 0.088 |  |  |  |  | 418.4 | 63 | 37 |  |  |  |  | 63 | 37 |
| Hex Cr | 0.012 | 0.0056 |  |  |  |  | 418.4 | 5.0 | 2.3 |  |  |  |  | 5.0 | 2.3 |

*The BPT limits for these constituents are applicable only if they are more stringent than BAT limits (see STEP 5) below).

STEP 5:	Calculate Amended BAT limits pursuant to 40 CFR § 419.43, for phenolic compounds (4AAP), total and hexavalent chromium.  The effluent limit is equal to the sum of the products of each effluent limitation factor times the applicable process feedstock rate.

| Pollutant | Process Category | BAT Effluent Limit Factors (lb/kbbl) |  | Feedstock (kbbl/d) | Effluent Limitation (lb/d) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Daily Max. | 30-d Average |  | Daily Max. | 30-d Average |
| Phenolic Compounds (4AAP) | Crude | 0.013 | 0.003 | 356.1 | 4.63 | 1.07 |
|  | Cracking & Coking | 0.147 | 0.036 | 264 | 38.81 | 9.50 |
|  | Reforming & Alkylation | 0.132 | 0.032 | 36.3 | 4.79 | 1.16 |
|  |  |  |  | TOTAL | 48.23 | 11.73 |
|  |  |  |  | (kg/d) | 21.9 | 5.32 |
| Total Chromium | Crude | 0.011 | 0.004 | 356.1 | 3.92 | 1.42 |
|  | Cracking & Coking | 0.119 | 0.041 | 264 | 31.42 | 10.82 |
|  | Reforming & Alkylation | 0.107 | 0.037 | 36.3 | 3.88 | 1.34 |
|  |  |  |  | TOTAL | 39.22 | 13.59 |
|  |  |  |  | (kg/d) | 17.79 | 6.17 |
| Hexavalent Chromium | Crude | 0.0007 | 0.0003 | 356.1 | 0.25 | 0.11 |
|  | Cracking & Coking | 0.0076 | 0.0034 | 264 | 2.01 | 0.90 |
|  | Reforming & Alkylation | 0.0069 | 0.0031 | 36.3 | 0.25 | 0.11 |
|  |  |  |  | TOTAL | 2.51 | 1.12 |
|  |  |  |  | (kg/d) | 1.14 | 0.51 |

STEP 6:	Compare Amended BAT limitations for phenolic compounds (4AAP), total chromium, and hexavalent chromium with BPT limitations.

Except for daily maximum limitation for phenolic compounds, the above BAT limits are more stringent than the BPT limits calculated in STEP 4.  Therefore, for these constituents, the above BAT limits, the BPT limit for phenolic compounds are considered for inclusion in the permit.