               CALIFORNIA REGIONAL WATER QUALITY CONTROL BOARD�PRIVATE ��

                           SAN FRANCISCO BAY REGION

�

�ORDER NO. 99-051

NPDES NO. CAG912003



GENERAL WASTE DISCHARGE REQUIREMENTS FOR:



Discharge or Reuse of Extracted and Treated Groundwater Resulting From the Cleanup of Groundwater Polluted by Volatile Organic Compounds, and Rescission of Order No. 94-087



The California Regional Water Quality Control Board, San Francisco Bay Region, (hereinafter the Board) finds that:



�seq level0 \h \r0 ��seq level1 \h \r0 ��seq level2 \h \r0 ��seq level3 \h \r0 ��seq level4 \h \r0 ��seq level5 \h \r0 ��seq level6 \h \r0 ��seq level7 \h \r0 ��seq level0 \*arabic�1�.	General: This National Pollutant Discharge Elimination System (NPDES) general permit regulates discharge or reuse of extracted and treated groundwater resulting from the cleanup of groundwater polluted by volatile organic compounds (VOC). All dischargers eligible for this general permit must submit a Notice of Intent (NOI) described in the attachment and appropriate annual fee to obtain coverage. Written authorization to initiate the discharge will be issued by the Executive Officer.



�seq level0 \*arabic�2�.	Authority: States may request authority to issue general NPDES permits pursuant to Code of Federal Regulations, Title 40, Chapter 1, Subchapter D, part 122.28 (40 CFR 122.28). On June 8, 1989, the State Water Resources Control Board (hereinafter State Board) submitted an application to the United States Environmental Protection Agency (hereinafter U.S.EPA) requesting revisions to its NPDES program in accordance with 40 CFR 122.28, 123.62 and 403.10. The application included a request to add general permit authority to its approved NPDES program. On September 22, 1989, the U.S.EPA, Region IX, approved the State Board's request and granted authorization for the State to issue general NPDES permits.



�seq level0 \*arabic�3�.	Types of Discharges: 40 CFR 122.28 provides for the issuance of general permits to regulate discharges of waste which result from similar operations, are the same types of waste, require the same effluent limitations, require similar monitoring, and are more appropriately regulated under a general permit rather than individual permits.



�seq level0 \*arabic�4�.	Eligibility for General Permit: A general permit for existing and proposed discharges of extracted and treated groundwater to surface waters of the San Francisco Bay Region (except for direct discharges to the Pacific Ocean) from groundwater cleanup projects meets the requirements of 40 CFR 122.28.  The discharges and proposed discharges:





	�seq level1 \*alphabetic�a�.	result from similar operations (all involve extraction, treatment, and discharge of groundwater),

	�seq level1 \*alphabetic�b�.	are the same types of waste (all are groundwater containing volatile organic compounds due to leaks and spills from industries which utilize VOC in manufacturing),

	�seq level1 \*alphabetic�c�.	require similar effluent limitations for the protection of the beneficial uses of surface waters in the San Francisco Bay Region (this general permit does not cover direct discharges to the Pacific Ocean),

	�seq level1 \*alphabetic�d�.	require similar monitoring, and

	�seq level1 \*alphabetic�e�.	are more appropriately regulated under a general permit rather than individual permits.



	Therefore, this Order establishes a general permit regulating extracted and treated groundwater discharges resulting from the cleanup of groundwater polluted by VOC and other related wastes. Entities which fall into this category are hereinafter referred to as discharger(s) and may be regulated by this Order. The following VOC-cleanup discharges are normally not eligible for coverage: discharges from cleanups involving significant contamination by metals, pesticides, or other conservative pollutants; discharges from cleanups involving reinjection of treated groundwater; and discharges from sites with other NPDES discharges (e.g. process waste or stormwater).



�seq level0 \*arabic�5�.�seq level1 \h \r0 �	Existing Permit: On July 20, 1994, the Board adopted Order No. 94-087 (NPDES No. CAG912003) allowing the VOC discharge or reuse of extracted and treated groundwater resulting from the cleanup of groundwater polluted by VOC.  The expiration date for Order 94-087 was July 20, 1999. During the period July 1994 to July 1999, more than 95 discharges were authorized under Order No. 94-087.



�seq level0 \*arabic�6�.	Benefits of General Permit: There are hundreds of VOC-contaminated sites within the San Francisco Bay Region.  Within the next five years, approximately 100 of these sites will be conducting groundwater cleanups by extracting contaminated groundwater, treating, and discharging treated groundwater, particularly in Santa Clara County.  Because some publicly owned treatment works (POTWs) do not accept new discharges from groundwater cleanups, many of these sites will require waste discharge requirements for discharge to surface water.  These cleanups will exceed the capacity of available staff to develop and bring individual waste discharge requirements to the Board for adoption. These circumstances create the need for an expedited system to process the anticipated numerous requests. The renewal of the 1994 general NPDES permit would: expedite the processing of requirements; enable the Board to better utilize limited staff resources; and permit cleanups to begin promptly.



�seq level0 \*arabic�7�.	Annual Fees: California Regulations establish an annual fee schedule dated May 18, 1995 based on the discharges' Threat To Water Quality and Complexity.  The dischargers to be regulated under this General Permit are classified as category 2-B:



	* Category 2 Threat To Water Quality - Those discharges of waste which could impair the designated beneficial uses of the receiving water, cause short-term violations of water quality objectives, cause secondary drinking water standards to be violated, or cause a nuisance; and



	* Category B Complexity - Any discharger not included in the major discharger category A, but has physical, chemical, or biological treatment system (except for septic systems with subsurface disposal), or any Class II or Class III waste management Units.



�seq level0 \*arabic�8�.	Basin Plan: The Board adopted a revised Water Quality Control Plan for the San Francisco Bay Basin (hereinafter called Basin Plan) on June 21,1995.  This updated and consolidated plan represents the Board's master water quality control planning document.  The revised Basin Plan was approved by the State Board on July 20, 1995 and the Office of Administrative Law on November 13, 1995.  The Office of Administrative Law's action is published in Section 3912 of Title 23 of the California Code of Regulations.  The Basin Plan defines beneficial uses and water quality objectives for waters of the State, including surface waters and groundwaters.



�seq level0 \*arabic�9�.	Beneficial Uses: The Basin Plan defines beneficial uses and water quality objectives for surface waters and groundwaters within the San Francisco Bay Region. Existing and potential beneficial uses are shown in the following table:

�PRIVATE ��Beneficial Uses�Surface waters �Ground-waters��Municipal and Domestic Supply�X�X��Industrial Service Supply�X�X��Industrial Process Supply�X�X��Agricultural Supply�X�X��Freshwater Replenishment��X��Groundwater Recharge�X���Water Contact and Non-Contact Recreation�X���Wildlife Habitat�X���Cold Freshwater and Warm Freshwater Habitat�X���Fish Migration and Fish Spawning�X���Navigation�X���Marine Habitat�X���Estuarine Habitat�X���Shellfish Harvesting�X���Ocean, Commercial, and Sport Fishing�X���Areas of Special Biological Significance�X���Preservation of Rare and Endangered Species�X���

�seq level0 \*arabic�10�.	Reuse Policy: The Board adopted Resolution No. 88�160 on October 19, 1988. The Resolution urges dischargers of extracted groundwater from site cleanup projects to reclaim their effluent and that when reclamation is not technically and/or economically feasible, to discharge to a publicly owned treatment works (POTW). If neither reclamation nor discharge to a POTW is technically or economically feasible and if beneficial uses of the receiving water are not adversely affected, it is the intent of the Board to authorize the discharge of treated extracted groundwater in accordance with the requirements of this Order.



�seq level0 \*arabic�11�.	Reuse Allowed: This Order permits reuse or reclamation of extracted treated groundwater in conjunction with the discharge to surface water, except for purposes of recharge or reinjection. Reuse of extracted treated groundwater can take many forms, such as:



	�seq level1 \*alphabetic�a�.	irrigation of landscaping or agriculture

	�seq level1 \*alphabetic�b�.	dust control or soil compaction on construction sites

	�seq level1 \*alphabetic�c�.	decorative pond or fountain supply

	�seq level1 \*alphabetic�d�.	industrial water supply



�seq level0 \*arabic�12�.�seq level1 \h \r0 �	Basin Plan Prohibition: The Basin Plan prohibits discharge of "wastewater which has particular characteristics of concern to beneficial uses": (a) "at any point at which the wastewater does not receive a minimum initial dilution of at least 10:1, or into any nontidal water, dead�end slough, similar confined waters, or any immediate tributaries thereof" and (b) at any point in "San Francisco Bay south of the Dumbarton Bridge". The Basin Plan allows for exceptions to this prohibition if a discharge is approved as part of a groundwater clean-up project in accordance with Resolution No. 88-160, it has been demonstrated that neither reclamation nor discharge to a POTW is technically and economically feasible, and the discharger has provided certification of the adequacy and reliability of treatment facilities and a plan that describes procedures for proper operation and maintenance of all treatment facilities.  The Basin Plan also prohibits discharge of "all conservative toxic and deleterious substances, above those levels which can be achieved by a program acceptable to the Board, to waters of the Basin."  Prior to discharge under this permit, dischargers must demonstrate to the satisfaction of the Executive Officer that their groundwater extraction and treatment systems and associated operation, maintenance, and monitoring plans constitute acceptable programs for minimizing the discharge of toxic substances to waters of the State.



�seq level0 \*arabic�13�.	Anti-degradation Policies: Federal Regulations (40 CFR 131.12) and State Board Resolution No. 68�16, "Statement of Policy with Respect to Maintaining High Quality of Waters in California" requires that any increase in pollutant loading to a receiving water shall be consistent with the following:



	�seq level1 \*alphabetic�a�.	Existing instream water uses and the level of water quality necessary to protect existing beneficial uses shall be maintained and protected; and



	�seq level1 \*alphabetic�b�.	Where the quality of the waters exceeds levels necessary to support propagation of fish, shellfish, and wildlife and recreation in and on the water, the quality shall be maintained and protected unless the State finds, after full satisfaction of the intergovernmental coordination and public participation provisions of the State's continuing planning process, that allowing lower water quality is necessary to accommodate important economic or social development in the area in which the waters are located.



�seq level0 \*arabic�14�.�seq level1 \h \r0 �	Anti-degradation Results: This permit complies with State and Federal "antidegradation" policies:



	�seq level1 \*alphabetic�a�.	The conditions and effluent limitations established in this Order for discharges of treated groundwater to surface waters in this Region ensure that the existing beneficial uses and quality of surface waters in this Region will be maintained and protected; and



	�seq level1 \*alphabetic�b�.	Discharges regulated by this Order should not lower water quality if the terms and conditions of this Order are met.



�seq level0 \*arabic�15�.�seq level1 \h \r0 �	No Preemption: This Order permits the discharge of treated groundwater to waters of the State subject to the prohibitions, effluent limitations, and provisions of this Order.  It does not pre�empt or supersede the authority of municipalities, flood control agencies, or other local agencies to prohibit, restrict, or control discharges of waste to storm drain systems or other watercourses subject to their jurisdiction.



�seq level0 \*arabic�16�.	Basis for Effluent Limits: Effluent limitations in this order are based on the existing permit, the Basin Plan, State plans and policies, U.S. EPA guidance, best available treatment technology economically achievable, best management practices, and best professional judgment.



�seq level0 \*arabic�17�.	CEQA: This Order serves as an NPDES Permit, adoption of which is exempt from the provisions of Chapter 3 (commencing with Section 21100) of Division 13 of the Public Resources Code (California Environmental Quality Act) pursuant to Section 13389 of the California Water Code.



�seq level0 \*arabic�18�.	Notice: The Board has notified interested agencies and persons of its intent to issue general waste discharge requirements for groundwater dewatering discharges resulting from the cleanup of groundwater polluted by VOC, and has provided them with an opportunity to submit their written views and recommendations.



�seq level0 \*arabic�19�.	Hearing: The Board, in a public meeting, heard and considered all comments pertaining to the discharge.



IT IS HEREBY ORDERED that dischargers of treated groundwater polluted by VOC, in order to  meet the provisions contained in Division 7 of the California Water Code and regulations adopted thereunder and the provisions of the Clean Water Act as amended and regulations and guidelines adopted thereunder, shall comply with the following:



A. Discharge Prohibitions

�

�seq level0 \h \r0 ��seq level1 \h \r0 ��seq level2 \h \r0 ��seq level3 \h \r0 ��seq level4 \h \r0 ��seq level5 \h \r0 ��seq level6 \h \r0 ��seq level7 \h \r0 ��seq level0 \*arabic�1�.	The discharge of extracted and treated groundwater polluted by VOC and related wastes to surface waters is prohibited unless an NOI application for proposed discharge for the discharge has been submitted and the Executive Officer has provided the discharger with written authorization to initiate the discharge.

�seq level0 \*arabic�2�.	The discharge shall be limited to extracted and treated groundwater and added treatment chemicals approved by the Executive Officer which do not adversely affect the environment and comply with the requirements of this Order.



�seq level0 \*arabic�3�.	The discharge of extracted and treated groundwater from a specific site in excess of the flow rate specified in each discharger's authorization letter from the Executive Officer is prohibited, unless an increase in gallons per day is approved by the Executive Officer.





B. Effluent Limitations (Surface water discharges only)



��seq level0 \h \r0 ��seq level1 \h \r0 ��seq level2 \h \r0 ��seq level3 \h \r0 ��seq level4 \h \r0 ��seq level5 \h \r0 ��seq level6 \h \r0 ��seq level7 \h \r0 ��seq level0 \*arabic�1�.	The effluent (at a point after full treatment but before it joins or is diluted by any other waste stream, body of water, or substance) shall not contain constituents in excess of the following:



	�seq level1 \*alphabetic�a�. ORGANIC COMPOUNDS - CONCENTRATION LIMITS



�PRIVATE ����ADVANCE \D 4.50�Constituent�Instantaneous Maximum Limit (ug/l)	�Method of Analysis��1�l,l,l�Trichloroethane�5.0�US EPA Method 8260*��2�Tetrachloroethylene�5.0���3�Trichloroethylene�5.0���4�l,l�Dichloroethylene�5.0���5�1,2�Dichloroethane�0.5���6�Vinyl Chloride�0.5���7�1,2�Dichloroethylene isomers�5.0���8�l,l�Dichloroethane�5.0���9�1,1,2�Trichloroethane�5.0���10�Methylene Chloride�5.0���11�Chloroform�5.0���12�Carbon Tetrachloride�0.5���13�Benzene�1.0���14�Toluene�5.0���15�Ethylbenzene�5.0���16�Total Xylenes�5.0���17�Any Other VOC�5.0���18��ADVANCE \D 4.50�Total Petroleum Hydrocarbons��ADVANCE \D 4.50�50.0�Modified US EPA Method 8015*��19��ADVANCE \D 4.50�Ethylene Dibromide ��ADVANCE \D 4.50�0.05�US EPA Method 504*��20��ADVANCE \D 4.50�Total Polynuclear Aromatic Hydrocarbons (PAHs)��ADVANCE \D 4.50�15.0�US EPA Method 610*��21��ADVANCE \D 4.50�Semi-VOC��ADVANCE \D 4.50�5.0�US EPA Method 625*��* or its equivalent�����



	�seq level1 \*alphabetic�b�. INORGANIC COMPOUNDS - MASS LIMITS



�PRIVATE ��	�ADVANCE \D 4.50���	Mass Limit (grams/day) by flow range*:����No.��ADVANCE \D 4.50�Constituent�Flows less than 10 gpm�Flows 10 to 100 gpm�Flows over 100 gpm��1�Arsenic�1�3�10��2�Cadmium�1�2�4��3�Chromium (VI)**�2�6�20��4�Copper�3�6�10��5�Lead�5�6�10��6�Mercury�0.01�0.1�0.5��7�Nickel�5�30�40��8�Selenium�2�20�45��9�Silver�1�3�10��10�Zinc�10�70�200��* Based on average flow computed from last 12 months of operation

** Dischargers, at their option, may meet this limit as total chromium������

�seq level0 \*arabic�2�.�seq level1 \h \r0 �	pH: The pH of the discharge shall not exceed 8.5 nor be less than 6.5.



�seq level0 \*arabic�3�.	Toxicity:  The survival of rainbow trout test fish in 96�hour static renewal bioassays of the discharge shall be a three sample moving median of 90% survival and a minimum value of not less than 70% survival.

�

C. Receiving Water Limitations

�

�seq level0 \h \r0 ��seq level1 \h \r0 ��seq level2 \h \r0 ��seq level3 \h \r0 ��seq level4 \h \r0 ��seq level5 \h \r0 ��seq level6 \h \r0 ��seq level7 \h \r0 ��seq level0 \*arabic�1�.	Narrative Limits: The discharge shall not cause the following conditions to exist in waters of the State at any place:



	�seq level1 \*alphabetic�a�.	Floating, suspended, or deposited macroscopic particulate matter or foam;

	�seq level1 \*alphabetic�b�.	Bottom deposits or aquatic growths;

	�seq level1 \*alphabetic�c�.	Alteration of temperature, turbidity, taste, odor, or apparent color beyond present natural background levels;

	�seq level1 \*alphabetic�d�.	Visible, floating, suspended, or deposited oil or other products of petroleum origin;

	�seq level1 \*alphabetic�e�.	Toxic or other deleterious substances to be present in concentrations or quantities that will cause deleterious effects on aquatic biota, wildlife, or waterfowl, or which render any of these unfit for human consumption either at levels created in the receiving waters or as a result of biological concentration.



�seq level0 \*arabic�2�.�seq level1 \h \r0 �	Numerical Limits: The discharge shall not cause the following limits to be exceeded in waters of the State in any place within one foot of the water surface:



	�seq level1 \*alphabetic�a�. Dissolved oxygen

		For all tidal waters:

			In the Bay downstream of Carquinez Bridge � 5.0 mg/l minimum

			Upstream of Carquinez Bridge � 7.0 mg/l minimum

		For nontidal waters:

			Waters designated as cold water habitat � 7.0 mg/l minimum

			Waters designated as warm water habitat � 5.0 mg/l minimum

		For all inland surface waters:

			The median of any three consecutive months shall not be less than 80% saturation. When natural factors cause lesser concentration(s) than those specified above, then this discharge shall not cause further reduction in the concentration of dissolved oxygen.



	�seq level1 \*alphabetic�b�. pH: Variation from natural ambient pH by more than 0.5 pH units.



�seq level0 \*arabic�3�.�seq level1 \h \r0 �	More Stringent Standards May Apply: The discharge shall not cause or contribute to a violation of any applicable water quality standard for receiving waters adopted by the Board or the State Water Resources Control Board as required by the Clean Water Act and regulations adopted thereunder. If more stringent applicable water quality standards are promulgated or approved pursuant to Section 303 of the Clean Water Act, or amendments thereto, the Board will revise and modify this Order in accordance with such more stringent standards.

�



D. Water Reclamation Specifications (water reuse only)



�seq level0 \h \r0 ��seq level1 \h \r0 ��seq level2 \h \r0 ��seq level3 \h \r0 ��seq level4 \h \r0 ��seq level5 \h \r0 ��seq level6 \h \r0 ��seq level7 \h \r0 ��seq level0 \*arabic�1�.	Water reclaimed for beneficial reuse as applied shall meet the following requirements:



�PRIVATE ���ADVANCE \D 4.50�Constituents�Instantaneous Maximum Limit (ug/l)�Method of Analysis��Benzene�0.5�US EPA Method 8260*��1,2�Dichloroethane�0.5���Vinyl Chloride�0.5���All other VOC�5.0����ADVANCE \D 4.50�Total Petroleum Hydrocarbons��ADVANCE \D 4.50�50.0�Modified US EPA Method 8015*���ADVANCE \D 4.50�Ethylene Dibromide ��ADVANCE \D 4.50�0.05�US EPA Method 504*��Semi-VOC per constituent��ADVANCE \D 4.50�5.0�US EPA Method 625*��* or its equivalent����

�seq level0 \*arabic�2�.	The water reclamation activities shall be those described in the discharger's NOI, including source of groundwater, method of treatment, and location and type of water reuse.



�seq level0 \*arabic�3�.	No reclaimed water shall be allowed to escape from the authorized use area by airborne spray, nor by surface flow except in minor amounts associated with good irrigation practice, nor from conveyance facilities.



�seq level0 \*arabic�4�.	Reclamation involving irrigation shall not occur when the ground is saturated.



�seq level0 \*arabic�5�.	The use of reclaimed water shall not impair the quality of waters of the State, nor shall it create a nuisance as defined by Section 13050(m) of the California Water Code.



�seq level0 \*arabic�6�.	Adequate measures shall be taken to minimize public contact with reclaimed water and to prevent the breeding of flies, mosquitos, and other vectors of public health significance during the process of reuse.



�seq level0 \*arabic�7�.	Appropriate public warnings must be posted to advise the public that the water is not suitable for drinking. Signs must be posted in the area, and all reclaimed water valves and outlets appropriatly labelled.



�seq level0 \*arabic�8�.	There shall be no cross-connection between the potable water supply and piping containing treated groundwater intended for reuse.



�seq level0 \*arabic�9�.	Water reclamation consisting of recharge or reinjection is not authorized under this Order.





E. Provisions

�

�seq level0 \h \r0 ��seq level1 \h \r0 ��seq level2 \h \r0 ��seq level3 \h \r0 ��seq level4 \h \r0 ��seq level5 \h \r0 ��seq level6 \h \r0 ��seq level7 \h \r0 ��seq level0 \*arabic�1�.	NOI Application: The NOI application for each point of proposed discharge to a storm sewer or storm channel shall contain the information required in attachment of this Order.



�seq level0 \*arabic�2�.	NOI Review: Upon receipt of a complete NOI application package for proposed discharge, the Executive Officer will review the application to determine whether the proposed discharger is eligible to discharge waste under this general permit:

	�seq level1 \*alphabetic�a�.	The proposed discharge results from the cleanup of groundwater polluted by VOC and similar wastes;

	�seq level1 \*alphabetic�b�.	The proposed discharge is to surface waters of the San Francisco Bay Region (except for direct discharges to the Pacific Ocean);

	�seq level1 \*alphabetic�c�.	The proposed discharger has met the provisions of Resolution No. 88�160; and

	�seq level1 \*alphabetic�d�.	The proposed treatment system and associated operation, maintenance, and monitoring plans are capable of ensuring that the discharge will meet the provisions, prohibitions, effluent limitations, and receiving water limitations of this Order.



�seq level0 \*arabic�3�.�seq level1 \h \r0 �	Discharge Authorization: If the Executive Officer determines that the proposed discharger is eligible to discharge waste under this general permit, the Executive Officer will authorize the proposed discharge. If the Executive Officer authorizes the discharge, a "discharge authorization letter" will be transmitted to the discharger authorizing the initiation of the discharge subject to the conditions of this Order and any other conditions necessary to protect the beneficial uses of the receiving waters. The discharge authorization letter from the Executive Officer will specify the maximum allowed discharge flow rate. The discharge authorization letter may be terminated or revised by the Executive Officer at any time.



�seq level0 \*arabic�4�.	Non-Compliance As A Violation: Upon receipt of the Executive Officer's discharge authorization letter, the discharger(s) shall comply with all conditions and limitations of this Order and the discharge authorization letter. Any permit noncompliance (violations of requirements in this Order or Self Monitoring Program) constitutes a violation of the Clean Water Act and the California Water Code and is grounds for enforcement action; for permit or authorization letter termination, revocation and reissuance, or modification; the issuance of an individual permit; or for denial of a renewal application.



�seq level0 \*arabic�5�.	Self�Monitoring Program: Dischargers shall comply with the Self�Monitoring Program as adopted by the Board (attached) and as amended by the Executive Officer.  Each discharger shall comply with the Self�Monitoring Program specified in the discharge authorization letter.



�seq level0 \*arabic�6�.	Proper O&M: Dischargers shall, at all times, properly operate and maintain all facilities and systems of treatment and control (and related appurtenances) which are installed or used by the dischargers to achieve compliance with the conditions in this Order and in the authorization letters from the Executive Officer.



�seq level0 \*arabic�7�.	Order Modification: This Order may be modified by the Board prior to the expiration date to include effluent or receiving water limitations for toxic constituents determined to be present in significant amounts in discharges regulated by this general permit (through the comprehensive monitoring program included as part of this Order).



�seq level0 \*arabic�8�.	Inorganic Limits Exceedance: If any inorganic effluent limit in Section B is exceeded, then the discharger shall take three additional samples for that constituent(s) during the following quarter.



	Case 1 - If the results of the three additional samples for the effluent do not exceed the effluent limit(s) the discharger shall report the results to the Executive Officer in the next Self-Monitoring Report, and shall return to the schedule of sampling and analysis in the Self-Monitoring Program.



	Case 2 - If the results of any one of the three additional samples exceed the effluent limit(s), the discharger shall perform the following:



	a)	Calculate the median and maximum concentration values for the constituent(s) of concern, using the three recent samples and all samples collected and analyzed for that constituent in the previous 12 month period.



	b)	Estimate the mass load discharged in the previous 12 month period for the constituent(s) of concern. Report the results in grams per day and in pounds per year, using the average flow rate for the previous 12 month period.



	c)	Report the results to the Executive Officer in the next Self-Monitoring Report, and return to the schedule of sampling and analysis in the Self-Monitoring Program.



	Case 3 - If the results of two or three of the additional samples exceed the effluent limit(s), the discharger shall perform the following:



	a)	Calculate median and maximum concentration values and mass load for the constituent(s) of concern, as described in Case 2 above.



	b)	Perform a cost analysis for treatment of the discharge for the constituent(s) of concern. The analysis should include, but need not be limited to, a discussion of various treatment technologies or pre-treatment filtration options, the cost and technical feasibility of increased treatment to reduce the constituent(s) of concern, and the amount of reduction in terms of concentration and average annual mass load. A joint effort may be undertaken and submitted by more than one discharger to evaluate cost and feasibility of treatment technologies or options.



		If the results of the cost analysis indicates that metals treatment of the discharge does not appear to be a feasible option, then: 



	c)	Perform an evaluation of the potential adverse impacts to the beneficial uses of the receiving water. The evaluation should include, but need not be limited to, description of the beneficial uses specific to the receiving water, physical and chemical characteristics of the water body and sediment, and the physical, chemical, or biological effects from the constituent(s) on the beneficial uses, including effects related to total or dissolved and hardness for metals with hardness-dependent objectives.



		If exceedances are only for metals with hardness-dependent objectives, then the discharger may conduct a hardness study prior to completing this task. The hardness study should assess receiving water hardness (as CaCO3) and compute a "no effect" concentration for affected metals, using (i) the  minimum of a statistically significant number of hardness samples, and (ii) hardness-dependent formula for US EPA freshwater criteria. If effluent metals concentrations fall below the computed "no effect" concentration, then the discharger need not complete the remainder of this task.



		If the receiving water study finds that the discharge is having potential adverse impacts to beneficial uses of the receiving water, then:



	d)	Evaluate control measures other than treatment to reduce the constituent(s) of concern in the discharge, such as re-evaluating options for re-use, discharge to POTW, or alternatives to groundwater extraction. 



	e)	Within 180 days of the discharger receiving results of the consecutive sampling, report the results of tasks (a) through (d) above to the Executive Officer, including:



		-	the proposed method to eliminate or minimize future non-compliance, or

		-	provide a rationale for why no change to the existing program should take place, and 

		-	return to the schedule of sampling and analysis in the Self-Monitoring Program.



	The discharger may be required to perform additional evaluations or take additional actions to minimize noncompliance, as deemed necessary by the Executive Officer. 



	If a violation of the same effluent limit occurs less than 60 months after completion of the required tasks in Cases 1, 2, or 3, then the Executive Officer may waive the evaluation required above. This waiver will not apply if a different inorganic constituent exceeds the effluent limit. In that case, the discharger shall perform an evaluation for that constituent.



�seq level0 \*arabic�9�.	Individual NPDES Permit May Be Required: The U.S.EPA Administrator may request the Board Executive Officer to require any discharger authorized to discharge waste by the general permit to subsequently apply for and obtain an individual NPDES permit. The Executive Officer of the Board may require any discharger authorized to discharge waste by a general permit to subsequently apply for and obtain an individual NPDES permit. An interested person may petition the Executive Officer or the Regional Administrator to take action under this provision. Cases where an individual NPDES permit may be required include the following:



	�seq level1 \*alphabetic�a�.	The discharger is not in compliance with the conditions of this Order or the discharge authorization letter from the Executive Officer;

	�seq level1 \*alphabetic�b�.	A change has occurred in the availability of demonstrated technology or practices for the control or abatement of pollutants applicable to the point source;

	�seq level1 \*alphabetic�c�.	Effluent limitation guidelines are promulgated for point sources covered by the general NPDES permit;

	�seq level1 \*alphabetic�d�.	A water quality control plan containing requirements applicable to such point sources is approved; or

	�seq level1 \*alphabetic�e�.	The requirements of 40 CFR 122.28(a), as explained in Finding No. 4, are not met.  



�seq level0 \*arabic�10�.�seq level1 \h \r0 �	This Order expires on July 21, 2004.  Dischargers, who need to discharge treated groundwater after July 21, 2004, must file an application for proposed discharge no later than January 21, 2004 as application for issuance of new waste discharge requirements.



�seq level0 \*arabic�11�.	The requirements prescribed by this Order supersede the requirements prescribed by Order No. 94�087.  Order No. 94�087 is hereby rescinded.  Dischargers who (i) were previously subject to Order No. 94�087, (ii) filed a complete NOI before the effective date of this Order, and (iii) have not yet received an Executive Officer authorization letter pursuant to this Order will remain subject to the requirements of Order No. 94�087 pending receipt of a new authorization letter.  This provision will assure no lapse in NPDES permit coverage for authorized discharges.



�seq level0 \*arabic�12�.	This Order shall serve as a general National Pollutant Discharge Elimination System Permit pursuant to Section 402 of the Clean Water Act or amendments thereto, and shall become effective 10 days after the date of its adoption provided the Regional Administrator, Environmental Protection Agency, has no objection. If the Regional Administrator objects to its issuance, the permit shall not become effective until such objection is withdrawn.





F. Standard Provisions

�



�seq level0 \h \r0 ��seq level1 \h \r0 ��seq level2 \h \r0 ��seq level3 \h \r0 ��seq level4 \h \r0 ��seq level5 \h \r0 ��seq level6 \h \r0 ��seq level7 \h \r0 ��seq level0 \*arabic�1�.	No Nuisance: Neither the treatment nor the discharge of pollutants shall create a pollution, contamination, or nuisance, as defined by Section 13050 of the California Water Code.



�seq level0 \*arabic�2�.	Duty to Comply



	�seq level1 \*alphabetic�a�.	If a toxic effluent standard or prohibition (including any schedule of compliance specified in such effluent standard or prohibition) is established under Section 307(a) of the Clean Water Act, or amendments thereto, for a toxic pollutant which is present in the discharge authorized herein and such standard or prohibition is more stringent than any limitation upon such pollutant in this Order, the discharger must comply with the new standard or prohibition.  The Board will revise the Order in accordance with such toxic effluent standard or prohibition and so notify the discharger.



	�seq level1 \*alphabetic�b�.	If more stringent applicable water quality standards are approved pursuant to Section 303 of the Clean Water Act, or amendments thereto, the discharger must comply with the new standard.  The Board will revise this Order in accordance with such more stringent standards.



	�seq level1 \*alphabetic�c�.	The filing of a request by the discharger for modification or termination of permit coverage, or a notification of planned changes or anticipated non-compliance does not stay any permit condition.





�seq level0 \*arabic�3�.�seq level1 \h \r0 �	Duty to Mitigate:  The discharger shall take all reasonable steps to minimize or prevent any discharge in violation of this Order which has a reasonable likelihood of adversely affecting public health or the environment, including such accelerated or additional monitoring as requested by the Board or Executive Officer to determine the nature and impact of the violation.



�seq level0 \*arabic�4�.	The discharger must notify the Board as soon as it knows or has reason to believe (1) that they have begun or expect to begin to use and discharge a pollutant not reported in the permit application, or (2) a discharge of toxic pollutants not limited by this permit has occurred or will occur in concentrations that exceed the limits specified in 40 CFR 122.42(a).



�seq level0 \*arabic�5�.	The discharge of any radiological, chemical, or biological warfare agent waste is prohibited.



�seq level0 \*arabic�6�.	Property rights:  This Order does not convey any property rights of any sort or any exclusive privileges.  The requirements prescribed herein do not authorize the commission of any act causing injury to the property of another, nor protect the discharger from liabilities under federal, state, or local laws, nor create a vested right for the discharger to continue the waste discharge, nor guarantee the discharger a capacity right in the receiving water.



�seq level0 \*arabic�7�. 	Inspection and Entry:  The Board or its authorized representatives shall be allowed:



	�seq level1 \*alphabetic�a�.	Entry upon premises where a regulated facility or activity is located or conducted, or where records are kept under the conditions of the Order;



	�seq level1 \*alphabetic�b�.	Reasonable access to and duplication of any records that must be kept under the conditions of the Order;



	�seq level1 \*alphabetic�c�.	To inspect at reasonable times any facility, equipment, practices, or operations regulated or required under the Order; and



	�seq level1 \*alphabetic�d�.	To photograph, sample, and monitor at reasonable times for the purpose of assuring compliance with the Order or as otherwise authorized by the Clean Water Act any substances or parameters at any locations.



�seq level0 \*arabic�8�.�seq level1 \h \r0 �	Duty to Provide Information:  The discharger shall furnish, within a reasonable time, any information the Board may request to determine whether cause exists for modifying, revoking and reissuing, or terminating the permit.  The discharger shall also furnish to the Board, upon request, copies of records required to be kept by its permit.



�seq level0 \*arabic�9�.	Bypass (the intentional diversion of waste streams from any portion of a treatment facility) is prohibited.  The Board may take enforcement action against the discharger for bypass unless:



	�seq level1 \*alphabetic�a�.	Bypass was unavoidable to prevent loss of life, personal injury, or severe property damage;



	�seq level1 \*alphabetic�b�.	There were no feasible alternatives to the bypass, such as shutting off the system, the use of auxiliary treatment facilities, retention of untreated waste, or maintenance during normal periods of equipment down time.  This condition is not satisfied if adequate back-up equipment should have been installed in the exercise of reasonable engineering judgement to prevent a bypass which occurred during normal periods of equipment down time or preventative maintenance; and



	�seq level1 \*alphabetic�c�.	The discharger submitted advance notice of the need for a bypass to the Board.  If the discharger knows in advance of the need for a bypass, it shall submit prior notice, if possible, at least 10 days before the bypass date.  The discharger shall submit notice of an unanticipated bypass as required elsewhere in the Self Monitoring Program (24-hour reporting).



		The discharger may allow a bypass to occur that does not cause effluent limitations to be exceeded, but only if it is for essential maintenance to assure efficient operation.  In such cases, the above bypass conditions are not applicable.



�seq level0 \*arabic�10�.�seq level1 \h \r0 �	Continuation of Expired Permit:  This permit continues in force and effect until a new permit is issued or the Board rescinds the permit.  Only those dischargers authorized to discharge under the expiring permit are covered by the continued permit.



�seq level0 \*arabic�11�.	Treatment Reliability:  The discharger shall, at all times, property operate and maintain all facilities which are used by the discharger to achieve compliance with this Order.  Proper operation and maintenance also includes adequate laboratory controls and appropriate quality assurance procedures.  All of these procedures shall be described in an Operation and Maintenance manual.  The discharger shall keep in a state of readiness all systems necessary to achieve compliance with the conditions of this Order.  All systems, both those in service and reserve, shall be inspected an maintained on a regular basis.  Records shall be kept of the tests and made available to the Board for at least five years.



�seq level0 \*arabic�12�.	Errata:  Should the discharger discover that it failed to submit any relevant facts or that it  submitted incorrect information in any report, it shall promptly submit the missing or correct information.



�seq level0 \*arabic�13�.	False Reporting: Any person who knowingly makes any false statement, representation, or certification in any record or other document submitted or required to be maintained under this permit, including monitoring reports or reports of compliance or noncompliance shall be subject to enforcement procedures as identified in Provision F.16.



�seq level0 \*arabic�14�.	Transfers:  Coverage by this permit is not transferrable to any person except after notice to the Executive Officer.  The Executive Officer may require modification of the discharge authorization letter to change the name of the permittee and incorporate such other requirements as may be necessary under the Clean Water Act.



�seq level0 \*arabic�15�.	Planned Changes:  The discharger shall file with the Executive Officer an amended Notice of Intent at least 60 days before making any material change in the character, location, or volume of the discharge.



�seq level0 \*arabic�16�.	Enforcement:  The provisions of this section shall not act as a limitation on the statutory or regulatory authority of the Board.



	�seq level1 \*alphabetic�a�.	Any violation of the permit constitutes violation of the California Water Code and regulations adopted thereunder and the provisions of the Clean Water Act and regulations adopted thereunder, and is the basis for enforcement action, revocation of permit coverage, denial of an application for continued permit coverage, or a combination thereof.



	�seq level1 \*alphabetic�b�.	The Board may impose administrative civil liability, may refer a discharger to the state Attorney General to seek civil monetary penalties, may seek injunctive relief or take other appropriate enforcement action as provided in the California Water Code or federal law for violation of this Order.



	�seq level1 \*alphabetic�c�.	It shall not be a defense for a discharger in an enforcement action that it would have been necessary to halt or reduce the permitted activity in order to maintain compliance with the conditions of this Order.



	�seq level1 \*alphabetic�d�.	A discharger seeking to establish the occurrence of an upset has the burden of proof.  A discharger who wishes to establish the affirmative defense of any upset in an action brought for non-compliance shall demonstrate through properly signed contemporaneous operating logs or other relevant evidence that: (i) an upset occurred and the permittee can identify the cause of the upset, (ii) the permitted facility was being properly operated at the time of the upset, (iii) the discharger submitted notice of the upset as required, and (iv) the discharger complied with any remedial measures required.



		No determination made before an action for non-compliance, such as during administrative review of claims that non-compliance was caused by an upset, is final administrative action subject to judicial review.



�seq level0 \*arabic�17�.�seq level1 \h \r0 �	Definitions



	�seq level1 \*alphabetic�a�.	Bypass means the intentional diversion of waste streams from any portion of the treatment facility.



	�seq level1 \*alphabetic�b�.	Overflow means the intentional or unintentional spilling or forcing out of untreated or partially treated wastes from a transport system upstream from any part of the treatment facility.



	�seq level1 \*alphabetic�c�.	Severe property damage means substantial physical damage to property, damage to the treatment facilities that causes them to become inoperable, or substantial and permanent loss of natural resources that can reasonably be expected to occur in the absence of a bypass.  It does not mean economic loss caused by delays in production.



	�seq level1 \*alphabetic�d�.	Toxic pollutant means any pollutant listed as toxic under Section 307(a) of the Clean Water Act or implementing regulations.



	�seq level1 \*alphabetic�e�.	Upset means an exceptional incident in which there is unintentional temporary non-compliance with technology-based effluent limits in the Order because of factors beyond the reasonable control of the discharger.  It does not include non-compliance to the extent caused by operational error, improperly designed treatment facilities, inadequate treatment facilities, lack of preventative maintenance, or careless or improper operation.



	�seq level1 \*alphabetic�f�.	Waste, waste discharge, discharge of waste, and discharge are used interchangeably in this Order.  The requirements of this Order apply to the entire volume of water, and the material therein, which is disposed of to surface and ground waters of the State of California.





I, Loretta K. Barsamian, Executive Officer, do hereby certify the foregoing is a full, true and correct copy of an Order adopted by the California Regional Water Quality Control Board, San Francisco Bay Region, on July 21, 1999.





                                      Loretta K. Barsamian

                                      Executive Officer











Attachments:



NOI Application Requirements

Self�Monitoring Program

�







 



 



�PAGE  �1�





�PAGE  �20�

�Order No. 99-051										    Page 

NPDES Permit No. CAG912003,  dated July 21, 1999


















