CALIFORNIA REGIONAL WATER QUALITY CONTROL BOARD

# COLORADO RIVER BASIN REGION

ORDER NO. 01-142

WASTE DISCHARGE REQUIREMENTS

FOR

RIVERSIDE COUNTY WASTE MANAGEMENT DEPARTMENT, OWNER/OPERATOR

MECCA II SANITARY LANDFILL,

CLASS III LANDFILL

East of Mecca – Riverside County

The California Regional Water Quality Control Board, Colorado River Basin Region, finds that:

- Riverside County Waste Management Department (hereinafter referred to as the discharger) 1995 Market Street, Riverside, CA 92501-1719, owns and operates the Mecca II Waste Management Facility (hereinafter referred to as the Landfill).

- The Landfill is located in Southern California approximately four (4) miles east of the town of Mecca on State Highway 195 as shown in Attachments A, and B.  It is located in the S ½ of the SW ¼ of Section 12, T7S, R9E, SBB&M as shown on Attachment C.

- Definitions of terms used in this Board Order:

  - Waste Management Facility (WMF) – The entire parcel of property at where waste discharge operations are conducted.

  - Waste Management Unit (WMU) – An area of land, or a portion of the Waste Management Facility, where waste is or was discharged. The term includes containment features and ancillary features for precipitation and drainage control and monitoring.

  - Landfill – A Waste Management Unit where waste is discharged to land. It does not include surface impoundments, waste piles, or land treatment and soil treatment.

d.  Discharger – discharger means any person who discharges waste that could affect the quality of the waters of the State, and includes any person who owns a Waste Management Unit or who is responsible for the operation of a Waste Management Unit.

    - The Mecca II Landfill consists of two (2) adjacent units: Phase I (19 acres), and Phase II (7.9 acres) as shown on Attachment D.

    - The site is on 80 acres of land. Landfilling occurs in the Phase I unit.  This active Phase is projected to receive an average of 69 tons per day (tpd) of non-hazardous waste.

    - The Landfill is located in the lower Coachella Valley, a narrow extension of the northwestern portion of the Salton Sea Trough. The Landfill is located in the northwestern part of this geological structural depression that also includes the Gulf of California. Mecca Hills, with an elevation of roughly 1,300 feet above mean sea level (MSL), borders the Landfill on the north. The Salton Sea, with an elevation of 235 feet below MSL is about 3.5 miles south of the Landfill. Elevation at the site ranges from 35 feet below MSL at the southwest corner to 40 feet above MSL at the northeast corner, a distance of about 0.5 miles.

    - The site is underlain by Quaternary alluvial fan deposits. These deposits typically consist of coarse-grained, poorly sorted sands and gravels.  The soils in the area are highly permeable.

    - The site is located near the San Andreas Fault Zone.  Large sub-parallel and branching faults present in Coachella Valley that are part of the San Andreas Fault Zone include the San Andreas, Mission Creek, Banning, Garnet Hill, Indio Hills and Mecca Hills faults.  These faults act as partial barriers to ground water movement.

    - The Landfill began accepting waste in 1982. In 1988, the Landfill became subject to Waste Discharge Requirements (WDRs) under Board Order No. 88-053. The WDRs were updated and superseded by Board Order No. 91-006. On September 15, 1993, the WDRs were amended when Board Order No. 93-071, amending all Municipal Solid Waste Landfill Board Orders to comply with federal regulations, was adopted by the Regional Board. The Discharger submitted a Report of Waste Discharge (ROWD) on October 16, 1995. The ROWD contained information to update the WDRs. The WDRs were again updated in Board Order No. 97-043 on June 25, 1997.

    - On May 3, 2001, the discharger submitted a request to modify the monitoring parameters and frequencies in Board Order No. 97-043.

    - This Board Order updates the WDRs in Board Order No. 97-043:

- To comply with current laws and regulations as set forth in the California Water Code and the combined SWRCB/CIWMB Regulations, Division 2, Title 27 (hereinafter referred to as Title 27), and

- To reflect the recent water quality related concerns and ground water monitoring reporting changes.

    - The Landfill presently accepts Class III non-hazardous solid waste. The waste types accepted at the Landfill are:

- Residential

- Commercial

- Plant Trimmings

- Agricultural

- Demolition/Construction

- Tires

- The discharger proposes to clean close the Phase II unit of the Landfill and place the excavated refuse in the Phase I unit.  Since the start of Landfill operation, approximately 348,579 cubic yards of waste have been placed in the Landfill.  The remaining capacity of the Phase I unit is approximately 29,901 cubic yards.

- The Landfill is not lined and does not have a leachate collection and removal system.

- Random load checking is performed at a minimum of 15 hours-per-month. Hazardous materials found are transported to the Central Accumulation Storage facility for temporary storage. Hazardous waste stored will be transported to an appropriate site within 90 days.

- Fuels and oils are stored on-site in storage facilities with secondary containment and then transported to a Treatment Storage and Disposal Facility.

  - The area fill method is used for waste disposal at the Landfill. Waste is compacted and covered daily.

  - The annual average precipitation and evaporation rates in the area are 3.3 inches and 100 inches, respectively.

  - The 100-year, 24-hour precipitation event for the site is 3.08 inches.

  - The site is not within the 100-year floodplain.

  - The current land use within a one (1) mile radius of the site is mainly agricultural, with some urban residential sections.

  - The main source of natural water in the Coachella Valley is percolation of surface water runoff from the adjacent mountain slopes to the ground water table.

  - The discharger reports that at least fourteen (14) ground water wells have been drilled within a one (1) mile of the site. Three (3) of these wells are used for domestic supply.

  - The Landfill is located in the Coachella Hydrologic Subunit.

  - The Water Quality Control Plan for the Colorado River Basin Region of California (Basin Plan) was adopted on November 17, 1993 and designates the beneficial uses of ground and surface waters in this Region.

  - The beneficial uses of ground waters in the Coachella Hydrologic Subunit are:

- Municipal supply (MUN)

- Industrial supply (IND)

- Agricultural supply (AGR)

- Relatively undisturbed and unconsolidated Holocene and late Pleistocene alluvial deposits are the principal reservoirs of ground water in the site area. It is within these water-bearing materials that storage of ground water occurs and from which water is readily extracted.

  - The discharger submitted a final Solid Waste Assessment Test (SWAT) report on July 2, 1990. During the SWAT investigation the discharger installed three (3) ground water monitoring wells: MII-1, MII-2, and MII-3 as shown on Attachment D.

  - Ground water occurs at a depth of approximately 167 to 195 feet below ground surface at the Landfill. The ground water elevation change between MII-1 and MII-3 is about 0.10 feet. Ground water flow direction is dependent on the pumping rate at nearby wells. The flow changes from southwesterly to northwesterly.

  - Analysis of water samples from monitoring wells during the SWAT investigation indicated that the Landfill is affecting the ground water quality in the vicinity of the site. The following is the list of parameters which exceed the drinking water standard (MCL):

Parameter	Result mg/L	Well No.

Nitrate	99	MII-2

Chloride	870	MII-3

- On April 14, 1995, the Regional Board received an Evidence of Release Notification from the Riverside County Waste Management Department indicating the release of the following volatile organic constituents (VOCs) from the Landfill:

Parameter	Result µg/L	Well No.

Dichlorodifluoromethane	2.1	MII-3

Trichlorofluoromethane	3.3	MII-3

The notification also stated that discrete retesting would be performed within 30 days (by May 12, 1995), in accordance with Section 13(f)(3) of Board Order No. 93-071.

- On July 10, 1995, the Regional Board received analytical results of discrete retesting of ground water samples taken from the Landfill on May 2, 1995 in accordance with Section (13)(c)(2) of Board Order No. 93-071. The samples indicated the presence of the following constituents in the ground water.

Parameter	Result µg/L	Well No.

Dichlorodifluoromethane	2.65	MII-3

Trichlorofluoromthane	1.72	MII-3

- On September 18, 1995, the Regional Board received analytical results of the Constituents of Concern for the Landfill indicating that the following constituents were detected in ground water:

Parameter	Result µg/L	Well No.

Dichlorodifluoromethane	2.65	MII-3

Trichlorofluoromthane	1.72	MII-3

- The discharger submitted an Evaluation Monitoring Program (EMP) on October 16, 1995. The EMP was reviewed and found to be incomplete. Additional information was requested from the discharger.

- Cleanup and Abatement Order (CAO) No. 95-128 addressing soil and ground water pollution at the Landfill was issued to the discharger on December 19, 1995.

- The discharger submitted a revised EMP on April 26, 1996 that includes the information requested by the Regional Board. The revised EMP was approved by the Regional Board’s Executive Officer on May 1, 1996.

- On June 19, 1997, CAO No. 95-128 was revised and CAO NO. 97-101 was issued by the Regional Board’s Executive Officer.

- In September 1998, the discharger installed a ground monitoring well, (MII-4) as required in CAO No. 97-101. Location of MII-4 is shown on Attachment D.

- The discharger has installed four (4) multi-level vadose zone gas detection probes around the perimeter of the Landfill in alluvium as part of the EMP work plan. The location of these probes is shown on Attachment E.

- The discharger submitted to the Regional Board, a Preliminary Closure and Post Closure Maintenance Plan (PCPCMP) on January 18, 1996, revised on December 16, 1996, and revised on August 21, 2001.

- The Regional Board finds that the 26.9-acre landfill project is in compliance with the provisions of Title 27, as well as 40 CFR Parts 257 and 258, the Federal RCRA Subtitle “D” (Subtitle D) Regulations promulgated by the U.S. Environmental Protection Agency (EPA) on October 9, 1991, effective October 9, 1993.

    - In accordance with Section 15301, Chapter 3, Division 6, Title 14 of the California Code of Regulations, the issuance of these Waste Discharges Requirements, which govern the operation of an existing facility involving negligible or no expansion of use beyond that previously existing, is exempt from the provisions of the California Environmental Quality Act (Public Resources Code, Section 21000 et. Seq.).

    - Federal Regulations for storm water discharges were promulgated by the U.S. Environmental Protection Agency on November 16, 1990 (40 CFR Parts 122, 123, and 124).  The regulations require that specific categories of facilities which discharge storm water associated with industrial activity obtain an NPDES Permit and implement Best Conventional Pollutant Technology (BCPT) to reduce or eliminate industrial storm water pollution.

    - The State Water Resources Control Board adopted Order No. 97-03-DWQ (General Permit No. CAS000001), specifying Waste Discharge Requirements for discharges of storm water associated with Industrial activities, excluding construction activities, and requiring submittal of a Notice of Intent by Industries to be covered under the Permit.

45.	The discharger has submitted to this Regional Board and to the California Integrated Waste Management Board, evidence of Financial Assurance for Closure and Post Closure, pursuant to Section 22207 and 22212 of Title 27.

- The Board has notified the discharger and all known interested agencies and persons of its intent to update Waste Discharge Requirements for said discharge and has provided them with an opportunity for a public meeting and an opportunity to submit comments.

- The Board, in a public meeting, heard and considered all comments pertaining to this discharge.

IT IS HEREBY ORDERED, that Board Order No. 97-043 is rescinded, and in order to meet the provisions contained in Division 7 of the California Water Code and regulations adopted thereunder, the Discharger shall comply with the following:

A.	Specifications

- The treatment or disposal of wastes at this facility shall not cause pollution or nuisance as defined in Section 13050 of Division 7 of the California Water Code.

- Waste materials shall be confined to the existing footprint of the Waste Management Facility as in the attached site maps.

- Waste material shall not be discharged on any ground surface which is less than five (5) feet above the highest anticipated ground water level.

- The discharge shall not cause degradation of any water supply.

- Surface drainage from tributary areas, and internal site drainage from surface or subsurface sources, shall not contact or percolate through the waste discharged at this site.

- The exterior surfaces of the disposal area, including the intermediate and final landfill covers, shall be graded and maintained to promote lateral runoff of precipitation and to prevent ponding.

- The discharger shall use the constituents listed in Monitoring and Reporting Program No. 01-142 and revisions thereto, as “Monitoring Parameters”. These Monitoring Parameters are subject to the most appropriate statistical or non-statistical test under Monitoring and Reporting Program No. 01-142, Part III, and any revised Monitoring and Reporting Program approved by the Regional Board’s Executive Officer.

- The discharger shall implement the attached Monitoring and Reporting Program No. 01-142 in order to detect, at the earliest opportunity, any unauthorized discharge of waste constituents from the Landfill, or any unreasonable impairment of beneficial uses associated with (caused by) discharges of waste to the Landfill.

- The discharge shall not cause the concentration of any Constituent of Concern or Monitoring Parameter to exceed its respective background value in any monitored medium at any Monitoring Point assigned to Detection Monitoring pursuant to Part II.B.4. of the attached Monitoring and Reporting Program No. 01-142 and revisions thereto.

- The discharger shall follow the Water Quality Protection Standards (WQPS) for detection monitoring established by the Regional Board in this Board Order pursuant to Title 27, Section 20390. The following are five (5) parts of WQPS as established by the Regional Board (the terms of art used in this Board Order regarding monitoring are defined in Part I of the attached Monitoring and Reporting Program No. 01-142 and revisions thereto, which is hereby incorporated by reference):

- The discharger shall test for the Monitoring Parameters and the Constituents of Concern (COC) listed in Monitoring and Reporting Program No. 01-142 and revisions thereto.

- Concentration Limit – The Concentration Limits for each Monitoring Parameter and Constituents of Concern for each Monitoring Point (as stated in Detection Monitoring Program Part II), shall be its background value as obtained during that reporting period.

- Monitoring Points and background Monitoring Points for Detection Monitoring shall be those listed in Part II.B of the attached Monitoring and Reporting Program No. 01-142, and any revised Monitoring and Reporting Program approved by the Regional Board’s Executive Officer. Monitoring and Background Monitoring Points are shown on Attachment D.

- The Points of Compliance are shown on Attachment D and extend down through the zone of saturation.

- Compliance Period – The estimated duration of the Compliance Period for this Landfill is six (6) years. Each time the Standard is not met (i.e., releases discovered), the Landfill begins a Compliance Period on the date the Regional Board directs the Discharger to begin an Evaluation Monitoring Program. If the Discharger’s Corrective Action Program (CAP) has not achieved compliance with the standard by the scheduled end of the Compliance Period, the Compliance Period is automatically extended until the Landfill has been in continuous compliance for at least three (3) consecutive years.

  - The discharger shall install at a minimum of two (2) permanent monuments to serve as reference points for monitoring refuse settlement at the Landfill. Also the entire permitted site shall be aerially photographed at the end of the closure activities and every five (5) years throughout the post-closure maintenance period.

  - The discharger shall remove and relocate any wastes that are discharged at this site in violation of these requirements.

  - Water used for site maintenance shall be limited to amounts necessary for dust control.

  - The discharger shall maintain a hazardous waste load-checking program at the Landfill. The Discharger shall report the results of the load-checking program in accordance with Monitoring and Reporting Program No. 01-142 and revisions thereto.

  - The Landfill shall be designed to prevent any washout or erosion of wastes or covering material, and from any inundation which could occur as a result of floods having a predicted frequency of once in 100 years.

  - The discharge shall not cause the release of pollutants, or waste constituents in a manner which could cause a condition of contamination, or pollution to occur, as indicated by the most appropriate statistical (or non-statistical) date analysis method and retest method listed in Part III of the attached Monitoring and Reporting Program No. 01-142 and revisions thereto.

B.	Prohibitions

- The discharge or deposit of hazardous waste at this site is prohibited.

- The discharge or deposit of designated waste as defined in Title 27 at this site is prohibited unless approved by the Regional Board’s Executive Officer.

- The co-disposal of incompatible wastes as defined in Title 27 is prohibited.

- The discharge of waste to land not owned or controlled by the Discharger is prohibited.

- The discharge of any waste to any surface waters or surface drainage courses is prohibited.

- The discharge of liquid or semi-solid waste (i.e., waste containing less than 50 percent solids) to the Landfill is prohibited unless approved by the Regional Board’s Executive Officer.

C.	Provisions

- The discharger shall comply with “Monitoring and Reporting Program No. 01-142”, and future revisions thereto, as specified by the Regional Board’s Executive Officer.

- Prior to any change in ownership or management of this operation, the Discharger shall transmit a copy of this Board Order to the succeeding owner/operator, and forward a copy of the transmittal letter to the Regional Board.

- The discharger shall ensure that all site-operating personnel are familiar with the content of this Board Order, and shall maintain a copy of this Board Order at the site.

- This Board Order does not authorize violation of any federal, state, or local laws or regulations.

- The discharger shall allow the Regional Board, or an authorized representative, upon presentation of credentials and other documents as may be required by law, to:

  - Enter upon the premises regulated by this Board Order, or the place where records must be kept under the conditions of this Board Order;

  - Have access to and copy, at reasonable times, any records that shall be kept under the conditions of this Board Order;

  - Inspect at reasonable times any facilities, equipment (including monitoring and control equipment), practices, or operations regulated or required under this Board Order; and

  - Sample or monitor at reasonable times, for the purpose of assuring compliance with this Board Order or as otherwise authorized by the California Water Code, any substances or parameters at this location.

- This Board Order does not convey any property rights of any sort or any exclusive privileges, nor does it authorize any injury to private property or any invasion of personal rights, nor any infringement of federal, state, or local laws or regulations.

- Unless otherwise approved by the Regional Board’s Executive Officer, all analyses shall be conducted at a laboratory certified for such analyses by the State Department of Health Services. All analyses shall be conducted in accordance with the latest edition of “Guidelines Establishing Test Procedures for Analysis of Pollutants”, promulgated by the United States Environmental Protection Agency.

- All regulated disposal systems shall be readily accessible for sampling and inspection.

- Adequate measures shall be taken to assure that flood or surface drainage waters do not erode or otherwise render portions of the discharge facilities inoperable.

- The discharger is the responsible party for the Waste Discharge Requirements and the Monitoring and Reporting Program for the facility. The Discharger shall comply with all conditions of these Waste Discharge Requirements. Violations may result in enforcement actions, including Regional Board Order or court orders, requiring corrective action or imposing civil monetary liability, or in modification or revocation of these Waste Discharge Requirements by the Regional Board.

- The discharge shall neither cause nor contribute to the contamination or pollution of ground water via the release of waste constituents in either liquid or gaseous Phase.

- The discharge shall not cause any increase in the concentration of waste constituents in soil-pore gas, soil-pore liquid, soil, or other geologic materials outside of the Landfill if such waste constituents could migrate to waters of the State, in either the liquid or the gaseous Phase, and cause a condition of contamination or pollution.

- All containment structures and erosion and drainage control systems shall be designed and constructed under direct supervision of a California Registered Civil Engineer or Certified Engineering Geologist, and shall be certified by the individual as meeting the prescriptive standards and performance goals of Title 27.

- The discharger shall within 48 hours of a significant earthquake event, submit to the Regional Board a detailed post-earthquake report describing any physical damages to the containment features, ground water monitoring and/or leachate control facilities and a corrective action plan to be implemented at the Landfill.

- The discharger shall immediately notify the Regional Board of any flooding, slope failure or other change in site conditions which could impair the integrity of waste containment facilities or of precipitation and drainage control structures.

- The discharger shall maintain legible records on the volume and type of each waste discharged at the site. These records shall be available for review by representatives of the Regional Board at any time during normal business hours. At the beginning of the post-closure maintenance period, copies of these records shall be sent to the Regional Board.

- The discharger shall maintain visible monuments identifying the boundary limits of the entire waste management facility.

- The discharger shall submit a Notice of Intent (NOI) to the State Water Resources Control Board to be covered under the Statewide General Permit General NPDES Permit for Storm Water Discharges Associated with Industrial Activities, Order NO. 97-03-DWQ, NPDES No. CAS000001. The discharger shall comply with all the discharge prohibitions, receiving water limitations, and provisions of the General Permit.

- The discharger shall submit a sampling and monitoring plan for storm water discharges to the Regional Board’s Executive Officer for review and approval no later than 90 days after the adoption of this Board Order.  The plan shall meet the minimum requirements of Section B, Monitoring Program and Reporting Requirements of the Statewide General NPDES Permit for Storm Water Discharges Associated with Industrial Activities, Order No. 97-03 DWQ.

- Within 180 days of the adoption of this Board Order, the Discharger shall submit to the Regional Board in accordance with Section 20380(b) of Title 27, assurance of financial responsibility acceptable to the Regional Board’s Executive Officer for initiating and completing corrective action for all known or reasonably foreseeable releases from the Landfill.

- The Preliminary and Final Closure and Post Closure Maintenance Plan (PCPCMP) shall satisfy all requirements of Title 27 as specified by the Regional Board’s Executive Officer.  The post-closure period shall be at least thirty (30) years. However, the post-closure maintenance period shall extend as long as the waste poses a threat to water Quality

- One (1) year prior to the anticipated closure of the facility or any unit (portion) thereof, the discharger shall submit to the Regional Board, for review and approval by the Regional Board’s Executive Officer, a final closure and a post-closure maintenance plan in accordance with Section 21769 of Title 27. The final closure and post-closure maintenance plan shall include seismicity studies.

- This Board Order is subject to final Regional Board review and updating, as necessary to comply with changing State or Federal laws, regulations, policies, or guidelines, or changes in the discharge characteristics.

I, Philip A. Gruenberg, Executive Officer, do hereby certify the foregoing is a full, true and correct copy of an Order adopted by the California Region Water Quality Control Board, Colorado River Basin Region, on November 14, 2001.

Executive Officer

mg/L = milligrams-per-Liter