Amend Title 27, Division 1, Subdivision 4, Chapter 1, Sections 15110, 15185, 15290, 15400, and 15400.3 of the California Code of Regulations:

§15110. What terms need to be clarified to understand the Unified Program regulations?

(a) For purposes of this Division, the following terms have the following meanings. All Health and Safety Code citations reference Division 20 unless otherwise specified.

Applicant Agency means a county, city or other local agency which is applying to the Secretary to become a Unified Program Agency.

Certified Unified Program Agency or CUPA is an agency which has been certified pursuant to the requirements of Chapter 6.11 and these regulations.

Consolidated Permit means those permits required to be consolidated pursuant to Health and Safety Code, Section 25404(c) and other permits not specifically required by Health and Safety Code Section 25404(c) but included within a Unified Program. The Consolidated Permit may be a single permit or multiple permits in a single package which shall minimize duplicate information.

Enforcement Actions. There are the following types of Unified Program enforcement actions:

(1) Formal Enforcement is an action which mandates compliance and initiates a civil, criminal, or administrative process which results in an enforceable agreement or order. Enforceable means the instrument creates an independent, affirmative obligation to comply and imposes sanctions for the prior failure to comply. Sanctions include fines and penalties as well as other tangible obligations, beyond returning to compliance, that are imposed upon the regulated. Examples include administrative orders and civil and criminal referrals to the appropriate prosecutor.

(2) Informal Enforcement is an action other than a formal enforcement action that notifies the regulated business of its non-compliance and establishes a date by which that non-compliance is to be corrected. Examples include a letter or notice of violation. Informal actions do not impose sanctions.

(3) Integrated or Multi-media Enforcement is a formal enforcement action involving either Unified Program and non-Unified Program Agencies or involving more than one environmental medium (i.e.; air, water, soil). An integrated or multimedia enforcement requires more than one agency's involvement or one medium-specific action to bring about compliance or to abate, investigate, prosecute, or remediate the violations.

Inspection Categories. There are four (4) categories of Unified Program inspections. One inspection may be reported in more than one category:

(1) Single Program Inspection is an inspection of a regulated business conducted by one inspector for only one Unified Program element.

(2) Combined Inspection is an inspection of a regulated business conducted by one inspector representing two or more Unified Program elements or other program elements.

(3) Joint Inspection is an inspection of a regulated business, conducted by two or more inspectors representing different Unified Program Agencies within a CUPA, specifically to evaluate operations within the expertise of those regulatory agencies.

(4) Integrated or Multi-media Inspection is an inspection either involving Unified Program and non-Unified Program Agencies and/or involving more than one environmental medium (i.e.; air, water, soil) of a regulated business that results in consolidating inspections.

Inspection Types. There are two (2) types of Unified Program inspections, which for reporting purposes are mutually exclusive:

(1) Routine Inspection is a regularly scheduled inspection to evaluate compliance pursuant to one or more program elements.

(2) Other Inspection includes, but is not limited to, regulatory field activity such as complaint investigations, enforcement follow-up, closures, tank installation and/or removal oversight, tank cleaning, and release investigations. It does not include routine inspections or field or site visits whose principle purposes are informational or educational, pollution prevention education, verification of administrative information, or orientation of new owners or operators. “Other Inspection” also includes verification inspections for the administrative requirement of subdivision (c) of Section 25270.5 for owners and operators of aboveground storage tanks to prepare a spill prevention control and countermeasure plan.

Participating Agency is an agency which has a formal agreement with the CUPA to implement and enforce a program element as part of the Unified Program.

Program Element is a program listed in Health and Safety Code, Section 25404(c), or it may be a program other than those mandated in Health and Safety Code, Section 25404(c) which is voluntarily consolidated into a Unified Program, to be carried out in conjunction with a program listed in 25404(c).

Regulated Business means any of the following:

(1) “person” as defined in: (A) the Hazardous Waste Management Program, Health and Safety Code, Chapter 6.5, Section 25118 and (B) the California Hazardous Substances Tax Law, Revenue and Taxation Code Part 22, Division 2, Section 43006.

(2) “business” as defined in the Hazardous Materials Emergency Response Plan and Inventory (Business Plan) program, Health and Safety Code, Chapter 6.95, Section 25501(d).

(3) “facility” as defined in the Underground Storage Tank Program, Health and Safety Code, Chapter 6.7, Section 25281(e).

(4) “tank facility” as defined in the Aboveground Storage Tank Program, Health and Safety Code, Chapter 6.67, Section 25270.2(l).

(5) “hazardous waste facility” as defined in the Hazardous Waste Management Program, Health and Safety Code, Chapter 6.5, Section 25117.1.

(6) “stationary source” as defined in the California Accidental Release Prevention Program, Health and Safety Code, Chapter 6.95, Section 25532(k).

Responsible Agency is the Certified Unified Program Agency (CUPA) or participating agency (PA), which implements a particular program element of the Unified Program.

Unified Program Data Dictionary (data dictionary) defines data elements, data field size and type, and edit criteria for regulatory data that shall be collected and retained by a CUPA. It has the following sections:

(1) Business Section: for information reported from businesses to CUPAs. [Appendix C Division 3, Subdivision 1, Chapters 1-4]

(2) CUPA Section: for CUPA-to-State reporting of CUPA activities or other information that shall be collected and retained by a CUPA and reported pursuant to Section 15290. [Appendix D Division 3, Subdivision 1, Chapter 5, Unified Agency Reporting]

Unified Program Consolidated Form (UPCF) is a form used by Unified Program regulated businesses to provide a standardized document to satisfy numerous business-to-CUPA reporting requirements. It consolidates information regulated businesses are required to provide for different program elements to the CUPA. It incorporates or replaces previous state and local forms for Unified Program program elements. The UPCF consists of the sections described in  Section 15400 and is found in Appendix E Division 3, Subdivision 1, Chapter 6, Forms.

Authority cited: Sections 25404(b), (c), (d) and (e) and 25404.6(c), Health and Safety Code. Reference: Sections 25117.1, 25118, 25270.2(d) and (e), 25281(i), 25404(c) and (d), 25404.1(a) and (b)(3), 25404.3(d)(4), 25404.5(a) and (b), 25404.6(c), 25501(d) and 25532(k), Health and Safety Code; Section 43006, Revenue and Taxation Code; and the 1996 United States Environmental Protection Agency Enforcement Response Policy for the Resource Conservation and Recovery Act.

§15185. What information must CUPAs collect, retain, and manage?

(a) CUPAs shall collect, retain, and manage information needed to implement the Unified Program. At a minimum, the CUPAs shall collect and retain all information defined in the Unified Program Data Dictionary. [Refer to Appendices C and D Division 3, Subdivision 1, Chapters 1-5]

(b) The data dictionary defines data elements, data field size and type, and edit criteria for regulatory data that shall be collected and retained by a CUPA. [Refer to definition in Section 15110] It consists of the following sections:

(1) The Business Section for information reported by businesses to CUPAs using the Unified Program Consolidated Form [refer to Section 15400] The Business Section of the Data Dictionary is organized into the following parts:

(A) Facility Information

(i) Business Activities

(ii) Business Owner/Operator Identification

(B) Hazardous Materials

(i) Hazardous Materials Inventory-Chemical Description

(C) Tanks

(i) Underground Storage Tank (UST) Facility

(ii) UST Tank


(iii) UST Installation-Certificate of Compliance

(D) Hazardous Waste

(i) Recyclable Materials Report

(ii) Onsite Hazardous Waste Treatment Notification-Facility

(iii) Onsite Hazardous Waste Treatment Notification-Unit

(iv) Certification Of Financial Assurance for Permit by Rule and Conditionally Authorized Operations

(v) Remote Waste Consolidation Site Annual Notification

(vi) Hazardous Waste Tank Closure Certification

(2) The CUPA Section for information that shall be collected and retained by a CUPA and reported to the State pursuant to Section 15290. [Appendix DDivision 3, Subdivision 1, Chapter 5, Unified Agency Reporting] The CUPA Section of the Data Dictionary is organized into the following sections:

(A) Compliance Activity Information

(B) Inspection Information

(C) Enforcement Information

(D) Tiered Permitting Release Information

(E) Household Hazardous Waste Information (reserved)

(c) CUPAs shall collect, retain, and manage any additional information required by state or federal law.

(d) CUPAs may satisfy these information collection, retention, and management requirements through agreements with Participating Agencies that serve as the repository of the information.

(e) Electronic reporting is optional and the CUPA is not required to store or maintain the data in the Unified Program Data Dictionary format (Appendices C and D Division 3, Subdivision 1, Chapters 1-5). The Data Dictionary data structures and formats must be used for electronic reporting by businesses to a CUPA pursuant to Section 15187 or by a CUPA to the State pursuant to Sections 15187 and 15290(f) and (g).

Authority cited: Sections 25404(b), (c), (d) and (e) and 25404.6(c), Health and Safety Code. Reference: Sections 25143.10, 25144.6, 25200.3, 25201, 25201.4.1, 25201.5, 25201.13, 25201.14, 25281.2, 25218.9, 25286, 25287, 25503.5, 25505, 25506 and 25509, Health and Safety Code.

§15290. What reports must the CUPA submit to the State?

(a) The CUPA shall submit the following reports for the previous fiscal year to the Secretary by September 30 of each year. The first reports shall be submitted by September 30 following a full State fiscal year of operation as a CUPA.

(1) The Annual Single Fee Summary Report using Report 2. It includes:

(A) The amount of the single fee billed and the amount collected.

(B) The amount of any funds due to participating agencies and the amount actually transmitted.

(C) The amount of surcharge billed, the amount of surcharge waived, and the amount of surcharge collected for each of the following categories:

(i) CUPA oversight

(ii) regulated underground tanks

(iii) California Accidental Release Prevention program

(D) If the CUPA believes that the number of regulated businesses will change significantly in the current year or in the next year, then estimates of those changes for each program element will be provided in a cover letter with Report 2.

(E) A count for the year of the report of the total regulated businesses, underground storage tank facilities, underground storage tanks, onsite hazardous waste treatment facilities (permit by rule, conditionally authorized, and conditionally exempt), CalARP program stationary sources, waivers granted to stationary sources, and businesses subject to the CalARP program surcharge.

(2) Annual Inspection Summary Report, using Report 3, provides summary information for each program element. The hazardous waste element is separated into parts for generators, large quantity generators, recyclers, and onsite treatment as shown on Report 3. The summary information includes the number of regulated businesses, total number of inspections, routine inspections, other inspections, and the inspected businesses that returned to compliance within established standards after routine inspections. Established standards vary by program element and are found in either state law or regulations, or the CUPA may adopt more stringent standards by local ordinance or in its application for certification. The report also collects total counts (not by program element) for these types of inspections: combined routine, joint, and integrated/multi-media; and a count of Risk Management Plan audits for the CalARP program.

(3) Annual Enforcement Summary Report, using Report 4, provides summary information for each program element. The hazardous waste element is separated into parts for generators, large quantity generators, recyclers, and onsite treatment, as shown on Report 4. The summary information includes the number of facilities with violations by type of violation; the number of informal enforcement actions; the total number of administrative actions, civil and criminal referrals and enforcement actions, and the total amount of fines and penalties initially assessed and collected. For the Class I and II violations within the hazardous waste program, it also provides a count of the total number of formal enforcement actions that were initiated within 135 days from the first day of a routine inspection or after making a determination of the violations for a complaint investigation. (This last count excludes minor violations.)

(b) The CUPA shall submit the Biennial Tiered Permitting Release Report, using Report 5, to the Secretary for the previous two-year period by August 30, every other year starting in 2000. This report provides a listing of all unauthorized and accidental releases to the environment from any onsite tiered permitting facility. This includes releases identified by the CUPA or other responding agencies or as self-reported to the CUPA by the regulated business. The report includes the EPA ID number, facility name and address, type of facility (based on the highest tier), date of the release, description of release, and whether the corrective action is completed at the time of the report.

(c)(i) Reports 2 through 4 shall be submitted by the CUPA to the:

SECRETARY FOR ENVIRONMENTAL PROTECTION

1001 I STREET

SACRAMENTO, CALIFORNIA 95814

(ii) Report 5 shall be submitted by the CUPA to the:

DEPARTMENT OF TOXIC SUBSTANCES CONTROL

STATE REGULATORY PROGRAMS DIVISION

P.O. BOX 806

SACRAMENTO, CALIFORNIA 95812-0806

(d) On a quarterly basis, each CUPA shall send information pertaining to local underground storage tank program implementation to the State Water Resources Control Board. This report shall satisfy the requirements of Health and Safety Code, Section 25299.7(b) and CCR Title 23, Section 2713.

(1) Quarterly Underground Storage Tank (UST) Program Report, using Report 6, provides information on quarterly changes to the count of regulated tank facilities; the number of active and permanently closed petroleum and hazardous substances tank systems; the completed UST facility inspections; and both a count and percent calculation of active UST systems with approved leak detection systems and the count and percent of UST systems that meet the 1998 upgrade or replacement requirements. This report is a turnaround document that is provided quarterly by the State Water Resources Control Board to each CUPA showing the previous quarter's information reported by the CUPA. The CUPA will also review and verify the information shown from the previous quarter and make any appropriate changes.

(2) The quarterly reports shall be submitted 60 days after the end of each quarter to the:

STATE WATER RESOURCES CONTROL BOARD 

DIVISION OF CLEAN WATER QUALITY PROGRAMS, UST PROGRAM 

P.O. BOX 944212 2231
SACRAMENTO, CA 94244-2120 95812
(e) The periodic reports required by Sections 15250 and 15290 shall be submitted in a paper form, unless the CUPA requests to submit the reports electronically and obtains the Secretary's prior approval of the file format.

(f) If the CUPA chooses to submit Reports 3-6, required by Section 15290, to the State in an electronic format, the CUPA shall:

(1) Meet the standards specified in Sections 15185 and 15187,

(2) Submit the data using the same layout and present the required information in the same order and general sequencing for each page as shown on each report, or use a facsimile version thereof.

(3) Collect and report all of the information found on the report that applies to the CUPA.

(g) Upon the written request of the Secretary or an authorized agent, or a state agency responsible for one or more program elements, the CUPA shall provide information listed in or derived from any part of the Unified Program Data Dictionary [refer to Appendices C and D Division 3, Subdivision 1, Chapters 1-5] to the person or agency making the request within 60 days. The scope of these requests by the Secretary for information on facilities and/or CUPA activities is limited to data included in the data dictionary. These additional data reports shall be submitted in a paper form, unless the person or agency making the request approves a CUPA's request to submit the reports electronically. CUPAs may request an extension upon showing good cause.

(h) Nothing in this section shall limit the authority of the Secretary to request records or documents that are normally maintained by the CUPA in the course of implementing the Unified Program or otherwise required by law to be retained by the CUPA. The CUPA shall provide this information to the Secretary within 60 days.

(i) Any other program reports required by federal or state law. The CUPA shall provide this information to the person or agency making the request within 60 days.

(j) The CUPAs shall report using the formats of Report 1 through 6, however, in 1998-99, CUPAs may submit incomplete Reports 3 and 4, if information on CUPA activities had not been collected using these categories for that entire reporting period. In those cases, the CUPA shall submit additional available information demonstrating inspection and enforcement activities for the 1998-99 fiscal year. Complete reports are required for the 1999-2000 fiscal year and subsequent submissions.

(k) An agency authorized to operate a program element or elements pursuant to Health and Safety Code Sections 25404.3(f) or 25533(f) shall only report information on the implementation of the program element or elements that particular agency is authorized to operate and shall not include information related to the surcharge or single fee system.

Authority cited: Sections 25404(b), (c), (d) and (e) and 25404.6(c), Health and Safety Code. Reference: Sections 25171, 25171.5, 25299.3(b), 25404(b), (c) and (d), 25404.4(a)(1) and 25404.5(b), Health and Safety Code.

§15400. What is the Unified Program Consolidated Form?

(a) The Unified Program Consolidated Form (UPCF) (1/99), defined in section 15110 and shown in Appendix E Division 3, Subdivision 1, Chapter 6, Forms, is a standardized form to be used by CUPAs in the Unified Program to collect information from regulated businesses. The UPCF is a single, comprehensive format that consolidates business-to-CUPA reporting requirements within the Unified Program.

(b) The UPCF may be reproduced or electronically duplicated as needed.

Authority cited: Sections 25404(b), (c), (d), and (e) and 25404.6(c), Health and Safety Code. Reference: Sections 25143.10, 25144.6, 25200.3, 25200.14, 25201, 25201.4.1, 25201.5, 25201.13, 25281.2, 25218.9, 25245.4, 25286, 25287, 25503.5, 25505, 25506 and 25509, Health and Safety Code.

§15400.3. When must a CUPA use and distribute the UPCF and when are CUPA alternative versions allowed?

(a) The CUPA shall distribute copies of the UPCF to any regulated business or member of the public upon request. A CUPA may add the name of the CUPA, a logo, and address, phone number, and other identifying information to the UPCF title or footer on one or more pages, without the customized UPCF being considered an alternative version subject to the conditions adopted by this section.

(b) The CUPA shall accept the UPCF as shown in Appendix E Division 3, Subdivision 1, Chapter 6, Forms from any regulated business that chooses to use it, even if the CUPA adopts one or more alternative versions of the UPCF.

(c) A CUPA may create alternative versions of the UPCF for local purposes such as streamlining for small businesses or addressing a specific type of industry. Any alternative version of the UPCF shall:

(1) Collect all of the information found on the UPCF that applies to the regulated businesses using the data element definitions established by the data dictionary.

(2) Be consistent with the data standards adopted throughout Article 5 through 10 of these regulations.

(3) Use the same section order as shown in Section 15400.1.

(4) Be developed in consultation with all other agencies within the CUPA's jurisdiction that are responsible for fire protection, emergency response, and environmental health.

(5) Not duplicate data elements between sections of the UPCF other than facility ID number and facility name.

(6) Comply with all applicable federal and state laws.

(d) Alternative versions of the UPCF adopted by a CUPA shall include this written disclaimer statement on a cover page or the front page, printed using a font larger than or equal to 8 points for readability: “This form was developed by the CUPA as an alternative version of the Unified Program Consolidated Form (UPCF). Businesses have the option to use it or the UPCF adopted in state regulations. The CUPA or Participating Agency (PA) must accept the state UPCF and cannot require a business to use the alternative version developed by the CUPA. The CUPA and PA can require businesses to provide additional information on either the UPCF or a supplemental page to that document.”

(e) Each CUPA shall provide instructions to the regulated businesses when distributing the UPCF and any alternative versions. These instructions must be consistent with the instructions adopted in California Code of Regulations Title 19, Article 4, Appendix B.

Authority cited: Sections 25404(b), (c), (d), and (e) and 25404.6(c), Health and Safety Code. Reference: Sections 25503.5(a) and (b)(1) and (2), 25505 and 25509, Health and Safety Code.

Repeal Title 27, Division 1, Subdivision 4, Chapter 1, Appendices C, D, E and F of the California Code of Regulations:

Appendix C 

UP Information Collection and Reporting Standards Unified Program Data Dictionary--Business Section

ACRONYMS

AA - Authorizing Agency

AN - Alpha-numeric

AST - Aboveground Storage Tank

ATG -  Automatic Tank Gauging

BOE - Board of Equalization

CA - Conditionally Authorized

Cal/EPA - California Environmental Protection Agency

Cal/ARP - California Accidental Release Prevention

CAS - Chemical Abstract Number

CCR - California Code of Regulations

CD - Certificate of Deposit

CE - Conditionally Exempt

CE-CL - Conditionally Exempt - Commercial Laundry

CEL - Conditionally Exempt – Limited

CESQT - Conditionally Exempt Small Quantity Treatment

CESW - Conditionally Exempt - Specified Wastestreams

CFO - Chief Financial Officer

CFR - Code of Federal Regulations

CUPA - Certified Unified Program Agency

DBA - Doing Business As

DOT - United States Department of Transportation

DTSC - Department of Toxic Substances Control

EHS - Extremely Hazardous Substance

EPA - United States Environmental Protection Agency

EPCRA - Emergency Planning and Community Right to Know Act

FRP - Fiberglass Reinforced Plastic

FTU - Fixed Treatment Unit

HDPE - High Density Polyethylene

HHW - Household Hazardous Waste  

HMRRP - Hazardous Materials Release Response Plans  

HSC - Health and Safety Code  

HW - Hazardous Waste  

ID - Identification  

IUPAC - International Union of Pure and Applied Chemistry  

LIA - Local Implementing Agency  

LQG - Large Quantity Generator  

MSDS - Material Safety Data Sheet  

MTG - Manual Tank Gauging  

NPDES - National Pollutant Discharge Elimination System  

OES - Office of Emergency Services  

PA - Participating Agency  

PBR - Permit by Rule  

POTW - Publicly Owned Treatment Works  

Q/A - Quality Assurance  

Q/C - Quality Control  

RCRA - Resource Conservation and Recovery Act  

RMP - Risk Management Plan  

SIC - Standard Industrial Classification  

SIR - Statistical Inventory Reconciliation  

SPCC - Spill Prevention Control and Countermeasures  

SWRCB - State Water Resources Control Board  

TTU - Transportable Treatment Unit  

U.S. EPA - United States Environmental Protection Agency  

UP - Unified Program  

UPCF - Unified Program Consolidated Form  

UST - Underground Storage Facility  

FACILITY INFORMATION

1.  Business Activities

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

2
EPA ID Number
12 digit identifier beginning with CA
12
AN
EPA Identification number for businesses that generate, recycle, or treat hazardous waste.  For facilities in California, the number usually starts with the letters CA.  The number can be obtained from the Telephone Information Center at

(916) 324-1781, (800) 61-TOXIC or (800) 618-6942.

3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of business.

4
Hazardous Materials

On Site
Y or N
1
AN
Business must report that it has hazardous materials on site if:

  - it is handled in quantities equal to or greater than 500 pounds, 55 gallons, or 200 cubic feet of gas (calculated at standard temperature and pressure),

  - it is handled in quantities equal to or greater than the applicable federal threshold planning quantity for an extremely hazardous substance listed in 40 CFR Part 355, Appendix A,

  - radioactive materials are handled in quantities for which an emergency plan is required to be adopted pursuant to Part 30, Part 40, or Part 70 of Chapter 10 of 10 CFR, or pursuant to any regulations adopted by the state in accordance with those regulations.

Triggers requirement for chemical description data elements.

5
Own or Operate Underground

Storage Tank
Y or N
1
AN
Facility must report if it owns or operates USTs containing hazardous substances defined in HSC 25316.  Triggers requirement for UST facility and tank data elements.

6
Upgrade/Install Underground

Storage Tank
Y or N
1
AN
Facility must report if it intends to install or upgrade USTs containing hazardous substances defined in HSC 25316.  Triggers requirement for UST installation data elements.

7
Underground

Storage Tank

Closure
Y or N
1
AN
Facility must report if a UST which held hazardous materials is being closed in place.  Triggers requirement for UST closure data elements.

8
Own or Operate Aboveground

Petroleum

Storage Tank
Y or N
1
AN
Facility must report if it stores petroleum in aboveground tanks.  "Petroleum" means crude oil or any fraction thereof, which is liquid at 60 degrees Fahrenheit temperature and 14.7 pounds per square inch absolute pressure (HSC 25270.2(g)).  The facility must report if any ASTs capacity exceeds 660 gallons or if the total facility storage capacity (aggregate) exceeds 1320 gal.  "Storage tank" does not include any of the  following:

  - a pressure vessel or boiler which is subject to Division 5 of the Labor Code,

  - a storage tank containing hazardous waste if a hazardous waste facilities permit has been issued for the storage tank by DTSC,

  - an aboveground oil production tank which is regulated by the Division of Oil and Gas, or

  - certain oil-filled electrical equipment including but not limited to transformers, circuit breakers, or capacitors.

9
Hazardous Waste Generator
Y or N
1
AN
Facility must report if it generates hazardous waste.  "Hazardous waste" means a waste that meets any of the criteria for the identification of a hazardous waste adopted by the department pursuant to HSC 25141. "Hazardous waste" includes, but is not limited to, RCRA hazardous waste.  Unless expressly provided otherwise, the term "hazardous waste" shall be understood to also include extremely hazardous waste and acutely hazardous waste.  Triggers requirement to obtain and provide EPA Identification number.

10
Recycle
Y or N
1
AN
Facility must report if it recycles more than 100 kilograms per month of recyclable material under a claim that the material qualifies for exclusion or exemption pursuant to HSC.25143.2.  This includes onsite and offsite facilities that recycle under this law.  Triggers requirement for Recyclable Materials data elements.  Persons that send recyclable material offsite to be recycled and that do not recycle onsite are not included in this category.

11
Onsite Hazardous

Waste Treatment
Y or N
1
AN
Facility must report if it treats hazardous waste under an onsite tier.  "Treatment" means any method, technique, or process which is designed to change the physical, chemical, or biological character or composition of any hazardous waste or any material contained therein, or removes or reduces its harmful properties or characteristics for any purpose.  "Treatment" does not include the removal of residues from manufacturing process equipment for the purposes of cleaning that equipment.  Amendments (effective 1/1/99) add exemptions from the definition of "treatment" for certain processes under specific, limited conditions.  Refer to HSC 25123.5(b) for these specific exemptions. 

Treatment of certain laboratory hazardous wastes do not require treatment.  Refer to HSC25200.3.1 for specific information.

Contact CUPA to determine if any exemptions or exclusions apply.  Triggers requirement for onsite hazardous waste treatment data elements.

12
Financial Assurance
Y or N
1
AN
Facilities that treat hazardous waste under PBR or CA tiers are required to provide financial assurance for closure costs (per 22 CCR 67450.13(b), HSC 25245.4), unless they are exempt.  Triggers requirement for financial assurance data elements.

13
Remote Waste Consolidation Site
Y or N
1
AN
Facilities must report if they collect hazardous waste initially at remote sites and subsequently transport the hazardous waste to a consolidation site they operate pursuant to HSC 25110.10.  Triggers requirement for remote hazardous waste consolidation data elements.

14
Hazardous Waste

Tank Closure
Y or N
1
AN
Facilities must report if the tank being closed would be classified as hazardous waste, after its contents are removed.  Classification could be based on:

- the facility's knowledge of the tank and its contents,

  - testing of the tank,

  - inability to remove hazardous materials stored in the tank,

  - the mixture rule, or

  - the listed wastes in 40 CFR 261.31, 40 CFR 261.32. 

Triggers requirement for hazardous waste data elements.

15
Local Requirements



For local use only.  This space may be used by the CUPA to collect any additional information necessary to meet the requirements of their individual programs.  Contact CUPA for guidance.

FACILITY INFORMATION

Business Owner / Operator Identification (OES Form 2730)

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

 1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

 3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of business.

100
Beginning Date
YYYYMMDD
8
D
Beginning year and date of report.

101
Ending Date
YYYYMMDD
8
D
Ending year and date of report.

102
Business Phone 
Area code + 7 digit phone number + extension
15
AN
Phone number of this site.

103
Business Site Address
Postal standard:

2 lines, 35 characters
70
AN
Street address where facility is located.  No post office box numbers are allowed.  This information must provide a means to geographically locate the facility.

104
City (Business)
Postal standard
20
AN
City or unincorporated area in which business site is located.

105
Zip Code (Business)
Postal standard
9
AN
Zip code of business site.

106
Dun & Bradstreet
D-U-N-S (data universal numbering system) 9 digit number
9
AN
Dun & Bradstreet D-U-N-S number for facility.  The Dun & Bradstreet number may be obtained by calling (610) 882‑7748 or by Internet.

107
SIC Code
Standard Industrial Classification (SIC) Code 4 digit number
4
AN
Standard Industrial Classification (SIC) Code number for primary business activity.  If code is more than 4 digits, report only the first four.

108
County

20
AN
County in which business site is located.

109
Business Operator Name

35
AN
Name of business operator.

110
Business Operator Phone
Area code + 7 digit phone number + extension
15
AN
Phone number of business operator, if different from business phone.

111
Business Owner Name

35
AN
Name of business owner, if different from business operator. 

112
Business Owner Phone
Area code + 7 digit phone number + extension
15
AN
Phone number of business owner, if different from business phone.

113
Business Owner

Mailing Address
Postal standard:

2 lines, 35 characters
70
AN
Mailing address of owner, if different from business site address.

114
Business Owner City
Postal standard
20
AN
City for owner's mailing address.

115
Business Owner State
Postal standard
2
AN
State for owner's mailing address.

116
Business Owner Zip Code
Postal standard
9
AN
Zip code for owner's mailing address.

117
Environmental

Contact Name

35
AN
Name of person, if different from the business owner/operator, who receives all environmental correspondence and will respond to enforcement activity.

118
Environmental

Contact Phone 
Area code + 7 digit phone number + extension
15
AN
Phone number of environmental contact, if different from business owner or operator.

119
Environmental Contact Mailing Address
Postal standard:

2 lines, 35 characters
70
AN
Mailing address for all environmental contact correspondence, if different from the site address.

120
Environmental

Contact City
Postal standard
20
AN
City for environmental contact's mailing address.

121
Environmental

Contact State
Postal standard
2
AN
State for environmental contact's mailing address.

122
Environmental

Contact Zip Code
Postal standard
9
AN
Zip code for environmental contact's mailing address.

123
Primary Emergency Contact Name

35
AN
Name of a representative that can be contacted in case of an emergency involving hazardous materials at the business site.  The contact shall have FULL facility access, site familiarity, and authority to make decisions for the business regarding incident mitigation.

124
Primary Emergency Contact Title

35
AN
Title of primary emergency contact.

125
Primary Emergency Contact Business

Phone Number
Area code + 7 digit phone number + extension
15
AN
Business phone number of primary emergency contact.

126
Primary Emergency Contact 24-Hour Phone
Area code + 7 digit phone number + extension
15
AN
Phone number for primary emergency contact which is answered 24 hours a day and, if not the contact's home phone number, then the service answering the phone must be able to immediately contact the above stated individual.

127
Primary Emergency Contact Pager Number
Area code + 7 digit phone number + extension
15
AN
Pager phone number for primary emergency contact, if available.

128
Secondary Emergency Contact Name

35
AN
Name of secondary representative that can be contacted in the event that the primary emergency contact is not available. The contact shall have FULL facility access, site familiarity, and authority to make decisions for the business regarding incident mitigation.

129
Secondary Emergency Contact Title

35
AN
Title of secondary emergency contact.

130
Secondary Emergency Contact Business Phone
Area code + 7 digit phone number + extension
15
AN
Business phone number of secondary emergency contact.

131
Secondary Emergency Contact 24-Hour Phone
Area code + 7 digit phone number + extension
15
AN
Phone number for secondary emergency contact which is answered 24 hours a day and, if not the contact's home phone number, then the service answering the phone must be able to immediately contact the above stated individual.

132
Secondary Emergency Contact Pager Number
Area code + 7 digit phone number + extension
15
AN
Pager phone number for secondary emergency contact, if available.

133
Additional Locally Collected Information
Narrative
255
AN
For local use only.  This space may be used for CUPAs or agencies authorized by the Secretary pursuant to HSC 25404.3(f)(2) to collect any additional information necessary to meet the requirements of their individual programs.  Contact local agency for guidance.

134
Date Identification Signed
YYYYMMDD
8
D
Date the document was signed.

135
Document Preparer Name (Identification)

35
AN
Full name of person who prepared the submittal information.

136
Name of Signer

of Identification

35
AN
Full name of person signing the page.  The signer certifies to a familiarity with the information submitted and that based on their inquiry of those individuals responsible for obtaining the information, all the information submitted is true, accurate and complete.

137
Title of Signer

of Identification

35
AN
Title of person signing the page.

HAZARDOUS MATERIALS

2.  Hazardous Materials Inventory - Chemical Description (OES Form 2731)

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction 

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

3
Business Name

70
AN
Full legal name of business.

200
Add / Delete / Revise
a = add

d = delete

r = revise
1

Indicates if material is being added to the inventory, deleted from the inventory or if the information previously submitted is being revised. Not required for electronic data collection.  NOTE: This field may be empty if entire inventory is resubmitted annually.

201
Chemical Location

(Inventory)
Narrative
140
AN
Building or outside/adjacent area where hazardous material is handled. A chemical that is stored at the same pressure and temperature, in multiple locations within a building, may be reported on a single page.  NOTE: This information is not subject to public disclosure pursuant to HSC 25506.

202
Chemical Location Confidential - EPCRA
Y or N
1
AN
If the business is subject to the Emergency Planning and Community Right to Know Act (EPCRA) this field indicates whether the business wishes to keep chemical location information confidential.

203
Map Number
Optional field
15
AN
If a map is included, number of map on which the location of the hazardous material is shown.

204
Grid Number
Optional field
15
AN
If grid coordinates are used, coordinates of map that correspond to the location of the hazardous material. If applicable, multiple grid coordinates can be listed.

205
Chemical Name
Narrative
60
AN
Proper chemical name associated to the Chemical Abstract Service (CAS) number of the hazardous material.  This should be the International Union of Pure and Applied Chemistry (IUPAC) name found on the Material Safety Data Sheet (MSDS).  NOTE: If the chemical is a mixture, do not complete this field; complete the common name" field instead.

206
Trade Secret
Y or N
1
AN
Indicates if information in this section is declared a trade secret.  If business is not subject to EPCRA, trade secret information is bound by State requirements, as defined in HSC 25511.  If business is subject to EPCRA, trade secret information is bound by Federal requirements, as defined in 40 CFR and business must submit a "Substantiation to Accompany Claims of Trade Secrecy" form (40 CFR 350.27) to U.S. EPA.

207
Common Name

(Inventory)

60
AN
Common name or trade name of hazardous material or mixture containing a hazardous material.

208
EHS
Y or N
1
AN
Indicates if hazardous material is an Extremely Hazardous Substance (EHS), as defined in 40 CFR Part 355, Appendix A. If the material is a mixture containing an EHS, do not complete this field; report on the individual hazardous components in the appropriate section below.

209
CAS #
Chemical Abstract Service number
15
AN
Chemical Abstract Service (CAS) number for the hazardous material. For mixtures, enter the CAS # of the mixture if it has been assigned a number distinct from its components. If the mixture has no CAS #, do not complete this field; report the CAS #s of the individual hazardous components in the appropriate section below.

210
Fire Code Hazard Classes
Narrative
60
AN
May be required by the CUPA.  Fire Code Hazard Classes describe to first responders the type and level of hazardous materials which a business handles. A list of the various hazard classes and instructions on how to determine which class a material falls under are included in the appendices of the Uniform Fire Code Article 80.  If a material has more than one applicable hazard class, include all. Contact CUPA for guidance.

211
Hazardous Material Type (Inventory)
a = pure

b = mixture

c = waste
1
AN
Type of hazardous material. If waste material, check only that box.  If mixture or waste, complete the individual hazardous components section below.

212
Radioactive
Y or N
1
AN
Indicates whether the hazardous material stored is radioactive.

213
Curies
9 digits with floating decimal
10
N
Activity in curies if the hazardous materials stored is radioactive.

214
Physical State
a = solid

b = liquid

c = gas
1
AN
Physical state of the hazardous material stored.

215
Largest Container
Maximum 13 digit number, report units in item 221.
13
N
Total capacity of largest container in which material is stored.

216a


Federal Hazard

Category = fire
Y or N
1
AN
Physical and health hazards associated with hazardous material.  FIRE: Flammable liquids and solids, combustible liquids, pyrophorics, oxidizers.

216b
Federal Hazard

Category = reactive
Y or N
1
AN
Physical and health hazards associated with hazardous material.  REACTIVE: Unstable reactive, organic peroxides, water reactive, radioactive.

216c
Federal Hazard

Category = pressure release
Y or N
1
AN
Physical and health hazards associated with hazardous material.  PRESSURE RELEASE: Explosives, compressed gases, blasting agents.

216d
Federal Hazard

Category = acute health
Y or N
1
AN
Physical and health hazards associated with hazardous material.  ACUTE HEALTH (Immediate): Highly toxic, toxic, irritants, sensitizers, corrosives, other hazardous chemicals with an adverse effect with short term exposure.

216e
Federal Hazard

Category = chronic health
Y or N
1
AN
Physical and health hazards associated with hazardous material.  CHRONIC HEALTH (Delayed): Carcinogens, other hazardous chemicals with an adverse effect with long term exposure.

217
Average Daily Amount
Maximum 15 digit number. This amount should be consistent with the units reported in item 221. NOTE: This amount should not exceed that of maximum daily amount.
15
N
Average daily amount of hazardous material or mixture containing a hazardous material in each building or adjacent/outside area. Calculations are based on previous year's inventory of material reported on this page by totaling all daily amounts and dividing by the number of days the chemical will be present on the site. If this is a material that has not previously been present at this location the amount is the average daily amount projected to be on hand during the course of the year.

218
Maximum Daily Amount
Maximum 15 digit number. This amount should be consistent with the units reported in item 221.
15
N
Maximum amount of each hazardous material or mixture containing a hazardous material handled in a building or adjacent/outside area at any one time over the course of the year. This amount must contain at a minimum last year's inventory of the material reported on this page, with the reflection of additions, deletions, or revisions projected for the current year. 

219
Annual Waste Amount
Maximum 15 digit number
15
N
Estimate of annual amount handled, if the hazardous material is a waste. 

220
State Waste Code
California 3-digit hazardous code
3
AN
California 3-digit hazardous waste code as listed on the back of the Uniform Hazardous Waste manifest, if the hazardous material is a hazardous waste.

221
Units

(Inventory)
a = cubic feet

b = pounds

c = tons

d = gallons
1
AN
Unit of measure which is most appropriate for the material being reported on this page.  NOTE: If the material is a federally defined Extremely Hazardous Substance (EHS), all amounts must be reported in pounds. If material is a mixture containing an EHS, report the units that the material is stored in (gallons, pounds, cubic feet, or tons).

222
Days on Site

3
N
Total number of days during the year material is on site.

223a
Storage Container = aboveground tank
Y or N
1
AN
Type of storage container in which hazardous material is stored.

223b
Storage Container = underground tank
Y or N
1
AN
See description in item 223a above.

223c
Storage Container =

tank inside building
Y or N
1
AN
See description in item 223a above.

223d
Storage Container =

steel drum
Y or N
1
AN
See description in item 223a above.

223e
Storage Container =

plastic / nonmetallic drum
Y or N
1
AN
See description in item 223a above.

223f
Storage Container = can
Y or N
1
AN
See description in item 223a above.

223g
Storage Container = carboy
Y or N
1
AN
See description in item 223a above.

223h
Storage Container = silo
Y or N
1
AN
See description in item 223a above.

223i
Storage Container =

fiber drum
Y or N
1
AN
See description in item 223a above.

223j
Storage Container = bag
Y or N
1
AN
See description in item 223a above.

223k
Storage Container = box
Y or N
1
AN
See description in item 223a above.

223l
Storage Container = cylinder
Y or N
1
AN
See description in item 223a above.

223m
Storage Container =

glass bottle
Y or N
1
AN
See description in item 223a above.

223n
Storage Container =

plastic bottle
Y or N
1
AN
See description in item 223a above.

223o
Storage Container = tote bin
Y or N
1
AN
See description in item 223a above.

223p
Storage Container =

tank truck, tank wagon
Y or N
1
AN
See description in item 223a above.

223q
Storage Container =

tank car, rail car
Y or N
1
AN
See description in item 223a above.

223r
Storage Container = other
Narrative
30
AN
See description in item 223a above.

224
Storage Pressure
a  = ambient

b = below ambient

c = above ambient
1
AN
Pressure at which hazardous material is stored.

225
Storage Temperature
a = ambient

b = below ambient

c = above ambient

d = cryogenic
1
AN
Temperature at which hazardous material is stored.

226
Hazardous Component 1

Percent by Weight
2.2 (implied decimal)
4
N
Percentage weight of hazardous component in a mixture. If a range of percentages is available, report the highest percentage in that range.

227
Hazardous Component 1 Name
Narrative
80
AN
Chemical name of hazardous component in a mixture (refer to MSDS or, in the case of trade secrets, refer to manufacturer).  All hazardous components in the mixture present at greater than 1% by weight if non-carcinogenic, or 0.1% by weight if carcinogenic, should be reported.  If more than five hazardous components are present above these percentages, the business may submit an  additional sheet of paper to capture the required information.  Information on more than five components is not submitted electronically unless the CUPA has established local standards.  When reporting a waste mixture, mineral and chemical composition should be listed.

228
Hazardous Component 1 EHS
Y or N
1
AN
Indicates if the component of the mixture is considered an Extremely Hazardous Substance as defined in 40 CFR Part 355.

229
Hazardous Component 1 CAS #

15
AN
Chemical Abstract Service (CAS) number related to hazardous component in the mixture.

230
Hazardous Component 2

Percent by Weight
2.2 (implied decimal)
4
N
See description in item 226.

231
Hazardous Component 2 Name

80
AN
See description in item 227.

232
Hazardous Component 2 EHS
Y or N
1
AN
See description in item 228.

233
Hazardous Component 2 CAS #

15
AN
See description in item 229.

234
Hazardous Component 3

Percent by Weight
2.2 (implied decimal)
4
N
See description in item 226.

235
Hazardous Component 3 Name

80
AN
See description in item 227.

236
Hazardous Component 3 EHS
Y or N
1
AN
See description in item 228.

237
Hazardous Component 3 CAS #

15
AN
See description in item 229.

238
Hazardous Component 4

Percent by Weight
2.2 (implied decimal)
4
N
See description in item 226.

239
Hazardous Component 4 Name

80
AN
See description in item 227.

240
Hazardous Component 4 EHS
Y or N
1
AN
See description in item 228.

241
Hazardous Component 4 CAS #

15
AN
See description in item 229.

242
Hazardous Component 5

Percent by Weight
2.2 (implied decimal)
4
N
See description in item 226.

243
Hazardous Component 5 Name

80
AN
See description in item 227.

244
Hazardous Component 5 EHS
Y or N
1
AN
See description in item 228.

245
Hazardous Component 5 CAS #

15
AN
See description in item 229.

If more than five hazardous components are present at greater than 1% by weight if non-carcinogenic, or 0.1% by weight if carcinogenic, the information is not submitted electronically unless the CUPA has established local data standards.

246
Additional Locally

Collected  Information

255
AN
For local use only.  This space may be used by the CUPA to collect any additional information necessary to meet the requirements of their individual programs. Contact CUPA for guidance.

III.
TANKS

A.
UST Facility Page

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

3
Business Name

70
AN
Full legal name of business.

400
Type of Action

(Tank Facility)
1 = new site permit

3 = renewal permit

4 = amended permit

5 = change of information

6 = temporary site closure

7 = permanently closed site

8 = tank removed
1
AN
Reason page is being submitted.

401
Nearest Cross Street

35
AN
Name of cross street nearest to site of the tank.

402
Facility Owner Type
1 = corporation

2 = individual 

3 = partnership

4 = local  agency / district

5 = county agency

6 = state agency

7 = federal agency
1
AN
Type of business ownership.

403
Business Type

(Tank Facility)
1 = gas station

2 = distributor

3 = farm

4 = processor

5 = commercial

6 = other
1
AN
Type of business.

404
Total Number of Tanks Remaining at Site

(Tank Facility)

4
N
Number of tanks remaining on the site after requested action.

405
Indian or Trust Land
Y or N
1
AN
Indicates if facility is located on Indian reservation or other trust lands.

406
Public Agency Supervisor Name

35
AN
Contact person for tank records, if facility owner is a public agency.

407
Property Owner Name

35
AN
Name of property owner, if different from business owner on Business Owner/Operator page.

408
Property Owner Phone
Area code + 7 digit phone number + extension
15
AN
Phone number of property owner, if different from business owner.

409
Property Owner Address
Postal standard:

2 lines, 35 characters
70
AN
Street or mailing address of property owner, if different from business owner.

410
Property Owner City
Postal standard
20
AN
City of property owner, if different from business owner.

411
Property Owner State
Valid 2-digit state code
2
AN
State of property owner, if different from business owner.

412
Property Owner Zip Code
Postal standard
9
AN
Zip code of property owner, if different from business owner.

413
Property Owner Type
1 = corporation

2 = individual 

3 = partnership

4 = local agency / district

5 = county agency

6 = state agency

7 = federal agency
1
AN
Type of property ownership.

414
Tank Owner Name (Facility)

35
AN
Name of tank owner, if different from business owner on Business Owner/Operator page.

415
Tank Owner Phone (Facility)
Area code + 7 digit phone number + extension
15
AN
Phone number of tank owner, if different from business owner.

416
Tank Owner Street (Facility)
Postal standard:

2 lines, 35 characters
70
AN
Street or mailing address of tank owner, if different from business owner.

417
Tank Owner City (Facility)
Postal standard
20
AN
City of tank owner, if different from business owner.

418
Tank Owner State (Facility)
Valid 2-digit state code
2
AN
State of tank owner, if different from business owner.

419
Tank Owner Zip Code (Facility)
Postal standard
9
AN
Zip code of tank owner, if different from business owner.

420
Tank Owner Type
1 = corporation

2 = individual 

3 = partnership

4 = local agency / district

5 = county agency

6 = state agency

7 = federal agency
1
AN
Type of tank ownership.

421
BOE Number
BOE 8 digit number, first two digits = 44
8
AN
Board of Equalization (BOE) UST storage fee account number.  This number is required before a permit application can be processed.  Registration with the BOE will ensure that you will receive a quarterly storage fee return in reporting the $0.012 per gallon fee due on the number of gallons placed in your USTs.  The BOE will code persons exempt from paying the storage fee so returns will not be sent.  If you do not have an account number with the BOE or if you have any questions regarding the fee or exemptions, please call the BOE at (916) 322-9669 or write to the BOE at the following address:

Board of Equalization

Fuel Taxes Division

P.O. Box 942879

Sacramento, CA 94279-0030

422-1
Petroleum UST Financial Responsibility Code = self-insured
Y or N
1
AN
Method(s) used by owner and/or operator in meeting the Federal and State financial responsibility requirements.  USTs owned by any Federal or State agency as well as non-petroleum USTs are exempt from this requirement.

422-2
Petroleum UST Financial Responsibility Code = guarantee
Y or N
1
AN
See description in item 422-1.

422-3
Petroleum UST Financial Responsibility Code = insurance
Y or N
1
AN
See description in item 422-1.

422-4
Petroleum UST Financial Responsibility Code = surety bond
Y or N
1
AN
See description in item 422-1.

422-5
Petroleum UST Financial Responsibility Code = letter of credit
Y or N
1
AN
See description in item 422-1.

422-6
Petroleum \UST Financial Responsibility Code = exemption
Y or N
1
AN
See description in item 422-1.

422-7
Petroleum UST Financial Responsibility Code = State Fund
Y or N
1
AN
See description in item 422-1.

422-8
Petroleum UST Financial Responsibility Code = State Fund and CFO letter
Y or N
1
AN
See description in item 422-1.

422-9
Petroleum UST Financial Responsibility Code = State Fund and CD
Y or N
1
AN
See description in item 422-1.

422-10
Petroleum UST Financial Responsibility Code = local government mechanism
Y or N
1
AN
See description in item 422-1.

422-99
Petroleum UST Financial Responsibility Code = other
Narrative
30
AN
See description in item 422-1.

423
Notification  Address
1 = facility address

2 = property owner address

3 = tank owner address
1
AN
Address to which legal notifications and mailings should be sent.

424
Date Certified

(Tank Facility)
YYYYMMDD
8
D
Date the page was signed.

425
Applicant Phone

(Tank Facility)
Area code + 7 digit phone number + extension
15
AN
Phone number of applicant (person certifying).

426
Applicant Name

(Tank Facility)

35
AN
Name of signatory.  The applicant certifies to a belief that all the information submitted is accurate and complete. The applicant may be the Owner/Operator or officially designated representative.

427
Applicant Title

(Tank Facility)

35
AN
Title of person signing the page.

428
State UST Facility Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
For local use only.  County and jurisdiction number from tax code list.  This number may be the same as the Facility ID number.

429
1998 Upgrade Certificate Number

6
AN
For local use only.  The State Water Resources Control Board 1998 Upgrade Certificate Number for the facility.

III.
TANKS

B.
UST Tank Pages 1 and 2

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number.
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of business.

430
Type of Action

(Tank Unit)
1 = new site permit

3 = renewal permit

4 = amended permit

5 = change of information

6 = temporary site closure

7 = permanently closed on site

8 = tank removed
1
AN
Reason page is being submitted.

431
Location Within Site

(Tank Unit)

70
AN
Optional.  Location of tank within site.

432
Tank ID #

(Tank Unit)

6
AN
Owner's tank ID#.  This is a unique tank number used by owner to identify the tank.

433
Tank Manufacturer

30
AN
Name of company that manufactured tank.

434
Compartmentalized Tank
Y or N
1
AN
Indicates whether the tank is compartmentalized. Each compartment is considered a separate tank and requires the completion of separate tank pages.

435
Date Tank Installed
YYYYMM
6
D
Year and month the tank installation was completed.

436
Tank Capacity

7
N
Tank capacity in gallons.

437
Number of Tank Compartments

2
AN
Number of compartments in compartmentalized tank.

438
Additional Description
Narrative
70
AN
For local use only.  Additional tank or location description.

439
Tank Use
01 = motor vehicle fuel

02 = non-fuel petroleum

03 = chemical product

04 = hazardous waste

        (includes used oil)

95 = unknown
2
AN
Type of hazardous materials stored.

440
Petroleum Type
1a = regular unleaded

1b = premium unleaded

1c = midgrade unleaded

02 = leaded

03 = diesel

04 = gasohol

05 = jet fuel

06 = aviation gas

99 = other
2
AN
Type of fuel if tank stores vehicle fuel.

441
Common Name

(Tank Unit)

30
AN
Common name of substance stored.  Same as on Hazardous Materials Inventory - Chemical Description page.

442
CAS # (Tank Unit)

15
AN
CAS # of chemical stored in UST.  Same as the CAS # on the Hazardous Material Inventory -  Chemical Description page.

443
Type of Tank
01 = single wall

02 = double wall

03 = single wall w/ exterior

        membrane liner

04 = single wall in a vault

05 = single wall w/ internal

        bladder system

95 = unknown

99 = other
2
AN
Type of tank construction.

444
Tank Material

(primary tank)
01 = bare steel

02 = stainless steel

03 = fiberglass / plastic

04 = steel clad w/ fiberglass

        reinforced plastic (frp)

05 = concrete

08 = frp compatible w/ 100%

        methanol

95 = unknown

99 = other
2
AN
Construction material of the primary tank.

445
Tank Material

(secondary tank)
01 = bare steel

02 = stainless steel

03 = fiberglass / plastic

04 = steel clad w/ fiberglass

        reinforced plastic (frp)

05 = concrete

08 = frp compatible w/ 100%

        methanol

09 = frp non-corrodible jacket

10 = coated steel

95 = unknown

99 = other
2
AN
Construction material of the secondary tank.

446
Tank Interior Lining

or Coating
01 = rubber‑lined

02 = alkyd lining

03 = epoxy lining

04 = phenolic lining

05 = glass lining

06 = unlined

95 = unknown

99 = other
2
AN
Construction material of the interior lining or coating.

447
Date Tank Interior

Lining Installed
YYYYMMDD
8
N
For local use only.  Date interior lining or coating installed.

448
Other Tank Corrosion Protection
01 = manufactured cathodic               protection

02 = sacrificial anode

03 = fiberglass reinforced                   plastic (frp)

04 = impressed current

95 = unknown

99 = other
2
AN
Other tank corrosion protection methods, if applicable.

449
Date Tank Corrosion Protection Installed
YYYYMMDD
8
N
For local use only. Date tank corrosion protection installed.

450-1
Year Spill and Overfill Installed = spill containment
YYYY
4
N
Year spill containment installed.

450-2
Year Spill and Overfill Installed = drop tube 
YYYY
4
N
Year drop tube installed.

450-3
Year Spill and Overfill Installed = striker plate
YYYY
4
N
Year striker plate installed.

451-1
Type of Spill Protection = spill containment
Narrative
15
AN
For local use only.  Type of tank spill protection.

451-2
Type of Spill Protection = drop tube
Narrative
15
AN
For local use only.  Type of tank spill protection.

451-3
Type of Spill Protection = striker plate
Narrative
15
AN
For local use only.  Type of tank spill protection.

452-1
Year Overfill Protection Equipment Installed = alarm
YYYY
4
N
Year alarm installed.

452-2
Year Overfill Protection Equipment Installed =

ball float
YYYY
4
N
Year ball float installed.

452-3
Year Overfill Protection Equipment Installed = fill tube shut off valve
YYYY
4
N
Year fill tube shut off valve installed.

452-4
Overfill Protection Equipment = exempt
Y or N
1
AN
Indicates exemption from overfill protection.

453-1


Tank Leak Detection (Single Wall) = visual (exposed portion only)
Y or N
1
AN
Type of tank leak detection.

453-2
Tank Leak Detection (Single Wall) = automatic tank gauging (ATG)
Y or N
1
AN
Type of tank leak detection.

453-3
Tank Leak Detection (Single Wall) = continuous ATG
Y or N
1
AN
Type of tank leak detection.

453-4
Tank Leak Detection (Single Wall) = statistical inventory reconciliation (SIR) + biennial tank testing
Y or N
1
AN
Type of tank leak detection.

453-5
Tank Leak Detection (Single Wall) = manual 

tank gauging (MTG)
Y or N
1
AN
Type of tank leak detection.

453-6
Tank Leak Detection (Single Wall) = vadose zone
Y or N
1
AN
Type of tank leak detection.

453-7
Tank Leak Detection (Single Wall) = groundwater
Y or N
1
AN
Type of tank leak detection.

453-8
Tank Leak Detection (Single Wall) = tank testing
Y or N
1
AN
Type of tank leak detection.

453-99
Tank Leak Detection (Single Wall) = other
Narrative
30
AN
Type of tank leak detection.

454-1
Tank Leak Detection (Double Wall) = visual (single wall in vault only)
Y or N
1
AN
Type of tank leak detection.

454-2
Tank Leak Detection (Double Wall) = continuous interstitial monitoring
Y or N
1
AN
Type of tank leak detection.

454-3
Tank Leak Detection (Double Wall) = manual monitoring
Y or N
1
AN
Type of tank leak detection.

455
Estimated Date Last Used
YYYYMMDD
8
D
Date tank last used (for closure in place).

456
Estimated Quantity of Substance Remaining

in Tank

7
N
Estimated quantity of hazardous substance remaining in gallons (for closure in place).

457
Tank Filled with 

Inert Material
Y or N


1
AN
Indicates whether tank was filled with an inert material prior to closure (for closure in place).

458-1


Piping System Type (Underground) = pressure
Y or N
1
AN
Type of underground piping system.

458-2
Piping System Type (Underground) = suction
Y or N
1
AN
Type of underground piping system.

458-3
Piping System Type (Underground) = gravity
Y or N
1
AN
Type of underground piping system.

459-1
Piping System Type (Aboveground) = pressure
Y or N
1
AN
Type of aboveground piping system.

459-2
Piping System Type (Aboveground) = suction
Y or N
1
AN
Type of aboveground piping system.

459-3
Piping System Type (Aboveground) = gravity
Y or N
1
AN
Type of aboveground piping system.

460-1


Piping Construction (Underground) = single wall
Y or N
1
AN
Type of underground piping construction.

460-2
Piping Construction (Underground) = double wall
Y or N
1
AN
Type of underground piping construction.

460-3
Piping Construction (Underground) = lined trench
Y or N
1
AN
Type of underground piping construction.

460-95
Piping Construction (Underground) = unknown
Y or N
1
AN
Type of underground piping construction.

460-99
Piping Construction (Underground) = other
Y or N
1
AN
Type of underground piping construction.

461
Piping Manufacturer (Underground)
Narrative
30
AN
Name of underground piping manufacturer.

462-1
Piping Construction (Aboveground) = single wall
Y or N
1
AN
Type of aboveground piping construction.

462-2
Piping Construction (Aboveground) = double wall
Y or N
1
AN
Type of aboveground piping construction.

462-95
Piping Construction (Aboveground) = unknown
Y or N
1
AN
Type of aboveground piping construction.

462-99
Piping Construction (Aboveground) = other
Y or N
1
AN
Type of aboveground piping construction.

463
Piping Manufacturer (Aboveground)
Narrative
30
AN
Name of aboveground piping manufacturer.

464-1
Piping Material and Corrosion Protection (Underground) = bare steel
Y or N
1
AN
Construction material and/or corrosion protection of underground piping.

464-2
Piping Material and Corrosion Protection (Underground) = stainless steel
Y or N
1
AN
Construction material and/or corrosion protection of underground piping.

464-3
Piping Material and Corrosion Protection (Underground) = plastic compatible with contents
Y or N
1
AN
Construction material and/or corrosion protection of underground piping.

464-4
Piping Material and Corrosion Protection (Underground) = fiberglass
Y or N
1
AN
Construction material and/or corrosion protection of underground piping.

464-5
Piping Material and Corrosion Protection (Underground) = steel w/ coating
Y or N
1
AN
Construction material and/or corrosion protection of underground piping.

464-6
Piping Material and Corrosion Protection (Underground) = frp compatible w/ 100% methanol
Y or N
1
AN
Construction material and/or corrosion protection of underground piping.

464-7
Piping Material and Corrosion Protection (Underground) =

galvanized steel
Y or N
1
AN
Construction material and/or corrosion protection of underground piping.

464-8
Piping Material and Corrosion Protection (Underground) = flexible (HDPE - high density polyethylene)
Y or N
1
AN
Construction material and/or corrosion protection of underground piping.

464-9
Piping Material and Corrosion Protection (Underground) =

cathodic protection
Y or N
1
AN
Construction material and/or corrosion protection of underground piping.

464-95
Piping Material and Corrosion Protection (Underground) = unknown
Y or N
1
AN
Construction material and/or corrosion protection of underground piping.

464-99
Piping Material and Corrosion Protection (Underground) = other
Y or N
1
AN
Construction material and/or corrosion protection of underground piping.

465-1
Piping Material and Corrosion Protection (Aboveground) = bare steel
Y or N
1
AN
Construction material and/or corrosion protection of aboveground piping.

465-2
Piping Material and Corrosion Protection (Aboveground) =

stainless steel
Y or N
1
AN
Construction material and/or corrosion protection of aboveground piping.

465-3
Piping Material and Corrosion Protection (Aboveground) = plastic compatible with contents
Y or N
1
AN
Construction material and/or corrosion protection of aboveground piping.

465-4
Piping Material and Corrosion Protection (Aboveground) = fiberglass
Y or N
1
AN
Construction material and/or corrosion protection of aboveground piping.

465-5
Piping Material and Corrosion Protection (Aboveground) =

steel w/ coating
Y or N
1
AN
Construction material and/or corrosion protection of aboveground piping.

465-6
Piping Material and Corrosion Protection (Aboveground) = frp compatible w/ 100% methanol
Y or N
1
AN
Construction material and/or corrosion protection of aboveground piping.

465-7
Piping Material and Corrosion Protection (Aboveground) =

galvanized steel
Y or N
1
AN
Construction material and/or corrosion protection of aboveground piping.

465-8
Piping Material and Corrosion Protection (Aboveground) = flexible (HDPE - high density polyethylene)
Y or N
1
AN
Construction material and/or corrosion protection of aboveground piping.

465-9
Piping Material and Corrosion Protection (Aboveground) =

cathodic protection
Y or N
1
AN
Construction material and/or corrosion protection of aboveground piping.

465-95
Piping Material and Corrosion Protection (Aboveground) = unknown
Y or N
1
AN
Construction material and/or corrosion protection of aboveground piping.

465-99
Piping Material and Corrosion Protection (Aboveground) = other
Y or N
1
AN
Construction material and/or corrosion protection of aboveground piping.

466-1
Piping Leak Detection (Underground - single wall) = electronic line leak detector + auto shutoff + alarms
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-2
Piping Leak Detection (Underground - single wall) = monthly 0.2 gph test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-3
Piping Leak Detection (Underground - single wall) = annual integrity test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-5
Piping Leak Detection (Underground - single wall) = daily visual monitoring + triennial integrity test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-7
Piping Leak Detection (Underground - single wall) = self monitoring
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-9
Piping Leak Detection (Underground - single wall) = biennial integrity test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-10a
Piping Leak Detection (Underground - secondarily contained) = sump sensor + alarms +  auto shutoff for leaks
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-10b
Piping Leak Detection (Underground - secondarily contained) = sump sensor + alarms +  auto shutoff for leaks, failure, and disconnect
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-10c
Piping Leak Detection (Underground - secondarily contained) = sump sensor + alarms + no auto shutoff
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-11
Piping Leak Detection (Underground - secondarily contained, pressure) = automatic leak detector + flow shutoff or restriction
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-12
Piping Leak Detection (Underground - secondarily contained) = annual integrity test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-13
Piping Leak Detection (Underground - secondarily contained, suction/gravity) = sump sensor + alarms
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-14
Piping Leak Detection (Underground - emergency generators) = sump sensor without auto shutoff + alarms
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-15
Piping Leak Detection (Underground - emergency generators) = automatic leak detector without flow shutoff or restriction
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-16
Piping Leak Detection (Underground - emergency generators) = annual integrity test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-17
Piping Leak Detection (Underground - emergency generators) = daily visual check
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

467-1


Piping Leak Detection (Aboveground - single wall) = electronic line leak detector + auto shutoff + alarms
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-2
Piping Leak Detection (Aboveground - single wall) = monthly 0.2 gph test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-3
Piping Leak Detection (Aboveground - single wall) = annual integrity test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-4
Piping Leak Detection (Aboveground - single wall, pressure) = daily visual check
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-5
Piping Leak Detection (Aboveground - single wall, suction) = daily visual monitoring
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-6
Piping Leak Detection (Aboveground - single wall) = triennial integrity test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-7
Piping Leak Detection (Aboveground - single wall) = self monitoring
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-8
Piping Leak Detection (Aboveground - single wall, gravity) = daily visual monitoring
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-9
Piping Leak Detection (Aboveground - single wall) = biennial integrity test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-10a
Piping Leak Detection (Aboveground - secondarily contained) = sump sensor + alarms + auto shutoff

for leaks
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-10b
Piping Leak Detection (Aboveground - secondarily contained) = sump sensor + alarms + auto shutoff for leaks, failure and disconnect
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-10c
Piping Leak Detection (Aboveground - secondarily contained, pressure) = sump sensor + alarms + no auto shutoff
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-11
Piping Leak Detection (Aboveground - secondarily contained) = automatic

leak detector
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-12
Piping Leak Detection (Aboveground - secondarily contained) = annual integrity test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-13
Piping Leak Detection (Aboveground - secondarily contained, suction/gravity) = sump sensor + alarms
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-14
Piping Leak Detection (Aboveground - emergency generators) = sump sensor without auto shutoff + alarms
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-15
Piping Leak Detection (Aboveground - emergency generators) = automatic leak detector
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-16
Piping Leak Detection (Aboveground - emergency generators) = annual integrity test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-17
Piping Leak Detection (Aboveground - emergency generators) = daily visual check
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

468
Date Dispenser Containment Installed
YYYYMMDD
8
N
Date dispenser containment installed.

469
Dispenser Containment Type
1 = float mechanism

2 = sensor + alarms

3 = sensor + auto shutoff +

      alarms

4 = daily visual check

5 = trench liner / monitoring

6 = none
1
AN
Type of dispenser containment.

470
Date Certified (Tank Unit)
YYYYMMDD
8
D
Date the document was signed.

471
Owner/ Operator Name (Tank Unit)

35
AN
Name of signatory.  The applicant certifies to a belief that all the information submitted is accurate and complete. The applicant may be the Owner/Operator or officially designated representative.

472
Owner/ Operator Title

(Tank Unit)

35
AN
Title of person signing the page.

473
Permit Number

9
AN
For local use only.  Permit number.

474
Permit Approved By

35
AN
For local use only.Name of person approving permit.

475
Permit Expiration Date
YYYYMMDD
8
D
For local use only.  Date of permit expiration.

TANKS

C.
UST Installation - Certificate of Compliance Page

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to permit cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of business.

476
Address

(Tank Installation)
Postal standard:

2 lines, 35 characters
70
AN
For local use only.  Address of tank installation.

477
Tank ID #

(Tank Installation)

6
AN
Owner's tank ID#, if there is a tank number used by owner to identify the tank.  Unique identifier of tank at site.  Same as data element # 432.

478
Trained and Certified

by Tank and Piping Manufacturer
Y or N
1
AN
Indicates whether installer was trained and certified by tank and piping manufacturer.

479
Registered Engineer Inspection
Y or N
1
AN
Indicates whether installation was certified by registered professional engineer.

480
Unified Program

Agency Approval
Y or N
1
AN
Indicates whether installation was approved by the Unified Program Agency.

481
Completion of Manufacturer's Checklist
Y or N
1
AN
Indicates whether work on manufacturer's installation checklist was completed.

482
Contractors State License Board Certification or License
Y or N
1
AN
Indicates whether contractor has been certified or licensed by the Contractors State License Board.

483
Installation Description
Narrative
30
AN
Description of installation.

484
Date Certified

(Tank Installation)
YYYYMMDD
8
D
Date tank installation certification was signed.

485
Tank Owner/Agent Name (Installation)

35
AN
Name of tank owner/agent, or officially designated representative of the owner/agent. The signer certifies to a belief that all the information submitted is accurate and complete.

486
Tank Owner/Agent Title (Installation)

35
AN
Title of person signing the page.

IV.
HAZARDOUS WASTE

A.
Recyclable Materials

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

2
EPA ID Number
12 digit identifier beginning with CA
12
AN
EPA Identification number for businesses that generate, recycle, or treat hazardous waste.  For facilities in California, the number usually starts with the letters CA.  The number can be obtained from the Telephone Information Center at

(916) 324-1781, (800) 61-TOXIC or (800) 618-6942.

3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of the business.

500
Beginning Date of

Reporting Period
YYYYMMDD
8
D
Beginning date of the reporting period for this report.  This report is for two calendar years and is due on July 1 of every even numbered year per HSC 25143.10(a).

501
Ending Date of

Reporting Period
YYYYMMDD
8
D
Ending date of the reporting period for this report. This report is for two calendar years and is due on July 1 of every even numbered year per HSC 25143.10(a).

I.  TYPE OF RECYCLING ACTIVITY

502
Onsite Recycling
Y or N
1
AN
Facility must report if it recycles onsite more than 100 kilograms per month of recyclable material under a claim that the material qualifies for exclusion or exemption pursuant to HSC 25143.2.  Triggers requirement to complete parts III and IV of the Recyclable Materials Report.

503
Offsite Recycling
Y or N
1
AN
Facility must report if it recycles more than 100 kilograms per month of recyclable material under a claim that the material qualifies for exclusion received from an offsite location pursuant to HSC 25143.2.  Triggers requirement to complete all parts of the Recyclable Materials Report.

II.  OFFSITE GENERATOR OF RECYCLABLE MATERIAL INFORMATION (repeats for each generator, EXCEPT for the signature information)

504
Offsite Generator Name

35
AN
Facility name of offsite generator of recyclable material.

505
Offsite Generator

EPA ID Number
12 digit identifier beginning with CA
12
AN
EPA Identification number of generator of recyclable material.

506
Street Address

(Offsite Generator)
Postal standard:

2 lines, 35 characters
70
AN
Street address for offsite generator of recyclable material.  No post office box numbers are allowed. This information must provide a means to geographically locate the facility.

507
Phone

(Offsite Generator)
Area code + 7 digit phone number + extension
15
AN
Phone number for offsite generator of recyclable material.

508
City

(Offsite Generator)
Postal standard
20
AN
City or unincorporated area for offsite generator of recyclable material.

509
State

(Offsite Generator)
Valid 2-digit state code
2
AN
State for offsite generator of recyclable material.

510
Zip Code

(Offsite Generator)
Postal standard
9
AN
Zip code for offsite generator of recyclable material.

511
Mailing Address 

(Offsite Generator)
Postal standard:

2 lines, 35 characters
70
AN
Mailing address for offsite generator of recyclable material, if different from street address.

512
City for Mailing Address 

(Offsite Generator)
Postal standard
20
AN
City or unincorporated area for mailing address of offsite generator of recyclable material, if different from street address.

513
State for Mailing Address

(Offsite Generator)
Valid 2-digit state code
2
AN
State for mailing address of offsite generator of recyclable material, if different from street address.

514
Zip Code for Mailing Address (Offsite Generator)
Postal standard
9
AN
Zip code for mailing address of  offsite generator of recyclable material, if different from street address.

III.   CERTIFICATION SECTION

515
Date (Recyclable Materials)
YYYYMMDD
8
D
Date the document was signed.

516
Name of Document Preparer (Recyclable Materials)

35
AN
Full name of person who prepared the Recyclable Materials Report.

517
Certifier Name 

(Recyclable Materials)

35
AN
Full name of person signing the Recyclable Materials Report.

518
Certifier Title

(Recyclable Materials)

35
AN
Title of person signing the Recyclable Materials Report.

IV.  RECYCLABLE MATERIAL INFORMATION
A.  Description

519
Total Number of

Recyclable Materials

3
AN
Total number of recyclable materials reported in this report.  Verify that the number of recyclable material reports (part IV) is the same as the total number listed here.

520
Recyclable Material

Number

3
AN
Unique identification number for each recyclable material reported. They can be numbered sequentially, or by any other system as long as the numbers are not repeated or duplicated.

521
Common Name

(Recyclable Materials)

60
AN
Common name of material recycled.  This is the same as the Common Name on the Hazardous Materials Inventory - Chemical Description page.

522
Quantity During Two

Year Reporting Period

(Recyclable Materials)

10
N
Total quantity of this recyclable material recycled during the two year reporting period.  Round to nearest whole number.

523
Units

(Recyclable Materials)
a = gallons

b = pounds

c = tons

d = kilograms
1
AN
Unit of measure for quantity reported.

524
Recyclable Material Description
Narrative
150
AN
Description of recyclable material.

525
Recycling Process and Beneficial Use
Narrative
150
AN
Description of the recycling process and beneficial use of recyclable material, i.e., how it was used to make or substitute for a product.

526
Authorizing Provision of

HSC Section 25143.2
Statutory citation
20
AN
Subdivision(s), and subparagraph(s) (if applicable) of HSC 25143.2 that served as the basis for the claim to exemption or exclusion.  For example: HSC 25143.2(d)(2)(C).

527
Basis for Claim to Exclusion

or Exemption
Narrative
50
AN
Explanation of the basis for the claim to an exclusion or exemption.

IV.  RECYCLABLE MATERIAL INFORMATION
B.  Product And Constituent Information:  Offsite Only

528
Hazardous Constituent 1

80
AN
Description of hazardous constituent (use common name, if appropriate).

529
Concentration

Recyclable Material 1
6 digits with floating decimal
7
N
Concentration of this hazardous constituent in this recyclable material.

530
Units

Recyclable Material 1
a = percentage

b = parts per million
1
AN
Unit of measure which is most appropriate for this hazardous constituent.

531
Concentration

Final Product 1
6 digits with floating decimal
7
N
Concentration of this hazardous constituent in the final product.

532
Units

Final Product 1
a  = percentage

b = parts per million
1
AN
Unit of measure which is most appropriate for this hazardous constituent.

533
Final Products/

Uses for Constituent 1

60
AN
Final use of the recyclable material.  Description of the product(s) that resulted from the recycling process and how that product is beneficially used.

534
Hazardous Constituent 2

80
AN
Description of hazardous constituent (use common name, if appropriate).

535
Concentration

Recyclable Material 2
6 digits with floating decimal
7
N
Concentration of this hazardous constituent in this recyclable material.

536
Units

Recyclable Material 2
a  = percentage

b = parts per million
1
AN
Unit of measure which is most appropriate for this hazardous constituent.

537
Concentration

Final Product 2
6 digits with floating decimal
7
N
Concentration of this hazardous constituent in the final product.

538
Units 

Final Product 2
a = percentage

b = parts per million
1
AN
Unit of measure which is most appropriate for this hazardous constituent.

539
Final Products/

Uses for Constituent 2

60
AN
Final use of the recyclable material.  Description of the product(s) that resulted from the recycling process and how that product is beneficially used.

540
Hazardous Constituent 3

80
AN
Description of hazardous constituent (use common name, if appropriate).

541
Concentration

Recyclable Material 3
6 digits with floating decimal
7
N
Concentration of this hazardous constituent in this recyclable material.

542
Units

Recyclable Material 3
a = percentage

b = parts per million
1
AN
Unit of measure which is most appropriate for this hazardous constituent.

543
Concentration

Final Product 3
6 digits with floating decimal
7
N
Concentration of this hazardous constituent in the final product.

544
Units

Final Product 3
a = percentage

b = parts per million
1
AN
Unit of measure which is most appropriate for this hazardous constituent.

545
Final Products/

Uses for Constituent 3

60
AN
Final use of the recyclable material.  Description of the product(s) that resulted from the recycling process and how that product is beneficially used.

546
Hazardous Constituent 4

80
AN
Description of hazardous constituent (use common name, if appropriate).

547
Concentration

Recyclable Material 4
6 digits with floating decimal
7
N
Concentration of this hazardous constituent in this recyclable material.

548
Units

Recyclable Material 4
a = percentage

b = parts per million
1
AN
Unit of measure which is most appropriate for this hazardous constituent.

549
Concentration

Final Product 4
6 digits with floating decimal
7
N
Concentration of this hazardous constituent in the final product.

550
Units

Final Product 4 
a = percentage

b = parts per million
1
AN
Unit of measure which is most appropriate for this hazardous constituent.

551
Final Products/

Uses for Constituent 4

60
AN
Final use of the recyclable material.  Description of the product(s) that resulted from the recycling process and how that product is beneficially used.

If more than four hazardous constituents are present in one recyclable material, submit this information on additional sheets.  The information is not submitted electronically unless the CUPA has established local data standards.

V.  DOCUMENTATION OF KNOWN MARKET

552
Documentation

(Offsite Recycling)
Y or N
1
AN
If the recycler is an offsite facility, indicates if documentation is provided.  Documentation is required pursuant to HSC 25143.10(a)(3)(A) to show that there was a known market for disposition of the recyclable material and any products manufactured from it.  A copy of this report must be submitted to the generator of the material.

IV.
HAZARDOUS WASTE

1.  Onsite Hazardous Waste Treatment Notification - Facility

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of business.

600
Notification Status

(Onsite Hazardous

Waste Facility)
a = amended

b = initial

c = renewal
1
AN
Notification status under the tiered permitting system.

601a


Permit Status =

facility permit
Y or N
1
AN
Permit status for State issued hazardous waste permits or grants of authorization.

601b
Permit Status =

interim status
Y or N
1
AN
Permit status for State issued hazardous waste permits or grants of authorization.

601c
Permit Status = standardized permit
Y or N
1
AN
Permit status for State issued hazardous waste permits or grants of authorization.

601d
Permit Status = variance
Y or N
1
AN
Permit status for State issued hazardous waste permits or grants of authorization.

601e
Permit Status =

consent agreement
Y or N
1
AN
Permit status for State issued hazardous waste permits or grants of authorization.

602a


Number of Units = CESQT
Maximum 3 digit number


3
AN
Number of units at facility.  Attach a unit specific notification page for EACH unit covered by this notification, except CE-CL units.

602b
Number of Units = CESW
Maximum 3 digit number
3
AN
Number of units at facility.  Attach a unit specific notification page for EACH unit covered by this notification, except CE-CL units.

602c
Number of Units = CA
Maximum 3 digit number
3
AN
Number of units at facility.  Attach a unit specific notification page for EACH unit covered by this notification, except CE-CL units.

602d
Number of Units = PBR
Maximum 3 digit number
3
AN
Number of units at facility.  Attach a unit specific notification page for EACH unit covered by this notification, except CE-CL units.

602e
Number of Units = CEL
Maximum 3 digit number
3
AN
Number of units at facility.  Attach a unit specific notification page for EACH unit covered by this notification, except CE-CL units.

602f
Number of Units = CE-CL
Maximum 3 digit number
3
AN
Number of CE-CL (commercial laundries) units at facility.  No unit specific notification is required for

CE-CL.

602g
Number of Units = total
Maximum 3 digit number
3
AN
Total number of units at facility.  Verify that this number equals the number of unit notification pages plus CE-CL units.

603
Date Certified

(Onsite Hazardous

Waste Facility)
YYYYMMDD
8
D
Date the page was signed.

604
Owner / Operator Name (Onsite Hazardous

Waste Facility)

35
AN
Full name of business owner/operator, or officially designated representative of the owner/operator. The signer certifies to a belief that all the information submitted is accurate and complete.  The person signing this page must be an owner or officer of the company who is authorized to make decisions for the facility and who has operational control.  In most companies, this is not the environmental compliance or technical staff.   The signer of the certifications attests to their accuracy under penalty of law for submitting false information.  The certifications cover waste minimization, the eligibility of the unit(s) for the indicated tier, the fact that the unit meets all of the operating requirements for that tier, and that the information is accurate.

605
Owner / Operator Title (Onsite Hazardous

Waste Facility)

35
AN
Title of person signing the page.

IV.
HAZARDOUS WASTE

3.  Onsite Hazardous Waste Treatment Notification - Unit 

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of business.

606
Unit ID Number

(Onsite Hazardous Waste Facility)

18
AN
Unique identification number for unit. The units can be numbered sequentially, or by any other system as long as the numbers are not repeated or duplicated.

607

Unit Type / Tier

(Onsite Hazardous Waste Facility)
a = CESQT

b = CESW

c = CA

d = PBR

e = CEL
1
AN
Unit type under the tiered permitting program.

608
Number of Tanks

(Onsite Hazardous Waste Facility)

2
AN
Number of tanks containing hazardous waste. "Tank" means a stationary device, designed to contain an accumulation of hazardous waste which is constructed primarily of non-earthen materials (e.g., wood, concrete, steel, plastic) which provide structural support.

609
Number Of Containers/

Treatment Areas

2
AN
Number of containers and container treatment areas for hazardous waste.  "Container" means any device that is open or closed, and portable in which a material can be stored, handled, treated, transported, recycled, or disposed of.  "Container treatment area" is the location set aside and used to treat containers, such as drums.

610
Unit Name

30
AN
Name of treatment unit.  A unit is defined as a tank, a container, or a combination of tanks or tank systems and/or containers located together that are used in sequence to treat or accumulate one or more compatible hazardous wastestreams. The devices are either plumbed together or otherwise linked so as to form one system.

611
Monthly Treatment Volume

8
AN
Estimated monthly total volume of hazardous waste treated in each unit. If the volume fluctuates significantly by month, enter the maximum or highest volume treated in any month.

612
Unit Of Measure
a = pounds

b = gallons
1
AN
Unit of measure for monthly treatment volume.

613
Specific Waste Type Treated
Narrative
150
AN
Description of the specific waste type(s) treated. For example, if waste qualifies as an aqueous waste with metal or organics, indicate the specific metals or organics.

614
Treatment Process Description
Narrative
150
AN
Description of treatment process(es) used.

615a


Basis for Not Needing Federal

Permit = treated waste is not a hazardous waste under federal law (California ‑ only waste)


Y or N


1
AN
Basis for determining that a hazardous waste permit is not required under RCRA and the federal regulations adopted under RCRA (40 CFR).  Only onsite treatment operations that are not required to obtain a federal permit are eligible for the onsite permitting tiers.  Indicate at least one reason to prove eligibility for the onsite treatment tiers. Contact the CUPA, DTSC Regional Office, the U.S. EPA's Region IX RCRA Information Line at (415)744-2074, or the U.S. EPA RCRA Hotline at (800)424-9346.

615b
Basis for Not Needing Federal

Permit = treated in waste water treatment units (tanks) and discharged to a publicly owned treatment works (POTW) / sewering agency or under an NPDES permit


Y or N
1
AN
Basis for determining that a hazardous waste permit is not required under RCRA and the federal regulations adopted under RCRA (40 CFR). Wastewater treatment unit means a device which (1) is part of a wastewater treatment facility regulated under section 402 or 307(b) of the Clean Water Act, and (2) receives and treats or stores an influent wastewater that is a hazardous waste or that generates and accumulates a wastewater treatment sludge that is a hazardous waste or that treats or stores a wastewater treatment sludge which is a hazardous waste, and (3) meets the definition of tank or tank system.  This term is defined in 40 CFR 260.10.

615c
Basis for Not Needing Federal

Permit = treatment in elementary neutralization units
Y or N
1
AN
Basis for determining that a hazardous waste permit is not required under RCRA and the federal regulations adopted under RCRA (40 CFR). Elementary neutralization unit means a device which: (1) is used for neutralizing wastes that are hazardous only because they exhibit the corrosivity characteristic or they are listed only for this reason: and (2) meets the definition of tank, tank system, container, transport vehicle, or vessel.  This term is defined in 40 CFR 260.10.

615d
Basis for Not Needing Federal

Permit = treatment in a totally enclosed treatment facility
Y or N
1
AN
Basis for determining that a hazardous waste permit is not required under RCRA and the federal regulations adopted under RCRA (40 CFR).  Totally enclosed treatment facility means a facility for the treatment of hazardous waste which is directly connected to an industrial production process and which is constructed and operated in a manner which prevents the release of any hazardous waste or any constituent thereof into the environment during treatment.  This term is defined in 40 CFR 260.10.

615e
Basis for Not Needing Federal

Permit = federal conditionally

exempt small quantity generator [generated 100 kg (approximately

27 gallons) or less] of hazardous waste in a calendar month
Y or N
1
AN
Basis for determining that a hazardous waste permit is not required under RCRA and the federal regulations adopted under RCRA (40 CFR).  See description in item 615a above.

615f
Basis for Not Needing Federal

Permit = treatment in an accumulation tank or container within 90 days for over 1000 kg/month generators and 180 or 270 days for generators of 100 to 1000 kg/month
Y or N
1
AN
Basis for determining that a hazardous waste permit is not required under RCRA and the federal regulations adopted under RCRA (40 CFR).  See description in item 615a above.

615g
Basis for Not Needing Federal

Permit = recyclable materials

are reclaimed to recover silver

or other precious metals
Y or N
1
AN
Basis for determining that a hazardous waste permit is not required under RCRA and the federal regulations adopted under RCRA (40 CFR).  See description in item 615a above.

615h
Basis for Not Needing Federal

Permit = empty container rinsing and/or treatment
Y or N
1
AN
Basis for determining that a hazardous waste permit is not required under RCRA and the federal regulations adopted under RCRA (40 CFR).  See description in item 615a above.

615i
Basis for Not Needing Federal

Permit = other
Narrative
30
AN
Description of basis for determining that a hazardous waste permit is not required under RCRA and the federal regulations adopted under RCRA (40 CFR).  See item 615a above.

616a


Residuals Management Description = discharge non-hazardous

aqueous waste to POTW or

sewer
Y or N
1
AN
Residual management options.  If any hazardous residuals are disposed or hauled offsite, indicates where the waste is sent.

616b
Residuals Management Description = discharge non-hazardous aqueous waste under a NPDES permit
Y or N
1
AN
See description in item 616a above.

616c
Residuals Management Description = dispose of non-hazardous solid waste residues at an offsite location
Y or N
1
AN
See description in item 616a above.

616d
Residuals Management Description = offsite recycling
Y or N
1
AN
See description in item 616a above.

616e
Residuals Management Description = thermal treatment
Y or N
1
AN
See description in item 616a above.

616f
Residuals Management Description = disposal to land
Y or N
1
AN
See description in item 616a above.

616g
Residuals Management Description = further treatment
Y or N
1
AN
See description in item 616a above.

616h
Residuals Management Description = other method of disposal
Narrative
30
AN
See description in item 616a above.

617
Secondary Containment

Installation Date
YYYYMMDD
8
D
Date secondary containment installed.

IV.
HAZARDOUS WASTE

C.
Onsite Tiered Permitting - Waste and Treatment Process Combinations

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE
INFORMATION DESCRIPTION

606
Unit ID Number

18
AN
Unique identification number for unit. The units can be numbered sequentially or by any other system as long as the numbers are not repeated or duplicated.

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to permit cross linking of data. County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

INFORMATION DESCRIPTION - Conditionally Exempt Small Quantity Treatment (CESQT) Waste and Treatment Process Combinations.  These are all of the eligible waste streams and treatment processes that are available within the tier.  NOTE: PBR codes are the same as CESQT.

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE

627-1a
Aqueous Waste ‑ Hexavalent Chromium Reduction
Y or N
1
AN

627-2a
Aqueous Waste w/Metals ‑ pH Adjustment / Neutralization
Y or N
1
AN

627-2b
Aqueous Waste w/Metals ‑ Precipitation or Crystallization
Y or N
1
AN

627-2c
Aqueous Waste w/Metals ‑ Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

627-2d
Aqueous Waste w/Metals ‑ Ion Exchange
Y or N
1
AN

627-2e
Aqueous Waste w/Metals ‑ Reverse Osmosis
Y or N
1
AN

627-2f
Aqueous Waste w/Metals ‑ Metallic Replacement
Y or N
1
AN

627-2g
Aqueous Waste w/Metals ‑ Plating onto an Electrode
Y or N
1
AN

627-2h
Aqueous Waste w/Metals ‑ Electrodialysis
Y or N
1
AN

627-2i
Aqueous Waste w/Metals ‑ Electrowinning or Electrolytic Recovery
Y or N
1
AN

627-2j
Aqueous Waste w/Metals ‑ Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

627-2k
Aqueous Waste w/Metals ‑ Evaporation
Y or N
1
AN

627-2l
Aqueous Waste w/Metals ‑ Adsorption
Y or N
1
AN

627-3a
Aqueous Waste w/Organics (<10% Organic and <1% Volatiles) ‑ Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

627-3b
Aqueous Waste w/Organics (<10% Organic and <1% Volatiles) ‑ Adsorption
Y or N
1
AN

627-3c
Aqueous Waste w/Organics (<10% Organic and <1% Volatiles) ‑ Distillation
Y or N
1
AN

627-3d
Aqueous Waste w/Organics (<10% Organic and <1% Volatiles) ‑ Biological Process Using Microorganisms
Y or N
1
AN

627-3e
Aqueous Waste w/Organics (<10% Organic and <1% Volatiles) ‑ Photodegradation in Enclosed System
Y or N
1
AN

627-3f
Aqueous Waste w/Organics (<10% Organic and <1% Volatiles) ‑ Air Stripping or Steam Stripping
Y or N
1
AN

IV.
HAZARDOUS WASTE

C.
Onsite Tiered Permitting - Waste and Treatment Process Combinations

INFORMATION DESCRIPTION - Conditionally Exempt Small Quantity Treatment (CESQT) Waste and Treatment Process Combinations.  These are all of the eligible waste streams and treatment processes that are available within the tier.  NOTE: PBR codes are the same as CESQT.

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE

627-4a
Sludges, Dusts, Solids w/Metal(s) - Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

627-4b
Sludges, Dusts, Solids w/Metal(s) - Grind, Shred, Crush, or Compact
Y or N
1
AN

627-4c
Sludges, Dusts, Solids w/Metal(s) - Drying to Remove Water
Y or N
1
AN

627-4d
Sludges, Dusts, Solids w/Metal(s) ‑ Separation by Size, Magnetism, or Density
Y or N
1
AN

627-5a
Sludges w/Alum, Gypsum, Lime, Sulfur, or Phosphate ‑ Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

627-5b
Sludges w/Alum, Gypsum, Lime, Sulfur, or Phosphate ‑ Drying to Remove Water
Y or N
1
AN

627-5c
Sludges w/Alum, Gypsum, Lime, Sulfur, or Phosphate - Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

627-6a
Special Waste (Sec. 66261.120) ‑ Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

627-6b
Special Waste (Sec. 66261.120) ‑ Drying to Remove Water
Y or N
1
AN

627-6c
Special Waste (Sec. 66261.120) ‑ Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

627-6d
Special Waste (Sec. 66261.120) ‑ Screening Based on Size
Y or N
1
AN

627-6e
Special Waste (Sec. 66261.120) ‑ Separation by Size, Magnetism, or Density
Y or N
1
AN

627-7a
Special Waste (Sec. 66261.124) ‑ Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

627-7b
Special Waste (Sec. 66261.124) ‑ Drying to Remove Water
Y or N
1
AN

627-7c
Special Waste (Sec. 66261.124) ‑ Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

627-7d
Special Waste (Sec. 66261.124) ‑ Magnetic Separation
Y or N
1
AN

627-8a
Inorganic Acid/Alkaline Waste ‑ pH Adjustment / Neutralization
Y or N
1
AN

627-9a
Soils w/Metal(s) ‑ Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

627-9b
Soils w/Metal(s) ‑ Separation by Size
Y or N
1
AN

627-9c
Soils w/Metal(s) ‑ Magnetic Separation
Y or N
1
AN

627-10a
Used Oil, Mixed Oil, Oily Water, Oil/Water Sludges - Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

627-10b
Used Oil, Mixed Oil, Oily Water, O/W Sludges ‑ Distillation
Y or N
1
AN

627-10c
Used Oil, Mixed Oil, Oily Water, O/W Sludges ‑ Neutralization
Y or N
1
AN

IV.
HAZARDOUS WASTE

C.
Onsite Tiered Permitting - Waste and Treatment Process Combinations

INFORMATION DESCRIPTION - Conditionally Exempt Small Quantity Treatment (CESQT) Waste and Treatment Process Combinations.  These are all of the eligible waste streams and treatment processes that are available within the tier.  NOTE: PBR codes are the same as CESQT.

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE

627-10d
Used Oil, Mixed Oil, Oily Water, O/W Sludges ‑ Separation by Size, Magnetism, or Density
Y or N
1
AN

627-10e
Used Oil, Mixed Oil, Oily Water, O/W Sludges ‑ Reverse Osmosis
Y or N
1
AN

627-10f
Used Oil, Mixed Oil, Oily Water, O/W Sludges ‑ Biological Process Using Microorganisms
Y or N
1
AN

627-11a
Containers (< 110 Gallons) or Liners - Rinsing with Liquid
Y or N
1
AN

627-11b
Containers (< 110 Gallons) or Liners - Crush, Shred, Grind, or Puncture
Y or N
1
AN

627-12a
Multi-component Resins - Mixing per Manufacturer's Instructions
Y or N
1
AN

627-13
Wastestream & Treatment Technology Combination Certified by DTSC per 

HSC 25200.1.5
Valid Certified Technology Number
10
AN

IV.
HAZARDOUS WASTE

C.
Onsite Tiered Permitting - Waste and Treatment Process Combinations

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE
INFORMATION DESCRIPTION

606
Unit ID Number

18
AN
Unique identification number for unit. The units can be numbered sequentially or by any other system as long as the numbers are not repeated or duplicated.

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to permit cross linking of data. County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

INFORMATION DESCRIPTION - Conditionally Exempt Specified Wastestreams (CESW) Waste and Treatment Process Combinations.  These are all of the elibible waste streams and Treatment processes that are available within the tier.

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE

628-1
Resins - Mixing or Curing per Manufacturer's Instructions
Y or N
1
AN

628-2
Containers (<110 Gallons) ‑ Rinse, Crush, Shred, Grind or Puncture
Y or N
1
AN

628-3
Special Waste (Sec. 66261.124) - Drying by Evaporation or Pressing to Remove Water
Y or N
1
AN

628-4
Special Waste (Sec. 66261.124) -  Magnetic Separation or Screening
Y or N
1
AN

628-8a
Gravity Settling ‑ Settling Solids w/Non-hazardous Liquid
Y or N
1
AN

628-8b
Gravity Settling ‑ Separation of Oil/Water Mix or Sludges (<25 Barrels/mo)
Y or N
1
AN

628-9
Neutralizing Acidic/Alkaline Material ‑ Certified or Educational Laboratory or

<1 Gallon/batch of Laboratory Waste
Y or N
1
AN

628-10
Hazardous Waste Treatment Carried Out in Q/C or Q/A Labs (not Offsite Facility)
Y or N
1
AN

628-11
Wastestream & Treatment Technology Combination Certified by DTSC per

HSC 25200.1.5
Valid Certified Technology Number
10
AN

628-12
Formaldehyde / Glutaraldehyde by Healthcare Facilities Using Equipment Certified by DTSC per HSC 25200.1.5
Valid Certified Technology Number
10
AN

IV.
HAZARDOUS WASTE

C.
Onsite Tiered Permitting - Waste and Treatment Process Combinations

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE
INFORMATION DESCRIPTION

606
Unit ID Number

18
AN
Unique identification number for unit. The units can be numbered sequentially or by any other system as long as the numbers are not repeated or duplicated.

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to permit cross linking of data. County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

INFORMATION DESCRIPTION - Conditionally Authorized (CA) Waste and Treatment Process Combinations.  These are all of the eligible waste streams and treatment processes that are available within the tier.

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE

629-1a
Aqueous Waste w/Inorganics <1,400 ppm - Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

629-1b
Aqueous Waste w/Inorganics <1,400 ppm - Ion Exchange / Metallic Replacement
Y or N
1
AN

629-1c
Aqueous Waste w/Inorganics <1,400 ppm - Reverse Osmosis
Y or N
1
AN

629-1d
Aqueous Waste w/Inorganics <1,400 ppm - Adsorption
Y or N
1
AN

629-1e
Aqueous Waste w/Inorganics <1,400 ppm ‑ pH Adjustment
Y or N
1
AN

629-1f
Aqueous Waste w/Inorganics <1,400 ppm - Electrowinning if no Hydrochloric Acid
Y or N
1
AN

629-1g
Aqueous Waste w/Inorganics <750 ppm ‑ Hexavalent Chromium Reduction
Y or N
1
AN

629-2a
Aqueous Waste w/Organics <750 ppm ‑ Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

629-2b
Aqueous Waste w/Organics <750 ppm ‑ Adsorption
Y or N
1
AN

629-3a
Sludges, Dusts, Solids w/Metal(s) <750 ppm ‑ Filter, Gravity, Grind, Crush, or Compact
Y or N
1
AN

629-3b
Sludges, Dusts, Solids w/Metal(s) <750 ppm ‑ Drying to Remove Water
Y or N
1
AN

629-3c
Sludges, Dusts, Solids w/Metal(s) <750 ppm ‑ Separation by Size, Magnetism, or Density
Y or N
1
AN

629-4a
Sludges w/ Alum, Gypsum, Lime, Phosphate <5,000 Gal / 45,000 Lb/mo ‑ Drying to Remove Water
Y or N
1
AN

629-4b
Sludges w/ Alum, Gypsum, Lime, Phosphate < 5,000 Gal / 45,000 Lb/mo ‑ Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

629-5a
Special Waste (Sec. 66261.120) w/<750 ppm - Drying to Remove Water
Y or N
1
AN

629-5b
Special Waste (Sec. 66261.120) w/<750 ppm - Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

629-5c
Special Waste (Sec. 66261.120) w/<750 ppm - Screening Based on Size
Y or N
1
AN

629-5d
Special Waste (Sec. 66261.120) w/<750 ppm - Separation by Size, Magnetism, or Density
Y or N
1
AN

IV.
HAZARDOUS WASTE

C.
Onsite Tiered Permitting - Waste and Treatment Process Combinations

INFORMATION DESCRIPTION - Conditionally Authorized (CA) Waste and Treatment Process Combinations.  These are all of the eligible waste streams and treatment processes that are available within the tier.

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE

629-6a
Special Waste (Sec. 66261.124) w/<750 ppm - Drying to Remove Water
Y or N
1
AN

629-6b
Special Waste (Sec. 66261.124) w/<750 ppm - Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

629-6c
Special Waste (Sec. 66261.124) w/<750 ppm - Magnetic Separation
Y or N
1
AN

629-7a
Soils w/Metals <45,000 Lb / < 5000 Gal/mo - Separation by Size
Y or N
1
AN

629-7b
Soils w/Metals <45,000 Lb / < 5000 Gal/mo - Magnetic Separation
Y or N
1
AN

629-8a
Oily Water, Oil/Water Sludges ‑ Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

629-8b
Oily Water, Oil/Water Sludges ‑ Separation by Size, Magnetism, or Density
Y or N
1
AN

629-8c
Oily Water, Oil/Water Sludges - Reverse Osmosis
Y or N
1
AN

629-9a
Alkaline/Acidic Waste <10% by Weight  - Neutralization
Y or N
1
AN

629-9b
Alkaline/Acidic Waste >10% by Weight (<500 Gallons/batch) - Neutralization 
Y or N
1
AN

629-12
Wastestream & Treatment Technology Combination Certified by DTSC per

HSC 25200.1.5
Valid Certified Technology Number
10
AN

IV.
HAZARDOUS WASTE

C.
Onsite Tiered Permitting - Waste and Treatment Process Combinations

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE
INFORMATION DESCRIPTION

606
Unit ID Number

18
AN
Unique identification number for unit. The units can be numbered sequentially or by any other system as long as the numbers are not repeated or duplicated.

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to permit cross linking of data. County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

INFORMATION DESCRIPTION - Permit by Rule (PBR) Waste and Treatment Process Combinations.  These are all of the eligible waste streams and treatment processes that are available within the tier.  NOTE: PBR codes are the same as CESQT.

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE

630-1a
Aqueous Waste ‑ Hexavalent Chromium Reduction
Y or N
1
AN

630-2a
Aqueous Waste w/Metals ‑ pH Adjustment / Neutralization
Y or N
1
AN

630-2b
Aqueous Waste w/Metals ‑ Precipitation or Crystallization
Y or N
1
AN

630-2c
Aqueous Waste w/Metals ‑ Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

630-2d
Aqueous Waste w/Metals ‑ Ion Exchange
Y or N
1
AN

630-2e
Aqueous Waste w/Metals ‑ Reverse Osmosis
Y or N
1
AN

630-2f
Aqueous Waste w/Metals ‑ Metallic Replacement
Y or N
1
AN

630-2g
Aqueous Waste w/Metals ‑ Plating onto an Electrode
Y or N
1
AN

630-2h
Aqueous Waste w/Metals ‑ Electrodialysis
Y or N
1
AN

630-2i
Aqueous Waste w/Metals ‑ Electrowinning or Electrolytic Recovery
Y or N
1
AN

630-2j
Aqueous Waste w/Metals ‑ Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

630-2k
Aqueous Waste w/Metals ‑ Evaporation
Y or N
1
AN

630-2l
Aqueous Waste w/Metals ‑ Adsorption
Y or N
1
AN

630-3a
Aqueous Waste w/Organics (<10% Organic and <1% Volatiles) ‑ Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

630-3b
Aqueous Waste w/Organics (<10% Organic and <1% Volatiles) ‑ Adsorption
Y or N
1
AN

630-3c
Aqueous Waste w/Organics (<10% Organic and <1% Volatiles) ‑ Distillation
Y or N
1
AN

630-3d
Aqueous Waste w/Organics (<10% Organic and <1% Volatiles) ‑ Biological Process Using Microorganisms
Y or N
1
AN

630-3e
Aqueous Waste w/Organics (<10% Organic and <1% Volatiles) ‑ Photodegradation in Enclosed System
Y or N
1
AN

630-3f
Aqueous Waste w/Organics (<1% Volatiles) - Air Stripping or Steam Stripping
Y or N
1
AN

IV
HAZARDOUS WASTE

C.
Onsite Tiered Permitting - Waste and Treatment Process Combinations

 INFORMATION DESCRIPTION - Permit by Rule (PBR) Waste and Treatment Process Combinations.  These are all of

 the eligible waste streams and treatment processes that are available within the tier.  NOTE: PBR codes are the

 same as CESQT.

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE

630-4a
Sludges, Dusts, Solids w/Metal(s) ‑ Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

630-4b
Sludges, Dusts, Solids w/Metal(s) ‑ Grind, Shred, Crush, or Compact
Y or N
1
AN

630-4c
Sludges, Dusts, Solids w/Metal(s) ‑ Drying to Remove Water
Y or N
1
AN

630-4d
Sludges, Dusts, Solids w/Metal(s) ‑ Separation by Size, Magnetism, or Density
Y or N
1
AN

630-5a
Sludges w/Alum, Gypsum, Lime, Sulfur, or Phosphate ‑ Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

630-5b
Sludges w/Alum, Gypsum, Lime, Sulfur, or Phosphate ‑ Drying to Remove Water
Y or N
1
AN

630-5c
Sludges w/Alum, Gypsum, Lime, Sulfur, or Phosphate - Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

630-6a
Special Waste (Sec. 66261.120) ‑ Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

630-6b
Special Waste (Sec. 66261.120) ‑ Drying to Remove Water
Y or N
1
AN

630-6c
Special Waste (Sec. 66261.120) ‑ Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

630-6d
Special Waste (Sec. 66261.120) ‑ Screening Based on Size
Y or N
1
AN

630-6e
Special Waste (Sec. 66261.120) ‑ Separation by Size, Magnetism, or Density
Y or N
1
AN

630-7a
Special Waste (Sec. 66261.124) ‑ Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

630-7b
Special Waste (Sec. 66261.124) ‑ Drying to Remove Water
Y or N
1
AN

630-7c
Special Waste (Sec. 66261.124) ‑ Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

630-7d
Special Waste (Sec. 66261.124) ‑ Magnetic Separation
Y or N
1
AN

630-8a
Inorganic Acid/Alkaline Waste ‑ pH Adjustment / Neutralization
Y or N
1
AN

630-9a
Soils w/Metal(s) - Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

630-9b
Soils w/Metal(s) - Separation by Size
Y or N
1
AN

630-9c
Soils w/Metal(s) ‑ Magnetic Separation
Y or N
1
AN

630-10a
Used Oil, Mixed Oil, Oily Water, Oil/W Sludges - Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

630-10b
Used Oil, Mixed Oil, Oily Water, O/W Sludges ‑ Distillation
Y or N
1
AN

630-10c
Used Oil, Mixed Oil, Oily Water, O/W Sludges ‑ Neutralization
Y or N
1
AN

IV
HAZARDOUS WASTE

C.
Onsite Tiered Permitting - Waste and Treatment Process Combinations

INFORMATION DESCRIPTION - Permit by Rule (PBR) Waste and Treatment Process Combinations.  These are all of the eligible waste streams and treatment processes that are available within the tier.  NOTE: PBR codes are the same as CESQT.

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE

630-10d
Used Oil, Mixed Oil, Oily Water, O/W Sludges ‑ Separation by Size, Magnetism, or Density
Y or N
1
AN

630-10e
Used Oil, Mixed Oil, Oily Water, O/W Sludges ‑ Reverse Osmosis
Y or N
1
AN

630-10f
Used Oil, Mixed Oil, Oily Water, O/W Sludges ‑ Biological Process Using Microorganisms
Y or N
1
AN

630-11a
Containers (< 110 Gallons) or Liners - Rinsing with Liquid
Y or N
1
AN

630-11b
Containers (< 110 Gallons) or Liners ‑ Crush, Shred, Grind, or Puncture
Y or N
1
AN

630-12a
Multi‑component Resins ‑ Mixing per Manufacturer's Instructions
Y or N
1
AN

630-13
Wastestream & Treatment Technology Combination Certified by DTSC per

HSC 25200.1.5
Valid Certified Technology Number
10
AN

IV.
HAZARDOUS WASTE

C.
Onsite Tiered Permitting - Waste and Treatment Process Combinations

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE
INFORMATION DESCRIPTION

606
Unit ID Number

18
AN
Unique identification number for unit. The units can be numbered sequentially or by any other system as long as the numbers are not repeated or duplicated.

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to permit cross linking of data. County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

INFORMATION DESCRIPTION - Conditionally Exempt - Limited (CEL) Waste and Treatment Process Combinations.  These are all of the eligible waste streams and treatment processes that are available within the tier.

ID
ELEMENT
EDIT CIRTERIA / CODE
LENGTH
TYPE

631-1
Aerosol Cans - Puncturing, Draining or Crushing (at Ambient Temperature) Using Wastestream & Treatment Technology Combination Certified by DTSC per

HSC 25200.1.5
Valid Certified Technology Number
10
AN

631-2a
Used Oil Mixed w/Water, Oily Water (no Gasoline and <2% Diesel) - Gravity Separation
Y or N
1
AN

631-2b
Used Oil Mixed w/Water, Oily Water (no Gasoline and <2% Diesel) - Centrifuge
Y or N
1
AN

631-2c
Used Oil Mixed w/Water, Oily Water (no Gasoline and <2% Diesel) - Membrane Technology
Y or N
1
AN

631-2d
Used Oil Mixed w/Water, Oily Water (no Gasoline and <2% Diesel) - Heating to Temperature not above 20 Deg F below Flashpoint of the Used Oil Component of the Mixture at Atmospheric Pressure.
Y or N
1
AN

631-2e
Used Oil Mixed w/Water, Oily Water (no Gasoline and <2% Diesel) - Addition of Demulsifiers
Y or N
1
AN

IV.
HAZARDOUS WASTE

2.  Certification Of Financial Assurance

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

2
EPA ID Number
12 digit identifier beginning with CA
12
AN
EPA Identification number for businesses that generate, recycle, or treat hazardous waste.  For facilities in California, the number usually starts with the letters CA.  The number can be obtained from the Telephone Information Center at (916)324-1781, (800) 61-TOXIC or (800) 618-6942.

3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of the business.

700
Certification Status
a = initial certification

b = amended certification

c = annual certification
1
AN
Reason for submitting certification.



701
Type of Operation (Hazardous Waste Financial Assurance)
a = PBR-FTU

b = CA

c = other
1
AN
Type of operation.

702
Estimated Closure Costs

12
AN
Estimated closure costs in dollars.  This section requires a written estimate of the cost of closing each treatment unit.  The estimated closure cost may be either the actual cost or the estimated cost when using your own staff and/or personal equipment.  The closure cost estimate may take into account any salvage value that may be realized from the sale of wastes, facility structure or equipment, land or other facility assets.  For PBR only: If you have operated under PBR for less than 30 days in any calendar year, do not enter any dollar amount.  You qualify for an exemption.  If eligible for the exemption, enter EXEMPT in this section.

703
Exemption From

Financial Assurance = cost estimate < $10,000
Y or N
1
AN
Indicates if reason for exemption from financial assurance requirements is total closure cost estimate not more than $10,000.  A model letter using the required certifications must be submitted to claim this exemption.

704
Exemption From

Financial Assurance = other
Narrative
30
AN
Description of other reason for exemption from financial assurance requirements.

705
Exemption From

Financial Assurance = TTU owner / operator

and operated < 30 days/yr
Y or N
1
AN
Indicates if reason for exemption from financial assurance requirements is for owner or operator under PBR only and operating no more than thirty days in any calendar year.

706
Requirement for

Financial Assurance
Y or N
1
AN
Indicates if financial assurance is required.

707
Date of Closure Assurance Mechanism
YYYYMMDD
8
D
Effective date of closure assurance mechanism.

708
Mechanism ID Number

25
AN
Number of closure assurance mechanism if applicable, for example, the insurance policy number.

709
Closure Assurance Mechanism
a = closure trust fund

b = surety bond

c = closure letter of credit

d = closure insurance

e = financial test and

      corporate guarantee

f  = alternative mechanism 

g = multiple financial

      mechanisms

h = certificate of deposit

I  = savings account
1
AN
Type of financial mechanism established to provide closure cost assurance.  Eligible types for FTUs and TTUs are contained in CCR Section 67450.13(a)(5).

a = closure trust fund, as provided in CCR Section

      66265.143(a)  (Attach DTSC Form 1154)

b = surety bond guaranteeing payment into a

      closure trust fund, as described in CCR Section

      66265.143(b) (Attach DTSC Form 1155 or 1156

      with DTSC Form 1154)

c = closure letter of credit, as described in CCR

      Section 66265.143(c)  (Attach DTSC Form

      1157)

d = closure insurance, as described in CCR Section

      66265.143(d) (Attach DTSC Form 1158)

e = financial test and corporate guarantee for

      closure, as described in CCR Section

      66265.143(e) (Attach either DTSC Form 1159 or

      1173)

f  = alternative mechanism for closure costs, as

      described in CCR Section 67450.13(c) (No

      form)

g = multiple financial mechanisms for closure costs,

      as described in CCR Section 66265.143(g) (No

      form)

h = certificate of deposit, as described in Section

      3-104(2)(c) of the Uniform Commercial Code

      (No form)

i  = savings account, as described in Section

      4-104(a) of the Uniform Commercial Code  (No

      form)

These mechanisms require use of the additional DTSC Financial Assurance forms referenced above. These forms are available from the CUPA or the DTSC Regional Office.

710
Financial Institution

or Surety Name

35
AN
Name of the financial institution, insurance company, surety company, or other appropriate organization used to establish the closure financial assurance.  Indicate your company if you are using a corporate guarantee and financial test.

711
Financial Institution

or Surety Address
Postal standard:

2 lines, 35 characters
70
AN
Address of financial institution, insurance company, surety company, or other appropriate organization used to establish the closure financial assurance.

712
Financial Institution

or Surety City
Postal standard
20
AN
City of financial institution, insurance company, surety company, or other appropriate organization used to establish the closure financial assurance.

713
Financial Institution

or Surety State
Valid 2-digit state code
2
AN
State of financial institution, insurance company, surety company, or other appropriate organization used to establish the closure financial assurance.

714
Financial Institution

or Surety Zip Code
Postal standard
9
AN
Zip code of financial institution, insurance company, surety company, or other appropriate organization used to establish the closure financial assurance.

715
Signer of Certification
a = owner

b = operator
1
AN
Indicates if signer of certification is the owner or operator of the facility.

716
Date Certified

(Financial Assurance)
YYYYMMDD
8
D
Date the document was signed.

717
Owner/ Operator Name (Financial Assurance)

35
AN
Full name of business owner/operator, or officially designated representative of the owner/operator. The person signing this page must be an owner or officer of the company who is authorized to make decisions for the facility and who has operational control.  The authorized signatory must be completed as specified in CCR Section 66270.11. In most companies, this is not the environmental compliance or technical staff.  The signer certifies to a belief that all the information submitted is accurate and complete.

718
Owner/ Operator Title (Financial Assurance)

35
AN
Title of person signing the page.

FACILITY INFORMATION

4.  Business Activities

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

2
EPA ID Number
12 digit identifier beginning with CA
12
AN
EPA Identification number for businesses that generate, recycle, or treat hazardous waste.  For facilities in California, the number usually starts with the letters CA.  The number can be obtained from the Telephone Information Center at

(916) 324-1781, (800) 61-TOXIC or (800) 618-6942.

3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of business.

4
Hazardous Materials

On Site
Y or N
1
AN
Business must report that it has hazardous materials on site if:

  - it is handled in quantities equal to or greater than 500 pounds, 55 gallons, or 200 cubic feet of gas (calculated at standard temperature and pressure),

  - it is handled in quantities equal to or greater than the applicable federal threshold planning quantity for an extremely hazardous substance listed in 40 CFR Part 355, Appendix A,

  - radioactive materials are handled in quantities for which an emergency plan is required to be adopted pursuant to Part 30, Part 40, or Part 70 of Chapter 10 of 10 CFR, or pursuant to any regulations adopted by the state in accordance with those regulations.

Triggers requirement for chemical description data elements.

5
Own or Operate Underground

Storage Tank
Y or N
1
AN
Facility must report if it owns or operates USTs containing hazardous substances defined in HSC 25316.  Triggers requirement for UST facility and tank data elements.

6
Upgrade/Install Underground

Storage Tank
Y or N
1
AN
Facility must report if it intends to install or upgrade USTs containing hazardous substances defined in HSC 25316.  Triggers requirement for UST installation data elements.

7
Underground

Storage Tank

Closure
Y or N
1
AN
Facility must report if a UST which held hazardous materials is being closed in place.  Triggers requirement for UST closure data elements.

8
Own or Operate Aboveground

Petroleum

Storage Tank
Y or N
1
AN
Facility must report if it stores petroleum in aboveground tanks.  "Petroleum" means crude oil or any fraction thereof, which is liquid at 60 degrees Fahrenheit temperature and 14.7 pounds per square inch absolute pressure (HSC 25270.2(g)).  The facility must report if any ASTs capacity exceeds 660 gallons or if the total facility storage capacity (aggregate) exceeds 1320 gal.  "Storage tank" does not include any of the  following:

  - a pressure vessel or boiler which is subject to Division 5 of the Labor Code,

  - a storage tank containing hazardous waste if a hazardous waste facilities permit has been issued for the storage tank by DTSC,

  - an aboveground oil production tank which is regulated by the Division of Oil and Gas, or

  - certain oil-filled electrical equipment including but not limited to transformers, circuit breakers, or capacitors.

9
Hazardous Waste Generator
Y or N
1
AN
Facility must report if it generates hazardous waste.  "Hazardous waste" means a waste that meets any of the criteria for the identification of a hazardous waste adopted by the department pursuant to HSC 25141. "Hazardous waste" includes, but is not limited to, RCRA hazardous waste.  Unless expressly provided otherwise, the term "hazardous waste" shall be understood to also include extremely hazardous waste and acutely hazardous waste.  Triggers requirement to obtain and provide EPA Identification number.

IV.
HAZARDOUS WASTE

5.  Remote Waste Consolidation Site Annual Notification

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

2
EPA ID Number
12 digit identifier beginning with CA
12
AN
EPA Identification number for businesses that generate, recycle, or treat hazardous waste.  For facilities in California, the number usually starts with the letters CA.  The number can be obtained from the Telephone Information Center at

(916) 324-1781, (800) 61-TOXIC or (800) 618-6942.

3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of business.

720
Notification Status

(Remote Consolidation)
a = initial

b = revised

c = annual
1
AN
Reason for submitting notification.

721
Address

(Remote Consolidation)
Postal standard:

2 lines, 35 characters
70
AN
Street address or legal description of consolidation site.

722
City (Remote Consolidation)
Postal standard
20
AN
City of consolidation site.

723
Zip Code

(Remote Consolidation)
Postal standard
9
AN
Zip Code of consolidation site.

724
Description Of Remote Consolidation Location(s)
Narrative
150
AN
Description of type of remote location(s) and sources(s) from which non-RCRA hazardous waste will be collected.

725
Description Of

Waste(s) Collected
Narrative
150
AN
Description of type of hazardous waste that may be collected.

726
Onsite Hazardous

Waste Treatment
Y or N
1
AN
Indicates if hazardous waste is treated at consolidation site.

727
Estimated Monthly

Volume Consolidated

10
N
Estimated monthly total volume consolidated.

728
Units

(Remote Consolidation)
a = pounds

b = gallons
1
AN
Units for estimated monthly total volume consolidated.

729a
Basis For Not Needing

A Federal Permit

(Remote Consolidation) =

non-RCRA hazardous waste
Y or N
1
AN
Indicates if activity is exempt from, or not otherwise regulated by, the federal Resource Conservation and Recovery Act (RCRA).

729b
Basis For Not Needing

A Federal Permit

(Remote Consolidation) = 

other
Narrative
60
AN
Description of other reason(s) activity is not subject to permitting requirements under federal law.

730
Date Certified

(Remote Consolidation)
YYYYMMDD
8
D
Date the document was signed.

731
Owner/Operator Name (Remote Consolidation)

35
AN
Full name of business owner/operator, or officially designated representative of the owner/operator. The person signing this page must be an owner or officer of the company who is authorized to make decisions for the facility and who has operational control.  In most companies, this is not the environmental compliance or technical staff.  The signer certifies to a belief that all the information submitted is accurate and complete.

732
Owner/Operator Title

(Remote Consolidation)

35
AN
Title of person signing the page.

IV.
HAZARDOUS WASTE

F.
Hazardous Waste Tank Closure Certification

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

 1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

3
Business Name

70
AN
Full legal name of the business.

740
Tank Owner Name (Closure)

35
AN
Name of tank owner, if different from business owner on Business Owner/Operator page.

741
Tank Owner Address (Closure)
Postal standard: 

2 lines, 35 characters
70
AN
Street or mailing address of tank owner, if different from business owner.

742
Tank Owner City (Closure)
Postal standard
20
AN
City of tank owner, if different from business owner.

743
Tank Owner State (Closure)
Postal standard
2
AN
State of tank owner, if different from business owner.

744
Tank Owner Zip Code (Closure)
Postal standard
9
AN
Zip code of tank owner, if different from business owner.

745
Tank ID # 1

(Closure)

6
AN
Owner's tank ID # for first tank closed, if there is a tank number used by owner to identify the tank. Unique identifier of tank at site.

746a
Concentration of Flammable Vapor 1 = top
4 digits with floating decimal
5
AN
Concentration of flammable vapor for first tank closed taken at top of tank.

746b
Concentration of Flammable Vapor 1 = center
4 digits with floating decimal
5
AN
Concentration of flammable vapor for first tank closed taken at center of tank.

746c
Concentration of Flammable Vapor 1 = bottom
4 digits with floating decimal
5
AN
Concentration of flammable vapor for first tank closed taken at bottom of tank.

747a
Concentration of

Oxygen 1 = top
4 digits with floating decimal
5
AN
Concentration of oxygen for first tank closed taken at top of tank.

747b
Concentration of

Oxygen 1 = center
4 digits with floating decimal
5
AN
Concentration of oxygen for first tank closed taken at center of tank.

747c
Concentration of

Oxygen 1 = bottom
4 digits with floating decimal
5
AN
Concentration of oxygen for first tank closed taken at bottom of tank.

748
Tank ID # 2

(Closure)

6
AN
Owner's tank ID # for second tank closed, if there is a tank number used by owner to identify the tank.  Unique identifier of tank at site.

749a
Concentration of Flammable Vapor 2 = top
4 digits with floating decimal
5
AN
Concentration of flammable vapor for second tank closed taken at top of tank.

749b
Concentration of Flammable Vapor 2 = center
4 digits with floating decimal
5
AN
Concentration of flammable vapor for second tank closed taken at center of tank.

IV.
HAZARDOUS WASTE

F.
Hazardous Waste Tank Closure Certification

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

749c
Concentration of Flammable Vapor 2 = bottom
4 digits with floating decimal
5
AN
Concentration of flammable vapor for second tank closed taken at bottom of tank.

750a
Concentration of

Oxygen 2 = top
4 digits with floating decimal
5
AN
Concentration of oxygen for second tank closed taken at top of tank.

750b
Concentration of

Oxygen 2 = center
4 digits with floating decimal
5
AN
Concentration of oxygen for second tank closed taken at center of tank.

750c
Concentration of

Oxygen 2 = bottom
4 digits with floating decimal
5
AN
Concentration of oxygen for second tank closed taken at bottom of tank.

751
Tank ID # 3

(Closure)

6
AN
Owner's tank ID # for third tank closed, if there is a tank number used by owner to identify the tank. Unique identifier of tank at site.

752a
Concentration of Flammable Vapor 3 = top
4 digits with floating decimal
5
AN
Concentration of flammable vapor for third tank closed taken at top of tank.

752b
Concentration of Flammable Vapor 3 = center
4 digits with floating decimal
5
AN
Concentration of flammable vapor for third tank closed taken at center of tank.

752c
Concentration of Flammable Vapor 3 = bottom
4 digits with floating decimal
5
AN
Concentration of flammable vapor for third tank closed taken at bottom of tank.

753a
Concentration of

Oxygen 3 = top
4 digits with floating decimal
5
AN
Concentration of oxygen for third tank closed taken at top of tank.

753b
Concentration of

Oxygen 3 = center
4 digits with floating decimal
5
AN
Concentration of oxygen for third tank closed taken at center of tank.

753c
Concentration of

Oxygen 3 = bottom
4 digits with floating decimal
5
AN
Concentration of oxygen for third tank closed taken at bottom of tank.

If more than three tanks are closed, complete additional copies of the Hazardous Waste Tank Closure Certification page.

754
Certifier Name

(Tank Closure)

35
AN
Full name of person signing the page.

755
Certifier Title 

(Tank Closure)

35
AN
Title of person signing the page.

756
Certifier Address

(Tank Closure)
Postal standard:

2 lines, 35 characters
70
AN
Address of person signing the page.

757
Certifier City

(Tank Closure)
Postal standard
20
AN
City of person signing the page.

758
Certifier Phone

(Tank Closure)
Area code + 7 digit phone number + extension
15
AN
Phone number of person signing the page.

759
Date Certified

(Tank Closure)
YYYMMDD
8
D
Date the document was signed.

IV.
HAZARDOUS WASTE

F.
Hazardous Waste Tank Closure Certification

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

760
Certifier Represents

Local Agency
Y or N
1
AN
Indicates if person certifying the tank is a representative of the CUPA, authorized agency, or LIA.

761
Name of Local Agency
Narrative
35
AN
Name of CUPA, authorized agency, or LIA.

762a
Affiliation of Certifying Person = Certified Industrial Hygienist
Y or N
1
AN
Certification, license or organization which the certifier holds or to which the certifier belongs, if the person does not represent the CUPA.

762b
Affiliation of Certifying Person = Certified

Safety Professional
Y or N
1
AN
Certification, license or organization which the certifier holds or to which the certifier belongs, if the person does not represent the CUPA.

762c
Affiliation of Certifying Person = Certified 

Marine Chemist
Y or N
1
AN
Certification, license or organization which the certifier holds or to which the certifier belongs, if the person does not represent the CUPA.

762d
Affiliation of Certifying Person = Registered Environmental Health Specialist
Y or N
1
AN
Certification, license or organization which the certifier holds or to which the certifier belongs, if the person does not represent the CUPA.

762e
Affiliation of Certifying Person = Professional Engineer
Y or N
1
AN
Certification, license or organization which the certifier holds or to which the certifier belongs, if the person does not represent the CUPA.

762f
Affiliation of Certifying Person = Class II Registered Environmental Assessor
Y or N
1
AN
Certification, license or organization which the certifier holds or to which the certifier belongs, if the person does not represent the CUPA.

762g
Affiliation of Certifying Person = Licensed Contractor
Y or N
1
AN
Certification, license or organization which the certifier holds or to which the certifier belongs, if the person does not represent the CUPA.

763
Tank Held Flammable or Combustible Materials
Y or N
1
AN
Indicates if tank previously held flammable or combustible materials.  If yes, the tank interior atmosphere shall be re-checked with a combustible gas indicator prior to work being conducted on the tank.

764
Management Instructions
Narrative
150
AN
Certifier's tank management instructions for scrap dealer, disposal facility, etc.

Appendix D UP Information Collection and Reporting Standards Unified Program Data Dictionary--CUPA Section

1 COMPLIANCE ACTIVITY INFORMATION

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

2
EPA ID Number
12 digit identifier beginning with CA
12
AN
EPA identification number for businesses that generate, recycle, or treat hazardous waste.  For facilities in California, the number should start with the letters CA.  If the handler is regulated under Federal RCRA requirements, this ID must be the U.S. EPA identification number.

3
Business Name
Postal standard:

2 lines, 35 character
70
AN
Full legal name of business.

900
RCRA Large Quantity Generator (LQG) of Hazardous Waste
Y or N
1
AN
Indicates if facility generates  1000 kg of RCRA hazardous waste in a calendar month.  Identification is based on the business notification of LQG activity to U.S. EPA.  If the designation is incorrect, the CUPA cannot change the designation unless the business notifies U.S. EPA.

901
Generator of Solely California Hazardous Waste
Y or N
1
AN
Indicates if facility generates solely California hazardous waste and does not generate any RCRA waste.

902
CalARP Program: Stationary Source
Y or N
1
AN
Indicates if facility is a stationary source as defined by the CalARP program.

903
CalARP Program: Multiple Stationary Sources
Y or N
1
AN
Indicates if business operates multiple locations in this CUPA jurisdiction that are stationary sources as defined by the CalARP program.

904
CalARP Program: RMP Waiver Determination
Y or N
1
AN
Indicates if the CUPA has waived the requirement for a Risk Management Plan for

this stationary source (a RMP waiver).

2.
INSPECTION INFORMATION (one record for each facility for each program element and inspection date)

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of business.

905
Program Element
a = Hazardous Materials Release     Response Plans (HMRRP)

b = California Accidental Release Prevention (CalARP)

c = Undergound Storage Tank  (UST)

d = Spill Prevention Control and           Countermeasures (SPCC) /            Aboveground Storage Tank

e = Hazardous Waste Generator

f  = Hazardous Waste Large Quantity Generator (LQG)               (subset of Hazardous Waste   Generator)

g = Hazardous Waste Recycler

h = Permit by Rule (PBR)

i  = Conditionally Authorized (CA)         (only available if PBR is not used)

j  = Conditionally Exempt (CE)              (only available if PBR and CA         are not used)

k = Household Hazardous Waste          (HHW)
1
AN
Program element inspected.  Enter one program element only.  For example, a facility inspected for compliance with UST, hazardous waste and hazardous materials storage regulations would have three records, one for each program element.  Each record would have a unique set of violations, even though all inspections took place on the same date.  For Tiered Permitting options enter the highest tier.

906
Inspection Date
YYYYMMDD
8
D
Date of completion of inspection.

907a
Inspection Type = Routine Inspection


Y or N


1
AN
Indicates if inspection is routine.  A routine inspection is a regularly scheduled inspection to evaluate compliance.   Does not include follow-up inspections.

907b
Inspection Type = Other
Y or N
1
AN
Indicates if inspection is not a routine inspection.  Other inspections include complaint investigations, closure, release investigations, tank installation and/or removal oversight, tank cleaning, and follow-up enforcement inspections, or other inspections that may be in addition to a regularly scheduled inspection.  This includes verification inspections for owners/operators of aboveground storage tanks having to prepare a spill prevention control and countermeasure plan.  It does not include regularly scheduled inspections, field or site visits whose principle purpose is informational or educational, pollution prevention education, or visits needed to verify administrative information or orient new owners or operators.  A complaint inspection is a service request originating from any outside party, including the public, that initiates a site visit outside of the routine inspection cycle.

908
CalARP Audit


Y or N
1
AN
Indicates if site visit is an audit of a CalARP Risk Management Plan for a stationary source.

909a
Inspection Category = Single Program
Y or N
1
AN
Indicates if inspection is a single program inspection.  Inspectors perform single program inspections alone.  If inspection is a single program inspection, do not enter codes for any other inspection category.

909b
Inspection Category = Combined Routine Inspection
Y or N
1
AN
Indicates if inspection is a combined routine inspection.  Combined routine inspections are regularly scheduled inspections to evaluate compliance conducted by one inspector for more than one program element.  This does not include other inspections performed outside the routine inspection cycle.

909c
Inspection Category = Joint Inspection
Y or N
1
AN
Indicates if inspection is a joint inspection.  Joint inspections may be routine or other inspections.  Joint inspections are conducted by more than one inspector from different Unified Program agencies within a CUPA, for more than one program element.

909d
Inspection Category = Integrated or Multi-media Inspections
Y or N
1
AN
Indicates if inspection is integrated or multi-media.  Integrated or multi-media inspections may be routine or other inspections.  Integrated inspections are conducted by one or more inspectors for the Unified Program and other programs not in the Unified Program.  Multi-media inspections are conducted by one or more inspectors for more than one medium, such as air, water, or soil.

910
Number of Class I Violations

2
N
For hazardous waste generators, number of Class I violations.  A Class I violation means a deviation that represents a significant threat to human health or safety or the environment because of the volume of the waste, the relative hazardousness of the waste, or the proximity of the population at risk.  The deviation must be significant enough that it could result in releases of hazardous waste or constituents to the environment, hazardous waste failing to be delivered to an authorized hazardous waste facility, failure to detect releases of hazardous waste or constituents, inadequate financial resources in the case of releases of hazardous waste or constituents, or inadequate financial resources to pay for facility closure, perform emergency cleanup operations or other corrective actions.  A Class I violation is also a deviation that is a chronic violation or committed by a recalcitrant violator.  A Class I violation is typically one that is referred to the District Attorney or City Attorney for formal enforcement action.  Sanctions are typically imposed for failure to correct the violation.  Class I violations are defined in the Health and Safety Code (HSC) section 25110.8.5.

911
Number of Class II Violations

2
N
For hazarous waste generators, number of Class II violations.  A Class II violation means a deviation that is not a Class I violation.  This count includes violations which would be considered minor, but are knowing, willful, or intentional, or enable the violator to benefit economically from noncompliance, either by reduced costs or competitive advantage.  Do not include minor violations in this count.  Class II violations are defined in 22 California Code of Regulations (CCR) 66260.10.

912
Number of Minor Violations

2
N
For hazardous waste generators, number of minor violations.  A minor violation means a deviation from any regulation, standard, requirement, or permit condition, that is not a Class I violation.  Exclude from this count all violations where the violation is knowing, willful, or intentional, or enables the violator to benefit economically from noncompliance, either by reduced costs or competitive advantage.  These are counted as Class II violations.  Also exclude any violation that is a chronic violation or that is committed by a reclacitrant violator, since these are counted as Class I violations.  A minor violation is defined in HSC 25117.6.

913
Number of Other Violations

2
N
For non-hazardous waste program elements, number of other violations.  Other violations are those that are not hazardous waste violations.

3.
ENFORCEMENT INFORMATION (one record for each facility for each program element and enforcement action)

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of business.

902
Program Element
a = Hazardous Materials Release             Response Plans (HMRRP)

b = California Accidental Release Prevention (CalARP)

c = Undergound Storage Tank                  (UST)

d = Spill Prevention Control and                Countermeasures (SPCC) /                Aboveground Storage Tank

e = Hazardous Waste Generator

f  = Hazardous Waste Large                     Quantity Generator (LQG)                   (subset of Hazardous Waste               Generator)

g = Hazardous Waste Recycler

h = Permit by Rule (PBR)

I  = Conditionally Authorized  (CA)  (only available if PBR is not used)

j  = Conditionally Exempt (CE)                  (only available if PBR and CA             are not used)

k = Household Hazardous Waste              (HHW) - Fixed
2
AN
Program element inspected.  Enter one program element only.  For example, a facility inspected for compliance with UST, hazardous waste and hazardous materials storage regulations would have three records, one for each program element.  Each record would have a unique set of violations, even though all inspections took place on the same date.  For Tiered Permitting options enter the highest tier.

903
Inspection Date
YYYYMMDD
8
D
Date of completion of inspection.

914
Type of Enforcement Action
a = Informal action

b = Referral to State agency

c = Formal order

d = Referral to Attorney General or           District Attorney
1
AN
Type of enforcement action.  Informal actions are actions that are not formal actions.  An informal enforcement action notifies the business of non-compliance and establishes a date by which the non-compliance is to be corrected.  Informal actions are made by a written document including, but not limited to, a letter or notice of violation.  Informal actions do not convey sanctions.   A formal order is an enforceable order or agreement which mandates compliance.  Examples include administrative orders and referrals for civil and/or criminal actions.  Sanctions are imposed for failure to comply.  If more than one enforcement action is taken, the type and date of each action should be recorded.

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

915
Date of Enforcement Action
YYYYMMDD
8
D
Date the enforcement action is taken.  The date of enforcement action is the date the order is sent to the business, a final referral is made to the Attorney General, District Attorney or County Attorney, the complaint is filed with the court, or the date the administrative order is issued.  If more than one enforcement action is taken, the type and date of each action should be recorded.

916
Type of Formal Enforcement Action
a = Administrative

b = Civil

c = Criminal
1
AN
Type of formal enforcement action. 

917
Date Returned to Compliance
YYYYMMDD
8
D


Date physical compliance was determined by the CUPA for all violations identified during the inspection.  This may not be based on a site visit, but is the date compliance was verified.  It may be based on correspondence received from the regulated business.

918
Docket Number
.
13
AN
Number assigned by the court for civil and criminal actions.

919
Final Fine or Penalty Assessed

8
AN
Dollar amount of fine or penalty assessed.  This is the final monetary penalty or fine assessed via court or administrative order, or the amount agreed upon in a formal legal settlement.  It is based on the value of fines / penalties excluding costs.  Round to nearest whole number.  Do not use decimal places.  Note the fine or penalty is by program element for each enforcement action at each facility, when available.

920
Fine or Penalty Collected

8
AN
Dollar amount of final fine or penalty actually collected by the CUPA.  Round to nearest whole number.  Do not use decimal places.

921
Date Fine or Penalty Collected
YYYYMMDD
8
D
Date when the final fine or penalty was completely collected.

4.
TIERED PERMITTING RELEASE INFORMATION

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of business.

922
Date of Release
YYYYMMDD
8
 D 
Date of release at a facility treating hazardous waste onsite under PBR, CA, or CE.  A reportable release is from any activity at the facility, not just from one of the Tiered Permitting units.

923
Description of Release and Consequences
Narrative
200
AN
Description of the circumstances of the release and actions subsequently taken.

924
Date Corrective Action Completed
YYYYMMDD
8
D
Date corrective action was completed.  If no corrective action was needed, then leave blank.

Appendix E Unified Program Consolidated Forms

http://www.calepa.ca.gov/CUPA/Pulications/

Appendix F [Reserved
Add Title 27, Division 3, Subdivision 1, Chapters 1, 2, 3, 4, 5 and 6 of the California Code of Regulations:

Chapter 1

Facility Information 

ACRONYMS

AA - Authorizing Agency

AN - Alpha-numeric

AST - Aboveground Storage Tank

ATG -  Automatic Tank Gauging

BOE - Board of Equalization

CA - Conditionally Authorized

Cal/EPA - California Environmental Protection Agency

Cal/ARP - California Accidental Release Prevention

CAS - Chemical Abstract Number

CCR - California Code of Regulations

CD - Certificate of Deposit

CE - Conditionally Exempt

CE-CL - Conditionally Exempt - Commercial Laundry

CEL - Conditionally Exempt – Limited

CESQT - Conditionally Exempt Small Quantity Treatment

CESW - Conditionally Exempt - Specified Wastestreams

CFO - Chief Financial Officer

CFR - Code of Federal Regulations

CUPA - Certified Unified Program Agency

DBA - Doing Business As

DOT - United States Department of Transportation

DTSC - Department of Toxic Substances Control

EHS - Extremely Hazardous Substance

EPA - United States Environmental Protection Agency

EPCRA - Emergency Planning and Community Right to Know Act

FRP - Fiberglass Reinforced Plastic

FTU - Fixed Treatment Unit

HDPE - High Density Polyethylene

HHW - Household Hazardous Waste  

HMRRP - Hazardous Materials Release Response Plans  

HSC - Health and Safety Code  

HW - Hazardous Waste  

ID - Identification  

IUPAC - International Union of Pure and Applied Chemistry  

LIA - Local Implementing Agency  

LQG - Large Quantity Generator  

MSDS - Material Safety Data Sheet  

MTG - Manual Tank Gauging  

NPDES - National Pollutant Discharge Elimination System  

OES - Office of Emergency Services  

PA - Participating Agency  

PBR - Permit by Rule  

POTW - Publicly Owned Treatment Works  

Q/A - Quality Assurance  

Q/C - Quality Control  

RCRA - Resource Conservation and Recovery Act  

RMP - Risk Management Plan  

SIC - Standard Industrial Classification  

SIR - Statistical Inventory Reconciliation  

SPCC - Spill Prevention Control and Countermeasures  

SWRCB - State Water Resources Control Board  

TTU - Transportable Treatment Unit  

U.S. EPA - United States Environmental Protection Agency  

UP - Unified Program  

UPCF - Unified Program Consolidated Form  

UST - Underground Storage Facility  

Chapter 1  FACILITY INFORMATION

1.  Business Activities

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

2
EPA ID Number
12 digit identifier beginning with CA
12
AN
EPA Identification number for businesses that generate, recycle, or treat hazardous waste.  For facilities in California, the number usually starts with the letters CA.  The number can be obtained from the Telephone Information Center at

(916) 324-1781, (800) 61-TOXIC or (800) 618-6942.

3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of business.

4
Hazardous Materials

On Site
Y or N
1
AN
Business must report that it has hazardous materials on site if:

  - it is handled in quantities equal to or greater than 500 pounds, 55 gallons, or 200 cubic feet of gas (calculated at standard temperature and pressure),

  - it is handled in quantities equal to or greater than the applicable federal threshold planning quantity for an extremely hazardous substance listed in 40 CFR Part 355, Appendix A,

  - radioactive materials are handled in quantities for which an emergency plan is required to be adopted pursuant to Part 30, Part 40, or Part 70 of Chapter 1 of 10 CFR, or pursuant to Section 25503.5(a) of the Health and Safety Code.
Triggers requirement for chemical description data elements.

5
Own or Operate Underground

Storage Tank
Y or N
1
AN
Facility must report if it owns or operates USTs containing hazardous substances defined in HSC 25316.  Triggers requirement for UST facility and tank data elements.

6
Upgrade/Install Underground

Storage Tank
Y or N
1
AN
Facility must report if it intends to install or upgrade USTs containing hazardous substances defined in HSC 25316.  Triggers requirement for UST installation data elements.

7
Underground

Storage Tank

Closure
Y or N
1
AN
Facility must report if a UST which held hazardous materials is being closed in place.  Triggers requirement for UST closure data elements.

8
Own or Operate Aboveground

Petroleum

Storage Tank
Y or N
1
AN
Facility must report if it stores petroleum in aboveground tanks.  "Petroleum" means crude oil or any fraction thereof, which is liquid at 60 degrees Fahrenheit temperature and 14.7 pounds per square inch absolute pressure (HSC 25270.2(g)).  The facility must report if any ASTs capacity exceeds 660 gallons or if the total facility storage capacity (aggregate) exceeds 1320 gal.  "Storage tank" does not include any of the  following:

  - a pressure vessel or boiler which is subject to Division 5 of the Labor Code,

  - a storage tank containing hazardous waste if a hazardous waste facilities permit has been issued for the storage tank by DTSC,

  - an aboveground oil production tank which is regulated by the Division of Oil and Gas, or

  - certain oil-filled electrical equipment including but not limited to transformers, circuit breakers, or capacitors.

9
Hazardous Waste Generator
Y or N
1
AN
Facility must report if it generates hazardous waste.  "Hazardous waste" means a waste that meets any of the criteria for the identification of a hazardous waste adopted by the department pursuant to HSC 25141. "Hazardous waste" includes, but is not limited to, RCRA hazardous waste.  Unless expressly provided otherwise, the term "hazardous waste" shall be understood to also include extremely hazardous waste and acutely hazardous waste.  Triggers requirement to obtain and provide EPA Identification number.

10
Recycle
Y or N
1
AN
Facility must report if it recycles more than 100 kilograms per month of recyclable material under a claim that the material qualifies for exclusion or exemption pursuant to HSC.25143.2.  This includes onsite and offsite facilities that recycle under this law.  Triggers requirement for Recyclable Materials data elements.  Persons that send recyclable material offsite to be recycled and that do not recycle onsite are not included in this category.

11
Onsite Hazardous

Waste Treatment
Y or N
1
AN
Facility must report if it treats hazardous waste under an onsite tier.  "Treatment" means any method, technique, or process which is designed to change the physical, chemical, or biological character or composition of any hazardous waste or any material contained therein, or removes or reduces its harmful properties or characteristics for any purpose.  "Treatment" does not include the removal of residues from manufacturing process equipment for the purposes of cleaning that equipment.  Amendments (effective 1/1/99) add exemptions from the definition of "treatment" for certain processes under specific, limited conditions.  Refer to HSC 25123.5(b) for these specific exemptions. 

Treatment of certain laboratory hazardous wastes do not require treatment.  Refer to HSC25200.3.1 for specific information.

Contact CUPA to determine if any exemptions or exclusions apply.  Triggers requirement for onsite hazardous waste treatment data elements.

12
Financial Assurance
Y or N
1
AN
Facilities that treat hazardous waste under PBR or CA tiers are required to provide financial assurance for closure costs (per 22 CCR 67450.13(b), HSC 25245.4), unless they are exempt.  Triggers requirement for financial assurance data elements.

13
Remote Waste Consolidation Site
Y or N
1
AN
Facilities must report if they collect hazardous waste initially at remote sites and subsequently transport the hazardous waste to a consolidation site they operate pursuant to HSC 25110.10.  Triggers requirement for remote hazardous waste consolidation data elements.

14
Hazardous Waste

Tank Closure
Y or N
1
AN
Facilities must report if the tank being closed would be classified as hazardous waste, after its contents are removed.  Classification could be based on:

- the facility's knowledge of the tank and its contents,

  - testing of the tank,

  - inability to remove hazardous materials stored in the tank,

  - the mixture rule, or

  - the listed wastes in 40 CFR 261.31, 40 CFR 261.32. 

Triggers requirement for hazardous waste data elements.

15
Local Requirements



For local use only.  This space may be used by the CUPA to collect any additional information necessary to meet the requirements of their individual programs.  Contact CUPA for guidance.

Chapter 1  FACILITY INFORMATION

Business Owner / Operator Identification (OES Form 2730)

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

 1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

 3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of business.

100
Beginning Date
YYYYMMDD
8
D
Beginning year and date of report.

101
Ending Date
YYYYMMDD
8
D
Ending year and date of report.

102
Business Phone 
Area code + 7 digit phone number + extension
15
AN
Phone number of this site.

103
Business Site Address
Postal standard:

2 lines, 35 characters
70
AN
Street address where facility is located.  No post office box numbers are allowed.  This information must provide a means to geographically locate the facility.

104
City (Business)
Postal standard
20
AN
City or unincorporated area in which business site is located.

105
Zip Code (Business)
Postal standard
9
AN
Zip code of business site.

106
Dun & Bradstreet
D-U-N-S (data universal numbering system) 9 digit number
9
AN
Dun & Bradstreet D-U-N-S number for facility.  The Dun & Bradstreet number may be obtained by calling (610) 882‑7748 or by Internet.

107
SIC Code
Standard Industrial Classification (SIC) Code 4 digit number
4
AN
Standard Industrial Classification (SIC) Code number for primary business activity.  If code is more than 4 digits, report only the first four.

108
County

20
AN
County in which business site is located.

109
Business Operator Name

35
AN
Name of business operator.

110
Business Operator Phone
Area code + 7 digit phone number + extension
15
AN
Phone number of business operator, if different from business phone.

111
Business Owner Name

35
AN
Name of business owner, if different from business operator. 

112
Business Owner Phone
Area code + 7 digit phone number + extension
15
AN
Phone number of business owner, if different from business phone.

113
Business Owner

Mailing Address
Postal standard:

2 lines, 35 characters
70
AN
Mailing address of owner, if different from business site address.

114
Business Owner City
Postal standard
20
AN
City for owner's mailing address.

115
Business Owner State
Postal standard
2
AN
State for owner's mailing address.

116
Business Owner Zip Code
Postal standard
9
AN
Zip code for owner's mailing address.

117
Environmental

Contact Name

35
AN
Name of person, if different from the business owner/operator, who receives all environmental correspondence and will respond to enforcement activity.

118
Environmental

Contact Phone 
Area code + 7 digit phone number + extension
15
AN
Phone number of environmental contact, if different from business owner or operator.

119
Environmental Contact Mailing Address
Postal standard:

2 lines, 35 characters
70
AN
Mailing address for all environmental contact correspondence, if different from the site address.

120
Environmental

Contact City
Postal standard
20
AN
City for environmental contact's mailing address.

121
Environmental

Contact State
Postal standard
2
AN
State for environmental contact's mailing address.

122
Environmental

Contact Zip Code
Postal standard
9
AN
Zip code for environmental contact's mailing address.

123
Primary Emergency Contact Name

35
AN
Name of a representative that can be contacted in case of an emergency involving hazardous materials at the business site.  The contact shall have FULL facility access, site familiarity, and authority to make decisions for the business regarding incident mitigation.

124
Primary Emergency Contact Title

35
AN
Title of primary emergency contact.

125
Primary Emergency Contact Business

Phone Number
Area code + 7 digit phone number + extension
15
AN
Business phone number of primary emergency contact.

126
Primary Emergency Contact 24-Hour Phone
Area code + 7 digit phone number + extension
15
AN
Phone number for primary emergency contact which is answered 24 hours a day and, if not the contact's home phone number, then the service answering the phone must be able to immediately contact the above stated individual.

127
Primary Emergency Contact Pager Number
Area code + 7 digit phone number + extension
15
AN
Pager phone number for primary emergency contact, if available.

128
Secondary Emergency Contact Name

35
AN
Name of secondary representative that can be contacted in the event that the primary emergency contact is not available. The contact shall have FULL facility access, site familiarity, and authority to make decisions for the business regarding incident mitigation.

129
Secondary Emergency Contact Title

35
AN
Title of secondary emergency contact.

130
Secondary Emergency Contact Business Phone
Area code + 7 digit phone number + extension
15
AN
Business phone number of secondary emergency contact.

131
Secondary Emergency Contact 24-Hour Phone
Area code + 7 digit phone number + extension
15
AN
Phone number for secondary emergency contact which is answered 24 hours a day and, if not the contact's home phone number, then the service answering the phone must be able to immediately contact the above stated individual.

132
Secondary Emergency Contact Pager Number
Area code + 7 digit phone number + extension
15
AN
Pager phone number for secondary emergency contact, if available.

133
Additional Locally Collected Information
Narrative
255
AN
For local use only.  This space may be used for CUPAs or agencies authorized by the Secretary pursuant to HSC 25404.3(f)(2) to collect any additional information necessary to meet the requirements of their individual programs.  Contact local agency for guidance.

134
Date Identification Signed
YYYYMMDD
8
D
Date the document was signed.

135
Document Preparer Name (Identification)

35
AN
Full name of person who prepared the submittal information.

136
Name of Signer

of Identification

35
AN
Full name of person signing the page.  The signer certifies to a familiarity with the information submitted and that based on their inquiry of those individuals responsible for obtaining the information, all the information submitted is true, accurate and complete.

137
Title of Signer

of Identification

35
AN
Title of person signing the page.

Chapter 2   HAZARDOUS MATERIALS

Hazardous Materials Inventory - Chemical Description (OES Form 2731)

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction 

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

3
Business Name

70
AN
Full legal name of business.

200
Add / Delete / Revise
a = add

d = delete

r = revise
1

Indicates if material is being added to the inventory, deleted from the inventory or if the information previously submitted is being revised. Not required for electronic data collection.  NOTE: This field may be empty if entire inventory is resubmitted annually.

201
Chemical Location

(Inventory)
Narrative
140
AN
Building or outside/adjacent area where hazardous material is handled. A chemical that is stored at the same pressure and temperature, in multiple locations within a building, may be reported on a single page.  NOTE: This information is not subject to public disclosure pursuant to HSC 25506.

202
Chemical Location Confidential - EPCRA
Y or N
1
AN
If the business is subject to the Emergency Planning and Community Right to Know Act (EPCRA) this field indicates whether the business wishes to keep chemical location information confidential.

203
Map Number
Optional field
15
AN
If a map is included, number of map on which the location of the hazardous material is shown.

204
Grid Number
Optional field
15
AN
If grid coordinates are used, coordinates of map that correspond to the location of the hazardous material. If applicable, multiple grid coordinates can be listed.

205
Chemical Name
Narrative
60
AN
Proper chemical name associated to the Chemical Abstract Service (CAS) number of the hazardous material.  This should be the International Union of Pure and Applied Chemistry (IUPAC) name found on the Material Safety Data Sheet (MSDS).  NOTE: If the chemical is a mixture, do not complete this field; complete the common name" field instead.

206
Trade Secret
Y or N
1
AN
Indicates if information in this section is declared a trade secret.  If business is not subject to EPCRA, trade secret information is bound by State requirements, as defined in HSC 25511.  If business is subject to EPCRA, trade secret information is bound by Federal requirements, as defined in 40 CFR and business must submit a "Substantiation to Accompany Claims of Trade Secrecy" form (40 CFR 350.27) to U.S. EPA.

207
Common Name

(Inventory)

60
AN
Common name or trade name of hazardous material or mixture containing a hazardous material.

208
EHS
Y or N
1
AN
Indicates if hazardous material is an Extremely Hazardous Substance (EHS), as defined in 40 CFR Part 355, Appendix A. If the material is a mixture containing an EHS, do not complete this field; report on the individual hazardous components in the appropriate section below.

209
CAS #
Chemical Abstract Service number
15
AN
Chemical Abstract Service (CAS) number for the hazardous material. For mixtures, enter the CAS # of the mixture if it has been assigned a number distinct from its components. If the mixture has no CAS #, do not complete this field; report the CAS #s of the individual hazardous components in the appropriate section below.

210
Fire Code Hazard Classes
Narrative
60
AN
May be required by the CUPA.  Fire Code Hazard Classes describe to first responders the type and level of hazardous materials which a business handles. A list of the various hazard classes and instructions on how to determine which class a material falls under are included in the appendices of the Uniform Fire Code Article 80.  If a material has more than one applicable hazard class, include all. Contact CUPA for guidance.

211
Hazardous Material Type (Inventory)
a = pure

b = mixture

c = waste
1
AN
Type of hazardous material. If waste material, check only that box.  If mixture or waste, complete the individual hazardous components section below.

212
Radioactive
Y or N
1
AN
Indicates whether the hazardous material stored is radioactive.

213
Curies
9 digits with floating decimal
10
N
Activity in curies if the hazardous materials stored is radioactive.

214
Physical State
a = solid

b = liquid

c = gas
1
AN
Physical state of the hazardous material stored.

215
Largest Container
Maximum 13 digit number, report units in item 221.
13
N
Total capacity of largest container in which material is stored.

216a


Federal Hazard

Category = fire
Y or N
1
AN
Physical and health hazards associated with hazardous material.  FIRE: Flammable liquids and solids, combustible liquids, pyrophorics, oxidizers.

216b
Federal Hazard

Category = reactive
Y or N
1
AN
Physical and health hazards associated with hazardous material.  REACTIVE: Unstable reactive, organic peroxides, water reactive, radioactive.

216c
Federal Hazard

Category = pressure release
Y or N
1
AN
Physical and health hazards associated with hazardous material.  PRESSURE RELEASE: Explosives, compressed gases, blasting agents.

216d
Federal Hazard

Category = acute health
Y or N
1
AN
Physical and health hazards associated with hazardous material.  ACUTE HEALTH (Immediate): Highly toxic, toxic, irritants, sensitizers, corrosives, other hazardous chemicals with an adverse effect with short term exposure.

216e
Federal Hazard

Category = chronic health
Y or N
1
AN
Physical and health hazards associated with hazardous material.  CHRONIC HEALTH (Delayed): Carcinogens, other hazardous chemicals with an adverse effect with long term exposure.

217
Average Daily Amount
Maximum 15 digit number. This amount should be consistent with the units reported in item 221. NOTE: This amount should not exceed that of maximum daily amount.
15
N
Average daily amount of hazardous material or mixture containing a hazardous material in each building or adjacent/outside area. Calculations are based on previous year's inventory of material reported on this page by totaling all daily amounts and dividing by the number of days the chemical will be present on the site. If this is a material that has not previously been present at this location the amount is the average daily amount projected to be on hand during the course of the year.

218
Maximum Daily Amount
Maximum 15 digit number. This amount should be consistent with the units reported in item 221.
15
N
Maximum amount of each hazardous material or mixture containing a hazardous material handled in a building or adjacent/outside area at any one time over the course of the year. This amount must contain at a minimum last year's inventory of the material reported on this page, with the reflection of additions, deletions, or revisions projected for the current year. 

219
Annual Waste Amount
Maximum 15 digit number
15
N
Estimate of annual amount handled, if the hazardous material is a waste. 

220
State Waste Code
California 3-digit hazardous code
3
AN
California 3-digit hazardous waste code as listed on the back of the Uniform Hazardous Waste manifest, if the hazardous material is a hazardous waste.

221
Units

(Inventory)
a = cubic feet

b = pounds

c = tons

d = gallons
1
AN
Unit of measure which is most appropriate for the material being reported on this page.  NOTE: If the material is a federally defined Extremely Hazardous Substance (EHS), all amounts must be reported in pounds. If material is a mixture containing an EHS, report the units that the material is stored in (gallons, pounds, cubic feet, or tons).

222
Days on Site

3
N
Total number of days during the year material is on site.

223a
Storage Container = aboveground tank
Y or N
1
AN
Type of storage container in which hazardous material is stored.

223b
Storage Container = underground tank
Y or N
1
AN
See description in item 223a above.

223c
Storage Container =

tank inside building
Y or N
1
AN
See description in item 223a above.

223d
Storage Container =

steel drum
Y or N
1
AN
See description in item 223a above.

223e
Storage Container =

plastic / nonmetallic drum
Y or N
1
AN
See description in item 223a above.

223f
Storage Container = can
Y or N
1
AN
See description in item 223a above.

223g
Storage Container = carboy
Y or N
1
AN
See description in item 223a above.

223h
Storage Container = silo
Y or N
1
AN
See description in item 223a above.

223i
Storage Container =

fiber drum
Y or N
1
AN
See description in item 223a above.

223j
Storage Container = bag
Y or N
1
AN
See description in item 223a above.

223k
Storage Container = box
Y or N
1
AN
See description in item 223a above.

223l
Storage Container = cylinder
Y or N
1
AN
See description in item 223a above.

223m
Storage Container =

glass bottle
Y or N
1
AN
See description in item 223a above.

223n
Storage Container =

plastic bottle
Y or N
1
AN
See description in item 223a above.

223o
Storage Container = tote bin
Y or N
1
AN
See description in item 223a above.

223p
Storage Container =

tank truck, tank wagon
Y or N
1
AN
See description in item 223a above.

223q
Storage Container =

tank car, rail car
Y or N
1
AN
See description in item 223a above.

223r
Storage Container = other
Narrative
30
AN
See description in item 223a above.

224
Storage Pressure
a  = ambient

b = below ambient

c = above ambient
1
AN
Pressure at which hazardous material is stored.

225
Storage Temperature
a = ambient

b = below ambient

c = above ambient

d = cryogenic
1
AN
Temperature at which hazardous material is stored.

226
Hazardous Component 1

Percent by Weight
2.2 (implied decimal)
4
N
Percentage weight of hazardous component in a mixture. If a range of percentages is available, report the highest percentage in that range.

227
Hazardous Component 1 Name
Narrative
80
AN
Chemical name of hazardous component in a mixture (refer to MSDS or, in the case of trade secrets, refer to manufacturer).  All hazardous components in the mixture present at greater than 1% by weight if non-carcinogenic, or 0.1% by weight if carcinogenic, should be reported.  If more than five hazardous components are present above these percentages, the business may submit an  additional sheet of paper to capture the required information.  Information on more than five components is not submitted electronically unless the CUPA has established local standards.  When reporting a waste mixture, mineral and chemical composition should be listed.

228
Hazardous Component 1 EHS
Y or N
1
AN
Indicates if the component of the mixture is considered an Extremely Hazardous Substance as defined in 40 CFR Part 355.

229
Hazardous Component 1 CAS #

15
AN
Chemical Abstract Service (CAS) number related to hazardous component in the mixture.

230
Hazardous Component 2

Percent by Weight
2.2 (implied decimal)
4
N
See description in item 226.

231
Hazardous Component 2 Name

80
AN
See description in item 227.

232
Hazardous Component 2 EHS
Y or N
1
AN
See description in item 228.

233
Hazardous Component 2 CAS #

15
AN
See description in item 229.

234
Hazardous Component 3

Percent by Weight
2.2 (implied decimal)
4
N
See description in item 226.

235
Hazardous Component 3 Name

80
AN
See description in item 227.

236
Hazardous Component 3 EHS
Y or N
1
AN
See description in item 228.

237
Hazardous Component 3 CAS #

15
AN
See description in item 229.

238
Hazardous Component 4

Percent by Weight
2.2 (implied decimal)
4
N
See description in item 226.

239
Hazardous Component 4 Name

80
AN
See description in item 227.

240
Hazardous Component 4 EHS
Y or N
1
AN
See description in item 228.

241
Hazardous Component 4 CAS #

15
AN
See description in item 229.

242
Hazardous Component 5

Percent by Weight
2.2 (implied decimal)
4
N
See description in item 226.

243
Hazardous Component 5 Name

80
AN
See description in item 227.

244
Hazardous Component 5 EHS
Y or N
1
AN
See description in item 228.

245
Hazardous Component 5 CAS #

15
AN
See description in item 229.

If more than five hazardous components are present at greater than 1% by weight if non-carcinogenic, or 0.1% by weight if carcinogenic, the information is not submitted electronically unless the CUPA has established local data standards.

246
Additional Locally

Collected  Information

255
AN
For local use only.  This space may be used by the CUPA to collect any additional information necessary to meet the requirements of their individual programs. Contact CUPA for guidance.

        Chapter 3    TANKS

A. UST Facility Page

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

3
Business Name

70
AN
Full legal name of business.

400
Type of Action

(Tank Facility)
1 = new site permit

3 = renewal permit

4 = amended permit

5 = change of information

6 = temporary site closure

7 = permanently closed site

8 = tank removed
1
AN
Reason page is being submitted.

401
Nearest Cross Street

35
AN
Name of cross street nearest to site of the tank.

402
Facility Owner Type
1 = corporation

2 = individual 

3 = partnership

4 = local  agency / district

5 = county agency

6 = state agency

7 = federal agency
1
AN
Type of business ownership.

403
Business Type

(Tank Facility)
1 = gas station

2 = distributor

3 = farm

4 = processor

5 = commercial

6 = other
1
AN
Type of business.

404
Total Number of Tanks Remaining at Site

(Tank Facility)

4
N
Number of tanks remaining on the site after requested action.

405
Indian or Trust Land
Y or N
1
AN
Indicates if facility is located on Indian reservation or other trust lands.

406
Public Agency Supervisor Name

35
AN
Contact person for tank records, if facility owner is a public agency.

407
Property Owner Name

35
AN
Name of property owner, if different from business owner on Business Owner/Operator page.

408
Property Owner Phone
Area code + 7 digit phone number + extension
15
AN
Phone number of property owner, if different from business owner.

409
Property Owner Address
Postal standard:

2 lines, 35 characters
70
AN
Street or mailing address of property owner, if different from business owner.

410
Property Owner City
Postal standard
20
AN
City of property owner, if different from business owner.

411
Property Owner State
Valid 2-digit state code
2
AN
State of property owner, if different from business owner.

412
Property Owner Zip Code
Postal standard
9
AN
Zip code of property owner, if different from business owner.

413
Property Owner Type
1 = corporation

2 = individual 

3 = partnership

4 = local agency / district

5 = county agency

6 = state agency

7 = federal agency
1
AN
Type of property ownership.

414
Tank Owner Name (Facility)

35
AN
Name of tank owner, if different from business owner on Business Owner/Operator page.

415
Tank Owner Phone (Facility)
Area code + 7 digit phone number + extension
15
AN
Phone number of tank owner, if different from business owner.

416
Tank Owner Street (Facility)
Postal standard:

2 lines, 35 characters
70
AN
Street or mailing address of tank owner, if different from business owner.

417
Tank Owner City (Facility)
Postal standard
20
AN
City of tank owner, if different from business owner.

418
Tank Owner State (Facility)
Valid 2-digit state code
2
AN
State of tank owner, if different from business owner.

419
Tank Owner Zip Code (Facility)
Postal standard
9
AN
Zip code of tank owner, if different from business owner.

420
Tank Owner Type
1 = corporation

2 = individual 

3 = partnership

4 = local agency / district

5 = county agency

6 = state agency

7 = federal agency
1
AN
Type of tank ownership.

421
BOE Number
BOE 8 digit number, first two digits = 44
8
AN
Board of Equalization (BOE) UST storage fee account number.  This number is required before a permit application can be processed.  Registration with the BOE will ensure that you will receive a quarterly storage fee return in reporting the $0.012 per gallon fee due on the number of gallons placed in your USTs.  The BOE will code persons exempt from paying the storage fee so returns will not be sent.  If you do not have an account number with the BOE or if you have any questions regarding the fee or exemptions, please call the BOE at (916) 322-9669 or write to the BOE at the following address:

Board of Equalization

Fuel Taxes Division

P.O. Box 942879

Sacramento, CA 94279-0030

422-1
Petroleum UST Financial Responsibility Code = self-insured
Y or N
1
AN
Method(s) used by owner and/or operator in meeting the Federal and State financial responsibility requirements.  USTs owned by any Federal or State agency as well as non-petroleum USTs are exempt from this requirement.

422-2
Petroleum UST Financial Responsibility Code = guarantee
Y or N
1
AN
See description in item 422-1.

422-3
Petroleum UST Financial Responsibility Code = insurance
Y or N
1
AN
See description in item 422-1.

422-4
Petroleum UST Financial Responsibility Code = surety bond
Y or N
1
AN
See description in item 422-1.

422-5
Petroleum UST Financial Responsibility Code = letter of credit
Y or N
1
AN
See description in item 422-1.

422-6
Petroleum \UST Financial Responsibility Code = exemption
Y or N
1
AN
See description in item 422-1.

422-7
Petroleum UST Financial Responsibility Code = State Fund
Y or N
1
AN
See description in item 422-1.

422-8
Petroleum UST Financial Responsibility Code = State Fund and CFO letter
Y or N
1
AN
See description in item 422-1.

422-9
Petroleum UST Financial Responsibility Code = State Fund and CD
Y or N
1
AN
See description in item 422-1.

422-10
Petroleum UST Financial Responsibility Code = local government mechanism
Y or N
1
AN
See description in item 422-1.

422-99
Petroleum UST Financial Responsibility Code = other
Narrative
30
AN
See description in item 422-1.

423
Notification  Address
1 = facility address

2 = property owner address

3 = tank owner address
1
AN
Address to which legal notifications and mailings should be sent.

424
Date Certified

(Tank Facility)
YYYYMMDD
8
D
Date the page was signed.

425
Applicant Phone

(Tank Facility)
Area code + 7 digit phone number + extension
15
AN
Phone number of applicant (person certifying).

426
Applicant Name

(Tank Facility)

35
AN
Name of signatory.  The applicant certifies to a belief that all the information submitted is accurate and complete. The applicant may be the Owner/Operator or officially designated representative.

427
Applicant Title

(Tank Facility)

35
AN
Title of person signing the page.

428
State UST Facility Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
For local use only.  County and jurisdiction number from tax code list.  This number may be the same as the Facility ID number.

429
1998 Upgrade Certificate Number

6
AN
For local use only.  The State Water Resources Control Board 1998 Upgrade Certificate Number for the facility.

    Chapter 3   TANKS

B. UST Tank Pages 1 and 2

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number.
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of business.

430
Type of Action

(Tank Unit)
1 = new site permit

3 = renewal permit

4 = amended permit

5 = change of information

6 = temporary site closure

7 = permanently closed on site

8 = tank removed
1
AN
Reason page is being submitted.

431
Location Within Site

(Tank Unit)

70
AN
Optional.  Location of tank within site.

432
Tank ID #

(Tank Unit)

6
AN
Owner's tank ID#.  This is a unique tank number used by owner to identify the tank.

433
Tank Manufacturer

30
AN
Name of company that manufactured tank.

434
Compartmentalized Tank
Y or N
1
AN
Indicates whether the tank is compartmentalized. Each compartment is considered a separate tank and requires the completion of separate tank pages.

435
Date Tank Installed
YYYYMM
6
D
Year and month the tank installation was completed.

436
Tank Capacity

7
N
Tank capacity in gallons.

437
Number of Tank Compartments

2
AN
Number of compartments in compartmentalized tank.

438
Additional Description
Narrative
70
AN
For local use only.  Additional tank or location description.

439
Tank Use
01 = motor vehicle fuel

02 = non-fuel petroleum

03 = chemical product

04 = hazardous waste

        (includes used oil)

95 = unknown
2
AN
Type of hazardous materials stored.

440
Petroleum Type
1a = regular unleaded

1b = premium unleaded

1c = midgrade unleaded

02 = leaded

03 = diesel

04 = gasohol

05 = jet fuel

06 = aviation gas

99 = other
2
AN
Type of fuel if tank stores vehicle fuel.

441
Common Name

(Tank Unit)

30
AN
Common name of substance stored.  Same as on Hazardous Materials Inventory - Chemical Description page.

442
CAS # (Tank Unit)

15
AN
CAS # of chemical stored in UST.  Same as the CAS # on the Hazardous Material Inventory -  Chemical Description page.

443
Type of Tank
01 = single wall

02 = double wall

03 = single wall w/ exterior

        membrane liner

04 = single wall in a vault

05 = single wall w/ internal

        bladder system

95 = unknown

99 = other
2
AN
Type of tank construction.

444
Tank Material

(primary tank)
01 = bare steel

02 = stainless steel

03 = fiberglass / plastic

04 = steel clad w/ fiberglass

        reinforced plastic (frp)

05 = concrete

08 = frp compatible w/ 100%

        methanol

95 = unknown

99 = other
2
AN
Construction material of the primary tank.

445
Tank Material

(secondary tank)
01 = bare steel

02 = stainless steel

03 = fiberglass / plastic

04 = steel clad w/ fiberglass

        reinforced plastic (frp)

05 = concrete

08 = frp compatible w/ 100%

        methanol

09 = frp non-corrodible jacket

10 = coated steel

95 = unknown

99 = other
2
AN
Construction material of the secondary tank.

446
Tank Interior Lining

or Coating
01 = rubber‑lined

02 = alkyd lining

03 = epoxy lining

04 = phenolic lining

05 = glass lining

06 = unlined

95 = unknown

99 = other
2
AN
Construction material of the interior lining or coating.

447
Date Tank Interior

Lining Installed
YYYYMMDD
8
N
For local use only.  Date interior lining or coating installed.

448
Other Tank Corrosion Protection
01 = manufactured cathodic               protection

02 = sacrificial anode

03 = fiberglass reinforced                   plastic (frp)

04 = impressed current

95 = unknown

99 = other
2
AN
Other tank corrosion protection methods, if applicable.

449
Date Tank Corrosion Protection Installed
YYYYMMDD
8
N
For local use only. Date tank corrosion protection installed.

450-1
Year Spill and Overfill Installed = spill containment
YYYY
4
N
Year spill containment installed.

450-2
Year Spill and Overfill Installed = drop tube 
YYYY
4
N
Year drop tube installed.

450-3
Year Spill and Overfill Installed = striker plate
YYYY
4
N
Year striker plate installed.

451-1
Type of Spill Protection = spill containment
Narrative
15
AN
For local use only.  Type of tank spill protection.

451-2
Type of Spill Protection = drop tube
Narrative
15
AN
For local use only.  Type of tank spill protection.

451-3
Type of Spill Protection = striker plate
Narrative
15
AN
For local use only.  Type of tank spill protection.

452-1
Year Overfill Protection Equipment Installed = alarm
YYYY
4
N
Year alarm installed.

452-2
Year Overfill Protection Equipment Installed =

ball float
YYYY
4
N
Year ball float installed.

452-3
Year Overfill Protection Equipment Installed = fill tube shut off valve
YYYY
4
N
Year fill tube shut off valve installed.

452-4
Overfill Protection Equipment = exempt
Y or N
1
AN
Indicates exemption from overfill protection.

453-1


Tank Leak Detection (Single Wall) = visual (exposed portion only)
Y or N
1
AN
Type of tank leak detection.

453-2
Tank Leak Detection (Single Wall) = automatic tank gauging (ATG)
Y or N
1
AN
Type of tank leak detection.

453-3
Tank Leak Detection (Single Wall) = continuous ATG
Y or N
1
AN
Type of tank leak detection.

453-4
Tank Leak Detection (Single Wall) = statistical inventory reconciliation (SIR) + biennial tank testing
Y or N
1
AN
Type of tank leak detection.

453-5
Tank Leak Detection (Single Wall) = manual 

tank gauging (MTG)
Y or N
1
AN
Type of tank leak detection.

453-6
Tank Leak Detection (Single Wall) = vadose zone
Y or N
1
AN
Type of tank leak detection.

453-7
Tank Leak Detection (Single Wall) = groundwater
Y or N
1
AN
Type of tank leak detection.

453-8
Tank Leak Detection (Single Wall) = tank testing
Y or N
1
AN
Type of tank leak detection.

453-99
Tank Leak Detection (Single Wall) = other
Narrative
30
AN
Type of tank leak detection.

454-1
Tank Leak Detection (Double Wall) = visual (single wall in vault only)
Y or N
1
AN
Type of tank leak detection.

454-2
Tank Leak Detection (Double Wall) = continuous interstitial monitoring
Y or N
1
AN
Type of tank leak detection.

454-3
Tank Leak Detection (Double Wall) = manual monitoring
Y or N
1
AN
Type of tank leak detection.

455
Estimated Date Last Used
YYYYMMDD
8
D
Date tank last used (for closure in place).

456
Estimated Quantity of Substance Remaining

in Tank

7
N
Estimated quantity of hazardous substance remaining in gallons (for closure in place).

457
Tank Filled with 

Inert Material
Y or N


1
AN
Indicates whether tank was filled with an inert material prior to closure (for closure in place).

458-1


Piping System Type (Underground) = pressure
Y or N
1
AN
Type of underground piping system.

458-2
Piping System Type (Underground) = suction
Y or N
1
AN
Type of underground piping system.

458-3
Piping System Type (Underground) = gravity
Y or N
1
AN
Type of underground piping system.

459-1
Piping System Type (Aboveground) = pressure
Y or N
1
AN
Type of aboveground piping system.

459-2
Piping System Type (Aboveground) = suction
Y or N
1
AN
Type of aboveground piping system.

459-3
Piping System Type (Aboveground) = gravity
Y or N
1
AN
Type of aboveground piping system.

460-1


Piping Construction (Underground) = single wall
Y or N
1
AN
Type of underground piping construction.

460-2
Piping Construction (Underground) = double wall
Y or N
1
AN
Type of underground piping construction.

460-3
Piping Construction (Underground) = lined trench
Y or N
1
AN
Type of underground piping construction.

460-95
Piping Construction (Underground) = unknown
Y or N
1
AN
Type of underground piping construction.

460-99
Piping Construction (Underground) = other
Y or N
1
AN
Type of underground piping construction.

461
Piping Manufacturer (Underground)
Narrative
30
AN
Name of underground piping manufacturer.

462-1
Piping Construction (Aboveground) = single wall
Y or N
1
AN
Type of aboveground piping construction.

462-2
Piping Construction (Aboveground) = double wall
Y or N
1
AN
Type of aboveground piping construction.

462-95
Piping Construction (Aboveground) = unknown
Y or N
1
AN
Type of aboveground piping construction.

462-99
Piping Construction (Aboveground) = other
Y or N
1
AN
Type of aboveground piping construction.

463
Piping Manufacturer (Aboveground)
Narrative
30
AN
Name of aboveground piping manufacturer.

464-1
Piping Material and Corrosion Protection (Underground) = bare steel
Y or N
1
AN
Construction material and/or corrosion protection of underground piping.

464-2
Piping Material and Corrosion Protection (Underground) = stainless steel
Y or N
1
AN
Construction material and/or corrosion protection of underground piping.

464-3
Piping Material and Corrosion Protection (Underground) = plastic compatible with contents
Y or N
1
AN
Construction material and/or corrosion protection of underground piping.

464-4
Piping Material and Corrosion Protection (Underground) = fiberglass
Y or N
1
AN
Construction material and/or corrosion protection of underground piping.

464-5
Piping Material and Corrosion Protection (Underground) = steel w/ coating
Y or N
1
AN
Construction material and/or corrosion protection of underground piping.

464-6
Piping Material and Corrosion Protection (Underground) = frp compatible w/ 100% methanol
Y or N
1
AN
Construction material and/or corrosion protection of underground piping.

464-7
Piping Material and Corrosion Protection (Underground) =

galvanized steel
Y or N
1
AN
Construction material and/or corrosion protection of underground piping.

464-8
Piping Material and Corrosion Protection (Underground) = flexible (HDPE - high density polyethylene)
Y or N
1
AN
Construction material and/or corrosion protection of underground piping.

464-9
Piping Material and Corrosion Protection (Underground) =

cathodic protection
Y or N
1
AN
Construction material and/or corrosion protection of underground piping.

464-95
Piping Material and Corrosion Protection (Underground) = unknown
Y or N
1
AN
Construction material and/or corrosion protection of underground piping.

464-99
Piping Material and Corrosion Protection (Underground) = other
Y or N
1
AN
Construction material and/or corrosion protection of underground piping.

465-1
Piping Material and Corrosion Protection (Aboveground) = bare steel
Y or N
1
AN
Construction material and/or corrosion protection of aboveground piping.

465-2
Piping Material and Corrosion Protection (Aboveground) =

stainless steel
Y or N
1
AN
Construction material and/or corrosion protection of aboveground piping.

465-3
Piping Material and Corrosion Protection (Aboveground) = plastic compatible with contents
Y or N
1
AN
Construction material and/or corrosion protection of aboveground piping.

465-4
Piping Material and Corrosion Protection (Aboveground) = fiberglass
Y or N
1
AN
Construction material and/or corrosion protection of aboveground piping.

465-5
Piping Material and Corrosion Protection (Aboveground) =

steel w/ coating
Y or N
1
AN
Construction material and/or corrosion protection of aboveground piping.

465-6
Piping Material and Corrosion Protection (Aboveground) = frp compatible w/ 100% methanol
Y or N
1
AN
Construction material and/or corrosion protection of aboveground piping.

465-7
Piping Material and Corrosion Protection (Aboveground) =

galvanized steel
Y or N
1
AN
Construction material and/or corrosion protection of aboveground piping.

465-8
Piping Material and Corrosion Protection (Aboveground) = flexible (HDPE - high density polyethylene)
Y or N
1
AN
Construction material and/or corrosion protection of aboveground piping.

465-9
Piping Material and Corrosion Protection (Aboveground) =

cathodic protection
Y or N
1
AN
Construction material and/or corrosion protection of aboveground piping.

465-95
Piping Material and Corrosion Protection (Aboveground) = unknown
Y or N
1
AN
Construction material and/or corrosion protection of aboveground piping.

465-99
Piping Material and Corrosion Protection (Aboveground) = other
Y or N
1
AN
Construction material and/or corrosion protection of aboveground piping.

466-1
Piping Leak Detection (Underground - single wall) = electronic line leak detector + auto shutoff + alarms
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-2
Piping Leak Detection (Underground - single wall) = monthly 0.2 gph test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-3
Piping Leak Detection (Underground - single wall) = annual integrity test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-5
Piping Leak Detection (Underground - single wall) = daily visual monitoring + triennial integrity test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-7
Piping Leak Detection (Underground - single wall) = self monitoring
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-9
Piping Leak Detection (Underground - single wall) = biennial integrity test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-10a
Piping Leak Detection (Underground - secondarily contained) = sump sensor + alarms +  auto shutoff for leaks
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-10b
Piping Leak Detection (Underground - secondarily contained) = sump sensor + alarms +  auto shutoff for leaks, failure, and disconnect
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-10c
Piping Leak Detection (Underground - secondarily contained) = sump sensor + alarms + no auto shutoff
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-11
Piping Leak Detection (Underground - secondarily contained, pressure) = automatic leak detector + flow shutoff or restriction
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-12
Piping Leak Detection (Underground - secondarily contained) = annual integrity test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-13
Piping Leak Detection (Underground - secondarily contained, suction/gravity) = sump sensor + alarms
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-14
Piping Leak Detection (Underground - emergency generators) = sump sensor without auto shutoff + alarms
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-15
Piping Leak Detection (Underground - emergency generators) = automatic leak detector without flow shutoff or restriction
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-16
Piping Leak Detection (Underground - emergency generators) = annual integrity test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

466-17
Piping Leak Detection (Underground - emergency generators) = daily visual check
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for underground piping.

467-1


Piping Leak Detection (Aboveground - single wall) = electronic line leak detector + auto shutoff + alarms
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-2
Piping Leak Detection (Aboveground - single wall) = monthly 0.2 gph test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-3
Piping Leak Detection (Aboveground - single wall) = annual integrity test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-4
Piping Leak Detection (Aboveground - single wall, pressure) = daily visual check
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-5
Piping Leak Detection (Aboveground - single wall, suction) = daily visual monitoring
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-6
Piping Leak Detection (Aboveground - single wall) = triennial integrity test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-7
Piping Leak Detection (Aboveground - single wall) = self monitoring
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-8
Piping Leak Detection (Aboveground - single wall, gravity) = daily visual monitoring
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-9
Piping Leak Detection (Aboveground - single wall) = biennial integrity test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-10a
Piping Leak Detection (Aboveground - secondarily contained) = sump sensor + alarms + auto shutoff

for leaks
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-10b
Piping Leak Detection (Aboveground - secondarily contained) = sump sensor + alarms + auto shutoff for leaks, failure and disconnect
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-10c
Piping Leak Detection (Aboveground - secondarily contained, pressure) = sump sensor + alarms + no auto shutoff
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-11
Piping Leak Detection (Aboveground - secondarily contained) = automatic

leak detector
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-12
Piping Leak Detection (Aboveground - secondarily contained) = annual integrity test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-13
Piping Leak Detection (Aboveground - secondarily contained, suction/gravity) = sump sensor + alarms
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-14
Piping Leak Detection (Aboveground - emergency generators) = sump sensor without auto shutoff + alarms
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-15
Piping Leak Detection (Aboveground - emergency generators) = automatic leak detector
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-16
Piping Leak Detection (Aboveground - emergency generators) = annual integrity test
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

467-17
Piping Leak Detection (Aboveground - emergency generators) = daily visual check
Y or N
1
AN
Leak detection system used to comply with monitoring requirements for aboveground piping.

468
Date Dispenser Containment Installed
YYYYMMDD
8
N
Date dispenser containment installed.

469
Dispenser Containment Type
1 = float mechanism

2 = sensor + alarms

3 = sensor + auto shutoff +

      alarms

4 = daily visual check

5 = trench liner / monitoring

6 = none
1
AN
Type of dispenser containment.

470
Date Certified (Tank Unit)
YYYYMMDD
8
D
Date the document was signed.

471
Owner/ Operator Name (Tank Unit)

35
AN
Name of signatory.  The applicant certifies to a belief that all the information submitted is accurate and complete. The applicant may be the Owner/Operator or officially designated representative.

472
Owner/ Operator Title

(Tank Unit)

35
AN
Title of person signing the page.

473
Permit Number

9
AN
For local use only.  Permit number.

474
Permit Approved By

35
AN
For local use only.Name of person approving permit.

475
Permit Expiration Date
YYYYMMDD
8
D
For local use only.  Date of permit expiration.

   Chapter 3   TANKS

C. UST Installation - Certificate of Compliance Page

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to permit cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of business.

476
Address

(Tank Installation)
Postal standard:

2 lines, 35 characters
70
AN
For local use only.  Address of tank installation.

477
Tank ID #

(Tank Installation)

6
AN
Owner's tank ID#, if there is a tank number used by owner to identify the tank.  Unique identifier of tank at site.  Same as data element # 432.

478
Trained and Certified

by Tank and Piping Manufacturer
Y or N
1
AN
Indicates whether installer was trained and certified by tank and piping manufacturer.

479
Registered Engineer Inspection
Y or N
1
AN
Indicates whether installation was certified by registered professional engineer.

480
Unified Program

Agency Approval
Y or N
1
AN
Indicates whether installation was approved by the Unified Program Agency.

481
Completion of Manufacturer's Checklist
Y or N
1
AN
Indicates whether work on manufacturer's installation checklist was completed.

482
Contractors State License Board Certification or License
Y or N
1
AN
Indicates whether contractor has been certified or licensed by the Contractors State License Board.

483
Installation Description
Narrative
30
AN
Description of installation.

484
Date Certified

(Tank Installation)
YYYYMMDD
8
D
Date tank installation certification was signed.

485
Tank Owner/Agent Name (Installation)

35
AN
Name of tank owner/agent, or officially designated representative of the owner/agent. The signer certifies to a belief that all the information submitted is accurate and complete.

486
Tank Owner/Agent Title (Installation)

35
AN
Title of person signing the page.

  Chapter 4   HAZARDOUS WASTE

A. Recyclable Materials

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

2
EPA ID Number
12 digit identifier beginning with CA
12
AN
EPA Identification number for businesses that generate, recycle, or treat hazardous waste.  For facilities in California, the number usually starts with the letters CA.  The number can be obtained from the Telephone Information Center at

(916) 324-1781, (800) 61-TOXIC or (800) 618-6942.

3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of the business.

500
Beginning Date of

Reporting Period
YYYYMMDD
8
D
Beginning date of the reporting period for this report.  This report is for two calendar years and is due on July 1 of every even numbered year per HSC 25143.10(a).

501
Ending Date of

Reporting Period
YYYYMMDD
8
D
Ending date of the reporting period for this report. This report is for two calendar years and is due on July 1 of every even numbered year per HSC 25143.10(a).

I.  TYPE OF RECYCLING ACTIVITY

502
Onsite Recycling
Y or N
1
AN
Facility must report if it recycles onsite more than 100 kilograms per month of recyclable material under a claim that the material qualifies for exclusion or exemption pursuant to HSC 25143.2.  Triggers requirement to complete parts III and IV of the Recyclable Materials Report.

503
Offsite Recycling
Y or N
1
AN
Facility must report if it recycles more than 100 kilograms per month of recyclable material under a claim that the material qualifies for exclusion received from an offsite location pursuant to HSC 25143.2.  Triggers requirement to complete all parts of the Recyclable Materials Report.

II.  OFFSITE GENERATOR OF RECYCLABLE MATERIAL INFORMATION (repeats for each generator, EXCEPT for the signature information)

504
Offsite Generator Name

35
AN
Facility name of offsite generator of recyclable material.

505
Offsite Generator

EPA ID Number
12 digit identifier beginning with CA
12
AN
EPA Identification number of generator of recyclable material.

506
Street Address

(Offsite Generator)
Postal standard:

2 lines, 35 characters
70
AN
Street address for offsite generator of recyclable material.  No post office box numbers are allowed. This information must provide a means to geographically locate the facility.

507
Phone

(Offsite Generator)
Area code + 7 digit phone number + extension
15
AN
Phone number for offsite generator of recyclable material.

508
City

(Offsite Generator)
Postal standard
20
AN
City or unincorporated area for offsite generator of recyclable material.

509
State

(Offsite Generator)
Valid 2-digit state code
2
AN
State for offsite generator of recyclable material.

510
Zip Code

(Offsite Generator)
Postal standard
9
AN
Zip code for offsite generator of recyclable material.

511
Mailing Address 

(Offsite Generator)
Postal standard:

2 lines, 35 characters
70
AN
Mailing address for offsite generator of recyclable material, if different from street address.

512
City for Mailing Address 

(Offsite Generator)
Postal standard
20
AN
City or unincorporated area for mailing address of offsite generator of recyclable material, if different from street address.

513
State for Mailing Address

(Offsite Generator)
Valid 2-digit state code
2
AN
State for mailing address of offsite generator of recyclable material, if different from street address.

514
Zip Code for Mailing Address (Offsite Generator)
Postal standard
9
AN
Zip code for mailing address of  offsite generator of recyclable material, if different from street address.

III.   CERTIFICATION SECTION

515
Date (Recyclable Materials)
YYYYMMDD
8
D
Date the document was signed.

516
Name of Document Preparer (Recyclable Materials)

35
AN
Full name of person who prepared the Recyclable Materials Report.

517
Certifier Name 

(Recyclable Materials)

35
AN
Full name of person signing the Recyclable Materials Report.

518
Certifier Title

(Recyclable Materials)

35
AN
Title of person signing the Recyclable Materials Report.

IV.  RECYCLABLE MATERIAL INFORMATION
A.  Description

519
Total Number of

Recyclable Materials

3
AN
Total number of recyclable materials reported in this report.  Verify that the number of recyclable material reports (part IV) is the same as the total number listed here.

520
Recyclable Material

Number

3
AN
Unique identification number for each recyclable material reported. They can be numbered sequentially, or by any other system as long as the numbers are not repeated or duplicated.

521
Common Name

(Recyclable Materials)

60
AN
Common name of material recycled.  This is the same as the Common Name on the Hazardous Materials Inventory - Chemical Description page.

522
Quantity During Two

Year Reporting Period

(Recyclable Materials)

10
N
Total quantity of this recyclable material recycled during the two year reporting period.  Round to nearest whole number.

523
Units

(Recyclable Materials)
a = gallons

b = pounds

c = tons

d = kilograms
1
AN
Unit of measure for quantity reported.

524
Recyclable Material Description
Narrative
150
AN
Description of recyclable material.

525
Recycling Process and Beneficial Use
Narrative
150
AN
Description of the recycling process and beneficial use of recyclable material, i.e., how it was used to make or substitute for a product.

526
Authorizing Provision of

HSC Section 25143.2
Statutory citation
20
AN
Subdivision(s), and subparagraph(s) (if applicable) of HSC 25143.2 that served as the basis for the claim to exemption or exclusion.  For example: HSC 25143.2(d)(2)(C).

527
Basis for Claim to Exclusion

or Exemption
Narrative
50
AN
Explanation of the basis for the claim to an exclusion or exemption.

IV.  RECYCLABLE MATERIAL INFORMATION
B.  Product And Constituent Information:  Offsite Only

528
Hazardous Constituent 1

80
AN
Description of hazardous constituent (use common name, if appropriate).

529
Concentration

Recyclable Material 1
6 digits with floating decimal
7
N
Concentration of this hazardous constituent in this recyclable material.

530
Units

Recyclable Material 1
a = percentage

b = parts per million
1
AN
Unit of measure which is most appropriate for this hazardous constituent.

531
Concentration

Final Product 1
6 digits with floating decimal
7
N
Concentration of this hazardous constituent in the final product.

532
Units

Final Product 1
a  = percentage

b = parts per million
1
AN
Unit of measure which is most appropriate for this hazardous constituent.

533
Final Products/

Uses for Constituent 1

60
AN
Final use of the recyclable material.  Description of the product(s) that resulted from the recycling process and how that product is beneficially used.

534
Hazardous Constituent 2

80
AN
Description of hazardous constituent (use common name, if appropriate).

535
Concentration

Recyclable Material 2
6 digits with floating decimal
7
N
Concentration of this hazardous constituent in this recyclable material.

536
Units

Recyclable Material 2
a  = percentage

b = parts per million
1
AN
Unit of measure which is most appropriate for this hazardous constituent.

537
Concentration

Final Product 2
6 digits with floating decimal
7
N
Concentration of this hazardous constituent in the final product.

538
Units 

Final Product 2
a = percentage

b = parts per million
1
AN
Unit of measure which is most appropriate for this hazardous constituent.

539
Final Products/

Uses for Constituent 2

60
AN
Final use of the recyclable material.  Description of the product(s) that resulted from the recycling process and how that product is beneficially used.

540
Hazardous Constituent 3

80
AN
Description of hazardous constituent (use common name, if appropriate).

541
Concentration

Recyclable Material 3
6 digits with floating decimal
7
N
Concentration of this hazardous constituent in this recyclable material.

542
Units

Recyclable Material 3
a = percentage

b = parts per million
1
AN
Unit of measure which is most appropriate for this hazardous constituent.

543
Concentration

Final Product 3
6 digits with floating decimal
7
N
Concentration of this hazardous constituent in the final product.

544
Units

Final Product 3
a = percentage

b = parts per million
1
AN
Unit of measure which is most appropriate for this hazardous constituent.

545
Final Products/

Uses for Constituent 3

60
AN
Final use of the recyclable material.  Description of the product(s) that resulted from the recycling process and how that product is beneficially used.

546
Hazardous Constituent 4

80
AN
Description of hazardous constituent (use common name, if appropriate).

547
Concentration

Recyclable Material 4
6 digits with floating decimal
7
N
Concentration of this hazardous constituent in this recyclable material.

548
Units

Recyclable Material 4
a = percentage

b = parts per million
1
AN
Unit of measure which is most appropriate for this hazardous constituent.

549
Concentration

Final Product 4
6 digits with floating decimal
7
N
Concentration of this hazardous constituent in the final product.

550
Units

Final Product 4 
a = percentage

b = parts per million
1
AN
Unit of measure which is most appropriate for this hazardous constituent.

551
Final Products/

Uses for Constituent 4

60
AN
Final use of the recyclable material.  Description of the product(s) that resulted from the recycling process and how that product is beneficially used.

If more than four hazardous constituents are present in one recyclable material, submit this information on additional sheets.  The information is not submitted electronically unless the CUPA has established local data standards.

V.  DOCUMENTATION OF KNOWN MARKET

552
Documentation

(Offsite Recycling)
Y or N
1
AN
If the recycler is an offsite facility, indicates if documentation is provided.  Documentation is required pursuant to HSC 25143.10(a)(3)(A) to show that there was a known market for disposition of the recyclable material and any products manufactured from it.  A copy of this report must be submitted to the generator of the material.

   Chapter 4   HAZARDOUS WASTE

1. Onsite Hazardous Waste Treatment Notification - Facility

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of business.

600
Notification Status

(Onsite Hazardous

Waste Facility)
a = amended

b = initial

c = renewal
1
AN
Notification status under the tiered permitting system.

601a


Permit Status =

facility permit
Y or N
1
AN
Permit status for State issued hazardous waste permits or grants of authorization.

601b
Permit Status =

interim status
Y or N
1
AN
Permit status for State issued hazardous waste permits or grants of authorization.

601c
Permit Status = standardized permit
Y or N
1
AN
Permit status for State issued hazardous waste permits or grants of authorization.

601d
Permit Status = variance
Y or N
1
AN
Permit status for State issued hazardous waste permits or grants of authorization.

601e
Permit Status =

consent agreement
Y or N
1
AN
Permit status for State issued hazardous waste permits or grants of authorization.

602a


Number of Units = CESQT
Maximum 3 digit number


3
AN
Number of units at facility.  Attach a unit specific notification page for EACH unit covered by this notification, except CE-CL units.

602b
Number of Units = CESW
Maximum 3 digit number
3
AN
Number of units at facility.  Attach a unit specific notification page for EACH unit covered by this notification, except CE-CL units.

602c
Number of Units = CA
Maximum 3 digit number
3
AN
Number of units at facility.  Attach a unit specific notification page for EACH unit covered by this notification, except CE-CL units.

602d
Number of Units = PBR
Maximum 3 digit number
3
AN
Number of units at facility.  Attach a unit specific notification page for EACH unit covered by this notification, except CE-CL units.

602e
Number of Units = CEL
Maximum 3 digit number
3
AN
Number of units at facility.  Attach a unit specific notification page for EACH unit covered by this notification, except CE-CL units.

602f
Number of Units = CE-CL
Maximum 3 digit number
3
AN
Number of CE-CL (commercial laundries) units at facility.  No unit specific notification is required for

CE-CL.

602g
Number of Units = total
Maximum 3 digit number
3
AN
Total number of units at facility.  Verify that this number equals the number of unit notification pages plus CE-CL units.

603
Date Certified

(Onsite Hazardous

Waste Facility)
YYYYMMDD
8
D
Date the page was signed.

604
Owner / Operator Name (Onsite Hazardous

Waste Facility)

35
AN
Full name of business owner/operator, or officially designated representative of the owner/operator. The signer certifies to a belief that all the information submitted is accurate and complete.  The person signing this page must be an owner or officer of the company who is authorized to make decisions for the facility and who has operational control.  In most companies, this is not the environmental compliance or technical staff.   The signer of the certifications attests to their accuracy under penalty of law for submitting false information.  The certifications cover waste minimization, the eligibility of the unit(s) for the indicated tier, the fact that the unit meets all of the operating requirements for that tier, and that the information is accurate.

605
Owner / Operator Title (Onsite Hazardous

Waste Facility)

35
AN
Title of person signing the page.

   Chapter 4   HAZARDOUS WASTE

3. Onsite Hazardous Waste Treatment Notification - Unit 

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of business.

606
Unit ID Number

(Onsite Hazardous Waste Facility)

18
AN
Unique identification number for unit. The units can be numbered sequentially, or by any other system as long as the numbers are not repeated or duplicated.

607

Unit Type / Tier

(Onsite Hazardous Waste Facility)
a = CESQT

b = CESW

c = CA

d = PBR

e = CEL
1
AN
Unit type under the tiered permitting program.

608
Number of Tanks

(Onsite Hazardous Waste Facility)

2
AN
Number of tanks containing hazardous waste. "Tank" means a stationary device, designed to contain an accumulation of hazardous waste which is constructed primarily of non-earthen materials (e.g., wood, concrete, steel, plastic) which provide structural support.

609
Number Of Containers/

Treatment Areas

2
AN
Number of containers and container treatment areas for hazardous waste.  "Container" means any device that is open or closed, and portable in which a material can be stored, handled, treated, transported, recycled, or disposed of.  "Container treatment area" is the location set aside and used to treat containers, such as drums.

610
Unit Name

30
AN
Name of treatment unit.  A unit is defined as a tank, a container, or a combination of tanks or tank systems and/or containers located together that are used in sequence to treat or accumulate one or more compatible hazardous wastestreams. The devices are either plumbed together or otherwise linked so as to form one system.

611
Monthly Treatment Volume

8
AN
Estimated monthly total volume of hazardous waste treated in each unit. If the volume fluctuates significantly by month, enter the maximum or highest volume treated in any month.

612
Unit Of Measure
a = pounds

b = gallons
1
AN
Unit of measure for monthly treatment volume.

613
Specific Waste Type Treated
Narrative
150
AN
Description of the specific waste type(s) treated. For example, if waste qualifies as an aqueous waste with metal or organics, indicate the specific metals or organics.

614
Treatment Process Description
Narrative
150
AN
Description of treatment process(es) used.

615a


Basis for Not Needing Federal

Permit = treated waste is not a hazardous waste under federal law (California ‑ only waste)


Y or N


1
AN
Basis for determining that a hazardous waste permit is not required under RCRA and the federal regulations adopted under RCRA (40 CFR).  Only onsite treatment operations that are not required to obtain a federal permit are eligible for the onsite permitting tiers.  Indicate at least one reason to prove eligibility for the onsite treatment tiers. Contact the CUPA, DTSC Regional Office, the U.S. EPA's Region IX RCRA Information Line at (415)744-2074, or the U.S. EPA RCRA Hotline at (800)424-9346.

615b
Basis for Not Needing Federal

Permit = treated in waste water treatment units (tanks) and discharged to a publicly owned treatment works (POTW) / sewering agency or under an NPDES permit


Y or N
1
AN
Basis for determining that a hazardous waste permit is not required under RCRA and the federal regulations adopted under RCRA (40 CFR). Wastewater treatment unit means a device which (1) is part of a wastewater treatment facility regulated under section 402 or 307(b) of the Clean Water Act, and (2) receives and treats or stores an influent wastewater that is a hazardous waste or that generates and accumulates a wastewater treatment sludge that is a hazardous waste or that treats or stores a wastewater treatment sludge which is a hazardous waste, and (3) meets the definition of tank or tank system.  This term is defined in 40 CFR 260.10.

615c
Basis for Not Needing Federal

Permit = treatment in elementary neutralization units
Y or N
1
AN
Basis for determining that a hazardous waste permit is not required under RCRA and the federal regulations adopted under RCRA (40 CFR). Elementary neutralization unit means a device which: (1) is used for neutralizing wastes that are hazardous only because they exhibit the corrosivity characteristic or they are listed only for this reason: and (2) meets the definition of tank, tank system, container, transport vehicle, or vessel.  This term is defined in 40 CFR 260.10.

615d
Basis for Not Needing Federal

Permit = treatment in a totally enclosed treatment facility
Y or N
1
AN
Basis for determining that a hazardous waste permit is not required under RCRA and the federal regulations adopted under RCRA (40 CFR).  Totally enclosed treatment facility means a facility for the treatment of hazardous waste which is directly connected to an industrial production process and which is constructed and operated in a manner which prevents the release of any hazardous waste or any constituent thereof into the environment during treatment.  This term is defined in 40 CFR 260.10.

615e
Basis for Not Needing Federal

Permit = federal conditionally

exempt small quantity generator [generated 100 kg (approximately

27 gallons) or less] of hazardous waste in a calendar month
Y or N
1
AN
Basis for determining that a hazardous waste permit is not required under RCRA and the federal regulations adopted under RCRA (40 CFR).  See description in item 615a above.

615f
Basis for Not Needing Federal

Permit = treatment in an accumulation tank or container within 90 days for over 1000 kg/month generators and 180 or 270 days for generators of 100 to 1000 kg/month
Y or N
1
AN
Basis for determining that a hazardous waste permit is not required under RCRA and the federal regulations adopted under RCRA (40 CFR).  See description in item 615a above.

615g
Basis for Not Needing Federal

Permit = recyclable materials

are reclaimed to recover silver

or other precious metals
Y or N
1
AN
Basis for determining that a hazardous waste permit is not required under RCRA and the federal regulations adopted under RCRA (40 CFR).  See description in item 615a above.

615h
Basis for Not Needing Federal

Permit = empty container rinsing and/or treatment
Y or N
1
AN
Basis for determining that a hazardous waste permit is not required under RCRA and the federal regulations adopted under RCRA (40 CFR).  See description in item 615a above.

615i
Basis for Not Needing Federal

Permit = other
Narrative
30
AN
Description of basis for determining that a hazardous waste permit is not required under RCRA and the federal regulations adopted under RCRA (40 CFR).  See item 615a above.

616a


Residuals Management Description = discharge non-hazardous

aqueous waste to POTW or

sewer
Y or N
1
AN
Residual management options.  If any hazardous residuals are disposed or hauled offsite, indicates where the waste is sent.

616b
Residuals Management Description = discharge non-hazardous aqueous waste under a NPDES permit
Y or N
1
AN
See description in item 616a above.

616c
Residuals Management Description = dispose of non-hazardous solid waste residues at an offsite location
Y or N
1
AN
See description in item 616a above.

616d
Residuals Management Description = offsite recycling
Y or N
1
AN
See description in item 616a above.

616e
Residuals Management Description = thermal treatment
Y or N
1
AN
See description in item 616a above.

616f
Residuals Management Description = disposal to land
Y or N
1
AN
See description in item 616a above.

616g
Residuals Management Description = further treatment
Y or N
1
AN
See description in item 616a above.

616h
Residuals Management Description = other method of disposal
Narrative
30
AN
See description in item 616a above.

617
Secondary Containment

Installation Date
YYYYMMDD
8
D
Date secondary containment installed.

   Chapter 4   HAZARDOUS WASTE

C. Onsite Tiered Permitting - Waste and Treatment Process Combinations

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE
INFORMATION DESCRIPTION

606
Unit ID Number

18
AN
Unique identification number for unit. The units can be numbered sequentially or by any other system as long as the numbers are not repeated or duplicated.

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to permit cross linking of data. County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

INFORMATION DESCRIPTION - Conditionally Exempt Small Quantity Treatment (CESQT) Waste and Treatment Process Combinations.  These are all of the eligible waste streams and treatment processes that are available within the tier.  NOTE: PBR codes are the same as CESQT.

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE

627-1a
Aqueous Waste ‑ Hexavalent Chromium Reduction
Y or N
1
AN

627-2a
Aqueous Waste w/Metals ‑ pH Adjustment / Neutralization
Y or N
1
AN

627-2b
Aqueous Waste w/Metals ‑ Precipitation or Crystallization
Y or N
1
AN

627-2c
Aqueous Waste w/Metals ‑ Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

627-2d
Aqueous Waste w/Metals ‑ Ion Exchange
Y or N
1
AN

627-2e
Aqueous Waste w/Metals ‑ Reverse Osmosis
Y or N
1
AN

627-2f
Aqueous Waste w/Metals ‑ Metallic Replacement
Y or N
1
AN

627-2g
Aqueous Waste w/Metals ‑ Plating onto an Electrode
Y or N
1
AN

627-2h
Aqueous Waste w/Metals ‑ Electrodialysis
Y or N
1
AN

627-2i
Aqueous Waste w/Metals ‑ Electrowinning or Electrolytic Recovery
Y or N
1
AN

627-2j
Aqueous Waste w/Metals ‑ Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

627-2k
Aqueous Waste w/Metals ‑ Evaporation
Y or N
1
AN

627-2l
Aqueous Waste w/Metals ‑ Adsorption
Y or N
1
AN

627-3a
Aqueous Waste w/Organics (<10% Organic and <1% Volatiles) ‑ Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

627-3b
Aqueous Waste w/Organics (<10% Organic and <1% Volatiles) ‑ Adsorption
Y or N
1
AN

627-3c
Aqueous Waste w/Organics (<10% Organic and <1% Volatiles) ‑ Distillation
Y or N
1
AN

627-3d
Aqueous Waste w/Organics (<10% Organic and <1% Volatiles) ‑ Biological Process Using Microorganisms
Y or N
1
AN

627-3e
Aqueous Waste w/Organics (<10% Organic and <1% Volatiles) ‑ Photodegradation in Enclosed System
Y or N
1
AN

627-3f
Aqueous Waste w/Organics (<10% Organic and <1% Volatiles) ‑ Air Stripping or Steam Stripping
Y or N
1
AN

   Chapter 4   HAZARDOUS WASTE

C. Onsite Tiered Permitting - Waste and Treatment Process Combinations

INFORMATION DESCRIPTION - Conditionally Exempt Small Quantity Treatment (CESQT) Waste and Treatment Process Combinations.  These are all of the eligible waste streams and treatment processes that are available within the tier.  NOTE: PBR codes are the same as CESQT.

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE

627-4a
Sludges, Dusts, Solids w/Metal(s) - Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

627-4b
Sludges, Dusts, Solids w/Metal(s) - Grind, Shred, Crush, or Compact
Y or N
1
AN

627-4c
Sludges, Dusts, Solids w/Metal(s) - Drying to Remove Water
Y or N
1
AN

627-4d
Sludges, Dusts, Solids w/Metal(s) ‑ Separation by Size, Magnetism, or Density
Y or N
1
AN

627-5a
Sludges w/Alum, Gypsum, Lime, Sulfur, or Phosphate ‑ Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

627-5b
Sludges w/Alum, Gypsum, Lime, Sulfur, or Phosphate ‑ Drying to Remove Water
Y or N
1
AN

627-5c
Sludges w/Alum, Gypsum, Lime, Sulfur, or Phosphate - Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

627-6a
Special Waste (Sec. 66261.120) ‑ Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

627-6b
Special Waste (Sec. 66261.120) ‑ Drying to Remove Water
Y or N
1
AN

627-6c
Special Waste (Sec. 66261.120) ‑ Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

627-6d
Special Waste (Sec. 66261.120) ‑ Screening Based on Size
Y or N
1
AN

627-6e
Special Waste (Sec. 66261.120) ‑ Separation by Size, Magnetism, or Density
Y or N
1
AN

627-7a
Special Waste (Sec. 66261.124) ‑ Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

627-7b
Special Waste (Sec. 66261.124) ‑ Drying to Remove Water
Y or N
1
AN

627-7c
Special Waste (Sec. 66261.124) ‑ Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

627-7d
Special Waste (Sec. 66261.124) ‑ Magnetic Separation
Y or N
1
AN

627-8a
Inorganic Acid/Alkaline Waste ‑ pH Adjustment / Neutralization
Y or N
1
AN

627-9a
Soils w/Metal(s) ‑ Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

627-9b
Soils w/Metal(s) ‑ Separation by Size
Y or N
1
AN

627-9c
Soils w/Metal(s) ‑ Magnetic Separation
Y or N
1
AN

627-10a
Used Oil, Mixed Oil, Oily Water, Oil/Water Sludges - Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

627-10b
Used Oil, Mixed Oil, Oily Water, O/W Sludges ‑ Distillation
Y or N
1
AN

627-10c
Used Oil, Mixed Oil, Oily Water, O/W Sludges ‑ Neutralization
Y or N
1
AN

   Chapter 4   HAZARDOUS WASTE

C. Onsite Tiered Permitting - Waste and Treatment Process Combinations

INFORMATION DESCRIPTION - Conditionally Exempt Small Quantity Treatment (CESQT) Waste and Treatment Process Combinations.  These are all of the eligible waste streams and treatment processes that are available within the tier.  NOTE: PBR codes are the same as CESQT.

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE

627-10d
Used Oil, Mixed Oil, Oily Water, O/W Sludges ‑ Separation by Size, Magnetism, or Density
Y or N
1
AN

627-10e
Used Oil, Mixed Oil, Oily Water, O/W Sludges ‑ Reverse Osmosis
Y or N
1
AN

627-10f
Used Oil, Mixed Oil, Oily Water, O/W Sludges ‑ Biological Process Using Microorganisms
Y or N
1
AN

627-11a
Containers (< 110 Gallons) or Liners - Rinsing with Liquid
Y or N
1
AN

627-11b
Containers (< 110 Gallons) or Liners - Crush, Shred, Grind, or Puncture
Y or N
1
AN

627-12a
Multi-component Resins - Mixing per Manufacturer's Instructions
Y or N
1
AN

627-13
Wastestream & Treatment Technology Combination Certified by DTSC per 

HSC 25200.1.5
Valid Certified Technology Number
10
AN

   Chapter 4   HAZARDOUS WASTE

C. Onsite Tiered Permitting - Waste and Treatment Process Combinations

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE
INFORMATION DESCRIPTION

606
Unit ID Number

18
AN
Unique identification number for unit. The units can be numbered sequentially or by any other system as long as the numbers are not repeated or duplicated.

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to permit cross linking of data. County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

INFORMATION DESCRIPTION - Conditionally Exempt Specified Wastestreams (CESW) Waste and Treatment Process Combinations.  These are all of the elibible waste streams and Treatment processes that are available within the tier.

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE

628-1
Resins - Mixing or Curing per Manufacturer's Instructions
Y or N
1
AN

628-2
Containers (<110 Gallons) ‑ Rinse, Crush, Shred, Grind or Puncture
Y or N
1
AN

628-3
Special Waste (Sec. 66261.124) - Drying by Evaporation or Pressing to Remove Water
Y or N
1
AN

628-4
Special Waste (Sec. 66261.124) -  Magnetic Separation or Screening
Y or N
1
AN

628-8a
Gravity Settling ‑ Settling Solids w/Non-hazardous Liquid
Y or N
1
AN

628-8b
Gravity Settling ‑ Separation of Oil/Water Mix or Sludges (<25 Barrels/mo)
Y or N
1
AN

628-9
Neutralizing Acidic/Alkaline Material ‑ Certified or Educational Laboratory or

<1 Gallon/batch of Laboratory Waste
Y or N
1
AN

628-10
Hazardous Waste Treatment Carried Out in Q/C or Q/A Labs (not Offsite Facility)
Y or N
1
AN

628-11
Wastestream & Treatment Technology Combination Certified by DTSC per

HSC 25200.1.5
Valid Certified Technology Number
10
AN

628-12
Formaldehyde / Glutaraldehyde by Healthcare Facilities Using Equipment Certified by DTSC per HSC 25200.1.5
Valid Certified Technology Number
10
AN

   Chapter 4   HAZARDOUS WASTE

C. Onsite Tiered Permitting - Waste and Treatment Process Combinations

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE
INFORMATION DESCRIPTION

606
Unit ID Number

18
AN
Unique identification number for unit. The units can be numbered sequentially or by any other system as long as the numbers are not repeated or duplicated.

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to permit cross linking of data. County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

INFORMATION DESCRIPTION - Conditionally Authorized (CA) Waste and Treatment Process Combinations.  These are all of the eligible waste streams and treatment processes that are available within the tier.

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE

629-1a
Aqueous Waste w/Inorganics <1,400 ppm - Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

629-1b
Aqueous Waste w/Inorganics <1,400 ppm - Ion Exchange / Metallic Replacement
Y or N
1
AN

629-1c
Aqueous Waste w/Inorganics <1,400 ppm - Reverse Osmosis
Y or N
1
AN

629-1d
Aqueous Waste w/Inorganics <1,400 ppm - Adsorption
Y or N
1
AN

629-1e
Aqueous Waste w/Inorganics <1,400 ppm ‑ pH Adjustment
Y or N
1
AN

629-1f
Aqueous Waste w/Inorganics <1,400 ppm - Electrowinning if no Hydrochloric Acid
Y or N
1
AN

629-1g
Aqueous Waste w/Inorganics <750 ppm ‑ Hexavalent Chromium Reduction
Y or N
1
AN

629-2a
Aqueous Waste w/Organics <750 ppm ‑ Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

629-2b
Aqueous Waste w/Organics <750 ppm ‑ Adsorption
Y or N
1
AN

629-3a
Sludges, Dusts, Solids w/Metal(s) <750 ppm ‑ Filter, Gravity, Grind, Crush, or Compact
Y or N
1
AN

629-3b
Sludges, Dusts, Solids w/Metal(s) <750 ppm ‑ Drying to Remove Water
Y or N
1
AN

629-3c
Sludges, Dusts, Solids w/Metal(s) <750 ppm ‑ Separation by Size, Magnetism, or Density
Y or N
1
AN

629-4a
Sludges w/ Alum, Gypsum, Lime, Phosphate <5,000 Gal / 45,000 Lb/mo ‑ Drying to Remove Water
Y or N
1
AN

629-4b
Sludges w/ Alum, Gypsum, Lime, Phosphate < 5,000 Gal / 45,000 Lb/mo ‑ Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

629-5a
Special Waste (Sec. 66261.120) w/<750 ppm - Drying to Remove Water
Y or N
1
AN

629-5b
Special Waste (Sec. 66261.120) w/<750 ppm - Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

629-5c
Special Waste (Sec. 66261.120) w/<750 ppm - Screening Based on Size
Y or N
1
AN

629-5d
Special Waste (Sec. 66261.120) w/<750 ppm - Separation by Size, Magnetism, or Density
Y or N
1
AN

   Chapter 4   HAZARDOUS WASTE

C. Onsite Tiered Permitting - Waste and Treatment Process Combinations

INFORMATION DESCRIPTION - Conditionally Authorized (CA) Waste and Treatment Process Combinations.  These are all of the eligible waste streams and treatment processes that are available within the tier.

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE

629-6a
Special Waste (Sec. 66261.124) w/<750 ppm - Drying to Remove Water
Y or N
1
AN

629-6b
Special Waste (Sec. 66261.124) w/<750 ppm - Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

629-6c
Special Waste (Sec. 66261.124) w/<750 ppm - Magnetic Separation
Y or N
1
AN

629-7a
Soils w/Metals <45,000 Lb / < 5000 Gal/mo - Separation by Size
Y or N
1
AN

629-7b
Soils w/Metals <45,000 Lb / < 5000 Gal/mo - Magnetic Separation
Y or N
1
AN

629-8a
Oily Water, Oil/Water Sludges ‑ Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

629-8b
Oily Water, Oil/Water Sludges ‑ Separation by Size, Magnetism, or Density
Y or N
1
AN

629-8c
Oily Water, Oil/Water Sludges - Reverse Osmosis
Y or N
1
AN

629-9a
Alkaline/Acidic Waste <10% by Weight  - Neutralization
Y or N
1
AN

629-9b
Alkaline/Acidic Waste >10% by Weight (<500 Gallons/batch) - Neutralization 
Y or N
1
AN

629-12
Wastestream & Treatment Technology Combination Certified by DTSC per

HSC 25200.1.5
Valid Certified Technology Number
10
AN

   Chapter 4   HAZARDOUS WASTE

C. Onsite Tiered Permitting - Waste and Treatment Process Combinations

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE
INFORMATION DESCRIPTION

606
Unit ID Number

18
AN
Unique identification number for unit. The units can be numbered sequentially or by any other system as long as the numbers are not repeated or duplicated.

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to permit cross linking of data. County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

INFORMATION DESCRIPTION - Permit by Rule (PBR) Waste and Treatment Process Combinations.  These are all of the eligible waste streams and treatment processes that are available within the tier.  NOTE: PBR codes are the same as CESQT.

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE

630-1a
Aqueous Waste ‑ Hexavalent Chromium Reduction
Y or N
1
AN

630-2a
Aqueous Waste w/Metals ‑ pH Adjustment / Neutralization
Y or N
1
AN

630-2b
Aqueous Waste w/Metals ‑ Precipitation or Crystallization
Y or N
1
AN

630-2c
Aqueous Waste w/Metals ‑ Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

630-2d
Aqueous Waste w/Metals ‑ Ion Exchange
Y or N
1
AN

630-2e
Aqueous Waste w/Metals ‑ Reverse Osmosis
Y or N
1
AN

630-2f
Aqueous Waste w/Metals ‑ Metallic Replacement
Y or N
1
AN

630-2g
Aqueous Waste w/Metals ‑ Plating onto an Electrode
Y or N
1
AN

630-2h
Aqueous Waste w/Metals ‑ Electrodialysis
Y or N
1
AN

630-2i
Aqueous Waste w/Metals ‑ Electrowinning or Electrolytic Recovery
Y or N
1
AN

630-2j
Aqueous Waste w/Metals ‑ Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

630-2k
Aqueous Waste w/Metals ‑ Evaporation
Y or N
1
AN

630-2l
Aqueous Waste w/Metals ‑ Adsorption
Y or N
1
AN

630-3a
Aqueous Waste w/Organics (<10% Organic and <1% Volatiles) ‑ Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

630-3b
Aqueous Waste w/Organics (<10% Organic and <1% Volatiles) ‑ Adsorption
Y or N
1
AN

630-3c
Aqueous Waste w/Organics (<10% Organic and <1% Volatiles) ‑ Distillation
Y or N
1
AN

630-3d
Aqueous Waste w/Organics (<10% Organic and <1% Volatiles) ‑ Biological Process Using Microorganisms
Y or N
1
AN

630-3e
Aqueous Waste w/Organics (<10% Organic and <1% Volatiles) ‑ Photodegradation in Enclosed System
Y or N
1
AN

630-3f
Aqueous Waste w/Organics (<1% Volatiles) - Air Stripping or Steam Stripping
Y or N
1
AN

   Chapter 4   HAZARDOUS WASTE

C. Onsite Tiered Permitting - Waste and Treatment Process Combinations

 INFORMATION DESCRIPTION - Permit by Rule (PBR) Waste and Treatment Process Combinations.  These are all of

 the eligible waste streams and treatment processes that are available within the tier.  NOTE: PBR codes are the

 same as CESQT.

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE

630-4a
Sludges, Dusts, Solids w/Metal(s) ‑ Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

630-4b
Sludges, Dusts, Solids w/Metal(s) ‑ Grind, Shred, Crush, or Compact
Y or N
1
AN

630-4c
Sludges, Dusts, Solids w/Metal(s) ‑ Drying to Remove Water
Y or N
1
AN

630-4d
Sludges, Dusts, Solids w/Metal(s) ‑ Separation by Size, Magnetism, or Density
Y or N
1
AN

630-5a
Sludges w/Alum, Gypsum, Lime, Sulfur, or Phosphate ‑ Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

630-5b
Sludges w/Alum, Gypsum, Lime, Sulfur, or Phosphate ‑ Drying to Remove Water
Y or N
1
AN

630-5c
Sludges w/Alum, Gypsum, Lime, Sulfur, or Phosphate - Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

630-6a
Special Waste (Sec. 66261.120) ‑ Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

630-6b
Special Waste (Sec. 66261.120) ‑ Drying to Remove Water
Y or N
1
AN

630-6c
Special Waste (Sec. 66261.120) ‑ Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

630-6d
Special Waste (Sec. 66261.120) ‑ Screening Based on Size
Y or N
1
AN

630-6e
Special Waste (Sec. 66261.120) ‑ Separation by Size, Magnetism, or Density
Y or N
1
AN

630-7a
Special Waste (Sec. 66261.124) ‑ Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

630-7b
Special Waste (Sec. 66261.124) ‑ Drying to Remove Water
Y or N
1
AN

630-7c
Special Waste (Sec. 66261.124) ‑ Phase Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

630-7d
Special Waste (Sec. 66261.124) ‑ Magnetic Separation
Y or N
1
AN

630-8a
Inorganic Acid/Alkaline Waste ‑ pH Adjustment / Neutralization
Y or N
1
AN

630-9a
Soils w/Metal(s) - Chemical Stabilization Using Silicates or Cementitious Reactions
Y or N
1
AN

630-9b
Soils w/Metal(s) - Separation by Size
Y or N
1
AN

630-9c
Soils w/Metal(s) ‑ Magnetic Separation
Y or N
1
AN

630-10a
Used Oil, Mixed Oil, Oily Water, Oil/W Sludges - Separation by Filter, Centrifuge, or Gravity Settling
Y or N
1
AN

630-10b
Used Oil, Mixed Oil, Oily Water, O/W Sludges ‑ Distillation
Y or N
1
AN

630-10c
Used Oil, Mixed Oil, Oily Water, O/W Sludges ‑ Neutralization
Y or N
1
AN

   Chapter 4   HAZARDOUS WASTE

C. Onsite Tiered Permitting - Waste and Treatment Process Combinations

INFORMATION DESCRIPTION - Permit by Rule (PBR) Waste and Treatment Process Combinations.  These are all of the eligible waste streams and treatment processes that are available within the tier.  NOTE: PBR codes are the same as CESQT.

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE

630-10d
Used Oil, Mixed Oil, Oily Water, O/W Sludges ‑ Separation by Size, Magnetism, or Density
Y or N
1
AN

630-10e
Used Oil, Mixed Oil, Oily Water, O/W Sludges ‑ Reverse Osmosis
Y or N
1
AN

630-10f
Used Oil, Mixed Oil, Oily Water, O/W Sludges ‑ Biological Process Using Microorganisms
Y or N
1
AN

630-11a
Containers (< 110 Gallons) or Liners - Rinsing with Liquid
Y or N
1
AN

630-11b
Containers (< 110 Gallons) or Liners ‑ Crush, Shred, Grind, or Puncture
Y or N
1
AN

630-12a
Multi‑component Resins ‑ Mixing per Manufacturer's Instructions
Y or N
1
AN

630-13
Wastestream & Treatment Technology Combination Certified by DTSC per

HSC 25200.1.5
Valid Certified Technology Number
10
AN

   Chapter 4   HAZARDOUS WASTE

C. Onsite Tiered Permitting - Waste and Treatment Process Combinations

ID
ELEMENT
EDIT CRITERIA / CODE
LENGTH
TYPE
INFORMATION DESCRIPTION

606
Unit ID Number

18
AN
Unique identification number for unit. The units can be numbered sequentially or by any other system as long as the numbers are not repeated or duplicated.

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to permit cross linking of data. County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

INFORMATION DESCRIPTION - Conditionally Exempt - Limited (CEL) Waste and Treatment Process Combinations.  These are all of the eligible waste streams and treatment processes that are available within the tier.

ID
ELEMENT
EDIT CIRTERIA / CODE
LENGTH
TYPE

631-1
Aerosol Cans - Puncturing, Draining or Crushing (at Ambient Temperature) Using Wastestream & Treatment Technology Combination Certified by DTSC per

HSC 25200.1.5
Valid Certified Technology Number
10
AN

631-2a
Used Oil Mixed w/Water, Oily Water (no Gasoline and <2% Diesel) - Gravity Separation
Y or N
1
AN

631-2b
Used Oil Mixed w/Water, Oily Water (no Gasoline and <2% Diesel) - Centrifuge
Y or N
1
AN

631-2c
Used Oil Mixed w/Water, Oily Water (no Gasoline and <2% Diesel) - Membrane Technology
Y or N
1
AN

631-2d
Used Oil Mixed w/Water, Oily Water (no Gasoline and <2% Diesel) - Heating to Temperature not above 20 Deg F below Flashpoint of the Used Oil Component of the Mixture at Atmospheric Pressure.
Y or N
1
AN

631-2e
Used Oil Mixed w/Water, Oily Water (no Gasoline and <2% Diesel) - Addition of Demulsifiers
Y or N
1
AN

   Chapter 4   HAZARDOUS WASTE

2. Certification Of Financial Assurance

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

2
EPA ID Number
12 digit identifier beginning with CA
12
AN
EPA Identification number for businesses that generate, recycle, or treat hazardous waste.  For facilities in California, the number usually starts with the letters CA.  The number can be obtained from the Telephone Information Center at (916)324-1781, (800) 61-TOXIC or (800) 618-6942.

3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of the business.

700
Certification Status
a = initial certification

b = amended certification

c = annual certification
1
AN
Reason for submitting certification.



701
Type of Operation (Hazardous Waste Financial Assurance)
a = PBR-FTU

b = CA

c = other
1
AN
Type of operation.

702
Estimated Closure Costs

12
AN
Estimated closure costs in dollars.  This section requires a written estimate of the cost of closing each treatment unit.  The estimated closure cost may be either the actual cost or the estimated cost when using your own staff and/or personal equipment.  The closure cost estimate may take into account any salvage value that may be realized from the sale of wastes, facility structure or equipment, land or other facility assets.  For PBR only: If you have operated under PBR for less than 30 days in any calendar year, do not enter any dollar amount.  You qualify for an exemption.  If eligible for the exemption, enter EXEMPT in this section.

703
Exemption From

Financial Assurance = cost estimate < $10,000
Y or N
1
AN
Indicates if reason for exemption from financial assurance requirements is total closure cost estimate not more than $10,000.  A model letter using the required certifications must be submitted to claim this exemption.

704
Exemption From

Financial Assurance = other
Narrative
30
AN
Description of other reason for exemption from financial assurance requirements.

705
Exemption From

Financial Assurance = TTU owner / operator

and operated < 30 days/yr
Y or N
1
AN
Indicates if reason for exemption from financial assurance requirements is for owner or operator under PBR only and operating no more than thirty days in any calendar year.

706
Requirement for

Financial Assurance
Y or N
1
AN
Indicates if financial assurance is required.

707
Date of Closure Assurance Mechanism
YYYYMMDD
8
D
Effective date of closure assurance mechanism.

708
Mechanism ID Number

25
AN
Number of closure assurance mechanism if applicable, for example, the insurance policy number.

709
Closure Assurance Mechanism
a = closure trust fund

b = surety bond

c = closure letter of credit

d = closure insurance

e = financial test and

      corporate guarantee

f  = alternative mechanism 

g = multiple financial

      mechanisms

h = certificate of deposit

I  = savings account
1
AN
Type of financial mechanism established to provide closure cost assurance.  Eligible types for FTUs and TTUs are contained in CCR, Title 22, Section 67450.13(a)(5).

a = closure trust fund, as provided in CCR, Title 22, Section

      66265.143(a)  (Attach DTSC Form 1154)

b = surety bond guaranteeing payment into a

      closure trust fund, as described in CCR, Title 22, Section

      66265.143(b) (Attach DTSC Form 1155 or 1156

      with DTSC Form 1154)

c = closure letter of credit, as described in CCR, Title 22,
      Section 66265.143(c)  (Attach DTSC Form

      1157)

d = closure insurance, as described in CCR, Title 22, Section

      66265.143(d) (Attach DTSC Form 1158)

e = financial test and corporate guarantee for

      closure, as described in CCR, Title 22,  Section

      66265.143(e) (Attach either DTSC Form 1159 or

      1173)

f  = alternative mechanism for closure costs, as

      described in CCR, Title 22,  Section 67450.13(c) (No

      form)

g = multiple financial mechanisms for closure costs,

      as described in CCR, Title 22,  Section 66265.143(g) (No

      form)

h = certificate of deposit, as described in Section

      3-104 (j) of the Uniform Commercial Code

      (No form)

i  = savings account, as described in Section

      4-104(a) of the Uniform Commercial Code  (No

      form)

These mechanisms require use of the additional DTSC Financial Assurance forms referenced above. These forms are available from the CUPA or the DTSC Regional Office.

710
Financial Institution

or Surety Name

35
AN
Name of the financial institution, insurance company, surety company, or other appropriate organization used to establish the closure financial assurance.  Indicate your company if you are using a corporate guarantee and financial test.

711
Financial Institution

or Surety Address
Postal standard:

2 lines, 35 characters
70
AN
Address of financial institution, insurance company, surety company, or other appropriate organization used to establish the closure financial assurance.

712
Financial Institution

or Surety City
Postal standard
20
AN
City of financial institution, insurance company, surety company, or other appropriate organization used to establish the closure financial assurance.

713
Financial Institution

or Surety State
Valid 2-digit state code
2
AN
State of financial institution, insurance company, surety company, or other appropriate organization used to establish the closure financial assurance.

714
Financial Institution

or Surety Zip Code
Postal standard
9
AN
Zip code of financial institution, insurance company, surety company, or other appropriate organization used to establish the closure financial assurance.

715
Signer of Certification
a = owner

b = operator
1
AN
Indicates if signer of certification is the owner or operator of the facility.

716
Date Certified

(Financial Assurance)
YYYYMMDD
8
D
Date the document was signed.

717
Owner/ Operator Name (Financial Assurance)

35
AN
Full name of business owner/operator, or officially designated representative of the owner/operator. The person signing this page must be an owner or officer of the company who is authorized to make decisions for the facility and who has operational control.  The authorized signatory must be completed as specified in CCR, Title 22,  Section 66270.11. In most companies, this is not the environmental compliance or technical staff.  The signer certifies to a belief that all the information submitted is accurate and complete.

718
Owner/ Operator Title (Financial Assurance)

35
AN
Title of person signing the page.

   Chapter 4   HAZARDOUS WASTE

4. Remote Waste Consolidation Site Annual Notification

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

2
EPA ID Number
12 digit identifier beginning with CA
12
AN
EPA Identification number for businesses that generate, recycle, or treat hazardous waste.  For facilities in California, the number usually starts with the letters CA.  The number can be obtained from the Telephone Information Center at

(916) 324-1781, (800) 61-TOXIC or (800) 618-6942.

3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of business.

720
Notification Status

(Remote Consolidation)
a = initial

b = revised

c = annual
1
AN
Reason for submitting notification.

721
Address

(Remote Consolidation)
Postal standard:

2 lines, 35 characters
70
AN
Street address or legal description of consolidation site.

722
City (Remote Consolidation)
Postal standard
20
AN
City of consolidation site.

723
Zip Code

(Remote Consolidation)
Postal standard
9
AN
Zip Code of consolidation site.

724
Description Of Remote Consolidation Location(s)
Narrative
150
AN
Description of type of remote location(s) and sources(s) from which non-RCRA hazardous waste will be collected.

725
Description Of

Waste(s) Collected
Narrative
150
AN
Description of type of hazardous waste that may be collected.

726
Onsite Hazardous

Waste Treatment
Y or N
1
AN
Indicates if hazardous waste is treated at consolidation site.

727
Estimated Monthly

Volume Consolidated

10
N
Estimated monthly total volume consolidated.

728
Units

(Remote Consolidation)
a = pounds

b = gallons
1
AN
Units for estimated monthly total volume consolidated.

729a
Basis For Not Needing

A Federal Permit

(Remote Consolidation) =

non-RCRA hazardous waste
Y or N
1
AN
Indicates if activity is exempt from, or not otherwise regulated by, the federal Resource Conservation and Recovery Act (RCRA).

729b
Basis For Not Needing

A Federal Permit

(Remote Consolidation) = 

other
Narrative
60
AN
Description of other reason(s) activity is not subject to permitting requirements under federal law.

730
Date Certified

(Remote Consolidation)
YYYYMMDD
8
D
Date the document was signed.

731
Owner/Operator Name (Remote Consolidation)

35
AN
Full name of business owner/operator, or officially designated representative of the owner/operator. The person signing this page must be an owner or officer of the company who is authorized to make decisions for the facility and who has operational control.  In most companies, this is not the environmental compliance or technical staff.  The signer certifies to a belief that all the information submitted is accurate and complete.

732
Owner/Operator Title

(Remote Consolidation)

35
AN
Title of person signing the page.

   Chapter 4   HAZARDOUS WASTE

F. Hazardous Waste Tank Closure Certification

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

 1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

3
Business Name

70
AN
Full legal name of the business.

740
Tank Owner Name (Closure)

35
AN
Name of tank owner, if different from business owner on Business Owner/Operator page.

741
Tank Owner Address (Closure)
Postal standard: 

2 lines, 35 characters
70
AN
Street or mailing address of tank owner, if different from business owner.

742
Tank Owner City (Closure)
Postal standard
20
AN
City of tank owner, if different from business owner.

743
Tank Owner State (Closure)
Postal standard
2
AN
State of tank owner, if different from business owner.

744
Tank Owner Zip Code (Closure)
Postal standard
9
AN
Zip code of tank owner, if different from business owner.

745
Tank ID # 1

(Closure)

6
AN
Owner's tank ID # for first tank closed, if there is a tank number used by owner to identify the tank. Unique identifier of tank at site.

746a
Concentration of Flammable Vapor 1 = top
4 digits with floating decimal
5
AN
Concentration of flammable vapor for first tank closed taken at top of tank.

746b
Concentration of Flammable Vapor 1 = center
4 digits with floating decimal
5
AN
Concentration of flammable vapor for first tank closed taken at center of tank.

746c
Concentration of Flammable Vapor 1 = bottom
4 digits with floating decimal
5
AN
Concentration of flammable vapor for first tank closed taken at bottom of tank.

747a
Concentration of

Oxygen 1 = top
4 digits with floating decimal
5
AN
Concentration of oxygen for first tank closed taken at top of tank.

747b
Concentration of

Oxygen 1 = center
4 digits with floating decimal
5
AN
Concentration of oxygen for first tank closed taken at center of tank.

747c
Concentration of

Oxygen 1 = bottom
4 digits with floating decimal
5
AN
Concentration of oxygen for first tank closed taken at bottom of tank.

748
Tank ID # 2

(Closure)

6
AN
Owner's tank ID # for second tank closed, if there is a tank number used by owner to identify the tank.  Unique identifier of tank at site.

749a
Concentration of Flammable Vapor 2 = top
4 digits with floating decimal
5
AN
Concentration of flammable vapor for second tank closed taken at top of tank.

749b
Concentration of Flammable Vapor 2 = center
4 digits with floating decimal
5
AN
Concentration of flammable vapor for second tank closed taken at center of tank.

   Chapter 4   HAZARDOUS WASTE

F. Hazardous Waste Tank Closure Certification

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

749c
Concentration of Flammable Vapor 2 = bottom
4 digits with floating decimal
5
AN
Concentration of flammable vapor for second tank closed taken at bottom of tank.

750a
Concentration of

Oxygen 2 = top
4 digits with floating decimal
5
AN
Concentration of oxygen for second tank closed taken at top of tank.

750b
Concentration of

Oxygen 2 = center
4 digits with floating decimal
5
AN
Concentration of oxygen for second tank closed taken at center of tank.

750c
Concentration of

Oxygen 2 = bottom
4 digits with floating decimal
5
AN
Concentration of oxygen for second tank closed taken at bottom of tank.

751
Tank ID # 3

(Closure)

6
AN
Owner's tank ID # for third tank closed, if there is a tank number used by owner to identify the tank. Unique identifier of tank at site.

752a
Concentration of Flammable Vapor 3 = top
4 digits with floating decimal
5
AN
Concentration of flammable vapor for third tank closed taken at top of tank.

752b
Concentration of Flammable Vapor 3 = center
4 digits with floating decimal
5
AN
Concentration of flammable vapor for third tank closed taken at center of tank.

752c
Concentration of Flammable Vapor 3 = bottom
4 digits with floating decimal
5
AN
Concentration of flammable vapor for third tank closed taken at bottom of tank.

753a
Concentration of

Oxygen 3 = top
4 digits with floating decimal
5
AN
Concentration of oxygen for third tank closed taken at top of tank.

753b
Concentration of

Oxygen 3 = center
4 digits with floating decimal
5
AN
Concentration of oxygen for third tank closed taken at center of tank.

753c
Concentration of

Oxygen 3 = bottom
4 digits with floating decimal
5
AN
Concentration of oxygen for third tank closed taken at bottom of tank.

If more than three tanks are closed, complete additional copies of the Hazardous Waste Tank Closure Certification page.

754
Certifier Name

(Tank Closure)

35
AN
Full name of person signing the page.

755
Certifier Title 

(Tank Closure)

35
AN
Title of person signing the page.

756
Certifier Address

(Tank Closure)
Postal standard:

2 lines, 35 characters
70
AN
Address of person signing the page.

757
Certifier City

(Tank Closure)
Postal standard
20
AN
City of person signing the page.

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

758
Certifier Phone

(Tank Closure)
Area code + 7 digit phone number + extension
15
AN
Phone number of person signing the page.

759
Date Certified

(Tank Closure)
YYYMMDD
8
D
Date the document was signed.

   Chapter 4   HAZARDOUS WASTE

F. Hazardous Waste Tank Closure Certification

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

760
Certifier Represents

Local Agency
Y or N
1
AN
Indicates if person certifying the tank is a representative of the CUPA, authorized agency, or LIA.

761
Name of Local Agency
Narrative
35
AN
Name of CUPA, authorized agency, or LIA.

762a
Affiliation of Certifying Person = Certified Industrial Hygienist
Y or N
1
AN
Certification, license or organization which the certifier holds or to which the certifier belongs, if the person does not represent the CUPA.

762b
Affiliation of Certifying Person = Certified

Safety Professional
Y or N
1
AN
Certification, license or organization which the certifier holds or to which the certifier belongs, if the person does not represent the CUPA.

762c
Affiliation of Certifying Person = Certified 

Marine Chemist
Y or N
1
AN
Certification, license or organization which the certifier holds or to which the certifier belongs, if the person does not represent the CUPA.

762d
Affiliation of Certifying Person = Registered Environmental Health Specialist
Y or N
1
AN
Certification, license or organization which the certifier holds or to which the certifier belongs, if the person does not represent the CUPA.

762e
Affiliation of Certifying Person = Professional Engineer
Y or N
1
AN
Certification, license or organization which the certifier holds or to which the certifier belongs, if the person does not represent the CUPA.

762f
Affiliation of Certifying Person = Class II Registered Environmental Assessor
Y or N
1
AN
Certification, license or organization which the certifier holds or to which the certifier belongs, if the person does not represent the CUPA.

762g
Affiliation of Certifying Person = Licensed Contractor
Y or N
1
AN
Certification, license or organization which the certifier holds or to which the certifier belongs, if the person does not represent the CUPA.

763
Tank Held Flammable or Combustible Materials
Y or N
1
AN
Indicates if tank previously held flammable or combustible materials.  If yes, the tank interior atmosphere shall be re-checked with a combustible gas indicator prior to work being conducted on the tank.

764
Management Instructions
Narrative
150
AN
Certifier's tank management instructions for scrap dealer, disposal facility, etc.

Chapter 5

UP Information Collection and Reporting Standards Unified Program Data Dictionary--CUPA Section

Chapter 5 UP INFORMATION COLLECTION AND REPORTING STANDARDS UNIFIED PRGRAM DATA DICTIONARY – CUPA SECTION

1.
COMPLIANCE ACTIVITY INFORMATION)

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

2
EPA ID Number
12 digit identifier beginning with CA
12
AN
EPA identification number for businesses that generate, recycle, or treat hazardous waste.  For facilities in California, the number should start with the letters CA.  If the handler is regulated under Federal RCRA requirements, this ID must be the U.S. EPA identification number.

3
Business Name
Postal standard:

2 lines, 35 character
70
AN
Full legal name of business.

900
RCRA Large Quantity Generator (LQG) of Hazardous Waste
Y or N
1
AN
Indicates if facility generates  1000 kg of RCRA hazardous waste in a calendar month.  Identification is based on the business notification of LQG activity to U.S. EPA.  If the designation is incorrect, the CUPA cannot change the designation unless the business notifies U.S. EPA.

901
Generator of Solely California Hazardous Waste
Y or N
1
AN
Indicates if facility generates solely California hazardous waste and does not generate any RCRA waste.

902
CalARP Program: Stationary Source
Y or N
1
AN
Indicates if facility is a stationary source as defined by the CalARP program.

903
CalARP Program: Multiple Stationary Sources
Y or N
1
AN
Indicates if business operates multiple locations in this CUPA jurisdiction that are stationary sources as defined by the CalARP program.

904
CalARP Program: RMP Waiver Determination
Y or N
1
AN
Indicates if the CUPA has waived the requirement for a Risk Management Plan for

this stationary source (a RMP waiver).

Chapter 5 UP INFORMATION COLLECTION AND REPORTING STANDARDS UNIFIED PRGRAM DATA DICTIONARY – CUPA SECTION

2.
INSPECTION INFORMATION (one record for each facility for each program element and inspection date)

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of business.

905
Program Element
a = Hazardous Materials Release         Response Plans (HMRRP)

b = California Accidental Release         Prevention (CalARP)

c = Undergound Storage Tank              (UST)

d = Spill Prevention Control and           Countermeasures (SPCC) /            Aboveground Storage Tank

e = Hazardous Waste Generator

f  = Hazardous Waste Large                 Quantity Generator (LQG)               (subset of Hazardous Waste           Generator)

g = Hazardous Waste Recycler

h = Permit by Rule (PBR)

i  = Conditionally Authorized (CA)         (only available if PBR is not            used)

j  = Conditionally Exempt (CE)              (only available if PBR and CA         are not used)

k = Household Hazardous Waste          (HHW)
1
AN
Program element inspected.  Enter one program element only.  For example, a facility inspected for compliance with UST, hazardous waste and hazardous materials storage regulations would have three records, one for each program element.  Each record would have a unique set of violations, even though all inspections took place on the same date.  For Tiered Permitting options enter the highest tier.

906
Inspection Date
YYYYMMDD
8
D
Date of completion of inspection.

907a
Inspection Type = Routine Inspection


Y or N


1
AN
Indicates if inspection is routine.  A routine inspection is a regularly scheduled inspection to evaluate compliance.   Does not include follow-up inspections.

907b
Inspection Type = Other
Y or N
1
AN
Indicates if inspection is not a routine inspection.  Other inspections include complaint investigations, closure, release investigations, tank installation and/or removal oversight, tank cleaning, and follow-up enforcement inspections, or other inspections that may be in addition to a regularly scheduled inspection.  This includes verification inspections for owners/operators of aboveground storage tanks having to prepare a spill prevention control and countermeasure plan.  It does not include regularly scheduled inspections, field or site visits whose principle purpose is informational or educational, pollution prevention education, or visits needed to verify administrative information or orient new owners or operators.  A complaint inspection is a service request originating from any outside party, including the public, that initiates a site visit outside of the routine inspection cycle.

908
CalARP Audit


Y or N
1
AN
Indicates if site visit is an audit of a CalARP Risk Management Plan for a stationary source.

909a
Inspection Category = Single Program
Y or N
1
AN
Indicates if inspection is a single program inspection.  Inspectors perform single program inspections alone.  If inspection is a single program inspection, do not enter codes for any other inspection category.

909b
Inspection Category = Combined Routine Inspection
Y or N
1
AN
Indicates if inspection is a combined routine inspection.  Combined routine inspections are regularly scheduled inspections to evaluate compliance conducted by one inspector for more than one program element.  This does not include other inspections performed outside the routine inspection cycle.

909c
Inspection Category = Joint Inspection
Y or N
1
AN
Indicates if inspection is a joint inspection.  Joint inspections may be routine or other inspections.  Joint inspections are conducted by more than one inspector from different Unified Program agencies within a CUPA, for more than one program element.

909d
Inspection Category = Integrated or Multi-media Inspections
Y or N
1
AN
Indicates if inspection is integrated or multi-media.  Integrated or multi-media inspections may be routine or other inspections.  Integrated inspections are conducted by one or more inspectors for the Unified Program and other programs not in the Unified Program.  Multi-media inspections are conducted by one or more inspectors for more than one medium, such as air, water, or soil.

910
Number of Class I Violations

2
N
For hazardous waste generators, number of Class I violations.  A Class I violation means a deviation that represents a significant threat to human health or safety or the environment because of the volume of the waste, the relative hazardousness of the waste, or the proximity of the population at risk.  The deviation must be significant enough that it could result in releases of hazardous waste or constituents to the environment, hazardous waste failing to be delivered to an authorized hazardous waste facility, failure to detect releases of hazardous waste or constituents, inadequate financial resources in the case of releases of hazardous waste or constituents, or inadequate financial resources to pay for facility closure, perform emergency cleanup operations or other corrective actions.  A Class I violation is also a deviation that is a chronic violation or committed by a recalcitrant violator.  A Class I violation is typically one that is referred to the District Attorney or City Attorney for formal enforcement action.  Sanctions are typically imposed for failure to correct the violation.  Class I violations are defined in the Health and Safety Code (HSC) section 25110.8.5.

911
Number of Class II Violations

2
N
For hazarous waste generators, number of Class II violations.  A Class II violation means a deviation that is not a Class I violation.  This count includes violations which would be considered minor, but are knowing, willful, or intentional, or enable the violator to benefit economically from noncompliance, either by reduced costs or competitive advantage.  Do not include minor violations in this count.  Class II violations are defined in 22 California Code of Regulations (CCR) 66260.10.

912
Number of Minor Violations

2
N
For hazardous waste generators, number of minor violations.  A minor violation means a deviation from any regulation, standard, requirement, or permit condition, that is not a Class I violation.  Exclude from this count all violations where the violation is knowing, willful, or intentional, or enables the violator to benefit economically from noncompliance, either by reduced costs or competitive advantage.  These are counted as Class II violations.  Also exclude any violation that is a chronic violation or that is committed by a reclacitrant violator, since these are counted as Class I violations.  A minor violation is defined in HSC 25117.6.

913
Number of Other Violations

2
N
For non-hazardous waste program elements, number of other violations.  Other violations are those that are not hazardous waste violations.

Chapter 5 UP INFORMATION COLLECTION AND REPORTING STANDARDS UNIFIED PRGRAM DATA DICTIONARY – CUPA SECTION

3.
ENFORCEMENT INFORMATION (one record for each facility for each program element and enforcement action)

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of business.

902
Program Element
a = Hazardous Materials Release             Response Plans (HMRRP)

b = California Accidental Release             Prevention (CalARP)

c = Undergound Storage Tank                  (UST)

d = Spill Prevention Control and                Countermeasures (SPCC) /                Aboveground Storage Tank

e = Hazardous Waste Generator

f  = Hazardous Waste Large                     Quantity Generator (LQG)                   (subset of Hazardous Waste               Generator)

g = Hazardous Waste Recycler

h = Permit by Rule (PBR)

I  = Conditionally Authorized (CA)             (only available if PBR is not                used)

j  = Conditionally Exempt (CE)                  (only available if PBR and CA             are not used)

k = Household Hazardous Waste              (HHW) - Fixed
2
AN
Program element inspected.  Enter one program element only.  For example, a facility inspected for compliance with UST, hazardous waste and hazardous materials storage regulations would have three records, one for each program element.  Each record would have a unique set of violations, even though all inspections took place on the same date.  For Tiered Permitting options enter the highest tier.

903
Inspection Date
YYYYMMDD
8
D
Date of completion of inspection.

914
Type of Enforcement Action
a = Informal action

b = Referral to State agency

c = Formal order

d = Referral to Attorney General or           District Attorney
1
AN
Type of enforcement action.  Informal actions are actions that are not formal actions.  An informal enforcement action notifies the business of non-compliance and establishes a date by which the non-compliance is to be corrected.  Informal actions are made by a written document including, but not limited to, a letter or notice of violation.  Informal actions do not convey sanctions.   A formal order is an enforceable order or agreement which mandates compliance.  Examples include administrative orders and referrals for civil and/or criminal actions.  Sanctions are imposed for failure to comply.  If more than one enforcement action is taken, the type and date of each action should be recorded.

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

915
Date of Enforcement Action
YYYYMMDD
8
D
Date the enforcement action is taken.  The date of enforcement action is the date the order is sent to the business, a final referral is made to the Attorney General, District Attorney or County Attorney, the complaint is filed with the court, or the date the administrative order is issued.  If more than one enforcement action is taken, the type and date of each action should be recorded.

916
Type of Formal Enforcement Action
a = Administrative

b = Civil

c = Criminal
1
AN
Type of formal enforcement action. 

917
Date Returned to Compliance
YYYYMMDD
8
D


Date physical compliance was determined by the CUPA for all violations identified during the inspection.  This may not be based on a site visit, but is the date compliance was verified.  It may be based on correspondence received from the regulated business.

918
Docket Number
.
13
AN
Number assigned by the court for civil and criminal actions.

919
Final Fine or Penalty Assessed

8
AN
Dollar amount of fine or penalty assessed.  This is the final monetary penalty or fine assessed via court or administrative order, or the amount agreed upon in a formal legal settlement.  It is based on the value of fines / penalties excluding costs.  Round to nearest whole number.  Do not use decimal places.  Note the fine or penalty is by program element for each enforcement action at each facility, when available.

920
Fine or Penalty Collected

8
AN
Dollar amount of final fine or penalty actually collected by the CUPA.  Round to nearest whole number.  Do not use decimal places.

921
Date Fine or Penalty Collected
YYYYMMDD
8
D
Date when the final fine or penalty was completely collected.

Chapter 5 UP INFORMATION COLLECTION AND REPORTING STANDARDS UNIFIED PRGRAM DATA DICTIONARY – CUPA SECTION

4.
TIERED PERMITTING RELEASE INFORMATION

ID
ELEMENT
EDIT CRITERIA / CODES
LENGTH
TYPE
INFORMATION DESCRIPTION

1
Facility ID Number
2 AN county

3 AN jurisdiction

6 AN facility number
11
AN
Number to allow cross linking of data.  County and jurisdiction number from tax code list.  This number is assigned by the CUPA.  This is the unique number which identifies the facility.

3
Business Name
Postal standard:

2 lines, 35 characters
70
AN
Full legal name of business.

922
Date of Release
YYYYMMDD
8
 D 
Date of release at a facility treating hazardous waste onsite under PBR, CA, or CE.  A reportable release is from any activity at the facility, not just from one of the Tiered Permitting units.

923
Description of Release and Consequences
Narrative
200
AN
Description of the circumstances of the release and actions subsequently taken.

924
Date Corrective Action Completed
YYYYMMDD
8
D
Date corrective action was completed.  If no corrective action was needed, then leave blank.

Chapter 6   Unified Program Consolidated Forms

http://www.calepa.ca.gov/CUPA/Publications/

Chapter 7  [Reserved]
3

