Monitoring Plan Update

2025-26 Monitoring Season

NPDES Permit No. CAS00003

Document No. CTSW-RT-25-438.01.03

November, 2025

California Department of Transportation

Division of Environmental Analysis

Storm Water Program MS-27

1120 N Street

Sacramento, California 95814

This page intentionally left blank

# CALTRANS DOCUMENTATION PAGE

1. Report No: CTSW-RT-25-438.01.03

2. Type of Report: Report

3. Report Phase and Edition: 	Final

4. Title and Subtitle: Monitoring Plan Update, 2025-26 Monitoring Season

5. Report Date: November 2025

6. Copyright Owner(s): California Department of Transportation

7. Caltrans Project Coordinator(s): Jarred Lakis

8. Performing Organization Names and Addresses:

Office of Water Programs

California State University, Sacramento

6000 J Street, 1001 Modoc Hall

Sacramento, CA 95819

www.owp.csus.edu

9. Contract No.: 43A0438

10. Task Order No.: 01

11. Sponsoring Agency Name and Address:

California Department of Transportation

Division of Environmental Analysis

Office of Stormwater Program Development

Stormwater Program MS-27

1120 N Street

Sacramento, CA 95814

www.dot.ca.gov/hq/env/stormwater/index.htm

12. Caltrans Functional Reviewer(s):

Allan Laca, Rohullah Mayar, Christian Figueroa, Ali Abrishamchi, Sayed Noorbakhsh, Jiatong Tang, Joshua Gualco

13. Supplementary Notes:

14. External Reviewer(s): Dipen Patel, Dakota Keene

15. Key Words:

TMDL, receiving water monitoring, runoff characterization, BMP effectiveness monitoring

16. Distribution Statement:

17. No. of pages:

18. Abstract:

The Caltrans National Pollutant Discharge Elimination System (NPDES) Permit requires development of a plan for monitoring receiving waters, characterizing runoff, and determining BMP effectiveness.  This Monitoring Plan Update describes updates to the proposed monitoring for the 2025-26 and 2026-27 fiscal years.

NOTICE

This document has been checked for ADA compliance using built-in checkers in Microsoft Word and Adobe Acrobat Pro; and checked against WCAG 2.0 guidelines using the PAC 2024 Accessibility Checker.

Accessibility Assistance: Caltrans makes every attempt to ensure our documents are accessible. Due to variances between assistive technologies, there may be portions of this document which are not accessible. Where documents cannot be made accessible, we are committed to providing alternative access to the content. Should you need additional assistance, please contact us at (916) 654-2852 and/or (916) 995-7050, dial 711 to use a relay service, or visit:

https://dot.ca.gov/request-ada-compliant-documents

TABLE OF CONTENTS

CALTRANS DOCUMENTATION PAGE	iii

Section 1 Introduction	7

1.1	Purpose	7

Section 3 Receiving Water Monitoring	8

3.2	Regional Monitoring Programs (Permit Sections F2.11.2, F2.11.4)	8

3.3	Cooperative Monitoring Programs (Permit Sections F2.11.1, F2.11.2, F2.11.5)	9

Section 4 Region-Specific TMDL Monitoring	11

4.2	Region-Specific TMDL Monitoring Options	11

4.2.1	North Coast Water Board Sediment TMDL (Permit Section F2.12.1)	11

4.2.2	San Francisco Bay Mercury & PCBs TMDLs (Permit Section F2.12.2)	13

San Francisco Bay Mercury TMDL (Permit Section F2.12.2.1)	13

San Francisco Bay PCBs TMDL (Permit Section F2.12.2.2)	14

4.2.3	Los Angeles Water Board TMDLs (Permit Section F2.12.3)	16

4.2.4	Central Valley Water Board TMDLs (Permit Section F2.12.4)	17

4.2.5	Lahontan Water Board TMDLs (Permit Section F2.12.5)	18

4.2.6	Colorado River Basin Water Board TMDLs (Permit Section F2.12.6)	18

4.2.7	Santa Ana Water Board TMDLs (Permit Section D5.13)	19

4.2.8	San Diego Water Board TMDLs (Permit Section F2.12.7)	20

Twenty Beaches & Creeks Bacteria TMDL (Permit Section F2.12.7.1)	20

Chollas Creek Dissolved Metals TMDL (Permit Section F2.12.7.2)	21

Section 5 Runoff Characterization Monitoring	23

5.2	Characterization Monitoring Sites (Permit Section F2.13)	23

5.4	Characterization Monitoring Schedule (Permit Section F2.13)	23

Section 6 BMP Effectiveness Monitoring	24

6.2	BMP Monitoring Sites (Permit Section F2.14)	24

6.2.1	North Coast Region (Permit Section F2.14)	24

6.2.2	San Francisco Bay Region (Permit Section F2.14)	24

6.2.3	Central Valley Region (Permit Section F2.14)	24

6.2.4	Los Angeles Region (Permit Section F2.14)	25

6.2.5	San Diego Region (Permit Section F2.14)	25

6.4	BMP Monitoring Schedule (Permit Section F2.14)	26

Section 7 Conditionally Exempt Non-Stormwater Discharge Monitoring	27

Appendix A	Sampling and Analysis Plan

Appendix B	Quality Assurance Project Plan

- Introduction

  - Purpose

This document is an update to the 2024-25 Monitoring Plan Update which was conditionally approved by the State Water Board in January, 2025.  This update fulfils the requirement in Section G3 of the Caltrans National Pollutant Discharge Elimination System Permit (Permit; State Water Board 2023).  Only sections that have been updated are included in this Monitoring Plan Update.  Also included is the proposed monitoring for the North Coast sediment TMDL watersheds that was requested in the State Water Board Executive Officer letter dated September 9, 2024 and email and attachment dated October 15, 2024.

The purpose of the Monitoring Plan is to present the California Department of Transportation (Caltrans) approach to comply with the monitoring requirements set forth in Attachment F of its  Permit.  The Monitoring Plan also complies with the State Water Resources Control Board (State Water Board) requirement in the Permit, cited below:

Attachment F, Section F2.  The Monitoring Plan shall include the elements required by section F2 through F2.15 of this Attachment. The Monitoring Plan shall be submitted within 12 months of the Effective Date of this Order. Annual updates shall be submitted by November 30 of each year. The Department shall submit updates to the Monitoring Plan and annual updates for review and consideration of approval by State Water Board Executive Director.

The Monitoring Plan shall address the requirements of this Attachment. The Monitoring Plan shall include all proposed monitoring and a monitoring schedule for the upcoming fiscal year and the following fiscal year, including monitoring of applicable water body reaches in total maximum daily load (TMDL) watersheds where the Department is named as a responsible party where required through region-specific monitoring requirements or, where there are no region-specific monitoring requirements, as needed to demonstrate compliance with TMDLs in accordance with the Department’s TMDL Compliance Plan. The Monitoring Plan and annual updates shall be implemented upon approval by the State Water Board Executive Director.

# Receiving Water Monitoring

  - Regional Monitoring Programs (Permit Sections F2.11.2, F2.11.4)

Caltrans works with regional monitoring groups to maximize monitoring resources.  Table 3-1 lists Caltrans participation in regional receiving water monitoring programs.  Caltrans will no longer participate in the Klamath Basin regional monitoring program because it intends to implement a self-monitoring program.

Table 3-1  
Caltrans Participation in Regional Monitoring Programs

| Program | Regional Board | Purpose/TMDLs | Major Analyte Categories | Document Links |
| --- | --- | --- | --- | --- |
| Phase I Methylmercury Delta Regional Monitoring Program | Central Valley | Trends Monitoring, TMDL compliance, characterization - Sacramento-San Joaquin Delta Methylmercury TMDL | TSS, methylmercury, nutrients, field tests, pesticides, constituents of emerging concern, toxicity, phytoplankton | Monitoring Workplan Fiscal Year, 2024-2025 Annual Regional Monitoring Report, 2023 - 2024 |
| Central Valley Region Salts Monitoring Program | Central Valley | Trends monitoring, characterization - Central Valley Salts Monitoring TMDL | Salinity, nitrate, TDS | Surveillance and Monitoring Program, 2023 Final Baseline Characterization Report, 2024 |
| Lake Tahoe Regional Stormwater Monitoring Program | Lahontan | Trends monitoring, TMDL compliance, characterization, BMP effectiveness - Lake Tahoe Sediment and Nutrients TMDL | TSS, turbidity, particle size distribution, nutrients, field tests | Monitoring Plan Update, 2023 Annual Stormwater Monitoring Report, Water Year 2024 |
| Southern California Stormwater Monitoring Coalition | All Southern California | Trends monitoring, TMDL compliance, characterization, BMP effectiveness - Multiple TMDLs in the Los Angeles, Santa Ana, and San Diego Regions1 | TSS, bacteria, nutrients, metals, organics, pesticides, field tests, sediment, biological indicators | Workplan, 2021 - 20251 Annual Report, 2023 - 2024 |
| San Francisco Bay Regional Monitoring Program | San Francisco | Trends Monitoring, TMDL compliance, characterization - San Francisco Bay PCBs and Mercury TMDLs | TSS, PCBs, mercury, contaminants of emerging concern, field tests, sediment, turbidity | Water Year 2024 Near-Field Sampling and Analysis Plan Regional Monitoring Program Update, 2025 |

1 - Table 7 of Bioassessment Survey of the Stormwater Monitoring Coalition, Workplan for Years 2021 through 2025. SCCWRP Technical Report #1174.

  - Cooperative Monitoring Programs (Permit Sections F2.11.1, F2.11.2, F2.11.5)

Caltrans works with local agencies, stakeholders, and permittees to maximize receiving water monitoring resources.  Tables 3-2 and 3-3 provide details on the cooperative receiving water monitoring programs, including coordinated integrated monitoring programs, that Caltrans is currently participating in the Los Angeles/Santa Ana region and San Diego region, respectively.

Table 3-2  
Caltrans Participation in Los Angeles and Santa Ana Cooperative Monitoring Programs

| Program | Regional Board | Group Name/Lead Agency | Purpose/TMDLs | Major Analyte Categories | Document Links |
| --- | --- | --- | --- | --- | --- |
| Calleguas Creek Watershed Total Maximum Daily Load Compliance Monitoring Program | Los Angeles | Stakeholders Implementing TMDLs in the Calleguas Creek Watershed | TMDL compliance - Calleguas Creek Watershed OC Pesticides and PCBs TMDL | Nutrients, metals, selenium, PCBs, pesticides, organics, field tests, sediment, toxicity | No online documents |
| Development of a Dominguez Channel Estuary Remediation Plan Phase I Work Plan1 | Los Angeles | Dominguez Channel Watershed Management Area Group/ Los Angeles County Public Works | TMDL compliance - TMDL for Toxic Pollutants in Dominguez Channel and Greater Los Angeles and Long Beach Harbor Waters | Sediment, sediment toxicity, benthic community | No online documents |
| Ventura River Algae TMDL Monitoring | Los Angeles | Ventura County Watershed Protection District/ TMDL Responsible Parties | TMDL compliance - Ventura River and its Tributaries Algae, Eutrophic Conditions and Nutrients TMDL | Nutrients, field tests, phytoplankton | No online documents |
| Ventura River Estuary Trash TMDL Monitoring | Los Angeles | Ventura County | TMDL compliance - Ventura River Estuary Trash TMDL | Trash | No online documents |
| Revolon Slough/Beardsley Wash Trash Monitoring | Los Angeles | Ventura County | TMDL compliance - Revolon Slough and Beardsley Wash Trash TMDL | Trash | No online documents |
| Lake Elsinore and Canyon Lake TMDL Task Force | Santa Ana | Lake Elsinore & San Jacinto Watersheds Authority | TMDL compliance - Lake Elsinore and Canyon Lake Nutrient TMDL | TSS, nutrients, field tests | Comprehensive Monitoring Work Plan, 2016 2024-2025 Annual Report |

1 -  Requested by Regional Board to address lack of information on spatial variability of sediment contamination and location of hot spots.

Table 3-3  
Caltrans Participation in San Diego Cooperative Monitoring Programs

| Program | Regional Board | Group Name/Lead Agency | Purpose/TMDLs | Major Analyte Categories | Document Links |
| --- | --- | --- | --- | --- | --- |
| Chollas Creek Diazinon and Dissolved Metals TMDL Monitoring1 | San Diego | San Diego Bay Responsible Parties | TMDL compliance - Chollas Creek Diazinon TMDL - Chollas Creek Dissolved Copper, Lead and Zinc TMDL | Diazinon, metals, toxicity | No online documents |
| Project I-Twenty Beaches and Creeks in the San Diego Region | San Diego | San Diego Bay Responsible Parties | TMDL compliance - Project I – Twenty Beaches and Creeks in the San Diego Region (including Tecolote Creek) Indicator Bacteria, Revised TMDL | Bacteria, field tests | No online documents |
| Los Peñasquitos Lagoon Sediment TMDL | San Diego | Los Peñasquitos Watershed Management Area Permittees | TMDL compliance - Los Peñasquitos Lagoon Sediment TMDL | TSS, field tests, sediment, vegetation | No online documents |
| San Diego River Human Fecal Material - Investigative Order | San Diego | County of San Diego | Investigative | Bacteria | No online documents |

1 -  Caltrans has continued participation in this program even though it has selected self-monitoring as the compliance option for this watershed

# Region-Specific TMDL Monitoring

## Region-Specific TMDL Monitoring Options

## North Coast Water Board Sediment TMDL (Permit Section F2.12.1)

Caltrans monitoring options for sediment in the North Coast Water Board region are:

- Cooperative Monitoring.  (a) Allocate a one-time funding contribution equivalent to ten percent of each TMDL sediment reduction project cost to maintain existing watershed-based status and trends monitoring programs or (b) contribute funding proportionate to its share of waste load allocation among stakeholders for each TMDL sediment reduction project, or

- Self-Monitoring.  Implement a watershed monitoring program associated with state highways within the North Coast region TMDL watersheds. State highways are broadly distributed and therefore the monitoring will be watershed-based to allow North Coast Water Board staff to assess water quality impacts from state highways and progress toward achieving TMDL targets from Caltrans implementation of TMDL sediment reduction projects.  The watershed-based monitoring program will include a number of monitoring locations and frequency of monitoring proportional to Caltrans sediment load in excess of its load allocation for each TMDL watershed.

Table 4-1 describes the Caltrans monitoring strategy for the North Coast Sediment TMDLs.

Table 4-1  
North Coast TMDL Monitoring

| TMDL(s) | Section in Permit | Monitoring Options | Caltrans Strategy |
| --- | --- | --- | --- |
| North Coast Sediment TMDLs | Attachment F, Section F2.12.1 Attachment D, Section D5.7 | Cooperative monitoring: - Provide 10% additional funding contribution for each Cooperative Implementation Agreement project in lieu of Caltrans monitoring - Contribute funding proportionate to its share of waste load allocation among stakeholders Self-monitoring: Monitor at a number of monitoring locations in each TMDL watershed | Caltrans has selected self- monitoring. Caltrans will develop and implement a monitoring program in the South Fork Eel River, Trinity River, and Redwood Creek watersheds. |

Caltrans has selected self-monitoring because there are no existing watershed-based status and trends monitoring programs in the North Coast region.  However, Caltrans has analyzed its share of waste load allocation among stakeholders to quantify its proportional responsibility for any monitoring effort.  Table 4-2 provides Caltrans responsibility for overall watershed health monitoring in each TMDL watershed based on its share of management waste load allocation.  As can be seen in Table 4-2, the relative impact of Caltrans discharges and Caltrans proportional responsibility for monitoring are minor in all watersheds.

For the self-monitoring program, Regional Board staff have proposed monitoring of Caltrans discharges from outfalls and receiving water locations immediately upstream and downstream of outfalls.  To satisfy these monitoring requirements, Caltrans is planning to identify and monitor 18 new locations in the South Fork Eel River, Trinity River, and Redwood Creek watersheds.  Caltrans will install and collect data at a total of 54 new stations at these locations, with each location comprising of an upstream station, a downstream station, and a discharge station.  Monitoring at these stations is expected to begin in the 2025-26 season.

In addition to self-monitoring in the South Fork Eel River, Trinity River, and Redwood Creek watersheds, Caltrans has commenced 6PPD-Q characterization monitoring at locations throughout the North Coast, including Redwood Creek, Garcia River, and South Fork Eel watershed.  Locations for 6PPD-Q monitoring are shown in Table 4 in Appendix B.

Caltrans has also commenced monitoring BMP effectiveness at one low impact development BMP in the Redwood Creek watershed.  Monitoring at site is expected to continue during the 2025-26 fiscal year.  Details on this additional monitoring are provided in Section 6 and Appendix B.

Table 4-2  
Caltrans Responsibility for North Coast Sediment TMDLs Watershed Monitoring

| Watershed | Watershed Area (mi2) | Caltrans Load Allocation1 (t/y) | Total Management Load in Watershed2 (t/mi2/y) | Management Load Reduction Required2 (%) | Management Load Allocation2 (t/mi2/y) | Management Load Allocation3 (t/y) | Proportional Responsibility for Watershed Health Monitoring Programs4 (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Albion River | 43 | 1.9 | 435 | 69% | 135 | 5799 | 0.03% |
| Big River | 181 | 43.4 | 314 | 75% | 79 | 14209 | 0.31% |
| Eel River, Upper Main | 708 | 70.9 | 152 | 49% | 78 | 54884 | 0.13% |
| Eel River, Middle Fork | 753 | 105.7 | 82 | 65% | 29 | 21611 | 0.49% |
| Eel River, Lower Main | 300 | 73.7 | 775 | 77% | 178 | 53475 | 0.14% |
| Eel River, South Fork | 689 | 4848.2 | 845 | 71% | 247 | 170376 | 2.85% |
| Garcia River | 114 | 100.0 | 1380 | 60% | 552 | 62928 | 0.16% |
| Gualala River | 299 | 20.6 | 840 | 89% | 96 | 28632 | 0.07% |
| Mad River | 374 | 515.0 | 1580 | 89% | 174 | 65001 | 0.79% |
| Navarro River | 316 | 1362.6 | 775 | 62% | 293 | 92572 | 1.47% |
| Noyo River | 113 | 32.8 | 274 | 70% | 82 | 9317 | 0.35% |
| Redwood Creek | 282 | 855.6 | 4360 | 65% | 1510 | 425820 | 0.20% |
| Scott River | 814 | 66.7 | 299 | 63% | 111 | 90053 | 0.07% |
| Ten Mile River | 120 | 1.1 | 318 | 75% | 80 | 9540 | 0.01% |
| Trinity River | 1694 | 88.9 | 548 | 68% | 178 | 301018 | 0.03% |
| Trinity River, South Fork | 931 | 352.3 | 371 | 68% | 119 | 110528 | 0.32% |
| Van Duzen River | 428 | 67.8 | 586 | 72% | 162 | 69368 | 0.10% |

1 – Table A-3, State Water Board (2023)

2 – Derived from TMDL staff reports.  Management Load Allocation (t/m2/y) is the Total Management Load in Watershed (t/mi2/y) multiplied by [100 – Management Load Reduction Required (%)]

3 – Management Load Allocation (t/y) is the Management Load Allocation (t/mi2/y) multiplied by the Watershed Area (mi2)

4 – Proportional monitoring responsibility assumed to be same as share of management waste load allocation.  The percent share of management waste load allocation is [Caltrans Load Allocation (t/y) divided by the Management Load Allocation (t/y)] multiplied by 100

## San Francisco Bay Mercury & PCBs TMDLs (Permit Section F2.12.2)

## San Francisco Bay Mercury TMDL (Permit Section F2.12.2.1)

Caltrans mercury monitoring options are as follows:

- Regional Monitoring.  Participate in mercury monitoring via the Regional Monitoring Program for Water Quality in San Francisco Bay; or

- Self-Monitoring.  Develop and implement a mercury monitoring plan to quantify the mercury loads or load reductions achieved through treatment, source control, and other management efforts.  Sample bedded fine sediment for a minimum of four wet weather events per year over the term of the Order.  Select sample locations at or near a point of discharge from the right-of-way and into the system that discharges stormwater into San Francisco Bay.

- Combination. Implement a combination of monitoring requirements in options 1 or 2, above, provided that the combination provides equivalent monitoring.

Table 4-3 describes the Caltrans monitoring strategy for the San Francisco Bay Mercury TMDL.

Table 4-3  
San Francisco Bay Mercury TMDL Monitoring

| TMDL(s) | Section in Permit | Monitoring Options | Caltrans Strategy |
| --- | --- | --- | --- |
| San Francisco Bay Mercury TMDL | Attachment F, Section F2.12.2.1 Attachment D, Section D5.8 | Regional monitoring: Participate in Regional Monitoring Program Self-monitoring: Monitor to quantify mercury loads or load reductions achieved through treatment, source control, and other management efforts. Include bedded fine sediment sampling Combination: Participate in Regional Monitoring Program and perform self-monitoring | Caltrans has selected combination monitoring. Caltrans will continue to contribute funding for the Regional Monitoring Program for Water Quality in San Francisco Bay and perform as needed self-monitoring. Monitoring for these projects may be performed by other stakeholders. |

In addition to participating in the Regional Monitoring Program, Caltrans will characterize mercury in its runoff and monitor BMP effectiveness at 3 hydrodynamic separator (HDS) sites and one multi gross solids removal device (MGSRD) site in the TMDL watershed.  Monitoring at the 3 HDS and one MGSRD sites is planned for the 2025-26 fiscal year.  Additional details on this self-monitoring are provided in Sections 5 and 6, and Appendix B.

## San Francisco Bay PCBs TMDL (Permit Section F2.12.2.2)

Caltrans PCBs monitoring options are as follows:

- Regional Monitoring.  Participate in PCBs monitoring via the Regional Monitoring Program for Water Quality in San Francisco Bay; or

- Self-Monitoring.  Develop, submit, and implement a Caltrans-specific monitoring plan to quantify PCBs stormwater runoff loads and the load reductions achieved through treatment, source control and other actions.  Sample bedded fine sediment a minimum of four wet weather events per year over the term of the Order.  Sample locations will be at/near a point of discharge from the right-of-way and into the conveyance system that discharges stormwater into San Francisco Bay.  Monitoring will be representative of pollutant concentrations or loadings in discharges from Caltrans right-of-way or will be representative of the effects of discharges from Caltrans right-of-way on water qualities in the TMDL waterbodies.

- Combination.  Implement a combination of monitoring requirements in options 1 or 2, above, provided that the combination provides equivalent monitoring.

Table 4-4 describes the Caltrans monitoring strategy for the San Francisco Bay PCBs TMDL.

Table 4-4  
San Francisco Bay PCBs TMDL Monitoring

| TMDL(s) | Section in Permit | Monitoring Options | Caltrans Strategy |
| --- | --- | --- | --- |
| San Francisco Bay PCBs TMDL | Attachment F, Section F2.12.2.2 Attachment D, Section D5.8 | Regional monitoring: Participate in Regional Monitoring Program Self-monitoring: Monitoring to quantify PCBs stormwater runoff loads and the load reductions achieved through treatment, source control and other actions. Include bedded fine sediment sampling Combination: Participate in Regional Monitoring Program and perform self-monitoring | Caltrans has selected combination monitoring. Caltrans will continue to contribute funding for the Regional Monitoring Program for Water Quality in San Francisco Bay and perform as needed self-monitoring. Monitoring for these projects may be performed by other stakeholders. |

In addition to participating in the Regional Monitoring Program, Caltrans will characterize PCBs in its runoff and monitor BMP effectiveness at 3 hydrodynamic separator (HDS) sites and one multi gross solids removal device (MGSRD) site in the TMDL watershed.  Monitoring at the 3 HDS and one MGSRD sites is planned for the 2025-26 fiscal year.  Additional details on this self-monitoring are provided in Sections 5 and 6, and Appendix B.

## Los Angeles Water Board TMDLs (Permit Section F2.12.3)

Caltrans monitoring options for TMDLs in the Los Angeles Water Board region (Los Angeles Region) are:

- Coordinated Integrated Monitoring.  Caltrans may continue to participate in Coordinated Integrated Monitoring Programs for individual TMDL watersheds or participate in other watershed cooperative monitoring programs in lieu of self- monitoring; or

- Self-Monitoring.  Caltrans will implement self-monitoring through development of a monitoring plan and schedule to monitor its rights-of-way.  The monitoring plan will be equivalent in methods, precision, accuracy, and quality to the (1) relevant Coordinated Integrated Monitoring Programs or other watershed cooperative monitoring programs and (2) the monitoring requirements in this Attachment.  The monitoring plan shall include a work plan and schedule to implement the monitoring. The watershed and TMDL shall be identified in the monitoring plan; or

- Combination.  Caltrans may implement a combination of requirements in options 1 or 2, above, provided that the combination is equivalent to the monitoring via watershed cooperative monitoring and right-of-way monitoring.

Table 4-5 describes the Caltrans monitoring strategy for the Los Angeles Region TMDLs.

Table 4-5  
Los Angeles Region TMDL Monitoring

| TMDL(s) | Section in Permit | Monitoring Options | Caltrans Strategy |
| --- | --- | --- | --- |
| Los Angeles Region TMDLs | Attachment F, Section F2.12.3 Attachment D, Section D5.10 | Coordinated Integrated monitoring: Participate in Coordinated Integrated Monitoring Programs Self-monitoring: Monitoring shall be equivalent in methods, precision, accuracy, and quality to the relevant Coordinated Integrated Monitoring Programs or other watershed cooperative monitoring programs Combination: Participate in Coordinated Integrated Monitoring Programs and perform self-monitoring providing that it is equivalent to the monitoring via watershed cooperative monitoring and right-of-way monitoring | Caltrans has selected combination monitoring. Caltrans will continue to contribute funding or enter into new Coordinated Integrated Monitoring Programs for individual TMDL watersheds and perform additional self-monitoring. Monitoring for these projects may be performed by other stakeholders. |

Caltrans is currently participating in a number of Coordinated Integrated Monitoring Programs in the Los Angeles Region and is planning to participate in new programs in the future.  Current Coordinated Integrated Monitoring Programs are listed in Table 3-2.  Caltrans anticipates participating in the new programs after management approval of terms and conditions of participation and upon acceptance from existing stakeholders.

As efforts continue to pursue participation in Coordinated Integrated Monitoring Programs, Caltrans will characterize its runoff at 7 sites in various Los Angeles Region TMDL watersheds and monitor BMP effectiveness at 1 site in the Peck Road Park watershed.  Monitoring at these sites, which are either ongoing monitoring sites or are currently in the planning phase, is expected to continue or commence during the 2025-26 fiscal year.  To characterize dry weather flows, monitoring will include field observations on the frequency of dry weather flows and dry weather flow sampling at select locations during the dry season.  Additional details on this self-monitoring are provided in Sections 5 and 6, and Appendix B.

## Central Valley Water Board TMDLs (Permit Section F2.12.4)

In the Central Valley Water Board region (Central Valley Region), Caltrans will continue to participate in the Central Valley Water Board approved Delta Regional Monitoring Program.  In the event the Delta Regional Monitoring Program is no longer approved by the Central Valley Water Board Executive Officer, Caltrans will submit a Methylmercury Monitoring Plan that assesses attainment with the TMDL allocations in its stormwater discharges and implement it within six months of the Central Valley Water Board Executive Officer approval.  The Methylmercury Monitoring Plan and Quality Assurance Project Plan that is to be included will meet the requirement of Section F2.12.4 of the Permit.

Table 4-6 describes the Caltrans monitoring strategy for the Central Valley Region.

Table 4-6  
Central Valley Region TMDL Monitoring

| TMDL(s) | Section in Permit | Monitoring Requirement | Caltrans Strategy |
| --- | --- | --- | --- |
| Sacramento-San Joaquin Delta Methylmercury TMDL | Attachment F, Section F2.12.4 Attachment D, Section D5.11 | Regional monitoring: Participate in Delta Regional Monitoring Program | Caltrans will continue to participate in Delta Regional Monitoring Program. Caltrans may perform as needed self-monitoring to quantify its loads and load reductions achieved through treatment, source control, and other management efforts. |

As required in Section F2.12.4 of the Permit, Caltrans will continue to implement turbidity monitoring for construction projects in the Clear Lake Nutrients TMDL watershed and the Cache Creek, Bear Creek, Sulphur Creek, and Harley Gulch Mercury TMDL watershed.  In addition to monitoring construction projects, Caltrans is monitoring 3 open-graded friction course (OGFC) sites together with one control (no OGFC) site in Merced County and has commenced 6PPD-Q characterization monitoring at 3 locations in the San Joaquin River watershed.

## Lahontan Water Board TMDLs (Permit Section F2.12.5)

Caltrans monitoring options in the Lahontan Water Board region (Lahontan Region) are:

- Cooperative Monitoring.  Caltrans will participate in the Lake Tahoe Regional Stormwater Monitoring Program; or

- Self-Monitoring.  Caltrans will prepare and submit a Stormwater Monitoring Plan for review and consideration of approval to the State Water Board Executive Director in consultation with the Lahontan Water Board Executive Officer. Caltrans monitoring plan will have the same monitoring parameters, locations, frequencies, and reporting as the Lake Tahoe Regional Stormwater Monitoring Program.

Table 4-7 describes the Caltrans monitoring strategy for the Lahontan Region.

Table 4-7  
Lahontan Region TMDL Monitoring

| TMDL(s) | Section in Permit | Monitoring Options | Caltrans Strategy |
| --- | --- | --- | --- |
| Lake Tahoe TMDL | Attachment F, Section F2.12.5 Attachment D, Section D5.12 | Cooperative monitoring: Participate in Lake Tahoe Regional Stormwater Monitoring Program Self-monitoring: Prepare and submit a Stormwater Monitoring Plan for review and approval that has the same monitoring parameters, locations, frequencies, and reporting as the Lake Tahoe Regional Stormwater Monitoring Program | Caltrans has selected cooperative monitoring. Caltrans will continue to participate in the Lake Tahoe Regional Stormwater Monitoring Program. |

## Colorado River Basin Water Board TMDLs (Permit Section F2.12.6)

For the Coachella Valley Stormwater Channel Bacterial Indicators TMDL, Caltrans is monitoring for Escherichia coli during a minimum of two qualifying precipitation events per calendar year that result in a discharge and for a minimum of eight sampled events over four years, excluding years less than two qualifying precipitation events.  Caltrans is sampling at the monitoring locations identified in the Caltrans document “Coachella Valley Stormwater Channel Bacterial Indicator Monitoring Quality Assurance Project Plan (QAPP), Document No. CTSW-PL-22-423.04.2, March 2022”.

Table 4-8 describes the Caltrans monitoring strategy for the Colorado River Basin.

Table 4-8  
Colorado River Basin TMDL Monitoring

| TMDL(s) | Section in Permit | Monitoring Requirement | Caltrans Strategy |
| --- | --- | --- | --- |
| Coachella Valley Stormwater Channel Bacterial Indicators TMDL | Attachment F, Section F2.12.6 | Sample at monitoring locations identified in Caltrans “Monitoring and Reporting Project Plan and Quality Assurance Project Plan for 2-Year Bacteria Indicator Monitoring in Conformance with Phase I Implementation for the Coachella Valley Stormwater Channel Total Maximum Daily Load, Riverside County, California” | Caltrans will monitor at Caltrans right-of-way sites stated in the Monitoring and Reporting Project Plan |

Caltrans is performing monitoring at four locations in the Coachella Valley Stormwater Channel watershed: I-10 West of Jackson Street, I-10 East of Jackson Street, I-10 West of Monroe Street, and SR-86 near the interchange.  Monitoring at these sites, which are existing monitoring sites, will continue during the 2025-26 fiscal year.  Additional details on these monitoring sites are provided in Section 5 and Appendix B.

## Santa Ana Water Board TMDLs (Permit Section D5.13)

The monitoring options for the Lake Elsinore and Canyon Lake Nutrient TMDL are:

- Cooperative Monitoring.  Caltrans will continue participation with the Lake Elsinore and Canyon Lake Nutrients TMDL Task Force for cooperative implementation actions, monitoring, and special studies; or

- Self-Monitoring.  Caltrans will develop and implement a program consistent with the Lake Elsinore and Canyon Lake TMDL Task force, which will include Canyon Lake in-lake monitoring consistent with the TMDL Task Force monitoring program and a monitoring program to evaluate the success of in-lake sediment reduction strategies that will be implemented.

Table 4-9 describes the Caltrans monitoring strategy for the Santa Ana Region.

Table 4-9  
Santa Ana Region TMDL Monitoring

| TMDL(s) | Section in Permit | Monitoring Options | Caltrans Strategy |
| --- | --- | --- | --- |
| Lake Elsinore and Canyon Lake Nutrient TMDL | Attachment D, Section D5.13 | Cooperative monitoring: Participate in Lake Elsinore and Canyon Lake Nutrients TMDL Task Force Self-monitoring: Prepare and implement a monitoring program that is consistent with that conducted by the Lake Elsinore and Canyon Lake TMDL Task force | Caltrans has selected cooperative monitoring. Caltrans will continue to participate in the Lake Elsinore and Canyon Lake Nutrients TMDL Task Force |

In addition to participating in the Lake Elsinore and Canyon Lake Nutrients TMDL Task Force, Caltrans is characterizing its runoff at 3 sites in the Big Bear Lake watershed.  Monitoring at two existing sites will continue during the 2025-26 fiscal year, while monitoring at a third site is expected to commence during the 2025-26 fiscal year.  Caltrans is also characterizing its runoff at 4 sites in the Newport Bay watershed.  Monitoring at the Newport Bay sites commenced during the 2024-25 fiscal year and is expected to continue during the 2025-26 fiscal year.  Additional details on this self-monitoring are provided in Section 5 and Appendix B.

## San Diego Water Board TMDLs (Permit Section F2.12.7)

## Twenty Beaches & Creeks Bacteria TMDL (Permit Section F2.12.7.1)

Caltrans monitoring options in the Project I – Twenty Beaches and Creeks Bacteria TMDL watersheds are:

- Cooperative Monitoring.  Caltrans will participate in cooperative watershed monitoring with the other responsible municipalities, as approved by the State Water Board Executive Director in consultation with the San Diego Water Board Executive Officer. Caltrans participation shall be a proportional responsibility that is calculated in accordance with Caltrans land use coverage in the watershed; or

- Self-Monitoring.  Caltrans will conduct compliance monitoring to demonstrate the effectiveness of best management practices (BMPs) in controlling bacteria loads.  Receiving water monitoring will be conducted in three representative watersheds annually for the Permit term.  For each of the twenty beaches and creeks watersheds, outfalls shall be monitored weekly during the dry season and a minimum of three rain events during one wet season.  This monitoring will be conducted in two different sampling years and at a minimum of twice per Permit term.

Table 4-10 describes the Caltrans monitoring strategy for the Twenty Beaches & Creeks Bacteria TMDL.

In addition, Caltrans will continue to self-monitor indicator bacteria in its runoff and discharge at 4 monitoring stations in the Chollas Creek watershed.  Monitoring at these stations, which are existing monitoring stations, is expected to continue during the 2025-26 fiscal year.  Additional details on this self-monitoring are provided in Sections 5 and 6, and Appendix B.

Table 4-10  
Twenty Beaches & Creeks Bacteria TMDL Monitoring

| TMDL(s) | Section in Permit | Monitoring Options | Caltrans Strategy |
| --- | --- | --- | --- |
| Twenty Beaches & Creeks Bacteria TMDL | Attachment F, Section F2.12.7.1 Attachment D, Section D5.14.1 | Cooperative monitoring: Participate in an approved cooperative watershed monitoring program Self-monitoring: Conduct annual receiving water in three representative watersheds, and outfall monitoring in each of the twenty beaches and creeks watersheds. May conduct compliance monitoring to demonstrate BMP effectiveness for bacteria | Caltrans has selected cooperative monitoring. Caltrans will continue to participate in Project I-Twenty Beaches and Creeks/ Chollas Creek Bacteria TMDL Monitoring in cooperation with the County and City of San Diego and other responsible municipalities |

## Chollas Creek Dissolved Metals TMDL (Permit Section F2.12.7.2)

Caltrans monitoring options in the Chollas Creek Dissolved Copper, Lead, and Zinc TMDL (Chollas Creek Metals TMDL) watershed are:

- Cooperative Monitoring.  Caltrans may participate in or contribute to a cooperative watershed monitoring program with the other responsible municipalities, as approved by the State Water Board Executive Director in consultation with the San Diego Water Board Executive Officer.  Receiving water will be sampled monthly during the wet season; or

- Self-Monitoring.  Caltrans may develop and conduct compliance monitoring to demonstrate the effectiveness of best management practices (BMPs) at outfalls.  Representative outfalls will be monitored for applicable metals for one rain event per year over three separate years during the wet season per the Permit term or per every five years, whichever is less.

Table 4-11 describes the Caltrans monitoring strategy for the Chollas Creek Metals TMDL.

Table 4-11  
Chollas Creek Dissolved Copper, Lead, and Zinc TMDL Monitoring

| TMDL(s) | Section in Permit | Monitoring Options | Caltrans Strategy |
| --- | --- | --- | --- |
| Chollas Creek Metals TMDL | Attachment F, Section F2.12.7.2 Attachment D, Section D5.14.2 | Cooperative monitoring: Participate in an approved cooperative receiving water monitoring program Self-monitoring: Perform representative outfall monitoring for applicable metals for one rain event per year over three separate years during the wet season per the Permit term or per every five years, whichever is less. Caltrans may develop and conduct compliance monitoring to demonstrate the effectiveness of BMPs | Caltrans has selected self-monitoring. Caltrans will continue monitoring at representative BMP sites that have been approved by the Water Board. The approved sites are an Austin sand filter and a bioswale. Monitoring at these approved sites will target a minimum of one rain event per year over three separate years during the wet season and will include north and south fork receiving water sampling for hardness and pH. Although not required by the Permit since self-monitoring is selected, Caltrans will continue to participate in the Regional Monitoring Program with the City of San Diego to assess compliance in the receiving water |

Since 2010, Caltrans has installed more than 20 structural BMPs within its right-of-way to treat stormwater runoff in the Chollas Creek watershed.  Four monitoring stations, located at representative Caltrans BMP locations on the north and south forks of Chollas Creek, have been selected for Caltrans runoff characterization and BMP effectiveness monitoring.  These locations include an Austin sand filter and one receiving water station on the north fork and a bioswale and one receiving water station on the south fork.  The two receiving water monitoring stations located on the north and south fork of Chollas Creek are being monitored to improve assessments of the dissolved metals TMDL acute and chronic thresholds.  Monitoring at these BMP and receiving water sites, which are existing monitoring sites approved as representative sites by the San Diego Water Board, is expected to continue during the 2025-26 fiscal year.  Additional details on this self-monitoring are provided in Sections 5 and 6, and Appendix B.

# Section 5
Runoff Characterization Monitoring

## Characterization Monitoring Sites (Permit Section F2.13)

Table 4 in the QAPP (Appendix B) lists the current and proposed runoff characterization monitoring sites, together with information on the station codes, where the sites are located, and the type of monitoring (Receiving Water Monitoring, Region-Specific TMDL Monitoring, Runoff Characterization Monitoring, BMP Effectiveness Monitoring, Conditionally Exempt Non-Stormwater Discharge Monitoring). Caltrans intends to monitor 57 runoff characterization sites during the 2025-26 fiscal year.

Site selection for runoff characterization focused on TMDL watersheds.  For each TMDL watershed, an analysis was performed during the 2023-24 season to determine the number of existing runoff monitoring sites and representative runoff sites that had at least five analytical results for the TMDL constituent.  Representative sites are sites that are located outside the TMDL watershed but are considered representative of sites within the TMDL watershed based on important factors such as traffic.  Traffic was the main factor found to govern Caltrans runoff quality in previous characterization studies (Section 4 in Caltrans 2003).  Following this approach, new runoff monitoring sites were identified in the following TMDL watersheds:  Big Bear Lake, Los Angeles River, Redwood Creek, Sacramento- San Joaquin River Delta Estuary, South Fork Eel River, San Francisco Bay, San Gabriel River, and some smaller watersheds.

In the North Coast sediment TMDL watersheds, Caltrans plans to monitor 18 new discharge sites in the South Fork Eel River, Trinity River, and Redwood Creek watersheds during the 2025-26 fiscal year.  Each discharge site will be paired with upstream and downstream receiving water stations.

## Characterization Monitoring Schedule (Permit Section F2.13)

Table 4 of the QAPP (Appendix B) provides the monitoring schedule for runoff characterization sites in each TMDL watershed.  Table 4 specifies monitoring that will be conducted during the 2025-26 fiscal year and monitoring that is proposed for the 2026-27 fiscal year.  The decision to continue monitoring these sites during the 2026-27 fiscal year will be made after the current season, when the total number of usable data points for the sites are known.

# Section 6
BMP Effectiveness Monitoring

## BMP Monitoring Sites (Permit Section F2.14)

Table 4 of the QAPP (Appendix B) lists the current and proposed BMP effectiveness monitoring sites, together with information on the influent and effluent station codes, where the sites are located, and key TMDL pollutants.  Caltrans intends to monitor a total of 13 BMP sites during the 2025-26 fiscal year.

Site selection for BMP effectiveness assessment focused on ecoregions.  For each ecoregion, an analysis was performed to determine the number of existing BMP monitoring sites of a given type that had at least five analytical results for the TMDL constituent.  Representative sites outside the ecoregion were also identified.  Representative sites outside an ecoregion are sites that are located outside the ecoregion but are considered representative of sites within the ecoregion based on traffic, the main factor found to govern Caltrans runoff quality in previous characterization studies (Section 4 in Caltrans 2003).  Following this approach, new BMP effectiveness monitoring sites were identified in the following water board regions:  North Coast, San Francisco Bay, Central Valley, Los Angeles, and San Diego.

## North Coast Region (Permit Section F2.14)

Caltrans intends to monitor the effectiveness of one bioswale near Lord Ellis Summit that drains to Redwood Creek.  Monitoring at this existing BMP site, which include influent and effluent stations, is expected to continue during the 2025-26 fiscal year.  Details on stations at this BMP site are given in Table 4 of the QAPP.

## San Francisco Bay Region (Permit Section F2.14)

In the San Francisco Bay region, Caltrans intends to commence BMP effectiveness monitoring at three HDS units during the 2025-26 fiscal year.  The HDS units that will be monitored are in Contra Costa at Willow Pass (stations 4-470 and 4-471), Santa Clara at Coyote Creek (4-472 and 4-473), and San Mateo at Edgewood Road (4-474 and 4-475).  In addition to HDS monitoring, Caltrans intends to monitor treatment by a multi gross solids removal device (MGSRD) in Santa Clara (stations 4-461 and 4-462).

## Central Valley Region (Permit Section F2.14)

Caltrans is currently monitoring the effectiveness of three types of Open Graded Friction Course (OGFC) near Santa Nella in the Central Valley: Non-Rubberized, Rubberized, and Modified Rubberized.  The Santa Nella OGFC study consists of four monitoring stations: three stations with OGFC overlays (10-307, 10-308, 10-309), while the remaining station (10-306) is a control station that collects runoff from standard dense-graded hot mix asphalt.  Monitoring of these stations will continue during the 2025-26 fiscal year.  Details on these BMP sites are given in Table 4 of the QAPP.

## Los Angeles Region (Permit Section F2.14)

Caltrans monitored the effectiveness of two Austin sand filters in the Los Angeles Region during the 2024-25 fiscal year, one in the Ballona Creek watershed and one in the Peck Road Park Lake (Los Angeles River) watershed.  Monitoring at the Ballona Creek filter, located off I-10, is now complete.  The Peck Road Park Lake filter, located off SR-210, will continue to be monitored during the 2025-26 fiscal year.  This filter includes influent station 7-422 and effluent station 7-423.  Details on this BMP are given in Table 4 of the QAPP.

Caltrans has collected sufficient monitoring data for pilot trash devices with biochar media at six locations in the Los Angeles Region, all in the Los Angeles River watershed.  No further monitoring is anticipated at these sites.

## San Diego Region (Permit Section F2.14)

Caltrans has constructed bioswales, modular infiltration trenches, and an Austin sand filter to address the Chollas Creek Diazinon TMDL, Dissolved Metals TMDL, and Twenty Beaches and Creeks Bacteria TMDL.  Caltrans has selected and is performing BMP effectiveness monitoring of one bioswale and one Austin sand filter.  The bioswale, located off SR-94, includes influent station 11-357 and effluent station 11-358.  The Austin sand filter, also located off SR-94, includes influent station 11-359 and effluent station 11-360.  In addition to BMP effectiveness monitoring, Caltrans is also conducting monitoring at two receiving water stations (11-355 and 11-356) to improve assessments of dissolved metals acute and chronic TMDL thresholds.  These BMP and receiving water sites will be monitored during the 2025-26 fiscal year.  Details on these sites are given in Table 4 of the QAPP.

During the 2024-25 fiscal year, Caltrans conducted a pilot pervious pavement BMP monitoring study at three sites near Encinitas in the Cottonwood Creek watershed.  The Encinitas Pervious Pavement Pilot Monitoring Project (Encinitas Pilot Project) consisted of three stations that evaluate the performance of pervious highway shoulders for runoff treatment.  One station (11-362) monitored untreated runoff while the other two monitored treated effluent (11-363 and 11-364).  Caltrans does not intend to continue monitoring at these sites because of operational issues.

## BMP Monitoring Schedule (Permit Section F2.14)

Table 4 of the QAPP (Appendix B) provides the monitoring schedule for BMP effectiveness sites in each TMDL watershed.  Table 4 specifies monitoring that will be conducted during the 2025-26 fiscal year and monitoring that is proposed for the 2026-27 fiscal year.  The decision to monitor these sites during the 2026-27 fiscal year will be made after the current season, when the total number of usable data points for the sites are known.

# Section 7
Conditionally Exempt Non-Stormwater Discharge Monitoring

Section F2.15 of the Caltrans Permit allows certain types of non-stormwater discharges that are not considered to be sources of pollutants.  However, if the State Water Board Executive Director determines that any category of allowed non-stormwater discharge is a source of pollutants, the State Water Board Executive Director may require the Department to conduct additional monitoring and submit a report on such discharges.  The State Water Board Executive Director may also order the Department to cease a non-stormwater discharge.

During the 2025-26 season, the State Water Board Executive Director has not identified any of the conditionally exempt non-stormwater discharges to be a source of pollutants.

Appendix A (Sampling and Analysis Plan)

Table SAP- 1.3 – Frequency of Field Quality Control Sample Collection

| QA/QC Sample Type | Minimum Sampling Frequency |
| --- | --- |
| Field Duplicate | Once every 20 samples collected at a given site or once per sampling station per season, whichever is more frequent. |
| Field Blank | Once every 20 samples collected at a given site or once per sampling station per season, whichever is more frequent. |
| Matrix Spike/Matrix Spike Duplicate | Once every 20 samples collected at a given site or once per sampling station per season, whichever is more frequent.1 |

1MS/MSD for 6PPD-Quinone to be analyzed only when requested by the Task Order Manager.

Table SAP- 4.1 - Methods and Reporting Limits

| Constituent | Analytical Method(s)1 | Units | Reporting Limit |
| --- | --- | --- | --- |
| Conventionals |  |  |  |
| Hardness as CaCO3 | SM 2340 B or C | mg/L | 1 |
| pH | Calibrated Field Instrument | pH Units | Field |
| Temperature | Calibrated Field Instrument | ° Celsius | Field |
| Flow Rate | Calibrated Field Instrument | ft3/s | Field |
| Salinity | SM 2520 A | ppt | 1 |
| Total Dissolved Solids | EPA 160.1 | mg/L | 1 |
| Total Suspended Solids | EPA 160.2 | mg/L | 1 |
| Settleable Solids | SM 2540 F | mL/L | 0.1 |
| Turbidity | Calibrated Field Instrument | NTU | Field |
| Total Organic Carbon | EPA 5310B or C | mg/L | 0.6 |
| Dissolved Organic Carbon | EPA 5310B or C | mg/L | 0.6 |
| Dissolved Oxygen | Calibrated Field Instrument | mg/L | Field |
| Alkalinity as CaCO3 | EPA 310.1 | mg/L | 1 |
| Conductivity | EPA 120.1 | uS/cm | 1 |
| Nutrients |  |  |  |
| Total Kjeldahl Nitrogen | EPA 351.3 | mg/L | 0.1 |
| Nitrate as N | EPA 300.0/EPA 300.1 | mg/L | 0.1 |
| Nitrite as N | EPA 300.0/EPA 300.1 | mg/L | 0.1 |
| Phosphorous as P | EPA 365.2 | mg/L | 0.03 |
| Orthophosphate (Total) | SM 4500-P, E | mg/L | 0.03 |
| Ammonia as N | EPA 350.1/SM 4500-NH3 D | mg/L | 0.1 |
| Metals (Total and Dissolved) |  |  |  |
| Aluminum | EPA 200.8/EPA 1640 | µg/L | 25 |
| Arsenic | EPA 200.8/EPA 1640 | µg/L | 0.01 |
| Cadmium | EPA 200.8/EPA 1640 | µg/L | 0.005 |
| Chromium | EPA 200.8/EPA 1640 | µg/L | 0.025 |
| Copper | EPA 200.8/EPA 1640 | µg/L | 0.01 |
| Iron | EPA 200.8/EPA 1640 | µg/L | 1 |
| Lead | EPA 200.8/EPA 1640 | µg/L | 0.05 |
| Manganese | EPA 200.8/EPA 1640 | µg/L | 0.01 |
| Mercury | EPA 1631E/EPA 1640 | µg/L | 0.0002 |
| Nickel | EPA 200.8/EPA 1640 | µg/L | 0.02 |
| Selenium | EPA 200.8/EPA 1640 | µg/L | 0.3 |
| Silver | EPA 200.8/EPA 1640 | µg/L | 0.02 |
| Zinc | EPA 200.8/EPA 1640 | µg/L | 5 |
| Hydrocarbons |  |  |  |
| Oil & Grease | EPA 1664B | mg/L | 1.4 |
| Organics |  |  |  |
| 6PPD-Quinone | EPA 1634 | ng/L | 2.0 |
| Polynuclear Aromatic Hydrocarbons (PAHs) | EPA 8310 | ng/L | See Note2 |
| Chlorinated Phenolics | EPA 625/EPA 8270 | µg/L | 1 |
| Phenolic Compounds (non-chlorinated) | EPA 625/EPA 8270 | µg/L | 30 |
| Organochlorine Pesticides | EPA 8081/EPA 625 | µg/L | See Note2 |
| Organochlorine Pesticides (low level) | EPA 1699 | pg/L | See Note2 |
| Organophosphorus Pesticides | EPA 8141/EPA 625 | ng/L | See Note2 |
| Pyrethroid Pesticides | EPA 625/EPA 8270 | ng/L | See Note2 |
| PCBs | EPA 8081/EPA 8082 | µg/L | See Note2 |
| PCB (low level) | EPA 1668C | pg/L | See Note3 |
| Microbiologicals |  |  |  |
| Total Coliform | SM 9221 C | MPN/100 mL | 2 |
| Fecal Coliform | SM 9221 C | MPN/100 mL | 2 |
| Enterococcus | EPA 1600/Enterolert | CFU/100 mL | 1 |
| E. Coli | SM 9221 C | MPN/100 mL | 2 |
| Other |  |  |  |
| Cyanide | EPA 335.2 | mg/L | 0.05 |
| Total Chlorine Residual | EPA 330.1/EPA 330.5 | mg/L | 0.2 |
| Toxicity |  |  |  |
| Acute Toxicity | EPA-821-R-02-012 | Pass/Fail | Not Applicable |
| Chronic Toxicity | EPA 821-R-02-013/ EPA-821-R-02-014 | Pass Fail | Not Applicable |

1 Alternate analytical methods may be used, but the use of any method not listed on this table must be approved in advance and in writing by the Caltrans Task Order Manager.

2 See Tables SAP-4.2 through SAP-4.6 for the individual compounds and their associated reporting limits.

3 PCBs are analyzed on a per-congener basis. MDLs are determined by the laboratory for each congener during each analysis.

Table SAP- 4.4 - Compound List/RLs for Organochlorine Pesticides

| Constituent Name | Units | Reporting Limit |
| --- | --- | --- |
| DDTs | ug/L | 0.005 |
| Toxaphene | ug/L | 0.005 |
| Dieldrin | ug/L | 0.005 |
| Chlordane | ug/L | 0.005 |
| DDTs (low level) | pg/L | 5 |
| Toxaphene (low level) | pg/L | 50 |
| Dieldrin (low level) | pg/L | 5 |
| Chlordane (low level) | pg/L | 5 |
| HCHs | µg/L | 0.002 |
| Endosulfan | µg/L | 0.002 |
| Endrin | µg/L | 0.002 |

Appendix B (Quality Assurance Project Plan)

A6.3	Geographical Locations

This section describes the physical locations (sites) at which monitoring will be conducted. All monitoring samples and measurements shall be representative of the monitored volume and characteristics of the monitored discharge at each monitoring site. At select monitoring locations, effluent from BMPs will be used as representative of the water quality of stormwater discharges from BMPs of the same type at other locations. Selection of representative BMP effectiveness monitoring sites are based on existing and proposed BMPs.

Table 4 lists all project monitoring sites, their geographical locations, and the type of monitoring (Receiving Water Monitoring, Region-Specific TMDL Monitoring, Runoff Characterization Monitoring, BMP Effectiveness Monitoring, Conditionally Exempt Non-Stormwater Discharge Monitoring) that will be conducted at each site. Table 4 summarizes the geographical information for all project monitoring sites.

Table 4 - Project Station Information

| Station Code | Station Name | Latitude | Longitude | RWQCB | County | Route | Postmile | Catchment (ha) | Position | Monitoring Type | Watershed Name | TMDL Pollutant List | Monitored 2025-26* | Monitored 2026-27* |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-351 | Lord Ellis Summit | 40.9303 | -123.855 | 1 | Humboldt | 299 | 18.42 | 0.54 | Influent | 6PPD-Q Characterization | Redwood Creek | Sediment | Yes | Planned |
| 1-352 | Lord Ellis Summit | 40.9305 | -126.855 | 1 | Humboldt | 299 | 18.42 | 0.54 | Effluent | 6PPD-Q Effectiveness (Bioswale) | Redwood Creek | Sediment | Yes | Planned |
| 1-362 | Mello Creek | 41.8986 | -124.1365 | 1 | Humboldt | 101 | 37.46 | 0.028 | Effluent | 6PPD-Q Effectiveness (Bioswale) | Smith River | None | Yes | No |
| 1-366 | Mello Creek | 41.8986 | -124.1365 | 1 | Humboldt | 101 | 37.46 | 0.028 | EOP | 6PPD-Q Characterization | Smith River | None | Yes | No |
| 1-344 | Circle Point Curve | 40.9231 | -123.8105 | 1 | Humboldt | 299 | 25.324 | 0.62 | Influent | 6PPD-Q Characterization | Redwood Creek | Sediment | Yes | Yes |
| 1-345 | Circle Point Curve | 40.9231 | -123.8105 | 1 | Humboldt | 299 | 25.324 | 0.62 | Effluent | 6PPD-Q Effectiveness (Bioswale) | Redwood Creek | Sediment | Yes | No |
| 1-321 | Wilson Creek | 41.6053 | -124.1020 | 1 | Del Norte | 101 | 12.74 | 0.47 | EOP | 6PPD-Q Characterization | Wilson Creek | Sediment | Yes | Yes |
| 1-364 | Chad Creek | 40.4068 | -123.9525 | 1 | Humboldt | 254 | 40.32 | 0.115 | EOP | 6PPD-Q Characterization | S. Eel River | Sediment | Yes | Yes |
| 1-353 | Fish Rock Gulch | 38.8056 | -123.5791 | 1 | Mendocino | 1 | 4.64 | 0.369 | Culvert from EOP | 6PPD-Q Characterization | Fish Rock Gulch | None | Yes | Yes |
| 1-354 | Fish Rock Gulch | 38.8056 | -123.5791 | 1 | Mendocino | 1 | 4.64 | 298 | RW Upstream | 6PPD-Q Receiving Water | Fish Rock Gulch | None | Yes | Yes |
| 1-355 | Fish Rock Gulch | 38.8056 | -123.5791 | 1 | Mendocino | 1 | 4.64 | 298 | RW Downstream | 6PPD-Q Receiving Water | Fish Rock Gulch | None | Yes | Yes |
| 1-356 | Creek 58.78 | 39.4056 | -123.8088 | 1 | Mendocino | 1 | 58.78 | 0.369 | Culvert from EOP | 6PPD-Q Effectiveness | Creek 58.78 | None | Yes | Yes |
| 1-357 | Creek 58.78 | 39.4056 | -123.8088 | 1 | Mendocino | 1 | 58.78 | 306 | RW Upstream | 6PPD-Q Runoff Characterization | Creek 58.78 | None | Yes | Yes |
| 1-358 | Creek 58.78 | 39.4056 | -123.8088 | 1 | Mendocino | 1 | 58.78 | 306 | RW Downstream | 6PPD-Q Runoff Characterization | Creek 58.78 | None | Yes | Yes |
| 1-367 | Little River | 39.270820 | -123.788615 | 1 | Mendocino | 1 | 47.72 | TBD | EOP | 6PPD-Q Runoff Characterization – rubberized | Little River | None | Yes | Yes |
| 1-### | Van Dam | 39.273976 | -123.790541 | 1 | Mendocino | 1 | 47.99 | TBD | EOP | 6PPD-Q Runoff Characterization - rubberized | Little River | None | Yes | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | RW Upstream | Receiving Water Impact | Redwood Creek | Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | RW Downstream | Receiving Water Impact | Redwood Creek | Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | EOP | Receiving Water Impact | Redwood Creek | Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | RW Upstream | Receiving Water Impact | Redwood Creek | Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | RW Downstream | Receiving Water Impact | Redwood Creek | Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | EOP | Receiving Water Impact | Redwood Creek | Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | RW Upstream | Receiving Water Impact | Redwood Creek | Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | RW Downstream | Receiving Water Impact | Redwood Creek | Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | EOP | Receiving Water Impact | Redwood Creek | Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | RW Upstream | Receiving Water Impact | Redwood Creek | Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | RW Downstream | Receiving Water Impact | Redwood Creek | Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | EOP | Receiving Water Impact | Redwood Creek | Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | RW Upstream | Receiving Water Impact | Redwood Creek | Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | RW Downstream | Receiving Water Impact | Redwood Creek | Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | EOP | Receiving Water Impact | Redwood Creek | Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | RW Upstream | Receiving Water Impact | Redwood Creek | Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | RW Downstream | Receiving Water Impact | Redwood Creek | Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | EOP | Receiving Water Impact | Redwood Creek | Sediment | Planned | Yes |
| 1-400 | North Coast | 40.35437 | -123.92622 | 1 | Humboldt | 101 | 35.94 | 0.51 | RW Upstream | Receiving Water Impact | South Eel Fork River | Temperature and Sediment | Yes | Yes |
| 1-401 | North Coast | 40.35437 | -123.92622 | 1 | Humboldt | 101 | 35.94 | 0.51 | RW Downstream | Receiving Water Impact | South Eel Fork River | Temperature and Sediment | Yes | Yes |
| 1-402 | North Coast | 40.35437 | -123.92622 | 1 | Humboldt | 101 | 35.94 | 0.51 | EOP | Receiving Water Impact | South Eel Fork River | Temperature and Sediment | Yes | Yes |
| 1-403 | North Coast | 40.26628 | -123.86755 | 1 | Humboldt | 101 | 27.91 | 0.75 | RW Upstream | Receiving Water Impact | South Eel Fork River | Temperature and Sediment | Yes | Yes |
| 1-404 | North Coast | 40.26628 | -123.86755 | 1 | Humboldt | 101 | 27.91 | 0.75 | RW Downstream | Receiving Water Impact | South Eel Fork River | Temperature and Sediment | Yes | Yes |
| 1-405 | North Coast | 40.26628 | -123.86755 | 1 | Humboldt | 101 | 27.91 | 0.75 | EOP | Receiving Water Impact | South Eel Fork River | Temperature and Sediment | Yes | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | RW Upstream | Receiving Water Impact | South Eel Fork River | Temperature and Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | RW Downstream | Receiving Water Impact | South Eel Fork River | Temperature and Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | EOP | Receiving Water Impact | South Eel Fork River | Temperature and Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | RW Upstream | Receiving Water Impact | South Eel Fork River | Temperature and Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | RW Downstream | Receiving Water Impact | South Eel Fork River | Temperature and Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | EOP | Receiving Water Impact | South Eel Fork River | Temperature and Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | RW Upstream | Receiving Water Impact | South Eel Fork River | Temperature and Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | RW Downstream | Receiving Water Impact | South Eel Fork River | Temperature and Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | EOP | Receiving Water Impact | South Eel Fork River | Temperature and Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | RW Upstream | Receiving Water Impact | South Eel Fork River | Temperature and Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | RW Downstream | Receiving Water Impact | South Eel Fork River | Temperature and Sediment | Planned | Yes |
| 1-### | North Coast | TBD | TBD | 1 | Humboldt/ Del Norte | TBD | TBD | TBD | EOP | Receiving Water Impact | South Eel Fork River | Temperature and Sediment | Planned | Yes |
| 2-406 | North Coast | 40.88931 | -123.59966 | 1 | Trinity | 299 | 0.13 | 0.11 | RW Upstream | Receiving Water Impact | Trinity River | Sediment | Yes | Yes |
| 2-407 | North Coast | 40.88931 | -123.59966 | 1 | Trinity | 299 | 0.13 | 0.11 | RW Downstream | Receiving Water Impact | Trinity River | Sediment | Yes | Yes |
| 2-408 | North Coast | 40.88931 | -123.59966 | 1 | Trinity | 299 | 0.13 | 0.11 | EOP | Receiving Water Impact | Trinity River | Sediment | Yes | Yes |
| 2-409 | North Coast | 40.65125 | -122.93997 | 1 | Trinity | 299 | 58.1 | 0.05 | RW Upstream | Receiving Water Impact | Trinity River | Sediment | Yes | Yes |
| 2-410 | North Coast | 40.65125 | -122.93997 | 1 | Trinity | 299 | 58.1 | 0.05 | RW Downstream | Receiving Water Impact | Trinity River | Sediment | Yes | Yes |
| 2-411 | North Coast | 40.65125 | -122.93997 | 1 | Trinity | 299 | 58.1 | 0.05 | EOP | Receiving Water Impact | Trinity River | Sediment | Yes | Yes |
| 2-### | North Coast | TBD | TBD | 1 | Trinity | TBD | TBD | TBD | RW Upstream | Receiving Water Impact | Trinity River | Sediment | Planned | Yes |
| 2-### | North Coast | TBD | TBD | 1 | Trinity | TBD | TBD | TBD | RW Downstream | Receiving Water Impact | Trinity River | Sediment | Planned | Yes |
| 2-### | North Coast | TBD | TBD | 1 | Trinity | TBD | TBD | TBD | EOP | Receiving Water Impact | Trinity River | Sediment | Planned | Yes |
| 2-### | North Coast | TBD | TBD | 1 | Trinity | TBD | TBD | TBD | RW Upstream | Receiving Water Impact | Trinity River | Sediment | Planned | Yes |
| 2-### | North Coast | TBD | TBD | 1 | Trinity | TBD | TBD | TBD | RW Downstream | Receiving Water Impact | Trinity River | Sediment | Planned | Yes |
| 2-### | North Coast | TBD | TBD | 1 | Trinity | TBD | TBD | TBD | EOP | Receiving Water Impact | Trinity River | Sediment | Planned | Yes |
| 2-### | North Coast | TBD | TBD | 1 | Trinity | TBD | TBD | TBD | RW Upstream | Receiving Water Impact | Trinity River | Sediment | Planned | Yes |
| 2-### | North Coast | TBD | TBD | 1 | Trinity | TBD | TBD | TBD | RW Downstream | Receiving Water Impact | Trinity River | Sediment | Planned | Yes |
| 2-### | North Coast | TBD | TBD | 1 | Trinity | TBD | TBD | TBD | EOP | Receiving Water Impact | Trinity River | Sediment | Planned | Yes |
| 2-### | North Coast | TBD | TBD | 1 | Trinity | TBD | TBD | TBD | RW Upstream | Receiving Water Impact | Trinity River | Sediment | Planned | Yes |
| 2-### | North Coast | TBD | TBD | 1 | Trinity | TBD | TBD | TBD | RW Downstream | Receiving Water Impact | Trinity River | Sediment | Planned | Yes |
| 2-### | North Coast | TBD | TBD | 1 | Trinity | TBD | TBD | TBD | EOP | Receiving Water Impact | Trinity River | Sediment | Planned | Yes |
| 4-470 | Willow Pass HDS Influent | 38.01308 | -121.99736 | 2 | Contra Costa | 4 | R16.83 | 46.9 | Influent | HDS influent | San Francisco Bay | PCBs, Mercury | Planned | Yes |
| 4-471 | Willow Pass HDS Effluent | 38.01306 | -121.99745 | 2 | Contra Costa | 4 | R16.83 | 46.9 | Effluent | HDS effluent | San Francisco Bay | PCBs, Mercury | Planned | Yes |
| 4-472 | Coyote Creek HDS Influent | 37.18977 | -121.69456 | 2 | Santa Clara | 101 | R21.26 | 32.46 | Influent | HDS influent | San Francisco Bay | PCBs, Mercury | Planned | Yes |
| 4-473 | Coyote Creek HDS Effluent | 37.18971 | -121.69449 | 2 | Santa Clara | 101 | R21.26 | 32.46 | Effluent | HDS effluent | San Francisco Bay | PCBs, Mercury | Planned | Yes |
| 4-474 | Edgewood Rd HDS Influent | 37.47124 | -122.29536 | 2 | San Mateo | 280 | 6.98 | 41.04 | Influent | HDS influent | San Francisco Bay | PCBs, Mercury | Planned | Yes |
| 4-475 | Edgewood Rd HDS Effluent | 37.47123 | -122.29533 | 2 | San Mateo | 280 | 6.98 | 41.04 | Effluent | HDS effluent | San Francisco Bay | PCBs, Mercury | Planned | Yes |
| 4-461 | SFBAY (MGSRD Influent) | 37.31523 | -121.91115 | 4 | Santa Clara | 280 | R3.8 EB | 34.9 | Influent | MGSRD influent | San Francisco Bay | PCBs, Mercury | Planned | Yes |
| 4-462 | SFBAY (MGSRD Effluent) | 37.31523 | -121.91140 | 4 | Santa Clara | 280 | R3.8 EB | 34.9 | Effluent | MGSRD effluent | San Francisco Bay | PCBs, Mercury | Planned | Yes |
| 6-314 | North Fork Road Bridge | 36.99214 | -119.71379 | 5 | Madera | North Fork Road | 700 feet NW of Fraint Cove Park and Ride | 0.161874 | Pipe | 6PPD-Q Characterization | San Joaquin River | 6PPD-Q | Yes | Planned |
| 6-315 | SR-41 Bridge | 36.87460 | -119.79230 | 5 | Fresno | 41 | 33.102 | 0.1011714 | Inlet | 6PPD-Q Characterization | San Joaquin River | 6PPD-Q | Yes | Planned |
| 6-316 | SR-99 Bridge | 36.84200 | -119.93159 | 5 | Fresno | 99 | 31.52 | 8.1503 | Pipe | 6PPD-Q Characterization | San Joaquin River | 6PPD-Q | Yes | Planned |
| 7-127 | Site 2, Interstate 210 mile post 40.8 | 34.12 | -117.89 | 4 | Los Angeles | 210 | 40.8 | 0.4 | EOP | Characterization | San Gabriel River | Nitrogen, Phosphorus, Copper, Lead, Zinc, Chlordane, DDT, PCBs, Mercury, Dieldrin, PAHs | Yes | No |
| 7-380 | SAD1060 | 34.0156 | -118.823 | 4 | Los Angeles | 1 | 55.2 | 3.181 | EOP | Characterization | Santa Monica Bay | Mercury, Bacterial Indicators | Yes | Yes |
| 7-421 | Legg Lake Characterization | 34.03978 | -118.06391 | 4 | Los Angeles | 164 | 3.24 | 0.6 | Pipe outlet | Characterization | Part B-Los Angeles Area North, Center & Legg Lake | Nitrogen, Phosphorus | Yes | Unknown |
| 7-422 | Peck Road Park Lake SF Influent | 34.13572 | -117.98787 | 4 | Los Angeles | 210 | 34.66 | 2.35 | Influent | Austin Sand Filter | Los Angeles Area Peck Road Park Lake | Nitrogen, Phosphorus, Chlordane, DDT, Dieldrin, and PCBs | Yes | Yes |
| 7-423 | Peck Road Park Lake SF Effluent | 34.13572 | -117.98787 | 4 | Los Angeles | 210 | 34.66 | 2.35 | Effluent | Austin Sand Filter | Los Angeles Area Peck Road Park Lake | Nitrogen, Phosphorus, Chlordane, DDT, Dieldrin, and PCBs | Yes | Yes |
| 7-425 | Machado Lake Characterization | 33.79089 | -118.29129 | 4 | Los Angeles | 1 | 12.15 | 0.55 | EOP | Characterization | Machado Lake | Nutrients /Pesticides and PCBs | Yes | Yes |
| 7-427 | Maxella Ave Characterization | 33.98649 | -118.44431 | 4 | Los Angeles | 90 | 31.45 | 0.84 | EOP | Characterization | Marina Del Rey Harbor | Toxic Pollutants (Cu, Pb, Zn, Chlordane and Total PCBs) | Yes | Unknown |
| 7-401 | Potrero Valley | 34.108493 | -118.855422 | 4 | Los Angeles | 23 | 7.14 | 0.1 | EOP | Characterization | Lake Sherwood, Santa Monica Bay | DDT, PCBs, Bacterial Indicators, Mercury | No | Planned |
| 7-470 | Route 33 | 34.349710 | -119.299330 | 4 | Ventura | 33 | 5.191 | 0.25 | Inlet | Characterization | Ventura River and its Tributaries | Nutrients | No | Planned |
| 8-313 | I-10 West of Jackson St, FES | 33.73723 | -116.21556 | 7 | Riverside | Inter | 55.812 | 3.5 | Flared End Section Outlet | Characterization | Coachella Valley Storm Water Channel | Bacterial Indicators | Yes | Yes |
| 8-314 | I-10 East of Jackson St, Inlet | 33.73828 | -116.2185 | 7 | Riverside | Inter | 55.629 | 1 | Drop Inlet in Median | Characterization | Coachella Valley Storm Water Channel | Bacterial Indicators | Yes | Yes |
| 8-315 | I-10 West of Monroe St, Inlet | 33.73882 | -116.22513 | 7 | Riverside | Inter | 55.25 | 1.1 | Drop Inlet in Median | Characterization | Coachella Valley Storm Water Channel | Bacterial Indicators | Yes | Yes |
| 8-316 | SR-86 near interchange, Inlet | 33.71645 | -116.1924 | 7 | Riverside | State | R23.051 | 1 | Drop Inlet | Characterization | Coachella Valley Storm Water Channel | Bacterial Indicators | Yes | Yes |
| 8-317 | Windy Point / Grout Bay | 34.259648 | -116.942638 | 8 | San Bernardino | SR 18 | 56.848 | 0.0404686 | Pipe | Characterization | Summit Creek | Nutrients for Dry Hydrological Conditions | Yes | Yes |
| 8-321 | Summit Creek | 34.243495 | -116.891312 | 8 | San Bernardino | SR 18 | 50.262 | 0.3520765 | Pipe | Characterization | Big Bear Lake | Nutrients for Dry Hydrological Conditions | Yes | Yes |
| 8-322 | Rathbun Creek | 34.24895 | -116.88298 | 8 | San Bernardino | SR 18 | 51.025 | 0.0404686 | Catch Basin Inlet | Characterization | Big Bear Lake | Nutrients for Dry Hydrological Conditions | Yes | Yes |
| 10-306 | SR 33-13.45-NB | 37.0598 | -121.01632 | 5 | Merced | SR 33 | 13.45 | 0.0526 | Edge of Pavement | Characterization | No TMDL | None | Yes | Planned |
| 10-307 | SR 33-14.33-NB | 37.07254 | -121.01615 | 5 | Merced | SR 33 | 14.33 | 0.0567 | Edge of Pavement | Porous Pavement (HMA-O) | No TMDL | None | Yes | Planned |
| 10-308 | SR 33-14.75-SB | 37.07877 | -121.01619 | 5 | Merced | SR 33 | 14.75 | 0.142 | Edge of Pavement | Porous Pavement (RHMA-O) | No TMDL | None | Yes | Planned |
| 10-309 | SR 33-14.75-NB | 37.07877 | -121.01610 | 5 | Merced | SR 33 | 14-48 NB | 0.028 | Edge of Pavement | Porous Pavement (RHMA-O-MG) | No TMDL | None | Yes | Planned |
| 11-355 | RW-NF | 32.71747 | -117.11505 | 9 | San Diego | 94 | 3.3 | 2286 | RW | Receiving Water | Chollas Creek | Hardness, TSS, Field Parameters | Yes | Unknown |
| 11-356 | RW-SF | 32.72306 | -117.07598 | 9 | San Diego | 94 | 5.78 | 567 | RW | Receiving Water | Chollas Creek | Hardness, TSS, Field Parameters | Yes | Unknown |
| 11-357 | 94E/College-INF | 32.7404 | -117.05211 | 9 | San Diego | 94 | 7.5 | 1.2626 | Influent | Biofiltration Swale | Chollas Creek | Dissolved Copper, Dissolved Lead, Dissolved Zinc, Total Diazinon | Yes | Unknown |
| 11-358 | 94E/College-EFF | 32.73943 | -117.05318 | 9 | San Diego | 94 | 7.5 | 1.2626 | Effluent | Biofiltration Swale | Chollas Creek | Dissolved Copper, Dissolved Lead, Dissolved Zinc, Total Diazinon | Yes | Unknown |
| 11-359 | 94E/Median ASF-INF | 32.71663 | -117.11878 | 9 | San Diego | 94 | 3 | 0.855 | Influent | Austin Sand Filter | Chollas Creek | Dissolved Copper, Dissolved Lead, Dissolved Zinc, Total Diazinon | Yes | Unknown |
| 11-360 | 94E/Median ASF-EFF | 32.71677 | -117.11844 | 9 | San Diego | 94 | 3 | 0.855 | Effluent | Austin Sand Filter | Chollas Creek | Dissolved Copper, Dissolved Lead, Dissolved Zinc, Total Diazinon | Yes | Unknown |
| 12-366 | Site 02 - NB SR55 and Baker Street offramp, north of SR73 | 33.686624 | -117.86939 | 8 | Orange | SR 405 | 8.49 | 4 | Runoff | Characterization | San Diego Creek/ Newport Bay | Cadmium, DDT, Chlordane, PCB, Copper, Lead, Zinc | Yes | Planned |
| 12-367 | Site 07 - NB SR55 Hospital Rd to first upstream shoulder inlet | 33.625867 | -117.927797 | 8 | Orange | SR 55 | 0.48 | 1 | Runoff | Characterization | San Diego Creek/ Newport Bay | Cadmium, DDT, Chlordane, PCB, Copper, Lead, Zinc | Yes | Planned |
| 12-369 | Site 09 - NB SR405 Between Bristol St and South Coast Drive | 33.688011 | -117.884574 | 8 | Orange | SR 405 | 9.46 | 7.5 | Runoff | Characterization | San Diego Creek/ Newport Bay | Cadmium, DDT, Chlordane, PCB, Copper, Lead, Zinc | Yes | Planned |
| 12-370 | Site 09 Alt- NB SR405 Between Bristol St and South Coast Drive (alt to station 12-369) | 33.688011 | -117.884574 | 8 | Orange | SR 405 | 9.46 | 1.6 | Runoff | Characterization | San Diego Creek/ Newport Bay | Cadmium, DDT, Chlordane, PCB, Copper, Lead, Zinc | Yes | Planned |
| 12-371 | Site 09 Control Point - South Coast Plaza Parking Lot | 33.688222 | -117.886944 | 8 | Orange | SR 405 | 9.46 | 3 | Runoff | Characterization | San Diego Creek/ Newport Bay | Cadmium, DDT, Chlordane, PCB, Copper, Lead, Zinc | Yes | Planned |

Planned - Monitoring at these sites will depend on a review of monitoring data collected to date.

Unknown - The decision to monitor these sites during the 2025-26 storm season will be made after the current season, when the total number of usable data points for the site are known.

HDS – Hydrodynamic Separator

MGSRD – Multiple Gross Solids Removal Device

HMA-O – Hot Mix Asphalt Open-Graded

PCP – Pervious Concrete Pavement

PAB – Permeable Aggregate Base

B1.3	Sample Count and Types

Table 5 lists the sample counts, sample types, and required analyses for each project site.  For samples collected in the San Francisco Bay PCBs TMDL area, Caltrans has selected a list of 40 PCB congeners that will be analyzed for all samples; similarly, Caltrans has selected the 55 PCB congeners listed in the Water Quality Control Plan for Enclosed Bays and Estuaries – Part 1, Sediment Quality Provisions to be analyzed for all samples collected in the Los Angeles Region.  The respective lists of congeners are included as footnotes to Table 5.

In addition to the analyses listed in Table 5, field tests will be conducted at every monitoring site during every event.  Field testing shall include temperature, pH, conductivity, turbidity, and dissolved oxygen.

Table 5 - Sample Counts, Types, and Analyses

| Station Code/Name | Analytical Constituent | Sample Type | Sample Frequency (per Year) |
| --- | --- | --- | --- |
| North Coast 6PPD-Q Characterization/ Receiving Water (8 stations); see Table 4 | TSS, SSC, Turbidity, 6PPD-Q | Grab | 3 |
| North Coast 6PPD-Q BMP Influent/Effluent (6 stations); see Table 4 | TSS, SSC, Turbidity, 6PPD-Q | Composite/Grab | 3 |
| North Coast Receiving Water Impact (54 stations); see Table 4 | TSS, SSC, Turbidity | Grab | 3 |
| SF BAY HDS (6 stations); see Table 4 | PCB3, Hg, SSC, TSS, PAH, Nutrients, Metals, organochlorine pesticides | Composite/Grab | 3 |
| SF BAY MGSRD (2 Stations); see Table 4 | Sediment test only: PCB3, Hg, TOC, % fines | Grab | 4 |
| District 6 6PPD-Q Characterization (3 stations); see Table 4 | TSS, SSC, Turbidity, 6PPD-Q | Grab | 3 |

| Station Code/Name | Analytical Constituent | Sample Type | Sample Frequency (per Year) |
| --- | --- | --- | --- |
| 7-127 | Nitrate as N | Composite | 2 |
|  | Nitrite as N | Composite | 2 |
|  | TKN | Composite | 2 |
|  | Phosphorus as P | Composite | 2 |
|  | Copper | Composite | 2 |
|  | Lead | Composite | 2 |
|  | Mercury | Grab | 2 |
|  | Zinc | Composite | 2 |
|  | Chlordane | Composite | 2 |
|  | DDTs, DDDs, DDEs (all forms) | Composite | 2 |
|  | PCBs2 | Composite | 2 |
|  | PAHs | Composite | 2 |
|  | Dieldrin | Composite | 2 |
|  | TSS | Composite | 2 |
|  | SSC | Grab | 2 |
|  | Total Organic Carbon | Composite | 2 |
|  | Dissolved Organic Carbon | Composite | 2 |
| 7-380 | Mercury | Grab | 2 |
|  | Bacterial Indicators (total coliform, fecal coliform, enterococcus) | Grab | 2 |
| 7-421 | Nitrate as N | Composite | 2 |
|  | Nitrite as N | Composite | 2 |
|  | TKN | Composite | 2 |
|  | Phosphorus as P | Composite | 2 |
|  | Bacterial Indicators (E.coli) | Grab | 2 |
| 7-422 |  |  |  |  |  |  |  |
|  | Chlordane | Composite | 2 |
|  | Dieldrin | Composite | 2 |
|  | PCBs2 | Composite | 2 |
|  | TSS | Composite | 2 |
|  | Bacterial Indicators (E.coli) | Grab | 2 |

| Station Code/Name | Analytical Constituent | Sample Type | Sample Frequency (per Year) |
| --- | --- | --- | --- |
| 7-423 |  |  |  |  |  |  |  |
|  | Chlordane | Composite | 2 |
|  | Dieldrin | Composite | 2 |
|  | PCBs2 | Composite | 2 |
|  | TSS | Composite | 2 |
|  | Bacterial Indicators (E.coli) | Grab | 2 |
| 7-425 |  |  |  |  |  |  |  |
|  | Chlordane (low level) | Composite | 2 |
|  | Dieldrin (low level) | Composite | 2 |
|  | DDTs, DDDs, DDEs (all forms low level) | Composite | 2 |
|  | PCBs2 | Composite | 2 |
|  | TSS | Composite | 2 |
| 7-427 |  |  |  |  |  |  |  |
|  | Chlordane | Composite | 2 |
|  | PCBs2 | Composite | 2 |
|  | TSS | Composite | 2 |
|  |  | Bacterial Indicators (total coliform, fecal coliform, enterococcus) |  | Grab |  | 2 |  |
| 7-401 | Mercury Constituents representative of other watersheds (to be determined) | Composite/Grab | 2 |

| Station Code/Name | Analytical Constituent | Sample Type | Sample Frequency (per Year) |
| --- | --- | --- | --- |
| 7-470 | Nitrate as N | Composite | 2 |
|  | Nitrite as N | Composite | 2 |
|  | TKN | Composite | 2 |
|  | Phosphorus as P | Composite | 2 |
|  | Constituents representative of other watersheds (to be determined) | Composite/Grab | 2 |
| 8-313 | Total Coliform | Grab | 2 |
|  | Fecal Coliform | Grab | 2 |
|  | E. Coli | Grab | 2 |
|  | Enterococcus | Grab | 2 |
| 8-314 | Total Coliform | Grab | 2 |
|  | Fecal Coliform | Grab | 2 |
|  | E. Coli | Grab | 2 |
|  | Enterococcus | Grab | 2 |
| 8-315 | Total Coliform | Grab | 2 |
|  | Fecal Coliform | Grab | 2 |
|  | E. Coli | Grab | 2 |
|  | Enterococcus | Grab | 2 |
| 8-316 | Total Coliform | Grab | 2 |
|  | Fecal Coliform | Grab | 2 |
|  | E. Coli | Grab | 2 |
|  | Enterococcus | Grab | 2 |
| 8-317 | Nitrate as N | Composite | 2 |
|  | Nitrite as N | Composite | 2 |
|  | TKN | Composite | 2 |
|  | Phosphorus as P | Composite | 2 |
|  | TSS | Composite | 2 |
| 8-321 | Nitrate as N | Composite | 2 |
|  | Nitrite as N | Composite | 2 |
|  | TKN | Composite | 2 |
|  | Phosphorus as P | Composite | 2 |
|  | TSS | Composite | 2 |
| 8-322 | Nitrate as N | Composite | 2 |
|  | Nitrite as N | Composite | 2 |
|  | TKN | Composite | 2 |
|  | Phosphorus as P | Composite | 2 |
|  | TSS | Composite | 2 |

| Station Code/Name | Analytical Constituent | Sample Type | Sample Frequency (per Year) |
| --- | --- | --- | --- |
| 10-306 | Cadmium | Composite | 3 |
|  | Chromium | Composite | 3 |
|  | Copper | Composite | 3 |
|  | Lead | Composite | 3 |
|  | Mercury | Composite | 3 |
|  | Zinc | Composite | 3 |
|  | Hardness as CaCO3 | Composite | 3 |
|  | Nitrate as N | Composite | 3 |
|  | Nitrite as N | Composite | 3 |
|  | TKN | Composite | 3 |
|  | Phosphorus as P | Composite | 3 |
|  | PCBs1 | Composite | 3 |
|  | TSS | Composite | 3 |
|  | SSC | Grab | 3 |
|  | Total Organic Carbon | Composite | 3 |
|  | Dissolved Organic Carbon | Composite | 3 |
| 10-307 | Cadmium | Composite | 3 |
|  | Chromium | Composite | 3 |
|  | Copper | Composite | 3 |
|  | Lead | Composite | 3 |
|  | Mercury | Composite | 3 |
|  | Zinc | Composite | 3 |
|  | Hardness as CaCO3 | Composite | 3 |
|  | Nitrate as N | Composite | 3 |
|  | Nitrite as N | Composite | 3 |
|  | TKN | Composite | 3 |
|  | Phosphorus as P | Composite | 3 |
|  | PCBs1 | Composite | 3 |
|  | TSS | Composite | 3 |
|  | SSC | Grab | 3 |
|  | Total Organic Carbon | Composite | 3 |
|  | Dissolved Organic Carbon | Composite | 3 |

| Station Code/Name | Analytical Constituent | Sample Type | Sample Frequency (per Year) |
| --- | --- | --- | --- |
| 10-308 | Cadmium | Composite | 3 |
|  | Chromium | Composite | 3 |
|  | Copper | Composite | 3 |
|  | Lead | Composite | 3 |
|  | Mercury | Composite | 3 |
|  | Zinc | Composite | 3 |
|  | Hardness as CaCO3 | Composite | 3 |
|  | Nitrate as N | Composite | 3 |
|  | Nitrite as N | Composite | 3 |
|  | TKN | Composite | 3 |
|  | Phosphorus as P | Composite | 3 |
|  | PCBs1 | Composite | 3 |
|  | TSS | Composite | 3 |
|  | SSC | Grab | 3 |
|  | Total Organic Carbon | Composite | 3 |
|  | Dissolved Organic Carbon | Composite | 3 |
| 10-309 | Cadmium | Composite | 3 |
|  | Chromium | Composite | 3 |
|  | Copper | Composite | 3 |
|  | Lead | Composite | 3 |
|  | Mercury | Composite | 3 |
|  | Zinc | Composite | 3 |
|  | Hardness as CaCO3 | Composite | 3 |
|  | Nitrate as N | Composite | 3 |
|  | Nitrite as N | Composite | 3 |
|  | TKN | Composite | 3 |
|  | Phosphorus as P | Composite | 3 |
|  | PCBs1 | Composite | 3 |
|  | TSS | Composite | 3 |
|  | SSC | Grab | 3 |
|  | Total Organic Carbon | Composite | 3 |
|  | Dissolved Organic Carbon | Composite | 3 |
| 11-355 | TSS | Composite | 1 |
|  | Hardness | Composite | 1 |
| 11-356 | TSS | Composite | 1 |
|  | Hardness | Composite | 1 |

| Station Code/Name | Analytical Constituent | Sample Type | Sample Frequency (per Year) |
| --- | --- | --- | --- |
| 11-357 | Dissolved Copper | Composite | 1 |
|  | Dissolved Lead | Composite | 1 |
|  | Dissolved Zinc | Composite | 1 |
|  | Diazinon | Composite | 1 |
| 11-358 | Dissolved Copper | Composite | 1 |
|  | Dissolved Lead | Composite | 1 |
|  | Dissolved Zinc | Composite | 1 |
|  | Diazinon | Composite | 1 |
| 11-359 | Dissolved Copper | Composite | 1 |
|  | Dissolved Lead | Composite | 1 |
|  | Dissolved Zinc | Composite | 1 |
|  | Diazinon | Composite | 1 |
| 11-360 | Dissolved Copper | Composite | 1 |
|  | Dissolved Lead | Composite | 1 |
|  | Dissolved Zinc | Composite | 1 |
|  | Diazinon | Composite | 1 |
| 12-366 | Cadmium, DDT, Chlordane, PCBs2, Copper, Lead, Zinc | Composite | 3 |
| 12-367 | Cadmium, DDT, Chlordane, PCBs2, Copper, Lead, Zinc | Composite | 3 |
| 12-369 | Cadmium, DDT, Chlordane, PCBs2, Copper, Lead, Zinc | Composite | 3 |
| 12-370 |  | Cadmium, DDT, Chlordane, PCBs2, Copper, Lead, Zinc |  | Composite |  | 3 |  |
| 12-371 |  | Cadmium, DDT, Chlordane, PCBs2, Copper, Lead, Zinc |  | Composite |  | 3 |  |

1Standard PCBs: Aroclors 1016 ,1221 ,1232 ,1242 ,1248, 1254, 1260

255 PCB congeners: 8, 18, 27, 28, 29, 31, 33, 44, 49, 52, 56, 60, 64, 66, 70, 74, 77, 87, 95, 97, 99, 101, 105, 110, 114, 118, 126, 128, 137, 138, 141, 146, 149, 151, 153, 156, 157, 158, 169, 170, 174, 177, 180, 183, 187, 189, 194, 195, 198, 199, 200, 201, 203, 206, and 209.

3PCBs 40 Congeners SF Bay List: PCB 004, 087, 153, 008, 095, 156, 018, 097, 158, 028, 099, 170, 031, 101, 174, 033, 105, 177, 044, 110, 180, 049, 118, 183, 052, 128, 187, 056, 132, 194, 060, 138, 195, 066, 141, 201, 070, 149, 203, 074, 151

PAHs: Acenaphthene, Chrysene, Acenaphthylene, Dibenzo(a,h)anthracene, Anthracene, Fluoranthene, Benzo(a)anthracene, Fluorene,Benzo(a)pyrene, Indeno(1,2,3-cd)pyrene, Benzo(b)fluoranthene, Naphthalene, Benzo(ghi)perylene, Phenanthrene, Benzo(k)fluoranthene, Pyrene

B2.1	Collection Methods

Sample collection will be performed using the methods provided in the project SAP (Appendix A). Methods in the SAP were developed in accordance with the guidelines in Chapters 4, 8, and 11 of the Stormwater Monitoring Guidance Manual (Caltrans 2020).

In general, composite samples will be collected using the methods provided in Section SAP-1.2 of Appendix A. Grab samples will be collected using the methods provided in Section SAP-1.3 of Appendix A.

Time-Weighted Sample Collection and Compositing at TMDL Sites

Time-weighted composite samples for TMDL sites may be collected using the following procedures.

Grab samples are collected manually and combined into a single composite sample in accordance with the procedure outlined in Section SAP-1.3 of Appendix A. Grab samples will be collected according to the following strategy:

- Storms with a duration greater than three hours: Samples shall be collected manually by taking at least three discrete grab samples during each of the first three hours of discharge.

- Storms with a duration of three hours or less: at least three discrete grab samples shall be collected during each hour of discharge for the entire duration of the storm event.

Samples shall be collected between 15 and 20 minutes apart. The actual time that it takes to collect an individual grab sample will not exceed 15 minutes. The first flow measurement will be taken approximately 10 minutes after the discharge has stabilized and the first sample will be collected approximately 15 minutes after the discharge has stabilized. The discharge will be considered to have stabilized when flow rate and turbidity appear to be reasonably constant and/or sheet flow is well established.

Field crews will take instantaneous flow measurements at the sample collection point prior to the collection of each grab sample. Flow measurements will be made using the measurement methods described in Section SAP-1.3.3 of Appendix A. One flow measurement will be made approximately 5 to 10 minutes prior to the collection of each grab sample, so that each grab sample is associated with a corresponding flow measurement.

At the end of the monitoring event, field technicians will use the flow measurements to prepare a compositing scheme, as shown in Figure 2. Field technicians or office staff will prepare this compositing scheme using the tool that has been provided by Caltrans for this purpose. Each grab sample must be associated with a corresponding flow measurement in order to us this tool correctly.

The field crew will then send all discrete grab samples to the laboratory along with the compositing scheme. The laboratory will use this flow compositing scheme to combine all discrete grab samples into a single composite sample for analysis.

Samples such as coliform bacteria and oil & grease must be collected as single discrete samples. These samples will be collected sometime during 45 to 90 minutes after start of sampling, or if the event lasts more than 3 hours, approximately in the middle of the sampling event. These tests will be performed on individual grab samples, not on the composite sample.

Table 6 - Example of a Sample Compositing Scheme for a Standard 3-Hour Event

|  | TOTAL SAMPLE VOLUME REQUIRED (Vc) |  |  |  | 20000 | mL |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DISCREET GRAB SIZE (Vgrab) |  |  | 4000 | mL |  |  |
| Aliquot # | Time of Flow Measurement | Time of Grab Sample | Measured Flow Rate (L/min) | Time Interval Between Flow Measurements (min) | Averaged Flow Rate (L/min) | Flow Volume (L) | Unadjusted Aliquot Volume | Final Aliquot Volume |
| -- | 00:00 | -- | 92 | -- | -- | -- | -- | -- |
| 1 | 00:18 | 00:20 | 100 | 18 | 96 | 1728 | 840 | 818 |
| 2 | 00:36 | 00:41 | 115 | 18 | 108 | 1935 | 941 | 916 |
| 3 | 00:57 | 00:59 | 201 | 21 | 158 | 3318 | 1613 | 1571 |
| 4 | 01:16 | 01:20 | 330 | 19 | 266 | 5045 | 2453 | 2388 |
| 5 | 01:30 | 01:33 | 401 | 14 | 366 | 5117 | 2488 | 2423 |
| 6 | 01:52 | 01:57 | 367 | 22 | 384 | 8448 | 4108 | 4000 |
| 7 | 02:08 | 02:10 | 209 | 16 | 288 | 4608 | 2241 | 2182 |
| 8 | 02:25 | 02:30 | 301 | 17 | 255 | 4335 | 2108 | 2053 |
| 9 | 02:47 | 02:54 | 299 | 22 | 300 | 6600 | 3209 | 3125 |
|  |  |  |  |  | Final Composite Volume: |  |  | 19476 |

Note: The start time of 00:00 reflects the time approximately 10 minutes after the flow appears to have stabilized, based on visual observation. During a storm event, actual times of day will be placed in these cells.

The volume of each grab sample that should be used to make the final composite (shown in Column 8 of Figure 2) is calculated as follows. These equations assume that the volume of the final composite will be 20 liters (20,000 ml) and that the target volume for each individual grab sample is 4 liters (4,000 ml).

First, for each discrete grab sample collected, determine the volume of discharge that it represents.

Equation B2.1

Flow Volume (L) = ∆Time * ((dQi t2 - dQi t1) / 2)

Where:	 Flow Volume = Volume of flow represented by the discrete grab sample

∆Time = Interval in minutes between the current and the previous instantaneous flow measurements

dQi t2 = Instantaneous flow measurement (L/min) associated with this grab sample

dQi t1 = Previous instantaneous flow measurement (L/min)

Next, determine the volume of each discrete grab sample to be used as an aliquot in the final composite sample. This is an unadjusted volume based on the assumption that all discrete grabs will have sufficient volume.

Equation B2.2

Unadjusted Aliquot Volume (mL) = Flow Volume * Final Composite Volume / ∑dQi

Where: 	Initial Aliquot Volume = Volume of the discrete grab used in the composite (mL)

Flow Volume = Flow volume for this sample, from Equation B2.1

Final Composite Volume = Target volume of the final composite (ml)

∑dQi = Sum of all flow volumes (L)

If all individual grab samples have sufficient volume, then these volumes will be used to make the final composite However, if some grab samples do not have sufficient volume, then adjusted volumes must be calculated so that a flow-proportioned composite can be made accurately. The tool provided by Caltrans does this job automatically using the following formula:

Equation B2.3

Final Aliquot Volume (ml) = Initial Aliquot Volume * Grab Volumetarget / Grab Volumemax

Where: 	Final Aliquot Volume = Volume of the discrete grab that will be used in the composite

Initial Aliquot Volume = Volume of the aliquot calculated in Equation B2.2

Grab Volumetarget = Target volume for discrete grab samples

Grab Volumemax = Volume of largest discrete grab sample or Grab Volumetarget, whichever is greater.

In the example shown in Figure 2, the final required volumes are shown in Column 9. These are the final volumes of each discrete grab sample that will be used to make the composite sample.

Receiving Water Sample Collection

Receiving water samples will be collected using the grab sample methods provided in Section SAP-1.3 of Appendix A. Time-weighted grab samples will be collected at three monitoring points at each location:

- From the receiving waterbody, upstream

- From the receiving waterbody, downstream

- From an edge-of-pavement (EOP) discharge location prior to the point at which it enters the receiving waterbody

In addition to sample collection, an onsite rain gauge will be installed at each location where receiving water monitoring will be conducted. Rainfall data will be recorded by the field technicians in their field logs.

Samples will be collected at the upstream and downstream sampling points directly into sample bottles provided by the laboratory. Each discrete sample will consist of two one-liter wide mouth HDPE sample bottles filled completely.

Samples will be collected as close as possible to the bottom of the water column to represent the bedload as accurately as possible. In wadable streams, field crews may opt to wade into the waterbody and immerse the bottles by hand. Where the waterbody is too deep or difficult to wade, or where the field team believes that this procedure may not be safe, sample bottles will be affixed to a pole and lowered into the stream.

The downstream receiving water samples must be collected at a point at or beyond the full mixing point. When monitoring turbulent waterbodies, these samples must be collected at about five stream widths downstream from where the discharge enters the stream. If the field technicians note any pooling of water in this area, samples must be collected from a point downstream of where the pooling occurs.

EOP samples will be collected manually by holding the uncapped sample container in the runoff stream and direct-filling.

The intent of receiving water monitoring is to characterize the impact on the waterbody of the rising and peak discharge portions of the hydrograph. Because storms vary widely in their duration, it may be necessary to modify the timing of sample collection based on the anticipated time span of the storm.

For storms anticipated to last three hours or more:

- Field crews will attempt to represent the “first flush” of discharge at both the receiving water and the EOP sites.

- Samples will be collected over the first three hours of the storm.

- One set of samples will be collected approximately every 15 minutes. The target is a total of 12 sets of samples collected.

- At each 15-minute interval, all three monitoring points (receiving water and EOP) will be collected as close together as possible, so that each set of samples represents a single point in time.

For storms with an anticipated duration of less than three hours:

- Field crews will attempt to represent the “first flush” of discharge at both the receiving water and the EOP sites.

- Samples will be collected over the entire duration of the storm.

- As with longer storms, the target is a total of 12 sets of samples collected. When planning the storm event, field technicians will use best professional judgement to anticipate what sample collection frequency will be required to reach this target.

As with longer storms, all three monitoring points (receiving water and EOP) will be collected as close together as possible, so that each set of samples represents a single point in time.

### B5.1	Quality Control

The quality of laboratory data is evaluated using the MQOs discussed in Section A7. Several kinds of samples, called QC samples are analyzed by the laboratory, and the results of these analyses are compared to the appropriate MQOs to verify that the data are of acceptable quality.

Some types of QC samples are produced in the field and submitted to the laboratory along with the normal field samples. Other types of QC samples are generated by the laboratory. The following sections discuss the various types of QC samples, their uses, and where they originate.

Section B5.1 of this document describes the various kinds of QC samples used for Caltrans stormwater monitoring projects. Table 9 shows the frequencies with which field QC samples will be collected. Laboratory QC samples shall be analyzed per laboratory SOP.

Table 9 - QC Sample Schedule

|  | Field Blanks | Field Duplicates | Matrix Spikes | Equipment Blanks |
| --- | --- | --- | --- | --- |
| Field |  |  |  |  |
| pH | None | 1 per event | None | None |
| Temperature | None | 1 per event | None | None |
| Dissolved Oxygen | None | 1 per event | None | None |
| Flow | None | None | None | None |
| Conventionals |  |  |  |  |
| Turbidity | 1 per 20 samples | 1 per 20 samples | None | None |
| Hardness as CaCO3 | 1 per 20 samples | 1 per 20 samples | 1 per 20 samples | None |
| Total Dissolved Solids | 1 per 20 samples | 1 per 20 samples | None | None |
| Total Suspended Solids | 1 per 20 samples | 1 per 20 samples | None | None |
| Nutrients |  |  |  |  |
| Kjeldahl Nitrogen | 1 per 20 samples | 1 per 20 samples | 1 per 20 samples | 1 per cleaning batch |
| Nitrate as N | 1 per 20 samples | 1 per 20 samples | 1 per 20 samples | 1 per cleaning batch |
| Nitrite | 1 per 20 samples | 1 per 20 samples | 1 per 20 samples | 1 per cleaning batch |
| Ammonia as N | 1 per 20 samples | 1 per 20 samples | 1 per 20 samples | 1 per cleaning batch |
| Phosphorus, Total | 1 per 20 samples | 1 per 20 samples | 1 per 20 samples | 1 per cleaning batch |
| Metals |  |  |  |  |
| Aluminum | 1 per 20 samples | 1 per 20 samples | 1 per 20 samples | 1 per cleaning batch |
| Chromium | 1 per 20 samples | 1 per 20 samples | 1 per 20 samples | 1 per cleaning batch |
| Copper | 1 per 20 samples | 1 per 20 samples | 1 per 20 samples | 1 per cleaning batch |
| Iron | 1 per 20 samples | 1 per 20 samples | 1 per 20 samples | 1 per cleaning batch |
| Lead | 1 per 20 samples | 1 per 20 samples | 1 per 20 samples | 1 per cleaning batch |
| Mercury | 1 per 20 samples | 1 per 20 samples | 1 per 20 samples | 1 per cleaning batch |
| Selenium | 1 per 20 samples | 1 per 20 samples | 1 per 20 samples | 1 per cleaning batch |
| Zinc | 1 per 20 samples | 1 per 20 samples | 1 per 20 samples | 1 per cleaning batch |
| Organics |  |  |  |  |
| 6PPD-Quinone | 1 per 20 samples | 1 per 20 samples | Note 1 | 1 per cleaning batch |
| PAHs | 1 per 20 samples | 1 per 20 samples | 1 per 20 samples | 1 per cleaning batch |
| Pesticides |  |  |  |  |
| DDT | 1 per 20 samples | 1 per 20 samples | 1 per 20 samples | 1 per cleaning batch |
| Chlordane | 1 per 20 samples | 1 per 20 samples | 1 per 20 samples | 1 per cleaning batch |
| Dieldrin | 1 per 20 samples | 1 per 20 samples | 1 per 20 samples | 1 per cleaning batch |
| Toxaphene | 1 per 20 samples | 1 per 20 samples | 1 per 20 samples | 1 per cleaning batch |
| PCBs |  |  |  |  |
| Aroclors | 1 per 20 samples | 1 per 20 samples | 1 per 20 samples | 1 per cleaning batch |
| Microbiologicals |  |  |  |  |
| Fecal Coliform | 1 per 20 samples | 1 per 20 samples | None | None |
| Enterococcus | 1 per 20 samples | 1 per 20 samples | None | None |
| Toxicity |  |  |  |  |
| Toxicity | None | None | None | None |

1 per 20 samples – Each monitoring consultant shall collect 1 sample per every 20 field samples collected on a monitoring project each season. A minimum of 1 set of QC samples per season will be collected by each monitoring consultant.

1 per cleaning batch - Each time a laboratory cleans a batch of monitoring equipment, such as tubing, strainers, and bailers, the lab will select one piece of cleaned equipment and blank-test it.

1MS/MSD for 6PPD-Quinone to be analyzed only when requested by the Task Order Manager