Imperial Valley Watershed

Municipal Stormwater Program

# Stormwater Management Plan

Principal Permittee

- City of Calexico

The permittee gratefully acknowledge and thank the Riverside County Flood Control and Water Conservation District, County of Riverside, and Sempra Energy Utilities for providing wording, resources, and assistance in the preparation of this document.

## January 2009 nfl

## City of Calexico SWMP

## Table of Contents

## Executive Summary									2

Program Structure									3

Section I   – Public Education and Outreach					7

Section II – Public Involvement/Participation

Sub-Section IIA – Residential					12

Sub-Section IIB – Commercial/Industrial Program		18

Section III – Illicit Discharge and Elimination					23

Section IV – Construction Activities						31

Section V -   New Development/Redevelopment Activities			34

Section VI – Pollution Prevention for Municipal Operators			37

Attachment A – Water Quality Construction Manual

Best Management Practices

## City of Calexico SWMP

## Executive Summary

Federal regulations implementing Clean Water Act section 402 (p) require operators of municipal separate storm sewer systems (MS4s) serving urbanized areas with populations of 100,000 or greater to obtain National Pollutant Discharge Elimination System (NPDES) permits for municipal stormwater discharges.  A MS4 is defined as a publicly owned conveyance or system of conveyances - including roadways, catch basins, curbs, gutters, ditches, man-made channels and storm drains – designed or used for collecting and conveying stormwater.  NPDES municipal stormwater permits require MS4 operators (permittee) to 1) effectively prohibit non-stormwater discharges to the MS4 and 2) implement controls to reduce the discharge of pollutants to the maximum extent practicable.

The Stormwater Phase II Final Rule is the next step in EPA’s effort to preserve, protect, and improve the Nation’s water resources from polluted stormwater runoff. The Phase II program expands the Phase I program by requiring additional operators of MS4’s in urbanized areas and operators of small construction sites, through the use of NPDES permits, to implement programs and practices to control stormwater runoff.

The Imperial Valley watershed is generally situated in Imperial County within the Imperial Valley Planning Area of the Colorado River Basin RWQCB.   The watershed is generally defined by the boundaries of the Imperial Hydrologic Unit as described in the Water Quality Control Plan for the Colorado River Basin RWQCB (Basin Plan).  Much of the watershed consists of sparsely populated mountains, desert, and agricultural lands.  Urbanized areas are principally located on the valley floor between Salton City and Calexico along State Highway 86, and from Calipatria to Brawley along State

Highway 111.

The Phase II federal storm water regulations (40 CFR 122.26) require municipal stormwater permittee to develop and implement a comprehensive stormwater management program to reduce pollutants in municipal stormwater discharges to the maximum extent practicable.  In 2003, the Permittee submitted a proposed Stormwater Management Plan (SWMP) to the RWQCB as part of their NPDES Permit application.  The SWMP includes Best Management Practices (BMPs) that are expected to be incorporated into the Permit by reference.

The SWMP presents each major program area as a separate section, including the identification of any proposals for program development during the term of the Permit, and corresponding BMPs.   Where applicable, performance measures are included to assess SWMP implementation and to measure progress toward the achievement of program goals.

City of Calexico SWMP

Program Structure

Introduction & Background

Polluted storm water runoff is often transported to municipal separate storm sewer systems (MS4s) and ultimately discharged into local rivers and streams without treatment.  A storm water management program can help improve the waterways by reducing the quantity of pollutants that storm water picks up and carries into storm sewer systems during storm events.  Common pollutants include oil and grease from roadways, pesticide from lawns, sediment from construction sites, and carelessly discarded trash, such as cigarette butts, paper wrappers, and plastic bottles.  When deposited into nearby waterways through MS4s discharges, these pollutants can impair the waterways, thereby discouraging recreational use of the resource, contaminating drinking water supplies, and interfering with the habitat for fish, other aquatic organisms, and wildlife.

Storm water discharges from MS4s in urbanized areas and due to urban development is a concern because of the high concentration of pollutants found in these discharges.  Concentrated development in urbanized areas substantially increases impervious surfaces, such as city streets, driveways, parking lots, and sidewalks, on which pollutants from concentrated human activities settle and remain until a storm event washes them into nearby storm drains.  Another concern is the possible illicit connections of sanitary sewers, which can result in fecal coliform bacteria entering the storm sewer system.  Storm water runoff picks up and transports these as well as other harmful pollutants, previously mentioned, then discharges them – untreated – to waterways via storm sewer systems.

Phase I of the U.S. Environmental Protection Agency’s (EPA) storm water program was promulgated in 1990 under the Clean Water Act.  Phase I relies on National Pollutant Discharge Elimination System (NPDES) permit coverage to address storm water runoff from (1) “medium” and “large” municipal separate storm sewer systems (MS4s) generally serving populations of 100,000 or greater, (2) construction activity disturbing 5 acres of land or greater, and (3) ten categories of industrial activity.

The Storm Water Phase II Final Rule is the next step in EPA’s effort to preserve, protect, and improve the Nation’s water resources from polluted storm water runoff.  The Phase II program expands the Phase I program by requiring additional operators of MS4’s in urbanized areas and operators of small construction sites to implement programs and practices to control polluted storm water runoff.

City of Calexico SWMP

The Phase II final rule set a deadline of March 10, 2003 to obtain permit coverage.  The final rule ‘automatically’ covers two classes of storm water dischargers on a nationwide basis:

- Operators of small MS4s located in ‘urbanized areas’ as delineated by the Bureau of the Census. A ‘small’ MS4 is any MS4 not already covered by Phase 1 of the NPDES storm water program.

- Operators of small construction activities that disturb equal to or greater than 1 (one) and less than 5 (five) acres of land.

The SWMP outlines Best Management Practices (BMPs) that will be used by the Permittee to manage and control stormwater runoff to the maximum extent practicable.

## Organization

The Permit identifies the City of Calexico and as the Principal Permittee. A working organizational framework developed during the application process which resulted in the Principal Permittee assuming responsibility for coordinating Permittee activities, including report preparation and submittals to the RWQCB.

To underpin the working framework among multiple agencies, the Permittee will be formally adopting an Implementation Agreement.  The Implementation Agreement will reinforce the roles and responsibilities of each Permittee as established by the Permit. Specific provisions of the agreement will include cost sharing for public education activities and water quality monitoring. The implementation agreement will expire coincident with expiration of the first Permit.

This Stormwater Management Plan is an integral component of the NPDES Permit and is organized into the following programs:

Public Education Program (Section I) – Public education is led by the Stormwater/Clean Water Protection Program (SWMP). The media campaign will take advantage of existing countywide resources to develop and increase public awareness of urban runoff issues.

Public Participation Program (section II) – Public participation is addressed by two components:  Residential Section IIA and Commercial Section IIB.  By splitting the public participation program into two subsections we will be able to focus specially on the unique characteristics of each group.

City of Calexico SWMP

Residential (Section IIA) – The Residential program area is focused mainly at public education, encouraging watershed awareness, individual responsibility, and offering practical alternatives for citizens to properly dispose of household hazardous waste (HHW). The program is implemented primarily through area-wide education activities and HHW material collection efforts.

Commercial/Industrial Program (Section IIB) – The Commercial/Industrial program area is implemented primarily through area-wide outreach, education, and subsequent facility visits. The program includes technical training for municipal staff regarding BMPs and stormwater management at industrial and commercial sites.

Illicit Connection/Illegal Discharge (IC/ID) Program (Section III) – This program involves screening, detection, and elimination of improper discharges to the storm drain system. This program is implemented at both the area-wide and individual municipal levels.

Construction Activities (Section IV) – The Construction program is closely linked to New Development/Redevelopment and will incorporate existing efforts by Sempra Energy Utilities for projects in the watershed.

New Development/Redevelopment (Section V) - New Development/Redevelopment program requirements will focus on integrating stormwater management measures into current development review processes within the Permittee’s Planning and General Services Departments.

Municipal Agency Activities (Section VI) – This program area is targeted at municipal operations and activities throughout the watershed, including various departments within the General Services framework. Employee training activities are a key aspect of stormwater management at the municipal agency level.

## Area-wide Programs

The Permittee employ two area-wide programs to implement certain BMPs. Each program is established by the execution of an agreement between the agencies.

City of Calexico SWMP

Hazardous Materials Spill Response

The Fire Department’s Hazardous Materials Emergency Spill Response Team (HAZMAT Team) is a major component of the Permittee’ area-wide source control efforts. The HAZMAT Team responds to incidents of spills and illegal dumping of hazardous material throughout the watershed.  HAZMAT directly oversees and directs incident response and clean up of hazardous material with the goal of preventing discharges into the municipal storm drain system.  The HAZMAT Team efforts support BMPs for the IC/ID and Commercial/Industrial program areas.  Details are presented in Section III (IC/ID).

HHW/Anti-freeze, Batteries, Oil, Latex Paint (ABOP)

The HHW and ABOP programs are principal components of the Permittee’ source control efforts.  Both programs are implemented by the Imperial Valley Waste Management Task Force, and provide practical alternatives to improper disposal of household hazardous wastes that might otherwise be disposed into the municipal storm drain system.  The HHW and ABOP programs directly support BMPs for Residential and IC/ID program areas.  Both programs are discussed further in Section IIA (Residential).

## Legal Authority

It is expected that the initial Permit will require the Permittee to establish adequate legal authority to implement the provisions of the Permit in accordance with federal regulations at 40 CFR 122.26.

Following Permit adoption, the Permittee will examine its respective ordinance framework and adopt or modify existing ordinances to ensure adequate legal authority in accordance with the Permit requirements.  The City of Calexico expects to rely on the principle of “combined legal authority” as outlined in the United States Environmental Protection Agency's (USEPA’s) Part 2 Permit Application Guidance and the Permittee’ application. Legal authority will then be maintained and exercised by the appropriate governmental entities with jurisdiction for the storm drain system.

## Fiscal Analysis

Fiscal resources for supporting the SWMP in the Imperial Valley watershed will be established during the Permit term.

City of Calexico SWMP

Section I
Public Education Program

The Permittee will develop a public education and outreach program, which is comprised of several BMPs that will be developed and implemented.  The implementation of these BMPs will be accomplished primarily through the activities of the SWMP.  Through the SWMP, the Permittee will develop outreach materials and methods that target both general public as well as specific niche audiences.   Significant outreach materials and methods include:

- Focused brochures and stormwater management guides

- Mass-media stormwater educational campaigns

- School programs and classroom presentations

- Stormwater presentations to adult audiences

- Participation at Community Events

Pollution prevention strategies that will be highlighted through the above educational tools include:

- Proper disposal of household hazardous wastes

- Proper disposal of used motor oil

- Practices to reduce excessive use of pesticides and fertilizers

- Good housekeeping BMPs for homeowners and business operators

- Proper disposal of pet waste

I.I		Best Management Practices

Each program area in the SWMP is supported by multiple public education BMPs. The BMPs are implemented at either the regional or local level.  Most of the BMPs are intended for multiple audiences and therefore apply to more than one stormwater management program area.  See Table I-1 for a listing of program areas and the educational BMPs that support those areas.

City of Calexico SWMP

Table I-1
Public Education Program BMP Summary

| Program Area | Applicable BMPs | Intended Audiences | Implementation | Responsible Party | Time Schedule |
| --- | --- | --- | --- | --- | --- |
| Illicit Connection/ Illegal Discharges | E1, E2, E3, E4 | - Residents - Groundskeepers - Nurseries - Commercial/Industrial facility owners/operators - Students | Local | I.V.W.M.T.F. City of Calexico | Ongoing |
| Commercial/ Industrial | E2, E3, E4, E9 | - Commercial/Industrial facility owners/operators | Local | I.V.W.M.T.F. City of Calexico | Ongoing |
| Development and Construction | E2, E3, E9, E11, E12 | - Contractors - Architects - Engineers - Plannning/ Inspection staff | Local | I.V.W.M.T.F. City of Calexico | Ongoing |
| Municipal Agency Activities | E1, E2, E3, E4, E11, E12 | - Municipal maintenance staff | Local | I.V.W.M.T.F. City of Calexico | Ongoing |
| Residential | E1, E2, E3, E4, E7, E9 | - Residents - Students | Local | I.V.W.M.T.F. City of Calexico | Ongoing |

The sections that follow describe the outreach methods, proposed outreach activities, and program evaluation assessments.  Sections II through VI provide more detail about how the Permittee customize outreach activities for those individual program areas.

I.II		Outreach Methods

The Permittee will develop outreach methods to effectively reach a variety of diverse audiences.  The table below presents the various outreach methods expected to be applied to targeted audiences.

City of Calexico SWMP

Table I-2
Outreach Methods

| Audience | Outreach Methods | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| Residents | - Flyers - Brochures - Radio, TV/Cable commercials - Utility bill inserts - Newspaper inserts and advertisements - Community events & presentations - Video presentations for adult audiences | I.V.W.M.T.F. City of Calexico | Ongoing |
| Groundskeepers ### Nurseries | - Focused Brochures - Posters - Workshops | I.V.W.M.T.F. City of Calexico | Ongoing |
| Commercial/Industrial | - Awareness Events (e.g. Pollution Prevention Week) - Brochures - Posters | I.V.W.M.T.F. City of Calexico | Ongoing |
| Students | - Classroom presentations - Teacher educational materials - Learning contests - Videos | I.V.W.M.T.F. City of Calexico | Ongoing |
| Contractors | - Brochures at Permittee public counters | I.V.W.M.T.F. City of Calexico | Ongoing |
| Architects/Developers | - Brochures at Permittee public counters - Workshops | I.V.W.M.T.F. City of Calexico | Ongoing |
| Permittee Engineers | - Training modules | I.V.W.M.T.F. City of Calexico | Ongoing |
| General Public | - Newspaper inserts - Television commercials - Radio campaigns - Adult presentations - Awareness Events (e.g. Pollution Prevention Week) | I.V.W.M.T.F. City of Calexico | Ongoing |

City of Calexico SWMP

I.III		Proposed Outreach Activities

I.III.I		Media Efforts

The SWMP will collaborate with Imperial County to conduct radio advertising campaigns addressing stormwater pollution prevention.  This effort will increase public awareness during the initial Permit term and will be supplemented by updated stormwater management information.

The SWMP will develop newspaper inserts, poly bags (plastic covers for newspapers used on rainy days) and other various advertisements to help increase public awareness.  The SWMP will develop a stenciling program. Brochures will also be developed addressing topics such as pool discharges and pet waste.

I.III.II		Presentations

The Permittee will conduct garden workshops at local nurseries and home improvement centers. At these workshops residents will learn about insectaria plants that help attract beneficial insects and other plants that help deter harmful pests.  The workshop will also include information on composting and mulching for erosion control.  The Permittee will also provide SWMP information and materials at workshops for landscape contractors, golf course superintendents and groundskeepers.

The Permittee will also provide presentations to rotary/civic club gatherings.  At the presentations club members will be informed about the problem of stormwater pollution and what they can do to help prevent it.  Presentations will conclude with material distribution and a question and answer period.  The Permittee will be developing a presentation outline that will be specific for homeowner associations.  The presentation will focus on the topic of pesticides/fertilizers and erosion control in the home garden.  The presentation will also cover the proper disposal of unwanted garden chemicals.

It is anticipated that the partnership with the California Conservation Corps (CCC) and the I.V.W.M.T.F. will be developed to conduct various community outreach for Permittees. The potential scope of work will include attending various events throughout the Imperial Valley area, distributing SWMP materials to residents, conducting classroom presentations, and conducting surveys to help measure residents’ knowledge of stromwater pollution.

City of Calexico SWMP

I.III.III 	Scholastic Events

Classroom presentations to earth science and automotive shop classes in middle and high schools within the Permittee’s service area will be conducted.  The presentation will identify the causes of stormwater pollution, its effects on surface waters, and what students can do to prevent it.  Topics will include motor oil recycling, responsible car wash events, proper use of material according to label instructions, and proper disposal of Household Hazardous Waste (HHW), trash, yard clippings, and pet waste.

I.IV		Evaluation / Assessment

The primary performance targets for public education and outreach activities are the number of the audiences reached and their level of knowledge for stormwater pollution prevention.  The evaluation and assessment of the Public Education Program will be accomplished by collecting a list of data, categorized by the types of audiences and distribution media as follows.

- Number of educational materials distributed at community events

- Attendance at community events

- Number of educational materials distributed at school presentations

- Attendance at school presentations

- Number of adult presentations provided

- Attendance at adult presentations

- Number of educational materials distributed at Permittee public counters

- Number of construction and development-focused workshops conducted

- Attendance at construction/development workshops

- Number of development staff training sessions conducted

- Attendance at development staff training sessions

- Local television, radio, and newspaper circulation estimates

City of Calexico SWMP

Section IIA
Residential

The Residential program is designed to educate the general public on the importance of stormwater pollution prevention. The program focuses on residential activities such as vehicle washing and maintenance, landscaping, home maintenance, illegal dumping, and pet ownership.

This section presents:

- Current Activities and Source Identification that are the focus of BMP implementation (IIA.I)

- BMPs to manage both stormwater and non-stormwater discharges for the next Permit term (IIA.II)

- Education for general public on residential related stormwater quality issues (IIA.III)

- Evaluation and Assessment of BMPs (IIA.IV)

IIA.I		Current Activities and Source Identification

IIA.I.I		Vehicle Washing and Maintenance

This category encompasses vehicle washing and maintenance activities that present opportunities for harmful pollutants such as soap, motor oil, radiator fluid, etc. to enter into the storm drain system. The SWMP will work cooperatively with the Permittees’ motor oil recycling programs to distribute informational materials that address the importance of proper disposal of used motor oil.  In addition, the SWMP will provide information on vehicle washing and maintenance related pollution prevention and control activities through distribution of brochures, utility inserts and newspaper advertisements, presentations to student and adult audiences, etc.  Please refer to Section I Public Education Program for more information.

City of Calexico SWMP

IIA.I.II	Landscaping

Landscaping related activities such as mowing, fertilizing, weeding, etc. are potential sources of stormwater pollution.  The improper handling of grass clippings, chemical fertilizers and pesticides can introduce pollutants into stormwater system and jeopardize the quality of the receiving waters.  The SWMP will develop a home garden care

brochure to inform residents of the adverse effects of stormwater pollution and to offer environmentally safe alternatives such as Integrated Pest Management, Companion Planting, Soil Solarization and Composting.  The brochures will be distributed to the general public via local nurseries, home improvement centers, garden workshops conducted by the Agricultural Commissioner and the UC Riverside Cooperative Extension. Other environmental educational tools such as video, newspaper insert, poly bag for newspaper, etc. will be developed to help increase public awareness in stormwater pollution prevention.  Please refer to Section I Public Education Program for more information.

IIA.I.III	Home Maintenance

This category encompasses BMPs to facilitate the proper use and disposal of common household products such as insecticides, batteries, latex paints, varnishes, cleaners, etc.  The SWMP will develop a brochure to educate residents on the importance of proper disposal of household hazardous wastes as well as offer less toxic alternatives to commonly used household products.  The IVWMTF sponsors HHW Collection Events and ABOP Centers to encourage the proper disposal of household hazardous wastes. Please refer to Section I Public Education Program for more information.

IIA.I.IV	Illegal Dumping

This category covers the full array of activities by residents that involve the improper disposal of waste materials into the storm drain system. Environmental educational tools that include brochures, video, public services announcements, etc. will be developed to inform the general public on the detrimental effects of stormwater pollution.  Another activity will include the use of "Notice of Violation" (NOV) door hanger notices specifically developed for the residential setting to inform residents of potential improper discharges from their properties.  In addition, a catch basin stenciling program will be implemented to remind residents that no dumping is allowed. Please refer to Section I Public Education Program for more information.

City of Calexico SWMP

IIA.I.V	Pet Ownership

The importance of proper clean-up and disposal of pet waste will be addressed and emphasized throughout the Public Education Program campaign in various formats including educational brochures, video presentations, radio commercials, etc.  The intent is to educate both adults and children on the adverse effects of improper disposal of pet waste and to cultivate awareness of responsible pet care to prevent stormwater pollution and potential health hazard.

IIA.II	Best Management Practices

Program activities for these residential activities and the corresponding BMPs are discussed in the following sections

IIA.II.I	Vehicle Washing and Maintenance

The Public Education Program will inform the general public of pollution prevention and control methods related to vehicle washing and maintenance.  The Permittee will continue the motor oil recycling programs to encourage the proper disposal of used motor oil.  Applicable BMPs are summarized in Table IIA-1 below.

Table IIA-1
Vehicle Washing and Maintenance BMP Summary

| BMPs | Description | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| E2 | Educate/inform the general public on the impacts of littering and other improper disposal. | I.V.W.M.T.F. City of Calexico | Ongoing |
| E3 | Educate/inform the general public on the impacts of dumping pollutants into the storm drainage system. | I.V.W.M.T.F. City of Calexico | Ongoing |
| E4 | Educate/inform the general public on impacts from leakage or dumping of gasoline, oil and grease, antifreeze and hydraulic fluid from vehicles into the streets. | I.V.W.M.T.F. City of Calexico | Ongoing |

City of Calexico SWMP

IA.II.II	Landscaping

The SWMP will offer home gardening workshops at local nurseries and home improvement centers to educate the general public on the importance to reduce/eliminate the use of pesticides and fertilizers, to handle landscape wastes properly, and to avoid excessive irrigation.  As part of the public education program, the Permittee will offer and conduct landscape training for contractors and groundskeepers through the University of California Cooperative Extension Workshops.  The SWMP will be placing display units with information on stormwater pollution, household hazardous waste, less toxic home gardening alternative products, etc. at public outreach events and libraries. Applicable BMPs are summarized in Table IIA-2 below.

Table IIA-2
Landscaping BMP Summary

| BMPs | Description | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| E1 | Educate/inform the general public on the proper application and management of pesticides, fertilizers and herbicides, as well as the proper management of irrigation systems to prevent runoff drainage to municipal storm drain system. Where appropriate, coordinate with the Soil Conservation Service, Resource Conservation Districts and UC Cooperative Extension. | I.V.W.M.T.F. City of Calexico | Ongoing |
| E3 | Educate/inform the general public on the impacts of dumping pollutants into the storm drainage system. | City of Calexico | Ongoing |
| SW3 | Support the efforts of the IVWMTF Household Hazardous Waste Program which provides a convenient means to properly dispose of oil, antifreeze, pesticides, herbicides, paints, solvents, and other potentially harmful chemicals. | City of Calexico | Ongoing |

IIA.II.III	Home Maintenance

The Permittee will address home maintenance related issues through the public education program, HHW Collection Events and ABOP Centers.  The public education efforts will include radio and cable television campaigns, utility bill inserts, newspaper inserts, brochures, presentations, etc. to inform the general public of the proper disposal of household hazardous wastes and to offer less toxic alternative products.  Home maintenance BMPs are shown below in Table IIA-3.

City of Calexico SWMP

Table IIA-3
Home Maintenance BMP Summary

| BMPs | Description | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| E3 | Educate/inform the general public on the impacts of dumping pollutants into the storm drainage system. | I.V.W.M.T.F. City of Calexico | Ongoing |
| SW3 | Continue to support the efforts of the IVWMTF Household Hazardous Waste Program which provides a convenient means to properly dispose of oil, antifreeze, pesticides, herbicides, paints, solvents, and other potentially harmful chemicals. | I.V.W.M.T.F. City of Calexico | Ongoing |

IIA.II.IV	Illegal Dumping

The Permittee will implement BMPs including public education, catch basin stenciling program, Notice of Violation (NOV) door hanger notices to curtail illegal dumping activities.  Please refer to Section 3 Illicit Connections and Illegal Discharges of this report for more information.  Table IIA-4 summarizes applicable BMPs.

Table IIA-4
Illegal Dumping BMP Summary

| BMPs | Description | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| E2 | Educate/inform the general public on the impacts of littering and other improper disposal. | I.V.W.M.T.F. City of Calexico | Ongoing |
| E3 | Educate/inform the general public on the impacts of dumping pollutants into the storm drainage system. | I.V.W.M.T.F. City of Calexico | Ongoing |
| SW1 | Provide, collect, and maintain litter receptacles in strategic public areas and during public events. | I.V.W.M.T.F. City of Calexico | Ongoing |
| OM1 | Develop a program, continue and/or expand an existing field program to detect and prevent dumping or routinely discharging pollutants into storm drains and drainage channels. | I.V.W.M.T.F. City of Calexico | Ongoing |

City of Calexico SWMP

IIA.II.V	Pet Ownership

The Permittee will use an area-wide public education program to inform pet owners of the importance of responsible pet care and to curtail the improper disposal of pet wastes. The SWMP will develop a "focused" brochure for pet owners on proper disposal of pet wastes.  Table IIA-5 describes applicable BMPs.

Table IIA-5
Pet Ownership BMP Summary

| BMPs | Description | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| E7 | Educate/inform the general public regarding the need to clean-up and properly dispose of pet waste. | City of Calexico | Ongoing |
| R3 | Implement and enforce leash laws and other pet laws (i.e., pet waste clean-up, no pets in public areas) in selected public-use areas. | City of Calexico | Ongoing |

IIA.III	Education

The area-wide public education program will present and offer environmental educational materials to the general public through various forms of media that include workshop, video, brochure, catch basin stenciling, NOV door hanger notices, radio and cable television campaigns, utility bill inserts and newspaper inserts.  Please refer to Section I Public Education Program for more information.

IIA.IV	Evaluation/Assessment

The Permittee will use the following targets to provide consistency among the Permittees’ programs to gauge effort and assess future outreach.  The proposed evaluation and assessment requirements for each BMP are shown below in Table IIA-6.

City of Calexico SWMP

Table IIA-6
Evaluation & Assessment Summary

| BMPs | Assessment Target | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| SW3 | Household Hazardous Waste Collection Program: (1) track event dates and number of days per event; (2) track type and amount of material collected; and (3) track advertisement expenditures by type (newspaper, television, radio, banners, flyers, etc.) for the Permittee’ internal use. | I.V.W.M.T.F. City of Calexico | Ongoing |

Section IIB
Commercial/Industrial Program

The purpose of this program area is to conduct source identification and outreach to reduce discharge of pollutants from both commercial businesses and industrial operations.

The Commercial/Industrial section presents:

- Current activities that are the focus of BMP implementation (IIB.I)

- BMPs to manage stormwater runoff and non-stormwater discharges for the Permit term (IIB.II)

- Training for municipal staff in Commercial/Industrial BMPs (IIB.III)

- Evaluation & Assessment of BMPs (IIB.IV)

IIB.I		Current Activities and Source Identification

Activities by the Permittee for stormwater management are presented in this section. Continual source identification is critical to effective program implementation, as it helps the Permittee focus their management efforts. Four sectors targeted for outreach are restaurants, automotive businesses, mobile business operators, and industrial facilities.

City of Calexico SWMP

IIB.I.I		Public Education

The Commercial/Industrial Compliance Assistance Program (C/ICAP) builds on the expertise of two existing inspection programs to provide technical assistance to commercial operators in stormwater management and compliance. Program implementation involves focused public outreach for food service and hazardous waste Permittee including auto repair, gas stations, etc.

The SWMP will develop a series of “focused” BMP brochures that target the commercial and industrial businesses. These brochures will provide useful guidelines on proper clean-up and waste disposal procedures.  Complementary posters illustrating recommended stormwater BMPs will also be developed for automotive service facilities and restaurants to serve as visual reminders for employees.  Distribution of these brochures and posters is expected to be accomplished through the C/ICAP, Permittee departments, and other agencies that regularly deal with commercial and industrial businesses.

A brochure will provide information on how to submit a Notice of Intent (NOI) form for industrial facilities and lists certain manufacturing operations that may require coverage under the Industrial Activities Storm Water General Permit. This brochure will be circulated via the Permittees’ permit counters and the C/ICAP.

IIB.I.II	General Industrial Permit Coordination

Many manufacturing and industrial operations are subject to the requirements of the State’s General Permit for Discharges of Storm Water Associated with Industrial Activities (General Industrial Permit). BMP R14 requires the Permittee to obtain proof of compliance prior to issuance of a business license or a certificate of occupancy for new industrial facilities.

IIB.II		Best Management Practices

Several of the BMPs that were developed support multiple program areas. Those that provide direct support to the commercial/industrial program are discussed in the subsections that follow.

BMP implementation in the commercial/industrial program area addresses:

Source Identification (IIB.II.II)

Outreach (IIB.II.III)

City of Calexico SWMP

IIB.II.I	Source Identification

Source identification of commercial/industrial operations will involve continuing the present program of tracking businesses specific to the Imperial Valley watershed that are potential pollutant sources. Table IIB-1 shows the primary BMPs for accomplishing source identification.

During the next Permit term:

- The Permittee will annually contact the RWQCB to obtain an updated list of commercial and industrial operations covered by the General Industrial Permit in their respective jurisdiction (BMP OM1C). The list will supplement the County Hazardous Materials Management Division Inspection Program list to target outreach for commercial and industrial activities.

- The Permittee will coordinate with County Department of Environmental Health to obtain an updated list of Commercial and Industrial Facilities in the Hazardous Materials Management Division Inspection Program (BMP OM1C).

- The Permittee will provide the Public Education Coordinator with the name, address, and telephone number of mobile cleaning services within their respective jurisdictions using available information sources including business license databases or telephone directories, etc., to conduct outreach

(BMP OM1C).

- The Permittee will require new industrial facilities subject to the General Industrial Permit to show proof of compliance prior to issuance of a business license or a certificate of occupancy (BMP R14).

City of Calexico SWMP

Table IIB-1
Source Identification BMP Summary

| ## BMPs | Description | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| OM1C | Coordinate with County Health Department, local fire departments, Colorado RWQCB, and other departments as necessary to develop a target list of industries which are known to be contributing substantial pollutant loads to the storm drain system. Develop and/or enhance inspection and notification procedures for these facilities. | City of Calexico | 24 Months |
| R14A | Develop, implement, and enforce regulations that require all new industrial facilities subject to the General Industrial Permit to show proof of compliance (such as a file number from submittal of a Notice of Intent) prior to: 1) issuance of a business license (applicable only to municipalities which require business licenses) or 2) issuance of a certificate of occupancy for new developments. | City of Calexico | 48 Months |

IIB.II.II	Outreach

The BMPs highlighted below in Table IIB-2 support the outreach component of the commercial/industrial program. The Permittee will distribute educational materials with content specific to controlling stormwater quality at restaurant, automotive service, and industrial facilities.

During the Permit term:

Outreach will be started by the C/ICAP. The C/ICAP will perform outreach to commercial and industrial facilities. During the first year of the Permit term, the Permittee will develop a target list of businesses likely to be contributing pollutant loads

(BMP OM1C). An implementation schedule for field visits will be developed using the County Environmental Health Department’s existing inspection schedules

(BMP OM1D). It is anticipated that restaurants will be visited once a year and HMMD facilities will be visited once every two years.

City of Calexico SWMP

Beginning in the second year of the Permit term, County Environmental Health inspections will include targeted commercial and industrial sites. These sites will be selected from the information base composed in BMP OM1C.

The site visits will be prioritized through BMP OM1D. The visits will include following elements:

- Confirm that a Stormwater Pollution Prevention Plan (SWPPP) is in place and being implemented if the activity is required to comply with the General Industrial Permit. If no SWPPP is on hand and being implemented, the owner will be informed of a potential violation and the need to comply.

- Provide general guidance on BMPs for the site.

Through the SWMP, target and direct outreach to specific activities based on the updated General Industrial Permit NOI list obtained from the RWQCB. Supplement the industrial activity list with commercial businesses from County Environmental Health Hazardous Materials Management Program Listing of Commercial and Industrial facilities.

Table IIB-2
Outreach BMPs Summary

| BMPs | Description | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| E2 | Educate/inform the general public on impacts from littering and improper waste disposal. | I.V.W.M.T.F. City of Calexico | 12 Months Ongoing |
| E3 | Educate/inform the general public on impacts from dumping pollutants into storm drainage system. | I.V.W.M.T.F. City of Calexico | 12 Months Ongoing |
| E4 | Educate/inform the general public on impacts from leakage or dumping of gasoline, oil and grease, antifreeze and hydraulic fluid from vehicles into the streets. | I.V.W.M.T.F. City of Calexico | 12 Months Ongoing |
| E9 | Educate/inform the landowners, tenants, business owners, and industrial operations (with special attention to facilities known to be contributing substantial pollutant loading to the storm drain system) regarding the need to provide covers (roofs, tarps) to keep rain off of areas which contain potential contaminants (such as chemical, waste and industrial storage areas) and keep runoff from draining through areas which contain contaminants. | I.V.W.M.T.F. City of Calexico | 24 Month Initial Ongoing |

City of Calexico SWMP

IIB.III	Training

Information on staff training during the Permit term is described in Section III.III. The General Services personnel who conduct inspections through the Commercial/Industrial Compliance Assistance Program will receive an orientation pertaining to the General Industrial Permit and related BMPs.

IIB.IV	Evaluation/Assessment

Evaluation and assessment of BMP performance and Permittee compliance will be accomplished primarily through reporting and documentation. Assessment measures shown in Table IIB-3 below will reflect progress for Commercial/Industrial BMPs.

Table IIB-3
Evaluation & Assessment Requirements

| BMPs | Assessment Target | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| OM1C | Number of commercial and industry facilities in the source database, by type: - Restaurant - Automotive Service - Industrial - Mobile Cleaning Business | City of Calexico | 18 Months |
| OM1E | Number of commercial and industrial facilities visited (during Permit year 3, to continue each year through the Permit term): - Implement BMPs - Maintain a SWPPP (if required by the General Industrial Permit) | City of Calexico | 24 Months |

City of Calexico SWMP

### Section III

## Illicit Connection/Illegal Discharge Program

The IC/ID program is designed to detect and eliminate improper discharges to the municipal storm drain system.  The IC/ID program is incorporated in the SWMP, and implemented at both the local and area-wide levels.  IC/ID issues specific to Commercial/Industrial operations are presented in Section 3.

The IC/ID program includes:

- Current activities focusing on BMP implementation (III.I)

- BMPs to manage stormwater runoff and non-stormwater discharges for the Permit term (III.II)

- Training for municipal staff in IC/ID BMPs (III.III)

- Evaluation & Assessment of BMPs (III.IV)

The federal stormwater regulations (40 CFR 122.26) require that the IC/ID program include: “a description of a program, including inspections, to implement and enforce an ordinance, orders, or similar means to prevent illicit discharges to the municipal separate storm sewer system; this program description shall address all types of illicit discharges…”

Three types of releases, generally referred to as "improper discharges" to the storm drain system are addressed under this IC/ID program:

Illicit discharge: An illicit discharge is any discharge that is not composed entirely of storm water except discharges pursuant to a NPDES Permit and other exempt discharges such as fire fighting flows and others.  Illicit discharges can be considered any non-storm water discharges, which enter or have the potential to enter the storm drain system through illicit connections or illegal dumping.

Illicit connection: An illicit connection is a direct physical connection to a storm drain, which allows prohibited discharges to enter the storm drain system.  Examples include sanitary sewer connections, industrial process waters, and floor drains.

City of Calexico SWMP

Illegal dumping: Illegal dumping is the intermittent discharge of pollutants into the storm drain system through either legal connections, such as catch basins, or by direct dumping into drains, road borrow pits, and channels.  This could include the disposal of used oil, paint, and wash water.  In addition, illegal dumping includes the illegal disposal of pollutant material in the road borrow pits, drains, and channels throughout the Imperial Valley drainage area such that stormwater can eventually mobilize it and carry it to other portions of the storm drain system and to receiving waters.

III.I	Current Activities and Source Identification

The Permittee will implement BMPs to perform public outreach, respond to field observations, and contain and clean up improper discharges when required.  The specific activities used to address IC/ID issues are presented below.

III.I.I		Outreach

The purpose of this activity is to inform the public about the detrimental effects that illegal discharges have on stormwater quality.  The SWMP is the primary mechanism for IC/ID outreach.  The SWMP includes public education on the full scope of stormwater pollution prevention including encouraging the public to report suspected incidents of IC/ID.  The SWMP outreach activities are further described in Section I, Public Education Program.

III.I.II		Field Screening/System Surveillance

Field Screening/System Surveillance consists of three elements:

- source identification

- routine field inspection/screening of storm drain

- routine inspection of facilities

Source identification will be performed during the initial Permit term in accordance with the NPDES Permit.  The source identification efforts are further described in section IIB.II.I and IIB.II.II.

City of Calexico SWMP

Current field inspection/screening of the storm drain system will be performed by Permittees’ department staff.  The Permittees’ code enforcement staff routinely patrol and inspect municipal facilities, and report IC/ID incidents.  The Permittee will instruct other General Services staff, such as building inspectors, fire department, and community service staff to report IC/ID incidents while in the field performing their specific duties.  The General Services department staff also inspects catch basins either as part of routine maintenance, or on a periodic (semi-annual/annual) basis.  The City of Calexico performs field inspections of its municipal storm drain system as part of ongoing facility maintenance programs.

Calexico staff also conducts periodic facilities reconnaissance targeting commercial/industrial areas and remote areas for IC/ID.  Routine facility inspections are further addressed through the Commercial/Industrial Compliance Assistance Program described in Section IIB.I.I.

In addition to field screening and surveillance, the Permittee will address swimming pool discharges. The City of Calexico has adequate legal authority to halt an IC/ID incident that results in the improper disposal of swimming pool discharges.

III.I.III		Reporting

The purpose of standardized reporting is to formalize the procedures followed for incident documentation and follow up.  During the initial Permit term, the Permittee will develop a program for reporting and documenting illegal discharges, spills, and dumping.

The Permittee will use internal log forms to document reports of IC/ID.  The log forms will also be used by staff to record observations in the field of improper discharge.  Incidents involving hazardous materials are documented using a standard California Hazardous Material Incident Report form.

III.I.IV	Incident Response, Investigation, and Clean up

The purpose of this activity is to address the procedures followed for incident response, investigation, and cleanup.  Presently, the Permittees’ General Services or code enforcement staff respond to notification of illegal discharges by taking phone calls, assessing the magnitude of the discharge, and either initiating direct follow-up or referring the call to HAZMAT (if a significant quantity of hazardous materials is reported).

City of Calexico SWMP

Staff of the Permittees’ agency handles response and initiate clean up in cases of minor spills involving non-hazardous materials.  Examples include paint spills into the city streets.  The HAZMAT team responds directly to accidental spills or illegal dumping of hazardous materials, and provides support services to other agencies that encounter hazardous materials in the routine of their duties.  An example of support services includes assisting police during illegal narcotics investigations.  The HAZMAT team operates 24 hours a day, seven days a week.  The team’s duties include, but are not limited to, directing the containment and clean up of spills that may enter surface waters or flow directly to the municipal storm drain system.

III.II	Best Management Practices

III.II.I		Outreach

During the Permit term, Permittee will develop informational materials that clearly define illegal discharges, explain their impacts on stormwater, and describe why such discharges should be prevented through the Public Education Program.  These efforts support the educational BMPs shown below in Table III-1.  Available informational materials may include audio and video Public Service Announcements, utility inserts, brochures, adult and classroom presentations, catch basin stenciling, water-drop magnets, print and radio advertisement campaigns, library displays, school programs, and workshops.  Permittee will distribute informational materials as detailed in Section I Public Education Program.

The Permittee also will support the Imperial Valley Waste Management Task Force (IVWMTF).  The IVWMTF sponsors a HHW Program that provides mobile collection events, while the ABOP provides a fixed facility for disposal.  Under these programs, residents can dispose of used motor oil and other types of HHW free of charge at both mobile and permanent collection facilities throughout the Permit area as detailed in Section I Public Education Program.

City of Calexico SWMP

Table III-1
Outreach BMP Summary

| BMPs | Description | #### Responsible #### Party | Time Schedule |
| --- | --- | --- | --- |
| E1 | Educate/inform the general public on the proper application and management of pesticides, fertilizers and herbicides, as well as the proper management of irrigation systems to prevent runoff drainage to municipal storm drain system. Where appropriate, coordinate with the Soil Conservation Service, Resource Conservation Districts and UC Cooperative Extension. | I.V.W.M.T.F. City of Calexico | Ongoing |
| E2 | Educate/inform the general public on the impacts of litter and improper waste disposal. | I.V.W.M.T.F. City of Calexico | Ongoing |
| E3 | Educate/inform the general public on impacts from dumping pollutants into storm drainage system. | I.V.W.M.T.F. City of Calexico | Ongoing |
| E4 | Educate/inform the general public on impacts from leakage or dumping of gasoline, oil and grease, antifreeze and hydraulic fluid from vehicles into the streets. | I.V.W.M.T.F. City of Calexico | Ongoing |

III.II.II		Field Screening/System Surveillance

The Permittee is expected to be required to report additions or modifications of new outfalls, major structural controls, and the list of industrial operations subject to storm water regulations.  The Permittee will report and update these items as required.  The list of industrial operations subject to storm water regulations can be updated through the C/ICAP and General Industrial Permit Coordination activities discussed in

Section IIB.  Table III-2 summarizes the BMPs for field screening/system surveillance.

I.	The Permittee will develop and use a standardized field inspection/reporting form for documenting IC/ID incidents (BMP OM1B).

II.	The Permittee will establish an implementation schedule

(BMP OM1C, BMP OM1D) for conducting IC/ID field inspections within their respective jurisdictions.

The schedule will address:

- Hazardous Materials Management facilities database

- RWQCB General Industrial Permit database

City of Calexico SWMP

- Major outfalls identified during the initial Permit term

The Permittee will conduct and document field inspections during the performance of existing field activities, recording IC/ID observations on the standardized inspection form (BMP OM1E).

Table III-2
Field Screening BMP Summary

| BMPs | Description | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| OM1B | Develop forms (or other mechanism) for reporting the observations of field personnel of unauthorized dumping or spills so the information can be used to help locate the source of pollutants. | City of Calexico | 12 Months |
| OM1C | Coordinate with County Health Department, local fire departments, Colorado RWQCB, and other departments as necessary to develop a target list of industries which are known to be contributing substantial pollutant loads to the storm drain system. Develop and/or enhance inspection and notification procedures for these facilities. | City of Calexico | 12 Months |
| OM1D | Develop implementation schedule for conducting field inspections of storm drain system and targeted industrial facilities. | City of Calexico | 12 Months |
| OM1E | Conduct field inspections to ensure identification and elimination of illegal dumping and discharge. | City of Calexico | 12 Months |

2.2.3 Reporting

For the Permit term, the Permittee will develop and use the standardized reporting form described in section III.II.II to document IC/ID incidents.  Table III-3 summarizes the related BMP for IC/ID reporting. The present system for internal reporting will remain in effect, supported by the existing framework of routine inspections by General Services staff.

For the Permit term:

The Permittees’ field personnel will use the standardized form described in III.II.II for reporting the IC/ID observations. (BMP OM1B).

City of Calexico SWMP

Table III-3
Reporting BMP Summary

| BMP | Description | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| OM1B | Develop forms (or other mechanism) for reporting the observations of field personnel of unauthorized dumping or spills so the information can be used to help locate the source of pollutants. | City of Calexico | 12 Months |

III.II.IV	Incident Response, Investigation, and Clean up

The HAZMAT Team responds directly to accidental spills and illegal dumping of hazardous materials and provides support to other agencies that encounter hazardous materials in the course of their duties.  The Permittees’ continued support of the HAZMAT Team helps ensure that spills or illegal dumping of hazardous material will be responded to in a timely and professional manner, thereby reducing the potential impact on municipal storm drain systems and receiving waters.

Over the Permit term, the Permittee will continue to contribute financially to support the County and local HAZMAT crews responding to IC/ID incidents.  The corresponding BMP is shown below in Table III-4.

Table III-4
Incident Response, Investigation, and Cleanup BMP Summary

| BMP | Description | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| SW4 | Continue to support the existing hazardous materials incident response programs implemented by the HAZMAT Team. | City of Calexico | Ongoing |

III.III		Training

During routine maintenance facility inspections, maintenance staff may come across evidence of potential stormwater pollution and/or illicit discharges.  Therefore, training personnel to recognize and respond appropriately to stormwater pollution problems is an integral part of the IC/ID program.

City of Calexico SWMP

During the Permit term:

- NPDES coordinators will develop and document their respective staff training activities and forward the information for inclusion in the Annual Report.  The documentation will include pertinent information such as, but not limited to, the number of staff in attendance and the type of training received.

- NPDES coordinators will attend at least one training session annually at any of the existing training forums available in the local area.

- NPDES coordinators will use municipal staff forums to hold briefings for appropriate staff on commercial/industrial stormwater issues.

The corresponding BMP is shown in Table III-5.  This BMP incorporates general training instruction for all aspects of municipal stormwater management.   Training for the IC/ID program may be conducted in conjunction with training for other portions of the municipal stormwater program discussed elsewhere in this manual.

Table III-5
Training BMP Summary

| BMP | Description | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| OM1A | Develop and conduct a training program for field personnel (including in-field workshops) to conduct inspections. Training program may include development of appropriate field manuals, training materials, and field sampling techniques and procedures. | City of Calexico | 12 Month Initial Ongoing |

III.IV		Evaluation/Assessment

Evaluation and assessment of BMP performance and Permittees’ compliance will be accomplished primarily through reporting and documentation.  The proposed evaluation and assessment requirements for each BMP are shown below in Table III-6.

City of Calexico SWMP

Table II-6
Evaluation & Assessment Summary

| BMPs | Assessment Target | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| OM1A, OM1B, OM1E | Staff training – number of employees trained, by department or function Meet present BMP standards, such as use of standardized IC/ID reporting form Collect information on IC/ID reports: Number of reports received Number of cases investigated/responded to Source of IC/ID Final outcome of case (e.g., spill/connection was terminated and cleaned up, source owner/operator educational visit, warning letter, referral to enforcement agency) Number of enforcement actions issued/taken (e.g., notice of non-compliance, notice of violation and order to comply, referral to District Attorney for prosecution) | City of Calexico | 12 Month Initial Ongoing |
| E2, E3, E4 | Number of education events conducted, by type (e.g., workshops, fairs, public counter assistance, etc.) Event attendance, by event type Number of outreach materials distributed through newspapers, direct mailings, and utility bill inserts. | I.V.W.M.T.F. City of Calexico | 12 Months Initial Ongoing |

City of Calexico SWMP

Section IV

Construction Activities

The Construction Activities section presents:

- Activities that are the focus of BMP implementation (IV.I)

- BMPs to manage stormwater and non-stormwater from new development, redevelopment, and construction activities for the Permit term (IV.II)

- Training for municipal staff in development/construction BMPs (IV.III)

- Evaluation & Assessment of BMPs (IV.IV)

IV.I		Current Activities and Source Identification

IV.I.I		Construction Activities

The Permittee will implement two processes for construction as requirements of the Imperial Valley Permit:

- For General Services projects, the Permittee will utilize BMP’s as defined in the manual entitled “Water Quality Construction Best Management Practices” (attachment A).

- For private sector (non-General Services projects) that require coverage under the General Construction Permit, the Permittee will verify that applicants have filed a NOI prior to the issuance of a grading permit.

IV.II		Best Management Practices

This section identifies and describes BMPs proposed by the Permittee.

IV.II.I		Construction Activities

Construction related activities conducted at project sites such as storage and handling of materials, fueling, equipment maintenance, etc. present opportunities for introducing potential pollutants into the storm drain system.  The Permittee will implement requirements set for General Services related construction activities as directed by the Permit that include:

City of Calexico SWMP

- Reporting General Services construction projects to the RWQCB that would otherwise necessitate coverage under the Statewide General Permit

- Preparing SWPPPs for General Services projects

- Inspecting General Services construction sites

- Verifying NOI submittal for non-General Services (private sector) construction projects that require coverage under the General Construction Permit prior to the issuance of grading permits.

BMPs shown in Table IV-2 will be used to reduce pollutant discharges from construction sites.

During the Permit term:

- The Permittee will require proof of permit coverage by private sector construction projects that require coverage under the General Construction Permit

- The Permittee will implement the requirements of the initial Municipal Permit for General Services projects.

Table IV-2
Construction BMP Summary

| BMPs | Description | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| E11 | Inform contractors, operators, and agency staff about upcoming educational and training workshops on construction site erosion control and construction materials management sponsored by professional organizations and public agencies. Make associated public education materials available at the public counter and staff bulletin boards, as appropriate. | I.V.W.M.T.F. City of Calexico | Ongoing |
| R6 | Review existing grading and erosion control ordinances, if any, to determine the adequacy of existing controls for construction site erosion, sedimentation, and construction material pollutants (i.e. paints, masonry wastes, etc.). The evaluation should also include an assessment of enforcement actions. | I.V.W.M.T.F. City of Calexico | 12 Months |
| R7 | Establish a model construction site control ordinance (if determined necessary based on BMP R6) for future adoption. The ordinance will require control of pollutants from construction sites including erosion and construction material. | I.V.W.M.T.F. City of Calexico | 24 Months |
| OM | Identify priorities for inspecting sites and enforcing control measures for construction projects that disturb greater that 5 acres (Phase I requirements), between 1 and 5 acres (Phase II requirements), and sites disturbing less than 1 acre. | I.V.W.M.T.F. City of Calexico | 24 Months |

IV.III		Training

To support effective stormwater and non-stormwater pollution prevention the Permittee will develop and provide informational and appropriate training sessions that can be provided to staff who conduct construction site inspections.  Training outlined in Section II.III will include construction related stormwater and non-stormwater issues.  New development/significant redevelopment and construction training modules will contain a construction stormwater orientation for plan check staff and field inspectors.  Topics will cover:

- Phase I and Phase II regulations

- SWPPP preparation

- Site inspection criteria and priorities

4.4	Evaluation/Assessment

Evaluation and assessment of BMP performance and Permittee compliance will be accomplished primarily through reporting and documentation.  The proposed evaluation and assessment measures shown in Table IV-3 will be used to evaluate progress for each BMP as follows:

City of Calexico SWMP

Table IV-3
Evaluation & Assessment Requirements

| BMPs | Assessment Target | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| E11 | Describe workshops and efforts to publicize them to the target audience. | I.V.W.M.T.F. City of Calexico | 12 Months |
| E12 | Describe educational materials developed in support of the New Development Guidelines. | I.V.W.M.T.F. City of Calexico | 24 Months |
| R6(C) | Confirm that existing ordinances adequately address Phase I and Phase II requirements. | I.V.W.M.T.F. City of Calexico | 24 Months |
| R7(C) | Adopt appropriate modifications, if necessary, to implement BMP R6. | I.V.W.M.T.F. City of Calexico | 36 Months |
| OM(C) | List priorities, develop implementation schedule for inspections. Conduct site inspections within existing building/grading inspection framework based on priorities. | I.V.W.M.T.F. City of Calexico | 12 Months |

Section V

New Development/Redevelopment and

Post Construction Activities

The New Development/Redevelopment and Post Construction Activities section presents:

- Activities that are the focus of BMP implementation (V.I)

- BMPs to manage stormwater and non-stormwater from new development, redevelopment, and construction activities for the Permit term (V.II)

- Training for municipal staff in development/construction BMPs (V.III)

- Evaluation & Assessment of BMPs (V.IV)

City of Calexico SWMP

V.I		Current Activities and Source Identification

V.I.I		New Development and Redevelopment

Both stormwater peak flows and volume increase as the community is progressively built out and impervious surfaces replace the natural topography.  In addition, new urban areas contribute to the runoff pollutant loads by creating new sources.

The new development and redevelopment program element is directed at reducing pollutant discharges from all types of land uses, including residential, commercial, and industrial. New development is considered construction on a previously undisturbed parcel.  A redevelopment project is any project where major modification to an existing site or structure requiring a permit is undertaken.  Routine maintenance, interior remodeling, re-roofing, and parking lot maintenance are not included.  A redevelopment project is not to be confused with the projects undertaken by a Redevelopment Agency.

V.II		Best Management Practices

This section indentifies and describes BMPs proposed by the Permittee.

V.II.I		New Development and Redevelopment

The purpose of this element is to reduce pollutant discharges from developing areas within the Permit area by incorporating applicable structural and non-structural BMPs into new development projects and significant redevelopment projects.

The Permittee will develop and implement “Construction and Development BMP Guidelines” to address Stormwater management measures during the planning, construction, and completion phases of a development/redevelopment project.  This guide will provide the Engineer, Architect, Contractor or Developer the following:

- Present procedures for approval of grading and drainage/erosion control plans will be modified to include necessary BMPs.

- The guide will be made available to development applicants as early in the permitting process as possible.

- Implementation procedures will be developed for the new development BMP guidelines, including training and education for employees who will need to implement the plan checking, approvals and inspections.

City of Calexico SWMP

- Implementing staff will notify the development applicant at the earliest possible opportunity if there is a known water quality problem, which might affect the proposed development.

Table V-1
New Development and Redevelopment BMP Summary

| BMPs | Description | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| E11 | Inform contractors, operators, and agency staff about upcoming educational and training workshops on construction site erosion control and construction materials management sponsored by professional organizations and public agencies. Make associated public education materials available at the public counter and staff bulletin boards, as appropriate. | City of Calexico | 12 Months Initial Ongoing |
| E12 | Inform architects, engineers, building department personnel, and local government officials on water quality problems associated with urban runoff and the requirements for meeting NPDES laws and program goals for properly managing the quality of stormwater runoff. Provide information on upcoming training workshops and distribute educational materials as appropriate. | City of Calexico | 18 Months Initial Ongoing |
| ## R | Adopt and implement the construction and development BMP guidelines. | City of Calexico | 24 Months |

V.III	Training

To support effective stormwater and non-stormwater pollution prevention the Permittee will develop and provide informational and appropriate stormwater pollution prevention training sessions that address municipal activities for staff. Training outlined in Section II.III will include who to construction related stormwater and non-stormwater issues.  New development/significant redevelopment and construction training modules will contain a construction stormwater orientation for plan check staff and field inspectors.

- Phase I and Phase II regulations

- SWPPP preparation

- Site inspection criteria and priorities

City of Calexico SWMP

V.IV	Evaluation/Assessment

Evaluation and assessment of BMP performance and Permittee compliance will be accomplished primarily through reporting and documentation.  The proposed evaluation and assessment measures shown in Table V.3 will be used to evaluate progress for each BMP as follows:

Table V-3
Evaluation & Assessment Requirements

| BMPs | Assessment Target | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| E11 | Describe workshops and efforts to publicize them to the target audience. | I.V.W.M.T.F. City of Calexico | 12 Months |
| E12 | Describe educational materials developed in support of the New Development Guidelines. | I.V.W.M.T.F. City of Calexico | 24 Months |
| R6(C) | Confirm that existing ordinances adequately address Phase I and Phase II requirements. | I.V.W.M.T.F. City of Calexico | 24 Months |
| R7(C) | Adopt appropriate modifications, if necessary, to implement BMP R6. | I.V.W.M.T.F. City of Calexico | 36 Months |
| OM(C) | List priorities, develop implementation schedule for inspections. Conduct site inspections within existing building/grading inspection framework based on priorities. | I.V.W.M.T.F. City of Calexico | 12 Months |

City of Calexico SWMP

Section VI

Municipal Agency Activities

The Municipal Agency Activities component is directed at reducing the potential for stormwater impacts from General Service’s operations.  Municipal and General Services staff who carry out regular maintenance and operations throughout the watershed are directly involved in both source control pollution management and IC/ID detection in the field.

The Municipal Agency Activities section presents:

- Current activities that are the focus of BMP implementation (VI.I)

- BMPs to manage both stormwater and non-stormwater for the next Permit term (VI.II)

- Training for municipal staff in relevant BMPs (VI.III)

- Evaluation & Assessment of BMPs (VI.IV)

Municipal operations can have various effects on stormwater quality.  For example, storm drain system maintenance may remove pollutants from the storm drain system before they reach receiving waters.  On the contrary, vehicle maintenance performed improperly or without adequate safeguards can result in pollutants entering the storm drain system.

This section addresses the following public facilities or municipal operations that may impact stormwater quality:

- Sewage Systems

- Municipal Facilities & Operations

- Streets and Roads

- Storm Drain Systems

City of Calexico SWMP

VI.I		Current Activities and Source Identification

VI.I.I		Sewage Systems

A release of untreated sewage contains potential pollutants that include organic matter, bacteria and other pathogens, soaps and detergents, oil and grease, and toxic substances, etc.  Accidental releases of untreated sewage can be conveyed into the storm drain system that drains directly into receiving waters.  Wastewater treatment and disposal plants are operated by the City of Calexico. The agency maintains sewage response plans to prevent unintentional sewage spills from entering the municipal storm drain system.  The sewage response plan includes procedures for notifying the RWQCB and other appropriate agencies, containing and cleaning up the spill.

VI.I.II		Municipal Facilities & Operations

The Permittee will conduct employee training in both stormwater and non-stormwater BMPs related to municipal facilities and operations.  The training will focus on numerous aspects of General Services activities as highlighted below.

Maintenance and material storage areas that are used to conduct vehicle and equipment maintenance and store materials in areas may be exposed to rainfall or stormwater runoff. Facilities include:

- Areas used for servicing, fueling, or washing vehicles or equipment.  Most often, these facilities are found at corporation yards, local or regional parks, golf courses, fire stations, police stations, and municipal facility areas.

- Outdoor storage areas for chemicals, construction materials, and waste products associated with municipal activities.  These materials may include fuels and equipment fluids, wash waters, paving and base materials, pesticides, herbicides, and other substances that may affect water quality if released to the storm drain system.

Maintenance activities for landscape areas have the potential to contribute several types of pollutants to stormwater runoff.  Municipal staff will be briefed on proper material use and handling during landscaping operations.  These topics will be included in the ongoing training for General Services staff.

Proposed BMPs for maintenance/materials storage and landscape maintenance are discussed in Sections VI.II.II and VI.II.III.

City of Calexico SWMP

VI.I.III	Streets and Roads

The SWMP will sponsor municipal employee training specifically designed to educate staff on pavement maintenance, repair, and cleaning.  Permittee sweeps the streets at regular intervals, usually on a daily, weekly or monthly basis.

VI.I.IV	Storm Drain Systems

The program elements discussed in sections VI.I.I through VI.I.III focus on reducing or eliminating the water quality impacts of various public agency activities.  This program element focuses on proactive BMPs the Permittee can implement to improve stormwater quality.  Specifically, the storm drain systems element focuses on inspecting, and where necessary cleaning retention/detention basins, debris basins, open channels, drains inlets, and catch basins.  The primary stormwater pollution control benefits include maintaining basin performance and removing sediments and other accumulated pollutants. This element shares a common purpose with prevention and inspection activities discussed in Section III, IC/ID.  As described in Section III.I.III, the City of Calexico performs regular maintenance and inspections of its storm drain systems.  Additionally, General Services staff observes and report improper discharges to the storm drain system in the course of their regular field duties.

VI.II		Best Management Practices

This section identifies and describes BMPs proposed by the Permittee to address Municipal Agency Activities.

VI.II.I		Sewage Systems

The purpose of this program activity is to prevent sewage spills from reaching storm drain systems and receiving waters.  This will be achieved by reviewing, modifying if necessary, and implementing sewage spill response plans for Permittee with jurisdiction over a wastewater collection system.

Table VI-1
Sewage Systems BMP Summary

| BMP | Description | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| OM | Review, revise if necessary, and implement sewage spill response plans | City of Calexico | 6 Months |

City of Calexico SWMP

VI.II.II	Municipal Facilities & Operations

The BMPs proposed for Municipal Facilities & Operations address activities primarily involving maintenance and material storage and landscape maintenance.  Two BMPs are proposed to reduce or eliminate pollutant discharges from outdoor facilities used by the Permittee for vehicle and equipment maintenance or materials storage.

Table VI-2
Areas for Maintenance and Materials Storage BMP Summary

| BMPs | Description | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| OM | Identify storm drain facilities that serve Permittee fleet vehicle storage/maintenance areas. | City of Calexico | 6 Months |
| OM(New) | Prepare a model pollution prevention plan for Permittee maintenance areas. Implement the plan at maintenance areas. | City of Calexico | 18 Months |

The Permittee will rely primarily on staff training and education to minimize pollutant discharges from publicly maintained landscaped areas such as parks, medians, and civic center areas.  However, the Permittee will adopt a BMP to reinforce state regulations for pesticide use.

Table VI-3
Landscape Maintenance BMP Summary

| BMPs | Description | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| E1B | Educate/inform municipal agency personnel responsible for channel, park, golf course, and highway right-of-way maintenance about the proper use and management of pesticides, fertilizers, and herbicides. Alternative methods for controlling insects and weeds such as biological controls and the use of less toxic chemicals will be encouraged through internal workshops and guidance documents. | I.V.W.M.T.F. City of Calexico | 12 Months Ongoing |
| OM | Permittee will require that pesticides are applied in conformance with existing state and federal regulations. | City of Calexico | 12 Months |

City of Calexico SWMP

VI.II.III	Streets and Roads

The focus of the streets and roads program is two-fold: reduce or eliminate pollutant discharges from road maintenance activities and continue the existing street sweeping program to remove accumulated pollutants that result from roadway operation.  To accomplish these goals, the Permittee will implement a combination of BMPs.

Table VI-4
Streets and Roads BMP Summary

| BMPs | Description | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| OM6 | Evaluate pavement repair and maintenance programs for public streets and parking areas (e.g., fill potholes, seal cracks, and apply surface treatments). | City of Calexico | 12 Months |
| E | Develop a model fact sheet of BMPs for common road maintenance activities. Permittee will require road maintenance personnel to review periodically and implement the BMPs. | City of Calexico | 24 Months |
| OM | Develop model specifications to incorporate appropriate elements from the fact sheet into road maintenance contracts. Permittee will incorporate applicable elements into road maintenance contracts. | City of Calexico | 24 Months |

VI.II.IV	Storm Drain Systems

The storm drain element focuses on a comprehensive combination of public education, illicit discharge/illegal dumping detection and cleanup, and facility maintenance to reduce the amount of pollutants that can enter receiving waters.

City of Calexico SWMP

Table VI-5
Storm Drain Systems BMP Summary

| BMPs | Description | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| E3 | Educate/inform the general public on impacts of dumping pollutants into storm drainage systems. | City of Calexico | 12 Months |
| OM1 | Develop a program, continue and/or expand an existing field program to detect and prevent dumping or routinely discharging pollutants into storm drains and drainage channels. | City of Calexico | 24 Months |
| OM2 | Determine effectiveness of existing stormwater drainage system maintenance programs. | City of Calexico | 18 Months |
| OM | Establish measurable goals for stormwater drainage system maintenance programs for basins, inlets, and open channels, based on the results of OM2. | City of Calexico | 36 Months |

VI.III		Training

To support effective stormwater education for municipal staff, the Permittee will provide stormwater pollution prevention training sessions that address municipal activities. The Permittee presently conducts comprehensive training sessions that focus on all aspects of municipal facilities and roadway operations for General Services staff. Annual refresher workshops are proposed, and will be held in conjunction with IC/ID training described in Section III.III.

VI.IV		Evaluation/Assessment

Evaluation and assessment of BMP performance and Permittee compliance will be accomplished primarily through reporting and documentation. The proposed evaluation and assessment requirements for each BMP are shown in Table VI-6.

City of Calexico SWMP

Table VI-6
Evaluation & Assessment Summary

| BMPs | Assessment Target | Responsible Party | Time Schedule |
| --- | --- | --- | --- |
| OM | Permittee with jurisdiction over a wastewater collection system to confirm the sewage spill response plans are developed and current. | City of Calexico | Ongoing |
| OM (MSA) | Show compliance by ensuring that vehicle maintenance & material storage yards develop and maintain a pollution prevention plan. | City of Calexico | 24 Months |
| OM (MSA) | Permittee to show compliance by listing vehicle maintenance & materials storage areas and indicating applicable sections of model plan (described above) that have been implemented. | City of Calexico | 24 Months |
| E1B (LM) | Develop and implement data collection form. Quantify percentage of Permittee landscape maintenance personnel that have received training/outreach. Document staff training activities. | City of Calexico | 24 Months |
| OM (LM) | Provide list of pesticide application personnel and verify certification. | City of Calexico | 12 Months |
| OM6 (SR) | Develop and implement data collection form. | City of Calexico | 12 Months |
| E(SR) | Show compliance by providing copy of BMP fact sheet. Document fact sheet review by Permittee staff. | City of Calexico | 12 Months |
| OM (SR) | Document model specification. Permittee to verify that specifications included in applicable contracts. | City of Calexico | 24 Months |
| E3B (SDS) | Verify that stencils are maintained on all inlets. | City of Calexico | 48 Months |
| OM1 (SDS) | See Section III. | City of Calexico | 24 Months |
| OM2 (SDS) | Monitor statewide Phase 1 and 2 municipal program activities. Participate in evaluation activities. | City of Calexico | 24 Months |
| OM (SDS) | Describe maintenance activities. | City of Calexico | 12 Months |

Legend:

SWG	Sewage Systems

MSA	Maintenance/Materials Storage Areas

LM	Landscape Maintenance

SR	Streets & Roads

SDS	Storm Drain System