# Los Angeles Regional Water Quality Control Board

Surface Water Ambient Monitoring Program

Workplan for FY 2000/01 & 2001/02

Draft September 26, 2001

Table of Contents

- Introduction	  1

- Sites to be Monitored (Problematic or Clean)	  2

II.1  Santa Clara	  2

II.2  Calleguas Creek	  4

II.3  Dominguez Channel and Los Angeles and Long Beach Harbors	  6

- Objectives of Monitoring Program	10

III.1  Program Wide	11

III.2  Santa Clara	17

III.3  Calleguas Creek	18

III.4  Dominguez Channel and Los Angeles and Long Beach Harbors	19

III.5  General Study Design	20

III.5.1  Overview of General Approach	20

III.5.2  Water Quality Indicators	22

- Specific Activities Planned for Fiscal Year 00/01 and FY 01/02	22

IV.1  List of Waterbodies to be Sampled	22

IV.1.1  FY 00/01 (Santa Clara and Calleguas Creek)	22

IV.1.2  FY 01/02 (Dominguez Channel and LA/LB Harbors)	22

IV.2  Review of Available Information	22

IV.2.1  FY 00/01 (Santa Clara and Calleguas Creek)	23

IV.2.2  FY 01/02 (Dominguez Channel and LA/LB Harbors)	23

IV.3  Specific Sampling Design & Sample Collection	23

IV.3.1  FY 00/01 (Santa Clara and Calleguas Creek)	23

IV.3.2  FY 01/02 (Dominguez Channel and LA/LB Harbors)	25

IV.4  Laboratory Analysis	26

IV.5  Data Quality Evaluation and Data Reporting	26

IV.6  Deliverable Products	26

IV.6.1  Field Report	26

IV.6.2  Final Data Report	27

IV.7  Desired Milestone Schedule	27

IV.8  Desired Sample Throughput Schedule	28

IV.9  Budget	28

- Working Relationships	28

Table of Contents

- Other Information & Attachments	29

VI.1  Statewide Monitoring Programs (Mussel Watch, TSM, etc)	29

VI.2  Statewide Effort to Identify Reference Sites & Conditions	30

VI.3  Funding for Equipment and Additional Studies	30

VI.4  Summary Tables for Watershed Monitoring	A-1

VI.5  “Services to be performed at each station/cost” Spreadsheet	B-1

VI.6  Applicable sections of the 1996 Water Quality Assessment

VI.7  Maps

VI.7.1  Regional Map

VI.7.2  Impaired Waterbodies in the Region (Map)

VI.7.3  Santa Clara Map

VI.7.4  Calleguas Creek Map

VI.7.5  Dominguez Channel and LA/LB Harbors Map

# Introduction

The Porter-Cologne Water Quality Control Act and the federal Clean Water Act (CWA) direct water quality programs to implement efforts intended to protect and restore the integrity of waters of the State.  California Assembly Bill (AB) 982 (Water Code Section 13192) requires the State Water Resources Control Board (SWRCB) to assess and report on the State monitoring programs and to prepare a proposal for a comprehensive surface water quality monitoring program.  Ambient monitoring is independent of other water quality programs, such as the National Pollutant Discharge Elimination Program which requires monitoring in direct relation to regulation of point source discharges.  Ambient monitoring can serve as a measure of (1) the overall quality of water resources and (2) the overall effectiveness of Regional Water Quality Control Boards’ (RWQCBs’) prevention, regulatory, and remedial actions.  Current monitoring and assessment capability at the SWRCB is limited and tends to be focused on specific program needs.  This has led to a fragmentation of monitoring efforts resulting in gaps in needed information and a lack of integrated analysis.  AB 982 mandates the SWRCB create and implement a comprehensive surface water quality monitoring program.  The SWRCB and the RWQCBs have responded to this mandate with the development and implementation of the Surface Water Ambient Monitoring Program (SWAMP).  The program goals of SWAMP are:

- Identify specific problems preventing the SWRCB, RWQCBs, and the public from realizing beneficial uses in targeted watersheds.

- Create an ambient monitoring program that addresses all hydrologic units of the State using consistent and objective monitoring, sampling and analysis methods; consistent data quality assurance protocols; and centralized data management.

- Document ambient water quality conditions in potentially “clean” and polluted areas.

- Provide the data to evaluate the effectiveness of water quality regulatory programs in protecting beneficial uses of waters of the State.

The overall goal of the Site-Specific Monitoring portion of SWAMP is to develop site-specific information on sites or water bodies that are (1) known or suspected to have water quality problems and (2) known or suspected to be clean.  This portion of SWAMP is focused on collecting information from sites in water bodies of the State that could be potentially listed or delisted under Clean Water Act Section 303(d).  This workplan has been developed to implement the Site-Specific Monitoring Requirements of SWAMP.  In Region 4, both the Site-Specific Monitoring goals and the Regional Monitoring goals have been integrated into one ambient monitoring program.

Per AB 982, monitoring is required in each hydrologic unit of the State at least once every five years.  Region 4 proposes to visit each hydrologic unit one-year ahead of the WMI schedule for targeted watersheds which rotate on a five-year cycle.  In this strategy, data will be gathered, analyzed, and interpreted in time to use the following year during NPDES permit renewals and other ongoing activities within the targeted watershed.  The following table below provides a summary of the schedule for upcoming years:

| Hydrologic Unit | WMI Timeline | SWAMP Timeline |
| --- | --- | --- |
| Santa Clara River Calleguas Creek | 2001-2002 2001-2002 | 2000-2001 |
| Dominguez Channel LA & LB Harbors | 2002-2003 2002-2003 | 2001-2002 |
| Santa Monica Bay | 2003-2004 | 2002-2003 |
| Los Angeles River San Gabriel River Los Cerritos Channel | 2004-2005 2005-2006 2005-2006 | 2003-2004 |
| Channel Islands Ventura River Misc. Ventura Coastal | 2005-2006 2006-2007 2006-2007 | 2004-2005 |

The timeline is altered a little from the original intent to provide for the fact that Region 4 is transferring from a 7-year rotating basin schedule to a 5-year schedule.  Therefore, slight shuffling of the arrangements of hydrologic units to be visited may occur.

- Sites to be Monitored (Problematic or Clean)

This section will summarize site-specific problems(s), potential problem(s), or clean water locations to be monitored.

II.1  Santa Clara River

Background

The Santa Clara River Watershed encompasses approximately 1,200 square miles and 100 miles in river length.  This is the largest river system in southern California that remains in a relatively natural state and is a high quality natural resource for much of its length.  The river originates in the northern slope of the San Gabriel Mountains in Los Angeles County, traverses Ventura County, and flows into the Pacific Ocean halfway between the cities of San Buenaventura and Oxnard.  Extensive patches of high quality riparian habitat are present along the length of the river and its tributaries.  The endangered fish, the unarmored stickleback, is a resident of the river.  One of the largest of the Santa Clara River’s tributaries, Sespe Creek, is designated a wild trout stream by the state of California and supports significant spawning and rearing habitat.  The Sespe Creek is also designated a wild and scenic river.  Piru and Santa Paula Creeks, which are tributaries to the Santa Clara River, also support good habitat for steelhead.  In addition, the river serves as an important wildlife corridor.  A lagoon exists at the mouth of the river and supports a large variety of wildlife.

Increasing loads of nitrogen and salts into supplies of ground water threaten beneficial uses including irrigation and drinking water.  Other threats to water quality include increasing development in floodplain areas which has necessitated flood control measures such as channelization that results in a cycle of increased runoff volumes and velocities, erosion, and loss of habitat.  In many of these highly disturbed areas the exotic giant reed (Arundo donax) is gaining a foothold.

Many of the smaller communities in this watershed remain unsewered.  In particular, in the Agua Dulce area of the upper watershed, impacts on drinking water wells from septic tanks is a major concern.  The community is undertaking a wellhead protection effort, with oversight by Board staff.  Development pressure, particularly in the upper watershed, threatens habitat and the water quality of the river.  The effects of septic system use in the Oxnard Forebay area is also of concern.

Beneficial Uses

Above the estuary:  contact and non-contact water recreation, wildlife habitat, migratory

habitat, wetlands habitat, municipal supply, industrial service supply,

industrial process supply, agricultural supply, groundwater recharge,

freshwater replenishment, warm water habitat, cold water habitat,

and preservation of rare and endangered species.

In estuary:	contact and non-contact water recreation, wildlife habitat,

preservation of rare and endangered species, migratory habitat,

wetlands habitat, spawning habitat, estuarine habitat, marine

habitat, navigation, and commercial and sport fishing.

Known Impairments

Various reaches of this watershed are 303(d) listed for historic pesticides, chloride, coliform, nitrogen, trash, and eutrophication.

Monitoring Design & Strategy

The primary objective of monitoring in the Santa Clara Watershed is to provide a broad baseline of the overall health of the watershed.  Existing data is inconsistent and incomplete for both parameters and sampling locations.  The monitoring done in SWAMP will fill in many of the data gaps and provide data where none exists at all.  A broad suite of parameters will be tested at the various stations.

Thirty randomly selected sites (following the USEPA Environmental Monitoring and Assessment Program design protocol) have been selected for monitoring in the Santa Clara River.  These thirty sites will be sampled for toxicity, bioassessment, and conventional water chemistry, including nutrients (phosphate, nitrate, sulfate, chloride, and nitrite), TDS, boron, ammonia, chlorophyll a, dissolved oxygen, pH, depth, temperature, flow, conductivity, and turbidity.  After the California Department of Fish and Game (CDFG) completes the necessary field reconnaissance, a list of sites with lattitude and longitude coordinates and a map of the locations of the randomly chosen sites will be provided.  Additionally, one station at the base of each of the six main subwatersheds (Santa Paula, Sespe, Piru, Castaic, San Francisquito, and Bouquet) will be selected and analyzed for the same parameters listed above, as well as, bioaccumulation, metals chemistry in the water column and sediment, sediment grain size, and ELISA testing for chlorpyrifos and diazinon.  One station in the middle of the watershed, at Blue Cut, will be sampled for the same constituents as the 30 random sites.  This station was chosen to represent the surface water and ground water interaction in the area.  A station at the very bottom of the watershed (prior to meeting the estuary) will be tested for trace organic chemistry (pesticides and aroclors in the sediment) in addition to the parameters listed above.

II.2  Calleguas Creek

Background

Calleguas Creek and its major tributaries, Revolon Slough, Conejo Creek, Arroyo Conejo, Arroyo Santa Rosa, and Arroyo Simi drain an area of 343 square miles in southern Ventura County and a small portion of western Los Angeles County.   This watershed, which is elongated along an east-west axis, is about 30 miles long and 14 miles wide.   The northern boundary of the watershed is formed by the Santa Susana Mountains, South Mountain, and Oak Ridge; the southern boundary is formed by the Simi Hills and Santa Monica Mountains.

Land uses vary throughout the watershed.  Urban developments are generally restricted to the city limits of Simi Valley, Moorpark, Thousand Oaks, and Camarillo.  Although some residential development has occurred along the slopes of the watershed, most upland areas are still open space, however, golf courses are becoming increasingly popular to locate in these open areas.  Agricultural activities, primarily cultivation of orchards and row crops, are spread out along valleys and on the Oxnard Plain.

Mugu Lagoon, located at the mouth of the watershed, is one of the few remaining significant saltwater wetland habitats in southern California.  The Point Mugu Naval Air Base is located in the immediate area and the surrounding Oxnard Plain supports a large variety of agricultural crops.  These fields drain into ditches which either enter the lagoon directly or through Calleguas Creek and its tributaries.  Other fields drain into tile drain systems which discharge to drains or creeks.  Also in the area of the base are freshwater wetlands created on a seasonal basis to support duck hunting clubs.  The lagoon borders on an Area of Special Biological Significance (ASBS) and supports a great diversity of wildlife including several endangered birds and one endangered plant species.  Except for the military base, the lagoon area is relatively undeveloped.

Supplies of ground water are critical to agricultural operations and industry (sand and gravel mining) in this watershed.  Moreover, much of the population in the watershed relies upon ground water for drinking.

Aquatic life in both Mugu Lagoon and the inland streams of this watershed has been impacted by pollutants from nonpoint sources.  DDT, PCBs, other pesticides, and some metals have been detected in both sediment and biota collected from surface waterbodies of this watershed.  Additionally, ambient toxicity has been revealed in several studies from periodic toxicity testing in the watershed (ammonia from POTWs and pesticides such as diazinon and chlorpyrifos are implicated).  Fish collected from Calleguas Creek and Revolon Slough exhibit skin lesions and have been found to have other histopathologic abnormalities.  High levels of minerals and nitrates are common in the water column as well as in the groundwater.  Sediment toxicity is also elevated in some parts of the lagoon.  Reproduction is impaired in the resident endangered species, the light-footed clapper rail due to elevated levels of DDT and PCBs.  Overall, this is a very impaired watershed.  It appears that the sources of many of these pollutants are agricultural activities (mostly through continued disturbance and erosion of historically contaminated soils), which cover approximately 25% of the watershed along the inland valleys and coastal plain, although the nearby naval facility has also been a contributor.  Other nonpoint sources include residential and urban activities, which are present over approximately 25% of the watershed.  The remaining 50% of the watershed is still open space although there is a severe lack of benthic and riparian habitat.

Mugu Lagoon as well as the Calleguas Creek Estuary is considered a candidate toxic hot spot under the Bay Protection and Toxic Cleanup Program for reproductive impairment (the endangered clapper rail), exceedance of the state Office of Environmental and Health Hazard Assessment (OEHHA) advisory level for mercury in fish, and exceedance of the National Academy of Science (NAS) guideline level for DDT in fish, sediment concentrations of DDT, PCB, chlordane, chlorpyrifos, sediment toxicity and degraded benthic infaunal community.

Primary issues related to POTW discharges include ammonia toxicity and high mineral content (i.e., salinity), the latter, in part, due to imported water supplies.

Beneficial Uses

Above the estuary:  wildlife habitat, contact and non-contact water recreation, industrial

service supply, industrial process supply, preservation of rare and

endangered species, agricultural supply, groundwater recharge,

wetlands habitat, freshwater replenishment, and warm water habitat.

In estuary:	wildlife habitat, contact and non-contact water recreation, estuarine

habitat, marine habitat, preservation of rare and endangered

species, navigation, preservation of biological habitats, wetlands

habitat, migratory and spawning habitat, and shellfish harvesting.

Known Impairments

Various reaches of this watershed are 303(d) listed for nitrogen, water-soluble pesticides, salts, historic pesticides, metals, PCBs, sediment-bound organics, and trash.

Monitoring Design and Strategy

The Calleguas Creek watershed has been studied much more comprehensively than the Santa Clara River watershed utilizing both an EMAP approach and pre-selected sites.  The POTWs in the area have participated in a comprehensive monitoring program, in addition to their separate monitoring programs, that has produced a large database.  A number of toxicity studies have been performed in this watershed.  Toxicity is a known impairment of the Calleguas Creek watershed.  Toxicity has severe consequences for the aquatic life of the area and is therefore a high priority for additional monitoring.  Calleguas Creek and the surrounding areas also have other identified impairments such as nutrients and chloride.  Monitoring to determine the extent of these parameters and their associated problems is included in this ambient monitoring plan.  Current efforts are underway at the Regional Board to address these constituents through TMDL development.  However, the main focus in the Calleguas Creek Watershed in relation to the SWAMP will be toxicity.  Chlorpyrifos and diazinon are suspected causes of the toxicity.

The program for this watershed is based on a directed approach and the focus of this is to gather more information on the chronic toxicity problem and the potential causes.  A sub-objective of the monitoring design is to obtain and fill in data where it is missing or non-existent.  There will be 12 stations and the following analyses will be performed: toxicity, bioassessment, chlorophyll a, ammonia, dissolved oxygen, pH, depth, flow, conductivity, turbidity, temperature, conventional water chemistry including nitrate, nitrite, phosphorous, sulfate, and chloride, TDS, boron, metals chemistry, and organophosphate chemistry including chlorpyrifos and diazinon.  One station at the base of the watershed will have the above mentioned analyses and additionally will be sampled for bioaccumulation.  Funding has also been set aside for Phase II Toxicity Identification Evaluation (TIE) work because toxicity is a known problem as documented by previous Regional Board studies conducted by UC Davis.  A list of the stations and a map of their locations will be provided in the near future.

II.3  Dominguez Channel / Los Angeles and Long Beach Harbors

Background

The Los Angeles and Long Beach Harbors are located in the southern portion of the Los Angeles Basin.  Along the northern portion of San Pedro Bay is a natural embayment formed by a westerly extension of the coastline which contains both harbors, with the Palos Verdes Hills the dominant onshore feature.  Historically, the area consisted of marshes and mudflats with a large marshy area, Dominguez Slough, to the north, and flow from the Los Angeles River entering where Dominguez Channel now drains.  Near the end of last century and during the beginning of this one, channels were dredged, marshes were filled, wharves were constructed, the Los Angeles River was diverted, and a breakwater was constructed in order to allow deep draft ships to be directly offloaded and products be swiftly moved.  The Dominguez Slough was completely channelized and became the drainage endpoint for runoff from a highly industrialized area.  Eventually, the greater San Pedro Bay was enclosed by two more breakwaters and deep entrance channels were dredged to allow for entry of ships with need of 70 feet of clearance.  The LA/LB Harbor complex together is now one of the largest ports in the country.

Both harbors are considered to be one oceanographic unit.  Despite its industrial nature, contaminant sources, and low flushing ability, the inner harbor area supports fairly diverse fish and benthic populations and provides a protected nursery area for juvenile fish.  The California least tern, an endangered species, nests in one part of the harbor complex.

Similar to LA Inner Harbor in many respects, LB Inner Harbor is dissimilar to the other Port in the higher number of privately owned waterfront parcels which the Port has recently been in the process of buying up and converting to Port-related uses, generally container terminals.  Also, basins and slips in LB Inner Harbor are somewhat more separated from each other than in LA Inner Harbor which may possibly prevent contamination from spreading easily.

The outer part of both harbors (the greater San Pedro Bay) has been less disrupted and supports a great diversity of marine life.  It is also open to the ocean at its eastern end and receives much greater flushing than the inner harbors.

- A POTW discharges secondary-treated effluent to the outer LA/LB Harbor and is under a time schedule order to remove the discharge.  The discharger's plan consists of achieving full reclamation (mostly for industrial reuse purposes) by 2020 which would eliminate the discharge completely.  They plan on achieving about 80% reclamation by 2005.  Two generating stations discharge to the inner harbor areas.  Many smaller, non-process waste discharges also occur into the harbors and Dominguez Channel drains a highly industrialized area of the city resulting in very poor water quality.

Two areas within Los Angeles Harbor are considered to be toxic hot spots under the BPTCP:  Dominguez Channel/Consolidated Slip, based on sediment concentrations of DDT, PCB, cadmium, copper, lead, mercury, zinc, dieldrin, chlordane (all exceed sediment quality guidelines), sediment toxicity, and degraded benthic infaunal community;  and Cabrillo Pier area, based on sediment concentrations of DDT, PCB and copper, sediment toxicity and issuance of a human health (fishing) advisory for DDT and PCB in white croaker and exceedances of NAS guidelines for DDT in fish and shellfish.  Several locations have been listed as sites of concern under the BPTCP:  Inner Fish Harbor, due to sediment concentrations of DDT, PCB, copper, mercury and zinc and sediment toxicity (not recurrent);  Kaiser International, due to sediment concentrations of DDT, PCB, PAH, copper and endosulfan;  Hugo Neu-Proler, due to PCB sediment concentrations;  Southwest Slip, due to sediment concentrations of DDT, PCB, PAH, mercury, and chromium, and sediment toxicity (not recurrent);  Shoreline Marina, due to sediment concentrations of zinc, DDT, PCB, chlordane and PAH, and sediment toxicity (not recurrent);  Long Beach Outer Harbor, due to sediment concentrations of DDT and chlordane and sediment toxicity (not recurrent);  West Basin, due to sediment concentrations of DDT and PCB, sediment toxicity (not recurrent) and accumulation in clam tissue;  Alamitos Bay, due to sediment concentrations of DDT and chlordane.  There is need for further monitoring in all of these areas to clarify their status.  Potential sources of these materials are considered to be historical deposition, discharges from the nearby POTW (especially for metals), spills from ships and industrial facilities, as well as stormwater runoff.  Many areas of the harbors have experienced soil and/or groundwater contamination, which may result in possible transport of pollutants to the harbors' surface waters.  Dredging and disposal of contaminated sediments and source control of pollutants in the harbors will be a major focal point for the Contaminated Sediment Task Force.

Los Angeles Inner Harbor

Although the area is dramatically cleaner now than twenty-five years ago, parts of LA Inner Harbor are still suffering the effects of historic deposits of pollutants in the sediment and current point and nonpoint source discharges.  Fish caught in the East Basin have exhibited histopathological abnormalities (liver lesions).  The abnormalities are indicative of aromatic and chlorinated hydrocarbon contamination.  There is also significant degradation in the biological community of a part of Inner Harbor with high levels of  PCB and DDT; and toxicity of the surface water microlayer of one part of the harbor to a test fish species (larval kelp bass).  Additionally, Cal-EPA's Office of Environmental Health Hazard Assessment now advises against consumption of white croaker in the harbor and recommends no more than one meal every two weeks of black croaker, queenfish, and surfperches if caught in the harbor.  On the other hand, the benthic community in many other areas of the inner harbor are healthy and sediments, though high in many pollutants, do not cause a great deal of toxicity in controlled lab tests.

LA Inner Harbor is on the 1998 303(d) list due to DDT, metals, PAHs, chlordane, TBT, and PCBs. Some of the contamination in sediment is historic with resuspension potential.  Dominguez Channel was the recipient of runoff from the Montrose Chemical Facility which manufactured DDT several decades ago.   There are also mostly nonpoint source inputs from several problem sites, spills, and storm drain runoff.  The problems tend to be exacerbated by the poor circulation and flushing.  The Port is in the process of filling in a large part of Outer Harbor and deepening some channels as part of their  "2020 Plan".  Pier 400, a 590-acre site of new land created by diking and filling harbor waters, was completed in April 2000.  As a result, the potential exists for greater stagnation and more problems from deposition of new contaminants.

Data from the State Mussel Watch (SMW) Program have documented high levels of metals, PCBs, TBT, and PAHs in mussel tissue at several locations in LA Inner Harbor.   The Bay Protection and Toxic Cleanup Program (BPTCP) has found a number of inner harbor areas with elevated pollutant levels but a smaller number of those have exhibited sediment toxicity.

Sediment data collected by Regional Board staff, the Port of LA, and various other researchers, have revealed several areas of heavy contamination with metals, PCBs, and DDT, and occasionally PAHs.  Regional Board data show that the level of contamination within particular regions of the inner harbor vary considerably from site to site.  Additionally, it is difficult to separate the effects of historic contamination from current inputs.  Bight’98 included samples within harbors, including a number of stations in LA/LB Harbor; toxicity, sediment chemistry, and benthic data reports should be available in 2001.

Dominguez Channel

Little recent data exist for the Channel itself even though considerable heavy industrial facilities (including the old Montrose site) are located within the watershed.  However, a consultant for Montrose conducted sediment sampling for DDT in the Channel during 1990.  EPA, in a letter to Montrose, cited this data and provided a comparison of those values with NOAA's "identified concentrations of DDT in sediment associated with adverse impacts.  A sediment level of 3 ppb was associated with adverse impacts in 10% (ER-L) of the data reviewed by NOAA and a level of 350 ppb total DDT was associated with adverse impacts in 50% (ER-M) of the data reviewed by NOAA" (EPA letter to Montrose Chemical Corporation, November 27, 1991).  The consultant found DDT levels of 300 - 13,000 ppb in the Channel.  EPA stated that adverse impacts in the biological community of Dominguez Channel and Consolidated Slip would be expected.

A Regional Board study conducted in 1975 found that the aquatic biota of the Channel were largely marine in origin and were a continuation of LA Inner Harbor biota.  The number and abundance of aquatic species declined with distance inland from the harbor.  A fairly abrupt decline in benthic species between Alameda and Wilmington Streets was attributed to the effects of pollution.  Capitella capitata was one of the most abundant benthic species in the area and is generally associated with polluted areas.  An absence of benthic fish species adjacent to one oil refinery was considered to be indicative of oxygen-poor bottom water.  There was a degraded benthic community at several stations in Consolidated Slip during BPTCP sampling.

Of major concern in the mid-1980s was discharge of zinc chromate as an additive in cooling water/boiler blowdown.  There may have been some justification for that concern.  Sediment sampling conducted by Regional Board staff in 1988 revealed zinc levels as high as 447 ppm, chromium as high as 67 ppm, and lead as high as 231 ppm.

Long Beach Inner Harbor

While historic contamination is a definite problem in the older parts of the harbor (including the naval base), Pier J has only recently been constructed, utilizing some highly contaminated dredge material.  Some other likely problem sites include: Cerritos Channel with its inputs at times from Consolidated Slip (water generally flows from LB to LA Harbors), a creosote manufacturing site, several oil terminals, a defunct ship repair yard (and several active ones), and the naval base, which is closed, while the attached shipyard remains open.

Contamination in the LB Inner Harbor is known to be sporadic.  Little information is available on contamination in Southeast Basin except for TBT water concentrations of up to 380 PPT found in a 1988 statewide study of harbors and low levels of PCBs found in mussel tissue in 1986.  The most recent SMW data for the Inner Harbor show some areas of elevated DDT, most notably at those stations located in or near Cerritos Channel.

Moderate PCB levels were found in mussel tissue in front of the creosote facility located in Channel 2 and somewhat higher levels were found in Cerritos Channel which is likely related to its proximity to Consolidated Slip and other LA Harbor point and nonpoint sources.  Long Beach Inner Harbor is on the 1998 303(d) list for DDT, PAHs, and PCBs, while San Pedro Bay is listed for DDT, PAHs, PCBs, and some metals.

Beneficial uses

Above the estuary:  non-contact recreation, preservation of rare and endangered

species

In estuary:	contact and non-contact recreation, preservation of rare and

endangered species, industrial water supply, navigation,

commercial and sport fishing, marine habitat, estuarine habitat,

wildlife habitat, migratory and spawning habitat.

Known Impairments

The area is 303(d) listed for PCBs, DDT, historic pesticides, PAHs, coliforms, metals, nitrogen, ammonia, TBT, and trash.  Currently, a TMDL is being developed to address the coliform contamination and is scheduled to be completed during fiscal year 2001-2002.

Monitoring Design and Strategy

Regional Board staff have divided the Dominguez Channel and LA/LB harbor area into the following six subareas based on characteristics of the area in order to simplify sampling design: (1) headwater streams, (2) the inner and outer harbors of LA and LB, (3) Madrona Marsh, (4) Lake Machado, (5) the Dominguez Channel estuary, and (6) the upper channelized Dominguez Channel above normal tidal influence.  The sampling design is still under development, however, if funding constraints are not restrictive, each of the six areas will be sampled to fill in the incomplete data relevant to each area.  For example,  the data for Lake Machado is old and limited and the lake is posted for fishing, therefore, studies will include fish tissue analysis in conjunction with water column chemistry and toxicity, sediment chemistry and toxicity, and pathogens.  The LA/LB harbor complex is another example which represents a different sampling strategy than the one proposed for Lake Machado.  The sampling of the harbor complex will include five weeks of coliform and pathogen testing in the summer and winter, water column toxicity and chemistry, metals chemistry, PAH analysis, and potential TIEs.  The ability to break down this watershed into subareas based on characteristics of the area identified allow staff to devise sampling plans and monitor for constituents in relation to each area.

A list of the specific sites and a map of the sampling locations is unavailable at this time, but will be provided in the near future.

- Objectives of Monitoring Program

In the development of a comprehensive monitoring program, the following steps should be followed where applicable:

- Identify overall objectives and sub-objectives (defined in SWAMP Proposal and Report to Legislature)

- Identify assessment questions (defined by Regional Board staff)

- Develop a conceptual model

- Select indicators (Regional Board staff per SWAMP documents)

- Design spatial and temporal aspects of an ideal monitoring program (Regional Board staff in conjunction with assistance from US EPA)

- Assess quality of data needed for results (being done at Statewide level)

- Examine reporting options (being done at Statewide level)

In the design of objectives, the following guidelines should be remembered:

- Objectives can be both wide-scale and/or site-specific

- Objectives can be incorporated into study through a multi-tiered design that addresses:

- baseline ecosystem level condition

- long term trend analysis

- hypothesized environmental problems

- flexibility for the future

The indicators chosen should be regionally responsive, unambiguously interpretable, allow for simple quantification (i.e., synoptic sampling), should have index period stability, low year to year variation (especially if used for trend analysis), and represent the environmental impact of concern.  Indicators should also have an available EPA approved method, historical use with database of results, be retrospective, anticipatory (provide early warning of widespread change in ecosystem) and cost effective.

III.1  Program Wide

A.  Beneficial Use:  Water Contact Recreation – Is it safe to swim?

i.  SWAMP Objective

At sites influenced by point sources (e.g., storm drains, publicly owned treatment works, etc.) or nonpoint sources of pathogenic contaminants, estimate the concentration of bacteria above screening values, health standards, or adopted water quality objectives.

ii.  Potential Assessment Questions

- What is the percent of streams in the watershed/region that support their designated beneficial use of water contact recreation?  Do they support their designated beneficial uses all year or a majority of the time?

- Is the percent of streams in the watershed/region which support the beneficial use of water contact recreation increasing or decreasing over time?  (For region-wide answer, will need to complete 10 year data cycle to answer… for watershed specific answer, can answer for first round of watersheds in year 6)

iii.  Potential Indicators:  Contaminant exposure:  total coliform bacteria, fecal coliform

bacteria, enterococcus bacteria, enteric viruses.  Another potential indicator not

mentioned in SWAMP is E. coli.

B.  Beneficial Use:  Municipal and Domestic Water Supply - Is it safe to drink the water?

i.  SWAMP Objective

At specific locations in lakes, rivers, and streams that are sources of drinking water and suspected to be contaminated, estimate the concentration of or verify previous estimates of the concentration of microbial and chemical contaminants above screening values, drinking water standards, or adopted water quality objectives used to protect drinking water quality.

ii.  Potential Assessment Questions

- What is the percent of streams that support their designated beneficial use of municipal and domestic water supply?

- Is the percent of streams in the watershed/region which support the beneficial use of municipal and domestic water supply increasing or decreasing over time?

iii. Potential Indicators:  Contaminant exposure:  inorganic water chemistry, nutrients,

organic water chemistry, total coliform bacteria, cryptosporidium, giardia

C.  Beneficial Uses:  Commercial and Sport Fishing, Shellfish Harvesting – Is it safe to

eat fish and other aquatic resources?

i.  SWAMP Objective

- At specific sites influenced by sources of bacterial contaminants, estimate the concentration of bacterial contaminants above health standards or adopted water quality objectives to protect shellfish harvesting areas.

- At specific sites influenced by sources of chemical contaminants, estimate the concentration of chemical contaminants in edible aquatic life tissues above advisory levels and critical thresholds of potential human health risk.

- At frequently fished sites, estimate the concentration of or verify previous estimates of chemical contaminants in commonly consumed fish and shellfish target species above advisory levels and critical thresholds of potential human health risk (adapted from USEPA, 1995).

- Throughout water bodies (streams, rivers, lakes, nearshore waters, enclosed bays and estuaries), estimate the concentration of chemical contaminants in fish and aquatic resources from year to year using several critical threshold values of potential human impact (advisory or action levels).

ii.  Potential Assessment Questions

- What is the percent of streams that support their designated beneficial uses of commercial fishing, sport fishing, and shellfish harvesting?

- Is the percent of streams in the watershed/region which support the beneficial uses of commercial fishing, sport fishing, and shellfish harvesting increasing or decreasing over time?

iii. Potential Indicators:  Contaminant exposure:  fish tissue chemistry, shellfish tissue

chemistry, coliform bacteria in shellfish

D.  Beneficial Uses:  Cold Freshwater Habitat, Estuarine Habitat, Marine Habitat,

Preservation of Biological Habitats (Preservation of Rare and Endangered Species), Rare, Threatened or Endangered Species, Warm Freshwater Habitat, Wildlife Habitat – Are aquatic populations, communities, and habitats protected?

i.  SWAMP Objective

- At sites influenced by point sources or nonpoint sources of pollutants, identify specific locations of degraded water or sediments in rivers, lakes, nearshore waters, enclosed bays, or estuaries using several critical threshold values of toxicity, water column or epibenthic community analysis, habitat condition, and chemical concentrations.

- Identify the areal extent of degraded sediment locations in rivers, lakes, nearshore waters, enclosed bays, and estuaries  using several critical threshold values of toxicity, benthic community analysis, habitat condition, and chemical concentration.

ii.  Potential Assessment Questions

- What is the percent of streams that support their designated beneficial uses of cold water habitat, estuarine habitat, marine habitat, preservation of rare and endangered species, warm freshwater habitat, and wildlife habitat?

- Is the percent of streams in the watershed/region which support the beneficial uses of cold water habitat, estuarine habitat, marine habitat, preservation of rare and endangered species, warm freshwater habitat, and wildlife habitat increasing or decreasing over time?

- What is the distribution of benthic conditions in streams of the watershed?

- What is the distribution of the total number of benthic species per site at each station sampled?

- What is the distribution of exotic species in the benthos in this watershed?

- What proportion of streams have an altered/degraded benthic community structure?

- What is the distribution of sediment contaminants in this watershed?

iii.  Potential Indicators:

Biological responses:  benthic infauna, fish assemblage, fish pathology,

interstitial water toxicity, macroinvertebrate assemblage, periphyton, sediment toxicity, water toxicity

Pollutant exposure:  acid volatile sulfides, debris, interstitial water metal chemistry, report gene system (RGS450), organic and inorganic sediment chemistry, shellfish or fish tissue chemistry, nutrients, inorganic and organic water chemistry

Habitat:  dissolved oxygen, sediment grain size, sediment organic carbon, water

flow, water temperature, channel morphology, wetland vegetation, riparian vegetation

E.  Beneficial Uses:  Spawning, Reproduction and/or Early Development – Are aquatic populations, communities, and habitats protected?

i.  SWAMP Objective

At sites influenced by point sources or nonpoint sources of pollutants, identify specific locations of or verify previous measurements identifying specific locations of degraded water or sediments in rivers, lakes, nearshore waters, enclosed bays, or estuaries using several critical threshold values of early life-stage toxicity, chemical concentrations, and physical characteristics.

ii.  Potential Assessment Questions

- What is the percent of streams that support their designated beneficial use of spawning, reproduction, and/or early development?

- Is the percent of streams in the watershed/region which support the beneficial use of spawning, reproduction, and/or early development increasing or decreasing over time?

- What is the distribution of benthic conditions in streams of the watershed?

- What is the distribution of the total number of benthic species per site at each station sampled?

- What is the distribution of exotic species in the benthos in this watershed?

- What proportion of streams have an altered/degraded benthic community structure?

- What is the distribution of sediment contaminants in this watershed?

- What is the distribution of toxicity in this watershed?

iii.  Potential Indicators:

Biological responses:  benthic infauna, fish assemblage, fish pathology, interstitial

water toxicity, macroinvertebrate assemblage, periphyton, sediment toxicity, water toxicity

Pollutant exposure:  acid volatile sulfides, debris, interstitial water metal chemistry, report gene system (RGS450), organic and inorganic sediment chemistry, shellfish or fish tissue chemistry, nutrients, inorganic and organic water chemistry

Habitat:  dissolved oxygen, sediment grain size, sediment organic carbon, water flow, water temperature, channel morphology, wetland vegetation, riparian vegetation

F.  Beneficial Uses:  Migration of Aquatic Organisms; Rare, Threatened or Endangered

Species;  Wildlife Habitat – Is water flow sufficient to protect fisheries?

i.  SWAMP Objective

At specific sties influenced by pollution, estimate the presence of conditions or verify previous estimates of the presence of conditions necessary for the migration and survival of aquatic organisms, such as anadromous fish, using measures of habitat condition including water flow, watercourse geomorphology, sedimentation, temperature, and biological communities.

ii.  Potential Assessment Questions

- What is the percent of streams that support their designated beneficial uses of migration of aquatic organisms, rare, threatened or endangered species, and wildlife habitat?

- Is the percent of streams in the watershed/region which support the beneficial uses of migration of aquatic organisms, rare, threatened or endangered species, and wildlife habitat increasing or decreasing over time?

- What is the distribution of benthic conditions in streams of the watershed?

- What is the distribution of the total number of benthic species per site at each station sampled?

- What is the distribution of exotic species in the benthos in this watershed?

- What proportion of streams have an altered/degraded benthic community structure?

- What is the distribution of sediment contaminants in this watershed?

- What is the distribution of toxicity in this watershed?

iii.  Potential Indicators:

Habitat:  water flow, suspended solids, channel morphology, water temperature

Another potential indicator not listed in SWAMP is dissolved oxygen levels.

Biological response:  fish assemblage, macroinvertebrate assemblage,  periphyton, wetland habitat, riparian habitat

G.  Beneficial Use:  Agricultural Supply – Is water safe for agricultural use?

i.  SWAMP Objective

At specific locations in lakes, rivers, and streams that are used for agricultural purposes, estimate the concentration of or verify previous estimates of the concentrations of chemical pollutants above screening values or adopted water quality objectives used to protect agricultural uses.

ii.  Potential Assessment Questions

- What is the percent of streams that support their designated beneficial use of agricultural supply?

- Is the percent of streams in the watershed/region which support the beneficial use of agricultural supply increasing or decreasing over time?

iii.  Potential Indicators:  None listed in the SWAMP documents.  Alternatives includeTDS, chloride, and heavy metals.

H.  Beneficial Use:  Industrial Source Supply, Industrial Process Supply – Is water safe for industrial use?

i.  SWAMP Objective

At specific locations in coastal waters, enclosed bays, estuaries, lakes, rivers and streams that are used for industrial purposes, estimate the concentration of or verify previous estimates of the concentration of chemical pollutants above screening values or adopted water quality objectives used to protect industrial use.

ii.  Potential Assessment Questions

- What is the percent of streams that support their designated beneficial uses of industrial source and process supply?

- Is the percent of streams in the watershed/region which support the beneficial uses of industrial source and process supply increasing or decreasing over time?

iii.  Potential Indicators:  None listed in the SWAMP documents.  Alternatives include

odor, scum, bacteria, trash, and algae.

I.  Beneficial Use:  Non-Contact Water Recreation – Are aesthetic conditions of the water protected?

i.  SWAMP Objective

At specific locations in coastal waters, enclosed bays, estuaries, lakes, rivers and streams, estimate the aesthetic condition or verify previous estimates of the aesthetic condition above screening values or adopted water quality objectives to protect non-contact water recreation.

ii.  Potential Assessment Questions

- What is the percent of streams that support their designated beneficial use of non-contact water recreation?

- Is the percent of streams in the watershed/region which support the beneficial use of non-contact water recreation increasing or decreasing over time?

iii.  Potential Indicators:  None listed in the SWAMP documents.

III.2  Santa Clara River

Because the design of this section of the monitoring program is a probability-based design, the Regional Board will be able to infer the status of the watershed based on the sampling of the thirty randomly selected stations.  The types of questions that we will be able to answer are as follows:

- What is the percent of streams that support their designated beneficial uses of commercial fishing and sport fishing?

- Is the percent of streams in the watershed/region which support the beneficial uses of commercial fishing and sport fishing increasing or decreasing over time?

- What is the percent of streams that support their designated beneficial uses of cold water habitat, estuarine habitat, marine habitat, preservation of rare and endangered species, warm freshwater habitat, and wildlife habitat?

- Is the percent of streams in the watershed/region which support the beneficial uses of cold water habitat, estuarine habitat, marine habitat, preservation of rare and endangered species, warm freshwater habitat, and wildlife habitat increasing or decreasing over time?

- What is the distribution of benthic conditions in streams of the watershed?

- What is the distribution of the total number of benthic species per site at each station sampled?

- What is the distribution of exotic species in the benthos in this watershed?

- What proportion of streams have an altered/degraded benthic community structure?

- What is the distribution of sediment contaminants in this watershed?

- What is the percent of streams that support their designated beneficial use of spawning, reproduction, and/or early development?

- Is the percent of streams in the watershed/region which support the beneficial use of spawning, reproduction, and/or early development increasing or decreasing over time?

- What is the distribution of toxicity in this watershed?

- What is the percent of streams that support their designated beneficial uses of migration of aquatic organisms, rare, threatened or endangered species, and wildlife habitat?

- Is the percent of streams in the watershed/region which support the beneficial uses of migration of aquatic organisms, rare, threatened or endangered species, and wildlife habitat increasing or decreasing over time?

The directed stations at the bottom of each major subwatershed in the Santa Clara River Watershed will provide the Regional Board with site specific information and allow a management decision to be made based on the data analysis and determination if each site has an impairment or not.  The presence of an impairment will be followed up by further monitoring both at the site already sampled and by monitoring upstream of the location as source identification is pursued.  Examples of site-specific questions can be seen below in the Calleguas Creek section.

III.3  Calleguas Creek

A directed study design has been selected for Calleguas Creek because a large sample data set already exists for this watershed.  The directed study will provide answers to questions regarding specific sites.  Because of the directed study design of the monitoring to be conducted in the Calleguas Creek area, potential watershed-wide assessment questions such as those mentioned above cannot be answered with any statistical confidence for the watershed as a whole.  The questions this program can answer are directed to a single site.  Examples are as follows:

- Does this site support its designated beneficial use of shellfish harvesting?

- Is the ability of this site to support the beneficial use of shellfish harvesting increasing or decreasing over time?

- Does this site support its designated beneficial uses of cold water habitat, estuarine habitat, marine habitat, preservation of rare and endangered species, warm freshwater habitat, and wildlife habitat?

- Is the ability of this site to support the beneficial uses of cold water habitat, estuarine habitat, marine habitat, preservation of rare and endangered species, warm freshwater habitat, and wildlife habitat increasing or decreasing over time?

- What is the distribution of benthic conditions at this site?

- What is the distribution of the total number of benthic species at this site?

- What is the distribution of exotic species in the benthos at this site?

- Does this site have an altered/degraded benthic community structure?

- Does this site support its designated beneficial use of spawning, reproduction, and/or early development?

- Is the ability of this site to support the beneficial use of spawning, reproduction, and/or early development increasing or decreasing over time?

- What is the distribution of toxicity at this site?

- What is the distribution of nutrients at this site?

- Does this site support its designated beneficial uses of migration of aquatic organisms, rare, threatened or endangered species, and wildlife habitat?

- Is the ability of this site to support the beneficial uses of migration of aquatic organisms, rare, threatened or endangered species, and wildlife habitat increasing or decreasing over time?

III.4  Dominguez Channel and Los Angeles and Long Beach Harbors

The sampling design for this area is going to be a mix of both directed and probabilistic.  Numerous sediment contamination and toxicity data presently exists and SWAMP monitoring efforts will be focused on directed sampling to follow up on the existing data.  However, water chemistry data is almost non-existent and SWAMP efforts will be designed in a random fashion for this effort in the inner and outer harbor areas.  Therefore, the Regional Board will be able to answer a mix of questions as evidenced above in both the Santa Clara and Calleguas Creek sections.

- Does this site support its designated beneficial use of water contact recreation?

- Is the ability of this site to support the beneficial use of water contact recreation increasing or decreasing over time?

- Does this site support its designated beneficial uses of commercial fishing and sport fishing?

- Is the ability of this site to support the beneficial uses of commercial fishing and sport fishing increasing or decreasing over time?

- What is the percent of streams that support their designated beneficial uses of cold water habitat, estuarine habitat, marine habitat, preservation of rare and endangered species, warm freshwater habitat, and wildlife habitat? (May also be site-specific.)

- Is the percent of streams in the watershed/region which support the beneficial uses of cold water habitat, estuarine habitat, marine habitat, preservation of rare and endangered species, warm freshwater habitat, and wildlife habitat increasing or decreasing over time?  (May also be site specific.)

- What is the distribution of benthic conditions at this site in the watershed? (May be the estuary, lake, or wetlands.)

- What is the distribution of the total number of benthic species per site at each station sampled? (May be the estuary, lake or wetlands.)

- What is the distribution of exotic species in the benthos in this watershed? (May be the estuary, lake or wetlands.)

- Does this site have an altered/degraded benthic community structure? (May be the estuary, lake or wetlands.)

- What is the distribution of sediment contaminants at this site or in this watershed?

- What is the percent of streams that support their designated beneficial use of spawning, reproduction, and/or early development?  Does this site meet its designated beneficial use of spawning, reproduction, and/or early development?

- Is the percent of streams in the watershed/region which support the beneficial use of spawning, reproduction, and/or early development increasing or decreasing over time?  Is the ability of this site to support this beneficial use increasing or decreasing over time?

- What is the distribution of toxicity in the harbors or at a specific site?

- What is the percent of the harbor that support their designated beneficial uses of migration of aquatic organisms, rare, threatened or endangered species, and wildlife habitat?  Does this site support its designated beneficial use?

- Is the percent of the harbors in the watershed/region which support the beneficial uses of migration of aquatic organisms, rare, threatened or endangered species, and wildlife habitat increasing or decreasing over time? (Can be site specific too.)

III.5  General Study Design

III.5.1.  Overview of General Approach

Overall General Approach

The sampling and analysis will be used to assess the ambient conditions of the watersheds in the Los Angeles and Ventura counties.  The sampling and analyses described in this Workplan will further delineate the nature, extent, and sources of toxic pollutants which have been detected or are suspected to be problematic for this region and its individual watersheds. Where applicable, a triad approach (benthic community analysis, water chemistry, and toxicity testing) has been used.  Ultimately, the information from these analyses will be used in the water quality assessment.  The bioaccumulation tests are being conducted in order to address possible human health concerns (contaminants in edible fish tissue) and ecologic concerns (benthic community impacts) which could result if the contaminants at a site were bioavailable for uptake by organisms.  This bioaccumulation test will help to demonstrate the bioavailability of contaminants at these stations.  There is also a large focus on bioassessment which historically has been overlooked.  The information gathered will be used in trend analysis as well as the potential identification of reference sites which could then potentially be used in the development of an index of biological integrity.

A focused, comprehensive sampling for chemistry, toxicity, bioaccumulation, and bioassessment is indicated as shown on the attached table “Services to Be Performed at Each Station/Cost.”

Santa Clara

The sampling in the Santa Clara watershed focuses on 30 randomly chosen stations in the EMAP fashion with sampling consisting of toxicity, bioassessment, and conventional water chemistry, including nutrients (phosphate, nitrate, sulfate, chloride, and nitrite), TDS, boron, ammonia, chlorophyll a, dissolved oxygen, pH, depth, flow, temperature, conductivity, and turbidity.  Additionally, one station at the base of each of the six main subwatersheds will be selected and analyzed for the same parameters listed above, as well as, bioaccumulation, metals chemistry in the water column and sediment, sediment grain size, and ELISA testing for chlorpyrifos and diazinon.  One station in the middle of the watershed, at Blue Cut, will be sampled for the same constituents as the 30 random sites.  This station was chosen to represent the surface water and ground water interaction in the area.  The station at the very bottom of the watershed (prior to meeting the estuary) will be tested for trace organic chemistry in addition to the parameters listed above.  Where applicable, a triad approach (benthic community analysis, water chemistry, and toxicity testing) has been used.

Calleguas Creek

The Calleguas Creek watershed has been studied much more comprehensively than the Santa Clara River watershed utilizing both an EMAP approach and pre-selected sites.  The POTWs in the area have participated in a comprehensive monitoring program, in addition to their separate monitoring programs, that has produced a large database.  The focus of the monitoring program is on known toxicity problems and the potential source identification.  The program for this watershed is based on a directed approach.  There will be 12 stations and the following analyses will be performed: toxicity, bioassessment, boron, TDS, nutrients including phosphate, nitrate, sulfate, chloride and nitrite, metals chemistry, chlorophyll a, ammonia, dissolved oxygen, pH, flow, temperature, conductivity, turbidity, depth, and organophosphate chemistry including chlorpyrifos and diazinon.  The thirteenth station at the base of the watershed will have the above mentioned analyses and additionally will be sampled for bioaccumulation. Funding has also been set aside for Phase II TIE work because toxicity is a known problem as documented by previous Regional Board studies performed by UC Davis.

Dominguez Channel and Los Angeles and Long Beach Harbors

Staff proposes to continue focusing on a randomized probabilistic design as modeled after the EPA EMAP program, especially for the inner and outer harbor areas.  The triad approach will be utilized where possible.  Sediment chemistry and toxicity studies will most likely be directed to sites which have previous data or to sites to fill in information where it does not exist today.  Analysis of sampling points will consist of bacteria, bioassessment, toxicity (sediment and water column), sediment chemistry, water chemistry, fish tissue studies, pathogens, DO, sediment grain size, flow, temperature, and physical habitat assessment, with the potential for TIE studies (Details to follow – plan still in developmental stages).

III.5.2.  Water Quality Indicators

Santa Clara  River

The following indicators will be used: toxicity, bioassessment (macroinvertebrate assemblage includes physical habitat assessment), and conventional water chemistry, including nutrients (phosphate, nitrate, sulfate, chloride, and nitrite), TDS, boron, ammonia, chlorophyll a, dissolved oxygen, pH, depth, temperature, conductivity, turbidity, bioaccumulation (shellfish tissue chemistry), metals chemistry in the water column and sediment, ELISA testing for chlorpyrifos and diazinon, and trace organic chemistry.

Calleguas Creek

The following indicators will be used:  toxicity, bioassessment (macroinvertebrate assemblage includes physical habitat assessment), metals chemistry, organophosphate chemistry including chlorpyrifos and diazinon, bioaccumulation (shellfish tissue chemistry), ammonia, chlorophyll a, dissolved oxygen, pH, depth, temperature, depth, conductivity, turbidity, boron, TDS, nutrients including phosphate, nitrate, chloride, sulfate, and nitrite, and Phase II TIE work because toxicity is a known problem as documented by previous Regional Board studies performed by UC Davis.

Dominguez Channel and Los Angeles and Long Beach Harbors

The following indicators will be used: bacteria, bioassessment (macroinvertebrate assemblage), toxicity (sediment and water), sediment chemistry, nutrients, water chemistry, DO, sediment grain size, flow, temperature, and physical habitat assessment.

- Specific Activities Planned for FY 2000-2001 and FY 2001-2002

IV.1.  List of Water Bodies to be Sampled

IV.1.1	FY 2000-2001

Santa Clara River Watershed Phase I

Calleguas Creek Watershed

IV.1.2.	FY 2001-2002

Dominguez Channel

Los Angeles and Long Beach Harbors

Santa Clara River Watershed, Phase II

IV.2.  Review of Available Information (analysis of data is incomplete at this time)

Please see Attachment VI.6 Applicable Sections of the 1996 Water Quality Assessment.  This will be updated as soon as the Water Quality Assessment currently under development is adopted by the State Board.

IV.2.1.	FY 2000-2001

Santa Clara River

The overall status of the Santa Clara watershed is not well known; historical assessment has been spotty.  Most data is limited to nitrogen and minerals.  The Santa Clara River Enhancement and Management Plan Steering Committee is a 26-member group currently directing preparation of an Enhancement and Management Plan.  Have they done any monitoring?

Calleguas Creek

The Calleguas Creek watershed has been studied much more comprehensively utilizing both an EMAP approach and pre-selected sites.  The POTWs in the area have participated in a comprehensive monitoring program, in addition to their separate monitoring programs, that has produced a large database.  Toxicity is a known problem as documented by previous Regional Board studies performed by UC Davis.

IV.2.2.	FY 2001-2002

Dominguez Channel and Los Angeles and Long Beach Harbors

This area has been sampled for sediment chemistry and toxicity during the Southern California Bight projects in 1994 and 1998 as well as in the Bay Protection Program.  However, water column chemistry has been largely overlooked.  Additionally, data does exist from the monitoring program that the refineries in the area partake in.

IV.3.  Specific Sampling Design / Sample Collection

IV.3.1.	FY 2000-2001

Santa Clara River

In this watershed, CDFG will utilize the information provided by USEPA and perform recon to narrow down the site selection from 240 potential sites to 30 stations.  USEPA provided the potential site population following EMAP procedures in randomly selecting from the following:

- The target population is all streams with flowing water within the Santa Clara watershed as defined by the USGS 4th field hydrologic unit 18070102.  The target population is the aquatic resource about which information is wanted – it must be clearly defined to everyone including the field crew whose understanding is critical.

- The sampling frame is RF3 restricted to Strahler 3rd order and higher.  Both perennial and non-perennial coded streams were included to make sure streams with flowing water during time of sampling would not be excluded.  A comparison was done to compare the difference if only perennial streams were chosen.

EMAP uses a Random Tessellation Stratified (RTS) design to spatially balance the sample across the resource which provides improved precision.  This design uses an oversample selection population to address non-target and inaccessible sites.  This random design was chosen in order to allow the Regional Board, in a statistically sound manner, to sample the watershed and develop an assessment of the overall health of the watershed.  The watershed is the geographic region of interest.  Fulfilling an additional goal of SWAMP, this approach theoretically will provide data both in clean and polluted areas, with sites near areas that have been sampled previously and those that have not.  By choosing a random sampling design, bias is removed.  The representative 30 sites is the recommended number of sites to provide a statistically defensible summary of the watershed.

The probabilistic design has a spatial component and can potentially have a time component built in as well.  EMAP experience has shown that the best design has a five-year cycle with five panels chosen.  A subset of the panel conducted in the first year is sampled in year two to tie together the panels and time.  Ten of the thirty random stations will be revisited on a random basis during Fiscal Year 2001/2002 in order to incorporate this temporal element into Region 4’s monitoring program.  The constituents analyzed will be the same as in the first year. See page 8 in June 13 EMAP notes for specifics.

The sampling effort is randomized to make an inference that is statistically defensible.  One of the reasons Region 4 decided to implement a probability based design in the Santa Clara River Watershed is because the area is so large and “A key point in favor of probability based designs is that they allow lower cost sampling programs because a smaller number of sites are able to support conclusions with known accuracy and precision about status and trends of a region.”  The probability survey design  fulfills one of the Regional Board’s objectives which is to determine the status of the watershed and to determine trend analysis over time.

At the same time, staff believes that a station at the bottom of each major subwatershed will provide valuable information about those said subwatersheds and the beginnings of a directed tributary-based monitoring program and will assist in TMDL development.  Funding has been set aside to follow up on “hot spot” stations in subsequent years and subwatersheds that have analyses which indicate problem areas.

Calleguas Creek

The program for this watershed is based on a directed approach as a large data set already exists for this watershed and because the watershed is smaller than the Santa Clara River watershed.  There will be 12 stations strategically placed by Regional Board staff in which the following analyses will be performed: toxicity, bioassessment, metals chemistry, TDS, flow, ammonia, chlorophyll a, temperature, conductivity, turbidity, dissolved oxygen, pH, boron, nutrients including phosphate, nitrate, chloride, nitrite, and sulfate, and organophosphate chemistry including chlorpyrifos and diazinon.  The thirteenth station at the base of the watershed will have the above-mentioned analyses and additionally will be sampled for bioaccumulation.  Funding has also been set aside for Phase II TIE work because toxicity is a known problem as documented in previous Regional Board studies conducted by UC Davis.  Map of station locations to be developed and distributed.

IV.3.2.	FY 2001-2002

Dominguez Channel and Los Angeles and Long Beach Harbors

Regional Board staff have divided the Dominguez Channel and LA/LB harbor area into the following six subareas based on characteristics of the area in order to simplify sampling design: (1) headwater streams, (2) the inner and outer harbors of LA and LB, (3) Madrona Marsh, (4) Lake Machado, (5) the Dominguez Channel estuary, and (6) the upper channelized Dominguez Channel above normal tidal influence.  The sampling design is still under development, however, if funding constraints are not restrictive, the following is a summary of monitoring goals:

- Headwater streams are not expected to have dry weather flow and therefore will not be sampled.

- The inner and outer LA and LB harbors will be sampled for coliform and other pathogens for five consecutive weeks in the summer and again in the winter at the water’s surface.  Water column toxicity and chemistry will be employed at the water’s surface and again at mid-depth.  Chemical analysis will include PAHs.  Funding will be set aside for TIEs because of the known sediment stations.  Staff proposes the use of a probabilistic monitoring design with 30 stations.  These analyses will further characterize the known impairments of the harbor which is very industrial as described above.

- Madrona Marsh is a 40-acre wetland and very little data exists relative to this waterbody.  Staff proposes 3 directed stations where bioassessment, water column toxicity, water column chemistry, sediment toxicity, and sediment chemistry will be employed to gather baseline data to determine the status of the waterbody.  One of the three stations should be at the outlet of the marsh which drains into the Torrance Lateral.

- Lake Machado will be sampled for water column toxicity, water column chemistry, sediment toxicity, sediment chemistry, fish tissue, and pathogens.  Staff proposes 5-10 directed stations and believes the information is necessary because the existing data is very old and very limited.  The lake is posted for swimming and also offers freshwater beaches.  Fish tissue and pathogen monitoring will provide information necessary to protect public health as well as aquatic life.

- Monitoring in the Dominguez Channel Estuary is currently done by the existing refineries as well as LA County.  Therefore, the goal of this monitoring is to supplement the information currently gathered for ambient water quality assessment.  Staff envisions utilizing a total of directed 3-9 stations at the same locations as the current monitoring.  Parameters to be sampled include bioassessment, water chemistry, pathogens, and one sediment toxicity station above the refineries.  A TSM station will also be employed during the summer of 2001.  Staff believe this monitoring will, combined with existing efforts, provide a detailed picture of the overall health of the Dominguez Channel Estuary.  SWAMP will focus on water column chemistry which has historically not been collected, as well as bioassessment, and the study of pathogens within the estuary.

- The upper Dominguez Channel will have one directed station above the tidal influence  which will be sampled for water column chemistry, water column toxicity, pathogens, metals, and organics.  The goal of this station will be to characterize the water quality coming into the estuary and the water quality of the channel above the heavy industry of the refineries.

A list of the specific sites and a map of the sampling locations is unavailable at this time, but will be provided in the near future.

IV.4  Laboratory Analysis

Actual analytical services that will be performed on each sample are shown on the attached table “Services to Be Performed at Each Station/Cost.”

IV.5.  Data Quality Evaluation and Data Reporting

Results from sampling shall be analyzed and reported in tabular and graphical format.  Analyses shall be compared to criteria supplied to CDFG by the Los Angeles Regional Board.  These criteria will consist of water quality criteria and water quality objectives.  Comparative analyses shall be performed in such a way to present the current state of health of the Santa Clara River and Calleguas Watersheds, as well as the Dominguez Channel and LA/LB Harbors.

Toxicity data will include test mean, standard deviation, and a determination of whether or not a sample is toxic at a statistically significant level of difference from the laboratory control samples.

Data from sampling in the Santa Clara River watershed, the Dominguez Channel and the LA/LB Harbors shall be analyzed and interpreted in a method consistent with the EMAP approach and protocol.

The California Stream Bioassessment Protocol developed by CDFG will be followed until a SWAMP-specific bioassessment protocol is established in 2002.

A statewide standardized SWAMP QAPP is being developed by CDFG and will be adopted by all Regional Boards.

IV.6.  Deliverable Products

IV.6.1.	Field Report: A field report will be prepared.  This field report will be provided to the Regional Board, with an additional copy provided to the State Board (one copy to each).  The field report will include a map with sufficient detail of stations sampled, including latitude and longitude coordinates and GPS coordinates.  Digital photographs of each sampling site shall also be included in the report and provided electronically to the Regional Board.

IV.6.2.	Final Data Report:  All data shall be reported in electronic file (Excel spreadsheet or Access database) on three 3.5" IBM-formatted diskettes, CDs, or zip discs, as well as on hard copy (three one-sided originals for copying, and three bound copies).  One of each type--electronic file, one-sided hardcopy original, and bound hardcopy-- shall go to the State Board and the Regional Board and DFG.  QA/QC evaluation reports and verification that data met QA criteria set forth in QA Project Plan must be provided with hardcopy data report.

The data report will include the following items, where applicable, but shall not necessarily be limited to the following items:

All station data including CDFG station name, station number, IDORG number, leg number, sample collection date, sample station longitude and latitude, sample GPS coordinates, sample station water depth, sample location characteristics, toxicity test endpoint mean and standard deviation, and all detection limits.   In addition to the above data, the following will also be reported for all stations indicated on the attached “Services to be performed at each station/cost” spreadsheet for bioassessment: raw data and computed biological indices.  Additionally, data from the bioaccumulation tests will be reported as tissue chemistry data for the specific chemical constituents shown on the attached “Services to be performed at each station/cost” spreadsheet.  A map (and GIS shape file) should be included showing the locations of each sampling station and an indication of the overall integrity of that site as excellent, good, marginal, or poor.  Another map (and GIS shape file) should further indicate the integrity of each site for biological, chemical, and toxicity data results expressed as a triad for each site.

QA/QC evaluation ranking by each analytical laboratory will be provided in the database.  In addition, appendices will include replicate data for toxicity tests, a database description and file structure description.  A QA/QC report will also be included in the final data report, containing an evaluation of how the data complied with actual QA/QC parameters.

IV.7.  Desired Milestone Schedule (significant dates for sample collection &

reporting)

The index period is when sampling occurs; in ecoregions where streams are perennial, sampling can occur in the spring or fall, each choice having pros and cons.  At this time, sample collection for FY 2000-2001 is scheduled to be conducted during the fall of 2001.  It has not been decided if following years will be sampled during a spring index period or a fall index period.  Time constraints and funding issues will impact these decisions in addition to sound science.

IV.8.  Desired “Sample Throughput Schedule”

Need to obtain from Fish and Game

IV.9.  Budget

FY 2000/2001

The maximum cost of all SWAMP services specified for FY 2000-2001, as shown in the attached table “Service to be performed at each station/cost”, shall not exceed $350,000.  This amount of $350,000 is from the Region 4 allocation for FY 2000-2001.

FY 2001/2002

The maximum cost of all SWAMP services specified for FY 2001-2002 shall not exceed $336,526.  This amount of $336,526 is from the Region 4 allocation for FY 2001-2002.  A table “Service to be performed at each station/cost” is under development and will be submitted as soon as it becomes available.

- Working Relationships

| Task |  | Responsible Organization |  |
| --- | --- | --- | --- |
|  | SWRCB | RWQCB | CDFG |
| Develop contract(s) for monitoring services |  |  |  |
| Identify water bodies or sites of concern and clean sites to be monitored |  |  |  |
| Identify site-specific locations with potential beneficial uses impacts or unimpacted conditions that will be monitored |  |  |  |
| Decide if concern is related to objectives focused on location or trends of impacts |  |  |  |
| Select monitoring objective(s) based on potential beneficial use impact(s) or need to identify baseline conditions |  |  |  |
| Identify already-completed monitoring and research efforts focused on potential problem, monitoring objective, or clean conditions |  |  |  |
| Make decision on adequacy of available information |  |  |  |
| Prepare site-specific study design based on monitoring objectives, the assessment of available information, sampling design, and indicators |  (Work Plan Review Role) |  |  |
| Implement study design (Collect and analyze samples) |  |  |  |
| Track study progress. Review quality assurance information and make assessments on data quality. Adapt study as needed |  (Review Role) |  |  |
| Report data through SWRCB web site |  |  (Coordination Role) |  |
| Prepare written report of data |  |  |  |

- Other Information and Attachments

VI.1.	Statewide Monitoring Efforts

The Toxic Substances Monitoring Program is a statewide program.  Each year CDFG in conjunction with Regional Board staff select sites and collect fish from these sites to analyze toxic substances that are being bioaccumulated by these species and stored in their muscle tissue.  The following sites were sampled during the summer of 2001:

- Ventura River

  - upstream of the Ojai Valley Sanitary District’s Wastewater Treatment Plant (near Foster Park)

  - downstream of the Ojai Valley Sanitary District’s Wastewater Treatment Plant

- Calleguas Creek

  - downstream of the Simi Valley Treatment Plant  (403.67.04) (Arroyo Simi)

  - downstream of the Hill Canyon Treatment Plant (403.64.04) (Arroyo Conejo)

  - at Lewis Road (403.12.06)

- Dominguez Channel

  - in the tidal prism*

- Santa Clara River Watershed

  - on Piru Creek just upstream of the confluence with the Santa Clara*

  - on Sespe Creek just upstream of the confluence with the Santa Clara*

  - on Santa Paula Creek upstream of the confluence with the Santa Clara at Steckel Park (403.21.10)*

  - on Castaic Creek upstream of the confluence with the Santa Clara below Castaic Lagoon (no fish were present)*

  - on Bouquet Creek just upstream of the confluence with the Santa Clara (no fish were observed)*

  - On San Francisquito Creek just upstream of the confluence with the Santa Clara (no water was present)*

  - downstream of the Santa Paula Treatment Plant

  - in the tidal prism (estuary at the Harbor Bridge) *

* These sites are being sampled in conjunction with the efforts being undertaken for SWAMP activities in FY 2000/01 and FY 2001/02.

It is the vision of the Los Angeles Regional Board that eventually, each of the SWRCB and RWQCBs existing monitoring programs (e.g., the State Mussel Watch Program, Toxics Substances Monitoring Program, toxicity studies, and fish/shellfish contamination studies) will be incorporated into the Regional SWAMP efforts to ensure a coordinated approach without duplication.  What about Bay Protection and Cleanup Program?

VI.2.	Statewide Effort and Identification of Reference Sites in California

Staff at the Los Angeles Regional Board also believes strongly in the necessity of a statewide effort to begin the identification of potential reference sites for a biomonitoring program.  Staff from Region 4 are members of and participate in the subcommittee on bioassessment which met in Tahoe in May of 2001 to establish preliminary goals and a guidance document which will be implemented in a pilot project in the Sacramento Region.  Reference sites are crucial in order to realize the potential health and community structure of a stream and to have a reference condition to compare other streams to.  Categories or levels of impairment can be discerned.  Lastly, it is imperative that information is gathered on what a truly healthy benthic biological community structure consists of so that we may have a goal to strive for.

VI.3	Funding for Equipment and Additional Studies

As described in detail in the Task Order for FY 2000/01, the Los Angeles Regional Board has opted to set aside funding for both the purchase of equipment for the region’s laboratory and to allow for follow-up studies to be conducted at study sites that are identified as impaired sites needing further analysis.  Equipment to be ordered for the Los Angeles Regional Board’s laboratory includes equipment and materials necessary to perform bacterial analysis using IDEXX kits, a multiparameter probe, and an ISCO automatic sampler to allow staff to measure pH, conductivity, Chlorophyll a, and temperature in the field.  The Los Angeles Board is also purchasing a Garmin GPS III Plus hand held unit and National Geographic TOPO software covering the State of California to aid in field studies and graphical interpretation of the data.  Further equipment may be ordered as a need is demonstrated.

State of California State Water Resources Control Board; Proposal for A Comprehensive Ambient Surface Water Quality Monitoring Program; Report to Legislature; November 30, 2000.

State of California Regional Water Quality Control Board, Los Angeles Region; Watershed Management Initiative Chapter; December 2000.

State of California Regional Water Quality Control Board, Los Angeles Region; Watershed Management Initiative Chapter; December 2000.

State of California Regional Water Quality Control Board, Los Angeles Region; Watershed Management Initiative Chapter; December 2000.

United States Environmental Protection Agency, EMAP West Coastal Technology Transfer Workshop; Moss Landing Marine Laboratory, June 13-14, 2001.

United States Environmental Protection Agency; Environmental Monitoring and Assessment Program: West – Research Strategy, February 2001.

United States Environmental Protection Agency, EMAP West Coastal Technology Transfer Workshop; Moss Landing Marine Laboratory, June 13-14, 2001.