Surface Water Ambient Monitoring Program for the Santa Ana Regional Board

for

Fiscal Years 2000-2005

Workplan

- Introduction:

Fiscal year (FY) 2001-02 will mark the first year of the coordinated implementation of the Surface Ambient Monitoring Program (SWAMP).  A description of the site-specific monitoring effort that will be implemented through SWAMP is presented in SectionV of the report to the Legislature titled “Proposal for a Comprehensive Ambient Surface Water Quality Monitoring Program.

The Surface Water Ambient-Monitoring program for the Santa Ana Regional Board for the fiscal years 2000 through 2005 involves sampling in Anaheim Bay, Huntington Harbor, Lake Elsinore, Canyon Lake, and Big Bear Lake.  The sampling activities that will take place in these water bodies will enable Regional Board staff to determine whether the beneficial uses in these water bodies are being attained.  The data will be used to prepare the Santa Ana Region’s 305 (b) report and possibly for updating the region’s 303 (d) list and to determine the need to do focused studies in the future.

# Section 305 (b) of the Clean Water Act requires the State to prepare and submit to EPA a report on the status of the state’s ambient water quality.  Each regional board prepares a regional water quality assessment (WQA), which then becomes a chapter in the statewide 305(b) report.  The WQA identifies a list of the water bodies assessed, the pollutants of concern, and the potential pollutant sources.  Water bodies identified in the 305(b) report as not supporting one or more beneficial uses are considered “impaired” and are then placed on the Clean Water Act Section 303 (d) list of impaired water bodies.  Once included on the 303(d) list, the Clean Water Act requires that total maximum daily loads (TMDL) be developed to address the parameters responsible for impairment.  Regional Boards prioritize the water bodies included in the 303(d) list for development of a TMDL.  Water bodies are prioritized based on the degree of impairment, the number and type of beneficial uses impaired.  The 305(b) report aims to answer the percent of area of a water body that meets the given standard.

A comprehensive monitoring program is needed to determine if the water quality objectives and / or beneficial uses are being met in the receiving water bodies in the Santa Ana Region.  In the past, monitoring programs used to prepare the water quality assessments have used sampling and analytical protocols that did not address large-scale questions of the entire water body.  Some of these questions involve defining the number of acres, or percent of acreage of that water body, that meets a water quality objective (threshold).  An appropriate monitoring program design that defines the percent area meeting a threshold has been used in offshore and other bay/harbor regions of Southern California.  This monitoring design is a stratified-random sampling design with a spatially systematic component.  This design randomly allocates sample sites throughout the water body of interest resulting in an unbiased representation of water quality.  Stratification within the water body enables us to compare one sub-region (sub-population or stratum) to another.  Consequently, the study design was chosen for the assessment of ambient water quality in the Anaheim Bay/Huntington Harbor Complex, Lake Elsinore, Canyon Lake and Big Bear Lake.  The overall goal of the study is to provide the information necessary to adequately assess the ambient water quality in these water bodies and to provide a baseline for future studies.

# The Southern California Coastal Water Research Project (SCCWRP) developed the study design.

Sampling sites were selected randomly within each stratum, rather than by investigator pre-selection, to ensure representative sampling.  The number of sites for each water body exceeds the original 30 sites per strata by 50%.  The reason for the increase is that it may not be possible to sample all of the randomly selected sites because of improper substrate type, depth restrictions, or dredging activities.  To prevent an unacceptable loss of statistical power due to lost samples the number of sites allocated was increased by 50%.  Thirty sites were allocated per stratum to ensure that the 95 % confidence interval is no larger than 15% of the subpopulation area assuming about 20% impairment.

Although sites were selected randomly, a systematic component was added to the selection process to minimize clustering of sample sites.  The systematic element was accomplished by using an extension of the sampling design used in the Southern California Coastal Bight Pilot Project and in EPA’s Environmental Monitoring and Assessment Program (EMAP).  A hexagonal grid was placed over a map of the sampling area.  The hexagonal grid structure ensures systematic separation of the sampling, while the random selection of sites within grid cells ensures an unbiased estimate of ecological condition.

Table 1 includes the priority of the monitoring activities in the Santa Ana Region and the objectives associated with each monitoring study.

The overall goal of these monitoring studies is to determine whether the beneficial uses are being attained at these water bodies.  The general monitoring approach involves applying a stratified-random sampling design with a spatially systematic component to each water body being studied.  This design randomly allocates sample sites throughout the water body of interest resulting in an unbiased representation of water quality.  Stratification within the water body enables us to compare one sub-region (sub-population or stratum) to another.  This design along with consistent sampling and analytical protocols will allow regional board staff to determine whether each water body is attaining the beneficial uses and will also allow for comparison among the different water bodies being studied in the Santa Ana Region.

Water bodies are complex systems.  Sediments interact with the water column.  The interaction includes deposition of heavy metals, organic chemicals, phosphorus and nitrogen into the bottom sediments.  The interaction can also include re-suspension of these materials from the sediment into the water column.  The interaction may take place as a result of changes in pH, dissolved oxygen, temperature, and wave patterns among others.  This interaction may include the potential release of various forms of nitrogen, phosphorus, sollubilized metals, and organic materials such as pesticides into the water column.  Sediments are also habitats for various organisms, which in turn are predated by higher organisms.

As a result of this interaction and to assess the overall water quality of the water bodies in the Santa Ana Region, each water quality assessment study includes sampling and analyses of the sediment and the water column.  In addition, the status of the organisms residing in the sediments will be investigated to gain an overall picture of the interaction between the water column and the sediments and help determine the impact on the sediment dwelling organisms.  Comparison of the sediment chemistry, biology, and toxicity with water column measurements and toxicity consist of the weight of evidence approach to water quality assessment.

Furthermore, the water quality indicators that will be used are specific for each water body type and relates to the specific beneficial use being studied.  For a complete list of water quality indicators please see tables 2 and 3.

The monitoring program for each water body will be implemented during the wet and dry seasons.  Therefore, the sampling periods will include August through the end of September for the dry season and March through the end of April for the wet season.  This takes into account changes in ambient water quality due to summer and winter months.

2.	Specific Activities Planned for FY 2001-02:

Section 1 of this document lists the water bodies that regional board staff plans to study over the next five years.  As stated, the first water bodies studied are Anaheim Bay and Huntington Harbor.  Sampling at these water bodies will be funded with SWAMP funds allocated to the Santa Ana Region for fiscal years 2000-2001 and 2001-2002.

Lake Elsinore is the next water body to be studied.  Sampling in Lake Elsinore will be funded with SWAMP funds allocated to the Santa Ana Region for fiscal year 2001-2002.

Existing information on Anaheim Bay/Huntington Harbor and Lake Elsinore is found below:

2a.	Anaheim Bay/Huntington Harbor:

# The Anaheim Bay/Huntington Harbor Complex is located on the northern edge of the Orange County Coast, approximately 20 miles southeast of Los Angeles.  It receives a multitude of potentially degrading pollutants, such as heavy metals and organic chemicals from a variety of sources.  These sources include boats and boating related activities; storm water/urban runoff; atmospheric deposition; agricultural runoff; and historical inputs.  These sources can result in impairment of the beneficial uses.  Beneficial uses include contact and non-contact recreation; navigation; biological habitat of special significance; wildlife habitat; rare, threatened or endangered species habitat; fish spawning; and marine habitat.

# Anaheim Bay was listed in the 303(d) list for pesticides and heavy metals.  Huntington Harbor was listed in the 303(d) list for pathogens, metals, and pesticides.  The data necessary to determine if the Anaheim Bay/Huntington Harbor Complex should be listed for additional pollutants is very limited and inconclusive.

A map of the study area is included in Appendix A.  This study does not include the Seal Beach Wildlife Refuge, the Bolsa Chica Ecological Preserve or Inner/Outer Bolsa Bay, which are separate water bodies that will be studied at a later time.  Furthermore, assessment of the ambient water quality of the Bolsa Chica Ecological Preserve and the Inner/Outer Bolsa Bay will be addressed for the water quality assessment by examining the data provided by CH2MHILL as a result of the Bolsa Chica Wetlands Restoration Project currently underway.  Additional sampling at the Bolsa Chica Wetlands may be conducted as a separate component of the upcoming Southern California Bight (Bight) 2002 Regional Monitoring Project.

# Study Design and Objectives

The overall goal of the study is to attain a comprehensive and current assessment of the water quality in Anaheim Bay/Huntington Harbor Complex.

The objectives of this monitoring study are:

- Define the extent (percent of area) and magnitude of deviation from thresholds.

- Describe and depict spatial gradients of contaminants

- Determine seasonal relationships (i.e. dry vs. wet seasons)

- Assess the relationship between biological responses and contaminant exposure

- Compare Anaheim Bay with the bays sampled by the Bight ‘98 Monitoring Program

Sampling will take place in August 2001 and March 2002.  These months were chosen to represent ambient water quality during both the wet and dry seasons.  Sampling in March will allow us to determine the ambient water quality in the wet season after storm events have occurred.  The sampling date for March will be chosen so that it does not coincide with a storm event, or immediately after a storm event so that the data will represent a period of time when the indicators are expected to remain stable (ambient water quality).  Sampling in August will allow for direct comparison with Bight ‘98 data.

The two strata selected for the study are Anaheim Bay and Huntington Harbor.  These strata were chosen because it is suspected that the water quality in the bay will be different than the water quality in the harbor.  For example, the bay is subject to more tidal influences, flushing and mixing than the harbor.

This monitoring study will involve sampling 60 sites in the Anaheim Bay/Huntington Harbor Complex (30 sites per stratum).  As stated above, the sampling sites were selected using stratified-random sampling design with a spatially systematic component.

Indicators

Uniform sampling and analytical methods will be conducted throughout Anaheim Bay/Huntington Harbor. Toxicity data will be correlated with chemistry and benthic infauna data for sediments and with chemistry for water column samples.

The Anaheim Bay/Huntington Harbor Water Quality Assessment Study will measure multiple indicators (Table 2) at each site to relate contaminant exposure with biological response, and habitat conditions.  These indicators were selected based on the following:

- the overall objectives of the study,

- the beneficial uses and the water quality objectives listed in the 1995 Basin Plan for the Santa Ana Region,

- the indicators for which a threshold is available, and

- the indicators sampled by the Bight ‘98 monitoring program.

Sampling and Analyses Methods:

Sampling methodology will be consistent with Bight ’98 sampling methodology found in the Field Operations Manual for the Southern California Bight 1998 Regional Marine Monitoring Survey dated July 1998.  Excerpts from the Field Operations Manual that are applicable to this study are available from Regional Board staff upon request.  Furthermore, sampling and analyses methodology will be consistent with the statewide SWAMP sampling and analyses methodology outlined in the statewide QAPP when it becomes available.

- Sediment Chemistry:

Chemical analyses of the sediment samples provide an assessment of chemical deposition. Sediment chemistry will allow us to determine the concentrations of the various chemicals present; which might not be in a bioavailable form.  (If these chemicals are not in bioavailable form then they are of no biological concern).  Sediment samples will be collected from the top 2cm using a 0.1m² modified Van Veen grab sampler. A list of the analyses that will be performed on the sediment samples is on Table 1.    Sediment chemistry samples will be collected and analyzed in August 2001.

- Sediment Toxicity:

Sediment toxicity will allow us to determine if there is a direct impact to the exposure of the chemicals found in the sediment.  Sediment samples for toxicity analyses will be collected from the top 2cm using a 0.1m² modified Van Veen grab sampler.  Amphipod (Eohaustorius estuarius) sediment bioassays will be conducted on the 60 sediment samples collected in August 2001 from Anaheim Bay/Huntington Harbor.  The endpoint of the test is survival of the amphipod after a ten days exposure to the sediment sample.

- Benthic Infauna:

Benthic infauna (organisms that live in the sediment) are an important part of the marine food web.  They generally reside in one location for most of their life and are chronically exposed to sediment contaminants.  Consequently, benthic infauna are excellent indicators of environmental water quality.  Samples for infauna analyses will be taken using a Van Veen grab sampler.  The benthic infauna analyses will consist of sorting and taxonomic identification, to the species level, of the organisms found in the sediment.  The sediment samples will be collected using a 0.1m² modified Van Veen grab sampler and sieved in the field to 1.0 mm.  The benthic infauna samples will be collected in August 2001.

- Water Column Chronic Toxicity:

Water column samples will be collected using a water-column depth integrator sampler.  Kelp (Macrocystis pyrifera) germination and growth, red abalone (Haliotus rufescens) larval development, and purple sea urchin (Strongylocentrotus purpuratus) fertilization bioassays will be performed on undiluted samples collected from Anaheim Bay in March 2001 and in August 2001.

The chronic toxicity analyses will include all required reference toxicant testing on the three species listed above.

The endpoints for these analyses are as follows:

| Species Tested | ###### End point of Analyses |
| --- | --- |
| Kelp | Germination and Growth after 48 hour exposure |
| Abalone Larva | Larval development after 48 hour exposure |
| Purple Sea Urchin | Embryo fertilization after 2 hour exposure. |

- Water Column Field Measurements:

As stated above, the interaction between the water column and the sediments results from changes in physical parameters such as pH, dissolved oxygen, and temperature.  Therefore, it is important to measure these parameters during the sampling activities.  A YSI 6920 multi-parameter probe will be used to measure pH, dissolved oxygen, temperature, pH, salinity, bottom depth, turbidity, total suspended solids, and chlorophyll a.  The multi-parameter probe will be calibrated the same day as the sampling activities.  These measurements will be taken in March and in August 2001.

Responsible Parties

The overall goal of this study is to obtain statistically significant data that is scientifically valid to assess the water quality in the Anaheim Bay/Huntington Harbor complex.  There is also an opportunity for public outreach including:

- educating the public about the water quality impacts to the bay and harbor from anthropogenic activities,

- informing the public of the water quality assessment report, and

- encouraging stewardship of the area.

In order to carry out the goal and objectives of this study, the Regional Board will be working with various agencies and contractors including SCCWRP, ABC Laboratories, CRG Marine Laboratories, and the Coast Keeper to carry out the monitoring program.  The following outlines the roles of each of the participating parties in this monitoring study:

- Santa Ana Regional Water Quality Control Board’s Role:

- Obtain the necessary permits to access the sampling sites and collecting samples

- Collect samples per protocols specified by the Quality Assurance Plan for Bight ‘98 and the laboratories participating in this study

- Ensure that all necessary chain of custody forms are completed prior to surrendering samples to the laboratory

- Obtain the necessary funding to carry out the study

- Coordinate with all parties involved in the study

- Coordinate with SCCWRP for data analyses and payment of analytical services.

- Coast Keeper’s Role:

- Organize community volunteers.

- Assemble informational pollution prevention material for the volunteers

- Coordinate meetings and training sessions with the volunteers

- Provide a registered boat with an A frame and winch

- SCCWRP’s Role:

- Provide the core monitoring design for the study (list of the sampling sites, list of indicators, map of the study area depicting the sampling sites, etc)

- Provide Van Veen grab sampler

- Coordinate with the laboratories for analyses of samples

- Coordinate with the laboratories to obtain the data for analyses

- Analyze the data

- Compile the data and write a report on the conclusions of the study.

- Provide the Regional Board with the database, data analyses and written report

- CRG Marine Laboratory’s Role:

- Provide training to regional board staff and volunteers for collection of samples

- Provide the necessary containers, preservatives, chain of custody forms for the samples.

- Oversee the sample collection.

- Transport the samples to the laboratory for processing.

- Analyze the samples for sediment chemistry, and water column chemistry.

- ABC Laboratory’s Role:

- Provide training to regional board staff and volunteers for collection of samples.

- Provide the necessary containers, preservatives, chain of custody forms for the samples.

- Oversee the sample collection.

- Transport the samples to the laboratory for processing.

- Analyze the samples for water column and sediment toxicity, and benthic infauna.

- Volunteers:

- Record data

- Label the sample bottles

- Assist in sample collection

- State Water Resources Control Board Volunteer Monitoring Coordinator:

- Assist Regional Board staff in training sessions of volunteers and participate in the sampling activities as needed

Equipment:

The boat necessary for the study will be equipped with an A-frame and a winch capable of handling the 0.1m² modified Van Veen grab sampler.  The Coast Keeper will supply the boat.  The Van Veen grab sampler will be used to collect the sediment samples for benthic infauna characterization, sediment chemistry, and sediment toxicity analyses.  The water column samples will be collected using a depth integrator sampler that collects a composite sample of the water column. Field measurements of the water column will be taken by using a YSI 6929 multi-parameter probe.  The probe will be calibrated the same day the measurements will be taken.  Calibration date and measurements will be recorded in a waterproof field log.  While on the boat, the locations will be verified by using a handheld GPS.

#### Chain of Custody or Field Methods

The Regional Board will be responsible for tracking all samples collected during the study.  Chain of custody forms will be used to track each sample from the time it is collected to its final destination in the laboratory.  The field crew will complete a chain of custody form in triplicate for each set of samples to be transferred to the laboratory.  This form will be signed by the crewmember transferring the samples.  Subsequently, the laboratory staff member will sign the chain of custody form.  A copy of the chain of custody form will be kept in the Regional Board Water Quality Assessment 2001 File and the original will accompany the samples.  The third copy will be forwarded to SCCWRP.

Quality Assurance and Quality Control

Quality assurance/quality control (QA/QC) is an important part of any environmental monitoring project.  A carefully planned QA/QC program ensures that the data collected are scientifically valid and adequate to meet the goals of the study.

Quality assurance activities for the study are outlined below:

- Standardization of sample collection, processing, and analytical methods

- Training workshops for field personnel by CRG Laboratories and ABC Labs.

The quality control activities are outlined below:

- CRG Marine Labs and ABC Labs staff will be present during the sample collection activities to ensure that the samples are collected in accordance with Bight ‘98 protocols.

- Sample processing and analyses will follow the Bight ‘98 Quality Assurance Plan.

- In the laboratory, the qc activities will consist of reference matrix samples, spiked samples laboratory duplicates and blanks at a frequency of 10% of all the batch of samples analyzed.  Also, initial calibration verification and continuous calibration verification will be performed.  The continuous calibration verification will be done at a frequency of 10% of the samples analyzed.  Please see attachment C for method detection limits.

Data Management and Data Availability

All data will be inputted into the database developed and designed for SWAMP when it becomes available.  In the mean time, field and laboratory data will be reported to SCCWRP by CRG Marine Laboratories and ABC Laboratories for input into their database in accordance with the Bight ‘98 Information Management Plan.  SCCWRP will store the data in the same database structure that was developed and used in the Bight ‘98 Monitoring Survey.

SCCWRP will forward the formatted data set to the Regional Board upon completion of a report describing the conclusions of the study.  The final data set will be stored in Microsoft Access 97 and in Storet Version 1.1 at the Santa Ana Regional Water Quality Control Board.

All data from this study will be available to the public via the Santa Ana Regional Water Quality Control Board’s website (WWW.SWRCB.CA.GOV/RWQCB8).  The SWRCB will receive a copy of all data and reports generated by this study.

Project Reporting

A water quality assessment report, in draft form, specific to the Anaheim Bay/Huntington Harbor Complex describing the conclusions of the study will be available through the Regional Board’s website for review and comment.  The final report will address any comments received and will be available to the public through the Regional Board’s website.

2b.	Lake Elsinore:

Lake Elsinore is located approximately 60 miles southeast of Los Angeles and approximately 22 miles south of the City of Riverside. The lake is located within the city of Lake Elsinore in Riverside County, and is a natural low point of the San Jacinto River and its drainage basin. The total drainage basin of the San Jacinto River watershed is approximately 782 square miles.  The local tributary area to the lake is approximately 47 square miles.

Lake Elsinore is a shallow lake, its depth ranging from a few feet to approximately 20 feet.  Current monitoring data indicates that the lake is well mixed, except during the brief periods of stratification.  According to the Santa Ana Region’s 1995 Basin Plan the beneficial uses designated for Lake Elsinore are body contact and non-body contact recreation, warm fresh water habitat, and wildlife habitat

Lake Elsinore was included on the 1998 Clean Water Act Section 303(d) list of impaired water bodies for unknown toxicity, low dissolved oxygen, turbidity, and observed algae blooms.  Limited amount of data is available to identify the percent area of Lake Elsinore affected by these stressors or to identify additional stressors.  The sources of these stressors are currently being studied through the TMDL process.   A comprehensive monitoring program is needed to determine the percent area of the lake that meets the water quality objectives and / or beneficial uses.

Existing Data Review:

- Toxicity:

A review of the Toxics Substances Monitoring Program 1978-1987 report and the 1994-1995 report data indicate that fresh water fish exceed the 85% EDL (elevated data level) for arsenic, and dacthal and exceed the maximum tissue residue levels for Total Chlordane, Total DDT, and Total PCB.  A Lake Elsinore Management Agency report dated 1996 by Black and Veatch consultants indicates that aluminum, arsenic, chromium, copper, lead, manganese, nickel, zinc, p’p’ DDD, p’p’ DDE, and total DDT were found in the sediment exceeding sediment criteria.  The threshold effects levels were the criteria most often exceeded.  Threshold effects levels are concentrations below which adverse effects are expected to occur only rarely.  The site-specific objective study conducted by the Regional Board in June 1991 indicates that copper and silver were found in the water column exceeding the California Toxics Rule standards.

- Nutrients and Dissolved Oxygen:

The nutrient concentrations in Lake Elsinore The data generated by the nutrient TMDL monitoring program for the years 2000 and 2001 indicate Lake Elsinore has periods of low dissolved oxygen in the hypolimnion.

# Focus of Work

This study focuses exclusively on determining the percent area of Lake Elsinore that meets the standards or thresholds.  A map of the study area is included in Appendix B.  Tributaries to Lake Elsinore are currently being studied during the source analyses of the TMDL process.

# Study Design and Objectives

The overall goal of the study is to attain a comprehensive and current assessment of the water quality at Lake Elsinore.

The objectives of this monitoring study are:

- Define the extent (percent of area) and magnitude of deviation from thresholds as these relate to beneficial uses and water quality objectives.

- Describe and depict spatial gradients of contaminants

- Determine seasonal relationships (i.e. dry vs. wet seasons)

- Assess the relationship between biological responses and contaminant exposure

- Compare the ambient water quality at Lake Elsinore with the ambient water quality at Canyon Lake and Big Bear Lake (studies to be done in later years).

Sampling will take place in March and August 2001.  These months were chosen to represent ambient water quality during both the wet and dry seasons.  Sampling in March will allow us to determine the ambient water quality in the wet season after storm events have occurred.  The sampling date for March will be chosen so that it does not coincide with a storm event, or immediately after a storm event so that the data will represent a period of time when the indicators are expected to remain stable (ambient water quality).

This monitoring study will involve sampling 30 sites in Lake Elsinore.  Table 3 lists the sampling sites in Lake Elsinore.  As stated above, the sampling sites were selected using stratified-random sampling design with a spatially systematic component.  Thirty sites were allocated to ensure that the 95% confidence interval is no larger than 15% of the sub-population area assuming about 20% impairment.

Sampling sites were selected randomly, rather than by investigator pre-selection, to ensure representative sampling.  The number of sites for Lake Elsinore exceeds the original 30 sites by 50%.  The reason for the increase is that it may not be possible to sample all of the randomly selected sites because of improper substrate type, depth restrictions, or dredging activities.  To prevent an unacceptable loss of statistical power due to lost samples the number of sites allocated was increased by 50%.

Although sites were selected randomly, a systematic component was added to the selection process to minimize clustering of sample sites.  The systematic element was accomplished by using an extension of the sampling design used in the Southern California Coastal Bight Pilot Project and in EPA’s Environmental Monitoring and Assessment Program (EMAP).  A hexagonal grid was placed over a map of the sampling area.  The hexagonal grid structure ensures systematic separation of the sampling, while the random selection of sites within grid cells ensures an unbiased estimate of ecological condition.

Indicators

The Lake Elsinore Water Quality Assessment Study will measure multiple indicators (Table 1) at each site to relate contaminant exposure with biological response, and habitat conditions.  These indicators were selected based on the following:

- the overall objectives of the study,

- the beneficial uses and the water quality objectives listed in the 1995 Basin Plan for the Santa Ana Region,

- the indicators for which a threshold is available, and

- the indicators known or suspected to be exceeded in Lake Elsinore.

It is expected that the benthic infauna identification won’t exhibit a significant difference between the March 2001 and the August 2001 months.  Consequently, the benthic infauna identification work will be performed only in August.  The March 2002 sampling activities will include sediment chemistry, toxicity, water column toxicity, and physical measurements.  The August sampling activities will include water column toxicity, physical measurements, benthic infauna identification, sediment chemistry and sediment toxicity.

Sampling and Analyses Methods

Uniform sampling and analytical methods will be conducted throughout Lake Elsinore. Toxicity data will be correlated with chemistry and benthic infauna data for sediments and with chemistry for water column samples.

- Sediment Chemistry:

Chemical analyses of the sediment samples provide an assessment of chemical deposition. Sediment chemistry will allow us to determine the chemicals that are present and their respective concentrations; which might not be in a bioavailable form.  (If these chemicals are not in bioavailable form then they are of no biological concern).  Sediment samples will be collected from the top 2cm using a 0.1m² modified Van Veen grab sampler. A list of the analyses that will be performed on the sediment samples is on Table 3.  Sediment chemistry samples will be collected and analyzed in August 2001 and March 2002.

- Sediment Toxicity:

Sediment toxicity will allow us to determine if there is a direct impact to the exposure of the chemicals found in the sediment.  Sediment samples for toxicity analyses will be collected from the top 2cm using a 0.1m² modified Van Veen grab sampler. Hyallela azteca and Chironomous tentans sediment bioassays will be conducted on the 30 sediment samples collected in August 2001 and March 2002 from Lake Elsinore.  The endpoint of the test is survival and growth after a 28 day exposure to the sediment sample.

- Benthic Infauna:

Benthic infauna (organisms that live in the sediment) are an important part of the marine food web.  They generally reside in one location for most of their life and are chronically exposed to sediment contaminants.  Consequently, benthic infauna are excellent indicators of environmental water quality.  Samples for infauna analyses will be taken using a Van Veen grab sampler.  The benthic infauna analyses will consist of sorting and taxonomic identification, to the species level, of the organisms found in the sediment.  The sediment samples will be collected using a 0.1m² modified Van Veen grab sampler and sieved in the field to 1.0 mm.  The benthic infauna samples will be collected in August 2001 and March 2002.  In absence of a reference condition for Lake Elsinore, the benthic infauna data will be used to compare one season to another and for comparison with future benthic infauna analyses.

- Water Column Chronic Toxicity:

Water column samples will be collected using a water-column depth integrator sampler.  Ceriodaphnia dubia, Pimephales promelas, and Selenastrum capricornatum survival and reproduction/growth toxicity tests will be performed on undiluted samples collected from Lake Elsinore in August 2001 and in March 2002.  The chronic toxicity analyses will include all required reference toxicant testing on the three species listed above.  The endpoints for these analyses are as follows:

| Species Tested | ###### End point of Analyses |
| --- | --- |
| ## Ceriodaphnia dubia | (71 or 6-8) day survival and reproduction |
| Selenastrum capricornatum | 96 hour growth |
| ## Pimephales promelas | 7 day survival and growth |

- Water Column Field Measurements:

As stated above, the interaction between the water column and the sediment results from changes in physical parameters such as pH, dissolved oxygen, and temperature.  Therefore, it is important to measure these parameters during the sampling activities.  A YSI 6920 multi-parameter probe will be used to measure pH, dissolved oxygen, temperature, pH, salinity, bottom depth, turbidity, total suspended solids, and chlorophyll a.  The multi-parameter probe will be calibrated the same day as the sampling activities.  These measurements will be taken in September  2001 and in March 2002.

Responsible Parties

The overall goal of this study is to obtain statistically significant data that is scientifically valid to assess the water quality in Lake Elsinore.  There is also an opportunity for public outreach including:

- educating the public about the water quality impacts to the bay and harbor from anthropogenic activities,

- informing the public of the water quality assessment report, and

- encouraging stewardship of the area.

In order to carry out the goal and objectives of this study, the Regional Board will be working with various agencies and contractors including SCCWRP, ABC Laboratories, CRG Marine Laboratories, and the Coast Keeper to carry out the monitoring program.  The following outlines the roles of each of the participating parties in this monitoring study:

Participants in this water quality source analyses monitoring program include Regional Water Quality Control Board staff who will collect the samples and conduct the field measurements, City of Lake Elsinore, and Elsinore Valley Municipal Water District staff who will facilitate entrance to the lakes for collection of samples.

- Santa Ana Regional Water Quality Control Board’s Role:

- Obtain the necessary permits to access the sampling sites and collecting samples

- Collect samples per protocols specified by the Quality Assurance Plan for Bight ‘98 and the laboratories participating in this study and ensure that all sampling and analyses methods are consistent with the statewide-SWAMP QAPP when it becomes available.

- Ensure that all necessary chain of custody forms are completed prior to surrendering samples to the laboratory

- Obtain the necessary funding to carry out the study

- Coordinate with all parties involved in the study

- Coordinate with SCCWRP for data analyses and payment of analytical services.

- Coast Keeper’s Role:

- Organize community volunteers.

- Assemble informational pollution prevention material for the volunteers

- Coordinate meetings and training sessions with the volunteers

- City of Lake Elsinore’s Role:

- Provide a registered boat with an A frame and winch

- SCCWRP’s Role:

- Provide the core monitoring design for the study (list of the sampling sites, list of indicators, map of the study area depicting the sampling sites, etc)

- Provide Van Veen grab sampler

- Coordinate with the laboratories for analyses of samples

- Coordinate with the laboratories to obtain the data for analyses

- Analyze the data

- Compile the data and write a report on the conclusions of the study.

- Provide the Regional Board with the database, data analyses and written report

- CRG Marine Laboratory’s Role:

- Provide training to regional board staff and volunteers for collection of samples

- Provide the necessary containers, preservatives, chain of custody forms for the samples.

- Oversee the sample collection.

- Transport the samples to the laboratory for processing.

- Analyze the samples for sediment chemistry, and water column chemistry.

- ABC Laboratory’s Role:

- Provide training to regional board staff and volunteers for collection of samples.

- Provide the necessary containers, preservatives, chain of custody forms for the samples.

- Oversee the sample collection.

- Transport the samples to the laboratory for processing.

- Analyze the samples for water column and sediment toxicity, and benthic infauna.

- Volunteers:

- Record data

- Label the sample bottles

- Assist in sample collection

- State Water Resources Control Board Volunteer Monitoring Coordinator:

- Assist Regional Board staff in training sessions of volunteers and participate in the sampling activities as needed.

Equipment:

The boat necessary for the study will be equipped with an A-frame and a winch capable of handling the 0.1m² modified Van Veen grab sampler.  The City of Lake Elsinore will supply the boat.  The Van Veen grab sampler will be used to collect the sediment samples for benthic infauna characterization, sediment chemistry, and sediment toxicity analyses.  The water column samples will be collected using a depth integrator sampler that collects a composite sample of the water column. Field measurements of the water column will be taken by using a YSI 6929 multi-parameter probe.  The probe will be calibrated the same day the measurements will be taken.  Calibration date and measurements will be recorded in a waterproof field log.  While on the boat, the locations will be verified by using a handheld GPS.

#### Chain of Custody and Field Methods

The Regional Board will be responsible for tracking all samples collected during the study.  Chain of custody forms will be used to track each sample from the time it is collected to its final destination in the laboratory.  The field crew will complete a chain of custody form in triplicate for each set of samples to be transferred to the laboratory.  This form will be signed by the crewmember transferring the samples.  Subsequently, the laboratory staff member will sign the chain of custody form.  A copy of the chain of custody form will be kept in the Regional Board Water Quality Assessment 2001 File and the original will accompany the samples.  The third copy will be forwarded to SCCWRP.

Quality Assurance and Quality Control

Quality assurance/quality control (QA/QC) is an important part of any environmental monitoring project.  A carefully planned QA/QC program ensures that the data collected are scientifically valid and adequate to meet the goals of the study.

Quality assurance activities for the study include but are not limited to:

- Standardization of sample collection, processing, and analytical methods

- Training workshops for field personnel by CRG Laboratories and ABC Labs.

The quality control activities are outlined below:

- CRG Marine Labs and ABC Labs staff will be present during the sample collection activities to ensure that the samples are collected in accordance with Bight ‘98 protocols and in accordance with the statewide SWAMP protocols when they become available.

- Sample processing and analyses will follow the Bight ‘98 Quality Assurance Plan and SWAMP QAPP when it becomes available.

- In the laboratory, the qc activities will consist of reference matrix samples, spiked samples laboratory duplicates and blanks at a frequency of 10% of all the batch of samples analyzed.  Also, initial calibration verification and continuous calibration verification will be performed.  The continuous calibration verification will be done at a frequency of 10% of the samples analyzed.  Please see attachment C for method detection limits.

Data Management and Data Availability

All data will be inputted into the database developed and designed for SWAMP when it becomes available.  In the mean time, field and laboratory data will be reported to SCCWRP by CRG Marine Laboratories and ABC Laboratories for input into their database in accordance with the Bight ‘98 Information Management Plan.  SCCWRP will store the data in the same database structure that was developed and used in the Bight ‘98 Monitoring Survey.

SCCWRP will forward the formatted data set to the Regional Board upon completion of a report describing the conclusions of the study.  The final data set will be stored in Microsoft Access 97 and in Storet Version 1.1 at the Santa Ana Regional Water Quality Control Board.

All data from this study will be available to the public via the Santa Ana Regional Water Quality Control Board’s website (WWW.SWRCB.CA.GOV/RWQCB8).

Project Reporting

A water quality assessment report, in draft form, specific to Anaheim Bay, Huntington Harbor and Lake Elsinore describing the conclusions of the study will be available through the Regional Board’s website for review and comment.  The reports will also be disseminated for peer review.  The final report will address any comments received and will be available to the public through the Regional Board’s website.

Deliverable Products:

The deliverable for this project will include the sample results in electronic format and in hard copy format from the analyses of sediment and water column samples from Anaheim Bay, Huntington Harbor, and Lake Elsinore, measurements of the water column in Anaheim Bay, Huntington Harbor and Lake Elsinore, and a report that will be prepared in conjunction with Regional Board staff for the interpretation of the data.

Sample Collection Dates and Laboratory Reporting:

The study for Anaheim Bay and Huntington Harbor has already started.  The next sample collection dates for Anaheim Bay and Huntington Harbor will be March 2002.  The sampling dates for Lake Elsinore will depend on the timeliness of the approval of the contract for this project.  The desired sample collection dates are October 2001 and April 2002.

Budget:

The overall funds that are available to the Santa Ana Region for the Fiscal Year 2001-2002 are 257,069.  Of this amount, $42,145 have been encumbered for the Anaheim Bay/Huntington Harbor study.  The funds necessary to cover the costs for the Lake Elsinore study is $185,130 for both dry and wet season sampling events.  Please see the approved and executed contract for the costs associated with the Anaheim Bay/Huntington Harbor study, for which funds for the 2001-2002 fiscal year will be used and have already been encumbered.  The remaining funds for the FY 2001-02 are $29,794.  These funds will be used to determine the attainability of Anaheim Bay and Huntington Harbor for body contact and non body contact recreation.

Table 1:  5 Year Monitoring Priorities for the Santa Ana Region

| Fiscal Year: | Water Bodies to be Monitored: | Beneficial Uses: | Monitoring Objective: |
| --- | --- | --- | --- |
| 2000-01 | Anaheim Bay and Huntington Harbor | Anaheim Bay: contact and non-contact recreation; navigation; biological habitat of special significance; wildlife habitat; rare, threatened or endangered species habitat; fish spawning; and marine habitat Huntington Harbor: Navigation; contact and non contact recreation; wildlife habitat; rare, threatened or endangered species habitat; fish spawning; and marine habitat | Are aquatic populations , communities protected? Does the water quality meet the non- body contact beneficial use? Does the water quality meet the body contact beneficial use? |
| 2001-02 | Completion of Anaheim Bay, and Huntington Harbor and beginning of sampling work for Lake Elsinore | Lake Elsinore: body contact and non-body contact recreation, warm fresh water habitat, and wildlife habitat | Does the water quality meet the body contact, non body contact , and habitat beneficial uses? |
| 2002-03 | Completion of Lake Elsinore and beginning of Canyon Lake sampling work | Canyon Lake: Municipal water supply, agricultural water supply, ground water recharge, body contact and non-body contact recreation, warm water habitat, and wild life habitat. | Does the water quality meet the body contact, non body contact , and habitat beneficial uses? Does the water quality meet the municipal water supply beneficial use? |
| 2003-04 | Completion of Canyon Lake and beginning of Big Bear Lake sampling work | Big Bear Lake: Municipal water supply; agricultural water supply; ground water recharge; body contact and non-body contact recreation; warm water habitat; rare, threatened or endangered species habitat; and wild life habitat. | Does the water quality meet the body contact, non body contact , and habitat beneficial uses? Does the water quality meet the municipal water supply beneficial use? |
| 2004-05 | Completion of Big Bear Lake sampling work | N/A | N/A |

Table 2:  List of Water Quality Indicators for the Lake Elsinore Water Quality Assessment Study

| Indicator | Sediment | Water Column |
| --- | --- | --- |
| Benthic infauna taxonomy |  |  |
| Toxicity |  |  |
| Arsenic |  |  |
| Cadmium |  |  |
| Chromium |  |  |
| Copper |  |  |
| Iron |  |  |
| Lead |  |  |
| Mercury |  |  |
| Nickel |  |  |
| Silver |  |  |
| Zinc |  |  |
| Acenaphthene |  |  |
| Acenaphthylene |  |  |
| Anthracene |  |  |
| Benz[a]anthracene |  |  |
| Benzo[a]pyrene |  |  |
| Benzo[b]flouranthene |  |  |
| Benzo[e]pyrene |  |  |
| Benzo[g,h,I]perylene |  |  |
| Benzo[k]flouranthene |  |  |
| Biphenyl |  |  |
| Chrysene |  |  |
| Dibenz[a,h]anthracene |  |  |
| Flouranthene |  |  |
| Flourene |  |  |
| Indeno(1,2,3-c,d)pyrene |  |  |
| Naphthalene |  |  |
| Perylene |  |  |
| Phenanthrene |  |  |
| Pyrene |  |  |
| 2,6-Dimethylnaphthalene |  |  |
| 1-Methylnaphthalene |  |  |
| 2-Methylnapthalene |  |  |
| 1-Methylphananthrene |  |  |
| 1,6,7-Trimethylnaphthalene |  |  |
| LMW PAH’s |  |  |
| HMW PAH/s |  |  |
| Total PAH/s |  |  |
| Chlordane |  |  |
| PCB Congeners |  |  |
| Total DDT |  |  |
| 4,4’-DDT |  |  |
| 2,4’-DDT |  |  |
| DDT |  |  |
| 4,4’-DDD |  |  |
| 2,4’-DDD |  |  |
| DDD |  |  |
| 4,4’-DDE |  |  |
| 2,4’-DDE |  |  |
| DDE |  |  |
| Salinity |  |  |
| Bottom Depth |  |  |
| Light Transmission (turbidity) |  |  |
| Total Suspended Solids |  |  |
| Oxygen Saturation |  |  |
| Bacteria |  |  |
| Visual inspection for sheen (oil & grease) |  |  |
| Total Organic Carbon |  |  |
| Grain size |  |  |
| Acid Volatile Sulfides and simultaneously extracted metals (SEM) |  |  |
| Percent Solids |  |  |
| Dissolved Oxygen (vertical profile) |  |  |
| pH (vertical profile) |  |  |
| Temperature (vertical profile) |  |  |
| Chlorophyll a (vertical profile) |  |  |
| Aluminum |  |  |
| Manganese |  |  |
| p’p’ DDD |  |  |
| p’p’ DDE |  |  |
| p’p’ DDT |  |  |
| dachtal |  |  |
| Total chlordane |  |  |
| Total PCB |  |  |
| Specific conductance (vertical profile) |  |  |

Table 3: List of Water Quality Indicators for the Anaheim Bay/Huntington Harbor Water Quality Assessment Study

| Indicator | Sediment | Water Column |
| --- | --- | --- |
| Benthic infauna taxonomy |  |  |
| Toxicity |  |  |
| Arsenic |  |  |
| Cadmium |  |  |
| Chromium |  |  |
| Copper |  |  |
| Iron |  |  |
| Lead |  |  |
| Mercury |  |  |
| Nickel |  |  |
| Silver |  |  |
| Zinc |  |  |
| Acenaphthene |  |  |
| Acenaphthylene |  |  |
| Anthracene |  |  |
| Benz[a]anthracene |  |  |
| Benzo[a]pyrene |  |  |
| Benzo[b]flouranthene |  |  |
| Benzo[e]pyrene |  |  |
| Benzo[g,h,I]perylene |  |  |
| Benzo[k]flouranthene |  |  |
| Biphenyl |  |  |
| Chrysene |  |  |
| Dibenz[a,h]anthracene |  |  |
| Flouranthene |  |  |
| Flourene |  |  |
| Indeno(1,2,3-c,d)pyrene |  |  |
| Naphthalene |  |  |
| Perylene |  |  |
| Phenanthrene |  |  |
| Pyrene |  |  |
| 2,6-Dimethylnaphthalene |  |  |
| 1-Methylnaphthalene |  |  |
| 2-Methylnapthalene |  |  |
| 1-Methylphananthrene |  |  |
| 1,6,7-Trimethylnaphthalene |  |  |
| LMW PAH’s |  |  |
| HMW PAH/s |  |  |
| Total PAH/s |  |  |
| Chlordane |  |  |
| PCB Congeners |  |  |
| Total DDT |  |  |
| 4,4’-DDT |  |  |
| 2,4’-DDT |  |  |
| DDT |  |  |
| 4,4’-DDD |  |  |
| 2,4’-DDD |  |  |
| DDD |  |  |
| 4,4’-DDE |  |  |
| 2,4’-DDE |  |  |
| DDE |  |  |
| Salinity |  |  |
| Bottom Depth |  |  |
| Light Transmission (turbidity) |  |  |
| Total Suspended Solids |  |  |
| Oxygen Saturation |  |  |
| Bacteria |  |  |
| Visual inspection for sheen (oil & grease) |  |  |
| Total Organic Carbon |  |  |
| Grain size |  |  |
| Acid Volatile Sulfides and simultaneously extracted metals (SEM) |  |  |
| Percent Solids |  |  |
| Dissolved Oxygen (vertical profile) |  |  |
| pH (vertical profile) |  |  |
| Temperature (vertical profile) |  |  |
| Chlorophyll a (vertical profile) |  |  |
| Specific conductance (vertical profile) |  |  |

State of California State Water Resources Control Board; Proposal for a Comprehensive Ambient Surface Water Quality Monitoring Program; Report to Legislature; November 30, 2000.