Recommended Field Equipment

See the list of recommended equipment for field sample collection and field data measurements at the end of this procedures guide.

## Monitoring Site Selection and Reconnaissance

See the file which discusses in detail the monitoring site selection and reconnaissance guidelines recommended for SWAMP, within Appendix D:  filename: “FieldSiteEvaluation_and_Recon_guidelines_1202”

## Field Data Logbook

A Field Data Logbook or a Field Folder is taken into the field on each sampling trip. The use of bound or loose-leaf notebooks is left up to the entity conducting the monitoring.  A good safety precaution against the loss of a bound field data logbook is to photocopy the current pages upon returning from the field. These pages are kept on file at the specific sample collection entity’s office. If a loose-leaf notebook is used, take care to remove original field data log sheets from the notebook and file in the office. Copies of the field data log sheets may be left in the notebook for future reference.  The entry’s discussed below are recorded at each sampling site.

Field Data Logbooks (bound or loose leaf sheets) are maintained on file indefinitely in each regional office or contract laboratory office. They are never discarded, since the logbook may be the only written record of field measurements. Field Data Logbooks are reviewed periodically during SWAMP QA site visits.  The information that must be completed for each individual station where field samples are collected is contained in several SWAMP Field Sheets, as shown in the forms on the following page.  At this point, these data sheets are not inclusive of the information that would be collected for biological assessment work, and several other data measurement types.

### Flow

Sampling crews should be notified on reconnaissance forms if it is known that there is an operational USGS gage is located at or nearby a sampling site.  If there is a USGS gage nearby, a gage height in feet is recorded and later converted to an instantaneous flow value and recorded in the logbook. The gage height is always to be reported to the USGS for conversion to flow. If a USGS gage is not available, a flow measurement should be taken, if requested. See Instantaneous Flow Measurement information towards the end of this document.  In addition, it is recommended that a flow severity value is recorded at each stream or river station that is not tidally influenced. See Flow Severity section towards the end of this document.  Centroid velocity measurements may be taken as a minimum acceptable rough characterization of the stream flow as requested also, although this measurement is not to be recorded as a flow, since it is only a velocity measurement.

### Record of Samples Collected for Purposes of Chemical Analysis

The general types of chemical samples to be collected are listed for each site, since this may vary from site-to-site: such as, metals-in-water, pesticides-in-sediments, routine water quality, etc. The method of preservation for each chemical sample is recorded, as appropriate.

### Record of Samples Collected for Biological Assessment Purposes

This information is described in SOP’s and QAPP’s in Appendix G.

### Record of Data Submission

A Field Data Logbook must indicate in some manner whether data recorded in the logbook has been transcribed onto data forms and submitted to the SWAMP data management staff.

###### Site Photo

Digital photos are taken to help document the actual sampling site. The convention is to take photos facing DOWNSTREAM, overlooking the site. Right bank and left bank are thus defined in this downstream-facing direction. Document any discrepancies from this convention. Only one photo is necessary, if both, left and right bank, fit into one frame. Record all photos in the field data sheet.

## Field Observations

Upon arrival at a sampling site, record visual observations on the appearance of the water and other information related to water quality and water use.

### Left Bank/Right Bank

Left bank is defined as the bank to the left of the observer when facing downstream, and the right bank is to the right of the observer when facing downstream.

### Other Observations

| Water Appearance Sediment Appearance | Color, unusual amount of suspended matter, debris or foam, etc. Color and sediment composition should be noted. |
| --- | --- |
| Weather | Note recent meteorological events that may have impacted water quality; heavy rains, cold front, very dry, very wet, etc. |
| Biological Activity | Note excessive macrophyte, phytoplankton or periphyton growth. The observation of water color and excessive algal growth is very important in explaining high chlorophyll-a values. Other observations such as presence of fish, birds and spawning fish are noted. |
| Unusual Odors | Note if hydrogen sulfide odor, musty odor, sewage odor, etc. This should be recorded for sediment as well. |
| Watershed or Instream Activities | Note instream or drainage basin activities or events that are impacting water quality; bridge construction, shoreline mowing, livestock watering upstream, etc. |
| Record of Pertinent Observations Related to Water Quality and Stream Uses | If the water quality conditions are exceptionally poor, note that standards are not met in the observations, for example, dissolved oxygen is below minimum criteria. Note uses-swimming, wading, boating, fishing, irrigation pumps, navigation, etc. Eventually, for setting water quality standards, the level of use will be based on comments related to the level of fishing and swimming activities observed at a station. |
| Specific Sample Information | Specific comments about the sample itself that may be useful in interpreting the results of the analysis; number of sediment grabs, or type and number of fish in a tissue sample. If the sample was collected for a complaint, or fish kill, make a note of this in the observation section. |
| Missing Parameters Habitat | If a scheduled parameter or group of parameters are not collected, make some note of this in the comments. Describe, if the habitat is a wadeable stream, standing water, etc. |

## Field Measurements

After collecting water samples (see procedures outlined in Appendix D), record appropriate field measurements. When field measurements are made with a multiparameter instrument, it is preferable to place the sonde in the body of water to be sampled and allow it to equilibrate in the D.O. mode while water samples are collected. Field measurements are made at the centroid of flow, if the stream visually appears to be completely mixed from shore to shore. Centroid is defined as the midpoint of that portion of the stream width which contains 50% of the total flow. For routine field measurements, the date, time and depth are reported as a grab.

Recommended Depths for Conducting Field Data Measurements

| Water Depth Less than 5 feet (<1.5m) | If the water depth is less than 5 feet (1.5m), grab samples for water are taken at approximately 0.1m (4 inches), and multi-probe measurements are taken at approximately 0.2m (8 in). This is because all sensors have to be submerged, so 0.1m would not be deep enough. But taking a grab sample at 0.2m is not always feasible, as it is difficult to submerge bottles to that depth, and in many cases the bottle will hit the stream bottom. |
| --- | --- |
| Water Depth Greater than 5 feet (>1.5m) | If the water depth at the sampling point exceeds 5 feet (1.5m) in depth, a vertical profile of dissolved oxygen, temperature, pH and specific conductance are made using the multiparameter probe equipment. The depth of the sonde at the time of measurement is most accurately determined from the depth sensor on the multiparameter sonde rather than depth labels on the cable. |
| Vertical Depth Profiles and Depth-Integrated Sample Collection | If depth integration sampling is being conducted, or if vertical profile measurements are requested, multi-probe measurements are made starting at a depth of 0.2m, and are then conducted at 1.0, 2.0, 3.0, 4.0, and 5.0m depths after that until 5.0m depth is reached. Beginning at 5.0m, measurements are made every 5.0m through depth profile. |

Field data for multiparameter vertical depth profiles are recorded in final form in a Field Data Logbook and submitted to the SWAMP data management staff. See the SWAMP Information System Managment Plan (SWAMP QAMP, Appendix J; 2002) for detailed information on data reporting.

### Water Temperature (OC)

Water temperature data are recorded for each SWAMP visit in final form in a Field Data Logbook and submitted to the SWAMP data management staff. See the SWAMP Information System Management Plan (SWAMP QAMP, Appendix J; 2002) for detailed information on data reporting.

- Temperature Measuring Equipment

	Centigrade Thermometer

	Electronic Temperature Sensor

- Temperature Sampling Procedures

Temperature is measured in-stream at the depth(s) specified in Field Measurements above. Measuring temperature directly from the stream by immersing a multiprobe instrument or thermometer is preferred.

- Hand Held Centigrade Thermometer

If an electronic meter is not available, the temperature is measured with a hand-held, centigrade thermometer (Rawson, 1982).

	In wadeable streams, stand so that a shadow is cast upon the site for temperature measurement.

	Hold the thermometer by its top and immerse it in the water. Position the thermometer so that the scale can be read.

	Allow the thermometer to stabilize for at least one minute, then without removing the thermometer from the water, read the temperature to the nearest 0.1o C and record.

	Do not read temperature with the thermometer out of the water. Temperature readings made with modern digital instruments are accurate to within + 0.1o C.

- Temperature Measurement from a Bucket

When temperature cannot be measured in-stream, it can be measured in a bucket-Nalgene or plastic. Care must be taken to insure a measurement representative of in-stream conditions.

The following conditions must be met when measuring temperature from a bucket:

	The bucket must be large enough to allow full immersion of the probe or thermometer.

	The bucket must be brought to the same temperature as the water before it is filled.

	The probe must be placed in the bucket immediately, before the temperature changes.

	The bucket must be shaded from direct sunlight and strong breezes prior to and during temperature measurement.

	The probe is allowed to equilibrate for at least one minute before temperature is recorded.

	After these measurements are made, this water is discarded and another sample is drawn for water samples which are sent to the laboratory.

pH (standard units)

pH data is recorded for each SWAMP visit in final form in a Field Data Logbook and submitted to the SWAMP data management staff. See the SWAMP Information System Management Plan (SWAMP QAMP, Appendix J; 2002) for detailed information on data reporting.

- pH Sampling Equipment

The pH meter should be calibrated according to the Recommended Procedures for Calibration and Maintenance of SWAMP Field Equipment, also found in Appendix D of the QAMP.

- pH Sampling Procedure

Calibrate the pH sensor. The pH function is calibrated each day of use for multiparameter instruments.

- In-stream Method

Preferably, pH is measured directly in-stream at the depth(s) specified earlier in this document. Allow the pH probe to equilibrate for at least one minute before pH is recorded to the nearest 0.1 pH unit.

- pH Measurement from a Bucket

When pH cannot be measured in-stream, it can be measured in a bucket-Nalgene or plastic, following precautions outlined under Temperature Measurement from a Bucket above.

Potential Problems

	If the pH meter value does not stabilize in several minutes, out gassing of 			carbon dioxide or hydrogen sulfide, or the settling of charged clay particles 			may be occurring (Rawson, 1982).

	If out gassing is suspected as the cause of meter drift, collect a fresh sample, immerse the pH probe and read pH at one minute.

	If suspended clay particles are the suspected cause of meter drift, allow the sample to settle for 10 minutes, then read the pH in the upper layer of sample without agitating the sample.

	With care, pH measurements can be accurately measured to the nearest 0.1 pH unit.

### Dissolved Oxygen (mg/L)

Dissolved oxygen (D.O.) data is recorded for each SWAMP visit in final form in a Field Data Logbook and submitted to the SWAMP data management staff. See the SWAMP Information System Management Plan (SWAMP QAMP, Appendix J; 2002) for detailed information on data reporting.

- Dissolved Oxygen Sampling Equipment

	Dissolved oxygen meter, calibrated according to the Recommended Procedures for Calibration and Maintenance of SWAMP Field Equipment, also found in Appendix D of the QAMP.

#### Multiprobe Instrument

Calibrate the D.O. sensor on the multiprobe instrument. Preferably, D.O. is measured directly in-stream at the depth(s) specified in Field Measurements section above. The D.O. probe must equilibrate for at least 90 seconds before D.O. is recorded to the nearest 0.1 mg/L. Care must be taken at profile stations to insure that the reading is stable for each depth. Since dissolved oxygen takes the longest to stabilize, record this parameter after temperature, conductivity and pH. If the D.O. probe has an operable, automatic stirrer attached, the D.O. probe does not have to be manually stirred. However, if the probe is not equipped with an automatic stirrer, manual stirring must be provided by raising and lowering the probe at a rate of 1 ft/s (0.3m/sec) without agitating the water surface. If the stream velocity at the sampling point exceeds 1 ft/s, the probe membrane can be pointed upstream into the flow and manual stirring can be avoided (Rawson, 1982).

#### D.O. Measurement from a Bucket

When D.O. cannot be measured in-stream, it can be measured in a bucket-Nalgene or plastic, following precautions outlined in Temperature Measurement from a Bucket on prior pages. During equilibration and reading, water should be moved past the membrane surface at a velocity of one ft/s (0.3m/sec), either by automatic stirrer or manual stirring. If stirred manually in a bucket, the water surface is not agitated (Rawson, 1982).

24-Hour Average D.O. (if requested in special study)

Unattended 24-Hour D.O. Data Collection

Why Collect 24-Hour Data

Dissolved oxygen sampling for standards compliance is targeted to water bodies where low instantaneous D.O. levels indicate partial or nonsupport of designated aquatic life uses. Intensive monitoring is conducted with automated equipment that is preset to record and store field measurements hourly over one 24-hour period. Four or more dissolved oxygen measurements may also be made manually at 4-6 hour intervals over one 24-hour period, as long as one is made near sunrise (0500-0900 hrs) to approximate the daily minimum. However, data collected with automated equipment is preferred.

When to Take Measurements

All 24-hour D.O. monitoring events must be spaced over an index period representing warm-weather seasons of the year (approx March 15-October 15), with between one-half to two-thirds of the measurements occurring during the critical period (July 1-September 30). The critical period of the year is when minimum stream flows, maximum temperatures, and minimum dissolved oxygen concentrations typically occur in area streams. A flow measurement must be taken at the time of deployment. In a perennial stream, a 24-hour data for standards compliance can not be used if the flow is less than the 7Q2. In perennial streams, the D.O. criterion to do not apply for flows under the 7Q2. A period of about one month must separate each 24-hour sampling event. Additional samples may be collected outside the index period to further characterize a water body, but that information is generally not used for assessing standards compliance.

Frequency of Measurements

The measurement interval should be no more than once per 15 minutes and no less than once per hour.

Where to Take Measurements

For purposes of determining standards compliance with the 24-hour average criteria, samples collected near the surface will be considered representative of the mixed surface layer. In deep streams, reservoirs, and tidally influenced water bodies, automated equipment is positioned between 1 foot (from the surface) to one-half the depth of the mixed surface layer. The mixed surface layer is defined on page 2-7. At least ten 24-hour monitoring events (using the 24-hour criteria and/or absolute minimum criteria) at each site within a 5-year period are recommended to provide adequate data for assessment.

When to Collect Other Routine Samples, if doing 24-hour D.O. measurements

Other routine field measurements and water samples should be collect at either the time of deployment, at the reference check, or when the multiprobe recording 24-hour data is retrieved.  When ever possible, flow must be measured at the 24-hour site.

Priority for Scheduling 24-hour Sampling Events

- 303d listed waterbodies

- Waterbodies with Concerns for DO problems (too few samples available for full use   assessment).

- Occurrence of low D.O. concentrations observed during the day

- Waterbodies with trends indicating declining D.O. concentrations

- Waterbodies which would contribute to an Ecoregion data set

Data Reporting for 24-hour D.O. measurements

Dissolved oxygen values recorded over the 24-hour period are summed and divided by the number of measurements to determine the average concentration, which is compared to the 24-hour criterion. The lowest D.O. value from each 24-hour set is compared to the minimum criterion. There will be occasions when a complete 24-hour data set wont be possible. For example, if there are 18 measurements instead of 24, a time weighted diurnal average needs to be calculated. This can be easily done using GW Basic.

Support of assigned aquatic life use is based on 24-hour D.O. average and minimum criteria for each monitoring event. Report the 24-hour average D.O. value, number of measurements over a 24-hour period, and the minimum, and maximum values. Report data as a time composite sample with a beginning and ending date and time, covering the 24-hour period measured.

Specific Conductance (mhos/cm)

Specific conductance should be recorded for each SWAMP visit in final form in a Field Data Logbook or field form and submitted to the SWAMP data management staff. See the SWAMP Information System Management Plan (SWAMP QAMP, Appendix J; 2002) for detailed information on data reporting.

#### Specific Conductance Sampling Equipment

	Conductivity meter, calibrated according to the Recommended Procedures for Calibration and Maintenance of SWAMP Field Equipment, also found in Appendix D of the QAMP.

#### Specific Conductance Sampling Procedure

Preferably, specific conductance is measured directly in-stream at the depth(s) specified in Field Measurements on prior pages. Allow the conductivity probe to equilibrate for at least one minute before specific conductance is recorded to three significant figures (if the value exceeds 100). The primary physical problem in using a specific conductance meter is entrapment of air in the conductivity probe chambers. The presence of air in the probe is indicated by unstable specific conductance values fluctuating up to _100 mhos/cm. The entrainment of air can be minimized by slowly, carefully placing the probe into the water; and when the probe is completely submerged, quickly move it through the water to release any air bubbles.

If specific conductance cannot be measured in-stream, it should be measured in the container used for collection of water samples (a bucket) using the precautions outlined on prior pages, Temperature Measurement from a Bucket.

### Salinity (parts per thousand--ppt, or ‰)

The value for salinity is computed from chloride concentration or specific conductance. The calculation assumes a nearly constant ratio for major ions in an estuary when seawater is diluted by river water. This assumption does not hold for cases where salinity is less than about three parts per thousand. Salinity determinations at such low values are only approximate. In estuarine waters, salinity is a relevant and meaningful parameter. Often the salinity may be low, approaching that of freshwater. Nevertheless, this is useful information.  Determine if a station is estuarine from historical records (i.e., experiences cases where salinity is >2.0 ppt) and always report salinity at this station, regardless of the salinity during periods of high flow.

Preferably, salinity is measured at the depth(s) specified in Field Measurements on prior pages. Salinity data should be recorded for each SWAMP visit in final form in a Field Data Logbook and submitted to the SWAMP data management staff. See the SWAMP Information System Management Plan (SWAMP QAMP, Appendix J; 2002) for detailed information on data reporting.

Values between 2.0 ppt and 1.0 ppt should be reported as <2.0 ppt rather than the actual value and values <1.0 ppt should be reported as <1.0 ppt. The field instruments compute salinity from specific conductance and temperature, and display the value in parts per thousand. Report salinity values above 2.0 ppt to the nearest 0.1 ppt.

### Secchi Disc Transparency (meters)--if requested in special study

Secchi disk transparency should be recorded for each SWAMP visit in final form in a Field Data Logbook and  submitted to the SWAMP data management staff. See the SWAMP Information System Management Plan (SWAMP QAMP, Appendix J; 2002) for detailed information on data reporting.

#### Secchi Disk Sampling Equipment

	Secchi disk, 20 cm in diameter

	Measuring tape

#### Secchi Disk Transparency Sampling Procedures

Preferably, Secchi disk transparency is measured directly in-stream wherever conditions allow. The Secchi disk should be clean, weighted and suspended with chain, wire or Dacron line (the line used to suspend the Secchi disk should not be nylon or cotton; stretching may cause erroneous readings). Another option is to attach the Secchi disk to a metal rod calibrated in metric units.

| Average Turbidity | The Secchi disk should be lowered vertically in a location shielded from direct sunlight. Glare from the water's surface will affect the accuracy of the measurement. Don't wear sunglasses. Slowly lower the disk until it disappears from view. The person viewing the disk should maintain an eye level of less than two meters above the water's surface. Note the depth at which the disk disappears from view. Slowly raise the disk until it becomes visible. Note the depth at which the disk reappears. Compute the mathematical average of the two depths noted and record the average value to two significant figures in the field logbook. The recorded average value is the Secchi disk transparency. |
| --- | --- |
| High Turbidity (Muddy Water) | In streams with very high turbidity, high velocity, and/or poor access, it may be necessary to measure Secchi disk transparency in a bucket. Fill the bucket from the centroid of flow being careful not to disturb the substrate. Follow steps above for measuring the Secchi disk depth within 30 seconds after raising the filled bucket from the water's surface. Or, resuspend the solids by stirring, then quickly make the measurement. Record Secchi disk transparency to two significant figures. |
| Low Turbidity (Clear Water) | Some bodies of water will be so clear and shallow that it will not be possible to lower the Secchi disk until it disappears from view. Measure and record the depth at the deepest point accessible. Report Secchi disk transparency as greater than the deepest depth measured. |

Example (Low Turbidity): South Fork Rocky Creek is a small (<1 ft3/s) clear stream. The stream in the vicinity of the sampling site was less than one meter deep and the bottom was clearly visible everywhere. However, a pool was located in the stream next to a bridge. The maximum depth of the pool was 2.6 meters at which depth the Secchi disk was still visible. Therefore, Secchi disk transparency for South Fork Rocky Creek was recorded as > 2.6 m.

Importance of Secchi Disk Data

Eutrophication, the natural aging process in reservoirs and lakes is accelerated by human activities which add nutrients to lakes, reservoirs and the surrounding watersheds. Section 314 of the Clean Water Act (CWA) of 1987 requires all states to classify lakes and reservoirs according to trophic state. Although chlorophyll a is the most direct measure of algal biomass, other indices and programs utilize Secchi disk depth as the primary factor.

###### Turbidity Measurement with Turbidity Meter

Nephelometric Turbidity can be determined by measuring the amount of scatter when light is passed through a sample using a turbidity meter. The LaMotte 2020 Turbidity meter is a suitable instrument for example.

Meters should be calibrated using a standard close to the expected sample value.

For instructions on how to operate the instruments refer to the manufacturer’s manual. Turbidity measurements can be executed together with water sampling. The turbidity sample has to be representative for the sampled water mass. Make sure that no gas bubbles are trapped in the vial for the reading and that the outside of the vial is wiped completely clean, meaning free of moisture, lint and fingerprints. Take several measurements to assure an accurate reading. Do not record values that vary greatly. If variations are small, record an average. If settling particles are present, record a reading before and one after settling. The meter might have to be recalibrated with a different standard, if the sample water readings are outside of the calibration standard limits.

### Significant Precipitation (days since last)--recommended new parameter

Significant precipitation is defined as any amount that visibly influences water quality. Water quality in small to medium streams and in the headwaters of many reservoirs is influenced by runoff during and immediately after rainfall events. This influence is site specific and poorly studied. As part of a new initiative to understand and regulate the adverse effects of runoff, the SWAMP Program would like to associate recent rains or melted snow with ambient water quality, using a parameter defined as "days since last significant precipitation". Record the number of days, rounded to the nearest whole number, since a rain has occurred that, in the best professional judgment of monitoring personnel, may have influenced water quality. If its raining when the sample is collected, or has rained within the last 24-hours, report a value of <1. If it has been a long time since a significant rain, record this as greater than that particular value, for example >7 days. If confidence about the recent history of precipitation is low, draw a line through the space on the data form.

### Flow Severity--recommended new parameter

Flow severity should be recorded for each SWAMP visit to non-tidally influenced flowing streams and submitted to the SWAMP data management staff. It should be recorded even if flow is not measured on that sampling visit. There are no numerical flow guidelines associated with flow severity. This is an observational measurement that is highly dependent on the knowledge of monitoring personnel. It is a simple but useful piece of information when assessing water quality data. For example, a bacteria value of 10,000 with a flow severity of 1 would represent something entirely different than same value with a flow severity of 5. The six flow severity values are; 1=No Flow, 2= Low Flow, 3 = Normal Flow, 4 = Flood, 5 = High Flow, and 6 = Dry. The following are detailed descriptions of severity values:

| 1 | No Flow When a flow severity of one (1 = no flow) is recorded for a sampling visit, then a flow value of zero ft3/s should also be recorded for that sampling visit. A flow severity of one (1) (no flow) describes situations where the stream has water visible in isolated pools. There should be no obvious shallow subsurface flow in sand or gravel beds between isolated pools. Low flow does not only apply to streams with pools. It also applies to long reaches of bayous and streams that have no detectable flow but may have water from bank to bank. |
| --- | --- |
| 2 | Low Flow When stream flow is considered low a flow severity value of two (2) is recorded for the visit and the corresponding flow measurement is also recorded for that visit. In streams to shallow for a flow measurement but water movement is detected, record a value of < 0.10 cfs. Note: Use a stick or other light object to verified the direction of water movement, i.e., movement is downstream and not the affect of wind. What is low for one stream could be high for another. |
| 3 | Normal Flow When stream flow is considered normal, a flow severity value of three (3) is recorded for the visit and the corresponding flow measurement is also be recorded for that visit. Normal is highly dependent on the stream. Like low flow, what is normal for one could be high or low for another stream. |
| 4 and 5 | Flood and High Flow Flow severity values for high and flood flows have long been established by USEPA and are not sequential. Flood flow is reported as a flow severity of four (4) and high flows are reported as a flow severity of five (5). High flows would be characterized by flows that leave the normal stream channel but stay within the stream banks. Flood flows are those which leave the confines of the normal stream channel and move out on to the flood plain. |
| 6 | Dry When the stream is dry a flow severity value of six (6 = dry) is recorded for the sampling visit. In this case the flow is not reported. This will indicate that the stream is completely dry with no visible pools. |

Flow information for over 200 USGS sites is available on the Internet. The address is http://water.usgs.gov/index.html. This is useful information in determining flow conditions prior to sampling. This information may be included in general observations.

Flow Measurement Method (Reporting)

The method (or instrument) used to measure flow is noted by reporting a method number. The method numbers are:

| 1- Flow Gage Station (USGS/IBWC) | 3- Electric (ex. Marsh-McBirney) |
| --- | --- |
| 2- Mechanical (ex. Pigmy meter) | 4- Weir/Flume |
| 5- Other (orange peel, etc.) |  |

### Flow (ft3/s)

If requested, flow data should be recorded for each monitoring visit to non-tidal, flowing streams. Flow data should be recorded in final form in a Field Data Logbook and submitted to the SWAMP data management staff. See the SWAMP Information System Management Plan (SWAMP QAMP, Appendix J; 2002) for detailed information on data reporting. The following are two exceptions to the flow reporting requirement:

| No Flow/ Pools | If there is no flow at a stream site and accessible, isolated pools remain in the stream bed, collect and report the required field data and laboratory samples from the pools and report instantaneous flow. Under these conditions, flow (ft3/s) should be reported as zero. The reported flow severity value should be one. Pools may represent natural low-flow conditions in some streams and the chemistry of these pools will reveal natural background conditions. |
| --- | --- |
| Dry | If the stream bed holds no water, the sampling visit is finished. Report that the stream was "dry" in the observations and record a value of six (meaning "dry") for flow severity. No value is reported for flow since there is no water. |

#### Flow Measurement

If a flow measurement is required at a site, measure and record flow after recording visual observations. The intent of measuring flow first, is to delay collection of chemical and biological water samples with limited holding times. Care must be taken not to collect water samples in the area disturbed during flow measurement.  There are several acceptable flow measurement methods that can be used.

##### U.S. Geological Survey (USGS) Gaging Station

Some SWAMP Stations are sampled at sites where the USGS maintains flow gaging equipment. On any type of sampling visit to a site that has a USGS flow gage, observe and record the gage height to the nearest hundredth of a foot in the field logbook. Upon return to the office, contact the USGS office responsible for maintaining the gage. USGS personnel can provide the flow value in cubic feet per second (ft3/s) that corresponds to the gage height. Although SWAMP personnel may have a rating curve available to them, shifts associated with changes in the stream bed may occur over time. Always call the USGS to determine the shift. At some sites the shift changes frequently. At others, the relation between stream flow and gage height is almost unchanging. If a gage is no longer maintained by USGS, cross out the recorded gage height and be prepared to measure flow by another method on the return visit to that site.

Several factors may influence the accuracy of the USGS rating curves that are used to convert gage height to flow. If there is any doubt about the accuracy of a USGS gage height reading or flow rating curve, sampling personnel should measure the flow if possible.

Gage height may be indicated at a USGS gage by one of three methods:

| Staff Gage | Staff gages are enameled steel plates (with the appearance of large measuring tapes) bolted to some stable structure. For example, staff gages may be bolted to concrete bridge abutments, pillars, or docks. The staff gage face is white with black lettering and gradations. The gradations shown are feet, tenths of a foot, and 0.02 of a foot. The point at which the water level crosses the staff gage should be recorded to the nearest hundredth of a foot. |
| --- | --- |
| Wire Weight Gage | Wire weight gages are locked, metal boxes with approximate dimensions of 15 inches long x 12 inches tall x 12 inches deep. Wire weight gages are usually affixed to bridge rails near mid-stream. They must be unlocked with a USGS key. The wire weight gages house a weight attached by wire cable to a graduated reel (gradations are tenths and hundredths of feet) with a counter at one end. |
|  | When the reel is released the weight can be gradually lowered until the bottom of the weight contacts the water surface. At the point of contact, the weight causes the water surface to ripple slightly. Maintaining the weight in that position, record the counter value to the nearest whole number and the point indicated by the stylus on the graduated reel to the nearest hundredth of a foot. Determine if the gage is the movable type that can be moved to multiple locations on the bridge. This type is common on braided streams. A correction value is stamped on the bridge near each point that the gage can be attached. Record the corrected value as the gage height in feet. |
| Bubble Gage | Bubble gages are locked in metal sheds that are approximately 4 feet wide x 4 feet deep x 6.5 feet tall. The gage houses are most frequently located on the shore near a bridge but sometimes are attached to bridge pillars near mid-stream or established on the stream bank far from any bridge. The gage house must be unlocked with a USGS key. Bubble gages in gage houses usually indicate the gage height in two or three locations. A counter attached to the manometer system indicates gage height in feet. Some gage houses have stilling wells that can be entered. Often there is a staff gage on the inside wall. |
|  | Most bubble gages are also equipped with digital recorders. Digital recorders consist of two white, coded discs, approximately four inches in diameter with a punch tape overlapping a portion of each disc. The discs are marked with 100 gradations. As the front of the digital recorder is viewed, the stylus at the disc on the left indicates height in feet. The stylus at the disc on the right indicates gage height in hundredths of feet. The gage height from both discs should be added and the number recorded in the field logbook as gage height to the nearest hundredth of a foot. |
|  | Many USGS metal sheds also contain a surface level recorder. This devise can be opened to determine how stable stream flow has been prior to the sampling event. Record observations concerning the flow hydrograph. |

Instantaneous Flow Measurement

Water quality monitoring visits to sites where there are no nearby USGS flow gauges will require water quality monitoring personnel to measure flow, when requested by RWQCB’s.

##### Flow Measurement Equipment

Flow meter

One of the following or an equivalent:

	Marsh-McBirney Electronic meter

	Montedoro-Whitney Electronic meter

	Price Pigmy meter (with timer and beeper)

	Price meter, Type AA (with Columbus weight)

Additional Equipment

	Top-setting wading rod (preferably measured in tenths of feet)(see Figure 1).

	Tape measure (with gradations every tenth of a foot).

##### Flow Measurement Procedure (USGS, 1969)

Select a stream reach with the following characteristics:

	Straight reach with laminar flow (threads of velocity parallel to each other) and bank to bank. These conditions are typically found immediately upstream of riffle areas or places where the stream channel is constricted.

	The site should have an even streambed free of large rocks, weeds, and protruding obstructions that create turbulence. The site should not have dead water areas near the banks, and a minimum amount of turbulence or back eddies.

##### Flat Streambed Profile (cross section)

Stretch the measuring tape across the stream at right angles to the direction of flow. When using an electronic flow meter, the tape does not have to be exactly perpendicular to the bank (direction of flow). When using a propeller or pigmy type meter, however, corrections for deviation from perpendicular must be made.

If necessary and possible, modify the measuring cross section to provide acceptable conditions by building dikes to cut off dead water and shallow flows, remove rocks, weeds, and debris in the reach of stream one or two meters upstream from the measurement cross section. After modifying a streambed, allow the flow to stabilize before starting the flow measurement.

Record the following information on the flow measurement form (see example Flow Measurement Forms at end of this document):

	Station Location and Station ID

	Date

	Time measurement is initiated and ended

	Name of person(s) measuring flow

	Note if measurements are in feet or meters

	Total Stream Width and Width of Each Measurement Section

	For each cross section, record the mid-point, section depth and flow velocity

##### Measuring the Stream Width

Measure and record the stream width between the points where the tape is stretched (waters edge to waters edge).

##### Determining the Number of Flow Cross Sections

Determine the spacing and location of flow measurement sections. Some judgement is required depending on the shape of the stream bed. Measurements must be representative of the velocity within the cross-section. If the stream banks are straight and the depth is nearly constant and the bottom is free of large obstructions, fewer measurements are needed, because the flow is homogeneous over a large section. Flow measurement sections do not have to be equal width. However, flow measurement sections should be of equal width, unless an obstacle or other obstruction prevents an accurate velocity measurement at that point.  No flow measurement section should have greater than 10% of the total flow.

If the stream width is less than 5 feet, use flow sections with a width of 0.5 feet. See Example 1 on page 2-34. If the stream width is greater than 5 feet, the minimum number of flow measurements is 10. The preferred number of flow measurement cross sections is 20-30. See Example 2 at the end of this document: The total stream width is 26 feet with 20 measurements, section widths will be 1.3 feet (26/20 = 1.3).

##### Determining the Mid-Point of the Cross Section

To find the mid-point of a cross section, divide the cross section width in half. Using Example 2 (see forms at end of document);

- The total stream width is 26 feet with 20 cross sections and each cross section width is equal to 1.3 feet.

- Divide 1.3 feet in half and the mid-point of the first section is 0.65 feet. In this example the tape at waters edge is set at zero (0) feet.

- By adding 0.65 to zero the mid-point of the first section is 0.65 feet.

- Each subsequent mid-point is found by adding the section width (1.3 feet) to the previous mid-point. For example; MIDPOINT #1 is 0.65 + 0.0 = 0.65; MIDPOINT #2 is 0.65 + 1.3= 1.95 feet; MIDPOINT #3 is 1.95 + 1.3 = 3.25 feet and ....MIDPOINT # 20 is 24.05 +1.3.

- Place the top setting wading rod at 0.65 feet for the first measurement.

- Using a top setting wading rod, measure the depth at the mid-point of the first flow measurement section and record to the nearest 0.01 feet.

Figure 1. Top-Setting Wading Rod

(Marsh-McBirney)

Adjusting the Sensor Depth at a Cross Section

Adjust the position of the sensor to the correct depth at each mid-point. The purpose of the top setting wading rod is to allow the user to easily set the sensor at 20%, 60% and 80% of the total depth. The total depth can be measured with the depth gage rod. Each single mark represents 0.10 foot, each double mark represents 0.50 foot, and each triple mark represents 1.00 foot (see Figure 2).

| For Depths  2.5 Feet | If the depth is less than 2.5 feet, only one measurement is required at each measurement section. To set the sensor at 60% of the depth, line up the foot scale on the sliding rod with the tenth scale, located on top of the depth gage rod. If, for example, the total depth is 2.7 feet (as shown on Figure 2), then line up the 2 on the foot scale with the 7 on the tenth scale (Marsh-McBirney 1990). |
| --- | --- |
| For Depths  2.5 Feet | If the depth is greater than 2.5 feet, two measurements should be taken at 20% and 80% of the total depth. To set the sensor at 20% of the depth, multiply the total depth by two. For example, if the total depth is 2.7 feet, the rod would be set at 5.4 feet (2.7 x 2). Line up the 5 on the sliding rod with the 4 on the tenth scale. |
| For Depths  2.5 Feet (cont) | To set the sensor at 80% of the depth, divide the total depth by two. For example, the total depth is 2.7 feet the rod would be set at 1.35 feet (2.7/2). Line up the 1 on the sliding rod with the 0.35 on the tenth scale. The average of the two velocity measurements is used in the flow calculation. See page 2-36 for an example of a flow form recording measurements for depths greater than 2.5 feet. |
|  | NOTE: The point where the rod is set for 20 and 80% of the depth will not equal values derived by calculating 20 and 80% of the total depth. |

Measuring Velocity (this has typically been measured at 6/10 of the total depth, for velocity-only measurements)

	Position the meter at the correct depth and place at the mid-point of the flow measurement section. Measure and record the velocity and depth. The wading rod is kept vertical and the flow sensor kept perpendicular to the tape rather than perpendicular to the flow while measuring velocity with an electronic flow meter. When using a propeller or pigmy-type meter, however, the instrument should be perpendicular to the flow.

	Permit the meter to adjust to the current for a few seconds. Measure the velocity for a minimum of 20 seconds with the Marsh-McBirney and Montedoro-Whitney meters. Measure velocity for a minimum of 40 seconds (preferably 2 minutes with the Price and pigmy meters).

	When measuring the flow by wading, stand in the position that least affects the velocity of the water passing the current meter. The person wading stands a minimum of 1.5 feet downstream and off to the side of the flow sensor.

	A flow sensor, equipped with cable and weight may be used to measure flows where the water is too deep to wade. Follow the procedure involving meters attached to wading rods.

	Report flow values less than 10 ft2 /s to two significant figures. Report flow values greater than 10 ft3/s to the nearest whole number, but no more than three significant figures.

	In cases where the flow is low and falling over an obstruction, it may be possible to measure the flow by timing how long it takes to fill a bucket of known volume.

Avoid measuring flow in areas with back eddies. The first choice would be to select a site with no back eddy development. However, this can not be avoided in certain situations. Measure the negative flows in the areas with back eddies. These negative values will be included in the final flow calculation.

##### Calculating Flow

To calculate flow, multiply the width x depth (ft2) to derive the area of the flow measurement section. The area of the section is then multiplied by the velocity (ft/s) to calculate the flow in cubic feet per second (cfs or ft3/sec) for that flow measurement section. When flow is calculated for all of the measurement sections, they are added together for the total stream flow (see Figure 2).

Q=Total Flow (or discharge), W=Width, D=Depth, V=Velocity.

Q = (W1 * D1 * V1) + (W2 *D2* V2) + ...... (Wn*Dn*Vn)

What to Do with Negative Values

Do not treat cross sections with negative flow values as zero. Negative values obtained from areas with back eddies should be subtracted during the summation of the flow for a site.

Flow Estimate (ft3/s)

Flow estimate data may be recorded for a non-tidally influenced stream when it is not possible to measure flows by one of the methods described above. Flow estimates are subjective measures based on field personnel's experience and ability to estimate distances, depths, and velocities. If flow can not be measured at a routine non-tidal station, a new site should be selected where flow can be measured.

Flow Estimate Procedure

	Observe the stream and choose a reach of the stream where it is possible to estimate the stream cross section and velocity.

	Estimate stream width (feet) at that reach and record.

	Estimate average stream depth (feet) at that reach and record.  Estimate stream velocity (ft/s) at that reach and record. A good way to do this is to time the travel of a piece of floating debris. If doing this method from a bridge, measure the width of the bridge. Have one person drop a floating object (something that can be distinguished from other floating material) at the upstream side of the bridge and say start. The person on the downstream side of the bridge will stop the clock when the floating object reaches the downstream side of the bridge. Divide the bridge width by the number of seconds to calculate the velocity. The velocity can be measured at multiple locations along the bridge. These velocities are averaged. If this is done alone, watch for road traffic.

	Multiply stream width (feet) times average stream depth (feet) to determine the cross sectional area (in ft2) which when multiplied by the stream velocity (in ft/s) and a correction constant, gives an estimated flow (ft3/s).

Example: A stream sampler conducted a sampling visit to a stream while the flow meter was being repaired. The sampler looked at the creek downstream from the bridge and saw a good place to estimate flow. The stream width was around 15 feet. It appeared the average depth on this reach was about 0.75 feet. The sampler timed a piece of floating debris as it moved a distance of ten feet in 25 seconds downstream over the reach. An estimated flow with a smooth bottom was calculated using the following formula.

Width x Depth x Velocity x A (correction factor)= estimated flow

15 ft (width) x 0.75 ft (depth) x 2.5 ft/s (velocity) x A =25 ft3/s (cfs)

A is a correction constant: 0.8 for rough bottom and 0.9 for smooth bottom

Estimated flow should be reported to one or two significant figures.

Experienced field personnel are able to estimate flow to within 20% of actual flow for total flows less than 50 ft3/s. The best way to develop this skill is to practice estimating flow before making measurements at all monitoring visits to non-tidally influenced flowing streams and then compare estimated flows with those obtained from USGS gages or from instantaneous flow measurements.

Figure 2. Stream Flow (Discharge) Measurement

Example 1.

Stream Flow (Discharge) Measurement

Small Stream < 5 Feet Wide and 2.5 Feet Deep

Stream:____OAK CREEK_____________________________________Date:__5/29/91_________

Station Description:_____at US Hwy 90A____________________________________

Time Begin:__1545______Time End:__1630_______Meter Type:__Marsh-McBirney_____________

Observers:_____BK/MK_______Stream Width*:____5 ft_____ Section Width:____0.5 ft__________

Observations:______________________________________________________________________________________________________________________________________________________________

| Section Midpoint (ft) | Section Depth (ft) | Observational Depth** Ft | Velocity |  | Area W x D (ft2) | Discharge (Q) V x A (ft3/s) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | At Point (ft/s) | Average (ft/s) |  |  |
| 0.25 | 0.55 |  |  | 0.05 |  | 0.01375 |
| 0.75 | 0.80 |  |  | 0.11 |  | 0.044 |
| 1.25 | 0.85 |  |  | 0.27 |  | 0.42635 |
| 1.75 | 0.90 |  |  | 0.49 |  | 0.2205 |
| 2.25 | 1.10 |  |  | 0.58 |  | 0.275 |
| 2.75 | 1.50 |  |  | 0.72 |  | 0.540 |
| 3.25 | 1.20 |  |  | 0.76 |  | 0.456 |
| 3.75 | 0.90 |  |  | 0.76 |  | 0.342 |
| 4.25 | 0.75 |  |  | 0.44 |  | 0.165 |
| 4.75 | 0.30 |  |  | 0.00 |  | 0.00 |
| m3/s x 35.3 =ft3/s |  |  | Total Discharge (Q) (ft3/s) |  |  | 2.4826 |

Example 2.

Stream Discharge Measurement Example (Larger Stream > 5 Feet and 2.5 Feet Deep)

Stream:____RED RIVER_______________________________________Date:__5/28/91____________

Station Description:_____Post Oak Creek 40 m Below Sherman WWTP Outfall__________________

Time Begin:__1542_____________Time End:__1601_____Meter Type:_Marsh-McBirney_________

Observers:_____CM, EW, DO_______Stream Width*:____26 ft___ Section Width:___1.3 ft_______

Observations:_________________________________________________________________________

| Section Midpoint (ft) | Section Depth (ft) | Observational Depth** ft | Velocity |  | Area W x D (ft2) | Discharge (Q) V x A (ft3/s) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | At Point (ft/s) | Average (ft/s) |  |  |
| 0.65 | 0.55 |  |  | 2.03 | 0.715 | 1.451 |
| 1.95 | 0.40 |  |  | 2.04 | 0.520 | 1.061 |
| 3.25 | 0.42 |  |  | 2.02 | 0.546 | 1.103 |
| 4.55 | 0.38 |  |  | 1.77 | 0.494 | 0.874 |
| 5.25 | 0.40 |  |  | 1.75 | 0.520 | 0.910 |
| 7.15 | 0.42 |  |  | 1.93 | 0.546 | 1.054 |
| 8.45 | 0.40 |  |  | 1.99 | 0.52 | 1.035 |
| 9.75 | 0.37 |  |  | 1.92 | 0.481 | 0.924 |
| 11.05 | 0.37 |  |  | 1.56 | 0.481 | 0.750 |
| 12.35 | 0.43 |  |  | 1.32 | 0.559 | 0.738 |
| 13.65 | 0.40 |  |  | 1.36 | 0.520 | 0.707 |
| 14.95 | 0.42 |  |  | 1.33 | 0.546 | 0.726 |
| 16.25 | 0.40 |  |  | 1.35 | 0.520 | 0.702 |
| 17.55 | 0.45 |  |  | 1.64 | 0.585 | 0.959 |
| 18.85 | 0.48 |  |  | 1.70 | 0.624 | 1.061 |
| 20.15 | 0.48 |  |  | 2.00 | 0.624 | 1.248 |
| 21.45 | 0.50 |  |  | 1.95 | 0.650 | 1.268 |
| 22.75 | 0.40 |  |  | 2.18 | 0.520 | 1.134 |
| 24.05 | 0.48 |  |  | 1.71 | 0.624 | 1.067 |
| 25.35 | 0.50 |  |  | 0.60 | 0.650 | 0.390 |
| m3/s x 35.3 =ft3/s |  |  | Total Discharge (Q) (ft3/s) |  |  | 19.162 |

Example 3.

Stream Flow (Discharge) Measurement (Larger Stream > 5 Feet and >2.5 Feet Deep)

Stream:____ARROYO COLORADO______________________________Date:__6/16/98___________

Station Description:_____Downstream of Harlingen WWTP__________________________________

Time Begin:__1400______Time End:__1445_____Meter Type:___Marsh-McBirney______________

Observers:_____JD, CK________Stream Width*:___47.5 ft___ Section Width:___2.375 ft________

Observations: *Note that the starting point is at 4.7 ft on the measuring tape and not zero.

| Section Midpoint (ft) | Section Depth (ft) | Observational Depth** ft | Velocity |  | Area W x D (ft2) | Discharge (Q) V x A (ft3/s) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | At Point (ft/sec) | Average (ft/sec) |  |  |
| 4.70 | 0.73 |  |  | 0.65 | 1.73 | 1.127 |
| 7.08 | 1.10 |  |  | 1.08 | 2.61 | 2.822 |
| 9.45 | 1.85 |  |  | 0.90 | 4.39 | 3.954 |
| 11.83 | 2.20 |  |  | 1.05 | 5.23 | 5.486 |
| 14.20 | 2.20 |  |  | 1.44 | 5.23 | 7.531 |
| 16.58 | 2.45 |  |  | 1.09 | 5.82 | 6.342 |
| 18.95 | 2.55 | 0.20 | 1.75 | 1.76 | 6.06 | 10.659 |
|  |  | 0.80 | 1.76 |  |  |  |
| 21.33 | 2.60 | 0.20 | 1.79 | 1.56 | 6.18 | 9.633 |
|  |  | 0.80 | 1.32 |  |  |  |
| 23.70 | 2.70 | 0.20 | 1.63 | 1.45 | 6.41 | 9.298 |
|  |  | 0.80 | 1.26 |  |  |  |
| 26.10 | 3.05 | 0.20 | 1.68 | 1.42 | 7.24 | 10.286 |
|  |  | 0.80 | 1.15 |  |  |  |
| 28.48 | 3.10 | 0.20 | 1.23 | 0.96 | 7.36 | 7.068 |
|  |  | 0.80 | 0.69 |  |  |  |
| 30.85 | 2.90 | 0.20 | 1.22 | 1.06 | 6.89 | 7.301 |
|  |  | 0.80 | 0.89 |  |  |  |
| 33.23 | 2.84 | 0.20 | 0.60 | 0.49 | 6.75 | 3.305 |
|  |  | 0.80 | 0.37 |  |  |  |
| 35.60 | 2.65 | 0.20 | 0.80 | 0.51 | 6.29 | 3.210 |
|  |  | 0.80 | 0.21 |  |  |  |
| 37.98 | 2.65 | 0.20 | 0.85 | 0.91 | 6.29 | 5.727 |
|  |  | 0.80 | 0.96 |  |  |  |
| 40.35 | 2.20 |  |  | 0.28 | 5.23 | 1.464 |
| 42.73 | 2.30 |  |  | 0.16 | 5.46 | 0.874 |
| 45.10 | 2.05 |  |  | 0.51 | 4.87 | 2.483 |
| 47.48 | 1.10 |  |  | 0.49 | 2.61 | 1.280 |
| 49.86 | 0.65 |  |  | 0.62 | 1.54 | 0.957 |
| m3/s x 35.3 =ft3/s |  |  | Total Discharge (Q) (ft3/s) |  |  | 100.8 |

Stream Flow (Discharge) Measurement Form

Stream:___________________________________________________________Date:_______________

Station Description:____________________________________________________________________

Time Begin:___________	Time End:_____________	Meter Type:__________________________

Observers:____________________	Stream Width*:______________	Section Width:___________

Observations:_________________________________________________________________________

| Section Midpoint (ft) (m) | Section Depth (ft) (m) (cm) | Observational Depth** ft-m-cm | Velocity |  | Area W x D (ft2) (m2) | Flow (Q) V x A (m3/s) (ft3/s) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | At Point (ft/s) (m/s) | Average (ft/s)(m/s) |  |  |
| m3/s x 35.3 =ft3/s |  |  | Total Flow (Discharge) (Q) (ft3/s) |  |  |  |

* 	Make a minimum of 10 measurements when the total width is > 5.0 ft, 20 measurements preferred.

**	When water is < 2.5 ft deep take one measurement at each cross section. When water is > 2.5 ft deep, take two measurements at each cross section; one at   the total depth and the other at 2 x the total depth. Average the two velocity measurements. See SWAMP Procedures Manual for a detailed flow measurement method.

Summary of Significant Figures for Reporting Field Parameters

| Parameter | Field Data Reporting Requirements |
| --- | --- |
| Water Temperature (oC) | Report temperature to the nearest tenth of a degree. Round insignificant figures 0 through 4 down and 5 thru 9 up. |
| pH (s.u.) | Report pH to the nearest tenth of a pH standard unit. |
| D.O. (mg/L) | Report dissolved oxygen to the nearest tenth of a mg/L. |
| Specific Conductance (mhos/cm) | Report specific conductance to only three significant figures if the value exceeds 100. Do not report ORP which is displayed by some multiprobes. |
| Salinity (ppt) | Report salinity values above 2.0 ppt to the nearest tenth of a part per thousand. In estuarine waters report the actual values displayed by the multiprobe above 2.0 ppt and values less than 2.0 as <2.0 or <1.0 only. Determine if a station is estuarine (i.e., experiences cases where salinity is >2.0 ppt) and always report salinity at this station, regardless of the salinity during periods of high flow. |
| Secchi Disk (meters) | Report Secchi depth transparency in meters to two significant figures. |
| Days Since Last Significant Precipitation(days) | Report whole numbers. If it is raining when the sample is collected or has rained within the last 24 hours, report a value of <1. If it has been over a week since a rainfall event, report a value of > 7. |
| Flow (ft3/s) | Report instantaneous flow values less than ten ft3/s to two significant figures. Report flow values greater than 10 ft3/s to the nearest whole number, but no more than three significant figures. When there is no flow (pools), report as 0.0. When there is no water, don't report a value, but report as "dry" in the observations. |
| Flow Severity (1-no flow, 2-low, 3-normal, 4-flood, 5-high, 6-dry) | When there is no flow (pools), report the severity as 1, and the instantaneous flow as 0.0 ft3/s. If the stream is dry, record only flow severity, as a value of 6. |