# 1.0 OBJECTIVE

In laboratory tests designed to determine the toxicity of low salinity water samples, fathead minnow juveniles are exposed to test solutions for 7 days, after which the survival and growth are determined in each toxicant concentration.  Observed effects may be related to the presence of contaminants or to naturally occurring factors.  In order to correctly interpret toxicity results, concentrations of chemical contaminants should be analyzed, as well as other water quality parameters, such as pH, dissolved oxygen, total dissolved solids, hardness, salinity, temperature, ammonia and conductivity.

In this procedure, water samples collected from field stations are divided into replicate beakers in the laboratory.  Ten fathead minnow juveniles are placed into each replicate container and monitored for mortality and growth.  After a 7-day exposure, daily survival and final growth are used to give an estimate of sample toxicity.  Because the test measures effects on an early life-stage of an ecologically important species possessing relatively stringent water quality requirements, the results constitute a good basis for decisions concerning either hazard evaluation or the suitability of estuarine waters for aquatic life (EPA 1994).

# 2.0 EQUIPMENT

The following equipment is necessary to conduct the toxicity test at the Marine Pollution Studies Laboratory at Granite Canyon (MPSL).  The word "clean" here and throughout this procedure means that the item has been cleaned according to the MPSL glassware cleaning procedures outlined in a separate standard operating procedure (SOP).

## 2.1 Culture

• Airstones and clean air system

• 4:1 water prepared from Perrier or Evian and distilled water (25 ± 1C.)

• Newly hatched Artemia for feeding

• Disposable plastic pipettes (for handling fish)

## 2.2 Test Initiation

• 500-ml beakers (4 per sample concentration)

• 1000-mL volumetric flasks (2) and pipettes for reference toxicant dilutions

• Water bath or environmental chamber

• Data sheets

• Gloves and appropriate safety gear (see MPSL lab safety manual)

• Sample vials for reference toxicant analysis (new polyethylene 30 ml, acid washed)

• Graduated pipettes: 1- and 10-ml

• Analytical balance

• Plastic squirt bottles

## 2.3 Water Quality

• Meters and probes for measuring pH, dissolved oxygen, hardness, alkalinity, ammonia, and conductivity

• Thermometers (glass mercury thermometer and continuously recording thermometer)

• Graduated pipettes (10-ml) and hand pipette pump for water quality sampling

• Gloves and appropriate safety gear (see MPSL lab safety manual)

# 2.4 Dilution Water

In every step of this procedure, use Granite Canyon Nanopure water mixed with Evian in a 4:1 ratio.

# 3.0 EXPERIMENTAL DESIGN

Aquatic toxicity tests can be used as screening tools or as part of more comprehensive studies to assess water quality.  Careful consideration must be given to site characteristics, reference site selection, field replication, choice of synoptic measures, seasonal factors, and comprehensive planning and peer review to determine that study designs are adequate to meet program objectives.

This laboratory toxicity test consists of ten replicate test cups for each sample concentration.   The quality of test animals and testing conditions is determined through concurrent testing of reference toxicants (positive controls) and control water (negative controls).  Testing of reference sites or receiving water is recommended to demonstrate the suitability of test sites in the absence of toxic contaminant concentrations.  Test conditions of temperature and photoperiod are controlled as indicated below, and pH, NH3, conductivity, and dissolved oxygen are measured at the beginning and end of the exposure.  Temperature is measured continuously, and hardness is measured at the beginning of the test.

# 4.0 Preparation of Samples for Testing

Because of the short holding time, tests will generally be initiated on the same day as sample receipt.  Place appropriate sample volume in the constant temperature room.   Allow particulates to settle out and oxygen concentrations to equilibrate below super-saturated levels.

# 5.0 CONTROLS

## 5.1 Dilution Controls

The dilution control should consist of 4:1 culture water.

## 5.2 Reference Toxicant Tests

A reference toxicant test must be conducted concurrently with every test to indicate the sensitivity of the organisms and the suitability of the test methodology.  Reagent grade copper chloride (CuCl2) should be used as the reference toxicant for fathead minnow tests, unless another toxicant is specified by the Regional Water Quality Control Board or other appropriate regulatory agency.  Prepare a 10,000 g/liter CuCl2 stock solution by adding 0.0268g reagent grade CuCl2 to one liter of distilled water in a volumetric flask.  Cap tightly and mix thoroughly.  Sample the reference toxicant stock solution at the beginning of the test for chemical verification of the copper concentration.  Acidify samples for analysis in clean sample vials with 1% by volume 14N-reagent grade nitric acid.

Reference toxicant solutions should be two to five replicates of 0 (control) 18, 32, 56, 100, and 180 g/liter.  Other concentrations may be added between these if greater precision is desired for quality control chart purposes.  Prepare concentrations according to dilution schedule.  Start with the control solutions and progress to the highest concentration to minimize contamination.

All tests (sample and reference toxicant) must use juveniles from the same culture.  They must be handled in the same way and delivered to the test containers at the same time.

# 6.0 TEST initiation

Fathead minnow juveniles are randomly placed in all test containers until each replicate contains ten fish.

# 7.0 daily test MONITORING and renewal

## 7.1 Measuring Water Quality in Test Containers

Measure temperature, dissolved oxygen, pH, ammonia, alkalinity, hardness, and conductivity in each sample at the beginning and end of the test.  Sample the initial test solutions at the time of dilution preparation.  Water quality should also be measured on old and new dilutions at the time of renewal.  Renewal water quality parameters include dissolved oxygen, pH, conductivity, and temperature.

## 7.2 Feeding

Feed each replicate twice per day.  Rinse newly hatched Artemia nauplii with fresh water to removes salts.  One mL of rinsed and concentrated nauplii is approximately equivalent to one gram.  Pipette 5 mL of concentrated nauplii into a small flask or beaker and add 10 mL control water.  Feed each replicate 3 drops (0.15g) in the morning, and at least 6 hours later.  On weekend days, replicates should be fed at the beginning and end of the work shift.  If survival rate in any test container falls below 50%, reduce feeding to one to two drops.

## 7.3 Counting and Renewal

Count the number of surviving fish daily.  As counts are being made, use a disposable transfer pipette to remove dead Artemia and dead fish.  After morning feeding and count, renew the test solutions.  Using a siphon (screened glass tube attached to plastic tubing), remove 80% of the test solution.  Take care not to damage fish by sucking them onto the siphon screen.

# 8.0 TERMINATING THE TOXICITY TEST

After 7 days of exposure final survival counts are made.  Fish are rinsed onto a screen, rinsed with distilled water, and transferred to pre-tared weighing pans for growth measurements.  Dry the fish overnight in an oven set to 60C.  After 24 hours place weigh pans in a dessicator to cool.  Measure final weight to the nearest 0.01 mg.  Final water quality must be sampled at the termination of the test.  Take the completed data sheet to the office for data entry and analysis.  Notify the data analyst that the data has arrived.  Make sure the data sheets are placed in the proper location and that the person keeping track of the data knows where it is.

# 9.0 DATA handling and Test Acceptability

Immediately after test termination, check the data sheet to determine whether dilution water and conductivity controls have acceptable survival (mean of > 80%).  If not, notify the project officer without delay.

This toxicity test procedure is considered acceptable if fathead minnow survival in controls is greater than or equal to 80%.  Average dry weight of surviving fish in controls must exceed 0.25mg.  Tests with temperature, salinity, or dissolved oxygen measurements outside the specified ranges, may be considered conditionally acceptable based on the project officer's best professional judgment.  Acceptable temperatures range from 25 ± 1C; acceptable dissolved oxygen concentration is 60-100% saturation.

# 10.0 REFERENCES

U.S. Environmental Protection Agency.  1994.  Short-term methods for estimating the chromic toxicity of effluents and receiving water to freshwater organisms.  EPA-600-4-91-002.  Office of Research and Development.  Washington, DC.

# 11.0 TEST SUMMARY

Species:			Pimephales promelas

Test Duration:			7 days

Renewals:			Daily

Organism Source		Aquatic Biosystems

Age of test organisms:		<24 hours

Test Temperature:		25 ± 1C

Light intensity:			Ambient laboratory illumination 10-20 µE/µ2/s

Photoperiod:			16 hour Light: 8 hour Dark; Replication:	5 replicates

Test Containers:		500-mL beakers

Test solution volume:		250-mL minimum

Loading:			10 fish per beaker

Feeding:	Newly hatched, rinsed Artemia

Water Quality:	pH, D.O. temp, conductivity, alkalinity, hardness, ammonia

Reference Toxicant:		Copper Chloride (CuCl2)

Stock Solution: 			0.0268 g in 1 liter of distilled water (= 10,000 mg/L).

Dilutions: 				0, 18, 32, 56, 100, 180 mg/L

Daily Monitoring:		Survival and Growth

Safety:		Wear protective clothing; read applicable MSDS, be familiar with the lab safety manual prior to testing.

Quality Control:	Fill out all data sheets completely. Be familiar with QA Project Plan prior to testing.

Acceptability Criteria:		Dilution Controls: >80%

Average dry weight of control fish >0.25mg

Temperature range: 		24° to 26°C.