# Surface Water Ambient Monitoring Program

# Site-Specific Monitoring Workplan

# Central Coast Regional Water Quality Control Board

## Introduction

Fiscal Year (FY) 2001-02 will mark the first year of the coordinated implementation of the Surface Water Ambient Monitoring Program (SWAMP).  The “site-specific” monitoring efforts described as one component of SWAMP is presented in Section VI of the report to the Legislature titled “Proposal for a comprehensive ambient surface water quality monitoring program.”  This workplan is intended to address that component of the SWAMP program. A description of the monitoring efforts that will be implemented through the Central Coast Ambient Monitoring Program (CCAMP) is described in this document.

State Board guidance intended that this portion of SWAMP be targeted at specific locations in each region but provides the RWQCBs significant flexibility in this site selection effort. The RWQCBs at their discretion may perform monitoring at clean sites to determine baseline conditions or in areas suspected or known to be polluted.  Because CCAMP has already been underway for three years and already includes both ambient and focused monitoring components, study design will follow that of previous years, where long-term “ambient” monitoring sites are selected at major tributary inputs and at the mainstem upstream of each tributary input, and “focused” monitoring sites are placed at other locations of interest in the watershed (such as above and below specific land uses, point sources, best management practices, or other areas in need of characterization).  Ambient site selection is not based on suppositions regarding water quality, but rather on hydrogeomorphology.  Focused monitoring sites are selected using a number of different criteria, including land use patterns, known problem areas, etc.

## Identify Sites to Monitor

Locations to be monitored are shown for each of five Watershed Rotation Years and for ongoing Coastal Confluences monitoring.  All sites (except offshore mussel watch sites) are monitored for conventional water quality.  A subset of these sites is monitored for Rapid Bioassessment, sediment quality, toxicity, and tissue bioaccumulation.  A preliminary site list for watershed and coastal confluence monitoring is shown in Table 1.

#### Table 1.  Central Coast Ambient Monitoring Program Site List

| Rotation Year | Hydrologic Subarea | Water Body | SiteTag | SiteName |
| --- | --- | --- | --- | --- |
| 2002 | 31012 | Arroyo de la Cruz Creek | 310ADU | Arroyo de la Cruz @ Burnett Creek Rd. |
| 2002 | 31031 | Arroyo Grande Creek | 310AGB | Arroyo Grande @ Biddle Park |
| 2002 | 31031 | Arroyo Grande Creek | 310AGC | Arroyo Grande Ck @ Hi Mntn. |
| 2002 | 31031 | Arroyo Grande Creek | 310AGH | Arroyo Grande @ Huasna Rd. |
| 2002 | 31031 | Arroyo Grande Creek | 310AGV | Arroyo Grande @ Bridge St |
| 2002 | 31024 | Avila | 310438 | Avila |
| 2002 | 30800 | Big Sur River | 308BSU | Big Sur @ Pfeiffer |
| 2002 | 30800 | Carmel River | 308CMD | Carmel River @ Robinson Cyn. |
| 2002 | 30800 | Carmel River | 308CMN | Carmel River at Nason Rd |
| 2002 | 30800 | Carmel River | 308CMU | Carmel River @ Esquiline Rd. |
| 2002 | 31016 | Cayucos Creek | 310CAY | Cayucos Ck @ Cayucos Ck Rd |
| 2002 | 31025 | Coon Creek | 310COO | Coon Creek |

| Rotation Year | Hydrologic Subarea | Water Body | SiteTag | SiteName |
| --- | --- | --- | --- | --- |
| 2002 | 30800 | Garapata Creek | 308GAR | Garapata Creek |
| 2002 | 31024 | Laguna Lake | 310LLK | Laguna Lake |
| 2002 | 30800 | Limekiln Creek | 308LIM | Limekiln Creek |
| 2002 | 30800 | Little Sur River | 308LSU | Little Sur River near mouth |
| 2002 | 31031 | Lopez Lake | 310LOL | Lopez Lake |
| 2002 | 31031 | Los Berros Creek | 310AGL | Los Berros @ Los Berros Rd. |
| 2002 | 30800 | Mill Creek | 308MIL | Mill Creek |
| 2002 | 31020 | Morro Bay | 310428.5 | Morro Bay/Virg's |
| 2002 | 31021 | Morro Creek | 310MOR | Morro Ck @ Quintana |
| 2002 | 31017 | Old Creek | 310OLD | Old Creek @ Old Creek Rd. |
| 2002 | 31025 | Port San Luis | 310437 | Point San Luis |
| 2002 | 31024 | Prefumo Creek | 310PRE | Prefumo @ Calle Joaquin |
| 2002 | 31011 | San Carpoforo Creek | 310SCP | San Carpoforo Creek @ Hwy 1 |
| 2002 | 30800 | San Jose Creek | 308SJC | San Jose Creek@ Hwy 1 |
| 2002 | 31024 | San Luis Obispo Creek | 310SLC | San Luis Obispo @ Cuesta Pk |
| 2002 | 31024 | San Luis Obispo Creek | 310SLV | San Luis Obispo @ Venado Tr |
| 2002 | 31013 | San Simeon Creek | 310SSU | San Simeon Creek d/s Steiner Creek |
| 2002 | 31014 | Santa Rosa Creek | 310SRU | Santa Rosa Creek @ Main St |
| 2002 | 31024 | Stenner Creek | 310SCN | Stenner Ck @ Nipomo |
| 2002 | 31018 | Toro Creek | 310TOR | Toro Ck. @ Toro Ck. Rd. |
| 2002 | 30800 | Tularcitos Creek | 308TUL | Tularcitos @ Carmel Valley |
| 2002 | 31015 | Villa Creek | 310VIA | Villa Creek @ Private Rd. Access |
| 2002 | 31017 | Whale Rock Reservoir | 310WRR | Whale Rock Reservoir |
| 2003 | 30420 | Ano Nuevo Island | 304400 | Ano Nuevo Island |
| 2003 | 30413 | Aptos Creek | 304APS | Aptos Ck. @ Soquel Rd. |
| 2003 | 30412 | Bear Creek | 304BEP | Bear Ck @ Elks Park |
| 2003 | 30412 | Boulder Creek | 304BH9 | Boulder Ck @ Highway 9 |
| 2003 | 30520 | Chesbro Reservoir | 305CHR | Chesbro Reservoir |
| 2003 | 30550 | Clear Creek | 305CCC | Clear Ck @ Clear Creek Rd. |
| 2003 | 30510 | Corralitos Creek | 305COR | Corralitos |
| 2003 | 30510 | Corralitos Creek | 305COR2 | Upper Corralitos |
| 2003 | 30510 | Harkins Slough | 304HAR | Harkins Sl @ Harkins Sl Rd |
| 2003 | 30550 | Hernandez Reservoir | 305HER | Hernandez Reservoir |
| 2003 | 30550 | Hernandez Reservoir | 305LAZ | Lazy 10 Marina |
| 2003 | 30550 | Laguna Creek | 305LAG | Laguna Creek |
| 2003 | 30530 | Llagas Creek | 305HOL | Llagas @ Holsclaw |
| 2003 | 30530 | Llagas Creek | 305LLA | Llagas @ Bloomfield |
| 2003 | 30412 | Loch Lomond | 304LCH | Loch Lomond |
| 2003 | 30412 | Newell Creek | 304NGA | Newell Creek@ Glen Arbor |
| 2003 | 30540 | Pacheco Creek | 305PAC | Pacheco Creek |
| 2003 | 30540 | Pacheco Lake | 305PAL | Pacheco Lake |
| 2003 | 30510 | Pajaro River | 305CHI | Pajaro @ Chittendon |
| 2003 | 30530 | Pajaro River | 305FRA | Pajaro @ Frazier Lake Rd. |
| 2003 | 30510 | Pajaro River | 305MUR | Pajaro @ Murphy's Crossing |
| 2003 | 30520 | Pajaro River | 305PAJ | Pajaro @ Betabel Rd. |
| 2003 | 30520 | Pajaro River | 305PES | Pescadero Ck |
| 2003 | 30510 | Pinto Lake | 305PIL | Pinto Lake |
| 2003 | 30550 | San Benito River | 305BRI | S Benito Bridge d/s/Willow Creek |

| Rotation Year | Hydrologic Subarea | Water Body | SiteTag | SiteName |
| --- | --- | --- | --- | --- |
| 2003 | 30550 | San Benito River | 305HRL | Below Hernandez Res. |
| 2003 | 30550 | San Benito River | 305HRU | Above Hernandez Reservoir |
| 2003 | 30550 | San Benito River | 305SAN | San Benito @ Y Rd. |
| 2003 | 30550 | San Benito River | 305SUR | Survey Marker/San Benito |
| 2003 | 30412 | San Lorenzo River | 304RIV | San Lorenzo River @ River St |
| 2003 | 30412 | San Lorenzo River | 304SLA | San Lorenzo River @ Alder |
| 2003 | 30412 | San Lorenzo River | 304SLE | San Lorenzo @ Elks Park |
| 2003 | 30412 | San Lorenzo River | 304SLP | San Lorenzo River @ Park |
| 2003 | 30530 | Santa Anna Creek | 305TES | Tequisquita Slough |
| 2003 | 30412 | Santa Cruz Harbor | 304400.7 | Santa Cruz Harbor/Inner |
| 2003 | 30411 | Scott Creek | 304SCM | Scott Ck d/s Mill Ck |
| 2003 | 30413 | Soquel Creek | 304SEO | Soquel East @ Olive Spring |
| 2003 | 30413 | Soquel Creek | 304SWO | Soquel West @ Olive Spring |
| 2003 | 30550 | Tres Pinos Creek | 305TRE | Tres Pinos |
| 2003 | 30530 | Uvas Creek | 305UVA | Uvas @ Bloomfield |
| 2003 | 30520 | Uvas Reservoir | 305UVR | Uvas Reservoir |
| 2003 | 30413 | Valencia Creek | 304VAL | Valencia Ck @ Soquel Rd. |
| 2003 | 30510 | Watsonville Slough | 305WSA | Watsonville Sl @ San Andreas |
| 2003 | 30550 | Willow Creek | 305WIL | Willow Creek |
| 2003 | 30412 | Zayante Creek | 304ZAY | Zayante Ck @ Graham Hill Rd |
| 2004 | 30970 | Alisal Creek | 309UAL | Upstream Salinas reclamation ditch at old Stage Road |
| 2004 | 30960 | Arroyo Seco River | 309SEC | Arroyo Seco River at the Elm St. bridge |
| 2004 | 30930 | Arroyo Seco River | 309SET | Arroyo Seco River at Thorne Rd. |
| 2004 | 30981 | Atascadero Creek | 309ATS | Atascadero Creek at Highway 41 bridge |
| 2004 | 30600 | Carneros Creek | 306CAR | Carneros Creek in Los Lomas at Blohm Road |
| 2004 | 31700 | Cholame Creek | 317CHO | Cholame Creek at Bitterwater Road |
| 2004 | 30600 | Espinosa Lake | 306ESP | Espinosa Lake |
| 2004 | 31700 | Estrella River | 317EST | Estrella River at Airport Road |
| 2004 | 30920 | Gabilan Creek | 309GAB | Gabilan Creek at Independence Road and East Boranda Road |
| 2004 | 30982 | Nacimiento Lake | 309NLK | Nacimiento Lake |
| 2004 | 30981 | Nacimiento River | 309NAC | Nacimiento River at Highway 101 |
| 2004 | 30910 | Old Salinas River | 309POT | Old Salinas River at Potrero Road |
| 2004 | 30920 | Quail Creek | 309QUA | Quail Creek at Potter Road |
| 2004 | 30920 | Quail Creek | 309UQA | Up stream Quail Creek at Old Stage Rd. |
| 2004 | 30910 | Salinas Reclamation Canal | 309ALD | Salinas Reclamation Canal at Boranda Road |
| 2004 | 30910 | Salinas Reclamation Canal | 309ALU | Salinas Reclamation Canal at Airport Road |
| 2004 | 30910 | Salinas Reclamation Canal | 309AXX | Salinas Reclamation Canal Storm Drain at and Airport Road |
| 2004 | 30910 | Salinas River (Lower) | 309DAV | Salinas River at Davis Road |
| 2004 | 30910 | Salinas River (Lower) | 309SAC | Salinas River at Chualar bridge on River Road |
| 2004 | 30910 | Salinas River (Lower) | 309SDR | Salinas Storm Drain up stream of Davis Road |
| 2004 | 30940 | Salinas River (Mid) | 309DSA | Salinas River down stream of San Ardo at Cattleman Road |
| 2004 | 30930 | Salinas River (Mid) | 309GRN | Salinas River at Elm Road in Greenfield |
| 2004 | 30940 | Salinas River (Mid) | 309KNG | Salinas River at Highway 101 in King City |
| 2004 | 30981 | Salinas River (Upper) | 309PSO | Salinas River at 13th st bridge in Paso Robles |
| 2004 | 30981 | Salinas River (Upper) | 309SAT | Salinas River at Highway 41 bridge |
| 2004 | 30981 | Salinas River (Upper) | 309SUN | Salinas River up stream of Nacimiento at Bradley Road bridge |
| 2004 | 30981 | Salinas River (Upper) | 309USA | Up stream San Ardo, at Highway 101 and Bradley Bridge |
| 2004 | 30983 | San Antonio Lake | 309SLK | San Antonio Lake |

| Rotation Year | Hydrologic Subarea | Water Body | SiteTag | SiteName |
| --- | --- | --- | --- | --- |
| 2004 | 30981 | San Antonio River | 309SAN | San Antonio River at Highway 101 |
| 2004 | 30940 | San Lorenzo Creek | 309LOK | San Lorenzo Creek at First Street in King City |
| 2004 | 30970 | San Lorenzo Creek | 309LOR | San Lorenzo Creek at Bitterwater Road east of King City |
| 2004 | 30981 | Santa Margarita Lake | 309SML | Santa Margarita Lake |
| 2004 | 30910 | Tembladero Slough | 309TEM | Tembladero Slough at Preston Road |
| 2004 | 30970 | Topo Creek | 309TOP | Topo Creek at Metz Road |
| 2005 | 31230 | Alamo Creek | 312ALA | Alamo Creek at Alamo Creek Road |
| 2005 | 31210 | Blosser Channel | 312BCD | Blosser Channel d/s of ponds |
| 2005 | 31210 | Bradley Channel | 312BCU | Bradley Channel d/s of ponds |
| 2005 | 31210 | Bradley Cyn Creek | 312BCF | Bradley Canyon Diversion Channel at Foxen Canyon |
| 2005 | 31210 | Bradley Cyn Creek | 312BCG | Bradley Canyon Creek at Orcut-Garey Road |
| 2005 | 31210 | Bradley Cyn Creek | 312MSD | Main Street ditch |
| 2005 | 31220 | Cat Cyn Creek | 312CAT | Cat Canyon Creek at Foxen Canyon Road |
| 2005 | 31230 | Cuyama River | 312CAV | Cuyama above Ventucopa at highway 33 bridge |
| 2005 | 31230 | Cuyama River | 312CCC | Cuyama at Cottonwood Canyon |
| 2005 | 31230 | Cuyama River | 312CKM | Cuyama above Kirschenmann Road at 166 |
| 2005 | 31230 | Cuyama River | 312CUT | Cuyama below Twitchell at White Rock Lane |
| 2005 | 31230 | Cuyama River | 312CUY | Cuyama below Buckhorn Road |
| 2005 | 31230 | Huasna River | 312HUA | Husana River at Husana Townsite Road |
| 2005 | 31220 | LaBrea Creek | 312BRE | LaBrea Creek at Rancho Sisquoc Vineyard |
| 2005 | 31210 | Nipomo Creek | 312NIP | Nipomo Creek at the speedway |
| 2005 | 31210 | Nipomo Creek | 312NIT | Nipomo Creek at Tefft Street |
| 2005 | 31210 | Orcutt Solomon Creek | 312OLA | Orcutt Solomon tributary at Betteravia Lakes |
| 2005 | 31210 | Orcutt Solomon Creek | 312ORB | Orcutt Solomon Creek at Black Road |
| 2005 | 31210 | Orcutt Solomon Creek | 312ORC | Orcutt Solomon Creek above Estuary |
| 2005 | 31210 | Orcutt Solomon Creek | 312ORI | Orcutt Solomon Creek at Highway 1 |
| 2005 | 31210 | Oso Flaco Creek | 312OFN | Oso Flaco north tributary at railroad crossing |
| 2005 | 31210 | Oso Flaco Lake | 312OFC | Oso Flaco Creek at Oso Flaco Lake Road |
| 2005 | 31210 | Oso Flaco Lake | 312OFL | Oso Flaco Lake at culvert |
| 2005 | 31210 | Oso Flaco Lake | 312OFL1 | Oso Flaco Lake Boardwalk at first alcove |
| 2005 | 31210 | Oso Flaco Lake | 312OFL2 | Oso Flaco Lake Boardwalk at second alcove |
| 2005 | 31210 | Oso Flaco Lake | 312OLK | Oso Flaco Lake |
| 2005 | 31230 | Salisbury Creek | 312SAL | Salisbury Creek at Branch Canyon Wash |
| 2005 | 31210 | Santa Maria River | 312SBC | Santa Maria River at Bull Canyon Rd |
| 2005 | 31210 | Santa Maria River | 312SBS | Santa Maria River at Bonita School Road |
| 2005 | 31210 | Santa Maria River | 312SMI | Santa Maria River at Highway 1 |
| 2005 | 31220 | Sisquoc River | 312SIS | Sisquoc above Fugler Point |
| 2005 | 31220 | Sisquoc River | 312SIV | Sisquoc at Rancho Sisquoc Vineyard |
| 2005 | 31100 | Soda Lake | 311SLE | Soda Lake Northeast |
| 2005 | 31100 | Soda Lake | 311SLN | Soda Lake North |
| 2005 | 31100 | Soda Lake | 311SOL | Soda Lake |
| 2005 | 31230 | Twitchell Reservoir | 312TWR | Twitchell Reservoir |
| 2006 | 31532 | Arroyo Burro Creek | 315ABH | Arroyo Burro Creek at Hope Street |
| 2006 | 31534 | Arroyo Paredon | 315APC | Arroyo Paredon Creek at Via Real |
| 2006 | 31531 | Atascadero Creek | 315ATU | Atascadero Creek u/s of Patterson |
| 2006 | 31510 | Bell Creek | 315BEL | Bell Creek on Bacara Resort Access Rd. |
| 2006 | 31452 | Cachuma, Lake | 314CHL | Cachuma, Lake |
| 2006 | 30000 | Canada de la Gaviota | 315GAI | Gaviota Creek at Highway 1 bridge |

| Rotation Year | Hydrologic Subarea | Water Body | SiteTag | SiteName |
| --- | --- | --- | --- | --- |
| 2006 | 30000 | Canada del Refugio | 315RSB | Canada del Refugio u/s Refugio State Beach |
| 2006 | 31534 | Carpinteria Creek | 315CAU | Carpenteria Creek at Highway 192 |
| 2006 | 30000 | Devereux Slough | 315DEV | Devereux Slough at the Golf Course culvert |
| 2006 | 31531 | Devereux Slough | 315DVS | Devereux Slough |
| 2006 | 30000 | Dos Pueblos Canyon Creek | 315DOS | Dos Pueblos Canyon Creek at Highway 101 |
| 2006 | 30000 | El Capitan Creek | 315CAP | El Capitan Creek above the campground |
| 2006 | 31534 | Franklin Creek | 315FRC | Franklin Creek at 7th St. |
| 2006 | 31451 | Gibraltar Reservoir | 314GBR | Gibraltar Reservoir |
| 2006 | 31531 | Glenn Annie Creek | 315ANN | Glenn Annie Creek u/s Holister |
| 2006 | 31510 | Jalama Creek | 315JAL | Jalama Creek at County Park |
| 2006 | 31451 | Jameson Lake | 314JML | Jameson Lake |
| 2006 | 30000 | Los Carneros Creek | 315LCR | Los Carneros Creek @ Hollister Rd. |
| 2006 | 30000 | Maria Ynacio Creek | 315MYC | Maria Ynacio Creek u/s Patterson Av. |
| 2006 | 31532 | Mission Creek | 315MIU | Mission Creek at Cathedral Oaks Rd. |
| 2006 | 31532 | Montecito Creek | 315MTC | Montecito Creek at Jamison Ln. |
| 2006 | 31533 | Romero Creek | 315ROM | Romero Creek at Jamison Ln. |
| 2006 | 30000 | Salsipuodes Creek | 313SAL | Salsipuodes Creek at Santa Rosa Road |
| 2006 | 31300 | San Antonio Creek | 313SAB | San Antonio Creek @ Rancho de las Flores Bridge & Hwy 135 |
| 2006 | 31300 | San Antonio Creek | 313SAE | San Antonio Creek at San Antonio Road East |
| 2006 | 31300 | San Antonio Creek | 313SAI | San Antonio Creek at San Antonio Road West |
| 2006 | 31531 | San Jose Creek | 315SJC | San Jose Creek at Kellogg Blvd |
| 2006 | 31610 | San Miguel Island | 316500 | San Miguel Island/West |
| 2006 | 30000 | San Miguelito Creek | 315MIG | San Miguelito Creek at Central and V st. |
| 2006 | 31531 | San Pedro Creek | 315SPC | San Pedro Creek down stream of Holister Rd. |
| 2006 | 31532 | San Ysidro Creek | 315YSI | San Ysidro Creek at Jamison Ln. |
| 2006 | 31532 | Santa Barbara Harbor | 315470 | Santa Barbara Harbor |
| 2006 | 31630 | Santa Cruz Island | 316502 | Santa Cruz Island |
| 2006 | 31410 | Santa Ynez River | 314SYC | Santa Ynez River d/s Chachuma on 154 |
| 2006 | 31410 | Santa Ynez River | 314SYF | Santa Ynez River d/s Lompoc at Floordale |
| 2006 | 31410 | Santa Ynez River | 314SYI | Santa Ynez River at Highway 101 |
| 2006 | 31410 | Santa Ynez River | 314SYL | Santa Ynez River u/s Lompoc at Highway 246 |
| 2006 | 31410 | Santa Ynez River | 314SYP | Santa Ynez River at Paradise Rd. |
| 2006 | 31533 | Sycamore Creek | 315SCC | Sycamore Creek at Punta Gorda |
| 2006 | 31510 | Tecolote Creek | 315TCI | Tecolote Creek at Bacara Resort access Rd. |
| 2006 | 31534 | Toro Canyon Creek | 315TOR | Toro Canyon Creek at Via Real |
| Ongoing | 30413 | Aptos Creek | 304APT | Aptos Creek lagoon |
| Ongoing | 31532 | Arroyo Burro Creek | 315ABU | Arroyo Burro |
| Ongoing | 31012 | Arroyo de la Cruz Creek | 310ADC | Arroyo de la Cruz lagoon |
| Ongoing | 31031 | Arroyo Grande Creek | 310ARG | Arroyo Grande Creek lagoon |
| Ongoing | 31531 | Atascadero Creek | 315ATA | Atascadero Creek |
| Ongoing | 30800 | Big Creek | 308BGC | Big Creek |
| Ongoing | 30800 | Big Sur River | 308BSR | Sur River |
| Ongoing | 31510 | Canada de la Gaviota | 315GAV | Canada de la Gaviota |
| Ongoing | 30700 | Carmel River | 307CML | Carmel River |
| Ongoing | 31534 | Carpinteria Creek | 315CRP | Carpinteria Creek |
| Ongoing | 31534 | Carpinteria Marsh | 315CAR | Carpinteria Marsh |
| Ongoing | 31022 | Chorro Creek | 310TWB | Chorro Creek mouth |
| Ongoing | 30600 | Elkhorn Slough | 306ELK | Elkhorn Slough |

| Rotation Year | Hydrologic Subarea | Water Body | SiteTag | SiteName |
| --- | --- | --- | --- | --- |
| Ongoing | 30420 | Gazos Creek | 304GAZ | Gazos Creek Lagoon |
| Ongoing | 31531 | Goleta Slough | 315GOL | Goleta Slough |
| Ongoing | 31023 | Los Osos Creek | 310SYB | Los Osos Creek mouth |
| Ongoing | 31532 | Mission Creek | 315MIS | Mission Creek @ Train Station |
| Ongoing | 30950 | Monterey Harbor | 309MON | Monterey Harbor |
| Ongoing | 30600 | Moro Cojo Slough | 306MOR | Moro Cojo Slough |
| Ongoing | 30600 | Moss Landing Harbor | 306404 | Sandholdt Bridge |
| Ongoing | 30600 | Moss Landing Harbor | 306MOS | Moss Landing Harbor |
| Ongoing | 30910 | Old Salinas River | 309OLD | Old Salinas River Estuary |
| Ongoing | 30950 | Pacific Grove Coast | 309414 | Pacific Grove |
| Ongoing | 30510 | Pajaro River | 305PJE | Pajaro River Estuary |
| Ongoing | 30510 | Pajaro River | 305THU | Pajaro at Thurwacher Br. |
| Ongoing | 31013 | Pico Creek | 310PCO | Pico Creek Lagoon |
| Ongoing | 31026 | Pismo Creek | 310PIS | Pismo Creek lagoon |
| Ongoing | 31510 | Point Conception | 315450 | Point Conception |
| Ongoing | 31534 | Rincon Creek | 315RIN | Rincon Creek near Hwy 101 |
| Ongoing | 30910 | Salinas River (Lower) | 309SBR | Salinas River |
| Ongoing | 31300 | San Antonio Creek | 313SAC | San Antonio Creek |
| Ongoing | 30412 | San Lorenzo River | 304LOR | San Lorenzo Estuary |
| Ongoing | 31025 | San Luis Obispo Creek | 310SLB | San Luis Obispos Creek |
| Ongoing | 31024 | San Luis Obispo Creek | 310SLO | San Luis Obispo Creek lagoon |
| Ongoing | 31013 | San Simeon Creek | 310SSC | San Simeon Creek lagoon |
| Ongoing | 30412 | Santa Cruz Harbor | 304SCH | Santa Cruz Yacht Harbor |
| Ongoing | 31210 | Santa Maria River | 312SMA | Santa Maria River at Estuary |
| Ongoing | 31534 | Santa Monica Creek | 315SMC | Santa Monica Creek |
| Ongoing | 31014 | Santa Rosa Creek | 310SRO | Santa Rosa Creek lagoon |
| Ongoing | 31410 | Santa Ynez River | 314SYN | Santa Ynez River Estuary |
| Ongoing | 30411 | Scott Creek | 304SCO | Scott Creek lagoon |
| Ongoing | 30413 | Soquel Creek | 304SOQ | Soquel Creek lagoon |
| Ongoing | 30910 | Tembladero Slough | 309TDW | Tembladero Slough |
| Ongoing | 30411 | Waddell Creek | 304WAD | Waddell Creek lagoon |
| Ongoing | 30510 | Watsonville Slough | 305WAT | Watsonville Slough |
| Ongoing | 30800 | Willow Creek | 308WLO | Willow Creek mouth |

## Objectives

Table 2 indicates the relationship between monitoring types and beneficial uses recognized in the Central Coast Basin Plan.  Monitoring approaches currently employed by CCAMP are shown in bold.  Though the program will become more robust as additional monitoring approaches are added, the current suite of monitoring activities address all beneficial uses to some degree.  Virtually all primary rivers and streams and their immediate tributaries in Region 3 are designated for cold water fisheries, commercial and sport fishing, contact and non-contact recreation, groundwater recharge, municipal and domestic supply, spawning, and migration beneficial uses.  Many also support threatened and endangered species and biological habitats of special significance.  Because these important beneficial uses tend to be universal in the Region and require most stringent water quality objectives, the CCAMP suite of indicators targets these beneficial uses particularly, and is intended to be applied uniformly to all sites (though funding at this point disallows such application).

The CCAMP program design includes monthly monitoring for conventional water quality (CWQ) at all sites.  At a subset of sites, generally selected based on hydrogeomorphological considerations or special interest, other monitoring approaches are applied.  These include sediment chemistry and toxicity, tissue chemistry, Rapid Bioassessment for benthic invertebrates, habitat assessment, and flow measurement.  Other approaches which have not yet been applied but which will be included as funding increases include water column chemistry, sedimentation assessment, habitat assessment, geomorphology and remote sensing.

#### Table 2.  Relationship between Beneficial Uses in Region 3 and Monitoring Activities  (currently employed monitoring approaches are shown in bold upper case)

|  | CWQ | SedChem | H2OChem | Tissue Chem | Rapid Bioassessment | Toxicity | Geomorphology | Habitat | Remote Sensing | ###### Flow | Sedimentation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Municipal & Domestic | X |  | x | X |  |  |  |  | x | X |  |
| Estuarine Habitat | X | X | x | X | X | X | x | x | x | X | x |
| Marine Habitat | X | X | x | X | X | X | x | x | x |  | x |
| Wildlife Habitat | X | X | x | X | X | X | x | x | x | X | x |
| Biological Habitat of Special Significance | X | X | x | X | X | X | x | x | x | X | x |
| Rare & Endangered Species | X | X | x | X | X | X | x | x | x | X | x |
| Fish Migration | X | X | x | X | X | X | x | x | x | X | x |
| Fish Spawning | X | X | x | X | X | X | x | x | x | X | x |
| Shellfishing | X |  |  | X |  |  |  |  |  |  |  |
| ASBS | X | X | x | X | X | X | x | x | x | X | x |
| Agricultural Supply | X | X | x |  |  |  |  |  |  | X |  |
| Industrial Process Supply | X |  | x |  |  |  |  |  |  |  |  |
| Industrial Service Supply | X |  |  |  |  |  |  |  | x |  | x |
| Groundwater Recharge | X |  | x |  |  |  | x |  | x | X |  |
| Fresh Water Replenishment | X |  | x |  |  |  | x |  | x | X |  |
| Navigation | X | X |  |  |  | X | x |  | x | X | x |
| Hydroelectric Power Generation | X |  |  |  |  |  | x |  | x | X |  |
| Water Contact Recreation | X |  |  |  |  |  |  |  |  |  |  |
| NonContact Recreation | X |  |  |  |  |  |  |  |  |  |  |
| Commercial and Sport Fishing | X | X | x | X | X | X | x | x | x | X |  |
| Aquaculture | X | X | x | X |  | X |  |  |  |  |  |
| Warm Water Habitat | X | X | x | X | X | X | x | x | x | X | x |
| Coldwater Habitat | X | X | x | X | X | X | x | x | x | X | x |

CCAMP Site-Specific Monitoring Objectives

The following objectives address questions posed in the SWAMP Site-Specific Monitoring Guidance related to beneficial use support.  Though the program will become more robust as additional monitoring approaches are added, the current suite of monitoring activities address all objectives to some degree.

Is it safe to swim?

Beneficial Use: Water Contact Recreation

Monitoring Aproach:  CWQ monthly monitoring for coliform organisms; compilation of other data sources

Objective(s):  At sites throughout water bodies that are used for swimming, or that drain to areas used for swimming, screen for indications of bacterial contamination by determining percent of samples exceeding adopted water quality objectives.

Compile pathogen indicator data collected by local agencies and organizations into a single database to describe shoreline and creek mouth conditions, and screen for geographic indications of bacterial contamination by determining percent of samples exceeding adopted water quality objectives.

Is it safe to drink the water?

Beneficial Use: Municipal and Domestic Water Supply

Monitoring Approach:  CWQ monthly sampling for coliform, nutrients, and minerals

Objective(s):  At sites throughout water bodies that are sources of drinking water, screen for percent of samples of microbial or chemical contaminants that exceed drinking water standards or adopted water quality objectives used to protect drinking water quality.

At sites throughout water bodies that are sources of drinking water, estimate the

concentration of microbial or chemical contaminants from month-to-month above

and below screening values, drinking water standards, or adopted water quality

objectives used to protect drinking water quality.

Is it safe to eat fish and other aquatic resources?

Beneficial Uses: Commercial and Sport Fishing, Shellfish Harvesting

Monitoring Approach:  Annual fish and mussel tissue collection and chemical analysis through the State Mussel Watch Program, Toxic Substances Monitoring Program and Coastal Fish Contamination Program.

Objective(s):  At sites located near the lower ends of streams and rivers, and in lakes, enclosed bays and estuaries, screen for contamination problems by estimating the concentration of chemical contaminants in fish and shellfish, and determining whether samples exceed several critical threshold values of potential human impact (advisory or action levels).

At long-term monitoring sites located near the lower ends of streams and rivers, in nearshore areas and in enclosed bays and estuaries, estimate the year-to-year concentration of chemical contaminants in fish and shellfish and determine whether samples exceed several critical threshold values of potential human impact (advisory or action levels).

At popular fishing locations at river mouths, lakes, nearshore waters, enclosed bays,

and estuaries, screen for contamination problems by estimating the concentration of chemical contaminants in sport fish species and determining whether samples exceed several critical threshold values of potential human impact (advisory or action levels).

Are aquatic populations, communities, and habitats protected?

Beneficial Uses: Cold Freshwater Habitat; Preservation of Biological Habitats; Warm Freshwater Habitat; Wildlife Habitat

Monitoring Approach:  Synoptic sampling for sediment and water column toxicity, sediment chemistry, and benthic invertebrate assemblages.  Toxicity Identification Evaluation and/or chemistry follow-through for toxic sites.

Objective(s):  At sites along the mainstem and at the lower ends of major tributaries of streams and  rivers, screen for indications of water quality and sediment degradation using several critical threshold values of toxicity, benthic community analysis, habitat condition, and chemical concentration.

Beneficial Use: Rare, Threatened or Endangered Species; Spawning

Monitoring Approach: CWQ monthly sampling for dissolved oxygen, nutrients, turbidity, temperature.  Pre-dawn sampling for dissolved oxygen sags.  Toxicity sampling of rainbow trout.  Benthic invertebrate assemblage assessment.

Objective(s):  At sites along the mainstem and at the lower ends of major tributaries of streams and  rivers, screen for indications of water quality degradation for anadromous fish species, using rainbow trout toxicity testing, benthic community analysis, habitat condition, and physical and chemical water condition.

Is water safe for agricultural use?

Beneficial Use: Agricultural supply

Monitoring Approach:  CWQ monthly sampling for nutrients, salts and TDS

Objective(s):  At sites throughout waterbodies that are used for agricultural purposes, estimate the concentration of chemical pollutants from month-to-month and year-to-year above or below screening values or adopted water quality objectives used to protect agricultural uses.

Are aesthetic conditions of the water protected?

Beneficial Use: Non-Contact Water Recreation

Monitoring Approach:  CWQ monthly qualitative assessment of % algal cover, presence of scum, odor, etc.

Objective(s):  At sites throughout water bodies, estimate the aesthetic condition from month-to-month and year-to-year above or below screening values used to protect non-contact water recreation.

Table 3.  Program Objectives and monitoring activities which can address them

## 3.1  General study design

## Overview of general approach

- The Central Coast Ambient Monitoring Program (CCAMP) monitoring strategy for watershed characterization calls for dividing the Region into five watershed rotation areas and conducting synoptic, tributary based sampling each year in one of the areas. Over a five-year period all of the Hydrologic Units in the Region are monitored and evaluated. In addition to the synoptic site selection approach, additional monitoring sites are established in each rotation area to provide focused attention on watersheds and waterbodies known to have water quality impairments.

- The CCAMP strategy of establishing and maintaining permanent long term monitoring sites provides a framework for trend analysis and detection of emergent water quality problems. CCAMP uses a variety of monitoring approaches to characterize status and trends of coastal watersheds, including:

- Rapid Bioassessment using benthic invertebrates

- Conventional water quality analysis

- Analysis of tissue, water, and sediment for organic chemicals and metals

- Toxicity evaluations

- Habitat assessments

- Sedimentation evaluations

- In order to develop a broad picture of the overall health of waters in Region 3, a similar baseline monitoring study design is applied in each watershed.  This provides compatibility across the Region and allows for prioritization of problems across a relatively large spatial scale.  However, it is important that each watershed analysis incorporate additional, watershed specific knowledge into the study design, so that questions which are narrower in focus can also be addressed.  For example, in watersheds where Total Maximum Daily Load assessments are being undertaken, other program funds can be applied to support additional monitoring for TMDL development.  Special studies are undertaken as funding and staffing permits to further focus monitoring on questions of interest specific in individual watersheds.

- Watershed characterization involves three major components: acquisition and evaluation of existing data, monitoring of surface water and habitat quality, and developing a watershed assessment based on findings.

- Initial Data Acquisition and Evaluation

Existing sources of data are evaluated for pollutants of concern, historic trends, data gaps, etc. These include Department of Health Services, USGS, Department of Fish and Game, Department of Pesticide Regulation, Toxic Substances Monitoring Program, STORET, NPDES discharge data, and other sources. Data from County, City, and other selected programs are acquired. Existing data is evaluated for standard exceedences, pollutant levels which warrant attention, beneficial use impairment, and other pertinent information.

- Watershed Monitoring

In order to provide water body assessments on the USEPA mandated reporting cycle, the Region has been divided into five watershed groups based on scientific, logistic, and financial considerations.  Figure 1 shows these watershed divisions.  This component of the CCAMP study design involves collection of surface water quality and habitat data which will provide information at a level of detail suitable for assessment reporting requirements, including Clean Water Act 305(b) and 303(d), and for supporting various statewide programs and initiatives (e.g. Nonpoint Source Program, Watershed Management Initiative).

#### Figure 1. Watershed Rotation in Region 3

Sampling sites are located at the primary discharge point of the watershed, and at the discharge of each major tributary into the watercourse which drains the watershed. For the purposes of site selection a "major tributary" is defined as a watercourse which drains a minimum percentage of the rotation area or which is the major watercourse that drains a Hydrological Area, Hydrological Subarea, or watershed of special concern. Some sites are also located above and below areas of significant human activity, including urban development, agriculture, and point source discharges. Site selection is constrained by site accessibility. In order to provide safe, all-weather access, conventional water quality sites are located preferably at bridges where sampling devices can be suspended during periods of high flow.  Benthic invertebrate sites are located upstream of conventional water quality sites, but out of the immediate influence of bridges.  Other sampling activities are conducted at a subset of conventional water quality sites.

Basic characterization includes collection of water chemistry, sediment chemistry, toxicity, tissue bioaccumulation, habitat assessment, and bioassessment data, as well as acquisition of basic GIS data layers, where available, describing land use, geology, soils, discharge locations, known problem sites, etc.

Conventional Water Quality - Basic water quality parameters, including nutrients, fecal and total coliform, dissolved oxygen, turbidity, pH, chlorophyll a, major ions and conductivity, are collected once a month.  Monthly sampling provides an opportunity to evaluate seasonal variability as well as a variety of flow conditions.  This program is not designed to be a stormwater monitoring program.  Sampling is maintained on an even monthly interval without regard for timing of weather events.  Even interval sampling permits use of certain time-series analysis techniques, such as the Mann-Kendall or seasonal Kendall tests described by the U.S. EPA National Monitoring Program (Monitoring Guidance for Determining the Effectiveness of Nonpoint Source Pollution Controls, EPA 1997).

A multianalyte probe is used to collect several of the parameters, including dissolved oxygen, pH, water temperature, turbidity, conductivity and chlorophyll a. Observations of air temperature, algal growth, scum, odor, and other indications of water and habitat conditions are also recorded.  Flow is estimated using a number of means.  Whereever possible, sites are located near existing county and USGS gages.  Flow is estimated elsewhere using cross-sectional flow measurements (pygmy gurley) or stream stage and calibrated discharge curves.  Water samples are bottled as appropriate and held at 4oC, before being transferred to a commercial laboratory for analysis. Chain-of-Custody documentation is maintained for all samples. Quality assurance procedures at the laboratory are consistent with SWAMP approved quality assurance requirements and follow U.S. EPA approved methods (BC Laboratories, 1998). Water quality parameters collected on a monthly basis are shown in Table 5.

#### Table 5.  Analytes for Monthly Conventional Water Quality Data Collection

| Analyte | Units | Typical MDL |
| --- | --- | --- |
| pH | pH units | n/a |
| Conductivity | US/cm | 1.0 |
| Turbidity | NTU | 0.1 |
| Dissolved Oxygen | Ppm | 0.01 |
| Oxygen Saturation | % Saturation | n/a |
| Water Temperature | Celsius | n/a |
| Air Temperature | Celsius | n/a |
| Total Coliform Bacteria | MPN/100 ml | 2 |
| Fecal Coliform Bacteria | MPN/100 ml | 2 |
| Nitrate-N | mg/l | 0.02 |
| Nitrite-N | mg/l | 0.01 |
| Total Kjeldahl Nitrogen | mg/l | 0.1 |
| Ammonia-NH3 | mg/l | 0.02 |
| Ortho Phosphate | mg/l | 0.01 |
| Total Phosphate | mg/l | 0.06 |
| Chlorophyll a | ug/l | 0.1 |
| Total Suspended Solids | mg/l | 0.5 |
| Fixed Suspended Solids | mg/l | 0.5 |
| Total Dissolved Solids | mg/l | 4.0 |
| Fixed Dissolved Solids | mg/l | 4.0 |
| Volatile Solids | mg/l | 5.0 |
| Volatile Suspended Solids | mg/l | 0.5 |
| Salinity | mg/l | 1 |
| CaCO3 | mg/l | 5 |
| Chloride | mg/l | 0.06 |
| Boron | mg/l | 0.05 |
| Calcium | mg/l | 0.01 |
| Magnesium | mg/l | 0.02 |
| Sodium | mg/l | 0.06 |
| Corridor Shading | % | n/a |
| Algal Cover | % | n/a |
| Plant Cover | % | n/a |
| Flow | Cfs | n/a |

Tissue Bioaccumulation - Fish or shellfish samples are collected to assess presence of chemicals which bioaccumulate in tissues. Sampling sites are generally located at the confluences of major tributaries and on the main stem. Corbicula clams or surrogate bioaccumulators are placed at selected sites during winter runoff by the State Mussel Watch Program. Clams are generally transplanted from the Russian River and held at Aptos Creek until deployment. They are placed in mesh bags at each station and secured to a rebar stake pounded into the substrate. Clams remain in the creeks for a minimum of four weeks before retrieval.  The standard analyte list for the State Mussel Watch Program and Toxic Substances Monitoring Program is shown in Table 6.

#### Table 6. Trace Metals, Synthetic Organic Compounds, and Polynuclear Aromatic Hydrocarbons Analyzed by the State Mussel Watch Program

| - Trace Elements |  |
| --- | --- |
| Aluminum | Nickel |
| Arsenic | Lead |
| Cadmium | Selenium |
| Chromium | Silver |
| Copper | Titanium |
| Mercury | Zinc |
| Manganese |  |
| - Synthetic Organic Compounds |  |
| aldrin | ethion |
| cis-chlordane | HCH, alpha |
| trans-chlordane | HCH, beta |
| chlordene, alpha | HCH, gamma |
| chlordene, gamma | HCH, delta |
| chlorpyrifos | heptachlor |
| dacthal | heptachlor epoxide |
| DDD, o, p' | HCB |
| DDD, p, p' | methoxychlor |
| DDE, o, p' | cis-nonachlor |
| DDE, p, p' | trans-nonachlor |
| DDMU, p, p' | oxadiazon |
| DDT, o, p' | oxychlordan |
| DDT, p, p' | parathion, ethyl |
| diazinon | parathion, methyl |
| dichlorobenzophenone-p, p' | PCB 1248 |
| dicofol (Kelthane) | PCB 1254 |
| dieldrin | PCB 1260 |
| endosulfan I | tetradifon (Tedion) |
| endosulfan II | toxaphene |
| endrin |  |
| - Polynuclear Aromatic Hydrocarbons (PAHs) |  |
| naphthalene | fluoranthene |
| 1-methylnaphthalene | pyrene |
| 2-methylnaphthalene | benz[a]anthracene |
| biphenyl | chrysene |
| 2,6-dimethlynaphthalene | benzo[b]fluoranthene |
| acenaphthylene | benzo[k]fluoranthene |
| 2,3,5-trimethylnaphthalene | benzo[e]pyrene |
| fluorene | benzo[a]pyrene |
| phenanthrene | perylene |
| anthracene | indeno[1,2,3-cd]pyrene |
| 1-methylphenanthrene | dibenz[a,h]anthracene |
| benz[ghi]perylene |  |

Benthic Macroinvertebrate Sampling - Benthic macroinvertebrate assemblages are indicators of stream health.  Different species of invertebrates respond differently to water pollution and habitat degradation and provide information on biological integrity.

Benthic invertebrate communities are sampled using California Rapid Bioassessment Protocols and quality assurance guidance for non-point source assessments (Harrington, 1996).  Three riffle locations are selected randomly from within stream reaches associated with water and sediment sampling.  Benthic invertebrates are collected in spring at selected sites.  When stream morphology limits riffle habitat, a low gradient protocol is adopted which includes sampling of stream margins.  The creek reach of interest is characterized according to geomorphic parameters, including bankful width, slope, drainage area, upstream river miles, particle size and other features. Geomorphic characteristics are considered during data evaluation.

Physical habitat quality is assessed at each sampling reach according to state protocols, using scoring as indicated in Table 7.  This assessment is qualitative in nature and can be subjective.  Therefore, field crews intercalibrate their assessments prior to conducting field work.

#### Table 7. Habitat Parameters used to assess habitat quality, from California Rapid Bioassessment Protocols

| Habitat Parameter | Condition |  |  |  |
| --- | --- | --- | --- | --- |
|  | Excellent | Good | Fair | Poor |
| Primary-Substrate and Instream Cover |  |  |  |  |
| 1. Instream Cover | 16-20 | 11-15 | 6-10 | 0-5 |
| 2. Epifaunal Substrate | 16-20 | 11-15 | 6-10 | 0-5 |
| 3. Embeddedness | 16-20 | 11-15 | 6-10 | 0-5 |
| 4. Channel Flow | 16-20 | 11-15 | 6-10 | 0-5 |
| Secondary - Channel Morphology |  |  |  |  |
| 5. Channel Alteration | 16-20 | 11-15 | 6-10 | 0-5 |
| 6. Sediment Deposition | 16-20 | 11-15 | 6-10 | 0-5 |
| 7. Riffle Frequency | 16-20 | 11-15 | 6-10 | 0-5 |
| Tertiary - Riparian and Bank Vegetation |  |  |  |  |
| 8. Bank Vegetation | 16-20 | 11-15 | 6-10 | 0-5 |
| 9. Bank Stability | 16-20 | 11-15 | 6-10 | 0-5 |
| 10. Riparian Zone | 16-20 | 11-15 | 6-10 | 0-5 |
| TOTALS | 151-200 | 101-150 | 51-100 | 0-50 |

Sediment Chemistry - Sediment samples are collected at each Rapid Bioassessment site, and are analyzed by a commercial laboratory. Sampling targets fine grain sediments. Laboratory analysis includes particle size distribution, as well as Total Organic Carbon. Laboratory analysis includes the parameters listed in Table 8.

#### Table 8.  Analyte List for Sediment Samples

| Analyte | EPAMethod | Units |  | Analyte | EPAMethod | Units |
| --- | --- | --- | --- | --- | --- | --- |
| % Clay | Plumb | % |  | Benzo(k)fluoranthene | 8270B | ug/Kg |
| % Sand | Plumb | % |  | beta-BHC | 8080A | ug/Kg |
| % Silt | Plumb | % |  | Chrysene | 8270B | ug/Kg |
| % Solids | 160.3 | % |  | delta-BHC | 8080A | ug/Kg |
| Azinphos-methyl | 8140 | ug/Kg |  | Dibenzo(a,h)anthracene | 8270B | ug/Kg |
| Bolstar | 8140 | ug/Kg |  | Dieldrin | 8080A | ug/Kg |
| Cadmium | 6020 | mg/kg |  | Endosulfan-I | 8080A | ug/Kg |
| Chlorpyrifos | 8140 | ug/Kg |  | Endosulfan-II | 8080A | ug/Kg |
| Chromium | 6020 | mg/kg |  | Endosulfan-Sulfate | 8080A | ug/Kg |
| Copper | 6020 | mg/kg |  | Endrin | 8080A | ug/Kg |
| Coumaphos | 8140 | ug/Kg |  | Endrin Aldehyde | 8080A | ug/Kg |
| Demeton-O | 8140 | ug/Kg |  | Endrin Ketone | 8080A | ug/Kg |
| Demeton-S | 8140 | ug/Kg |  | Flouranthene | 8270B | ug/Kg |
| Diazinon | 8140 | ug/Kg |  | Flourene | 8270B | ug/Kg |
| Dichlorvos | 8140 | ug/Kg |  | gamma-BHC | 8080A | ug/Kg |
| Disulfoton | 8140 | ug/Kg |  | gamma-Chlordane | 8080A | ug/Kg |
| Ethoprop | 8140 | ug/Kg |  | Heptachlor | 8080A | ug/Kg |
| Fensulfothion | 8140 | ug/Kg |  | Heptachlor-Epoxide | 8080A | ug/Kg |
| Fenthion | 8140 | ug/Kg |  | Indeno(1,2,3-cd)pyrene | 8270B | ug/Kg |
| Lead | 6020 | mg/kg |  | Methoxychlor | 8080A | ug/Kg |
| Mercury | 7471 | mg/kg |  | Naphthalene | 8270B | ug/Kg |
| Merphos | 8140 | ug/Kg |  | PCBs | 8080A | ug/Kg |
| Mevinphos | 8140 | ug/Kg |  | Phenanthrene | 8270B | ug/Kg |
| Naled | 8140 | ug/Kg |  | Pyrene | 8270B | ug/Kg |
| Nickel | 6020 | mg/kg |  | Toxaphene | 8080A | ug/Kg |
| Parathion-methyl | 8140 | ug/Kg |  | 4,4'-DDD | 608 | ug/L |
| Particle Size Wt.(<0.002mm) | Plumb | gm |  | 4,4'-DDE | 608 | ug/L |
| Particle Size Wt.(>.0156mm) | Plumb | gm |  | 4,4'-DDT | 608 | ug/L |
| Particle Size Wt.(>0.002mm) | Plumb | gm |  | Aldrin | 608 | ug/L |
| Particle Size Wt.(>0.0039mm) | Plumb | gm |  | alpha-BHC | 608 | ug/L |
| Particle Size Wt.(>0.0078mm) | Plumb | gm |  | alpha-Chlordane | 608 | ug/L |
| Particle Size Wt.(>0.0313mm) | Plumb | gm |  | Azinphos-methyl | 614 | ug/L |
| Particle Size Wt.(>0.0625mm) | Plumb | gm |  | beta-BHC | 608 | ug/L |
| Particle Size Wt.(>0.125mm) | Plumb | gm |  | Cadmium | 200.8 | ug/L |
| Particle Size Wt.(>0.25mm) | Plumb | gm |  | Chromium | 200.8 | ug/L |
| Particle Size Wt.(>0.5mm) | Plumb | gm |  | Copper | 200.8 | ug/L |
| Particle Size Wt.(>16mm) | Plumb | gm |  | delta-BHC | 608 | ug/L |
| Particle Size Wt.(>1mm) | Plumb | gm |  | Demeton-O | 614 | ug/L |
| Particle Size Wt.(>2mm) | Plumb | gm |  | Diazinon | 614 | ug/L |
| Particle Size Wt.(>32mm) | Plumb | gm |  | Dieldrin | 608 | ug/L |
| Particle Size Wt.(>4mm) | Plumb | gm |  | Disulfoton | 614 | ug/L |
| Particle Size Wt.(>8mm) | Plumb | gm |  | Endosulfan-I | 608 | ug/L |
| Phorate | 8140 | ug/Kg |  | Endosulfan-II | 608 | ug/L |
| Ronnel | 8140 | ug/Kg |  | Endosulfan-Sulfate | 608 | ug/L |
| Stirphos | 8140 | ug/Kg |  | Endrin | 608 | ug/L |
| Tokuthion | 8140 | ug/Kg |  | Endrin Aldehyde | 608 | ug/L |
| Trichloronate | 8140 | ug/Kg |  | Endrin Ketone | 608 | ug/L |
| Weight Coarse | Plumb | gm |  | Ethion | 614 | ug/L |
| Weight Fine | Plumb | gm |  | gamma-BHC | 608 | ug/L |
| Weight Total | Plumb | gm |  | gamma-Chlordane | 608 | ug/L |

| Analyte | EPAMethod | Units |  | Analyte | EPAMethod | Units |
| --- | --- | --- | --- | --- | --- | --- |
| Zinc | 6020 | mg/kg |  | Hardness | 130.2 | mg/L |
| 4,4'-DDD | 8080A | ug/Kg |  | Heptachlor | 608 | ug/L |
| 4,4'-DDE | 8080A | ug/Kg |  | Heptachlor-Epoxide | 608 | ug/L |
| 4,4'-DDT | 8080A | ug/Kg |  | Lead | 200.8 | ug/L |
| Acenaphthene | 8270B | ug/Kg |  | Malathion | 614 | ug/L |
| Acenaphthylene | 8270B | ug/Kg |  | Mercury | 200.8 | ug/L |
| Aldrin | 8080A | ug/Kg |  | Methoxychlor | 608 | ug/L |
| alpha-BHC | 8080A | ug/Kg |  | Nickel | 200.8 | ug/L |
| alpha-Chlordane | 8080A | ug/Kg |  | Parathion-ethyl | 614 | ug/L |
| Antracene | 8270B | ug/Kg |  | Parathion-methyl | 614 | ug/L |
| Benzo(a)fluoranthene | 8270B | ug/Kg |  | PCBs | 608 | ug/L |
| Benzo(a)pyrene | 8270B | ug/Kg |  | Toxaphene | 608 | ug/L |
| Benzo(b)fluoranthene | 8270B | ug/Kg |  | Zinc | 200.8 | ug/L |
| Benzo(g,h,i)perylene | 8270B | ug/Kg |  |  |  |  |

Watershed Assessment

Monitoring data is evaluated in consideration of hydrogeomorphic information, land use information, and other data to develop a watershed scale assessment.   The assessment is utilized to update the 305(b) report, 303(d) report, and support permit review and nonpoint source activities, etc.  In addition to surface water quality, relationships between groundwater and surface water quality will be examined by Basin Planning staff.

Data is also used to develop recommendations for action and for future research and monitoring needs.  For example, when benthic invertebrate sampling results indicate possible chemical impairment from upstream sources, sediment and/or water toxicity studies and Toxicity Identification Evaluations may be conducted to identify the nature of the problem and potential sources.

Watershed assessments are made available on the CCAMP web site.  Data is also made available online, and can be downloaded for further analysis.

## Water Quality Indicators

#### Table 9.  List of indicators for CCAMP Monitoring

| Beneficial Use | Monitoring Objectives | Category | Indicator |
| --- | --- | --- | --- |
| Water Contact | 1 | Contaminant exposure | Total coliform bacteria Fecal coliform bacteria |
| Drinking Water | 2 and 3 | Contaminant exposure | Inorganic water chemistry Nutrients Total coliform bacteria Fecal coliform bacteria |
| Fish and Shellfish Contamin-ation | 4, 5, 6, 7, and 8 | Contaminant exposure | Fish tissue chemistry Shellfish tissue chemistry Fecal coliform bacteria in water |
| Aquatic Life | 9, 10, 12, and 13 | Biological response | Chlorophyll-a Macroinvertebrate assemblages Periphyton (qualitative) Sediment interface toxicity |
|  |  | Pollutant exposure | Sediment interface toxicity Reporter Gene System (RGS 450) Organic and inorganic sediment chemistry Total organic carbon Shellfish or fish tissue chemistry Nutrients Turbidity Inorganic water chemistry |
|  |  | Habitat | Dissolved oxygen Sediment grain size Sediment organic carbon Water flow Water temperature Channel morphology Instream structure Substrate composition Riparian vegetation Electrical conductivity Salinity Ammonia |
| Sufficient Flow | 14 and 15 | Habitat | Water flow Instream habitat Water temperature Dissolved Oxygen |
|  |  | Biological response | Macroinvertebrate assemblages |
| Agricultural Supply | 16 and 17 | Pollutant Exposure | Inorganic chemistry Electrical conductivity |
| Industrial Supply | 18 and 19 | Pollutant Exposure | Organic and inorganic chemistry Temperature Electrical conductivity Turbidity |
| Aesthetic Condition | 20 and 21 | Pollutant Exposure | Presence of odor, scum, nuisance algae |

Adapted from:  SWRCB, 1993; SPARC, 1997; SCCWRP, 1998; Stephenson et al., 1994; CalEPA, 1998; CABW, 1998; CDFG, 1998; Noble et al., 1999; AB 982 Scientific Advisory Group, personal communication, August, 2000.

- Specific Activities Planned For FY 2001-02:  Specific technical approach and scope of work to be performed

The work planned for FY 2001-02 is described in this section.  The scale of sampling activities may be modified based on available funding.  Planned sampling includes funding from several sources other than SWAMP, including State Mussel Watch Program, Toxic Substances Monitoring Program,  CCAMP Endowment, and State Mussel Watch Endowment.

## List of Water Bodies to be sampled in FY 2001-02

Table 10 includes a list of water bodies to be sampled in FY 2001-02.  Because this sampling rotation includes many small coastal streams, some of these water bodies will be sampled at the mouth only.

#### Table 10.  Water bodies to by sampled during FY 2001-02 sampling rotation.

| Shed | H2OBody |
| --- | --- |
| 30700 | Carmel River |
| 30800 | Big Creek |
| 30800 | Big Sur River |
| 30800 | Carmel River |
| 30800 | Garapata Creek |
| 30800 | Limekiln Creek |
| 30800 | Little Sur River |
| 30800 | Mill Creek |
| 30800 | San Jose Creek |
| 30800 | Tularcitos Creek |
| 30800 | Willow Creek |
| 31011 | San Carpoforo Creek |
| 31012 | Arroyo de la Cruz Creek |
| 31013 | Pico Creek |
| 31013 | San Simeon Creek |
| 31014 | Santa Rosa Creek |
| 31015 | Villa Creek |
| 31016 | Cayucos Creek |
| 31017 | Old Creek |
| 31017 | Whale Rock Reservoir |
| 31018 | Toro Creek |
| 31020 | Morro Bay |
| 31021 | Morro Creek |
| 31022 | Chorro Creek |
| 31023 | Los Osos Creek |
| 31024 | Avila |
| 31024 | Laguna Lake |
| 31024 | Prefumo Creek |
| 31024 | San Luis Obispo Creek |
| 31024 | Stenner Creek |
| 31025 | Coon Creek |
| 31025 | Port San Luis |
| 31025 | San Luis Obispo Creek |
| 31026 | Pismo Creek |
| 31031 | Arroyo Grande Creek |
| 31031 | Lopez Lake |
| 31031 | Los Berros Creek |

  - Review of available information

The Santa Lucia watershed rotation area is located west of the Santa Lucia mountain ranges in San Luis Obispo and Monterey counties and is characterized by small coastal streams which flow directly to the ocean.  This is the smallest of the five CCAMP rotation areas.  The largest watershed in this 843,000 acre area is the Carmel River, at 164,000 acres.  Because a good portion of this rotation area is along the remote Big Sur coastline, many of the watersheds have little or no disturbance by agricultural or urban activities.  However, there are areas of higher human impact, particularly in the Carmel River watershed and drainages from San Simeon Creek south to Arroyo Grande Creek.  The communities of Carmel, Big Sur, San Simeon, Cambria, Cayucos, Morro Bay, Los Osos, San Luis Obispo, Pismo Beach, Arroyo Grande, and Oceano are found in the area.  Some areas include intensive agricultural cropping activities, particularly in the lower watersheds of the Carmel River, San Simeon Creek, Morro Creek, Chorro Creek, Los Osos Creek, San Luis Obispo Creek, Pismo Creek, and Arroyo Grande Creek.  Steelhead trout is an important resource in most of these creeks, and the southern portion of this area is typically considered the southern extent of their range.

TMDL listings in this areas include Morro Bay (for metals, pathogens, and sedimentation), Chorro Creek (for metals, nutrients, and sedimentation), Los Osos Creek (for nutrients, priority organics, and sedimentation), and San Luis Obispo Creek (for nutrients, pathogens, and priority organics).

The largest body of data available within the Santa Lucia watershed rotation area is from the Morro Bay National Monitoring Program.  This program has been collecting basic water quality and benthic invertebrate data in the Morro Bay watershed since 1993.  Conventional water quality has been collected every two weeks continuously, with sampling increased to a weekly interval for 20 consecutive weeks each winter.  Benthic invertebrate and habitat data have been collected annually in the spring.  Stormwater data has been collected from 10 storms per year from two small subdrainages in the Chorro watershed.  Flow has been collected from several locations.  This database has served as the basis for TMDL development and for modeling activities associated with the Morro Bay National Estuary Program.  This database has been incorporated into the CCAMP database.

Other federal data available in the area include stream survey information collected recently by the National Forest Service as part of their endangered species monitoring activities.  Although we have met with NFS staff to discuss this data, they do not currently have it available in an accessible format.  Data available from the USGS has been downloaded off of the Internet and incorporated into the CCAMP database, however, no stream water quality data was available for this particular area.

Sampling will be conducted at a number of sites during the sampling year.  Sampling will be conducted for both the ongoing Coastal Confluences sampling, and for the Santa Lucia watershed rotation.  Sites to be sampled are indicated in Table 12.  Conventional water quality sampling is conducted once a month, Sites which are listed as part of the Coastal Confluences project are sampled continuously once a month, those which are part of the Santa Lucia watershed rotation will be sampled beginning in January 2002 and ending in March 2003.  Details related to conventional water quality sampling are found in Table 13.  Rapid bioassessment will be conducted from March 15 through April 30th, 2002.  Because all bioassessment sites are sampled for two consecutive springs, Santa Barbara rotation area sites will be sampled for their second round of bioassessment during this spring as well.  Sediment chemistry is only being collected at a subset of the bioassessment sites because of funding constraints, and will be collected synoptically.  State Mussel Watch and Toxic Substances Monitoring Program sampling will be done as per the time frame of these programs; TSM sampling will actually take place during August and September of 2001; SMW sampling will take place during early winter of 2001/2002.

#### Table 11.  Monitoring activities in the Santa Lucia watershed rotation area.

| Program | Monitoring Activity | Status |
| --- | --- | --- |
| Federal |  |  |
| Morro Bay National Monitoring Program | 10 year program. Conventional water quality and benthic inverts | Program reaching completion 2002. Data acquired |
| NOAA Status and Trends | Morro Bay is identified as a site | No recent data available from this site |
| EMAP | California nearshore monitoring in 2003. Currently considering whether to include Morro Bay in estuarine EMAP monitoring program. Inland sampling did not include sites in this area. | Undetermined |
| NOAA Monterey Bay Marine Sanctuary Integrated Monitoring Network (SIMON) | This is an ecological monitoring program which primary coordinates existing research, and initiates new monitoring in Monterey Bay | Program is new and has not yet begun collecting data. CCAMP has coordinated with the program and is considered a primary water quality data gathering program within SIMON |
| State |  |  |
| RWQCB | TMDL monitoring for loading assessments in Chorro, Los Osos, San Luis Obispo Creeks | Data being collected over the next several years. Data in electronic format. |
| State Mussel Watch Program | Endowment monitoring at one site (Point Lobos) | Ongoing. Data acquired. |
| State Mussel Watch Program | Carmel Area WWTP monitoring | Recently terminated due to MRP changes. Data acquired |
| State Mussel Watch Program | PG&E Diablo Canyon nearshore monitoring | Monitoring program to end pending outcome of entrainment and thermal studies. Data acquired |
| State Mussel Watch Program, Univ of California, Santa Barbara | Sand Crab tissue bioaccumulation monitoring | This program will be utilizing the CCAMP data format and will provide data. |
| Department of Health Services | DHS samples Morro Bay as part of the National Shellfish Protection Program for toxic phytoplankton. DHS also samples shellfish | Ongoing |
| California Cooperative Fisheries Investigations | CalCOFI has conducted marine surveys for basic water quality parameters offshore southern and central California for many years | Ongoing, but most data collection is south of the study area. Data is extractable online. |
| California Department of Fish and Game | CDFG has monitoring data from stream surveys and from Marine Protected Areas | CCAMP has provided an electronic format for stream survey data that is currently being used for habitat typing work in the Morro Bay watershed. |

| Local |  |  |
| --- | --- | --- |
| Carmel Area WWTP | Receiving water monitoring program recently incorporated into Central Coast Long-term Environmental Assessment Program (CCLEAN). Includes monitoring at Carmel River mouth. | Data not yet available, but will be electronically reported in CCAMP format as part of CCLEAN requirements. |
| Cambria Sanitary District | Percolation ponds adjacent to San Simeon Creek | Limited monitoring of San Simeon Creek and lagoon. |
| California Men’s Colony WWTP | Discharges directly to Chorro Creek. Monitoring data includes upstream and downstream sites. | Ongoing data. Data has been collected for use in Chorro Creek TMDL development |
| City of Morro Bay | Discharges to Pacific Ocean. Extensive nearshore monitoring as a result of 301(h) waiver. | Data has not been collected electronically |
| Pismo and South County WWTPs | Discharge to nearshore areas. Plants have minimal receiving water monitoring activities | Data has not been collected electronically |
| County of San Luis Obispo AB 411 monitoring | Funds have recently been allocated to begin coastal sampling | Program is new; data acquisition has not yet been coordinated |
| Pacific Gas and Electric | Many years of intensive monitoring of Diablo Cove for impacts associated with the Diablo Canyon Nuclear Power Plant | Data has not been collected electronically |
| University |  |  |
| PISCO – Univ of California, Santa Cruz and Santa Barbara | Intertidal and subtidal monitoring along the California Coast | Ongoing, with a number of stations in the Central Coast area. Data will be available online. |
| California Polytechnic State University | Buoyed sampling device monitoring chlorophyll a, turbidity, and other water quality parameters at Avila Bay in real time | Program currently being installed; data collection to be ongoing. |
| Volunteer |  |  |
| Friends of the Estuary Volunteer Monitoring Program | Ongoing Chorro and Los Osos Creek and Morro Bay water quality sampling, some habitat sampling. | Data currently being entered by volunteers into CCAMP format. New funding has resulted in completion of QA document, better data management |
| San Luis Obispo Land Conservancy | Water quality data on San Luis Obispo creek | Ongoing. Data has not yet been acquired but is readily available |
| Central Coast Salmon Enhancement | New water quality monitoring program developing | Starting up. CCAMP data management system has been provided. |

#### Table 12.  Sampling sites and activities to be conducted for

#### Fiscal Year 2001/2002 monitoring

| Shed | H2OBody | SiteName | Coastal Confluences | SantaLucia | Conventional Water Quality | Rapid Bioassessment | Sediment Chemistry | State Mussel Watch Program | Toxic Substances Monitoring Program |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30411 | Scott Creek | 304SCO-Scott Creek lagoon | 1 |  | 1 | 1 |  |  |  |
| 30411 | Waddell Creek | 304WAD-Waddell Creek lagoon | 1 |  | 1 | 1 |  |  |  |
| 30412 | San Lorenzo River | 304LOR-San Lorenzo Estuary | 1 |  | 1 | 1 |  |  |  |
| 30413 | Aptos Creek | 304APT-Aptos Creek lagoon | 1 |  | 1 | 1 |  |  |  |
| 30413 | Soquel Creek | 304SOQ-Soquel Creek lagoon | 1 |  | 1 | 1 |  |  |  |
| 30420 | Gazos Creek | 304GAZ-Gazos Creek Lagoon | 1 |  | 1 | 1 |  |  |  |
| 30510 | Pajaro River | 305THU-Pajaro at Thurwacher Br. | 1 |  | 1 | 1 |  |  |  |
| 30700 | Carmel River | 307CML-Carmel River | 1 | 1 | 1 | 1 | 1 |  |  |
| 30800 | Big Creek | 308BGC-Big Creek | 1 | 1 | 1 | 1 | 1 |  |  |
| 30800 | Big Creek | 308BIG-Big Creek near mouth |  | 1 | 1 |  |  |  |  |
| 30800 | Big Sur River | 308BSR-Big Sur River | 1 | 1 | 1 | 1 | 1 |  | 1 |
| 30800 | Big Sur River | 308BSU-Big Sur @ Pfeiffer |  | 1 | 1 | 1 |  |  |  |
| 30800 | Carmel River | 308CMU-Carmel River @ Esquiline Rd. |  | 1 | 1 | 1 |  |  |  |
| 30800 | Carmel River | 308CMD-Carmel River @ Robinson Cyn. |  | 1 | 1 | 1 | 1 |  |  |
| 30800 | Carmel River | 308CMN-Carmel River at Nason Rd |  | 1 | 1 | 1 |  |  |  |
| 30800 | Garapata Creek | 308GAR-Garapata Creek near mouth |  | 1 | 1 |  |  |  |  |
| 30800 | Limekiln Creek | 308LIM-Limekiln Creek near mouth |  | 1 | 1 |  |  |  |  |
| 30800 | Little Sur River | 308LSU-Little Sur River near mouth |  | 1 | 1 |  |  |  |  |
| 30800 | Mill Creek | 308MIL-Mill Creek near mouth |  | 1 | 1 |  |  |  |  |
| 30800 | San Jose Creek | 308SJC-San Jose Creek@ Hwy 1 |  | 1 | 1 |  |  |  |  |
| 30800 | Tularcitos Creek | 308TUL-Tularcitos @ Carmel Valley |  | 1 | 1 |  |  |  |  |
| 30800 | Willow Creek | 308WLO-Willow Creek mouth | 1 | 1 | 1 | 1 | 1 |  |  |
| 30910 | Old Salinas River | 309OLD-Old Salinas River Estuary | 1 |  | 1 |  |  |  |  |

| Shed | H2OBody | SiteName | Coastal Confluences | SantaLucia | CWQ | Bioassessment | Sediment Chem | SMW | TSM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30910 | Salinas River (Lower) | 309DAV-Salinas River at Davis Road | 1 |  | 1 | 1 |  |  |  |
| 30910 | Tembladero Slough | 309TDW-Tembladero Slough | 1 |  | 1 |  |  |  |  |
| 31011 | San Carpoforo Creek | 310SCP-San Carpoforo Creek @ hwy 1 |  | 1 | 1 | 1 |  |  |  |
| 31012 | Arroyo de la Cruz Creek | 310ADC-Arroyo de la Cruz lagoon | 1 | 1 | 1 | 1 | 1 |  |  |
| 31012 | Arroyo de la Cruz Creek | 310ADU-Arroyo de la Cruz @ Burnett Creek Rd. |  | 1 | 1 |  |  |  |  |
| 31013 | Pico Creek | 310PCO-Pico Creek Lagoon |  | 1 | 1 | 1 | 1 |  |  |
| 31013 | San Simeon Creek | 310SSC-San Simeon Creek lagoon | 1 | 1 | 1 | 1 | 1 |  | 1 |
| 31013 | San Simeon Creek | 310SSU-San Simeon Creek ds Steiner Creek |  | 1 | 1 | 1 |  |  |  |
| 31014 | Santa Rosa Creek | 310SRO-Santa Rosa Creek lagoon | 1 | 1 | 1 | 1 | 1 |  |  |
| 31014 | Santa Rosa Creek | 310SRU-Santa Rosa Creek @ Main St |  | 1 | 1 | 1 |  |  |  |
| 31015 | Villa Creek | 310VIA-Villa Creek @ Private Rd. Access |  | 1 | 1 |  |  |  |  |
| 31016 | Cayucos Creek | 310CAY-Cayucos Ck @ CayucosCk Rd |  | 1 | 1 |  |  |  |  |
| 31017 | Old Creek | 310OLD-Old Creek @ Old Creek Rd. |  | 1 | 1 |  |  |  |  |
| 31018 | Toro Creek | 310TOR-Toro Ck. @ Toro Ck. Rd. |  | 1 | 1 |  |  |  |  |
| 31020 | Morro Bay | 310428.5-Morro Bay/Virg's |  | 1 |  |  |  | 1* |  |
| 31021 | Morro Creek | 310MOR-Morro Ck @ Quintana |  | 1 | 1 |  |  |  |  |
| 31022 | Chorro Creek | 310TWB-Chorro Creek mouth | 1 | 1 | 1 | 1 | 1 |  |  |
| 31023 | Los Osos Creek | 310SYB-Los Osos Creek mouth |  | 1 | 1 | 1 | 1 |  |  |
| 31024 | Avila | 310438-Avila |  | 1 |  |  |  | 1* |  |
| 31024 | Prefumo Creek | 310PRE-Prefumo @ Calle Joaquin |  | 1 | 1 |  |  |  |  |
| 31024 | San Luis Obispo Creek | 310SLO-San Luis Obispo Creek lagoon | 1 | 1 |  |  |  |  | 1* |
| 31024 | San Luis Obispo Creek | 310SLC-San Luis Obispo @ Cuesta Pk |  | 1 | 1 | 1 |  |  |  |
| 31024 | San Luis Obispo Creek | 310SLV-San Luis Obispo @ Venado Tr |  | 1 | 1 | 1 |  |  |  |
| 31024 | Stenner Creek | 310SCN-Stenner Ck @ Nipomo |  | 1 | 1 |  |  |  |  |
| 31025 | Coon Creek | 310COO - Coon Creek |  | 1 | 1 | 1 |  |  |  |
| 31025 | San Luis Obispo Creek | 310SLB-San Luis Obispo Creek above lagoon | 1 | 1 | 1 | 1 | 1 |  |  |
| 31026 | Pismo Creek | 310PIS-Pismo Creek lagoon | 1 | 1 | 1 | 1 | 1 |  | 1* |
| 31031 | Arroyo Grande Creek | 310ARG-Arroyo Grande Creek lagoon | 1 | 1 | 1 | 1 | 1 |  | 1* |
| 31031 | Arroyo Grande Creek | 310AGB-Arroyo Grande @ Biddle Park |  | 1 | 1 | 1 |  |  |  |
| 31031 | Arroyo Grande Creek | 310AGV-Arroyo Grande @ Bridge St |  | 1 | 1 | 1 | 1 |  |  |
| 31031 | Arroyo Grande Creek | 310AGH-Arroyo Grande @ Huasna Rd. |  | 1 | 1 |  |  |  |  |
| 31031 | Arroyo Grande Creek | 310AGC-Arroyo Grande Ck @ Hi Mntn. |  | 1 | 1 | 1 |  |  |  |
| 31031 | Lopez Lake | 310LOL-Lopez Lake |  | 1 |  |  |  |  | 1* |
| 31031 | Los Berros Creek | 310AGL-Los Berros @ Los Berros Rd. |  | 1 | 1 |  |  |  |  |
| 31210 | Santa Maria River | 312SMA-Santa Maria River at Estuary | 1 |  | 1 | 1 |  |  |  |
| 31300 | San Antonio Creek | 313SAC-San Antonio Creek | 1 |  | 1 |  |  |  |  |
| 31410 | Santa Ynez River | 314SYN-Santa Ynez River Estuary | 1 |  | 1 |  |  |  |  |
| 31510 | Canada de la Gaviota | 315GAV-Canada de la Gaviota | 1 |  | 1 | 1 |  |  |  |
| 31531 | Atascadero Creek(315) | 315ATA-Atascadero Creek | 1 |  | 1 | 1 |  |  |  |
| 31532 | Arroyo Burro Creek | 315ABU-Arroyo Burro | 1 |  | 1 | 1 |  |  |  |
| 31532 | Mission Creek | 315MIS-Mission Creek | 1 |  | 1 | 1 |  |  |  |
| 31534 | Carpinteria Creek | 315CRP-Carpinteria Creek | 1 |  | 1 | 1 |  |  |  |
| 31534 | Rincon Creek | 315RIN-Rincon Creek | 1 |  | 1 | 1 |  |  |  |
| 31534 | Santa Monica Creek | 315SMC-Santa Monica Creek | 1 |  | 1 |  |  |  |  |

#### Table 13.  Specific sampling information associated with monthly conventional water quality sampling

(Methods are those which have been used for CCAMP sampling prior to SWAMP funding.  New methods and detection limits to be determined by the SWAMP Quality Assurance Plan)

| Analyte | Units | MDLs | Sampling Approach | Sample volume/comments/Preservation |
| --- | --- | --- | --- | --- |
| PH | pH units | N/a | DataSonde 4a | Sample taken in sufficent depth to submerge datasonde, preferably at approximately 0.6 of total depth, in thalweg or at least away from bank in areas of higher flow velocities |
| Conductivity | US/cm | 1.0 | DataSonde 4a | “ |
| Turbidity | NTU | 0.1 | DataSonde 4a | “ |
| Dissolved Oxygen | Ppm | 0.01 | DataSonde 4a | “ |
| Oxygen Saturation | % Saturation | n/a | DataSonde 4a | “ |
| Water Temperature | Celsius | n/a | DataSonde 4a | “ |
| Air Temperature | Celsius | n/a | Thermometer (oC) | Taken in shade at least two feet from ground surface. |
| Total Coliform Bacteria | MPN/100 ml | 2 | 25-tube dilution | Sealed, sterile, 100-ml plastic bacteria containers with Na2S2O3 preservative. Kept at 4oC. 6 hour holding time desirable; 24 hour required. |
| Fecal Coliform Bacteria | MPN/100 ml | 2 | 25-tube dilution | “ |
| Nitrate-N | mg/l | 0.02 | EPA 300.0 | 1-liter plastic bottle. Kept at 4oC. 48 hour holding time. |
| Nitrite-N | mg/l | 0.01 | EPA 353.2 | “ |
| Ortho Phosphate | mg/l | 0.01 | EPA 365.1 | “ |
| Total Kjeldahl Nitrogen | mg/l | 0.1 | EPA 351.2 | 1-liter plastic bottle. Kept at 4oC. 28 day holding time. |
| Total Ammonia-NH3 | mg/l | 0.02 | EPA 350.1 | “ |
| Total Phosphate | mg/l | 0.06 | EPA 365.4 | “ |
| Chlorophyll a | ug/l | 0.1 | DataSonde 4a | 500 ml plastic amber bottle. Kept in dark at 4oC |
| Total Suspended Solids | mg/l | 0.5 | EPA 160.2 | 1-liter plastic bottle. Kept at 4oC. 7 day holding time. |
| Fixed Suspended Solids | mg/l | 0.5 | EPA 160.2 | “ |
| Total Dissolved Solids | mg/l | 4.0 | EPA 160.1 | “ |
| Fixed Dissolved Solids | mg/l | 4.0 | EPA 160.4 | “ |
| Volatile Dissolved Solids | mg/l | 5.0 | EPA 160.4 | “ |
| Volatile Suspended Solids | mg/l | 0.5 | EPA 160.4 | “ |
| CaCO3 | mg/l | 5 | SM-2340B | 1-liter plastic bottle. Kept at 4oC. 28 day holding time. |
| Chloride | mg/l | 0.06 | EPA 300.0 | “ |
| Boron | mg/l | 0.05 | EPA 6010 | “ |
| Calcium | mg/l | 0.01 | EPA 6010 | “ |
| Magnesium | mg/l | 0.02 | EPA 6010 | “ |
| Sodium | mg/l | 0.06 | EPA 6010 | “ |
| Corridor Shading | % | n/a | Visual estimate | Percent of flowing stream channel that is shaded |
| Algal Cover | % | n/a | Visual estimate | Percent of flowing stream channel that is covered with algae (periphyton or filamentous) |
| Instream Plant Cover | % | n/a | Visual estimate | Percent of flowing stream channel that is covered with rooted vegetation |
| Flow | Cubic Feet per Second | n/a | Pygmy gurley or Cross-sectional profile with stage height, 3-point calibration | Flow measurements taken each month. For sites estimated using cross sectional profile, transects are surveyed each spring. At least 3 flow measurements are collected at low, medium and high flows to calibrate curve. Each month height is measured from fixed point to water surface to estimate water surface height and flow. |

## Laboratory Analysis

Laboratory analytical work to be performed is as described in Section 3.1.1.  Because the CCAMP program is striving to develop a broad ambient picture of water quality conditions in the Region, consistency is applied in the suite of analytical parameters collected from year to year and site to site.

## Data quality evaluation and data reporting

CCAMP data quality considerations are (or will be) described in the SWAMP QAPP and Region 3 QAPP/SOP.  The maximum variation that is acceptable within the limits of quality assurance is discussed in these documents.  Any variation greater than these limits is brought to the attention of the project manager who will evaluate data, calibration and maintenance records for trends or possible outliers. The project manager will decide if any results will be discarded due to instrument or laboratory error.

Field equipment is calibrated prior to each sampling day according to CCAMP Standard Operating Procedures.  Post calibration checks of field equipment are conducted at the contract laboratory upon return from the field.  A blank sample is tested using field probes and subsequently tested for pH, conductivity, turbidity and salinity using the facilities at the commercial laboratory.  Measurements should vary less than 10%. Instrument maintenance or replacement will occur if the standard variation requirement is not met following basic maintenance procedures.

Duplicate samples and split samples are collected and analyzed to insure performance and validity of laboratory data.  Duplicate samples measure both field and laboratory variability.  Split samples are more directly a measure of laboratory performance.  Duplicate field samples are collected at 5% of all sites sampled.  5% of all conventional water samples are also collected as field splits to measure performance of the contract laboratory.  Duplicate and split data is evaluated for percent difference, and where this exceeds acceptable performance standards (see QAPP) appropriate action is taken depending upon the parameter.  A quality assurance report will be included with each annual report and hardcopy data reports.

To the extent possible data is entered into the CCAMP database utilizing electronic transfer protocols to reduce the incidence of data entry errors.  All commercial laboratory data is electronically uptaken, and information which is stored (for each data element) includes probable quantification limit, minimum detection limit or reporting limit, units of measurement, and method.  Data from multi-analyte probes is both collected in hard copy and downloaded and electronically uptaken into the CCAMP database.  The hard copy is compared with the electronic download to ensure that site tags are correctly associated with data.  The only data which is entered by hand are field parameters like air temperature, % algal cover, and notes.

Data is screened for obvious errors both by graphing data and by screening for exceedance of user-defined minimum and maximum values.  All data is checked against hard copy lab data sheets for errors.

The CCAMP database is currently being adapted as the prototype data entry system for SWAMP.

## Deliverable products

Hardcopy Data Reports

Commercial Laboratory Contracts

Annual Task Order

Master Contractor Data Report

QA/QC Summary Report

Draft Annual Report

Final Annual Report

QA/QC and SOPs

  - Desired milestone schedule for the Santa Lucia water rotation year (2002)

| Start of conventional water quality sampling | January 1 2002 |
| --- | --- |
| Benthic Invertebrate sampling for Santa Barbara and Santa Lucia watershed rotations | March 15, 2002 – May 1, 2002 |
| Pre-dawn DO measurements | July, August, September |
| Calibration of flow curves | January, March, June (at high, medium, low flow events) |
| Sediment Sampling | Preferably synoptic with benthic invertebrate sampling, but may be dictated by fund availability at end of fiscal year (June, 2002) |
| Santa Lucia hardcopy data report (draft) | March 2003 |
| Santa Lucia data delivery (final) | March 2004 (depends on delivery schedule from Master Contractor) |
| Santa Lucia Report delivery (draft) | July 1, 2003 |
| Santa Lucia Report delivery (final) | July 1, 2004 (depends on data delivery schedule from Master Contractor) |

## Desired “sample throughput schedule”

| Conventional Water Quality data (comm. lab) | 30 days from time of collection |
| --- | --- |
| Benthic Invertebrate data (Master Contract) | 6 months from time of delivery |
| Fish and Mussel tissue data (SMW, TSM) | 9 months from time of collection |
| Sediment Chemistry data (Master Contract) | 9 months from time of collection |
| Sediment Chemistry data (commercial lab) | 2 months from time of collection |
| Toxicity data (Master Contract) | 4 months from time of collection |

## Budget

The Region 3 allotment from the SWAMP program for FY 2001/2002 is $289,787.  This is an approximate reduction of $20,000 from the previous fiscal year.  Other funding sources applied toward monitoring activities include State Mussel Watch, State Mussel Watch endowment, and CCAMP endowment.  Budget allocations to different elements of the program are shown.  Because funds are provided on a fiscal year basis and sampling rotation years are managed on a calendar year basis, the budget reflects completion of the 2000/2001 sampling year in the Santa Barbara watershed rotation area and initiation of the sampling year in the Santa Lucia watershed rotation area.  Table 14 shows the CCAMP budget for FY 2001/2002, including all currently available funding sources.

#### Table 14.  CCAMP 2001/2002 Budget

| CCamp Budget 01/02 | Sites | Quality Assurance Samples per sampling event | $/Sample | Sampling Events/ fiscal year | Total Samples (inlcuing Quality Assurance samples) | Total Cost | CCAMP Endowment | State Mussel Watch | SWAMP funds | RWQCB General Lab Budget | State Mussel Watch Endowment | SWRCB Fish Contamination | SWRCB Toxicity | San Jose State Foundation | SUBTOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SantaBarb Watershed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Conventional Water Quality | 33 | 3 | 192 | 9 | 324 | 62,208 |  |  | 60,008 |  |  |  |  |  | 62,208 |
| Rapid Bioassessment | 13 |  | 756 | 1 | 13 | 9,828 |  |  | 9,828 |  |  |  |  |  | 9,828 |
| Santa Lucia Watershed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Conventional Water Quality | 33 | 3 | 192 | 6 | 198 | 38,016 |  |  | 38,016 | 2,200 |  |  |  |  | 38,016 |
| Bioaccumulation(bivalves) SMW + PAHs | 4 |  | 4,017 | 1 | 4 | 16,068 |  | 16,068 |  |  |  |  |  |  |  |
| Bioaccumulation(fish) TSM | 8 |  | 3,156 | 1 | 8 | 25,248 |  | 25,248 |  |  |  |  |  |  | 25,248 |
| Bioaccumulation(fish) TSM + PAHs | 5 |  | 3,755 | 1 | 5 | 18,775 |  | 18,775 |  |  |  |  |  |  |  |
| Rapid Bioassessment | 15 |  | 756 | 1 | 15 | 11,340 |  |  | 11,340 |  |  |  |  |  | 11,340 |
| Watershed Characterization subtotal |  |  |  |  |  | $181,483 | $0 | $60,091 | $119,192 | $2,200 | $0 | $0 | $0 | $0 | $181,483 |
| Coastal Confluences |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Conventional Water Quality | 34 | 3 | 192 | 12 | 408 | 78,336 |  |  | 78,336 |  |  |  |  |  | 78,336 |
| Rapid Bioassessment | 26 |  | 756 | 1 | 26 | 19,656 |  |  | 19,656 |  |  |  |  |  | 19,656 |
| Coastal Confluences subtotal |  |  |  |  |  | $97,992 | $0 | $0 | $97,992 | $0 | $0 | $0 | $0 | $0 | $97,992 |
| Nearshore |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bioaccumulation SWRCB BCP | 10 |  | 5,000 | 1 | 10 | 50,000 |  |  |  |  |  | 50,000 |  |  | 50,000 |
| Bioaccumulation Mussel Watch Endowment | 3 |  | 3,418 | 1 | 3 | 10,254 |  |  |  |  | 10,254 |  |  |  | 10,254 |
| Bioaccumulation Sea Otters | 50 | 2 | 600 | 1 | 50 | 30,000 | 30,000 |  |  |  |  |  |  |  | 30,000 |
| Nearshore Subtotal |  |  |  |  |  | $90,254 | $30,000 | $0 | $0 | $0 | $10,254 | $50,000 | $0 | $0 | $90,254 |
| CCamp Budget 01/02 | Sites |  | $/Sample | Samples/yr | Total Samples | Total | CCAMP Endowment | State Mussel Watch | SWAMP funds | RWQCB General Lab Budget | State Mussel Watch Endowment | SWRCB Fish Contamination | SWRCB Toxicity | San Jose State Foundation | SUBTOTAL |
| Special Studies |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Avila Beach PAH study |  |  |  |  |  | 50,000 |  |  |  |  |  |  |  | 50,000 | 50,000 |
| Toxicity triad study - Water Chemistry |  |  |  |  |  | 20,500 | 8,000 |  | 12,500 |  |  |  |  |  | 20,500 |
| Toxicity triad study - Toxicity, Benthics |  |  |  |  |  | 86,000 |  |  |  |  |  |  | 86,000 |  |  |
| Special Studies Subtotal |  |  |  |  |  | $156,500 | $8,000 | $0 | $12,500 | $0 | $0 | $0 |  | $50,000 | $70,500 |
| Hardware |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Misc. |  |  |  |  |  | 5,000 | 5,000 |  |  |  |  |  |  |  | 5,000 |
| Garmin 175 GPS |  |  |  |  |  | 2,000 | 2,000 |  |  |  |  |  |  |  | 2,000 |
| Hardware Subtotal |  |  |  |  |  | $7,000 | $7,000 |  |  |  |  |  |  |  | $7,000 |
| Support Staff |  |  | $/Hr | Hrs/Yr | Total |  |  |  |  |  |  |  |  |  |  |
| Field team leader | 1 |  | 15 | 1,300 | 19,500 | 20,000 | 20,000 |  |  |  |  |  |  |  | 20,000 |
| Students | 4 |  | 11 | 1,300 | 15,000 | 60,000 |  |  | 60,000 |  |  |  |  |  | 60,000 |
| Data management | 1 |  |  |  | 25,000 | 25,000 | 25,000 |  |  |  |  |  |  |  | 25,000 |
| MBNMS Volunteer Monitoring Support | 1 |  |  |  | 10,000 | 10,000 | 10,000 |  |  |  |  |  |  |  | 10,000 |
| Support Staff Subtotal |  |  |  |  |  | $115,000 | $55,000 | $0 | $60,000 | $0 | $0 | $0 | $0 | $0 | $115,000 |
| CCAMP TOTAL EXPENDITURES |  |  |  |  |  | $648,229 | $100,000 | $60,091 | $289,684 | $2,200 | $10,254 | $50,000 | 86,000 | $50,000 | $648,229 |

## Working Relationships

#### Table 15.  Working relationships between State Water Resources Control Board staff, Regional Water Quality Control Board staff and Contractors.

| Task | Responsible Organization |  |  |
| --- | --- | --- | --- |
|  | SWRCB | RWQCBs | Contractors |
| Develop contract(s) for monitoring services. |  |  |  |
| Identify water bodies or sites of concern and clean sites to be monitored. |  |  |  |
| Identify site-specific locations with potential beneficial use impacts or unimpacted conditions that will be monitored. |  |  |  |
| Decide if concern is related to objectives focused on location or trends of impacts. |  |  |  |
| Select monitoring objective(s) based on potential beneficial use impact(s) or need to identify baseline conditions. |  |  |  |
| Identify already-completed monitoring and research efforts focused on potential problem, monitoring objective, or clean conditions. |  |  |  |
| Make decision on adequacy of available information. |  |  |  |
| Prepare site-specific study design based on monitoring objectives, the assessment of available information, sampling design, and indicators. |  (Work Plan Review Role) |  |  |
| Implement study design. (Collect and analyze samples.) |  |  |  |
| Track study progress. Review quality assurance information and make assessments on data quality. Adapt study as needed. |  (Review Role) |  |  |
| Report data through SWRCB web site. |  |  (Coordination Role) |  |
| Prepare written report of data. |  |  |  |

## Other information and list of attachments

Attachment 1 – SWAMP Program Objectives

Attachment 2 – CCAMP Data Management

Attachment 1.  SWAMP Program Objectives

From the November, 2000 Report to the Legislature, “Proposal for a Comprehensive Ambient Surface Water Quality Monitoring Program”

Monitoring Objectives

In developing the SWAMP monitoring objectives, the SWRCB used a modified version

of the model proposed by Bernstein et al. (1993) for developing clear monitoring

objectives. The model makes explicit the assumptions and/or expectations that are often

embedded in less detailed statements of objectives such as those presented in the

SWRCB Report to the Legislature on comprehensive monitoring submitted in February

2000 (SWRCB, 2000). This section is organized by each major question posed in the

January 2000 report.

Is it safe to swim?

Beneficial Use: Water Contact Recreation

1. Throughout water bodies that are used for swimming, estimate the concentration

of pathogenic contaminants above and below screening values, health standards,

or adopted water quality objectives.

2. Estimate the percent of beach area that poses potential health risks of exposure to

pathogens in streams, rivers, lakes, nearshore waters, enclosed bays, and estuaries

using several critical threshold values of potential human impact (pathogen

indicators).

3. Throughout water bodies that are used for swimming, estimate the concentration

of bacterial contaminants from month-to-month above and below screening

values, health standards, or adopted water quality objectives.

Is it safe to drink the water?

Beneficial Use: Municipal and Domestic Water Supply

4. Throughout water bodies, estimate the area of lakes, rivers, and streams that are

sources of drinking water where the concentration of microbial or chemical

contaminants are above and below screening values, drinking water standards, or

adopted water quality objectives used to protect drinking water quality.

5. Throughout water bodies that are used as a source of drinking water, estimate the

concentration of microbial or chemical contaminants from month-to-month above

and below screening values, drinking water standards, or adopted water quality

objectives used to protect drinking water quality.

Is it safe to eat fish and other aquatic resources?

Beneficial Uses: Commercial and Sport Fishing, Shellfish Harvesting

6. Estimate the area of streams, rivers, lakes, nearshore waters, enclosed bays, and

estuaries where the concentration of chemical contaminants in edible fish or

shellfish tissue exceeds several critical threshold values of potential human impact

(screening values or action levels).

7. Assess the geographic extent of chemical contaminants in selected size classes of

commonly consumed target species that exceed several critical threshold values of

potential human impact (screening values or action levels) (Adapted from

USEPA, 1995).

8. Throughout water bodies (streams, rivers, lakes, nearshore waters, enclosed bays,

and estuaries), estimate the concentration of chemical contaminants in fish and

aquatic resources from year to year using several critical threshold values of

potential human impact (advisory or action levels).

9. Throughout water bodies that are used for shellfish harvesting, estimate the

concentration of bacterial contaminants from month to month above and below

health standards or adopted water quality objectives.

10. Throughout water bodies that are used for shellfish harvesting, estimate the

concentration of bacterial contaminants above and below health standards or

adopted water quality objectives.

Are aquatic populations, communities, and habitats protected?

Beneficial Uses: Cold Freshwater Habitat; Estuarine Habitat; Inland Saline

Water Habitats; Marine Habitat; Preservation of Biological Habitats; Rare,

Threatened or Endangered Species; Warm Freshwater Habitat; Wildlife

Habitat

11. Estimate the percent of degraded water area in lakes, nearshore waters, enclosed

bays, and estuaries using several critical threshold values of toxicity, water or

benthic community analysis, habitat condition, and chemical concentration.

12. Estimate the percent of degraded sediment area in rivers, lakes, nearshore waters,

enclosed bays, and estuaries using several critical threshold values of toxicity,

benthic community analysis, habitat condition, and chemical concentration.

13. Identify the areal extent of degraded sediment locations in rivers, lakes, nearshore

waters, enclosed bays, and estuaries using several critical threshold values of

toxicity, benthic community analysis, habitat condition, and chemical

concentration.

14. Estimate the percent of degraded sediment area from year to year in rivers, lakes,

nearshore waters, enclosed bays, and estuaries using several critical threshold

values of toxicity, benthic community analysis, habitat condition, and chemical

concentration.

15. Estimate the percent of degraded water area from year to year in rivers, lakes,

nearshore waters, enclosed bays, and estuaries using several critical threshold

values of toxicity, water column or benthic community analysis, habitat condition,

and chemical concentration.

Beneficial Use: Spawning, Reproduction and/or Early Development

16. Estimate the area of degraded spawning locations and water or sediment toxicity

associated with toxic pollutants in rivers, lakes, nearshore waters, enclosed bays,

and estuaries using critical threshold values of early life-stage toxicity, chemical

concentration, and physical characteristics

17. Estimate the area degraded spawning locations and water or sediment toxicity

associated with toxic pollutants from year to year in rivers, lakes, nearshore

waters, enclosed bays, and estuaries using critical threshold values of early life-stage

toxicity, chemical concentration, and physical characteristics.

Is water flow sufficient to protect fisheries?

Beneficial Use: Migration of Aquatic Organisms; Rare, Threatened or

Endangered Species; Wildlife Habitat

18. Throughout water bodies, estimate the area with the conditions necessary for the

migration of aquatic organisms, such as anadromous fish, using measures of

habitat condition including water flow, watercourse geomorphology,

sedimentation, temperature, and biological communities.

19. Throughout water bodies, estimate the area with the conditions from month to

month necessary for the migration of aquatic organisms, such as anadromous fish,

using measures of habitat condition including water flow, watercourse

geomorphology, sedimentation, temperature, and biological communities.

Is water safe for agricultural use?

Beneficial Use: Agricultural supply

20. Throughout water bodies, estimate the area of lakes, rivers and streams that are

used for agricultural purposes where the concentration of chemical pollutants are

above or below screening values or adopted water quality objectives used to

protect agricultural uses.

21. Throughout waterbodies that are used for agricultural purposes, estimate the

concentration of chemical pollutants from year-to-year above or below screening

values or adopted water quality objectives used to protect agricultural uses.

Is water safe for industrial use?

Beneficial Use: Industrial Process Supply; Industrial Service Supply

22. Throughout water bodies, estimate the area of coastal waters, enclosed bays,

estuaries, lakes, rivers and streams that are used for industrial purposes where the

concentration of chemical pollutants are above or below screening values or

adopted water quality objectives used to protect industrial uses.

23. Throughout water bodies that are used for industrial purposes, estimate the

concentration of chemical pollutants from year to year above or below screening

values or adopted water quality objectives used to protect industrial uses.

Are aesthetic conditions of the water protected?

Beneficial Use: Non-Contact Water Recreation

24. Throughout water bodies, estimate the area of coastal waters, enclosed bays,

estuaries, lakes, rivers and streams where the aesthetic conditions are above or

below screening values or adopted water quality objectives used to protect non-contact

water recreation.

25. Throughout water bodies, estimate the aesthetic condition from year-to-year

above or below screening values or adopted water quality objectives used to

protect non-contact water recreation.

The number refers to the monitoring objective discussed in Attachment 1, SWAMP site-specific monitoring objectives.