California Regional Water Quality Control Board, Los Angeles Region

#### Tissue, Sediment and Benthic Infauna Data

Conejo Creek R1 (confl Call to Santa Rosa Rd) and R2 (Santa Rosa Rd to Thousand Oaks city limit) and R3 (Thousand Oaks city limit to Lynn Rd) and R4 (abv Lynn Rd)

# Summary of Proposed Action

## Proposed New Listings

- “Not Supporting” (Impaired) for chlordane in tissue due to exceedances of Maximum Tissue Residue Levels (MTRLs) in new Calleguas CreekReach 13 only.

- “Not Supporting” (Impaired) for dieldrin in tissue due to exceedances of MTRLs in new Calleguas Creek Reach 13 only.

- “Not Supporting” (Impaired) for HCH in tissue due to exceedances of MTRLs in new Calleguas Creek Reach 13 only.

- “Not Supporting” (Impaired) for PCBs in tissue due to exceedances of MTRLs in new Calleguas Creek Reach 13 only.

## Proposed New Delistings

- Delist dacthal in tissue in old Calleguas Creek Reaches 1, 2, 3 and 4 because the listings were based on Elevated Data Levels (EDLs) which no longer represent valid assessment guidelines.

- Delist silver in tissue in old Calleguas Creek Reaches 1, 2 and 3 because the listings were based on EDLs which no longer represent valid assessment guidelines.

- Delist cadmium in tissue in old Calleguas Creek Reaches 1, 2 and 3 because the listings were based on EDLs which no longer represent valid assessment guidelines.

- Delist chromium in tissue in old Calleguas Creek Reaches 1, 2 and 3 because the listings were based on EDLs which no longer represent valid assessment guidelines.

- Delist nickel in tissue in old Calleguas Creek Reaches 1, 2 and 3 because the listings were based on EDLs which no longer represent valid assessment guidelines.

These actions all affect the downstream commercial and sport fishing beneficial uses.

## Table 1. 303(d) Listing/TMDL Information

| ## Waterbody Name | Conejo Creek, a tributary to Calleguas Creek | Pollutants/Stressors | Add: Chlordane (Tissue) [R1]; Dieldrin (Tissue) [R1]; HCH (Tissue) [R1]; PCB (Tissue) [R1] Delete: Dacthal (Tissue) [R1, R2, R3, R4]; Silver (Tissue) [R1, R2, R3]; Cadmium (Tissue) [R1, R2, R3]; Chromium (Tissue) [R1, R2, R3]; Nickel (Tissue) [R1, R2, R3]; |
| --- | --- | --- | --- |
| Hydrologic Unit | 403.64 | Source(s) | Historic use of pesticides and lubricants. |
| Total Waterbody Size | 6.51 | TMDL Priority | Chlordane, Dieldrin, HCH, PCB, Chem A, Dacthal: 5 Silver, Cadmium, Chromium, Nickel: 6 |
| Size Affected | Reach 1 only | TMDL Start Date (Mo/Yr) | 2002 |
| Extent of Impairment | R1 | TMDL End Date (Mo/Yr) | 2005 |

# Watershed Characteristics

{Insert text describing the watershed – could be taken from WMI chapter or draft TMDL documents}

Calleguas Creek and its major tributaries, Revolon Slough, Conejo Creek, Arroyo Conejo, Arroyo Santa Rosa, and Arroyo Simi drain an area of 343 square miles in southern Ventura County and a small portion of western Los Angeles County.   This watershed, which is elongated along an east-west axis, is about 30 miles long and 14 miles wide.   The northern boundary of the watershed is formed by the Santa Susana Mountains, South Mountain, and Oak Ridge; the southern boundary is formed by the Simi Hills and Santa Monica Mountains.

Land uses vary throughout the watershed.  Urban developments are generally restricted to the city limits of Simi Valley, Moorpark, Thousand Oaks, and Camarillo.  Although some residential development has occurred along the slopes of the watershed, most upland areas are still open space, however, golf courses are becoming increasingly popular to locate in these open areas.  Agricultural activities, primarily cultivation of orchards and row crops, are spread out along valleys and on the Oxnard Plain.

Mugu Lagoon, located at the mouth of the watershed, is one of the few remaining significant saltwater wetland habitats in southern California.  The Point Mugu Naval Air Base is located in the immediate area and the surrounding Oxnard Plain supports a large variety of agricultural crops.  These fields drain into ditches which either enter the lagoon directly or through Calleguas Creek and its tributaries.  Other fields drain into tile drain systems which discharge to drains or creeks.  Also in the area of the base are freshwater wetlands created on a seasonal basis to support duck hunting clubs.  The lagoon borders on an Area of Special Biological Significance (ASBS) and supports a great diversity of wildlife including several endangered birds and one endangered plant species.  Except for the military base, the lagoon area is relatively undeveloped.

# Water Quality Objectives Not Attained

Tissue MTRLs

EDLs have been determined to be an insufficient basis for impairment determination.

# Beneficial Uses Affected

Downstream commercial and sport fishing beneficial uses.

# Data Assessment

Tissue (98):  chlordane, DDT, dieldrin, HCH, PCB, toxaphene (MTRL)

## Table 2. Summary of Tissue Data for Conejo Creek, a tributary to Calleguas Creek

| Dates of Sampling | 6/25/98 |
| --- | --- |
| Number of Samples (n) | 2 (fish tissue) |
| Minimum Data Value | Total chlordane: 39.7 ppb p,p’-DDD: 34.6 ppb p,p’-DDE: 844 ppb p,p’-DDT: 94 ppb dieldrin: 16.5 ppb gamma-HCH: 4.0 ppb total PCB: 20.3 ppb toxaphene: 819 ppb |
| Maximum Data Value | Total chlordane: 42.1 ppb p,p’-DDD: 33.9 ppb p,p’-DDE: 932 ppb p,p’-DDT: 100 ppb dieldrin: 17.2 ppb gamma-HCH: 4.0 ppb total PCB: 22.0 ppb toxaphene: 874 ppb |
| Median Data Value |  |
| Arithmetic Mean Value |  |
| Standard Deviation |  |
| Number (Percent) above Objective | Chlordane: 2 (100 %) DDTs: 4 (67 %) Dieldrin: 2 (100 %) HCH: 2 (100 %) PCB: 2 (100 %) Toxaphene: 2 (100 %) |

This table may summarize additional data not relevant to this factsheet that supports a continued listing for this waterbody.

# Potential Sources

Historical use of pesticides and lubricants.

# References

Toxic Substances Monitoring Program database