California Regional Water Quality Control Board, Los Angeles Region

Tissue, Sediment and Benthic Infauna Data

Coyote Creek

# Summary of Proposed Action

Proposed New Delistings

- Delist silver in tissue because the listing was based on Elevated Data Levels (EDLs) which no longer represent valid assessment guidelines.

This action affects the aquatic life beneficial uses.

Delist copper, selenium, silver and zinc for Aquatic Life (partially supporting designation).  These were based on EDLs, which do not represent valid assessment guidelines.

List chlordane and HCH for Aquatic Life.  Based on 1998 TSM sampling, tissue concentrations in arroyo chub for total chlordane (8.4 ppb) and gamma-HCH (5.8 ppb) exceeded MTRLs at the Ventura River station just downstream of the Ojai Valley Sanitation District’s discharge point.  In 1993 TSM sampling, arroyo chub tissue concentrations for total chlordane (23.8 ppb) and gamma-HCH also exceeded MTRLs.

## Table 1. 303(d) Listing/TMDL Information

| ## Waterbody Name | Coyote Creek | Pollutants/Stressors | Delete: Ag (Tissue) |
| --- | --- | --- | --- |
| Hydrologic Unit | 405.15 | Source(s) | Historical use of pesticides |
| Total Waterbody Size | 13.45 | TMDL Priority |  |
| Size Affected | 13.45 | TMDL Start Date (Mo/Yr) |  |
| Extent of Impairment | Entire reach. | TMDL End Date (Mo/Yr) |  |
| Upstream Extent Latitude |  | Upstream Extent Longitude |  |
| Downstream Extent Latitude |  | Downstream Extent Longitude |  |

# Watershed Characteristics

The San Gabriel River receives drainage from a large area of eastern Los Angeles County; its headwaters originate in the San Gabriel Mountains.  The watershed consists of extensive areas of undisturbed riparian and woodland habitats in its upper reaches.  Much of the watershed of the West Fork and East Fork of the river is set aside as a wilderness area; other areas in the upper watershed are subject to heavy recreational use.  The upper watershed also contains a series of flood control dams.  Further downstream, towards the middle of the watershed, are large spreading grounds utilized for groundwater recharge.  The watershed is hydraulically connected to the Los Angeles River through the Whittier Narrows Reservoir (normally only during high storm flows).   The lower part of the river flows through a concrete-lined channel in a heavily urbanized portion of the county before becoming an soft bottom channel once again near the ocean in the city of Long Beach.  Large electrical power poles line the river along the channelized portion and nurseries, small stable areas, and a large poultry farm are located in these areas.

{Insert text describing the watershed – could be taken from WMI chapter or draft TMDL documents}

# Water Quality Objectives Not Attained

Tissue MTRLs

# Beneficial Uses Affected

Aquatic Life

# Data Assessment

Tissue (97):  metals and organics were low, except for gamma-HCH, which exceeded MTRL.

## Table 2. Summary of Tissue Data for Coyote Creek

| Dates of Sampling | 7/18/97 |
| --- | --- |
| Number of Samples (n) | 1 (fish tissue) |
| Minimum Data Value |  |
| Maximum Data Value | Gamma-HCH: 6.5 ppb |
| Median Data Value |  |
| Arithmetic Mean Value |  |
| Standard Deviation |  |
| Number (Percent) above Objective |  |

# Potential Sources

Historical use of pesticides.

# References

Toxic Substances Monitoring Program database.