California Regional Water Quality Control Board, Los Angeles Region

Tissue, Sediment and Benthic Infauna Data

Lake Lindero

# Summary of Proposed Action

Proposed New Delistings

- Delist selenium in tissue because the listing was based on Median International Standards (MIS) for trace elements which are outdated and no longer represent valid assessment guidelines.

This action affects the aquatic life beneficial uses.

Delist copper, selenium, silver and zinc for Aquatic Life (partially supporting designation).  These were based on EDLs, which do not represent valid assessment guidelines.

List chlordane and HCH for Aquatic Life.  Based on 1998 TSM sampling, tissue concentrations in arroyo chub for total chlordane (8.4 ppb) and gamma-HCH (5.8 ppb) exceeded MTRLs at the Ventura River station just downstream of the Ojai Valley Sanitation District’s discharge point.  In 1993 TSM sampling, arroyo chub tissue concentrations for total chlordane (23.8 ppb) and gamma-HCH also exceeded MTRLs.

## Table 1. 303(d) Listing/TMDL Information

| ## Waterbody Name | Lake Lindero | Pollutants/Stressors | Delete: Se (Tissue) |
| --- | --- | --- | --- |
| Hydrologic Unit | 404.23 | Source(s) | Historical use of pesticides |
| Total Waterbody Size | 13.56 | TMDL Priority |  |
| Size Affected | 13.56 | TMDL Start Date (Mo/Yr) |  |
| Extent of Impairment | Entire lake | TMDL End Date (Mo/Yr) |  |
| Upstream Extent Latitude |  | Upstream Extent Longitude |  |
| Downstream Extent Latitude |  | Downstream Extent Longitude |  |

# Watershed Characteristics

{Insert text describing the watershed – could be taken from WMI chapter or draft TMDL documents}

The Santa Monica Bay Watershed Management Area (WMA), which encompasses an area of 414 square miles, is quite diverse.  Its borders reach from the crest of the Santa Monica Mountains on the north and from the Ventura-Los Angeles County line to downtown Los Angeles.  From there it extends south and west across the Los Angeles plain to include the area east of Ballona Creek and north of the Baldwin Hills.  South of Ballona Creek the natural drainage area is a narrow strip of wetlands between Playa del Rey and Palos Verdes.  The WMA includes several watersheds the two largest being Malibu Creek to the north and Ballona Creek to the south.  While the Malibu Creek area contains mostly undeveloped mountain areas, large acreage residential properties and many natural stream reaches; Ballona Creek is predominantly channelized, and highly developed with both residential and commercial properties.

# Water Quality Objectives Not Attained

Tissue MTRLs

# Beneficial Uses Affected

Aquatic Life

# Data Assessment

Tissue (98): chlordane, toxaphene

## Table 2. Summary of Tissue Data for Lake Lindero

| Dates of Sampling | 6/24/98 |
| --- | --- |
| Number of Samples (n) | 1 (fish tissue) |
| Minimum Data Value |  |
| Maximum Data Value | Total chlordane: 15.1 ppb Toxaphene: 26.2 ppb |
| Median Data Value |  |
| Arithmetic Mean Value |  |
| Standard Deviation |  |
| Number (Percent) above Objective | Chlordane: 1 (100 %) Toxaphene: 1 (100 %) |

# Potential Sources

Historical use of pesticides.

# References

Toxic Substances Monitoring Program database.