California Regional Water Quality Control Board, Los Angeles Region

Santa Clara River Reach 3 (Freeman Diversion to Fillmore Street A)

Nitrite and Nitrate as Nitrogen, Nitrite as Nitrogen, Total Dissolved Solids

# Summary of Proposed Action

Listing is proposed for Reach 3 (Freeman Diversion to Fillmore Street A) on the Santa Clara River for nutrients and their effects, and Total Dissolved Solids, which affect agriculture and municipal drinking supplies.  This Reach will be listed as “Partially Supporting (Impaired)” for agriculture and “Partially Supporting (Impaired)” for groundwater recharge.

## Table 1. 303(d) Listing/TMDL Information

| ## Waterbody Name | Reach 3 (Below Fillmore at Santa Paula) | Pollutants/Stressors | Nitrite and Nitrate as Nitrogen, Nitrite as Nitrogen, Total Dissolved Solids |
| --- | --- | --- | --- |
| Hydrologic Unit | 403.21 & 403.31 | Source(s) | non point and point sources |
| Total Waterbody Size |  | TMDL Priority | Nutrient: TMDL Analytical Unit 32 TDS: low |
| Size Affected | 13.24 | TMDL Start Date (Mo/Yr) | Nutrients: July 2001 TDS 2012 |
| Extent of Impairment | Entire Reach | TMDL End Date (Mo/Yr) | Nutrients: March 2003 TDS 2014 |

# Watershed Characteristics

The Santa Clara River is the largest river system in southern California that remains in a relatively natural state; this is a high quality natural resource for much of its length.  The river originates in the northern slope of the San Gabriel Mountains in Los Angeles County, traverses Ventura County, and flows into the Pacific Ocean halfway between the cities of San Buenaventura and Oxnard.

Extensive patches of high quality riparian habitat are present along the length of the river and its tributaries.  The endangered fish, the unarmored stickleback, is resident in the river.  One of the largest of the Santa Clara River's tributaries, Sespe Creek, is designated a wild trout stream by the state of California and supports significant spawning and rearing habitat.  The Sespe Creek is also designated a wild and scenic river.  Piru and Santa Paula Creeks, which are tributaries to the Santa Clara River, also support good habitats for steelhead.  In addition, the river serves as an important wildlife corridor.  A lagoon exists at the mouth of the river and supports a large variety of wildlife.

# Water Quality Objectives Not Attained

Nitrite and Nitrate as Nitrogen: 10 mg/L, Nitrite as Nitrogen: 1 mg/L, Total Dissolved Solids: 1300 mg/L

# Beneficial Uses Affected

Agriculture, Groundwater Recharge (and associated MUN beneficial use of underlying groundwater basin)

# Data Assessment

## Table 2. Summary of Nitrite and Nitrate as Nitrogen, Nitrite as Nitrogen, Total Dissolved Solids Data (in mg/L) for Santa Clara River Reach 3 (Below Fillmore at Santa Paula)

|  | Nitrite and Nitrate as Nitrogen | Nitrite as Nitrogen | Total Dissolved Solids |
| --- | --- | --- | --- |
| Dates of Sampling | 1997-2000 | 1997-2000 | 1997-2000 |
| Number of Samples (n) | 45 | 30 | 189 |
| Minimum Data Value | .3 | 0 | 400 |
| Maximum Data Value | 33 | 1.7 | 1630 |
| Median Data Value | 3.7 | .45 | 1080 |
| Arithmetic Mean Value | 5.16 | .53 | 1081 |
| Standard Deviation | 5.68 | .483 | 221 |
| Percent above Objective | 11 | 17 | 20 |

# Potential Sources

Point and Non point sources

# References

1994 Basin Plan

California Regional Water Quality Control Board, Los Angeles Region

Pole Creek/Canyon

Tributary to Santa Clara River Reach 3

(Freeman Diversion to Fillmore Street A)

Sulfate, Total Dissolved Solids

# Summary of Proposed Action

Listing as “Not Supporting (Impaired)” is proposed for Pole Creek on the Santa Clara River for Sulfate and Total Dissolved Solids, which affect agricultural beneficial use.

## Table 1. 303(d) Listing/TMDL Information

| ## Waterbody Name | Pole Creek/ Santa Clara River | Pollutants/Stressors | Sulfate, Total Dissolved Solids |
| --- | --- | --- | --- |
| Hydrologic Unit | 403.31 | Source(s) | Non point sources |
| Total Waterbody Size |  | TMDL Priority | Low |
| Size Affected | 5.5 | TMDL Start Date (Mo/Yr) | 2012 |
| Extent of Impairment | Entire creek | TMDL End Date (Mo/Yr) | 2014 |

# Watershed Characteristics

The Santa Clara River is the largest river system in southern California that remains in a relatively natural state; this is a high quality natural resource for much of its length.  The river originates in the northern slope of the San Gabriel Mountains in Los Angeles County, traverses Ventura County, and flows into the Pacific Ocean halfway between the cities of San Buenaventura and Oxnard.

Extensive patches of high quality riparian habitat are present along the length of the river and its tributaries.  The endangered fish, the unarmored stickleback, is resident in the river.  One of the largest of the Santa Clara River's tributaries, Sespe Creek, is designated a wild trout stream by the state of California and supports significant spawning and rearing habitat.  The Sespe Creek is also designated a wild and scenic river.  Piru and Santa Paula Creeks, which are tributaries to the Santa Clara River, also support good habitats for steelhead.  In addition, the river serves as an important wildlife corridor.  A lagoon exists at the mouth of the river and supports a large variety of wildlife.

# Water Quality Objectives Not Attained

Sulfate: 650 mg/L, Total Dissolved Solids: 1300 mg/L

# Beneficial Uses Affected

Agriculture

# Data Assessment

## Table 2. Summary of Sulfate, TDS Data for Pole Creek/ Santa Clara River

|  | Sulfate (mg/L) | TDS (mg/L) |
| --- | --- | --- |
| Dates of Sampling | 1997-2000 | 1997-2000 |
| Number of Samples (n) | 12 | 12 |
| Minimum Data Value | 310 | 630 |
| Maximum Data Value | 850 | 1700 |
| Median Data Value | 753 | 1390 |
| Arithmetic Mean Value | 723 | 1374 |
| Standard Deviation | 135 | 256 |
| Percent above Objective | 97. | 91.7 |

# Potential Sources

Non Point Sources

# References

1994 Basin Plan

California Regional Water Quality Control Board, Los Angeles Region

Todd Barranca-Wheeler Creek/Canyon

Tributary to Santa Clara River Reach 3

(Freeman Diversion to Fillmore Street A)

Sulfate, Total Dissolved Solids

# Summary of Proposed Action

Listing as “Not Supporting (Impaired)” is proposed for Todd Barranca-Wheeler Creek/Canyon on the Santa Clara River for Sulfate and Total Dissolved Solids, which affect agricultural beneficial use.

## Table 1. 303(d) Listing/TMDL Information

| ## Waterbody Name | Todd Barranca-Wheeler Creek/Canyon | Pollutants/Stressors | Sulfate, Total Dissolved Solids |
| --- | --- | --- | --- |
| Hydrologic Unit | 403.21 | Source(s) | Non point sources |
| Total Waterbody Size |  | TMDL Priority | Low |
| Size Affected | 4.17 | TMDL Start Date (Mo/Yr) | 2012 |
| Extent of Impairment | Entire creek | TMDL End Date (Mo/Yr) | 2014 |

# Watershed Characteristics

The Santa Clara River is the largest river system in southern California that remains in a relatively natural state; this is a high quality natural resource for much of its length.  The river originates in the northern slope of the San Gabriel Mountains in Los Angeles County, traverses Ventura County, and flows into the Pacific Ocean halfway between the cities of San Buenaventura and Oxnard.

Extensive patches of high quality riparian habitat are present along the length of the river and its tributaries.  The endangered fish, the unarmored stickleback, is resident in the river.  One of the largest of the Santa Clara River's tributaries, Sespe Creek, is designated a wild trout stream by the state of California and supports significant spawning and rearing habitat.  The Sespe Creek is also designated a wild and scenic river.  Piru and Santa Paula Creeks, which are tributaries to the Santa Clara River, also support good habitats for steelhead.  In addition, the river serves as an important wildlife corridor.  A lagoon exists at the mouth of the river and supports a large variety of wildlife.

# Water Quality Objectives Not Attained

Sulfate: 650 mg/l., Total Dissolved Solids: 1300 mg/L  (Waterbody tributary to Santa Clara River Reach 3 between Freeman Diversion and Fillmore Street A with objectives in Basin Plan Table 3-8)

# Beneficial Uses Affected

Agriculture

# Data Assessment

## Table 2. Summary of Sulfate, Total Dissolved Solids Data for Todd Barranca-Wheeler Creek/ Santa Clara River

|  | Sulfate (mg/L) | TDS (mg/L) |
| --- | --- | --- |
| Dates of Sampling | 1997-2000 | 1997-2000 |
| Number of Samples (n) | 12 | 12 |
| Minimum Data Value | 650 | 1410 |
| Maximum Data Value | 1380 | 2650 |
| Median Data Value | 875.5 | 1920 |
| Arithmetic Mean Value | 905 | 1952 |
| Standard Deviation | 176 | 302 |
| Percent above Objective | 91.7 | 100 |

# Potential Sources

Non Point Sources

# References

1994 Basin Plan

California Regional Water Quality Control Board, Los Angeles Region

Hopper Creek

Tributary to Santa Clara River Reach 4

(Fillmore Street A to Blue Cut Gauging Station)

Sulfate/Total Dissolved Solids

# Summary of Proposed Action

Listing as “Not Supporting (Impaired)” is proposed for Hopper Creek, a tributary of the Santa Clara River, Reach 4, for Sulfate and Total Dissolved Solids, which affect agricultural beneficial use, due to greater than 25 percent exceedance of the objective.

{Insert text describing the proposed listing or de-listing action, including the waterbody reach name and description of its location, pollutant or stressor causing impairment, and beneficial uses affected}

## Table 1. 303(d) Listing/TMDL Information

| ## Waterbody Name | Hopper Creek/Santa Clara River | Pollutants/Stressors | sulfate/ total dissolved solids |
| --- | --- | --- | --- |
| Hydrologic Unit | 403.41 | Source(s) | nonpoint sources, point sources |
| Total Waterbody Size |  | TMDL Priority | Low |
| Size Affected | 13.65 | TMDL Start Date (Mo/Yr) | 2012 |
| Extent of Impairment | Entire reach | TMDL End Date (Mo/Yr) | 2014 |

# Watershed Characteristics

The Santa Clara River is the largest river system in southern California that remains in a relatively natural state; this is a high quality natural resource for much of its length.  The river originates in the northern slope of the San Gabriel Mountains in Los Angeles County, traverses Ventura County, and flows into the Pacific Ocean halfway between the cities of San Buenaventura and Oxnard.

Extensive patches of high quality riparian habitat are present along the length of the river and its tributaries.  The endangered fish, the unarmored stickleback, is resident in the river.  One of the largest of the Santa Clara River's tributaries, Sespe Creek, is designated a wild trout stream by the state of California and supports significant spawning and rearing habitat.  The Sespe Creek is also designated a wild and scenic river.  Piru and Santa Paula Creeks, which are tributaries to the Santa Clara River, also support good habitats for steelhead.  In addition, the river serves as an important wildlife corridor.  A lagoon exists at the mouth of the river and supports a large variety of wildlife.

# Water Quality Objectives Not Attained

Sulfate: 600 mg/L; Total Dissolved Solids: 1300 mg/L (Table 3-8 in LA Regional Board Basin Plan)

Elizabeth – Renee has comments stating Sulfate is 500 MUN and TDS is 500 Sec MCL.  Can you clarify?

# Beneficial Uses Affected

Agriculture

# Data Assessment

## Table 2. Summary of Sulfate/TDS Data for Hopper Creek

|  | Sulfate (mg/L) | TDS (mg/L) |
| --- | --- | --- |
| Dates of Sampling | 1997-2000 | 1997-2000 |
| Number of Samples (n) | 12 | 11 |
| Minimum Data Value | 580 | 1220 |
| Maximum Data Value | 801 | 1700 |
| Median Data Value | 714 | 1430 |
| Arithmetic Mean Value | 717 | 1444 |
| Standard Deviation | 63 | 131 |
| Percent above Objective | 91.7 | 91.7 |

# Potential Sources

Non Point sources and Point sources

# References

1994 Basin Plan

California Regional Water Quality Control Board, Los Angeles Region

Piru Creek

Tributary to Santa Clara River Reach 4

(Fillmore A Street and Blue Cut Gauging Station)

pH

# Summary of Proposed Action

Listing as “Partially Supporting” (impaired) is proposed for Piru Creek on the Santa Clara River for pH, which affects aquatic life beneficial use because the objective is exceeded 17 percent of the time.

## Table 1. 303(d) Listing/TMDL Information

| ## Waterbody Name | Piru Creek/ Santa Clara River | Pollutants/Stressors | pH |
| --- | --- | --- | --- |
| Hydrologic Unit | 403.41 | Source(s) | Non point sources, Conservation discharge |
| Total Waterbody Size |  | TMDL Priority | Analytical Unit 32 |
| Size Affected |  | TMDL Start Date (Mo/Yr) | 2001 |
| Extent of Impairment | Entire creek | TMDL End Date (Mo/Yr) | 2003 |

# Watershed Characteristics

The Santa Clara River is the largest river system in southern California that remains in a relatively natural state; this is a high quality natural resource for much of its length.  The river originates in the northern slope of the San Gabriel Mountains in Los Angeles County, traverses Ventura County, and flows into the Pacific Ocean halfway between the cities of San Buenaventura and Oxnard.

Extensive patches of high quality riparian habitat are present along the length of the river and its tributaries.  The endangered fish, the unarmored stickleback, is resident in the river.  One of the largest of the Santa Clara River's tributaries, Sespe Creek, is designated a wild trout stream by the state of California and supports significant spawning and rearing habitat.  The Sespe Creek is also designated a wild and scenic river.  Piru and Santa Paula Creeks, which are tributaries to the Santa Clara River, also support good habitats for steelhead.  In addition, the river serves as an important wildlife corridor.  A lagoon exists at the mouth of the river and supports a large variety of wildlife.

# Water Quality Objectives Not Attained

pH: <6.5 or > 8.5

# Beneficial Uses Affected

Aquatic Life

# Data Assessment

## Table 2. Summary of pH Data for Piru Creek/ Santa Clara River

| Dates of Sampling | 1997-2000 |
| --- | --- |
| Number of Samples (n) | 24 |
| Minimum Data Value | 7.6 |
| Maximum Data Value | 8.8 |
| Median Data Value | 8.4 |
| Arithmetic Mean Value | 8.29 |
| Standard Deviation | .36 |
| Percent above Objective | 17 |

# Potential Sources

Non Point Sources, Conservation Releases

# References

1994 Basin Plan

The samples are from R1 and R2. I would put R1 and R2 together because there is not sufficient data to do otherwise and leave R3 unassessed.

California Regional Water Quality Control Board, Los Angeles Region

Sespe Creek

Tributary to Santa Clara River Reach 3

(Freeman Diversion to Fillmore Street A)

Chloride, pH

# Summary of Proposed Action

Listing is proposed for Sespe Creek on the Santa Clara River for chloride and pH, which affect agricultural and aquatic life beneficial uses.  The proposed listing would be “Not Supporting” for the agricultural beneficial use and “Partially Supporting” for the aquatic life beneficial use.

## Table 1. 303(d) Listing/TMDL Information

| ## Waterbody Name | Sespe Creek/ Santa Clara River | Pollutants/Stressors | Chloride, pH |
| --- | --- | --- | --- |
| Hydrologic Unit | 403.31, 403.32 | Source(s) | non point sources |
| Total Waterbody Size | 41.5 | TMDL Priority | Chloride: Analytical Unit 31 pH: Analytical Unit 32 |
| Size Affected |  | TMDL Start Date (Mo/Yr) | Chloride: 1998 pH: 2001 |
| Extent of Impairment | Entire creek | TMDL End Date (Mo/Yr) | Chloride: 2002 pH: 2003 |

# Watershed Characteristics

{Insert text describing the watershed – could be taken from WMI chapter or draft TMDL documents}

The Santa Clara River is the largest river system in southern California that remains in a relatively natural state; this is a high quality natural resource for much of its length.  The river originates in the northern slope of the San Gabriel Mountains in Los Angeles County, traverses Ventura County, and flows into the Pacific Ocean halfway between the cities of San Buenaventura and Oxnard.

Extensive patches of high quality riparian habitat are present along the length of the river and its tributaries.  The endangered fish, the unarmored stickleback, is resident in the river.  One of the largest of the Santa Clara River's tributaries, Sespe Creek, is designated a wild trout stream by the state of California and supports significant spawning and rearing habitat.  The Sespe Creek is also designated a wild and scenic river.  Piru and Santa Paula Creeks, which are tributaries to the Santa Clara River, also support good habitats for steelhead.  In addition, the river serves as an important wildlife corridor.  A lagoon exists at the mouth of the river and supports a large variety of wildlife.

Increasing loads of nitrogen and salts in supplies of ground water threaten beneficial uses including irrigation and drinking water.  Other threats to water quality include increasing development in floodplain areas which has necessitated flood control measures such as channelization that results in increased runoff volumes and velocities, erosion, and loss of habitat.  In many of these highly disturbed areas the exotic giant reed (Arundo donax) is gaining a foothold.

Many of the smaller communities in this watershed remain unsewered.  In particular, in the Agua Dulce area of the upper watershed, impacts on drinking water wells from septic tanks is a major concern.  The community is undertaking a wellhead protection effort, with oversight by Board staff.  Development pressure, particularly in the upper watershed, threatens habitat and the water quality of the river.  The effects of septic system use in the Oxnard Forebay area is also of concern.

# Water Quality Objectives Not Attained

Chloride: 60 mg/L, pH <6.5 or >8.5 (Table 3-8 of the LA Regional Board Basin Plan)

# Beneficial Uses Affected

Agriculture, Aquatic Life

# Data Assessment

## Table 2. Summary of Chloride and pH Data for Sespe Creek / Santa Clara River

|  | Chloride (mg/L) | pH (units) |
| --- | --- | --- |
| Dates of Sampling | 1997-2000 | 1997-2000 |
| Number of Samples (n) | 16 | 24 |
| Minimum Data Value | 6 | 7.2 |
| Maximum Data Value | 118 | 9 |
| Median Data Value | 34 | 8.3 |
| Arithmetic Mean Value | 49 | 8.25 |
| Standard Deviation | 35 | .39 |
| Percent above Objective | 44 | 25 |

# Potential Sources

Non point sources

# References

Basin Plan (1994)