California Regional Water Quality Control Board, Los Angeles Region

Tissue, Sediment and Benthic Infauna Data

Ventura River Estuary

# Summary of Proposed Action

## Proposed New Delistings

- Delist DDT in tissue as the original listing appears to have been based on DDT concentrations found in shiner surfperch in 1993 (TSM);  however, the level of 23 ppb for p,p’-DDE is below the MTRL (which equals 32.0 ppb).

These actions all affect the fish consumption and aquatic life beneficial uses as previously listed in the 1996 Water Quality Assessment.

## Table 1. 303(d) Listing/TMDL Information

| ## Waterbody Name | Ventura River Estuary | Pollutants/Stressors | See Above |
| --- | --- | --- | --- |
| Hydrologic Unit | 402.10 | Source(s) | n/a |
| Total Waterbody Size | 0.35 mi | TMDL Priority | 87 |
| Size Affected |  | TMDL Start Date (Mo/Yr) |  |
| Extent of Impairment |  | TMDL End Date (Mo/Yr) |  |

# Watershed Characteristics

{Insert text describing the watershed – could be taken from WMI chapter or draft TMDL documents}

The Ventura River and its tributaries drain a coastal watershed in western Ventura County.  The watershed covers a fan-shaped area of 235 square miles, which is situated within the western Transverse Ranges (the only major east-west mountain ranges in the continental U.S.).  From the upper slopes of the Transverse Ranges, the surface water system in the Ventura River watershed generally flows in a southerly direction to an estuary, located at the mouth of the Ventura River.  Groundwater basins composed of alluvial aquifers deposited along the surface water system, are highly interconnected with the surface water system and are quickly recharged or depleted, according to surface flow conditions.  Topography in the watershed is rugged and as a result, the surface waters that drain the watershed have very steep gradients, ranging from 40 feet per mile at the mouth to 150 feet per mile at the headwaters.

Precipitation varies widely in the watershed.  Most occurs as rainfall during just a few storms, between November and March.  Summer and fall months are typically dry.  Although snow occurs at higher elevations, melting snowpack does not sustain significant runoff in warmer months.  The erratic weather pattern, coupled with the steep gradients throughout most of the watershed, result in high flow velocities with most runoff reaching the ocean.

# Water Quality Objectives Not Attained

N/A

# Beneficial Uses Affected

Aquatic Life, as previously listed in the 1996 Water Quality Assessment.

Fish Consumption.

# Data Assessment

## Table 2. Summary of Tissue and Sediment Data for the Ventura River Estuary

| Dates of Sampling | 2/10/93 6/21/93 6/20/98 |
| --- | --- |
| Number of Samples (n) | 1993: 1 (sediment) + 1 (fish tissue) 1998: 2 (sediment) |
| Minimum Data Value |  |
| Maximum Data Value |  |
| Median Data Value |  |
| Arithmetic Mean Value |  |
| Standard Deviation |  |
| Number (Percent) above Objective |  |

This table may summarize additional data not relevant to this factsheet that supports a continued listing for this waterbody.

# Potential Sources

N/A{Indicate if you can any and all potential sources}

# References

Toxic Substances Monitoring Program Database

Bay Protection and Toxic Cleanup Program Database

Ojai Valley Sanitation Districts NPDES Monitoring