California Regional Water Quality Control Board

Santa Ana Region

October 26, 2001

Item:	  9

Subject:      Update of the Clean Water Act Section 303(d) List of Impaired Waterbodies

INTRODUCTION

Section 303(d) of the Clean Water Act requires states to update the list of surface waterbodies for which water quality standards are not attained, or are not expected to be attained with the implementation of technology-based controls.  These waterbodies are considered “impaired”.  The resulting 303(d) list of impaired waterbodies includes a description of the pollutants causing impairment and a schedule for developing a Total Maximum Daily Load (TMDL) for each pollutant.  The TMDL is the maximum load of a pollutant that can be discharged and still ensure the attainment of applicable water quality standards.  Placing a waterbody on the Section 303(d) list of impaired waterbodies requires the development of a TMDL(s) to address the source(s) of impairment.  Federal TMDL regulations require states to update the Section 303(d) list of impaired waterbodies and submit the list to US Environmental Protection Agency  (USEPA).  The Santa Ana Regional Water Quality Control Board, on behalf of the State Water Resources Control Board (State Board), has compiled recommended changes to the 303(d) list.  The State Board will review recommendations from all the Regional Boards, hold a public hearing to consider public comments, and adopt a statewide 303(d) list for submittal to the USEPA by April 2002.

The Santa Ana Region last reviewed and updated the 303(d) list in 1998.   The proposed revised 303(d) list is shown in Attachment A.

# WATERBODIES ASSESSED

To update the 303(d) list, staff solicited information from the public on the water quality condition of waterbodies within the Region and reviewed additional data from recent investigations.  The waterbodies assessed thereby included coastal beaches, as well as coastal and inland rivers and streams.  These waterbodies are shown in Table 1.  The data obtained from the public and additional data reviewed are summarized in Worksheets prepared for each waterbody assessed.  These waterbody Worksheets are contained in Attachment B.

Table 1

2001/2002 List of Waterbodies Assessed

| Coastal Waterbodies | Inland Valley Waterbodies | Mountain Area Waterbodies |
| --- | --- | --- |
| - Seal Beach | - San Timoteo Creek | - Big Bear Lake |
| - Anaheim Bay | - Cucamonga Creek | - Metcalf Creek |
| - Huntington Harbour | - Chino Creek | - Boulder Creek |
| - Newport Bay | - Mill Creek (Prado Area) | - Knickerbocker Creek |
| - San Diego Creek, Reach 1 | - Santa Ana River, Reaches 2 & 3 | - Grout Creek |
| - San Diego Creek, Reach 2 | - Temescal Creek | - San Jacinto River, Reaches 6 & 7 |
| - Pelican Point Creek | - Canyon Lake | - Strawberry Creek |
| - Los Trancos Creek (Crystal Cove Cr.) |  |  |
| - Muddy Canyon Creek - Near-shore ocean waters |  |  |
| - Crystal Cove Beaches |  |  |
| - Huntington State/ City Beaches |  |  |
| - Bolsa Chica State Beach |  |  |
| - Corona State Beach |  |  |
| - Newport Beaches |  |  |
| - Off-shore ocean waters |  |  |

PROPOSED SECTION 303(d) LIST CHANGES

### Listing/Delisting Strategy

State Board guidance to the Regional Boards on listing and delisting pursuant to Section 303(d) has changed over time.  In 1998, the Santa Ana Regional Board staff participated on an interagency task force to develop new listing/delisting criteria for use by the State Board and Regional Boards.  These criteria generally require more concrete, quantitative information for listing than past criteria (i.e., listing on the basis of “best professional judgment” or “estimated assessments”).  The task force discussed but did not reach consensus on the number of samples, or the number of violations of standards, required in order to list a specific waterbody.  In general, water bodies may be delisted from the 303(d) list if data demonstrate that the waterbody is not impaired, if there is a TMDL in place, or if there are specific and planned measures to be undertaken that will address the impairment (e.g, a Cleanup and Abatement Order).   Specific delisting criteria were included as part of the 1998 Listing Guidance. The State Board has been making plans to revise the 1998 Guidance; however, no updated guidelines are available for use in this update of the 303(d) list.

Given the lack of specific State guidance on revising the 303(d) list, Board staff generally utilized an approach that consisted of evaluating available data and determining if the data were adequate to support a listing decision.  Data types evaluated included numeric water column and/or sediment chemistry data, bioassessment data (e.g., benthic infaunal richness and abundance), water column and/or sediment toxicity data.   In addition to the numeric data, staff also reviewed other types of information that provide an indication of the status of a waterbody.  Examples include a history of algal blooms and/or fish kills, and beach posting information.  For this assessment, the majority of the data available were numeric water column chemistry data, numerical data from fish or mussel bioaccumulation studies, and information about beach or river reach closure or postings due to bacterial contamination.  Numeric data were compared to an appropriate numeric standard for that waterbody; other information was evaluated to determine if there was clear information demonstrating impairment of a beneficial use.   In many cases, the data that were submitted or available were insufficient, or not conclusive as to whether an impairment exists.  In this case, staff recommends implementing a prioritized monitoring program for those waterbodies.

In evaluating waterbodies, staff determined the water quality indicator (parameter or beneficial use) to be evaluated and the minimum required sample size per parameter (e.g., comparison of the bacteriological data to the Basin Plan objective for the protection of swimming requires a minimum of 5 samples for fecal coliform to be taken during a 30-day period).  Staff also determined the number of sampling locations for each waterbody and the number of times each location was sampled.  Staff believes that, because of the implications of placing a waterbody on the 303(d) list, identifying a waterbody as impaired should not be based on a limited amount of data.  Therefore, staff’s approach was to define the minimum sample size requirement that would allow an assessment to be completed for a waterbody (including all locations) as 10 data points during the 1997-2001 time period (this requirement was based on USEPA 305(b) guidance, 1998).   There was no standard “frequency of exceedance” that staff utilized to make a determination that a waterbody is impaired. For the most part, staff relied on a weight of evidence approach on a waterbody by waterbody basis that took into consideration the number of exceedances of the applicable objectives, the beneficial uses threatened or impaired, the magnitude of the exceedances from the numerical objective, knowledge of the land use history (as it affects water quality) and the quality of the data reviewed.  Where the data were adequate to make a determination, staff also attempted to identify seasonal impairment, e.g., whether the impairment occurs primarily during the rainy season.  This will help to focus any subsequent TMDL development efforts.

Beach posting data, the only narrative information reviewed, were assessed in a different manner.  Staff reviewed this information to determine the number of times a beach was posted by the Orange County Health Care Agency due to bacterial contamination.  The Health Care Agency monitors bacterial quality of the beaches pursuant to the California Heath and Safety Code, Section  115880 (AB 411 requirements), and posts ocean waters when the bacteriological standards established by the Department of Health Services in the California Code of Regulations (Title 17, Section 7958) are exceeded.  The Health Care Agency is required to apply these standards to determine whether it is necessary to restrict the use of public beaches (or portions thereof).  Staff believes that posting of the ocean waters indicates at least the threat of impairment to recreation beneficial uses.  If a beach was posted for more than one week (seven consecutive days) per year during the assessment period (1997-2001) for reasons other than a spill or illegal dumping, then the beach was included on the proposed list of impaired water bodies.  Staff recognized that at times, beaches may have been posted for several days, then reopened for several days and then posted again, etc.  Staff determined that this sporadic posting of a beach did not demonstrate a consistent exceedance, and thus the beach is not proposed to be included on the 303(d) list of impaired waterbodies.  In addition to the beach posting information, staff also evaluated available bacteriological data for comparison with the California Ocean Plan bacteriological objectives.  In most cases, however, the data were collected in a manner that is inconsistent with the Ocean Plan objectives.  Therefore, staff only used the bacteriological data to support the beach posting information.  Finally, staff also reviewed the environmental group Heal the Bay’s Beach Report Card information to support 303(d) listing recommendations.  Heal the Bay evaluated the bacteriological data from the Orange County Health Care Agency’s monitoring program in comparison to the AB 411 bacteriological thresholds and the Santa Monica Bay Restoration Project’s epidemiological study on swimmers at urban-runoff influenced beaches.  The grading system of the Report Card takes into consideration the magnitude and frequency of exceedances of the thresholds during both the wet and dry season.  Staff used the Report Card to confirm the beach posting information i.e., beaches which have threatened or impacted recreation uses due to bacterial contamination.

As part of the toxic parameter TMDL development process for Newport Bay and its watershed, Regional Board staff reviewed available data and prepared a problem statement (December 2000).  The intent was to identify the specific toxic pollutants for which TMDLs are required.  (The 303(d) listing for Newport was too general, broadly identifying metals, pesticides, etc., as the causes of impairment.)   The specific pollutants identified included selenium, diazinon and chlorpyrifos.  Board staff is developing TMDLs for those constituents now.  The USEPA is also in the process of reviewing additional data sets as well as new data not available to Board staff at the time of the problem statement development.  A final list of toxic substances requiring TMDLs has not yet been formulated.  Once the list is finalized, USEPA will assure that TMDLs are developed for all the listed pollutants and will promulgate all the toxic substance TMDLs, including those for selenium, diazinon and chlorpyrifos, by April 15, 2002.  Once the TMDLs are promulgated, the 303(d) list can be modified appropriately, i.e., Newport Bay and its watershed can be removed from the 303(d) list of impaired waters.  This revision will be addressed in a future update of the 303(d) list.

To recommend delisting a waterbody from the 303(d) list, staff relied on the 1998 Listing/Delisting Guidance criteria that state that waterbodies may be removed from the 303(d) list if data (10 sample minimum) demonstrate that objectives are being met or if a TMDL has been developed and approved by USEPA.

Table 2 contains the list of waterbodies proposed to be delisted from the 303(d) list with a brief summary of the justification.

Table 3 contains the list of waterbodies proposed to be added to the 303(d) list with a brief summary of the justification.

For each proposed delisted and/or listed waterbody, staff has prepared Waterbody Worksheets that provide the data source, a summary of the data evaluated and justification for the proposed listing/delisting.  The Worksheets are included in Attachment B.

### Proposed TMDL  Priorities

- Pursuant to federal regulations (40 CFR 130.7), the Regional Board is required to provide priority rankings for the development of TMDLs for the Region’s 303(d) listed waterbodies.  To develop TMDL priorities, Board staff considered the 1998 Listing/Delisting Guidance, USEPA’s “directive” to complete all TMDLs within the next 13 years (note that there is no specific time frame specified in either the statute or regulation), and the following criteria.

“High” priority waterbodies are targeted for TMDL development in the next 2 to 5 years.  Waters are identified as “High” priority for TMDL development when one or more of the following criteria are met:

- there is current involvement in watershed planning activities affecting the waterbody, pursuant to the Watershed Management Initiative adopted by the Regional Board in March, 2001;

- TMDL development activities are currently underway;

- there is litigation that is driving the TMDL development process;

- the waterbody is of significant concern because of its regionally important beneficial uses, including municipal drinking water supply (MUN), habitat for rare or endangered species (RARE) or body contact recreation (REC1), one or more of which may be affected by the pollutant(s) of concern;

- there  is a high degree of public concern;

- there is a high potential for beneficial use recovery upon implementation of the TMDL; and

- there is a high potential for state or federal funding or stakeholder funding to support TMDL development.

“Medium” priority waterbodies are targeted for TMDL development in the next 5 to 10 years.  Waters are identified as “Medium” priority for TMDL development when one or more of the following criteria are met:

- Board staff is planning to conduct watershed planning activities involving the waterbody in the next 3 to 4 years, pursuant to the Watershed Management Initiative adopted by the Regional Board in March, 2001;

- there is a moderate potential for beneficial use recovery;

- there is a moderate degree of public concern; and

- there is a moderate potential for state or federal funding or stakeholder funding to support TMDL development in the future.

“Low” priority waterbodies are targeted for TMDL development in the next 9-11 years.  Data collection efforts to be undertaken in these waterbodies and watersheds may ultimately result in the delisting of many of these waterbodies from the 303(d) list.  Therefore, until a more thorough monitoring and assessment program is conducted, staff does not believe effort should be spent developing TMDLs for the “Low” priority waterbodies. Waterbodies are considered to be “Low” priority for TMDL planning because of the following:

- Board staff is not planning to conduct watershed planning activities affecting the waterbody until 7 to 10 years in the future, pursuant to the Watershed Management Initiative adopted by the Regional Board in March, 2001;

- there is minimal public concern;

- there is a low potential for beneficial use recovery upon implementation of the TMDL (the cost of developing TMDL would likely be greater than the expected benefit); and

- there is minimal potential for state or federal funding or stakeholder funding to support TMDL development.

- TMDL priorities and schedules for new waterbodies proposed for inclusion on the 303(d) list are shown on Table 3.  TMDL priorities and schedules for waterbodies already on the 303(d) list are shown on the proposed 303(d) list in Attachment A.

# Priority Monitoring Strategy

In several cases, the data available for review were insufficient, or not conclusive as to whether impairment exists.  In these cases, staff recommends implementing a prioritized monitoring program for those waterbodies and the parameters of concern.  Two priority lists are proposed.  Priority 1 waterbodies are those where the data assessed do not exceed a standard, but are close enough to the standard to be of concern, or where the data assessed occasionally exceed a standard, but there are not enough data points to indicate consistent exceedances.  For Priority 1 waterbodies, additional data or information are needed to confirm an impaired water status.  Focused monitoring for these waterbodies will take place sooner than for other waterbodies.

Priority 2 waterbodies are those waterbodies where the data assessed do not exceed a standard and are not of concern at this time, or for which less than five data points exist and therefore there are not enough data available to conclude there is impairment or a threat of impairment.  Monitoring for these waterbodies and parameters would likely be carried out as part of other agency monitoring programs. Board staff would utilize these data and information to make a determination on the waterbody status in a subsequent 303(d) list update.

Tables 4a and 4b contain the list of Priority 1 and Priority 2 waterbodies, the respective parameters of concern and the proposed monitoring schedule.

Table 2

Waterbodies Proposed to be Delisted from Section 303(d) List of Impaired Waters

| Waterbody | ##### Pollutant | ## Justification |
| --- | --- | --- |
| San Diego Creek, Reach 1 | Nutrients1 | TMDL incorporated into the Basin Plan; TMDL approved by USEPA |
|  | Siltation2 | TMDL incorporated into the Basin Plan; TMDL approved by USEPA |
| San Diego Creek, Reach 2 | Nutrients1 | TMDL incorporated into the Basin Plan; TMDL approved by USEPA |
|  | Siltation2 | TMDL incorporated into the Basin Plan; TMDL approved by USEPA |
| Upper Newport Bay | Nutrients1 | TMDL incorporated into the Basin Plan; TMDL approved by USEPA |
|  | Siltation2 | TMDL incorporated into the Basin Plan; TMDL approved by USEPA |
|  | Fecal coliform3 | TMDL incorporated into the Basin Plan; TMDL approved by USEPA |
| Lower Newport Bay | Nutrients1 | TMDL incorporated into the Basin Plan; TMDL approved by USEPA |
|  | Siltation2 | TMDL incorporated into the Basin Plan; TMDL approved by USEPA |
|  | Fecal coliform3 | TMDL incorporated into the Basin Plan; TMDL approved by USEPA |
| Santa Ana River, Reach 3 | Total Dissolved Solids4 | Data demonstrate objective being met |
|  | Nitrogen5 | TMDL incorporated into the Basin Plan; TMDL approved by USEPA Data demonstrate objective being met |

1 Resolution No. 98-100			 4  See Attachment B for Worksheet

2 Resolution No. 98-101 			 5  Resolution No. 91-125

3 Resolution No. 99-10

Table 3

Waterbodies Proposed to be Added to the Section 303(d) List of Impaired Waters

| ### Waterbody | #### Pollutant | ### TMDL Priority | ### TMDL Development |  |
| --- | --- | --- | --- | --- |
|  |  |  | ### Start Date | ### End Date |
| San Diego Creek, Reach 1 | Fecal coliform | Medium | 2010 | 2015 |
| Pelican Point Creek | Total/Fecal Coliform | Medium | 2009 | 2011 |
| Buck Gully Creek | Total/ Fecal coliform | Medium | 2008 | 2011 |
| Los Trancos Creek (Crystal Cove Cr.) | Total /Fecal coliform | Medium | 2008 | 2011 |
| Muddy Creek | Total/ Fecal coliform | Medium | 2008 | 2011 |
| Seal Beach 1st Street to Main Street Pier | Bacteria (wet season)* | High | 2007 | 2011 |
| Seal Beach Breakwater | Bacteria (wet season) * | High | 2007 | 2011 |
| Huntington Beach – Dog Beach | Bacteria (wet season) * | High | 2007 | 2011 |
| Huntington State Beach – from Newland Avenue to Santa Ana River | Bacteria (wet and dry seasons) * | High | 2005 | 2009 |
| Newport Beach 19th Street to 43 rd Street Beach | Bacteria (wet and dry seasons) * | High | 2005 | 2009 |
| Newport Beach 1000 feet down coast of Santa Ana River | Bacteria (wet season) * | High | 2007 | 2011 |
| Little Corona Beach | Bacteria (wet season) * | High | 2007 | 2011 |
| Canyon Lake – East Bay | Sediment | medium | 2008 | 2011 |

*  Orange County Health Care Agency bases beach postings on the following bacterial indicators:  total coliform, fecal coliform and enterococcus.  Wet season extends from October to April.

Table 4a

Monitoring Priority 1 Water Bodies

| Waterbody | Parameter of Concern | Monitoring Schedule (year) 1 |
| --- | --- | --- |
| Ocean Waters | Dieldrin, mercury, p,pDDE (fish tissue) | 2004 |
| Seal Beach | Mercury, p,pDDE( fish tissue) | 2004 |
| Huntington Beach State Park | Mercury, p,pDDE (fish tissue) | 2004 |
| Anaheim Bay | Mercury, p,pDDE, nickel, copper, dieldrin, PCB | 20012 |
| Huntington Harbour | Copper, Nickel, dieldrin, toxaphene | 20012 |
| Bolsa Chica | Copper, Nickel | 2004 |
| San Jacinto River, Reaches 6 and 7 | Hardness, TDS, Chloride, aluminum , sodium | 2004 |
| Strawberry Creek | General mineral constituents | 2004 |
| Big Bear Lake | Inorganic nitrogen, phosphorus | 20023 |
| Knickerbocker Creek | Inorganic nitrogen | 20023 |
| Metcalf Creek | Inorganic nitrogen | 20023 |
| Boulder Creek | Inorganic nitrogen | 20023 |
| Knickerbocker Creek | Inorganic nitrogen | 20023 |

Table 4b

###### Monitoring Priority 2 Water bodies

| Waterbody | Parameter of Concern | Monitoring Schedule (year) 1 |
| --- | --- | --- |
| Anaheim Bay | Zinc, Nickel, Lead, Chromium, Cadmium | 20012 |
| San Timoteo Creek | General water quality parameters | 2006 |
| Temescal Creek | Metals | 2007 |
| Cucamonga Creek | Metals | 2006 |
| Chino Creek Reach 1 | Metals | 2006 |
| Mill Creek (Prado area) | Metals | 2006 |
| Santa Ana River Reaches 3, 4 and 5 | Metals | 2006 |

- monitoring schedule is contingent upon  funding availability

- these waterbodies will be assessed as part of the current Anaheim Bay/Huntington Harbour Water Quality Assessment study

- these waterbodies will be assessed as part of the current studies being conducted to develop TMDLs for the Big Bear Lake watershed.

STAFF RECOMMENDATION

Direct staff to transmit this report, comments received and all other relevant materials to the State Water Resources Control in support of the Statewide Section 303(d) list adoption.

ATTACHMENTS

Attachment A:	Santa Ana Region 2001/2002 Section 303(d) List

Attachment B: 	Data Analyses, Summary and Waterbody Worksheets

Attachment A  - Proposed 2002 303(d) List of Impaired Waterbodies for the Santa Ana Region

Attachment B – Data Analyses Summary and Waterbody Worksheets

List of Abbreviations/Acronyms

| Ag | Silver |
| --- | --- |
| Hg | Mercury |
| Cu | Copper |
| Ni | Nickel |
| Cd | Cadmium |
| Cr | Chromium |
| TDS | Total dissolved solids |
| TIN | Total inorganic nitrogen |
| TN | Total nitrogen |
| Zn | Zinc |
| MCL | Maximum contaminant level |
| MTRL | Maximum Tissue Residence Level |
| DHS | CA Department of Health Services |
| NAS | National Academy of Science |
| FDA | Food and Drug Administration |
| EBE | Enclosed Bays and Estuaries (Cal Toxics Rule) |
| CTR | California Toxics Rule |
| REC1 | Water contact recreation beneficial use |
| REC2 | Non-water contact recreation beneficial use |
| MUN | Municipal drinking water supply beneficial use |
| IND | Industrial service supply beneficial use |
| PROC | Industrial process supply beneficial use |
| GWR | Groundwater recharge |
| COMM | Commercial and sport fishing beneficial use |
| NAV | Navigation beneficial use |
| BIOL | Biological habitat beneficial use |
| RARE | Habitat for rare or endangered species (beneficial use) |
| WILD | Wildlife habitat beneficial use |
| EST | Estuarine habitat beneficial use |
| SPWN | Spawning, reproduction, development habitat beneficial use |
| SHEL | Shellfish harvesting beneficial use |
| MAR | Marine aquatic habitat beneficial use |
| WARM | Warm water aquatic habitat beneficial use |
| LWRM | Limited warm water aquatic habitat beneficial use |
| COLD | Cold water aquatic habitat beneficial use |