Salmon Forever's Sediment Data Package for:

2012 California Integrated Report – Surface Water Quality Assessment and List of Impaired Waters [Clean Water Act Sections 305(b) and 303(d)]

July 30, 2010

Photo: Thomas Dunklin

Continuous Suspended Sediment Concentrations, Turbidity and Discharge; Suspended Sediment Loads, Sand Fractions and Loads, Rainfall, Channel Surveys

Elk River and Freshwater Creek, Humboldt County, California

Submitted to State Water Resources Control Board

Division of Water Quality

Submitted By Salmon Forever

Project Director: Jesse Noell

Analyst: Jack Lewis

INTRODUCTION

This document is best viewed electronically as it contains hyperlinks to all the essential elements of this package. The package consists of water, sediment discharge, and ancillary data and analyses for HY2003 through HY2008 from four stream gaging stations on Freshwater Creek and Elk River, tributaries to Humboldt Bay, in the North Coast District.  Two gaging stations in each watershed are operated using the Turbidity Threshold Sampling (TTS) system (Lewis and Eads, 2009).  Gaging station locations are South Fork Elk River at Jesse Noell's house (SFM), North Fork Elk River at Kristi Wrigley's house (KRW), Freshwater Creek at Terry Roelofs' house (FTR), and Freshwater Creek at Howard Heights Bridge (HHB).  Maps and aerial photos can be found in the folder Maps to roughly locate stations and cross sections. Suspended sediment loads were determined for 64 storm events at SFM, KRW, and FTR, and 44 events at HHB, which was started later in HY2005.  Inter-storm loads were also computed and added to storm loads to obtain annual sediment loads.  Suspended sediment concentration (SSC) from more than 5000 pumped samples were included in the computation of storm loads.  A subset of nearly 700 samples was analyzed for sand fraction, and the percent sand (> 0.063 mm) of the suspended load was determined for 3 years at FTR and HHB, and 2 years at SFM and KRW.  As a byproduct of the analysis, a record of SSC was produced at 10-minute intervals for all stations, from which quantiles of SSC and exceedence durations were extracted at various levels.  Newcombe and Jensen’s (1996) severity-of-ill-effects index was computed for a matrix of concentrations and continuous exposure periods at each station and year.  In addition to the gaging station data, cross-sections have been surveyed at many locations on both streams, and differences in mean elevation and cross-sectional area were computed between successive surveys at the same location.

WATERSHEDS

Freshwater Creek.  The Freshwater Creek watershed drains into the northern end of Humboldt Bay in Northern California just north of Eureka. The Redwood and Douglas-fir forested watershed trends southeast to northwest. The watershed is mainly underlain by Franciscan, Yager and Wildcat geological formations. Portions of the northeast watershed are composed of Franciscan melange formation. Up until recently, Maxxam Corporation was the major landowner in the Freshwater Creek Watershed. Since their bankruptcy, these areas have been taken over by Humboldt Redwoods Co. Salmon Forever maintains two continuous TTS monitoring stations in Freshwater Creek.  Station HHB is in the lower portion of the watershed at Howard Heights bridge and the FTR station is higher on the mainstem Freshwater Creek 400 yards above Freshwater Park. The watershed area above Site FTR covers 13.2 square miles. The watershed area draining to site HHB is 27.8 square miles. TTS Monitoring commenced at site FTR in 1999 and at site HHB in 2005.  The average suspended sediment yield from sites HHB (HY2005-2008) and FTR (HY 2003-2008) has been 285 and 470 tons per square mile, respectively.

Elk River. The Elk River Watershed drains into Humboldt Bay just south of Eureka. The watershed area is 56.1 square miles. The Redwood and Douglas-fir forested watershed also trends northwest to southeast. The main geologic units are the Wildcat Group underlain by the Yager Formation. Maxxam Corporation, Green Diamond Corporation, and the BLM / Headwaters Preserve were the primary landowners in Elk River watershed until recently when Maxxam lands were acquired by Humboldt Redwoods Co. Elk River is the largest watershed to drain into Humboldt Bay. Salmon-Forever operates two continuous TTS monitoring stations in Elk River. TTS monitoring commenced at sites SFM and KRW in HY 2003. Site KRW is located on the North Fork Elk River 1.0 miles above the confluence of North and South Fork Elk Rivers. The watershed area above site KRW is 22.2 square miles. Site SFM is located on the South Fork Elk River approximately 0.5 miles above the confluence. The watershed area above site SFM is 19.3 square miles. The average suspended sediment yield from sites SFM and KRW for HY 2003-2008 has been 807 and 499 tons per square mile, respectively.

GAGING STATION OPERATIONS

All four gaging stations are operated as described in the TTS Implementation guide (Lewis and Eads, 2009) and field sampling has been undertaken in accordance with the following Standard Operating Procedures provided with this data package.

- Depth-Integrated Sampling

- Discharge Measurements

- Field Instrumentation

- Turbidity Threshold Sampling

During the period HY03-08 each gaging station had a Campbell CR10X or CR510 data logger, an ISCO Model 3700, 6700 or 6712 pumping sampler; Druck 1830 pressure transducers were standard. Turbidity sensors were suspended from a bridge-mounted or bank-mounted boom. OBS-3 turbidity sensors were used prior to HY05 at KRW and SFM, and prior to HY04 at FTR. Beginning in HY05, DTS-12 sensors were standard, but an OBS-3 was substituted occasionally during malfunctions. The DTS-12 sensors generally produce higher quality data because they have built-in mechanical wipers that clean the optics before each reading.  The DTS-12 sensors also can record water temperature as well as turbidity.  If turbidity is to be analyzed as a measure of water quality, it is important to remember that these two types of sensors operate according to different principles and their output is not equivalent without adjustment (Lewis, 2007).  In developing relationships between turbidity and SSC, data from different sensor types are never combined without adjustment.  Once SSC has been determined, differences in sensor types are no longer relevant.

Site FTR had continuous rainfall data recorded by a Campbell TR525I tipping bucket rain gage through HY08.

LABORATORY OPERATIONS

Samples from water years HY03-06 were processed at the Sunnybrae Sediment Laboratory in Arcata, managed by Clark Fenton.  Samples from water years HY07-08 were processed at the Laboratory in Elk River, managed by Kristi Wrigley.  SSC is determined by vacuum filtration through tared 1-micron glass fiber filters. Filters are oven-dried at 105º C, cooled in a dessicator, and weighed on a Precisa XB-120-A balance to the nearest 0.0001 g. Sample water weight and sediment weight is used to calculate SSC in mg/L.  A subset of samples is washed through a 0.063 mm sieve prior to vacuum filtration for determination of sand content.  Laboratory methods are in accordance with the Standard Operating Procedures for Laboratory SSC provided with this data package.

CROSS SECTION SURVEYS

Stream cross sections have been established in low gradient reaches of Freshwater Creek and Elk River as a result of a number of different efforts over the years.

During 2001, cross sections were established by Ed Schillinger, licensed surveyor, on the North Fork of Elk River, the South Fork of Elk River, and along Elk River below the confluence. During 2002, Keith Barnard resurveyed the Schillinger cross sections. Salmon Forever volunteers began surveying Elk River cross sections during 1999 and have continued to date. Other cross section locations have been surveyed by the Army Corp of Engineers, ACOE (USGS gage site, 1957-67), Pacific Lumber Company and HSU student Bill Conroy.

In Freshwater Creek cross section were established in the low gradient reaches in conjunction with the FEMA flood mapping (circa 1975). Some ACOE sections have been resurveyed in 1999 by Cafferata of CalFire and at various intervals by Salmon Forever. HSU professor Andre Lehre established some sections in 1999 along the stream reach defined by the property of Dr. Terry Roelofs. Many of these have been resurveyed by Salmon Forever. Paul Bigelow, HSU student, established more cross sections during 1999 and 2000 along the Roelofs reach and at another study area upstream.

Analysis of cross sections includes comparisons of cross section area change between surveys for both the entire cross section surveyed and just the active channel area. These cross section scour and aggradation change calculations are then summed to provide an estimate of average channel change.

FILE FORMATS

Data files in a TTS database consist of plain ASCII text only.  These files have various extensions but are simply text files that can be opened with any text editor or easily be imported into any spreadsheet, database, or statistical program. See File Formats.doc for a description of the standard files in a TTS database.

PLOTTING THE DATA

The stage and turbidity data in the appended/corrected .flo data files can be plotted using the TTS Adjuster program, which can be started from the preceding hyperlink or from the Start TTS Adjuster shortcut.  When prompted for a starting month, enter August.  On the initial screen, click on the “Browse” button, and specify the “Data” directory of this package as the TTS Home directory.  After selecting a station, file, and start/end dates or dumps, click correspondingly on “Read Dates” or “Read Dumps” to view a plot.  Zooming in and out can be accomplished by dragging the mouse. Scatterplots of SSC and turbidity for the displayed time window can be obtained by clicking on the”Scatterplot” button. Additional instructions are found in the TTS Adjuster manual.

PRODUCTS

The primary data products of this report are water and sediment discharge for both storm events and water years, as well as a 10-minute record of flow and SSC for each gaging station.  Channel changes are also evaluated, and weekly time series plots of rainfall, turbidity and stage are provided.

In addition to the data products, this report includes an analysis of errors in suspended sediment loads, an analysis of trends in storm event loads, calculations of SSC distributions, computation of severity-of-ill-effects indexes relating to salmonids, and calculation of selected cross-section changes between 2001 and 2008.

Sediment loads

All sediment loads and associated sample sizes are tabled in the spreadsheet Event totals.xls in the “Data” folder.  Sediment loads were computed in the R software environment using R procedures developed for TTS, which are described at a general level in the TTS Implementation Guide (Lewis and Eads, 2009).  Detailed documentation including commands, scatterplots and estimated loads and statistics for alternative models, as well as graphs of discharge, turbidity, and SSC for each event are provided in this data package by water year within the Data Correction and Load Calculations folder in documents titled “…Event Load.doc”.  Similar documentation is provided for annual loads in documents titled “…Annual Load.doc”.  Annual loads were estimated by summing storm loads and inter-storm loads. Inter-storm loads were calculated using a single relationship between SSC and turbidity, and if any turbidity data were missing, a single relationship between SSC and discharge was employed wherever turbidity was missing.

Sand fractions

Sand fractions and associated sample sizes are tabled in the spreadsheet Event totals.xls in the “Data” folder.  The proportion of sand in the annual load was calculated by first computing the annual load of fines (< 0.063 mm), then subtracting the annual load of fines from the total annual load to arrive at annual sand load, and finally dividing by the total annual load.  The reason for arriving at the sand loads by subtraction is that fine sediment is much better correlated with turbidity than sand is, therefore fine sediment can be more reliably estimated using the turbidity data.  Both the fines and total loads were estimated from the subset of samples for which sand and fine fractions were available (about 35% of all samples).   The fines and total loads were each calculated using a single relationship between SSC and turbidity, and if any turbidity data were missing, an additional relationship between SSC and discharge was employed.  Detailed documentation including commands, scatterplots, estimated loads, and statistics for alternative models are provided in this data package by water year within the Data Correction and Load Calculations folder in documents titled and “…Annual Sand Load.doc.

Scatterplots showing sand fraction plotted against total SSC and water discharge are provided in the document Sand Fraction Plots.doc.  There appear to be no relationships between sand fraction and either SSC or discharge at the Elk River stations, SFM and KRW.  There appear to be weak positive correlations, at least in 2006 and 2008, between sand fraction and both SSC and discharge at the Freshwater stations, FTR and HHB.

Analysis of errors

A detailed analysis of errors in the annual suspended sediment loads was undertaken and is described in the document Error Analysis.doc.  Total error in the annual loads is estimated to average about 13%.  TTS successfully reduced errors due to sampling and modeling so that they are smaller than the uncertainty in discharge. By contrast, in typical sampling programs with no turbidity monitoring and infrequent storm sampling, errors due to sampling and modeling dominate and can easily reach or exceed 50% of the estimated load.

Trend analysis

The storm event loads can be used to investigate trends in sediment by computing a regression of storm event loads between two stations then looking for a trend in residuals versus time.  This has been done for KRW/SFM, HHB/FTR, FTR/ SFM, and FTR/KRW.  Among tests for linear trends, none are significant at the 0.05 level.  Results of the analysis are contained in the file Trend Analysis Results.doc. Results along with the commands used in the analysis are contained in the file Trend Analysis with Commands.doc.  Equivalent trend analyses for KRW/SFM and HHB/FTR are also shown in the worksheets of Event totals.xls.

Near-continuous record of SSC

In the process of estimating sediment loads, SSC was estimated for each 10-minute interval during the study period, except when stations were down for the summer.  The result has been compiled into data files that contain the final flow, turbidity, and estimated SSC at 10-minute intervals. These data files are located in the Ten minute SSC folder under “Data”.  This product is highly unusual (perhaps the only data set of its kind in the world) in that the SSC estimates are not simply a rescaled version of turbidity or flow; instead many relationships representative of individual storms or sections of storms were used to produce the estimated SSC for each station and year, resulting in relatively accurate, nearly continuous representations of SSC that have considerable value for scientific purposes such as model validation and sampling simulation.  These SSC data are a new product and many analyses remain to be performed.

Further details about the contents of these files are given in File Formats.doc.  In addition, all the R commands needed to recreate these output files have been saved in the R commands folder within the “Ten minute SSC” folder.

SSC statistics and graphs

Tables and graphs summarizing the 10-minute SSC data are found in the document SSC Graphs and Stats.doc.  Because the stations have been started up and closed down at varying dates, but often opening in November and closing in May, all statistics are computed for a winter season from Dec. 1 through Apr 30.  The tables in the document show

- SSC exceeded various percentages of the winter season

- Total hours that various concentrations were exceeded for each station and year.

- Maximum continuous hours that various concentrations were exceeded for each station and year.

The graphs in SSC Graphs and Stats.doc show the cumulative frequency distributions of SSC for each year by station. In most years the ordering of stations by SSC from lowest to highest is FTR, HHB, KRW, and SFM.

Newcombe and Jensen’s severity of ill-effect scores

The table of maximum annual duration of continuous exposure at different levels of SSC is interesting to compare with severity-of-stress indexes developed by Newcombe and MacDonald (1991) and Newcombe and Jensen (1996), which relate the duration and concentration of sediment exposure to stress on aquatic organisms.  The latter publication includes 4 models of severity for different groupings of salmonid life stages.  The severity of ill-effect scores can be computed directly from our tables of annual maximum continuous exposure, and we have done so in Severity index tables.doc.

We did not have SSC data for the specific size fractions to which the models are said to pertain, therefore our calculations are based on the total SSC without regard to grain size.  Our SSC data exclude some particles finer than 1 but may include sizes coarser than 250; the former bias is more important at low concentrations and the latter more important at high concentrations.  It might be possible to improve these calculations using our sand fraction data, since our sand break at 63m is not far from the 75m break used to define the upper limit of particle sizes for models 3 and 4.  Adjusting the concentrations would be subject to significant errors, however, because there are no strong relationships between sand fraction and either total SSC or discharge.  See Sand Fraction Plots.doc.

Applying Newcombe and Jensen’s model 2 for adult salmonids we find a maximum severity of 8.8 for our period of record occurring at SFM in HY03.  A severity of 9 is defined as a sublethal effect associated with reduced growth and population density. A severity of 8 was exceeded at SFM in all years except HY04.  The same severity of 8 was exceeded in 3 of 6 years at KRW, 2 of 6 years at FTR, and 0 of 4 years at HHB.  Newcombe and Jensen (1996) defined severity 8 as indicating major physiological stress with long-term reductions in feeding success.  A severity of 7, indicating moderate habitat degradation and impaired homing, was exceeded every year at all stations.

Newcombe and Jensen’s model 3 indicates that conditions for juvenile salmonids are not as stressful as for adults.  Annual maximum severity scores for juveniles varied from 7.4 to 8.6 at SFM, 7.3 to 7.9 at KRW, 6.8 to 7.6 at FTR, and 6.7 to 7.2 at HHB.

Their model 4 for salmonid eggs and larvae assigns a maximum severity of 12.6 to conditions occurring at SFM in HY06 and HY08.  A severity of 12 is defined as a lethal effect with 40-60% mortality and a severity of 13 is associated with 60-80% mortality.  A severity of 11, associated with 20-40% mortality, was exceeded at SFM in all years but HY04, and at KRW in all years, but was only exceeded at the Freshwater stations in HY06.  A severity of 10 was exceeded every year at all stations, suggesting 0-20% mortality, increased predation, and moderate to severe habitat degradation.

Turbidity

Continuous turbidity is a secondary product of this data set.  The turbidity data have gaps and two different sensor types were used.  However, the gaps are relatively few and of limited duration, and could be filled with estimates using relationships between turbidity and SSC.  In addition, although the OBS-3 and DTS-12 sensors do not produce equivalent output, it is possible to convert the OBS-3 turbidity to DTS equivalents or vice versa.  Such conversions are occasionally prone to large errors (Lewis, 2007), because they depend upon the sediment being measured. However using the Lewis (2007) data set, formulas have been developed specifically for each of the Salmon Forever watersheds.  These formulas were applied by Klein et al. (2008) to an earlier (substantially similar) version of these data sets through HY05 when they compared turbidity duration and exceedence levels in 28 north coast watersheds.

Rainfall plots

Cumulative rainfall measured at Station FTR has been plotted beneath stage and turbidity from stations FTR, KRW, and SFM for years 2003, 2006, 2007, and station HHB for years 2006 and 2007.  Generally, each plot represents a 6 or 7 day storm period. The plots display stage and turbidity data that have been reviewed and corrected using procedures described in the TTS Implementation Guide. They are included in this package to facilitate exploratory analysis of the lag times between rainfall bursts and responses of turbidity and discharge. They are located in the folder Rainfall plots under individual subfolders for each year.

Cross section changes over time

Changes in cross-sectional area and bed elevation have been determined through a series of surveys at 23 cross-sections on the mainstem, and North and South Forks of Elk River. The survey data and analyses can be found in the Elk Cross Sections subfolder under “Data”.  Tables and graphs in Elk Cross Section Changes.xls show the aggradation that has occurred. Bed elevation in the active channel has increased an average of 0.64 ft in the North Fork at the 5 cross sections that were measured in both 2001 and 2007, and aggradation has reduced the common surveyed cross-sectional areas by an average of 19 ft2.  (The common surveyed area is wider than the active area, and only 2 of the 5 cross sections aggraded significantly).  In the most recently surveyed South Fork cross sections, the mean bed elevation in the active channel increased 1.05 ft from 2001-2005 at SA1 and 0.93 ft from 2001-2007 at SA5. The corresponding changes in the common surveyed cross-sectional areas are 42 ft2 and 100 ft2, respectively. Changes in the main stem appear to be smaller. Of the three main stem cross sections that were surveyed in 2001 or 2002 and 2007, the mean bed elevation in the active channel increased 0.19 ft, and aggradation has reduced the common surveyed cross-sectional areas by an average of 24 ft2.

REFERENCES

Klein, R.D., Trush, W.J., and M. Buffleben. 2008. Watershed condition, turbidity, and implications for anadromous salmonids in north coastal California streams. Report to North Coast Regional Water Quality Control Board, Santa Rosa, CA. 105 p.

Lewis, Jack. 2007.  Comparisons of turbidity data collected with different instruments.  Report on a cooperative agreement between the California Department of Forestry and Fire Protection and USDA Forest Service--Pacific Southwest Research Station (PSW Agreement # 06-CO-11272133-041); July 6, 2007.

Lewis, Jack and Rand E.Eads. 2009. Implementation Guide for Turbidity Threshold Sampling: Principles, Procedures, and Analysis.  General Technical Report PSW-GTR-212, U.S. Department of Agriculture, Forest Service, Pacific Southwest Research Station.

Newcombe, C.P. and D. D. Macdonald. 1991. Effects of suspended sediments on aquatic ecosystems.  North American Journal of Fisheries Management 11: 72-82.

Newcombe, C.P. and J.O.T. Jensen. 1996. Channel suspended sediment and fisheries: a synthesis for quantitative assessment of risk and impact.  North American Journal of Fisheries Management 16: 693-727.