KRW07 Event Loads

stn,storm,sdate,stime,edate,etime

KRW,701,061213,0500,061214,2140   93531 kg  Very good

KRW,702,061225,2220,061229,1200  607840 kg  Excellent

KRW,703,070103,0600,070106,1330  649458 kg  Very good

KRW,704,070210,0510,070213,1200  550687 kg  Very good

KRW,705,070220,1450,070224,1420 1975972 kg  Very good

KRW,706,070224,1450,070227,0840  572629 kg  Excellent

KRW,707,070227,0850,070303,1200  288123 kg  Good

KRW,708,070326,1020,070329,1200  322224 kg  Very good

-----------------------------------------------------

SUM of events:                  5060464 kg  Very good

-----------------------------------------------------

krw07.flo <- read.flo("krw",07)

krw07.lab <- read.lab("krw",07)

mismatches("krw",07)

krw07.sed <- merge.flo("krw",07)

Storm 701: dump 8: KRW,701,061213,0500,061214,2140

Excellent stage and turbidity data and very good sample coverage

Might include bots 7 and 8 to extend recession coverage

four()

turbsscplot("krw",07,061213,0500,061214,2140)

turbsscplot("krw",07,061213,0500,061214,2140,type="power")

turbsscplot("krw",07,dump=8,bot=1:8)

turbsscplot("krw",07,dump=8,bot=1:8,type="power")

turb1 <- turbsrc("krw",07,061213,0500,061214,2140)

turb2 <- turbsrc("krw",07,061213,0500,061214,2140,type="power")

turb3 <- turbsrc("krw",07,061213,0500,061214,2140,dump=8,bot=1:8)

turb4 <- turbsrc("krw",07,061213,0500,061214,2140,dump=8,bot=1:8,type="power")

turb5 <- turbsrc("krw",07,061213,0500,061214,2140,type="pairs")

total(turb1,turb2,turb3,turb4,turb5)

start            end   type surr     yhat n    r2      s    cv

turb1 12/13/06 05:00 12/14/06 21:40 linear turb 92080.67 6 0.951 20.905 8.170

turb2 12/13/06 05:00 12/14/06 21:40  power turb 92678.50 6 0.958 19.240 6.730

turb3 12/13/06 05:00 12/14/06 21:40 linear turb 93530.67 8 0.963 17.655 5.839

turb4 12/13/06 05:00 12/14/06 21:40  power turb 92719.73 8 0.969 16.108 5.351

turb5 12/13/06 05:00 12/14/06 21:40  pairs turb 94343.37 6    NA     NA    NA

# Range in estimates is less than 2% of the mean

# turb5 doesn't represent the peak well

# There is a risk that the peak is underestimated based on sample 3

# Hence my preference for turb3

ttsplot("krw",07,turb3)

total(turb3)

start            end   type surr     yhat n    r2      s    cv

turb3 12/13/06 05:00 12/14/06 21:40 linear turb 93530.67 8 0.963 17.655 5.839

Total load = 93531 kg,  CV = 5.84 %  Very good

Storm 702: dumps 8-9: KRW,702,061225,2220,061229,1200

Large event with two peaks

Excellent stage and turbidity data and sample coverage

four()

turbsscplot("krw",07,061225,2220,061229,1200)

# a bit of hysteresis on second peak, probably not worth modeling

turbsscplot("krw",07,061225,2220,061229,1200,type="pairs")

turbsscplot("krw",07,061225,2220,061229,1200,type="power")

turbsscplot("krw",07,061225,2220,061229,1200,type="logxy")

turb1 <- turbsrc("krw",07,061225,2220,061229,1200)

turb2 <- turbsrc("krw",07,061225,2220,061229,1200,type="pairs")

turb3 <- turbsrc("krw",07,061225,2220,061229,1200,type="power")

turb4 <- turbsrc("krw",07,061225,2220,061229,1200,type="logxy",bias="qmle")

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat  n    r2      s    cv

turb1 12/25/06 22:20 12/29/06 12:00     linear turb 606144.2 17 0.970 37.178 3.416

turb2 12/25/06 22:20 12/29/06 12:00      pairs turb 596224.7 17    NA     NA    NA

turb3 12/25/06 22:20 12/29/06 12:00      power turb 604066.1 17 0.971 36.392 3.102

turb4 12/25/06 22:20 12/29/06 12:00 logxy:qmle turb 608378.3 17 0.946  0.226 6.230

# Range in estimates is 2% of the mean

# Linear and power model are both excellent

# Pairs model is likely lower because other models overestimate recession.

# Best approach would fit two models because of separation at low flow

four()

turbsscplot("krw",07,061225,2220,061227,1020,type="power")

turbsscplot("krw",07,061225,2220,061227,1020,type="logxy")
turbsscplot("krw",07,061227,0950,061229,1200,type="power")

turbsscplot("krw",07,061227,0950,061229,1200,type="logxy")

turb5 <- turbsrc("krw",07,061225,2220,061227,1020,type="power")

turb6 <- turbsrc("krw",07,061227,1030,061229,1200,061227,0950,061229,1200,type="logxy")

ttsplot("krw",07,turb5,turb6)

total(turb5,turb6)

start            end       type surr     yhat n    r2      s    cv

turb5 12/25/06 22:20 12/27/06 10:20      power turb 378460.3 9 0.973 38.236 3.776

turb6 12/27/06 10:30 12/29/06 12:00 logxy:mvue turb 229379.6 9 0.990  0.119 4.276

Total load = 607840 kg,  CV = 2.85 %  Excellent

Storm 703: dump 9-10: KRW,703,070103,0600,070106,1330

Very good stage data and turbidity data.

Very good sample coverage.

Initial filter weights were lost for all but first 3 samples, replaced by an average.

Field notes indicated a possible bottle labeling problem.  First bottle of dump 1 corresponds

to bottle 20 of dump 9 in Campbell data and FLO file. Needed to decrement bottle numbers in ISC

dump 10 to match FLO file.  Empty bottle 3 becomes empty bottle 2. Last bot becomes 16.  This

improved things a great deal.  Lost filter weights are not a big problem.

four()

turbsscplot("krw",07,070103,0600,070106,1330)

turbsscplot("krw",07,070103,0600,070106,1330,type="power")

turbsscplot("krw",07,070103,0600,070106,1330,type="loess",degree=2)

turbsscplot("krw",07,070103,0600,070106,1330,type="pairs")

turb1 <- turbsrc("krw",07,070103,0600,070106,1330)

turb2 <- turbsrc("krw",07,070103,0600,070106,1330,type="power")

turb3 <- turbsrc("krw",07,070103,0600,070106,1330,type="loess",degree=2)

turb4 <- turbsrc("krw",07,070103,0600,070106,1330,type="pairs")

total(turb1,turb2,turb3,turb4)

start            end   type surr     yhat  n    r2      s    cv

turb1 01/03/07 06:00 01/06/07 13:30 linear turb 640629.3 16 0.954 46.018 4.575

turb2 01/03/07 06:00 01/06/07 13:30  power turb 622235.3 16 0.968 39.615 3.776

turb3 01/03/07 06:00 01/06/07 13:30  loess turb 649458.5 16 0.978 34.240    NA

turb4 01/03/07 06:00 01/06/07 13:30  pairs turb 650493.3 16    NA     NA    NA

# Range of estimates is 4.6%

ttsplot("krw",07,turb3)

total(turb3)

start            end  type surr     yhat  n    r2     s cv

turb3 01/03/07 06:00 01/06/07 13:30 loess turb 649458.5 16 0.978 34.24 NA

Total load = 649458 kg   Very good

Storm 704: dump 12: KRW,704,070210,0510,070213,1200

Excellent turbidity and stage data and excellent sample coverage

four()

turbsscplot("krw",07,070210,0510,070213,1200)

# Eliminate outlier bottles 17 and 21

turbsscplot("krw",07,dump=12,bot=c(10:16,18:20,22:24))

turbsscplot("krw",07,dump=12,bot=c(10:16,18:20,22:24),type="power")

turbsscplot("krw",07,dump=12,bot=c(10:16,18:20,22:24),type="logxy")

turb1 <- turbsrc("krw",07,070210,0510,070213,1200,dump=12,bot=c(10:16,18:20,22:24))

turb2 <- turbsrc("krw",07,070210,0510,070213,1200,dump=12,bot=c(10:16,18:20,22:24),type="power")

turb3 <- turbsrc("krw",07,070210,0510,070213,1200,dump=12,bot=c(10:16,18:20,22:24),type="logxy",bias="qmle")

turb4 <- turbsrc("krw",07,070210,0510,070213,1200,type="logxy",bias="qmle")

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat  n    r2      s    cv

turb1 02/10/07 05:10 02/13/07 12:00     linear turb 547289.7 13 0.978 39.023 5.136

turb2 02/10/07 05:10 02/13/07 12:00      power turb 549509.5 13 0.980 36.899 4.056

turb3 02/10/07 05:10 02/13/07 12:00 logxy:qmle turb 550686.7 13 0.984  0.132 3.827

turb4 02/10/07 05:10 02/13/07 12:00 logxy:qmle turb 554032.0 15 0.917  0.281 7.631

# Range of estimates with reduced sample is 1%

# The full sample set gives only a scarcely higher estimate

ttsplot("krw",07,turb3)

total(turb3)

start            end       type surr     yhat  n    r2     s    cv

turb3 02/10/07 05:10 02/13/07 12:00 logxy:qmle turb 550686.7 13 0.984 0.132 3.827

Total load = 550687 kg,  CV = 3.83 %  Very good

Storm 705: dump 14-15: KRW,705,070220,1450,070224,1420

Very large event.

Excellent turbidity data and excellent sample coverage.

Stage data were NOT adjusted to match OR (difference = 0.078)

four()

turbsscplot("krw",07,070220,1450,070224,1420)

turbsscplot("krw",07,070220,1450,070224,1420,type="power")

turbsscplot("krw",07,070220,1450,070221,1310,type="pairs")

turbsscplot("krw",07,070221,1320,070224,1420)

turb0 <- turbsrc("krw",07,070220,1450,070224,1420)

total(turb0)

start            end   type surr    yhat  n   r2      s    cv

turb0 02/20/07 14:50 02/24/07 14:20 linear turb 2031211 17 0.93 95.857 5.181

Total load = 2031211 kg,  CV = 5.18 %

endpoints(turb0)

start       end

-16.75513  43.26879

turb0a <- turbsrc("krw",07,070220,1450,070224,1420,type="power")

turb0b <- turbsrc("krw",07,070220,1450,070224,1420,type="logxy",bias="qmle")

total(turb0,turb0a,turb0b)

start            end       type surr    yhat  n    r2      s    cv

turb0  02/20/07 14:50 02/24/07 14:20     linear turb 2031211 17 0.930 95.857 5.181

turb0a 02/20/07 14:50 02/24/07 14:20      power turb 2007210 17 0.936 92.199 4.891

turb0b 02/20/07 14:50 02/24/07 14:20 logxy:qmle turb 1984516 17 0.918  0.264 7.461

# Break into two parts

turb1 <- turbsrc("krw",07,070220,1450,070221,1310,type="pairs")

turb2 <- turbsrc("krw",07,070221,1320,070224,1420)

ttsplot("krw",07,turb1,turb2)

total(turb1,turb2)

start            end   type surr    yhat  n    r2     s   cv

turb1 02/20/07 14:50 02/21/07 13:10  pairs turb  290374  6    NA    NA   NA

turb2 02/21/07 13:20 02/24/07 14:20 linear turb 1685598 11 0.986 34.23 2.49

Total load = 1975972 kg  Very good to excellent

# Only 2% less than power regression estimate (turb0a)

# Uncertainty around peak isn't very sensitive to how it's handled

Storm 706: dump 15: KRW,706,070224,1450,070227,0840

Very good turbidity data and very good sample coverage

Stage data look good but were NOT adjusted to match Feb 23 OR (diff=0.078)

four()

turbsscplot("krw",07,070224,1450,070227,0840)

turbsscplot("krw",07,070224,1450,070227,0840,type="power")

turbsscplot("krw",07,070224,1450,070227,0840,type="logxy")

turbsscplot("krw",07,070224,1450,070227,0840,type="loess")

turb1 <- turbsrc("krw",07,070224,1450,070227,0840)

turb2 <- turbsrc("krw",07,070224,1450,070227,0840,type="power")

turb3 <- turbsrc("krw",07,070224,1450,070227,0840,type="logxy",bias="qmle")

turb4 <- turbsrc("krw",07,070224,1450,070227,0840,type="loess")

turb5 <- turbsrc("krw",07,070224,1450,070227,0840,type="pairs")

total(turb1,turb2,turb3,turb4,turb5)

start            end       type surr     yhat  n    r2      s    cv

turb1 02/24/07 14:50 02/27/07 08:40     linear turb 554825.6 10 0.971 27.126 4.429

turb2 02/24/07 14:50 02/27/07 08:40      power turb 554502.7 10 0.982 21.827 3.134

turb3 02/24/07 14:50 02/27/07 08:40 logxy:qmle turb 565281.8 10 0.979  0.087 2.803

turb4 02/24/07 14:50 02/27/07 08:40      loess turb 572629.2 10 0.984 21.694    NA

turb5 02/24/07 14:50 02/27/07 08:40      pairs turb 567762.8 10    NA     NA    NA

# Range of estimates is 3.3%

# Loess is highest because it's the best fit to bottle 6 w/o underestimating low end

ttsplot("krw",07,turb4)

total(turb4)

start            end  type surr     yhat  n    r2      s cv

turb4 02/24/07 14:50 02/27/07 08:40 loess turb 572629.2 10 0.984 21.694 NA

Total load = 572629 kg    Excellent

Storm 707: dump 15: KRW,707,070227,0850,070303,1200

Small event, very good stage data

Good turbidity data (but noise was smoothed on recession)

Only 3 samples, but can supplement from preceding samples in storm 706

four()

turbsscplot("krw",07,dump=15,bot=9:14)

turbsscplot("krw",07,dump=15,bot=9:14,type="power")

turbsscplot("krw",07,dump=15,bot=9:14,type="loess",degree=2)

turbsscplot("krw",07,dump=15,bot=7:14,type="loess",degree=2)

turb1 <- turbsrc("krw",07,070227,0850,070303,1200,dump=15,bot=9:14)

turb2 <- turbsrc("krw",07,070227,0850,070303,1200,dump=15,bot=9:14,type="power")

turb3 <- turbsrc("krw",07,070227,0850,070303,1200,dump=15,bot=9:14,type="logxy",bias="qmle")

turb4 <- turbsrc("krw",07,070227,0850,070303,1200,dump=15,bot=9:14,type="loess",degree=2)

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat n    r2     s    cv

turb1 02/27/07 08:50 03/03/07 12:00     linear turb 262757.7 6 0.976 8.819 5.278

turb2 02/27/07 08:50 03/03/07 12:00      power turb 261376.0 6 0.980 8.142 4.245

turb3 02/27/07 08:50 03/03/07 12:00 logxy:qmle turb 269800.2 6 0.971 0.069 3.415

turb4 02/27/07 08:50 03/03/07 12:00      loess turb 288122.7 6 0.999 2.433    NA

# Range of estimates is 10% with loess being the extreme

# But loess is the best fit and it's not an overfit

ttsplot("krw",07,turb4)

total(turb4)

start            end  type surr     yhat n    r2     s cv

turb4 02/27/07 08:50 03/03/07 12:00 loess turb 288122.7 6 0.999 2.433 NA

Total load = 288123 kg  Good

Storm 708: dump 16: KRW,708,070326,1020,070329,1200

Isolated small event with good stage data

Turbidity data very good except for easily correctible spikes

Very good sample coverage

four()

turbsscplot("krw",07,070326,1020,070329,1200)

# Have to model the hystereis

turbsscplot("krw",07,070326,1020,070329,1200,type="pairs")

turbsscplot("krw",07,070326,1020,070327,1350)

turbsscplot("krw",07,070327,1350,070329,1200,type="loess",degree=2)

turb1 <- turbsrc("krw",07,070326,1020,070329,1200)

turb2 <- turbsrc("krw",07,070326,1020,070329,1200,type="pairs")

turb3 <- turbsrc("krw",07,070326,1020,070327,1350)

turb4 <- turbsrc("krw",07,070327,1400,070329,1200,type="loess",degree=2)

turb5 <- turbsrc("krw",07,070327,1400,070329,1200,type="power")   # to get a CV

total(turb1,turb2)

start            end   type surr     yhat  n    r2      s    cv

turb1 03/26/07 10:20 03/29/07 12:00 linear turb 326435.3 12 0.954 44.567 5.602

turb2 03/26/07 10:20 03/29/07 12:00  pairs turb 323695.4 12    NA     NA    NA

total(turb3,turb5)

start            end   type surr      yhat n    r2      s    cv

turb3 03/26/07 10:20 03/27/07 13:50 linear turb  99610.05 4 0.997 15.357 2.618

turb5 03/27/07 14:00 03/29/07 12:00  power turb 219240.24 8 0.995 14.066 2.171

Total load = 318850 kg,  CV = 1.7 %

ttsplot("krw",07,turb3,turb4)

total(turb3,turb4)

start            end   type surr      yhat n    r2      s    cv

turb3 03/26/07 10:20 03/27/07 13:50 linear turb  99610.05 4 0.997 15.357 2.618

turb4 03/27/07 14:00 03/29/07 12:00  loess turb 222614.34 8 0.999  8.486    NA

Total load = 322224 kg  Very good (would be excellent if more samples)