HHB rating curve analysis:

hhbq <- read.csv("hhb/HHB Q summary.csv")

> hhbq

HY          ID       Date Stage       Q Code

1  2000  12/10/1999 12/10/1999  2.46  114.30    9

2  2000   2/28/2000  2/28/2000  3.87  201.60    9

3  2001  10/29/2000 10/29/2000  1.12    4.30    0

4  2001    3/3/2001   3/3/2001  1.70   30.21    0

5  2001   3/11/2001  3/11/2001  1.45   16.03    0

6  2002   9/16/2001  9/16/2001  0.91    0.21    0

7  2002  11/23/2001 11/23/2001  1.52   16.83    0

8  2002   1/22/2002  1/22/2002  2.99  228.38    0

9  2002   1/26/2002  1/26/2002  3.54  289.10    0

10 2002 2/23/2002 A  2/23/2002  5.17  643.99    0

11 2002   2-23-02 B  2/23/2002  6.98  979.41    0

12 2003   11/9/2002  11/9/2002  1.38    8.74    0

13 2003    1/5/2003   1/5/2003  2.30   83.16    0

14 2003   1/18/2003  1/18/2003  1.81   37.10    0

15 2003   3/15/2003  3/15/2003  4.73  496.02    0

16 2003   4/25/2003  4/25/2003  7.65  885.14    9

17 2004  10/25/2003 10/25/2003  0.97    0.72    0

18 2004   11/8/2003  11/8/2003  1.11    2.41    0

19 2004   1/11/2004  1/11/2004  1.90   40.65    0

20 2005   9/19/2004  9/19/2004  0.83    0.54    0

21 2005  12-15-04 A 12/15/2004  1.51   14.57    0

22 2005  12-15-04 B 12/15/2004  1.51   12.68    0

23 2005    1/4/2005   1/4/2005  2.65   79.55    0

24 2005    1/5/2005   1/5/2005  2.23   54.16    0

25 2005    1/6/2005   1/6/2005  2.15   40.93    0

26 2005   3-20-05 A  3/20/2005  2.73   80.92    0

27 2005   3-20-05 B  3/20/2005  7.37  705.81    9

28 2005   3/21/2005  3/21/2005  3.99  283.14    0

29 2005   3-29-05 a  3/29/2005  9.57 1159.57    8

30 2005  3-29-05 ov  3/29/2005  9.57 1265.65    0

31 2005   3-29-05 b  3/29/2005  6.65  350.75    0

32 2006  10/29/2005 10/29/2005  1.32    2.14    0

33 2007   9/10/2006  9/10/2006  1.21    0.51    0

34 2007  11/18/2006 11/18/2006  1.60    8.67    0

35 2007  12/29/2006 12/29/2006  2.66   70.93    0

36 2007   2/22/2006  2/22/2006  6.17  651.13    0

Codes

----------------------------------

9=poor

8=does not include floodplain flow

0=fair or better

----------------------------------

# Exclude code 8 data

hhbq <- hhbq[hhbq$Code != 8, ]

# Plot the data using last digit of water year

hhbq$sym <- as.character(hhbq$HY %% 1000)

attach(hhbq)

plot(Stage, Q, type="p", pch=sym)

Outlier datum is 3-29-05 b: (6.65,350.75)

Remarks from electronic data sheet:

1: "cell width corrected"

2: "used 0.6 method by mistake"

The value was corrected to 344.09 and included in Clark's "final 0 intercept" model. I'm excluding it.  It's too much of an anomaly.

hhbq <- hhbq[ID != "3-29-05 b", ]

detach(hhbq)

attach(hhbq)

plot(Stage, Q, type="p", pch=sym)

# Exclude "poor" measurements from smooth model and display them in red

scatter.smooth(Stage[Code!=9], Q[Code!=9], type="p", pch=sym[Code!=9])

points(Stage[Code==9], Q[Code==9], pch=sym[Code==9], col=2)

Suggests that 2002 may have been higher for a given stage.

# Plot only 2003-2007 data

detach(hhbq)

attach(hhbq[hhbq$HY > 2002, ])

scatter.smooth(Stage[Code!=9], Q[Code!=9], type="p", pch=sym[Code!=9],span=0.4,degree=2)

points(Stage[Code==9], Q[Code==9], pch=sym[Code==9], col=2)

This suggests two linear segments. It's not clear that the "poor" measurements should be excluded, but I will do so.

Fit a segment regression using function segmented from package segmented.

detach(hhbq)

attach(hhbq[ HY > 2002 & Code !=9, ])

obj <- lm(Q ~ Stage)

segreg <- segmented(obj, Z=Stage, psi=3)

Call: segmented.lm(obj = obj, Z = Stage, psi = 3)

Meaningful coefficients of the linear terms:

(Intercept)       Stage     U.Stage

-64.92203    57.76400   109.23012

Estimated Break-Point(s) for the variable(s) Stage : 2.507

detach(3)

attach(hhbq[hhbq$HY > 2002, ])

plot(Stage[Code!=9], Q[Code!=9], type="p", pch=sym[Code!=9])

abline(-64.922,57.764,col=2)

abline(-64.922-2.507*109.23, 57.764+109.23,col=2)

The lower segment shouldn't really be linear.  Fit a power function below 2.5 and a line above 2.5.

detach(2)

attach(hhbq)

# I had to increase the cutpoint to 3.5 in next statement to get an intersection

data1 <- hhbq[HY > 2002 & Code !=9 & Stage <= 3.5, ]

data2 <- hhbq[HY > 2002 & Code !=9 & Stage > 2.5, ]

detach(hhbq)

fit0 <- lm(log(Q) ~ log(Stage), data= data1)

(Intercept)   log(Stage)

-0.07762      5.36747

alldata <- rbind(data1,data2)

plot(alldata$Stage,alldata$Q,pch=alldata$sym,cex=0.6)

fit1 <- nls(Q ~ a*(Stage)^b, data=data1, start=list(a=exp(0.07), b=4.8))

x0 <- seq(0,3.5,0.1)

y0 <- predict(fit1,newdata=data.frame(Stage=x0))

lines(x0,y0)

fit2 <- lm(Q ~ Stage, data=data2)

abline(fit2)

Trouble is, if you look closer the power function is a poor fit (see next graph).

Clark's original quadratic fit had the same problem.  It just wasn't visible at the scale he used.

Here's a kind of kludge that fits a little better.

attach(hhbq)

# Increase the cutpoint to 3.5 in next statement to get an intersection

data1 <- hhbq[HY > 2002 & Code !=9 & Stage <= 5, ]

data2 <- hhbq[HY > 2002 & Code !=9 & Stage > 2.5, ]

detach(hhbq)

alldata <- rbind(data1,data2)

plot(alldata$Stage,alldata$Q,pch=alldata$sym,cex=0.6,xlim=c(0,3),ylim=c(0,100))

fit1 <- lm(log(Q) ~ log(Stage), data= data1)

y1 <- exp(predict(fit1,newdata=data.frame(Stage=x0)))

lines(x0,y1)

fit2 <- lm(Q ~ Stage, data=data2)

abline(fit2)

# The curve is nearly tangent to the line.  Doesn't fit some of the

# 2003 and 2004 data points, but that won't hurt in 2005.  (Maybe the

# problem is trying to include too many years in one model.)

# Now zoom back out

plot(alldata$Stage,alldata$Q,pch=alldata$sym,cex=0.6)

lines(x0,y1)

abline(fit2)

The line and the curve come to within 0.996 cfs but don't intersect.

Since the curve is a bit of a kludge anyway I'll adjust the intercept

of fit1 to get a tangent point.

fit1

-----------------------------------------

Call:

lm(formula = log(Q) ~ log(Stage), data = data1)

Coefficients:

(Intercept)   log(Stage)

0.2689       4.2415

-----------------------------------------

Lowering the intercept to 0.2604 makes the curves tangent (to within 0.009 cfs). The closest point is at stage=2.891.

plot(alldata$Stage,alldata$Q,pch=alldata$sym,cex=0.6,xlim=c(0.5,3),ylim=c(0,120))

x1 <- seq(0.5,3,0.01)

y1 <- exp(0.2604 + 4.2415*log(x1))

lines(x1,y1)

abline(fit2,col=2)

This is indistinguishable from the plot above that used the original coefficients.

New multiplier is exp(0.2604) = 1.2974

FINAL equations for HY03-HY07 (although HY05 is the first TTS year)

---------------------------------------

q = 1.2974*s^4.2415        for s<=2.891

q = -379.751 + 171.8645*s  for s>2.891

# Compare with Clark's original curve

plot(alldata$Stage,alldata$Q,pch=alldata$sym,cex=0.6)

x1 <- seq(0,2.89,0.01)

y1 <- 1.2974 * x1^4.2415

lines(x1,y1)

x2 <- seq(2.9,10,0.01)

y2 <- -379.751 + 171.8645*x2

lines(x2,y2)

x3 <- c(x1,x2)

y3 <- 13.122*x3^2 - 7.7527*x3

lines(x3,y3,col=2)

points(7.37,705.81,col=2)

points(6.65,350.75,col=2)

points(9.57,1159.57,col=2)

legend(locator(1),c("Lewis","Clark"),col=1:2,lty=1)

This is a big change. Why so different?  Clark used only 2005 data, didn't include the floodplain flow in the highest measurement, and included 2 points that I excluded:

"3-20-05 B" (poor quality)

"3-29-05 b" (outlier below both curves)

The 3 points just referred to are shown in red in the above plot.