SFM 2008 Annual load estimation

sfm08.flo <- read.flo("sfm",08)

sfm08.lab <- read.lab("sfm",08)

sfm08.sed <- merge.flo("sfm",08)

nonstorms <- drop.storms("sfm",08)

dim(nonstorms)

[1] 33733     4

head(nonstorms, 5)

chr     q turb turbcode

1 (08/12/07 15:30:00) 5.465   NA        2

2 (08/12/07 15:40:00) 5.465   NA        2

3 (08/12/07 15:50:00) 5.465   NA        2

4 (08/12/07 16:00:00) 5.438   NA        2

5 (08/12/07 16:10:00) 5.465   NA        2

> table(nonstorms$turbcode)

0     2     5     6     7

4730 22086  5166  1748     3

22086 values of bad turbidity out of 33733 intervals to be estimated

# This reflects all the fouling of the OBS-3

trb08.flo <- nonstorms[!is.na(nonstorms$turb), ]

flo08.flo <- nonstorms[is.na(nonstorms$turb), ]

dim(trb08.flo)

[1] 11647     4

dim(flo08.flo)

[1] 22086   4

11647+22086

[1] 33733

# Develop model for good turb data

range(trb08.flo$turb)

[1]   9 235

table(sfm08.sed$labcode2)

0   1   2   4   6   7   8   9

188  13   1   4   3   2   1   2

# Codes 2,4 are minor errors

# Codes 5,7,8,9 are major errors

# Code 1 is < 15% organics

# Code 6 is > 15% organics

# The only outlier with labcode2 > 4 is dump 5 bottle 4

# Eliminate bad turbidity and dump 5 bottle 4

# Eliminate dump5 bottle 11, SSC=0 is bogus, was not processed.

# And eliminate turbidity over 300

trb08.sed <- sfm08.sed[!is.na(sfm08.sed$turb) & sfm08.sed$turb <= 300, ]

trb08.sed <- trb08.sed[trb08.sed$dump!=5 | trb08.sed$bottle!=4,]

trb08.sed <- trb08.sed[trb08.sed$dump!=5 | trb08.sed$bottle!=11,]

table(trb08.sed$turbcode)

0   1   5   6   7

93   1  54   8   1

turbsscplot("trb",08)

turbsscplot("trb",08,type="loess") # slightly curved

turbsscplot("trb",08,type="power")

turbsscplot("trb",08,type="logxy")

# We won't be able to use linear or loess becaues of negative predictions

# The power model looks good

# Try a few different models for comparison

turb1 <- turbsrc("trb",08,070801,0000,080731,2400,type="loess",var=F)

turb2 <- turbsrc("trb",08,070801,0000,080731,2400,type="power",var=F)

turb3 <- turbsrc("trb",08,070801,0000,080731,2400,type="logxy",var=F)

total(turb1,turb2,turb3)

start            end       type surr    yhat   n    r2       s cv

turb1 11/09/07 19:50 03/17/08 22:20      loess turb 1196763 157 0.861 106.172 NA

turb2 11/09/07 19:50 03/17/08 22:20      power turb 1285533 157 0.865 104.426 NA

turb3 11/09/07 19:50 03/17/08 22:20 logxy:mvue turb 1330328 157 0.758   0.762 NA> sum(turb1$predssc < 0)

[1] 3833

# Can't use the loess model because of the negative predictions

# The power model has the higher r-square of the remaining 2 models

total(turb2)

start            end  type surr    yhat   n    r2       s cv

turb2 11/09/07 19:50 03/17/08 22:20 power turb 1285533 157 0.865 104.426 NA

Total load = 1285533 kg

# A fair estimate.  There's a lot of scatter in the regression

====================================================================

# Now fit a discharge sediment rating curve for the remaining data

> range(flo08.flo$q)

[1]   1.42 152.00

> range(sfm08.sed$q)

[1]   43.15 1125.00

# Problem is no samples at flows under 43cfs.

qsscplot("sfm",08)

# Eliminate flows over 200 cfs and outlier (dump 3 bottle 16)

flo08.sed <- sfm08.sed[sfm08.sed$q < 200,]

flo08.sed <- flo08.sed[flo08.sed$dump !=3 | flo08.sed$bottle !=16, ]

qsscplot("flo",08)

qsscplot("flo",08,type="power")

qsscplot("flo",08,type="loess")

flow1 <- flowsrc("flo",08,040801,0000,080731,2400,type="logxy",var=F)

flow2 <- flowsrc("flo",08,040801,0000,080731,2400,type="power",var=F)

flow3 <- flowsrc("flo",08,040801,0000,080731,2400,type="loess",var=F)

total(flow1,flow2,flow3)

start            end       type surr     yhat   n    r2      s cv

flow1 08/12/07 15:30 04/22/08 10:10 logxy:mvue flow 361202.6 159 0.602  0.752 NA

flow2 08/12/07 15:30 04/22/08 10:10      power flow 390819.9 159 0.493 82.095 NA

flow3 08/12/07 15:30 04/22/08 10:10      loess flow 223466.7 159 0.483 83.155 NA

sum(flow3$predssc <0)

[1] 16328

# Loess model cannot be used because of negative predictions

# Agreement is pretty good among the first two models.  8% difference.

#logxy looks like the best model to me, highest r2 by far

start            end       type surr     yhat   n    r2     s cv

flow1 08/12/07 15:30 04/22/08 10:10 logxy:mvue flow 361202.6 159 0.602 0.752 NA

Total load = 361203 kg

==================================================

# Total non-storm load

1285533 + 361203 = 1646736 kg = 1647 metric tons

# Total storm load = 6073312 kg = 6073 metric tons

# Total annual load

6073 + 1647 = 7720 metric tons total annual load

# Storm load is 6073/7720 = 78.6 % of annual load