SFM 2006 Annual load estimation

sfm06.flo <- read.flo("sfm",06)

sfm06.lab <- read.lab("sfm",06)

sfm06.sed <- merge.flo("sfm",06)

nonstorms <- drop.storms("sfm",06)

dim(nonstorms)

[1] 19990     4

head(nonstorms, 5)

chr     q turb turbcode

1 (11/02/05 16:20:00) 3.686    1        0

2 (11/02/05 16:30:00) 3.577    1        0

3 (11/02/05 16:40:00) 3.577    1        0

4 (11/02/05 16:50:00) 3.577    1        0

5 (11/02/05 17:00:00) 3.577    1        0

table(nonstorms$turbcode)

0     2     5

19844   127    19

Only 127 values of bad turbidity out of 19990 intervals to be estimated

These were all on a small peak Jan 7 - Jan 8, 2006

trb06.flo <- nonstorms[!is.na(nonstorms$turb), ]

flo06.flo <- nonstorms[is.na(nonstorms$turb), ]

dim(trb06.flo)

[1] 19863     4

dim(flo06.flo)

[1] 127     4

19863 + 127

[1] 19990

# Develop model for good turb data

range(trb06.flo$turb)

[1]   1 1425

table(sfm06.sed$labcode2)

0   1   2   6

440   4   1   2

# Just two bad lab codes to worry about

# (6=greater than 15% org debris, not necessarily a problem)

# Eliminate bad turbidity

# Note I had already eliminated dump 19, bottle 4 from sfm06.sed

trb06.sed <- sfm06.sed[!is.na(sfm06.sed$turb), ]

table(trb06.sed$turbcode)

0

446

turbsscplot("trb",06,type="loess")

turbsscplot("trb",06,type="power")

turbsscplot("trb",06,type="logxy")

turb1 <- turbsrc("trb",06,050801,0000,060731,2400,type="power",var=F)

turb2 <- turbsrc("trb",06,050801,0000,060731,2400,type="logxy",var=F)

turb3 <- turbsrc("trb",06,050801,0000,060731,2400,type="loess",var=F)

turb4 <- turbsrc("trb",06,050801,0000,060731,2400,type="linear",var=F)

start            end       type surr    yhat   n    r2       s cv

turb1 11/02/05 16:20 05/10/06 16:50      power turb 3999001 446 0.944 139.143 NA

turb2 11/02/05 16:20 05/10/06 16:50 logxy:mvue turb 3481246 446 0.957   0.244 NA

turb3 11/02/05 16:20 05/10/06 16:50      loess turb 3529033 446 0.946 136.710 NA

turb4 11/02/05 16:20 05/10/06 16:50     linear turb 3902027 446 0.943 140.898 NA

15% difference between largest and smallest

> sum(turb3$predssc < 0)

[1] 3973

> sum(turb4$predssc < 0)

[1] 0

# Can't use loess: too many negative predictions

# Tough to choose between the power model or logxy model

# Power model gives higher load because curve is higher at lower SSC

# But logxy model fits better at lower SSC, so use that one

start            end       type surr    yhat   n    r2     s cv

turb2 11/02/05 16:20 05/10/06 16:50 logxy:mvue turb 3481246 446 0.957 0.244 NA

Total load = 3481246 kg

# A relatively good estimate

====================================================================

# Now fit a discharge sediment rating curve for the remaining data

> range(flo06.flo$q)

[1]   115.6 136.3

> range(sfm06.sed$q)

[1]   16.8 1225.0

qsscplot("sfm",06,type="loess")

The data to be estimated are in a narrow range near 125cfs = 3.5cumecs

# Limit the regression data to between 1 and 10 cumecs

flo06.sed <- sfm06.sed[sfm06.sed$q > 35 & sfm06.sed$q < 350, ]

qsscplot("flo",06)

flow1 <- flowsrc("flo",06,050801,0000,060731,2400,type="logxy",var=F)

flow2 <- flowsrc("flo",06,050801,0000,060731,2400,type="power",var=F)

flow3 <- flowsrc("flo",06,050801,0000,060731,2400,type="loess",var=F)

total(flow1,flow2,flow3)

start            end       type surr     yhat   n    r2       s cv

flow1 01/07/06 18:20 01/08/06 15:20 logxy:mvue flow 83953.43 343 0.372   0.827 NA

flow2 01/07/06 18:20 01/08/06 15:20      power flow 97062.50 343 0.181 418.762 NA

flow3 01/07/06 18:20 01/08/06 15:20      loess flow 92604.97 343 0.197 415.112 NA

> sum(flow3$predssc <0)

[1] 0

#The logxy model looks good to me

start            end       type surr     yhat   n    r2     s cv

flow1 01/07/06 18:20 01/08/06 15:20 logxy:mvue flow 83953.43 343 0.372 0.827 NA

Total load = 83953 kg

==================================================

# Total non-storm load

3481246 + 83953 = 3565199 kg = 3565 metric tons

# Total storm load = 21647968 kg

# Total annual load

21468 + 3565 = 25033 metric tons total annual load

# Storm load is 21468/25033 = 85.8% of annual load