HHB rating curve analysis update with 2008 data: 3 new points

hhbq <- read.csv("hhb/Q and rating curves/HHB Q summary.csv")

> hhbq

HY          ID       Date Stage       Q Code

1  2000  12/10/1999 12/10/1999  2.46  114.30    9

2  2000   2/28/2000  2/28/2000  3.87  201.60    9

3  2001  10/29/2000 10/29/2000  1.12    4.30    0

4  2001    3/3/2001   3/3/2001  1.70   30.21    0

5  2001   3/11/2001  3/11/2001  1.45   16.03    0

6  2002   9/16/2001  9/16/2001  0.91    0.21    0

7  2002  11/23/2001 11/23/2001  1.52   16.83    0

8  2002   1/22/2002  1/22/2002  2.99  228.38    0

9  2002   1/26/2002  1/26/2002  3.54  289.10    0

10 2002 2/23/2002 A  2/23/2002  5.17  643.99    0

11 2002   2-23-02 B  2/23/2002  6.98  979.41    0

12 2003   11/9/2002  11/9/2002  1.38    8.74    0

13 2003    1/5/2003   1/5/2003  2.30   83.16    0

14 2003   1/18/2003  1/18/2003  1.81   37.10    0

15 2003   3/15/2003  3/15/2003  4.73  496.02    0

16 2003   4/25/2003  4/25/2003  7.65  885.14    9

17 2004  10/25/2003 10/25/2003  0.97    0.72    0

18 2004   11/8/2003  11/8/2003  1.11    2.41    0

19 2004   1/11/2004  1/11/2004  1.90   40.65    0

20 2005   9/19/2004  9/19/2004  0.83    0.54    0

21 2005  12-15-04 A 12/15/2004  1.51   14.57    0

22 2005  12-15-04 B 12/15/2004  1.51   12.68    0

23 2005    1/4/2005   1/4/2005  2.65   79.55    0

24 2005    1/5/2005   1/5/2005  2.23   54.16    0

25 2005    1/6/2005   1/6/2005  2.15   40.93    0

26 2005   3-20-05 A  3/20/2005  2.73   80.92    0

27 2005   3-20-05 B  3/20/2005  7.37  705.81    9

28 2005   3/21/2005  3/21/2005  3.99  283.14    0

29 2005   3-29-05 a  3/29/2005  9.57 1159.57    8

30 2005  3-29-05 ov  3/29/2005  9.57 1265.65    0

31 2005   3-29-05 b  3/29/2005  6.65  350.75    0

32 2006  10/29/2005 10/29/2005  1.32    2.14    0

33 2007   9/10/2006  9/10/2006  1.21    0.51    0

34 2007  11/18/2006 11/18/2006  1.60    8.78    0

35 2007  12/29/2006 12/29/2006  2.66   70.24    0

36 2007   2/22/2007  2/22/2007  6.16  656.78    0

37 2008   1/31/2008  1/31/2008 11.33 1796.00    0

38 2008    1/4/2008   1/4/2008  8.64  883.07    0

39 2008   2/26/2008  2/26/2008  2.78  114.22    0

# Note very minor changes to discharges in rows 34-36

# Row 37 includes a crude estimate of overbank flow (353cfs)

# based on the shape of the cross section at the road surface

Codes

----------------------------------

9=poor

8=does not include floodplain flow

0=fair or better

----------------------------------

# Exclude pre-2003 and bad data identified last year

hhbq <- hhbq[hhbq$Code != 8 & hhbq$HY >=2003, ]

hhbq <- hhbq[hhbq$ID != "3-29-05 b", ]

# Plot the data using last digit of water year

hhbq$sym <- as.character(hhbq$HY %% 1000)

plot(hhbq$Stage, hhbq$Q, type="p", pch=sym, cex=0.7)

================================================================

Observing that the 2003 data are slightly above the others, Jesse and

Jack decided to eliminate 2003 due to suspected subsequent aggradation.

Remember the rating curve will only be applied to HY05 and later.

================================================================

hhbq <- hhbq[hhbq$HY > 2003, ]

attach(hhbq)

# Exclude "poor" measurements from smooth model and display them in red

scatter.smooth(Stage[Code!=9], Q[Code!=9], span=0.8, degree=2, type="p", pch=sym[Code!=9],cex=0.7)

points(Stage[Code==9], Q[Code==9], pch=sym[Code==9], cex=0.7, col=2)

Suggests that the one poor datum may not really be so poor

# Plot smooth fit to 2004-2008 data including "poor" datum

scatter.smooth(Stage, Q, type="p", pch=sym, cex=0.7,span=0.8,degree=2)

#  That's very weird behavior.  Include the point and the curve jumps away from it

Last year's analysis used a curved lower segment and a linear upper segment.

This year's will use two curved segments. One curve doesn't fit the whole range.

Keep the "poor" code 9 data.  Not shown: neither a power function nor a quadratic

provides a good fit throughout the range. Results below were arrived at by

trial and error.

plot(Stage,Q,main="HHB HY04-08",xlab="stage (ft)",ylab="discharge (cfs)",type="n")

text(Stage,Q,HY%%1000,cex=0.7)

fit1 <- lm(Q ~ Stage + I(Stage^2), data=hhbq, subset=Stage < 3)

x1 <- seq(0,2.90,0.2)

y1 <- predict(fit1, newdata=data.frame(Stage=x1))

# Had to force the intercept to get a smooth transition between the fits

fit2 <- lm(I(Q+80) ~ -1 + Stage + I(Stage^2), data=hhbq, subset=Stage > 2.5)

x2 <- seq(2.90,12,0.2)

y2 <- predict(fit2, newdata=data.frame(Stage=x2)) - 80

lines(x1,y1)

lines(x2,y2,col=2)

legend(locator(1),c("Q = 12.606 -36.016s + 23.568s^2", "Q = -80 + 31.458s+ 11.310s^2"), lty=1, col=1:2, cex=0.7)

# Calculation of intersection

uniroot(

function(x) 12.606-36.016*x + 23.568*x^2 - (-80 + 31.458*x+ 11.310*x^2),

interval=c(2.8,2.9)

)

$root

[1] 2.894034

# Compare with other rating equations

plot(Stage,Q,main="HHB rating curve comparison",xlab="stage (ft)",ylab="discharge (cfs)",type="n")

text(Stage,Q,HY%%1000,cex=0.7)

lines(x1,y1,col=2)

lines(x2,y2,col=2)

# Add lines for earlier rating equations

x3 <- seq(0,2.68,0.02)

x4 <- seq(2.68,12,0.02)

y3 <- 1.2974*x3^4.2415

y4 <- -379.751+171.8645*x4

lines(x3,y3,col=3)

lines(x4,y4,col=3)

x5 <- seq(0, 12, 0.5)

y5 <- (13.122*x5^2)-(7.7527*x5)

lines(x5,y5,col=4)

legend(locator(1),c("Jack: HY0408","Jack: HY0307","Clark: HY05"), lty=1,col=2:4,cex=0.75)

equation"), lty=1,col=1:3,cex=0.75)

# Equation for HY05 to HY08 will be the red curve, based on HY04-08 data

Stage < 2.89403:  Q =  12.606 - 36.016s + 23.568s^2

Stage > 2.89403:  Q = -80.000 + 31.458s + 11.310s^2

# Beware this equation goes negative at stages <= 0.985

# In HY05 the lowest recorded stage was 1.010