FTR07 Event Loads

stn,storm,sdate,stime,edate,etime

FTR,701,061212,2350,061214,2040  100213 kg  Very good

FTR,702,061225,2220,061229,1200  241810 kg  Excellent

FTR,703,070103,0640,070106,1010  394393 kg  Excellent

FTR,704,070210,0510,070213,1200  157637 kg  Very good

FTR,705,070220,1510,070224,1340 1148011 kg  Excellent

FTR,706,070224,1350,070227,0740  171312 kg  Very good

FTR,707,070227,0750,070303,1200   75065 kg  Good

FTR,708,070326,0920,070329,1200  148970 kg  Very good

-----------------------------------------------------

SUM of events:                  2437411 kg  Excellent

-----------------------------------------------------

ftr07.flo <- read.flo("ftr",07)

ftr07.lab <- read.lab("ftr",07)

mismatches("ftr",07)

# Unmatched samples in flo file: 1(1), 3(1-14), 15(1-6)

ftr07.sed <- merge.flo("ftr",07)

-----------------------------------------------------

Storm 701: dump5,061212,2350,061214,2040

# Excellent stage and turbidity data

# Excellent sample coverage, still could be useful to bring in a bottle

# or two before and after the defined storm to solidify the lower end

four()

turbsscplot("ftr",07,061212,2350,061214,2040)

# A little bit of hysteresis is evident

turbsscplot("ftr",07,dump=5,bot=7:12)

turbsscplot("ftr",07,dump=5,bot=12:17)

turbsscplot("ftr",07,dump=c(rep(5,6),6),bot=c(12:17,1))

turb0 <- turbsrc("ftr",07,061212,2350,061214,2040)

total(turb0)

start            end   type surr     yhat  n    r2      s     cv

turb0 12/12/06 23:50 12/14/06 20:40 linear turb 100131.3 10 0.924 44.339 14.595

Total load = 100131 kg,  CV = 14.6 %

turb1 <- turbsrc("ftr",07,061212,2350,061213,1120,dump=5,bot=7:12)

turb2 <- turbsrc("ftr",07,061213,1220,0061214,2040,dump=c(rep(5,6),6),bot=c(12:17,1))

tail(turb1$predssc,4)

[1] 439.8827 466.5120 513.1132 523.7649

time1 <- lineartime("ftr",07,061213,1130,061213,1210,ssc1=520)

ttsplot("ftr",07,turb1,time1,turb2)

total(turb1,time1,turb2)

start            end   type surr     yhat n    r2      s     cv

turb1 12/12/06 23:50 12/13/06 11:20 linear turb 30990.56 6 0.969 39.081 12.231

time1 12/13/06 11:30 12/13/06 12:10 linear time 14141.12 1    NA     NA     NA

turb2 12/13/06 12:20 12/14/06 20:40 linear turb 57046.00 7 0.962 30.711 14.885

Total load = 102178

turb3 <- turbsrc("ftr",07,061212,2350,061213,1120,dump=5,bot=7:12,type="logxy")

time2 <- lineartime("ftr",07,061213,1130,061214,2040,ssc1=603)

ttsplot("ftr",07,turb3,time2)

start            end       type surr     yhat n    r2     s     cv

turb3 12/12/06 23:50 12/13/06 11:20 logxy:mvue turb 31097.07 6 0.957 0.318 15.211

time2 12/13/06 11:30 12/14/06 20:40     linear time 69116.18 6    NA    NA     NA

Total load = 100213 kg  Very good

# Despite the high CV all estimates agree within 2%

-----------------------------------------------------

Storm 702: dump 6,061225,2220,061229,1200

# Two peaks in this storm

# Excellent turbidity and stage data

# Excellent sample coverage

# Some mild hysteresis in the turb/ssc plots

four()

turbsscplot("ftr",07,061225,2220,061229,1200)

turbsscplot("ftr",07,061225,2220,061227,0700)

turbsscplot("ftr",07,061225,2220,061227,0700,type="sqrt")

turbsscplot("ftr",07,061227,0900,061229,1200)

turb01 <- turbsrc("ftr",07,061225,2220,061229,1200)

turb02 <- turbsrc("ftr",07,061225,2220,061229,1200,type="power")

turb03 <- turbsrc("ftr",07,061225,2220,061229,1200,type="sqrt")

turb04 <- turbsrc("ftr",07,061225,2220,061229,1200,type="loess")

total(turb01,turb02,turb03,turb04)

start            end      type surr     yhat  n    r2      s    cv

turb01 12/25/06 22:20 12/29/06 12:00    linear turb 240235.3 18 0.962 31.178 4.283

turb02 12/25/06 22:20 12/29/06 12:00     power turb 242700.8 18 0.963 30.910 3.917

turb03 12/25/06 22:20 12/29/06 12:00 sqrt:duan turb 239036.1 18 0.982  0.870    NA

turb04 12/25/06 22:20 12/29/06 12:00     loess turb 241264.4 18 0.963 31.882    NA

# All estimates agree within 1.5%

# Still it seems like we should model the hysteresis

turb1 <- turbsrc("ftr",07,061225,2220,061227,0700,type="sqrt")

start.8270   end.8466

1.135058 720.047622

turb2 <- turbsrc("ftr",07,061227,0900,061229,1200)

endpoints(turb2)

start        end

416.851174   6.876538

# Turb has to drop 303 in 12 intervals, or 25 per interval

time1 <- lineartime("ftr",07,061227,0710,061227,0850,ssc1=720-25,ssc2=417+25)

ttsplot("ftr",07,turb1,time1,turb2)

total(turb1,time1,turb2)

start            end      type surr      yhat  n    r2     s    cv

turb1 12/25/06 22:20 12/27/06 07:00 sqrt:duan turb 109191.15 11 0.990 0.688    NA

time1 12/27/06 07:10 12/27/06 08:50    linear time  49915.25  0    NA    NA    NA

turb2 12/27/06 09:00 12/29/06 12:00    linear turb  82703.95  7 0.999 4.533 1.729

Total load = 241810 kg  Excellent

# Agrees with earlier single model estimates to within 1.2%

-----------------------------------------------------

Storm 703: dump7,070103,0640,070106,1010

# Excellent turbidity and stage data

# Excellent sample coverage

four()

turbsscplot("ftr",07,070103,0640,070106,1010)

turbsscplot("ftr",07,070103,0640,070106,1010,type="power")

turbsscplot("ftr",07,070103,0640,070106,1010,type="loess")

turbsscplot("ftr",07,070103,0640,070106,1010,type="sqrt")

turb1 <- turbsrc("ftr",07,070103,0640,070106,1010)

turb2 <- turbsrc("ftr",07,070103,0640,070106,1010,type="power")

turb3 <- turbsrc("ftr",07,070103,0640,070106,1010,type="loess")

turb4 <- turbsrc("ftr",07,070103,0640,070106,1010,type="sqrt")

total(turb1,turb2,turb3,turb4)

start            end      type surr     yhat  n    r2      s    cv

turb1 01/03/07 06:40 01/06/07 10:10    linear turb 391667.1 14 0.995 17.453 2.118

turb2 01/03/07 06:40 01/06/07 10:10     power turb 402326.8 14 0.995 17.807 1.694

turb3 01/03/07 06:40 01/06/07 10:10     loess turb 393892.5 14 0.995 17.640    NA

turb4 01/03/07 06:40 01/06/07 10:10 sqrt:duan turb 394393.0 14 0.997  0.433    NA

# All estimates agree to within 2.6%

# The power estimate overestimates the low end of recession a bit

# The linear and loess estimates start negative

ttsplot("ftr",07,turb4)

start            end      type surr     yhat  n    r2     s cv

turb4 01/03/07 06:40 01/06/07 10:10 sqrt:duan turb 394393.0 14 0.997 0.433 NA

Total load = 394393 kg  Excellent

-----------------------------------------------------

Storm 704: dump 8-9,070210,0510,070213,1200

# Double peak storm

# Stage data was shifted up by 0.05 for this whole event to match staff plate

# Transducer was buried in the mud, so uncertainty here is in the discharge

# Excellent turbidity data and excellent sample coverage

four()

turbsscplot("ftr",07,070210,0510,070213,1200)

# Bring in an extra sample at start

turbsscplot("ftr",07,dump=c(8,rep(9,12)),bot=c(13,1:12))

turbsscplot("ftr",07,dump=c(8,9,9,9),bot=c(13,1:3))

turbsscplot("ftr",07,dump=9,bot=4:12,type="power")

turb01 <- turbsrc("ftr",07,070210,0510,070213,1200)

turb02 <- turbsrc("ftr",07,070210,0510,070213,1200,type="power")

turb03 <- turbsrc("ftr",07,070210,0510,070213,1200,type="logxy",bias="qmle")

turb04 <- turbsrc("ftr",07,070210,0510,070213,1200,type="sqrt")

total(turb01,turb02,turb03,turb04)

start            end       type surr     yhat  n    r2      s     cv

turb01 02/10/07 05:10 02/13/07 12:00     linear turb 154500.1 12 0.956 44.217 12.762

turb02 02/10/07 05:10 02/13/07 12:00      power turb 164444.6 12 0.957 44.329  8.900

turb03 02/10/07 05:10 02/13/07 12:00 logxy:qmle turb 158449.8 12 0.972  0.206  6.288

turb04 02/10/07 05:10 02/13/07 12:00  sqrt:duan turb 157938.0 12 0.973  1.179     NA

# turb01 starts and ends negative; the logxy model would be best

# Really need to model the hysteresis

turb1 <- turbsrc("ftr",07,070210,0510,070210,2130,dump=c(8,9,9,9),bot=c(13,1:3))

turb2 <- turbsrc("ftr",07,070210,2140,070213,1200,dump=9,bot=4:12,type="power")

ttsplot("ftr",07,turb1,turb2)

ttsplot("ftr",07,turb1,turb2)

start            end   type surr      yhat n    r2      s    cv

turb1 02/10/07 05:10 02/10/07 21:30 linear turb  54915.71 4 0.999 11.794 2.142

turb2 02/10/07 21:40 02/13/07 12:00  power turb 102721.14 9 0.997 11.530 3.631

Total load = 157637 kg,  CV = 2.48 %  Very good

# Agrees remarkably well with single-model estimates

-----------------------------------------------------

Storm 705: dump 10-11,070220,1510,070224,1340

# Very good stage and turbidity data

# A couple of brief spikes removed

# Excellent sample coverage

four()

turbsscplot("ftr",07,070220,1510,070224,1340)

turbsscplot("ftr",07,070220,1510,070224,1340,type="power")

turbsscplot("ftr",07,070220,1510,070224,1340,type="logxy")

turbsscplot("ftr",07,070220,1510,070224,1340,type="sqrt")

turb1 <- turbsrc("ftr",07,070220,1510,070224,1340)

turb2 <- turbsrc("ftr",07,070220,1510,070224,1340,type="power")

turb3 <- turbsrc("ftr",07,070220,1510,070224,1340,type="logxy",bias="qmle")

turb4 <- turbsrc("ftr",07,070220,1510,070224,1340,type="sqrt")

total(turb1,turb2,turb3,turb4)

start            end       type surr    yhat  n    r2      s    cv

turb1 02/20/07 15:10 02/24/07 13:40     linear turb 1145450 20 0.992 32.192 1.935

turb2 02/20/07 15:10 02/24/07 13:40      power turb 1147890 20 0.991 35.016 2.124

turb3 02/20/07 15:10 02/24/07 13:40 logxy:qmle turb 1153348 20 0.980  0.206 5.773

turb4 02/20/07 15:10 02/24/07 13:40  sqrt:duan turb 1132347 20 0.990  0.999    NA

# Estimates vary by 1.9% but linear model has negative endpoints

endpoints(turb1)

start       end

-46.11600 -11.22615

ttsplot("ftr",07,turb2)

total(turb2)

start            end  type surr    yhat  n    r2      s    cv

turb2 02/20/07 15:10 02/24/07 13:40 power turb 1148011 20 0.991 35.016 2.124

Total load = 1148011 kg,  CV = 2.12 %   Excellent

-----------------------------------------------------

Storm 706: dump 11,070224,1350,070227,0740

# Two peaks to this storm

# Stage data was shifted down by about 0.02 but no staff plate readings in this event

# Excellent turbidity data and very good sample coverage

# But probably should grab bottle 10 which occurred right after storm

four()

turbsscplot("ftr",07,dump=11,bot=1:10)

turbsscplot("ftr",07,dump=11,bot=1:10,type="power")

turbsscplot("ftr",07,dump=11,bot=1:10,type="logxy")

turbsscplot("ftr",07,dump=11,bot=1:10,type="sqrt")

turb1 <- turbsrc("ftr",07,070224,1350,070227,0740,dump=11,bot=1:10)

turb2 <- turbsrc("ftr",07,070224,1350,070227,0740,dump=11,bot=1:10,type="power")

turb3 <- turbsrc("ftr",07,070224,1350,070227,0740,dump=11,bot=1:10,type="logxy",bias="qmle")

turb4 <- turbsrc("ftr",07,070224,1350,070227,0740,dump=11,bot=1:10,type="sqrt")

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat  n    r2      s    cv

turb1 02/24/07 13:50 02/27/07 07:40     linear turb 164172.9 10 0.977 16.190 5.586

turb2 02/24/07 13:50 02/27/07 07:40      power turb 171662.6 10 0.980 15.404 4.237

turb3 02/24/07 13:50 02/27/07 07:40 logxy:qmle turb 171312.2 10 0.985  0.089 3.000

turb4 02/24/07 13:50 02/27/07 07:40  sqrt:duan turb 168687.3 10 0.984  0.538    NA

# The logxy only has the best CV because the upper samples appear to be close

# Estimates vary by 4.3%, not really clear which is best

ttsplot("ftr",07,turb3)

total(turb3)

start            end       type surr     yhat  n    r2     s cv

turb3 02/24/07 13:50 02/27/07 07:40 logxy:qmle turb 171312.2 10 0.985 0.089  3

Total load = 171312 kg,  CV = 3 %  Very good

-----------------------------------------------------

Storm 707: dump 11,070227,0750,070303,1200

# An extended event with 2 or 3 hydrograph peaks

# Stage was adjusted down by about 0.03 to match staff plate readings

# But none of the readings were taken during this event

# Very good turbidity data

# Sample coverage is good on the main peak, then only 1 sample for a 3-day stretch

four()

turbsscplot("ftr",07,070227,0750,070303,1200)

turbsscplot("ftr",07,070227,0750,070303,1200,type="power")

turbsscplot("ftr",07,070227,0750,070303,1200,type="logxy")

turbsscplot("ftr",07,070227,0750,070303,1200,type="loess")

turb1 <- turbsrc("ftr",07,070227,0750,070303,1200)

turb2 <- turbsrc("ftr",07,070227,0750,070303,1200,type="power")

turb3 <- turbsrc("ftr",07,070227,0750,070303,1200,type="logxy",bias="qmle")

turb4 <- turbsrc("ftr",07,070227,0750,070303,1200,type="loess")

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat n    r2     s    cv

turb1 02/27/07 07:50 03/03/07 12:00     linear turb 72122.76 6 0.960 6.939 8.982

turb2 02/27/07 07:50 03/03/07 12:00      power turb 72391.34 6 0.974 5.604 6.187

turb3 02/27/07 07:50 03/03/07 12:00 logxy:qmle turb 75818.57 6 0.946 0.132 5.902

turb4 02/27/07 07:50 03/03/07 12:00      loess turb 75064.64 6 0.976 6.312    NA

# Estimates vary by 5.1%

# The loess or logxy estimate is best, whichever comes closest to bottle 15

ttsplot("ftr",07,turb4)

total(turb4)

start            end  type surr     yhat n    r2     s cv

turb4 02/27/07 07:50 03/03/07 12:00 loess turb 75064.64 6 0.976 6.312 NA

Total load = 75065 kg  Good

# Would be very good but for the 3 days without a sample

-----------------------------------------------------

Storm 708: dump 12,070326,0920,070329,1200

# Excellent stage and turbidity data

# Stage was shifted down a bit to match a staff plate reading

# Good sample coverage but would've been nice to get one at the peak

four()

turbsscplot("ftr",07,070326,0920,070329,1200)

turbsscplot("ftr",07,070326,0920,070329,1200,type="power")

turbsscplot("ftr",07,070326,0920,070329,1200,type="logxy")

turbsscplot("ftr",07,070326,0920,070329,1200,type="loess")

turb1 <- turbsrc("ftr",07,070326,0920,070329,1200)

turb2 <- turbsrc("ftr",07,070326,0920,070329,1200,type="power")

turb3 <- turbsrc("ftr",07,070326,0920,070329,1200,type="logxy",bias="qmle")

turb4 <- turbsrc("ftr",07,070326,0920,070329,1200,type="loess")

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat  n    r2      s     cv

turb1 03/26/07 09:20 03/29/07 12:00     linear turb 149148.4 10 0.988 12.329  3.138

turb2 03/26/07 09:20 03/29/07 12:00      power turb 148969.7 10 0.999  4.209  1.024

turb3 03/26/07 09:20 03/29/07 12:00 logxy:qmle turb 155624.8 10 0.969  0.251 12.120

turb4 03/26/07 09:20 03/29/07 12:00      loess turb 149232.0 10 0.997  6.624     NA

# Estimates vary by 4.4%

# Power model is the hands-down winner

ttsplot("ftr",07,turb2)

total(turb2)

start            end  type surr     yhat  n    r2     s    cv

turb2 03/26/07 09:20 03/29/07 12:00 power turb 148969.7 10 0.999 4.209 1.024

Total load = 148970 kg,  CV = 1.02 %  Very good