SFM05 Event Loads

These loads are based on the following rating equation, built from 03-05 measurements:

(s<2.4302)*(1.2326*s^3.1572) + (s>2.4302)*(2.1831*s^2+28.325*s-61.386)

Note turbidity data was excellent all winter

stn,storm,sdate,stime,edate,etime

SFM,501,041206,0920,041207,1940     75620 kg: Excellent

SFM,502,041207,1950,041211,0000    832999 kg: Very good

SFM,503,041230,1800,050104,0000   1239292 kg: Good

SFM,504,050107,0400,050108,0850    354553 kg: Fair

SFM,505,050108,0900,050111,0000    929896 kg: Good

SFM,506,050319,0640,050323,0120    627413 kg: Good to fair

SFM,507,050327,1230,050331,1200   1571031 kg: Good

SFM,508,050406,2350,050408,0650    249203 kg: Excellent

SFM,509,050408,0700,050412,0600   2059936 kg: Excellent

SFM,510,050507,1920,050511,1200    329998 kg: Good

-----------------------------------------------------

SUM of events                     8269941 kg: Very good

-----------------------------------------------------

sfm05.flo <- read.flo("sfm",05)

sfm05.lab <- read.lab("sfm",05)

mismatches("sfm",05)

# Many unmatched samples in flo file; none in lab file

sfm05.sed <- merge.flo("sfm",05)

Storm 501: dump 6:  SFM,501,051206,0920,051207,1940

Very good sample coverage and nearly perfect relation between SSC and turbidity

four()

turbsscplot("sfm",05,041206,0920,041207,1940)

turbsscplot("sfm",05,041206,0920,041207,1940,type="power")

turbsscplot("sfm",05,041206,0920,041207,1940,type="logxy")

turbsscplot("sfm",05,041206,0920,041207,1940,type="sqrt")

turb1 <- turbsrc("sfm",05,041206,0920,041207,1940)

endpoints(turb1)

start       end

-31.89194  36.42649

turb2 <- turbsrc("sfm",05,041206,0920,041207,1940,type="power")

ttsplot("sfm",05,turb2)

total(turb2)

start            end  type surr     yhat  n    r2      s   cv

turb2 12/06/04 09:20 12/07/04 19:40 power turb 75619.94 12 0.992 19.438 2.56

Total load = 75620 kg,  CV = 2.56 %  Excellent quality

Storm 502: dump 7: SFM,502,041207,1950,041211,0000

Good sample coverage except for 5 hour gap at the peak; excellent regressions

four()

turbsscplot("sfm",05,041207,1950,041211,0000)

turbsscplot("sfm",05,041207,1950,041211,0000,type="power")

turbsscplot("sfm",05,041207,1950,041211,0000,type="logxy")

turbsscplot("sfm",05,041207,1950,041211,0000,type="sqrt")

# All have r-squared > 0.99

turb1 <- turbsrc("sfm",05,041207,1950,041211,0000)

endpoints(turb1)

start       end

25.962189  8.389451

turb2 <- turbsrc("sfm",05,041207,1950,041211,0000,type="power")

start     end   type surr     yhat  n    r2      s    cv

turb1 12/07/04 19:50 2/11/04 linear turb 830572.5 14 0.993 50.660 3.451

turb2 12/07/04 19:50 2/11/04  power turb 832998.6 14 0.994 45.788 2.793

ttsplot("sfm",05,turb2)

total(turb2)

start     end  type surr     yhat  n    r2      s    cv

turb2 12/07/04 19:50 2/11/04 power turb 832998.6 14 0.994 45.788 2.793

Total load = 832999 kg,  CV = 2.79 %  Very good quality

Storm 503: dumps 11-12: SFM,503,041230,1800,050104,0000

Excellent sample coverage on first peak, not on second peak

Dump 11 bot 10 is extreme outlier and has anomalous (56%) sand fraction.

(Was double checked according to spreadsheet)

Dump 11 bots 7-9 have high SSC for the turb, but normal sand fractions.

four()

turbsscplot("sfm",05,041230,1800,050104,0000)

turbsscplot("sfm",05,041230,1800,050104,0000,type="power")

turbsscplot("sfm",05,041230,1800,050104,0000,type="logxy")

turbsscplot("sfm",05,041230,1800,050104,0000,type="sqrt")

Need to eliminate samples 7-10 from regressions. Use lineartime on 7-9.

turbsscplot("sfm",05,dump=rep(11:12,c(6,4)),bot=c(1:6,1:4))

turb1 <- turbsrc("sfm",05,041230,1800,041231,2240,dump=rep(11:12,c(6,4)),bot=c(1:6,1:4))

time1 <- lineartime("sfm",05,041231,2250,050101,0140)

turb2 <- turbsrc("sfm",05,050101,0150,050104,0000,dump=rep(11:12,c(6,4)),bot=c(1:6,1:4))

ttsplot("sfm",05,turb1,time1,turb2)

total(turb1,time1,turb2)

start            end   type surr     yhat  n    r2      s    cv

turb1 12/30/04 18:00 12/31/04 22:40 linear turb 571928.4 10 0.996 35.779 2.077

time1 12/31/04 22:50 01/01/05 01:40 linear time 196417.4  3    NA     NA    NA

turb2 01/01/05 01:50 01/04/05 00:00 linear turb 470946.7 10 0.996 35.779 6.009

Total load = 1239292 kg   Quality good

Quality of samples is suspect, and third peak has poor coverage

But a reasonable alternative disregarding samples 7-9 entirely is only 6% less (see below)

turb3 <- turbsrc("sfm",05,041230,1800,050104,0000,dump=rep(11:12,c(6,4)),bot=c(1:6,1:4))

total(turb3)

start     end   type surr    yhat  n    r2      s    cv

turb3 12/30/04 18:00 1/04/05 linear turb 1173347 10 0.996 35.779 3.468

Total load = 1173347 kg,  CV = 3.47 %

Storm 504: dump 12: SFM,504,050107,0400,050108,0850

Dump 12, no samples on turbidity recession (includes hydro peak)

four()

turbsscplot("sfm",05,050107,0400,050108,0850)

turbsscplot("sfm",05,050107,0400,050108,0850,type="power")

turbsscplot("sfm",05,050107,0400,050108,0850,type="logxy")

turbsscplot("sfm",05,050107,0400,050108,0850,type="sqrt")

turb1 <- turbsrc("sfm",05,050107,0400,050108,0850,type="power")

ttsplot("sfm",05,turb1)

total(turb1)

start            end  type surr     yhat n    r2      s    cv

turb1 01/07/05 04:00 01/08/05 08:50 power turb 354553.4 7 0.994 40.988 3.561

Total load = 354553 kg,  CV = 3.56 %  Quality fair (sample coverage)

Storm 505: dump 13: SFM,505,050108,0900,050111,0000

Good sample coverage except for last 36 hours

four()

turbsscplot("sfm",05,050108,0900,050111,0000)

# Calls for separate regression of falling and rising turbidity

turbsscplot("sfm",05,050108,0900,050108,1800)

turbsscplot("sfm",05,050108,1800,050111,0000)

turb1 <- turbsrc("sfm",05,050108,0900,050108,1800)

turb2 <- turbsrc("sfm",05,050108,1810,050111,0000)

ttsplot("sfm",05,turb1,turb2)

total(turb1,turb2)

start            end   type surr     yhat n    r2      s    cv

turb1 01/08/05 09:00 01/08/05 18:00 linear turb 244405.3 4 0.997 26.427 2.245

turb2 01/08/05 18:10 01/11/05 00:00 linear turb 685490.6 7 0.999  9.655 1.297

Total load = 929896 kg,  CV = 1.12 %  Quality good

About 180000/930000 = 19% occurred after 01/09/05:1200 i.e. when no samples

Storm 506: dumps 16-17: SFM,506,050319,0640,050323,0120

Good coverage except at peak, but turbidity had already dropped by then

four()

turbsscplot("sfm",05,050319,0640,050323,0120)

turbsscplot("sfm",05,050319,0640,050323,0120,type="power")

turbsscplot("sfm",05,050319,0640,050323,0120,type="logxy")

turbsscplot("sfm",05,050319,0640,050323,0120,type="loess")

# sample 1 is an outlier

turb1 <- turbsrc("sfm",05,050319,0640,050323,0120)

endpoints(turb1)

start       end

-8.156883 71.628717

# 15 points are negative at start (2.5 hrs)

turb2 <- turbsrc("sfm",05,050319,0640,050323,0120,type="power")

# Omit outlier bottle 1

turb3 <- turbsrc("sfm",05,050319,0640,050323,0120,type="power", dump=rep(16:17,c(18,2),bot=c(7:24,2:3))

start            end   type surr     yhat  n    r2      s    cv

turb1 03/19/05 06:40 03/23/05 01:20 linear turb 603121.4 21 0.990 72.438 4.609

turb2 03/19/05 06:40 03/23/05 01:20  power turb 620496.3 21 0.990 73.330 4.363

turb3 03/19/05 06:40 03/23/05 01:20  power turb 608278.3 20 0.993 64.328 3.956

ttsplot("sfm",05,turb3)

# Looks OK but this could benefit from splitting out the first peak

par(mfrow=c(1,2))

turbsscplot("sfm",05,050319,0640,050320,1230,type="power")

turbsscplot("sfm",05,dump=rep(16:17,c(12,2)),bot=c(13:24,2:3),type="power")

turb1 <- turbsrc("sfm",05,050319,0640,050320,1230,type="power")

turb2 <- turbsrc("sfm",05,050320,1240,050323,0120,dump=rep(16:17,c(12,2)),bot=c(13:24,2:3),type="power")

ttsplot("sfm",05,turb1,turb2)

total(turb1,turb2)

start            end  type surr      yhat  n    r2      s    cv

turb1 03/19/05 06:40 03/20/05 12:30 power turb  22028.70  6 0.981 10.217 3.644

turb2 03/20/05 12:40 03/23/05 01:20 power turb 605383.99 14 0.992 65.573 4.360

Total load = 627413 kg,  CV = 4.21 %  Quality good to fair

# Seems good but 64% of this load occurred after sample 24

Storm 507: dump 18-19 SFM,507,050327,1230,050331,1200

No samples collected during peak of event

four()

turbsscplot("sfm",05,050327,1230,050331,1200)

turbsscplot("sfm",05,050327,1230,050331,1200,type="loess",degree=2)

turbsscplot("sfm",05,dumps=16:19)

turbsscplot("sfm",05,dumps=16:22)

The second plot shows that other dumps would likely underpredict SSC

for this event.  We're probably better off extrapolating, but there

is going to be considerable uncertainty.

turb1 <- turbsrc("sfm",05,050327,1230,050331,1200)

endpoints(turb1)

start       end

-17.61203  52.84627

The first 19 points are negative

turb2 <- turbsrc("sfm",05,050327,1230,050331,1200,type="power")

turb3 <- turbsrc("sfm",05,050327,1230,050331,1200,type="loess")

turb4 <- turbsrc("sfm",05,050327,1230,050331,1200,type="loess",degree=2)

turb5 <- turbsrc("sfm",05,050327,1230,050331,1200,dumps=16:19)

total(turb1,turb2,turb3,turb4,turb5)

start            end   type surr    yhat  n    r2      s    cv

turb1 03/27/05 12:30 03/31/05 12:00 linear turb 1608773 17 0.995 32.201 1.407

turb2 03/27/05 12:30 03/31/05 12:00  power turb 1640384 17 0.993 36.342 1.560

turb3 03/27/05 12:30 03/31/05 12:00  loess turb 1605505 17 0.995 31.822    NA

turb4 03/27/05 12:30 03/31/05 12:00  loess turb 1571031 17 0.996 30.854    NA

turb5 03/27/05 12:30 03/31/05 12:00 linear turb 1350593 64 0.981 79.518 1.778

# turb5 is just for sensitivity analysis

# Use the 2nd degree loess fit: it has the most conservative extrapolation

ttsplot("sfm",05,turb4)

total(turb4)

start            end  type surr    yhat  n    r2      s cv

turb4 03/27/05 12:30 03/31/05 12:00 loess turb 1571031 17 0.996 30.854 NA

Total load = 1571031 kg	   Quality good

Storm 508: dump 20: SFM,508,050406,2350,050408,0650

Very good sample coverage

four()

turbsscplot("sfm",05,050406,2350,050408,0650)

turbsscplot("sfm",05,050406,2350,050408,0650,type="power")

turbsscplot("sfm",05,050406,2350,050408,0650,type="logxy")

turbsscplot("sfm",05,050406,2350,050408,0650,type="loess")

turb1 <- turbsrc("sfm",05,050406,2350,050408,0650)

turb2 <- turbsrc("sfm",05,050406,2350,050408,0650,type="power")

turb3 <- turbsrc("sfm",05,050406,2350,050408,0650,type="loess")

turb3 <- turbsrc("sfm",05,050406,2350,050408,0650,type="logxy")

total(turb1,turb2,turb3,turb4)

start            end       type surr     yhat  n    r2      s    cv

turb1 04/06/05 23:50 04/08/05 06:50     linear turb 248644.7 13 0.980 55.217 4.889

turb2 04/06/05 23:50 04/08/05 06:50      power turb 249202.6 13 0.980 54.518 4.187

turb3 04/06/05 23:50 04/08/05 06:50      loess turb 247975.5 13 0.980 57.209    NA

turb4 04/06/05 23:50 04/08/05 06:50 logxy:mvue turb 243373.8 13 0.969  0.134 3.782

# Only 2.4% diff between least and greatest. Logxy misses upper data slightly, use power.

ttsplot("sfm",05,turb2)

total(turb2)

start            end  type surr     yhat  n   r2      s    cv

turb2 04/06/05 23:50 04/08/05 06:50 power turb 249202.6 13 0.98 54.518 4.187

Total load = 249203 kg,  CV = 4.19 %  Quality excellent

Storm 509: dumps 20-21: SFM,509,050408,0700,050412,0600

Very good sample coverage

four()

# Go back to midnight to pick up bottle 15 from dump 20

turbsscplot("sfm",05,050408,0000,050412,0600)

# The hysteresis makes this a good candidate for a pairs plot

turbsscplot("sfm",05,050408,0700,050412,0600,type="pairs")

turbsscplot("sfm",05,050408,0000,050408,2050,type="loess",degree=2)

turbsscplot("sfm",05,050408,2050,050412,0600,type="loess",degree=2,span=0.4)

turb1 <- turbsrc("sfm",05,050408,0700,050412,0600)

turb2 <- turbsrc("sfm",05,050408,0700,050412,0600,type="pairs")

turb3 <- turbsrc("sfm",05,050408,0700,050408,2150,050408,0700,050408,2050,type="loess",degree=2)

turb4 <- turbsrc("sfm",05,050408,2200,050412,0600,050408,2050,050412,0600,type="loess",degree=2,span=0.4)

total(turb1,turb2)

start            end   type surr    yhat  n   r2       s    cv

turb1 04/08/05 07:00 04/12/05 06:00 linear turb 2111679 20 0.91 182.369 6.123

turb2 04/08/05 07:00 04/12/05 06:00  pairs turb 2013750 20   NA      NA    NA

ttsplot("sfm",05,turb3,turb4)

total(turb3,turb4)

start            end  type surr      yhat  n    r2      s cv

turb3 04/08/05 07:00 04/08/05 21:50 loess turb  727903.6  6 0.993 95.047 NA

turb4 04/08/05 22:00 04/12/05 06:00 loess turb 1332032.0 15 0.996 64.012 NA

Total load = 2059936 kg  Quality excellent

Storm 510: dump 22-23: SFM,510,050507,1920,050511,1200

Three peaks in this event, missed 4 samples on 1st and highest turb peak

Might need to include samples from dump 21

four()

turbsscplot("sfm",05,050507,0800,050512,1200)

turbsscplot("sfm",05,dump=21:23)

# Include bottles 9-12 from dump 21 to extend range on first peak

turbsscplot("sfm",05,dumps=rep(21:23,c(4,4,2)),bot=c(9:12,6:9,1:2))

# Exclude bottle 14 since it doesn't fit and occurs after storm

turbsscplot("sfm",05,050508,1830,050511,1200)

turb1 <- turbsrc("sfm",05,050507,1920,050508,1820,dumps=rep(21:23,c(4,4,2)),bot=c(9:12,6:9,1:2))

endpoints(turb1)

start       end

-15.12694  81.72460

turbsscplot("sfm",05,dumps=rep(21:23,c(4,4,2)),bot=c(9:12,6:9,1:2),type="power")

turb2 <- turbsrc("sfm",05,050507,1920,050508,1820,dumps=rep(21:23,c(4,4,2)),bot=c(9:12,6:9,1:2),type="power")

turb3 <- turbsrc("sfm",05,050508,1830,050511,1200)

ttsplot("sfm",05,turb2,turb3)

total(turb2,turb3)

start            end   type surr     yhat  n    r2       s     cv

turb2 05/07/05 19:20 05/08/05 18:20  power turb  67332.9 10 0.916 104.821 11.912

turb3 05/08/05 18:30 05/11/05 12:00 linear turb 262664.8 11 0.949  37.482  6.141

Total load = 329998 kg,  CV = 5.46 %  Quality good