qsfm <- read.csv("sfm/Q and rating curves/sfmratingdata0308.csv")

qsfm

date   hy   stg       q

1  16-Nov-02 2003  1.21    0.92

2  15-Dec-02 2003  4.89   93.51

3  16-Dec-02 2003 12.41  544.72

4  31-Dec-02 2003 16.85 1064.72

5   1-Jan-03 2003  6.73  233.23

6  19-Jan-03 2003  2.80   34.71

7   3-Jan-04 2004  5.33  145.92

8  12-Dec-04 2005  2.88   32.21

9  20-Mar-05 2005  5.61  154.76

10 20-Mar-05 2005  8.14  396.93

11 21-Mar-05 2005  6.45  239.79

12 29-Mar-05 2005 10.07  398.81

13  9-Apr-05 2005  9.18  425.16

14 20-Nov-05 2006  1.90    6.38

15  9-Dec-05 2006  2.65   19.82

16 20-Dec-05 2006  8.48  163.55

17 24-Dec-05 2006  4.68   56.68

18 28-Dec-05 2006 16.87  606.48

19  2-Feb-06 2006 10.87  459.99

20  4-Feb-06 2006  6.27  201.09

21 11-Feb-06 2006  2.86   25.66

22  5-Mar-06 2006 15.71  898.23

23 27-Aug-06 2007  1.50    0.31

24 19-Nov-06 2007  1.89    5.29

25 13-Feb-07 2007  4.60   97.60

26 25-Feb-07 2007 10.63  398.65

27 10-Apr-07 2007  2.78   10.95

28 25-Feb-08 2008  6.30  181.24

# Excel model is in Q_CV_SFM050607.xls, sheet "SFM 03-07 int -32 final"

# Reproduce previous Excel model with forced intercept of -32

attach(qsfm[-c(16:18,27:28),])

lm(I(q+32) ~ -1+stg+I(stg^2))

Coefficients:

stg  I(stg^2)

19.266     2.583

plot(stg,q)

x0 <- 0:18

y0 <- 19.266*x0 + 2.583*x0^2 - 32

lines(x0,y0)

# Add 2 new data points in red

detach(2)

points(qsfm$stg[27:28], qsfm$q[27:28], col=2)

# Calc new model

sfmfit <- lm(I(q+32) ~ -1+stg+I(stg^2), data=qsfm[-(16:18),])

Coefficients:

Estimate Std. Error t value Pr(>|t|)

stg       18.6925     2.7153   6.884 5.10e-07 ***

I(stg^2)   2.6237     0.2215  11.848 2.85e-11 ***

---

Residual standard error: 36.14 on 23 degrees of freedom

Multiple R-Squared: 0.992,      Adjusted R-squared: 0.9913

F-statistic:  1432 on 2 and 23 DF,  p-value: < 2.2e-16

# Add new line

x0 <- 0:18

y0 <- 18.692*x0 + 2.6237*x0^2 - 32

lines(x0,y0,col=2,lty=2)

The 2 points seem to fit the old model fairly well and the new curve is

indistinguishable from the old one.

Question is whether or not to recalculate any of the old loads.  HY03-08.  The equation goes negative for stages < 1.427, which occurred in

HY03, HY04, and HY05, but not in HY06-HY08.  So do not apply to HY03-05.  Could be applied to HY08 but the change would be negligible.

Decision: due to limited funding and the negligible change, HY2008 loads will not be recalculated.