Data corrections SFM07 - Jack Lewis

FLO file corrections - Jack Lewis

Had to changed last record of dump 8 to dump 9 because the sample

was listed as dump 9 bottle 1.

Had to add an extra column for those records logged when the OBS3 was in place to hold the place of water temp and provide a constant number of columns per record.

Dump 10 bottles 3 and 4 were pumped when the battery terminals were jiggled during the field form dated 09Feb2007:1620. There were 2 more records that dump, at 1630 and 1640.  Bottle 3 labored through 2 cycles then a plug of algae came thru. I've excluded bottle 3 and assigned bottle 4 to the record at 1630.

Observer record corrections – Jack Lewis

Rating equation –

Use rating based on measurements from HY03 to HY07 -- same one that was used for HY06.

Stage corrections - Jack Lewis

Reconstructed 19Nov2006:1150 to 20Nov2006:1430 from KRW using quadratic method.

A few other short periods of missing data were interpolated.

From Dec 5 to Mar 7 there is a significant stage-related bias:

Coefficients:

Estimate Std. Error t value Pr(>|t|)

(Intercept)  0.069536   0.008520   8.162 1.79e-06 ***

elecstg     -0.016813   0.001473 -11.417 3.79e-08 ***

0.0695-0.0168*seq(1,10)

0.0527 0.0359 0.0191 0.0023 -0.0145

-0.0313 -0.0481 -0.0649 -0.0817 -0.0985

Errors are positive below 4 feet and negative above 4 ft.

Applied two stage-proportional adjustments from 10Dec2006:0800 to 06Mar2007:1640 (no change at 4 ft to -0.10 at 10 ft).

There are 2 large OR differences after 06Mar2007 but these seem like anomalies.  All the others are small.  No other stage adjustments were made.

Turbidity corrections - Jack Lewis

Interpolated the spike at 13Dec2006:1310, as debris was noted during the

following visit 30 minutes later.

There are some lengthy periods of missing or bad turbidity:

DTS12 turbidity data bad until OBS3 was installed on 22Nov2006:1250

27Nov to 08Dec: OBS3 apparently fouled

15Dec to 19Dec: OBS3 fouled then missing after DTS12 installed on 17Dec.

06Jan to 10Feb: DTS12 stuck at 49 NTU (sensor address problem)