HHB DISCHARGE ERROR ANALYSIS HY05-08

# The following function was applied for 05-08

# It’s based on data from 04-08

qfunhhb <- function(s) ifelse(s < 2.89403, 12.606 - 36.016*s + 23.568*s^2, -80 + 31.458*s+ 11.310*s^2)

avg.err <- function(e) (exp(e) - exp(-e))/2

hhbq <- read.csv("hhb/Q and rating curves/HHB Q summary.csv")

names(hhbq) <- c(“HY”,”ID”,”date”,”stg”,”q”,”Code”)

hhbq <- hhbq[hhbq$Code != 8 & hhbq$HY >=2003, ]

hhbq <- hhbq[hhbq$ID != "3-29-05 b", ]

hhbq$sym <- as.character(hhbq$HY %% 1000)

hhbq <- hhbq[hhbq$HY > 2003, ]

attach(hhbq)

plot(stg,q)

x0 <- seq(0.4,11.6,0.2)

y0 <- qfunhhb(x0)

lines(x0,y0,col=1)

# Develop model for log(q)

fit1 <- lm(log(q) ~ log(stg))

fit2 <- lm(log(q) ~ log(stg) + I((log(stg))^2))

fit3 <- lm(log(q) ~ log(stg) + I((log(stg))^2) + I((log(stg))^3))

fit4 <- lm(log(q) ~ log(stg) + I((log(stg))^2) + I((log(stg))^3) + I((log(stg))^4) + I((log(stg))^4))

y1 <- predict(fit1,newdata=data.frame(stg=x0))

y2 <- predict(fit2,newdata=data.frame(stg=x0))

y3 <- predict(fit3,newdata=data.frame(stg=x0))

y4 <- predict(fit4,newdata=data.frame(stg=x0))

lines(x0,exp(y1),col=2)

lines(x0,exp(y2),col=3)

lines(x0,exp(y3),col=4)

lines(x0,exp(y4),col=5)

legend(locator(1),c(”Rating equation”,”log(q) ~ poly(1)”,”log(q) ~ poly(2)”,”log(q)  ~ poly(3)”,”log(q)  ~ poly(4)”),lty=c(1,1,1,1,1),col=1:5)

Unable to find a polynomial in log(stg) close to the original rating.  fit4 is the closest.

q1 <- qfunhhb(stg)

q2 <- exp(predict(fit4,newdata=data.frame(stg)))

sqrt(mean((q-q1)^2))

[1] 48.24829

sqrt(mean((q-q2)^2))

[1] 52.63909

# Maybe this is close enough to work with

m1 <- cbind(log(stg),log(stg)^2,log(stg)^3, log(stg)^4)

HY05

m2 <- cbind(log(hhb05.flo$stg), log(hhb05.flo$stg)^2, log(hhb05.flo$stg)^3, log(hhb05.flo$stg)^4)

weighthalfwidth(fit4, m1, m2, hhb05.flo$q)

[1] 0.2379037

avg.err(0.2379037)

[1] 0.2401542

# Adjust for RMSE difference

0.2401542 * 48.25/52.64

[1] 0.2201261

# Adjusted estimated average error = 22.0%

HY06

m2 <- cbind(log(hhb06.flo$stg), log(hhb06.flo$stg)^2, log(hhb06.flo$stg)^3, log(hhb06.flo$stg)^4)

weighthalfwidth(fit4, m1, m2, hhb06.flo$q)

[1] 0.2539677

avg.err(0.2539677)

[1] 0.2567067

# Adjust for RMSE difference

0.2567067 * 48.25/52.64

[1] 0.2352982

# Adjusted estimated average error = 23.5%

HY07

m2 <- cbind(log(hhb07.flo$stg), log(hhb07.flo$stg)^2, log(hhb07.flo$stg)^3, log(hhb07.flo$stg)^4)

weighthalfwidth(fit4, m1, m2, hhb07.flo$q)

[1] 0.2380182

avg.err(0.2380182)

[1] 0.2402720

# Adjust for RMSE difference

0.2402720 * 48.25/52.64

[1] 0.2202341

# Adjusted estimated average error = 22.0%

HY08

m2 <- cbind(log(hhb08.flo$stg), log(hhb08.flo$stg)^2, log(hhb08.flo$stg)^3, log(hhb08.flo$stg)^4)

weighthalfwidth(fit4, m1, m2, hhb08.flo$q)

[1] 0.2346394

avg.err(0.2346394)

[1] 0.2367984

# Adjust for RMSE difference

0.2367984 * 48.25/52.64

[1] 0.2170502

# Adjusted estimated average error = 21.7%

# ALTERNATIVE: Try a two-part model for log(q)

plot(stg,q)

x0 <- seq(0.4,11.6,0.2)

y0 <- qfunhhb(x0)

lines(x0,y0,col=1)

# Develop model for log(q)

split=2.25

overlap=0.25

fit1 <- lm(log(q) ~ log(stg), subset=stg < split+overlap)

fit2 <- lm(log(q) ~ log(stg) + I(log(stg)^2) + I(log(stg)^3) , subset=stg >= split-overlap)

x1 <- x0[x0<=split+overlap]

x2 <- x0[x0>split-overlap]

y1 <- predict(fit1,newdata=data.frame(stg=x1))

y2 <- predict(fit2,newdata=data.frame(stg=x2))

lines(x1,exp(y1),col=2)

lines(x2,exp(y2),col=3)

# Now the log model is reasonable similar to the original

# Find the intersection of the two log models

> fit1

Call:

lm(formula = log(q) ~ log(stg), subset = stg < split + overlap)

Coefficients:

(Intercept)     log(stg)

-0.1303       5.2485

> fit2

Call:

lm(formula = log(q) ~ log(stg) + I(log(stg)^2) + I(log(stg)^3),     subset = stg >= split - overlap)

Coefficients:

(Intercept)       log(stg)  I(log(stg)^2)  I(log(stg)^3)

-0.5966         7.6525        -3.0547         0.5212

uniroot(

f = function(x) -0.1303 + 5.2485*log(x)

– (-0.5966 + 7.6525*log(x) – 3.0547*log(x)^2 + 0.5212*log(x)^3),

interval = c(1.75,2.25))$root

[1] 1.792115

=====================================

# RMSE

q1 <- qfunhhb(stg)

q2 <- numeric(length(q1))

q2[stg < 1.7921] <- exp(predict(fit1,newdata=data.frame(stg=stg[stg < 1.7921])))

q2[stg > 1.7921] <- exp(predict(fit2,newdata=data.frame(stg=stg[stg > 1.7921])))

sqrt(mean((q-q1)^2))

[1] 48.24829

sqrt(mean((q-q2)^2))

[1] 46.22217

x1 <- stg[stg <= split+overlap]

x2 <- stg[stg >  split-overlap]

m11 <- log(x1)

m12 <- cbind(log(x2),log(x2)^2,log(x2)^3)

# HY05

stg1 <- hhb05.flo$stg[hhb05.flo$stg < 1.7921]

q1 <- hhb05.flo$q[hhb05.flo$stg < 1.7921]

m21 <- log(stg1)

weighthalfwidth(fit1, m11, m21, q1)

[1] 0.2222144

stg2 <- hhb05.flo$stg[hhb05.flo$stg > 1.7921]

q2 <- hhb05.flo$q[hhb05.flo$stg > 1.7921]

m22 <- cbind(log(stg2),log(stg2)^2,log(stg2)^3)

weighthalfwidth(fit2, m12, m22, q2)

[1] 0.09066635

# Now find the weighted average of these two estimates

(0.2222144*sum(q1) + 0.09066635*sum(q2)) / (sum(q1)+sum(q2))

[1] 0.1012944

avg.err(0.1012944)

[1] 0.1014677

# Adjust for RMSE

0.1014677 * 48.25/46.22

[1] 0.1059242

# HY06

stg1 <- hhb06.flo$stg[hhb06.flo$stg < 1.7921]

q1 <- hhb06.flo$q[hhb06.flo$stg < 1.7921]

m21 <- log(stg1)

weighthalfwidth(fit1, m11, m21, q1)

0.2283681

stg2 <- hhb06.flo$stg[hhb06.flo$stg > 1.7921]

q2 <- hhb06.flo$q[hhb06.flo$stg > 1.7921]

m22 <- cbind(log(stg2),log(stg2)^2,log(stg2)^3)

weighthalfwidth(fit2, m12, m22, q2)

[1] 0.08909134

# Now find the weighted average of these two estimates

(0.2283681*sum(q1) + 0.08909134*sum(q2)) / (sum(q1)+sum(q2))

[1] 0.09297031

avg.err(0.09297031)

[1] 0.0931043

# Adjust for RMSE

0.0931043 * 48.25/46.22

[1] 0.09719348

# HY07

stg1 <- hhb07.flo$stg[hhb07.flo$stg < 1.7921]

q1 <- hhb07.flo$q[hhb07.flo$stg < 1.7921]

m21 <- log(stg1)

weighthalfwidth(fit1, m11, m21, q1)

0.2234364

stg2 <- hhb07.flo$stg[hhb07.flo$stg > 1.7921]

q2 <- hhb07.flo$q[hhb07.flo$stg > 1.7921]

m22 <- cbind(log(stg2),log(stg2)^2,log(stg2)^3)

weighthalfwidth(fit2, m12, m22, q2)

0.09295417

# Now find the weighted average of these two estimates

(0.2234364*sum(q1) + 0.09295417*sum(q2)) / (sum(q1)+sum(q2))

[1] 0.1033648

avg.err(0.1033648)

[1] 0.1035490

# Adjust for RMSE

0.1035490 * 48.25/46.22

[1] 0.1080969

# HY08

stg1 <- hhb08.flo$stg[hhb08.flo$stg < 1.7921]

q1 <- hhb08.flo$q[hhb08.flo$stg < 1.7921]

m21 <- log(stg1)

weighthalfwidth(fit1, m11, m21, q1)

0.2242249

stg2 <- hhb08.flo$stg[hhb08.flo$stg > 1.7921]

q2 <- hhb08.flo$q[hhb08.flo$stg > 1.7921]

m22 <- cbind(log(stg2),log(stg2)^2,log(stg2)^3)

weighthalfwidth(fit2, m12, m22, q2)

0.09105905

# Now find the weighted average of these two estimates

(0.2242249*sum(q1) + 0.09105905*sum(q2)) / (sum(q1)+sum(q2))

[1] 0.1016231

avg.err(0.1016231)

[1] 0.1017981

# Adjust for RMSE

0.1017981 * 48.25/46.22

[1] 0.1062691