Data corrections KRW06 - Jack Lewis

Observer record corrections – Jack Lewis

Rating equation –

Stage corrections - Jack Lewis

plot.or("krw",06,reread=F,ylim=c(-0.3,0.2),sdate=051001,edate=060531)

There is a downward trend until Feb 20, when the stage offset was changed.  There is also a trend with stage, both before and after the offset adjustment.

plot.or("krw",06,reread=F,ylim=c(-0.3,0.2), sdate=051001,edate=060220,x="stage")

plot.or("krw",06,reread=F,ylim=c(-0.3,0.2), sdate=060221,edate=060531,x="stage")

So I used a series of y-proportional shift to correct most of the data.  This took care of everything except the two -0.3 anomalies in the above charts.

Reconstructed missing storm of Jan 15 from SFM using linear ratio method.

Turbidity corrections - Jack Lewis

Removed sunspikes and fouling spikes.

Sunspikes were evident on sunny days during most of the year and were generally handled using midnight-to-midnight interpolation.

Tprobe maxed out 28Dec2005:0920-1210.  Fixed-time sampling should have kicked in but did not! Note the hydrograph was starting to round off at the time of the maxout; judging by what happened at SFM during this event, the SSC probably did not rise dramatically during this period and may have started falling about one-third of the way through.  I reconstructed this section by freehand drawing. That is the best hope for estimating the load here.

Tprobe again maxed out 30Dec2005:2030-2130 and possible starting as early as 1900, again with no samples. Again the hydrograph was rounding off so the SSC probably did not get much higher.  I'll accept the flat top as useable, coded questionable.