qkrw <- read.csv("krw/Q and rating curves/KRWratingdata0308.csv")

qkrw

date   hy   stg      q qual

1  10/24/1999 2000  0.43   0.28 fair

2   12/5/1999 2000  1.86  22.86 fair

3   12/2/2000 2001  1.16   7.67 fair

4   1/20/2001 2001  0.98   5.68 poor

5   3/11/2001 2001  1.47  15.84 good

6    4/2/2001 2001  1.36  15.23 fair

7   1/19/2002 2002  1.49  17.35 fair

8   1/22/2002 2002  4.43 141.00 fair

9   2/19/2002 2002  8.95 391.61 poor

10  2/20/2002 2002 12.81 687.56 poor

11  2/20/2002 2002 12.58 644.07 poor

12  2/21/2002 2002  5.13 168.01 fair

13 11/16/2002 2003  0.65   1.23 fair

14 12/15/2002 2003  6.16 205.00 poor

15 12/16/2002 2003 14.37 811.70 poor

16   1/1/2003 2003  6.49 229.70 fair

17  3/16/2003 2003  4.62 127.70 fair

18  3/21/2005 2005  6.69 264.24 poor

19  3/28/2005 2005  8.94 389.91 fair

20  3/29/2005 2005 12.62 608.28 fair

21   4/8/2005 2005 15.08 417.54 fair (but major outlier)

22  4/10/2007 2007  1.88   9.21

23  4/18/2007 2007  3.63  72.54

24   1/7/2008 2008  6.82 296.27

# Notes: Discharge is from NF bridge.  Stage at station KRW

1. Stage for first 12 measurements estimated using Randy's equation:

Skrw = 0.0151*Snfe^2 + 0.6154*Snfe + 0.1898

2. Stage for remaining measurements chosen using lag times based on mean velocity

# Clark's Excel model is in "Q_CV_KRW 2005.xls", based on just 8 measurements

# Had forced intercept of 0

attach(qkrw[13:20,])

lm(q ~ -1+stg+I(stg^2))

stg  I(stg^2)

19.317     2.501

This differs slightly from Clark's model, q= 19.577s + 2.394s^2, because

1. Randy used 3600 feet between stations, I used 4012 as per Kristi

2. I extracted stages from the corrected FLO files, no raw stages

plot(stg,q)

x0 <- 0:15

y0 <- 2.394*(x0^2)+19.577*x0

y1 <- 2.501*(x0^2)+19.317*x0

lines(x0,y0,col=1)

lines(x0,y1,col=2)

legend(locator(1),c("Old equation", "New equation"),lty=1,col=1:2)

# Add early data points in green, new points in blue

detach(2)

attach(qkrw)

plot(stg,q,type="n",xlim=c(0,16))

points(stg[1:12],q[1:12],col=3)

points(stg[13:20],q[13:20])

# Plot omitted outlier in red

points(stg[21],q[21],col=2)

points(stg[22:24], q[22:24], col=4)

# Calc new model

lm(q ~ stg+I(stg^2), data=qkrw[-21,])

Coefficients:

(Intercept)          stg     I(stg^2)

-27.321       26.928        2.134

# Add lines for old and new model

lines(x0,y0,col=1)

y2 <- -27.321 + 26.928*x0 + 2.134*x0^2

lines(x0,y2,col=3,lty=1)

legend(locator(1),c("Clark's old model", "New model HY00-08"),lty=1,col=c(1,3))

Clark's old model doesn't fit any of the low stage data points. New eqn is not bad, but curve needs to stay positive at lower end. Also HY07 measurements raise question of whether there was a shift, but HY08 measurement is shifted the opposite way. Jesse says the 2008 data point is probably correct and represents the impact of a 9ft diameter RW log that impacted the left bank sometime around 2006 or 2007. It was part of a debris jam that formed upstream in 2005.

# Fit lower end to low stage data except the HY07 data point

lm(q ~ -1+stg+I(stg^2), data=qkrw, subset = stg < 2.5 & hy < 2007)

stg  I(stg^2)

-1.304     7.883

plot(stg,q,type="n",xlim=c(0,16))

points(stg[1:12],q[1:12],col=3)

points(stg[13:20],q[13:20])

points(stg[22:24], q[22:24], col=4)

lines(x0,y2,col=2)

y3 <- -1.304*x0 + 7.883*x0^2

lines(x0[x0<6],y3[x0<6],col=1)

legend(locator(1),c("Segment 1", "Segment 2"),lty=1,col=c(1,2))

We can combine these models, by breaking at the lower intersection

uniroot(

function(x) -1.304*x + 7.883*x^2 - (-27.321 + 26.928*x + 2.134*x^2),

interval=c(1,3)

)

$root

[1] 1.325516

plot(stg,q,type="n",xlim=c(0,16))

text(stg[-21],q[-21],hy[-21]%%1000,cex=0.7)

lines(x0,y0,col=3)

x1 <- seq(0,1.32,0.04)

y1 <- -1.304*x1 + 7.883*x1^2

lines(x1,y1,col=1,lty=1)

x2 <- seq(1.32,16,1)

y2 <- -27.321+ 26.928*x2 + 2.134*x2^2

lines(x2,y2,col=2)

legend(locator(1),c("q = -1.304*s + 7.883*s^2","q = -27.321+ 26.928*s + 2.134*s^2","Old rating: q = 19.577*s + 2.394*s^2"),lty=1,col=1:3,cex=0.75)

title("KRW: HY00-08")

# Updated KRW equations for HY03 to HY08 are

Stage < 1.3255:  Q = -1.304s + 7.883s^2

Stage > 1.3255:  Q = -27.321 + 26.928s + 2.134s^2

# Compare to Klein's rating equation for HY03-04

plot(stg,q,type="n",xlim=c(0,16))

text(stg[-21],q[-21],hy[-21]%%1000,cex=0.7)

x1 <- seq(0,1.32,0.04)

y1 <- -1.304*x1 + 7.883*x1^2

lines(x1,y1,col=1,lty=1)

x2 <- seq(1.32,16,1)

y2 <- -27.321+ 26.928*x2 + 2.134*x2^2

lines(x2,y2,col=2)

x0 <- c(x1,x2)

y0 <- (2.9415*(x0^2))+(15.033*x0)+0.3

lines(x0,y0,col=3)

legend(locator(1),c("q = -1.304*s + 7.883*s^2","q = -27.321+ 26.928*s + 2.134*s^2","Klein's rating: q = 2.9415*s^2 + 15.033*s + 0.3"),lty=1,col=1:3,cex=0.75)

# Randy's equation doesn't filt the lower swarm of data points either

# The new curves should probably be applied retroactively to all the data