SFM 2007 Annual SAND and FINES load estimation

sfm07.flo <- read.flo("sfm",07)

sfm07.lab <- read.sand("sfm",07)

sfm07.sed <- merge.flo.sand("sfm",07)

# Now create the ssc column: it will represent only FINES

sfm07.sed$ssc <- sfm07.sed$totssc - sfm07.sed$sand

head(sfm07.sed,6)

chr dump bottle  sand totssc turb turbcode     q labcode1 labcode2    ssc

1  (12/10/06 09:00:00)    6     13 25.08 260.62  191        0 119.2        0        0 235.54

2  (12/10/06 12:40:00)    6     14 25.11 310.07  222        0 168.6        0        0 284.96

3  (12/10/06 16:30:00)    6     15 11.76 192.16  154        0 158.9        0        0 180.40

4  (12/10/06 19:20:00)    6     16  7.96 114.69  104        0 142.1        0        0 106.73

5  (12/13/06 11:00:00)    7      4 27.31 316.58  201        0 204.9        0        0 289.27

6  (12/13/06 13:40:00)    7      5 61.31 515.26  284        0 255.5        0        0 453.95

dim(sfm07.flo)

[1] 37803     9

table(sfm07.flo$turbcode)

0     2     5

21353 16347   103

There are 16347 values of bad turbidity out of 37803 intervals to be estimated

trb07.flo <- sfm07.flo[!is.na(sfm07.flo$turb), ]

flo07.flo <- sfm07.flo[is.na(sfm07.flo$turb), ]

dim(trb07.flo)

[1] 21456     9

dim(flo07.flo)

[1] 16347     9

21456 + 16347

[1] 37803

# Develop model for good turb data

range(trb07.flo$turb)

[1]    2 1621

range(sfm07.sed$turb,na.rm=t)

[1]   62 1600

table(sfm07.sed$labcode2)

0  1  3  5  9

72 30  1  2  1

# Just three bad lab codes to worry about

# (5=low volume, 9=spilled > 15% of sediment)

attach(sfm07.sed)

sfm07.sed[labcode2 > 4, ]

chr dump bottle  sand totssc turb turbcode     q labcode1 labcode2    ssc

84 (02/25/07 02:30:00)   15     11 18.39 210.67  156        0 254.8        0        5 192.28

94 (02/26/07 09:00:00)   15     21 18.82 117.91  170        0 276.1        0        5  99.09

95 (02/26/07 09:30:00)   15     22 21.11 178.39  144        0 273.8        0        9 157.28

turbsscplot("sfm",07,type="logxy",txt="dump")

points(turb[labcode2 > 4], ssc[labcode2 > 4], pch=16,col=2)

detach(2)

# The points aren't terrible outliers.  Keep em, but eliminate bad (NA) turbidity

trb07.sed <- sfm07.sed[!is.na(sfm07.sed$turb), ]

four()

turbsscplot("trb",07,type="power",txt="dump")

turbsscplot("trb",07,type="logxy",txt="dump")

turbsscplot("trb",07,type="linear",txt="dump")

turbsscplot("trb",07,type="loess",txt="dump")

turb1 <- turbsrc("trb",07,060801,0000,070731,2400,type="power",var=F)

turb2 <- turbsrc("trb",07,060801,0000,070731,2400,type="logxy",var=F)

turb3 <- turbsrc("trb",07,060801,0000,070731,2400,type="linear",var=F)

turb4 <- turbsrc("trb",07,060801,0000,070731,2400,type="loess",var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr    yhat   n    r2       s cv

turb1 11/22/06 13:30 06/09/07 12:30      power turb 5863096 104 0.951 115.413 NA

turb2 11/22/06 13:30 06/09/07 12:30 logxy:mvue turb 6235212 104 0.907   0.245 NA

turb3 11/22/06 13:30 06/09/07 12:30     linear turb 5548410 104 0.944 122.279 NA

turb4 11/22/06 13:30 06/09/07 12:30      loess turb 6138164 104 0.953 112.694 NA

> sum(turb3$predssc < 0)

[1] 16252

> sum(turb4$predssc < 0)

[1] 4931

# Can't use linear or loess models: too many negative predictions

# Power model is best fit

total(turb1)

start            end  type surr    yhat   n    r2       s cv

turb1 11/22/06 13:30 06/09/07 12:30 power turb 5863096 104 0.951 115.413 NA

Total fines load by turbidity = 5863096 kg

# A fairly good estimate

====================================================================

# Now fit a discharge sediment rating curve for the remaining data

> range(flo07.flo$q)

[1]   0.8853 149.0000

> range(sfm07.sed$q)

[[1]   76.95 1029.00

attach(sfm07.sed)

sfm07.sed[labcode2 > 4, ]

chr dump bottle  sand totssc turb turbcode     q labcode1 labcode2    ssc

84 (02/25/07 02:30:00)   15     11 18.39 210.67  156        0 254.8        0        5 192.28

94 (02/26/07 09:00:00)   15     21 18.82 117.91  170        0 276.1        0        5  99.09

95 (02/26/07 09:30:00)   15     22 21.11 178.39  144        0 273.8        0        9 157.28

qsscplot("sfm",07)

sfm07.sed[labcode2 > 4, ]

points(q[labcode2 > 4]/35.315, ssc[labcode2 > 4], pch=16,col=2)

detach(sfm07.sed)

The data to be estimated are below 149cfs= 4.2cumecs.

# Limit the regression data to less than 250cfs, and eliminate leverage point

# That will eliminate the high labcodes as well

flo07.sed <- sfm07.sed[sfm07.sed$ssc > 50 & sfm07.sed$q < 250, ]

four()

qsscplot("flo",07,txt="dump")

qsscplot("flo",07,type="power",txt="dump")

qsscplot("flo",07,type="linear",txt="dump")

qsscplot("flo",07,type="loess",txt="dump")

flow1 <- flowsrc("flo",07,060801,0000,070731,2400,type="logxy",var=F)

flow2 <- flowsrc("flo",07,060801,0000,070731,2400,type="power",var=F)

flow3 <- flowsrc("flo",07,060801,0000,070731,2400,type="loess",var=F)

flow4 <- flowsrc("flo",07,060801,0000,070731,2400,type="linear",var=F)

total(flow1,flow2,flow3,flow4)

start            end       type surr      yhat  n    r2       s cv

flow1 09/20/06 00:10 02/10/07 14:00 logxy:mvue flow 1261078.9 39 0.111   0.484 NA

flow2 09/20/06 00:10 02/10/07 14:00      power flow 1377625.4 39 0.082 170.012 NA

flow3 09/20/06 00:10 02/10/07 14:00      loess flow  942736.8 39 0.105 169.891 NA

flow4 09/20/06 00:10 02/10/07 14:00     linear flow 1590300.1 39 0.076 170.580 NA

> sum(flow3$predssc <0)

[1] 7529

> sum(flow4$predssc <0)

[1] 0

# Eliminate loess due to negative predictions

# Very difficult to say which model is best; estimates vary by 26%

# But most prediction is needed at low flows so use the logxy model

total(flow1)

start            end       type surr    yhat  n    r2     s cv

flow1 09/20/06 00:10 02/10/07 14:00 logxy:mvue flow 1261079 39 0.111 0.484 NA

Total fines load by discharge = 1261079 kg

==================================================

# Total fines load

5863096 + 1261079 = 7124175 kg

==================================================

# Compute total load based on the reduced sample (i.e. those with sand fractions only)

trb07.sed$ssc <- trb07.sed$totssc

flo07.sed$ssc <- flo07.sed$totssc

four()

turbsscplot("trb",07,type="power",txt="dump")

turbsscplot("trb",07,type="logxy",txt="dump")

turbsscplot("trb",07,type="linear",txt="dump")

turbsscplot("trb",07,type="loess",txt="dump")

turb1 <- turbsrc("trb",07,060801,0000,070731,2400,type="power",var=F)

turb2 <- turbsrc("trb",07,060801,0000,070731,2400,type="logxy",var=F)

turb3 <- turbsrc("trb",07,060801,0000,070731,2400,type="loess",var=F)

turb4 <- turbsrc("trb",07,060801,0000,070731,2400,type="linear",var=F)

total(turb1,turb2,turb3,turb4)

start            end       type surr    yhat   n    r2       s cv

turb1 11/22/06 13:30 06/09/07 12:30      power turb 6662381 104 0.961 111.244 NA

turb2 11/22/06 13:30 06/09/07 12:30 logxy:mvue turb 7055359 104 0.934   0.198 NA

turb3 11/22/06 13:30 06/09/07 12:30      loess turb 7124901 104 0.964 107.616 NA

turb4 11/22/06 13:30 06/09/07 12:30     linear turb 6374685 104 0.956 117.163 NA

> sum(turb3$predssc < 0)

[1] 0

> sum(turb4$predssc < 0)

[1] 14927

# Cannot use linear model; too many negative predictions

# Remaining models vary by 7%; for consistency with fines load, stay with power model

total(turb1)

start            end  	type surr    yhat   n    r2       s cv

turb1 11/22/06 13:30 06/09/07 12:30      power turb 6662381 104 0.961 111.244 NA

Total load by turbidity = 6662381 kg

four()

qsscplot("flo",07,txt="dump")

qsscplot("flo",07,type="power",txt="dump")

qsscplot("flo",07,type="linear",txt="dump")

qsscplot("flo",07,type="loess",txt="dump")

flow1 <- flowsrc("flo",07,060801,0000,070731,2400,type="logxy",var=F)

flow2 <- flowsrc("flo",07,060801,0000,070731,2400,type="power",var=F)

flow3 <- flowsrc("flo",07,060801,0000,070731,2400,type="loess",var=F)

flow4 <- flowsrc("flo",07,060801,0000,070731,2400,type="linear",var=F)

total(flow1,flow2,flow3,flow4)

start            end       type surr    yhat  n    r2       s cv

flow1 09/20/06 00:10 02/10/07 14:00 logxy:mvue flow 1666369 39 0.059   0.499 NA

flow2 09/20/06 00:10 02/10/07 14:00      power flow 1730114 39 0.052 205.895 NA

flow3 09/20/06 00:10 02/10/07 14:00      loess flow 1255340 39 0.074 205.895 NA

flow4 09/20/06 00:10 02/10/07 14:00     linear flow 2001682 39 0.046 206.455 NA

sum(flow3$predssc < 0)

sum(flow4$predssc < 0)

> sum(flow3$predssc < 0)

[1] 0

> sum(flow4$predssc < 0)

[1] 0

# Again it is very difficult to say which model is best; estimates vary by 59%

# But most prediction is needed at low flows so use the logxy model

# which has the virtue of being in the middle of the range of estimates

total(flow1)

start            end       type surr    yhat  n    r2     s cv

flow1 09/20/06 00:10 02/10/07 14:00 logxy:mvue flow 1666369 39 0.059 0.499 NA

Total load by discharge = 1666369 kg

==================================================

# Total load

6662381 + 1666369 = 8328750 kg

This total differs quite a lot (15% higher) from the previous estimate:

7251068 based on sum of storms and inter-storm loads

# The range of this data is quite limited relative to the entire data set

# The entire data set has many samples between 20 and 100 NTU (10-100 mg/L)

sfm07.lab <- read.lab("sfm",07)

sfm07.sed.all <- merge.flo("sfm",07)

sfm07.sed.all <- sfm07.sed.all[!is.na(sfm07.sed.all$turb), ]

sfm07.sed <- sfm07.sed[!is.na(sfm07.sed$turb), ]

plot(sfm07.sed.all$turb,sfm07.sed.all$ssc,log="xy",pch=16)

points(sfm07.sed$turb,sfm07.sed$totssc,col=2)

logline(sfm07.sed.all$turb,sfm07.sed.all$ssc)

logline(sfm07.sed$turb,sfm07.sed$totssc,col=2)

legend(locator(1),c("all samples","sand frac samples"), lty=1,pch=c(16,1),col=1:2, cex=0.8)

# This makes it clear why using only bottles with sand fractions gives a higher load

# The red line must be extrapolated down and overestimates most of the concentrations

# This total should be used only to get a ratio of sand to total load

# The idea is to estimate both the fines and total load using similar/compatible data

==================================================

==================================================

Total load 8328750

Total fines 7124175

Total sand  1204575

Percent sand = 14.5%

Percent fines= 85.5%

Applying these percentages to the more reliable total load (7251068) gives

Total fines = 6202357

Total sand = 1048711