KRW08 Event Loads

stn,storm,sdate,stime,edate,etime

KRW,801,080104,0310,080106,1550 1007376 kg  Excellent

KRW,802,080108,0510,080111,1200  241621 kg  Very good

KRW,803,080130,2230,080202,0640 1528455 kg  Very good

KRW,804,080202,0650,080206,0000  711907 kg  Very good

KRW,805,080224,0100,080227,0000  215491 kg  Very good

KRW,806,080312,2000,080317,1200  141866 kg  Very good

-----------------------------------------------------

SUM of events:                  3846716 kg  Very good

-----------------------------------------------------

krw08.flo <- read.flo("krw",08)

krw08.lab <- read.lab("krw",08)

mismatches("krw",08)

krw08.sed <- merge.flo("krw",08)

Storm 801: dump 6: KRW,801,080104,0310,080106,1550

Excellent turbidity and excellent sample coverage

four()

turbsscplot("krw",08,080104,0310,080106,1550)

turbsscplot("krw",08,080104,0310,080104,1220)

turbsscplot("krw",08,080104,1230,080106,1550)

turbsscplot("krw",08,dump=6,bot=4:7,type="power")

turb1 <- turbsrc("krw",08,080104,0310,080106,1550)

turb2 <- turbsrc("krw",08,080104,0310,080106,1550,type="pairs")

time1 <- lineartime("krw",08,080104,0310,080106,1550)

turb3 <- turbsrc("krw",08,080104,0310,080104,1220)

turb4 <- turbsrc("krw",08,080104,1230,080106,1550)

total(turb1,time1,turb2)

start            end   type surr      yhat  n    r2      s   cv

turb1 01/04/08 03:10 01/06/08 15:50 linear turb  984637.8 15 0.971 81.696 6.52

time1 01/04/08 03:10 01/06/08 15:50 linear time 1008060.2 15    NA     NA   NA

turb2 01/04/08 03:10 01/06/08 15:50  pairs turb 1023075.8 15    NA     NA   NA

# The pairs plot is pretty good except it's not continuous between samples 8 & 9

# the linear time plot is pretty good but truncates the peak

total(turb3,turb4)

start            end   type surr     yhat  n    r2      s    cv

turb3 01/04/08 03:10 01/04/08 12:20 linear turb 317056.5  5 0.993 56.170 2.969

turb4 01/04/08 12:30 01/06/08 15:50 linear turb 700871.8 10 0.998 19.694 2.482

Total load = 1017928 kg,  CV = 1.94 %

ttsplot("krw",08,turb3,turb4)

The peak isn't right. Try using cubictime to fit the peak.  Need a power

function for the rising limb to keep it positive.

turb5 <- turbsrc("krw",08,080104,0310,080104,1040,dump=6,bot=4:7,type="power")

turb6 <- turbsrc("krw",08,080104,1350,080106,1550,dump=6,bot=9:16)

time2 <- cubictime("krw",08,080104,1050,080104,1340)

ttsplot("krw",08,turb5,time2,turb6)

total(turb5,time2,turb6)

start            end   type surr     yhat n    r2      s    cv

turb5 01/04/08 03:10 01/04/08 10:40  power turb 103169.2 4 0.997 31.052 3.340

time2 01/04/08 10:50 01/04/08 13:40 linear time 376863.3 4    NA     NA    NA

turb6 01/04/08 13:50 01/06/08 15:50 linear turb 527343.7 8 0.997 21.560 5.191

Total load = 1007376 kg  Quality excellent

Storm 802: dumps 6-7: KRW,802,080108,0510,080111,1200

Excellent turbidity and very good sample coverage

Extend time to pick up dump 6 bottle 19 from previous recession

two()

turbsscplot("krw",08,080107,1200,08011,1200)

turbsscplot("krw",08,080108,0510,08011,1200)

turb1 <- turbsrc("krw",08,080108,0510,080111,1200)

ttsplot("krw",08,turb1)

total(turb1)

start            end   type surr     yhat n    r2      s    cv

turb1 01/08/08 05:10 01/11/08 12:00 linear turb 241620.5 7 0.988 14.057 4.799

Total load = 241621 kg,  CV = 4.84 %  Very good

#For comparison, including dump 6, bot 19 raises the total by  5%

turb2 <- turbsrc("krw",08,080108,0510,080111,1200,080107,1200,080111,1200)

total(turb2)

start            end   type surr     yhat n    r2      s    cv

turb2 01/08/08 05:10 01/11/08 12:00 linear turb 254409.8 8 0.972 19.879 5.859

Total load = 254410 kg,  CV = 5.86 %

Storm 803: dump 8: KRW,803,080130,2230,080202,0640

Excellent turbidity and sample coverage

turbsscplot("krw",08,080130,2230,080202,0640)

# Outlier bottle 12 is colored yellow in Chris' spreadsheet with a note

# saying: "used QC 100 as a silt filter; used 943 as the QC"

# Not sure the significance, but bottles 10 and 12 balance each other

turb1 <- turbsrc("krw",08,080130,2230,080202,0640)

ttsplot("krw",08,turb1)

total(turb1)

start            end   type surr    yhat  n   r2      s    cv

turb1 01/30/08 22:30 02/02/08 06:40 linear turb 1528455 12 0.94 78.345 4.666

Total load = 1528455 kg,  CV = 4.67 %  Quality very good

Storm 804: dump 8-9: KRW,804,080202,0650,080206,0000

Excellent turbidity and sample coverage

Extend to include bot 19 from before and bot 4 from after

four()

turbsscplot("krw",08,080202,0000,080206,1400)

turbsscplot("krw",08,080202,0650,080206,0000)

turbsscplot("krw",08,dump=8,bot=19:22)

turbsscplot("krw",08,dump=c(8,8,rep(9,4)),bot=c(23:24,1:4))

Sample 22 occurred right at the peak, so can use a regression for the rise

and lineartime or cubictime for the recession.  Ending SSC should be about the

mean SSC of bottles 3 and 4.

turb1 <- turbsrc("krw",08,080202,0650,080203,0310,dump=8,bot=19:22)

time1 <- lineartime("krw",08,080203,0320,080206,0000,ssc2=38)

time2 <- cubictime("krw",08,080203,0320,080206,0000,ssc2=38)

ttsplot("krw",08,turb1,time1)

# The segment between bots 24 and 1 should be curved, so this overestimates

total(turb1,time1)

start            end   type surr     yhat n    r2      s    cv

turb1 02/02/08 06:50 02/03/08 03:10 linear turb 206367.5 4 0.991 18.611 3.896

time1 02/03/08 03:20 02/06/08 00:00 linear time 514142.9 6    NA     NA    NA

Total load = 720510 kg

ttsplot("krw",08,turb1,time2)

Shouldn't have a hump on the recession

turb2 <- turbsrc("krw",08,080203,0540,080206,0000,dump=c(8,8,rep(9,4)),bot=c(23:24,1:4))

time2 <- lineartime("krw",08,080203,0320,080203,0530)  # Connects bots 22-23

ttsplot("krw",08,turb1,time2,turb2)

total(turb1,time2,turb2)

start            end   type surr      yhat n    r2      s    cv

turb1 02/02/08 06:50 02/03/08 03:10 linear turb 206367.53 4 0.991 18.611 3.896

time2 02/03/08 03:20 02/03/08 05:30 linear time  83179.32 2    NA     NA    NA

turb2 02/03/08 05:40 02/06/08 00:00 linear turb 422360.02 6 0.980 28.199 6.086

Total load = 711907 kg  Very good

Storm 805: dump 9: KRW,805,080224,0100,080227,0000

Excellent turbidity, good sample coverage (except no sample at peak)

The hydrograph has a weird jump on 24Feb:0800, a speculative adjustment was made

four()

turbsscplot("krw",08,080224,0100,080227,0000)

turbsscplot("krw",08,080224,0100,080227,0000,type="power")

turbsscplot("krw",08,080224,0100,080227,0000,type="logxy")

turbsscplot("krw",08,080224,0100,080227,0000,type="sqrt")

The power function is the best fit

turb1 <- turbsrc("krw",08,080224,0100,080227,0000,type="power")

ttsplot("krw",08,turb1)

total(turb1)

start     end  type surr     yhat  n    r2     s    cv

turb1 02/24/08 01:00 2/27/08 power turb 215490.7 10 0.995 9.798 1.474

Total load = 215491 kg,  CV = 1.47 kg,  CV = 1.48 %  Very good

Storm 806: dump 10: KRW,806,080312,2000,080317,1200

Excellent turbidity and sample coverage.

There are a couple of weird jumps in the hydrograph that were removed

Include sample 10 from before the initial rise and 20 from

four()

turbsscplot("krw",08,dump=10,bot=10:20)

turbsscplot("krw",08,dump=10,bot=10:13)

turbsscplot("krw",08,dump=10,bot=10:13,type="power")

turbsscplot("krw",08,dump=10,bot=14:20)

turb1 <- turbsrc("krw",08,080312,2000,080317,1200,dump=10,bot=10:20)

ttsplot("krw",08,turb1)

total(turb1)

start            end   type surr   yhat  n    r2      s    cv

turb1 03/12/08 20:00 03/17/08 12:00 linear turb 145493 11 0.977 11.387 4.453

Total load = 145493 kg,  CV = 4.45 %

This can be improved upon by splitting the hyrographs

turb2 <- turbsrc("krw",08,080312,2000,080314,2100,dump=10,bot=10:13,type="power")

turb3 <- turbsrc("krw",08,080314,2110,080317,1200,dump=10,bot=14:20)

ttsplot("krw",08,turb2,turb3)

# The curves don't meet up very well, but they intersect at a higher turbidity

# Apply the regression right up to sample 14

turb2 <- turbsrc("krw",08,080312,2000,080315,0400,dump=10,bot=10:13,type="power")

turb3 <- turbsrc("krw",08,080315,0410,080317,1200,dump=10,bot=14:20)

total(turb2,turb3)

start            end   type surr     yhat n    r2     s    cv

turb2 03/12/08 20:00 03/15/08 04:00  power turb  34843.7 4 0.998 3.584 4.396

turb3 03/15/08 04:10 03/17/08 12:00 linear turb 102381.8 7 0.984 9.606 4.538

Total load = 141866 kg,  CV = 3.52 %  Very good

# The 3 estimates are within 3% of each other despite esthetic differences.